| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0060 |
| --- | --- |

| Title: | Response to the EE3-D0046 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Robert, F. Le Léannec, T. Poirier 75 Avenue des Champs Blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33 2 99 27 34 41 {antoine.robert, fabrice.leleannec, tangi.poirier}@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution responds to the Exploratory Experiments 3, part D0046. It tests independently the features of D0046, then reduces the complexity by adjusting the number and the place in the FRUC lists of the added candidates and by removing several candidates from the FRUC lists of sub-blocks of merge CUs. High precision refinement improves only Low Delay P configuration (0.13%), but increases the encoding and decoding runtimes (1% to 3%). Candidate addition provides BD-rate gains (0.23% RA, 0.64% LDB and 0.77% LDP) with increases of encoding (1% RA, 5% LDB and 3% LDP) and decoding (1-2%) runtimes. However, in Test #3, the coding gains are preserved even improved with a complexity reduction of both encoding and decoding runtimes that remain close to 100%.

Overall, four (2 AMVP and 2 spatial) and two spatial additional motion vector candidates are used for AMVP and merge CUs respectively. Two additional spatial candidates are used for each sub-block of a merge CU, but several of initial ones have been removed (zero and unilateral motion vectors, bottom-right co-located candidates and up to 24 ATMVP candidates).

It is suggested that the FRUC improvements corresponding to Test #3 are integrated into the JEM.

# Introduction

The technology proposed in D0046 [1] initially contains:

- Increase of the precision in FRUC until the finer internal one (1/16-pel accuracy is used for Motion Compensation);

- Addition of Motion Vector Candidates in FRUC, by adding the 2 AMVP candidates in the front of the FRUC candidate list for AMVP CUs, and up to 5 Spatial Candidate(s) in the end of FRUC candidate list, both for entire CUs, and FRUC CUs’ sub-blocks.

The Exploration Experiments 3 [2] recommends to answer the following questions:

- Individual performance of each modification needs to be reported.

  - Increase of the precision in FRUC up to 1/16-pel accuracy?

  - Candidate addition as initially proposed?

- How many "Additional Motion Vector Candidates" in total?

- Can some candidates (ex. spatial) be re-ordered?

- Can some candidates (among initial FRUC candidates or added spatial candidates) be removed?

The associated tests are described in the following table.

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.1 | Increase of the FRUC precision until the finer internal one. | Technicolor |  |
| 1.2 | Candidate addition as initially proposed. | Technicolor |  |
| 2 | Evaluate the performance using various number of additional spatial MV candidates (3, 2, 1 or 0 added spatial candidates), at various positions in the FRUC list. | Technicolor |  |
| 3 | Test 2, plus removing some other candidates for merge sub-blocks: limiting to 4 ATMVP candidates, remove unilateral MV, zero MV, and some temporal MV | Technicolor |  |

# Test #1

Test #1 aims at evaluating each proposed improvement of D0046 [1] independently. This test is thus divided into two parts corresponding to (i) increasing the precision of the FRUC refinement until the finer internal one (currently 1/16-pel accuracy), and (ii) adding motion vector candidates in the FRUC lists.

BD-rate performances of each of these improvements are presented and discussed. All provided results have been obtained with the JEM-4.0 [3].

## Test #1.1 - High precision FRUC

As presented in D0046 [1], the final single FRUC refinement step has been updated to exactly fit the motion compensation precision. This step has been replaced by a loop of single refinement steps at a finer precision to the motion compensation one. For now, the motion compensation precision has been extended to 1/16-pel accuracy [5], thus this final step performs a refinement at 1/8-pel then at 1/16-pel accuracy.

As shown in Table 1, High precision refinement provides 0.13% BD-rate gain in Luma for Low Delay P configuration. The impact is negligible in Random Access and Low Delay B configurations.

However, this feature impacts slightly the encoding (increased by 1% to 3%) and decoding runtimes (increased by 1% to 3%).

Table 1: Impact of high precision refinement over FRUC in JEM4.0

## Test #1.2 - Candidate addition

D0046 [1] proposes to add several candidates into the FRUC lists.

For AMVP CUs, AMVP candidates are added at the beginning of the list. For all entire CUs (AMVP and merge), motion vectors from five spatial neighbors (top, left, top-left, top-right and bottom-left) are added at the end of the list and in the same way as for those already present for sub-blocks. Then, the FRUC list is pruned to obtain a list of unique motion vectors (the proposed additional candidates are in italics):

- AMVP candidates if current CU is in AMVP mode,

- merge candidates,

- FRUC unilateral candidates, and

- top, left, top-left, top-right and bottom-left neighboring motion vectors.

And, for sub-blocks of a merge CU, the three missing spatial candidates (top-left, top-right and bottom-left) are added at the end of the list, which becomes a list of unique motion vectors taken from:

- the best candidate of the corresponding entire CU,

- a zero motion vector,

- scaled versions of co-located motion vectors,

- scaled versions of bottom-right co-located motion vectors,

- ATMVP candidates (up to 16),

- ATMVPext candidates (up to 16),

- scaled version of a FRUC unilateral candidate,

- top and left neighboring motion vectors, and

- top-left, top-right and bottom-left neighboring motion vectors.

Results are reported in Table 2. Adding candidates to the FRUC list on top of the JEM4.0 provides coding gains of 0.23% in Luma and around 0.30% in Chroma in Random Access configuration. It gives 0.64% gain in Luma and around 0.93% in Chroma in Low Delay B configuration. Finally, 0.77% gain in Luma and around 1.08% gain in Chroma are obtained in Low Delay P configuration.

However, this feature impacts slightly the encoding time, increased by 1% in Random Access configuration, by 5% in Low Delay B configuration, and by 3% in Low Delay P configuration. The decoding time is also slightly impacted whatever the used configuration (increased by 1% to 2%).

Table 2: Impact of candidate addition over FRUC in JEM4.0

# Test #2

Test #2 aims at adjusting the number of added candidates and their place in the FRUC list independently for each type of block (AMVP CUs, merge CUs, or sub-blocks of merge CUs). To this end, statistics on selected candidates from the FRUC list have been performed. These statistics have then been refined to focus more particularly on the spatial candidates.

First, for each type of blocks (AMVP, merge CUs and sub-blocks of merge CUs), the statistics on the type of selected candidates over all selected ones can be obtained as presented on the subsequent Tables.

|  | AMVP | Merge | Unilateral | Spatial |
| --- | --- | --- | --- | --- |
| RA | 61.58% | 26.97% | 5.32% | 6.13% |
| LDB | 72.54% | 17.27% | 5.88% | 4.30% |
| LDP | 72.16% | 19.12% | 4.38% | 4.34% |
| Average | 68.76% | 21.12% | 5.19% | 4.93% |

Table 3: Selection percentage of each type of candidates for AMVP CUs

From Table 3, it can be seen that the added AMVP candidates for AMVP CUs are largely selected (68%), three times more than the merge candidates. Such AMVP candidates are really relevant and need to stay at the first place of the associated FRUC list.

|  | Merge | Unilateral | Spatial |
| --- | --- | --- | --- |
| RA | 81.02% | 12.98% | 6.00% |
| LDB | 78.81% | 16.30% | 4.89% |
| LDP | 88.32% | 5.33% | 6.35% |
| Average | 82.71% | 11.54% | 5.75% |

Table 4: Selection percentage of each type of candidates for merge CUs

For AMVP (Table 3) and merge CUs (Table 4), the added spatial candidates are not so much selected (around 5-6%), but in the same way as the unilateral candidates for AMVP CUs and twice less for merge CUs. These spatial candidates can be kept and at the same place in the FRUC lists (the last one).

|  | Start | Zero | Temp0 | Temp1 | ATMVP | ATMVPext | Unilateral | Spatial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA | 79.90% | 0.76% | 4.72% | 1.53% | 1.55% | 2.84% | 0.56% | 8.13% |
| LDB | 85.33% | 0.63% | 4.67% | 1.43% | 0.37% | 1.67% | 0.50% | 5.38% |
| LDP | 81.33% | 1.39% | 3.17% | 1.08% | 0.18% | 1.72% | 0.64% | 10.49% |
| Average | 82.19% | 0.93% | 4.18% | 1.35% | 0.70% | 2.08% | 0.57% | 8.00% |

Table 5: Selection percentage of each type of candidates for sub-blocks of merge CUs
(Temp0 stands for the co-located candidates and Temp1 for the bottom-right co-located ones)

For sub-blocks of merge CUs, after the motion vector candidate coming from the entire merge CU (82%), this is the spatial candidates that are mostly selected (8%, twice more than the following one). Thus, from these statistics, it is clear that the spatial candidates are useful and must be promoted to the second place of the FRUC lists of sub-blocks of merge CUs.

Then, by focusing on spatial candidates, it is possible to output statistics on the selection of each spatial candidate independently among all selected spatial candidates as presented on the following Table 6, Table 7 and Table 8.

|  | Top | Left | Top-left | Top-right | Bottom-left |
| --- | --- | --- | --- | --- | --- |
| RA | 46.38% | 50.92% | 2.70% | 0% | 0% |
| LDB | 46.20% | 49.86% | 3.93% | 0% | 0% |
| LDP | 47.37% | 50.44% | 2.19% | 0% | 0% |
| Average | 46.65% | 50.41% | 2.94% | 0% | 0% |

Table 6: Selection percentage of each spatial candidates for AMVP CUs

|  | Top | Left | Top-left | Top-right | Bottom-left |
| --- | --- | --- | --- | --- | --- |
| RA | 47.26% | 48.85% | 3.88% | 0% | 0% |
| LDB | 46.96% | 48.18% | 4.86% | 0% | 0% |
| LDP | 50.18% | 47.67% | 2.16% | 0% | 0% |
| Average | 48.13% | 48.23% | 3.63% | 0% | 0% |

Table 7: Selection percentage of each spatial candidates for merge CUs

Table 6 and Table 7 show, for AMVP and merge CUs, that the two last spatial candidates are never present and so never selected, and that the third one is only selected around 3% from all the spatial candidates. Spatial candidates for such CUs are selected 5-6% of the time, so the top-left candidate is selected less than 0.20% of the whole candidates. Thus it can be removed from the FRUC lists of AMVP and merge CUs.

|  | Top | Left | Top-left | Top-right | Bottom-left |
| --- | --- | --- | --- | --- | --- |
| RA | 43.47% | 36.46% | 8.22% | 7.73% | 4.12% |
| LDB | 46.08% | 37.88% | 6.73% | 5.90% | 3.41% |
| LDP | 52.55% | 37.41% | 4.20% | 3.65% | 2.19% |
| Average | 47.37% | 37.25% | 6.38% | 5.76% | 3.24% |

Table 8: Selection percentage of each spatial candidates for sub-blocks of merge CUs

For sub-blocks of merge CUs, on Table 8, the two first spatial candidates are largely selected (more than 37%), the two following ones are selected around 6% and the last one about 3%. Once again, these spatial candidates are selected 8% of the whole candidates and so the bottom-left one less than 0.25%. This last candidate can be removed from the FRUC lists of sub-blocks of merge CUs with a limited impact.

To sum up this statistical study, the FRUC list becomes, for AMVP and merge CUs, a list of unique motion vectors from (the proposed additional candidates are in italics):

- AMVP candidates if current CU is in AMVP mode,

- merge candidates,

- FRUC unilateral candidates, and

- top and left neighboring motion vectors.

And, for sub-blocks of merge CUs, the FRUC list becomes a list of unique motion vectors taken from the following ordered set:

- the best candidate of the corresponding entire CU,

- top and left neighboring motion vectors,

- top-left and top-right neighboring motion vectors,

- a zero motion vector,

- scaled versions of co-located motion vectors,

- scaled versions of bottom-right co-located motion vectors,

- ATMVP candidates (up to 16),

- ATMVPext candidates (up to 16), and

- scaled version of a FRUC unilateral candidate.

Results are reported in Table 9. By limiting the number of added spatial candidates and by moving them to the second place of the FRUC list for sub-blocks of merge CUs, the coding gains presented in test #1.2 are conserved (even slightly improved).

This feature decreases the encoding time close to 100%. However, the decoding time is not clearly modified and stays slightly greater than 100%.

Table 9: Controlling the number and the place of added candidates over FRUC in JEM4.0

# Test #3

Test #3 aims at reducing the complexity of test #2 by removing several candidates from the FRUC lists. As done previously, finer statistics can be obtained for each type of candidates. Such statistics may allow discarding the least selected candidates, thus reducing the complexity without a huge impact on observed coding gains.

For entire CUs (AMVP and merge), from Table 3 and Table 4, one can see that all types of candidate are selected more than 5% showing that all these types are useful.

As for spatial candidates in test #2, it possible to refine statistics for particular candidate types as the four unilateral ones of AMVP and merge CUs, presented on Table 10 and Table 11.

|  | (0, 0) | (w/2, 0) | (0, h/2) | (w/2, h/2) |
| --- | --- | --- | --- | --- |
| RA | 51.62% | 23.90% | 15.65% | 8.83% |
| LDB | 54.62% | 23.90% | 14.21% | 7.26% |
| LDP | 57.67% | 23.45% | 12.64% | 6.24% |
| Average | 54.64% | 23.75% | 14.17% | 7.44% |

Table 10: Selection percentage of each unilateral candidates for AMVP CUs

|  | (0, 0) | (w/2, 0) | (0, h/2) | (w/2, h/2) |
| --- | --- | --- | --- | --- |
| RA | 51.97% | 23.59% | 14.26% | 10.18% |
| LDB | 57.61% | 21.51% | 13.00% | 7.88% |
| LDP | 59.56% | 21.43% | 12.84% | 6.15% |
| Average | 56.38% | 22.18% | 13.37% | 8.07% |

Table 11: Selection percentage of each unilateral candidates for merge CUs

Both for AMVP (Table 10) and merge CUs (Table 11), each unilateral candidate is selected more than 6% of the time. These unilateral candidates need to be kept at the same place in these CUs’ FRUC lists.

Then, for sub-blocks of merge CUs, and from Table 5, it can be seen that the zero motion vector and the unilateral candidate are selected less than 1%. Both these candidates can be removed from the FRUC lists with a limited impact on performances.

As previously, it is also possible to refine statistics for particular candidate types as the temporal ones. Corresponding statistics are presented, for co-located candidates, on Table 12 and, for bottom-right co-located candidates, on Table 13. On these Tables, results are presented in descending ref_idx order (as in the FRUC lists) where odd candidates corresponds to the co-located (or bottom-right co-located respectively) motion vector from reference frame list L0 and even ones from reference frame list L1. Here, candidate 7 corresponds to the motion vector picked from L0 of the co-located block inside the frame at ref_idx0, and candidate 8 to the motion vector from L1 of the same co-located block inside frame at ref_idx0, then the same for candidates 5 and 6 but inside the frame at ref_idx1, …

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA | NA | NA | NA | NA | 24.52% | 13.67% | 37.49% | 24.32% |
| LDB | 8.35% | 3.58% | 10.06% | 4.29% | 15.66% | 6.19% | 38.78% | 13.07% |
| LDP | 16.28% | NA | 18.26% | NA | 23.19% | NA | 42.27% | NA |
| Average | 12.31% | 3.58% | 14.16% | 4.29% | 21.12% | 9.93% | 39.51% | 18.69% |

Table 12: Selection percentage of each co-located candidate for sub-blocks of merge CUs

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA | NA | NA | NA | NA | 24.52% | 13.83% | 36.16% | 25.48% |
| LDB | 9.56% | 4.12% | 11.57% | 5.07% | 16.76% | 6.70% | 33.85% | 12.36% |
| LDP | 17.97% | NA | 20.26% | NA | 26.53% | NA | 35.24% | NA |
| Average | 13.76% | 4.12% | 15.92% | 5.07% | 22.60% | 10.27% | 35.08% | 18.92% |

Table 13: Selection percentage of each bottom-right co-located candidate for sub-blocks of merge CUs

These Table 12 and Table 13 show that all these temporal candidates are selected more than 4% of the time. But, from Table 5, one can get that co-located candidates are selected 4.18% compared to bottom-right co-located ones with only 1.35%, i.e. more than 3 times less. If this factor is applied on Table 13 to be able to compare it with Table 12, then this shows that the bottom-right co-located candidates are only selected between 1.33% and 11.33%, so close to the third least selected of the co-located candidates (9.93%). Thus, since all bottom-right co-located candidates are almost less selected than those of the co-located ones, it is possible to remove these bottom-right co-located candidates from the FRUC lists of sub-blocks of merge CUs with a limited impact on BD-rates enhancements.

Finally, statistics of all ATMVP candidates can also be refined. These statistics are presented, for ATMVP candidates, on Table 14 and, for ATMVPext candidates, on Table 15.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA | 83.9% | 6.34% | 4.26% | 3.03% | 1.69% | 1.03% | 0.39% | 0.37% | 1.66% | 0.35% | 0.38% | 0.53% | 1.07% | 0.36% | 0.52% | 0.71% |
| LDB | 82.2% | 5.46% | 4.49% | 3.55% | 1.03% | 0.71% | 0.49% | 0.25% | 2.19% | 0.25% | 0.30% | 0.21% | 0.56% | 0.21% | 0.35% | 0.48% |
| LDP | 85.9% | 4.40% | 3.15% | 2.78% | 0.64% | 0.54% | 0.19% | 0.16% | 0.85% | 0.16% | 0.18% | 0.13% | 0.35% | 0.11% | 0.20% | 0.42% |
| Average | 84.0% | 5.40% | 3.97% | 3.12% | 1.12% | 0.76% | 0.35% | 0.26% | 1.57% | 0.25% | 0.29% | 0.29% | 0.66% | 0.23% | 0.35% | 0.54% |

Table 14: Selection percentage of each ATMVP candidates for sub-blocks of merge CUs

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA | 77.8% | 9.19% | 4.32% | 6.11% | 1.20% | 2.61% | 0.39% | 0.75% | 0.62% | 0.43% | 0.29% | 0.48% | 0.46% | 0.31% | 0.28% | 2.08% |
| LDB | 84.7% | 5.91% | 3.21% | 4.71% | 0.67% | 1.26% | 0.18% | 0.34% | 0.30% | 0.23% | 0.12% | 0.30% | 0.15% | 0.11% | 0.22% | 0.86% |
| LDP | 93.2% | 2.94% | 1.55% | 1.82% | 0.19% | 0.34% | 0.12% | 0.12% | 0.08% | 0.06% | 0.04% | 0.06% | 0.04% | 0.03% | 0.06% | 0.26% |
| Average | 85.2% | 6.01% | 3.03% | 4.21% | 0.69% | 1.41% | 0.23% | 0.40% | 0.33% | 0.24% | 0.15% | 0.28% | 0.22% | 0.15% | 0.19% | 1.07% |

Table 15: Selection percentage of each ATMVPext candidates for sub-blocks of merge CUs

Both on Table 14 and Table 15, the first ATMVP or ATMVPext candidate is the most selected one (84%), but the three following ones are also selected more than 3% compared to all other that are selected less than 1.5% (and less than 1% for most of them). It may be interesting to discard the twelve last AMVP and ATMVPext candidates to save complexity without affecting the performances.

To summarize this second statistical study, for AMVP and merge CUs, the FRUC list stays as in test #2 a list of unique motion vectors from (the proposed additional candidates are in italics):

- AMVP candidates if current CU is in AMVP mode,

- merge candidates,

- FRUC unilateral candidates, and

- top and left neighboring motion vectors.

And, for sub-blocks of merge CUs, the FRUC list becomes a list of unique motion vectors taken from the following ordered set:

- the best candidate of the corresponding entire CU,

- top and left neighboring motion vectors,

- top-left and top-right neighboring motion vectors,

- a zero motion vector,

- scaled versions of co-located motion vectors,

- scaled versions of bottom-right co-located motion vectors,

- up to 4 ATMVP candidates,

- up to 4 ATMVPext candidates, and

- scaled version of a FRUC unilateral candidate.

Results are reported in Table 16. By removing some candidates from FRUC lists of sub-blocks of merge CUs, the coding gains presented in tests #1.2 and #2 are conserved (even slightly improved).

As expected, the encoding and decoding runtimes are decreased compared to tests #1.2 and #2. With this feature, the encoding time stays at 100% in Random Access configuration, and is increased to 104% in Low Delay B configuration, and to 103% in Low Delay P configuration. The decoding time is decreased under 100% whatever the used configuration.

Table 16: Removing some sub-blocks candidates over FRUC in JEM4.0

# Conclusion

The Exploratory Experiments have been performed with independent features of D0046. Then, in order to reduce the complexity of the candidate addition, the number and the place of spatial candidates have been updated. And, in a final test, still to reduce the complexity, several candidates have been discarded or limited in the FRUC lists of sub-blocks of merge CUs.

The high precision refinement of D0046 provides 0.13% BD-rate gain in Luma for Low Delay P configuration, but is negligible for Random Access and Low Delay B configurations. However, this feature impacts the encoding and decoding runtimes (increased by 1% to 3%).

Candidate addition of D0046 provides coding gains of 0.23% in Luma and 0.30% in Chroma in Random Access configuration. It gives 0.64% gain in Luma and 0.93% in Chroma in Low Delay B, and 0.77% gain in Luma and 1.08% gain in Chroma in Low Delay P configuration. Although, this addition increases slightly the encoding time by 1% in Random Access, by 5% in Low Delay B, and by 3% in Low Delay P configuration. The decoding time is also increased by 1% to 2% whatever the configuration used.

For entire CUs, by limiting the number of added spatial candidates to 2 (top and left), and, for sub-blocks of merge CUs, by limiting the number of added spatial candidates to 4 (top, left, top-left and top-right) and moving them to the second place of the FRUC lists, by discarding the zero motion vector, the unilateral and the bottom-right co-located candidates, and by limiting the number of all ATMVP candidates to 4; the BD-rate gains are kept and the complexity reduced. Indeed, this provides coding gains 0.01% better than candidate addition of D0046 either for Luma and Chroma. It also drecreases the encoding runtime by 1% and the decoding runtime by 1% to 2%, up to 4% in Low Delay B configuration.

In test #3, the proposed FRUC improvements provide coding gains (0.24% RA, 0.64% LDB and 0.79% LDP) with a negligible impact on the encoding runtime and a faster decoding, so it is suggested that this test #3 will be integrated into the next version of the JEM.

|  | Test #1.2 | Test #2 | Test #3 |
| --- | --- | --- | --- |
| AMVP | +7 +2 AMVP (beginning) +5 Spatial (end) | +4 +2 AMVP (beginning) +2 Spatial (end) | +4 +2 AMVP (beginning) +2 Spatial (end) |
| Merge | +5 +5 Spatial (end) | +2 +2 Spatial (end) | +2 +2 Spatial (end) |
| Merge Sub-blocks | +3 +3 Spatial (end) | +2 +2 Spatial (2nd place) | -28 (RA / -32 LD) +2 Spatial (2nd place) -1 Zero -1 Unilateral -4/8 B-R co-located (RA/LD) -24 ATMVP |
| BD-rate | -0.23% -0.64% -0.77% | -0.22% -0.66% -0.78% | -0.24% -0.64% -0.79% |
| EncT | 101% 105% 103% | 97% 101% 99% | 100% 104% 103% |
| DecT | 101% 102% 101% | 100% 101% 101% | 99% 98% 100% |
|  | Y-RA Y-LDB Y-LDP | Y-RA Y-LDB Y-LDP | Y-RA Y-LDB Y-LDP |

# References

[1] JVET-D0046_r1, “High precision FRUC with additional candidates”, Antoine Robert, Fabrice Le Léannec, Tangi Poirier, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 4rd Meeting: Chengdu, CN, 15–21 October 2016

[2] JVET-D1011_r2_v2, “Description of Exploration Experiments on Coding Tools”, Elena Alshina, Jill Boyce, Li Zhang, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 4rd Meeting: Chengdu, CN, 15–21 October 2016

[3] JVET-D1001_v3, “Algorithm Description of Joint Exploration Test Model 4”, Jianle Chen, Elena Alshina, Gary J. Sullivan, Jens-Rainer Ohm, Jill Boyce, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 4rd Meeting: Chengdu, CN, 15–21 October 2016

[4] VCEG-AZ07, “Further improvements to HMKTA-1.0”, Jianle Chen, Wei-Jung Chien, Marta Karczewicz, Xiang Li, Hongbin Liu, Amir Said, Li Zhang, Xin Zhao, ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 52nd Meeting: Warsaw, PL, 19–26 June 2015

[5] JVET-B0058, “Modification of merge candidate derivation”, Wei-Jung Chien, Jianle Chen, Sungwon Lee, Marta Karczewicz,  Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 2nd Meeting: San Diego, USA, 20–26 February 2016

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).