| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F0037 |
| --- | --- |

| Title: | AHG8: Padding method for Segmented Sphere Projection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuanyi Zhang, Yao Lu, Jisheng Li, Ziyu Wen | Email: | chuanyi.zhang@owlreality.com yao.lu@owlreality.com jisheng.li@owlreality.com ziyu.wen@owlreality.com |
| Source: | OwlReality |  |  |

_____________________________

- Abstract

This contribution describes a padding method for the SSP format in order to mitigate the seam artifact. Experiments result are listed, comparing with/without padding, including subjective comparison and objective metrics.

Introduction

Seam artifacts are common in different formats, especially when QP is large. The main reason is discontinuity between video content, borders and grey area surrounding non-rectangular tiles. This phenomena is especially obvious in SSP’s round tiles under large QP condition. Via padding around video content, this seam artifact can be mitigated.

It should be noticed that the seam artifact caused by discontinuity is a common phenomenon in formats without padding scheme. The seam on 180° latitude line can be seen in ERP format under large QP and a lot of moving objects. Please see attached video files of ERP for further evaluation.

Padding scheme

The packing for Segmented Sphere Projection (SSP) format contains 2 circle tiles and 1 rectangular tile. The discontinuity is observed along red borders depicted in Figure 1.

Figure 1. Discontinuity borders in SSP

In order to mitigate this artifact, padding pixels are added to border area.

Figure 2. Padding scheme

The padding area is added inside the active area, which will decrease the amount of pixels of video content. This may cause variance in values of objective metrics, which will be discussed in Section 4. The width of propose padding area can be set in the source file of SSP.

Subjective effect

Tests are done based on 360Lib 1.1. Figure 3 shows zoomed viewport of test sequence “GasLamp”, using viewport angle (180∘, 45∘). Left column shows original viewport without overlapping, from QP22 to QP37. When QP is 32 and 37, a vertical seam can be seen at the right of the content and a curved seam can be found at the top. Right column shows result with padding area of 8 pixel wide. It can be seen that after padding using extend area, subjective effect is improved.

It should be noticed that when QP is small, subjectively speaking, the padding area is not necessary.

Besides, it is observed that when most of the video content is static (the camera is static) then 8 pixel is almost enough for mitigating the seam artifact. However, if the content contains a lot of moving objects (the camera is moving), then the width of padding area shall be larger. A series of padding width are tested, and the subjective quality is good enough from QP16. Table 1 shows the classification of static/moving camera for each sequence. The “DrivingInCity” and “DirvingInCountry” sequences are classified as static camera because the camera is stationary relative to the roof, and the movement is slow.

Table 1. Classification of static/moving camera

| Sequence | St/Mv | Padding Width |
| --- | --- | --- |
| Trolley | Static | 8 |
| GasLamp | Static | 8 |
| KiteFlite | Static | 8 |
| Harbor | Static | 8 |
| PoleVault | Static | 8 |
| AerialCity | Static | 8 |
| DrivingInCity | Static | 8 |
| DirvingInCountry | Static | 8 |
| Skateboarding_in_lot | Moving | 16 |
| Chairlift | Moving | 16 |

Subjective comparison of moving camera are depicted in Figure 4.

Figure 3 Example of SSP using GasLamp, 
(1-1) Without padding QP22; (1-2) With padding QP 22 (width = 8 pixel)
(2-1) Without padding QP27; (2-2) With padding QP 27 (width = 8 pixel)
(3-1) Without padding QP32; (3-2) With padding QP 32 (width = 8 pixel)
(4-1) Without padding QP37; (4-2) With padding QP 37 (width = 8 pixel)

Figure 4. Example of SSP using Skateboarding_in_lot, 
(1-1) Without padding QP22; (1-2) With padding QP 22 (width = 16 pixel)
(2-1) Without padding QP27; (2-2) With padding QP 27 (width = 16 pixel)
(3-1) Without padding QP32; (3-2) With padding QP 32 (width = 16 pixel)
(4-1) Without padding QP37; (4-2) With padding QP 37 (width = 16 pixel)

Objective effect

Table 2. Comparison of objective metrics with/without padding method for SSP (static camera)

|  | E2ESPSNR-NN Y | E2ESPSNR-NN U | E2ESPSNR-NN V | E2ESPSNR-I Y | E2ESPSNR-I U | E2ESPSNR-I V | E2ESPSNR-CPP Y | E2ESPSNR-CPP U | E2ESPSNR-CPP V | PSNR-DYN-VP0 Y | PSNR-DYN-VP0 U | PSNR-DYN-VP0 V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pad | -6.67% | -1.65% | -1.61% | -6.45% | -1.77% | -1.73% | -6.52% | -1.89% | -1.85% | -13.82% | -5.97% | -5.89% |
| No-pad | -8.08% | -1.69% | -2.17% | -7.86% | -1.88% | -2.28% | -8.00% | -1.90% | -2.40% | -15.61% | -7.04% | -6.80% |
| Loss | 1.41% | 0.04% | 0.56% | 1.41% | 0.11% | 0.54% | 1.48% | 0.01% | 0.55% | 2.10% | 1.20% | 1.00% |
|  | PSNR-DYN-VP1 Y | PSNR-DYN-VP1 U | PSNR-DYN-VP1 V | CFSPSNR-NN Y | CFSPSNR-NN U | CFSPSNR-NN V | CFSPSNR-I Y | CFSPSNR-I U | CFSPSNR-I V | CFCPP-PSNR Y | CFCPP-PSNR U | CFCPP-PSNR V |
| Pad | -11.57% | -3.28% | -4.31% | -24.51% | -0.03% | -0.16% | -6.23% | -1.60% | -1.57% | -6.30% | -1.96% | -1.92% |
| No-pad | -13.14% | -3.58% | -4.92% | -24.86% | 0.83% | -0.44% | -7.64% | -1.39% | -1.99% | -7.77% | -1.80% | -2.46% |
| Loss | 1.57% | 0.30% | 0.61% | 0.35% | -0.86% | 0.28% | 1.40% | -0.21% | 0.41% | 1.46% | -0.16% | 0.54% |

Row “Pad” contains SSP’s average metric value of 8 “static” test sequence (static camera) using 8-pixel padding scheme, and row “No-pad” contains SSP’s original metric value without padding. Row “Loss” finds the difference of metric value introduced by the padding method. Due to the decrease of the pixels of video content to be used, objective values also decrease by about 1.5%.

Table 4. Comparison of objective metrics with/without padding method for SSP (moving camera)

|  | E2ESPSNR-NN Y | E2ESPSNR-NN U | E2ESPSNR-NN V | E2ESPSNR-I Y | E2ESPSNR-I U | E2ESPSNR-I V | E2ESPSNR-CPP Y | E2ESPSNR-CPP U | E2ESPSNR-CPP V | PSNR-DYN-VP0 Y | PSNR-DYN-VP0 U | PSNR-DYN-VP0 V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pad | -14.77% | -12.11% | -12.38% | -14.82% | -12.19% | -12.45% | -14.88% | -12.25% | -12.54% | -23.36% | -19.12% | -18.47% |
| No-pad | -18.25% | -13.47% | -13.93% | -18.27% | -13.48% | -13.91% | -18.31% | -13.48% | -14.02% | -27.96% | -23.14% | -21.79% |
| Loss | 3.48% | 1.36% | 1.55% | 3.45% | 1.29% | 1.46% | 3.43% | 1.23% | 1.47% | 2.10% | 1.20% | 1.00% |
|  | PSNR-DYN-VP1 Y | PSNR-DYN-VP1 U | PSNR-DYN-VP1 V | CFSPSNR-NN Y | CFSPSNR-NN U | CFSPSNR-NN V | CFSPSNR-I Y | CFSPSNR-I U | CFSPSNR-I V | CFCPP-PSNR Y | CFCPP-PSNR U | CFCPP-PSNR V |
| Pad | -12.21% | -6.39% | -1.84% | -41.44% | -9.86% | -9.94% | -14.79% | -12.15% | -12.46% | -14.81% | -12.37% | -12.66% |
| No-pad | -15.66% | -7.28% | -2.83% | -42.74% | -10.38% | -10.92% | -18.24% | -13.28% | -13.79% | -18.24% | -13.55% | -14.10% |
| Loss | 3.45% | 0.89% | 0.99% | 1.30% | 0.52% | 0.98% | 3.45% | 1.13% | 1.33% | 3.43% | 1.18% | 1.44% |

Row “Pad” contains SSP’s average metric value of 2 “moving” test sequence (moving camera) using 16-pixel padding scheme, and row “No-pad” contains SSP’s original metric value without padding. Row “Loss” finds the difference of metric value introduced by the padding method. Objective values decrease by about 3%.

Patent rights declaration

OwlReality may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).