| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F0052 |
| --- | --- |

| Title: | AHG 8: EAP-based segmented sphere projection with padding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ya-Hsuan Lee, Jian-Liang Lin, Shen-Kai Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {louise.lee, jl.lin, shenkai.chang, cc.ju }@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes an EAP-based segmented sphere projection which uses equal-area projection (EAP) on the equatorial segment and applies padding on the north and south poles. The experimental results reportedly demonstrate that the proposed EAP-based segmented sphere projection achieves 11.33%, 11.21%, 11.28% and 11.35% BD-rate reduction in S-PSNR-NN, S-PSNR-I, CPP-PSNR, and WS-PSNR, respectively, as compared to ERP.

# Introduction

The segmented sphere projection proposed in JVET-E0025 [1] segments a sphere into two caps and one ring along the latitude of ±45°. The caps are flatten into circles, and the ring is unfolded into a rectangle which is the same as an equirectangular projection. These three segments are combined vertically as shown in Fig. 1.

- (b)

Fig. 1 (a) Segmentation of a sphere in JVET-E0025 [1] (b) Vertical SSP layout

However, the quality of the original SSP lacks consistency especially at latitude = ±45° where the projection format is changed. An EAP-based SSP with padding is then proposed to overcome this drawback.

# EAP-based Segmented Sphere Projection with Padding

In the quality metric WS-PSNR, the weighting of SSP is related to the latitude. However, there exists a gap at latitude = ±45° which encounters quality inconsistency between the south/north pole and the ring as shown in Fig. 2(a). Changing the projection format of the ring from ERP to EAP results in smoother WS-PSNR weighting and better quality consistency as illustrated in Fig. 2(b). The comparison between the WS-PSNR weighting image for the original SSP and the EAP-based SSP is shown in Fig. 3.

(a)	(b)

Fig. 2 (a) WS-PSNR weighting of the original SSP (b) WS-PSNR weighting of the EAP-based SSP

(a)	(b)

Fig. 3 (a) WS-PSNR weighting image of the original SSP (b) WS-PSNR weighting image of the EAP-based SSP

Furthermore, after video coding, a reconstructed viewport frame as shown in Fig. 4 has visual artifacts along the latitude of ±45° due to the quantization operation where the non-active pixels around the poles introduce errors.

Fig. 4 Reconstructed viewport frame of AerialCity with QP = 37, FOV = 80°x80° and (yaw, pitch) = (-90, 45)

To efficiently reduce the artifacts around the boundary of the poles caused by DCT transform and quantization, padding is applied to reduce the effect from the non-active pixels before encoding. A padding region, which surrounds the pole with a specified padding radius, is generated. The value of a padding pixel is derived from calculating its longitude and mapping to the pixel with the same longitude but the latitude of ±45° as shown in Fig. 5(a). The pixel in the padding region is then blended with the non-active pixel according to the distance to the boundary of the pole and the distance to the non-active region as shown in Fig. 5(b).

(a)	(b)

Fig. 5 (a) The padding region with size n maps to the pixel on the boundary of the pole (b) Blend the padding region with the non-active pixels

The size of the padding region is set to 32 pixels due to the trade-off between the bit-rate and the quality of the reconstructed viewport frame. An image example for the proposed EAP-based SSP with padding is shown in Fig. 6. The snapshot images shown in Fig. 7 are the comparisons of the reconstructed frames encoded with QP = 37 after viewport rendering with FOV = 80°x80°.

Fig. 6 An example image of the proposed EAP-based SSP

| AerialCity (Yaw = -90°, Pitch= 45°) | Without padding |  |
| --- | --- | --- |
|  | With padding |  |
| DrivingInCountry (Yaw = 0°, Pitch= -45°) | Without padding |  |
|  | With padding |  |
| Harbor (Yaw = -75°, Pitch= -45°) | Without padding |  |
|  | With padding |  |
| KiteFlite (Yaw = 0°, Pitch= 45°) | Without padding |  |
|  | With padding |  |

Fig. 7 Comparisons between reconstructed viewport frame with and without padding

# Experimental Result

The original face size of SSP for 4K sequences is 1008 pixels both for the width and the height according to the JVET common test conditions [2]. However, due to the padding, the active pixel count exceeds 3% of the specified size in JVET common test conditions [2]. In these experiments, the face size of EAP-based SSP is adjusted to 992x992 for 4K sequences, as shown in Table I.

|  | 4K |  | 8K |  |
| --- | --- | --- | --- | --- |
|  | Width | Height | Width | Height |
| Original Size in SSP | 1008 | 1008 | 1216 | 1216 |
| Modified Size in EAP-based SSP | 992 | 992 | 1216 | 1216 |

Table I. Face size for EAP-based SSP

The experiments on the proposed EAP-based SSP with padding are conducted on 360Lib-2.1 [3] and HM version 16.15[4] followed by the JVET common test conditions and evaluation procedures for 360° video [2]. The averaged BD-rate reduction over the ten test sequences with various quality metrics is shown in Table II, where anchor is ERP.

|  | Original vertical SSP vs. ERP |  |  | Proposed vs. ERP |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Quality Metric | Y | U | V | Y | U | V |
| End to End S-PSNR-NN | -10.24% | -4.06% | -4.61% | -11.33% | -6.53% | -6.96% |
| End to End S-PSNR-I | -10.06% | -4.21% | -4.69% | -11.21% | -6.61% | -7.02% |
| End to End CPP-PSNR | -10.17% | -4.39% | -4.87% | -11.28% | -6.72% | -7.12% |
| End to End WS-PSNR | -10.28% | -4.02% | -4.61% | -11.35% | -6.58% | -7.00% |

Table II. Averaged BD-rate performance

# Conclusion

This contribution proposes an EAP-based segmented sphere projection with padding. By applying the equal-area projection on the equatorial segment with padding on the poles, better quality consistency can be reached and virtual artifacts are efficiently reduced. This proposed EAP-based SSP demonstrates 11.33%, 11.21%, 11.28% and 11.35% BD-rate reduction as compared to ERP in S-PSNR-NN, S-PSNR-I CPP-PSNR, and WS-PSNR, respectively.

# References

- C. Zhang, Y. Lu, J. Li, Z. Wen, “AHG8: Segmented Sphere Projection for 360-degree video”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-E0025, Jan. 2017

- J. Boyce, E. Alshina, A. Abbas, Y. Ye, “JVET common test conditions and evaluation procedures for 360° video”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-E1030, Jan. 2017

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- HM reference software, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).