| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F0080-r1 |
| --- | --- |

| Title: | Cross-check of JVET-F0033 : UWP with Constrained PDPC and Constrained ARSS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Krit Panusopone Seungwook Hong Yue Yu Limin Wang | Email: | krit.panusopone@arris.com seungwook.hong@arris.com yue.yu@arris.com limin.wang@arris.com |
| Source: | ARRIS |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for JVET-F0033, when UW planar is used together with constrained PDPC and constrained ARSS. For 4-mode version of constrained PDPC, simulation results of test 8 reportedly show on average 0.14% luma BD-rate loss and 77% encoding time for AI. Decoding time is 100%.

# Introduction

In EE1 [1], interactions between Unequal Weight Planar prediction (UW-Planar), Position Dependent Intra Prediction Combination (PDPC), and Adaptive Reference Sample Smoothing (ARSS) were investigated. Tests in EE1 were studied with one tool change at a time.

JVET-F0033 [2] proposes to constrain both PDPC and ARSS, together with UW-Planar. Two variations with different number of modes allowed to use PDPC were studied in JVET-F0033. Specifically, test 8 employs 4-mode version of constrained PDPC, similar to that employed by EE1 test 2. Test 9 employs 3-mode version of constrained PDPC, similar to that employed by EE1 test 4.

# Simulation Results

EE1 software was downloaded from SVN repository with configuration settings provided by the proponent. Tests 8 and 9 were simulated on Linux cluster for AI configurations following CTC. Simulation results are summarised in the tables 1 and 2, for tests 8 and 9, respectively, and it was confirmed that it matches the results distributed by the proponent.

Table 1: Coding Performance of F-0033 test 8

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.26% | -0.08% | 0.03% | 77% | 101% |
| Class A2 | 0.22% | 0.01% | 0.03% | 79% | 100% |
| Class B | 0.14% | 0.06% | 0.13% | 76% | 100% |
| Class C | 0.06% | 0.00% | 0.08% | 77% | 101% |
| Class D | 0.00% | -0.08% | -0.14 | 77% | 100% |
| Class E | 0.21% | 0.11% | 0.32% | 76% | 100% |
| Overall | 0.14% | 0.00% | 0.07% | 77% | 100% |
| Class F | -.0.06% | -0.09% | -0.23% | 77% | 100% |

Table 2: Coding Performance of F-0033 test 9

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.29% | -0.02% | 0.05% | 77% | 102% |
| Class A2 | 0.24% | 0.06% | -0.02% | 79% | 102% |
| Class B | 0.16% | 0.04% | 0.16% | 75% | 102% |
| Class C | 0.04% | 0.08% | 0.02% | 76% | 102% |
| Class D | 0.01% | -0.03% | -0.19% | 77% | 101% |
| Class E | 0.23% | 0.09% | 0.32% | 76% | 101% |
| Overall | 0.16% | 0.04% | 0.05% | 77% | 102% |
| Class F | -0.10% | -0.01% | -0.19% | 76% | 101% |

# References

[1] K. Panusopone, S. Hong, Y. Yu, and L. Wang, “Unequal Weight Planar Prediction and Constrained PDPC,” JVET-F0024, April 2017.

[2] H. Jang, J. Lim, J. Nam, S.-H. Kim, “EE1-Related : UWP with Constrained PDPC and Constrained ARSS,” JVET-F0033, April 2017.