| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Macao, CN, 18–24 Oct. 2017 | Document: JVET-H0043 |
| --- | --- |

| Title: | AHG5: How to measure memory bandwidth |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ryoji HASHIMOTO Seiji Mochizuki Tomohiro Ikai | Tel: Email: | ryoji.hashimoto.te@renesas.com seiji.mochizuki.zc@renesas.com ikai.tomohiro@sharp.co.jp |
| Source: | Renesas Electronics Corporation, Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution suggests how to measure the memory bandwidth in JEM. Focusing on access to reference pictures makes it easy to model a cache in JEM. The implementation of a cache in JEM enables performance check of the cache without any external tool, and makes crosscheck easy by removing implementation dependency. The result will be available in the next meeting.

Introduction

The video applications in various devices become much popular today. In some devices such as mobile devices, it is very important to save power consumption in order to expand battery life. The memory bandwidth is one of the biggest factors in power consumption. Moreover, rapid increase of bandwidth consumed by video applications makes system design difficult. Therefore, a contribution of AHG5 has checked memory bandwidth in JEM by using a profiling tool (Valgrind) [1]. However, at the 7th meeting in Turin, some experts worried about implementation dependency and mismatch caused by environment such as compiler. Moreover, it is not suitable to check the bandwidth in terms of hardware development because the results of Valgrind includes not only reference pixel data but also constant/table access in the program. Motivated by this tendency, we propose a way to measure the memory bandwidth in JEM. Our proposal focuses on reading reference picture, which is dominant factor in external memory access. The goal of this proposal in this meeting is to make a consensus of how to measure the bandwidth in JEM.

How to measure the memory bandwidth

Figure 1 shows decoder memory access model defined in [1]. The external memory access includes following data: (except intermediate value in operation and constant/table in the program)

- Read

  - Reference picture

  - Bitstream

  - Motion vector of reference picture

- Write

  - Decoded picture

  - Motion vector of current picture

As for bitstream and motion vector, the size of “decoder total data access” would be the same as that of “external memory access” thanks to its straightforward access. The data size of them can be calculated simply from the bitrate and the resolution of a picture, respectively since the cache data is anyway accessed from external data. On the other hand, in accessing reference pictures, there is the difference between “decoder total data access” and “external memory access” due to its arbitrary access, which may use cached data or may use data in external memory, and it can be too complex to calculate the size of “decoder total data access” for reference pictures without a software implementation. The cache makes it more complex to calculate that of “external memory access”. As for data amount of each access, accessing reference pictures is dominant. Based on these, we propose focusing on the access to reference pictures. In this case, we do not need to take care of write access to external memory so that it is easy to model a cache in JEM. Preparing a cache for reference picture is ordinary in hardware development of video codec. Moreover, the implementation of the cache on JEM solves implementation dependency and makes it easy to crosscheck. On the implementation, we will use set associative cache, which is the most general cache.

On measuring access to reference pictures, we will parameterize following cache parameters:

- Size of cache line

  - Size of cache block (when a cache miss occurs, the cache reads this size from external memory)

  - This affects not only ratio of cache hit/miss but also efficiency of external memory access

- Cache size

  - Size of cache memory

  - Number of cache entry is defined by (cache size / size of cache line)

- Associativity

  - Number of way

On each access to reference pictures in JEM, JEM checks cache hit/miss as shown in Figure 2. CACHE_ACCESS is a function to check cache hit/miss and manage data in the cache, and it will be enabled by the corresponding define to minimize the effect to runtime. The result will be available in the next meeting.

Figure 1 Decoder memory access model (with cache)

// TComPrediction::gradFilter2DVer() for ( Int y = iHeight; y != 0; y-- ) { for ( Int x = 0; x < iWidth; x++ ) { iSum = m_lumaGradientFilter[iMV][0]* piSrcTmp[x] +m_lumaGradientFilter[iMV][1]*piSrcTmp1[x] +m_lumaGradientFilter[iMV][2]*piSrcTmp2[x] +m_lumaGradientFilter[iMV][3]*piSrcTmp3[x] +m_lumaGradientFilter[iMV][4]*piSrcTmp4[x] +m_lumaGradientFilter[iMV][5]*piSrcTmp5[x]; CACHE_ACCESS( &piSrcTmp[x] ); CACHE_ACCESS( &piSrcTmp1[x] ); CACHE_ACCESS( &piSrcTmp2[x] ); CACHE_ACCESS( &piSrcTmp3[x] ); CACHE_ACCESS( &piSrcTmp4[x] ); CACHE_ACCESS( &piSrcTmp5[x] ); iSum = (iSum>=0)?((iSum +iOffSet)>>iShift):-(((-iSum +iOffSet)>>iShift)); piDst[x ] = (Pel) (iSum); } …

Figure 2 Sample code to check cache hit/miss

Conclusion

This contribution proposes how to measure the memory bandwidth in JEM, which focuses on access to reference pictures. This implementation enables performance check of cache without any external tool and makes crosscheck easy by removing implementation dependency. The result will be available in the next meeting.

[1] JVET-G0061 AHG5 External Memory Access Evaluation with the Consideration of Cache “http://phenix.int-evry.fr/jvet/doc_end_user/current_document.php?id=3161”

- Patent rights declaration(s)

Renesas Electronics Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).