| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Gwangju, KR, 20–26 Jan. 2018 | Document: JVET-I0025 |
| --- | --- |

| Title: | Clarifications on HDR Test of CfP on Video Compression beyond HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peng Yin, Walt Husak, Taoran Lu, Fangjun Pu, Tao Chen 432 Lakeside Drive, Sunnyvale, CA 94085, USA Shunsuke Iwamura 1-10-11 Kinuta Setagaya-ku Tokyo 157-8510, Japan; Tel: Email:; +81-(0)3-5494-3353 iwamura.s-gc@nhk.or.jp. | Tel: Email: | +1 4083303251 {pyin,wjh}@dolby.com +81-(0)3-5494-3353 iwamura.s-gc@nhk.or.jp |
| Source: | Dolby Laboratories, Inc., NHK |  |  |

_____________________________

# Abstract

This contribution seeks several clarifications regarding to High Dynamic Range Video test described in JVET-H1002 “Joint Call for Proposals on Video Compression with Capability beyond HEVC”. The clarification relates to two aspects: 1) the input video format to the HDR display for coded test material submission; 2) reference software and configuration to be used to compute HDR objective metrics.

# Introduction

The Joint Call for Proposals on Video Compression with Capability beyond HEVC has been published in JVET-H1002 [1].  This contribution seeks some clarification for High Dynamic Range video test. It includes two aspects: 1) the input video format to the HDR display for coded test material submission; 2) the reference software and configurations to be used to compute HDR objective metrics.

# Clarifications

## Input video format to the HDR display for coded test material submission

For HDR video test, the CfP states the following in Annex C.4:

- For the HDR tests, an appropriate display will be use (e.g. for the SONY BVM X300 for HDR-A, and the SIM2 for HDR-B)

### HDR-A (HDR HLG content) Test

If SONY BVM X300 is going to be used for HDR HLG content, two issues are to be clarified.

1) BVM X300 does not accept 4:2:0 10b YUV input video directly. The input must be at least 4:2:2 10b YUV. The question is if proponent needs to upsample the decoded 4:2:0 10b YUV into 4:2:2 10b YUV for coded test material submission. If so, what chroma upsampler and reference software should the proponent use?

2) BVM X300 supports two HLG modes: HLG SG 1.2 (HDR) and HLG SG Variable (HDR) (default system gamma value is 1.2). Since HLG SG Variable (HDR) is aligned with ITU-R BT 2100, it is suggested to use HLG SG Variable mode for HDR HLG content test.

### HDR-B (HDR PQ content) Test

If SIM2 is going to be used for HDR PQ content, two questions to be clarified.

1) SIM2 does not accept 4:2:0 10b YUV input video directly. The question is if proponent needs to convert the decoded 4:2:0 10b YUV into SIM2 AVI file format. In MPEG HDR CfE [2], the following are stated:

- F.1.1.2.	Conversion of decoded videos for the SIM2 monitor

  - Proponents need to provide AVI files for driving the SIM2 monitor. These files shall be generated using the Sim2Convert tool provided in the HDRTools software using the OpenEXR reconstructed sequences. Sim2Convert is available in version 0.9 of HDRTools. Configuration files are available in directory ‘bin\CfE_cfgFiles’ of the HDRTools package.

2) There are 5 clips in HDR-B category. Four of them are in 1920x1080 resolution and one (Cosmos) is 1920x856. The question is if proponents need to pad 1920x856 into 1920x1080 for the SIM2 AVI file. If needed, what are the padding configuration parameters?

## Reference software and configurations used to compute objective metrics

### Reference software HDRTools

In Annex D, it states the following:

- In this Annex, further information about the wPSNR and deltaE100, and PSNR-L100 metrics are provided. The metrics deltaE100 and PSNR-L100 shall be calculated using the HDRMetrics v0.15 utility available at https://gitlab.com/standards/HDRTools.

Annex D does not define which software to compute wPSNR metrics. A newer version v0.17 is available now and can compute wPSNR metrics too. It is suggested to update the HDRTools version into v0.17 and use this version to calculate wPSNR, deltaE100 and PSNR-L100.

### Configuration settings for HDR-A category (HLG content)

In JVET-H0068, it is commented that for HLG graded contents, display peak luminance of 1,000cd/m2 should be assumed to align with the nominal display peak luminance noted in BT.2100. Therefore, it is suggested to use a peak luminance of 1,000cd/m2 for HLG graded contents whereas 10,000cd/m2 for PQ graded contents when calculating deltaE100 and PSNR-L100 [3]. The issue was discussed in last meeting and notes are kept in JVET-H0095 [4]. There seems no clear action.

The related parameter is MaxSampleValue in HDRMetric_CfE.cfg.

MaxSampleValue=10000.0                                         # Maximum sample value for floating point (openEXR) data files.

Setting MaxSampleValue to 1000 in HDRMetrics_CfE.cfg for HDR-A(HLG) is also aligned with EXR file conversion from HLG yuv file where the source peak luminance is set to 1000nits.

If this is agreed, it is suggested to revise the following sentence in D.2 deltaE100-based metric and D.3 PSNR-100 from

- where PeakValue is set to 10,000.

to

- where PeakValue is set to 1,000 for HDR-A content and 10,000 for HDR-B content.

# References

- A. Segall, V. Baroncini, J. Boyce, J. Chen and T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC”, JVET-H1002, Oct. 2017, Macao, CN

- A. Luthra, E. François, and W. Husak (Editors), “Call for Evidence (CfE) for HDR and WCG Video Coding”, N 15083, 110th MPEG meeting, Geneva

- S. Iwamura, S. Nemoto, A. Ichigaya,  “AHG7: HLG rate points for CfP anchor generation”, JVET-H0068, Oct. 2017, Macao, CN

- A. Segall, “BoG Report: Extended Colour Volume and High Dynamic Range”, JVET-H0095, Oct. 2017, Macao, CN

# Patent rights declaration(s)

Dolby Laboratories, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.

NHK does not have any current or pending patent rights relating to the technology described in this contribution.