| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Gwangju, KR, 20–26 Jan. 2018 | Document: JVET-I0026 |
| --- | --- |

| Title: | AHG7: HLG anchor generation of S31 and S32 for Joint Call for Proposals on Video Compression with Capability beyond HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2782 Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution provides information regarding HLG anchor generation of the DayStreet (S31) and the PeopleInShoppingCenter (S32) for Joint Call for Proposals on Video Compression with Capability beyond HEVC. Encoding results are reported for HM-16.16 as well as JEM-7.1 at given rate points which was identified in last meeting.

Introduction

The coding conditions for the anchor generation of both HM and JEM are shown in JVET-H1002 [1].  The following Table 1-4 provide summaries of information of test sequence which are thumbnail, basic video information, md5sums and the bit rate.

Table 1		HDR-A test sequence example pictures

| DayStreet | PeopleInShoppingCenter | SunsetBeach |
| --- | --- | --- |

Table 2		HDR-A test sequences

| Sequence name | Resolution | Frame count | Frame rate | Chroma format | Bit depth |
| --- | --- | --- | --- | --- | --- |
| DayStreet | 3840x2160 | 600 | 60 | 4:2:0 | 10 |
| PeopleInShoppingCenter | 3840x2160 | 600 | 60 | 4:2:0 | 10 |
| SunsetBeach | 3840x2160 | 600 | 60 | 4:2:0 | 10 |

Table 3		HDR-A test sequence md5sums

| Sequence name | Clip Identifier | MD5Sum | Source (or Owner) |
| --- | --- | --- | --- |
| DayStreet | S31 | 2bd7bbcfb80ab329118a0a82101e2715 | SONY |
| PeopleInShoppingCenter | S32 | 40395778572368438c0202fe3c017fa9 | SONY |
| SunsetBeach | S33 | 7cc25ae4249203427c2f83db2d9f09f7 | NHK |

Table 4		Target bit rate points not to be exceeded for the HDR-A category

|  | Target bit rates [kbit/s] |  |  |  |
| --- | --- | --- | --- | --- |
| Sequences | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| DayStreet | 680 | 1100 | 1750 | 2800 |
| PeopleInShoppingCenter | 640 | 960 | 1450 | 2150 |
| SunsetBeach | 1500 | 3000 | 6000 | 10000 |

Encoding results

Table 5 provides the target and matched rates with the QP parameters, based QP and QPIncrementFrame (QPIF), for two HLG sequences. For each rate the rate difference is about 0.7% at maximum as shown in Table 5. The objective values are obtained by the HDRMetric (v0.16) as shown in Table 6 and 7. A peak luminance of 1,000cd/m2 for HLG graded contents whereas 10,000cd/m2 for PQ graded contents was used. The number is being clarified in JVET-I0025 [2]. Those results were crosschecked by NHK.

Table 5		Bit rate of HM-16.6 and JEM-7.1

| Sequence | Target rate (kbps) | HM-16.16 |  |  |  | JEM-7.1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | kbps | baseQP | QPIF | Diff | kbps | baseQP | QPIF | Diff |
| DayStreet | 680 | 679.8 | 46 | 598 | 0.0% | 679.0 | 43 | 254 | -0.1% |
|  | 1100 | 1094.9 | 41 | 64 | -0.5% | 1098.2 | 40 | 512 | -0.2% |
|  | 1750 | 1749.5 | 38 | 280 | 0.0% | 1749.8 | 36 | 64 | 0.0% |
|  | 2800 | 2800.0 | 35 | 532 | 0.0% | 2798.0 | 33 | 176 | -0.1% |
|  | QP32 | 4402.5 | 32 |  |  |  |  |  |  |
| PeopleInShoppingCenter | 640 | 636.3 | 46 | 574 | -0.6% | 635.5 | 44 | 318 | -0.7% |
|  | 960 | 959.9 | 42 | 359 | 0.0% | 959.9 | 41 | 551 | 0.0% |
|  | 1450 | 1449.8 | 39 | 516 | 0.0% | 1450.0 | 37 | 184 | 0.0% |
|  | 2150 | 2148.8 | 35 | 39 | -0.1% | 2141.5 | 34 | 320 | -0.4% |
|  | QP33 | 3094.9 | 33 |  |  |  |  |  |  |

Table 6		Encoding results of HM-16.16 for CS1

|  | Target rates (kbps) | kbps | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DayStreet | QP32 | 4402.47 | 24.19 | 28.00 | 43.04 | 49.37 | 51.93 | 38.91 | 43.63 | 46.62 |
|  | 2800 | 2799.96 | 23.80 | 27.23 | 41.57 | 48.71 | 51.33 | 37.27 | 42.90 | 45.91 |
|  | 1750 | 1749.49 | 23.33 | 26.37 | 39.96 | 47.97 | 50.60 | 35.51 | 42.10 | 45.10 |
|  | 1100 | 1094.92 | 22.97 | 25.51 | 38.31 | 47.51 | 50.13 | 33.77 | 41.60 | 44.57 |
|  | 680 | 679.83 | 22.46 | 24.55 | 36.51 | 46.86 | 49.44 | 31.93 | 40.90 | 43.78 |
| PeopleInShoppingCenter | QP33 | 3094.90 | 24.64 | 28.32 | 44.62 | 47.87 | 51.20 | 39.58 | 42.80 | 43.87 |
|  | 2150 | 2148.76 | 24.05 | 27.51 | 43.01 | 46.59 | 49.89 | 37.90 | 41.57 | 42.54 |
|  | 1450 | 1449.84 | 23.53 | 26.62 | 41.22 | 45.62 | 48.87 | 36.07 | 40.63 | 41.54 |
|  | 960 | 959.95 | 23.09 | 25.68 | 39.35 | 45.05 | 48.24 | 34.15 | 40.05 | 40.93 |
|  | 640 | 636.30 | 22.59 | 24.70 | 37.46 | 44.34 | 47.51 | 32.24 | 39.30 | 40.21 |

Table 7		Encoding results of JEM-7.1

|  | Target rates (kbps) | kbps | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DayStreet | QP32 |  |  |  |  |  |  |  |  |  |
|  | 2800 | 2797.97 | 24.49 | 27.93 | 42.86 | 49.69 | 52.64 | 38.65 | 43.96 | 47.43 |
|  | 1750 | 1749.83 | 24.11 | 27.16 | 41.39 | 49.10 | 52.13 | 37.01 | 43.28 | 46.79 |
|  | 1100 | 1098.15 | 23.73 | 26.38 | 39.89 | 48.52 | 51.58 | 35.39 | 42.65 | 46.13 |
|  | 680 | 679.05 | 23.33 | 25.53 | 38.29 | 47.98 | 51.06 | 33.70 | 42.05 | 45.49 |
| PeopleInShoppingCenter | QP33 |  |  |  |  |  |  |  |  |  |
|  | 2150 | 2141.48 | 24.88 | 28.28 | 44.51 | 48.41 | 51.68 | 39.45 | 43.29 | 44.34 |
|  | 1450 | 1449.96 | 24.40 | 27.42 | 42.75 | 47.55 | 50.71 | 37.62 | 42.41 | 43.34 |
|  | 960 | 959.94 | 23.95 | 26.48 | 40.86 | 46.95 | 50.01 | 35.68 | 41.78 | 42.64 |
|  | 640 | 635.52 | 23.38 | 25.52 | 38.97 | 46.09 | 49.03 | 33.76 | 40.87 | 41.66 |

Conclusion

This contribution provides information regarding HLG anchor generation of the DayStreet (S31) and the PeopleInShoppingCenter (S32) for Joint Call for Proposals on Video Compression with Capability beyond HEVC. In addition, it summarizes the encoding parameters and results, matched bit rate and DE100/PSNRL100/wPsnr/Psnr. The generated bitstream files are available at the FTP site.

Reference

- A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC,” JVET-H1002, July 2017.

- P. Yin, W. Husak, T.Lu, F. Pu, T. Chen (Dolby), S. Iwamura (NHK), “Clarifications on HDR Test of CfP on Video Compression beyond HEVC”, JVT-I0025, January 2018.

Patent rights declaration(s)

Sony Corporation does not have any current or pending patent rights relating to the technology described in this contribution.