| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Gwangju, KR, 20–26 Jan. 2018 | Document: JVET-I0033 |
| --- | --- |

| Title: | AHG5: Measurement result of memory bandwidth with anchor streams |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ryoji HASHIMOTO Seiji Mochizuki | Tel: Email: | ryoji.hashimoto.te@renesas.com seiji.mochizuki.zc@renesas.com |
| Source: | Renesas Electronics Corporation |  |  |

_____________________________

- Abstract

This contribution shows the measurement result of the memory bandwidth with JEM7.1. At 8th JVET meeting in Macau, JVET-H0043 [1] proposes how to measure memory bandwidth. Based on the proposal, we have measured memory bandwidth with JEM 7.1 by utilizing anchor streams for CfP. This can check memory bandwidth in the test model for next video standardization.

Introduction

The video applications in various devices become much popular today. In some devices such as mobile devices, it is very important to save power consumption in order to expand battery life. The memory bandwidth is one of the biggest factors in power consumption. Moreover, rapid increase of bandwidth consumed by video applications makes system design difficult. Therefore, a contribution of AHG5 has checked memory bandwidth in JEM by using a profiling tool (Valgrind) [2]. However, at the 7th meeting in Turin, some experts worried about implementation dependency and mismatch caused by environment such as compiler. To prevent this, JVET-H0043 proposes how to measure the memory bandwidth on JEM, which is focusing on reading reference picture. This contribution shows the measurement result of memory bandwidth with JEM7.1 by utilizing anchor streams for CfP.

How to measure the memory bandwidth

Figure 1 shows decoder memory access model defined in [1]. On the measurement, we used the settings as shown Table 1. Tree PLRU is a simple but an effective cache replacement algorithm. Compared to LRU, PLRU is slightly worse miss rate and much lower overheads in hardware implementation. There are several configuration sets of cache as shown in Table 2. To avoid environmental difference, kept memory is aligned with 256 byte (largest cache line size in the measurement).

On each access to reference pictures, JEM checks cache hit/miss as shown in Figure 2. AHG5_CACHE_ACCESS is a function to check cache hit/miss and manage data in the cache, and it will be enabled by the corresponding define, that is “AHG5_CACHE_ENABLE”.

Figure 1 Decoder memory access model (with cache)

Table 1 cache configuration

| parameter | Value |
| --- | --- |
| cache type | n-way set associative |
| cache replacement algorithm | Tree Pseudo-Least Recently Used (Tree PLRU) |
| cache line size | 128, 256 [Byte] |
| # of way | 4, 8 |
| # of cache line | 32 |
| total cache size | 32, 64 [KByte] |

Table 2 cache configuration set

| Configuration set | Cache line size [Byte] | # of way | # of cache line | Total cache size [KByte] |
| --- | --- | --- | --- | --- |
| A | 128 | 4 | 64 | 32 |
| B | 128 | 4 | 128 | 64 |
| C | 128 | 8 | 64 | 64 |
| D | 256 | 4 | 64 | 64 |

// TComPrediction::gradFilter2DVer() for ( Int y = iHeight; y != 0; y-- ) { for ( Int x = 0; x < iWidth; x++ ) { iSum = m_lumaGradientFilter[iMV][0]* piSrcTmp[x] +m_lumaGradientFilter[iMV][1]*piSrcTmp1[x] +m_lumaGradientFilter[iMV][2]*piSrcTmp2[x] +m_lumaGradientFilter[iMV][3]*piSrcTmp3[x] +m_lumaGradientFilter[iMV][4]*piSrcTmp4[x] +m_lumaGradientFilter[iMV][5]*piSrcTmp5[x]; AHG5_CACHE_ACCESS( &piSrcTmp[x] ); AHG5_CACHE_ACCESS( &piSrcTmp1[x] ); AHG5_CACHE_ACCESS( &piSrcTmp2[x] ); AHG5_CACHE_ACCESS( &piSrcTmp3[x] ); AHG5_CACHE_ACCESS( &piSrcTmp4[x] ); AHG5_CACHE_ACCESS( &piSrcTmp5[x] ); iSum = (iSum>=0)?((iSum +iOffSet)>>iShift):-(((-iSum +iOffSet)>>iShift)); piDst[x ] = (Pel) (iSum); } …

Figure 2 Sample code to check cache hit/miss

Measurement result

We have tested with anchor stream for CfP and JEM7.1. To compare the difference between encoder options, we used Class SDR-B constraint set 1 and 2 (random access and low delay).

In all tests (Figure 3, Figure 5, Figure 7, and Figure 9), it is clear that low delay constraint requires much lower memory bandwidth than random access (from 1/3 to 1/10). This means that encoder tools for random access such as bi prediction requires higher bandwidth. This can be also observed from Figure 11. This figure shows total access number to the cache in a frame. Random access accesses much bigger than low delay (from 3 times to 18 times). There is another fact that as bitrate becomes higher, number of access becomes smaller in both configuration. To analyze cause of this, we need to gather statistical information such as block size, prediction mode and so on. Moreover, this number is equal to bandwidth in no cache operation and reading 1 pixel unit from external memory (As for 10 bit sequence, we need to multiply 2 to change to byte).

Configuration set A (Figure 3) has highest bandwidth among test sets due to the smallest cache size.

Between configuration sets B and C, there is little difference in both memory bandwidth (Figure 5 and Figure 7) and cache hit ratio (Figure 6 and Figure 8). In a strict sense, set B is a little better than C. On the other hand, sets B and C (Figure 5 and Figure 7) requires lower bandwidth than set D (Figure 9) even if set D (Figure 10) has higher hit ratio than B and C (Figure 6 and Figure 8). The difference of cache line size leads this result. For 128x128 block, cache line size is equal to 256 is too big so that this causes big overhead on cache miss.

Figure 3 average memory bandwidth in configuration set A

Figure 4 average cache hit ratio in configuration set A

Figure 5 average memory bandwidth in configuration set B

Figure 6 average cache hit ratio in configuration set B

Figure 7 average memory bandwidth in configuration set C

Figure 8 average cache hit ratio in configuration set C

Figure 9 average memory bandwidth in configuration set D

Figure 10 average cache hit ratio in configuration set D

Figure 11 total access number to cache

Conclusion

This contribution shows the measurement result of memory bandwidth with several cache configurations. On the measurement, JEM7.1 and anchor streams for CfP is used. Measurement result shows encoder tools in random access configuration requires higher bandwidth. Big cache line size leads large penalty so that, for 128x128 block, cache line size = 128 is more suitable than 256. This cache implementation is useful to check the bandwidth in tool on/off test in the next test model.

[1] JVET-H0043AHG5: How to measure memory bandwidth “http://phenix.int-evry.fr/jvet/doc_end_user/current_document.php?id=3315”

[2] JVET-G0061 AHG5 External Memory Access Evaluation with the Consideration of Cache “http://phenix.int-evry.fr/jvet/doc_end_user/current_document.php?id=3161”

Patent rights declaration(s)

Renesas Electronics Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).