| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, CA, USA, 10–20 April 2018 | Document: JVET-J0012-v1 |
| --- | --- |

| Title: | Description of SDR and HDR video coding technology proposal by Ericsson and Nokia |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Response to Call for Proposals |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Kenneth Andersson Ruoyang Yu Zhi Zhang Per Wennersten Ericsson Jani Lainema Antti Hallapuro Alireza Aminlou Miska Hannuksela Ramin Ghaznavi-Youvalari Justin Ridge Nokia | Tel: Email: Tel: Email: | first.last@ericsson.com first.last@nokia.com |
| Source: | Ericsson, Nokia |  |  |

_____________________________

# Abstract

This document describes Ericsson’s and Nokia’s response to VCEG’s and MPEG’s joint Call for Proposals on video compression technologies with capability beyond HEVC. The proposal, referred to as “ENC”, is based on JEM 7.0 software and includes additional tools asserted to provide subjective improvements, coding efficiency gains and complexity benefits over the JEM model. The following additional tools or modifications of JEM 7.0 tools are included in the proposal:

- Wide-angle intra prediction extending the JEM prediction directions beyond 45-degree top-right and bottom-left directions

- Multiple boundary filtering for planar and DC mode uses two or three tap distance-adaptive filtering of prediction border samples for the planar and DC intra modes

- Motion vector look back modifies the merge candidate list generation by including additional neighboring motion vectors in case the spatial and advanced TMVP candidates do not fill the list

- Motion vector prediction between list exploits correlation between the two motion vectors when the current block is bi-predictively coded using the AMVP mode

- Motion vector difference sign data hiding hides the sign of the MVD x component

- Restricted OBMC is reported to reduce the computational complexity by disabling OBMC when the spatial activity of the prediction is below a threshold or when LIC flags differ

- Affine flexing which resamples affine sub-PU motion boundaries in order to compensate for the sample level motion

- Improved in-loop deblocking filter which extends the JEM deblocking filter with longer filters. The tool also filters sub-PU boundaries, LIC boundaries and boundaries caused by CCLM prediction

In the case of SDR category tests it is reported that that proposed software provides -33.73% and -24.66% BD-rate impact with respect to the HM anchor for CS1 and CS2 configurations, respectively. The reported impacts over JEM anchor for these configurations are -0.90% and -0.17%, respectively.

In the case of HDR category tests it is reported that that proposed software provides -43.6% BR-rate impact over the HM anchor and -0.9% BD-rate impact over the JEM anchor.

Reported encoder runtimes compared to JEM anchor are 103%, 99% and 103% for SDR CS1, SDR CS2 and HDR configurations, respectively. Reported decoder runtimes for the same configurations are 98%, 95% and 99%, respectively.

# Contents

Abstract	i

Contents	ii

1	Introduction	1

2	Decoder algorithm description	1

2.1	General algorithm description	1

2.1.1	Bitstream structure	1

2.1.2	Entropy decoding	1

2.1.3	Conversion process from entropy decoded data to symbols	1

2.1.4	Decoder-side motion refinement	1

2.1.5	Inverse quantization	1

2.1.6	Inverse transforms	1

2.1.7	Motion compensation	1

2.1.8	Intra prediction	5

2.1.9	In-loop filtering	9

2.1.10	Additional tools	12

2.1.11	Additional algorithmic description topic(s)	12

2.2	360° Video	12

2.2.1	Projection format (including padding)	12

2.2.2	Quantization handling	12

2.2.3	360° video specific decoding tools	13

2.3	HDR	13

2.3.1	Post-processing	13

2.3.2	Quantization handling	13

2.3.3	HDR specific decoding tools	13

3	Encoder algorithm description	13

3.1	General algorithm description	13

3.1.1	Rate-distortion optimization	13

3.1.2	Partitioning structure selection	13

3.1.3	Intra prediction mode estimation and type selection	13

3.1.4	Motion estimation and motion segment selection	13

3.1.5	Mode decisions	14

3.1.6	In-loop filtering type selection and parameter estimation	14

3.1.7	Transforms and transform type selection	14

3.1.8	Quantization and quantization type selection	14

3.1.9	Entropy coding	14

3.1.10	Additional tools	14

3.1.11	Additional encoder optimization	14

3.1.12	Additional algorithmic description topic(s)	14

3.2	360°	14

3.2.1	Usage of projection format adaptation	14

3.2.2	360° Video specific encoding tools	14

3.2.3	Additional encoder optimization	15

3.3	HDR	15

3.3.1	Pre-processing	15

3.3.2	HDR specific encoding tools	15

3.3.3	Additional encoder optimization	15

4	Compression performance	15

4.1	SDR: Constraint set 1 configuration relative to HM anchor	15

4.2	SDR: Constraint set 2 configuration relative to HM anchor	15

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	16

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	16

4.5	Visual examples	16

4.6	HDR: Constraint set 1 configuration relative to HM anchor	19

4.7	HDR: Constraint set 1 configuration relative to JEM anchor	20

5	Decoder complexity analysis	20

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	20

5.2	Description of computing platform used to determine decoding times reported in section 5.1	20

5.3	Memory usage of decoder	21

5.4	Complexity characteristics of decoder entropy decoding operation	21

5.5	Complexity characteristics of decoder inverse quantization	21

5.6	Complexity characteristics of decoder inverse transform operation	21

5.7	Complexity characteristics of decoder-side motion refinement	21

5.8	Complexity characteristics of decoder motion compensation	21

5.9	Complexity characteristics of decoder intra-frame prediction operation	21

5.10	Complexity characteristics of decoder in-loop filtering operation	21

5.11	Complexity characteristics of additional decoder tools	22

5.12	Complexity characteristics of 360° video specific decoding tools	22

5.13	Complexity characteristics of HDR specific decoding tools	22

5.14	Degree of capability for decoder parallel processing	22

6	Encoder complexity analysis	22

6.1	Encoding time and measurement methodology	22

6.2	Description of computing platform used to determine encoding times reported in section 6.1	22

6.3	Memory usage of encoder	22

6.4	Complexity characteristics of encoder intra-frame prediction type selection	22

6.5	Complexity characteristics of encoder motion estimation and motion segmentation selection	23

6.5.1	General	23

6.5.2	Complexity characteristics of usage of decoder-side motion refinement	23

6.6	Complexity characteristics of encoder transforms and transform type selection	23

6.7	Complexity characteristics of encoder quantization and quantization type selection	23

6.8	Complexity characteristics of encoder in-loop filtering type selection and parameter estimation	23

6.9	Complexity characteristics of encoder entropy coding type selection	23

6.10	Complexity characteristics of additional encoder tools	23

6.11	Complexity characteristics of 360° video specific encoding tools	23

6.12	Complexity characteristics of HDR specific encoding tools	23

6.13	Degree of capability for encoder parallel processing	23

7	Algorithmic characteristics	23

7.1	Random access characteristics	23

7.2	Delay characteristics	23

7.3	Additional characteristics discussion topic(s)	24

8	Software implementation description	24

9	Highlighted aspects discussion	24

10	Closing remarks	24

11	Patent rights declaration(s)	24

12	Annex 1: Questionnaire	24

12.1	Codec level questions	24

12.2	Tool level questions	25

# Introduction

The response described in this contribution is based on JEM 7.0. Thus, in the following sections we concentrate on describing the differences between the proposal and JEM 7.0.

# Decoder algorithm description

## General algorithm description

### Bitstream structure

As in JEM 7.0, except that there are some new syntax elements as described in the following sections.

### Entropy decoding

As in JEM 7.0, except for added syntax elements.

### Conversion process from entropy decoded data to symbols

#### Intra prediction parameters

As in JEM 7.0, except that there is a new syntax element indicating usage of wide-angle intra prediction. That one bin syntax element is read from the bitstream if the signalled intra prediction mode has an associated wide-angle mode as described in section 2.1.8.1. One new CABAC context is introduced for this flag.

#### Motion parameters

As in JEM 7.0, except that the sign of the x component of the MVD is not signaled in the bitstream when the MVD sign data hiding tool (see 2.1.7.5) is enabled.

#### Quantization parameters

As in JEM 7.0.

#### Transform coefficients

As in JEM 7.0.

#### In-loop filter parameters

As in JEM 7.0.

### Decoder-side motion refinement

As in JEM 7.0.

### Inverse quantization

As in JEM 7.0.

### Inverse transforms

As in JEM 7.0.

### Motion compensation

As in JEM 7.0, except for the changes described in the following sections.

#### Affine flexing

The affine flexing tool calculates the horizontal and vertical motion vector differences on the affine sub-PU boundaries. Based on whether the nature of motion is more rotational or zooming, the method decides to apply either lateral compensation (moving the sub-PU boundary samples laterally with respect to the neighboring sub-PU samples) or perpendicular compensation (moving the sub-PU boundary samples away or closer to the samples of the neighboring sub-PU), respectively.

Figure af-1: Reference blocks for block based affine motion compensation. Left side: illustration of rotating motion - ideal border sample locations are offset laterally across the sub-PU borders. Right side: illustration of zooming motion – now the ideal sample locations are offset perpendicularly across the sub-PU borders.

Figure af-2: Prediction blocks after block based affine motion compensation. Left side (rotating motion): final border samples can be obtained by resampling the border lines laterally. Right side (zooming motion): now the final samples are obtained by perpendicular resampling of the PU lines.

In the proposed configuration of the affine flexing tool, OBMC is switched off and flexing is applied for all the affine PUs. That is, there is no bitstream signaling associated with the tool. However, if the motion difference between the sub-PUs is very small, the displacement associated with flexing may quantize to zero and effectively flexing is then switched off. Also, if the motion difference is large enough, flexing is switched off as it doesn’t seem to provide benefits in extreme scenarios. The limits for the displacement are full pixel and half pixel distance in the case of lateral and perpendicular compensation, respectively.

Line displacement parameters are calculated separately for the border sample lines and for lines one sample away from the border. The resulting displacement parameters are evaluated at 1/64 sample accuracy and the flexing (or “resampling”) operation is performed with the 8-tap affine interpolation filters of the KTA software. Implementation includes also a possibility to use 1/16 sample accuracy and JEM motion compensation filters for the process.

Further details:

- Flexing is applied to both horizontal and vertical sub-PU boundaries and the result of the flexing in first dimension is used as an input to the second dimension

- Flexing is applied only for the luma channel in the case of 4:2:0 video

- Line ends are padded with the last available samples

- OBMC is switched off for affine blocks, but still applied for non-affine blocks

On the encoder side the proposed implementation applies flexing only in the final motion compensation stage and not in the affine motion search phase to minimize encoder complexity.

#### Restricted OBMC

This proposed tool identifies cases when OBMC is less beneficial and restricts the OBMC usage for those cases. The modification consists of two parts. The first part is to calculate the spatial activity for a prediction block after motion compensation, and decide whether to apply OBMC or not by comparing the spatial activity of the prediction block against a predefined threshold determined by the QP.

The spatial activity is computed as the averaged 2-D Laplacian activity for luma samples of the prediction block. The samples at the block boundary are not considered during the calculation. For a block with a size of , the averaged activity  is calculated as:

The spatial activity threshold is derived as , where  is fetched from a look-up table using the slice QP as the entry. OBMC is disabled if La is lower than the threshold.

| QP | 51 | 50 | 49 | 48 | 47 | 46 | 45 | 44 | 43 | 42 | 41 | 40 | 39 | 38 | 37 | 36 | 35 | 34 | 33 | 32 | 31 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| K | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 4 | 4 | 4 |
| QP | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| K | 4 | 6 | 6 | 8 | 8 | 10 | 12 | 14 | 14 | 16 | 16 | 16 | 16 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| QP | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| K | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The second part operates on the sub-PU level of a prediction block. For a current sub-PU, its local illumination (LIC) flag is compared with the LIC flags of neighboring sub-PUs. When the LIC flags are different, the MV of that neighboring sub-PU will not be used for OBMC of the current sub-PU. The neighboring sub-PU is the sub-PU which shares the same boundary with the current block that OBMC is operating on. When OBMC operates on the left boundary of the current sub-PU, the LIC flag of the left neighboring sub-PU is checked. When OBMC operates on the above boundary of the current sub-PU, the LIC flag of the above neighboring sub-PU is checked. When OBMC operates on both the left and above boundary of the current sub-PU, then the LIC flags of both the left and above neighboring sub-PU are checked.

It should be noted that the restricted OBMC is enabled only when the OBMC flag of the current block is implicitly derived to be equal to 1, which means the current block is either coded with merge flag equal to 1 or has a size larger than 256 luma samples.

#### Motion vector look back

This proposed tool modifies the merge candidate list generation by including additional neighboring motion vectors. The additional candidates are fetched from two places. One from the closest non-neighboring block in straight horizontal spatial distance and the other one from the closest non-neighboring block in straight vertical spatial distance. The horizontal spatial distance is measured using the left-bottom neighboring sub-PU as the reference position. The vertical spatial distance is measured using the right-above neighboring sub-PU as the reference point. The reference positions are A1 and B1 for horizontal and vertical direction respectively are shown in the figure below.

Figure mvlb-1: Illustration of the maximum distance used for motion vector look back

The fetching process starts with tracing the previous decoded blocks in the vertical direction. The vertical inverse tracing stops when an inter block is encountered or the traced back distance reaches 1 CTU size. The fetching process then traces the previous decoded blocks in the horizontal direction and stops when an inter block is encountered or the traced back distance reaches 1 CTU size.

The fetched motion vectors are then added before the normal TMVP candidate in the merge candidate list, i.e. after the normal spatial candidates and the advanced TMVP candidates. It should be noted that the fetched motion vectors will not be added when there is already enough number of normal spatial candidates, currently that number is set to 4. The modified merge candidate list generation process is shown as follows:

Figure mvlb-2: Proposed merge candidate list generation process

#### Motion vector prediction between list

This proposed tool exploits correlation between the two motion vectors for bi-prediction when the current block is coded using the AMVP mode. The tool allows a motion vector in a bi-prediction pair to be scaled and used as a predictor for the second vector. Furthermore, the tool makes the reconstruction order of the bi-prediction pair adaptively depending on how promising the L1 motion vector prediction candidate list is. A promising candidate list here means a list containing a non-scaled prediction candidate. When FRUC is enabled, the FRUC candidate is counted as a non-scaled candidate.

It should be noted that no change is made to the parsing of the motion vector differences. The reconstruction process of the bi-prediction pair starts with deriving the two candidate lists for L0 and L1 as normal. If the L1 candidate list does not contain a scaled candidate, the L1 vector is determined to be promising and therefore get reconstructed first. After that, if the L0 prediction list contains a scaled candidate, the first scaled candidate in the L0 candidate list is replaced by a scaled version of the L1 vector. Otherwise, if the L1 candidate list does contain a scaled candidate, the L0 vector is reconstructed first. After that, the first scaled candidate in the L1 candidate list is replaced with a scaled version of the L0 vector.

The tool is not enabled when the two reference picture lists contains the same reference pictures with mvd_l1_zero_flag being enabled.

#### Motion vector difference sign data hiding

In this tool, the sign of the x component of the MVD is derived based on the magnitude of both components as well as the reference picture index and the size information of the current block. The sign of the x component is not signaled to the decoder but derived using the formula below. Inside the formula, Magnitude represents the value of the magnitudes. RefPicIdx represents the value of the reference picture index. PartWidth and PartHeight represents the width and height of the current block.

The tool is enabled for the AMVP mode. When the block is coded with affine inter (AF_INTER) mode or with the IMV flag larger than 0 (MVD in one luma sample resolution or 4 luma sample resolution), the tool is disabled.

### Intra prediction

As in JEM 7.0, except for the following changes.

#### Wide-angle intra prediction

In the case of non-square intra prediction blocks, the reference samples for the intra prediction process can sometimes be relatively far away from the locations of the predicted samples. It is noted that a similar directional texture structure that is generated by a certain prediction angle can be created using closer reference samples by “flipping” the indicated prediction direction. This is illustrated in figure below.

Figure wi-1: Illustration of intra prediction for a block having larger width than height. Comparing the average distance of the reference samples with the indicated prediction direction one can notice that the reference samples on the above row are closer than the reference samples on the left border (but the angularity with respect to the top row references is too large to be represented with the existing JEM prediction directions).

In order to take advantage of this phenomenon, it is proposed to allow intra prediction from angles extending beyond the traditional 45 degree top-right and bottom-left directions. In order to keep signalling overhead and computational complexity low it is further proposed to allow these angles only for non-square blocks as follows:

- Angles going beyond 45 degrees in top-right direction if block width is larger than block height

- Angles going beyond 45 degrees in bottom-left direction if block height is larger than block width

Signalling for the tool is implemented by sending a 1-bin identifier for those directions that have an available wide-angle “flip mode”. In the case of the 65-direction JEM angular intra prediction, the availability of the new modes is limited to the 10 directional modes closest to the 45-degree diagonal top-right and bottom-left modes.

The actual sample prediction process follows the one in JEM, but the effective prediction direction for the wide-angle modes is calculated by inverting the signalled prediction direction and switching the reference arrays (using top instead of left or left instead of top).

The figure below illustrates the process for the hypothetical case of 33 HEVC directions and limiting the operation to two modes closest to the diagonal modes. In this example, if the block width is larger than height, horizontal +21 and +26 modes would have extra flags indicating whether to use the indicated mode or corresponding vertical directions with +48 and +39 directions, respectively. Similarly, if the block height was larger than width, vertical +21 and +26 modes would receive that signalling instead.

Figure wi-2: Illustration of the signalled modes (directional parameters < 32) and corresponding wide-angle modes (directional parameters > 32)

Further details:

- Reference samples outside of the existing reference sample arrays are padded

- Context based arithmetic coding of the flag uses one context

- JEM intra slices have their own chroma QTBT. In this case chroma has its own wide-angle mode signalling. In the case of inter slices (and if QTBT is #defined off) chroma uses wide-angle mode of the corresponding luma block.

- Intra mode prediction and prediction error coding are not modified. Thus, those assume the direction of the signalled original mode (not the final wide-angle mode, but naturally both the modes tend to generate a texture structure with similar directionality).

- There are no extra RDO checks in the encoder – the new directions are input in the initial fast search and enter the final RDO stage with the same rules as traditional directions.

#### Multiple boundary filtering for planar and DC mode

This proposed MBF method filters a sample in the boundary area for the planar and DC Intra modes as follows:

- The top-left part of the boundary area is using a distance adaptive three tap filter using the closest reference sample from the block above, the closest reference sample from the left block and the current sample

- The top-right part of the boundary area is using a distance adaptive two tap filter using the closest reference sample from the block above and the current sample

- The bottom-left part of the boundary area is using a distance adaptive two tap filter using the closest reference sample from the left block and the current sample

Figure mbf-1: Illustration of the samples used by the proposed multi boundary filter

PDPC planar is proposed to be replaced by a planar mode using multiple boundary filtering (MBF) which only filters the left and the top boundaries of a block predicted by planar prediction. The reference sample filtering approach from PDPC planar is still used. The filtering of respective area of the boundary is done as follows:

Top-left area (Distance adaptive 3 tap filtering):

S2’ = (S3*Cleft+S1*Ctop+S2*(256-Cleft-Ctop) + 128) >> 8

Where Cleft = g(M-1-i) and Ctop = g(M-1-j)

Bottom-left area (Distance adaptive 2 tap filtering):

S7’ = (S6*Cleft+S7*(256-Cleft) + 128) >> 8

Where Cleft = f(M-1-i)

Top-right area (Distance adaptive 2 tap filtering):

S5’ = (S4*Ctop+S5*(256-Ctop) + 128) >> 8

Where Ctop = f(M-1-j)

Where for min(CUwidth,CUheight) >= 8:

g = (32, 58, 88, 118) and f = (32, 64, 96, 128)

Otherwise:

g = (64, 118) and f = (64, 128)

The one sample boundary smoothing for the DC mode in JEM 7.0 is proposed to be replaced by multiple boundary filtering (MBF) where the length of the MBF filter is determined by the size (minimum of width and height of the block) and the use of non-separable transforms (NSST) as indicated in the following table:

| size | filter length NSST used | filter length NSST not used |
| --- | --- | --- |
| 32 | 15 | 7* |
| 16 | 7 | 7* |
| 8 | 4 | 1 |
| 4 | 2 | 1 |

If NSST is used a strong filter is used, otherwise a weaker filter is used.

The filtering of the respective area of the boundary is done as follows:

Top-left area (Distance adaptive 3 tap filtering):

S2’ = (S3*Cleft+S1*Ctop+S2*(28-Cleft-Ctop) + 28-1) >> 8

Where Cleft = g(M-1-i)+j*d(M-1-i) and Ctop = g(M-1-j)+i*d(M-1-j)

Bottom-left area (Distance adaptive 2 tap filtering):

S7’ = (S6*Cleft+S7*(2N-Cleft) + 2N-1) >> N

Where Cleft = f(M-1-i)

Top-right area (Distance adaptive 2 tap filtering):

S5’ = (S4*Ctop+S5*(2N-Ctop) + 2N-1) >> N

Where Ctop = f(M-1-j)

Where the filters are described in the following table:

| filter length(M) | filter top-left area (g) | delta filter top-left area (d) | filter top-right and bottom-left area (f) | Scale factor N NNST used | Scale factor N NNST not used |
| --- | --- | --- | --- | --- | --- |
| 15 | (8, 16, 24, 32, 40, 48, 56, 64, 72, 80, 88, 96, 104, 112, 120) | (1, 1, 1, 2, 2, 3, 4, 4, 5, 5, 6, 6, 7, 8, 8) | Same as top-left | 8 | - |
| 7 | (2, 24, 46, 64, 82, 104, 126) | (5, 7, 8, 10, 13, 16, 17) *(2, 2, 2, 2, 2, 2, 1) | (16, 32, 48, 64, 80, 96, 112) | 7 | 8 |
| 4 | (5, 43, 87, 123) | (13, 20, 23, 29) | (25, 51, 77, 103) | 7 | - |
| 2 | (14, 114) | (58, 70) | (43, 85) | 7 | - |
| 1 | (64) | 0 | (64) | - | 8 |

The effect for the DC mode is improved details and a reduction of blocking artifacts, one example is shown below.

Figure mbf-2: Effect from FoodMarket, frame 0 at R1, using the proposed MBF for DC
(Left side: JEM 7.0. Right side: JEM 7.0 with MBF for DC)

For Planar mode, stripe like artifacts from PDPC planar can be suppressed, one example is shown below.

Figure mbf-3: Effect from Campfire, frame 21 at R1, using the proposed MBF for planar mode
(Left side: JEM 7.0. Right side: JEM 7.0 with MBF for planar)

### In-loop filtering

As in JEM 7.0, except for the following deblocking filter modifications:

- Extended to include longer filters for luma

- Filtering of sub-PU boundaries for luma

- Filtering of LIC boundaries for both luma and chroma

- Filtering of internal boundaries of chroma block in intra slices when cross-component prediction results in prediction from multiple luma blocks

- Filtering is made parallel-friendly.

#### Super-strong deblocking filters and decisions

The super-strong deblocking filters are designed to linearly interpolate from a virtual sample value on respective side of the block boundary (refQ, refP) towards a virtual value centered in the middle of the block boundary (refMiddle) along a line of sample values perpendicular to the block boundary. The filtering of one sample along the i:th line of samples is done as follows:

pi’(x) = Clip3(pi(x)-tc, pi(x)+tc, (f(x)*refMiddlei + (64-f (x))*refPi + 32) >> 6)

qi’(x) = Clip3(qi(x)-tc, qi(x)+tc, (f(x)*refMiddlei + (64-f(x))*refQi + 32) >> 6)

where pi(0) is the sample value closest to the block boundary in block P and qi(0) is the sample value closest to the block boundary in block Q for line i.

The values of refPi, refQi and refMiddlei for each filter size are computed as shown in the table below. Filtering is done for x ranging from 0 to size-1.

| size | refPi | refQi | refMiddlei | f |
| --- | --- | --- | --- | --- |
| 3 | pi3+pi2+1≫1 | qi3+qi2+1≫1 | k=023·pik+qi(k)-pi2-pi2+8≫4 | 53,32,11 |
| 5 | pi5+pi4+1≫1 | qi5+qi4+1≫1 | k=04pik+qi(k)+k=02pik+qi(k)+8≫4 | 58,45,32,19,6 |
| 7 | pi7+pi6+1≫1 | qi7+qi6+1≫1 | k=16pik+qi(k)+2·pi0+2·qi0+8≫4 | 59,50,41,32,23,14,5 |

The filter with size 7 can be used if both the current CU and the neighbouring CU has a width >= 32 and a height >=32. The boundary is partitioned into segments with the length of 16 samples. For a segment to be filtered, 8 lines are checked and all 8 checks need to pass. The check consists of the HEVC weak/strong filter decision which needs to indicate strong filtering, and some additional smoothness checks (1-D Laplacian) that need to indicate that the line is smooth.

Otherwise, the filter with size 5 can be used if the CUs are of size >= 16. Similar checks as for the size 7 filter are used, but with fewer checks for smoothness.

Otherwise, the filter with size 3 can be used if the CUs are of size >= 8. Similar checks as for the size 7 and 5 filters are used, but with fewer checks for smoothness.

If none of above super-strong filter decisions are true, the HEVC deblocking filter and decisions are used.

It is also ensured that no filter size is used such that CUs can not be filtered in parallel.

The effect of the super-strong filters is a significant reduction in blocking artifacts for large blocks, one example for an intra picture is shown below.

Figure db-1: Effect from FoodMarket, frame 64 at R1, using longer deblocking filters
(Left side: JEM 7.0. Right side: JEM 7.0 with the proposed deblocking filter)

#### Deblocking of sub-PU boundaries

Sub-PU boundaries originate from one of FRUC, MV prediction or AFFINE coding modes and those are not deblocked in JEM 7.0. They can cause blocking artifacts so therefore we propose to set the boundary strength equal to 1 for a sub-PU boundary if any of the following conditions are true:

- if any of the absolute value differences between motion vector components for the two sides of the sub-PU boundary exceeds a threshold

- if different reference pictures are used for the two sides of the sub-PU boundary

- if one side of the sub-PU boundary uses uni-directional prediction and the other uses bi-prediction

Deblocking of sub-PU boundaries only reads and modifies samples within the luma coded block of the CU. Thus, the filtering of sub-PU boundaries for different CUs can be done in parallel. Sub-PU boundaries that are separated by only 4 samples needs to be filtered from left to right for vertical sub-PU boundaries and from top to bottom for horizontal sub-PU boundaries. The filtering of sub-PU boundaries that are separated by more than 4 samples read and modify at most half the minimal distance between the current sub-PU boundary and the closest neighbouring sub-PU boundary. Thus, if the minimal distance between sub-PU boundaries is larger than 4 the filtering of sub-PU boundaries in a CU can be performed in parallel.

The effect is a significant reduction of artifacts from discontinuities at sub-PU boundaries, one example is shown below.

Figure db-2: Effect from FoodMarket, frame 1 at R1, filtering sub-PU boundaries
(Left side: JEM 7.0. Right side: JEM 7.0 with the proposed deblocking filter)

#### Deblocking of LIC block boundaries

Local illumination compensation (LIC) scales or offsets the prediction block which can cause discontinuities across blocks. Therefore, if any block adjacent to the block boundary uses LIC, the boundary strength is set to 1 for luma and 2 for chroma. One example of the effect is shown below.

Figure db-3: Effect from Campfire, frame 30 at R1, deblocking LIC boundaries
(Left side: JEM 7.0. Right side: JEM 7.0 with the proposed deblocking filter)

#### Deblocking of internal boundaries of chroma blocks that have been predicted from luma

Luma and chroma are coded with separate coding structures for intra slices which causes internal boundaries in chroma blocks that has been predicted from luma using CCLM prediction. If a chroma block in an intra coded CU in an intra slice has been predicted from a luma region and there are internal boundaries in that luma region for which the luma boundary strength is larger than 0, then the corresponding internal boundaries in the chroma block are deblocked using a strength of 1. The filtering is performed at an 8 sample grid for horisontal internal boundaries with block height larger than 8 chroma samples and for vertical internal boundaries with block width larger than 8 chroma samples.

The effect is a reduction in artifacts from discontinuities caused by prediction from luma. The improvements can be seen in still pictures, an example is shown below.

Figure db-4: Effect from Campfire, frame 0 at R1, deblocking cross-component predicted chroma blocks
(Left side: JEM 7.0. Right side: JEM 7.0 with the proposed deblocking filter)

Figure db-5: Effect on the Cr component from Campfire, frame 0 at R1, deblocking predicted chroma blocks
(Left side: JEM 7.0. Right side: JEM 7.0 with the proposed deblocking filter)

### Additional tools

No additional tools, except the ones described in earlier sections.

### Additional algorithmic description topic(s)

N/A

## 360° Video

### Projection format (including padding)

N/A

### Quantization handling

N/A

### 360° video specific decoding tools

N/A

## HDR

### Post-processing

As in JEM 7.0.

### Quantization handling

As in JEM 7.0.

### HDR specific decoding tools

As in JEM 7.0.

# Encoder algorithm description

## General algorithm description

### Rate-distortion optimization

#### Distortion metrics

As in JEM 7.0.

#### Rate-distortion dependency models

As in JEM 7.0.

### Partitioning structure selection

As in JEM 7.0.

### Intra prediction mode estimation and type selection

As in JEM 7.0, except that the new wide-angle modes (available for non-square CUs) are inserted in the initial fast mode search. The new modes can make it to the final RDO stage if comparing competitively against the traditional modes in the fast search stage.

### Motion estimation and motion segment selection

#### General

As in JEM 7.0 except for the following modifications when MV prediction between lists (see 2.1.7.4) or MVD sign data hiding (see 2.1.7.4) is enabled.

When the MVD sign data hiding tool is enabled, the JEM’s default quarter-pel refinement in the final ME stage is replaced with a new refinement module. The search range of the new module is the same as in the JEM 7.0 quarter-pel refinement module. It consists of nine MV points including the best MV point so-far and the 8 surrounding quarter-pel MV points. For each MV point, its MVD against each MVP inside the AMVP list is calculated. If the MVD satisfies the sign data hiding formula, then the MV point and the MVP constitute a valid pair. The valid pair that gives the minimum RD cost will be selected. The RD cost is calculated as SATD + lambda * (bits(MVP index) + bits(MVD)).

The MV prediction between list tool adds a MV refinement module after motion estimation. The module starts when a bi-predictive MV pair is chosen. An initial assumption is that the MV in L0 is reconstructed first so that it may be used for predicting the L1 MV. The module first loops over all the reference pictures inside L1. For each reference picture, the corresponding AMVP list is checked to see whether the list consists of a scaled MVP candidate or not. If yes, the scaled MVP candidate is replaced with a scaled version of the reconstructed L0 MV. Subsequently, the previously selected MV (from the ME) for that reference picture is further adjusted using the same motion search method in ME but with search range set to 1 integer luma sample. After the looping over the L1’s reference pictures is done, the resulted best MV is checked. If the MV’s corresponding AMVP list aligns with the initial assumption, the refinement module stops. Otherwise, the module keeps that MV as the final MV for L1 and starts the same loop for the reference pictures in L0.

#### Usage of decoder-side motion refinement

As in JEM 7.0.

### Mode decisions

As in JEM 7.0.

### In-loop filtering type selection and parameter estimation

As in JEM 7.0. The deblocking filtering is modified but the encoder and decoder use the same processes without the encoder additionally selecting filter types or estimating parameters.

### Transforms and transform type selection

As in JEM 7.0.

### Quantization and quantization type selection

As in JEM 7.0.

### Entropy coding

As in JEM 7.0.

### Additional tools

N/A

### Additional encoder optimization

N/A

### Additional algorithmic description topic(s)

N/A

## 360°

### Usage of projection format adaptation

N/A

### 360° Video specific encoding tools

N/A

### Additional encoder optimization

N/A

## HDR

### Pre-processing

As in JEM 7.0.

### HDR specific encoding tools

As in JEM 7.0.

### Additional encoder optimization

As in JEM 7.0.

# Compression performance

## SDR: Constraint set 1 configuration relative to HM anchor

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -33.92% | -44.27% | -47.54% | 1.69 | 1.29 | 1.50 |
| CatRobot1 | -39.72% | -52.57% | -46.02% | 1.56 | 0.96 | 1.30 |
| DaylightRoad2 | -40.80% | -57.53% | -41.70% | 1.14 | 1.24 | 0.77 |
| ParkRunning3 | -31.87% | -26.36% | -28.75% | 1.02 | 0.45 | 0.41 |
| Campfire | -34.74% | -34.70% | -49.92% | 1.02 | 1.29 | 1.53 |
| BQTerrace | -31.05% | -45.71% | -55.15% | 0.93 | 0.83 | 1.04 |
| RitualDance | -28.08% | -37.99% | -41.74% | 1.50 | 1.09 | 1.37 |
| MarketPlace | -29.58% | -47.91% | -44.49% | 1.04 | 0.84 | 0.81 |
| BasketballDrive | -31.92% | -46.36% | -44.03% | 1.15 | 1.03 | 1.33 |
| Cactus | -35.63% | -45.76% | -41.68% | 1.47 | 0.73 | 1.08 |
| Average SDR-A | -36.21% | -43.09% | -42.79% | 1.29 | 1.04 | 1.10 |
| Average SDR-B | -31.25% | -44.75% | -45.42% | 1.22 | 0.91 | 1.13 |
| Average all | -33.73% | -43.92% | -44.10% | 1.25 | 0.98 | 1.12 |
| Enc Time[%] | 777% |  |  |  |  |  |
| Dec Time[%] | 777% |  |  |  |  |  |

Note: Encoding and decoding time computed on same compute cluster but not same machine. The time comparisons with JEM is more accurate since it is on same machine.

## SDR: Constraint set 2 configuration relative to HM anchor

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -26.47% | -46.77% | -53.21% | 0.79 | 0.55 | 0.64 |
| RitualDance | -21.98% | -35.31% | -36.47% | 1.12 | 0.85 | 1.01 |
| MarketPlace | -20.76% | -43.02% | -41.04% | 0.66 | 0.56 | 0.59 |
| BasketballDrive | -25.56% | -46.15% | -44.39% | 0.96 | 0.99 | 1.31 |
| Cactus | -28.51% | -42.98% | -40.56% | 1.14 | 0.62 | 0.91 |
| Average SDR-B | -24.66% | -42.85% | -43.13% | 0.93 | 0.72 | 0.89 |
| Enc Time[%] | 828% |  |  |  |  |  |
| Dec Time[%] | 582% |  |  |  |  |  |

Note: Encoding and decoding time computed on same compute cluster but not same machine. The time comparisons with JEM is more accurate since it is on same machine.

## SDR: Constraint set 1 configuration relative to JEM anchor

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM 7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -0.94% | -1.16% | -1.41% | 0.04 | 0.02 | 0.03 |
| CatRobot1 | -0.76% | -0.98% | -0.77% | 0.02 | 0.01 | 0.01 |
| DaylightRoad2 | -0.86% | -0.95% | -0.97% | 0.02 | 0.01 | 0.01 |
| ParkRunning3 | -1.11% | -1.65% | -1.90% | 0.03 | 0.02 | 0.02 |
| Campfire | -0.59% | -0.16% | -0.74% | 0.01 | 0.01 | 0.01 |
| BQTerrace | -1.02% | -1.03% | -1.47% | 0.02 | 0.01 | 0.02 |
| RitualDance | -1.05% | -1.68% | -2.01% | 0.05 | 0.03 | 0.05 |
| MarketPlace | -0.56% | -1.15% | -0.14% | 0.02 | 0.01 | 0.00 |
| BasketballDrive | -1.10% | -1.71% | -1.17% | 0.03 | 0.02 | 0.03 |
| Cactus | -1.00% | -0.98% | -0.92% | 0.03 | 0.01 | 0.02 |
| Average SDR-A | -0.85% | -0.98% | -1.16% | 0.02 | 0.01 | 0.02 |
| Average SDR-B | -0.95% | -1.31% | -1.14% | 0.03 | 0.02 | 0.02 |
| Average all | -0.90% | -1.14% | -1.15% | 0.03 | 0.02 | 0.02 |
| Enc Time[%] | 103% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  |  |  |  |

## SDR: Constraint set 2 configuration relative to JEM anchor

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM 7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -0.27% | 1.57% | -4.74% | 0.01 | -0.01 | 0.03 |
| RitualDance | -0.36% | -1.51% | -1.51% | 0.02 | 0.03 | 0.03 |
| MarketPlace | -0.08% | -0.62% | -0.63% | 0.00 | 0.01 | 0.01 |
| BasketballDrive | 0.14% | -0.70% | -0.92% | 0.00 | 0.01 | 0.02 |
| Cactus | -0.30% | -1.13% | -1.86% | 0.01 | 0.01 | 0.03 |
| Average SDR-B | -0.17% | -0.48% | -1.93% | 0.01 | 0.01 | 0.02 |
| Enc Time[%] | 99% |  |  |  |  |  |
| Dec Time[%] | 95% |  |  |  |  |  |

## Visual examples

Here we have collected some examples of improvements of subjective performance of the proposal in comparison to JEM 7.0 for some sequences at the lowest bitrate (R1) of the CfP.

Figure ve-1: Campfire, SDR RA, frame 53 at R1
(Left side: JEM 7.0. Right side: Proposal)

Figure ve-2: FoodMarket, SDR RA, frame 86 at R1
(Left side: JEM 7.0. Right side: Proposal)

Figure ve-3: ParkRunning, SDR RA, frame 40 at R1
(Left side: JEM 7.0. Right side: Proposal)

Figure ve-4: RitualDance, SDR LD, frame 141 at R1
(Left side: JEM 7.0. Right side: Proposal)

Figure ve-5: MarketPlace, SDR LD, frame 599 at R1
(Left side: JEM 7.0. Right side: Proposal)

Figure ve-6: BasketballDrive, SDR LD, frame 75 at R1
(Left side: JEM 7.0. Right side: Proposal)

## HDR: Constraint set 1 configuration relative to HM anchor

|  | BD-rate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| DayStreet | -58.2% | -36.7% | -35.4% | -55.6% | -68.3% | -34.5% | -54.1% | -66.4% |
| PeopleInShoppingCenter | -50.1% | -29.9% | -28.8% | -58.5% | -54.4% | -28.6% | -56.2% | -53.9% |
| SunsetBeach | -55.4% | -26.7% | -21.5% | -81.2% | -79.0% | -22.4% | -76.4% | -79.6% |
| Market | -40.2% | -24.7% | -25.3% | -46.7% | -44.9% | -23.7% | -39.6% | -36.2% |
| ShowGirl2 | -29.5% | -26.9% | -28.2% | -62.4% | -33.7% | -26.4% | -59.6% | -29.7% |
| Hurdles | -49.4% | -35.0% | -34.1% | -59.9% | -55.6% | -31.4% | -54.8% | -51.2% |
| Starting | -34.4% | -30.8% | -30.3% | -34.2% | -37.7% | -28.7% | -31.9% | -23.7% |
| Cosmos1 | -31.8% | -25.1% | -25.2% | -67.4% | -79.6% | -23.8% | -66.8% | -79.6% |
| Average HDR-A | -54.6% | -31.1% | -28.6% | -65.1% | -67.3% | -28.5% | -62.2% | -66.7% |
| Average HDR-B | -37.1% | -28.5% | -28.6% | -54.1% | -50.3% | -26.8% | -50.5% | -44.1% |
| Average all | -43.6% | -29.5% | -28.6% | -58.2% | -56.6% | -27.4% | -54.9% | -52.5% |
| Enc Time[%] | 610% |  |  |  |  |  |  |  |
| Dec Time[%] | 827% |  |  |  |  |  |  |  |

Note: Encoding and decoding time computed on same compute cluster but not same machine. The time comparisons with JEM is more accurate since it is on same machine.

## HDR: Constraint set 1 configuration relative to JEM anchor

|  | BD-rate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| DayStreet | -0.5% | -0.5% | -0.4% | -0.5% | -1.0% | -0.6% | -0.5% | -0.8% |
| PeopleInShoppingCenter | -0.7% | -0.6% | -0.6% | -0.7% | -0.7% | -0.6% | -0.8% | -0.7% |
| SunsetBeach | -0.2% | -0.7% | -0.5% | 0.2% | -0.3% | -0.5% | 0.1% | -0.4% |
| Market | -0.9% | -0.4% | -0.3% | -1.1% | -1.9% | -0.3% | -1.0% | -1.3% |
| ShowGirl2 | -2.7% | -0.7% | -1.0% | -4.3% | -5.6% | -1.0% | -5.3% | -5.9% |
| Hurdles | -0.1% | -0.5% | -0.8% | -0.8% | -0.7% | -0.9% | -0.5% | -0.5% |
| Starting | -0.5% | 0.0% | 0.0% | 0.1% | -1.2% | 0.0% | 0.0% | -0.9% |
| Cosmos1 | -1.9% | -1.3% | -1.4% | -2.8% | -4.2% | -1.4% | -2.5% | -3.2% |
| Average HDR-A | -0.5% | -0.6% | -0.5% | -0.3% | -0.7% | -0.5% | -0.4% | -0.7% |
| Average HDR-B | -1.2% | -0.6% | -0.7% | -1.8% | -2.7% | -0.7% | -1.9% | -2.3% |
| Average all | -0.9% | -0.6% | -0.6% | -1.2% | -2.0% | -0.7% | -1.3% | -1.7% |
| Enc Time[%] | 103% |  |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |  |  |

# Decoder complexity analysis

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

The decoder complexity comparisons were done against JEM 7.0 since that is what the proposal is based on. The complexity is as follows:

99% decoding time for HDR.

98% decoding time for SDR RA.

95% decoding time for SDR LD.

## Description of computing platform used to determine decoding times reported in section 5.1

64-bit Linux executables were used to run reference encoder/decoder and test encoder/decoder on the same core with randomized order of reference and test. The code was compiled using gcc 6.2.0.

## Memory usage of decoder

Similar to JEM 7.0.

## Complexity characteristics of decoder entropy decoding operation

As in JEM 7.0.

## Complexity characteristics of decoder inverse quantization

As in JEM 7.0.

## Complexity characteristics of decoder inverse transform operation

As in JEM 7.0.

## Complexity characteristics of decoder-side motion refinement

Reduced compared to JEM 7.0 due to use of OBMC skip for relatively flat prediction blocks.

## Complexity characteristics of decoder motion compensation

Reduced compared to JEM 7.0 due to use of OBMC skip for relatively flat prediction blocks when implicit OBMC is used.

Affine flexing tool adds extra computations as an 8-tap FIR filtering is performed for each sample in the prediction block horizontally and vertically. However, overall complexity associated with predicting affine blocks is asserted to be lower than in JEM 7.0 as the OBMC tool is completely switched off for these blocks.

## Complexity characteristics of decoder intra-frame prediction operation

Simplified for PDPC planar since MBF planar only filters 4 or 2 sample boundary of the block. DC mode is made more complex since MBF DC filters of up to 15 sample boundary but since the DC mode is not the most complex mode the average complexity is expected to be reduced.

There is one extra block level check to determine if wide-angle intra prediction is applied. In the case of active wide-angle intra prediction, the directionality parameter for angular intra prediction is calculated using the same inverse angle equation that is used for projecting the side reference samples for negative prediction angles. However, as there is no need for the side sample projection (wide-angle intra is only applicable to positive angles) the worst-case complexity of generating prediction with wide-angle intra directions is lower than that of the HEVC intra prediction directions.

## Complexity characteristics of decoder in-loop filtering operation

The bilateral, ALF, and SAO filters are all identical with JEM 7.0.

The deblocking filter is more computationally complex compared to JEM 7.0 since filter lengths up to 7 samples on each side of a block boundary is supported.

Compared with JEM 7.0, the proposal additionally filters the following boundaries:

- LIC blocks may be filtered

- Internal sub-PU boundaries in blocks using sub-PU motion compensation may be filtered

- Internal chroma block boundaries that are predicted from luma in intra slices may be filtered

The proposed deblocking filter is more parallel-friendly compared with JEM 7.0 since all CU boundaries can be filtered in parallel.

## Complexity characteristics of additional decoder tools

The MV look back tool increases the logical complexity of the merge candidate list derivation as it fetches motion vectors from non-neighboring blocks by scanning the previous decoded blocks in vertical or horizontal direction. However, the logical complexity increase is asserted to be marginal as the sample scan distance is restricted to 1 CTU size.

The MV prediction between list tool makes the reconstruction order of a bi-prediction MV pair (MV for L0 and MV for L1) adaptive by checking whether the AMVP list for L1 consists of a scaled MVP candidate or not. Additionally, it modifies one of the AMVP list by replacing one of the MVP candidates. The complexity of these operations is asserted to be marginal.

The MVD sign data hiding tool derives the sign of the MVD x component from other parameter values instead of reading it from the bit stream. The derivation is done by simple add and operations.

## Complexity characteristics of 360° video specific decoding tools

N/A

## Complexity characteristics of HDR specific decoding tools

N/A

## Degree of capability for decoder parallel processing

As in JEM 7.0 but deblocking of CU boundaries can be filtered in parallel.

# Encoder complexity analysis

## Encoding time and measurement methodology

The encoder complexity comparisons were done against JEM 7.0 since that is what the proposal is based on. The complexity is as follows:

103% encoding time for HDR.

103% encoding time for SDR RA.

99% encoding time for SDR LD.

## Description of computing platform used to determine encoding times reported in section 6.1

64-bit Linux executables were used to run reference encoder/decoder and test encoder/decoder on the same core with randomized order of reference and test. The code was compiled using gcc 6.2.0.

## Memory usage of encoder

Similar to JEM 7.0.

## Complexity characteristics of encoder intra-frame prediction type selection

Slightly more complex than JEM 7.0 as the initial fast search has more candidates due to the wide-angle intra prediction tool.

## Complexity characteristics of encoder motion estimation and motion segmentation selection

### General

As in JEM 7.0 except for the additional MV refinements (see 3.1.4.1) when MV prediction between lists (see 2.1.7.4) or MVD sign data hiding (see 2.1.7.5) is enabled.

### Complexity characteristics of usage of decoder-side motion refinement

As in JEM 7.0.

## Complexity characteristics of encoder transforms and transform type selection

As in JEM 7.0.

## Complexity characteristics of encoder quantization and quantization type selection

As in JEM 7.0.

## Complexity characteristics of encoder in-loop filtering type selection and parameter estimation

As in JEM 7.0.

## Complexity characteristics of encoder entropy coding type selection

As in JEM 7.0.

## Complexity characteristics of additional encoder tools

N/A

## Complexity characteristics of 360° video specific encoding tools

N/A

## Complexity characteristics of HDR specific encoding tools

N/A

## Degree of capability for encoder parallel processing

As in JEM 7.0.

# Algorithmic characteristics

## Random access characteristics

As in JEM 7.0.

## Delay characteristics

As in JEM 7.0.

## Additional characteristics discussion topic(s)

N/A

# Software implementation description

JEM 7.0 with all tools enabled was used as basis for the proposal. In the implementation, each modification to JEM software is separated with a corresponding compile time switch defined in the TypeDef.h header file.

# Highlighted aspects discussion

# Closing remarks

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex 1: Questionnaire

## Codec level questions

- Maximum number of reference frames used by encoder

As in JEM 7.0.

- Maximum coding unit (e.g. CTU in HEVC)

As in JEM 7.0.

- Minimum and maximum transform block size

As in JEM 7.0

- Minimum and maximum intra prediction block

As in JEM 7.0

- Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length

As in JEM 7.0

- List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category)

As in JEM 7.0

- List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples)

None (or as in JEM 7.0)

- List all tools that use non-rectangular (e.g. triangle) partitions

None.

- Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa)

No.

- Indicate whether luma and chroma use separate partitioning structures

As in JEM 7.0

- Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes)

As in JEM 7.0

- If the coded picture resolution is different from the input picture resolution, list the coded picture resolution for those sequences (e.g. 360° video, input is 8K ERP, coded video is in 4K CMP converted from the 8K input)

N/A

## Tool level questions

- Partitioning structure

  - Whether the prediction unit partitioning is different from transform unit partitioning

  - Maximum tree depth

As in JEM 7.0

- Entropy coding

  - Total number of context models

  - Memory storage size for each context model

  - Range table size if table look up is used for range update

  - Initialization table size

  - Other memory associated with context model adaptation

As in JEM 7.0, except that the proposal uses one additional context for signaling the wide-angle Intra mode.

- Control data derivation process

  - Temporal motion data storage granularity

  - Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-predictive reference picture are used, count this picture as 2)

As in JEM 7.0

- Decoder-side motion refinement

  - Line buffer size in terms of motion vector block lines (assuming horizontal motion vector block size is 4 luma samples)

  - Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement

  - Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size

As in JEM 7.0

- Intra prediction

  - Line buffer size in terms of luma and chroma sample lines.

As in JEM 7.0

  - List all the filters used for intra prediction

For MBF used for the planar and the DC mode, two or three tap distance adaptive filter using 7 or 8 bit accuracy of filter coefficients. The filter coefficients can be derived on the fly by table lookup using a table size of 10 for the planar mode and 77 for the DC mode.

  - Indicate whether there are cross-component dependencies in intra prediction reconstruction.

As in JEM 7.0

- Inter prediction

  - Motion vector accuracy

  - Storage size for a motion vector component

  - List all the interpolation filters used for luma/chroma motion compensation

As in JEM 7.0

- Quantization

  - Storage size associated with quantization

As in JEM 7.0

- Transforms

- List all the combinations of transform block size and transform type for primary transforms

- List all the combinations of transform block size and transform type for secondary transforms

- List all the non-separable transforms used

- Indicate whether transforms can be fully/partially implemented using direct matrix multiply

- List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output)

As in JEM 7.0

- In-loop filtering

- List line buffer size in terms of luma and chroma lines (if any)

The proposed luma deblocking modifications increase the line buffer size for deblocking from 4 lines in JEM 7.0 to 8 lines.

- List all the filters used for in-loop filtering

Luma: First a center, left and right virtual sample value is calculated. The center value is calculated from 6, 10, 14 sample values for deblocking filter size 3, 5, 7 respectively using the filters (2,3,3,3,3,2)/16, (1,1,2,2,2,2,2,2,1,1)/16 or (1,1,1,1,1,1,2,2,1,1,1,1,1,1)/16. The left and right virtual sample values are calculated from two sample values using (1,1)/2. Then interpolation between the virtual samples is done using one of seven two tap-filters: (59,5)/64, (50,14) /64, (41,23) /64, (58,6) /64, (45,19) /64, (53,11) /64, (32,32)/64.

Chroma: Same as JEM 7.0.

- Minimum processing unit size.

Same as JEM 7.0.

- “tool-off” and “tool-on” tests

The tool tests are in comparison with JEM-7.0 and encoder/decoder runtime and coding gain can be found in Table 2 to Table 7.

Table 2. Summary of “tool-off” and “tool-on” tests for ENC vs. JEM 7.0 for SDR RA

| Tested tool name | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| MBF DC | 0.1% | 0.4% | 0.2% | 98%* | 100%* | -0.1% | -0.2% | -0.3% | 100% | 100% |
| MBF PLANAR | 0.0% | 0.2% | -0.0% | 99%* | 102%* | 0.0% | 0.1% | -0.1% | 100% | 100% |
| MV PBL | 0.2% | 0.1% | 0.1% | 94%* | 100%* | -0.2% | -0.1% | -0.1% | 104% | 100% |
| MV LB | 0.3% | 0.4% | 0.5% | 100%* | 100%* | -0.3% | -0.3% | -0.5% | 100% | 100% |
| MVD SDH | 0.1% | 0.1% | 0.1% | 99%* | 99%* | 0.0% | 0.1% | 0.0% | 100% | 100% |
| OBMC SKIP | 0.0% | 0.0% | 0.0% | 102% | 101% | 0.0% | 0.1% | 0.1% | 99% | 98% |
| DEBLOCKING | 0.1% | 0.1% | 0.2% | 100% | 98% | 0.0% | 0.0% | -0.2% | 100% | 101% |

Note: *Encoding and decoding time computed on same compute cluster but not same machine.

Table 3 Summary of “tool-off” and “tool-on” tests for ENC vs. JEM.7.0 for SDR LD

| Tested tool name | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| MBF DC | 0.0% | 0.3% | 0.8% | 100% | 101% | -0.0% | 0.0% | -1.0% | 100% | 99% |
| MBF PLANAR | 0.0% | -0.1% | 0.7% | 100% | 100% | 0.0% | 0.3% | 0.4% | 101% | 98% |
| MV PBL | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | 0.0% | 0.1% | 100% | 99% |
| MV LB | 0.1% | 0.5% | 0.3% | 101% | 100% | -0.1% | 0.5% | -0.1% | 100% | 101% |
| MVD SDH | 0.1% | 0.2% | -0.0% | 98% | 100% | -0.1% | 0.8% | 0.2% | 102% | 100% |
| OBMC SKIP | -0.0% | 0.1% | 0.1% | 102% | 102% | 0.1% | 1.0% | 1.0% | 98% | 97% |
| DEBLOCKING | 0.0% | 0.5% | 0.8% | 100% | 98% | 0.1% | 0.2% | -1.3% | 100% | 102% |

Table 4 Summary of “tool-on” tests for ENC vs. JEM.7.0 for HDR (HLG and PQ)

| Tested tool name | “tool-on” test results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  |  |  |  |  |  | Runtime |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | Y | Cb | Cr | Enc | Dec |
| MBF DC | -0.3% | -0.1% | -0.1% | -0.6% | -0.1% | -0.1% | -0.6% | -0.1% | 98%* | 102%* |
| MBF PLANAR | 0.0% | 0.0% | 0.0% | -0.6% | 0.1% | 0.0% | -0.7% | 0.2% | 99%* | 101%* |
| MV PBL | -0.2% | -0.1% | -0.1% | -0.3% | -0.3% | -0.1% | -0.3% | -0.3% | 101%* | 102%* |
| MV LB | -0.4% | -0.3% | -0.3% | -0.7% | -1.0% | -0.3% | -0.7% | -0.7% | 97%* | 100%* |
| MVD SDH | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 98%* | 100%* |
| OBMC SKIP | 0.1% | 0.0% | 0.0% | 0.2% | 0.5% | 0.0% | 0.1% | 0.7% | 96%* | 99%* |
| DEBLOCKING | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.3% | 98%* | 102%* |

Note: *Encoding and decoding time computed on same compute cluster but not same machine.

Table 5 Summary of “tool-off” tests for ENC vs. JEM.7.0 for HDR (HLG and PQ)

| Tested tool name | “tool-off” test results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  |  |  |  |  |  | Runtime |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | Y | Cb | Cr | Enc | Dec |
| MBF DC | 0.1% | 0.1% | 0.1% | 0.2% | 0.3% | 0.1% | 0.2% | 0.3% | 97%* | 102%* |
| MBF PLANAR | 0.1% | 0.0% | 0.0% | 0.3% | 0.6% | 0.0% | 0.2% | 0.5% | 97%* | 102%* |
| MV PBL | 0.2% | 0.1% | 0.1% | -0.2% | 0.4% | 0.1% | -0.1% | 0.4% | 93%* | 100%* |
| MV LB | 0.4% | 0.3% | 0.3% | 0.7% | 1.2% | 0.3% | 0.6% | 1.1% | 98%* | 99%* |
| MVD SDH | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | 0.0% | 96%* | 100%* |
| OBMC SKIP | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | -0.1% | 99%* | 102%* |
| DEBLOCKING | 0.0% | 0.0% | 0.0% | -0.1% | 0.3% | 0.0% | 0.0% | 0.4% | 98%* | 98%* |

Note: *Encoding and decoding time computed on same compute cluster but not same machine.

Table 6. Selected “tool-on” tests for ENC vs. JEM.7.0 for All Intra coding using JVET CTC testset.

| Tested tool name | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec |
| WIDE ANGLE | -0.25% | -0.20% | -0.50% | 104% | 101% |

Table 7. Selected “tool-on” tests for ENC vs. JEM.7.0 for 17 frames Random Access coding using a testset with affine motion (CatRobot, Cactus, Jets, SpinCalendar, TractorZoom, Station2, ShieldsZoom, BlueSky).

| Tested tool name | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec |
| AFFINE FLEX1 | -1.49% | -0.99% | -0.72% | 102% | 102% |
| AFFINE FLEX2 | -0.07% | -0.01% | 0.10% | 99% | 86% |

Note1: Anchor JEM 7.0 with OBMC off, test with OBMC off.

Note2: Anchor JEM 7.0.

[end of document]