| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0013-v1 |
| --- | --- |

| Title: | Description of SDR video coding technology proposal by ETRI and Sejong University |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jung Won Kang, Hahyun Lee, Sung-Chang Lim, Jinho Lee, Hui Yong Kim ETRI 218 Gajeong-ro, Yuseong-gu Daejeon, 34129, South Korea Nam-Uk Kim, Yung-Lyul Lee Sejong University 209 Neungdong-ro, Gwangjin-gu Seoul, 05006, South Korea | Tel: Email: Tel: Email: | +82-42-860-5137 jungwon@etri.re.kr +82-2-3408-3753 yllee@sejong.ac.kr |
| Source: | Electronics and Telecommunications Research Institute (ETRI) and Sejong University |  |  |

_____________________________

- Abstract

This document describes the SDR video coding technology proposal by ETRI and Sejong University in response to the call for proposal issued jointly by MPEG and ITU-T. This proposal is based on JEM7.0 with several modifications to reduce decoder complexity while maintaining the coding efficiency of the proposed codec comparable to the coding efficiency of JEM7.0. PMMVD, DMVR, AMT, adaptive clipping and control and signalling the probability updating speed for the context model adopted in JEM7.0 are disabled in the proposal. For inter prediction, two special merge modes, which are motion refined mode (MRM) and template matched merge (TMM), based on decoder-side motion refinement are newly introduced. For intra prediction, combined filer (CF) combining interpolation filter with reference sampling smoothing filter and multi-line based intra prediction (MIP) method are included. In addition, DST-VII with residual flipping to replace AMT in JEM7.0 and modified ALF called reduced complexity-ALF (RC-ALF) are included. For constraint set 1, average BD-rates are -32.74% compared to HM anchor and 0.64% compared to JEM anchor. For constraint set 2, the average BD-rates are -23.93% compared to HM anchor and 0.82% compared to JEM anchor. It is reported that the average decoding time of the proposed codec is 4.04 times and 3.31 times of HM16.16 decoder for constraint set 1 and for constraint set 2, respectively. It is also reported that the average encoding time of the proposed codec is 8.41 times and 8.18 times of HM16.16 encoder for constraint set 1 and for constraint set 2, respectively.

- Contents

Abstract	i

Contents	ii

1	Introduction	1

2	Decoder algorithm description	2

2.1	General algorithm description	2

2.1.1	Bitstream structure	2

2.1.2	Entropy decoding	3

2.1.3	Conversion process from entropy decoded data to symbols	3

2.1.4	Decoder-side motion refinement	3

2.1.5	Inverse quantization	5

2.1.6	Inverse transforms	5

2.1.7	Motion compensation	7

2.1.8	Intra prediction	7

2.1.9	In-loop filtering	8

3	Encoder algorithm description	10

3.1	General algorithm description	10

3.1.1	Rate-distortion optimization	10

3.1.2	Partitioning structure selection	10

3.1.3	Intra prediction mode estimation and type selection	11

3.1.4	Motion estimation and motion segment selection	11

3.1.5	Mode decisions	11

3.1.6	In-loop filtering type selection and parameter estimation	11

3.1.7	Transforms and transform type selection	12

3.1.8	Quantization and quantization type selection	13

3.1.9	Entropy coding	13

4	Compression performance	14

4.1	SDR: Constraint set 1 configuration relative to HM anchor	14

4.1.1	Class SDR-A (RD-curves)	14

4.1.2	Class SDR-B (RD-curves)	15

4.1.3	Overall (BD-rate summary tables)	15

4.2	SDR: Constraint set 2 configuration relative to HM anchor	16

4.2.1	Class SDR-B	16

4.2.2	Overall	17

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	18

4.3.1	Class SDR-A	18

4.3.2	Class SDR-B	19

4.3.3	Overall	19

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	20

4.4.1	Class SDR-B	20

4.4.2	Overall	21

5	Decoder complexity analysis	22

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	22

5.2	Description of computing platform used to determine decoding times reported in section 5.1	23

5.3	Memory usage of decoder	23

5.4	Complexity characteristics of decoder entropy decoding operation	23

5.5	Complexity characteristics of decoder inverse quantization	23

5.6	Complexity characteristics of decoder inverse transform operation	24

5.7	Complexity characteristics of decoder-side motion refinement	24

5.8	Complexity characteristics of decoder motion compensation	25

5.9	Complexity characteristics of decoder intra-frame prediction operation	26

5.10	Complexity characteristics of decoder in-loop filtering operation	26

5.11	Degree of capability for decoder parallel processing	27

6	Encoder complexity analysis	27

6.1	Encoding time and measurement methodology	27

6.2	Description of computing platform used to determine encoding times reported in section 6.1	28

6.3	Memory usage of encoder	28

6.4	Complexity characteristics of encoder intra-frame prediction type selection	28

6.5	Complexity characteristics of encoder motion estimation and motion segmentation selection	29

6.5.1	General	29

6.5.2	Complexity characteristics of usage of decoder-side motion refinement	29

6.6	Complexity characteristics of encoder transforms and transform type selection	29

6.7	Complexity characteristics of encoder quantization and quantization type selection	29

6.8	Complexity characteristics of encoder in-loop filtering type selection and parameter estimation	29

6.9	Complexity characteristics of encoder entropy coding type selection	30

6.10	Degree of capability for encoder parallel processing	30

7	Algorithmic characteristics	30

7.1	Random access characteristics	30

7.2	Delay characteristics	30

8	Software implementation description	30

9	Highlighted aspects discussion	30

10	Closing remarks	31

11	References	31

12	Patent rights declaration(s)	31

13	Annex: Questionnaire	33

13.1	Codec level questions	33

13.2	Tool level questions	34

Introduction

This document describes the SDR video coding technology proposal by ETRI and Sejong University in response to the call for proposal [1]. This proposal is based on JEM7.0 [2] with several modifications to reduce decoder complexity while maintaining the coding efficiency of the proposed codec comparable to the coding efficiency of JEM7.0. PMMVD, DMVR, AMT, adaptive clipping and control and signalling the probability updating speed for the context model adopted in JEM7.0 are disabled for the proposal. For inter prediction, two special merge modes, which are motion refined mode (MRM) and template matched merge (TMM), based on decoder-side motion refinement are newly introduced. For intra prediction, combined filer (CF) combining interpolation filter with reference sampling smoothing filter and multi-line based intra prediction (MIP) method are included. In addition, DST-VII with residual flipping to replace AMT in JEM7.0 and modified ALF called reduced complexity-ALF (RC-ALF) are included.

Key features of the proposed codec are summarized in Figure 1-1, and a short overview of the key features is as follows:

Figure 1-1: Key features of the proposed codec

- Motion Refined Mode (MRM): a special merge mode based on bilateral template matching used in JEM7.0 and priority-based candidate search is proposed to increase coding efficiency by minimize signalling syntax.

- Template Matched Merge (TMM): to reduce decoder complexity while minimizing coding efficiency degradation compared to pattern matched motion vector derivation (PMMVD) in JEM7.0, another special merge mode called Template Matched Merge (TMM) mode is proposed. TMM mode performs CU-level template matching based motion refinement around an initial MV derived from the signalled TMM index.

- DST-VII with residual flipping: For the core transform, DST-VII with residual data flipping is introduced for the intra- and inter-prediction instead of adaptive multiple transform (AMT) scheme in order to reduce the number of transform matrix compared to AMT.

- Reduced Complexity-ALF (RC-ALF): block classification method and filter shapes are modified to reduce complexity compared to ALF in JEM7.0. 4×4 block classification based on subsampled sum-modified-Laplacian (SSML) is used and four types of 5×5 filter shapes, which are 5×5 square, octagon, snowflake, and diamond filter shapes, are employed instead of 9×9, 7×7, and 5×5 diamond filter shapes.

- Multi-line based Intra Prediction (MIP): to increase the performance of intra coding, the weighted combination of two reference lines for angular intra prediction is used.

- Combined Filter (CF): to reduce the number of operations for angular intra prediction mode, 6-tap interpolation filter that combines 4-tap interpolation filter with 3-tap reference sample smoothing filter is applied.

Decoder algorithm description

General algorithm description

Bitstream structure

Bitstream structure of the proposed codec is basically same as JEM7.0 except the following aspects.

MRM and TMM mode introduce new syntax elements, which are an MRM flag, a TMM flag and a TMM index as shown in Figure 2-1. When the skip flag is true, an MRM flag is signalled to indicate whether MRM is used. When the MRM flag is true, no more syntax element is signalled. When the MRM flag is false, a TMM flag is signalled to indicate whether TMM mode is used. If the TMM flag is true, a TMM index is additionally signalled. Otherwise, general merge index is signalled to perform general skip process. The bitstream structure for skip mode is also applied to merge mode in the same way. On the other hand, since MIP is always applied to angular intra prediction mode, syntax element for intra prediction is not changed. DST-VII with residual flipping and RC-ALF use the similar syntax elements as in JEM7.0 without changing bitstream structure.

Figure 2-1: Bitstream structure changed by the proposed codec

Entropy decoding

In this proposal, multi-hypothesis probability estimation is applied without enabling a method changing the probability updating speed [2]. Except this aspect, entropy decoding is the same as in JEM7.0.

Conversion process from entropy decoded data to symbols

Conversion process from entropy decoded data to symbols is the same as CABAC in JEM7.0.

Intra prediction parameters

It is the same as in JEM7.0.

Motion parameters

It is the same as in JEM7.0.

Quantization parameters

It is the same as in JEM7.0.

Transform coefficients

It is the same as in JEM7.0.

In-loop filter parameters

It is the same as in JEM7.0.

Decoder-side motion refinement

In this proposal, two special merge modes, which are motion refined mode (MRM) and template matched merge (TMM) mode, based on decoder-side motion refinement are presented to improve coding efficiency.

MRM is a newly added merge mode based on decoder-side motion vector refinement method using bilateral template matching which is decoder-side motion vector refinement (DMVR) [2] of JEM7.0 and priority-based candidate search to increase coding efficiency by minimize signalling syntax.

TMM mode is a special merge mode based on CU-level decoder-side motion refinement method around an initial MV derived from the signalled TMM index. In TMM mode, template matching method of pattern matched motion vector derivation (PMMVD) [2] is employed as the CU-level decoder-side motion refinement method. It is claimed that TMM mode reduces decoder complexity by signalling the initial MV information while minimizing coding efficiency degradation compared to PMMVD in JEM7.0.

Motion Refined Mode (MRM)

MRM is a special merge mode based on decoder-side motion vector refinement method using bilateral template matching which is DMVR [2] of JEM7.0 and priority-based candidate search.

An MRM flag is signalled when a skip flag or merge flag is true. When the MRM flag is false, a TMM flag is signalled to indicate whether TMM mode is used or general merge mode is used. When the MRM flag is true, MRM process is performed without additional syntax elements.

In this mode, at the decoder, bilateral template matching is applied to the first available bi-directional MVs among the MRM candidates in the priority order of S0  S1  S2  T0  T2  zero MV shown in Figure 2-2. If there are more than one MRM candidates having bi-directional MVs which are not zero MV, the first available MVs in the given order are chosen to be two initial MVs for bilateral template matching process. If there is no MRM candidate satisfying the condition, zero MVs are used as two initial MVs for bilateral template matching process. Reference picture index for reference picture list0 and list1 to derive temporal MRM candidates (i.e., T0 and T2) is set to median value among reference indices of spatial neighboring candidates for reference picture list0 and list1, respectively.

(a) Positions of spatial MRM candidate         (b) Positions of temporal MRM candidate

Figure 2-2: Positions of MRM candidate

The derived two initial MVs are refined by the bilateral template matching process same as DMVR in JEM7.0. However, unlike the configuration of DMVR in JEM7.0, in this proposal, DMVR with 1/2-pel MV refinement precision is applied. So, in the first round of search, up to 8 MV candidates (1 integer-pel displacement in vertical and horizontal directions) are checked. The second round of search refines MV with 1/2-pel precision.

Template Matched Merge (TMM)

TMM mode is another special merge mode with a CU-level motion refinement process based on template matching at the decoder side. In this mode, initial motion vector is derived from a signalled TMM index, then, CU-level motion refinement around the initial MV based on template matching is performed.

A TMM flag is signalled when a skip flag or merge flag is true and the MRM flag is false for a CU. When the TMM flag is false, a merge index is signalled and the general merge mode is used. When the TMM flag is true, a TMM index (i.e., TMMIndex) is signalled to indicate which TMM candidate is to be used to derive initial motion information for the CU.

Motion derivation process in TMM mode has two steps: 1) initial MV derivation process based on TMM index and 2) CU-level motion refinement process based on the template matching.

In the initial MV derivation process, first, TMM candidate list is constructed in the order of spatial merge candidates, temporal merge candidates, combined bi-predictive merge candidates, zero motion vector merge candidates. A maximum number of TMM candidates (i.e., MaxNumTMMMergeCand) which is signalled in slice header can be up to seven.. In the derivation of TMM spatial merge candidates, a maximum of seven candidates can be selected among TMM spatial merge candidates located in positions as shown in Figure 2-3 (a). The order of derivation is A  B  C  D  E  F  G. The remaining derivation process for TMM spatial merge candidates is same as in HEVC [3]. If the number of derived TMM spatial merge candidates is less than MaxNumTMMMergeCand, at most one TMM temporal merge candidate is selected from two candidates (Figure 2-3 (b)) that are derived as same as in HEVC. If the number of derived TMM spatial and temporal merge candidates does not reach to MaxNumTMMMergeCand, the combined bi-predictive merge candidates and zero motion vector candidates are added at the end of the list as in HEVC.

(a) Positions of spatial merge candidate    (b) Positions of temporal merge candidate

Figure 2-3: Positions of TMM candidate

At the decoder, the MV is derived from the TMM candidate indicated by the signalled TMM index and is set to an initial MV. In the CU-level motion refinement process, a local search based on the template matching around the initial MV is performed and the MV with the minimum matching cost (i.e., SAD) is taken as the MV for the whole CU. The template matching method applied in TMM mode is the same template matching method of PMMVD [2] in JEM7.0. The search range of motion refinement for the CU-level is set equal to 8 luma samples. Also, it should be noted that, in TMM mode, a CU-level motion search is only performed without additional sub-CU level motion refinement.

Similarly, template matching is also applied to AMVP mode if there is an AMVP candidate derived from spatial or temporal neighboring block.

Inverse quantization

Inverse quantization process is the same as in JEM7.0.

Inverse transforms

As a secondary transform, mode-dependent non-separable secondary transform (MDNSST) [2] in JEM7.0 is applied to the intra predicted CUs in this proposal.

As core transform, DST-VII with residual data flipping as shown in Figure 2-4 is applied to the intra- and inter-predicted CUs instead of adaptive multiple transform (AMT) scheme [2] in order to reduce complexity. The DST-VII transform matrices used in this proposal are the same as defined in JEM7.0.

Figure 2-4: Proposed decoding process using DST-VII with residual flipping

A CU-level flag is signalled whether DCT-VII with residual flipping is used or not. When the CU-level flag is false, DCT-II is used. When the CU-level flag is true, DST-VII is used and an index is signalled to indicate which type of residual flipping method is to be used with the DST-VII.

There are four types of residual flipping method as shown in Figure 2-5: No-Flip (no flipping), H-Flip (horizontal flipping), V-Flip (vertical flipping), HV-Flip (horizontal & vertical flipping). Each residual flipping method is implemented as follows:

No-Flip: 𝑝′(𝑥, 𝑦) = 𝑝(𝑥, 𝑦)

H-Flip: 𝑝′(𝑥, 𝑦) = 𝑝(𝑊−1−𝑥, 𝑦)

V-Flip: 𝑝′(𝑥, 𝑦) = 𝑝(𝑥, 𝐻−1−𝑦)

HV-Flip: 𝑝′(𝑥, 𝑦) = 𝑝(𝑊−1−𝑥, 𝐻−1−𝑦)

where 𝑝(𝑥, 𝑦) is residual data, 𝑝′(𝑥, 𝑦) is residual-flipped data, and 𝑊 and 𝐻 are the width and height of the CU, respectively. Since inverse residual flipping process is exactly same as forward residual flipping process, residual flipping method performed at the encoder is performed at the decoder in the same way.

Figure 2-5: Four types of residual flipping

For intra residual data, due to the different residual statistics of different intra prediction modes, a mode-dependent residual flipping candidate selection process is used similar to AMT in JEM7.0. The four residual flipping candidate sets are pre-defined for intra prediction mode as shown in Table 2-1. One of the residual flipping candidates in the pre-defined residual flipping candidate set for the given intra mode is selected based on the signalled index. For inter residual data, one of four residual flipping methods is selected based on the signalled index.

Table 2-1: Four pre-defined residual flipping candidate sets

| Intra Mode | Residual Flipping Candidates |
| --- | --- |
| Diagonal modes | No-Flip |
| Horizontal direction modes | No-Flip, V-Flip |
| Vertical direction modes | No-Flip, H-Flip |
| Others | No-Flip, H-Flip, V-Flip, HV-Flip |

Also, same as in JEM7.0, large block transforms, up to 128 × 128 in size, are enabled and high frequency transform coefficients are zeroed out for the transform blocks with size (width or height, or both width and height) larger than or equal to 64. In addition, like AMT in JEM7.0, DST-VII with residual flipping applies to the CUs with both width and height smaller than or equal to 64.

Due to DST-VII with residual flipping and MDNSST in the intra predicted CUs, energy tends to converge to low frequency domain. So, in this proposal, up-right diagonal scan order is only used as a scanning method for transform coefficients of the intra predicted CUs without considering intra prediction mode and block size.

Motion compensation

Basically, it is the same as in JEM7.0 except a few aspects as described below.

- When a CU is coded with MRM, both OBMC [2] and LIC [2] are not applied to the CU.

- When a CU is coded with TMM mode, OBMC is not applied to the CU and LIC flag is signalled for the CU to indicate whether LIC applies or not.

- Sub-CU block size for ATMVP and STMVP are configured as 8×8 block instead of 4×4 block defined in JEM7.0.

Intra prediction

Combined Filter (CF)

In intra prediction process of JEM7.0, there is a filtering process called mode-dependent intra smoothing (MDIS) in which, for luma component, reference samples are filtered before performing intra prediction when NSST index is not equal to zero. This reference sample filtering process is controlled by a given intra prediction mode and block size. After applying MDIS, 4-tap interpolation filter for fractional position calculation is applied to the filtered reference samples to generate predicted samples.

In combined filter (CF), two filters which are 3-tap smoothing filter defined in MDIS and 4-tap interpolation filters are combined and represented as 6-tap interpolation filter. So, in the proposed method, only 6-tap interpolation filter is applied to the reference samples for angular intra prediction mode without additional filtering process. Like 4-tap interpolation filter defined in JEM7.0, there are two sets of 6-tap interpolation filter are defined as described in Table 2-2.

Table 2-2: Combined filter (1/32-pel precision)

| Filter A [4] (width <= 8) | Filter B (width > 8) |
| --- | --- |
| { 0, 0, 256, 0, 0, 0 }, { 0, -4, 253, 9, -2, 0 }, { 1, -7, 249, 17, -4, 0 }, { 1, -10, 245, 25, -6, 1 }, { 1, -13, 241, 34, -8, 1 }, { 2, -16, 235, 44, -10, 1 }, { 2, -18, 229, 53, -12, 2 }, { 2, -20, 223, 63, -14, 2 }, { 2, -22, 217, 72, -15, 2 }, { 3, -23, 209, 82, -17, 2 }, { 3, -24, 202, 92, -19, 2 }, { 3, -25, 194, 101, -20, 3 }, { 3, -25, 185, 111, -21, 3 }, { 3, -26, 178, 121, -23, 3 }, { 3, -25, 168, 131, -24, 3 }, { 3, -25, 159, 141, -25, 3 }, { 3, -25, 150, 150, -25, 3 } | { 47, 255, 416, 256, 49, 1 } { 43, 247, 416, 264, 53, 1 } { 40, 240, 414, 270, 58, 2 } { 37, 233, 413, 277, 62, 2 } { 34, 226, 412, 284, 66, 2 } { 31, 218, 410, 292, 71, 2 } { 28, 210, 407, 299, 77, 3 } { 26, 203, 404, 306, 82, 3 } { 23, 195, 401, 313, 88, 4 } { 21, 188, 398, 320, 93, 4 } { 19, 180, 393, 327, 100, 5 } { 17, 173, 389, 333, 106, 6 } { 16, 167, 385, 339, 111, 6 } { 14, 159, 380, 346, 118, 7 } { 13, 153, 375, 351, 124, 8 } { 11, 145, 370, 358, 131, 9 } { 10, 138, 364, 364, 138, 10 } |

Multi-line based Intra Prediction (MIP)

In intra prediction mode except LM mode of JEM7.0 and HEVC, only the nearest reference sample line to the CU is used for intra prediction. However, increasing the number of reference lines may provide better prediction accuracy. For this reason, multi-line based intra prediction (MIP) is proposed.

MIP uses the weighted combination of two reference lines, the nearest reference line and the second nearest reference line to the CU, for intra prediction. Weighting factor 3/4 is used for the nearest reference line and weighting factor 1/4 is used for the second nearest reference line as shown in Figure 2-6.

Figure 2-6: Multi-line based intra prediction

The MIP is applied when intra prediction mode is horizontal, vertical or angular mode. For horizontal and vertical mode, prediction value is calculated as follows:

Vertical mode:    pred[x][y] = (3×r1[x][-1] + r2[x][-2] + 2) >> 2,

Horizontal mode:  pred[x][y] = (3×r1[-1][y] + r2[-2][y] + 2) >> 2

where pred[x][y] is the prediction value of each pixel, in where x and y are the row and column location index, respectively, r1 is a sample value in the nearest reference line and r2 is a sample value in the second nearest reference line.

For angular mode where interpolation filter applies, CF filter is applied to the nearest reference line and bi-linear interpolation filter defined in HEVC is applied to the second nearest reference line. Then, prediction value is calculated with the interpolated sample values as follows:

pred[x][y] = (3×r1[6-tap] + r2[2-tap] + 2) >> 2

where r1[6-tap] is a 6-tap interpolated sample value in the nearest reference line and r2[2-tap] is a bi-linear interpolated sample value in the second nearest reference line as shown in Figure 2-6.

In-loop filtering

Deblocking filter and SAO are the same as in JEM7.0.

The proposed ALF called reduced complexity-ALF (RC-ALF) method is basically same to JEM7.0 while block classification method and filter shapes are modified as follows.

Reduced complexity ALF (RC-ALF)

4×4 level block classification based on subsampled sum-modified-Laplacian (SSML)

In the proposed method, 4×4 level luma block classification is performed instead of 2×2 block classification as in JEM7.0, so, ALF coefficients may switch for each 4×4 luma block.

To classify each 4×4 block where filtering operation is performed, 4×4 block classification using subsampled sum-modified-Laplacian (SSML) is used. Each 4×4 block is categorized into one out of 25 classes. The classification index C is derived based on its directionality, D, and a quantized value of activity, A, as follows:

| C=5D+A. |  |
| --- | --- |

To calculate D and A, gradients of the horizontal, vertical and two diagonal directions are calculated using 1-D SSML:

gv=k=-12l=-25Vi+2k,j+l,  Vp,q=2Rp,q-Rp,q-1-Rp,q+1,

gh=k=-25l=-12Hi+k,j+2l,  Hp,q=2Rp,q-Rp-1,q-Rp+1,q,

gd1=k=-12l=-12(D1i+2k,j+2l+D1i+2k+1,j+2l+1), D1p,q=2Rp,q-Rp-1,q-1-Rp+1,q+1,

gd2=k=-12j=-12(D2i+2k+1,j+2l+D2i+2k,j+2l+1),D2p,q=2Rp,q-Rp-1,q+1-Rp+1,q-1,

where indices i and j refer to the coordinates of the upper left sample in the 4×4 block and R(i,j) indicates a reconstructed sample at coordinate (i,j).

Figure 2-7 (a), (b), (c), and (d) show positions of calculating 1-D Laplacian, Vp,q, Hp,q, D1p,q and D2p,q for gradients, gv, gh, gd1 and gd2, respectively, for a 4×4 block. Since 1-D Laplacian values, Vp,q, Hp,q, D1p,q and D2p,q are calculated at the subsampled positions within 8×8 window, it can be claimed that the computational complexity in calculating gradients is reduced. The rest of derivation process for D and A with the calculated gradients, gv, gh, gd1 and gd2, is the same as in JEM7.0.

(a) Calculation of vertical gradient, gv      (b) Calculation of horizontal gradient, gh

(c) Calculation of diagonal gradients, gd1       (d) Calculation of diagonal gradients, gd2

Figure 2-7: Calculation of gradients

Multiple filter shapes with max 5×5 filter size

Instead of 9×9, 7×7 and 5×5 diamond filter shapes as in JEM7.0, four types of 5×5 filter shapes, which are 5×5 square filter, 5×5 octagon filter, 5×5 snowflake filter, and 5×5 diamond filter, as shown in Figure 2-8 are used for a luma component. For chroma component in the picture, the 5×5 diamond filter shape is always used as in JEM7.0.

Figure 2-8: RC-ALF filter shapes

An index is signalled at the picture level to indicate the filter shape used for the luma component. At the decoder, the index is decoded to select one used for the luma component among four types of 5×5 filter shapes. For chroma components, 5×5 diamond filter shape is always used as same as in JEM7.0. The other processes and signalling method for filter coefficients in RC-ALF of this proposal are the same as in JEM7.0.

Encoder algorithm description

General algorithm description

Rate-distortion optimization

Distortion metrics

Distortion metrics are the same as in JEM7.0.

Rate-distortion dependency models

Rate-distortion dependency models are the same as in JEM7.0.

Partitioning structure selection

CTU size is configured to 256 instead of 128 as shown in Table 3-1. Except this aspect, it is the same as in JEM7.0.

Table 3-1: QTBT parameter settings of the proposed codec

| QTBT parameter | I slice | P/B slice |
| --- | --- | --- |
| CTU size | 256×256 | 256×256 |
| minQTSizeLuma | 8×8 | 8×8 (init) |
| minQTSizeChroma | 4×4 | n/a |
| maxBTSizeLuma | 32×32 | 128×128 (init) |
| maxBTSizeChroma | 64×64 | n/a |
| maxBTDepthLuma | 3 | 3 |
| maxBTDepthChroma | 3 | n/a |
| minBTSize | 4 | 4 |

Intra prediction mode estimation and type selection

In JEM7.0, RD cost is calculated for 3 intra prediction modes derived from rough mode decision and 2 or 3 modes in MPM list. However, unlike JEM7.0, in this proposal, 6 intra prediction modes in the MPM list are candidates for RD cost calculation.

The other process of the intra prediction is the same as in JEM7.0.

Motion estimation and motion segment selection

General

Motion estimation and motion segmentation selection are the same as in JEM7.0.

Usage of decoder-side motion refinement

Motion-Refined Mode (MRM)

MRM is performed in the same way at the encoder and the decoder. Therefore, this section refers to section 2.1.4.1 in the decoder algorithm description.

Template Matched Merge (TMM)

As described in section 2.1.4.2, TMM candidate list is first constructed for TMM mode. At the encoder side, the CU-level motion refinement process with the template matching is performed around MV derived from each TMM candidate in the TMM candidate list. TMM candidates are checked for a CU by using RD cost selection. The one leading to minimum RD cost is selected as the best TMM candidate and signals the index indicating the best TMM candidate.

Mode decisions

Mode decision process is based on rate distortion optimization (RDO) process defined in JEM7.0. For encoding a CU, MRM and TMM mode are checked in mode decision process.

In-loop filtering type selection and parameter estimation

Deblocking filter and SAO are the same as in JEM7.0.

At the encoder, filter type selection and parameter estimation in the RC-ALF are basically the same as in JEM7.0 while block classification method and filter shapes are modified as follows:

Reduced complexity ALF (RC-ALF)

4×4 level block classification based on subsampled sum-modified-Laplacian (SSML)

To derive filter coefficients based on wiener filter at the encoder, 4×4 block classification using SSML is used instead of 2×2 block classification as in JEM7.0. The detailed description of 4×4 block classification using SSML is the same as described in section 2.1.9. Therefore, when filtering process is applied, ALF coefficients may switch for each 4×4 luma block.

Multiple filter shapes with max 5×5 filter size

As described in section 2.1.9, four types of 5×5 filter shapes, which are 5×5 square filter, 5×5 octagon filter, 5×5 snowflake filter, and 5×5 diamond filter, are used for a luma component. However, the filter type selection process for selecting one of four types of 5×5 filter size is the same as in JEM7.0.

Transforms and transform type selection

As a core transform, DST-VII with the four types of residual flipping methods as shown in Figure 3-1 is used instead of AMT in JEM7.0, and, as a secondary transform, MDNSST in JEM7.0 is used.

Figure 3-1: Proposed encoding process using DST-VII with residual flipping

When intra prediction is performed for a CU, the prediction errors usually increase as the distance from the reference sample increases. Also, when inter prediction is performed for a CU and a MV of the CU is different from a neighboring CU, the prediction errors are usually higher near the CU boundary than in the middle of the CU. On the other hand, the N-point DST-VII is efficient for the residual data distribution having the shape of the v0 basis vector (low frequency component) as shown in Figure 3-2. So, in this proposal, it is asserted that, for the CU having large prediction error in the CU boundary, coding efficiency can be improved if DST-VII is applied after residual data is flipped to be suitable for DST-VII.

Figure 3-2: Basis vectors of DST-VII for input samples

Figure 3-3 shows a practical example of the proposed core transform method. For residual data of Figure 3-3 (a), the DCT-II is selected since they are uniformly distributed in a relative sense compared with the residual data in Figure 3-3 (b), (c), (d) and (e). The residual data of Figure 3-3 (b) are concentrated at the bottom-right of the CU and the residual data at the top-left and bottom-left are relatively small, therefore, the DST-VII is applied without a residual flipping (No-Flip). However, the residual data of Figures 3-3 (c), (d), and (e) are concentrated at the bottom-left, top-right and top-left sides of the CU, respectively. In order to the residual data energy to the low frequency basis components of DST-VII as shown in Figure 3-3, the residual flipping to shift the residual data of Figures 3-3 (c), (d), and (e) into the bottom-right side of the CU are carried out; H-Flip is applied to the residual data of Figure 3-3 (c), V-Flip is applied to the residual data of Figure 3-3 (d), and HV-Flip is applied to the residual data of Figure 3-3 (e).

Figure 3-3: Transform with residual flipping method for four types of residual data in (b), (c), (d) and (e)

In this proposal, the method of selecting the optimal core transform among DCT-II and DST-VII with four types of residual flipping method is performed in the same manner as AMT of JEM7.0.

In addition, among DST-VII with residual flipping, the optimal transform selection process is as follows. For intra residual data, a mode-dependent residual flipping candidate selection process is used. More specifically, RD cost is calculated by applying DST-VII with each residual flipping candidate method in the pre-defined residual flipping candidate set for a given intra mode as shown Table 2-1. For inter residual data, since four types of residual flipping can be applied, RD cost is calculated by applying DST-VII with each residual flipping method out of four types of flipping methods. Then, encoder selects the residual flipping method corresponding to the lowest RD cost and signals the index to indicate the chosen residual flipping method.

As described in section 2.1.6 of the decoder algorithm description, up-right diagonal scan order is only used as a scanning method for transform coefficients of CUs in the proposal.

Quantization and quantization type selection

Quantization and quantization type selection process are the same as in JEM7.0.

Entropy coding

In this proposal, multi-hypothesis probability estimation is applied without enabling a method changing the probability updating speed [2]. Except this aspect, entropy coding is the same as in JEM7.0.

Compression performance

SDR: Constraint set 1 configuration relative to HM anchor

Class SDR-A (RD-curves)

RD-curves of Class SDR-A using constraint set 1 relative to HM anchor are shown in below.

Class SDR-B (RD-curves)

RD-curves of Class SDR-B using constraint set 1 relative to HM anchor are shown in below.

Overall (BD-rate summary tables)

Table 4-1 shows the summarized performance of the proposed codec relative to HM anchor for constraint set 1. The average BD-rate for all sequences in constraint set 1 is -32.74% compared to HM anchor. The average BD-rates for Class SDR-A is -35.48% and the average BD-rates for Class SDR-B is -30.00%. The best BD-rates are -39.93% and -33.64% for DaylightRoad2 in Class SDR-A and Cactus in Class SDR-B, respectively. The worst BD-rates are -31.22% and -26.94% for ParkRunning3 in Class SDR-A and RitualDance in Class SDR-B.

Table 4-1: Results of BD-rate for constraint set 1

| Class | Sequences | Over HM16.16 |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-rate (Y) | BD-rate (U) | BD-rate (V) |
| SDR-A | FoodMarket4 | -33.10% | -44.05% | -47.47% |
|  | CatRobot1 | -38.53% | -52.88% | -46.41% |
|  | DaylightRoad2 | -39.93% | -57.61% | -41.46% |
|  | ParkRunning3 | -31.22% | -24.70% | -27.30% |
|  | Campfire | -34.63% | -34.10% | -50.71% |
| SDR-B | BQTerrace | -29.96% | -46.12% | -55.26% |
|  | RitualDance | -26.94% | -37.15% | -40.80% |
|  | MarketPlace | -28.94% | -47.53% | -44.92% |
|  | BasketballDrive | -30.51% | -45.74% | -43.49% |
|  | Cactus | -33.64% | -44.91% | -41.09% |
| Average SDR-A |  | -35.48% | -42.67% | -42.67% |
| Average SDR-B |  | -30.00% | -44.29% | -45.11% |
| Average all |  | -32.74% | -43.48% | -43.89% |

SDR: Constraint set 2 configuration relative to HM anchor

Class SDR-B

RD-curves of Class SDR-B using constraint set 2 relative to HM anchor are shown in below.

Overall

Table 4-2 shows the summarized performance of the proposed codec relative to HM anchor for constraint set 2. The average BD-rate for all sequences in constraint set 2 is -23.93% compared to HM anchor. The best BD-rate is -27.58% for Cactus. The worst BD-rate is -20.29% for MarketPlace.

Table 4-2: Results of BD-rate for constraint set 2

| Class | Sequences | Over HM16.16 |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-rate (Y) | BD-rate (U) | BD-rate (V) |
| SDR-B | BQTerrace | -25.30% | -48.15% | -53.55% |
|  | RitualDance | -21.49% | -35.43% | -36.73% |
|  | MarketPlace | -20.29% | -43.18% | -41.64% |
|  | BasketballDrive | -25.00% | -46.68% | -44.64% |
|  | Cactus | -27.58% | -42.57% | -40.14% |
| Average all |  | -23.93% | -43.20% | -43.34% |

SDR: Constraint set 1 configuration relative to JEM anchor

Class SDR-A

RD-curves of Class SDR-A using constraint set 1 relative to JEM anchor are shown in below.

Class SDR-B

RD-curves of Class SDR-B using constraint set 1 relative to JEM anchor are shown in below.

Overall

Table 4-3 shows the summarized performance of the proposed codec relative to JEM anchor for constraint set 1. The average BD-rate for all sequences in constraint set 1 is 0.64% compared to JEM anchor. The average BD-rates for Class SDR-A is 0.34% and the average BD-rates for Class SDR-B is 0.95%. The best BD-rates are -0.43% and 0.42% for Campfire in Class SDR-A and MarketPlace in Class SDR-B, respectively. The worst BD-rates are 1.27% and 2.25% for CatRobot1 in Class SDR-A and Cactus in Class SDR-B.

Table 4-3: Results of BD-rate for constraint set 1

| Class | Sequences | Over JEM7.0 |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-rate (Y) | BD-rate (U) | BD-rate (V) |
| SDR-A | FoodMarket4 | 0.36% | -0.72% | -1.28% |
|  | CatRobot1 | 1.27% | -1.63% | -1.49% |
|  | DaylightRoad2 | 0.62% | -1.08% | -0.38% |
|  | ParkRunning3 | -0.14% | 0.64% | 0.17% |
|  | Campfire | -0.43% | 0.74% | -2.37% |
| SDR-B | BQTerrace | 0.54% | -1.74% | -2.22% |
|  | RitualDance | 0.55% | -0.12% | -0.35% |
|  | MarketPlace | 0.42% | -0.33% | -0.90% |
|  | BasketballDrive | 1.00% | -0.40% | -0.23% |
|  | Cactus | 2.25% | 0.71% | 0.16% |
| Average SDR-A |  | 0.34% | -0.41% | -1.07% |
| Average SDR-B |  | 0.95% | -0.38% | -0.71% |
| Average all |  | 0.64% | -0.39% | -0.89% |

SDR: Constraint set 2 configuration relative to JEM anchor

Class SDR-B

RD-curves of Class SDR-B using constraint set 2 relative to JEM anchor are shown in below.

Overall

Table 4-4 shows the summarized performance of the proposed codec relative to JEM anchor for constraint set 2. The average BD-rate for all sequences in constraint set 2 is 0.82% compared to JEM anchor. The best BD-rate is 0.29% for RitualDance. The worst BD-rate is 1.39% for BQTerrace.

Table 4-4: Results of BD-rate for constraint set 2

| Class | Sequences | Over JEM7.0 |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-rate (Y) | BD-rate (U) | BD-rate (V) |
| SDR-B | BQTerrace | 1.39% | -1.98% | -6.59% |
|  | RitualDance | 0.29% | -1.72% | -1.96% |
|  | MarketPlace | 0.49% | -0.78% | -1.57% |
|  | BasketballDrive | 0.90% | -1.70% | -1.47% |
|  | Cactus | 1.05% | -0.16% | -0.90% |
| Average all |  | 0.82% | -1.27% | -2.50% |

Decoder complexity analysis

Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

The 64-bit executables of the proposed decoder and HM16.16 were generated by Microsoft® Visual Studio® Professional 2013. The decoding times were collected from log files generated by the decoder. The decoding times written to the log files were obtained using the same process as used by HM16.16 software. The I/O times (for output YUV file generation) are included in the reported decoding times.

As shown in Table 5-1 and Table 5-2, the average decoding time of the proposed codec is 4.04 times of HM16.16 decoder for constraint set 1 and 3.31 times of HM16.16 decoder for constraint set 2.

Table 5-1: Results of decoding time measurement for constraint set 1

| Class | Sequences | HM16.16 (sec) | Proposed (sec) | Decoding Time Ratio |
| --- | --- | --- | --- | --- |
| SDR-A | FoodMarket4 | 207.29 | 748.12 | 3.61 |
|  | CatRobot1 | 173.30 | 870.56 | 5.02 |
|  | DaylightRoad2 | 173.78 | 662.19 | 3.81 |
|  | ParkRunning3 | 152.94 | 610.09 | 3.99 |
|  | Campfire | 90.07 | 255.99 | 2.84 |
| SDR-B | BQTerrace | 40.27 | 168.74 | 4.19 |
|  | RitualDance | 44.83 | 172.57 | 3.85 |
|  | MarketPlace | 40.85 | 181.55 | 4.44 |
|  | BasketballDrive | 37.66 | 158.82 | 4.19 |
|  | Cactus | 33.09 | 163.31 | 4.97 |
| Average SDR-A |  | 153.78 | 583.01 | 3.79 |
| Average SDR-B |  | 39.14 | 168.61 | 4.31 |
| Average all |  | 77.59 | 313.72 | 4.04 |

Table 5-2: Results of decoding time measurement for constraint set 2

| Class | Sequences | HM16.16 (sec) | Proposed (sec) | Decoding Time Ratio |
| --- | --- | --- | --- | --- |
| SDR-B | BQTerrace | 39.52 | 97.39 | 2.46 |
|  | RitualDance | 43.82 | 140.47 | 3.21 |
|  | MarketPlace | 39.02 | 137.82 | 3.53 |
|  | BasketballDrive | 36.91 | 127.16 | 3.45 |
|  | Cactus | 30.91 | 127.60 | 4.13 |
| Average all |  | 37.79 | 125.06 | 3.31 |

Description of computing platform used to determine decoding times reported in section 5.1

The configuration of the computing platform is described as below.

| CPU | Intel Xeon E5-2697 v2 (2.60GHz) |
| --- | --- |
| Memory | DDR4 192GB (32.0GB used) |
| Hard Drive | SATA3 Samsung SSD 850 Pro 1TB |
| OS | Windows Server 2008 R2 64bit |
| Parallel processing | No |
| Compiler | Microsoft® Visual Studio® Professional 2013 |
| Executable | 64bit |

Memory usage of decoder

The memory usage of the proposed decoder and the HM/JEM decoder is shown in Table 5-3. It should be noted that the memory usage is obtained by observing the Resident Set Size (RSS) of the decoder process in the Linux system. It is reported that memory usage of the proposed decoder is higher than JEM7.0 because CTU size is configured to 256 instead of 128 in the proposed codec.

Table 5-3: Memory usage of HM, JEM and the proposed decoder

| Constraint Set | Sequences | HM16.16 (GB) | JEM7.0 (GB) | Proposed (GB) |
| --- | --- | --- | --- | --- |
| Constraint Set1 | FoodMarket4 | 0.99 | 1.20 | 1.30 |
|  | CatRobot1 | 0.99 | 1.20 | 1.30 |
|  | DaylightRoad2 | 0.99 | 1.20 | 1.30 |
|  | ParkRunning3 | 0.99 | 1.20 | 1.30 |
|  | Campfire | 0.99 | 1.20 | 1.30 |
| Constraint Set2 | BQTerrace | 0.22 | 0.34 | 0.43 |
|  | RitualDance | 0.22 | 0.34 | 0.43 |
|  | MarketPlace | 0.22 | 0.34 | 0.43 |
|  | BasketballDrive | 0.22 | 0.34 | 0.43 |
|  | Cactus | 0.22 | 0.34 | 0.43 |

Complexity characteristics of decoder entropy decoding operation

Since, as described in section 2.1.2, multi-hypothesis probability estimation is applied without enabling a method changing the probability updating speed, entropy decoding complexity decreases accordingly compared to JEM7.0.

Complexity characteristics of decoder inverse quantization

Since there is no modification or change in the quantization process compared to JEM7.0, the complexity characteristics of decoder inverse quantization is the same as in JEM7.0.

Complexity characteristics of decoder inverse transform operation

While four additional transforms are needed for AMT as core transforms, one additional transform and residual flipping are needed in the proposed method. Memory usage estimation for core transforms in HEVC, JEM7.0, and the proposed method is shown in Table 5-4 and memory usage for additional core transforms, in addition to DCT-II, in JEM7.0 and the proposed method is shown in Table 5-5. The proposed method saves 37.5% memory for transform coefficients storage compared to JEM7.0 and 75% memory for additional transform coefficients storage in addition to DCT-II compared to JEM7.0.

On residual flipping, inverse residual flipping can be implemented by simply reordering the output of inverse DST-VII transform without any additional process.

Table 5-4: Memory usage for HEVC, JEM, and proposed method

| Transform size | Matrix size | HEVC |  |  | JEM7.0 |  |  | Proposed |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | # of kernels | bits per elements | memory | # of kernels | bits per elements | memory | # of kernels | bits per elements | memory |
| 2 | 4 | 0 | 7 | 0 | 1 | 10 | 5 | 1 | 10 | 5 |
| 4 | 16 | 2 | 7 | 28 | 5 | 10 | 100 | 2 | 10 | 40 |
| 8 | 64 | 1 | 7 | 56 | 5 | 10 | 400 | 2 | 10 | 160 |
| 16 | 256 | 1 | 7 | 224 | 5 | 10 | 1600 | 2 | 10 | 640 |
| 32 | 1024 | 1 | 7 | 896 | 5 | 10 | 6400 | 2 | 10 | 2560 |
| 64 | 4096 | 0 | 7 | 0 | 5 | 10 | 25600 | 2 | 10 | 10240 |
| 128 | 16384 | 0 | 7 | 0 | 1 | 10 | 20480 | 1 | 10 | 20480 |
| Total (bytes) |  |  |  | 1204 |  |  | 54585 |  |  | 34125 |

Table 5-5: Memory usage for additional core transform in JEM and proposed method

| Transform size | Matrix size | JEM7.0 |  |  | Proposed |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | # of kernels | bits per elements | memory | # of kernels | bits per elements | memory |
| 2 | 4 | 0 | 10 | 0 | 0 | 10 | 0 |
| 4 | 16 | 4 | 10 | 80 | 1 | 10 | 20 |
| 8 | 64 | 4 | 10 | 320 | 1 | 10 | 80 |
| 16 | 256 | 4 | 10 | 1280 | 1 | 10 | 320 |
| 32 | 1024 | 4 | 10 | 5120 | 1 | 10 | 1280 |
| 64 | 4096 | 4 | 10 | 20480 | 1 | 10 | 5120 |
| 128 | 16384 | 0 | 10 | 0 | 0 | 10 | 0 |
| Total (bytes) |  |  |  | 27280 |  |  | 6820 |

Complexity characteristics of decoder-side motion refinement

Motion Refined Mode (MRM)

In MRM, the first available bi-directional MVs are searched by checking MVs of each MRM candidate among six MRM candidates in the given order. The first available MVs in the given order are chosen to be two initial MVs for bilateral template matching process. Then, bilateral template matching with 1/2-pel MV refinement precision is applied to the derived two initial MVs.

Since, in this proposal, bilateral template matching with 1/2-per MV refinement precision is applied, memory access required for MRM is (W+7+4) × (H+7+4) while bilateral template matching with integer-pel precision needs (W+7+2) × (H+7+2). In terms of computational complexity, for the first round of search, up to 8 MV candidates (1 integer-pel displacement in vertical and horizontal directions) are checked. So, up to 9 calculations for SAD and up to 8 comparisons are needed. For the second round of search that refines MV with 1/2-pel precision, additionally up to 8 calculations for SAD and up to 8 comparisons are needed.

Template Matched Merge (TMM)

In TMM mode, a CU-level refinement is performed around a MV derived from the signalled TMM index. The CU-level refinement process is exactly the same as template matching method defined for PMMVD in JEM7.0. Therefore, in the MV refinement, the MV is directly searched at quarter luma sample MV accuracy, and this is followed by one-eighth luma sample MV refinement. Also, the search range of MV refinement is set equal to 8 luma samples same as template matching in JEM7.0.

Unlike PMMVD which has 13 initial MVs for a CU and 17 initial MVs for a sub-CU in the FRUC merge mode, since TMM mode has one initial MV for the CU-level motion refinement and no sub-CU level motion refinement, memory access for per a (W × H) CU in TMM mode can be constrained as follows. In the motion refinement process of TMM mode, search range of the MV refinement is set equal to 8 luma samples, a template consists of (W × 4) top neighbouring block and (4 × H) left neighboring block of the current CU, and bi-linear interpolation is performed for motion compensated interpolation. Given these considerations, memory access required for motion refinement process of TMM mode is (W+16+4) x (H+16+4). If the motion compensation (MC) process is considered on top of the motion refinement process, memory access required for the TMM mode becomes (W+16+4+4) × (H+16+4+4) as shown in Figure 5-1.

On the other hand, like PMMVD, since the MV refinement process is terminated when SAD cost no longer decreases, number of SAD calculation and additional MC interpolations per a CU for MV refinement process is not constrained. However, it is asserted that computational complexity at decoder is also reduced by performing only CU-level refinement around one initial MV without sub-CU level motion refinement in TMM compared to PMMVD in JEM7.0.

Figure 5-1: Memory access for TMM mode

Complexity characteristics of decoder motion compensation

Complexity characteristics of decoder motion compensation are the same as in JEM7.0.

Complexity characteristics of decoder intra-frame prediction operation

In CF, since two-step operations for filtering are reduced to one by combining the interpolation filter with the reference sample smoothing filter, it can be claimed that computational complexity for filtering process is reduced. In terms of memory usage, while LUT size for 4-tap interpolation filter is 256 bytes, LUT size for proposed CF filter that is 6-tap interpolation filter is 408 bytes.

MIP calculates weighted combination of the nearest reference line and the second nearest reference line and bi-linear interpolation filter is applied to the second nearest reference line. So, compared to using one reference sample line for intra prediction, 2 multiplications, 4 additions and 2 shifts per sample are additionally needed by using two reference lines.

Complexity characteristics of decoder in-loop filtering operation

Since deblocking filter and SAO are the same as in JEM7.0, complexity characteristics for deblocking filter and SAO remain the same as in JEM7.0.

In RC-ALF, the proposed two methods, 4×4 block classification based on SSML and multiple filter shapes with 5×5 filter size, can reduce complexity compared to ALF in JEM7.0 as follows.

In the proposed 4×4 block classification based on SSML, 1-D Laplacian values, Vp,q, Hp,q, D1p,q and D2p,q calculated at the subsampled positions within 8×8 window are summed up for each 4×4 block to derive gradients, gv, gh, gd1 and gd2, respectively. Therefore, 32 × (2 additions + 1 shift) + 31 additions are required to derive each gradient for 4×4 block by applying the proposed block classification method while 36 × (2 additions + 1 shift) + 31 additions are required to derive each gradient for a 2×2 block by applying block classification method in JEM7.0, which is the 2×2 block classification based on SML. If the computational complexity for a given block size (e.g., 128×128 block) are compared, the proposed block classification method reduces approximately 77% of additions and shifts compared to block classification method in JEM7.0.

Instead of 9×9, 7×7, and 5×5 diamond filter shapes as in JEM7.0, the four types of 5×5 filter shapes are used for a luma component in RC-ALF. When performing filtering on reconstructed samples, the maximum number of multiplications per pixel is 21 for ALF in JEM7.0 and the proposed method can reduce the maximum number of multiplications per pixel to 13. Also, since ALF coefficients of reference picture and ALF coefficients for a set of 16 fixed filters are stored to be reused as ALF coefficients of a current picture, it is asserted that the proposed method whose maximum number of filter coefficients is 13 instead of 21 can reduce memory usage for storing ALF coefficients.

In addition, RC-ALF reduces the line buffer size, which requires eight in ALF of JEM7.0, to seven as shown in Figure 5-2.

Figure 5-2: Required line buffer size in RC-ALF

Degree of capability for decoder parallel processing

Degree of capability for decoder parallel processing is the same as JEM7.0.

Encoder complexity analysis

Encoding time and measurement methodology

The 64-bit executables of the proposed decoder and HM16.16 were generated by GCC4.7.2 compiler. The encoding times were collected from log files generated by the encoder. The encoding times written to the log files were obtained using the same process as used by HM16.16 software. It should be noted that encoding log files are parsed for each IRAP segment to measure the encoding time of the proposed encoder. Total encoding time is calculated by adding the encoding time of all IRAP segments and then subtracting the encoding times of the first picture for all IRAP segments except the first.

As shown in Table 6-1, the average encoding time of the proposed codec is 8.41 times of HM16.16 encoder for constraint set 1 and 8.18 times of HM16.16 for constraint set 2.

Table 6-1: Results of encoding time measurement for constraint set 1

| Class | Sequences | HM16.16 (sec) | Proposed (sec) | Encoding Time Ratio |
| --- | --- | --- | --- | --- |
| SDR-A | FoodMarket4 | 67655.49 | 399683.06 | 5.91 |
|  | CatRobot1 | 54093.07 | 402327.83 | 7.44 |
|  | DaylightRoad2 | 56095.67 | 428892.59 | 7.65 |
|  | ParkRunning3 | 52027.51 | 453545.57 | 8.72 |
|  | Campfire | 31025.93 | 419707.69 | 13.53 |
| SDR-B | BQTerrace | 11451.15 | 59768.39 | 5.22 |
|  | RitualDance | 17990.27 | 215833.21 | 12.00 |
|  | MarketPlace | 14524.94 | 111094.59 | 7.65 |
|  | BasketballDrive | 13871.57 | 150642.15 | 10.86 |
|  | Cactus | 10822.70 | 92446.24 | 8.54 |
| Average SDR-A |  | 50590.20 | 420380.39 | 8.31 |
| Average SDR-B |  | 13504.93 | 114821.63 | 8.50 |
| Average all |  | 26138.42 | 219701.53 | 8.41 |

Table 6-2: Results of encoding time measurement for constraint set 2

| Class | Sequences | HM16.16 (sec) | Proposed (sec) | Encoding Time Ratio |
| --- | --- | --- | --- | --- |
| SDR-B | BQTerrace | 14352.88 | 63718.52 | 4.44 |
|  | RitualDance | 18062.51 | 218995.52 | 12.12 |
|  | MarketPlace | 15757.86 | 115631.76 | 7.34 |
|  | BasketballDrive | 14702.86 | 159263.91 | 10.83 |
|  | Cactus | 12710.42 | 108517.93 | 8.54 |
| Average all |  | 15015.98 | 122766.36 | 8.18 |

Description of computing platform used to determine encoding times reported in section 6.1

The encoding was executed on a homogeneous computer cluster with computing platform described as below.

| Specification of homogeneous computer cluster |  |
| --- | --- |
| CPU | Intel Xeon E5-1620 v2 (3.7GHz) |
| Memory | DDR3 8GB x 4 (32GB) |
| Hard Drive | Seagate 4TB SATA3 7200rpm |
| OS | CentOS release 6.7 |
| Compiler | GCC 4.7.2 |
| Executable | 64bit |

Memory usage of encoder

The memory usage of the proposed encoder and the HM/JEM encoder is shown in the table below. It should be noted that the memory usage is obtained by observing the Resident Set Size (RSS) of the encoder process in the Linux system. It is reported that memory usage of the proposed encoder is higher than JEM7.0 because CTU size is configured to 256 instead of 128 in the proposed codec

Table 6-3: Memory usage of HM, JEM and the proposed encoder

| Constraint Set | Sequences | HM16.16 (GB) | JEM7.0 (GB) | Proposed (GB) |
| --- | --- | --- | --- | --- |
| Constraint Set1 | FoodMarket4 | 1.90 | 6.00 | 6.80 |
|  | CatRobot1 | 1.90 | 6.00 | 6.80 |
|  | DaylightRoad2 | 1.90 | 6.00 | 6.80 |
|  | ParkRunning3 | 1.90 | 6.00 | 6.80 |
|  | Campfire | 1.90 | 6.00 | 6.80 |
| Constraint Set2 | BQTerrace | 0.24 | 0.90 | 1.40 |
|  | RitualDance | 0.24 | 0.90 | 1.40 |
|  | MarketPlace | 0.24 | 0.90 | 1.40 |
|  | BasketballDrive | 0.24 | 0.90 | 1.40 |
|  | Cactus | 0.24 | 0.90 | 1.40 |

Complexity characteristics of encoder intra-frame prediction type selection

Complexity characteristics for decoder described in section 5.9, can be applied to this section. In addition, as described in section 3.1.3, three additional calculation of RD cost are required.

Complexity characteristics of encoder motion estimation and motion segmentation selection

General

Complexity characteristics of encoder motion estimation and motion segmentation selection are the same as in JEM7.0 except applying MRM and TMM mode.

Complexity characteristics of usage of decoder-side motion refinement

Motion Refined Mode (MRM)

MRM is performed in the same way at the encoder and the decoder. Therefore, complexity characteristics at encoder are the same as complexity characteristics at decoder described in section 5.7.

Template Matched Merge (TMM)

As described in the section 3.1.4.2.2, in order to select best TMM candidate in the candidate list, the CU-level motion refinement process with the template matching is performed around MV derived from each TMM candidate in the TMM candidate list and the RD cost for each TMM candidate is calculated. Then, encoder selects the best TMM candidate with the refined MV corresponding to the lowest RD cost. Since the MV refinement process is performed for the candidates in the candidate list while it is performed for candidate indicated by the signalled TMM index, it can be claimed that encoder complexity is higher than the decoder complexity.

Complexity characteristics of encoder transforms and transform type selection

Basically, the method of selecting the optimal core transform among DCT-II and DST-VII with residual flipping is performed in the same manner as AMT of JEM7.0. However, for intra prediction, while, in AMT, there are four types of combinations for each intra prediction mode to calculate RD cost, in DST-VII with residual flipping method, there are up to four types of flipping candidates in the pre-defined residual flipping candidate set for each intra prediction mode. Therefore, it is claimed that the average computational complexity of the proposed method could be reduced.

The memory usage for the proposed method at the encoder is the same as the memory usage at the decoder in section 5.6. In addition, it is claimed that residual flipping could be implemented by simply reordering the input of DST-VII transform without any additional process.

Complexity characteristics of encoder quantization and quantization type selection

The complexity characteristics of encoder quantization and quantization type selection are the same as in JEM7.0 since there is no modification in quantization and quantization type selection processes.

Complexity characteristics of encoder in-loop filtering type selection and parameter estimation

Since deblocking filter and SAO are the same as in JEM7.0, complexity characteristics for deblocking filter and SAO remain the same as in JEM7.0.

Complexity characteristics of RC-ALF described in section 5.10 is also applied to this section.

In addition, computational complexity of ALF coefficients derivation process for a set of 16 fixed filters should be considered. The fixed filter for ALF in JEM7.0 has up to 21 filter coefficients and the fixed filter for RC-ALF has up to 13 filter coefficients. Therefore, it is claimed that the RC-ALF also reduces computational complexity of ALF coefficients derivation process for a set of 16 fixed filters.

Complexity characteristics of encoder entropy coding type selection

Since, as described in section 3.1.9, multi-hypothesis probability estimation is applied without enabling a method changing the probability updating speed, entropy encoding complexity decreases accordingly compared to JEM7.0.

Degree of capability for encoder parallel processing

Degree of capability for encoder parallel processing is the same as JEM7.0.

Algorithmic characteristics

Random access characteristics

Random access characteristics are the same as in JEM7.0.

Delay characteristics

Delay characteristics are the same as in JEM7.0.

Software implementation description

The proposal is based on the JEM software (version 7.0) [5]. All the source codes are written in C++ programming language. The following JEM tool specific encoder parameters were set for all simulations.

- SelectiveRDOQ		=  true

- LCTUFast		=  false

- NUM_MRG_SATD_CAND = 4

Highlighted aspects discussion

For inter prediction, two special merge modes called MRM and TMM mode are newly introduced. MRM is based on decoder-side motion refinement method using bilateral template matching in JEM 7.0 and priority-based candidate search to increase coding efficiency by minimize signalling syntax. TMM mode is based on CU-level decoder-side motion refinement around an initial MV derived the signalled TMM index to reduce decoder complexity while minimizing coding efficiency degradation compared to PMMVD in JEM7.0. It is reported that MRM provides BD-rate saving of 3.46% in “tool on” and 0.45% in “tool off” test, respectively, and TMM provides BD-rate savings of 5.42% in “tool on” and 2.58% in “tool off” test, respectively.

For intra prediction, 6-tap interpolation filter called CF that combines 4-tap interpolation filter with 3-tap reference sample smoothing filter is proposed to reduce the number of operations for angular intra prediction mode. In addition, the weighted combination of two reference lines for angular intra prediction called MIP is proposed to increase the performance of intra coding. It is reported that CF provides BD-rate saving of 0.53% in “tool on” and 0.16% in “tool off” test, respectively and MIP provides BD-rate saving of 0.15% in “tool on” and 0.07% in “tool off” test, respectively.

For core transform, DST-VII with residual flipping is introduced for the intra- and inter-prediction instead of AMT scheme in order to reduce the number of transform matrix and memory usage compared to AMT while minimizing coding efficiency degradation compared to AMT in JEM7.0. It is reported that DST-VII with residual flipping saves 75% memory for additional transform coefficients storage in addition to DCT-II compared to JEM 7.0. In coding efficiency, it is reported that DST-VII with residual flipping provides BD-rate saving of 2.86% in “tool on” and 1.94% in “tool off” test, respectively.

For ALF, RC-ALF which modifies block classification method and filter shapes is proposed to reduce complexity compared to ALF in JEM7.0. In RC-ALF, 4×4 block classification based on subsampled sum-modified-Laplacian is used and four types of 5×5 filter shapes, which are 5×5 square, octagon, snowflake, and diamond filter shapes, are employed instead of 9×9, 7×7, and 5×5 diamond filter shapes. It is reported that the RC-ALF reduces computational complexity and line buffers compared to ALF in JEM7.0. In coding efficiency, it is reported that RC-ALF provides BD-rate savings of 3.43% in “tool on” and 2.43% in “tool off” test, respectively.

Closing remarks

ETRI and Sejong University’s SDR video coding technology proposal in response to the call for proposal issued jointly by MPEG and ITU-T has been described. The coding efficiency of the proposed codec is summarized as follows. For constraint set 1, average BD-rates are -32.74% compared to HM anchor and 0.64% compared to JEM anchor. For constraint set 2, the average BD-rates are -23.93% compared to HM anchor and 0.82% compared to JEM anchor. The average decoding time of the proposed codec is 4.04 times and 3.31 times of HM16.16 decoder for constraint set 1 and for constraint set 2, respectively. The average encoding time of the proposed codec is 8.41 times and 8.18 times of HM16.16 encoder for constraint set 1 and for constraint set 2, respectively. From the analysis of results with the proposed codec, it is recommended to investigate techniques related to decoder side motion refinement process, core transform, in-loop filtering process, and intra prediction process through tool experiments.

References

- “Joint Call for Proposals on Video Compression with Capability beyond HEVC”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-H1002, Oct. 2012.

- “Algorithm Description of Joint Exploration Test Model 7 (JEM7)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-G1001, Jul. 2017.

- High Efficiency Video Coding (HEVC), Rec. ITU-T H.265 and ISO/IEC 23008-2, Jan. 2013.

- X. Zhao, V. Seregin, and M. Karczewicz, “Six tap intra interpolation filter”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-D0119, Oct. 2016.

- JEM reference software JEM7.0, https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-7.0/

Patent rights declaration(s)

Electronics and Telecommunications Research Institute (ETRI) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sejong University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Annex: Questionnaire

Codec level questions

| Codec Level Q&A |  | HM | JEM | Proposed |
| --- | --- | --- | --- | --- |
| 1 | Maximum number of reference frames used by encoder | 4 | 4 | Same as JEM7.0 |
| 2 | Maximum coding unit (e.g. CTU in HEVC) | luma 64×64 chroma 32×32 | luma 128×128 chroma 64×64 | luma 256×256 chroma 128×128 |
| 3 | Minimum and maximum transform block size | luma 4×4/32×32 chroma 4×4/16×16 | luma 4×4/128×128 chroma 2×2/64×64 | Same as JEM7.0 |
| 4 | Minimum and maximum intra prediction block | luma 4×4 chroma 4×4 | luma 4×4 chroma 2×2 | Same as JEM7.0 |
| 5 | Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length | luma 4×8/8×4 (uni-pred) and 8×8 (bi-pred), 8-tap Chroma 2×4/4×2 (uni-pred) and 4×4 bi-pred, 4-tap | luma 4×4 (uni/bi-pred), 8-tap Chroma 2×2 (uni/bi-pred), 4-tap | Same as JEM7.0 |
| 6 | List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category) | n/a | template matching in FRUC, LIC | TMM, LIC |
| 7 | List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples, position unforeseeable) | n/a | 64-bit arithmetic for LIC, BIO and LMC | Same as JEM7.0 |
| 8 | List all the tools that use non-rectangular (e.g. triangle) partitions | n/a | n/a | n/a |
| 9 | Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa) | n/a | n/a | n/a |
| 10 | Indicate whether luma and chroma use separate partitioning structures | no | Yes, in I-slices | Same as JEM7.0 |
| 11 | Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes) | Entropy parsing depends on intra prediction mode reconstruction, which requires an intra MPM list derivation | Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation | Same as JEM7.0 |

Tool level questions

- Partitioning structure

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1. Partitioning structure |  | HM | JEM | Proposed |
| 1.a | Whether the prediction unit partitioning is different from transform unit partitioning | Yes, PU and TU partitioning for a CU can differ | No | No |
| 1.b | Maximum tree depth | CU maximum depth 4 TU maximum depth 3 PU maximum depth 1 | QT maximum depth 5 BT maximum depth 3 | Same as JEM7.0 |

- Entropy coding

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 2. Entropy coding |  | HM | JEM | Proposed |
| 2.a | Total number of context models | 186 | 329 | 337 |
| 2.b | Memory storage size for each context model | 8-bit | 30-bit | Same as JEM7.0 |
| 2.c | Range table size if table look up is used for range update | LPSTable[64×4×8bits]; RenormTable[32× 3bits] | LPSTable[512×64×9bits]; RenormTable[128×3bits] | Same as JEM7.0 |
| 2.d | Initialization table size | 3×186×8 bits | 3×329×8 + 329×15×5 bits | 3×337×8 + 337×15×5 bits |
| 2.e | Other memory associated with context model adaptation | LPS transition table: 64*6 bits MPS transition table: 64*6 bits | n/a | Same as JEM7.0 |

- Control data derivation process

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 3. Control data derivation process |  | HM | JEM | Proposed |
| 3.a | Temporal motion data storage granularity | 16×16 | 4×4 | Same as JEM7.0 |
| 3.b | Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-directional reference picture are used, count this picture as 2) | 1 | 4 | Same as JEM7.0 |

- Decoder-side motion refinement

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 4. Decoder-side motion refinement |  | HM | JEM | Proposed |
| 4.a | Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement. | n/a | - BIO 2×2D separable 6 taps filter (for gradients), mults per sample in refinement calculation 5+5*4 +1/8+2, minimum processing unit same as regular bi-pred, no SAD calculation - FRUC minimum processing unit 44, sequential PU followed by sub-PU level refinement, number of SAD calculations and additional MC interpolations per unit for MV refinement is not constrained (refinement is terminated when cost no longer decreases); bi-linear filter at MV refinement stage - DMVR: minimum processing unit 4x4. 9 SAD calculations, 8 comparisons | - BIO: same as BIO in JEM7.0 - TMM: minimum processing unit 44, number of SAD calculations and additional MC interpolations per unit for MV refinement is not constrained (refinement is terminated when cost no longer decreases); bi-linear filter at MV refinement stage (same as FRUC in JEM7.0) - MRM: minimum processing unit 4x4. 17 SAD calculations, 16 comparisons |
| 4.b | Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size. | n/a | - DMVR needs to access (H+7+2)×(W+7+2) MC instead of (H+7)×(W+7) in HEVC MC - FRUC not constrained | - TMM mode: (H+7+17 )×(W+7+17) - MRM: (H+7+4) ×(W+7+4) |

- Intra prediction

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 5. Intra prediction |  | HM | JEM | Proposed |
| 5.a | Line buffer size in terms of luma and chroma sample lines | luma 1, chroma 1 | luma 4, chroma 2 | Same as JEM7.0 |
| 5.b | List all the filters used for intra prediction | Reference sample smoothing: {1,2,1}, Strong intra smoothing filter: 2-tap bilinear filter (only for 32×32 luma TU) Interpolation: 2×32×5 bits (bi-linear 32 phases) | Reference sample smoothing: {1,2,1}, {3,7,14,16,14,7,3}, {1,6,15,20,15,6,1} Interpolation: 2×4×32×8 bits (4 taps, 2 sets, 32 phase) | Reference sample smoothing for P-PDPC: {1,2,1}, {3,7,14,16,14,7,3}, {1,6,15,20,15,6,1} Interpolation: 6×32×(8 bits + 9bits) (6 taps, 2 sets, 32 phase) 2×32×5 bits (bi-linear 32 phases) |
| 5.c | Indicate whether there are cross-component dependencies in intra prediction reconstruction | n/a | LM-CHROMA (Cb depends on Y, and Cr depends on Cb&Y) | Same as JEM7.0 |

- Inter prediction

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 6. Inter prediction |  | HM | JEM | Proposed |
| 6.a | Motion vector accuracy | luma ¼ pel chroma 1/8 pel | luma 1/16 pel chroma 1/32 pel | Same as JEM7.0 |
| 6.b | Storage size for a motion vector component | 16-bit | 16-bit | Same as JEM7.0 |
| 6.c | List all the interpolation filters used for luma/chroma motion compensation | luma 8-tap, 8×4×6 bits chroma 4-tap, 4×8×6 bits | luma 8-tap, 8×16×6 bits chroma 4-tap, 4×32×6 bits | Same as JEM7.0 |

- Quantization

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 7. Quantization |  | HM | JEM | Proposed |
| 7.a | Storage size associated with quantization | n/a | n/a | n/a |

- Transforms

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 8. Transforms |  | HM | JEM | Proposed |
| 8.a | List all the combinations of transform block size and transform type for primary transforms | - 8-bit 4×4, 8×8, 16×16 and 32×32 DCT-II, - 8-bit 4×4 DST VII | HEVC transform mode (10-bit DCT-II), transform size combinations: - (4, 8, 16, 32, 64, 128) × (4, 8, 16, 32, 64, 128) TUs for luma CUs - (2, 4, 8, 16, 32, 64) × (2, 4, 8, 16, 32, 64) TUs for inter-coded chroma CTUs - (2, 4, 8, 16, 32, 64,) × (2, 4, 8, 16, 32, 64) TUs for intra-coded chroma CUs EMT transform mode (10-bit DST VII, DCT-V, DCT-VIII, DST-I), only applied to CUs up to 64×64, transform size combinations: - (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TUs for luma CUs | HEVC transform mode (10-bit DCT-II), transform size combinations: same as JEM7.0 DST-VII transform mode (10-bit), only applied to CUs up to 64×64, transform size combinations: (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TUs for luma CUs |
| 8.b | List all the combinations of transform block size and transform type for secondary transforms | n/a | 35×3 4×4 and 8×8 non-separable Hypercube-Givens Transform (HyGT) | Same as JEM7.0 |
| 8.c | List all the non-separable transforms used | n/a | 4×4/8×8 HyGT | Same as JEM7.0 |
| 8.d | Indicate whether transforms can be fully/partially implemented using direct matrix multiply | partial factorization equivalent to matrix multiplication | factorized form not equivalent to matrix multiplication | Same as JEM7.0 |
| 8.e | List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output) | n/a | 4×4 HyGT 2-interations, 8×8 HyGT 4-interations | Same as JEM7.0 |

- In-loop filtering

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 9. In-loop filtering |  | HM | JEM | Proposed |
| 9.a | List line buffer size in terms of luma and chroma lines (if any) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) ALF (luma 8, chroma 4) | Deblocking: same as JEM7.0 SAO: same as JEM7.0 ALF: (luma 7, chroma 4) |
| 9.b | List all the filters used for in-loop filtering | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. ALF: 9×9/7×7/5×5 diamond filter support for luma; and 5×5 diamond filter support for chroma | Deblocking: same as JEM7.0 ALF: 5×5 square, octagon, snowflake, and diamond filter support for luma; and 5×5 diamond filter support for chroma |
| 9.c | Minimum processing unit size | Deblocking (luma: 8×8 grid with 4×4 filter decision, chroma 8×8 grid with 4×4 filter decision) | Deblocking (luma 4×4 grid with 4×4 filter decision, chroma 8×8 grid with 2×2 filter decision) ALF: for luma, filter decision is based on 2×2 block Bilateral filter: sample level filter switch (luma only) | Deblocking: same as JEM7.0 ALF: for luma, filter decision is based on 4×4 block Bilateral filter: same as JEM7.0 |

- “tool-off” and “tool-on” tests

| Tool Level Q&A |  |  |
| --- | --- | --- |
| Category 10. “tool-off” and “tool-on” tests |  | Proposed |
| 10.a | List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools | TMM, DST-VII with residual flipping, MRM, CF, MIP, RC-ALF, AFFINE, LIC, BIO, OBMC, MV16, ATMVP, IMV, IBF, LM-CHROMA, P-PDPC, NSST, BLF, INT_PREV, COEFF-COD |

Table. Summary of “tool-off” and “tool-on” tests for the proposed vs. HM

| Tested tool name | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| TMM mode | 2.6% | 2.3% | 2.5% | 77% | 98% | -5.4% | -5.1% | -5.3% | 152% | 169% |
| MRM | 0.4% | 0.4% | 0.4% | 96% | 95% | -3.5% | -2.7% | -2.9% | 111% | 183% |
| DST-VII with residual flipping | 1.9% | 0.7% | 0.8% | 81% | 100% | -2.9% | -0.9% | -0.9% | 135% | 100% |
| CF | 0.2% | 0.2% | 0.1% | 99% | 100% | -0.5% | -0.6% | -0.7% | 101% | 100% |
| MIP | 0.1% | 0.1% | 0.1% | 99% | 100% | -0.2% | -0.2% | -0.2% | 101% | 100% |
| RC-ALF | 2.4% | 1.0% | 0.8% | 99% | 83% | -3.4% | -1.3% | -1.0% | 111% | 195% |
| AFFINE | 2.3% | 1.8% | 1.7% | 94% | 96% | -4.6% | -3.4% | -3.6% | 140% | 119% |
| LIC | 1.0% | 0.6% | 0.6% | 82% | 100% | -0.5% | -0.2% | -0.1% | 119% | 101% |
| BIO | 1.7% | 0.5% | 0.5% | 94% | 73% | -3.2% | -1.1% | -0.9% | 108% | 208% |
| OBMC | 0.8% | 1.1% | 1.3% | 95% | 87% | -0.6% | -1.5% | -1.6% | 107% | 120% |
| MV16 | 0.0% | 0.1% | -0.1% | 99% | 95% | 0.0% | 0.0% | 0.0% | 101% | 102% |
| ATMVP | 1.3% | 1.4% | 1.5% | 95% | 84% | -2.1% | -2.0% | -2.1% | 103% | 115% |
| IMV | 1.4% | 1.9% | 2.1% | 89% | 102% | -4.8% | 7.9% | 7.4% | 116% | 95% |
| IBF | 0.1% | 0.2% | 0.2% | 101% | 100% | -0.1% | -0.2% | -0.1% | 101% | 100% |
| LM-CHROMA | 4.0% | 10.1% | 6.7% | 100% | 100% | -1.6% | -19.6% | -17.0% | 101% | 101% |
| P-PDPC | 0.2% | 0.3% | 0.2% | 100% | 100% | -0.1% | 0.2% | 0.2% | 100% | 101% |
| NSST | 1.6% | 4.1% | 4.2% | 89% | 100% | -3.1% | -7.1% | -7.2% | 117% | 102% |
| BLF | 0.4% | 0.0% | 0.0% | 99% | 99% | -0.3% | 0.0% | 0.1% | 101% | 103% |
| INT-PREV | 0.9% | 0.6% | 0.6% | 99% | 98% | -0.4% | -0.2% | -0.3% | 101% | 100% |
| COEFF-COD | 0.4% | 0.3% | 0.1% | 99% | 100% | -0.7% | -0.2% | -0.4% | 101% | 100% |

[end of document]