| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0015-v1 |
| --- | --- |

| Title: | Description of SDR, HDR, and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu Philippe Hanhart Rahul Vanam Yuwen He Yan Ye Taoran Lu Fangjun Pu Peng Yin Walt Husak Tao Chen | Tel: Email: | {xiaoyu.xiu, philippe.hanhart, rahul.vanam, yuwen.he, yan.ye}@interdigital.com {tlu, fpu, pyin, whj,tchen}@dolby.com |
| Source: | InterDigital Communications, Inc. Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This response to the Joint Call for Proposals on Video Compression with Capability beyond HEVC was jointly developed by InterDigital Communications and Dolby Laboratories. It answers all three categories covered in the joint CfP, the SDR category, the 360° category, and the HDR category. The software of this response is written based on the JEM and the 360Lib software.

Significant coding efficiency improvements and reasonable decoding complexity are factors important to the success of the new video coding standard. Design of the core SDR codec in this response took both factors into careful consideration: many of the compression technologies in the JEM are simplified to reduce average and worst case complexity with negligible coding performance loss, and two additional compression technologies are added to further improve coding efficiency.  The objective performance and complexity of the proposed SDR codec are summarized as follows:

Compared to the HM anchors, the proposed SDR codec achieves:

- For constraint set 1, {Y, U, V} BD-rate savings: {35.72%, 44.75%, 44.95%}, Enc time: 1710%, Dec time: 263%

- For constraint set 2, {Y, U, V} BD-rate savings: {27.18%, 43.81%, 44.51%}, Enc time: 1827%, Dec time: 301%

Compared to the JEM anchors, the proposed SDR codec achieves:

- For constraint set 1, {Y, U, V} BD-rate savings: {3.98%, 3.28%, 3.16%}, Enc time: 205%, Dec time: 33%

- For constraint set 2, {Y, U, V} BD-rate savings: {3.64%, 2.55%, 4.48%}, Enc time: 203%, Dec time: 45%

The proposed SDR codec is used as the core coding engine in the 360° category and the HDR category. For the 360° category, projection formats customized to the input video are used in this response as a “coding tool” to improve coding efficiency. Additional 360° specific compression technologies are proposed to improve the subjective quality, especially in terms of alleviating the often observable “face seam” artifacts for this type of video. In terms of objective coding performance using the end-to-end WSPSNR metric, this response achieves average BD-rate savings for the {Y, U, V} components of {-33.87%, -54.04%, -56.79%} and {-13.51%, -18.29%, -20.96%} over the HM and JEM anchors, respectively.

For the HDR category, this proposal includes two major coding tools: an in-loop reshaper and Luma-based QP Prediction. The in-loop reshaper maps (“reshapes”) Luma sample values inside the coding loop according to a 1-D LUT for better HDR coding efficiency. The reshaping process does not require any changes to the existing DPB handling mechanism. Luma-based QP Prediction reduces the bitrate overhead of the deltaQP syntax when spatial Luma-dependent QP adaptation (LumaDQP) is used for HDR coding. Pre- and post-processing are explicitly excluded in this proposal since all additional tools reside inside the core codec. The HDR bitstreams submitted for the CfP target better subjective performance using the above tools. The in-loop reshaper can also be configured to maximize HDR objective metrics.

In summary, this response provides a complete set of solutions for all three types of video formats essential to the video applications targeted by the new video coding standard. The codec in this response is shown to achieve large coding efficiency gain and substantial subjective quality improvement in all three categories. The decoding complexity increase compared to HEVC is asserted to be reasonable.

# Contents

Abstract	i

Contents	iii

1	Introduction	1

2	Decoder algorithm description	2

2.1	General algorithm description	2

2.1.1	Bitstream structure	2

2.1.2	Entropy decoding	2

2.1.3	Conversion process from entropy decoded data to symbols	2

2.1.4	Decoder-side motion refinement	3

2.1.5	Inverse quantization	7

2.1.6	Inverse transforms	8

2.1.7	Motion compensation	8

2.1.8	Intra prediction	13

2.1.9	In-loop filtering	14

2.1.10	Additional tools	15

2.1.11	Additional algorithmic description topic(s)	15

2.2	360° Video	16

2.2.1	Projection format (including padding)	16

2.2.2	Quantization handling	19

2.2.3	360° video specific decoding tools	19

2.3	HDR	24

2.3.1	Post-processing	25

2.3.2	Quantization handling	25

2.3.3	HDR specific decoding tools	25

3	Encoder algorithm description	27

3.1	General algorithm description	27

3.1.1	Rate-distortion optimization	27

3.1.2	Partitioning structure selection	27

3.1.3	Intra prediction mode estimation and type selection	29

3.1.4	Motion estimation and motion segment selection	29

3.1.5	Mode decisions	29

3.1.6	In-loop filtering type selection and parameter estimation	29

3.1.7	Transforms and transform type selection	30

3.1.8	Quantization and quantization type selection	30

3.1.9	Entropy coding	30

3.1.10	Additional tools	30

3.1.11	Additional encoder optimization	30

3.1.12	Additional algorithmic description topic(s)	31

3.2	360°	31

3.2.1	Usage of projection format adaptation	31

3.2.2	360° Video specific encoding tools	33

3.2.3	Additional encoder optimization	33

3.3	HDR	33

3.3.1	Pre-processing	33

3.3.2	HDR specific encoding tools	33

3.3.3	Additional encoder optimization	34

4	Compression performance	35

4.1	SDR: Constraint set 1 configuration relative to HM anchor	35

4.1.1	Class SDR-A (RD-curves)	35

4.1.2	Class SDR-B (RD-curves)	36

4.1.3	Overall (BD-rate summary tables)	37

4.2	SDR: Constraint set 2 configuration relative to HM anchor	38

4.2.1	Class SDR-B	38

4.2.2	Overall	39

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	40

4.3.1	Class SDR-A	40

4.3.2	Class SDR-B	41

4.3.3	Overall	42

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	43

4.4.1	Class SDR-B	43

4.4.2	Overall	44

4.5	HDR: Constraint set 1 configuration relative to HM anchor	45

4.5.1	Class HDR-A	45

4.5.2	Class HDR-B	45

4.5.3	Overall	45

4.6	HDR: Constraint set 1 configuration relative to JEM anchor	46

4.6.1	Class HDR-A	46

4.6.2	Class HDR-B	46

4.6.3	Overall	46

4.7	360° Video: Constraint set 1 configuration relative to HM anchor	46

4.7.1	Class 360	46

4.7.2	Overall	47

4.8	360° Video: Constraint set 1 configuration relative to JEM anchor	48

4.8.1	Class 360	48

4.8.2	Overall	49

5	Decoder complexity analysis	49

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	49

5.2	Description of computing platform used to determine decoding times reported in Section 5.1	51

5.3	Memory usage of decoder	51

5.4	Complexity characteristics of decoder entropy decoding operation	53

5.5	Complexity characteristics of decoder inverse quantization	53

5.6	Complexity characteristics of decoder inverse transform operation	53

5.7	Complexity characteristics of decoder-side motion refinement	53

5.8	Complexity characteristics of decoder motion compensation	53

5.9	Complexity characteristics of decoder intra-frame prediction operation	54

5.10	Complexity characteristics of decoder in-loop filtering operation	54

5.11	Complexity characteristics of additional decoder tools	54

5.12	Complexity characteristics of 360° video specific decoding tools	54

5.13	Complexity characteristics of HDR specific decoding tools	54

5.14	Degree of capability for decoder parallel processing	55

6	Encoder complexity analysis	55

6.1	Encoding time and measurement methodology	55

6.2	Description of computing platform used to determine encoding times reported in Section 6.1	56

6.3	Memory usage of encoder	57

6.4	Complexity characteristics of encoder intra-frame prediction type selection	58

6.5	Complexity characteristics of encoder motion estimation and motion segmentation selection	58

6.5.1	General	58

6.5.2	Complexity characteristics of usage of decoder-side motion refinement	58

6.6	Complexity characteristics of encoder transforms and transform type selection	58

6.7	Complexity characteristics of encoder quantization and quantization type selection	58

6.8	Complexity characteristics of encoder in-loop filtering type selection and parameter estimation	59

6.9	Complexity characteristics of encoder entropy coding type selection	59

6.10	Complexity characteristics of additional encoder tools	59

6.11	Complexity characteristics of 360° video specific encoding tools	59

6.12	Complexity characteristics of HDR specific encoding tools	59

6.13	Degree of capability for encoder parallel processing	59

7	Algorithmic characteristics	59

7.1	Random access characteristics	59

7.2	Delay characteristics	59

7.3	Additional characteristics discussion topic(s)	59

8	Software implementation description	59

9	Highlighted aspects discussion	60

9.1	SDR	60

9.2	360° Video	61

9.3	HDR	65

10	Closing remarks	67

11	Patent rights declaration(s)	68

12	Annex: Questionnaire	69

12.1	Codec level questions	69

12.2	Tool level questions	70

12.2.1	Category 1. Partitioning structure	70

12.2.2	Category 2. Entropy coding	70

12.2.3	Category 3. Control data derivation process	70

12.2.4	Category 4. Decoder-side motion refinement	71

12.2.5	Category 5. Intra prediction	71

12.2.6	Category 6. Inter prediction	72

12.2.7	Category 7. Quantization	72

12.2.8	Category 8. Transforms	72

12.2.9	Category 9. In-loop filtering	73

12.2.10	Category 10. “tool-off” and “tool-on” tests	74

13	Reference	76

# Introduction

This document describes the response to the Joint Call for Proposals on Video Compression with Capability beyond HEVC [1]. This response was jointly developed by InterDigital Communications and Dolby Laboratories. It contains video compression technologies for all three categories in the Joint CfP: the SDR category, the 360° category, and the HDR category.

The core SDR codec in this response is based on the Joint Exploration Model (JEM) design [3]. The JEM achieves impressive coding efficiency gain over HEVC but at the cost of significant complexity increase. Take Constraint Set 1 for example. Compared to the HM anchors, the JEM anchors achieve 33% BD rate reduction, but with almost 8 times the encoding time and 7.6 times the decoding time [2]. The complexity increase at the decoder side is of particular concern, as in many mainstream video applications, a video content is encoded once or a small number of times, but decoded many times and on different end devices, a lot of which with limited battery power. Therefore, a main design goal of this response is to reduce the decoding complexity without causing performance loss, considering not only average complexity reduction, but also worst case complexity reduction and reduction in memory bandwidth. A second design goal of this response is to explore other compression technologies that can further improve coding efficiency.

Corresponding to the main design goal, i.e., decoder complexity reduction, significant simplifications to the following compression technologies in the JEM are proposed for the SDR category:

- Simplified motion search for frame-rate up conversion (Section 2.1.4.1)

- Simplified decoder side motion vector refinement (Section 2.1.4.2)

- Simplified bi-directional optical flow (Section 2.1.7.1)

- Simplified overlapped block motion compensation (Section 2.1.7.2)

- Simplified local illumination compensation (Section 2.1.7.3)

- Simplified ATMVP and STMVP merge modes (Section 2.1.7.4)

- Simplified adaptive loop filters (Section 2.1.9.1)

Corresponding to the second design goal, i.e., additional compression technologies, the following technologies are proposed for the SDR category:

- Multi-type tree (MTT) (Section 2.1.10.1)

- Decoder-side intra mode derivation (DIMD) (Section 2.1.8.1)

Simulation results show the effectiveness of the proposed technologies. As will be detailed in Sections 4.1 and 5.1, compared to the HM anchors, the SDR codec in this response achieves average{Y, U, V} BD-rate savings of {35.72%, 44.75%, 44.95%} for constraint set 1, and {27.18%, 43.81%, 44.51%} for constraint set 2. The average encoding time and decoding time are 1710% and 263% for constraint set 1, and 1827% and 301% for constraint set 2. Compared to the JEM anchors, the average {Y, U, V} BD-rate savings of the SDR codec are {3.98%, 3.28%, 3.16%} and {3.64%, 2.55%, 4.48%} for constraint set 1 and constraint set 2, respectively. The average encoding time and decoding time are respectively 205% and 33% for constraint set 1, and respectively 203% and 45% for constraint set 2.

The SDR codec in this response is used as the core coding engine for the 360° and HDR categories. For the 360° category, additional compression technologies that fall into two categories are proposed. In the first category of technologies, projection formats are customized based on the input video content and used as a “coding tool” to achieve higher coding efficiency:

- Hybrid angular cubemap (HAC) (Section 2.2.1.1)

- Adaptive frame packing (AFP) (Section 2.2.1.2)

In the second category of technologies, special attention is paid to improve subjective quality by alleviating/eliminating the commonly observed “face seam” artifacts in rendered viewport video. This is achieved by considering the special neighboring relationship of samples and blocks based on the spherical geometry for the 360° video. This category of technologies includes the following:

- Geometry padding of the reference pictures (Section 2.2.3.1)

- Face discontinuity handling (FDH) (Section 2.2.3.2)

- Post filtering (Section 2.2.3.3)

For the 360° category, in terms of the end-to-end WSPSNR metric, this response achieves average BD-rate savings for the {Y, U, V} components of {-33.87%, -54.04%, -56.79%} and {-13.51%, -18.29%, -20.96%} over the HM and JEM anchors, respectively. Subjective quality is also noticeably improved with better texture and edges with reduced blocking artifacts compared to the HM anchor. Visual quality improvements are also observed over the JEM anchor, particularly for the Chairlift sequence. No face seam artifact was observed in our expert viewing.

For the HDR category, this proposal includes two major coding tools: an in-loop reshaper (Section 2.3.3.1) and Luma-based QP Prediction (Section 2.3.3.2). Luma sample values are modified inside the coding loop according to a 1-D LUT for better HDR coding efficiency. DPB handling is in the non-reshaped domain. Inter prediction is performed in the non-reshaped domain and the reconstructed pictures are stored in the DPB directly. Intra prediction is performed in the reshaped domain and the reconstructed pictures are inverse reshaped before being placed into the DPB. Therefore, the existing DPB handling mechanism is not altered by this reshaping process. Luma-based QP Prediction reduces the bitrate overhead of the deltaQP syntax when coupled with spatial Luma-dependent QP adaptation (LumaDQP). These tools are located within the core codec avoiding any pre- and post- processing, thereby creating a cleaner implementation. The in-loop reshaper can be configured to maximize either objective or subjective metrics. For this response, the proponents chose better subjective performance for the HDR bitstreams submitted for the CfP.

# Decoder algorithm description

## General algorithm description

### Bitstream structure

This is the same as in the JEM.

### Entropy decoding

This is the same as in the JEM.

### Conversion process from entropy decoded data to symbols

#### Intra prediction parameters

This is the same as in the JEM.

#### Motion parameters

This is the same as in the JEM.

#### Quantization parameters

This is the same as in the JEM.

#### Transform coefficients

This is the same as in the JEM.

#### In-loop filter parameters

This is the same as in the JEM.

### Decoder-side motion refinement

#### Simplified frame-rate up conversion motion search

The frame-rate up conversion (FRUC) in the JEM has been significantly simplified in this response. Like the FRUC in JEM [3], the motion information of one CU is derived based on either template-matching or bilateral matching at both the encoder and the decoder. The FRUC motion derivation process is composed of four steps:  CU-level motion search, CU-level motion refinement, sub-CU-level motion search and sub-CU-level motion refinement.

To reduce the complexity of the FRUC in JEM, the following changes are made in this response:

- For the CU-level motion search and sub-CU-level motion search, an MV clustering method is proposed to reduce the number of tested MVs searched at these two stages.

- For the CU-level motion refinement and the sub-CU-level motion refinement, a multi-stage motion refinement method is proposed to find the optimal MV within a limited number of search iterations.

- Early termination methods are proposed to conditionally skip the entire sub-CU-level motion derivation process, that is, to skip both the sub-CU-level motion search and the sub-CU-level motion refinement.

- One constrained search range is proposed to reduce memory bandwidth requirement by ensuring that the entire motion search in the FRUC can be performed by fetching the reference samples within one fixed size search window only once.

##### CU-/sub-CU-level motion search

To reduce the number of tested MV candidates, one clustering-based motion search is proposed, which consists of the following steps: 1) initial MV candidate generation; 2) reference picture selection; 3) MV scaling and pruning and 4) MV clustering.

##### Initial MV candidate generation

The generation of initial MV candidates in JEM-7.0 [3] for the CU-level motion search and the sub-CU-level motion search are modified with the following two changes:

- The JEM-7.0 includes a picture-level motion field interpolation process that generates motion information for all 4×4 blocks. This process is completely removed. Correspondingly, the four MV candidates from the picture-level MV field interpolation process are removed from the initial MV candidate list.

- The MV candidates obtained from advanced temporal motion vector prediction (ATMVP) and spatial-temporal motion vector prediction (STMVP) are not included in the CU-level initial candidates. The total number of the merge candidates that are used as FRUC MV candidates remains the same as that used in JEM-7.0 (i.e., 7).

##### Reference picture selection

The CU- and sub-CU-level searches in the JEM could include searching multiple reference pictures. This is undesirable because it increases memory bandwidth access. To address this problem, one reference picture selection method is proposed in this response to pick one single refence picture from each reference picture list L0/L1 for the CU-level motion search and the sub-CU-level motion search.

For the template matching mode, the proposed method is based on the reference pictures of the initial MV candidates, and the reference picture which is referenced the most frequently in L0/L1 by the initial MV candidates is selected. When two or more reference pictures have the same usage, the reference picture with the smallest distance to the current picture is selected.

For the bilateral-matching mode, the selected reference pictures in L0 and L1 must fulfill the condition necessary to enable bilateral-matching, that is, for non-low-delay B pictures, the sum of the POC values of the two reference pictures is twice the POC value of the current picture; and for low-delay B pictures, the temporal distance to the current picture is minimized in one reference picture list. The proposed method selects two reference pictures (one in L0 and the other in L1) by considering the usages of the reference pictures in both lists. Specifically, for each initial MV candidate in L0, the usage of the corresponding L0 reference picture is incremented by 1. For each initial MV candidate in L1, its paired reference picture in L0 (based on the bilateral-matching condition) is identified and the usage of that paired L0 reference picture is incremented by 1. The above process is repeated for each MV candidate in the initial MV list, and the L0 reference picture with the highest usage, along with its paired L1 reference picture are selected.

##### MV scaling and pruning

After the reference picture in each reference picture list L0/L1 is determined, the initial MV candidate list is updated by scaling the original MV candidates to the selected reference picture. The same MV scaling process used in HEVC is used in this proposal. Additionally, because the values of multiple MV candidates may become the same after MV scaling, pruning is also performed to remove any redundant MV candidates and to only keep unique MV candidates in the final candidate list.

##### MV clustering

To reduce the computational complexity of the initial motion search, one MV clustering method is applied to reduce the total number of the MV candidates to be tested at both CU-level and sub-CU-level. This is achieved by grouping similar MV candidates into multiple sets if the distance between a given MV candidate in a set and the centroid of the set is below a predefined threshold. Denote the MV candidates after the reference picture selection and the MV scaling and pruning steps as MV0, MV1, …, MVN-1, where N is the size of the MV candidate list. In the proposed method, the MV candidates are clustered based on the following steps:

- Cluster initialization: create the first cluster by adding the first MV, MV0, into the cluster.

- Cluster assignment: for each subsequent MV candidate, calculate the distance between the current MV candidate and the centroid of each existing MV cluster. Find the cluster which yields the least distance. If the least distance is sufficiently small (i.e., below the threshold), then the MV candidate is added into the selected cluster; otherwise, a new cluster is created and the current MV candidate is added into the new cluster.

- Cluster update: if a new MV candidate is added into a cluster at step 2), then the centroid of that cluster is updated considering the impact of the newly added element as follows:

μi(t+1)=1Si(t)∙MVj∈Si(t)MVj

where Si(t) represents the number of the elements in the i-th cluster after adding the t-th MV candidate into the cluster; μi(t+1) is the updated centroid of Si. This process is repeated until all MV candidates have been scanned and added to a cluster. The proposed clustering process considers each candidate MV only once. Because there is no iteration, the proposed process has very low complexity. The centroids of the resulting clusters are included in the final MV candidate list for CU-level and sub-CU-level motion search.

##### CU-/sub-CU-level motion refinement

Unlike the CU-level and sub-CU-level motion refinement processes in the FRUC in JEM, which does not limit the number of searches and could invoke more searches that require interpolations (if the MVs from the motion search stage have fractional precision), the proposed motion refinement process uses a multi-stage strategy to reduce the number of interpolations. An upper bound of how many motion refinement iterations can be performed is also imposed. The proposed motion refinement method consists of two steps, i.e., integer-pel motion refinement and fractional-pel motion refinement.

##### Integer-pel motion refinement

At the integer-pel motion refinement stage, the proposed method searches the MVs at integer sample positions and maintain the best integer MV that minimizes the matching cost. It works by firstly rounding the initial MV obtained from the CU-/sub-CU-level motion search to the integer-pel precision and using it as the search center. Then, the matching costs of a set of integer MVs based on a certain search pattern around the search center are calculated and the best integer MV that minimize the cost is maintained and used as the new search center. The above search process continues until either of two conditions is met: 1) the search center is not updated for two consecutive search iterations; or 2) the number of the search iterations reaches its maximum. In this response, the maximum is set to 12. Like in JEM-7.0, two different search patterns, i.e., center-biased diamond search and cross search, are used for the integer motion search at the CU-level and the sub-CU-level, respectively.

##### Fractional-pel motion refinement

After the optimal integer-pel MV is determined, the precision of the derived MV is improved progressively at fractional sample positions (including 1/2-, 1/4- and 1/8-pel precisions) based on the cross search. At most 3 search iterations are performed. Additionally, to reduce the search complexity, the following conditions are applied to completely or partially disable the fractional-pel motion search at the CU-level and sub-CU-level motion refinement stages.

- For the sub-CU-level motion refinement, the fractional-pel motion refinement is always completely disabled.

- For the CU-level motion refinement, when the current CU is coded by bilateral-matching mode, an early termination method is applied for fractional-pel motion refinement. Specifically, the sum of absolute difference (SAD) between the two prediction signals (L0 and L1 prediction) using the current best MVs is calculated. If the SAD is no larger than a predefined threshold, the subsequent fractional-pel motion refinement is skipped. In this response, the threshold is set to 2BDepth-10 per sample.

Figure 1 depicts the block diagram of the fractional-pel motion refinement after the above early termination conditions are applied. Because the SAD value between two prediction signals are already available when calculating the matching cost for the bilateral-matching mode, the proposed early termination method does not introduce any extra computations to the encoder and the decoder.

Figure 1. The proposed early termination of the fractional-sample motion refinement

##### Early termination of the sub-CU-level motion derivation

Like the early termination of the sub-CU-level refinement in Section 2.1.4.1.2, it is also proposed to completely disable the entire sub-CU-level motion derivation process (including the sub-CU-level motion search and the sub-CU-level motion refinement) under the following conditions:

- The sub-CU-level motion derivation is always completely disabled for the template-matching mode.

- When the current CU is coded by the bilateral-matching mode, the sub-CU-level motion derivation is conditionally skipped when the SAD between two prediction signals as derived at the CU-level is no larger than a predefined threshold. In this response, the threshold is set to 2BDepth-9 per sample.

Figure 2 illustrates the block diagram of the FRUC motion derivation process after the proposed conditions are applied to early terminate the sub-CU-level motion derivation.

Figure 2. The proposed early termination of the sub-CU-level motion derivation

##### Constrained search range for FRUC motion derivation

In this response, one constrained search range is applied to the whole FRUC motion search process such that only one single memory access to one reference picture in each reference picture list L0/L1 is needed to obtain all reference samples necessary to derive the MVs of the current CU. More specifically, based on the CU-level MV clustering as discussed in Section 2.1.4.1.1, one search center is determined from the centroids of the derived MV clusters. This is done by selecting the centroid of the largest MV cluster. Then, one search region of a fixed size is determined around this selected search center. The search region is set to 96×96 luma samples in this response. After that, each MV that is tested at the CU-/sub-CU-level motion search and CU-/sub-CU-level motion refinement stages are clipped such that all reference samples needed in the search (for interpolation and/or for the SAD calculation) are within the specified search range.

#### Decoder side motion vector refinement simplification

The decoder-side motion vector refinement (DMVR) tool included in JEM-7.0 [3] is used in this response. To reduce the DMVR complexity, an early termination method is applied to terminate DMVR if the two prediction signals from the two prediction lists L0 and L1 are sufficiently similar. The same early termination method is used for the BIO and is as described in Section 2.1.7.1.1, with the exception that only the CU-level early termination needs to be applied. This is because the DMVR is only applied to the conventional merge candidates at the CU-level in JEM-7.0.

### Inverse quantization

This is the same as in the JEM.

### Inverse transforms

This is the same as in the JEM.

### Motion compensation

#### Bi-directional optical flow simplification

The bi-directional optical flow (BIO) coding tool in JEM-7.0 [3] is used in this response. In order to reduce BIO’s complexity, the following two modifications are applied in this response:

- Early termination of the BIO process at both CU-level and sub-CU-level.

- Modifications to the BIO refinement derivation process for reducing the complexity.

##### Two-stage early termination of the BIO

It has been observed that the prediction improvement due to BIO is limited when the L0 and L1 prediction signals are sufficiently similar. Therefore, a two-stage early termination method is used in this response to conditionally disable the BIO operations depending on the similarity between the two prediction signals. The two stage scheme is first applied at the CU-level and then at the sub-CU-level. Specifically, the proposed method first calculates the SAD between the two prediction signals at the CU level. Given that the BIO is only applied to luma [3], only the luma samples need to be considered for the SAD calculation. If the CU-level SAD is no larger than a predefined threshold, the BIO process is completely disabled for the whole CU. The CU-level threshold is set to 2BDepth-9  per sample in this response. If the BIO process is not disabled at the CU level, and if the current CU contains multiple sub-CUs, the SAD of each sub-CU inside the CU will be calculated. Then, the decision on whether to enable or disable the BIO process is made at the sub-CU-level based on a predefined sub-CU-level SAD threshold, which is set to 3∙2BDepth-10 per sample in this response. Figure 3 illustrates the block diagram of the modified prediction generation process after the proposed BIO early terminations are applied.

Figure 3. The proposed two-stage early termination for the BIO

##### The modified BIO refinement derivation

If the BIO is still applied after the decision according to Figure 3, a modified BIO derivation is applied in this response. Two main modifications are made to reduce the computational and implementational complexity.

- The BIO process applies filters to derive the BIO horizontal and vertical gradients. In JEM-7.0, if horizontal gradients are being derived, then a vertical interpolation filtering is followed by a horizontal gradient filtering; otherwise, if vertical gradients are being derived, then a vertical gradient filtering is followed by a horizontal interpolation filter. In this response, the order of applying the two filters is modified such that a horizontal filtering process is always followed by a vertical filtering process for both horizontal and vertical gradient derivations. Specifically, when calculating horizontal gradients, the horizontal gradient filter is firstly applied, followed by the vertical interpolation filter; when deriving vertical gradients, the interpolation filter is firstly applied in the horizontal direction, followed by the gradient filter being applied vertically. This is to make the BIO gradient filtering process better aligned with the existing filtering processes in the HEVC, such as, the interpolation in motion compensation (MC) and deblocking, where a horizontal filtering is always applied first, followed by a vertical filtering. In terms of the interpolation and gradient filters, this response uses the same 2-D separable filters as in JEM-7.0 (i.e., one 6-tap gradient filter and one 6-tap interpolation filter).

- For the BIO in JEM-7.0, the rounding operations (e.g. rounding is used in gradient derivation as well as local motion refinement calculation) are based on absolute values. To further reduce the BIO complexity, it is proposed to modify the BIO rounding operations by changing the right-shifts used for the rounding to be arithmetic right-shifts (i.e., the sign of the input is kept unchanged during the right-shifts). With such change, the calculations of the absolute values can be removed.

#### Overlapped block motion compensation simplification

Modifications are made to the overlapped block motion compensation (OBMC) coding tool in JEM-7.0 [3] in this response to reduce its complexity. The following modifications are made:

- Block grouping is applied in order toto reduce the number of motion compensations invoked during the OBMC.

- Skip some motion compensation steps in the OBMC based on the difference between the MVs of the current block and those of its spatial neighbor.

##### Block grouping in OBMC

Different from the OBMC in JEM-7.0 where the OBMC is applied to the 4×4 sub-blocks inside a CU in the raster scan order, the OBMC process in this response is performed in two steps. In the first step,  referred to as “boundary OBMC,” the samples on the top/left boundaries of the current CU are adjusted using the MVs from its spatial neighboring CUs. If the CU is not coded in sub-block mode, the OMBC process ends after the first step. Otherwise (if the CU is coded in sub-block mode), the second step,  referred as “internal OBMC,” is performed. In the second step, the samples on the top, left, bottom and right boundaries of each sub-block, except the samples on the CU boundaries that have already been adjusted in the first step, are adjusted using the MVs of its neighboring sub-blocks in the same CU. Note that due to the modified OBMC process, the order in which the boundary samples of the current CU are adjusted during the OBMC process is different from that in the JEM-7.0. Assuming the current CU is in the size of 16×4, Figure 4 compares the OBMC process in JEM-7.0 and the proposed OBMC process, where the numbers specify the order in which the boundary samples within the CU are processed. As shown in Figure 4, the OBMC using the MVs of the four neighboring sub-blocks on top of the current CU are grouped into the boundary OBMC process, i.e., OBMC #0, #3, #6 and #9 in JEM-7.0 now become OBMC #0, #1, #2 and #3 in the proposed process. Given that some of those sub-blocks may belong to the same neighboring CU, their MVs are more likely to be identical, in which case the OBMC using those MVs can be combined to one single MC with a larger block size for reducing the complexity.  Based on such consideration, block grouping is applied to reduce the number of motion compensations invoked during the proposed OBMC process. Unlike the JEM-7.0 where the OBMC is always performed on 4×4 units, variable sub-block sizes are used in the proposed method. Specifically, for the boundary OBMC process, the adjacent 4×4 neighboring blocks that are located along the same CU boundary (i.e., either top or left) and have the same motion are grouped into one block group; for each block group, only one single MC is performed. For the internal OBMC process, it is proposed to align the sub-block size with that used for the sub-block-level motion derivation (e.g., FRUC, affine, ATMVP). Figure 5 illustrates the proposed OBMC process where the bold arrows and the patterned areas indicate the used MVs and the sample areas that are adjusted at each step.

Figure 4. The order of adjusting boundary samples during the OBMC process in JEM-7.0 (left) and the proposed OBMC process (right).

Figure 5. Illustration of the proposed OBMC process

Additionally, in this response, the internal OBMC process is always disabled for the CUs that are coded by the STMVP mode.

##### Conditional skipping of motion compensation during the OBMC

In this response, some motion compensations are skipped during the OBMC process based on the motion similarity between one current block group and its neighboring block groups. Specifically, each time before motion compensation is invoked using the motion information from a given neighboring block group, the neighboring motion is compared to the current block’s motion based on the following three conditions:

- If the neighboring motion and the current motion are both predicted using uni-pred or both predicted using bi-pred;

- If the same reference picture(s) are used by the neighboring motion and the current motion;

- If MV difference between the neighboring motion and the current motion is no larger than 1/2-pel (both L0 and L1 MVs are checked if bi-pred is used).

If all the 3 conditions are met, then motion compensation is not performed (this is applicable to luma and chroma) using that given neighboring motion. This is because when the neighboring motion is substantially similar to the current motion, the perceived benefits of OBMC will be limited.

#### Local illumination compensation simplification

In this response, the existing design of local illumination compensation (LIC) is modified in three ways:

- The LIC method is modified for bi-predicted blocks.

- For blocks coded using OBMC, the LIC parameters are derived at the regular MC stage.

- Completely disable the LIC for the FRUC bilateral-matching mode, including removal of LIC flag signaling for CUs coded using FRUC bilateral-matching mode.

##### The improved LIC for bi-prediction

For bi-predicted blocks, in JEM-7.0 [3], LIC parameters (weight and offset) are estimated for each prediction direction separately. The estimated LIC parameters are applied to each prediction signal separately, and then the two LIC modified prediction signals are combined to form the bi-prediction signal. In other words, the JEM-7.0 design requires two individual LIC processes for bi-predicted CUs. To reduce the LIC complexity, in this response, the LIC process is modified for bi-prediction. Specifically, the proposed method first generates the bi-prediction signal; then, the LIC parameters are estimated only once using the bi-prediction signal; finally, the derived LIC parameters are applied to the bi-prediction signal of the current CU based on the LIC linear model. Figure 6 compares the LIC method in JEM-7.0 and the proposed LIC method for bi-prediction.

Figure 6. The comparison of the LIC method in JEM-7.0 and the proposed LIC method for bi-prediction

##### The simplified LIC process for the OBMC

In JEM-7.0 [3], the LIC is applied at the OBMC stage. In other words, during OBMC, for each of the MVs of the neighboring CUs and/or neighboring sub-blocks, the LIC parameters are separately estimated, resulting in multiple invocations of the LIC estimation process during the OBMC stage. To simplify the OBMC design, it is proposed to reuse the LIC parameters that are derived at the MC stage for the LIC operations at the OBMC stage. Specifically, the LIC parameters are estimated only once for the regular MC prediction signal of the current CU; then, instead of calculating the parameters again, the LIC parameters derived from the MC stage are applied at the OBMC stage.

#### Sub-block merge mode simplification

In this response, the following modifications are made to the ATMVP and STMVP sub-block merge modes in JEM-7.0 [3]:

- Signaling is added at the picture/slice-level to allow various sub-block sizes to be used for the ATMVP/STMVP-based motion derivation.

- Simplified ATMVP-based motion derivation by allowing one collocated picture to be used.

##### Picture/slice-level adaptation of the sub-block-size for the motion derivation of the ATMVP and STMVP

In this response, it is proposed to support the picture/slice-level adaptation of the granularity of the motion field derived by the ATMVP and STMVP modes. Specifically, it is proposed to signal in sequence parameter set (SPS) the default value of the sub-block size that is used to derive the motion parameters for the ATMVP and the STMVP in the slices that refers to the SPS. Additionally, one flag is signaled in slice segment header. If the flag is equal to 0, it indicates the default sub-block size as specified in the SPS is used for the ATMVP/STMVP-based motion derivation in the current slice. Otherwise (i.e., the flag is set to 1), another syntax element is signaled in the slice segment header to specify the ATMVP/STMVP sub-block size used for the slice.

In this response, a simple encoder method is used to decide the ATMVP/STMVP sub-block size for the current picture. One of two ATMVP/STMVP sub-block sizes, 4 and 8, are selected based on the average size of the ATMVP/STMVP blocks from the last coded picture in the same temporal layer. Assume the picture just coded is the i-th picture in the k-th temporal layer and it contains N CUs coded by the ATMVP and STMVP modes. Moreover, assume the sizes of those CUs are S0, S1, …, SN-1. The average size of the ATMVP/STMVP CUs is calculated as σk=i=0N-1SiN. Then, when coding the (i+1)-th picture in the same k-th temporal layer, the corresponding ATMVP/STMVP sub-block size gi+1k is determined according to the following:

gi+1k=4,σk<278σk≥27

For the first picture in each temporal layer, the ATMVP/STMVP sub-block size is always set to 4.

##### Simplified ATMVP-based motion derivation with one fixed collocated picture

ATMVP is derived using the following two steps in JEM-7.0:

- Add motion vectors from spatial neighboring CUs if neighboring CU is available and the MV of that CU is different from those MVs in the existing candidate list, in the order of A1, B1 , B0, and A0 , as shown in Figure 7. Denote the number of available and unique spatial candidates as N0;

- The first MV candidate from the N0 spatial candidates will be used to determine the collocated picture and the position from which to fetch the motion for each sub-block (by adding the displacement according to the MV). If N0 is equal to 0, the collocated picture signaled in the slice header and the collocated position with zero motion are used to fetch the motion for each sub-block.

Figure 7. Spatial merge candidates in JEM

The collocated picture of different CUs for ATMVP will not be always the same if multiple reference pictures are used. For different CUs in the current picture, having different collocated pictures for ATMVP derivation means that multiple reference pictures’ motion fields need to be fetched, which is undesirable due to the increase in memory bandwidth. In this response, a simplified design is proposed to use the same collocated picture as in HEVC, which is signaled at the slice header, as the collocated picture for ATMVP derivation. At the block level, if the reference picture of a neighboring block A is different from this collocated picture, the MV of block A is scaled using the HEVC temporal MV scaling method, and the scaled MV of block A is used in ATMVP.

Denote the motion vector used to fetch the motion field in the collocated picture Rcol as MVcol. In order to minimize the impact due to MV scaling, the MV in the spatial candidate list used to derive MVcol is selected in the following way in this response. If the reference picture of a candidate MV is the collocated picture, this MV is selected and used as MVcol without any scaling. Otherwise, the MV having a reference picture closest to the collocated picture is selected to derive MVcol with scaling.

### Intra prediction

#### Decoder-side intra mode derivation

The decoder-side intra mode derivation (DIMD) method as proposed in [4][5] is used in this response.

In the DIMD, the intra mode of the current CU is derived at both the encoder and the decoder using the CU’s neighboring reconstructed samples (i.e., template) before loop filtering [4], as illustrated in Figure 8. Specifically, for each selected intra mode candidate, the predicted samples of the template are calculated using that intra mode and the reference samples of the template; the SAD between the template samples and the predicted samples of the template is calculated; the intra mode that minimizes the SAD is selected as the DIMD intra mode of the CU. As no intra mode signaling is needed in DIMD, the number of the angular directions is increased to 129 when the DIMD is enabled.

Figure 8. Illustration of the DIMD

The DIMD is used as one additional intra mode and is adaptively enabled/disabled for each intra CU. For inter slices, one DIMD flag is signaled for each intra CU. When the DIMD flag is set, the chroma components always reuse the same intra mode as that derived for the luma component. For intra slices, the signaling of the DIMD flag is also at the CU level, except that the maximum BT/TT depth for signaling the DIMD flag is restricted to one, i.e., for BTs/TTs with partition depth larger than one, the DIMD flag is not signaled but shares the same value as the DIMD flag signaled from the upper level. Additionally, for intra slices, the DIMD is only applied to the luma component while the intra modes of the chroma components are always explicitly signaled.

One fast intra mode search method is used to identify the optimal DIMD intra mode. Firstly, the intra modes of five spatial neighbors (the same neighbors as those in the merge mode) are checked. If all the spatial candidates are coded by DC or Planar, then only DC and Planar will be considered. Otherwise, one initial candidate list, which consists of DC, Planar and every 4-th mode of the 33 HEVC angular intra directions, is tested to find one starting DIMD intra mode. If the starting DIMD intra mode is either DC or Planar, it is used as the optimal DIMD mode without further testing. Otherwise, one refinement process is applied where the final DIMD intra mode is identified through one progressive search process.

Because intra modes of neighboring CUs are used in the parsing stage, care needs to be taken avoid introducing any parsing dependency due to the DIMD. In this response, the intra mode propagation method in [5] is used to remove the interdependency between the DIMD intra mode derivation and parsing. Specifically, the intra modes of neighboring normal intra blocks (which are explicitly signaled) are firstly used to generate one intra mode for each DIMD CU. The generated intra mode is then used at the parsing stage, e.g., in determining the scanning order of transform coefficients, in parsing the NSST index and in forming the MPM candidate list. At the reconstruction stage, the intra mode of each DIMD CU is derived based on the process described above (as depicted in Figure 8) and used to form the prediction signal of the CU.

### In-loop filtering

#### Adaptive Loop Filtering

In block classification of JEM-7.0, for each sample in a 2×2 luma block, four derivatives are computed, namely the horizontal, the vertical, and two diagonal derivatives.  The derivative for each 2×2 block, G2×2dir, is the sum of those sample derivatives within the 2×2 block and calculated as

| - G2×2dir=i,j∈2×2 window¦gdir(i,j)¦ | (1) |
| --- | --- |

where dir is the direction such as horizontal, vertical, diagonal and anti-diagonal, and gdiri,j is the derivative at luma sample position (i, j).

In this response, the derivative calculation of 2×2 block for those pictures at the highest temporal layer is simplified by calculating the derivative only at top-left sample position (the gray circles in Figure 9) of each 2×2 block. Eq. (1) is simplified as

| - G2×2dir=gdiri0,j0 | (2) |
| --- | --- |

where (i0, j0) is the top-left position of each 2×2 block.

Figure 9. Block classification for a 2×2 block.  Gray circles represent pixels for which derivatives are calculated

### Additional tools

#### Multi-type tree (MTT)

In JEM, quadtree plus binary tree (QTBT) as shown in Figure 10 (a)(b)(c) is supported for coding block partitioning. In the response, the multi-type tree (MTT) partition based on [6] is implemented to increase tree partitioning flexibility. Besides the QT and BT that are already supported in JEM-7.0, MTT allows the triple-tree (TT) partitions as shown in Figure 10 (d)(e). The TT partitions split the block into three parts: the vertical TT as shown in Figure 10 (d) partitions the block in vertical direction, where the widths of the three partitioned sub-blocks are equal to 1/4, 1/2 and 1/4 of the width of the block, respectively, and the horizontal TT as shown in Figure 10 (e) partitions the block in the horizontal direction, where the heights of the three partitioned sub-blocks are equal to 1/4, 1/2 and 1/4 of the height of the block, respectively. No additional transforms are needed in MTT. Figure 11 shows the tree partition signaling in MTT framework. For all QT leaf nodes, if it is further split further by BT or TT, two flags will be signaled further: the first flag is partition type indicating whether it is a BT or TT partition, and the second flag is direction type indicating whether it is horizontal or vertical partition.

Figure 10. MTT partitioning types:(a) quad-tree partitioning (b) vertical binary-tree partitioning (c) horizontal binary-tree partitioning (d) vertical triple-tree partitioning (e) horizontal triple-tree partitioning

Figure 11. Illustration of the tree partition signaling in MTT framework

### Additional algorithmic description topic(s)

None.

## 360° Video

### Projection format (including padding)

#### Hybrid angular cubemap

Figure 12. Testing work flow for 360º video coding

The 360º video coding work flow used by the JVET is shown in Figure 12. The source video is converted to the coding projection format for encoding, and the reconstructed video is converted back from the coding projection format to the source projection format for end-to-end quality measurement.

Projection format can be used as a coding tool to improve compression efficiency. The specific projection format parameters can be optimized based on the input video content. In this response, a hybrid angular cubemap (HAC) projection format is proposed to improve coding performance by customizing the equal-angular cubemap (EAC) projection format based on the input signal. The EAC projection format [7] is based on the widely used cubemap (CMP) projection format, but adjusts the sampling distribution in the CMP face in such a way that sampling on the sphere is distributed with equal angle spacing in between neighboring sampling points. The 360Lib software package [8] supports the EAC format.

Although EAC can improve coding efficiency compared to the conventional CMP projection, the sampling distribution of EAC is fixed. In this response, the HAC format is proposed to generalize the EAC format and allow EAC to be customized based on the input video content. The HAC format is related to CMP with forward and backward mapping functions. Assuming (x, y) is defined in a CMP face with side length equal to 2, and (x’, y’) is the corresponding point in an HAC face, Equations (3) (4) are the forward mapping functions from CMP to HAC for the horizontal and vertical directions, respectively.

| x'=fxx=4π∙atan-1x∙tanπ4∙a | (3) |
| --- | --- |
| y'=fyy=4π∙btan-1y∙tanπ4∙b | (4) |

Equations (5) (6) are the backward mapping from HAC to CMP, for the horizontal and vertical directions, respectively.

| x=gxx'=tanπ4∙x'∙atanπ4∙a | (5) |
| --- | --- |
| y=gyy'=tanπ4∙x'∙btanπ4∙b | (6) |

where a and b are the HAC horizontal and vertical mapping function parameters, and are defined within the range of [0.3, 1.5]. The sampling distribution of HAC (that is, parameters a and b) for each face can be adaptively selected based on the content. As the parameter a or b gets closer to 0, the sampling distribution in that corresponding direction becomes closer to that of CMP.

Table 1. Face definition of HAC

| Face index | Notes |
| --- | --- |
| 0 | Front face |
| 1 | Back face |
| 2 | Top face |
| 3 | Bottom face |
| 4 | Right face |
| 5 | Left face |

HAC has 6 faces, with face index and orientation as defined in Table 1. In this response, the 3×2 layout is used in HAC frame packing. Figure 13 shows an example for the frame packed picture. Denote Fi as the i-th face of HAC, and (ai, bi) as the parameters defining the mapping functions of face Fi. In order to keep sampling continuities at face boundaries in between two neighboring faces, the following constraint is applied in HAC.

| b4=b0=b5 | (7) |
| --- | --- |
| b2=b1=b3 | (8) |

In other words, the first row faces F4, F0 and F5 will share the same vertical mapping function, and the second row faces F2, F1 and F3 will share the same vertical mapping function.

Figure 13. Frame packed picture in 3×2 layout for SkateboardInLot

The HAC parameters are determined at the encoder side at every IRAP (see Section 3.2.1.1) and signaled in the PPS. When the HAC parameters of the reference picture are different from that of the current picture, the reference picture needs to be converted into the HAC format of the current picture.

#### Adaptive frame packing

To reduce the visibility of face seam artifacts, adaptive frame packing is proposed in this response to periodically modify the location of discontinuous face boundaries in the frame packed pictures. The advantage of changing frame packing parameters at regular time intervals is that, for a discontinuous face boundary in the current picture, if there is a reference picture which uses a different frame packing where that same face boundary is continuous, then better prediction may be achieved for the areas located near the discontinuous face boundary in the current picture. For example, if the reference and current pictures are packed using different frame packing configurations as depicted in Figure 14(a) and Figure 14(b), respectively, then the face boundary between face #1 and face #4 in 3D geometry, marked by the red dashed lines, can be better predicted from the corresponding areas in the reference picture. This is because the areas near that face boundary in the reference picture, marked by the green line in Figure 14(a), are continuous and therefore generally easier to be coded at a higher quality; thus, better prediction may be provided for the current picture around these areas, reducing the visibility of the seam artifact in the current picture. Additionally, by changing frame packing parameters at regular intervals, locations of the discontinuous face boundaries in the frame packed pictures will change periodically. As a result, though face seams may still appear at some point in time, the duration of a face seam at the same location will be reduced. The result is visually more pleasant compared to having a face seam artifact always at the same location, which could catch the viewer’s attention and degrade perceived video quality when watching the viewport video.

As depicted in Figure 14, when the frame packing configuration of the reference picture is different from that of the current picture, the reference picture needs to be first converted into the frame packing configuration of the current picture. This conversion is straightforward and only involves “reshuffling” of the faces within the frame packed pictures. An example of the converted reference picture is shown in Figure 14(c).

The adaptive frame packing parameters are determined at the encoder side at every IRAP (see Section 3.2.1.2) and signaled in the PPS.

Figure 14. Inter frame prediction in the case of different frame packing configurations between the reference (a) and current (b) pictures. The converted reference picture used to predict the current picture is shown in (c).

#### Signaling of projection format and HAC parameters

The high level syntax is modified to include projection format and frame packing parameters signalling. The SPS and PPS are modified as described below. The SPS and PPS are signalled at the beginning of each IRAP segment.

- 360° video flag, coding projection format, and frame packing parameters as listed in Table 2, i.e., number of faces in horizontal and vertical directions in the frame packed picture, and each face’s position and orientation in the frame packed picture in the SPS and/or the PPS.

- If HAC projection format is used, then the horizontal and vertical HAC parameters of each face as listed in Table 3 in the PPS.

Table 2. Frame packing parameters signalling

num_face_rows

num_face_columns

for( i = 0; i < num_face_rows; i++ ) {

for( j = 0; j < num_face_columns; j++ ) {

face_idx[ i ][ j ]

face_rotation_idc[ i ][ j ]

}

}

Table 3. HAC sampling parameters signalling in PPS

for( i = 0; i < num_face_rows; i++ ) {

for( j = 0; j < num_face_columns; j++ ) {

HAC_sampling_dir_x[ i ][ j ]

HAC_sampling_dir_y[ i ][ j ]

}

}

### Quantization handling

The same static quantization parameter setting as that used in the anchors are used in this response.

### 360° video specific decoding tools

#### Geometry padding of the reference pictures

Existing video codecs are designed only considering conventional 2D video captured on a plane. When motion compensated prediction refers to any samples outside of a reference picture’s boundaries, padding will be performed by simply copying the sample values from the picture boundaries. However, the 360º video encompasses information on a whole sphere, and thus intrinsically has a cyclic property. When considering this cyclic property, the reference pictures of a 360º video no longer have “boundaries”, as the information they contain is all wrapped around a sphere. This cyclic property holds regardless of which projection format and frame packing scheme are used in the coding projection format. Considering this very specific 360º video property, geometry padding of reference pictures was previously proposed in [9], which introduces a different padding method for motion compensated prediction in 360º video coding. In this response, the same geometry padding scheme is proposed.

In the proposed geometry padding scheme, face-based padding is applied to the reference pictures.  In motion compensated prediction, when the reference sample is outside of the face that the current block belongs to, padding according to the coding projection format is applied to derive the value of the reference sample. Considering the example given in Figure 15, in geometry padding, if block B0 is located inside face F0 (front face) and its reference block B0’ is partially outside of face F0, the samples in the shaded region are not derived from samples in face F1, which is the neighboring face in the frame-packed picture, but derived from samples in face F3, which is the neighboring face on the sphere.

Figure 15. Motion compensated prediction for the pictured coded in HAC 3×2 frame packing

Figure 16 shows how geometry padding is performed for a given face of a reference picture. Point P is on face F1, but outside of face F1’s boundaries. Thus, it needs to be padded. Point O is the center of the sphere. R is the left boundary point closest to P and is inside face F1. Point Q is the projection point of point P on face F2 from the center point O. Rather than using sample value at point R to fill the sample value at point P (which is referred to as repetitive padding), geometry padding uses the sample value at point Q to fill the sample value at point P.

Figure 16. Geometry padding for CMP. Point P is outside of face F1 and needs to be padded. Point Q is the projected point on face F2 of point P from the center of the sphere (point O). The sample value at point P will be filled with the sample value at point Q on face F2

When point Q is located at a fractional position, bilinear interpolation is used in this response to derive the padded value of P. In the current software implementation, geometry padding is performed on the reference pictures and the padded reference pictures are stored in the memory. In practical decoder implementation, geometry padding can be performed on-the-fly; in other words, padding can be performed only when necessary, that is, when reference samples outside of a face boundary are used in motion compensated prediction.

Figure 17 shows examples of a padded reference picture using the repetitive padding method and using the proposed geometry padding method. Compared to the repetitive padding method currently used in the JEM, the proposed geometry padding can provide more meaningful reference samples for areas outside of the face boundaries. In this response, the padding size is set to 144 (that is, 128 + 16) luma samples on each side of the face.

(a)

(b)

Figure 17. Extended picture for HAC 3×2 frame packing using repetitive padding (a) and the proposed geometry padding (b). Padded areas are outside of the red boxes

Using the proposed geometry padding method, slight modification to the motion vector clipping process is made in this response. In the JEM, each motion vector is clipped such that the corresponding reference CU or sub-block may not be outside of the padded picture. In this response, each motion vector is clipped such that the corresponding reference CU or sub-block may not be outside of the padded face to which it belongs. Further, unlike in the JEM, where motion vector clipping is based on the CTU coordinates, in this response, each motion vector is clipped based on the CU or sub-block coordinates. Compared to the CTU based MV clipping, the CU or sub-block based MV clipping allows the use of larger-magnitude MVs that will access more padded areas on the padded faces. For a given padding size, this allows the codec to maximize the usage of the padded areas, therefore improving compression efficiency.

#### Face discontinuity handling

For projection formats composed of a plurality of faces, no matter what kind of compact frame packing arrangement is used, discontinuities appear between two or more adjacent faces in the frame packed picture. For example, considering the 3×2 frame packing configuration depicted in Figure 18, the three faces in the top half are horizontally continuous in the 3D geometry, the three faces in the bottom half are horizontally continuous in the 3D geometry, but the top and bottom halves of the frame packed picture are discontinuous in the 3D geometry. In this response, face discontinuity is defined as a face boundary on the frame packed picture where samples on two sides of the face boundary are not neighboring samples in the 3D geometry. For 360° video, because of these discontinuities, a neighboring block in the frame packed picture may not be its neighbor in the 3D geometry. In this response, blocks that are neighbors of the current block in the frame packed picture are referred to as “frame packed neighbors,” whereas blocks that are neighbors of the current block in the 3D geometry are referred to as “spherical neighbors.” Considering the example depicted in Figure 18, block D is the frame packed neighbor located above block A, and block E is the spherical neighbor located “above” block A. Due to the inherent discontinuity between D and A, information from D may not be appropriate to be used to code A, and may degrade coding efficiency and even introduce artifact. For example, if improper neighboring samples are used in in-loop filters, face seam artifacts may become visible in the reconstructed video. In this response, information from a frame packed neighbor block is used to code the current block only when the frame packed neighbor block is also the spherical neighbor block of the current block. For the example depicted in Figure 18, only block B is both a frame packed and spherical neighbor with respect to block A, whereas blocks C and D are frame packed neighbors to block A, but not spherical neighbors of block A. Therefore, only block B is used to code block A.

Figure 18. Current block (A) and neighboring blocks: frame packed (B, C, D) and spherical (B, E) neighbors

The following processes are modified to handle face discontinuities.

- Intra prediction

For intra prediction (DC, planar, and angular modes, as well as cross-component linear model), the use of reference samples is modified as follows. If a CU does not cross a face discontinuity, then it is determined whether each of the available reference samples is located on the same side of the face discontinuity to which the current CU belongs, and only those available reference samples that are located on the same side of the face discontinuity are used in intra prediction.

- Most probable modes

For most probable modes (MPM) in intra prediction, the construction of the MPM list is modified as follows. If a CU does not cross a face discontinuity, then it is determined if each available spatial neighbor in the frame packed picture is also a spherical neighbor, and only the available frame packed neighbors that are also spherical neighbors are used to build the MPM list.

- Decoder-side intra mode derivation

For the proposed DIMD method described in 2.1.8.1, determination of the template availability is modified as follows. If a CU does not cross a face discontinuity, a template is used only if all its samples and all the references samples used to predict the template are located on the same side of the face discontinuity to which the current CU belongs.

- Motion vector prediction

For motion vector prediction (MVP), the construction of the MVP candidate list is modified as follows. If a CU does not cross a face discontinuity, then it is determined if each available spatial neighbor in the frame packed picture is also a spherical neighbor, and only the available frame packed neighbors that are also spherical neighbors are used to build the MVP candidate list.

- Merge mode

For merge mode, the construction of the merge candidate list is modified as follows. If a CU does not cross a face discontinuity, then it is determined if each available spatial neighbor in the frame packed picture is also a spherical neighbor, and only the available frame packed neighbors that are also spherical neighbors are used to build the merge candidate list.

- Frame-rate up conversion

For FRUC template mode, determination of template availability is modified as follows. If a CU does not cross a face discontinuity, a template is considered available only if all its samples are located on the same side of the face discontinuity to which the current CU belongs.

- Local illumination compensation

The LIC process is modified as follows. If the CU crosses a face boundary (whether continuous or discontinuous), LIC is disabled (the flag is set to 0 but still signaled). When LIC is enabled (i.e. the CU does not cross a face boundary), a template is used only if all its samples are located on the same side of the face discontinuity to which the current CU belongs.

- Overlapped block motion compensation

To reduce face seam artifacts, the OBMC process is modified at all face boundaries (whether continuous or discontinuous). Only the portions of the top and left CU boundaries that are within the same face as the CU’s top left corner are adjusted. In the example given in Figure 19, a horizontal face boundary is crossing the current CU. The top CU boundary is adjusted. For the left CU boundary, the part located above the face boundary is adjusted, but the part located below the face boundary is not adjusted. Additionally, a CU boundary is adjusted only if it is not aligned with a face boundary. For the sub-block adjustment within a CU, a sub-block boundary is adjusted only if it is not aligned with a face boundary and not crossed by a face boundary.

Figure 19. OBMC near a face boundary

- Deblocking filter

To reduce face seam artifacts, deblocking is only applied where, considering the largest filter, all samples used in the filtering process are located on the same side of a face discontinuity.

- Sample adaptive offset

To reduce face seam artifacts, in SAO edge offset mode, only the samples that are located on the same side of the face discontinuity to which the current sample belongs are used for filtering the current sample.

- Adaptive loop filter

To reduce face seam artifacts, ALF is only applied where, considering the largest filter, all samples used in the filtering process are located on the same side of a face discontinuity.

#### Post-filtering

Padded ERP (PERP) is used in the HM and JEM anchors to reduce the face seam artifacts in reconstructed viewports that encompass the left and right boundaries of the ERP picture. In PERP, padding of redundant samples is applied on the left and the right sides of the picture. The PERP pictures containing those redundant padded samples are encoded. After decoding, the reconstructed PERP pictures are converted back to ERP by blending the redundant samples. Similar idea of padding (pre-processing) and blending (post-processing) are also used in some other projection formats. However, this padding method increases the number of coded samples and consequently increases the coded bitrate. For ERP, for a resolution of 4096×2048 luma samples, if padding of 8 luma samples is applied to the left and right boundaries, respectively, then the number of coded samples is increased by 0.39%. For cubemap-like projection formats, for a face resolution of 1184×1184 luma samples using the 3×2 frame packing configuration, if padding of 8 luma samples is applied to the boundaries of each face row (assuming that the faces are continuous within each face row), then the number of coded samples is increased by 1.81%. Additionally, as reported in [10], padding is not sufficient to completely remove face seam artifacts.

In this response, instead of padding the discontinuous boundaries with additional redundant samples, encoding the increased number of samples, and blending the padded regions at the decoder, it is proposed to encode at the original coding resolution without increasing number of coded samples, and then apply post filtering to the discontinuous face boundaries using the coded samples available on each side of the face boundary after decoding. Figure 20 illustrates the discontinuous face boundary filtering for the 3×2 frame packing configuration, where arrows of the same color indicate the samples involved in the post filtering process. With this solution, the number of coded samples does not increase.

Figure 20. Discontinuous face boundaries processed by post-filtering

In this response, the same deblocking filter as in HEVC is applied to filter the discontinuous face boundaries. The filter is applied using a fixed boundary strength of 2. The slice level QP is used to determine the parameters tC and β. Same as in HEVC, for the luma component, the filter is applied on to 4 sample lines/columns on each side of the discontinuous face boundary, and the samples located on each side of the discontinuous face boundary are used to determine whether strong or weak filtering is applied.

Because the HAC parameters may be different between two neighboring faces, both faces located on each side of the discontinuous face boundary are first padded by 4 samples using geometry padding. Then, filtering is applied independently to each side of the discontinuous face boundary. The padded samples are used in the filtering process (e.g., when the filter is applied at a sample position at which a padded sample would contribute to the filtering result), but the padded samples themselves are not filtered.

## HDR

All SDR decoding tools developed in this proposal have been applied to HDR decoding. In addition, the proposal also includes two major HDR specific decoding tools: the in-loop reshaper and Luma-based QP prediction. At the decoder, all tools are implemented using fix-point operations. All proposed tools reside inside core codec while quantization handling at the decoder is the same as the SDR case.

### Post-processing

This proposal does not use any post-processing tools for HDR.

### Quantization handling

There is nothing special regarding Quantization handling for this proposal.

### HDR specific decoding tools

#### The In-loop Luma reshaper

An in-loop reshaper remaps (“reshapes”) Luma code values inside the coding loop for better HDR coding efficiency. Two modes are supported: default mode and adaptive mode. The default reshaper uses a pre-defined Luma mapping curve derived from the weights in the wPSNR metric (Annex D.1 in [1]). The adaptive reshaper allows adaptive adjustment to the Luma mapping curve for better HDR subjective quality. The default mode can be used to evaluate the in-loop reshaping architecture using current wPSNR metrics, while the adaptive mode enables future commercial optimization using better HVS models. For CfP submission bitstreams, adaptive mode is used for better HDR subjective quality.

The concept of the reshaper was introduced in JCTVC-ETM (exploratory test model) [12] in order to provide better coding efficiency for HDR signals. The ETM reshaper provided better coding efficiency by employing pre/post processing outside of the coding loop. The major difference introduced in this proposal is an in-loop reshaper with the advantage of not requiring any change to the existing DPB handling. All the reconstructed samples stored in the DBP buffer can be interpreted in the same way as the original input video samples. For example, for HDR-A (HLG), the DPB buffer stores 10-bit HLG video samples ready for output; for HDR-B (PQ), the DPB buffer stores 10-bit PQ video samples ready for output. The proposal does not need any further post processing or colour volume transformation after decoding for the intended HDR output.

The in-loop Luma reshaper is implemented and configured as a pair of one dimensional, 10-bit, 1024-entry mapping tables (1D-LUT). The first LUT is referred to as FwdLUT, which can be used to map an input Luma code value Yi to an altered value Yr: Yr=FwdLUT[Yi]. The second LUT is referred to as InvLUT, which can be used to map altered code value Yr for “the reconstruction of Yi” as Yi: Yi=InvLUT[Yr]. Only one of the two LUTs need to be signalled as the other one can be computed from the signaled LUT. Figure 21 shows an example of a pair of FwdLUT and InvLUT.

Figure 21. An example of FwdLUT and InvLUT

The in-loop reshaper maps the Luma code values based on the slice type received by the decoder.

For an intra slice, reshaping is performed in an explicit way. The HDR intra reconstruction process is performed in the same manner as in SDR decoding until the reconstructed samples are about to be stored into the DPB buffer. The reconstructed Yr samples are modified by the inverse reshaping process before the final intra reconstructed samples Yi are stored into the DPB buffer by Yi=InvLUT[Yr]. The simplified HDR intra slice decoding workflow is shown in Figure 22.

Figure 22. HDR Intra slice reconstruction with in-loop Luma Reshaper

For an inter slice, reshaping is implicitly performed. The decoding reconstruction process (e.g., Yr= Yres+Ypred) is modified so that reconstructed sample values are produced with the influence of the reshaper. The reconstructed samples are generated by Yr=InvLUT[Yres+FwdLUT[Ypred]]. All the other inter decoding tools are used in the same way as the SDR decoding process, such as entropy decoding, motion compensation, reference picture handling, etc. The simplified HDR inter slice decoding workflow is shown in Figure 23.

Figure 23. HDR Inter slice reconstruction with in-loop Luma Reshaper

The in-loop Luma reshaper is signaled with a parametric model for codeword distribution. The Luma channel is divided into 32 bins or intervals. The parametric model signals bin importance and pre-defines how codewords are allocated for Luma intervals based on bin importance. The typical size of the reshaper syntax in the slice header is around 30 bits. In the CfP submission bitstreams, the syntax is only sent in intra slices. At the decoder side, the FwdLUT is constructed using the received parameters and then the FwdLUT is used to compute the corresponding InvLUT. If the default reshaper is enabled, there is no slice level reshaper syntax signalling or parsing. The pre-defined default LUTs are used to process the entire video sequence.

The in-loop Luma reshaper in this proposal also includes several configurable coding tools:

- Region-of-Interest reshaping: a rectangular area in multiples of CTU size that can be specified as a region of interest (ROI), where reshaping is enabled for the coding units inside the ROI. The ROI location is signaled in the SPS and can be modified at the intra slice level if needed. To avoid boundary artifacts in the intra slice, the inverse reshaping is applied for the ROI before the loop filter.

- Luma-based Chroma QP Offset: to compensate for the Luma signal interaction with the Chroma signal, an additional Chroma QP Offset is used to adjust Chroma quantization parameters for Chroma component decoding. The Chroma QP Offset is derived as follows:

cQPO=-6*log2(FwdLUT'YCU)

where FwdLUT' is the slope (first order derivative) of the FwdLUT and YCU is the average Luma value of the CU.

- Loop Filter Optimization: If the control flag for Loop Filter optimization is enabled in the SPS, then the reconstructed Luma component Yr is first forward reshaped to apply Loop Filtering, then inverse reshaped to store Yf in the DPB. This only applies for inter slices.

Yf=InvLUT[LFFwdLUTYr]

#### Luma-based QP Prediction

Because the spatially varying quantization settings (LumaDQP) are used for HDR coding, delta QP syntax must be signalled in the bitstreams for each quantization group. The deltaQP syntax elements create a signaling cost that raises the total bitrate by some number of percentage points per earlier studies [13]. In the existing HM/JEM framework, predicted QP is derived based on the left and above spatially neighbouring QPs. While for HDR, the QP is adapted at each CU based on its Luma value. The Luma-based QP Prediction tool in the proposal is designed to reduce the signalling overhead of the delta QP syntax by minimizing the difference between the predicted QP and the coding QP. It is based on the assumption that CU average predicted luma value is very close to that of the original Luma value. After receiving the deltaQP syntax, the decoder workflow derives the actual coding QP value for a CU as shown in Figure 24:

Figure 24. QP Prediction: QP calculation

The LumaDQPLUT is the Luma Level to delta QP mapping table. In order to use the LumaDQPLUT at the decoder side, it is signalled in the SPS when Luma-based QP Prediction is enabled.

Luma-based QP prediction can be combined with neighbor-based QP prediction. Therefore, this proposal allows commercial products to implement additional adjustments based on other parameters, such as variance etc.

# Encoder algorithm description

## General algorithm description

### Rate-distortion optimization

#### Distortion metrics

This is the same as in the JEM.

#### Rate-distortion dependency models

This is the same as in the JEM.

### Partitioning structure selection

In order to reduce the encoding complexity of MTT, fast TT direction determination is applied based on RD cost from BT partition testing stage. Using the RD costs of the BT partition in each direction, the encoder determines the likelihood of horizontal TT partition and vertical TT partition being selected, and skips the direction in which the TT partition is considered unlikely to be selected. This allows the RD cost checking of the TT partition in that direction to be skipped. As an example, the likelihood checking for the horizontal TT partition is shown in Figure 25. If the horizontal BT partition is not the best partition after the horizontal BT partition is tested, then the likelihood of horizontal TT partition being selected is considered to be low, and the flag Hor_TT_Test is set to 0 to indicate that RD cost checking of the horizontal TT partition will be skipped. Otherwise, the RD cost of the horizontal BT partition will be compared with the RD cost without partition. If the RD cost of the horizontal BT partition is much greater than the RD cost without partition (e.g. the ratio is greater than a pre-defined threshold A_TH), then the horizontal TT is considered not valuable to test, and the flag Hor_TT_Test is set to 0. In remaining cases, the likelihood of the horizontal TT partition being selected may be high and the flag Hor_TT_Test is set to 1. The same process is also applied to determined the flag Ver_TT_Test for the vertical TT partition.

After the likelihood checking for TT partition, the candidate directions for TT partition checking will be determined according to Figure 26. If both horizontal partition and vertical TT partition need to be tested (that is, both Hor_TT_Test and Ver_TT_Test are set to 1), we will compare the RD cost of horizontal BT and that of vertical BT. If the ratio of the two RD costs are smaller than a pre-defined threshold D_TH, then both directions are considered as further candidate directions for TT partition checking. Otherwise, only the direction with the smaller BT partition RD cost will be considered as the candidate direction for further TT partition checking. If only one TT partition at certain direction needs to be tested (that is, only either Hor_TT_Test or Ver_TT_Test is set to 1), it is treated as the only candidate direction for futher TT partition checking. Using this proposed candidate TT direction determination method, the encoder does not always perform RD cost checking of both TT partition directions.

Figure 25. Likelihood checking for horizontal TT partition, Hor_TT_Test flag indicates whether the encoder needs to further test horizontal TT partition or not

Figure 26. Determination of the candidate directions for TT partition testing

### Intra prediction mode estimation and type selection

This is the same as in the JEM.

### Motion estimation and motion segment selection

#### General

This is the same as in the JEM.

#### Usage of decoder-side motion refinement

This is the same as in the JEM, except the decoder specific changes discussed in Section 2.1.4.

### Mode decisions

This is the same as in the JEM.

### In-loop filtering type selection and parameter estimation

ALF parameter determination scheme at the encoder in JEM-7.0 is slightly modified in this response. For pictures in the highest and second highest temporal layers, an additional condition is added to bias against using ALF, as described in Figure 27. For pictures at the highest and second highest temporal layers, both luma and chroma ALF are disabled if the picture satisfies the following condition

| Orig_SSE≤T∙ALF_SSE | (9) |
| --- | --- |

where Orig_SSE is the sum of the squared error (SSE) between the original picture and the reconstructed picture without ALF filtering applied, and ALF_SSE is the SSE between the original picture and the reconstructed picture with ALF filtering applied, and T is a bias factor.

Figure 27. Slightly modified ALF decision applied at the encoder in this response

### Transforms and transform type selection

This is the same as in the JEM.

### Quantization and quantization type selection

This is the same as in the JEM.

### Entropy coding

This is the same as in the JEM.

### Additional tools

None.

### Additional encoder optimization

In this response, two modifications are applied to the RA configuration used by the anchors [1] :

- The total number of temporal layers is reduced from 5 to 3. Specifically, the pictures at the lowest three temporal layers in the RA setting used in anchor generation are merged into one temporal layer in this response; and the pictures at the 4-th temporal layer and the 5-th temporal layer in the JEM-7.0 RA setting remain the same and become the 2nd temporal layer and the 3rd temporal layer, respectively, in the new RA configuration.

- The number of reference pictures used for the pictures whose POCs are a multiple of 16 is increased from 2 to 3 by including one additional reference picture with (POC-20) for temporal prediction. Specifically, the previous coded pictures with (POC-16), (POC-20), (POC-24) and (POC-32) are included in the reference picture lists of those pictures.

Note that in this response, the maximum DPB size (i.e., 6 pictures) and the maximum picture reordering latency (i.e., 16 pictures) are kept the same as those of the RA configuration of the anchors.

### Additional algorithmic description topic(s)

None.

## 360°

### Usage of projection format adaptation

#### HAC parameter determination

Taking into account the constraints in (7)(8), eight parameters need to be determined for the 3×2 HAC projection format used in this response, including six parameters in the horizontal direction, and two parameters in the vertical direction.

The HAC parameter derivation process is separated into two sub-processes: (a) deriving parameters for the first face row consisting of F4, F0 and F5; (b) deriving parameters for the second face row consisting of F2, F1 and F3. For each face row, the following iterative parameters searching algorithm for horizontal and vertical mapping functions is applied:

Step 1:	search parameters for the horizontal direction while fixing parameters for the vertical direction, and update the parameters for the horizontal direction with the optimal parameters found in the search; if there is no update, the search process stops; otherwise, go to Step 2;

Step 2:	search parameters for the vertical direction while fixing parameters for the horizontal direction, and update the parameters for the vertical direction with the optimal parameters found in the search; if there is no update, the search process stops; otherwise, go to Step 1.

The search algorithm usually converges after 3~4 iterations. In Step 1 and Step 2, the HAC parameters are selected by minimizing end-to-end weighted conversion-only error calculated as in Equation (10), where wSSE(x, y) is the weighted SSE calculation using sample set x and y, and the weights are the same as those used in WS-PSNR calculation for ERP in 360Lib.

| Dist=Fi∈face_rowwSSE(Fi(ERP), Fi(ERP')) | (10) |
| --- | --- |

where Fi(ERP) is the portion of source ERP corresponding to face Fi in HAC, and Fi(ERP') is the portion of reconstructed ERP corresponding to face Fi in HAC.

In this response, only projection format conversion distortion is considered. The HAC parameters are selected by the encoder using the first picture of each IRAP and signalled once per IRAP.

#### Adaptive frame packing determination

Considering a 3×2 layout of the HAC projection format, there are six different combinations to select two face groups of three faces each. Using the face definitions from the 360Lib, these combinations are listed in Table 4. Considering that the top and bottom parts of the spherical video are typically less interesting and/or easier to encode, the encoder in this response considers only the combinations where three faces along the equators are grouped together. This reduces the problem to four different combinations (combinations 1-4 in Table 4).

Table 4. Combinations to select two face groups of three faces

| Combination | Group #1 | Group #2 |
| --- | --- | --- |
| 1 | PX, PZ, NX | PY, NZ, NY |
| 2 | PX, NZ, NX | PY, PZ, NY |
| 3 | PZ, PX, NZ | PY, NX, NY |
| 4 | PZ, NX, NZ | PY, PX, NY |
| 5 | PX, PY, NX | PZ, NY, NZ |
| 6 | PZ, PY, NZ | PX, NY, NX |

For any of these four combinations, the three equatorial faces in group #1 are placed horizontally, whereas the remaining three face in group #2 may be rotated in the frame packed picture. However, rotations of ±90° may impact coding efficiency, especially for intra coding. Indeed, a better prediction can be achieved for a pattern composed of diagonal lines, i.e., from upper left to lower right, than for a pattern composed of anti-diagonal lines, i.e., from upper right to lower left, because prediction directions close to the intra modes 2 and 66 in JEM-7.0 require reference samples located below left and above right of the current block, respectively, which are more likely to be unavailable.

To determine which rotation is optimal for a face, the horizontal, vertical, diagonal, and anti-diagonal gradients are computed on the luma component using 1-D Laplacian. Then, gradient classification is applied on an 8×8 block basis to determine the dominant gradient in each block. The gradient values are accumulated over all samples within a block. The accumulated horizontal, vertical, diagonal, and anti-diagonal gradient values, denoted gh, gv, gd, and ga, respectively, are compared to each other’s to determine the block’s dominant gradient following the flowchart depicted in Figure 28, where α is a parameter used to classify between active and inactive gradients and is set to 2.

Figure 28. Flowchart for determining the dominant gradient for an 8×8 block

For each face, a cost function is built as

| ci = wh∙gh,i+wv∙gv,i+wd∙gd,i+wa∙ga,i | (11) |
| --- | --- |

Where i is the face index, wh, wv, wd, and wa are the weights assigned to the horizontal, vertical, diagonal, and anti-diagonal dominant gradient counts gh, gv, gd, and ga, respectively. Considering that intra prediction for patterns composed of anti-diagonal lines may be less efficient than for other directions, the following set of weights are used in this response: wh=wv=wa=-1, wd=1. Then, the overall cost for each combination (see Table 4) is computed as the sum of the individual cost of each face, considering the orientation of each face.

In this response, a subset of the first four adaptive frame packing combinations shown in Table 4 is used in a rotating manner at the beginning of each IRAP. Among the four combinations, the cost of each combination is calculated using the first picture of the input video sequence. If the cost of any combination is significantly higher than that of the best combination, then that combination is not used in coding the input video sequence to avoid reducing coding efficiency. Since only the first picture in the sequence is used in the determination, this solution can be used together with parallel encoding. Without loss of generality, after analyzing the first picture, assume that all four combinations are determined to be viable, and they are ranked based on their cost as C1, C2, C3, C4. For the first picture, the best combination, i.e., C1, is used. For the subsequent IRAP pictures, two sets of combinations are formed, i.e., {C1,C2} and {C3,C4}, and the optimal combination is selected as

| Copt=arg min{C1,C2}cif t % 2T=0arg min{C3,C4}cotherwise | (12) |
| --- | --- |

where T is the IRAP period, % is the modulo operator, and c is the cost function computed for the given IRAP picture. In this response, adaptive frame packing parameters are selected by the encoder using the first picture of each IRAP and signaled once per IRAP. The adaptive frame packing determination is performed before the HAC parameters determination. This is because the vertical HAC parameter needs to be the same for all faces within a face row, and hence the frame packing arrangement needs to be known before the HAC parameters can be determined.

### 360° Video specific encoding tools

None.

### Additional encoder optimization

None.

## HDR

This proposal does not use any pre-processing for HDR. Section 3.3.2 discusses encoder-side operation for two major coding tools: the in-loop reshaper and Luma-based QP prediction. Additional HDR encoder optimizations are explained in Section 3.3.3. The first discussion is joint reshaper and LumaDQP optimization, and the second is chromaQPOffset derivation.

### Pre-processing

This proposal is entirely in-loop and therefore no pre-processing is used.

### HDR specific encoding tools

#### In-loop Luma reshaper

If the default reshaper is enabled, there is no derivation process needed at the encoder side. The pre-defined LUTs are used across the coding of the entire sequence. Otherwise, an adaptive reshaper derivation process is invoked. When coding an Intra slice, the adaptive reshaper derivation process consists of the following steps:

1) Perform picture analysis to generate the parametric reshaping model which signals bin importance;

2) Construct the forward reshaping LUT (FwdLUT) based on pre-defined rules describing how codewords are allocated for bins, relative to the bin importance;

3) Compute the inverse reshaping LUT (InvLUT) from FwdLUT. Details about FwdLUT derivation can be found in [14].

The reshaper derivation process does not require any picture look-ahead and therefore will not introduce extra delay.

The in-loop Luma reshaper does not require any multi-pass encoding or extra rate-distortion optimization over and above those required by SDR coding tools. For Intra slices, forward reshaping is applied on Luma samples before Intra coding starts. During mode decision and rate-distortion optimization, the reshaped Luma sample values are used in the distortion computation. The remainder of the Intra coding workflow is like the decoder-side workflow described in Section 2.3.3.1. For Inter slices, the input Inter pictures start block Inter/Intra prediction as a normal encoder would until such time the residue is generated. In this proposal, the residue is generated by Yres=FwdLUT[Yi]-FwdLUT[Ypred] instead of the traditional method where the residue is the direct difference between the input and prediction (e.g., Yres=Yi-Ypred). The residue produced by using this method is implicitly reshaped due to the mathematical operations occurring in the reshaped domain. The remainder of the Inter coding workflow is like the decoder-side workflow described in Section 2.3.3.1.

#### Luma-based QP Prediction

The deltaQP value to be signaled in bitstream is derived as follows: for each coding unit, get the actual CodingQP = SliceQP – LumaDQPLUT[Yorg], where Yorg is the average original input Luma sample value for the CU; then calculate the PredQP = SliceQP – LumaDQPLUT[Ypred] where Ypred is the average predicted Luma sample value of the CU. The deltaQP to be signalled is calculated as deltaQP = CodingQP – PredQP = LumaDQPLUT[Ypred] – LumaDQPLUT[Yorg]. The LumaDQPLUT is signaled in the SPS for decoding.

### Additional encoder optimization

#### Joint optimization of reshaper and luma QP Adaptation

Additional subjective gain can be achieved by joint optimization of the adaptive reshaper and the Luma dependent adaptive quantization inside the codec (lumaDQP) [15]. In this proposal, a spatially varying quantization setting was applied for the generation of the HDR-B bitstreams, where the variation was an explicit function of the average local Luma value for a given block. The method is the same as the Luma-based quantization adjustment used to generate the HDR-B Anchors.

HDR-B Anchors use weighted SSE distortion computation and use a pre-determined Luma dependent weight (Annex D.1 in [1]) in RD decision for all components. In this proposal, SSE distortion is used for Intra slices. Weighted SSE distortion is used for Inter slices but with different weights derived from the reshaping LUTs. The weighting computation consists of two parts: weights from Luma delta QP: wQPY=2(dQP[Y]3)  and weights from Luma reshaping: wrspY=(FwdLUT'[Y])2, where FwdLUT' is the slope (first order derivative) of the FwdLUT. The total weights are calculated as wY=Clip3 (0, TH, wQPY*wrspY). TH is an empirically chosen threshold to clip the weight values into a reasonable range. The weights are updated when an adaptive reshaping LUT is updated (at each Intra slice).

#### Chroma QP offset adjustment

For HDR-A (HLG content), Chroma QP Offset adjustment method is used based on the technical report [11]. The model in the TR is expressed as:

QPoffsetCb = Clip3(-12,0,Round(c_cb  * (k * QP+l)))

QpoffsetCr = Clip3(-12,0,Round(c_cr  * (k * QP+l)))

The configuration for HDR-A is shown as follows.

WCGPPSEnable             : 1

WCGPPSChromaQpScale      : -0.2      # k

WCGPPSChromaQpOffset     : 7        # l

WCGPPSCbQpScale          : 1.00     # c_cb

WCGPPSCrQpScale          : 1.00     # c_cr

For HDR-B (PQ content), the model and parameters are exactly the same as those being used in generating the HDR-B Anchors.

# Compression performance

The compression performance is reported using the test conditions and the measurement metrics as required by the CfP [1][2]. The actual bit rates of all bitstreams satisfy the CfP requirement that they shall not exceed the target bit rates defined in [1][2]. Summary of compression performance of the response is discussed in this section. Readers are referred to the accompanying spreadsheets for further details.

## SDR: Constraint set 1 configuration relative to HM anchor

### Class SDR-A (RD-curves)

Figure 29 compares the luma rate-distortion (RD) curves of the response and the HM anchor for 5 Class SDR-A sequences under the constraint set 1 configuration. The response significantly outperforms the HM anchor with an average BD-PSNR gain of 1.38 dB.

Figure 29. RD-curves of the response and the HM anchor for the Class SDR-A sequences under constraint set 1 configuration

### Class SDR-B (RD-curves)

Figure 30 compares the luma RD curves of the response and the HM anchor for 5 Class SDR-B sequences under the constraint set 1 configuration. The response significantly outperforms the HM anchor with an average BD-PSNR gain of 1.30 dB.

Figure 30. RD-curves of the response and the HM anchor for the Class SDR-B sequences under constraint set 1 configuration

### Overall (BD-rate summary tables)

Table 5 shows the average BD-rate improvement of the response compared to the HM anchor under the constraint set 1 configuration. For the SDR-A sequences, the average BD-rate savings for {Y, U, V} components of the response are {38.42%, 43.95%, 43.90%}. For the SDR-B sequences, the average BD-rate savings for {Y, U, V} components are {33.01%, 45.55%, 46.01%}. The maximum luma BD-rate gain is 42.87% achieved by the sequence “DaylightRoad2.”

Table 5. Comparison of the response with the HM anchor under the constraint set 1 configuration

|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| --- | --- | --- | --- |
| FoodMarket4 | -35.70% | -42.16% | -46.22% |
| CatRobot1 | -41.50% | -55.96% | -48.31% |
| DaylightRoad2 | -42.87% | -60.19% | -44.56% |
| ParkRunning3 | -34.88% | -25.66% | -28.09% |
| Campfire | -37.12% | -35.77% | -52.31% |
| BQTerrace | -33.45% | -47.24% | -55.88% |
| RitualDance | -29.80% | -38.73% | -42.22% |
| MarketPlace | -30.98% | -46.73% | -44.35% |
| BasketballDrive | -33.75% | -47.48% | -44.34% |
| Cactus | -37.09% | -47.55% | -43.25% |
| Average SDR-A | -38.42% | -43.95% | -43.90% |
| Average SDR-B | -33.01% | -45.55% | -46.01% |
| Average all | -35.72% | -44.75% | -44.95% |
| Enc Time[%] | 1710% |  |  |
| Dec Time[%] | 263% |  |  |

## SDR: Constraint set 2 configuration relative to HM anchor

### Class SDR-B

Figure 31 compares the luma RD curves of the response and the HM anchor for 5 Class SDR-B sequences under the constraint set 2 configuration. The response significantly outperforms the HM anchor with an average BD-PSNR gain of 1.04 dB.

Figure 31. RD-curves of the response and the HM anchor for the Class SDR-B sequences under constraint set 2 configuration

### Overall

Table 6 shows the average BD-rate improvement of the response compared to the HM anchor under the constraint set 2 configuration. The average BD-rate savings for {Y, U, V} components are {27.18%, 43.18%, 44.15%}. The maximum luma BD-rate gain is 30.80% achieved by the sequence “Cactus.”

Table 6. Comparison of the response with the HM anchor under the constraint set 2 configuration

|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| --- | --- | --- | --- |
| BQTerrace | -28.90% | -48.18% | -55.50% |
| RitualDance | -24.32% | -36.67% | -38.15% |
| MarketPlace | -23.62% | -42.22% | -41.74% |
| BasketballDrive | -28.26% | -48.89% | -46.29% |
| Cactus | -30.80% | -43.09% | -40.88% |
| Average SDR-B | -27.18% | -43.81% | -44.51% |
| Enc Time[%] | 1827% |  |  |
| Dec Time[%] | 301% |  |  |

## SDR: Constraint set 1 configuration relative to JEM anchor

### Class SDR-A

Figure 32 compares the luma RD curves of the response and the JEM anchor for 5 Class SDR-A sequences under the constraint set 1 configuration. As can be seen in Figure 32, the response provides consistent coding improvement compared the JEM anchor. The average BD-PSNR gain is 0.11 dB.

Figure 32. RD-curves of the response and the JEM anchor for the Class SDR-A sequences under constraint set 1 configuration

### Class SDR-B

Figure 33 compares the luma RD curves of the response and the JEM anchor for 5 Class SDR-B sequences under the constraint set 1 configuration. As can be seen in Figure 33, the response provides consistent coding improvements compared the JEM anchor. The average BD-PSNR gain is 0.11 dB.

Figure 33. RD-curves of the response and the JEM anchor for the Class SDR-B sequences under constraint set 1 configuration

### Overall

Table 7 shows the average BD-rate improvement of the response compared to the JEM anchor under the constraint set 1 configuration. For the SDR-A sequences, the average BD-rate savings for {Y, U, V} components of the response are {4.38%, 3.46%, 3.45%}. For the SDR-B sequences, the average BD-rates savings for {Y, U, V} components are {3.58%, 3.11%, 2.87%}. The maximum luma BD-rate gain is 5.59% achieved by the sequence “ParkRunning3.”

Table 7. Comparison of the response with the JEM anchor under the constraint set 1 configuration

|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| --- | --- | --- | --- |
| FoodMarket4 | -3.71% | 2.53% | 0.80% |
| CatRobot1 | -3.78% | -8.89% | -5.49% |
| DaylightRoad2 | -4.40% | -8.40% | -5.87% |
| ParkRunning3 | -5.59% | -0.74% | -1.01% |
| Campfire | -4.44% | -1.81% | -5.68% |
| BQTerrace | -4.49% | -4.26% | -5.13% |
| RitualDance | -3.49% | -2.85% | -2.85% |
| MarketPlace | -2.65% | 0.01% | -0.63% |
| BasketballDrive | -3.88% | -3.79% | -1.78% |
| Cactus | -3.40% | -4.64% | -3.97% |
| Average SDR-A | -4.38% | -3.46% | -3.45% |
| Average SDR-B | -3.58% | -3.11% | -2.87% |
| Average all | -3.98% | -3.28% | -3.16% |
| Enc Time[%] | 205% |  |  |
| Dec Time[%] | 33% |  |  |

## SDR: Constraint set 2 configuration relative to JEM anchor

### Class SDR-B

Figure 34 compares the luma RD curves of the response and the JEM anchor for 5 Class SDR-B sequences under the constraint set 2 configuration. As can be seen in Figure 34, the response provides consistent coding improvement compared the JEM anchor. The average BD-PSNR gain is 0.12 dB.

Figure 34. RD-curves of the response and the JEM anchor for the Class SDR-B sequences under constraint set 2 configuration

### Overall

Table 8 shows the average BD-rate improvement of the response compared to the JEM anchor under the constraint set 2 configuration. The average BD-rate savings for {Y, U, V} components are {3.64%, 2.55%, 4.48%}. The maximum luma BD-rate gain is 3.78% achieved by the sequence “BQTerrace.”

Table 8. Comparison of the response with the JEM anchor under the constraint set 2 configuration

|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| --- | --- | --- | --- |
| BQTerrace | -3.78% | -2.48% | -9.80% |
| RitualDance | -3.38% | -3.60% | -4.24% |
| MarketPlace | -3.74% | 0.40% | -1.74% |
| BasketballDrive | -3.65% | -5.92% | -4.39% |
| Cactus | -3.65% | -1.15% | -2.22% |
| Average SDR-B | -3.64% | -2.55% | -4.48% |
| Enc Time[%] | 203% |  |  |
| Dec Time[%] | 45% |  |  |

## HDR: Constraint set 1 configuration relative to HM anchor

HDR Constraint set 1 bitstreams are generated under target bitrates for each rate point. The Random-Access configuration similar to the one used for generating SDR bitstreams in this proposal is also used for HDR. Both HDR specific coding tools and all SDR coding tools included in this proposal have been used for the HDR coding. Section 4.5.1 to 4.5.3 report the objective metrics relative to HM anchor, and Section 4.6.1 to 4.6.3 report the objective metrics relative to JEM anchor.

### Class HDR-A

Table 9. Comparison of the response with the HM anchor under the HDR-A constraint set 1

|  | BD-rate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | L100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| DayStreet | -59.01% | -34.56% | -26.84% | -58.22% | -71.51% | -31.11% | -56.84% | -69.39% |
| PeopleInShoppingCenter | -51.69% | -27.61% | -23.21% | -62.17% | -58.35% | -24.43% | -59.79% | -58.01% |
| SunsetBeach | -60.36% | -28.11% | -22.67% | -88.27% | -87.96% | -23.49% | -81.92% | -88.44% |
| Average HDR-A | -57.02% | -30.09% | -24.24% | -69.55% | -72.60% | -26.34% | -66.18% | -71.95% |
| Enc Time[%] | 1272% |  |  |  |  |  |  |  |
| Dec Time[%] | 277% |  |  |  |  |  |  |  |

### Class HDR-B

Table 10. Comparison of the response with the HM anchor under the HDR-B constraint set 1

|  | BD-rate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | L100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | -29.73% | -28.41% | -24.29% | -31.06% | -24.24% | -11.85% | -30.70% | -21.95% |
| ShowGirl2 | -25.12% | -23.65% | -16.79% | -50.05% | -22.84% | -4.78% | -35.91% | -9.70% |
| Hurdles | -42.31% | -39.78% | -33.14% | -54.87% | -47.38% | -20.94% | -45.32% | -38.59% |
| Starting | -38.70% | -30.31% | -24.60% | -26.68% | -30.75% | -20.20% | -27.03% | -23.31% |
| Cosmos1 | -32.81% | -30.04% | -17.67% | -61.25% | -76.56% | -8.49% | -56.43% | -72.87% |
| Average HDR-B | -33.73% | -30.44% | -23.30% | -44.78% | -40.35% | -13.25% | -39.08% | -33.28% |
| Enc Time[%] | 1589% |  |  |  |  |  |  |  |
| Dec Time[%] | 301% |  |  |  |  |  |  |  |

### Overall

Table 11. Comparison of the response with the HM anchor under the HDR constraint set 1

|  | BD-rate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | L100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Average all | -42.47% | -30.31% | -23.65% | -54.07% | -52.45% | -18.16% | -49.24% | -47.78% |
| Enc Time[%] | 1462% |  |  |  |  |  |  |  |
| Dec Time[%] | 292% |  |  |  |  |  |  |  |

## HDR: Constraint set 1 configuration relative to JEM anchor

### Class HDR-A

Table 12. Comparison of the response with the JEM anchor under the HDR-A constraint set 1

|  | BD-rate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | L100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| DayStreet | -2.01% | 2.91% | 13.51% | -7.15% | -8.60% | 4.82% | -7.05% | -7.43% |
| PeopleInShoppingCenter | -3.72% | 2.55% | 7.15% | -12.94% | -10.98% | 5.13% | -11.03% | -11.20% |
| SunsetBeach | -9.96% | -2.71% | -2.01% | -14.95% | -22.84% | -1.86% | -11.93% | -22.84% |
| Average HDR-A | -5.23% | 0.91% | 6.21% | -11.68% | -14.14% | 2.70% | -10.01% | -13.82% |
| Enc Time[%] | 160% |  |  |  |  |  |  |  |
| Dec Time[%] | 30% |  |  |  |  |  |  |  |

### Class HDR-B

Table 13. Comparison of the response with the JEM anchor under the HDR-B constraint set 1

|  | BD-rate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | L100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | 15.67% | -5.33% | 0.99% | 27.24% | 36.84% | 15.74% | 13.23% | 19.00% |
| ShowGirl2 | 4.00% | 3.79% | 14.96% | 27.47% | 15.41% | 28.23% | 58.51% | 29.87% |
| Hurdles | 13.94% | -8.09% | 0.66% | 17.39% | 18.87% | 14.55% | 25.06% | 28.64% |
| Starting | -7.70% | 0.75% | 8.58% | 12.37% | -0.71% | 12.45% | 7.40% | -4.22% |
| Cosmos1 | -3.49% | -7.84% | 8.80% | 14.77% | 1.21% | 18.53% | 26.96% | 20.02% |
| Average HDR-B | 4.48% | -3.34% | 6.80% | 19.85% | 14.32% | 17.90% | 26.23% | 18.66% |
| Enc Time[%] | 199% |  |  |  |  |  |  |  |
| Dec Time[%] | 35% |  |  |  |  |  |  |  |

### Overall

Table 14. Comparison of the response with the JEM anchor under the HDR constraint set 1

|  | BD-rate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | L100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Average all | 0.84% | -1.75% | 6.58% | 8.02% | 3.65% | 12.20% | 12.64% | 6.48% |
| Enc Time[%] | 183% |  |  |  |  |  |  |  |
| Dec Time[%] | 33% |  |  |  |  |  |  |  |

## 360° Video: Constraint set 1 configuration relative to HM anchor

### Class 360

Figure 35 shows the RD curves for the response and the HM anchor for the 5 class 360 test sequences considering the E2E WS-PSNR metric computed on the luma component. The response outperforms the HM anchor by 1.00 dB to 2.02 dB, with an average E2E WS-PSNR gain of 1.44 dB.

Figure 35. RD-curves reporting the luma E2E WS-PSNR of the response and the HM anchor

### Overall

Table 15 reports the BD-rate of the response compared to the HM anchor. The response can effectively improve the coding performance of HEVC by providing an average BD-rate savings of 33.87%, 54.04%, and 56.79% for the Y, U, and V components, respectively, based on the E2E WS-PSNR metric. The maximum BD-rate savings of the response can reach 47.19% (sequence Chairlift). Larger BD-rate savings are observed for sequences recorded using a moving camera than for sequences recorded using a static camera.

Table 15. Comparison of the response with the HM anchor in terms of coding performance

|  | E2E WS-PSNR |  |  | E2E SPSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Balboa | -43.85% | -53.01% | -57.54% | -43.90% | -53.05% | -57.56% |
| Chairlift | -47.19% | -62.52% | -60.10% | -47.17% | -62.54% | -60.08% |
| KiteFlite | -25.41% | -55.89% | -61.71% | -25.51% | -55.92% | -61.76% |
| Harbor | -28.92% | -56.21% | -56.47% | -28.78% | -56.21% | -56.44% |
| Trolley | -23.97% | -42.56% | -48.14% | -24.00% | -42.60% | -48.09% |
| Average 360 | -33.87% | -54.04% | -56.79% | -33.87% | -54.06% | -56.78% |
| Enc Time[%] | 1332% |  |  |  |  |  |
| Dec Time[%] | 350% |  |  |  |  |  |

## 360° Video: Constraint set 1 configuration relative to JEM anchor

### Class 360

Figure 36 shows the RD curves for the response and the JEM anchor for the 5 class 360 sequences considering the E2E WS-PSNR metric computed on the luma component. The response outperforms the JEM anchor by 0.29dB to 0.91dB, with an average E2E WS-PSNR gain of 0.50 dB.

Figure 36. RD-curves reporting the E2E WS-PSNR of the response and the JEM anchor

### Overall

Table 16 reports the BD-rate of the response compared to the JEM anchor. The response can effectively improve the coding performance of the JEM by providing an average BD-rate savings of 13.51%, 18.29%, and 20.96% for the Y, U, and V components, respectively, based on the E2E WS-PSNR metric. The maximum BD rate savings of the response can reach 23.98% (sequence Chairlift). Larger BD rate savings are observed for sequences recorded using a moving camera than for sequences recorded using a static camera.

Table 16. Comparison of the response with the JEM anchor in terms of coding performance

|  | E2E WS-PSNR |  |  | E2E SPSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Balboa | -13.49% | -18.51% | -19.80% | -13.55% | -18.52% | -19.82% |
| Chairlift | -23.98% | -25.78% | -28.57% | -23.98% | -25.80% | -28.64% |
| KiteFlite | -9.61% | -15.82% | -20.15% | -9.70% | -15.83% | -20.24% |
| Harbor | -10.82% | -20.05% | -19.52% | -10.75% | -20.13% | -19.65% |
| Trolley | -9.65% | -11.30% | -16.76% | -9.64% | -11.25% | -16.77% |
| Average 360 | -13.51% | -18.29% | -20.96% | -13.52% | -18.31% | -21.03% |
| Enc Time[%] | 241% |  |  |  |  |  |
| Dec Time[%] | 57% |  |  |  |  |  |

# Decoder complexity analysis

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

Table 17 to Table 22 report the relative decoding time for each sequence and rate point when compared to the HM and JEM anchors for the different sequence classes and constraint sets. The decoding time of the anchors and the response was measured using the PC workstations as specified in Section 5.2. The decoding time was measured with YUV output enabled and reference input disabled. For the SDR category, compared to the HM anchor, the average decoding time of the response is 263% and 301% for the constraint set 1 and the constraint set 2, respectively; compared to the JEM anchor, the average decoding time of the response is 33% and 45% for the constraint set 1 and the constraint set 2, respectively. For the 360° category, the average decoding time of the response is 350% and 57% of that of the HM and JEM anchors, respectively. For the HDR category, compared to the HM anchor, the average decoding time of the response is 292% for the constraint set 1; compared to the JEM anchor, the average decoding time of the response is 33% for the constraint set 1.

Table 17. Relative decoding time of each sequence and rate point for class SDR-A constraint set 1

|  | Relative decoding time |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | vs. HM anchor |  |  |  | vs. JEM anchor |  |  |  |
| Sequence | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| FoodMarket4 | 232% | 236% | 232% | 227% | 27% | 28% | 29% | 32% |
| CatRobot1 | 257% | 284% | 274% | 251% | 27% | 28% | 28% | 29% |
| DaylightRoad2 | 231% | 241% | 251% | 257% | 30% | 31% | 33% | 35% |
| ParkRunning3 | 267% | 262% | 267% | 278% | 33% | 32% | 33% | 35% |
| Campfire | 238% | 234% | 234% | 244% | 31% | 35% | 38% | 43% |
| Average | 249% |  |  |  | 32% |  |  |  |

Table 18. Relative decoding time of each sequence and rate point for class SDR-B constraint set 1

|  | Relative decoding time |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | vs. HM anchor |  |  |  | vs. JEM anchor |  |  |  |
| Sequence | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| BQTerrace | 203% | 218% | 242% | 241% | 26% | 28% | 32% | 33% |
| RitualDance | 299% | 276% | 281% | 274% | 37% | 39% | 41% | 44% |
| MarketPlace | 277% | 277% | 288% | 272% | 31% | 33% | 35% | 36% |
| BasketballDrive | 327% | 316% | 300% | 304% | 36% | 38% | 41% | 44% |
| Cactus | 306% | 305% | 307% | 275% | 30% | 31% | 32% | 32% |
| Average | 277% |  |  |  | 35% |  |  |  |

Table 19. Relative decoding time of each sequence and rate point for class SDR-B constraint set 2

|  | Relative decoding time |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | vs. HM anchor |  |  |  | vs. JEM anchor |  |  |  |
| Sequence | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| BQTerrace | 211% | 232% | 238% | 246% | 39% | 45% | 46% | 49% |
| RitualDance | 295% | 294% | 307% | 315% | 44% | 46% | 50% | 52% |
| MarketPlace | 283% | 296% | 306% | 311% | 41% | 41% | 44% | 46% |
| BasketballDrive | 309% | 313% | 322% | 333% | 44% | 47% | 49% | 54% |
| Cactus | 383% | 375% | 375% | 355% | 39% | 41% | 45% | 48% |
| Average | 301% |  |  |  | 45% |  |  |  |

Table 20. Relative decoding time of each sequence and rate point for class HDR-A constraint set 1

|  | Relative decoding time |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | vs. HM anchor |  |  |  | vs. JEM anchor |  |  |  |
| Sequence | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| DayStreet | 285% | 275% | 276% | 273% | 29% | 27% | 27% | 25% |
| PeopleInShoppingCenter | 231% | 228% | 243% | 242% | 30% | 28% | 28% | 26% |
| SunsetBeach | 333% | 326% | 322% | 324% | 43% | 37% | 33% | 27% |
| Average | 277% |  |  |  | 30% |  |  |  |

Table 21. Relative decoding time of each sequence and rate point for class HDR-B constraint set 1

|  | Relative decoding time |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | vs. HM anchor |  |  |  | vs. JEM anchor |  |  |  |
| Sequence | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| Market3 | 287% | 263% | 259% | 252% | 39% | 34% | 29% | 28% |
| ShowGirl2 | 302% | 295% | 296% | 300% | 42% | 39% | 37% | 36% |
| Hurdles | 336% | 325% | 316% | 307% | 36% | 34% | 32% | 30% |
| Starting | 294% | 269% | 251% | 244% | 35% | 30% | 26% | 25% |
| Cosmos1 | 391% | 386% | 364% | 347% | 53% | 48% | 45% | 42% |
| Average | 301% |  |  |  | 35% |  |  |  |

Table 22. Relative decoding time of each sequence and rate point for class 360 constraint set 1

|  | Relative decoding time |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | vs. HM anchor |  |  |  | vs. JEM anchor |  |  |  |
| Sequence | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| Balboa | 376% | 404% | 389% | 414% | 50% | 55% | 53% | 55% |
| Chairlift | 333% | 331% | 355% | 391% | 47% | 48% | 50% | 56% |
| Harbor | 335% | 360% | 333% | 333% | 51% | 55% | 57% | 59% |
| KiteFlite | 343% | 339% | 333% | 336% | 55% | 57% | 61% | 65% |
| Trolley | 322% | 346% | 315% | 339% | 64% | 67% | 66% | 68% |
| Average | 350% |  |  |  | 57% |  |  |  |

## Description of computing platform used to determine decoding times reported in Section 5.1

The PC workstation as specified in Table 23 was used for measuring the decoding time for SDR and 360°, and the PC workstation as specified in Table 24 was used for measuring the decoding time for HDR. For accurate decoding time measurement, bitstreams were decoded in a serial manner.

Table 23. Technical specification of the PC workstation for SDR and 360° categories

| CPU | Intel Core i7-5960X / Clock speed 3.00GHz |
| --- | --- |
| Memory | 64 GB |
| SDD characteristics | Samsung 960 PRO -Capacity: 1TB -Interface: NVME |
| Compiler | Visual Studio 2010 |
| Operating System (OS) | Windows 7 64 bit |

Table 24. Technical specification of the PC workstation for HDR category

| CPU | Intel Core i7-3930K / Clock speed 3.20GHz |
| --- | --- |
| Memory | 64 GB |
| SDD characteristics | Intel SSD 520 Series -Capacity: 240GB -Interface: SATA 3.0 6Gb/s |
| Compiler | Visual Studio 2015 |
| Operating System (OS) | Windows 7 64 bit |

## Memory usage of decoder

Table 25 to Table 29 report the peak decoder memory usage of the response and anchors for each category and constraint set, for the sequence class of the largest resolution at the highest target rate point. The peak decoder memory usage of the anchors and the response was measured using the Linux clusters as specified in Section 6.2. In the SDR category, for the constraint set 1, the peak decoder memory usages of the HM, the JEM and the response are 0.97GB, 1.24GB and 1.19GB, respectively; for the constraint set 2, the peak decoder memory usages are 0.22GB, 0.33GB and 0.34GB for the HM, the JEM and the response, respectively. For the 360° category, the peak decoder memory usage of the response is 1.69GB, compared to 0.99GB and 1.27GB for the HM and JEM anchors, respectively. For the HDR category, the peak decoder memory usage of the response is 1.06GB, compared to 0.97GB and 1.22GB for the HM and JEM anchors, respectively.

Table 25. Peak decoder memory usage for class SDR-A constraint set 1

|  | Peak decoder memory usage [GB] |  |  |
| --- | --- | --- | --- |
| Sequence | Response | HM Anchor | JEM Anchor |
| FoodMarket4 | 1.19 | 0.97 | 1.24 |
| CatRobot1 | 1.18 | 0.97 | 1.22 |
| DaylightRoad2 | 1.19 | 0.97 | 1.22 |
| ParkRunning3 | 1.18 | 0.97 | 1.22 |
| Campfire | 1.19 | 0.97 | 1.22 |

Table 26. Peak decoder memory usage for class SDR-B constraint set 2

|  | Peak decoder memory usage [GB] |  |  |
| --- | --- | --- | --- |
| Sequence | Response | HM Anchor | JEM Anchor |
| BQTerrace | 0.34 | 0.22 | 0.33 |
| RitualDance | 0.34 | 0.22 | 0.33 |
| MarketPlace | 0.34 | 0.22 | 0.33 |
| BasketballDrive | 0.34 | 0.22 | 0.33 |
| Cactus | 0.34 | 0.22 | 0.33 |

Table 27. Peak decoder memory usage for class HDR-A constraint set 1

|  | Peak decoder memory usage [GB] |  |  |
| --- | --- | --- | --- |
| Sequence | Response | HM Anchor | JEM Anchor |
| DayStreet | 1.06 | 0.97 | 1.22 |
| PeopleInShoppingCenter | 1.06 | 0.97 | 1.22 |
| SunsetBeach | 1.06 | 0.97 | 1.22 |

Table 28. Peak decoder memory usage for class HDR-B constraint set 1

|  | Peak decoder memory usage [GB] |  |  |
| --- | --- | --- | --- |
| Sequence | Response | HM Anchor | JEM Anchor |
| Market3 | 0.39 | 0.25 | 0.37 |
| ShowGirl2 | 0.39 | 0.25 | 0.37 |
| Hurdles | 0.39 | 0.25 | 0.37 |
| Starting | 0.39 | 0.25 | 0.37 |
| Cosmos1 | 0.26 | 0.21 | 0.31 |

Table 29. Peak decoder memory usage for class 360 constraint set 1

|  | Peak decoder memory usage [GB] |  |  |
| --- | --- | --- | --- |
| Sequence | Response | HM Anchor | JEM Anchor |
| Balboa | 1.64 | 0.99 | 1.27 |
| Chairlift | 1.69 | 0.99 | 1.26 |
| Harbor | 1.69 | 0.99 | 1.26 |
| KiteFlite | 1.69 | 0.99 | 1.26 |
| Trolley | 1.69 | 0.99 | 1.26 |

## Complexity characteristics of decoder entropy decoding operation

This is the same as in the JEM.

## Complexity characteristics of decoder inverse quantization

This is the same as in the JEM.

## Complexity characteristics of decoder inverse transform operation

This is the same as in the JEM.

## Complexity characteristics of decoder-side motion refinement

As discussed in Section 2.1.4, FRUC motion search and DMVR in the JEM-7.0 have been significantly simplified in this response. The complexity of the decode-side motion refinement tools, is greatly reduced. Complexity reduction in this response includes the following aspects:

- Average computational complexity reduction:

- One MV clustering method for the CU-level and sub-CU-level FRUC motion search

- One multi-stage local motion search for the CU-level and sub-CU-level FRUC motion refinement

- Early termination methods for the sub-CU-level FRUC motion search and motion refinement

- Early termination method of the DMVR

- Worst-case complexity and memory bandwidth reduction:

- Reference picture selection in the MV clustering method reduces in the worst-case the number of reference pictures used in FRUC motion search from 4 reference pictures to 1 reference picture

- The maximum number of search iterations performed during the CU-level and sub-CU-level motion refinement is set to 12 for integer-pel refinement and 3 for fractional-pel refinement

- Constrained search window 96×96 luma sample block to reduce the worst-case memory bandwidth of the FRUC

As detailed in Section 12.2.10.1 in the “tool on” and “tool off” tests for FRUC simplifications, the proposed simplifications can achieve significant complexity reduction: the total decoding time is reduced by almost 1/3 on average.

## Complexity characteristics of decoder motion compensation

As discussed in Section 2.1.7.3, both the LIC and the BIO are significantly simplified in this response.

The following changes are proposed to significantly reduce the average and worst-case complexity of the LIC:

- Simplified LIC method for bi-prediction by reducing the number of LIC parameter derivations from 2 to 1

- Simplified LIC for OBMC by reducing the worst-case number of LIC parameter derivations from 5 per sub-block to 1 per sub-block

- Disable the LIC for FRUC bilateral-matching mode

The following changes are proposed to significantly reduce the average complexity of the BIO:

- Two-stage early termination of the BIO at CU level and sub-CU level based on prediction signal similarity;

- Modified BIO filtering process.

As detailed in Section 12.2.10.1 in the “tool on” and “tool off” tests for BIO simplifications, the proposed simplifications can achieve significant complexity reduction: the total decoding time is reduced by more than 1/5 on average.

The following changes are proposed to significantly reduce the average complexity of the OBMC:

- Sub-block grouping to reduce the average number of OBMC operations

- Conditional skipping of certain OBMC operations based on MV difference

As detailed in Section 12.2.10.1 in the “tool on” and “tool off” tests for OBMC simplifications, the proposed simplifications can achieve significant complexity reduction: the total decoding time is reduced by 13%.

## Complexity characteristics of decoder intra-frame prediction operation

This is the same as in the JEM.

## Complexity characteristics of decoder in-loop filtering operation

As discussed in Section 2.1.9.1, the derivative calculation at highest temporal layer is simplified.

## Complexity characteristics of additional decoder tools

The proposed MTT partition increases partition types for the coding tree but does not increase decoding complexity.

## Complexity characteristics of 360° video specific decoding tools

To convert a reference picture from one HAC format to a different HAC format (see Section 2.2.1.1), Lanczos 3 and Lanczos 2 interpolation in the 2D plane is used in this response for the luma and chroma components, respectively. A look-up table is used to store the sample positions and corresponding index of the phase of the interpolation filter. This is performed on one IRAP picture per IRAP period.

To convert a reference picture from one frame packing configuration to a different frame packing configuration (see Section 2.2.1.2), a simple “reshuffling” of the faces within the frame packed pictures is involved. Thus, the adaptive frame packing has no impact on the decoder complexity.

To get the corresponding value of the padded samples with geometry padding (see Section 2.2.3.1), bilinear interpolation in the 2D plane is used in this response. For a face of size 1184×1184 and padding size of 144, the padded region is 4×144×(1184+144). In other words, using geometry padding, the padded region is equivalent to 55% of the coding picture size. In comparison, if repetitive padding is used for the same 3×2 frame packing arrangement, the padded region is equivalent to 22% of the coding picture size. In the decoder implementation of this response, each face in each reference picture is entirely padded. In practical implementation, this padding operation may be performed on the fly at the decoder side.

For post-filtering (see Section 2.2.3.3), only a very limited number of samples are affected. The decoding complexity increase due to post filtering is negligible.

## Complexity characteristics of HDR specific decoding tools

For the in-loop Luma reshaper, the major computational complexity is the construction of FwdLUT and InvLUT and the mapping for Luma samples using the pair of LUTs. The pair of reshaping LUTs are implemented using 2-byte (10-bit), 1024 entry integer arrays. All the related operations, such as mapping and LUT construction, are performed with fixed point arithmetic at the decoder side. The in-loop reshaper performs Luma sample mapping in-place, meaning no extra storage buffer is needed for Luma samples.

For Luma-based QP Prediction, the major computational complexity is the LumaDQPLUT table lookup. It reuses the existing deltaQP parsing framework and does not require any additional memory allocation.

## Degree of capability for decoder parallel processing

This is the same as in the JEM.

# Encoder complexity analysis

## Encoding time and measurement methodology

Table 30 to Table 35 report the relative encoding time for each sequence and rate point when compared to the HM and JEM anchors for the different classes and constraint sets. The encoding time of the anchors and the response was measured using the Linux clusters as specified in Section 6.2. For the SDR category, compared to the HM anchor, the average encoding time of the response is 1710% and 1827% for the constraint set 1 and the constraint set 2, respectively; compared to the JEM anchor, the average encoding time of the response is 205% and 203% for the constraint set 1 and the constraint set 2, respectively. For the 360° category, the average encoding time of the response is 1332% and 241% of that of the HM and JEM anchors, respectively. For the HDR category, the average encoding time of the response is 1462% and 183% of that of the HM and JEM anchors, respectively.

Table 30. Relative encoding time of each sequence and rate point for class SDR-A constraint set 1

|  | Relative encoding time |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | vs. HM anchor |  |  |  | vs. JEM anchor |  |  |  |
| Sequence | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| FoodMarket4 | 976% | 1136% | 1283% | 1624% | 174% | 189% | 203% | 217% |
| CatRobot1 | 1183% | 1373% | 1547% | 1918% | 183% | 190% | 200% | 217% |
| DaylightRoad2 | 1197% | 1386% | 1655% | 2009% | 180% | 190% | 203% | 214% |
| ParkRunning3 | 1467% | 1586% | 1812% | 2103% | 190% | 203% | 205% | 223% |
| Campfire | 2079% | 2538% | 2964% | 3300% | 186% | 204% | 217% | 225% |
| Average | 1669% |  |  |  | 200% |  |  |  |

Table 31. Relative encoding time of each sequence and rate point for class SDR-B constraint set 1

|  | Relative encoding time |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | vs. HM anchor |  |  |  | vs. JEM anchor |  |  |  |
| Sequence | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| BQTerrace | 922% | 1025% | 1227% | 1453% | 188% | 196% | 208% | 221% |
| RitualDance | 1964% | 2260% | 2382% | 2656% | 189% | 197% | 206% | 215% |
| MarketPlace | 1325% | 1475% | 1725% | 1896% | 195% | 201% | 209% | 221% |
| BasketballDrive | 1788% | 2018% | 2207% | 2583% | 198% | 205% | 219% | 237% |
| Cactus | 1499% | 1709% | 1977% | 2345% | 204% | 216% | 230% | 245% |
| Average | 1751% |  |  |  | 209% |  |  |  |

Table 32. Relative encoding time of each sequence and rate point for class SDR-B constraint set 2

|  | Relative encoding time |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | vs. HM anchor |  |  |  | vs. JEM anchor |  |  |  |
| Sequence | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| BQTerrace | 852% | 909% | 1003% | 1162% | 181% | 187% | 188% | 197% |
| RitualDance | 2257% | 2593% | 2757% | 2949% | 189% | 209% | 210% | 224% |
| MarketPlace | 1335% | 1671% | 1785% | 1984% | 186% | 209% | 197% | 213% |
| BasketballDrive | 2069% | 2225% | 2594% | 2792% | 195% | 204% | 214% | 221% |
| Cactus | 1705% | 1868% | 1990% | 2386% | 200% | 204% | 207% | 225% |
| Average | 1827% |  |  |  | 203% |  |  |  |

Table 33. Relative encoding time of each sequence and rate point for class HDR-A constraint set 1

|  | Relative encoding time |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | vs. HM anchor |  |  |  | vs. JEM anchor |  |  |  |
| Sequence | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| DayStreet | 1441% | 1201% | 1038% | 923% | 181% | 151% | 130% | 116% |
| PeopleInShoppingCenter | 1313% | 1147% | 969% | 919% | 165% | 144% | 122% | 115% |
| SunsetBeach | 2171% | 2022% | 1566% | 1169% | 272% | 254% | 196% | 147% |
| Average | 1272% |  |  |  | 160% |  |  |  |

Table 34. Relative encoding time of each sequence and rate point for class HDR-B constraint set 1

|  | Relative encoding time |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | vs. HM anchor |  |  |  | vs. JEM anchor |  |  |  |
| Sequence | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| Market3 | 1964% | 1639% | 1280% | 1197% | 246% | 206% | 161% | 150% |
| ShowGirl2 | 2092% | 1753% | 1544% | 1260% | 262% | 220% | 194% | 158% |
| Hurdles | 2227% | 1956% | 1081% | 1328% | 279% | 245% | 136% | 167% |
| Starting | 1468% | 1404% | 825% | 1078% | 184% | 176% | 104% | 135% |
| Cosmos1 | 3244% | 2823% | 2253% | 1272% | 407% | 354% | 283% | 160% |
| Average | 1589% |  |  |  | 199% |  |  |  |

Table 35. Relative encoding time of each sequence and rate point for class 360 constraint set 1

|  | Relative encoding time |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | vs. HM anchor |  |  |  | vs. JEM anchor |  |  |  |
| Sequence | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| Balboa | 1086% | 1274% | 1557% | 1648% | 154% | 164% | 173% | 172% |
| Chairlift | 1192% | 1367% | 1677% | 1898% | 186% | 194% | 209% | 213% |
| Harbor | 969% | 1243% | 1481% | 1750% | 265% | 295% | 311% | 339% |
| KiteFlite | 780% | 909% | 1166% | 1482% | 226% | 261% | 289% | 326% |
| Trolley | 1122% | 1299% | 1609% | 1869% | 268% | 284% | 318% | 336% |
| Average | 1332% |  |  |  | 241% |  |  |  |

## Description of computing platform used to determine encoding times reported in Section 6.1

The Linux cluster as specified in Table 36 and Table 37 was used for measuring the encoding time for SDR/360° and HDR, respectively. The Sun Grid Engine accounting information was check using the qacct command to get the ru_maxrss entry for all categories.

Table 36. Technical specification of the Linux cluster for SDR and 360° categories

| CPU | Intel Xeon E5-2698 v3 @ 2.30GHz |
| --- | --- |
| Memory | 16 GB per core |
| HDD characteristics | -Hard Drive Capacity: 1.8TB -Interface: Serial Attached SCSI -External Data Transfer Rate: 12.0 GB/sec -Rotational Speed: 10,000 rpm |
| Compiler | GCC version 4.4.5 |
| Operating System (OS) | Linux 64 bit (Red Hat 6.9) |

Table 37. Technical specification of the Linux cluster for HDR category

| CPU | Intel Xeon E5-2690 v4 @ 2.60GHz |
| --- | --- |
| Memory | 16 GB per core |
| HDD characteristics | -Hard Drive Capacity: 14TB |
| Compiler | GCC version 4.4.7 |
| Operating System (OS) | Linux 64 bit (CentOS 6.5) |

## Memory usage of encoder

Table 38 to Table 42 report the peak encoder memory usage of the response and anchors for each category and constraint set, for the sequence class of the largest resolution at the highest target rate point. The peak encoder memory usage of the anchors and the response was measured using the Linux clusters as specified in Section 6.2. In the SDR category, for the constraint set 1, the peak encoder memory usages of the HM, the JEM and the response are 1.92GB, 6.01GB and 7.88GB, respectively; for the constraint set 2, the corresponding peak encoder memory usages are 0.24GB, 0.90GB and 2.50GB for the HM, the JEM and the response, respectively. For the 360° category, the peak encoder memory usage of the response is 9.72GB, compared to 3.74GB and 7.94GB for the HM and JEM anchors. For the HDR category, the peak encoder memory usage of the response is 5.81GB, compared to 1.91.GB and 6.01GB for the HM and JEM anchors.

Table 38. Peak encoder memory usage for class SDR-A constraint set 1

|  | Peak encoder memory usage [GB] |  |  |
| --- | --- | --- | --- |
| Sequence | Response | HM Anchor | JEM Anchor |
| FoodMarket4 | 7.87 | 1.90 | 6.01 |
| CatRobot1 | 7.87 | 1.91 | 6.01 |
| DaylightRoad2 | 7.87 | 1.92 | 6.01 |
| ParkRunning3 | 7.88 | 1.90 | 6.01 |
| Campfire | 7.86 | 1.88 | 6.01 |

Table 39. Peak encoder memory usage for class SDR-B constraint set 2

|  | Peak encoder memory usage [GB] |  |  |
| --- | --- | --- | --- |
| Sequence | Response | HM Anchor | JEM Anchor |
| BQTerrace | 2.49 | 0.24 | 0.90 |
| RitualDance | 2.49 | 0.24 | 0.89 |
| MarketPlace | 2.49 | 0.24 | 0.89 |
| BasketballDrive | 2.50 | 0.24 | 0.89 |
| Cactus | 2.49 | 0.24 | 0.90 |

Table 40. Peak encoder memory usage for class HDR-A constraint set 1

|  | Peak encoder memory usage [GB] |  |  |
| --- | --- | --- | --- |
| Sequence | Response | HM Anchor | JEM Anchor |
| DayStreet | 5.81 | 1.90 | 6.01 |
| PeopleInShoppingCenter | 5.81 | 1.89 | 6.01 |
| SunsetBeach | 5.81 | 1.91 | 6.01 |

Table 41. Peak encoder memory usage for class HDR-B constraint set 1

|  | Peak encoder memory usage [GB] |  |  |
| --- | --- | --- | --- |
| Sequence | Response | HM Anchor | JEM Anchor |
| Market3 | 3.51 | 0.52 | 1.65 |
| ShowGirl2 | 3.51 | 0.53 | 1.66 |
| Hurdles | 3.51 | 0.53 | 1.66 |
| Starting | 3.51 | 0.53 | 1.66 |
| Cosmos1 | 2.85 | 0.43 | 1.35 |

Table 42. Peak encoder memory usage for class 360 constraint set 1

|  | Peak encoder memory usage [GB] |  |  |
| --- | --- | --- | --- |
| Sequence | Response | HM Anchor | JEM Anchor |
| Balboa | 9.29 | 3.26 | 7.47 |
| Chairlift | 9.72 | 3.71 | 7.94 |
| Harbor | 9.71 | 3.73 | 7.90 |
| KiteFlite | 9.72 | 3.74 | 7.90 |
| Trolley | 9.72 | 3.73 | 7.94 |

## Complexity characteristics of encoder intra-frame prediction type selection

This is the same as in the JEM.

## Complexity characteristics of encoder motion estimation and motion segmentation selection

### General

This is the same as in the JEM.

### Complexity characteristics of usage of decoder-side motion refinement

This is the same as in the JEM.

## Complexity characteristics of encoder transforms and transform type selection

This is the same as in the JEM.

## Complexity characteristics of encoder quantization and quantization type selection

This is the same as in the JEM.

## Complexity characteristics of encoder in-loop filtering type selection and parameter estimation

For ALF as discussed in Section 3.1.6, an additional condition is added to bias against using ALF for pictures at the highest and second highest temporal layers. The bias does not affect encoding complexity.

## Complexity characteristics of encoder entropy coding type selection

This is the same as in the JEM.

## Complexity characteristics of additional encoder tools

The proposed MTT partition discussed in Section 2.1.10.1 increases encoding complexity, as the encoder needs to select from a larger set of partition types. To control encoding complexity increase, the fast encoding method as discussed in Section 3.1.2 is applied for MTT.

## Complexity characteristics of 360° video specific encoding tools

The HAC parameters and frame packing parameters are determined at the encoder side at every IRAP. There is almost no impact on the encoding complexity because these parameter estimation processes do not invoke any additional coding loops.

## Complexity characteristics of HDR specific encoding tools

For the in-loop Luma reshaper, the additional computational complexity at encoder side compared to the decoder is picture analysis. Block based analysis (16×16) is used to gather image statistics: such as mean, variance and histogram, and to determine bin importance. The picture analysis is only performed once in each intra slice.

For Luma-based QP Prediction, the major computational complexity is the LumaDQPLUT table lookup which is the same as the decoder side operation.

## Degree of capability for encoder parallel processing

Parallel encoding at slice level is supported in the same way as in the JEM.

# Algorithmic characteristics

## Random access characteristics

This is the same as in the JEM.

## Delay characteristics

The delay is same as the HM/JEM anchor.

## Additional characteristics discussion topic(s)

None.

# Software implementation description

The response was developed using C++ based on the JEM-7.0 reference software for the SDR and HDR category and the 360Lib software for the 360° category.

# Highlighted aspects discussion

## SDR

For the SDR category, the main goal of this response is to reduce the decoder complexity of some of the most complex JEM coding tools without incurring coding performance loss. Table 43 highlights the most effective complexity reduction algorithms proposed in this response.

Table 43. Summary of the main complexity reduction algorithms in the SDR category

| Tool name | Decoding time reduction (%) | Performance loss (BD rate) | Worst case complexity reduction? (Y/N) | Memory bandwidth reduction? (Y/N) |
| --- | --- | --- | --- | --- |
| FRUC-related simplifications | 33% | 0.14% | Y | Y |
| BIO-related simplifications | 21% | 0.13% | N | N |
| OBMC-related simplifications | 13% | 0.05% | N | N |
| LIC-related simplifications | 1% | 0.00% | Y | Y |

Further complexity reduction aspects have been considered since the final bit-streams were submitted.

- Further simplification on the LIC

In the CfP submission, the LIC parameters derived at MC stage are reused at the LIC operations at the OBMC stage (see Section 2.1.7.3). Although such method significantly reduces the number of times that the LIC parameter derivation process is invoked, the CfP response still performs the LIC-based sample adjustment (i.e., the linear mode based on one scaling factor and one offset) multiple times during the MC and the OBMC stages.  After the bit-streams were submitted, we further simplified the LIC by applying the LIC sample adjustment only one time to the prediction signal generated after the OBMC stage. It is reported that the method can significantly reduce in the worst-case the number of the LIC-based sample adjustments in the JEM with no coding performance impact. To understand the reduction on the worst-case LIC complexity, let’s assume W and H are the width and the height of the current block that is divided into 4×4 sub-blocks, each being coded by bi-directional prediction with unique motion; additionally, it is assumed that the OBMC is applied to the block. Therefore, to generate the prediction of the block, there are in total 2×=10×W4×H4-2×W4-2×H4 LIC parameter derivations and 2×W4×H4×5-W4-H4= 10×W4×H4-2×W4-2×H4 LIC-based sample adjustments that are performed by the JEM. Finally, when both the LIC simplifications in Section 2.1.7.3 and in this section are applied, both the numbers of the LIC parameter derivations and the LIC-based sample adjustments can be reduced to W4×H4, i.e., by almost 10 times compared to the JEM.

- Encoding/decoding latency reduction on the template-based inter prediction methods

In the JEM, there are two template-based inter prediction modes, including the FRUC template-matching mode and the LIC mode. The encoding/decoding of these two modes need to refer to the already reconstructed samples of spatial neighbors in order to generate the prediction signal of the current block. Such dependency introduces very undesirable latency for the encoding and decoding processes among spatial neighboring blocks. To reduce such latency, instead of using the reconstructed samples, we use the prediction signal (specifically, the prediction signal generated after the MC but before the OBMC) as the template for the template-based inter prediction methods. By this way, the encoding/decoding of one template block can start as soon as the prediction of its neighboring samples become available. Additionally, one DC offset (based on the parsed DC coefficient at the entropy decoding stage) is added to the prediction samples to mitigate the potential coding loss caused by using prediction signal as template. Simulation results show that the proposed method maintains most of the coding performance of the FRUC template-matching mode and the LIC mode, and only introduces small coding loss. At the same time, the encoding/decoding latency is significantly reduced compared to the existing template-based inter prediction modes in the JEM. Readers are referred to [16] for more details on the proposed method.

## 360° Video

For the 360° category, face seam artifact reduction is a major goal of this response, because if care is not taken, these artifacts can be visible even at medium bit rates. The technologies proposed in this response, e.g., adaptive frame packing, geometry padding, and post-filtering, aim at minimizing the face seam artifacts. Further, most of the face discontinuity handling methods proposed in this response aim at slightly modifying the SDR tools in such a way that these SDR tools will not cause face seam artifacts to be generated when applied to 360° video. As illustrated in Figure 37, face seam artifacts can be observed in the “tool-on” anchor (see Section 12.2), i.e., with the proposed face discontinuities handling disabled, whereas no face seam artifact is observed with the proposed face discontinuities handling enabled. From our informal viewing, no face seam artifact is observed in the reconstructed viewport video sequences encoded with this response, whereas a face seam artifact is clearly visible in both the HM and JEM anchors in the Chairlift sequence, where the video appears to flicker around the face seams and making the face seams more objectionable visually.

- “tool-on” anchor with face discontinuities handling disabled

- Face discontinuities handling enabled

Figure 37. Screen shot illustrating face seam artifacts when face discontinuities handling is disabled for Chairlift@R2

Compared to the HM anchor, visual quality improvements from this response, for example in the form of better texture and edges with reduced blocking artifacts, are clearly visible in real-time playback. Visual quality improvements are also observed over the JEM anchor for several sequences and rate points. The overall visual quality improvement over the JEM anchor is particularly visible for the Chairlift sequence. Figure 38 to Figure 40 illustrate different examples where this response improves visual quality over the HM and JEM anchors.

- Viewport extracted from uncompressed sequence

- HM anchor

- JEM anchor

- This response

Figure 38. Screen shot illustrating visual quality improvements over the 360° anchors for Balboa@R1

- Viewport extracted from uncompressed sequence

- HM anchor

- JEM anchor

- This response

Figure 39. Screen shot illustrating visual quality improvements over the 360° anchors for Chairlift@R2

| - Viewport extracted from uncompressed sequence | - HM anchor |
| --- | --- |
| - JEM anchor | - This response |

Figure 40. Screen shot illustrating visual quality improvements over the 360° anchors for Trolley@R3

## HDR

For HDR category, as described in Section 2.3.3.1, the in-loop reshaper supports two modes: default mode and adaptive mode. Adaptive mode is used in submitted CfP bitstreams for better subjective quality. Additional BD-rate results are shown in Table 44 and Table 45 by replacing the adaptive in-loop reshaper with the default in-loop reshaper, which targets on maximizing wPSNR metrics.

Table 44. Comparison of the default reshaper with the HM anchor under the HDR-B constraint set 1

|  | BD-rate over HM16.16 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | L100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | -38.81% | -28.82% | -28.83% | -41.50% | -43.18% | -25.36% | -44.69% | -43.08% |
| ShowGirl2 | -37.99% | -29.67% | -30.65% | -69.30% | -52.74% | -27.70% | -70.64% | -56.59% |
| Hurdles | -47.49% | -38.94% | -37.71% | -57.76% | -53.13% | -33.98% | -54.05% | -49.43% |
| Starting | -36.68% | -34.43% | -33.00% | -36.02% | -34.90% | -30.33% | -37.25% | -30.03% |
| Cosmos1 | -36.45% | -26.39% | -26.21% | -71.12% | -83.84% | -24.19% | -70.95% | -83.94% |
| Average HDR-B | -39.48% | -31.65% | -31.28% | -55.14% | -53.56% | -28.31% | -55.52% | -52.61% |

Table 45. Comparison of the default reshaper with the JEM anchor under the HDR-B constraint set 1

|  | BD-rate over JEM7.0 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | L100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | 1.66% | -5.93% | -5.22% | 8.92% | 4.19% | -2.60% | -8.87% | -16.47% |
| ShowGirl2 | -15.23% | -4.47% | -4.38% | -23.45% | -35.25% | -2.73% | -33.97% | -46.05% |
| Hurdles | 4.55% | -6.98% | -6.54% | 6.73% | 11.30% | -4.71% | 3.21% | 8.08% |
| Starting | -3.79% | -5.32% | -4.00% | -2.48% | 4.44% | -2.36% | -7.95% | -9.00% |
| Cosmos1 | -8.66% | -2.95% | -2.75% | -13.87% | -30.66% | -1.92% | -14.42% | -29.30% |
| Average HDR-B | -4.29% | -5.13% | -4.58% | -4.83% | -9.19% | -2.86% | -12.40% | -18.55% |

The picture quality of Class HDR-B content has been evaluated by the proponents on both Pulsar display and Sim2. On each display, the response is able to show subjective quality improvement over the JEM Anchor for the majority of the test sequences. Figure 41 and Figure 42 illustrate two examples where this response improves visual quality over the JEM anchors. Due to presentation media limitation, the example is represented by display-mapped SDR. And the improvement looks more prominent on HDR display than what is shown in the mapped SDR results.

| - JEM anchor | - This response |  |
| --- | --- | --- |

Figure 41. Screen shot illustrating visual quality improvements over the HDR JEM anchor for Market@R2

- JEM anchor

- This response

Figure 42. Screen shot illustrating visual quality improvements over the HDR JEM anchor for Starting@R1

# Closing remarks

This response provides a complete set of solutions for all three categories. The codec in this response is reported to achieve large coding efficiency gain and substantial subjective quality improvement in all three categories. Great attention is paid to ensure that the decoding complexity of the core SDR codec is reasonable: for the constraint set 1 class SDR sequences, the decoding complexity is ~2.6x compared to HM anchor and ~0.3x compared to JEM anchor. As an experimental platform, the JEM provides evidence that substantial compression performance gain can be achieved over the HEVC standard. However, close attention was not paid to study effective components of the JEM coding tools and the interactions among different tools, leading to significant decoding complexity increase. As the next generation video standard project gets underway, maintaining reasonable decoder complexity will become an essential design consideration.  From that point of view, this response provides good candidate technologies that are shown to achieve the same coding efficiency but with significantly reduced complexity; they are suitable candidate technologies for inclusion in the Test Model to be used as the basis for developing the next generation video compression standard.

In addition, the proponents believe that HDR tools should be part of the base codec due to the wide commercial success of HDR in the marketplace. The concept of the reshaper has proven to be beneficial in solving a number of HDR challenges, such as increasing compression efficiency. This proposal presents a simplified method to include the in-loop reshaper in the core codec and an enhanced Luma-based QP prediction mechanism. The tools are appropriate candidates to be included in the Test Model.

As for 360° video, the proponents believe that special care needs to be taken when using SDR coding tools to code this type of video content. The spherical neighboring relationship should be taken into account in order to avoid the 360° specific subjective quality problems such as face seam artifacts. Further, coding tools designed for 360° video, including projection formats that provide good coding efficiency, should be included in the Test Model as they have been shown to provide significant compression efficiency gain compared to the equirectangular projection format used in the anchors.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: Questionnaire

## Codec level questions

| Codec Level Q&A |  | HM | JEM | The response |
| --- | --- | --- | --- | --- |
| 1 | Maximum number of reference frames used by encoder | 4 | 4 | same as the JEM |
| 2 | Maximum coding unit (e.g. CTU in HEVC) | luma 64×64 chroma 32×32 | luma 128×128 chroma 64×64 | same as the JEM |
| 3 | Minimum and maximum transform block size | luma 4×4/32×32 chroma 4×4/16×16 | luma 4×4/128×128 chroma 2×2/64×64 | same as the JEM |
| 4 | Minimum and maximum intra prediction block | luma 4×4/32×32 chroma 4×4/16×16 | luma 4×4/32×32 chroma 2×2/32×32 | same as the JEM |
| 5 | Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length | luma 4×8/8×4 (uni-pred) and 8×8 (bi-pred), 8-tap Chroma 2×4/4×2 (uni-pred) and 4×4 bi-pred, 4-tap | luma 4×4 (uni/bi-pred), 8-tap Chroma 2×2 (uni/bi-pred), 4-tap | same as the JEM |
| 6 | List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category) | n/a | template matching in FRUC, LIC | same as the JEM |
| 7 | List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples, position unforeseeable) | n/a | 64-bit arithmetic for LIC, BIO and LMC | same as the JEM |
| 8 | List all the tools that use non-rectangular (e.g. triangle) partitions | n/a | n/a | n/a |
| 9 | Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa) | n/a | n/a | n/a |
| 10 | Indicate whether luma and chroma use separate partitioning structures | no | yes, in I-slices | same as the JEM |
| 11 | Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes) | Entropy parsing depends on intra prediction mode reconstruction, which requires an intra MPM list derivation | Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation | For blocks coded by the proposed DIMD mode, entropy parsing depends on the spatial neighbors’ intra modes (this dependency is simpler than parsing of conventional intra modes); for all other blocks, entropy parsing is the same as the JEM. |

## Tool level questions

### Category 1. Partitioning structure

| Category 1. Partitioning structure |  | HM | JEM | The response |
| --- | --- | --- | --- | --- |
| 1.a | Whether the prediction unit partitioning is different from transform unit partitioning | Yes, PU and TU partitioning for a CU can differ | No | No |
| 1.b | Maximum tree depth | CU maximum depth 4 TU maximum depth 3 PU maximum depth 1 | QT maximum depth 5 BT maximum depth 3 | QT maximum depth 5 BT/TT maximum depth is 4 for intra slices and 3 for inter slices |

### Category 2. Entropy coding

| Category 2. Entropy coding |  | HM | JEM | The response |
| --- | --- | --- | --- | --- |
| 2.a | Total number of context models | 186 | 329 | 329+8 |
| 2.b | Memory storage size for each context model | 8-bit | 30-bit | same as the JEM |
| 2.c | Range table size if table look up is used for range update | LPSTable[64×4×8bits]; RenormTable[32× 3bits] | LPSTable[512×64×9bits]; RenormTable[128×3bits] | same as the JEM |
| 2.d | Initialization table size | 3×186×8 bits | 3×329×8 + 329×15×5 bits | same as the JEM |
| 2.e | Other memory associated with context model adaptation | LPS transition table: 64*6 bits MPS transition table: 64*6 bits | n/a | n/a |

### Category 3. Control data derivation process

| Category 3. Control data derivation process |  | HM | JEM | The response |
| --- | --- | --- | --- | --- |
| 3.a | Temporal motion data storage granularity | 16×16 | 4×4 | same as the JEM |
| 3.b | Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-directional reference picture are used, count this picture as 2) | 1 | 4 | 1 |

### Category 4. Decoder-side motion refinement

| Category 4. Decoder-side motion refinement |  | HM | JEM | The response |
| --- | --- | --- | --- | --- |
| 4.a | Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement. | n/a | - BIO 2×2D separable 6 taps filter (for gradients), mults per sample in refinement calculation 5+5*4 +1/8+2, minimum processing unit same as regular bi-pred, no SAD calculation - FRUC minimum processing unit 44, sequential PU followed by sub-PU level refinement, number of SAD calculations and additional MC interpolations per unit for MV refinement is not constrained (refinement is terminated when cost no longer decreases); bi-linear filter at MV refinement stage | - BIO is kept the same as the JEM except SAD between two prediction signals is calculated for each minimum processing unit 4×4 to determine whether to conditionally skip BIO. - For DMVR, one SAD calculation is added for each block - For FRUC, the worst-case number of SAD calculations and the worst-case number of MC interpolation for MV refinement are both bounded. The maximum number of MC interpolation for MV refinement in this response is 12 integer-pel motion refinement and 3 fractional-pel motion refinement. |
| 4.b | Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size. | n/a | - DMVR needs to access (H+7+2)×(W+7+2) MC instead of (H+7)×(W+7) in HEVC MC - FRUC needs to access up to 4 reference pictures for motion search; the search window is not constrained | - DMVR is kept the same as the JEM - For FRUC, only one single reference picture is used for the motion search for list L0/L1; the search window in the reference picture is also restricted to be within 96×96 luma sample block |

### Category 5. Intra prediction

| Category 5. Intra prediction |  | HM | JEM | The response |
| --- | --- | --- | --- | --- |
| 5.a | Line buffer size in terms of luma and chroma sample lines | luma 1, chroma 1 | luma 4, chroma 2 | luma 5, chroma 2 |
| 5.b | List all the filters used for intra prediction | Reference sample smoothing: {1,2,1}, Strong intra smoothing filter: 2-tap bilinear filter (only for 32×32 luma TU) Interpolation: 2×32×5 bits (bi-linear 32 phases) | Reference sample smoothing: {1,2,1}, {3,7,14,16,14,7,3}, {4,9,12,14,12,9,4} Interpolation: 2×4×32×8 bits (4 taps, 2 sets, 32 phase) | same as the JEM |
| 5.c | Indicate whether there are cross-component dependencies in intra prediction reconstruction | n/a | LM-CHROMA (Cb depends on Y, and Cr depends on Cb&Y) | same as the JEM |

### Category 6. Inter prediction

| Category 6. Inter prediction |  | HM | JEM | The response |
| --- | --- | --- | --- | --- |
| 6.a | Motion vector accuracy | luma ¼ pel chroma 1/8 pel | luma 1/16 pel chroma 1/32 pel | same as the JEM |
| 6.b | Storage size for a motion vector component | 16-bit | 16-bit | same as the JEM |
| 6.c | List all the interpolation filters used for luma/chroma motion compensation | luma 8-tap, 8×4×6 bits chroma 4-tap, 4×8×6 bits | luma 8-tap, 8×16×6 bits chroma 4-tap, 4×32×6 bits | same as the JEM for SDR and HDR; bi-linear filter is used to generate reference samples in the padded region for 360 |
| 6.d | Worst-case number of operations associated with accessing the neighboring reconstructed samples for the generation of the prediction of one block | n/a | The LIC parameters are derived twice for bi-prediction, i.e., L0 and L1 separately; different LIC parameters are used at MC and OBMC; the worst-case number of the LIC derivations is 2*((W/4)x(H/4)x5–(W/4)-(H/4)) | The LIC parameters are derived once for bi-prediction; the same LIC parameters are used at both MC and OBMC; the worst-case number of the LIC derivations is (W/4)x(H/4) |

### Category 7. Quantization

| Category 7. Quantization |  | HM | JEM | The response |
| --- | --- | --- | --- | --- |
| 7.a | Storage size associated with quantization | n/a | n/a | n/a |

### Category 8. Transforms

| Category 8. Transforms |  | HM | JEM | The response |
| --- | --- | --- | --- | --- |
| 8.a | List all the combinations of transform block size and transform type for primary transforms | - 8-bit 4×4, 8×8, 16×16 and 32×32 DCT-II, - 8-bit 4×4 DST VII | HEVC transform mode (10-bit DCT-II), transform size combinations: - (4, 8, 16, 32, 64, 128) × (4, 8, 16, 32, 64, 128) TUs for luma CTUs - (2, 4, 8, 16, 32, 64) × (2, 4, 8, 16, 32, 64) TUs for inter-coded chroma CTUs - (2, 4, 8, 16, 32, 64,) × (2, 4, 8, 16, 32, 64) TUs for intra-coded chroma CTUs EMT transform mode (10-bit DST VII, DCT-V, DCT-VIII, DST-I), only applied to CTUs up to 64×64, transform size combinations: - (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TUs for luma CTUs | same as the JEM |
| 8.b | List all the combinations of transform block size and transform type for secondary transforms | n/a | 35×3 4×4 and 8×8 non-separable Hypercube-Givens Transform (HyGT) | same as the JEM |
| 8.c | List all the non-separable transforms used | n/a | 4×4/8×8 HyGT | same as the JEM |
| 8.d | Indicate whether transforms can be fully/partially implemented using direct matrix multiply | partial factorization equivalent to matrix multiplication | factorized form not equivalent to matrix multiplication | same as the JEM |
| 8.e | List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output) | n/a | 4×4 HyGT 2-interations, 8×8 HyGT 4-interations | same as the JEM |

### Category 9. In-loop filtering

| Category 9. In-loop filtering |  | HM | JEM | The response |
| --- | --- | --- | --- | --- |
| 9.a | List line buffer size in terms of luma and chroma lines (if any) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) ALF (luma 8, chroma 4) | same as the JEM |
| 9.b | List all the filters used for in-loop filtering | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. ALF: 9×9/7×7/5×5 diamond filter support for luma; and 5×5 diamond filter support for chroma | same as the JEM |
| 9.c | Minimum processing unit size | Deblocking (luma: 8×8 grid with 4×4 filter decision, chroma 8×8 grid with 4×4 filter decision) | Deblocking (luma 4×4 grid with 4×4 filter decision, chroma 8×8 grid with 2×2 filter decision) ALF: for luma, filter decision is based on 2×2 block Bilateral filter: sample level filter switch (luma only) | same as the JEM |

### Category 10. “tool-off” and “tool-on” tests

#### SDR

Table 46 reports the results of the “tool-off” and “tool-on” tests for the major coding tools that contribute to either further coding efficiency improvement or decoding complexity reduction in the SDR category of this response. Specifically, the following tools are tested:

- FRUC SIMP: simplification methods proposed to the FRUC as described in Section 2.1.4.1

- BIO SIMP: simplification methods proposed to the BIO as described in Section 2.1.7.1

- OBMC SIMP: simplification methods proposed to the OBMC as described in Section 2.1.7.2

- MTT: multi-type tree as described in Section 2.1.10.1

- DIMD: decoder-side intra mode derivation as described in Section 2.1.8.1

The constraint set 1 configuration is used for the “tool-off” and “tool-on” tests. The encoding and decoding times were measured using the computing platforms as specified in Section 6.2 and Section 5.2, respectively. The anchor of the “tool-off” test is the CfP response software. The anchor for the “tool-on” test is the CfP response software with the five selected tools being disabled. The relative performance of the “tool-on” anchor compared to the HM anchor, the JEM anchor and the CfP response is shown in Table 47.

Table 46. Summary of “tool-off” and “tool-on” tests for major SDR coding tools

| Tested tool name | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Run time |  | BD-rate |  |  |  | Run time |  |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc |  | Dec |
| FRUC SIMP | 0.00% | -0.16% | -0.15% | 105% | 156% | 0.14% | 0.23% | 0.27% | 95% |  | 67% |
| BIO SIMP | -0.17% | -0.01% | -0.03% | 104% | 146% | 0.13% | 0.00% | -0.02% | 95% |  | 79% |
| OBMC SIMP | -0.03% | 0.02% | -0.08% | 104% | 120% | 0.05% | 0.05% | 0.05% | 93% |  | 87% |
| MTT | 4.35% | 1.91% | 1.49% | 43% | 101% | -3.26% | -1.10% | -0.61% | 236% |  | 102% |
| DIMD | 0.41% | 0.21% | 0.35% | 90% | 99% | -0.41% | -0.17% | -0.28% | 107% |  | 100% |

Table 47. Performance of the “tool-on” anchor compared to the HM anchor, the JEM anchor and the CfP response results

| Compared to | BD-rate |  |  | Run time |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec |
| HM | -34.03% | -46.75% | -46.69% | 786% | 597% |
| JEM | -0.66% | -2.38% | -2.56% | 94% | 75% |
| The CfP response | 3.46% | 1.14% | 0.78% | 45% | 227% |

#### HDR

Table 48 reports the results of the “tool-off” tests for the in-loop reshaper as described in Section 3.3.2.1. that contributes to HDR coding efficiency improvement of this response. The constraint set 1 configuration for Class HDR-B is used for the “tool-off” tests. Default reshaper is configured to perform the test. The encoding and decoding times were measured using the computing platforms as specified in Section 6.2 and Section 5.2, respectively. The anchor of the “tool-off” test is the CfP response using default in-loop reshaper.

Table 48. Summary of “tool-off” tests for major HDR coding tool

| Tested tool name | “tool-off” test results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  |  |  |  |  |  | Run time |  |
|  | DE | L100 | wY | wU | wV | Y | U | V | Enc | Dec |
| IN LOOP RESHAPER | 8.2% | 5.3% | 3.1% | 8.4% | 8.1% | -0.3% | 1.9% | 1.3% | 96% | 94% |

#### 360° Video

Table 49 reports the results of the “tool-off” and “tool-on” tests for the major 360° video specific coding tools. The BD-rate are based on the end-to-end WS-PSNR measurements. The encoding and decoding times were measured using the computing platforms as specified in Section 6.2 and Section 5.2, respectively. The following tools were tested:

- AFP: adaptive frame packing as described in Section 2.2.1.2

- HAC: hybrid angular cubemap as described in Section 2.2.1.1

- FDH: face discontinuity handling as described in Section 2.2.3.2

- GP: geometry padding as described in Section 2.2.3.1

The anchor for the “tool-on” test is encoded in the EAC projection formation using our SDR codebase (with post-filtering as described in Section 2.2.3.3 enabled). The performance of the “tool-on” anchor over the HM and JEM anchors is reported in Table 50. As can be observed, the “tool-on” anchor, i.e., with major 360° video specific coding tools disabled, already outperforms the HM and JEM anchors by -32.43% and -11.45%, respectively. A large portion of these gains is due to encoding in EAC projection format. Though the objective performance is high, the “tool-on” anchor still has some 360° specific subjective quality issues such as the face seam artifacts.

Table 49. Summary of “tool-off” and “tool-on” tests for major 360° video specific coding tools

| Tested tool name | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| AFP | 0.32% | 1.91% | 1.03% | 98% | 99% | -0.54% | -2.10% | -1.42% | 101% | 100% |
| HAC | 0.16% | 0.46% | -0.01% | 95% | 80% | -0.32% | -0.54% | -0.19% | 104% | 137% |
| FDH | 0.48% | 1.51% | 0.71% | 99% | 99% | -0.33% | -1.12% | -0.60% | 101% | 101% |
| GP | 1.58% | 2.38% | 2.30% | 99% | 72% | -1.62% | -2.23% | -2.32% | 99% | 137% |

Table 50. Performance of the “tool-on” anchor over the HM and JEM anchors

|  | Compared to HM anchor |  |  |  |  | Compared to JEM anchor |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| “tool-on” anchor | -32.43% | -51.94% | -55.32% | 1320% | 188% | -11.45% | -13.58% | -17.39% | 239% | 30% |

# Reference

- A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC,” JVET-H1002, Oct. 2017.

- M. Zhou, J. Chen, E. Francois, P. Hanhart, E. Alshina, “Template for Proposal Description Documents for Responses to the Joint CfP on Video Compression with Capability beyond HEVC,” JVET-I1003, Jan. 2018.

- J. Chen, E. Alshina, G. J. Sullivan, J. R. Ohm, J. Boyce, “Algorithm description of joint exploration test model 7,” JVET-G1001, August, 2017, Torino, Italy.

- X. Xiu, Y. He, Y. Ye, “EE8: Decoder-side intra mode derivation,” JVET-D0097, October, 2016, Chengdu, China.

- X. Xiu, Y. He, Y. Ye, “EE8-related: Decoupling decoder-side intra mode derivation from entropy decoding,” JVET-D0098, October, 2016, Chengdu, China.

- X. Li, H.-C. Chuang, J. Chen, M. Karczewicz, L. Zhang, X. Zhao, A. Said, “Multi-Type-Tree,” JVET-D0117, Oct. 2016, Chengdu, CN.

- Equi-Angular Cubemaps (EAC), https://www.blog.google/products/google-vr/bringing-pixels-front-and-center-vr-video/

- Y. Ye, E. Alshina, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 5),” JVET-H1004, Oct. 2017, Macau, CN.

- Y. He, Y. Ye, P. Hanhart, X. Xiu, “AHG8: Geometry padding for 360 video coding,” JVET-D0075, Oct. 2016, Chengdu, CN.

- P. Hanhart, Y. He, Y. Ye, “AHG8: Geometry padding for PERP,” JVET-J0044, Apr. 2018, San Diego, USA.

- J. Samuelsson, C. Fogg, A. Norkin, A. Segall, J. Ström, G. Sullivan, P. Topiwala, and A. Tourapis, “Conversion and Coding Practices for HDR/WCG Y′CbCr 4:2:0 Video with PQ Transfer Characteristics (Draft 4)”, JCTVC-Z1017, Geneva, CH, 12–20 January 2017.

- K. Minoo (Arris), T. Lu, P. Yin (Dolby), L. Kerofsky (InterDigital), D. Rusanovskyy (Qualcomm), E. Francois (Technicolor), “Description of the Exploratory Test Model (ETM) for HDR/WCG extension of HEVC”, JCTVC-W0092, San Diego, USA, 19–26 February 2016.

- J. Zhao, A. Segall, S.-H. Kim (Sharp), “EE2.4: De-quantization and Scaling for Next Generation Containers”, JVET-C0095, Geneva, CH, 26–31 May 2016.

- T. Lu, F. Pu, P. Yin, J. Pytlarz, T. Chen, W. Husak, “Adaptive Reshaper for High Dynamic Range and Wide Color Gamut Video Compression”, SPIE Optical Engineering + Applications, Proceedings Volume 9971, Applications of Digital Image Processing XXXIX; 99710B, San Diego, California, United States, 2016.

- T. Lu, F. Pu, P. Yin, T. Chen, W. Husak (Dolby), Y. He, L. Kerofsky, Y. Ye (InterDigital), “HDR CE2: CE2.a-2, CE2.c, CE2.d and CE2.e-3”, JCTVC-W0084, San Diego, USA, 19–26 February 2016.

- X. Xiu, Y. He, Y. Ye, “On low-latency reduction for template-based inter prediction,” JVET-J0045, Apr. 2018, San Diego, USA

[end of document]