| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0018 |
| --- | --- |

| Title: | Description of SDR video coding technology proposal by MediaTek |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Ching-Yeh Chen, Tzu-Der Chuang, Han Huang, Shih-Ta Hsiang, Chun-Chia Chen, Man-Shu Chiang, Chen-Yen Lai, Chia-Ming Tsai, Yu-Chi Su, Zhi-Yi Lin, Yu-Ling Hsiao Jan Klopp, I-Hsiang Wang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City, 30078, Taiwan | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 yuwen.huang@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution describes MediaTek’s proposal in response to the standard dynamic range (SDR) category of the “Joint Call for Proposals on Video Compression with Capability beyond HEVC” jointly issued by ITU-T SG16 Q.6 (VCEG) and ISO/IEC JTC1/SC29/WG11 (MPEG). The goal of this proposal is to provide a video codec with higher compression capability than HEVC, especially for ultra high-definition (UHD) and full high-definition (FHD) video contents. To achieve this goal, a number of tools are proposed covering several aspects of video compression technology, including coding block structure, inter/intra prediction, transform, quantization, in-loop filtering, and entropy coding. The proposed video codec achieves -43.81%/-45.61%/-47.41% Y/U/V BD-rates and -31.27%/-37.54%/-38.27% Y/U/V BD-rates compared to HM-16.16 for constraint set 1 (CS1) configuration containing 5 UHD and 5 FHD video sequences under random access condition and constraint set 2 (CS2) configuration containing 5 FHD video sequences under low delay B condition, respectively. When compared to JEM-7.0, the proposed video codec achieves -16.60%/-6.75%/-10.43% Y/U/V BD-rates with 1.52x encoding time and 2.27x decoding time for CS1 configuration and -9.41%/-1.92%/-3.35% Y/U/V BD-rates with 1.31x encoding time and 1.71x decoding time for CS2 configuration. To reduce encoding time, the proposed encoder is further accelerated with encoder-only non-normative changes. After the speed-up, the proposed codec achieves -42.38%/-44.64%/-46.37% Y/U/V BD-rates and -29.69%/-35.82%/-36.60% BD-rates compared to HM-16.16 for CS1 configuration and CS2 configuration, respectively. When compared to JEM-7.0, the proposed video codec achieves -14.40%/-5.13%/-8.82% Y/U/V BD-rates with 0.77x encoding time for CS1 configuration and -7.20%/+0.23%/-0.96% Y/U/V BD-rates with 0.60x encoding time for CS2 configuration.

# Contents

Abstract	i

Contents	ii

1	Introduction	1

2	Decoder algorithm description	1

2.1	Block structure	1

2.1.1	Block partition for internal CTU	1

2.1.2	Block partition for frame boundary CTU	3

2.1.3	Block partition for large CTU	4

2.2	Intra prediction for luma component	4

2.2.1	Intra prediction mode	4

2.2.2	Intra boundary filters	4

2.3	Intra prediction for chroma component	4

2.3.1	Multi-DM modes	4

2.3.2	LM-based modes	4

2.3.3	Signaling of intra chroma modes	5

2.4	Inter prediction	6

2.4.1	Unified merge modes	6

2.4.2	Affine inter mode	8

2.4.3	Pattern-matched motion vector refinement	10

2.4.4	Merge assistant prediction	11

2.4.5	Adaptive motion vector resolution	12

2.4.6	Local illumination compensation	13

2.4.7	Generalized bi-prediction	13

2.4.8	Decoder-side motion vector refinement	13

2.4.9	Bi-directional optical flow	13

2.4.10	Overlapped block motion compensation	14

2.4.11	Motion candidate reorder	14

2.5	Transform	16

2.5.1	Large block-size transform with high-frequency zeroing	16

2.5.2	Adaptive multi-core transform	16

2.5.3	Non-separable secondary transform	19

2.5.4	Transform syntax reorder	20

2.6	Quantization	22

2.7	Reconstruction	22

2.7.1	Bilateral filter	22

2.7.2	Content adaptive clipping	22

2.8	In-loop filters	22

2.8.1	Deblocking filter	23

2.8.2	Non-local mean loop filter	27

2.8.3	Sample adaptive offset	29

2.8.4	Adaptive loop filter	29

2.8.5	Convolution neural network loop filter	31

2.9	Entropy coding	32

2.9.1	Multi-parameter CABAC	32

3	Encoder algorithm description	33

3.1	GOP structure	33

3.2	Adaptive encoder flow for QT/BT	33

3.3	One additional encoding pass for CNN training	33

3.4	Settings in operation point 1 and operation point 2	33

4	Compression performance	34

4.1	SDR: Constraint set 1 configuration relative to HM anchor (OP1)	34

4.1.1	Class SDR-A	34

4.1.2	Class SDR-B	36

4.1.3	Overall	38

4.2	SDR: Constraint set 2 configuration relative to HM anchor (OP1)	39

4.2.1	Class SDR-B	39

4.2.2	Overall	41

4.3	SDR: Constraint set 1 configuration relative to JEM anchor (OP1)	42

4.3.1	Class SDR-A	42

4.3.2	Class SDR-B	44

4.3.3	Overall	47

4.4	SDR: Constraint set 2 configuration relative to JEM anchor (OP1)	48

4.4.1	Class SDR-B	48

4.4.2	Overall	50

4.5	SDR: Constraint set 1 configuration relative to HM anchor (OP2)	50

4.5.1	Overall	51

4.6	SDR: Constraint set 2 configuration relative to HM anchor (OP2)	51

4.6.1	Overall	51

4.7	SDR: Constraint set 1 configuration relative to JEM anchor (OP2)	52

4.7.1	Overall	52

4.8	SDR: Constraint set 2 configuration relative to JEM anchor (OP2)	52

4.8.1	Overall	52

5	Decoder complexity analysis	53

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	53

5.2	Description of computing platform used to determine decoding times reported in section 5.1	53

5.3	Memory usage of decoder	53

6	Encoder complexity analysis	54

6.1	Encoding time and measurement methodology	54

6.2	Description of computing platform used to determine encoding times reported in section 6.1	54

6.3	Memory usage of encoder	54

7	Software implementation description	54

8	Closing remarks	55

9	Patent rights declaration(s)	55

10	Annex 1: Questionnaire	55

10.1	Codec level questions (mandatory)	55

10.2	Tool level questions (optional)	58

11	Reference	61

# Introduction

In response to the SDR category of the Joint Call for Proposals (CfP) on Video Compression with Capability beyond HEVC, MediaTek has submitted the required coded test material. This contribution is a technical description of MediaTek’s SDR proposal. Many effective tools are integrated together to achieve significantly better coding efficiency than the CfP anchors, including HM-16.16 and JEM-7.0.

The integrated tools include tools in the joint exploration test model (JEM), e.g., quad-tree binary-tree (QTBT) for block structure, 65 angular modes, 4-tap interpolation filters, smooth and bi-boundary filters for intra prediction, and local illumination compensation (LIC), overlapped block motion compensation (OBMC) for inter prediction. A few JEM tools are further improved, e.g., affine inter mode, pattern-matched motion vector derivation (PMVD), and bidirectional optical flow (BIO). Moreover, several tools are also developed for better coding efficiency, e.g., triple tree (TT), merge assistant prediction (MAP), motion candidate reorder (MCR), non-local mean loop filter (NLMLF), and convolution neural network loop filter (CNNLF), and for better subjective quality, e.g., length-adaptive deblocking filter (DF).

A software integrated with all the above tools is developed with no mismatch between the encoder and the decoder throughout all the tests performed, including those required by the CfP. The source code of this software is based on JEM, where each coding tool can be easily enabled or disabled by using the corresponding control flag in the sequence parameter set (SPS). Moreover, double Z-scan addressing in JEM is removed, and only raster scan addressing is used to simplify the operations of address mapping. The memory usage including reference frame buffer, coding tree unit (CTU) buffer, and data buffer, is also optimized to reduce the memory requirement when encoding one video sequence or decoding one bitstream. This software can be a good starting framework for the development of the new standard since it already includes many promising techniques up to date.

# Decoder algorithm description

## Block structure

### Block partition for internal CTU

In this proposal, one slice is partitioned into multiple non-overlapping coding tree units (CTUs). Inside the CTU, a multi-type-tree structure is applied to partition a CTU into one or multiple non-overlapping coding units (CUs). The CU is used as the basic unit for both prediction and transform coding, i.e., the prediction unit (PU) and transform unit (TU) concepts in HEVC are removed.

The multi-type-tree structure consists of quad-tree (QT), binary-tree (BT), and triple-tree (TT) structures. A CTU is first partitioned by the QT structure, where a CTU block can be non-split or recursively split into 4 equal-size child-blocks. Then, the QT leaf nodes can be further partitioned by the BT or TT structure. The BT and TT partitioning can be interleaved and recursively applied. However, once BT or TT partitioning is applied, QT partitioning is disabled for the subsequent nodes. There are two splitting types in both BT and TT partitioning. For BT partitioning, the two splitting types are both symmetric. For TT partitioning, the splitting is in a ratio of 1:2:1. The different splitting types are illustrated in Figure 1, and an exemplary partition is shown in Figure 2.

Figure 1: Illustration of splitting types.

Figure 2: Example of multi-type-tree partition.

Signaling of the multi-type-tree structure is shown in Figure 3 and described as follows. A qt_split_flag is first signaled to indicate whether QT split is applied. If qt_split_flag is equal to 1, then the current CU is split into 4 child-CUs, and the signaling is repeated for the child-CUs. If qt_split_flag is equal to 0, then the QT partitioning stops, and a ct_split_mode can be signaled. ct_split_mode equal to 0 indicates the partitioning stops, and the current CU is the leaf node CU. ct_split_mode equal to 1 indicates the current CU is horizontal split, and a ct_split_type can be further signaled. ct_split_mode equal to ‘2’ indicates the current CU is vertical split, and a ct_split_type can be further signaled. ct_split_type equal to 0 indicates BT split, and ct_split_type equal to 1 indicates TT split.

A CU consists of coding blocks (CBs) of one, two, or three colour components. For 4:2:0 chroma format, in P and B slices, one CU contains one luma CB and two chroma CBs; in I slices, one CU contains only one luma CB or two chroma CBs. It means that the luma and chroma components can have different block partitioning structure in I slices.

For P and B slices, some redundant partitions are avoided.

- Further vertical BT split in the 2nd child-CU of horizontal BT split is disabled when the 1st child-CU of horizontal BT split is further vertical BT split.

- Further horizontal BT split in the 2nd child-CU of vertical BT split is disabled when the 1st child-CU of vertical BT split is further horizontal BT split and when QT split is available (i.e., at QT node).

- Further horizontal BT split in the 2nd child-CU of horizontal TT split is disabled.

- Further vertical BT split in the 2nd child-CU of vertical TT split is disabled.

- Further horizontal TT split in the 2nd child-CU of vertical BT split is disabled when the 1st child-CU is further horizontal TT split.

- Further vertical TT split in the 2nd child-CU of horizontal BT split is disabled when the 1st child-CU is further vertical TT split.

The following parameters are defined for the multi-type-tree scheme.

–	CtuSize: the root node size of a quadtree, the same concept as in HEVC

–	MinQTSize: the minimum allowed quadtree leaf node size

–	MaxCTSize: the maximum allowed root node size for BT/TT partitioning

–	MaxCTDepth: the maximum allowed tree depth in BT/TT partitioning

–	MinCTSize: the minimum allowed leaf node size in BT/TT, i.e., minimum CU size

In one example of the multi-type-tree partitioning structure, the CTU size is set as 256×256 luma samples with two corresponding 128×128 blocks of chroma samples, the MinQTSize is set as 8×8, the MaxCTSize is set as 256×256, the MinCTSize (for both width and height) is set as 4, and the MaxCTDepth is set to 4 for intra slices and 3 for inter slices. The quadtree partitioning is applied to the CTU first to generate quadtree leaf nodes. The quadtree leaf nodes may have a size from 8×8 (i.e., the MinQTSize) to 256×256 (i.e., the CTU size). If the leaf quadtree node is larger than MaxCTSize, it will not be further split by the BT/TT (but this does not happen in this example). Otherwise, the leaf quadtree node could be further partitioned by the BT/TT (this always happens in this example). Therefore, the quadtree leaf node is also the root node for the BT/TT, and it has the coding tree depth as 0. When the coding tree depth reaches MaxCTDepth, it implies no further splitting. When the coding tree node has width equal to MinCTSize (i.e., 4), it implies no further vertical splitting. Similarly, when the coding tree node has height equal to MinCTSize, it implies no further horizontal splitting. The leaf nodes of the coding tree are namely CUs further processed by prediction and transform without any further partitioning. When the coding tree node has width equal to 2* MinCTSize, it implies no further vertical TT splitting. Similarly, when the coding tree node has height equal to 2 * MinCTSize, it implies no further horizontal TT splitting. This example is the setting used in our CfP response at operation point 1 submitted for subjective evaluation.

Figure 3: Signaling of multi-type-tree partition.

### Block partition for frame boundary CTU

When a CTU corresponds to a region across a current frame boundary, it is forced to be further split into smaller-size child-CUs using QT split or BT split. When BT split is selected, the split direction, either one from horizontal or vertical split, is implicitly derived from the boundary conditions.

### Block partition for large CTU

In this proposal, CTU with size up to 256x256 and transform with size up to 128 are supported.

In intra slices, each 256x256 luma CTU in units of luma samples is forced to be divided into four 128x128 luma CUs, and 128x128 chroma CUs in units of chroma samples are supported.

In non-low-delay inter slices, for CUs with luma width or height larger than 128, skip mode is inferred and always applied.

In low-delay inter slices (all reference frames are prior to the current frame in display order), for CUs with luma width or height larger than 128, skip mode and advanced motion vector prediction (AMVP) mode with all-zero residuals are supported, where the coded block flag (CBF) of the AMVP mode is not signaled and inferred as 0.

## Intra prediction for luma component

### Intra prediction mode

In this proposal, 65 intra angular modes, DC mode, 4-tap interpolation filters, and smooth filters are supported, which are the same as JEM-7.0. Planar mode is replaced by unequal weight planar (UWP), which is the same as JVET-E0068.

### Intra boundary filters

In this proposal, gradient filters and bi-boundary filters are supported, which are the same as JEM-7.0.

## Intra prediction for chroma component

In this proposal, 15 intra modes are allowed for chroma mode coding. Similar to JEM-7.0, these modes can be classified into 5 direct modes (DMs) and 10 linear model (LM) modes, which are described as in the following sections.

### Multi-DM modes

There are 5 DMs which are the same as those in JEM-7.0.

### LM-based modes

In addition to the Cb-to-Cr residual prediction for non-LM intra modes and the 6 LM-based modes in JEM-7.0 (i.e., one LM mode similar to the LM mode in HM-7.0 (LM), one multi-model LM (MMLM), and four multi-filter LM (MFLM) modes), four new LM-based modes are added, including LM-left and LM-top modes, LM-CbCr mode, and LM fusion mode.

LM-left mode uses 2 columns of left neighboring CU reconstructed samples to determine the linear model parameters between down-sampled luma and chroma components. Similarly, LM-top mode uses 2 rows of above neighboring CU reconstructed samples to derive the linear model parameters. The model derivation process of LM-left and LM-top modes is the same as the LM in JEM-7.0.

LM-CbCr mode derives two separate linear models. One is between the neighboring reconstructed luma and Cb components for predicting Cb samples from luma samples, and the other is between the neighboring reconstructed Cb and Cr components for predicting Cr samples from Cb samples. The model derivation process of LM-CbCr mode is the same as the LM mode in JEM-7.0.

LM fusion mode is used to combine the predictions from LM and DM as a new prediction,

Px,y=0.5×P1x,y+0.5×P2x,y    ,

(1)

where P1 and P2 are the predictions from LM and DM, respectively. One flag is used right after a DM is signaled to indicate whether to apply the LM fusion mode or not.

A division-free derivation process of linear model parameters is used in JEM-7.0. The number of samples for model derivation needs to be power of 2. Therefore, for a non-square chroma CB to select samples for model derivation, samples at the longer side are uniformly sub-sampled to match the sample number at the shorter side. In this proposal, the samples that are at the longer side and far most from the shorter side are discarded, and the number of discarded samples at the longer side is equal to the samples at the shorter side. An example is shown in Figure 4.

| (a) | (b) |
| --- | --- |

Figure 4: For a 16x4 chroma CB, only the yellow samples at the longer side and the blue samples at the shorter side are used in model derivation process. (a) The chosen samples in JEM-7.0. (b) The chosen samples in this proposal with 4 samples discarded at the longer side.

### Signaling of intra chroma modes

The comparisons of syntax designs and the supported intra chroma modes between HEVC, JEM-7.0, and this proposal are summarized in Table 1.

Table 1: The syntax designs of intra chroma modes

|  | HEVC |  | JEM-7.0 |  | Proposed |  |
| --- | --- | --- | --- | --- | --- | --- |
| Non-LM Modes | Planar, Ver, Hor, DC, DM (or VerDIA) |  | Multi-DM modes: - Intra-slice → 5 collocated luma modes Inter-slice → collocated luma mode - Fill up to 5 DMs from neighbors/derived/default modes |  |  |  |
| Binarization of Non-LM Modes | DM Planar Ver Hor DC | 0 100 101 110 111 | DM0 DM1 DM2 DM3 DM4 |  | 10 110 1110 11110 111110 (the 0 should be removed) |  |
| Residual Prediction | N/A |  | For DM (non-LM intra) modes, Cb-to-Cr residual prediction is always applied. |  |  |  |
| LM-based Modes | N/A |  | LM, MMLM, 4 MFLMs |  | LM, MMLM, 4 MFLMs, LM-left, LM-top, LM-CbCr, LM fusion |  |
| Signaling of LM-based Modes | N/A |  | MMLM LM MFLMs | 01 001 00000 00001 00010 00011 | MMLM LM LM-left LM-top LM-CbCr MFLMs | 01 001 0001 00001 000001 00000000 00000001 00000010 00000011 |
|  |  |  |  |  | LM fusion mode is indicated by a flag right after a DM is signaled |  |

## Inter prediction

### Unified merge modes

A merge mode derives the motion information from spatial or temporal neighboring blocks and transmits a merge index to convey which one from the candidate list is selected. The candidate list of the proposed unified merge modes consists of spatial type candidates, temporal type candidates, and affine type candidates. In this proposal, for unification, affine merge is also indicated by a merge index, which is different from JEM-7.0 that uses an additional flag for affine merge. Figure 5 shows the insertion order for the different types of candidates. Before adding one candidate to the list, an availability check is performed to make sure the motion data is available, and a redundancy check is performed to prevent candidates from having exactly the same motion information. The maximum number of merge candidates is 10. If the number of merge candidates is larger than 10, only the first 10 candidates are added. Otherwise, if the number of merge candidates is smaller than 10, zero motion vectors associated with reference indices from zero to the number of reference frames minus one are generated until the number of merge candidates is equal to 10. Different types of unified merge candidates are described in the following sections.

Figure 5: Insertion order for merge candidates; (*) denotes the maximum number of candidates that may stay in the final candidate list of size 10

#### Spatial type candidates

The spatial type candidates consist of spatial candidates, middle spatial candidates, and a multi-average candidate. Figure 6 shows the positions of five spatial candidates and two middle spatial candidates. The spatial candidates are inserted by following the order {A1, B1, B0, A0}. Spatial candidate B2 is only inserted when the number of merge candidates in the current list is smaller than a threshold: 7 if alternative temporal motion vector prediction (ATMVP) is enabled; 5, otherwise. For middle spatial candidates, two more candidates are inserted from the middle positions of left and top edges by following the order {L, T}. Multi-average candidate is generated by averaging all candidates with reference indices equal to 0 in the current list, and the averaged motion vectors are calculated separately for each reference list.

Figure 6: Positions for spatial and temporal candidates

#### Temporal type candidates

The temporal type candidates consist of temporal candidates, ATMVP candidates, and pair-wise average candidates. Figure 6 shows the positions of four temporal candidates. The temporal candidates are inserted by following the order {RB, CT, LB, RT}. ATMVP is basically the same as JEM-7.0; however, there are two differences. First, two temporal vectors instead of one are used to generate two ATMVP candidates. Second, the derivations for the temporal vectors differ. For the first temporal vector, it is derived by averaging all available motion vectors with the same reference frame as the first available motion vector. The second temporal vector is the second available motion vector. The definitions of first and second available motion vectors are shown in Table 2, where list 0 is represented as L0 and list 1 is represented as L1. Moreover, the proposed ATMVP stops initial vector scaling when the reference index is larger than or equal to 4 for reducing the number of scaling operations in the worst case, though the worst case does not happen in the CfP tests. Pair-wise average candidates are generated by averaging every pair of candidates in the current list using a predefined order, and the averaged motion vectors are calculated separately for each reference list. For example, if there are 4 candidates in the current list, the pair order is (0, 1) (1, 0) (0, 2) (2, 0) (1, 2) (2, 1) (0, 3) (3, 0) (1, 3) (3, 1) (2, 3) (3, 2), where 4 candidates are indexed from 0 to 3.

Table 2: Definitions of first and second available motion vectors

| Prediction direction of first candidate | First available motion vector | Second available motion vector |
| --- | --- | --- |
| Bi | Motion vector of L0 | Motion vector of L1 |
| Uni (L0/L1) | Motion vector of L0/L1 | Motion vector of second candidate (default is L0 first then L1, or decided by a slice flag) |

#### Affine type candidates

The affine type candidates consist of spatial inherited affine candidates, middle spatial inherited affine candidates, and corner derived affine candidates. Spatial inherited affine candidates insert spatial candidates which are coded using affine mode with the order {A1, B1, B0, A0, B2} as shown in Figure 6. For example, if the left block A1 is coded with affine mode, the control points of the CU which contains block A1 are fetched. And control points of the current CU are derived according to the A1 control points. The definitions of control points are disclosed in the following section 2.4.2. Middle spatial inherited affine candidates insert spatial candidates at the middle of left and top edges which are coded using affine mode with the order {L, T}. Then it follows the same procedure to derive affine parameters as spatial inherited affine candidates. Corner derived affine candidates simply set three spatial candidates as control points. Available motion triplets of spatial neighbors with the same reference frame are inserted by following the order {(B2, A1, B1), (B2, A0, B1), (B2, A1, B0), (B2, A0, B0)} for each reference list.

### Affine inter mode

A four-parameter (two-control-point) affine inter mode is used to predict the content with the motion model of zoom-in/out and rotation. As shown in Figure 7, the affine motion field of the block is described by two-control-point motion vectors.

Figure 7: Two-control-point affine motion model.

The motion vector field (MVF) of a block is described by the following equation:

,

(2)

where (v0x, v0y) is the motion vector of the top-left corner control point, (v1x, v1y) is the motion vector of the top-right corner control point, and w is the CU width. The block-based motion compensation is used. The current CU is divided into several 4x4 sub-blocks. The motion vector of each sub-block is calculated by Equ. (2). The luma motion vector precision is rounded to 1/16-sample precision. In HEVC, the advanced motion vector prediction (AMVP) mode needs to signal a motion vector prediction (MVP) index and motion vector differences (MVDs). When the AMVP mode is applied in this proposal, an affine_flag is signaled to indicate whether the affine prediction is used. If the affine prediction is applied, the syntax of inter_dir, ref_idx, mvp_index, and two MVDs (mvd_x and mvd_y) are signaled. An affine MVP pair candidate list containing two affine MVP pairs is generated. The signaled mvp_index is used to select one of them. The affine MVP pair is generated by two kinds of affine MVP candidates. One is the spatial inherited affine candidate, and the other is the corner derived affine candidate. If the neighboring CUs are coded with affine mode, the spatial inherited affine candidates can be generated. The affine motion model of the neighboring affine coded block is used to generate the motion vectors of the two-control-point MVP pair. The MVs of the two-control-point MVP pair of the spatial inherited affine candidate are derived by using the following equations.

V0x = VB0x + (VB2_x - VB0x ) * ( posCurCU_Y - posRefCU_Y ) / RefCU_height
+ (VB1x - VB0x ) * (posCurCU_X - posRefCU_X) / RefCU_width

(3)

V0y = VB0y + (VB2_y - VB0y ) * (posCurCU_Y - posRefCU_Y) / RefCU_height
+ (VB1y - VB0y ) * (posCurCU_X - posRefCU_X) / RefCU_width    ,

(4)

where VB0, VB1, and VB2 can be replaced by the top-left MV, top-right MV, and bottom-left MV of any reference/neighboring CU, (posCurCU_X, posCurCU_Y) is the position of the top-left sample of the current CU relative to the top-left sample of the frame, (posRefCU_X, posRefCU_Y) is the position of the top-left sample of the reference/neighboring CU relative to the top-left sample of the frame.

V1x = VB0x + (VB1x - VB0x) * CU_width / RefCU_width

(5)

V1y = VB0y + (VB1y - VB0y) * CU_width / RefCU_width

(6)

If the number of the MVP pairs is smaller than 2, the corner derived affine candidate is used. The neighboring motion vectors, as shown in Figure 8, are used to derive the affine MVP pair. For the first corner derived affine candidate, the first available MV in set A (A0, A1, and A2) and first available MV in set B (B0 and B1) are used to construct the first MVP pair. For the second corner derived affine candidate, the first available MV in set A and first available MV in set C (C0 and C1) are used to calculate the MV of top-right control point. The first available MV in set A and the calculated top-right control point MV are the second MVP pair.

Figure 8: The neighboring MV of a current block.

### Pattern-matched motion vector refinement

In the pattern-matched motion vector derivation (named PMMVD in JEM encoder description, shortened to PMVD in this document) of JEM, the decoder needs to evaluate several motion vector (MV) candidates to determine a starting MV candidate for CU-level search. In sub-CU-level search, in addition to the best CU-level MV, several MV candidates are added. The decoder needs to evaluate these MV candidates to find the best MV, which requires a lot of memory bandwidth. In the proposed pattern-matched motion vector refinement (PMVR), the concepts of template matching and bilateral matching in PMVD in JEM are adopted. One PMVR_flag is signaled when skip mode or merge mode is selected to indicate PMVR is enabled or not. To reduce the memory bandwidth requirement significantly in comparison with PMVD, a MV candidate list is generated, and a starting MV candidate index is explicitly signaled if PMVR is applied.

The candidate list is generated by using merge candidate list generation process, but the sub-CU merge candidates, e.g., the affine candidates and ATMVP candidates, are excluded. For bilateral matching, only the uni-prediction MV candidate is included. A bi-prediction MV candidate is divided into two uni-prediction MV candidates. Also, similar MV candidates (MV differences smaller than a predefined threshold) are also removed. For the CU-level search, a diamond search MV refinement is performed starting from the signaled MV candidate.

The sub-CU-level search is only enabled for the bilateral matching merge mode. To reduce memory bandwidth, only the MV determined from the CU-level search is evaluated. The search window of the sub-CU-level search for all sub-CUs is the same as the search window of the CU-level search. Therefore, no additional bandwidth is required for sub-CU-level search.

The template matching is also used to refine the MVP in AMVP mode. In AMVP mode, two MVPs are generated by using HEVC MVP generation process, and one MVP index is signaled to select one of them. The selected MVP is further refined by using template matching in PMVR. If the adaptive motion vector resolution (AMVR) is applied, the MVP is rounded to the corresponding precision before template matching refinement. This refinement process is named as pattern-matched motion vector predictor refinement (PMVPR). In the rest of this document, if not particularly specified, PMVR includes template matching PMVR, bilateral matching PMVR, and PMVPR.

To reduce the memory bandwidth requirement, the PMVR is disabled for 4x4, 4x8, and 8x4 CUs. To further reduce the memory bandwidth requirement, the search range of {template matching, bilateral matching} for CU area equal to 64 is reduced to {±2, ±4}, and the search range of {template matching, bilateral matching} for CU area larger than 64 is reduced to {±6, ±8}. By using all the above methods described in this PMVR section, the required memory bandwidth is reduced from 45.9x in PMVD of JEM-7.0 to 3.1x in PMVR, compared to the worst case in HEVC.

### Merge assistant prediction

In this proposal, merge assistant prediction (MAP) is proposed to improve the existing prediction modes in HEVC inter frame, including uni-prediction of AMVP mode, skip mode, merge mode, and intra mode. The general concept of MAP is to combine an existing prediction mode with an extra merge indexed prediction. The merge indexed prediction is performed as in merge mode, where a merge index is signaled to acquire motion information for the motion compensated prediction. The final prediction is the weighted average of the merge indexed prediction and the prediction generated by the existing prediction mode, where different weights are applied depending on the combinations.

When applied to improve uni-prediction of AMVP mode, one MAP flag is signaled to enable or disable MAP for inter_dir equal to 0 or 1, where 0, 1, and 2 represent list 0, list 1, and bi-prediction, respectively. Moreover, one more merge index is signaled when the MAP flag is true. In this way, MAP turns uni-prediction into bi-prediction, where one motion is acquired using the original syntax elements in AMVP mode while the other is acquired using the merge scheme, as shown in Figure 9. The final prediction uses 1:1 weights to combine these two predictions as in bi-prediction. The merge candidate list is first derived from merge mode with sub-CU candidates (e.g., affine, ATMVP) excluded. Next, it is separated into two individual lists, one for list 0 containing all list 0 motions from the candidates, and the other for list 1 containing all list 1 motions. After rearrangement for filling vacancy and removing redundancy, two merge lists are generated for list 0 and list 1 respectively, as shown in Figure 10. The maximum number of merge candidates is set to 4.

Figure 9: Illustration of MAP when applied to uni-prediction.

Figure 10: Example of merge list construction for MAP when applied to uni-prediction of AMVP.

When applied to improve skip or merge mode, whether to enable MAP is explicitly signaled. An extra merge indexed prediction is selected in addition to the original one. However, it is implicitly decided as the succeeding merge candidate (i.e., the already signaled merge index plus one) without signaling any additional merge index. Therefore, motions from a pair of consecutive two merge candidates are acquired to generate the final prediction, where 5:3 weights are applied to the 1st and 2nd merge indexed predictions, respectively. Moreover, sub-CU candidates (e.g., affine, ATMVP) are excluded from the pair.

When applied to improve intra mode, MAP combines one intra prediction and one merge indexed prediction. It can be achieved using two variations of signaling for both inter and intra CUs as follows. In an inter CU, one MAP flag is signaled for merge mode, and one intra mode index is additionally signaled to select an intra mode from an intra candidate list when the MAP flag is true. The intra candidate list is derived from 19 intra prediction modes including DC and planar modes, and 17 angular modes which are subsampled from 65 angular modes. For an intra CU, one MAP flag is signaled for luma component, and one merge index is additionally signaled to select a merge mode from a merge candidate list when the MAP flag is true. The merge candidate list is derived from merge modes with sub-CU candidates (e.g., affine, ATMVP) excluded and the merge candidate list size is set to 4. In both of these two signaling schemes, one intra prediction selected by the intra mode index and one merge indexed prediction selected by the merge index are combined using weighted average. For chroma, DM is always applied without extra signaling. When DC or planar mode is selected or the CU width or height is smaller than 4, equal weights are applied. For other angular modes, they are first divided into two groups, which are near HOR or near VER. Next, horizontal or vertical separate regions are given for these two groups to define different weight ratios for combining intra and merge indexed predictions as shown in Figure 11. For regions (1) and (2), which are closer to the reference samples in the neighboring CU, the weights for merge indexed prediction are smaller than those for intra prediction. For regions (3) and (4), which are relatively far away from the reference samples in the neighboring CU, the weights for merge indexed prediction are larger than those for intra prediction. And for all regions, the weights for merge indexed prediction get larger while those for intra prediction get smaller when the current to-be-processed sample gets far from the reference sample.

Figure 11: Weights for combining intra and merge indexed predictions

### Adaptive motion vector resolution

Locally adaptive motion vector difference resolutions (LAMVR) is adopted in JEM, and it supports three kinds of motion vector resolutions, including quarter-luma samples, integer-luma samples, and four-luma samples, to reduce the side information of motion vector differences (MVDs). In this contribution, one modification of releasing the constraint of signaling LAMVR flag is added on top of LAMVR and this tool is renamed as adaptive motion vector resolution (AMVR). In JEM, LAMVR flag is used to indicate the resolution of MVD, so this flag is coded only when there is at least one non-zero MVD in AMVP mode. Otherwise (when LAMVR flag is not presented), the resolution of MVD and motion vector predictor (MVP) is inferred as quarter-luma samples. In AMVR, the AMVR flag is always coded in AMVP mode to indicate the selection of MVD and MVP resolution, even if the coded MVD is zero.

### Local illumination compensation

Local illumination compensation (LIC) is supported and the same as JEM-7.0.

### Generalized bi-prediction

Generalized bi-prediction (GBi) is proposed in JVET-C0047. The concept of GBi is to allow using different weights for predictors from L0 and L1, respectively, instead of using equal weights in traditional bi-prediction. The predictor generation can be summarized in Equ. (7) and (8).

PTraditionalBiPred = (PL0 + PL1 + RoundingOffset) >> shiftNum

(7)

PGBi = (w0 * PL0 + w1 * PL1 + RoundingOffsetGBi) >> shiftNumGBi    ,

(8)

where PTraditionalBiPred and PGBi are final predictors for traditional bi-prediction and GBi respectively, PL0 and PL1 are predictors from L0 and L1, w0 and w1 are selected GBi weights for L0 and L1, and RoundingOffset, shiftNum, RoundingOffset1, and shiftNum1 are used to normalize the final predictors in traditional bi-prediction and GBi respectively. The supported weights of w0 and w1 are {6, 5, 4, 3, 2} and {2, 3, 4, 5, 6}, which is the same as the setting of EE3-Test2 in JVET-D0102, and the binarization is the same as that in random access (RA) case. For AMVP mode, the weight selection in GBi is explicitly signaled for one CU if this CU is coded by bi-prediction. For merge mode, the weight selection is inherited from the merge candidate, except the bilateral matching mode in PMVR. In the bilateral matching mode in PMVR, only equal weight is allowed.

### Decoder-side motion vector refinement

Decoder-side motion vector refinement (DMVR) is adopted in JEM and used to refine the motion vectors of merge candidates. In this contribution, DMVR is utilized to refine not only the motion vectors of merge candidates but also those of template matching mode in PMVR. Moreover, the half-sample refinement process is allowed in DMVR after integer-sample refinement. One early termination mechanism is also introduced to reduce the computation complexity of DMVR. If the motion vector in L0 after the refinement process is the same as that before the refinement process, then the refinement process in L1 is skipped.

### Bi-directional optical flow

Bi-directional optical flow (BIO) is proposed in VCEG-AZ07 and adopted in JEM. The concept of BIO is to utilize the assumption of optical flow and steady motion to further refine the final predictor in bi-prediction. The decoding process of the BIO in this proposal is similar to the BIO in JEM-7.0. A block-based design of BIO is used instead of the sample-based design. The block size is adaptively switched between 2x2 and 4x4, according to the video resolution and OBMC condition. If the video resolution is smaller than or equal to 720p or the current MC block is used for OBMC process, then the BIO subblock size is 2x2. Otherwise, the BIO subblock size is set to 4x4. Moreover, in order to reduce the computation complexity of BIO, the division operation and the gradient calculation are simplified as follows.

Assume the current CU size is equal to PxQ, and the BIO subblock size is MxM, as shown in Figure 12. The required gradients for one BIO subblock are reduced from (M+4)x(M+4) to (M+2)x(M+2). And the gradients in x and y directions are directly calculated by using the filter with coefficients equal to [-1, 0, 1] on MC results, instead of using one additional 6-tap gradient filter set and one new 6-tap interpolation filter set to generate gradients in JEM. Therefore, the size of the required MC samples become (P+4)x(Q+4), which increases memory bandwidth. In order to avoid increasing memory bandwidth, only the reference samples in the fractional sample MC region, (3+P+4)x(3+Q+4), are allowed to be accessed. If the reference samples out of this region are required, then the sample values are generated by padding instead of accessing real reference samples out of this region, as shown in Figure 12. As for the division operation, it is replaced by using right shift only.

Figure 12: The required memory access of BIO.

The concept of BIO is also extended to chroma components in this contribution. For chroma components, the displacements, vx and vy, are reused from luma component, and the gradient filter is also modified from 3-tap filter to 2-tap filter with coefficients equal to [-1, 1] at CU boundaries, to avoid the increase of memory bandwidth.

### Overlapped block motion compensation

Overlapped block motion compensation (OBMC) is supported and the same as JEM-7.0.

### Motion candidate reorder

For one CU, motion candidate reorder (MCR) reorders the possible motion candidates in a list to achieve efficient syntax representation. The reorder rule depends on the template matching costs which are calculated between the current CU and those to-be-tested motion candidates. Figure 13 illustrates the template matching process. The left-hand side shows that the template of the current CU is formed with reconstructed samples to the left and above of the current CU. The right-hand side shows that the template of the reference block pointed by integer-sample MV of the motion candidate is formed with reference samples to the left and above of the reference block. The template of the current CU and that of the motion candidate have the same shape and size. The lower the cost, the higher the probability for the candidate to be chosen. Finally, the candidates with smaller costs are put at the front of the list.

MCR is applied to merge mode, PMVR mode, and AMVP mode, where the possible motion candidates are defined in Table 3. The detailed operations for each mode are described in the following sections.

Figure 13: Illustration of template matching process for MCR

Table 3: Possible motion candidates for different modes in MCR.

| Mode | Possible Motion Candidates |
| --- | --- |
| Merge | Spatial merge candidates (non-LIC) |
| PMVR | PMVR merge candidates in the front (non-LIC + index <=4) |
| AMVP | MVD sign pairs: (++), (+-), (-+), (--) |

#### MCR for merge mode

For merge mode, the motion candidates are spatial candidates with LIC inherited flag equal to 0. The line number of the L-shape template is adaptively decided. If the CU area is smaller than or equal to 2048, one line on the above and one line on the left are used. Otherwise, two lines on the above and two lines on the left are used. The L-shape template matching uses sum of absolute differences (SAD) as cost. When template costs are determined for all motion candidates, the candidates with smaller costs are reordered to the front of the list.

#### MCR for PMVR mode

For PMVR mode, the motion candidates are all non-sub-CU candidates with LIC inherited flag == 0 but with merge index <= 4. The line number of the L-shape template is adaptively decided. If the CU area is smaller than or equal to 2048, one line on the above and one line on the left are used. Otherwise, two lines on the top and two lines on the left are used. The L-shape template matching uses SAD as cost. Moreover, cost adjustment is performed. The cost for bi-prediction candidates is multiplied by a constant less than 1, which can give higher priority to bi-prediction candidates than uni-prediction ones. After template costs are determined for all motion candidates, the candidates with smaller costs are reordered to the front of the list.

#### MCR for AMVP mode

For AMVP mode, motion vectors are formed by adding MVPs and MVDs. When the signs of MVDs are unkown, there are four possibilities for the final result if both components of MVD are non-zero. To apply MCR to AMVP mode, the possible motion candidates are the four motion vectors generated by all possible combinations of sign pairs, i.e., (++), (+-), (-+), and (--) for x and y components of MVD, respectively. The line number of the L-shape template is always one. The L-shape template matching uses SAD as cost. Instead of coding two signs using two fixed bins, the codeword is reordered depending on the template cost for each sign pair. When one sign pair with the smallest template cost is found, codeword for all sign pairs are determined as shown in Table 4. For example, if (+-) has the smallest template cost, the codeword is reordered as (0) for (+-), (10) for (++), (110) for (--) and (111) for (-+), respectively. Instead of bypass bins, context coding bins are used for coding signs of MVDs.

Table 4: Codeword assignment for sign pairs according to template costs.

| Smallest Template Cost position | Hit (0) | Vertical flip (10) |  |  | Horizontal flip (110) | Diagonal flip (111) |
| --- | --- | --- | --- | --- | --- | --- |
| (++) | (++) |  | (+ -) | (- +) |  | (- -) |
| (+ -) | (+ -) |  | (++) | (- -) |  | (- +) |
| (- +) | (- +) |  | (- -) | (++) |  | (+ -) |
| (- -) | (- -) |  | (- +) | (+ -) |  | (++) |

## Transform

Transforms consist of core transform and secondary transform. Compared to the core transform in HEVC, the core transform in this proposal includes the following new features: large block-size transform with high-frequency zeroing, adaptive multi-core transform. Moreover, both core transform and secondary transform contain transform syntax reorder.

### Large block-size transform with high-frequency zeroing

In this proposal, up to 128x128 transforms are enabled to improve coding efficiency. The high frequency transform coefficients are zeroed out for the transform blocks with size (width or height, or both width and height) larger than 32, so that only the low-frequency coefficients at the topmost 32 rows and the leftmost 32 columns of transform coefficients are maintained. However, when transform skip mode is used, zeroing out is not performed.

### Adaptive multi-core transform

The adaptive multi-core transform (AMT) can select the horizontal transform and vertical transform from DCT-II, DST-VII and flipped DST-VII (FDST-VII). Table 5 shows the basis functions for the three transforms. Note that, let fDST-VIIj be the transformed signal using DST-VII, then the flipped DST-VII is simply calculated as fDST-VIIN-1-j, where N is the transform size. An AMT flag is defined. AMT flag equal to “0” indicates that DCT-II is used for both horizontal transform and vertical transform, AMT flag equal to “1” indicates that another transform type is used depending on an indicator, AMT index.

Table 5:  Transform basis functions of DCT-II, DST-VII and flipped DST-VII for N-point input.

| Transform Type | Basis function Ti(j), i, j=0, 1,…, N−1 |
| --- | --- |
| DCT-II | Ti(j)=ω0∙2N∙cosπ∙i∙2j+12N where ω0=2Ni=01i≠0 |
| DST-VII | Ti(j)=42N+1∙sinπ∙(2i+1)∙(j+1)2N+1 |
| FDST-VII | Ti(j)=42N+1∙sinπ∙(2i+1)∙(N-j)2N+1 |

In this proposal, AMT is only allowed when both CU width and height are smaller than or equal to 64.

AMT flag is signaled when the number of non-zero coefficients is larger than a threshold. For inter CUs, the threshold is set to 2. For intra CUs, it is set to 0. AMT index is signaled if the number of non-zero coefficients is larger than 2; otherwise, it is inferred as 0.

Moreover, for a CU with width, W, and height, H, the number of AMT index, the corresponding AMT types, and the codewords are shown in Table 6. Note that for an inter CU with AMT enabled, when at least one of the W and H is larger than 4 and the CU area is smaller than or equal to 1024, the codeword for AMT index is dynamically generated with transform syntax reorder (TSR), which is described in section 2.5.4. The decoding processes to parse the AMT syntax for inter CUs and intra CUs are shown in Figure 14 (a) and (b), respectively.

Table 6: The transform sets for intra and inter coding and the mapping code words.

| CU |  |  | Number of AMT | AMT type | Code word |
| --- | --- | --- | --- | --- | --- |
| Intra CU | W > 4 and H > 4 |  | 3 | Index 0 (DST-VII, DST-VII) | 0 |
|  |  |  |  | Index 1 (DCT-II, DST-VII) | 10 |
|  |  |  |  | Index 2 (DST-VII, DCT-II) | 11 |
|  | W > 4 and H <= 4 |  | 2 | Index 0/1 | 0/1 |
|  | W <= 4 and H > 4 |  | 2 | Index 0/2 | 0/1 |
|  | W <= 4 and H <= 4 |  | 1 | Index 0 | - |
| Inter CU | Area > T3 (Area = W x H) (T3 = 1024) |  | 4 | Index 0 (FDST-VII, FDST-VII) | 00 |
|  |  |  |  | Index 1 (DST-VII, FDST-VII) | 10 |
|  |  |  |  | Index 2 (FDST-VII, DST-VII) | 01 |
|  |  |  |  | Index 3 (DST-VII, DST-VII) | 11 |
|  | Area <= T3 | W > 4 and H > 4 | 4 | Index 0/1/2/3 | Dynamic by TSR |
|  |  | W > 4 and H <= 4 | 2 | Index 0/1 |  |
|  |  | W <= 4 and H > 4 | 2 | Index 0/2 |  |
|  |  | W <= 4 and H <= 4 | 1 | Index 0 | - |

(a)

(b)

Figure 14: (a) The decoding process to parse the AMT syntax for an inter CU. (b) The decoding process to parse the AMT syntax for an intra CU.

### Non-separable secondary transform

The process of non-separable secondary transform (NSST) is the same as JEM-7.0, but the syntax is different. In intra slices, for an intra luma CU, an NSST index needs to be signaled when the number of non-zero coefficients of luma component is larger than 1, while for an intra chroma CU, an NSST index is signaled when the number of non-zero AC coefficients of chroma components is larger than 0. In inter slices, for an intra luma CU, an NSST index is signaled when the number of non-zero coefficients of luma plus the number of non-zero AC coefficients of chroma components is larger than 2, while for an intra chroma CU, the NSST index is dependent on luma and not signaled. When coding the NSST index, the codeword is the same as JEM-7.0. However, for an intra luma CU with area smaller than or equal to 512, the codeword is dynamic and has to be generated by transform syntax reorder (TSR), which is described in the next section.

### Transform syntax reorder

Transform syntax reorder (TSR) is proposed to re-map the syntax for signaling multiple transforms. For an inter CU with AMT enabled, TSR is provided to re-map the AMT index. As for an intra CU with NSST enabled, TSR is employed to re-map the NSST index. To re-map the index, costs are given to each transform, which is specified by AMT or NSST index. The one with the smallest cost is chosen as the predicted transform with the shortest codeword. For the rest of transforms, their codewords are assigned depending on the order created by sorting the costs. The TSR flowcharts for 4 AMT indexes and 4 NSST indexes are shown in the Figure 15 and Figure 16, respectively.

Figure 15: The TSR flowchart for 4 candidate AMT modes.

Figure 16: The TSR flowchart for 4 candidate NSST modes.

Boundary matching is used to generate the costs. Assuming at CU boundaries the reconstructed samples in the current CU are highly correlated with the reconstructed samples of the neighboring CUs, a cost is calculated by measuring the difference between the reconstructed samples of neighboring CUs and the reconstructed boundary samples of the current CU with one particular transform. For one TU, coefficients are de-quantized and inverse transformed with one particular transform to generate the corresponding reconstructed residuals. By adding those reconstructed residuals to the predictors, its reconstructed samples are acquired. Figure 17 demonstrates one example for a 4x4 CU with 4 candidate transforms and Equ. (9) is employed to compute the cost for each candidate transform.

Figure 17: The reconstruction of current CU with each candidate transform mode and the reconstruction of top and left neighboring CUs.

cost=x=0W-12px, -1 -px, -2-px,0+y=0H-12p-1, y -p-2, y-p0, y    ,

(9)

where W and H indicate the width and height. px, 0 and p0,y mean the reconstructed samples, generated with one particular transform, within the topmost row and leftmost column of the current CU. px, -1 and px, -2 mean the reconstructed samples within the two nearest rows of the above neighboring CU. p-1, y and p-2, y mean the reconstructed samples within the two nearest columns of the left neighboring CU. The samples used to calculate the cost are marked with the red dash lines. After sorting those costs of multiple transforms, the order is created, and the codewords for the corresponding candidate transform modes are decided. Table 7 shows an example for mapping 4 candidate transforms to their corresponding code words, where the cost for index 3 is the smallest followed by index 0, 1, 2.

Table 7: The coded indices for 4 candidate transforms with the corresponding real transform mode indices and codewords

| Mapped transform index | Real transform index | The codeword |
| --- | --- | --- |
| 0 | 3 | 0 |
| 1 | 0 | 10 |
| 2 | 1 | 110 |
| 3 | 2 | 111 |

For inter CUs with AMT enabled, TSR is applied when the AMT flag is equal to 1 and the CU area is smaller than or equal to 1024. As for intra CUs with NSST enabled, TSR is applied when the CU area is smaller than or equal to 512.

## Quantization

Our proposal uses the same quantization method as HEVC. However, the maximum quantization parameter (QP) in this proposal is further extended to 57 instead of being 51. Furthermore, a modified mapping table, as shown in Table 8, is employed for deriving the chroma quantization parameter, QpC, from the associated luma quantization parameter, qPi.

Table 8. Specification of QpC as a function of qPi.

| qPi | < 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | > 43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | = qPi − 2 |

## Reconstruction

### Bilateral filter

The process of bilateral filter in this contribution is almost the same as that in JEM-7.0. Bilateral filter is applied between reconstruction stage and deblocking filter to improve the reconstructed samples, especially for small CUs with non-zero residuals. The difference between the JEM version and this version is the table of center weight coefficients. The proposed center weight coefficients are shown in Table 9.

Table 9. The center weight values based on the prediction mode and CU area size.

|  | CU Area |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 16 | 32 | 64 | >=128 and <256 | >=256 |
| Intra Prediction | 65 | 65 | 81 | 113 | 113 |
| Inter Prediction | 81 | 113 | 113 | 196 |  |

### Content adaptive clipping

Content adaptive clipping is proposed in JVET-C0040, and it is also adopted in JEM. In this contribution, this technology is also used to replace the clipping operation in HEVC. The signaling method and operations are the same as those in JEM-7.0.

## In-loop filters

In this contribution, there are five in-loop filters in the video codec system. The processing order is deblocking filter (DF), non-local mean loop filter (NLMLF), sample adaptive offset (SAO), adaptive loop filter (ALF), and convolution neural network loop filter (CNNLF), sequentially. In the following subsections, these five in-loop filter are introduced one by one.

### Deblocking filter

In general, deblocking filter (DF) consists of 2 main parts: filtering decisions and filtering operations. Filtering decisions are used to determine the filter types (e.g., no filtering, or weak/strong filtering) for each block edge. Filtering operations are then applied to filter the samples at both sides of block boundaries based on the filtering decisions. In this proposal, the DF is similar to that in HEVC, however, two major extensions are proposed. First, to handle the blocky artifacts for large blocks, length-adaptive filtering with three new long tap filters for HEVC strong filtering cases is proposed. Second, a modification to support parallel processing for 4xN or Nx4 blocks is proposed.

#### Length-adaptive filtering for strong filtering

The filtering decisions for deciding weak and strong filters are the same as those in HEVC. However, 3 new long tap filters based on side lengths are added for strong filtering, where lengths refer to CU width and height for vertical and horizontal edges, respectively. The decisions and operations are described as follows.

##### Filter decisions of length-adaptive filtering

When the following conditions are true, the proposed length-adaptive filtering is applied:

- Strong filter is applied

- Slice type is inter-slice

- The gap value which represents the smoothness of the signal on both sides of the block boundary is larger than a threshold

The gap value is defined as p0i-q1i+p1i-q0i≫1, where i = 0 and 3. That is, for each block boundary, only the first and fourth lines of 4-sample length are used to obtain the gap value. The threshold value, gap_th, is derived from Figure 18 and depends on QP values.

Figure 18. Illustration of gap_th values depending on QP.

If the proposed length-adaptive filtering is applied, three new filters, L4, L8, and L16 filters, are adaptively selected according to the following rules.

If “lenp≥128 and lenq≥32, or lenp≥32 and lenq≥128” and Cond. (10) is true (meaning the “sum of absolute differences between the 4th sample and a farther sample from the boundary on the first line and the 4th line” is small; also meaning the samples that are more than 4 samples away from the boundary are smooth and close to the 4th sample), then the current 4-sample length boundary is labeled as L16/L8; otherwise, the current 4-sample length boundary is labeled as L8/L4/HEVC-Strong. The notations lenp and lenq are the CU lengths at two sides, and evaluation of Cond. (10) needs calculation of Equ. (11).

s=4m-1psi-p3i+t=4n-1qti-q3i<ssth    ,

(10)

where ssth=β×m-4+n-4≫4    ,

(11)

i = 0 and 3, and m = 16 and n = 16, if lenp≥128 and lenq≥128 m = 16 and n = 8 ,if lenp≥128 and 128>lenq m = 8 and n = 16, if 128>lenp and lenq≥128,

Next, filter decision coherence between neighboring 4-sample length boundaries is checked using a sliding window approach, where for simplicity in the following a 4-sample length boundary is also called a segment. 4-segment sliding window checks for segments are first performed and followed by 2-segment sliding window checks for segments. For 4-segment sliding window checks, when a current segment is labeled as L16/L8, as illustrated in Figure 19, if the current segment is “d”, then four windows, {a, b, c, d}, {b, c, d, e}, {c, d, e, f}, and {d, e, f, g}, are further checked. If none of the four windows is entirely labeled as L16/L8 for its four segments, the current segment is re-labeled to L8/L4/HEVC-Strong. For 2-segment sliding window checks, when a current segment is labeled as L8/L4/HEVC-Strong, as illustrated in Figure 19, two windows, {c, d} and {d, e} are further checked. If none of the two windows is entirely L8/L4 for its two segments, the current segment is re-labeled to HEVC-Strong.

Figure 19. An example of checking filtering decision coherence. “d” is the current 4-sample length boundary (or also called current segment), and P and Q are the two sides of the block boundary.

If a segment is labeled as L16/L8, the following filter selection process is applied.

- For p-side, if lenp≥128, then L16 is applied; otherwise, L8 is applied.

- For q-side, if lenq≥128, then L16 is applied; otherwise, L8 is applied.

If a segment is labeled as L8/L4, the following filter selection process is applied.

- For p-side, if lenp≥16, then L8 is applied. If 16>lenp≥8, then L4 is applied. Otherwise, the strong filtering in HEVC is applied.

- For q-side, if lenq≥16, then L8 is applied. If 16>lenq≥8, then L4 is applied. Otherwise, the strong filtering in HEVC is applied.

If a segment is labeled as HEVC-Strong, the strong filtering in HEVC is applied.

##### Filter operations of length-adaptive filtering

If the proposed L16 is selected at p-side, then the filtered sample values from p'15i to p'0i are calculated as follows:

p'15i=18p15i+2p14i+2p13i+p12i+p11i+p10i+p9i+p8i+p7i+p6i+p5i+p4i+p3i+16≫5
p'14i=2p15i+16p14i+2p13i+2p12i+p11i+p10i+p9i+p8i+p7i+p6i+p5i+p4i+p3i+p2i+16≫5
p'13i=2p15i+2p14i+14p13i+2p12i+2p11i+p10i+p9i+p8i+p7i+p6i+p5i+p4i+p3i+p2i+p1i+16≫5
p'12i=p15i+2p14i+2p13i+13p12i+2p11i+2p10i+p9i+p8i+p7i+p6i+p5i+p4i+p3i+p2i+p1i+p0i+16≫5
p'11i=p15i+p14i+2p13i+2p12i+12p11i+2p10i+2p9i+p8i+p7i+p6i+p5i+p4i+p3i+p2i+p1i+p0i+q0i+16≫5
p'10i=p15i+p14i+p13i+2p12i+2p11i+11p10i+2p9i+2p8i+p7i+p6i+p5i+p4i+p3i+p2i+p1i+p0i+q0i+q1i+16≫5
p'9i=p15i+p14i+p13i+p12i+2p11i+2p10i+10p9i+2p8i+2×p7i+p6i+p5i+p4i+p3i+p2i+p1i+p0i+q0i+q1i+q2i+16≫5
p'8i=p15i+p14i+p13i+p12i+p11i+2p10i+2p9i+9p8i+2p7i+2×p6i+p5i+p4i+p3i+p2i+p1i+p0i+q0i+q1i+q2i+q3i+16≫5
p'7i=p15i+p14i+p13i+p12i+p11i+p10i+2p9i+2p8i+8p7i+2p6i+2p5i+p4i+p3i+p2i+p1i+p0i+q0i+q1i+q2i+q3i+q4i+16≫5
p'6i=p15i+p14i+p13i+p12i+p11i+p10i+p9i+2p8i+2p7i+7p6i+2p5i+2p4i+p3i+p2i+p1i+p0i+q0i+q1i+q2i+q3i+q4i+q5i+16≫5
p'5i=p15i+p14i+p13i+p12i+p11i+p10i+p9i+p8i+2p7i+2p6i+6p5i+2p4i+2p3i+p2i+p1i+p0i+q0i+q1i+q2i+q3i+q4i+q5i+q6i+16≫5
p'4i=p15i+p14i+p13i+p12i+p11i+p10i+p9i+p8i+p7i+2p6i+2p5i+5p4i+2p3i+2p2i+p1i+p0i+q0i+q1i+q2i+q3i+q4i+q5i+q6i+q7i+16≫5
p'3i=p15i+p14i+p13i+p12i+p11i+p10i+p9i+p8i+p7i+p6i+2p5i+2p4i+4p3i+2p2i+2p1i+p0i+q0i+q1i+q2i+q3i+q4i+q5i+q6i+q7i+q8i+16≫5
p'2i=p14i+p13i+p12i+p11i+p10i+p9i+p8i+p7i+p6i+p5i+2p4i+2p3i+4p2i+2p1i+2×p0i+q0i+q1i+q2i+q3i+q4i+q5i+q6i+q7i+q8i+q9i+16≫5
p'1i=p13i+p12i+p11i+p10i+p9i+p8i+p7i+p6i+p5i+p4i+2p3i+2p2i+4p1i+2p0i+2q0i+q1i+q2i+q3i+q4i+q5i+q6i+q7i+q8i+q9i+q10i+16≫5
p'0i=p12i+p11i+p10i+p9i+p8i+p7i+p6i+p5i+p4i+p3i+2p2i+2p1i+4p0i+2q0i+2q1i+q2i+q3i+q4i+q5i+q6i+q7i+q8i+q9i+q10i+q11i+16≫5
If the proposed L16 is selected at q-side, then the filtered sample values from q'15i to q'0i are calculated as follows:

q'15i=18q15i+2q14i+2q13i+q12i+q11i+q10i+q9i+q8i+q7i+q6i+q5i+q4i+q3i+16≫5
q'14i=2q15i+16q14i+2q13i+2q12i+q11i+q10i+q9i+q8i+q7i+q6i+q5i+q4i+q3i+q2i+16≫5
q'13i=2q15i+2q14i+14q13i+2q12i+2q11i+q10i+q9i+q8i+q7i+q6i+q5i+q4i+q3i+q2i+q1i+16≫5
q'12i=q15i+2q14i+2q13i+13q12i+2q11i+2q10i+q9i+q8i+q7i+q6i+q5i+q4i+q3i+q2i+q1i+q0i+16≫5
q'11i=q15i+q14i+2q13i+2q12i+12q11i+2q10i+2q9i+q8i+q7i+q6i+q5i+q4i+q3i+q2i+q1i+q0i+p0i+16≫5
q'10i=q15i+q14i+q13i+2q12i+2q11i+11q10i+2q9i+2q8i+q7i+q6i+q5i+q4i+q3i+q2i+q1i+q0i+p0i+p1i+16≫5
q'9i=q15i+q14i+q13i+q12i+2q11i+2q10i+10q9i+2q8i+2q7i+q6i+q5i+q4i+q3i+q2i+q1i+q0i+p0i+p1i+p2i+16≫5
q'8i=q15i+q14i+q13i+q12i+q11i+2q10i+2q9i+9q8i+2q7i+2q6i+q5i+q4i+q3i+q2i+q1i+q0i+p0i+p1i+p2i+p3i+16≫5
q'7i=q15i+q14i+q13i+q12i+q11i+q10i+2q9i+2q8i+8q7i+2q6i+2q5i+q4i+q3i+q2i+q1i+q0i+p0i+p1i+p2i+p3i+p4i+16≫5
q'6i=q15i+q14i+q13i+q12i+q11i+q10i+q9i+2q8i+2q7i+7q6i+2q5i+2q4i+q3i+q2i+q1i+q0i+p0i+p1i+p2i+p3i+p4i+p5i+16≫5
q'5i=q15i+q14i+q13i+q12i+q11i+q10i+q9i+q8i+2q7i+2q6i+6q5i+2q4i+2q3i+q2i+q1i+q0i+p0i+p1i+p2i+p3i+p4i+p5i+p6i+16≫5
q'4i=q15i+q14i+q13i+q12i+q11i+q10i+q9i+q8i+q7i+2q6i+2q5i+5q4i+2q3i+2q2i+q1i+q0i+p0i+p1i+p2i+p3i+p4i+p5i+p6i+p7i+16≫5
q'3i=q15i+q14i+q13i+q12i+q11i+q10i+q9i+q8i+q7i+q6i+2q5i+2q4i+4q3i+2q2i+2q1i+q0i+p0i+p1i+p2i+p3i+p4i+p5i+p6i+p7i+p8i+16≫5
q'2i=q14i+q13i+q12i+q11i+q10i+q9i+q8i+q7i+q6i+q5i+2q4i+2q3i+4q2i+2q1i+2q0i+p0i+p1i+p2i+p3i+p4i+p5i+p6i+p7i+p8i+p9i+16≫5
q'1i=q13i+q12i+q11i+q10i+q9i+q8i+q7i+q6i+q5i+q4i+2q3i+2q2i+4q1i+2q0i+2q0i+p1i+p2i+p3i+p4i+p5i+p6i+p7i+p8i+p9i+p10i+16≫5
q'0i=q12i+q11i+q10i+q9i+q8i+q7i+q6i+q5i+q4i+q3i+2q2i+2q1i+4q0i+2p0i+2p1i+p2i+p3i+p4i+p5i+p6i+p7i+p8i+p9i+p10i+p11i+16≫5

If the proposed L8 is selected at p-side, then the filtered sample values from p'6i to p'0i are calculated as follows:

p'6i=3p7i+4p6i+3p5i+2p4i+p3i+p2i+p1i+p0i+8≫4
p'5i=p7i+3p6i+4p5i+2p4i+p3i+p2i+p1i+p0i+q0i+q1i+8≫4
p'4i=p7i+2p6i+2p5i+3p4i+2p3i+2p2i+p1i+p0i+q0i+q1i+8≫4
p'3i=p6i+p5i+2p4i+3p3i+3p2i+2p1i+p0i+q0i+q1i+q2i+8≫4
p'2i=p5i+p4i+2p3i+3p2i+3p1i+2p0i+q0i+q1i+q2i+q3i+8≫4
p'1i=p5i+p4i+2p3i+2p2i+2p1i+2p0i+2q0i+2q1i+q2i+q3i+8≫4
p'0i=p4i+2p3i+2p2i+2p1i+2p0i+2q0i+2q1i+q2i+q3i+q4i+8≫4

If the proposed L8 is selected at q-side, then the filtered sample values from q'6i to q'0i are calculated as follows:

q'6i=3q7i+4q6i+3q5i+2q4i+q3i+q2i+q1i+q0i+8≫4
q'5i=q7i+3q6i+4q5i+2q4i+q3i+q2i+q1i+q0i+p0i+p1i+8≫4
q'4i=q7i+2q6i+2q5i+3q4i+2q3i+2q2i+q1i+q0i+p0i+p1i+8≫4
q'3i=q6i+q5i+2q4i+3q3i+3q2i+2q1i+q0i+p0i+p1i+p2i+8≫4
q'2i=q5i+q4i+2q3i+3q2i+3q1i+2q0i+p0i+p1i+p2i+p3i+8≫4
q'1i=q5i+q4i+2q3i+2q2i+2q1i+2q0i+2p0i+2p1i+p2i+p3i+8≫4
q'0i=q4i+2q3i+2q2i+2q1i+2q0i+2p0i+2p1i+p2i+p3i+p4i+8≫4

If the proposed L4 is selected at p-side, then the filtered sample values from p'3i to p'0i are calculated as follows:

p'3i=5p3i+4p2i+3p1i+p0i+q0i+q1i+q2i+8≫4
p'2i=3p3i+4p2i+3p1i+2p0i+q0i+q1i+q2i+q3i+8≫4
p'1i=2p3i+3p2i+3p1i+2p0i+2q0i+2q1i+q2i+q3i+8≫4
p'0i=2p3i+2p2i+3p1i+2p0i+2q0i+2q1i+2q2i+q3i+8≫4

If the proposed L4 is selected at q-side, then the filtered sample values from q'3i to q'0i are calculated as follows:

q'3i=5q3i+4q2i+3q1i+q0i+p0i+p1i+p2i+8≫4
q'2i=3q3i+4q2i+3q1i+2q0i+p0i+p1i+p2i+p3i+8≫4
q'1i=2q3i+3q2i+3q1i+2q0i+2p0i+2p1i+p2i+p3i+8≫4
q'0i=2q3i+2q2i+3q1i+2q0i+2p0i+2p1i+2p2i+p3i+8≫4

Finally, the clipping function is applied to filtered sample values, which is the same as those in HEVC and bounds the change of sample value within -2tc to 2tc.

#### Parallel deblocking process for 4xN or Nx4 blocks

To support parallel deblocking for 4xN and Nx4 blocks, p1i and q1i are used to replace the pixels outside p1i and q1i when doing filtering decision and operations. For example, the Equ. (12) is the condition for deciding whether the deblocking filter is applied or not.

p20-2×p10+p00+p23-2×p13+p03+q20-2×q10+q00+q23-2×q13+q03<β

(12)

If the lengths of two sides are both 4, then the filtering decision is changed to

p10-2×p10+p00+p13-2×p13+p03+q10-2×q10+q00+q13-2×q13+q03<β

(13)

If only the length of p side is 4, then the filtering decision is changed to

p10-2×p10+p00+p13-2×p13+p03+q20-2×q10+q00+q23-2×q13+q03<β

(14)

If only the length of q side is 4, then the filtering decision is changed to

p20-2×p10+p00+p23-2×p13+p03+q10-2×q10+q00+q13-2×q13+q03<β

(15)

### Non-local mean loop filter

Non-local mean (NLM) is often applied in image denoising. In this contribution, non-local mean is utilized as one in-loop filter to reduce the compression error in the video coding system and is called non-local mean loop filter (NLMLF). The basic flow of NLMLF is described as follows. First, the current frame is divided into non-overlapping 8x8 patches. Based on the received on/off flags, NLMLF is performed on those enabled patches. For each enabled patch, one search region with range equal to [-16, 16] x [-16, 16] in units of luma samples is allowed, and 15 best matched reference patches are found in this search range, where matching criterion is sum of squared differences (SSD) between current patch samples and reference patch samples. One current patch and 15 best reference patches are formed as one patch group. Two denoising methods, NLM and singular value decomposition with low-rank optimization (LowRank-SVD), are conditionally applied on patch groups. After applying NLM or LowRank-SVD, one filtered patch group with corresponding weights are derived, and the weighted filtered value and the corresponding weight of one sample in this patch group are accumulated into one frame buffer according to the sample position. After processing all enabled patches, each final NLMLF output sample value is derived by normalizing the accumulation of weighted filtered values with the accumulation of weights. The decoding process of each step in details is introduced as follows.

The 15 best reference patches are found by using multi-subsampling pattern search (MSPS). In MSPS, the [-16, 16] x [-16, 16] search region is centered at zero motion vector (MV) and divided into two sub-regions and different subsampling ratios are used in different regions. Sub-region 1 is within [-4, 4] x [-4, 4] and searched by using a search position subsampling ratio equal to 2 in each direction (motion vector x % 2 == 0 && motion vector y % 2 == 0). Sub-region 2 ([-16, 16] x [-16, 16] without sub-region 1) is searched by using a search position subsampling ratio equal to 4 (motion vector x % 4 == 0 && motion vector y % 4 == 0). After finding 15 initial best candidates, a refinement process is performed. For initial candidates located in sub-region 1, a refinement range, [-1, 1] x [-1, 1], centered at the initial candidate, is applied. For initial candidates located in sub-region 2, a refinement range, [-2, 2] x [-2, 2], centered at the initial candidate, is applied. Only integer MVs are searched, and the search algorithm is only applied to luma component. For chroma components, luma MVs are reused after performing MV scaling according to the chroma subsampling format.

Two methods can be used to modify samples in one patch group, NLM and LowRank-SVD. The selection is dependent on the SSD calculated in the search process. If the SSD is small, then NLM is used. Otherwise, LowRank-SVD is used. In NLM, the filtered samples and weights in the current patch are generated by the following equations,

AccFilteredSampleInCurPatch [m, n]= i=015weighti[m, n]*RecSampleim, n ,

(16)

AccFilteredWeightInCurPatch m, n=i=015weighti[m, n]  ,

(17)

where [m, n] is the position of a to-be-processed sample in one patch, RecSampleim, n is the sample value at position [m, n] in the ith patch, weighti[m, n] is the corresponding weight of to-be-processed samples. As for the samples in the reference patch, the filtered results and weights are generated by the following equations,

AccFilteredSampleInRefPatchi [m, n]= weighti[m, n]*RecSamplecurPatchm, n ,

(18)

AccWeightInRefPatchi m, n=weighti[m, n]  ,

(19)

where [m, n] is the position of the to-be-processed sample in one patch, RecSamplecurPatchm, n is the sample value at position [m, n] in current patch, weighti[m, n] is the corresponding weight of to-be-processed samples. The weight, weightim, n, is derived based on the estimated quantization noise and SSD between the current patch and the ith reference patch. In order to estimate the quantization noise, one compression noise model is introduced, and the quantization parameters, prediction type, and standard deviation of the current patch are used in this model. In LowRank-SVD, the filtered patch group is derived by using the following equations,

Yp=UΛV* ,

(20)

FilteredYp=UΛ'V* ,

(21)

where Yp is one 2D matrix formed by one patch group, U and V are unitary matrices generated by performing SVD, Λ is the singular value matrix with non-negative real numbers on the diagonal axis, Λ' is the modified singular value matrix, and FilteredYp is the filtered patch group. Note that in order to reduce the computation complexity of LowRank-SVD, when it is used, without changing the motion search process (each current patch of size 8x8 has 15 best reference patches of size 8x8), one 8x8 patch is divided into four 4x4 subblocks and LowRank-SVD is applied to each subblock set including 16 subblocks from one current patch and 15 reference patches, independently. Therefore, YP is a 16x16 matrix, where each column is a 16x1 vector representing one 4x4 subblock. Λ' is derived by performing hard thresholding on Λ, where the threshold is derived from the estimated quantization noise, and the corresponding weight depends on the number of non-zero ranks in the matrix, Λ'.

The side information of NLMLF includes two parts, parameter selection in noise model and multi-level on/off control flags. The former is used to decide the filtering strength of NLMLF and coded in slice header only; the latter is used to indicate whether NLMLF is enabled or disabled for each current patch and signaled at slice header and CTB level. At slice header, one syntax is signaled to indicate the on/off mode selection among 3 supported modes, {SliceAllOff, SliceAllOn, CTBOnOff}. If SliceAllOff is selected, the NLMLF is disabled for the current component in current slice. If SliceAllOn is selected, the NLMLF is applied for all samples of the current component in the current slice. If CTBOnOff is selected, whether the NLMLF is applied or not depends on the syntax elements signaled at CTB level. At CTB level, there are also three on/off modes, {CTBAllOff, CTBAllOn, BlockOnOff}. If CTBAllOff is selected, the NLMLF is disabled for the current component in the current CTB. If CTBAllOn is selected, the NLMLF is applied for all samples of the current component in the current CTB. If BlockOnOff is selected, whether the NLMLF is applied or not depends on the on/off flags signaled at 32x32 block level.

### Sample adaptive offset

Sample adaptive offset (SAO) is adopted as one in-loop filter in HEVC. The concept of SAO is to classify to-be-processed samples into different groups first, and based on the classification result, one corresponding offset is added on top of a to-be-processed sample to reduce the sample distortion. In HEVC, SAO supports 4 classes in edge offset (EO) and 32 different sets of selected bands in band offset (BO). In this contribution, there are two modifications related to SAO. One is to add 3 EO classes, as shown in Figure 20 (e), (f), and (g), and the other is to remove the constraint related to the sign of EO offsets. The signs of EO offsets are coded to indicate whether the sign of EO offset is positive or negative.

Figure 20. Supported EO classes in SAO.

### Adaptive loop filter

Adaptive loop filter (ALF) was utilized as one in-loop filter during HEVC standardization. The concept is to utilize Wiener filter to reduce sample distortions. In this contribution, ALF is also integrated. The basic decoding process of ALF is similar to that of SAO. First, sample classification is performed for each to-be-processed sample to select a corresponding filter, if necessary. Next, the filtered results are generated by applying filtering with the corresponding filters on to-be-processed samples. The side information of ALF is signaled at slice level and CTB level. At slice header, one flag to indicate whether ALF is enabled or disabled, the selection of sample classifiers, coefficients of slice filters, and the supported CTB modes, are signaled for each color component. For each CTB, one on/off flag and the selected CTB mode with related information are coded. In the following, the details of ALF will be introduced.

For each CTB, one on/off flag is used to indicate whether ALF is applied on the current CTB or not. If ALF is enabled for the current CTB, three CTB modes, NewFilterMode, MergeFiterMode, and SliceFilterMode, can be used. The number of supported CTB modes is dependent on two flags signaled at slice header. One flag is used to indicate whether SliceFilterMode is allowed or not, and the other flag is used to indicate whether NewFilterMode and MergeFiterMode are forbidden or not when SliceFilterMode is allowed. If the selected CTB mode is NewFilterMode, then one new filter is signaled and used for all samples in the current CTB. If the selected CTB mode is MergeFilterMode, then one filter is inherited from a neighboring CTB and used for all samples in the current CTB. If the selected CTB mode is SliceFilterMode, then the slice filters signaled at slice header are used for all samples in the current CTB, and sample classification may be used to select the corresponding filter for each to-be-processed sample in the current CTB. There is only one filter shape but with two kinds of symmetries, as shown in Figure 21. The filter shape is 9x9Cross + 3x3Square with half symmetry or full symmetry. For one filter, 10 or 5 coefficients are required to be signaled according to the filter symmetry. The center coefficient is implicitly derived.

Figure 21. The filter shape in ALF with two kinds of symmetries. (a) 9x9Cross + 3x3Square with half symmetry, 11 coefficients. (b) 9x9Cross + 3x3Square with full symmetry, 6 coefficients.

At the slice header, besides the flag used to indicate whether ALF is enabled or disabled and the two flags used to select the supported CTB modes, additional information related to slice filters is also signaled for each color component as follows. First, the selection of slice filters is signaled. An encoder can signal one new set of slice filters or signal one index to indicate which decoded set of slice filters is used for the current slice. The new set of slice filters will be added into the decoded sets of slice filters after applying ALF. The number of decoded sets of slice filters is limited to 4, and referencing these decoded sets of slice filters is disallowed to cross random access points. That is, these decoded sets of slice filters will be reset when the decoder resets the reference frame buffer. In one set of slice filters, multiple filters with one selected sample classification are supported for luma component. For chroma components, only single filter without sample classification is allowed. In ALF, three kinds of sample classifications, IntensityClassifier, SimilarityClassifier, and RotBAClassifier, are supported. IntensityClassifier is the same as band offset in SAO. The range of sample intensity is divided into 32 groups, and the group index is decided based on the intensity of to-be-processed sample. In SimilarityClassifier, neighboring samples in one 5x5 diamond shape are used to compare with the to-be-processed sample, as shown in Figure 22. The group index of the to-be-processed sample is first reset to zero. If the difference between a neighboring sample and the to-be-processed sample is larger than one pre-defined threshold, then the group index is increased by 1. If the difference between the neighboring sample and the to-be-processed sample is larger than two times of the pre-defined threshold, then additional 1 is added to the group index. Totally, there are 25 groups in SimilarityClassifier. RotBAClassifier is almost the same as the block classification with geometric transform in JEM ALF, except the size of activity window for one 2x2 block is reduced from 6x6 to 4x4. In this classification, there are 25 groups at most. Although the number of groups is up to 25 or 32 in different classifiers, the number of filters in one set of slice filters is limited to 16 at most. That is, the encoder merges successive groups to keep the number of merged groups smaller than or equal to 16.

Figure 22. The classification of SimilarityClassifier.

### Convolution neural network loop filter

Convolution neural network (CNN) is successful in many applications, e.g., face/people/vehicle/object/… detection/recognition, image signal processing including denoising, sharpening, and super-resolution, and so on. In this contribution, CNN is applied as one in-loop filter stage and called convolutional neural network loop filter (CNNLF). The inputs of CNNLF include three kinds of data types, reconstructed samples, prediction samples, and residual samples. The reconstructed samples (Reco.) are ALF output samples, the prediction samples (Pred.) are inter or intra prediction output samples, and the residual samples (Resi.) are inverse quantization and inverse transform output samples. The outputs of CNNLF are the restored reconstructed samples. An 8-layer neural network for luma component and another 8-layer neural network for chroma components are designed, as shown in Figure 23 (a) and (b), respectively, where the numbers in a layer, MxMxNxP, are used to represent HorizontalSpatialFilterSizeInOneChannel x VerticalSpatialFilterSizeInOneChannel x NumberOfInputChannels x NumberOfFiltersInOneLayer. The operation in each layer is filtering with a rectified linear unit (ReLu) right after each filter, except the last layer. For the luma neural network, the 18 input channels are 4 x {Y-Reco., Y-Pred., Y-Resi.}, {U-Reco., U-Pred., U-Resi.}, and {V-Reco., V-Pred., V-Resi.}. The data of 4 x {Y-Reco., Y-Pred., Y-Resi.} are generated by 2x2 subsampling a luma block into 4 different subblocks, and each subblock is treated as one input channel. For example, if one to-be-processed luma block is Width x Height, then 2x2 subsampling is applied to generate 4 subblocks of size Width/2 x Height/2, in Packing for Luma. After finishing the processes of 8 layers, unPackingLuma generates a filtered block with size equal to Width x Height by integrating the 4 subblocks into one block and adding the filtered results on top of Y-Reco. As for chroma componenets, U and V are processed simultaneously. The 36 input channels are 4 x {AvgY-Reco., AvgY-Pred., AvgY-Resi.}, 4 x {U-Reco., U-Pred., U-Resi.}, and 4 x {V-Reco., V-Pred., V-Resi.}, where AvgY-Xxxx. is generated by averaging 4 luma samples first if the YUV format is 420. unPackingChroma generates the two filtered chroma blocks, after finishing the processes of 8 layers.

The parameters of CNNLF are signaled in the picture parameter set (PPS), and multi-level on/off control is applied at slice level and CTB level for each color component. At the slice header, one syntax element is signaled to indicate the mode selection among 3 supported modes, {SliceAllOff, SliceAllOn, CTBOnOff}. If SliceAllOff is selected, the CNNLF is disabled for the current component in the current slice. If SliceAllOn is selected, the CNNLF is applied for all samples of the current component in the current slice. If CTBOnOff is selected, whether the CNNLF is applied or not depends on the syntax elements signaled at CTB level. At CTB level, there are also three on/off modes, {CTBAllOff, CTBAllOn, BlockOnOff}. If CTBAllOff is selected, the CNNLF is disabled for the current component in the current CTB. If CTBAllOn is selected, the CNNLF is applied for all samples in the current CTB for the current component. If BlockOnOff is selected, whether the CNNLF is applied or not depends on the on/off flags signaled at 32x32 block level.

Figure 23. The neural network design of CNNLF. (a) Luma neural network. (b) Chroma neural network.

## Entropy coding

### Multi-parameter CABAC

The JEM CABAC modifications, which include multi-hypothesis probability estimation with context-dependent updating speed and adaptive initialization for context models, are applied. In JEM, a 9-bits*64-columns*512-rows look-up-table (LUT) is used for deriving rangeOne. The LUT is 144x of the rangeLPS LUT in HEVC. To reduce the LUT size, the rangeOne derivation is modified as follows.

P = (P1 + P2) >> 1

(22)

probLPS = (P >= 16384) ? 32767 – P : P

(23)

rangeLPS  = LUT[probLPS>>9][(range>>5)&7]

(24)

rangeOne = (P >= 16384) ? range – rangeLPS : rangeLPS

(25)

rangeZero = (P >= 16384) ? rangeLPS : range – rangeLPS    ,

(26)

where the P1 and P2 are the two probabilities of a context in multi-hypothesis probability model, and the range is the current range in binary arithmetic coder (BAC). An 8-bits*8-columns*32-rows LUT, which is as small as the LUT used in HEVC, is used to derived the rangeOne and rangeZero in the proposed CABAC modification.

# Encoder algorithm description

## GOP structure

Our proposal uses the same hierarchical GOP structure as JEM with configuration parameters specified in [1]. However, the configuration parameter QPoffset is set to 0 and 3 for temporal_id equal to 0 and 2, respectively, for encoding constraint set 1 (CS1).

## Adaptive encoder flow for QT/BT

In this proposal, QT, BT, and TT partitions are tested in an adaptive order. If the size of a current CU is larger than 32 or the current QT depth is not larger than the minimum QT depth of its neighboring blocks, then QT is tested before BT. Otherwise, BT is tested before QT.

In the case that QT is tested before BT, BT and TT tests can be skipped based on the QT result. In the case that BT is tested before QT, QT and TT tests can be skipped based on the BT result. BT is always tested before TT, and the testing order of horizontal TT and vertical TT is decided based on the BT result. In the case that horizontal TT is tested before vertical TT, vertical TT test can be skipped based on the horizontal TT result. Similarly, in the case of vertical TT is tested before horizontal TT, horizontal TT can be skipped based on the vertical TT result.

## One additional encoding pass for CNN training

A two-pass encoding algorithm is utilized to achieve a better coding efficiency for CS1 configuration. In the first pass, the data used in the CNN training process, i.e., the ALF output, the prediction samples, and the residual samples, are generated by the encoder with CNNLF disabled. After deriving CNNLF parameters in the CNN training process, the parameters of CNNLF are coded in the PPS, and the encoder with CNNLF enabled is performed to generate the final bitstream in the second pass. The purpose of the first pass is to generate the data for CNN training only, and the training process takes shorter than 2 hours when a GPU is used. No other information is collected and used in the second-pass encoding. In real applications, the first pass intermediate and final encoding decisions can be stored to significantly reduce the encoding time of the second pass, but this was not implemented in this proposal. For constraint set 2 (CS2) configuration, CNNLF is disabled directly, in order to meet the low-delay constraint.

## Settings in operation point 1 and operation point 2

In this proposal, two operation points, operation point 1 (OP1) and operation point 2 (OP2) are provided for different purposes. OP1 pursues ultra high coding efficiency with longer encoding time than JEM-7.0, while OP2 pursues high coding efficiency with shorter encoding time than JEM-7.0. The differences between the two operation points are merely non-normative encoder-only changes. The OP1 bitstreams were submitted for subjective evaluation. The detailed performance regarding BD-rates and run time percentages for both OP1 and OP2 are shown in the following sections.

# Compression performance

## SDR: Constraint set 1 configuration relative to HM anchor (OP1)

### Class SDR-A

The RD-curves of each sequence in class SDR-A in CS1 relative to HM anchor are shown as follows. The solid-lines are the proposed method, and the dash-lines are the HM anchor.

### Class SDR-B

The RD-curves of each sequence in class SDR-B in CS1 relative to HM anchor are shown as follows. The solid-lines are the proposed method, and the dash-lines are the HM anchor.

### Overall

The BD-rate summary of the proposed method at OP1 in CS1 relative to HM anchor is shown in Table 10.

Table 10. BD-rate summary of the proposed method at OP1 in CS1 relative to HM anchor.

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -39.99% | -36.21% | -34.85% | 2.07 | 1.02 | 1.06 |
| CatRobot1 | -48.11% | -57.27% | -50.55% | 1.95 | 1.07 | 1.49 |
| DaylightRoad2 | -48.08% | -65.92% | -58.63% | 1.38 | 1.53 | 1.25 |
| ParkRunning3 | -49.48% | -22.05% | -38.91% | 1.86 | 0.36 | 0.61 |
| Campfire | -59.04% | -16.42% | -69.66% | 1.96 | 0.54 | 2.55 |
| BQTerrace | -39.91% | -61.48% | -67.02% | 1.24 | 1.38 | 1.51 |
| RitualDance | -34.41% | -39.63% | -40.86% | 1.92 | 1.16 | 1.39 |
| MarketPlace | -36.51% | -47.81% | -33.26% | 1.35 | 0.93 | 0.65 |
| BasketballDrive | -40.49% | -51.91% | -39.08% | 1.52 | 1.21 | 1.17 |
| Cactus | -42.09% | -57.34% | -41.23% | 1.81 | 1.04 | 1.12 |
| Average SDR-A | -48.94% | -39.58% | -50.52% | 1.84 | 0.90 | 1.39 |
| Average SDR-B | -38.68% | -51.63% | -44.29% | 1.57 | 1.14 | 1.17 |
| Average all | -43.81% | -45.61% | -47.41% | 1.71 | 1.02 | 1.28 |
| Enc Time[%] | 1190% |  |  |  |  |  |
| Dec Time[%] | 1302% |  |  |  |  |  |
| Max Delta Rate[%] | 0.00% |  |  |  |  |  |
| Min Delta Rate[%] | -0.70% |  |  |  |  |  |
| Max Delta PSNR (YUV) | 2.64 |  | 1.65 |  | 2.72 |  |
| Min Delta PNSR (YUV) | 0.87 |  | 0.27 |  | 0.22 |  |

## SDR: Constraint set 2 configuration relative to HM anchor (OP1)

### Class SDR-B

The RD-curves of each sequence in class SDR-B in CS2 relative to HM anchor are shown as follows. The solid-lines are the proposed method, and the dash-lines are the HM anchor.

### Overall

The BD-rate summary of the proposed method at OP1 in CS2 relative to HM anchor is shown in Table 11.

Table 11. BD-rate summary of the proposed method at OP1 in CS2 relative to HM anchor

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -33.16% | -48.93% | -60.78% | 1.02 | 0.65 | 0.83 |
| RitualDance | -27.24% | -35.28% | -35.58% | 1.45 | 0.94 | 1.12 |
| MarketPlace | -27.94% | -22.61% | -19.67% | 0.93 | 0.33 | 0.32 |
| BasketballDrive | -32.50% | -46.22% | -42.27% | 1.27 | 1.09 | 1.37 |
| Cactus | -35.52% | -34.64% | -33.05% | 1.49 | 0.55 | 0.80 |
| Average SDR-B | -31.27% | -37.54% | -38.27% | 1.23 | 0.71 | 0.89 |
| Enc Time[%] | 1126% |  |  |  |  |  |
| Dec Time[%] | 894% |  |  |  |  |  |
| Max Delta Rate | 0.00% |  |  |  |  |  |
| Min Delta Rate | -0.51% |  |  |  |  |  |
| Max Delta PSNR (YUV) | 1.76 |  | 1.42 |  | 1.75 |  |
| Min Delta PNSR (YUV) | 0.72 |  | -0.10 |  | -0.13 |  |

## SDR: Constraint set 1 configuration relative to JEM anchor (OP1)

### Class SDR-A

The RD-curves of each sequence in class SDR-A in CS1 relative to JEM anchor are shown as follows. The solid-lines are the proposed method, and the dash-lines are the JEM anchor.

### Class SDR-B

The RD-curves of each sequence in class SDR-B in CS1 relative to JEM anchor are shown as follows. The solid-lines are the proposed method, and the dash-lines are the JEM anchor.

### Overall

The BD-rate summary of the proposed method at OP1 in CS1 relative to JEM anchor is shown in Table 12.

Table 12. BD-rate summary of the proposed method at OP1 in CS1 relative to JEM anchor.

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -10.60% | 10.57% | 16.10% | 0.41 | -0.24 | -0.41 |
| CatRobot1 | -15.12% | -11.07% | -9.49% | 0.41 | 0.12 | 0.20 |
| DaylightRoad2 | -13.06% | -21.75% | -30.11% | 0.25 | 0.30 | 0.49 |
| ParkRunning3 | -27.26% | 4.92% | -14.71% | 0.87 | -0.07 | 0.22 |
| Campfire | -40.18% | 26.84% | -41.49% | 0.96 | -0.75 | 1.03 |
| BQTerrace | -13.42% | -30.13% | -30.27% | 0.33 | 0.56 | 0.48 |
| RitualDance | -9.98% | -4.95% | -3.12% | 0.48 | 0.11 | 0.07 |
| MarketPlace | -10.68% | -6.99% | 7.27% | 0.32 | 0.10 | -0.15 |
| BasketballDrive | -14.06% | -12.29% | 5.12% | 0.41 | 0.21 | -0.14 |
| Cactus | -11.63% | -22.61% | -3.64% | 0.37 | 0.32 | 0.05 |
| Average SDR-A | -21.24% | 1.90% | -15.94% | 0.58 | -0.13 | 0.31 |
| Average SDR-B | -11.95% | -15.39% | -4.93% | 0.38 | 0.26 | 0.06 |
| Average all | -16.60% | -6.75% | -10.43% | 0.48 | 0.07 | 0.18 |
| Enc Time[%] | 152% |  |  |  |  |  |
| Dec Time[%] | 227% |  |  |  |  |  |
| Max Delta Rate[%] |  |  |  |  |  |  |
| Min Delta Rate[%] |  |  |  |  |  |  |
| Max Delta PSNR (YUV) | 1.19 |  | 0.81 |  | 1.11 |  |
| Min Delta PNSR (YUV) | 0.16 |  | -0.85 |  | -0.88 |  |

## SDR: Constraint set 2 configuration relative to JEM anchor (OP1)

### Class SDR-B

The RD-curves of each sequence in class SDR-B in CS2 relative to JEM anchor are shown as follows. The solid-lines are the proposed method, and the dash-lines are the JEM anchor.

### Overall

The BD-rate summary of the proposed method at OP1 in CS2 relative to JEM anchor is shown in Table 13.

Table 13. BD-rate summary of the proposed method at OP1 in CS2 relative to JEM anchor.

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -9.98% | -11.31% | -22.65% | 0.24 | 0.10 | 0.21 |
| RitualDance | -7.23% | -5.82% | -5.60% | 0.34 | 0.11 | 0.14 |
| MarketPlace | -9.32% | 12.61% | 13.28% | 0.28 | -0.22 | -0.26 |
| BasketballDrive | -9.65% | -5.95% | -2.67% | 0.30 | 0.11 | 0.07 |
| Cactus | -10.84% | 0.88% | 0.88% | 0.36 | -0.06 | -0.08 |
| Average SDR-B | -9.41% | -1.92% | -3.35% | 0.30 | 0.01 | 0.02 |
| Enc Time[%] | 131% |  |  |  |  |  |
| Dec Time[%] | 171% |  |  |  |  |  |
| Max Delta Rate |  |  |  |  |  |  |
| Min Delta Rate |  |  |  |  |  |  |
| Max Delta PSNR (YUV) | 0.53 |  | 0.60 |  | 0.73 |  |
| Min Delta PNSR (YUV) | 0.12 |  | -0.75 |  | -0.89 |  |

## SDR: Constraint set 1 configuration relative to HM anchor (OP2)

### Overall

The BD-rate summary of the proposed method at OP2 in CS1 relative to HM anchor is shown in Table 14.

Table 14. BD-rate summary of the proposed method at OP2 in CS1 relative to HM anchor

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -38.57% | -34.92% | -33.44% | 1.98 | 0.97 | 1.00 |
| CatRobot1 | -46.61% | -55.91% | -48.78% | 1.88 | 1.04 | 1.42 |
| DaylightRoad2 | -46.19% | -64.92% | -57.47% | 1.32 | 1.49 | 1.21 |
| ParkRunning3 | -48.18% | -20.53% | -37.73% | 1.79 | 0.33 | 0.58 |
| Campfire | -57.70% | -15.05% | -68.90% | 1.91 | 0.49 | 2.50 |
| BQTerrace | -38.46% | -61.63% | -67.27% | 1.19 | 1.38 | 1.51 |
| RitualDance | -32.95% | -38.73% | -39.98% | 1.82 | 1.12 | 1.34 |
| MarketPlace | -35.16% | -47.18% | -32.40% | 1.28 | 0.92 | 0.64 |
| BasketballDrive | -39.18% | -51.05% | -37.65% | 1.46 | 1.18 | 1.12 |
| Cactus | -40.84% | -56.53% | -40.11% | 1.74 | 1.02 | 1.08 |
| Average SDR-A | -47.45% | -38.27% | -49.26% | 1.77 | 0.87 | 1.34 |
| Average SDR-B | -37.32% | -51.02% | -43.48% | 1.50 | 1.12 | 1.14 |
| Average all | -42.38% | -44.64% | -46.37% | 1.64 | 0.99 | 1.24 |
| Enc Time[%] | 606% |  |  |  |  |  |
| Dec Time[%] | 1330% |  |  |  |  |  |
| Max Delta Rate[%] | 0.00% |  |  |  |  |  |
| Min Delta Rate[%] | -1.05% |  |  |  |  |  |
| Max Delta PSNR (YUV) | 2.54 |  | 1.62 |  | 2.66 |  |
| Min Delta PNSR (YUV) | 0.84 |  | 0.24 |  | 0.18 |  |

## SDR: Constraint set 2 configuration relative to HM anchor (OP2)

### Overall

The BD-rate summary of the proposed method at OP2 in CS2 relative to HM anchor is shown in Table 15.

Table 15. BD-rate summary of the proposed method at OP2 in CS2 relative to HM anchor.

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -31.71% | -48.68% | -59.97% | 0.97 | 0.66 | 0.81 |
| RitualDance | -25.61% | -33.45% | -33.66% | 1.34 | 0.88 | 1.04 |
| MarketPlace | -26.07% | -19.99% | -18.35% | 0.86 | 0.28 | 0.29 |
| BasketballDrive | -30.99% | -44.09% | -40.15% | 1.20 | 1.02 | 1.29 |
| Cactus | -34.07% | -32.86% | -30.90% | 1.41 | 0.51 | 0.74 |
| Average SDR-B | -29.69% | -35.82% | -36.60% | 1.16 | 0.67 | 0.83 |
| Enc Time[%] | 515% |  |  |  |  |  |
| Dec Time[%] | 899% |  |  |  |  |  |
| Max Delta Rate | -0.01% |  |  |  |  |  |
| Min Delta Rate | -0.57% |  |  |  |  |  |
| Max Delta PSNR (YUV) | 1.65 |  | 1.34 |  | 1.69 |  |
| Min Delta PNSR (YUV) | 0.66 |  | -0.13 |  | -0.17 |  |

## SDR: Constraint set 1 configuration relative to JEM anchor (OP2)

### Overall

The BD-rate summary of the proposed method at OP2 in CS1 relative to JEM anchor is shown in Table 16.

Table 16. BD-rate summary of the proposed method at OP2 in CS1 relative to JEM anchor.

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -8.37% | 12.61% | 18.26% | 0.32 | -0.29 | -0.47 |
| CatRobot1 | -12.54% | -8.39% | -6.51% | 0.34 | 0.09 | 0.13 |
| DaylightRoad2 | -9.88% | -19.13% | -28.02% | 0.19 | 0.27 | 0.46 |
| ParkRunning3 | -25.35% | 6.76% | -13.06% | 0.80 | -0.09 | 0.20 |
| Campfire | -38.14% | 28.92% | -39.87% | 0.90 | -0.80 | 0.98 |
| BQTerrace | -11.42% | -30.42% | -30.82% | 0.28 | 0.56 | 0.48 |
| RitualDance | -7.93% | -3.37% | -1.64% | 0.37 | 0.07 | 0.02 |
| MarketPlace | -8.71% | -6.51% | 8.04% | 0.26 | 0.09 | -0.17 |
| BasketballDrive | -12.06% | -10.81% | 7.28% | 0.35 | 0.18 | -0.19 |
| Cactus | -9.59% | -20.98% | -1.92% | 0.30 | 0.30 | 0.01 |
| Average SDR-A | -18.86% | 4.16% | -13.84% | 0.51 | -0.16 | 0.26 |
| Average SDR-B | -9.94% | -14.42% | -3.81% | 0.31 | 0.24 | 0.03 |
| Average all | -14.40% | -5.13% | -8.82% | 0.41 | 0.04 | 0.14 |
| Enc Time[%] | 77% |  |  |  |  |  |
| Dec Time[%] | 232% |  |  |  |  |  |
| Max Delta Rate[%] |  |  |  |  |  |  |
| Min Delta Rate[%] |  |  |  |  |  |  |
| Max Delta PSNR (YUV) | 1.12 |  | 0.79 |  | 1.07 |  |
| Min Delta PNSR (YUV) | 0.13 |  | -0.92 |  | -0.91 |  |

## SDR: Constraint set 2 configuration relative to JEM anchor (OP2)

### Overall

The BD-rate summary of the proposed method at OP2 in CS2 relative to JEM anchor is shown in Table 17.

Table 17. BD-rate summary of the proposed method at OP2 in CS2 relative to JEM anchor.

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -7.87% | -12.21% | -20.99% | 0.18 | 0.10 | 0.19 |
| RitualDance | -5.09% | -3.88% | -3.45% | 0.24 | 0.06 | 0.06 |
| MarketPlace | -6.90% | 16.50% | 15.51% | 0.20 | -0.28 | -0.29 |
| BasketballDrive | -7.52% | -2.85% | -0.09% | 0.23 | 0.04 | -0.01 |
| Cactus | -8.63% | 3.57% | 4.21% | 0.28 | -0.09 | -0.15 |
| Average SDR-B | -7.20% | 0.23% | -0.96% | 0.23 | -0.03 | -0.04 |
| Enc Time[%] | 60% |  |  |  |  |  |
| Dec Time[%] | 172% |  |  |  |  |  |
| Max Delta Rate |  |  |  |  |  |  |
| Min Delta Rate |  |  |  |  |  |  |
| Max Delta PSNR (YUV) | 0.43 |  | 0.53 |  | 0.65 |  |
| Min Delta PNSR (YUV) | 0.05 |  | -0.78 |  | -0.92 |  |

# Decoder complexity analysis

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

As shown in Table 10 and Table 11, the decoding time of the proposed proposal is 1302% in CS1 and 894% in CS2 compared with HM-16.16. As shown in Table 12 and Table 13, the decoding time of the proposed proposal is 227% in CS1 and 171% in CS2 compared with JEM-7.0. The bitstreams of HM-16.16, JEM-7.0, and this proposal are all decoded with YUV output enabled and reference input disabled. Also, the MD5 check sum is also disabled.

The bitstreams of this proposal are generated by using parallel encoding scripts. The bitstreams of all sequence segments are cascaded into one single bitstream. The decoding time is measured by decoding the cascaded bitstream.

## Description of computing platform used to determine decoding times reported in section 5.1

The decoder is compiled in 64-bit Linux environment (version 2.6.32-573.8.1.el6.x86_64) with GCC version 5.2.0. The Intel Xeon E312xx CPU with 3400MHz is used. The DRAM size of each CPU is 10 GB. Single instruction multiple data (SIMD) is enabled in this proposal. The serial attached SCSI (SAS) hard disk drives with redundant array of independent disks 1 (RAID1) configuration are used.

## Memory usage of decoder

For this proposal, the used memory of decoding the largest resolution at the highest target rate point in CS1, i.e., FoodMarket4 at base QP 32, is 871.9MB. The used memory of decoding the largest resolution at the highest target rate point in CS2, i.e., RitualDance at base QP 29, is 272.2MB.

For HM-16.16, the used memory of decoding the largest resolution at the highest target rate point in CS1, i.e., FoodMarket4 at base QP 34, is 982.3MB. The used memory of decoding the largest resolution at the highest target rate point in CS2, i.e., RitualDance at base QP 30, is 218.2MB.

For JEM-7.0, the used memory of decoding the largest resolution at the highest target rate point in CS1, i.e., FoodMarket4 at base QP 32, is 1.23GB. The used memory of decoding the largest resolution at the highest target rate point in CS2, i.e., RitualDance at base QP 29, is 339.1MB.

# Encoder complexity analysis

## Encoding time and measurement methodology

As shown in Table 10 and Table 11, the encoding time of this proposal at OP1 is 1190% in CS1 and 1126% in CS2 compared with HM-16.16. As shown in Table 12 and Table 13, the encoding time of this proposal at OP1 is 152% in CS1 and 131% in CS2 compared with JEM-7.0.

As shown in Table 14 and Table 15, the encoding time of this proposal at OP2 is 606% in CS1 and 515% in CS2 compared with HM-16.16. As shown in Table 16 and Table 17, the encoding time of this proposal at OP2 is 77% in CS1 and 60% in CS2 compared with JEM-7.0.

For JEM-7.0 and this proposal, in CS1, parallel encoding is used to generate the bitstreams. To calculate the encoding time, encoding log files is parsed for each random access point (RAP) segment, and the total encoding time is calculated by adding the encoding times of all RAP segments and then subtracting the encoding times of the first frames of all non-first RAP segments.

## Description of computing platform used to determine encoding times reported in section 6.1

The encoder is compiled in 64-bit Linux environment (version 2.6.32-573.8.1.el6.x86_64) with GCC version 5.2.0. The Intel Xeon E312xx CPU with 3400MHz is used. The DRAM size of each CPU is 10 GB. SIMD is enabled in this proposal. The SAS hard disk drives with RAID1 configuration are used.

## Memory usage of encoder

For this proposal, the used memory of encoding the largest resolution at the highest target rate point in CS1, i.e., FoodMarket4 at base QP 32, is 3.29GB. The used memory of encoding the largest resolution at the highest target rate point in CS2, i.e., RitualDance at base QP 29, is 701.8MB.

For HM-16.16, the used memory of encoding the largest resolution at the highest target rate point in CS1, i.e., FoodMarket4 at base QP 34, is 1.84GB. The used memory of encoding the largest resolution at the highest target rate point in CS2, i.e., RitualDance at base QP 30, is 253.5MB.

For JEM-16.16, the used memory of encoding the largest resolution at the highest target rate point in CS1, i.e., FoodMarket4 at base QP 32, is 5.57GB. The used memory of encoding the largest resolution at the highest target rate point in CS2, i.e., RitualDance at base QP 29, is 903.3MB.

# Software implementation description

The source code of this software is based on JEM with QTBT macro removed (i.e., QTBT is always activated with configurable settings of depths and sizes and cannot be deactivated by a macro). Each coding tool can be enabled or disabled by using the related control flag in the SPS. That is, tool on/off experiments can be easily tested by setting control flags in the SPS. Moreover, double Z-scan addressing in JEM is removed and only raster scan addressing is kept to simplify the operations of address mapping. The memory usage including reference frame buffer, CTU buffer, and data buffer, is also optimized to reduce the memory requirement when encoding one video sequence or decoding one bitstream.

# Closing remarks

MediaTek has submitted all required materials for the CfP. There is no mismatch between our encoder and decoder. Bitrate values are all computed from encoded bitstreams. PSNR values are all computed from decoded video sequences. The coding efficiency of our codec is summarized as follows. When compared to HM-16.16, the proposed video codec achieves 43.81% and 31.27% luma BD-rate savings for constraint set 1 and 2 configurations, respectively. When compared to JEM-7.0, it achieves 16.60% and 9.41% BD-rate savings for constraint set 1 and 2 configurations, respectively. In conclusion, we think that our codec can be a good starting framework for the development of the standard since it already includes many promising techniques up to date.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex 1: Questionnaire

## Codec level questions (mandatory)

- Maximum number of reference frames used by encoder

4 for list 0 and 4 for list 1

- Maximum coding unit (e.g. CTU in HEVC)

256x256 luma samples

- Minimum and maximum transform block size

Luma:

Maximum: 128x128

Minimum: 4x4

Chroma:

Maximum: 128x128

Minimum: 2x2

- Minimum and maximum intra prediction block

Luma:

Maximum: 128x128

Minimum: 4x4

Chroma:

Maximum: 128x128

Minimum: 2x2

- Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length

Luma:

4x4 bi-predicted luma samples with 8-tap interpolation filter

8x8 four-hypothesis luma samples with 8-tap interpolation filter

Chroma:

2x2 bi-predicted chroma samples with 4-tap interpolation filter

4x4 four-hypothesis chroma samples with 4-tap interpolation filter

- List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category)

Template matching mode in PMVR, local illumination compensation, motion candidate reorder, transform syntax reorder

- List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples)

Floating point operations used in LowRank-SVD in NLM

64-bit arithmetic for LIC, BIO, LM chroma, UWP

- List all tools that use non-rectangular (e.g. triangle) partitions

None

- Indicate whether reference frame resampling is used (e.g. the current frame is in 1080p, while the reference frame is in 720p, and vice versa)

No

- Indicate whether luma and chroma use separate partitioning structures

Intra slices use separate partitioning structures.

Inter slices use the same partitioning structures.

- Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes)

Entropy parsing depends on intra prediction mode reconstruction, which is the same as HEVC and requires an intra MPM list derivation.

- If the coded frame resolution is different from the input frame resolution, list the coded frame resolution for those sequences (e.g. 360° video, input is 8K ERP, coded video is in 4K CMP converted from the 8K input)

Same for input and coded frame resolution

| Codec Level Q&A |  | HM | JEM | JVET-J0018 |
| --- | --- | --- | --- | --- |
| 1 | Maximum number of reference frames used by encoder | 4 | 4 | 4 |
| 2 | Maximum coding unit (e.g. CTU in HEVC) | luma 64×64 chroma 32×32 | luma 128×128 chroma 64×64 | luma 256×256 chroma 128×128 |
| 3 | Minimum and maximum transform block size | luma 4×4/32×32 chroma 4×4/16×16 | luma 4×4/128×128 chroma 2×2/64×64 | luma 4×4/128×128 chroma 2×2/128×128 |
| 4 | Minimum and maximum intra prediction block | luma 4×4 chroma 4×4 | luma 4×4 chroma 2×2 | luma 4×4/128×128 chroma 2×2/128×128 |
| 5 | Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length | luma 4×8/8×4 (uni-pred) and 8×8 (bi-pred), 8-tap Chroma 2×4/4×2 (uni-pred) and 4×4 bi-pred, 4-tap | luma 4×4 (uni/bi-pred), 8-tap chroma 2×2 (uni/bi-pred), 4-tap | luma: 8-tap, 4×4 (bi-pred), 8×8 (four-hypothesis) chroma: 4-tap, 2×2 (bi-pred), 4×4 (four-hypothesis) |
| 6 | List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category) | n/a | template matching in PMVD, LIC | template matching in PMVD, LIC, motion candidate reorder, transform syntax reorder |
| 7 | List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples, position unforeseeable) | n/a | 64-bit arithmetic for LIC, BIO and LMC | 64-bit arithmetic for LIC, BIO, LM chroma, and UWP, Floating-point operation in LowRank-SVD |
| 8 | List all the tools that use non-rectangular (e.g. triangle) partitions | n/a | n/a | n/a |
| 9 | Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa) | n/a | n/a | n/a |
| 10 | Indicate whether luma and chroma use separate partitioning structures | no | Yes, in I-slices | Yes, in I-slices |
| 11 | Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes) | Entropy parsing depends on intra prediction mode reconstruction, which requires an intra MPM list derivation | Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation | Entropy parsing depends on intra prediction mode reconstruction, which requires an intra MPM list derivation |

## Tool level questions (optional)

- Partitioning structure

  - Whether the prediction unit partitioning is different from transform unit partitioning

  - Maximum tree depth

- Entropy coding

  - Total number of context models

  - Memory storage size for each context model

  - Range table size if table look up is used for range update

  - Initialization table size

  - Other memory associated with context model adaptation

- Control data derivation process

  - Temporal motion data storage granularity

  - Worst case number of reference frames which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-predictive reference frame are used, count this frame as 2)

- Decoder-side motion refinement

  - Line buffer size in terms of motion vector block lines (assuming horizontal motion vector block size is 4 luma samples)

  - Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement

  - Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size

- Intra prediction

  - Line buffer size in terms of luma and chroma sample lines

  - List all the filters used for intra prediction

  - Indicate whether there are cross-component dependencies in intra prediction reconstruction

- Inter prediction

  - Motion vector accuracy

  - Storage size for a motion vector component

  - List all the interpolation filters used for luma/chroma motion compensation

- Quantization

  - Storage size associated with quantization

- Transforms

- List all the combinations of transform block size and transform type for primary transforms

- List all the combinations of transform block size and transform type for secondary transforms

- List all the non-separable transforms used

- Indicate whether transforms can be fully/partially implemented using direct matrix multiply

- List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output)

- In-loop filtering

- List line buffer size in terms of luma and chroma lines (if any)

- List all the filters used for in-loop filtering

- Minimum processing unit size

- “tool-off” and “tool-on” tests

- List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools

Answers to Question #10

The proposed software is based on JEM with QTBT macro removed (i.e., QTBT is always activated with configurable settings of depths and sizes and cannot be deactivated by a macro). Each coding tool can be enabled or disabled by using the related control flags in the SPS except QTBT. Therefore, in the following single-tool-off and single-tool-on tests, QTBT is not included. Moreover, with limited computing resource, all the tests were run with 1-second short length sequences. CNNLF requires signalling of parameters in the PPS and was only trained for full length instead of short length. Therefore, CNNLF is not included in these tests, either.

The single-tool-off and single-tool-on tests for CS1 are summarized in Table 18, and those for CS2 are summarized in Table 19.

Table 18. Summary of “single-tool-off” and “single-tool-on” tests for CS1

| Tested tool name | “single-tool-off” test results (anchor: our OP1 CfP response without CNNLF) |  |  |  |  | “single-tool-on” test results (anchor: single-tool-off anchor with all the listed tools off) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| TT | 2.53% | 4.35% | 4.61% | 60% | 100% | -2.36% | -4.27% | -4.06% | 160% | 103% |
| MERGE-S | 0.09% | 0.00% | 0.00% | 101% | 98% | 0.03% | -0.11% | 0.18% | 100% | 101% |
| MERGE-T | 0.53% | 0.42% | 0.58% | 102% | 96% | -1.44% | -1.23% | -1.34% | 102% | 141% |
| MERGE-A | 0.72% | 0.44% | 0.57% | 101% | 95% | -1.58% | -1.70% | -1.40% | 100% | 107% |
| AFFINE | 2.07% | 1.52% | 1.69% | 95% | 96% | -3.91% | -2.89% | -2.86% | 120% | 103% |
| PMVR | 2.02% | 2.13% | 2.18% | 89% | 91% | -7.01% | -6.31% | -6.47% | 125% | 201% |
| MAP | 0.72% | 0.63% | 0.64% | 87% | 98% | -1.61% | -1.74% | -1.78% | 132% | 104% |
| AMVR | 1.36% | 1.98% | 2.03% | 94% | 98% | -3.16% | -3.63% | -3.57% | 110% | 100% |
| LIC | 0.53% | 0.03% | 0.04% | 100% | 96% | -0.80% | -0.02% | -0.14% | 101% | 100% |
| GBI | 0.07% | 0.01% | 0.09% | 96% | 98% | -0.23% | -0.40% | -0.11% | 110% | 101% |
| DMVR | 0.68% | 0.45% | 0.55% | 96% | 94% | -3.70% | -3.07% | -3.14% | 116% | 151% |
| BIO | 1.07% | 0.77% | 0.82% | 96% | 86% | -2.98% | -2.38% | -2.35% | 108% | 134% |
| OBMC | 0.84% | 0.96% | 1.08% | 96% | 85% | -0.66% | -1.32% | -1.17% | 103% | 112% |
| MCR | 0.41% | 0.82% | 0.47% | 100% | 98% | -0.72% | -0.63% | -0.80% | 101% | 102% |
| AMT | 2.40% | 1.79% | 1.93% | 77% | 96% | -3.36% | -2.93% | -2.85% | 119% | 100% |
| NSST | 1.48% | 3.32% | 3.29% | 87% | 97% | -2.68% | -4.87% | -4.43% | 117% | 100% |
| TSR | 0.19% | 0.01% | -0.16% | 97% | 96% | -0.16% | -0.41% | 0.05% | 104% | 106% |
| BF | 0.22% | -0.07% | -0.11% | 98% | 98% | -0.20% | -0.15% | 0.57% | 101% | 101% |
| NLMLF | 1.11% | 5.81% | 5.62% | 100% | 61% | -2.07% | -5.12% | -4.76% | 106% | 329% |
| ALF | 1.65% | 3.43% | 3.28% | 100% | 94% | -3.26% | -2.72% | -2.58% | 102% | 124% |

Table 19. Summary of “single-tool-off” and “single-tool-on” tests for CS2

| Tested tool name | “single-tool-off” test results (anchor: our OP1 CfP response without CNNLF) |  |  |  |  | “single-tool-on” test results (anchor: single-tool-off anchor with all the listed tools off) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| TT | 2.36% | 5.17% | 5.16% | 59% | 99% | -2.20% | -5.81% | -6.22% | 166% | 100% |
| MERGE-S | -0.03% | 0.29% | 0.16% | 101% | 103% | -0.02% | 0.12% | -0.06% | 100% | 100% |
| MERGE-T | 0.97% | 1.30% | 1.00% | 102% | 95% | -0.72% | -1.48% | -0.83% | 105% | 130% |
| MERGE-A | 0.62% | 1.19% | 1.24% | 101% | 95% | -1.42% | -1.00% | -0.87% | 101% | 104% |
| AFFINE | 2.18% | 2.11% | 2.22% | 88% | 97% | -3.23% | -2.71% | -2.49% | 144% | 102% |
| PMVR | 0.91% | 1.12% | 1.38% | 89% | 97% | -2.47% | -3.20% | -3.28% | 152% | 148% |
| MAP | 0.87% | 1.27% | 1.51% | 91% | 102% | -1.04% | -2.96% | -3.22% | 135% | 106% |
| AMVR | 0.78% | 1.31% | 1.37% | 90% | 101% | -1.61% | -2.38% | -2.41% | 132% | 99% |
| LIC | 0.93% | 0.23% | 0.39% | 101% | 98% | -1.34% | -0.69% | -0.32% | 101% | 100% |
| GBI | 0.07% | -0.32% | 0.51% | 92% | 99% | -0.19% | -0.26% | -0.25% | 125% | 101% |
| OBMC | 0.96% | 1.09% | 1.28% | 97% | 87% | -0.82% | -1.33% | -1.39% | 105% | 112% |
| MCR | 0.29% | 0.55% | 0.14% | 100% | 98% | -0.38% | -0.57% | -0.33% | 103% | 103% |
| AMT | 2.09% | 0.01% | 0.24% | 76% | 98% | -2.67% | -1.63% | -1.44% | 126% | 100% |
| NSST | 1.17% | 3.55% | 4.02% | 89% | 100% | -2.06% | -4.78% | -4.27% | 125% | 101% |
| TSR | 0.28% | 0.27% | 0.25% | 97% | 97% | -0.25% | 0.31% | 0.33% | 104% | 103% |
| BF | 0.16% | -0.26% | 0.21% | 99% | 99% | -0.16% | -0.07% | 0.13% | 102% | 101% |
| NLMLF | 1.30% | 9.73% | 9.37% | 100% | 49% | -2.23% | -6.83% | -7.02% | 111% | 379% |
| ALF | 2.32% | 9.58% | 6.62% | 101% | 94% | -3.03% | -4.07% | -3.75% | 102% | 124% |

Preliminary results of QTBT can be found in ITU-T SG16 Document COM16-C966, JVET-B0023, and JVET-C0024. A preliminary QTBT implementation reportedly achieved about 6% luma BD-rate saving and 15% chroma BD-rate saving for both random access (RA) condition and low delay B (LB) condition with 155%-160% encoding time and 105%-110% decoding time when compared to HM-13.0.

Our internal experiments show that enabling CNNLF can achieve 7.2% YUV BD-rate saving for CS1 when the anchor is our OP1 CfP response with CNNLF disabled. The encoding time is about 100% (<101% excluding the training pass; training needs shorter than 2 hours on a GPU), and the decoding time without using any GPUs is 150%. Further complexity reduction on the proposed CNNLF is being researched. It is assumed that in the years including and after 2020 most devices (personal computers, tablets, smartphones, BD players, TVs …) will have GPUs, so CNNLF decoding processes can be easily accelerated by 100x, as an example shows in JVET-I0022 that gives 196x speed-up (12787% => 65%). Therefore, it is suggested that CNNLF is a promising direction to go for the new standard. Besides, it is suggested to standardize predefined CNNLF parameter sets in addition to allowing signaled CNNLF parameter sets. Real-time encoders at the device end can select a predefined CNNLF parameter set and signal an index of the selected set (perhaps at slice level) to meet low complexity or low delay requirements, while multi-pass encoders at the cloud end can train segments of video sequences and signal CNNLF parameters of each segment in the bitstream to achieve higher compression.

In the NLMLF of our CfP response submission, two denoising methods, NLM and LowRank-SVD, can be applied. A simplified NLMLF has been developed. In the simplified NLMLF, only the NLM is applied, and the LowRank-SVD is disabled to reduce about 1/3 of the NLMLF runtime at the decoder side. When the anchor is our OP1 CfP response without CNNLF, the simplified NLMLF results in less than 0.2% BD-rate increases for both CS1 and CS2, 12% decoding time reduction for CS1, and 16% decoding time reduction for CS2. Further complexity reduction on the simplified NLMLF is being researched. The BD-rate difference between enabling LowRank-SVD and disabling LowRank-SVD was 0.6% on an early software with few coding tools integrated, but now the BD-rate difference is less than 0.2% on the current software with many coding tools integrated. Therefore, it is suggested that the simplified NLMLF is a better choice for the new standard than the NLMLF in our CfP response submission.

# Reference

[1] “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T VCEG (Q6/16) and ISO/IEC MPEG (JTC 1/SC 29/WG 11), 8th Meeting, Macao, China, October 2017, Doc. JVET-H1010.

[end of document]