| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0019-v2 |
| --- | --- |

| Title: | Description of 360° video coding technology proposal by MediaTek |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin ,Ya-Hsuan Lee, Cheng-Hsuan Shih, Sheng-Yen Lin, Hung-Chih Lin, Shen-Kai Chang, Peng Wang, Lin Liu, and Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {jl.lin, louise.lee, CH.Shih, Sheng-Yen.Lin, hungchih.lin, shenkai.chang, ben.wang, Lin.Liu, cc.ju} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution describes MediaTek’s proposal, in response to the joint call for proposals (CfP) issued jointly by VCEG and MPEG, for the 360° video category. This contribution includes a Modified Cubemap Projection (MCP) and 360° specific coding tools. The proposed MCP is arranged into a compact 3x2 layout, which has one discontinuous edge and none of padding pixels. To address the geometric continuity in 360° video, the 360° specific coding tools are proposed to appropriately process data in inter prediction, intra prediction, and in-loop filters.

The default face resolution in MCP is set to 1184x1184 to match the number of coded samples in the anchors. Compared to the HM anchor, the experimental results reportedly show this contribution achieves the average 35.5%, 69.5%, 71.6%, and 44.3% BD-rate reduction in terms of end-to-end WS-PSNR-Y, WS-PSNR-U, WS-PSNR-V, and WS-PSNR-YUV, respectively. As compared to the JEM anchor, the results report the average 15.8%, 50.7%, 52.6%, and 24.8% BD-rate reduction in terms of end-to-end WS-PSNR-Y, WS-PSNR-U, WS-PSNR-V, and WS-PSNR-YUV, respectively. In addition, the face with a resolution set to 1280x1280, which is a multiple of a LCU size, is also tested. Compare to the HM anchors, the results report the average -36.2%, -69.4%, -71.7%, and -44.8% BD-rate reduction in terms of end-to-end WS-PSNR-Y, WS-PSNR-U, WS-PSNR-V, and WS-PSNR-YUV, respectively. As compared to the JEM anchor, the results report the average -16.8%, -51.3%, 53.4%, and -25.7% BD-rate reduction in terms of end-to-end WS-PSNR-Y, WS-PSNR-U, WS-PSNR-V, and WS-PSNR-YUV, respectively.

- Contents

Abstract	i

Contents	ii

1	Introduction	1

2	Decoder algorithm description	1

2.1	General algorithm description	1

2.2	360° Video	1

2.2.1	Modified Cubemap Projection format	1

2.2.2	Quantization handling	4

2.2.3	360° video specific decoding tools	4

3	Encoder algorithm description	12

3.1	General algorithm description	12

3.2	360°	12

3.2.1	Usage of projection format adaptation	12

3.2.2	360° Video specific encoding tools	12

3.2.3	Additional encoder optimization	13

4	Compression performance	13

4.1	360° Video: Constraint set 1 configuration relative to HM anchor	13

4.1.1	Class 360	14

4.2	360° Video: Constraint set 1 configuration relative to JEM anchor	14

4.2.1	Class 360	14

5	Decoder complexity analysis	18

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	18

5.2	Description of computing platform used to determine decoding times reported in section 5.1	20

5.3	Memory usage of decoder	20

5.3.1	Memory usage of HM and JEM decoders	20

5.3.2	Memory usage of decoder in proposed implementation	21

5.4	Complexity characteristics of 360° video specific decoding tools	21

5.4.1	CU partition	21

5.4.2	Inter prediction	22

5.4.3	Intra prediction	22

5.4.4	360°-based In-loop filtering	22

6	Encoder complexity analysis	22

6.1	Encoding time and measurement methodology	22

6.2	Description of computing platform used to determine encoding times reported in section 6.1	23

6.3	Memory usage of encoder	23

6.3.1	Memory usage of HM and JEM anchors	23

6.3.2	Memory usage of proposed encoder	24

6.4	Complexity characteristics of 360° video specific encoding tools	24

6.4.1	CU partition	24

6.4.2	360°-based In-loop filtering	24

7	Software implementation description	24

8	Closing remarks	24

9	Acknowledgement	24

10	Patent rights declaration(s)	25

11	References	25

12	Annex: Questionnaire	25

12.1	Codec level questions	25

12.2	Tool level questions	27

Introduction

This contribution describes the video encoder and decoder proposed by MediaTek in response to the “Joint Call for Proposals on Video Compression with Capability beyond HEVC” [1] issued jointly by ITU-T SG16 Q6 (VCEG) and ISO/IEC JTC 1/SC 29/WG11 (MPEG) for the 360° video category. All the coded test materials required in the 360º video category have been submitted.

The proposed 360º video coding system includes a Modified Cubemap Projection (MCP) and encoder/decoder specific coding tools for 360° video, integrated into the 360Lib-5.0 [2] and JVET-J0018 [3]. The proposed mapping functions applied in the MCP have highly efficient representation for 360° video. In the face arrangement, the MCP adopts the compact 3x2 layout, which has one discontinuous edge and none of padding pixels.

Geometric continuity is one of the major properties of 360° video because the whole surrounding environment is represented to create an immersive experience. To accomplish the geometric continuity, the proposed algorithm carries out proper modifications on inter prediction, intra prediction, and in-loop filters. In inter and intra prediction, the reference data are replaced with those obtained by the geometry padding when the referenced data is outside layout arrangement. Moreover, the referenced motion vector outside layout arrangement is also adjusted according to the projection mapping. For the in-loop filters, the layout boundary and the discontinuous edge are appropriately filtered as well. With these modified implementations, the proposed contribution noticeably enhances compression efficiency and eliminates seam artifacts.

Decoder algorithm description

General algorithm description

The proposed 360º video coding system is integrated into JVET-J0018 [3] and the general algorithm description could be referred to JVET-J0018, except the De-blocking filter and Quantization. The general algorithms for de-blocking filter and quantization parameter (QP) mapping table are identical to HEVC.

360° Video

Modified Cubemap Projection format

In this contribution, a Modified Cubemap Projection (MCP) which consists of six square faces is proposed. These six faces are arranged into a compact rectangular layout without any padding pixels as shown in Fig. 1(a). Fig. 1(b) illustrates each face defined by a two-dimensional u,v plane [4], where u and v are in the range of [-1, 1] and the face definition of the six faces is listed in Table I.

| (a) | (b) |
| --- | --- |

Fig. 1 (a) Compact MCP layout (b) Coordinate of MCP [4]

Table I. Face index definition [4]

| Face index | Face label | Description |
| --- | --- | --- |
| 0 | PX | Front face with positive X axis value |
| 1 | NX | Back face with negative X axis value |
| 2 | PY | Top face with positive Y axis value |
| 3 | NY | Bottom face with negative Y axis value |
| 4 | PZ | Right face with positive Z axis value |
| 5 | NZ | Left face with negative Z axis value |

The conventional cube map projection (CMP) [5] has two major drawbacks that results in inefficient compression performance, which mainly due to the inefficient sampling rate inside a face and turning points appeared when a straight line crosses face boundaries as the example image shown in Fig. 2. To improve the compression performance, the equal-angular cube (EAC) projection applies an equal-angular mapping function [6][7] on u-axis and v-axis, separately, to achieve uniform sampling distribution for each axis as follows:

|  | 2D-to-3D: | fu=tan⁡(π4u)fv=tan⁡(π4v) | (1) |
| --- | --- | --- | --- |
|  | 3D-to-2D: | gu=4πtan-1(u)gv=4πtan-1(v) | (2) |

However, the turning points still appeared on the face boundaries in the EAC projection as shown in Fig. 3.

Fig. 2 An image of CMP in 3x2 layout

Fig. 3 An image of EAC in 3x2 layout

To overcome these drawbacks, the proposed MCP thus introduces a coordinate mapping function used in the 2D-to-3D and 3D-to-2D conversions to improve the EAC projection and reduce the distortion across faces. For the Front, Back, Right and Left faces, the proposed MCP applies the proposed mapping function fv on v-axis (corresponding to the latitude) to replace the equal-angular mapping function in EAC. The proposed mapping function fv is defined as

|  | 2D-to-3D: | fv=v1+0.4(1-u2)(1-v2) | (3) |
| --- | --- | --- | --- |
|  | 3D-to-2D: | gv=v , if t=01-1-4t(v-t)2t, otherwise where t=0.4vgu2-1 | (4) |

, wWamee fined as f istortion across facesosed mapping fucntion places the equalhile the Top and Bottom faces apply the equal-angular mapping on both u-axis and v-axis to keep the continuity with other faces. Fig. 4 provides an example of MCP image, which consists of two 3x1 face regions with one discontinuous edge.

Fig. 4 An image of MCP in 3x2 layout

Quantization handling

No 360° video specific quantization handling is applied in this contribution.

360° video specific decoding tools

Geometric continuity is one major property of 360° video. Based on this property, any coding process in conventional coding system that requires the reference information across the frame boundary (layout boundary) or the discontinuous edge may need to be appropriately changed to improve the coding efficiency. This section explains the modifications in the proposed design.

CU partition

According to the layout arrangement in Fig. 4, the proposed implementation determines the location of the discontinuous edge. With this information, a CTU could optimally select a partition structure to avoid a CU containing the discontinuous edge. Moreover, the syntax bits that represent automatic CU split for the discontinuous edge could be reduced because such a CU partition can be inferred by the decoder. Fig. 5 illustrates the two cases of automatic CU split for the discontinuous edge.

- Case 1: For a CU with the horizontal discontinuous edge inside as shown in Fig. 5(a), only the quadtree (QT) partition or the horizontal binary tree (BT) partition is allowed for further split. In this example, the flag of BT is not necessarily coded because only one BT partition is allowed.

- Case 2: For a CU with the horizontal discontinuous edge and a vertical frame boundary (layout boundary) as shown in Fig. 5(b), only the QT partition is allowed for further split. In this example, the splitting flag is not necessarily coded because only QT partition is allowed.

(a)                                  (b)

Fig. 5. Proposed partition rule for CU split (a) CU containing the horizontal discontinuous edge (b) CU containing the horizontal discontinuous edge and a vertical frame boundary

Inter prediction

Geometry padding

The inter prediction in this proposal replaces the replication padding with the geometry padding when the reference pixels are outside the frame boundary (layout boundary) or across the discontinuous edge. Based on the property of geometric continuity, these reference pixels can be determined from other faces. The geometry padding in Fig. 6 demonstrates a pixel position qb outside the Bottom face is represented by its geometric pixel position qa, which is located at the Front face. This geometric pixel position is obtained according to the proposed MCP mapping functions as described in Section 2.2.1. When the derived coordinate qa is fractional, the 6-tap and 4-tap Lanczos interpolation filters [8] are applied for the luminance and chrominance components, respectively.

Fig. 6 Geometry padding process

To achieve the geometry padding in inter prediction, the proposed implementation requires two sub-frame buffers, the top sub-frame and the bottom sub-frame, for each reference frame data as illustrated in Fig. 7. Each sub-frame buffer has one 3x1 face region with an extended padding area. The pixel data in the padding area are obtained by the geometry padding process as described above. Note that the padding size is the LCU size with additional 16 pixels, which yields the width of the padding area is 272 pixels.

Fig. 7 Top and bottom sub-frame buffers: 3x1 face region with geometry padding area

Motion vector projection

In MV coding, the motion vectors from the neighboring CUs are used as the potential candidates during the derivation of the motion vector predictor. However, when a CU is located beside the frame boundary (layout boundary) or the discontinuous edge, its neighboring CUs in the proposed MCP layout should be replaced by its geometric neighboring CUs, located at other faces. As the example shown in Fig. 8, similar with the geometry padding process, a geometric neighboring CU located at the Front face can be found according to the packing layout in MCP. Besides, the motion vector v of this geometric neighboring CU is also adjusted accordingly by projecting the motion vector onto the extended Bottom face at which the current CU is located. This projected motion vector v’ is then regarded as a candidate of the motion vector predictor for the current CU. Note that the projected motion vector may be unavailable because this geometric neighboring CU may not been coded yet.

Fig. 8 Geometric neighboring CU and motion projection

Intra prediction

The intra prediction constructs efficient predictors by using the neighboring reconstructed pixels of the current CU. In the proposed implementation, when a CU is located beside the frame boundary (layout boundary) or the discontinuous edge, its neighboring pixels are replaced by the geometric neighboring reconstructed pixels, which are located at other faces. As the example shown in Fig. 9, the neighboring pixels of the current CU are replaced by the top and left geometric neighboring pixels which are located at the Left face and Front face, respectively. Note that the geometric neighboring reference pixels are unavailable when these pixels have not been coded yet.

Fig. 9 Geometric neighboring pixels

Neighboring CU availability

The intra MPM set is derived from the intra prediction modes of the neighboring CUs. In this proposal, when the neighboring CUs are located outside the frame boundary (layout boundary) or across the discontinuous edge, the coding information of those neighboring CU is considered as unavailable to avoid inappropriate reference. Fig. 10 demonstrates that the coding information in the top-right CU is considered as unavailable because the referenced coding information in this location is spatially irrelevant. In the proposed implementation, the neighboring CU’s availability check is also applied to all the other tools when the reference information of the neighboring CUs are located outside boundary (layout boundary) or across the discontinuous edge, except the motion vector prediction. For motion vector prediction, the geometric neighboring CU derivation is applied as described in Section 2.2.3.2.2.

Fig. 10 Neighboring CU availability

360°-based In-loop filtering

Based on the concept of the geometric continuity, the in-loop filtering processes should be modified accordingly when the accessed pixels (to be referenced and/or filtered) are outside the frame boundary (layout boundary) or across the discontinuous edge. In this case, the proposed modification makes use of the pixels data located at the geometric positions for filtering so that the frame boundary (layout boundary) or the discontinuous edge can be properly processed (filtered). As shown in Fig. 11 to Fig. 13, this concept implemented in the in-loop filters significantly reduces the seam artifacts to improve subjective quality. In this contribution, the modified in-loop filters include the de-blocking (DB) filter, sample adaptive offset (SAO) filter, adaptive loop filter (ALF), and the non-local means (NLM) and convolutional neural network (CNN) filters proposed in JVET-J0018 [3].

| (a) |  | (b) |
| --- | --- | --- |

Fig. 11 Subjective quality cropped from the viewport centered in degree (135, 35), frame 29 of Balboa  (a) Use neighboring pixels for filtering (b) Use geometric pixel positions for filtering

| (a) | (b) |  |
| --- | --- | --- |

Fig. 12 Subjective quality cropped from the viewport centered in degree (-45, -35), frame 211 of ChairliftRide (a) Use neighboring pixels for filtering (b) Use geometric pixel positions for filtering

| (a) | (b) |
| --- | --- |

Fig. 13 Subjective quality cropped from the viewport center in degree (135, 35), frame 171 of Trolley  (a) Use neighboring pixels for filtering (b) Use geometric pixel positions for filtering

360°-based De-blocking Filter

The conventional implementation of the de-blocking filter is used to reduce blocky artifacts appeared at CU boundaries inside frame. For an MCP frame, Fig. 14 illustrates that the modified implementation in the de-blocking filtering fetches the geometric pixel positions located at another face for edge filtering when the filtered pixels cross the frame boundary (layout boundary) or the discontinuous edge. The pixel data in the geometric positions are copied directly from the corresponding face. After the pixel data are collected for edge filtering, the motion vector located at the geometric pixel positions is also adjusted by the MV projection (described in Section 2.2.3.2.2) for appropriate boundary strength calculation.

Fig. 14 Geometric pixel positions from another face for 360°-based DB filter

360°-based Sample Adaptive Offset Filter

In the SAO filtering process, each pixel is adjusted by referencing its neighboring pixels. In Fig. 15, when the referenced neighboring pixels cross the discontinuous edge, the proposed implementation uses the geometric pixel positions located at another face for SAO filtering. Moreover, the pixel data in the geometric positions are copied directly from the corresponding face.

Fig. 15 Geometric pixel positions from another face for 360°-based SAO filter

360°-based Adaptive Loop Filter

In the conventional ALF, the replication padding is used when the accessed pixels for filtering are located outside the frame boundary (layout boundary). In Fig. 16, when the referenced pixels are across the frame boundary (layout boundary) or the discontinuous edge, the proposed implementation in the ALF replaces the replication padding with the geometric padding process (as described in 2.2.3.2.1), where the pixel data are derived from another face.

Fig. 16 Geometric padding used for 360°-based ALF

360°-based Non-Local Means Filter

The NLM filter is an efficient de-noising algorithm collects multiple similar group of pixels (one group is called a patch) within a pre-defined search range to reduce the coding error of the current processing region. In the NLM filtering process implemented in [3], any of the collected patches outside the frame boundary is not allowed. However, this restriction in patch collection is not necessarily followed for 360° video. In the proposed implementation, the accessed pixels across the frame boundary (layout boundary) or the discontinuous edge are obtained directly from other faces, as shown in Fig. 17, where the search range used in the proposed implementation is [-16, 16].

Fig. 17 Geometric pixel positions from another face for 360°-based NLM filter

360°-based Convolutional Neural Network Filter

The processing unit of the CNN filter is one LCU in JVET-J0018 [3]. However, due to the layout arrangement, certain LCUs in the MCP frame contain the discontinuous edge. The pixel data in such an LCU should not be jointly processed in the CNN filtering. To address this problem, the proposed implementation in this contribution divides such an LCU into two small processing units that are separately processed by the CNN filter as shown in Fig. 18. Fig. 19 demonstrates the predicted residual data provided by the CNN filtering process are used for improvement in coding efficiency, where the trained weights are explicitly transmitted in the bit-stream.

Fig. 18 Two divided processing units for an LCU containing the discontinuous edge

Fig. 19 360°-based CNN filter process for reconstructed frame

Encoder algorithm description

General algorithm description

The proposed 360º video coding system is integrated into JVET-J0018 [3] and the general algorithm description could be referred to JVET-J0018, except the De-blocking filter and Quantization. The general algorithms for de-blocking filter and quantization parameter (QP) mapping table are identical to HEVC.

360°

Usage of projection format adaptation

The projection format adaptation is not used in this contribution. The projection format is MCP with a fixed parameter.

360° Video specific encoding tools

Most of the proposed coding tools are the same in encoder and decoder, except for the ALF and CNN filters. The encoding process in the ALF filter estimates the filter coefficients while the weights used in the CNN filter is pre-trained. Moreover, the necessary information, including the filter coefficients and weights, required by the ALF and CNN filters is transmitted by the proposed encoder via bit-stream.

360°-based Adaptive Loop Filter

For encoder, the filter coefficients in the ALF need to be determined by the pixel classification methods and the Wiener filter approach. During the determination of the filter coefficients, the neighboring pixels that cross the frame boundary (layout boundary) or the discontinuous edge are changed to use the pixels provided by the geometry padding process (as described in Section 2.2.3.2.1).

After the filter coefficients are determined, the filtering process in the ALF is the same as that described in Section 2.2.3.5.3. Moreover, these coefficients are encoded in bit-stream so that the ALF in decoder can perform the same reconstruction process as that in encoder.

360°-based Convolutional Neural Network Filter

The original YUV frames, the reconstructed YUV frames, the reconstructed residual frames, and the prediction frames are the training materials used for the weighting parameter determination in the CNN filter. This offline training process is illustrated in Fig. 20. Each frame in the training materials is divided into two 3x1 face regions since there is a discontinuous edge in the MCP.

After the weighting parameters are determined, the filtering process in the CNN is the same as that described in Section 2.2.3.5.5. Moreover, these weights are encoded in bit-stream so that the decoder can perform the same reconstruction process as that in encoder.

Fig. 20 Offline training process for 360°-based CNN filter

Additional encoder optimization

No additional encoder optimization is used in this proposed contribution.

Compression performance

360° Video: Constraint set 1 configuration relative to HM anchor

The simulation results of the proposed implementation, integrated into 360Lib-5.0 [2] and JVET-J0018 [3], are conducted followed by the constraint set 1 [1]. Each face resolution in MCP is 1184x1184, which yields the frame resolution of MCP is 3552x2368 i.e., the coded samples of the MCP is about 99.88% of the anchor PERP (4112x2048). The coding rates after bit-rate matching and the Qp information are detailed in Table II.

Table II. The quantization settings for each sequence in the four rate points

| Sequence | Target bit rate (kbps) | Matched bit rate (kbps) | baseQP | QPIncrementFrame | Rate Diff |
| --- | --- | --- | --- | --- | --- |
| Balboa | 10000 | 9993.1768 | 28 | 184 | -0.1% |
|  | 6000 | 5996.3560 | 31 | 104 | -0.1% |
|  | 3300 | 3299.3480 | 35 | 196 | 0.0% |
|  | 2000 | 1999.9832 | 38 | 19 | 0.0% |
| Chairlift | 7000 | 6999.5864 | 26 | 224 | 0.0% |
|  | 4000 | 3999.5616 | 29 | 34 | 0.0% |
|  | 2400 | 2394.6608 | 33 | 272 | -0.2% |
|  | 1500 | 1496.5184 | 35 | 16 | -0.2% |
| KiteFlite | 7000 | 6998.8528 | 30 | 276 | 0.0% |
|  | 4000 | 3994.8288 | 34 | 192 | -0.1% |
|  | 2400 | 2385.7440 | 38 | 288 | -0.6% |
|  | 1200 | 1197.7376 | 42 | 96 | -0.2% |
| Harbor | 3300 | 3298.2264 | 28 | 60 | -0.1% |
|  | 2000 | 1999.0152 | 32 | 232 | 0.0% |
|  | 1200 | 1193.1880 | 35 | 96 | -0.6% |
|  | 700 | 699.0224 | 39 | 144 | -0.1% |
| Trolley | 7000 | 6999.7352 | 28 | 107 | 0.0% |
|  | 4000 | 3988.5544 | 33 | 224 | -0.3% |
|  | 2400 | 2395.6096 | 37 | 288 | -0.2% |
|  | 1500 | 1499.9032 | 40 | 170 | 0.0% |

Class 360

The experimental results in Table III demonstrate that, as compared to the HM anchors [9], the average BD-rate reduction provided by this contribution achieves 35.5%, 69.5%, 71.6%, and 44.3% in terms of end-to-end WS-PSNR-Y, WS-PSNR-U, WS-PSNR-V, and WS-PSNR-YUV, respectively.

In addition, the face with the resolution of 1280x1280, which is a multiple of a LCU size, is also tested, and the results are reported in Table IV. Compared to the HM anchors, the average BD-rate reduction achieves -36.2%, -69.4%, -71.7%, and -44.8% BD-rate reduction in terms of end-to-end WS-PSNR-Y, WS-PSNR-U, WS-PSNR-V, and WS-PSNR-YUV, respectively.

Table III. BD-rate performance compared to the HM anchors

|  | E2E WS-PSNR |  |  |  | E2E SPSNR-NN |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Balboa | -46.6% | -73.2% | -77.0% | -53.7% | -46.6% | -73.2% | -77.0% | -53.7% |
| Chairlift | -49.3% | -73.1% | -69.3% | -54.8% | -49.3% | -73.1% | -69.3% | -54.7% |
| KiteFlite | -26.8% | -71.1% | -74.4% | -38.3% | -26.7% | -71.1% | -74.3% | -38.2% |
| Harbor | -30.3% | -68.7% | -70.2% | -40.1% | -30.0% | -68.8% | -70.2% | -39.9% |
| Trolley | -24.6% | -61.4% | -67.3% | -34.5% | -24.5% | -61.5% | -67.3% | -34.4% |
| Average | -35.5% | -69.5% | -71.6% | -44.3% | -35.4% | -69.5% | -71.6% | -44.2% |

Table IV. BD-rate performance with the face resolution of 1280x1280 compared to the HM anchors

|  | E2E WS-PSNR |  |  |  | E2E SPSNR-NN |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Balboa | -47.0% | -74.1% | -78.2% | -54.3% | -47.0% | -74.1% | -78.2% | -54.3% |
| Chairlift | -50.1% | -73.7% | -69.6% | -55.5% | -50.0% | -73.7% | -69.6% | -55.4% |
| KiteFlite | -27.2% | -70.9% | -74.3% | -38.6% | -27.2% | -70.9% | -74.2% | -38.5% |
| Harbor | -31.3% | -68.2% | -69.5% | -40.7% | -31.0% | -68.2% | -69.5% | -40.5% |
| Trolley | -25.4% | -60.2% | -66.7% | -34.9% | -25.4% | -60.3% | -66.7% | -34.9% |
| Average | -36.2% | -69.4% | -71.7% | -44.8% | -36.1% | -69.4% | -71.6% | -44.7% |

360° Video: Constraint set 1 configuration relative to JEM anchor

Class 360

As compared to the JEM anchors [10], the experimental data in Table V report that the average BD-rate reduction provided by this contribution achieves 15.8%, 50.7%, 52.6%, and 24.8% in terms of end-to-end WS-PSNR-Y, WS-PSNR-U, WS-PSNR-V, and WS-PSNR-YUV, respectively. The RD-curves of each sequence regarding to the end-to-end WS-PSNR are also shown in Fig. 21, Fig. 22, Fig. 23, Fig. 24 and Fig. 25 to compare the performance of the proposed contribution with the HM and JEM anchors.

In addition, the results of the experiments with the face resolution set to 1280x1280 are also reported in Table VI, where the average BD-rate reduction achieves -16.8%, -51.3%, 53.4%, and -25.7% in terms of end-to-end WS-PSNR-Y, WS-PSNR-U, WS-PSNR-V, and WS-PSNR-YUV, respectively, as compared to the JEM anchors.

Table V. BD-rate performance compared to the JEM anchors

|  | E2E WS-PSNR |  |  |  | E2E SPSNR-NN |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Balboa | -17.8% | -61.1% | -63.3% | -28.9% | -17.7% | -61.1% | -63.3% | -28.9% |
| Chairlift | -27.1% | -51.7% | -49.0% | -32.9% | -27.0% | -51.7% | -49.0% | -32.9% |
| KiteFlite | -11.3% | -49.2% | -50.1% | -20.9% | -11.2% | -49.1% | -50.0% | -20.8% |
| Harbor | -12.6% | -46.5% | -49.4% | -21.4% | -12.4% | -46.6% | -49.5% | -21.3% |
| Trolley | -10.4% | -44.8% | -51.2% | -19.8% | -10.2% | -44.8% | -51.2% | -19.7% |
| Average | -15.8% | -50.7% | -52.6% | -24.8% | -15.7% | -50.7% | -52.6% | -24.7% |

Table VI. BD-rate performance with the face resolution of 1280x1280 compared to the JEM anchors

|  | E2E WS-PSNR |  |  |  | E2E SPSNR-NN |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Balboa | -18.6% | -62.7% | -65.4% | -29.9% | -18.5% | -62.7% | -65.5% | -29.9% |
| Chairlift | -28.2% | -54.1% | -50.6% | -34.2% | -28.1% | -54.1% | -50.6% | -34.2% |
| KiteFlite | -11.9% | -49.6% | -50.8% | -21.5% | -11.8% | -49.4% | -50.7% | -21.4% |
| Harbor | -13.9% | -46.7% | -49.2% | -22.4% | -13.6% | -46.8% | -49.3% | -22.2% |
| Trolley | -11.3% | -43.3% | -50.7% | -20.2% | -11.3% | -43.2% | -50.7% | -20.2% |
| Average | -16.8% | -51.3% | -53.4% | -25.7% | -16.7% | -51.3% | -53.3% | -25.6% |

(a)

(b)                                          (c)

Fig. 21 RD-curves for sequence Balboa (a) End-to-end WS-PSNR Y (b) End-to-end WS-PSNR U (c) End-to-end WS-PSNR V

(a)

(b)                                          (c)

Fig. 22 RD-curves for sequence ChairliftRide (a) End-to-end WS-PSNR Y (b) End-to-end WS-PSNR U (c) End-to-end WS-PSNR V

(a)

(b)                                          (c)

Fig. 23 RD-curves for sequence KiteFlite (a) End-to-end WS-PSNR Y (b) End-to-end WS-PSNR U (c) End-to-end WS-PSNR V

(a)

(b)                                          (c)

Fig. 24 RD-curves for sequence Harbor (a) End-to-end WS-PSNR Y (b) End-to-end WS-PSNR U (c) End-to-end WS-PSNR V

(a)

(b)                                          (c)

Fig. 25 RD-curves for sequence Trolley (a) End-to-end WS-PSNR Y (b) End-to-end WS-PSNR U (c) End-to-end WS-PSNR V

Decoder complexity analysis

Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

Each of the 20 bit-streams contains 600 (Balboa sequence) or 300 frames (the other four sequences) for complexity measure, as listed in Table VII. For our submitted bit-streams and the HM anchors, the one-frame processing time, including bit-stream decoding and projection conversion from MCP to high-fidelity ERP, is evaluated under the same test environment. Note that the bit-stream decoding time of the JEM is estimated from that of the HM [8]. As shown in Table VII, the complexity of the bit-stream decoding required by the proposed method needs about 25.4x and 3.3x of the HM and JEM anchors, respectively.

Moreover, the complexity of the projection conversion from MCP to high-fidelity ERP is reported in Table VIII. The projection conversion for MCP is implemented based on 360Lib-5.0 [2] with C-level optimizations to reduce the complexity. Compared to the anchor which converts the PERP to a high-fidelity ERP using 360Lib-5.0 [2], the projection conversion time of MCP converter (converts the MCP to a high-fidelity ERP) needs only about 77.0% of the anchor.

Combined with bit-stream decoding and projection conversion, the average one-frame processing time is listed in Table IX. The complexity of the proposed implementation is about 2.3x and 1.6x of the HM and JEM anchors, respectively.

Table VII. Bit-stream decoding time comparisons

|  | Rate Point | HM anchors [sec/frame] | JEM anchors [sec/frame] | Proposed [sec/frame] | Proposed vs. HM anchors | Proposed vs. JEM anchors |
| --- | --- | --- | --- | --- | --- | --- |
| Balboa | R4 | 0.29 | 2.24 | 8.20 | 2789.4% | 366.1% |
|  | R3 | 0.29 | 2.25 | 7.58 | 2571.2% | 337.4% |
|  | R2 | 0.28 | 2.13 | 7.27 | 2602.7% | 341.6% |
|  | R1 | 0.24 | 1.84 | 6.80 | 2822.0% | 370.3% |
| ChairliftRide | R4 | 0.33 | 2.51 | 7.59 | 2302.3% | 302.1% |
|  | R3 | 0.31 | 2.36 | 7.01 | 2261.7% | 296.8% |
|  | R2 | 0.29 | 2.23 | 6.50 | 2221.5% | 291.5% |
|  | R1 | 0.29 | 2.19 | 6.30 | 2194.9% | 288.0% |
| KiteFlite | R4 | 0.22 | 1.66 | 6.06 | 2786.3% | 365.7% |
|  | R3 | 0.25 | 1.87 | 5.68 | 2315.2% | 303.8% |
|  | R2 | 0.24 | 1.81 | 5.63 | 2369.8% | 311.0% |
|  | R1 | 0.21 | 1.58 | 5.35 | 2575.1% | 337.9% |
| Harbor | R4 | 0.20 | 1.49 | 5.47 | 2796.6% | 367.0% |
|  | R3 | 0.19 | 1.47 | 5.27 | 2731.9% | 358.5% |
|  | R2 | 0.19 | 1.43 | 5.21 | 2781.3% | 365.0% |
|  | R1 | 0.21 | 1.59 | 5.15 | 2461.8% | 323.1% |
| Trolley | R4 | 0.22 | 1.71 | 5.62 | 2504.5% | 328.7% |
|  | R3 | 0.21 | 1.62 | 5.52 | 2593.0% | 340.3% |
|  | R2 | 0.20 | 1.51 | 5.32 | 2687.0% | 352.6% |
|  | R1 | 0.19 | 1.47 | 5.19 | 2695.6% | 353.8% |
| Average |  | 0.24 | 1.82 | 6.07 | 2544.6% | 333.9% |

Table VIII. Projection conversion time comparisons

|  | Rate Point | PERP to high-fidelity ERP [sec/frame] | MCP to high-fidelity ERP [sec/frame] | MCP convertor vs. PERP convertor |
| --- | --- | --- | --- | --- |
| Balboa | R4 | 2.35 | 1.82 | 77.5% |
|  | R3 | 2.35 | 1.81 | 77.1% |
|  | R2 | 2.36 | 1.89 | 80.2% |
|  | R1 | 2.35 | 1.89 | 80.4% |
| ChairliftRide | R4 | 4.27 | 3.00 | 70.2% |
|  | R3 | 4.03 | 3.07 | 76.2% |
|  | R2 | 3.88 | 3.05 | 78.6% |
|  | R1 | 4.01 | 3.11 | 77.5% |
| KiteFlite | R4 | 4.18 | 3.24 | 77.6% |
|  | R3 | 4.23 | 3.23 | 76.4% |
|  | R2 | 4.27 | 3.08 | 72.0% |
|  | R1 | 4.19 | 3.04 | 72.6% |
| Harbor | R4 | 4.03 | 3.33 | 82.8% |
|  | R3 | 4.09 | 3.34 | 81.6% |
|  | R2 | 4.09 | 3.17 | 77.6% |
|  | R1 | 4.21 | 3.21 | 76.2% |
| Trolley | R4 | 4.19 | 3.34 | 79.8% |
|  | R3 | 4.21 | 3.32 | 79.0% |
|  | R2 | 4.19 | 3.16 | 75.4% |
|  | R1 | 4.21 | 3.10 | 73.8% |
| Average |  | 3.70 | 2.85 | 77.1% |

Table IX. Processing time comparisons (bit-stream decoding and projection conversion)

|  | Rate Point | HM anchors [sec/frame] | JEM anchors [sec/frame] | Proposed [sec/frame] | Proposed vs. HM anchors | Proposed vs. JEM anchors |
| --- | --- | --- | --- | --- | --- | --- |
| Balboa | R4 | 2.65 | 4.59 | 10.02 | 378.6% | 218.2% |
|  | R3 | 2.64 | 4.59 | 9.39 | 355.8% | 204.5% |
|  | R2 | 2.64 | 4.49 | 9.16 | 347.3% | 204.2% |
|  | R1 | 2.59 | 4.19 | 8.69 | 335.1% | 207.5% |
| ChairliftRide | R4 | 4.60 | 6.78 | 10.58 | 230.1% | 156.1% |
|  | R3 | 4.34 | 6.39 | 10.08 | 232.5% | 157.8% |
|  | R2 | 4.17 | 6.10 | 9.54 | 228.9% | 156.3% |
|  | R1 | 4.30 | 6.20 | 9.41 | 219.0% | 151.8% |
| KiteFlite | R4 | 4.40 | 5.84 | 9.31 | 211.6% | 159.4% |
|  | R3 | 4.48 | 6.10 | 8.92 | 199.1% | 146.1% |
|  | R2 | 4.51 | 6.08 | 8.70 | 193.1% | 143.2% |
|  | R1 | 4.40 | 5.77 | 8.39 | 190.9% | 145.4% |
| Harbor | R4 | 4.22 | 5.52 | 8.80 | 208.4% | 159.5% |
|  | R3 | 4.28 | 5.56 | 8.61 | 201.0% | 154.8% |
|  | R2 | 4.28 | 5.52 | 8.38 | 195.8% | 151.8% |
|  | R1 | 4.42 | 5.80 | 8.36 | 189.2% | 144.1% |
| Trolley | R4 | 4.41 | 5.90 | 8.96 | 203.2% | 152.0% |
|  | R3 | 4.42 | 5.83 | 8.84 | 200.0% | 151.6% |
|  | R2 | 4.38 | 5.69 | 8.47 | 193.3% | 148.8% |
|  | R1 | 4.40 | 5.68 | 8.29 | 188.5% | 146.1% |
| Average |  | 3.95 | 5.59 | 9.03 | 228.5% | 161.5% |

Description of computing platform used to determine decoding times reported in section 5.1

A 64-bit executable file for decoding is generated by gcc version 4.8.4 on Linux environment. The specification of the test platform is a dual CPU of Intel® Xeon® Processor E5-2667 v3, which has total 16 cores operating at 3.20 GHz, 40 MB cache, and 125.4GB RAM.

Memory usage of decoder

This section analyzes the memory usage of the proposed decoders and compare it with the HM and JEM anchors. For precise evaluation, the malloc, calloc, and free functions are re-implemented so that the memory size in each memory management function could be collected (increased by malloc and calloc; decreased by free). The peak memory usage of a coding system could then be measured according to the collection information in memory allocation.

Memory usage of HM and JEM decoders

Based on the measurement method described above, the peak memory usage required by the HM and JEM decoder while decoding the anchor bit-streams is reported in Table X.

Table X. Peak memory usage in HM and JEM decoders

| Decoder | Memory usage (in MB) |
| --- | --- |
| HM | 1044.7 |
| JEM | 1362.0 |

Memory usage of decoder in proposed implementation

The memory usage in the proposed decoder implementation is listed in Table XI. As shown, the proposed 360° video specific decoding tools described in Section 2.2.3 additionally need about 280MB to accomplish the reconstruction process in the proposed design. The additional memory usage mainly comes from sub-frame buffers used in Inter prediction and the 360°-based in-loop filters, and CU information. The sub-frame buffer in the proposed implementation consists of one 3x1-face region with additional padding pixels.

Table XI. Peak memory usage in proposed decoder

| Decoder configuration | Memory usage (in MB) |
| --- | --- |
| Proposed decoder | 1208.6 |
| Proposed decoder without 360°specific decoding tools | 929.1 |

Based on the prediction structure in [3], the proposed decoder has to reserve reference frame buffers for Inter prediction used in the successive frames. The proposed implementation requires five additional reference buffers, each of which is composed of one top and bottom sub-frame buffers. Furthermore, each sub-frame has 272-pixel wide of padding area. Therefore, the memory usage of the five reference buffers is

1184×3+272×2×1184+272×2×22 bytes per pixel×1.5Sub-frame×2×5≈212.3(MB).

The 360°-based in-loop filters require one top and one bottom sub-frame buffers with 24-pixel wide of padding area in the proposed implementation. The two sub-frame buffers used to store the processed pixels by in-place computation in each stage needs

1184×3+24×2×1184+24×2×22 bytes per pixel×1.5Sub-frame×2≈26.6(MB).

The remained memory usage (about 40MB) is used for the necessary CU information storage.

The memory usage used in the projection conversion is also reported. The conversion process needs about 707MB for converting the MCP (3552x2368) to ERP (8192x4096). As for the anchor, the conversion process required for converting the PERP (4112x2048) to ERP (8192x4096) needs about 701MB. However, in practical implementation, the render engine may produce the viewport image directly from the decoded projection frames, instead of projection conversion.

Complexity characteristics of 360° video specific decoding tools

CU partition

As mentioned in Section 2.2.3.1, the edge checking on the frame boundary or the discontinuous edge, which is determined according to the projection layout, is necessarily performed before CU decoding (syntax parsing). For example, the parsing process of splitting flag is skipped when there exist with the horizontal discontinuous edge and a vertical frame boundary inside the current CU.

Inter prediction

Geometry padding

The geometry padding in the proposed implementation is achieved by coordinate conversion and Lanczos filter interpolation. The coordinate conversion for each pixel in the padding area is performed by using the proposed MCP mapping functions. The pixel value at the obtained coordinate is then calculated by applying the two-dimensional Lanczos interpolation filter. The computation complexity of the Lanczos interpolation process for each padding pixel is analyzed in Table XII.

Table XII. Computation analysis of interpolation filter

| Component | Coordinate mapping | Interpolation |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Filter coefficients | Multiplication | Addition | Shift |
| Luma | Floating-point operation | Lanczos 6x6 | 36 | 35 | 1 |
| Chroma |  | Lanczos 4x4 | 16 | 15 |  |

Motion vector projection

As mentioned in Section 2.2.3.2.2, the derivation process for the projected motion vector applies the MCP mapping functions to the starting point and the ending point of the referenced motion vector.

Intra prediction

For intra prediction, the geometric neighboring pixels are copied directly from other faces and the complexity depends on the number of geometric positions accessed when the reference pixels used in intra prediction are across the frame boundary (layout boundary) or the discontinuous edge.

360°-based In-loop filtering

The complexity in the proposed modifications for in-loop filters comes from the corresponding filtering process and the pixel padding process. The pixel padding process is performed when the accessed pixels (to be referenced and/or filtered) outside the frame boundary (layout boundary) or the discontinuous edge. For the de-blocking filter, the SAO filter and the NLM filter, the accessed pixels located at geometric positions are copied directly from other faces and the complexity depends on the number of geometric positions accessed while processing. Moreover, the ALF filter uses the geometric padding process, where the complexity is described in Section 5.4.2.1.

In addition to the padding process, the de-blocking filter uses the projected motion vector for appropriate boundary strength calculation and the additional complexity of motion vector projection is described in Section 5.4.2.2. For the CNN filter, an LCU is divided into two small processing units in the filtering process when containing the discontinuous edge.

Encoder complexity analysis

Encoding time and measurement methodology

Each test sequence is separated into 10 IRAP segments for parallel encoding. The encoding time of each frame is then collected after the all 10 encoding jobs are finished, where the encoding time of the first picture is discarded for each job except the first. For our submitted bit-streams and the HM anchors, the average one-frame encoding time is evaluated under the same test environment. In Table XIII, the complexity of the proposed method has about 9.0x and 1.1x of the HM and JEM anchors, respectively. Note that the bit-stream encoding time of the JEM is estimated from that of the HM [11].

Table XIII. Bit-stream encoding time comparisons

|  | Rate Point | HM anchors [sec/frame] | JEM anchors [sec/frame] | Proposed [sec/frame] | Proposed vs. HM anchors | Proposed vs. JEM anchors |
| --- | --- | --- | --- | --- | --- | --- |
| Balboa | R4 | 226.68 | 1806.66 | 3006.49 | 1326.3% | 166.4% |
|  | R3 | 210.70 | 1679.27 | 2424.88 | 1150.9% | 144.4% |
|  | R2 | 195.00 | 1554.16 | 1814.16 | 930.3% | 116.7% |
|  | R1 | 177.73 | 1416.48 | 1497.81 | 842.8% | 105.7% |
| ChairliftRide | R4 | 213.40 | 1700.82 | 3080.55 | 1443.5% | 181.1% |
|  | R3 | 195.37 | 1557.07 | 2364.06 | 1210.1% | 151.8% |
|  | R2 | 183.46 | 1462.19 | 1884.71 | 1027.3% | 128.9% |
|  | R1 | 169.24 | 1348.85 | 1474.27 | 871.1% | 109.3% |
| KiteFlite | R4 | 163.34 | 1301.81 | 2040.73 | 1249.4% | 156.8% |
|  | R3 | 157.39 | 1254.39 | 1519.86 | 965.7% | 121.2% |
|  | R2 | 151.86 | 1210.32 | 1232.90 | 811.9% | 101.9% |
|  | R1 | 142.26 | 1133.80 | 909.76 | 639.5% | 80.2% |
| Harbor | R4 | 155.17 | 1236.70 | 1440.07 | 928.1% | 116.4% |
|  | R3 | 152.29 | 1213.76 | 1089.19 | 715.2% | 89.7% |
|  | R2 | 149.24 | 1189.46 | 796.21 | 533.5% | 66.9% |
|  | R1 | 139.72 | 1113.53 | 598.39 | 428.3% | 53.7% |
| Trolley | R4 | 164.22 | 1308.85 | 2055.31 | 1251.5% | 157.0% |
|  | R3 | 158.34 | 1261.97 | 1618.77 | 1022.3% | 128.3% |
|  | R2 | 156.04 | 1243.65 | 1268.46 | 812.9% | 102.0% |
|  | R1 | 150.79 | 1201.78 | 1007.71 | 668.3% | 83.9% |
| Average |  | 168.94 | 1346.48 | 1521.70 | 900.7% | 113.0% |

Description of computing platform used to determine encoding times reported in section 6.1

A 64-bit executable file for encoding is generated by gcc version 4.8.4 on Linux environment. The test platform is the same as that described in Section 5.2.

Memory usage of encoder

This section analyzes the memory usage of the proposed encoders and compare it with the HM and JEM anchors by using the memory usage measurement method as described in Section 5.3. Note that memory usage of the conversion from high-fidelity ERP to target projection format (ERP in HM/JEM anchors or MCP in proposed encoder) is also included.

Memory usage of HM and JEM anchors

The peak memory usage required by the HM and JEM encoders while encoding the anchor bit-streams is reported in Table XIV.

Table XIV. Peak memory usage in HM and JEM encoders

| Encoder | Memory usage (in MB) |
| --- | --- |
| HM anchor | 3977.0 |
| JEM anchor | 8517.8 |

Memory usage of proposed encoder

The memory usage in the proposed encoder implementation is listed in Table XV. As shown, the proposed design additionally needs about 268MB to accomplish the proposed 360° video specific encoding tools described in Section 3.2.2. The additional memory usage used in the encoder is the similar with that required by the decoder.

Table XV. Peak memory usage in proposed encoder

| Encoder configuration | Memory usage (in MB) |
| --- | --- |
| Proposed encoder | 5517.6 |
| Proposed encoder without 360°specific encoding tools | 5250.0 |

Complexity characteristics of 360° video specific encoding tools

CU partition

Although the edge checking is performed for each CU to skip certain unnecessary partition modes, the complexity of CU partitioning in the proposed encoder is similar to that of JVET-J0018 [3].

360°-based In-loop filtering

Compared to the proposed decoder, the additional complexity in encoder comes from the determination of ALF filter coefficients and the CNN training process. The derivation process of ALF filter coefficients is the same as that of traditional ALF. Moreover, the additional complexity in the CNN training process requires that each frame is divided into two 3x1 face regions for weighting calculation.

Software implementation description

The proposed 360º video coding system is implemented based on the JEM-based software and 360Lib-5.0 with C-level optimizations to reduce the conversion complexity, and the programming language used in this contribution is standard C++. The proposed system is suitable as a reference software for 360º video to explore the projections and 360° video specific coding tools.

Closing remarks

In this contribution, the proposed algorithm and its implementation for the 360° video category are explained in detail. A MCP format, arranged into a compact 3x2 layout, is applied for efficient representation of 360° video. To address the geometric continuity in 360° video, the proposed coding tools are provided to appropriately process pixel data in inter prediction, intra prediction, and in-loop filters. Compared with the HM and JEM anchors, the proposed implementation achieves significant improvement both in objective and subjective quality.

Acknowledgement

We would like to thank Ching-Yeh Chen, Tzu-Der Chuang, Chih-Wei Hsu, and Shih-Ta Hsiang for their help on integrating the proposed 360º video coding system into JVET-J0018 [3].

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

- A. Segall, V. Baroncini, J. Boyce, J. Chen, and T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-H1002, Oct. 2017

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei, “Description of SDR video coding technology proposal by MediaTek,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J0018, Apr. 2018

- Y. Ye, E. Alshina, and J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-H1004, Oct. 2017

- Cube mapping, https://en.wikipedia.org/wiki/Cube_mapping

- M. Zhou, “AHG8: A study on Equi-Angular Cubemap projection (EAC),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-G0056, July 2017

- Equi-Angular Cubemap Projection (EAC), https://blog.google/products/google-vr/bringing-pixels-front-and-center-vr-video/

- Lanczos resampling, https://en.wikipedia.org/wiki/Lanczos_resampling

- HM reference software, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/

- JEM reference software, https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/

- M. Zhou, J. Chen, E. Francois, P. Hanhart, E. Alshin, “Template for Proposal Description Documents for Responses to the Joint CfP on Video Compression with Capability beyond HEVC,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-I1003, Jan. 2018

Annex: Questionnaire

Proponents of CfP responses are required to answer the codec level questions, and are also encouraged to fill out the optional tool level questions in addition to the complexity analysis of their proposals. If some question is not applicable to your proposal, you may express that as well.

Codec level questions

- Maximum number of reference frames used by encoder

Identical to JVET-J0018.

- Maximum coding unit (e.g. CTU in HEVC)

Identical to JVET-J0018.

- Minimum and maximum transform block size

Identical to JVET-J0018.

- Minimum and maximum intra prediction block

Identical to JVET-J0018.

- Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length

Identical to JVET-J0018.

- List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category)

Identical to JVET-J0018.

- List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples)

For 360° video specific coding tools, the geometry padding in Section 2.2.3.2.1 and motion vector prediction in 2.2.3.2.2 require floating point operations to derive geometric pixel position.

- List all tools that use non-rectangular (e.g. triangle) partitions

Identical to JVET-J0018.

- Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa)

Identical to JVET-J0018.

- Indicate whether luma and chroma use separate partitioning structures

Identical to JVET-J0018.

- Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes)

Identical to JVET-J0018.

- If the coded picture resolution is different from the input picture resolution, list the coded picture resolution for those sequences (e.g. 360° video, input is 8K ERP, coded video is in 4K CMP converted from the 8K input)

The input picture resolution and the coded picture resolution used in the proposed contribution are listed in Table XIV.

Table XIV. Input and coded picture resolutions

| Sequence | Input picture resolution | Coded picture resolution |
| --- | --- | --- |
| Balboa | 6144x3072 | 3552x2368 |
| ChairliftRide | 8192x4096 | 3552x2368 |
| KiteFlite | 8192x4096 | 3552x2368 |
| Harbor | 8192x4096 | 3552x2368 |
| Trolley | 8192x4096 | 3552x2368 |

Tool level questions

The proposed 360º video coding system is integrated into JVET-J0018. The tool level questions related to 360° video specifically are expressed as follows:

- Intra prediction

  - Line buffer size in terms of luma and chroma sample lines

The line buffer size for intra prediction is identical to JVET-J0018. However, when encoding the bottom 3x1 face region, the reference pixels across frame boundary (layout boundary) or discontinuous edge are accessed from the top 3x1 face region, where the pixel data are not performed by the in-loop filtering process. The un-filtered pixel data may need to be stored in an extra memory buffer.

- Inter prediction

  - List all the interpolation filters used for luma/chroma motion compensation

Before the motion compensation, the reference pixels, across frame boundary (layout boundary) and discontinuous edge, are obtained by the geometry padding process, which is achieved by MCP mapping functions and two-dimensional Lanczos interpolation filters. Then, the motion compensation in luma/chroma components uses the same interpolation filters as described in JVET-J0018.

- In-loop filtering

- List line buffer size in terms of luma and chroma lines (if any)

The line buffer size for in-loop filtering is identical to JVET-J0018. The accessed pixels (to be referenced and/or filtered), outside the frame boundaries (layout boundary) or discontinuous edge, are obtained from other faces. In the proposed implementation, two sub-frame buffers with padding pixels are used to store the input pixel data.

- List all the filters used for in-loop filtering

Identical to JVET-J0018.

- Minimum processing unit size

Identical to JVET-J0018.

- “tool-off” and “tool-on” tests

- List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools (see Table 1 below for test procedure)

The major tools in the proposed implementation includes:

- Modified cubemap projection (MCP)

- 360° video specific decoding tools

  - Inter prediction

  - Intra prediction

- 360°-based in-loop filters

For the Tool-on tests, we first turn on the MCP and keep all the other tools in the test candidates list off to test the performance of MCP projection. The performance of MCP on is then used as an anchor to conduct the tool-on test for each of the 360° video specific coding tools and in-loop filters with MCP enabled, since those tools are currently implemented on the cube projection. For the 360°-based in-loop filters, the designed modifications are mainly for reducing the visual artifacts.

For the Tool-off tests, we turn off each tool in the 360° video specific coding tools and in-loop filters with MCP enabled. After that, we turn off the MCP with all the other 360° video specific coding tools and in-loop filters disabled.

Table XV. Tool-on test for the MCP

|  | MCP |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Balboa | -5.29% | -11.55% | -11.51% |
| Chairlift | -16.40% | -15.30% | -12.46% |
| KiteFlite | -7.56% | -21.03% | -24.25% |
| Harbor | -8.72% | -16.25% | -15.86% |
| Trolley | -6.74% | -20.66% | -21.40% |
| Average | -8.94% | -16.96% | -17.09% |

Table XVI. Tool-on tests for the 360° video specific coding tools relative to the MCP-on anchor

|  | Inter Prediction |  |  | Intra Prediction |  |  | 360°-based in-loop Filters |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Balboa | -2.68% | -5.30% | -5.23% | -0.65% | -1.32% | -1.91% | -0.42% | -1.92% | -2.00% |
| Chairlift | -3.39% | -3.99% | -3.60% | -0.99% | -1.43% | -1.84% | -0.32% | -0.88% | -1.40% |
| KiteFlite | 0.15% | -0.21% | 0.43% | -0.11% | -1.13% | -0.76% | -0.06% | -1.00% | -0.88% |
| Harbor | 0.55% | 0.00% | -0.44% | -0.33% | -1.19% | -2.28% | -0.14% | -1.73% | -2.60% |
| Trolley | -0.03% | -0.41% | -0.46% | -0.17% | -1.58% | -1.48% | -0.07% | -2.05% | -1.69% |
| Average | -1.08% | -1.98% | -1.86% | -0.45% | -1.33% | -1.65% | -0.20% | -1.52% | -1.71% |

Table XVII. Tool-off test for the MCP with all 360° coding tools

|  | MCP + all 360° coding tools |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Balboa | 9.37% | 21.06% | 20.30% |
| Chairlift | 24.63% | 24.88% | 20.92% |
| KiteFlite | 8.37% | 29.21% | 34.08% |
| Harbor | 9.97% | 21.91% | 22.77% |
| Trolley | 7.64% | 30.27% | 31.27% |
| Average | 11.99% | 25.47% | 25.87% |

Table XVIII. Tool-off tests for the 360° video specific coding tools

|  | Inter Prediction |  |  | Intra Prediction |  |  | 360°-based in-loop Filters |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Balboa | 2.65% | 4.52% | 3.81% | 0.37% | 0.91% | 0.52% | 0.12% | 1.04% | 0.25% |
| Chairlift | 2.93% | 3.49% | 2.65% | 0.32% | 0.38% | 0.80% | 0.05% | 0.69% | 1.09% |
| KiteFlite | 0.04% | 0.34% | 0.13% | 0.09% | 0.96% | 1.05% | 0.02% | 0.72% | 0.45% |
| Harbor | 0.01% | 0.00% | -0.14% | 0.21% | 0.41% | 0.63% | 0.07% | 0.77% | 0.55% |
| Trolley | 0.18% | 0.59% | 0.71% | 0.13% | 1.05% | 1.01% | 0.03% | 1.21% | 0.83% |
| Average | 1.16% | 1.79% | 1.43% | 0.23% | 0.74% | 0.80% | 0.06% | 0.89% | 0.63% |

Table 1. Test procedure of “tool-off” and “tool-on” tests for a CfP response

| step | “tool-off” test | “tool-on” test |
| --- | --- | --- |
| 1 | Identify what you consider major tools in a CfP response that contribute to compression efficiency gain, and create a test candidates list |  |
| 2 | Turn on all the tools in the test candidates list, run the CfP response software and compute PSNRs, bit rates, and run times | Turn off all the tools in the test candidates list, run the CfP response software, compute PSNRs, bit rates, and run times, and compute the BD-rate and run-time numbers relative to the HM anchor |
| 3 | Turn off a tool in the test candidates list, and keep all the other tools in the test candidates list on, run the CfP response software and compute the BD-rate and run-time numbers relative to the CfP response software with all tools on | Turn on a tool in the test candidates list, and keep all the other tools in the test candidates list off, run the CfP response software and compute the BD-rate and run-time numbers relative to the CfP response software with all tools of the test candidates list off (i.e. step 2) |
| 4 | Repeat step 3 until all the tools in the test candidates list are tested | Repeat step 3 until all the tools in the test candidates list are tested |
| 5 | Compose the test data |  |