| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0020-v1 |
| --- | --- |

| Title: | Description of SDR video coding technology proposal by Panasonic |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Panasonic’s answer to Call for Proposals |  |  |
| Author(s) or Contact(s): | Tadamasa Toma Takahiro Nishi Kiyofumi Abe Ryuichi Kanoh ChongSoon Lim Jingya Li Ruling Liao Sughosh Pavan Haiwei Sun Hanboon Teo Virginie Drugeon | Tel: Email: | +81-80-3594-3378 toma.tadamasa@jp.panasonic.com nishi.takahiro@jp.panasonic.com abe.kiyo@jp.panasonic.com kanoh.ryuichi@jp.panasonic.com +65-6550-5487 chongsoon.lim@sg.panasonic.com jingya.li@sg.panasonic.com ruling.liao@sg.panasonic.com sughosh.pavan@sg.panasonic.com haiwei.sun@sg.panasonic.com hanboon.teo@sg.panasonic.com +49-6103-766240 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

PEM (Panasonic Exploration Model) is the Panasonic answer to the call for proposals on video coding with capability beyond HEVC [1] in the SDR category. The software and syntax are based on JEM7.0. The main design principles in PEM development have been lower algorithmic complexity, especially for the decoder, and hardware friendliness for a better coding performance in average compared to JEM7.0. PEM provides an average of 2.3% coding gain compared to JEM7.0 for constraint set 1 at 107% of encoder runtime and 56% of decoder runtime, and an average of 2% coding gain for constraint set 2. It corresponds to a coding gain of 34.6% compared to HM16.16 for constraint set 1 and 26% for constraint set 2. Among others, coding tools include tri-tree block partitioning, triangle prediction blocks for motion compensation, a modified combination of inter prediction and transform tools, modifications to the algorithms of some coding tools from JEM, asymmetric deblocking filter, tools contributed by other organisations to JVET in the past, and bug fixes. Additionally, a version of PEM with lower complexity is also available that provides similar coding performances to JEM7.0 for an encoder runtime that is 60% that of JEM7.0.

# Contents

Abstract	i

Contents	i

1	Introduction	1

2	Decoder algorithm description	2

2.1	General algorithm description	2

2.1.1	Bitstream structure: tri-tree partition structure and syntax flow	2

2.1.2	Entropy decoding: Multi-hypothesis CABAC window size for each context	4

2.1.3	Conversion process from entropy decoded data to symbols	4

2.1.4	Decoder-side motion refinement	4

2.1.5	Inverse quantization	10

2.1.6	Inverse transforms	10

2.1.7	Motion compensation	11

2.1.8	Intra prediction: improved filtering	16

2.1.9	In-loop filtering	17

2.1.10	Additional tools	18

2.1.11	Additional algorithmic description topics	18

2.2	360° Video: N/A	20

2.3	HDR: N/A	20

3	Encoder algorithm description	20

3.1	General algorithm description	20

3.1.1	Rate-distortion optimization	20

3.1.2	Partitioning structure selection: fast algorithms	21

3.1.3	Intra prediction mode estimation and type selection	26

3.1.4	Motion estimation and motion segment selection	26

3.1.5	Mode decisions	28

3.1.6	In-loop filtering type selection and parameter estimation	28

3.1.7	Transforms and transform type selection	28

3.1.8	Quantization and quantization type selection	28

3.1.9	Entropy coding	29

3.1.10	Additional tools	29

3.1.11	Additional encoder optimization	29

3.1.12	Additional algorithmic description topic(s): Tools combinations for inter prediction	29

3.2	360°: N/A	29

3.3	HDR: N/A	29

4	Compression performance	30

4.1	SDR: Constraint set 1 configuration relative to HM anchor	30

4.1.1	Class SDR-A (RD-curves)	30

4.1.2	Class SDR-B (RD-curves)	31

4.1.3	Overall (BD-rate summary tables)	32

4.2	SDR: Constraint set 2 configuration relative to HM anchor	33

4.2.1	Class SDR-B	33

4.2.2	Overall	34

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	35

4.3.1	Class SDR-A	35

4.3.2	Class SDR-B	36

4.3.3	Overall	37

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	38

4.4.1	Class SDR-B	38

4.4.2	Overall	39

5	Decoder complexity analysis	39

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	39

5.2	Description of computing platform used to determine decoding times reported in section 5.1	39

5.3	Memory usage of decoder	39

5.4	Complexity characteristics of decoder entropy decoding operation: multi-hypothesis CABAC window size for each context	40

5.5	Complexity characteristics of decoder inverse quantization	40

5.6	Complexity characteristics of decoder inverse transform operation	40

5.7	Complexity characteristics of decoder-side motion refinement	40

5.7.1	Simplification of FRUC motion vector refinement process	40

5.7.2	Simplification of bi-directional optical flow	41

5.8	Complexity characteristics of decoder motion compensation	44

5.8.1	Complexity characteristics of triangular prediction unit mode	44

5.8.2	Simplification of overlapped block motion compensation	46

5.9	Complexity characteristics of decoder intra-frame prediction operation	46

5.10	Complexity characteristics of decoder in-loop filtering operation	46

5.11	Complexity characteristics of additional decoder tools	47

5.11.1	Block partitioning	47

5.11.2	Tools combinations for inter prediction	47

5.12	Complexity characteristics of 360° video specific decoding tools: N/A	49

5.13	Complexity characteristics of HDR specific decoding tools: N/A	49

5.14	Degree of capability for decoder parallel processing	49

6	Encoder complexity analysis	49

6.1	Encoding time and measurement methodology	49

6.2	Description of computing platform used to determine encoding times reported in section 6.1	49

6.3	Memory usage of encoder	50

6.4	Complexity characteristics of encoder intra-frame prediction type selection	50

6.5	Complexity characteristics of encoder motion estimation and motion segmentation selection	50

6.5.1	General: triangular prediction unit mode	50

6.5.2	Complexity characteristics of usage of decoder-side motion refinement	51

6.6	Complexity characteristics of encoder transforms and transform type selection	51

6.7	Complexity characteristics of encoder quantization and quantization type selection	51

6.8	Complexity characteristics of encoder in-loop filtering type selection and parameter estimation	51

6.9	Complexity characteristics of encoder entropy coding type selection	51

6.10	Complexity characteristics of additional encoder tools: block partition structure selection	51

6.11	Complexity characteristics of 360° video specific encoding tools: N/A	52

6.12	Complexity characteristics of HDR specific encoding tools: N/A	52

6.13	Degree of capability for encoder parallel processing	52

7	Algorithmic characteristics	53

7.1	Random access characteristics	53

7.2	Delay characteristics	53

7.3	Additional characteristics discussion topic(s)	53

8	Software implementation description	53

9	Highlighted aspects discussion	53

9.1	Low complexity version of PEM	53

9.2	Panasonic’s NextSoftware	54

10	Closing remarks	55

11	Patent rights declaration(s)	55

12	Annex 1: Questionnaire	55

12.1	Codec level questions	55

12.2	Tool level questions	56

12.2.1	Partitioning structure	56

12.2.2	Entropy coding	56

12.2.3	Control data derivation process	57

12.2.4	Decoder-side motion refinement	57

12.2.5	Intra prediction	58

12.2.6	Inter prediction	58

12.2.7	Quantization	59

12.2.8	Transforms	59

12.2.9	In-loop filtering	60

12.2.10	“tool-off” and “tool-on” tests	61

13	Annex 2: Syntax	62

14	References	69

# Introduction

PEM (Panasonic Exploration Model) is an answer to the call for proposals on video coding with capability beyond HEVC [1] in the SDR category only. It is heavily based on previous JVET’s work. The software and syntax are based on JEM7.0 with modifications to achieve major complexity reduction for both software and hardware deployment with some further coding gain at almost the same encoder complexity.

PEM decoder runtime is 56% of JEM7.0 decoder runtime thanks to optimised combinations of coding tools and complexity reduction technologies. PEM provides an average of 2.3% coding gain compared to JEM7.0 for constraint set 1 at 107% of encoder runtime and 56% of decoder runtime, and an average of 2% coding gain for constraint set 2. It corresponds to a coding gain of 34.6% compared to HM16.16 for constraint set 1 and 26% for constraint set 2.

The coding performance improvement of PEM is, among others, due to improved block partitioning including tri-tree partitioning, triangle prediction blocks for improved motion compensation and asymmetric deblocking filter. Many new coding tools introduced in PEM are based on previous work contributed to JVET by various organisations, but not included in JEM so far. PEM also fixes bugs discovered in JEM7.0. Significant complexity reduction could be achieved by optimising combinations of existing inter prediction and transform tools from JEM, as well as improvements to the algorithms of some tools.

In addition, an even lower complexity version of PEM is also available that provides similar coding performances to JEM7.0 for an encoder runtime that is 60% that of JEM7.0. The difference between the two versions of PEM is that tri-tree partitioning has been disabled in the low complexity version.

In this document, the following terminology is used:

- ALF: Adaptive Loop Filter

- AMVP: Adaptive Motion Vector Prediction

- AMVR: Adaptive Motion Vector Resolution

- ATMVP: Advanced Temporal Motion Vector Prediction

- BIO: Bidirectional Optical flow

- BT: Binary Tree

- BLF: Bilateral Filter

- CABAC: Context-Adaptive Binary Arithmetic Coding

- CS1: Constraint Set 1

- CS2: Constraint Set 2

- CTU: Coding Tree Unit

- CU: Coding Unit

- DBF: Deblocking Filter

- DCT: Discrete Cosine Transform

- DMVR: Decoder-side Motion Vector Refinement

- DST: Discrete Sine Transform

- EMT: Explicit Multiple-core Transform

- FRUC: Frame Rate Up Conversion

- HEVC: High Efficiency Video Coding

- HM: HEVC test Model

- JEM: Joint Exploration Model

- LIC: Local Illumination Compensation

- MC: Motion Compensation

- MDIS: Mode Dependent Intra Smoothing

- MPM: Most Probably Mode

- MV: Motion Vector

- NSST: Non-Separable Secondary Transform

- OBMC: Overlapped Block Motion Compensation

- PEM: Panasonic Exploration Model

- P-PDPC: Planar Position Dependent Prediction Combination

- QP: Quantization Parameter

- QT: Quad Tree

- SAD: Sum of Absolute Differences

- SAO: Sample Adaptive Offset

- SDR: Standard Dynamic Range

- SPS: Sequence Parameter Set

- STMVP: Spatial-Temporal Motion Vector Prediction

- TMVP: Temporal Motion Vector Prediction

- TT: Tri Tree

# Decoder algorithm description

## General algorithm description

### Bitstream structure: tri-tree partition structure and syntax flow

Tri-tree (TT) partitioning is implemented on top of quad-tree (QT) partitioning and binary-tree (BT) partitioning from JEM, where a block from QT partitioning may be further partitioned using a BT or TT structure. TT partitioning is applied in horizontal direction (Figure 1 (d)), or vertical direction (Figure 1 (e)). This technology is conceptually similar to the technology proposed in JVET-D0117 [2].

Figure 1: Possible splitting of a block from QT partitioning: (a) no-split (b) horizontal BT partitioning (c) vertical BT partitioning (d) horizontal TT partitioning (e) vertical TT partitioning (f) QT partitioning

As shown in Figure 2, a Coding Tree Unit (CTU) is firstly partitioned using a QT structure. The blocks from QT partitioning are further partitioned using a BT or TT structure in horizontal or vertical direction. In the case of BT partitioning, a block is split into two symmetric partitions (see Figure 1 (b) and (c)). In the case of TT partitioning, a block is split into three partitions where the center partition is twice as big as the other two partitions (see Figure 1 (d) and (e)). The leaf node is named as Coding Units (CUs), which is used for prediction and transform without any further partitioning. QT partitioning is invalid on a block which is obtained from BT or TT partitioning. Both BT and TT partitioning are valid on a block which is obtained from QT, BT or TT partitioning.

Figure 2: Illustration of a combination of QT, BT and TT partition structures

Figure 3 shows the signaling of QT, BT, and TT partitioning. A flag qt_split_flag signals if a block is further partitioned using a QT structure. When qt_split_flag is equal to 0, the block may be a leaf node (if bt_tt_split_flag is equal to 0) or may be further partitioned using a BT or TT structure (if bt_tt_split_flag is equal to 1), where the flag bt_tt_split_flag signals if a block is further partitioned using a BT or TT structure. When bt_tt_split_flag is equal to 1, two other flags are needed to signal if the partitioning is BT or TT partitioning (tt_flag is equal to 1 for TT partitioning, 0 for BT partitioning) and if it is horizontal or vertical (split_dir_flag is equal to 1 for vertical partitioning, 0 for horizontal partitioning). Please refer to the syntax in section 13 Annex 2 for further information. The flags qt_split_flag, bt_tt_split_flag and split_dir_flag either already exist in JEM or extend the meaning of existing flags in JEM. Therefore, these flags are coded using the same context models as already used in JEM. One new context model was defined for coding of tt_flag.

TT vertical partitioning is invalid when the block width is smaller than the larger value between MinBTTTSize and 16, or when the block height is greater than twice the block width. When TT vertical partitioning is invalid, TT_HOR (TT horizontal partitioning) is automatically chosen. TT horizontal partitioning is invalid when the block height is smaller than the larger value between MinBTTTSize and 16 or when the block width is greater than twice the block height. When TT horizontal partitioning is invalid, TT_VER (TT vertical partitioning) is automatically chosen. TT partitioning is invalid when both TT horizontal partitioning and TT vertical partitioning are invalid, or when the block width and height are larger than MaxBTTTSize. When TT partitioning is invalid, BT partitioning is automatically chosen. Please note that both BT and TT partitioning increase the depth of the BT/TT common tree, so that there is only one parameter MaxBTTTDepth to limit the depth of the two partitioning methods. When the common depth for BT/TT partitioning reaches MaxBTTTDepth, both BT and TT partitioning are invalid, which stops the partitioning process. In PEM, MinBTTTSize equals to MinBTSize, MaxBTTTDepth equals to MaxBTDepth, and MaxBTTTSize equals to MaxBTSize. MinBTSize, MaxBTDepth and MaxBTSize have the same values as in JEM7.0.

Figure 3: Signalling for block partitioning

### Entropy decoding: Multi-hypothesis CABAC window size for each context

The technology described in JVET-H0061 [3] Test 2 is included into PEM. It consists in extending the JEM double window estimation technique to allow different pairs of window parameters for each coding context. In JVET-H0061 Test2, the window size for each context model is pre-defined and is fixed for all QPs and slice types.

### Conversion process from entropy decoded data to symbols

#### Intra prediction parameters

The technology described in JVET-H0024 [4] is included into PEM. As described in JVET-H0024, it consists in modifications to the lists of MPM and selected modes.

#### Motion parameters

See section 2.1.7.1 for decoding of triangle_merge_flag and triangle_idx.

#### Quantization parameters

Same as JEM7.0.

#### Transform coefficients

Same as JEM7.0.

#### In-loop filter parameters

Same as JEM7.0.

### Decoder-side motion refinement

#### FRUC bandwidth reduction

This technique targets at reducing memory bandwidth required for FRUC.

Worst-case memory bandwidth is approximately 7% of that estimated in JEM7.0 as shown in Table 1, where FRUC worst-case memory bandwidth is compared to that of JEM7.0 (i.e. 4×4 bi-prediction) for motion compensation.

Table 1: FRUC motion estimation worst-case memory bandwidth of JEM and PEM

| JEM7.0 | PEM |
| --- | --- |
| 5.6 times worst-case bandwidth of MC in JEM7.0 | 0.4 times worst-case bandwidth of MC in JEM7.0 |

In JEM7.0, when performing the process of selecting the best motion vector for a whole coding unit using FRUC, memory needs to be retrieved independently for each candidate motion vector in the candidate list, which results in 5.6 times worst-case bandwidth of MC in JEM7.0. In PEM, each CU in one CTU shares a common pre-defined memory block from each reference frame. When a coding unit uses FRUC in PEM, only the reference samples within the pre-defined memory block of the corresponding CTU can be used. A brief illustration of the design is highlighted in Figure 4, where the gray CU is only allowed to use motion vectors that point to blocks contained within the blue memory blocks in the reference frames.

Figure 4: FRUC memory block design

The size of a memory block is pre-defined and identical for all CTUs within a picture. This pre-defined size depends on the number of reference frames of the current frame so that total bandwidth per CTU is 0.4 times worst-case bandwidth of MC in JEM7.0. The number of 0.4 was experimentally selected in order not to degrade coding gain while reducing the bandwidth.

The size of a pre-defined memory block for a given number of reference frames was calculated as follows:

- The worst-case bandwidth of motion compensation in JEM7.0 is 350208 × 10 bits (calculated assuming that all CUs are of minimum size and using bi-prediction).

- The target bandwidth of FRUC per CTU was experimentally set as 0.4 times the worst-case bandwidth of MC in JEM7.0, which is equal to 0.4 × 350208 × 10 bits = 140083 × 10 bits.

- Therefore, the size of the predefined memory block (needed for luma only) is 140083/number_of_reference_frames.

Details of memory block size are given in Table 2.

Table 2: FRUC memory block size

| Number of reference frames | Pre-defined memory block size |
| --- | --- |
| 1 | 374×374 |
| 2 | 264×264 |
| 3 | 216×216 |
| 4 | 187×187 |
| 5 | 167×167 |
| 6 | 152×152 |
| 7 | 141×141 |
| 8 | 132×132 |

The location of a memory block in a target reference frame target_ref_pic for a current CTU located at [i, j], wherein i and j indicate horizontal and vertical positions of left top corner of the CTU within the picture, is calculated as explained below.

(MVX, MVY), which represent intermediate parameters for calculating the position of the pre-defined memory block, is calculated as the average values of a set of motion vectors U. This set of motion vectors U includes the motion vectors of the CUs from the left and/or top neighboring CTUs of the current CTU that point to the target reference frame target_ref_pic. Equations (1) and (2) below show the calculation of (MVX, MVY):

MVX=CU_MV∈U CU_MVx×CU_area/ CU_MV∈U CU_area	(1)

MVY=CU_MV∈U CU_MVy×CU.area/ CU_MV∈U CU_area	(2)

where U denotes the set of motion vectors pointing to the target reference frame target_ref_pic and CU_area denotes the number of pixels in the corresponding CU. The definition of U varies depending on the position of the current CTU as shown below:

If the current CTU is the top left CTU of a frame,

U includes only a zero motion vector;

Otherwise, if the current CTU is located on the top boundary of the frame,

U includes all the motion vectors of the left neighboring CTU that point to target_ref_pic;

Otherwise, if the current CTU is located on the left boundary of the frame,

U includes all the motion vectors of the top neighboring CTU that point to target_ref_pic;

Otherwise,

U includes all the motion vectors of the top and left neighboring CTUs that point to target_ref_pic.

Given a pre-defined memory block with N×N size, current CTU location as [i, j], and the motion vector (MVX, MVY) calculated above, the location of the top-left pixel of the pre-defined memory block within each reference frame target_ref_pic is calculated in equations (3) and (4), where CTUsize is equal to 128 in PEM.

location X=i-((N-CTUsize-1)≫1)+MVX	(3)

location Y=j-((N-CTUsize-1)≫1)+MVY	(4)

#### Simplification of FRUC motion vector refinement process

Compared with FRUC existing technique in JEM, there are two main changes to the MV refinement process:

- Redundant refinement process is removed;

- Motion vector refinement search is simplified.

In the first change, the MV refinement process is skipped if all of the following conditions are satisfied:

- Current CU is in FRUC template matching mode;

- The motion vector points to a reference frame in reference list 1;

- The motion vector pointing to a reference frame in reference list 1 is identical to the motion vector pointing to a reference frame in reference list 0.

In the second change, simplification is applied to unrestricted center-biased diamond search (UCBDS). A flow chart and an example of the proposed motion vector refinement search are shown in Figure 5 and Figure 6, respectively.

Figure 5: proposed process for FRUC simplified refinement search

Figure 6: An example of FRUC simplified refinement search

#### MV candidate list simplification for CU level FRUC

Figure 7 shows the MV candidate list generation process for FRUC at CU level together with the number of MV candidates at each step for both JEM and PEM to highlight the differences.

The first change is related to inclusion of ATMVP/STMVP candidates. In JEM, the merge candidate list is generated including ATMVP and STMVP candidates, despite the fact that they are removed in the next step. On the other hand, PEM generates merge candidate list without ATMVP and STMVP candidates.

The second change removes FRUC unilateral candidates. This change reduces the complexity by making the generation process of interpolated MV field for the picture unnecessary.

Figure 7: flowchart of CU level FRUC MV candidate list generation in JEM (left) and PEM (right)

#### AMVP candidate list simplification

Figure 8 shows AMVP candidate list generation process with AMVR for Normal Inter mode for both JEM and PEM to highlight the differences. In JEM, a rounding process for each MV candidate is applied a first time just before adding FRUC best MV to the candidate list and a second time after reordering the candidate list. Therefore, both intermediate list1 and intermediate list2 in Figure 8 will be changed depending on AMVR precision. On the other hand, in PEM, the rounding process for each MV candidate is applied only once after reordering the candidate list. Intermediate list2 is always the same independently from AMVR precision, and it can be reused easily.

Figure 8: flowchart of AMVP candidate list generation in JEM (left) and PEM (right)

#### Removal of FRUC sub-CU level process

For complexity reasons, the sub-CU level process of FRUC has been removed in PEM. FRUC is applied only at the CU level.

#### Simplification of bi-directional optical flow

In PEM, gradients for BIO are calculated by applying a 5-tap gradient filter, with coefficients {2, -9, 0, 9, -2}, to the prediction samples obtained from motion compensation interpolation process. Given a CU with M×N block size, a reference block with (M+i)×(N+j) block size is obtained according to the motion vector of the CU, wherein i and j indicate the number of additional reference samples needed for performing interpolation process. The reference block is firstly padded into (M+i+4)×(N+j+4) block size, and then the interpolation process is performed to output a (M+4)×(N+4) prediction block. The 5-tap gradient filter is applied to the prediction block to generate gradients. The process for calculating gradients is illustrated in Figure 9.

Figure 9: The process for calculating gradients in PEM

Furthermore in PEM, BIO is applied only to bi-predicted blocks when the two predictions are from reference pictures with different time directions, i.e. one reference picture in the past and one in the future. But BIO is not applied to bi-predicted blocks using reference pictures in the same time direction, i.e. both reference pictures in the past or both in the future. Therefore, BIO is never applied under low delay B conditions.

In addition, BIO is disabled when ATMVP/STMVP is applied.

#### BIO modification on displacement calculation

Compared to current BIO motion refinement technique in JEM, a different equation for calculating block-wise displacement parameters vx,vy is proposed. This method is based on [5].

In the proposed technique, calculation for vx,vy is as follows:

vx=Clip1(((sGy2×sGxdI-sGxGy×sGydI≫shift-12)×value)≫27)	(5)

vy=Clip1(((sGx2×sGydI-sGxGy×sGxdI≫shift-12)×value)≫27)	(6)

where:

Gx=τ0×Ix0+τ1×Ix1;    Gy=τ0×Iy0+τ1×Iy1;     ∆I=I0-I1	(7)

sGxGy=[i,j]∈Ωw[i,j]×Gx[i,j]×Gy[i,j]	(8)

sGx2=[i,j]∈Ωw[i,j]×Gx[i,j]×Gx[i,j]	(9)

sGy2=[i,j]∈Ωw[i,j]×Gy[i,j]×Gy[i,j]	(10)

sGxdI=[i,j]∈Ωw[i,j]×Gx[i,j]×∆I[i,j]	(11)

sGydI=[i,j]∈Ωw[i,j]×Gy[i,j]×∆I[i,j]	(12)

shift=bit_length(sGx2×sGy2-sGxGy×sGxGy)-6	(13)

value=BIOShift[ClipsGx2×sGy2-sGxGy×sGxGy≫shift]	(14)

BIOShifti=1≪15+i2i;     i∈[1,63]	(15)

Ix0 and Ix1 are pixel-wise horizontal gradient values by applying a horizontal gradient filter to pixels in two reference frames, as described in section 2.1.4.6. Iy0 and Iy1 are pixel-wise vertical gradient values by applying a vertical gradient filter to pixels in two reference frames, as described in section 2.1.4.6. I0 and I1 are pixel values in two reference frames. τ0 and τ1 denote the distances to the reference frames and are calculated based on the POC values of the two reference frames and the current frame. Ω denotes a window that contains a block of pixels. w is a weight matrix of the same size as window Ω, which is a pre-defined 2D array used to assign different weights to each element of the multiplication operation according to its position. It is the same weight matrix as used in JEM7.0.

### Inverse quantization

Same as JEM7.0.

### Inverse transforms

EMT in both JEM and PEM uses three syntax elements, called “emt_cu_flag” and “{hor/ver}_emt_type” (see syntax in section 13 Annex 2). In JEM, whether DCT2 is selected or not is signaled as “emt_cu_flag” (when emt_cu_flag is equal to 0, EMT is not used, which means that DCT2 is selected). If DCT2 is not selected (emt_cu_flag equal to 1), selected transform kernel type, which may include two of DST7, DCT8, DST1 or DCT5 depending on the signalled intra prediction mode, is coded as “{hor/ver}_emt_type”. However in PEM, if the width or height of the block is less than or equal to a threshold, DST7 is selected by default as the only candidate for EMT. Therefore, hor_emt_type is not signalled if the width of the block is less than or equal to the threshold, and ver_emt_type is not signalled if the height of the block is less than or equal to the threshold. In PEM, the threshold is set to be 4.

NSST in both PEM and JEM uses one syntax element called “nsst_idx” (see syntax in section 13 Annex 2). When nsst_idx is equal to 0, NSST is not applied. Otherwise, nsst_idx indicates which secondary transform should be applied. In PEM, NSST is used only when DCT2 is used as primary transform. Therefore, if DCT2 is not used as primary transform, i.e. if EMT is used, nsst_idx is not signalled and NSST is not applied.

Table 3: Signaling condition of {hor/ver}_emt_type and nsst_idx based on block shape

| Block shape |  | Is the syntax element signaled in the bitstream? |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | emt_cu_flag = 0 | emt_cu_flag = 1 |  |  |
|  |  | Signaling nsst_idx | Signaling hor_emt_type | Signaling ver_emt_type | Signaling nsst_idx |
| width <= threshold | height <= threshold | yes | no | no | no |
| width <= threshold | height > threshold | yes | no | yes | no |
| width > threshold | height <= threshold | yes | yes | no | no |
| width > threshold | height > threshold | yes | yes | yes | no |

These skip methods for both EMT and NSST are applied to luma component in intra mode only. The transform kernels themselves are the same as those of JEM7.0.

### Motion compensation

#### Triangular prediction unit mode

PEM introduces new triangular partitions for motion compensated prediction. A CU can be splitted using two triangular prediction units, either in diagonal or inverse diagonal direction. The triangular prediction unit mode is only applied to CUs in skip or merge mode. Each triangular prediction unit in the CU has its own motion vectors and reference frame index. Triangular partitioning is only applied to motion compensated prediction, which means that the transform and quantization process is applied to the whole CU formed by combining together the two triangles. This technique shares some similarities with JVET-H0087 [6].

In PEM, a SPS flag is signalled to indicate whether the triangular prediction unit mode is enabled or not. For a CU coded in a skip or a merge mode, a CU level flag (triangle_merge_flag) is signalled to indicate whether the triangular prediction unit mode is applied or not for the current CU. The flag is CABAC-coded with 3 context models. The context model selection is based on the coding mode of the left and the above neighbouring block as follows:

If both left and above neighbouring block use triangular prediction unit mode,

the third context model is used;

Otherwise, if either left or above neighbouring block use triangular prediction unit mode,

the second context model is used;

Otherwise,

the first context model is used.

When the triangular prediction unit mode is applied to the CU, an index (triangle_idx) indicating the direction of splitting the CU into two triangles and the motion vectors of the two triangles is signalled. The triangle index ranges from 0 to 61 and is binarized using EG-1. The first bin of the triangle index is CABAC-coded with 1 context model, and the rest bins are bypass-coded. A look-up table is used for deriving splitting direction and motion vectors from the index, as shown in Table 4.

Table 4: The look-up table of triangle index

| Triangle index | Splitting direction (from top-left corner to bottom right corner (TL-BR) / from top-right corner to bottom left corner (TR-BL)) | The candidate selected from the motion vector candidate list for PU1 | The candidate selected from the motion vector candidate list for PU2 |
| --- | --- | --- | --- |
| 0 | TL-BR | 1 | 0 |
| 1 | TR-BL | 0 | 1 |
| 2 | TL-BR | 1 | 2 |
| 3 | TR-BL | 1 | 0 |
| 4 | TR-BL | 0 | 2 |
| 5 | TR-BL | 1 | 2 |
| 6 | TL-BR | 2 | 0 |
| … |  |  |  |

When a CU is coded with the triangular prediction unit mode, the CU is firstly split into two triangles, either from top-left corner to bottom-right corner or from top-right corner to bottom-left corner, according to the triangle index.

Figure 10: Examples of splitting a CU into two triangles

Then, two prediction blocks are inter-predicted using two different motion vectors. The motion vectors are derived from a motion vector candidate list which is obtained from the merge candidate list used for the skip/merge mode. The candidates in the motion vector candidate list are copied from the candidates in the merge candidate list, but not including sub-CU based motion vector prediction (i.e. ATMVP mode and STMVP mode). The inter prediction process may include interpolation, LIC or BIO, but excludes DMVR and OBMC.

An adaptive weighting process is applied to the diagonal edge between the two triangular prediction units to derive the final prediction for the whole CU using the two prediction blocks. As illustrated in Figure 11, one out of two weighting factors group is selected based on the comparison of the two motion vectors, which are denoted as MV1 and MV2.

Figure 11: The selection process based on the comparison of two motion vectors

The two prediction blocks are denoted as P1 and P2. As illustrated in Figure 12, when the first weighting factors group is selected, {7/8, 6/8, 4/8, 2/8, 1/8} and {7/8, 4/8, 1/8} are used for the luminance and the chrominance samples in P1, respectively. {1/8, 2/8, 4/8, 6/8, 7/8} and {1/8, 4/8, 7/8} are used for the luminance and the chrominance samples in P2, respectively. As illustrated in Figure 13, when the second weighting factors group is selected, {7/8, 6/8, 5/8, 4/8, 3/8, 2/8, 1/8} and {6/8, 4/8, 2/8} are used for the luminance and the chrominance samples in P1, respectively. {1/8, 2/8, 3/8, 4/8, 5/8, 6/8, 7/8} and {2/8, 4/8, 6/8} are used for the luminance and the chrominance samples in P2, respectively.

Figure 12: An example of a CU applying the first weighting factors group

Figure 13: An example of a CU applying the second weighting factors group

Results for the triangular prediction unit mode can be found in Table 5 for different coding conditions. The same methodology has been used as for the results in section 12.2.10 (which means that only the first second of the test sequences is used).

Table 5: Results of "tool-off" and "tool-on" tests for triangular prediction unit mode

| Condition | Tool-off test |  |  |  |  | Tool-on test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| Constraint set 1 | 0.40% | 0.46% | 0.64% | 93% | 101% | -0.27% | -0.31% | -0.54% | 101% | 101% |
| Constraint set 2 | 0.64% | 1.21% | 0.75% | 95% | 102% | -0.62% | -0.53% | -0.33% | 99% | 100% |
| Low Delay P | 0.99% | 0.88% | 1.13% | 93% | 100% | -0.67% | -1.41% | -1.43% | 101% | 99% |

#### Simplification of overlapped block motion compensation

In the OBMC process of PEM, only prediction blocks based on motion vectors of above and left neighbouring sub-blocks are used to derive a prediction for the current sub-block.

Figure 14 shows an illustration of the sub-blocks where OBMC applies.

Figure 14: Illustration of sub-blocks where OBMC applies

Furthermore, in PEM, OBMC is disabled for FRUC merge, LIC or triangular prediction unit mode.

#### SubCU size restriction for ATMVP/STMVP

In JEM, the size of the sub-CUs in ATMVP/STMVP is always equal to 4x4 even for large CUs. However, in most cases splitting a CU into such a small size is not needed. PEM restricts the size of each sub-CU’s width and height using the following equations:

SubCUwidth=maxCUwidth≫depth, 4 	(16)

SubCUheight=maxCUheight≫depth, 4	(17)

where depth (indicating the depth from CU size) is set to 3 in PEM.

#### Modification of TMVP derivation for STMVP

Figure 15 and Figure 16 show the difference of STMVP derivation between JEM and PEM. The motion vector for sub-CU A is calculated from the average of a (spatial left), b (spatial upper) and c (TMVP). In JEM, TMVP is derived from bottom-right position of collocated Sub-CU. Therefore, there may be significant differences between the motion vectors of the sub-CUs located on the right or the bottom of the whole CU and the motion vectors of the sub-CUs in the rest of the CU. This creates subjective image quality problems. To solve these problems, PEM uses the collocated sub-CU as prediction for TMVP in STMVP, so that all sub-CUs in the CU tend to have similar motion vectors.

Figure 15: STMVP derivation in JEM

Figure 16: STMVP derivation in PEM

#### Constraint on usage of LIC

In PEM, LIC is disabled for normal inter prediction, i.e. LIC can only be used together with skip/merge modes.

### Intra prediction: improved filtering

In both HEVC and JEM, reference samples for angular intra prediction that are located to the left of and above the block are sometimes smoothed with filter [1 2 1]/4. The smoothing filter is applied or not based on the angular intra prediction mode used (MDIS). After this smoothing operation, and depending on the angular prediction mode used, an array of reference samples is prepared that is subsequently used to calculate the prediction signal for the block. For vertical prediction modes, the samples located left of the block are projected in the above row (see Figure 17). For horizontal prediction modes, the samples located above the block are projected in the left column.

Figure 17: Projection of reference samples for a vertical mode

PEM introduces two modifications to this process: the projection process of reference samples is improved to be more accurate, and the smoothing filter is applied after projection rather than before. Specifically, projection of reference samples is performed by using bilinear interpolation to calculate the value of a projected sample using equation (18). Equation (18) is based on equation (8-54) in the HEVC standard to extend the main reference array but includes bilinear interpolation instead of simply copying the closest pixel.

refx=256-x×invAngle&255×p-1-1+x×invAngle≫8+x×invAngle&255×p-1x×invAngle≫8≫8

(18)

The same smoothing filter as in HEVC and JEM is then applied to the projected array of reference samples, thereby smoothing only the samples actually used for prediction (see Figure 18). Furthermore, in JEM, MDIS is applied for a block only when the nsst_idx is non-zero (i.e. NSST is applied). In PEM, MDIS is always applied independently from the value of nsst_idx.

Figure 18: Improved projection and smoothing after projection

In addition, PEM harmonises the calculation of prediction compared to JEM by using 4-tap interpolation filters to calculate the prediction of all the pixels in the block, including the ones that use integer reference samples as references. While it does not make any difference compared to JEM for the cubic 4-tap interpolation filter, it does make a difference when the gaussian 4-tap interpolation filter is used.

### In-loop filtering

#### Asymmetric deblocking filter

Generally, the image quality on the top or left boundaries of a CU is likely to be better than the image quality on the bottom or right boundaries. Therefore, the symmetric deblocking filter of HEVC and JEM tends to increase the SAD of the top and left boundaries, whereas the SAD of the bottom and right boundaries tends to decrease.

Figure 19: SAD change caused by symmetric deblocking filter used in HEVC and JEM

The deblocking filter in HEVC and JEM changes the pixel value as p'=p+∆, where p' and p are filtered and original pixel values respectively, and ∆ is the displacement of the deblocking filter. In PEM, in order to further reduce SAD both in the bottom/right part and in the top/left part of the CU, an asymmetric weight is introduced to the original displacement. A pixel value is filtered as p'=p+w*∆, where w is a weight for the displacement of the deblocking filter. w for the top/left boundary is set to be 0.75. w for the bottom/right boundary is set to be 1.25. This asymmetric weight is applied only to the weak deblocking filter for luma component. Calculation for ∆ is the same as in HEVC and JEM.

#### Bs derivation process for deblocking filter

The derivation process on Bs, which controls the deblocking filter operation, is modified by adding a decision based on LIC usage.

Table 6: Bs decision criteria

| Conditions | Bs |
| --- | --- |
| At least one of the blocks uses Intra prediction | 2 |
| At least one of the blocks has non-zero coded residual coefficient | 1 |
| Absolute difference between corresponding spatial motion vector components of the two blocks are >=1 in units of integer pixels | 1 |
| Motion-compensated prediction for the two blocks refers to different reference pictures or the number of motion vectors is different for the two blocks | 1 |
| At least one of the blocks uses LIC | 1 |
| Otherwise | 0 |

### Additional tools

None.

### Additional algorithmic description topics

#### Tools combinations for inter prediction

PEM uses almost all JEM tools. But some combination rules have been introduced to reduce the complexity especially for worst process path. Table 7 shows inter tools combinations for each inter mode.

Table 7: Tools combinations for Inter CU in PEM

| Tools | Normal Inter |  | Normal Merge |  |  |  | FRUC Merge |  | Affine Inter |  | Affine Merge |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CU |  | SubCU |  |  |  |  |  |  |
| FRUC(CU) | Y |  |  |  |  |  | Y |  |  |  |  |
| FRUC(SubCU) |  |  |  |  |  |  | N |  |  |  |  |
| AMVR | Y |  |  |  |  |  |  |  |  |  |  |
| LIC | N |  | 0 | 1 | 0 | 1 | 0 | 1 |  |  |  |
| DMVR |  |  | Y |  |  |  |  |  |  |  |  |
| BIO | Y |  | Y |  | N |  | Y |  |  |  |  |
| OBMC | 0 | 1 | Y | N | Y | N | N | N | 0 | 1 | Y |
| EMT | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y |

Gray cells highlight the differences to JEM. PEM has 6 additional combination rules summarized below:

- Remove SubCU level FRUC process.

- Disable LIC for Normal Inter.

- Disable BIO for SubCU Normal Merge (ATMVP/STMVP).

- Disable OBMC when LIC is used.

- Disable OBMC for FRUC Merge.

- Enable EMT for all inter mode.

1) and 5) shorten the worst process path, 3) and 4) reduce overlapping of tools functionalities, 2) removes a combination of incompatible tools, 6) simplifies the algorithm.

#### Syntax simplification for skip mode

Both JEM and PEM have a lot of inter prediction tools compared to HEVC. Therefore, a lot of syntax elements are necessary even for Skip mode (Merge mode without residual). In order to simplify the syntax and increase the coding gain for content with easy scenes, PEM prohibits coding Affine Merge and FRUC Merge using LIC as a Skip mode even if it doesn’t have residual. Skip mode of JEM needs to signal fruc_flag, fruc_type, affine_flag, merge_idx, and lic_flag. affine_flag and lic_flag are removed from the syntax of the skip mode in PEM. A more detailed version of the syntax in PEM is available in section 13 Annex 2.

#### Bug fix for P-PDPC

A subjective quality issue has been noticed in PEM, where vertical or horizontal lines appear in the middle of blocks encoded with the intra P-PDPC  mode (see Figure 20).

Figure 20: Subjective artefacts in the form of horizontal lines in Intra P-PDPC predicted blocks

The equation to calculate the P-PDPC prediction for a pixel is (copied from JVET-G1001 [7]):

px,y=c1(v)≫y/dyrx,-1-c2(v)≫y/dyr-1,-1+ c1(h)≫x/dxr-1,y-c2(h)≫x/dxr-1,-1+bx,yqx,y+64≫7

(19)

This is implemented using bit shifts in JEM. Unfortunately, when the height or width of a block gets bigger than 64, the value used for the shift gets bigger than the number of bits in the variable. On our platform, the effect is to give a very high value to the coefficients applied to the unfiltered reference samples in the equation above, instead of 0. It creates a line artefact that closely follows the pattern of the upper row of reference samples. Another minor problem in the implementation of P-PDPC is that bit shifts are used for division of negative numbers, which gives wrong results. These two problems in the code of P-PDPC were fixed in PEM.

#### Code optimization of BIO

The BIO process in PEM is optimized in terms of software implementation to reduce complexity without changing the algorithm itself. This optimization mainly consists of skipping calculation of BIO intermediate parameter when they are not used and optimization of reading arrays, which are summarized in Table 8.

Table 8: PEM code optimization of BIO

| Optimized items | JEM 7.0 | PEM |
| --- | --- | --- |
| skipping calculation of BIO intermediate parameters when they are not used | Perform calculation for every BIO intermediate parameter | Judging if some of the intermediate parameters will be used in later calculation, if not, skip calculation for those parameters. |
| optimization of reading arrays | Locations of array element needs to be specifically calculated during matrix multiplications | Padding matrix boundary and perform straightforward matrix multiplication without special calculation of element location |

#### Code optimization of ALF

The ALF process in PEM is optimized in terms of software implementation to reduce complexity without changing the algorithm itself. This optimization mainly consists of separating luma and chroma process and changing filter selection from a pixel basis to 2x2 pixels basis, which are summarized in Table 9.

Table 9: PEM code optimization of ALF

| Optimized items | JEM 7.0 | PEM |
| --- | --- | --- |
| Switching between luma and chroma process | Luma and chroma process are in the same function. Decision if current process is luma or chroma happens every pixel. | Luma and chroma process are separated. No decision is needed. |
| Writing of filter index | Pixel basis operation resulting in writing the same index for 2x2 pixels. | 2x2 pixels basis operation. |
| Reading of filter index | Pixel basis operation resulting in reading the same index for 2x2 pixels. | 2x2 pixels basis operation. |

## 360° Video: N/A

There is nothing specific to 360° Video in this proposal.

## HDR: N/A

There is nothing specific to HDR in this proposal.

# Encoder algorithm description

## General algorithm description

### Rate-distortion optimization

#### Distortion metrics

Same as JEM7.0.

#### Rate-distortion dependency models

Same as JEM7.0.

### Partitioning structure selection: fast algorithms

Figure 21 shows the basic process flow of partitioning selection for a coding unit at the encoder, where BT, TT and QT are candidate partition structures for the CU. Horizontal TT partitioning (TT Horizontal) and vertical TT partitioning (TT Vertical) selections are applied after BT horizontal and BT vertical selections, but before QT partitioning selection. To reduce encoding complexity, fast partitioning schemes are applied, in which BT, TT or QT partitioning selections from Figure 21 may be skipped. The subsections 3.1.2.1 to 3.1.2.4 details the fast partitioning selection schemes used in PEM.

Figure 21: Basic process flow of partitioning selection

#### Disabling BT, TT and QT partitioning selection based on block shape or information from current and lower depths

BT, TT or QT partitioning selections may be skipped according to partition structure constraints, or according to the total number of skip modes from current and parent CUs. The decision point for skipping these processes is located after “no split” selection, but before BT partitioning selection as shown in Figure 22, which is a simplification of Figure 21.

Figure 22: Decision point of fast partitioning before BT partitioning selection

When a block uses TT partitioning, constraints are introduced for the middle block. Indeed, if this middle block was further partitioned using BT partitioning in the same direction, the block partitioning structure obtained would be the same as applying BT partitioning twice, as shown in Figure 23 for vertical partitions. To avoid applying rate distortion search twice for the same partitioning structure, BT partitioning selection is skipped for the middle block of a TT partition, when the splitting direction of the BT partition is the same as the splitting direction of the TT partition. However, this constraint is introduced only when the best mode for this middle block has been selected as being the skip mode during the “no-split” selection in Figure 22.

Figure 23: Fast partitioning selection according to partition structure constraints

BT and TT partitioning selections can be skipped using knowledge about the best selected modes for partition structures of current and lower depths. Specifically, when the skip mode has been selected as the best mode for the current CU at current depth (depth equal to 2 for example) during the “no split” selection from Figure 22, and the skip mode has been selected as the best mode during the “no split” selections of the two parent CUs at lower depths (depth equal to 0 and 1 for example), then BT, TT and QT partitioning selections are skipped for the current CU (see Figure 24 where CU3 is the current CU, and CU1 and CU2 are the parents CUs). This technique is an extension of a similar technique used in JVET-H0084 [8] (HHI’s NextSoftware).

Figure 24: Fast BT, TT, QT partitioning selection according to the total number of skip modes from current CU (CU3) and its parent CUs(CU1 and CU2)

#### Disabling BT and TT partitioning selection when QT selection is applied before BT selection

In some cases, the order of partitioning selections shown in Figure 22 can be modified as in Figure 25 by performing QT partitioning selection before BT and TT partitioning selection. This alternative order of partitioning selection is applied when the CUs located above and left of the current CU use only QT partitioning (see Figure 26 (a) for an example). In such cases, information from QT partitioning selection is used to determine if BT and TT partitioning selection is applied, as shown in Figure 25.

Figure 25: Decision point of fast partitioning before BT partitioning selection when QT selection is applied before BT selection

Specifically, BT and TT partitioning selections are skipped when the best partition structure of current CU contains only QT partitioning and the partitioning depth is greater than 1 (see Figure 26 (b) for an example).  This technique is an extension of a similar technique used in JVET-H0084 [8] (HHI’s NextSoftware).

Figure 26: Examples, (a) both above and left CUs use QT partitioning, (b) partitioning depth is > 1, (c) partitioning depth is 0 or 1

#### Disabling TT partitioning selection using information from BT selection or from original content or from block size

TT partitioning selection may be skipped according to information from BT partitioning selection, or according to gradients in the original image, or according to block sizes. The decision point for skipping TT partitioning selection is located after BT partitioning selection as shown in Figure 27, which is a simplification of Figure 21.

Figure 27: Decision point of fast partitioning before TT partitioning selection

TT partitioning selection is skipped after BT partitioning selection when the partitioning depth of the best partition structure equals to 0 or 1, as shown in Figure 28.

Figure 28: Fast TT partitioning according to the best partition structure after BT selection

TT partitioning selection in either the horizontal or the vertical direction may be skipped according to block size and gradient information from the original content. Block gradients are calculated in horizontal, vertical and diagonal directions using sum of pixel differences from the original content for the current CU. Figure 29 illustrates the conditions used for skipping TT partitioning selection for non-square blocks. If the block’s height is greater than its width and the vertical gradient of the block is greater than the horizontal gradient, then vertical TT partitioning selection is skipped. Similarly, if the block’s width is greater than its height and the horizontal gradient of the block is greater than the vertical gradient, then horizontal TT partitioning selection is skipped. There are similar rules for square blocks, where the horizontal gradient is compared to the vertical and diagonal gradients to decide if horizontal TT partitioning selection is skipped, or where the vertical gradient is compared to the horizontal and diagonal gradients to decide if vertical TT partitioning selection is skipped. This technique is an extension of a similar technique used in JVET-H0084 [8] (HHI’s NextSoftware).

Figure 29: Fast TT partitioning according to block gradient

TT partitioning selection in one direction is also skipped for some block sizes. When the width of a block is greater than twice its height, horizontal TT partition selection is skipped (see example in Figure 30). Similarly, when the height of a block is greater than twice its width, vertical TT partitioning selection is skipped. Please note that this particular condition is also reflected in the syntax, as described in section 2.1.1 and in the syntax of section 13 Annex 2.

Figure 30: Fast TT partitioning according to block size

#### Disabling QT partitioning selection based on information from BT and TT partitioning selection

QT partitioning selection may be skipped according to information from BT and TT partitioning selection. The decision for skipping QT selection is located after BT and TT partitioning selection, as shown in Figure 31, which is a simplification of Figure 21. QT partitioning selection is skipped, when the partitioning depth of the best partition structure after BT and TT partitioning selection is equal to 1, as shown in Figure 32.

Figure 31: Decision point of fast partitioning before QT

Figure 32: Examples of fast QT partitioning scheme using BT and TT partitioning information

### Intra prediction mode estimation and type selection

Same as JEM7.0.

### Motion estimation and motion segment selection

#### General: Fast algorithm for triangular prediction unit mode

In PEM, a prediction block denoted as PN is obtained for the triangular prediction unit mode based on the Nth motion vector in the motion vector candidate list. Since the motion vector candidate list is derived from a merge candidate list for a skip/merge mode, the prediction blocks obtained for the skip/merge mode may be reused for PN. The prediction block obtained for the skip/merge mode is denoted as QM, with M indicating the Mth candidate in the merge candidate list. When QM is based on a motion vector that is uni-prediction or DMVR is not applied to QM, QM is reused for PN. PN and QM here refer to the same prediction block and only the index is different. Otherwise, PN is inter predicted with the corresponding motion vector.

A number X, which indicates the number of triangle indices being tested, is determined for a CU based on its size and, if available, the coding mode of a previously tested CU with the same size and position which was obtained using a different splitting method. An example of such a CU is illustrated in Figure 33. The triangle indices tested at the encoder are the first X from Table 4.

Figure 33: An example of the same CU using different splitting method

The number X is determined as follows:

If the coding mode of a previously tested CU with the same size and position is available and is not triangular prediction unit mode,

the number X is set to 7;

Otherwise, if the coding mode of a previously tested CU with the same size and position is available and is a skip mode,

the number X is set to 7;

Otherwise, if CU width CU height=32 or CU heightCU width=32,

the number X is set to 7;

Otherwise, if CU width CU height=16 or CU heightCU width=16,

the number X is set to 14;

Otherwise, if CU width CU height=8 or CU heightCU width=8,

the number X is set to 40;

Otherwise,

the number X is set to 62.

#### Usage of decoder-side motion refinement: AMVP candidate list simplification

Figure 34 shows the encoding process with AMVR for Normal Inter mode. In JEM, the intermediate list of AMVP mode in the figure will change depending on AMVR precision, so candidate list generation including FRUC is needed for each evaluation of AMVR precision. On the other hand, in PEM, the intermediate list of AMVP mode in the figure is always the same for all AMVR precision, and it can be reused in the AMVR loop.

Figure 34: flowchart of Normal Inter encoding process with AMVR in JEM (left) and PEM (right)

### Mode decisions

Same as JEM7.0.

### In-loop filtering type selection and parameter estimation

Same as JEM7.0.

### Transforms and transform type selection

#### Usage restriction on NSST

Encoder search to select the best NSST kernel is skipped if the primary transform type is not DCT2 (see section 2.1.6).

#### Usage restriction on EMT

For both intra and inter block, PEM uses two-pass fast search algorithm for the EMT kernel selection. The algorithm itself is the same as the one used in JEM. However, JEM only uses fast search for intra blocks when the coding condition is All Intra. PEM uses fast search algorithm for all intra coded blocks, such as intra blocks in I slices in Random Access condition, or intra blocks in B or P slices.

### Quantization and quantization type selection

Same as JEM7.0.

### Entropy coding

Same as JEM7.0.

### Additional tools

None.

### Additional encoder optimization

None.

### Additional algorithmic description topic(s): Tools combinations for inter prediction

PEM disables LIC in Normal Inter, so that the mode decision loop for LIC can be removed from Normal Inter encoding process as shown in Figure 35.

Figure 35: flowchart of Normal Inter encoding process with LIC in JEM (left) and PEM (right)

## 360°: N/A

There is nothing specific to 360° Video in this proposal.

## HDR: N/A

There is nothing specific to HDR in this proposal.

# Compression performance

## SDR: Constraint set 1 configuration relative to HM anchor

### Class SDR-A (RD-curves)

Figure 36: RD curves comparing the performances of PEM and HM16.16 for sequences of class SDR-A coded with constraint set 1 configuration

### Class SDR-B (RD-curves)

Figure 37: RD curves comparing the performances of PEM and HM16.16 for sequences of class SDR-B coded with constraint set 1 configuration

### Overall (BD-rate summary tables)

Table 10: Compression performance results for constraint set 1 configuration relative to HM anchor

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16(GCC6.2.0, 1 core for decode, dec YUV output) |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -34.66% | -44.83% | -48.56% | 1.74 | 1.31 | 1.55 |
| CatRobot1 | -40.71% | -54.01% | -47.38% | 1.60 | 0.99 | 1.35 |
| DaylightRoad2 | -42.04% | -58.59% | -42.85% | 1.18 | 1.27 | 0.80 |
| ParkRunning3 | -33.13% | -27.22% | -30.25% | 1.07 | 0.47 | 0.43 |
| Campfire | -35.82% | -36.19% | -51.69% | 1.05 | 1.37 | 1.61 |
| BQTerrace | -31.69% | -47.18% | -56.49% | 0.95 | 0.87 | 1.08 |
| RitualDance | -29.47% | -39.14% | -42.97% | 1.59 | 1.13 | 1.42 |
| MarketPlace | -30.48% | -48.71% | -46.93% | 1.08 | 0.87 | 0.87 |
| BasketballDrive | -32.99% | -47.77% | -45.42% | 1.19 | 1.07 | 1.39 |
| Cactus | -35.29% | -46.91% | -42.70% | 1.45 | 0.76 | 1.12 |
| Average SDR-A | -37.27% | -44.17% | -44.15% | 1.33 | 1.08 | 1.15 |
| Average SDR-B | -31.99% | -45.94% | -46.90% | 1.25 | 0.94 | 1.18 |
| Average all | -34.63% | -45.06% | -45.52% | 1.29 | 1.01 | 1.16 |
| Enc Time[%] | 817% |  |  |  |  |  |
| Dec Time[%] | 384% |  |  |  |  |  |

## SDR: Constraint set 2 configuration relative to HM anchor

### Class SDR-B

Figure 38: RD curves comparing the performances of PEM and HM16.16 for sequences of class SDR-B coded with constraint set 2 configuration

### Overall

Table 11: Compression performance results for constraint set 2 configuration relative to HM anchor

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16(GCC6.2.0, 1 core for decode, dec YUV output) |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -27.39% | -47.34% | -54.06% | 0.82 | 0.56 | 0.65 |
| RitualDance | -23.69% | -35.04% | -36.57% | 1.23 | 0.85 | 1.02 |
| MarketPlace | -22.62% | -42.91% | -41.64% | 0.73 | 0.56 | 0.60 |
| BasketballDrive | -27.41% | -47.70% | -45.64% | 1.04 | 1.04 | 1.36 |
| Cactus | -28.98% | -42.46% | -38.80% | 1.16 | 0.61 | 0.86 |
| Average SDR-B | -26.02% | -43.09% | -43.34% | 0.99 | 0.72 | 0.90 |
| Enc Time[%] | 830% |  |  |  |  |  |
| Dec Time[%] | 346% |  |  |  |  |  |

## SDR: Constraint set 1 configuration relative to JEM anchor

### Class SDR-A

Figure 39: RD curves comparing the performances of PEM and JEM7.0  for sequences of class SDR-A coded with constraint set 1 configuration

### Class SDR-B

Figure 40: RD curves comparing the performances of PEM and JEM7.0  for sequences of class SDR-B coded with constraint set 1 configuration

### Overall

Table 12: Compression performance results for constraint set 1 configuration relative to JEM anchor

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0(GCC6.2.0, 1 core for decode, dec YUV output) |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -2.08% | -2.18% | -3.32% | 0.08 | 0.04 | 0.07 |
| CatRobot1 | -2.40% | -4.43% | -3.49% | 0.06 | 0.05 | 0.07 |
| DaylightRoad2 | -2.93% | -4.16% | -3.19% | 0.05 | 0.05 | 0.04 |
| ParkRunning3 | -2.97% | -2.86% | -4.05% | 0.08 | 0.04 | 0.05 |
| Campfire | -2.34% | -2.38% | -4.39% | 0.05 | 0.08 | 0.09 |
| BQTerrace | -1.94% | -3.98% | -5.12% | 0.05 | 0.05 | 0.05 |
| RitualDance | -3.01% | -3.76% | -4.25% | 0.14 | 0.08 | 0.10 |
| MarketPlace | -1.94% | -2.93% | -4.39% | 0.06 | 0.04 | 0.06 |
| BasketballDrive | -2.72% | -4.42% | -3.64% | 0.08 | 0.06 | 0.08 |
| Cactus | -0.44% | -3.27% | -2.95% | 0.01 | 0.04 | 0.05 |
| Average SDR-A | -2.54% | -3.20% | -3.69% | 0.07 | 0.05 | 0.06 |
| Average SDR-B | -2.01% | -3.67% | -4.07% | 0.07 | 0.05 | 0.07 |
| Average all | -2.28% | -3.44% | -3.88% | 0.07 | 0.05 | 0.07 |
| Enc Time[%] | 107% |  |  |  |  |  |
| Dec Time[%] | 56% |  |  |  |  |  |

## SDR: Constraint set 2 configuration relative to JEM anchor

### Class SDR-B

Figure 41: RD curves comparing the performances of PEM and JEM7.0  for sequences of class SDR-B coded with constraint set 2 configuration

### Overall

Table 13: Compression performance results for constraint set 2 configuration relative to JEM anchor

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0(GCC6.2.0, 1 core for decode, dec YUV output) |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -1.63% | -0.19% | -6.94% | 0.04 | 0.00 | 0.04 |
| RitualDance | -2.57% | -1.12% | -1.77% | 0.12 | 0.02 | 0.04 |
| MarketPlace | -2.47% | -0.23% | -1.17% | 0.07 | 0.00 | 0.02 |
| BasketballDrive | -2.45% | -3.57% | -3.17% | 0.08 | 0.05 | 0.07 |
| Cactus | -1.00% | 0.02% | 1.23% | 0.03 | 0.00 | -0.02 |
| Average SDR-B | -2.02% | -1.02% | -2.36% | 0.07 | 0.02 | 0.03 |
| Enc Time[%] | 100% |  |  |  |  |  |
| Dec Time[%] | 61% |  |  |  |  |  |

# Decoder complexity analysis

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

Please refer to Table 10 and Table 11 for the decoding time comparison to HM anchors: PEM requires 384% of the decoding time of HM16.16 for constraint set 1 and 346% of the decoding time of HM16.16 for constraint set 2. In addition, Table 12 and Table 13 also give information about the decoding time of PEM compared to JEM anchors: PEM requires 56% of the decoding time of JEM7.0 for constraint set 1 and 61% of the decoding time of JEM7.0 for constraint set 2. All the decoding time given in Table 10, Table 11, Table 12, and Table 13 are calculated with YUV output enabled.

As additional information, the decoding time ratios with YUV output disabled are: 444% of HM16.16 decoding time for constraint set 1 and 385% of HM16.16 decoding time for constraint set 2, which corresponds to 57% of JEM7.0 decoding time for constraint set 1 and 60% of JEM7.0 decoding time for constraint set 2.

## Description of computing platform used to determine decoding times reported in section 5.1

Decoding times are measured using the following platform:

CPU: Intel Core i7-6800K, 3.4GHz, 6Core

RAM: 64GB

HDD: SATA 1TB

OS: Ubuntu Linux 16.04.3 LTS (64bit OS)

Compiler: GCC6.2.0 (64bit executable)

Parallel decoding: No (decode a bitstream after concatenation)

Parallel execution: No (use 1 core only per 1 PC)

## Memory usage of decoder

The test conditions used are the same as in section 5.2. The value reported in Table 14 is VSZ (Virtual Set Size) measured by “ps” command on linux platform.

Table 14: Decoder maximum memory usage of PEM and JEM7.0

|  | PEM | JEM7.0 |
| --- | --- | --- |
| Constraint set 1 (Class SDR-A/R4) | 1.33GB | 1.32GB |
| Constraint set 2 (Class SDR-B/R4) | 0.37GB | 0.37GB |

## Complexity characteristics of decoder entropy decoding operation: multi-hypothesis CABAC window size for each context

In JEM, the multi-hypothesis probability update model applied to binary arithmetic coder is shown as follows:

(20)

where the variable Mi is equal to 4, 5, 6 or 7 and the variable Mj is equal to 8. 10×3 (for I, P and B slices) bits are required for each context model to store the probability (8 bits) and the variable Mi (2 bits).

In PEM, the probability and the variables Mi and Mj are the same for I, P and B slices. The variables Mi and Mj range from 2 to 9. Therefore, 14 bits are required for each context model to store the probability (8 bits), the variable Mi (3 bits) and the variable Mj (3 bits).

## Complexity characteristics of decoder inverse quantization

Same as JEM7.0.

## Complexity characteristics of decoder inverse transform operation

Same as JEM7.0.

## Complexity characteristics of decoder-side motion refinement

### Simplification of FRUC motion vector refinement process

In a typical case, the number of search points is reduced by 74% for class SDR-A and SDR-B in constraint set 1 and 87% for class SDR-B in constraint set 2 in the proposed technique, as shown in Table 15.

Table 15: Average number of search point comparison between JEM7.0 and the proposed technique

| Category | JEM7.0 | Proposed technique | Ratio |
| --- | --- | --- | --- |
| Class SDR-A (CS1) | 11707812 | 8716299 | 74% |
| Class SDR-B (CS1) | 3336564 | 2462375 | 74% |
| Class SDR-B (CS2) | 2753850 | 2392418 | 87% |

Average reduction of decoding time is shown in Table 16 with JEM7.0 as anchor. An average decoding time reduction of 2.9% for class SDR-A in constraint set 1, 2.5% for class SDR-B in constraint set 1 and 1.8% for class SDR-B in constraint set 2 is achieved without significant impact on coding performance.

Table 16: Decoding time comparison between JEM7.0 and the proposed technique

| 1sec results | JEM7.0+Proposed technique |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Runtime |  | BD-rate |  |  |
|  | Enc | Dec | Y | Cb | Cr |
| Class SDR-A (CS1) | 96% | 97% | 0.06% | 0.23% | -0.03% |
| Class SDR-B (CS1) | 98% | 98% | 0.04% | 0.25% | 0.18% |
| Class SDR-B (CS2) | 97% | 98% | -0.07% | 0.06% | 0.29% |

### Simplification of bi-directional optical flow

In JEM, a group of 8-tap interpolation filter is used for predicting luminance samples. A group of 6-tap gradient filters and a group of 6-tap interpolation filters are used for calculating gradients of BIO. Figure 42 and Figure 43 show an example of performing BIO for a 4×4 block in JEM.

Figure 42: An example of predicting 4×4 block in JEM

Figure 43: An example of calculating 4×4 gradients in JEM

In PEM, an 8-tap interpolation filter is used for predicting luminance samples and a 5-tap gradient filter is used for calculating gradients of BIO. Figure 44 and Figure 45 show an example of performing BIO for a 4×4 block in PEM.

Figure 44: An example of predicting 4×4 block in PEM

Figure 45: An example of calculating 4×4 gradients in PEM

The number of multiplication and addition operations required for a N×M CU coded with the above simplification of BIO are shown in Table 17 and Table 18, respectively.

Table 17: Number of multiplications required for a NxM CU coded with BIO in JEM and JEM modified as explained above

| Given a N×M bi-predicted CU |  | JEM | With simplification |
| --- | --- | --- | --- |
| N×M prediction block | L0 prediction | 8×(N×(M+7))+8×(N×M) | 8×((N+4)×(M+11))+8×((N+4)×(M+4)) |
|  | L1 prediction | 8×(N×(M+7))+8×(N×M) | 8×((N+4)×(M+11))+8×((N+4)×(M+4)) |
|  | averaging | 2×(N×M) | 2×(N×M) |
| N×M horizontal gradients | L0 | 6×((N+5)×M)+6×(N×M) | 5×(N×M) |
|  | L1 | 6×((N+5)×M)+6×(N×M) | 5×(N×M) |
| N×M vertical gradients | L0 | 6×((N+5)×M)+6×(N×M) | 5×(N×M) |
|  | L1 | 6×((N+5)×M)+6×(N×M) | 5×(N×M) |
| Total |  | 82×N×M+112×N+120×M | 54×N×M+240×N+128×M+960 |

Table 18: Number of additions required for a NxM CU coded with BIO in JEM and JEM modified as explained above

| Given a N×M bi-predicted CU |  | JEM | With simplification |
| --- | --- | --- | --- |
| N×M prediction block | L0 prediction | 7×(N×(M+7))+7×(N×M) | 7×((N+4)×(M+11))+7×((N+4)×(M+4)) |
|  | L1 prediction | 7×(N×(M+7))+7×(N×M) | 7×((N+4)×(M+11))+7×((N+4)×(M+4)) |
|  | averaging | 1×(N×M) | 1×(N×M) |
| N×M horizontal gradients | L0 | 5×((N+5)×M)+5×(N×M) | 4×(N×M) |
|  | L1 | 5×((N+5)×M)+5×(N×M) | 4×(N×M) |
| N×M vertical gradients | L0 | 5×((N+5)×M)+5×(N×M) | 4×(N×M) |
|  | L1 | 5×((N+5)×M)+5×(N×M) | 4×(N×M) |
| Total |  | 69×N×M+98×N+100×M | 45×N×M+210×N+112×M+840 |

The number of multiplication and addition operations required for each block size are shown in Table 19.

Table 19: Number of operations required for a CU coded with BIO in JEM and JEM modified as explained above

| CU size |  | Multiplication |  |  | Addition |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| width | height | JEM | With simplification | Difference | JEM | With simplification | Difference |
| 4 | 4 | 2240 | 3296 | 1056 | 1896 | 2848 | 952 |
|  | 8 | 4032 | 4672 | 640 | 3400 | 4016 | 616 |
|  | 16 | 7616 | 7424 | -192 | 6408 | 6352 | -56 |
|  | 32 | 14784 | 12928 | -1856 | 12424 | 11024 | -1400 |
|  | 64 | 29120 | 23936 | -5184 | 24456 | 20368 | -4088 |
|  | 128 | 57792 | 45952 | -11840 | 48520 | 39506 | -9464 |
| 8 | 4 | 4000 | 5120 | 1120 | 3392 | 4408 | 1016 |
|  | 8 | 7104 | 7360 | 256 | 6000 | 6296 | 296 |
|  | 16 | 13312 | 11840 | -1472 | 11216 | 10072 | -1144 |
|  | 32 | 25728 | 20800 | -4928 | 21648 | 17624 | -4024 |
|  | 64 | 50560 | 38720 | -11840 | 42512 | 32728 | -9784 |
|  | 128 | 100224 | 74560 | -25664 | 84240 | 62936 | -21304 |
| 16 | 4 | 7520 | 8768 | 1248 | 6384 | 7528 | 1144 |
|  | 8 | 13248 | 12736 | -512 | 11200 | 10856 | -344 |
|  | 16 | 24704 | 20672 | -4032 | 20832 | 17512 | -3320 |
|  | 32 | 47616 | 36544 | -11072 | 40096 | 30824 | -9272 |
|  | 64 | 93440 | 68288 | -25152 | 78624 | 57448 | -21176 |
|  | 128 | 185088 | 131776 | -53312 | 155680 | 110696 | -44984 |
| 32 | 4 | 14560 | 16064 | 1504 | 12368 | 13768 | 1400 |
|  | 8 | 25536 | 23488 | -2048 | 21600 | 19976 | -1624 |
|  | 16 | 47488 | 38336 | -9152 | 40064 | 32392 | -7672 |
|  | 32 | 91392 | 68032 | -23360 | 76992 | 57224 | -19768 |
|  | 64 | 179200 | 127424 | -51776 | 150848 | 106888 | -43960 |
|  | 128 | 354816 | 246208 | -108608 | 298560 | 206216 | -92344 |
| 64 | 4 | 28640 | 30656 | 2016 | 24336 | 26248 | 1912 |
|  | 8 | 50112 | 44992 | -5120 | 42400 | 38216 | -4184 |
|  | 16 | 93056 | 73664 | -19392 | 78528 | 62152 | -16376 |
|  | 32 | 178944 | 131008 | -47936 | 150784 | 110024 | -40760 |
|  | 64 | 350720 | 245696 | -105024 | 295296 | 205768 | -89528 |
|  | 128 | 694272 | 475072 | -219200 | 584320 | 397256 | -187064 |
| 128 | 4 | 56800 | 59840 | 3040 | 48272 | 51208 | 2936 |
|  | 8 | 99264 | 88000 | -11264 | 84000 | 74696 | -9304 |
|  | 16 | 184192 | 144320 | -39872 | 155456 | 121672 | -33784 |
|  | 32 | 354048 | 256960 | -97088 | 298368 | 215624 | -82744 |
|  | 64 | 693760 | 482240 | -211520 | 584192 | 403528 | -180664 |
|  | 128 | 1373184 | 932800 | -440384 | 1155840 | 779336 | -376504 |

Additionally, with the improvement of the BIO equation described in 2.1.4.7, the number of multiplications increases by 7 for a 4x4 sized block. The impact of this change on complexity is negligible.

## Complexity characteristics of decoder motion compensation

### Complexity characteristics of triangular prediction unit mode

In JEM, the memory access bandwidth for a CU coded with skip/merge mode includes memory access for motion compensation interpolation process, DMVR, BIO and OBMC.

In PEM, two prediction blocks, which are predicted excluding DMVR and OBMC processes, are needed for a CU coded with triangular prediction unit mode. As shown in Figure 46, since DMVR and OBMC are not performed for triangular prediction unit mode, memory access bandwidth for a CU coded with triangular prediction unit mode is less than two times of that for a CU coded with skip/merge mode.

Figure 46: An illustration of prediction process for merge mode and triangular prediction unit mode

Besides, 2 multiplications and 1 addition are required for each sample where the adaptive weighting process is applied. Table 20 shows the number of operations for each block size.

Table 20: Number of operations for the adaptive weighting process

| CU size |  | First weighting factors group |  |  |  | Second weighting factors group |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | number of weighting samples |  | number of operation |  | number of weighting samples |  | number of operation |  |
| width | height | luma | chroma | multiplication | addition | luma | chroma | multiplication | addition |
| 4 | 4 | 14 | 4 | 44 | 22 | 16 | 4 | 48 | 24 |
|  | 8 | 28 | 8 | 88 | 44 | 32 | 8 | 96 | 48 |
|  | 16 | 56 | 16 | 176 | 88 | 64 | 16 | 192 | 96 |
|  | 32 | 112 | 32 | 352 | 176 | 128 | 32 | 384 | 192 |
|  | 64 | 224 | 64 | 704 | 352 | 256 | 64 | 768 | 384 |
|  | 128 | 448 | 160 | 1536 | 768 | 512 | 160 | 1664 | 832 |
| 8 | 4 | 28 | 7 | 84 | 42 | 32 | 8 | 96 | 48 |
|  | 8 | 34 | 10 | 108 | 54 | 44 | 10 | 128 | 64 |
|  | 16 | 68 | 20 | 216 | 108 | 88 | 20 | 256 | 128 |
|  | 32 | 136 | 40 | 432 | 216 | 176 | 40 | 512 | 256 |
|  | 64 | 272 | 80 | 864 | 432 | 352 | 80 | 1024 | 512 |
|  | 128 | 544 | 160 | 1728 | 864 | 704 | 160 | 2048 | 1024 |
| 16 | 4 | 56 | 14 | 168 | 84 | 64 | 16 | 192 | 96 |
|  | 8 | 68 | 17 | 204 | 102 | 88 | 22 | 264 | 132 |
|  | 16 | 74 | 22 | 236 | 118 | 100 | 22 | 288 | 144 |
|  | 32 | 148 | 44 | 472 | 236 | 200 | 44 | 576 | 288 |
|  | 64 | 296 | 88 | 944 | 472 | 400 | 88 | 1152 | 576 |
|  | 128 | 592 | 176 | 1888 | 944 | 800 | 176 | 2304 | 1152 |
| 32 | 4 | 112 | 28 | 336 | 168 | 128 | 32 | 384 | 192 |
|  | 8 | 136 | 34 | 408 | 204 | 176 | 44 | 528 | 264 |
|  | 16 | 148 | 37 | 444 | 222 | 200 | 50 | 600 | 300 |
|  | 32 | 154 | 46 | 492 | 246 | 212 | 46 | 608 | 304 |
|  | 64 | 308 | 92 | 984 | 492 | 424 | 92 | 1216 | 608 |
|  | 128 | 616 | 184 | 1968 | 984 | 848 | 184 | 2432 | 1216 |
| 64 | 4 | 224 | 152 | 1056 | 528 | 256 | 208 | 1344 | 672 |
|  | 8 | 272 | 68 | 816 | 408 | 352 | 88 | 1056 | 528 |
|  | 16 | 296 | 74 | 888 | 444 | 400 | 100 | 1200 | 600 |
|  | 32 | 308 | 77 | 924 | 462 | 424 | 106 | 1272 | 636 |
|  | 64 | 314 | 94 | 1004 | 502 | 436 | 94 | 1248 | 624 |
|  | 128 | 628 | 188 | 2008 | 1004 | 872 | 188 | 2496 | 1248 |
| 128 | 4 | 448 | 144 | 1472 | 736 | 512 | 192 | 1792 | 896 |
|  | 8 | 544 | 136 | 1632 | 816 | 704 | 176 | 2112 | 1056 |
|  | 16 | 592 | 148 | 1776 | 888 | 800 | 200 | 2400 | 1200 |
|  | 32 | 616 | 154 | 1848 | 924 | 848 | 212 | 2544 | 1272 |
|  | 64 | 628 | 157 | 1884 | 942 | 872 | 218 | 2616 | 1308 |
|  | 128 | 634 | 190 | 2028 | 1014 | 884 | 190 | 2528 | 1264 |

### Simplification of overlapped block motion compensation

For JEM in the case of a 64×64 CTU size and ATMVP mode, the total number of motion compensation process calls in OBMC process for 4×4 sub-block is 60 (CU boundary) × 2 (bi-prediction) × 2 (neighbour sub-blocks) and 14×14 (inside sub-blocks) × 2 (bi-prediction) × 4 (neighbour sub-blocks).

For PEM, the number of neighbour sub-blocks for SubCU OBMC blending is reduced to 2 instead of 4, which divides the previous multiplication by 2.

In the worst-case scenario, this tool reduces by 43% the total number of 4×4 motion compensation process calls required in OBMC process.

As shown in Table 21, decoding time reduction of 4% (CS1) at the cost of 0.12% loss and 7% (CS2) at the cost of 0.30% loss are achieved.

Table 21: Tool on/off results for simplified OBMC

| 1 second results | "tool-off" test results |  |  |  |  | "tool-on" test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Runtime |  | BD-rate |  |  | Runtime |  | BD-rate |  |  |
|  | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr |
| Constraint set 1 | 100% | 104% | -0.12% | 0.18% | 0.07% | 97% | 94% | 0.27% | 0.25% | 0.10% |
| Constraint set 2 | 101% | 107% | -0.30% | -0.34% | -0.22% | 97% | 93% | 0.32% | 0.53% | 0.50% |

## Complexity characteristics of decoder intra-frame prediction operation

The improved projection of reference samples in PEM adds 2 multiplications and 1 addition to the calculation of some intra prediction modes for one whole block. This projection only happens for half of the prediction directions.

Smoothing of the reference samples after projection does not add any additional complexity, since the smoothing filter used is the same as the one used in HEVC and JEM. It does however remove the need to switch between smoothing of reference row and column, since the samples to smooth are all on one line.

## Complexity characteristics of decoder in-loop filtering operation

Same as JEM7.0.

## Complexity characteristics of additional decoder tools

### Block partitioning

There are no additional variations of prediction block size and transform matrices for TT partitions. Therefore, the complexity of block partitioning at the decoder is not increased in PEM.

### Tools combinations for inter prediction

FRUC and LIC need to access the reconstructed image after bilateral filter (BLF). Therefore, the target CU needs to wait for the previous CU process during the worst feedback loop period. In JEM, FRUC Merge would be the worst path (Figure 47). On the other hand, since PEM introduces additional tool combination rules, the worst feedback loop is greatly shortened (Figure 48). It greatly helps to achieve parallel processing.

Figure 47: JEM decoding block diagram

Figure 48: PEM decoding block diagram

Table 22 and Table 23 show the detail tool on/off test results of tools combinations for Inter CU. Test conditions are the same as in section 12.2.10. Every tool can reduce the encoding or decoding runtime significantly with small coding loss. FRUC SIMP and OBMC COMB mainly reduce the decoding runtime, whereas LIC SIMP mainly reduces the encoding runtime. Especially, FRUC SIMP that removes SubCU level refinement from FRUC alone achieves decoding time reduction of 22% in Tool-off test and 20% in Tool-on test. This significantly reduces the worst path of FRUC Merge mode and helps reducing the worst feedback loop.

Table 22: Detail tool on/off test results of tool combination for Inter CU (CS1)

| Tool name | Tool-off test |  |  |  |  | Tool-on test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| FRUC SIMP | -0.02% | -0.21% | -0.17% | 104% | 122% | 0.26% | 0.60% | 0.48% | 94% | 80% |
| LIC SIMP | -0.09% | 0.06% | 0.21% | 113% | 100% | 0.22% | 0.37% | 0.18% | 87% | 101% |
| BIO SIMP | -0.10% | -0.07% | 0.01% | 101% | 105% | 0.05% | 0.03% | 0.09% | 99% | 99% |
| OBMC COMB | 0.11% | -0.12% | 0.00% | 99% | 105% | 0.16% | 0.52% | 0.33% | 94% | 89% |
| All tools | -0.16% | -0.40% | -0.38% | 120% | 133% | 0.50% | 0.99% | 0.62% | 77% | 74% |

Table 23: Detail tool on/off test results of tool combination for Inter CU (CS2)

| Tool name | Tool-off test |  |  |  |  | Tool-on test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| FRUC SIMP | 0.00% | 0.06% | -0.20% | 106% | 114% | 0.22% | 0.72% | 0.47% | 93% | 85% |
| LIC SIMP | -0.19% | -0.01% | -0.46% | 124% | 101% | 0.19% | 0.57% | 0.48% | 79% | 102% |
| BIO SIMP | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| OBMC COMB | -0.06% | 0.07% | 0.06% | 101% | 109% | 0.25% | 0.63% | 0.96% | 94% | 88% |
| All tools | -0.17% | -0.80% | -0.83% | 133% | 124% | 0.49% | 1.42% | 1.81% | 70% | 79% |

FRUC SIMP	: Remove SubCU level FRUC process.

LIC SIMP	: Disable LIC for Normal Inter.

BIO SIMP	: Disable BIO for SubCU Normal Merge (ATMVP/STMVP).

OBMC COMB	: Disable OBMC when LIC is used.

Disable OBMC for FRUC Merge.

Enable EMT for all inter mode.

## Complexity characteristics of 360° video specific decoding tools: N/A

There is nothing specific to 360° Video in this proposal.

## Complexity characteristics of HDR specific decoding tools: N/A

There is nothing specific to HDR in this proposal.

## Degree of capability for decoder parallel processing

PEM has the same degree of capability for decoder parallel processing as JEM7.0. However, PEM is more suitable to hardware pipeline architecture due to the reduced worst feedback loop in inter prediction (see section 5.11.2 for more details).

# Encoder complexity analysis

## Encoding time and measurement methodology

Please refer to Table 10 and Table 11 for the encoding time comparison to HM anchors: PEM requires 817% of the encoding time of HM16.16 for constraint set 1 and 830% of the encoding time of HM16.16 for constraint set 2. In addition, Table 12 and Table 13 also give information about the encoding time of PEM compared to JEM anchors: PEM requires 107% of the encoding time of JEM7.0 for constraint set 1 and 100% of the encoding time of JEM7.0 for constraint set 2.

## Description of computing platform used to determine encoding times reported in section 6.1

Encoding times are measured using the following platform:

CPU: Intel Core i7-6800K, 3.4GHz, 6Core

RAM: 64GB

HDD: SATA 1TB

OS: Ubuntu Linux 16.04.3 LTS (64bit OS)

Compiler: GCC6.2.0 (64bit executable)

Parallel encoding: Yes (split a sequence at 1 second intervals and remove duplicated I picture)

Parallel execution: Yes (use all 6 cores per 1 PC)

## Memory usage of encoder

The test conditions used are the same as in section 6.2. The value reported in Table 24 is VSZ (Virtual Set Size) measured by ps command on linux platform.

Table 24: Encoder maximum memory usage of PEM and JEM7.0

|  | PEM | JEM7.0 |
| --- | --- | --- |
| Constraint set 1 (Class SDR-A/R4) | 6.35GB | 6.34GB |
| Constraint set 2 (Class SDR-B/R4) | 0.98GB | 0.96GB |

## Complexity characteristics of encoder intra-frame prediction type selection

Same as JEM7.0.

## Complexity characteristics of encoder motion estimation and motion segmentation selection

### General: triangular prediction unit mode

In the worst-case scenario, prediction blocks for skip/merge mode cannot be reused for triangular prediction unit mode. In that case, additional 7 inter prediction processes which exclude DMVR and OBMC processes are performed, as shown in Figure 49. Compared to the memory access bandwidth of the skip/merge mode in JEM, PEM requires additionally 1 time or less than 1 time the memory access bandwidth.

Figure 49: An illustration of encoder process for merge mode and triangular prediction unit mode

In addition, additional multiplication and addition operations are required for the adaptive weighting process. Table 20 shows the number of operations for performing one adaptive weighting process. The numbers in Table 20 may be multiplied by 7, 14, 40 or 62 based on the fast algorithm described in section 3.1.4.1.

### Complexity characteristics of usage of decoder-side motion refinement

See sections 5.8.2 and 5.8.3.

## Complexity characteristics of encoder transforms and transform type selection

PEM has a lower complexity characteristic than JEM7.0 for transform type selection due to many checks being skipped, as described in section 3.1.7. Since NSST can be used only in intra blocks, large complexity reduction is observed especially in All Intra configuration, as shown in Table 25.

Table 25: Tool on/off results for simplified transform

| Condition | Tool-off test |  |  |  |  | Tool-on test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| All Intra | -0.35% | 0.44% | -0.05% | 144% | 101% | 0.48% | -0.54% | -0.26% | 68% | 100% |
| Constraint set 1 | -0.23% | -0.11% | 0.27% | 117% | 100% | 0.32% | -0.05% | 0.01% | 91% | 101% |
| Constraint set 2 | -0.32% | 0.13% | 0.01% | 109% | 101% | 0.19% | 0.37% | 0.27% | 96% | 100% |

For evaluating performances in All Intra conditions, QPs and sequences are the same as in CS2 tool on/off tests.

## Complexity characteristics of encoder quantization and quantization type selection

Same as JEM7.0.

## Complexity characteristics of encoder in-loop filtering type selection and parameter estimation

Same as JEM7.0.

## Complexity characteristics of encoder entropy coding type selection

Same as JEM7.0.

## Complexity characteristics of additional encoder tools: block partition structure selection

Table 26 and Table 27 summarize the tool off and tool on tests results of individual speedup tools described in section 3.1.2 compared to PEM with all the speedup tools enabled (for tool off test) or with all the speedup tools disabled (for tool on test). The test conditions are the same as the conditions described in sections 5.2 and 6.2, except that only the first second of the test sequences is used. The results show an average of all the class SDR-A and SDR-B sequences for constraint set 1, and an average of all the class SDR-B sequences for constraint set 2.

For the speedup tools from section 3.1.2.1 and section 3.1.2.4, there is significant difference between tool off and tool on tests performance. This difference is due to overlap in performance between the two tools. Section 3.1.2.4 is based on choosing “no split” as the best mode for blocks of lower depth. Section 3.1.2.1 is based on choosing “skip mode” as the best mode for blocks of lower depth. Section 3.1.2.4 has larger coverage compared to section 3.1.2.1. As a result, section 3.1.2.1 has some independent coverage of 6% encoder time increase in tool off test.

For the speedup tools from section 3.1.2.2, although only 2% reduction in encoder time is seen in tool on tests, performance is significantly better when used on top of other speedup tools. This tool is based on a similar tool from HHI’s NextSoftware JVET-H0084 [8] where QT selection is checked before BT selection, and extended to TT. There is almost no loss in BD-Rate(Y) with 9% increase in encoder time in tool off test.

The speedup tools from section 3.1.2.3 are important speedup tools for speeding up TT selection process based on BT selection, activity in original content and block size. Although some coding loss occurs, the speedup performance is extremely large.

All speedup tools from section 3.1.2.1 to section 3.1.2.4 together achieve about 81% encoding time reduction with 1.49% BD-Rate(Y) loss for constraint set 1 configuration, and about 77% encoding time reduction with 1.46% BD-Rate(Y) loss for constraint set 2 configuration in tool on tests.

Table 26: Summary of "tool off/on" tests for block partition speedup tools (constraint set 1)

| Condition | Tool-off test |  |  |  |  | Tool-on test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| Section 3.1.2.1 | -0.04% | 0.12% | 0.28% | 106% | 100% | 0.04% | -0.20% | -0.16% | 51% | 100% |
| Section 3.1.2.2 | 0.00% | -0.08% | 0.27% | 109% | 100% | 0.09% | 0.12% | -0.27% | 98% | 102% |
| Section 3.1.2.3 | -1.32% | -2.02% | -1.82% | 214% | 102% | 1.24% | 2.15% | 1.92% | 37% | 100% |
| Section 3.1.2.4 | -0.05% | 0.02% | 0.00% | 122% | 101% | 0.01% | -0.35% | -0.34% | 56% | 100% |
| All tools | -1.46% | -2.00% | -1.63% | 526% | 102% | 1.49% | 2.05% | 1.66% | 19% | 100% |

Table 27: Summary of "tool off/on" tests for block partition speedup tools (constraint set 2)

| Condition | Tool-off test |  |  |  |  | Tool-on test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| Section 3.1.2.1 | -0.09% | 0.45% | 0.05% | 101% | 101% | 0.09% | -0.77% | -0.20% | 61% | 101% |
| Section 3.1.2.2 | -0.07% | -0.46% | 0.30% | 110% | 100% | 0.07% | -0.71% | -0.02% | 97% | 100% |
| Section 3.1.2.3 | -1.34% | -2.58% | -2.38% | 219% | 102% | 1.42% | 2.07% | 2.91% | 40% | 101% |
| Section 3.1.2.4 | 0.07% | -0.01% | -0.10% | 123% | 101% | -0.04% | -0.91% | 0.41% | 62% | 101% |
| All tools | -1.44% | -1.70% | -2.55% | 437% | 101% | 1.46% | 1.74% | 2.63% | 23% | 103% |

## Complexity characteristics of 360° video specific encoding tools: N/A

There is nothing specific to 360° Video in this proposal.

## Complexity characteristics of HDR specific encoding tools: N/A

There is nothing specific to HDR in this proposal.

## Degree of capability for encoder parallel processing

PEM has the same degree of capability for encoder parallel processing as JEM7.0.

# Algorithmic characteristics

## Random access characteristics

Same as JEM7.0.

## Delay characteristics

Same as JEM7.0.

## Additional characteristics discussion topic(s)

None.

# Software implementation description

JEM7.0 was used as basis for the development of PEM. All modifications to JEM7.0, including the bug fixes, are clearly implemented using macros, so that disabling all the macros reverts to the code of JEM7.0. Therefore, the code of PEM will be generally familiar to most JVET participants, and tests on individual tools is facilitated by the macros that can be easily disabled.

# Highlighted aspects discussion

## Low complexity version of PEM

As additional information, an even lower complexity version of PEM is also available that provides similar coding performances to JEM7.0 for an encoder runtime that is 60% that of JEM7.0 for constraint set 1 and 54% that of JEM7.0 for constraint set 2. The only difference to the algorithm of PEM described in this document is that tri-tree partitioning has been disabled in the low complexity version. All other coding tools are the same as in the normal version of PEM. The main advantage of this low complexity version is that encoder runtime can be significantly decreased compared to JEM7.0. Table 28 and Table 29 give the results of the low complexity version of PEM compared to JEM7.0 for constraint set 1 and constraint set 2, respectively, using the same methodology and platform as in sections 5.1 and 6.1.

Table 28: Compression performance results of low complexity version of PEM for constraint set 1 configuration relative to JEM anchor

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0(GCC6.2.0, 1 core for decode, dec YUV output) |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -0.47% | 0.80% | 0.20% | 0.02 | -0.02 | 0.00 |
| CatRobot1 | 0.42% | 1.34% | 1.28% | -0.01 | -0.01 | -0.02 |
| DaylightRoad2 | 0.15% | 2.28% | 1.79% | 0.00 | -0.03 | -0.02 |
| ParkRunning3 | -1.10% | -0.10% | -1.06% | 0.03 | 0.00 | 0.01 |
| Campfire | -0.26% | -0.67% | -0.72% | 0.01 | 0.02 | 0.01 |
| BQTerrace | 0.65% | 0.75% | -0.55% | -0.01 | -0.01 | 0.01 |
| RitualDance | -1.01% | 0.37% | 0.68% | 0.05 | -0.01 | -0.02 |
| MarketPlace | -0.22% | 1.63% | 1.41% | 0.01 | -0.02 | -0.02 |
| BasketballDrive | -0.53% | 1.42% | 0.83% | 0.01 | -0.02 | -0.02 |
| Cactus | 1.80% | 1.31% | 1.35% | -0.05 | -0.01 | -0.02 |
| Average SDR-A | -0.25% | 0.73% | 0.30% | 0.01 | -0.01 | 0.00 |
| Average SDR-B | 0.14% | 1.10% | 0.74% | 0.00 | -0.01 | -0.01 |
| Average all | -0.06% | 0.91% | 0.52% | 0.00 | -0.01 | -0.01 |
| Enc Time[%] | 60% |  |  |  |  |  |
| Dec Time[%] | 55% |  |  |  |  |  |

Table 29: Compression performance results of low complexity version of PEM for constraint set 2 configuration relative to JEM anchor

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0(GCC6.2.0, 1 core for decode, dec YUV output) |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | 0.61% | 5.20% | -2.03% | -0.01 | -0.04 | 0.01 |
| RitualDance | -0.82% | 2.16% | 1.89% | 0.04 | -0.04 | -0.04 |
| MarketPlace | -0.78% | 3.03% | 1.30% | 0.02 | -0.03 | -0.01 |
| BasketballDrive | -0.62% | 0.35% | 0.36% | 0.02 | -0.01 | -0.01 |
| Cactus | 1.17% | 3.64% | 4.52% | -0.04 | -0.03 | -0.07 |
| Average SDR-B | -0.09% | 2.87% | 1.21% | 0.01 | -0.03 | -0.02 |
| Enc Time[%] | 54% |  |  |  |  |  |
| Dec Time[%] | 59% |  |  |  |  |  |

## Panasonic’s NextSoftware

In JVET-H0084 [8], HHI provided an alternative implementation of JEM called NextSoftware. As additional information, Panasonic has implemented all the PEM tools mentioned in this document in the code base of NextSoftware, as provided by HHI. Table 30 and Table 31 give the results of this alternative software compared to JEM7.0 for constraint set 1 and constraint set 2, respectively, using the same methodology and platform as in sections 5.1 and 6.1, except that only the first second of the test sequences and different compilers are used. GCC5.4.0 is used for compiling this alternative software, however GCC6.2.0 is used for compiling JEM7.0. The results obtained are slightly better than the results of PEM in terms of complexity, while being similar in terms of coding performance. Therefore, these results show that the technologies described in this document result in consistent behaviour independently from the implementation.

Table 30: Compression performance results of PEM tools in the code base of HHI’sNextSoftware for constraint set 1 configuration relative to JEM anchor

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0(GCC6.2.0, 1 core for decode, dec YUV output) |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -2.47% | -2.47% | -3.03% | 0.09 | 0.05 | 0.08 |
| CatRobot1 | -2.01% | -4.85% | -4.05% | 0.05 | 0.05 | 0.08 |
| DaylightRoad2 | -3.00% | -5.61% | -5.33% | 0.06 | 0.07 | 0.07 |
| ParkRunning3 | -2.15% | -2.35% | -4.06% | 0.06 | 0.04 | 0.05 |
| Campfire | -2.51% | -3.24% | -4.22% | 0.07 | 0.10 | 0.09 |
| BQTerrace | -1.50% | -2.74% | -2.45% | 0.04 | 0.04 | 0.04 |
| RitualDance | -2.63% | -4.89% | -5.60% | 0.13 | 0.11 | 0.15 |
| MarketPlace | -2.56% | -4.29% | -4.61% | 0.07 | 0.05 | 0.06 |
| BasketballDrive | -2.49% | -4.11% | -4.25% | 0.07 | 0.06 | 0.10 |
| Cactus | -0.81% | -4.55% | -2.63% | 0.03 | 0.05 | 0.05 |
| Average SDR-A | -2.43% | -3.71% | -4.14% | 0.07 | 0.06 | 0.07 |
| Average SDR-B | -2.00% | -4.12% | -3.91% | 0.07 | 0.06 | 0.08 |
| Average all | -2.21% | -3.91% | -4.02% | 0.07 | 0.06 | 0.08 |
| Enc Time[%] | 98% |  |  |  |  |  |
| Dec Time[%] | 52% |  |  |  |  |  |

Table 31: Compression performance results of PEM tools in the code base of HHI’s NextSoftware for constraint set 2 configuration relative to JEM anchor

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0(GCC6.2.0, 1 core for decode, dec YUV output) |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -1.63% | -4.08% | -5.06% | 0.04 | 0.04 | 0.06 |
| RitualDance | -2.39% | -4.45% | -3.53% | 0.12 | 0.10 | 0.09 |
| MarketPlace | -2.21% | -3.84% | -5.84% | 0.06 | 0.04 | 0.07 |
| BasketballDrive | -2.43% | -4.08% | -2.84% | 0.07 | 0.07 | 0.07 |
| Cactus | -1.65% | -4.03% | -4.08% | 0.05 | 0.04 | 0.06 |
| Average SDR-B | -2.06% | -4.10% | -4.27% | 0.07 | 0.06 | 0.07 |
| Enc Time[%] | 92% |  |  |  |  |  |
| Dec Time[%] | 52% |  |  |  |  |  |

# Closing remarks

PEM was designed based on JEM7.0 with increased coding performance for a lower algorithmic complexity and better hardware friendliness. PEM was developed using previous JVET work as much as possible.

Panasonic believes that PEM would be a good basis for further video coding developments within JVET. The code being based on JEM7.0 is expected to be very familiar to JVET participants, while the optimisation of tools and decreased complexity will facilitate tests for further coding tools.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex 1: Questionnaire

Proponents of CfP responses are required to answer the codec level questions, and are also encouraged to fill out the optional tool level questions in addition to the complexity analysis of their proposals. If some question is not applicable to your proposal, you may express that as well.

## Codec level questions

| Codec Level Q&A |  | HM | JEM | PEM |
| --- | --- | --- | --- | --- |
| 1 | Maximum number of reference frames used by encoder | 4 | 4 | Same as JEM |
| 2 | Maximum coding unit (e.g. CTU in HEVC) | luma 64×64 chroma 32×32 | luma 128×128 chroma 64×64 | Same as JEM |
| 3 | Minimum and maximum transform block size | luma 4×4/32×32 chroma 4×4/16×16 | luma 4×4/128×128 chroma 2×2/64×64 | same as JEM |
| 4 | Minimum and maximum intra prediction block | luma 4×4/64×64 chroma 4×4/32×32 | luma 4×4/128×128 chroma 2×2/64×64 | Same as JEM |
| 5 | Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length | luma 4×8/8×4 (uni-pred) and 8×8 (bi-pred), 8-tap Chroma 2×4/4×2 (uni-pred) and 4×4 bi-pred, 4-tap | luma 4×4 (uni/bi-pred), 8-tap Chroma 2×2 (uni/bi-pred), 4-tap | Same as JEM |
| 6 | List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category) | n/a | template matching in FRUC, LIC | Same as JEM |
| 7 | List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples, position unforeseeable) | n/a | 64-bit arithmetic for LIC, BIO and LMC | Same as JEM |
| 8 | List all the tools that use non-rectangular (e.g. triangle) partitions | n/a | n/a | Triangular prediction unit mode |
| 9 | Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa) | n/a | n/a | n/a |
| 10 | Indicate whether luma and chroma use separate partitioning structures | no | Yes, in I-slices | Same as JEM |
| 11 | Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes) | Entropy parsing depends on intra prediction mode reconstruction, which requires an intra MPM list derivation | Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation | Same as JEM |

## Tool level questions

### Partitioning structure

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1. Partitioning structure |  | HM | JEM | PEM |
| 1.a | Whether the prediction unit partitioning is different from transform unit partitioning | Yes, PU and TU partitioning for a CU can differ | No | In general no, except for CUs encoded with the triangular prediction unit mode |
| 1.b | Maximum tree depth | CU maximum depth 4 TU maximum depth 3 PU maximum depth 1 | QT maximum depth 5 BT maximum depth 3 | QT maximum depth 5 BT & TT maximum depth 3 |

### Entropy coding

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 2. Entropy coding |  | HM | JEM | PEM |
| 2.a | Total number of context models | 186 | 329 (actually fewer, because about 20% of the context models are not used) | 335 (actually 267, because 68 context models are not used) |
| 2.b | Memory storage size for each context model | 8×3-bit | 10×3-bit | 14-bit |
| 2.c | Range table size if table look up is used for range update | LPSTable[64×4×8bits]; RenormTable[32× 3bits] | LPSTable[512×64×9bits]; RenormTable[128×3bits] | Same as JEM |
| 2.d | Initialization table size | 3×186×8 bits | 3×329×8 + 329×15×5 bits | 335×14 + 335×15×5 bits |
| 2.e | Other memory associated with context model adaptation | LPS transition table: 64*6 bits MPS transition table: 64*6 bits | n/a | n/a |

### Control data derivation process

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 3. Control data derivation process |  | HM | JEM | PEM |
| 3.a | Temporal motion data storage granularity | 16×16 | 4×4 | Same as JEM |
| 3.b | Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-directional reference picture are used, count this picture as 2) | 1 | 4 | Same as JEM |

### Decoder-side motion refinement

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 4. Decoder-side motion refinement |  | HM | JEM | PEM |
| 4.a | Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement. | n/a | - BIO 2×2D separable 6 taps filter (for gradients), mults per sample in refinement calculation 5+5*4 +1/8+2, minimum processing unit same as regular bi-pred, no SAD calculation - FRUC minimum processing unit 44, sequential PU followed by sub-PU level refinement, number of SAD calculations and additional MC interpolations per unit for MV refinement is not constrained (refinement is terminated when cost no longer decreases); bi-linear filter at MV refinement stage | - BIO 5 taps filter (for gradients) - FRUC minimum processing unit 44, number of SAD calculations and additional MC interpolations per unit for MV refinement is not constrained (refinement is terminated when cost no longer decreases around a search point); bi-linear filter at MV refinement stage |
| 4.b | Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size. | n/a | - DMVR needs to access (H+7+2)×(W+7+2) MC instead of (H+7)×(W+7) in HEVC MC - FRUC not constrained | - DMVR needs to access (H+7+2)×(W+7+2) MC instead of (H+7)×(W+7) in HEVC MC - FRUC is constrained by size of pre-defined memory block (see section 2.1.4.1) |

### Intra prediction

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 5. Intra prediction |  | HM | JEM | PEM |
| 5.a | Line buffer size in terms of luma and chroma sample lines | luma 1, chroma 1 | luma 4, chroma 2 | Same as JEM |
| 5.b | List all the filters used for intra prediction | Reference sample smoothing: {1,2,1}, Strong intra smoothing filter: 2-tap bilinear filter (only for 32×32 luma TU) Interpolation: 2×32×5 bits (bi-linear 32 phases) | Reference sample smoothing: {1,2,1}, {3,7,14,16,14,7,3}, {4,9,12,14,12,9,4} Interpolation: 2×4×32×8 bits (4 taps, 2 sets, 32 phase) | Same as JEM |
| 5.c | Indicate whether there are cross-component dependencies in intra prediction reconstruction | n/a | LM-CHROMA (Cb depends on Y, and Cr depends on Cb&Y) | Same as JEM |

### Inter prediction

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 6. Inter prediction |  | HM | JEM | PEM |
| 6.a | Motion vector accuracy | luma ¼ pel chroma 1/8 pel | luma 1/16 pel chroma 1/32 pel | Same as JEM |
| 6.b | Storage size for a motion vector component | 16-bit | 16-bit | Same as JEM |
| 6.c | List all the interpolation filters used for luma/chroma motion compensation | luma 8-tap, 8×4×6 bits chroma 4-tap, 4×8×6 bits | luma 8-tap, 8×16×6 bits chroma 4-tap, 4×32×6 bits | Same as JEM |

### Quantization

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 7. Quantization |  | HM | JEM | PEM |
| 7.a | Storage size associated with quantization | n/a | n/a | n/a |

### Transforms

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 8. Transforms |  | HM | JEM | PEM |
| 8.a | List all the combinations of transform block size and transform type for primary transforms | - 8-bit 4×4, 8×8, 16×16 and 32×32 DCT-II, - 8-bit 4×4 DST VII | HEVC transform mode (10-bit DCT-II), transform size combinations: - (4, 8, 16, 32, 64, 128) × (4, 8, 16, 32, 64, 128) TUs for luma CTUs - (2, 4, 8, 16, 32, 64) × (2, 4, 8, 16, 32, 64) TUs for inter-coded chroma CTUs - (2, 4, 8, 16, 32, 64,) × (2, 4, 8, 16, 32, 64) TUs for intra-coded chroma CTUs EMT transform mode (10-bit DST VII, DCT-V, DCT-VIII, DST-I), only applied to CTUs up to 64×64, transform size combinations: - (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TUs for luma CTUs | HEVC transform mode (10-bit DCT-II), transform size combinations: - (4, 8, 16, 32, 64, 128) × (4, 8, 16, 32, 64, 128) TUs for luma CTUs - (2, 4, 8, 16, 32, 64) × (2, 4, 8, 16, 32, 64) TUs for inter-coded chroma CTUs - (2, 4, 8, 16, 32, 64,) × (2, 4, 8, 16, 32, 64) TUs for intra-coded chroma CTUs EMT transform mode (10-bit DST-VII, DCT-VIII), only applied to CTUs up to 64×64, transform size combinations: - (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TUs for luma CTUs EMT transform mode (10-bit DCT-V, DST-I), only applied to CTUs up to 64×64, transform size combinations: - (8, 16, 32, 64) × (8, 16, 32, 64) TUs for intra-coded luma CTUs |
| 8.b | List all the combinations of transform block size and transform type for secondary transforms | n/a | 35×3 4×4 and 8×8 non-separable Hypercube-Givens Transform (HyGT) | Same as JEM |
| 8.c | List all the non-separable transforms used | n/a | 4×4/8×8 HyGT | Same as JEM |
| 8.d | Indicate whether transforms can be fully/partially implemented using direct matrix multiply | partial factorization equivalent to matrix multiplication | factorized form not equivalent to matrix multiplication | Same as JEM |
| 8.e | List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output) | n/a | 4×4 HyGT 2-interations, 8×8 HyGT 4-interations | Same as JEM |

### In-loop filtering

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 9. In-loop filtering |  | HM | JEM | PEM |
| 9.a | List line buffer size in terms of luma and chroma lines (if any) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) ALF (luma 8, chroma 4) | Same as JEM |
| 9.b | List all the filters used for in-loop filtering | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. ALF: 9×9/7×7/5×5 diamond filter support for luma; and 5×5 diamond filter support for chroma | Filters are the same as JEM. Deviations on deblocking weak-filter for luma are weighted by a constant. |
| 9.c | Minimum processing unit size | Deblocking (luma: 8×8 grid with 4×4 filter decision, chroma 8×8 grid with 4×4 filter decision) | Deblocking (luma 4×4 grid with 4×4 filter decision, chroma 8×8 grid with 2×2 filter decision) ALF: for luma, filter decision is based on 2×2 block Bilateral filter: sample level filter switch (luma only) | Same as JEM |

### “tool-off” and “tool-on” tests

Table 32 and Table 33 summarize the results of individual tools in terms of coding efficiency and encoder/decoder runtime compared to the anchor in the first row of the tables. The test conditions are the same as the conditions described in sections 5.2 and 6.2, except that only the first second of the test sequences is used. The results have been obtained as an average of all the class SDR-A and SDR-B sequences for constraint set 1, and as an average of all the class SDR-B sequences for constraint set 2. The anchor for the “tool-off” tests is completely the same as PEM, and the anchor for the “tool-on” tests generates completely the same bitstream as JEM7.0.

Table 32: Summary of "tool-on" and "tool off" tests for PEM vs JEM7.0 (constraint set 1)

| Tested tool name | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| Anchor (vs HM16.16) | -34.57% | -45.45% | -44.47% | 842% | 401% | -33.02% | -43.49% | -42.15% | 815% | 707% |
| TRI_TREE | 1.98% | 4.15% | 4.26% | 64% | 97% | -1.95% | -4.14% | -4.13% | 159% | 104% |
| INTERTOOL_COMB | -0.16% | -0.40% | -0.38% | 120% | 133% | 0.50% | 0.99% | 0.62% | 77% | 74% |
| TRIANGLE | 0.40% | 0.46% | 0.64% | 93% | 101% | -0.27% | -0.31% | -0.54% | 101% | 101% |
| FRUC_LOWBW | 0.01% | 0.14% | 0.26% | 100% | 99% | 0.01% | -0.08% | -0.25% | 101% | 102% |
| FRUC_LOWCOMP | -0.03% | -0.06% | 0.11% | 109% | 110% | 0.15% | 0.11% | 0.08% | 90% | 89% |
| OBMC_LOWCOMP | -0.12% | 0.18% | 0.07% | 100% | 104% | 0.27% | 0.25% | 0.10% | 97% | 95% |
| BIO_LOWCOMP | -0.03% | 0.06% | 0.11% | 100% | 101% | 0.06% | 0.04% | 0.08% | 99% | 99% |
| BIO_IMPROVE | 0.18% | 0.01% | 0.03% | 100% | 100% | -0.08% | 0.03% | 0.11% | 100% | 101% |
| ATMVP_LOWCOMP | 0.00% | -0.01% | 0.02% | 99% | 105% | 0.05% | 0.11% | -0.02% | 98% | 95% |
| SKIP_SIMP | 0.10% | 0.06% | 0.31% | 100% | 101% | -0.01% | 0.18% | -0.19% | 98% | 100% |
| INTRA_IMPROVE | 0.20% | 0.33% | 0.47% | 98% | 100% | -0.12% | 0.10% | 0.00% | 100% | 101% |
| TRANS_LOWCOMP | -0.23% | -0.11% | 0.27% | 117% | 100% | 0.32% | -0.05% | 0.01% | 91% | 101% |
| DBF_IMPROVE | 0.15% | 0.02% | -0.02% | 100% | 100% | -0.15% | 0.08% | -0.09% | 100% | 101% |
| CABAC_IMPROVE | 0.38% | -0.14% | 0.50% | 99% | 101% | -0.38% | -0.04% | -0.25% | 101% | 98% |
| BIO_CODEREFINE | 0.00% | 0.00% | 0.00% | 100% | 105% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| ALF_CODEREFINE | 0.00% | 0.00% | 0.00% | 99% | 105% | 0.00% | 0.00% | 0.00% | 100% | 98% |

Table 33: Summary of "tool-on" and "tool off" tests for PEM vs JEM7.0 (constraint set 2)

| Tested tool name | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| Anchor (vs HM16.16) | -28.11% | -54.34% | -55.42% | 838% | 346% | -26.40% | -52.71% | -53.08% | 835% | 572% |
| TRI_TREE | 1.86% | 3.72% | 4.48% | 57% | 96% | -1.92% | -4.92% | -4.83% | 178% | 103% |
| INTERTOOL_COMB | -0.17% | -0.80% | -0.83% | 133% | 124% | 0.49% | 1.42% | 1.81% | 70% | 79% |
| TRIANGLE | 0.64% | 1.21% | 0.75% | 95% | 102% | -0.62% | -0.53% | -0.33% | 99% | 100% |
| FRUC_LOWBW | -0.08% | 0.09% | -0.22% | 100% | 100% | 0.03% | 0.19% | 0.22% | 99% | 102% |
| FRUC_LOWCOMP | -0.10% | 0.31% | -0.22% | 114% | 113% | -0.04% | 0.26% | 0.54% | 85% | 88% |
| OBMC_LOWCOMP | -0.30% | -0.34% | -0.22% | 101% | 107% | 0.32% | 0.53% | 0.50% | 97% | 93% |
| BIO_LOWCOMP | -0.09% | 0.06% | 0.02% | 101% | 101% | 0.00% | 0.39% | 0.35% | 99% | 98% |
| BIO_IMPROVE | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| ATMVP_LOWCOMP | -0.14% | -0.15% | 0.21% | 100% | 105% | 0.07% | 0.10% | 0.06% | 97% | 95% |
| SKIP_SIMP | -0.22% | 0.10% | 0.04% | 100% | 101% | 0.08% | 0.14% | 0.20% | 98% | 99% |
| INTRA_IMPROVE | 0.03% | -0.14% | -0.46% | 99% | 100% | -0.12% | 0.62% | 0.16% | 100% | 100% |
| TRANS_LOWCOMP | -0.32% | 0.13% | 0.01% | 109% | 101% | 0.19% | 0.37% | 0.27% | 96% | 100% |
| DBF_IMPROVE | -0.01% | 0.12% | 0.04% | 100% | 100% | -0.22% | 0.01% | 0.24% | 101% | 100% |
| CABAC_IMPROVE | 0.36% | -0.37% | 0.05% | 99% | 101% | -0.50% | 0.83% | -0.42% | 101% | 98% |
| BIO_CODEREFINE | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| ALF_CODEREFINE | 0.00% | 0.00% | 0.00% | 100% | 105% | 0.00% | 0.00% | 0.00% | 100% | 96% |

The correspondence between the names of the coding tools in Table 32 and Table 33 and the sections in which these tools are presented is given below:

- TRI_TREE: Tri-tree partition structure in section 2.1.1

- INTERTOOL_COMB: Tools combinations for inter prediction in section 2.1.11.1

- TRIANGLE: Triangular prediction unit mode in section 2.1.7.1

- FRUC_LOWBW: FRUC bandwidth reduction in section 2.1.4.1

- FRUC_LOWCOMP: FRUC complexity reduction in sections 2.1.4.2, 2.1.4.3, 2.1.4.4 and 2.1.4.5

- OBMC_LOWCOMP: OBMC complexity reduction in section 2.1.7.2

- BIO_LOWCOMP: BIO complexity reduction in section 2.1.4.6

- BIO_IMPROVE: BIO modification on displacement calculation in section 2.1.4.7

- ATMVP_LOWCOMP: ATMVP/STMVP complexity reduction in sections 2.1.7.3 and 2.1.7.4

- SKIP_SIMP: Syntax simplification for skip mode in section 2.1.11.2

- INTRA_IMPROVE: Improvements to intra mode coding and intra filtering in sections 2.1.3.1 and 2.1.8

- TRANS_LOWCOMP: Transforms complexity reduction in section 2.1.6

- DBF_IMPROVE: Improvements to deblocking filter in sections 2.1.9.1 and 2.1.9.2

- CABAC_IMPROVE: CABAC window size for each context in section 2.1.2

- BIO_CODEREFINE: Code optimisation of BIO in section 2.1.4.8

- ALF_CODEREFINE: Code optimization of ALF in section 2.1.9.3.

# Annex 2: Syntax

Table 34 includes block partitioning, coding unit, prediction and transform information syntax in a PEM bitstream. The names have been chosen to be as close as possible to the syntax from the HEVC standard. The modifications introduced in PEM on top of JEM are shown in red in Table 34.

Table 34: PEM syntax

| Syntax | Semantics |
| --- | --- |
| coding_tree_unit() { |  |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| if( slice_sao_luma_flag ¦¦ slide_sao_chroma_flag ) |  |
| sao( xCtb >> CtbLog2SizeY, yCtb >> CtbLog2SizeY ) |  |
| coding_quadtree( xCtb, yCtb, CtbLog2SizeY, 0, 0 ) |  |
| if( slice_type == I_SLICE && ChromaArrayType != 0 ) |  |
| coding_quadtree( xCtb, yCtb, CtbLog2SizeY, 0, 1 ) |  |
| } |  |
| coding_quadtree (x0, y0, log2CqtbSize, cqtDepth, chroma) { |  |
| forceQT = width > MAX_QT_SIZE |  |
| xr = x0 + ( 1 << log2CqtbSize ) |  |
| yb = y0 + ( 1 << log2CqtbSize ) |  |
| boundary = ( xr > pic_width_in_luma_samples ¦¦ yb > pic_height_in_luma_samples ) |  |
| if( !boundary && !forceQT && log2CqtbSize > minLog2CqtbSize) |  |
| qt_split_flag | QT flag; inferred equal to 1 (i.e. further splitting) if not present and log2CqtbSize > minLog2CqtbSize, otherwise inferred to be equal to 0 |
| if( qt_split_flag ) { |  |
| x1 = x0 + ( 1 << (log2CqtbSize -1) ) |  |
| y1 = y0 + ( 1 << (log2CqtbSize - 1) ) |  |
| coding_quadtree( x0, y0, log2CqtbSize - 1, cqtDepth + 1, chroma ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| coding_quadtree( x1, y0, log2CqtbSize - 1, cqtDepth + 1, chroma ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x0, y1, log2CqtbSize - 1, cqtDepth + 1, chroma ) |  |
| if( x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x1, y1, log2CqtbSize - 1, cqtDepth + 1, chroma ) |  |
| } |  |
| else |  |
| coding_binary_tri_tree( x0, y0, 1 << log2CqtbSize, 1 << log2CqtbSize, 0, 0, chroma ) |  |
| } |  |
| coding_binary_tri_tree( x0, y0, width, height, btTtDepth, splitConstrain, chroma ) { |  |
| btTtValid = ( ( height > minBTTTSize ¦¦ width > minBTTTSize ) |  |
| && width <= maxBTTTSize && height <= maxBTTTSize && |  |
| btTtDepth < MaxBTTTD ) |  |
| if( btTtValid ) |  |
| bt_tt_split_flag | BT/TT flag; inferred equal to 0 (i.e. no splitting) if not present |
| if( bt_tt_split_flag ) { |  |
| if( width > 8 ¦¦ height > 8 ) { |  |
| tt_flag | 0 if BT is used, 1 is TT is used; inferred to be equal to 0 (BT) if not present |
| if( ( tt_flag && ( width < 16 ¦¦ height > 2*width ) ) ¦¦ |  |
| ( !tt_flag && ( width < 8 ¦¦ splitConstrain == 2 ) ) ) |  |
| split_dir_flag = 0 | Horizontal BT/TT splitting |
| else if( ( tt_flag && ( height < 16 ¦¦ width > 2*height ) ) ¦¦ |  |
| ( !tt_flag && ( height < 8 ¦¦ splitConstrain == 1 ) ) ) |  |
| split_dir_flag = 1 | Vertical BT/TT splitting |
| else |  |
| split_dir_flag | BT or TT splitting direction: 0 for horizontal, 1 for vertical |
| if( tt_flag ) { /* TT partitioning */ |  |
| if( split_dir_flag == 0 ) { | Horizontal TT splitting |
| y1 = y0 + (height >> 2) |  |
| y2 = y0 + 3 * (height >> 2) |  |
| coding_binary_tri_tree( x0, y0, width, height >> 2, btTtDepth + 1, 0, chroma ) |  |
| coding_binary_tri_tree( x0, y1, width, height >> 1, btTtDepth + 1, 0, chroma ) |  |
| coding_binary_tri_tree( x0, y2, width, height >> 2, btTtDepth + 1, 0, chroma ) |  |
| } |  |
| else { /* if( split_dir_flag == 1 ) */ | Vertical TT splitting |
| x1 = x0 + (width >> 2) |  |
| x2 = x0 + 3 * (width >> 2) |  |
| coding_binary_tri_tree( x0, y0, width >> 2, height, btTtDepth + 1, 0, chroma ) |  |
| coding_binary_tri_tree( x1, y0, width >> 1, height, btTtDepth + 1, 0, chroma ) |  |
| coding_binary_tri_tree( x2, y0, width >> 2, height, btTtDepth + 1, 0, chroma ) |  |
| } |  |
| } |  |
| else { /* BT partitioning */ |  |
| if( split_dir_flag == 0 ) { | Horizontal BT splitting |
| y1 = y0 + (height >> 1) |  |
| firstSplitDir = coding_binary_tri_tree( x0, y0, width, height >> 1, btTtDepth + 1, |  |
| 0, chroma ) |  |
| if( firstSplitDir == 2 && slice_type != I_SLICE ) | First block was splitted vertically |
| splitConstrain = 2 |  |
| else |  |
| splitConstrain = 0 |  |
| coding_binary_tri_tree( x0, y1, width, height >> 1, btTtDepth + 1, |  |
| splitConstrain, chroma ) |  |
| } |  |
| else { /* if( split_dir_flag == 1 ) */ | Vertical BT splitting |
| x1 = x0 + (width >> 1) |  |
| firstSplitDir = coding_binary_tri_tree( x0, y0, width >> 1, height, |  |
| btTtDepth + 1, 0, chroma ) |  |
| qtValid = (CTUSize >> cqtDepth) > minQTSize |  |
| if( firstSplitDir == 1 && slice_type != I_SLICE && qtValid ) | First block was splitted horizontally |
| splitConstrain = 1 |  |
| else |  |
| splitConstrain = 0 |  |
| coding_binary_tri_tree( x1, y0, width >> 1, height, btTtDepth + 1, |  |
| splitConstrain, chroma ) |  |
| } |  |
| } |  |
| } /* end of split flag loop */ |  |
| else |  |
| coding_unit( x0, y0, width, height, chroma ) |  |
| if( !bt_tt_split_flag ) |  |
| splitDir = 0 |  |
| else if( !tt_flag ) |  |
| splitDir = split_dir_flag + 1 |  |
| else |  |
| splitDir = split_dir_flag + 3 |  |
| return splitDir |  |
| } |  |
| coding_unit( x0, y0, width, height, chroma ) { |  |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag |  |
| if( slice_type =! I_SLICE ) |  |
| cu_skip_flag | Inferred to be equal to 0 when not present |
| if( cu_skip_flag ) |  |
| prediction( x0, y0, width, height, 1 ) |  |
| else { |  |
| if( slice_type != I_SLICE ) |  |
| pred_mode_flag | 0 for MODE_INTER, 1 for MODE_INTRA (default when not present) |
| if( pred_mode_flag ) { /* MODE_INTRA */ |  |
| if( pcm_enabled_flag && width >= Log2MinIpcmCbSizeY |  |
| && width <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[x0][y0] | Inferred to be equal to 0 when not present |
| if( pcm_flag[x0][y0] ) { |  |
| while( !byte_aligned() ) |  |
| pcm_alignment_zero_bit |  |
| for( i = 0; i < width*height; i++ ) |  |
| pcm_sample_luma[i] |  |
| for( i = 0; i < width*height/2; i++ ) |  |
| pcm_sample_chroma[i] |  |
| } |  |
| else { /* normal intra */ |  |
| if( !chroma ) { |  |
| mpm_flag |  |
| if( mpm_flag ) |  |
| mpm_idx |  |
| else { |  |
| selected_modes_flag |  |
| if( selected_modes_flag ) |  |
| selected_intra_luma_pred_mode_idx |  |
| else |  |
| non_selected_intra_luma_pred_mode_idx |  |
| } |  |
| } |  |
| if( chroma ¦¦ slice_type != I_SLICE ) { |  |
| non_lm_flag | 0 for LM chroma mode; 1 otherwise |
| if( non_lm_flag ) |  |
| intra_chroma_pred_mode |  |
| else { |  |
| if( (width/2 + height/2) >= minSizeMMLM ) |  |
| mmlm_chroma_flag | 0 for LM chroma mode; 1 for MMLM chroma; inferred to be equal to 0 if not present |
| if( !mmlm_chroma_flag && |  |
| (width/2 + height/2) >= minSizeMFLM ) |  |
| downsampling_filter_idx |  |
| } |  |
| } |  |
| } /* end of normal intra loop */ |  |
| } /* end of MODE_INTRA */ |  |
| else /* MODE_INTER */ |  |
| prediction( x0, y0, width, height, 0 ) |  |
| if( !pcm_flag[x0][y0] ) { |  |
| if( !pred_mode_flag && ( !merge_flag[x0][y0] ¦¦ |  |
| affine_flag[x0][y0] ¦¦ lic_flag[x0][y0] ) ) |  |
| rqt_root_cbf | Inferred to be equal to 1 when not present |
| if( rqt_root_cbf ) |  |
| transform( x0, y0, width, height, chroma ) |  |
| } /* end of non-skip loop */ |  |
| } |  |
| prediction( x0, y0, width, height, skip ) { |  |
| if( skip ) { |  |
| fruc_flag | 0 if FRUC not used, 1 otherwise |
| if( fruc_flag ) { |  |
| if( slice_type != P_SLICE ) |  |
| fruc_type | 0 for template, 1 for bilateral |
| else |  |
| fruc_type = 0 |  |
| } |  |
| else { |  |
| triangle_merge_flag |  |
| if( triangle_merge_flag ) |  |
| triangle_idx |  |
| else |  |
| if( isAffineMrgFlagCoded() ) | True only if width x height >=64 and one of left, above, above right, bottom left or above left CU uses affine |
| affine_flag | Inferred to be equal to 0 if not present |
| if( !affine_flag ) |  |
| merge_idx |  |
| } |  |
| if( lic_enabled_flag && fruc_flag ) |  |
| lic_flag |  |
| } /* end of skip loop */ |  |
| else { |  |
| merge_flag |  |
| if( merge_flag ) { |  |
| fruc_flag | 0 if FRUC not used, 1 otherwise |
| if( fruc_flag ) { |  |
| if( slice_type != P_SLICE ) |  |
| fruc_type | 0 for template, 1 for bilateral |
| else |  |
| fruc_type = 0 |  |
| } |  |
| else { |  |
| triangle_merge_flag |  |
| if( triangle_merge_flag ) |  |
| triangle_idx |  |
| else { |  |
| if( isAffineMrgFlagCoded() ) | True only if width x height >=64 and one of left, above, above right, bottom left or above left CU uses affine |
| affine_flag | Inferred to be equal to 0 if not present |
| if( !affine_flag ) |  |
| merge_idx |  |
| } |  |
| } |  |
| } /* end of merge loop */ |  |
| else { /* Normal inter */ |  |
| if( slice_type == B_SLICE ) |  |
| inter_pred_idc[x0][y0] |  |
| if( width > 8 && height > 8 ) |  |
| affine_flag |  |
| if( inter_pred_idc[x0][y0] != PRED_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[x0][y0] |  |
| if( affine_flag ) |  |
| affine_mvd_coding( x0, y0, 0 ) |  |
| else |  |
| mvd_coding( x0, y0, 0 ) |  |
| mvp_l0_idx[x0][y0] |  |
| } |  |
| if( inter_pred_idc[x0][y0] != PRED_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[x0][y0] |  |
| if( affine_flag ) |  |
| affine_mvd_coding( x0, y0, 0 ) |  |
| else |  |
| mvd_coding( x0, y0, 1 ) |  |
| mvp_l1_idx[x0][y0] |  |
| } |  |
| if( !affine_flag ) |  |
| amvr_resolution |  |
| if( width*height <= 256 ) |  |
| obmc_flag |  |
| } /* end of normal inter loop */ |  |
| if( lic_enabled_flag && (!merge_flag ¦¦ fruc_flag ) && !affine_flag ) |  |
| lic_flag |  |
| if( !merge_flag && width * height <= 256 && !lic_flag ) |  |
| obmc_flag |  |
| } |  |
| } |  |
| transform( x0, y0, width, height, chroma ) { |  |
| if( ChromaArrayType != 0 ) { |  |
| cbf_cb | Inferred to be equal to 0 if not present |
| if( cbf_cb ) |  |
| residual_coding( x0, y0, width, height, 1 ) |  |
| cbf_cr | Inferred to be equal to 0 if not present |
| if( cbf_cr ) |  |
| residual_coding( x0, y0, width, height, 2 ) |  |
| } |  |
| if( pred_mode_flag[x0][y0] ¦¦ cbf_cb ¦¦ cbf_cr ) |  |
| cbf_luma | Inferred to be equal to 1 if not present |
| if( cbf_luma ) { |  |
| if( emt_enabled_flag && width <= 64 && height <= 64 && !chroma ) |  |
| emt_cu_flag | Inferred to be equal to 0 if not present |
| residual_coding( x0, y0, width, height, 0 ) |  |
| if( emt_cu_flag ) { |  |
| if( width > 4 ) |  |
| hor_emt_type | Inferred to be equal to 0 if not present |
| if( height > 4 ) |  |
| ver_emt_type | Inferred to be equal to 0 if not present |
| } |  |
| } |  |
| if( cbf_cb ¦¦ cbf_cr ¦¦ cbf_luma ) |  |
| delta_qp() |  |
| if( ( cbf_cb ¦¦ cbf_cr ) && !cu_transquant_bypass_flag ) |  |
| chroma_qp_offset() |  |
| if( nsst_enabled_flag && !cu_transquant_bypass_flag && |  |
| iNonZeroCoeff > iNonZeroCoeffThr && pred_mode_flag ) |  |
| && ( chroma ¦¦ !emt_cu_flag ) ) |  |
| nsst_idx |  |
| } |  |

# References

- “Joint Call for Proposals on Video Compression with Capability beyond HEVC”, JVET, 8th meeting, Macao, China, October 2017, doc. JVET-H1002

- X. Li, H.-C. Chuang, J. Chen, M. Karczewicz, L. Zhang, X. Zhao, A. Said, “Multi-Type-Tree”, JVET, 4th meeting, Chengdu, China, October 2016, doc. JVET-D0117

- A. Said, M. Karczewicz, V. Seregin, H. Egilmez, L. Zhang, X. Zhao, “EE2: Arithmetic coding with context-dependent double-window adaptation response”, JVET, 8th meeting, Macao, China, October 2017, doc. JVET-H0061

- Yu Han, Jicheng An, Jianhua Zheng, “EE1: Improvements for Intra Prediction Mode Coding”, JVET, 8th meeting, Macao, China, October 2017, doc. JVET-H0024

- A. Alshin, E. Alshina, T. Lee, “Bi-Directional Optical Flow for improving Motion Compensation”, 28th Picture Coding Symposium

- Y. Ahn, H. Ryu, D. Sim, “Diagonal motion partitions on top of QTBT block structure”, JVET, 8th meeting, Macao, China, October 2017, doc. JVET-H0087

- “Algorithm Description of Joint Exploration Test Model 7 (JEM 7)”, JVET, 7th meeting, Torino, Italy, July 2017. Doc. JVET-G1001

- A. Wieckowski, T. Hinz, V. George, J. Brandenburg, J. Ma, B. Bross, H. Schwarz, D. Marpe, T. Wiegand, “NextSoftware: An alternative implementation of the Joint Exploration Model (JEM)”, JVET, 8th meeting, Macao, China, October 2017, doc. JVET-H0084

[end of document]