| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0022r1 |
| --- | --- |

| Title: | Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – medium complexity version |  |  |
| --- | --- | --- | --- |
| Status: | Input document approved by JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P. Bordes, Y. Chen, C. Chevance, E. Francois, F. Galpin, M. Kerdranvat, F. Hiron, P. de Lagrange, F. Le Léannec, K. Naser, T. Poirier, F. Racapé, G. Rath, A. Robert, F. Urban, T. Viellard (Technicolor) Y. Chen, W.-J. Chien, H.-C. Chuang, M. Coban, J. Dong, H. E. Egilmez, N. Hu, M. Karczewicz, A. Ramasubramonian, D. Rusanovskyy, A. Said, V. Seregin, G. Van Der Auwera, K. Zhang, L. Zhang (Qualcomm) | Tel: Email: | edouard.francois@technicolor.com martak@qti.qualcomm.com wchien@qti.qualcomm.com vseregin@qti.qualcomm.com dmytror@qti.qualcomm.com hchuang@qti.qualcomm.com |
| Source: | Qualcomm, Technicolor |  |  |

_____________________________

# Abstract

This contribution describes the Medium Complexity version of the joint Qualcomm-Technicolor response to the joint Call for Proposal from ISO/IEC JTC1/SC29/WG11 MPEG and ITU-T SG16/Q.6 VCEG. This version is based on the same Multiple Type Tree (MTT) software model as proposed in JVET-J0021, with several additional or adapted coding tools and encoder evolutions. This implementation contains most of the tools implemented into the JEM. In addition, one of the main features of the MTT codec is the introduction of new coding unit (CU) topologies on top of QTBT, via two tools: Triple Tree (TT) and Asymmetric Binary Tree (ABT). TT allows splitting a CU of size S in width or height into three rectangular CUs (S/4, S/2, S/4) while ABT allows a recursive splitting of a CU of size S into two non-symmetric rectangular CUs (S/4, 3S/4) or (3S/4, S/4). Both are applicable in the horizontal and vertical dimensions.

In this contribution, only ABT is activated on top of QTBT. The other additional or adapted tools proposed in this response are: multi-reference intra prediction, bi-directional intra prediction, unicity in motion information candidate lists process, extended affine motion compensation, extended template merge modes, generalized OBMC, simplified EMT design, bi-directional illumination compensation, and SAO palette. Fast encoding methods, including a deep-learning based to drive the partitioning in Intra slices, as well as heuristics and caching mechanisms in RD decisions, are reported to offer a wide range of reachable trade-offs between the encoding complexity and the coding gains. For this response the trade-off was set to 90% and 82% of respectively JEM 7.0 encoder and decoder runtimes in SDR constraint set 1. For HDR, pre-/post-dynamic range adaptation, applied in the Y’CbCr 4:2:0 sample domain, is used. A post-filtering refinement is also employed. For 360° video, the encoding is performed on padded ERP (PERP) content, and a normative spatially adaptive quantization is used.

Objective performance is summarized in the tables below.

| SDR | Over HM anchors |  |  |  |  | Over JEM anchors |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate PSNR |  |  | Time variation |  | BD-rate PSNR |  |  | Time variation |  |
|  | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |
| CS1 | -41.9% | -44.8% | -46.1% | 728% | 582% | -13.6% | -3.8% | -5.6% | 90% | 82% |
| CS2 | -33.8% | -51.6% | -52.5% | 1035% | 524% | -12.7% | -16.7% | -18.8% | 119% | 89% |

| HDR | Over HM anchors |  |  |  | Over JEM anchors |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate HDR metrics |  | Time variation |  | BD-rate HDR metrics |  | Time variation |  |
|  | PSNRL | DE100 | Enc | Dec | PSNRL | DE100 | Enc | Dec |
| CS1 | -38.3% | -52.2% | 528% | 604% | -13.3% | -17.0% | 96% | 90% |

| 360 | Over HM anchors |  |  |  |  | Over JEM anchors |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate WS-PSNR |  |  | Time variation |  | BD-rate WS-PSNR |  |  | Time variation |  |
|  | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |
| CS1 | -32.1% | -51.6% | -53.6% | 579% | 519% | -11.7% | -13.1% | -14.1% | 97% | 102% |

# Contents

Abstract	i

Contents	ii

1	Introduction	1

1.1	General	1

1.2	Specific coding tools	1

1.3	Specific encoding adaptations	1

2	Decoder algorithm description	3

2.1	General algorithm description	3

2.1.1	Bitstream structure	3

2.1.2	Entropy decoding	9

2.1.3	Conversion process from entropy decoded data to symbols	9

2.1.4	Decoder-side motion refinement	11

2.1.5	Inverse quantization	11

2.1.6	Inverse transforms	13

2.1.7	Motion compensation	16

2.1.8	Intra prediction	25

2.1.9	In-loop filtering	28

2.2	360° Video	29

2.2.1	Projection format (including padding)	29

2.2.2	Quantization handling	29

2.2.3	360° video specific decoding tools	30

2.3	HDR	31

2.3.1	Post-processing	31

2.3.2	Quantization handling	32

2.3.3	HDR specific decoding tools	32

3	Encoder algorithm description	32

3.1	General algorithm description	32

3.1.1	Rate-distortion optimization	32

3.1.2	ABT and coding tree configuration for Medium Complexity (Random Access)	32

3.1.3	Low-Delay B encoder configuration	34

3.1.4	Alternative High Efficiency Encoder Configuration for Random-Access	35

3.1.5	Alternative Low Complexity Encoder Configuration for Random-Access	36

3.1.6	Intra prediction mode estimation and type selection	38

3.1.7	Motion estimation and motion segment selection	38

3.1.8	Mode decisions	39

3.1.9	In-loop filtering type selection and parameter estimation	39

3.1.10	Transforms and transform type selection	39

3.1.11	Quantization and quantization type selection	39

3.1.12	Entropy coding	41

3.1.13	Additional tool – CNN-based driving of partitioning decisions	41

3.1.14	Additional encoder optimization	43

3.2	360°	43

3.2.1	Usage of projection format adaptation	43

3.2.2	360° Video specific encoding tools	43

3.2.3	Additional encoder optimization	43

3.3	HDR	44

3.3.1	Pre-processing	44

3.3.2	HDR specific encoding tools	45

3.3.3	Additional encoder optimization	45

4	Compression performance	46

4.1	SDR: Constraint set 1 configuration relative to HM anchor	46

4.1.1	Class SDR-A (RD-curves)	46

4.1.2	Class SDR-B (RD-curves)	47

4.1.3	Overall (BD-rate summary tables)	48

4.2	SDR: Constraint set 2 configuration relative to HM anchor	48

4.2.1	Class SDR-B	48

4.2.2	Overall	49

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	49

4.3.1	Class SDR-A	49

4.3.2	Class SDR-B	50

4.3.3	Overall	51

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	51

4.4.1	Class SDR-B	52

4.4.2	Overall	53

4.5	HDR: Constraint set 1 configuration relative to HM anchor	53

4.5.1	Class HDR-A	53

4.5.2	Class HDR-B	53

4.5.3	Overall	54

4.6	HDR: Constraint set 1 configuration relative to JEM anchor	55

4.6.1	Class HDR-A	55

4.6.2	Class HDR-B	55

4.6.3	Overall	56

4.7	360° Video: Constraint set 1 configuration relative to HM anchor	56

4.7.1	Class 360	57

4.7.2	Overall	58

4.8	360° Video: Constraint set 1 configuration relative to JEM anchor	58

4.8.1	Class 360	58

4.8.2	Overall	59

5	Decoder complexity analysis	59

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	59

5.2	Description of computing platform used to determine decoding times reported in section 5.1	59

5.3	Memory usage of decoder	60

5.4	Complexity characteristics of decoder entropy decoding operation	60

5.5	Complexity characteristics of decoder inverse quantization	60

5.6	Complexity characteristics of decoder inverse transform operation	60

5.7	Complexity characteristics of decoder-side motion refinement	62

5.8	Complexity characteristics of decoder motion compensation	62

5.9	Complexity characteristics of decoder intra-frame prediction operation	62

5.10	Complexity characteristics of decoder in-loop filtering operation	63

5.11	Complexity characteristics of additional decoder tools	63

5.12	Complexity characteristics of 360° video specific decoding tools	63

5.13	Complexity characteristics of HDR specific decoding tools	63

5.14	Degree of capability for decoder parallel processing	63

6	Encoder complexity analysis	64

6.1	Encoding time and measurement methodology	64

6.2	Description of computing platform used to determine encoding times reported in section 6.1	64

6.3	Memory usage of encoder	64

6.4	Complexity characteristics of encoder intra-frame prediction type selection	65

6.5	Complexity characteristics of encoder motion estimation and motion segmentation selection	65

6.5.1	General	65

6.5.2	Complexity characteristics of usage of decoder-side motion refinement	65

6.6	Complexity characteristics of encoder transforms and transform type selection	66

6.7	Complexity characteristics of encoder quantization and quantization type selection	66

6.8	Complexity characteristics of encoder in-loop filtering type selection and parameter estimation	66

6.9	Complexity characteristics of encoder entropy coding type selection	66

6.10	Complexity characteristics of additional encoder tools	66

6.11	Complexity characteristics of 360° video specific encoding tools	66

6.12	Complexity characteristics of HDR specific encoding tools	66

6.13	Degree of capability for encoder parallel processing	67

7	Algorithmic characteristics	67

7.1	Random access characteristics	67

7.2	Lower Delay characteristics	67

8	Software implementation description	67

8.1	Software structure	67

8.2	Software statistics and portability	67

9	Closing remarks	69

9.1	General	69

9.2	Proposed new tools or adaptations	69

9.3	Control of the encoding combinatory	69

9.4	Objective performance in the CfP response	69

9.5	Software availability and characteristics	69

10	Patent rights declaration(s)	70

11	Annex 1: Questionnaire	71

11.1	Codec level questions	71

11.2	Tool level questions	72

11.2.1	Partitioning structure	72

11.2.2	Entropy coding	72

11.2.3	Control data derivation process	72

11.2.4	Decoder-side motion refinement	72

11.2.5	Intra prediction	73

11.2.6	Inter prediction	73

11.2.7	Quantization	74

11.2.8	Transforms	74

11.2.9	In-loop filtering	75

11.2.10	“tool-off” and “tool-on” tests	75

12	Annex 2: References	77

# Introduction

## General

This document describes a joint Qualcomm-Technicolor proposal made in response to the Call for Proposal issued jointly by ISO/IEC JTC 1/SC 29/WG 11 (MPEG) and ITU-T SG 16 Question 6. The proposal is based on the same Multiple Type Tree (MTT) software model as proposed in JVET-J0021. The MTT codec implements most of the tools already adopted in the JVET reference software, plus several additional tools. JVET-J0021 describes two versions of this codec named Low and High Complexity while JVET-J0022 describes a Medium Complexity version.

## Specific coding tools

Compared to JVET-J0021, the MTT codec variant proposed in this contribution includes the following additional or adapted coding tools applying for all CfP categories:

- Picture topology:

  - One of the main concepts brought by the MTT codec is the introduction of new CU topologies. Two splitting tools are specified on top of the JEM QTBT splitting modes: Triple Tree (TT) that allows splitting of a CU of size S in width or height into 3 rectangular CUs (S/4, S/2, S/4); Asymmetric Binary Tree (ABT) that allows recursive splitting of a CU into 2 non-symmetric rectangular CUs (S/4, 3S/4) or (3S/4, S/4). TT and ABT are both applicable in the horizontal or vertical dimensions. In the current contribution, only ABT is activated. Some coding tool adaptions for ABT are also introduced, including coding tree configuration, LM/IC, transforms, quantization and entropy coding.

  - Adapted picture border splitting allowing RD optimized binary split on the picture border.

- Intra tools:

  - Multi-reference intra prediction.

  - Bi-directional intra prediction.

- Inter tools:

  - Unicity in motion information candidate lists.

  - Extended template merge modes.

  - Extended affine motion compensation.

  - Template affine mode.

  - Generalized OBMC.

  - Bi-directional illumination compensation.

- Transforms:

  - Reduced pool of primary transforms in EMT.

  - Interaction constraints on EMT/NSST/Skip Transforms.

- Loop filtering:

  - SAO palette.

In the HDR category, dynamic range adaptation as pre-/post-filtering is used as in JVET-J0021. In addition, a post-filtering mapping refinement is included in the current proposal.

As for the 360° video category, the encoding is performed on the padded ERP (PERP) content. A normative spatially adaptive quantization algorithm is used to adapt the bitrate allocation to the equirectangular projection's sampling.

## Specific encoding adaptations

JVET developments, also applicable to MTT, result in the increasing of the coding choices combinatory (e.g. by increasing the partition types) which may result in a noticeable increase of the encoding complexity (run time). In the current proposed software model, fast encoding solutions have been developed to offer a wide range of reachable trade-offs between the encoding complexity and the coding gains. In the CfP bitstreams delivered in February 2018, a medium complexity setting was used. Figure 1 depicts the other MTT trade-offs (low and high complexity) also delivered, corresponding to JVET-J0021.

The nature of the fast encoding methods is of two types.

- Different encoding solutions have been employed for controlling the encoding complexity. Strategies for adapting the combinatory allocation to the slice type and the temporal level of the picture are implemented. To reduce redundant computations, caching mechanisms are used. The testing combinatory is also reduced by using various heuristics in the RD-cost based decisions. The fast encoding methods used to reach the medium encoder complexity are depicted in section 3.

- A deep-learning based prediction of split modes in Intra slices has been developed. The algorithm can be controlled to reach different encoding complexity/coding gains trade-offs. Typically, this solution allows saving around 30% of the intra encoding runtime without BD rate penalty. The deep-learning based prediction of Intra CU split mode is depicted in 3.1.13 and described in more details in JVET-J0034.

Figure 1. Different MTT encoding complexity/BDR gain trade-offs.

# Decoder algorithm description

## General algorithm description

### Bitstream structure

#### General

The bitstream structure is similar to the structure used in HEVC or in the JEM. The same concept of NAL Units, that can be of various types, such as VPS, SPS, PPS, slice and slice segment, is defined. A slice segment is made of a header and of data representative of successive coding tree units (CTUs).

In the proposed response, the CTU size is 256x256 (in luma samples). A CTU is represented according to a coding tree, which is depicted in the following.

In the JEM, QTBT introduces flexibility in the Coding Unit (CU) structure to divide the picture in square or rectangular blocks (see illustration in Figure 2). CUs are no more sub-divided into Prediction Units and/or Transform Units, as was the case in HEVC, then until JEM2.

Figure 2. Quad-Tree plus Binary-Tree (QTBT) CTU representation adopted from JVET-C0024.

The coding tree used in this response extends the coding tree representation of QTBT, through an extended set of available CU sizes and shapes.

Four new Binary Tree splitting modes were introduced into the QTBT framework, so as to allow new splitting configurations. So-called Asymmetric Binary Tree split modes (ABT) are employed in addition to the splitting modes already available in QTBT, as shown by Figure 3.

According to the added splitting modes, a coding unit with size S is divided into 2 sub-CU with sizes S/4 and 3.S/4, either in the horizontal or in the vertical direction. In practice the added available CU sizes are 6, 12, 24 and 48.

Overall, a CTU can be divided into square or rectangle CUs, according to various split options, as illustrated in Figure 3: quadtree (QT), symmetric binary tree (BT), and a new type of split, asymmetric binary tree (ABT).

Figure 3. Split modes (tree types) supported in the proposed codec.

A typical picture representation obtained with this QT+BT+ABT representation is illustrated on Figure 4.

Figure 4. Example of picture block division obtained with the QT+BT+ABT coding tree used in this response.

The syntax to signal the coding structure is of two types.

First some parameters signaled in the slice header enable controlling the maximum depths, respectively of the quad-tree and the binary-tree used to represent the coded CTUs.

Next, on the CTU level, the coding tree is signaled as illustrated in Figure 5. The coding tree is made of two parts: the coding quad-tree and then the coding binary-tree. The coding quadtree employs a flag to signal if a current CU is split in a quad-tree fashion or not.

The coding binary-tree syntax arrangement aims at coding a binary tree type (or split mode), which can be equal to NO_SPLIT, HOR, VER, HOR_UP, HOR_DOWN, VER_LEFT or VER_RIGHT. It employs the syntax elements which correspond to the nodes of the coding binary-tree shown in Figure 5.

Figure 5. Overall syntax arrangement to signal the coding tree.

An example of block partitioning using QT (dark lines), symmetric BT (yellow lines) and asymmetric BT (green lines), is shown in Figure 6.

Figure 6. Example of block partitioning by using QT, symmetric and asymmetric BT.

The ABT tool impacts other parts of the codec design, which need to accommodate to new block sizes of shape 3×2n,n≥1. These include transform coefficient entropy coding (section 2.1.3.4), scaling factors in the transform/quantization process (section 2.1.5), separable primary transform sets (section 2.1.6.3), LM/IC parameters derivation (section 2.1.7.7).

#### Block size adaptive spatial partitioning

A CU resulting from an asymmetric splitting may be further split in a recursive way, as shown on Figure 7. This includes binary symmetric split, asymmetric binary split along the direction which size is a power of two, and optionally asymmetric binary split in the orientation where the CU dimension is a multiple of 3. In the latest case, the divided CU horizontal or vertical sizes are respectively equal to S/3 and 2.S/3, where S is the parent CU horizontal or vertical size.

Figure 7. Spatial splitting optionally adaptive to the parent CU Size.

#### Non-redundancy between successions of splits

As the splitting process is recursive and given the multiple supported CU split modes, several recursive splitting paths may lead to the same partitioning of a CTU or of a part of a CTU.

Some normative restrictions are introduced to ensure that any topology in the representation of a CTU can be reached through at most one series of quad-tree plus binary-tree recursive splitting steps.

These restrictions consist in forbidding some splitting modes for a CU, given the split modes associated to the parent CU (Figure 8) or the neighboring sub-CU already processed in the same parent CU (Figure 9).

- First, no succession of 2 asymmetric binary splits can be used, if it produces a topology than can be obtained with the binary tree (see Figure 8). The non-desired successions of binary tree are forbidden in a normative way, i.e. all syntax elements that lead to a non-admissible case are omitted from the bitstream, and their value is inferred by the decoder.

Figure 8. Restriction to avoid redundancies between asymmetric and symmetric binary trees.

- Second, another type of normative restriction extends the concept of anti-emulation already present in JEM7, which prevents successive symmetric binary tree splits from emulating the quad-tree. This extension is shown on Figure 9. The basic principle is to forbid some horizontal Triple Tree or Asymmetric Binary Tree splitting of the second sub-CU of a parent CU, when the parent CU has undergone a vertical TT or binary split. If a triple or binary split mode is assigned to the first sub-CU, then the same split mode cannot be given to the second sub-CU. This prevents from producing topologies that can be reached through a horizontal triple/binary split of the parent CU, followed by two identical vertical splits of its sub-CUs.

Figure 9. Restriction of split modes allowed for a CU, given the split mode of the parent CU and of the neighboring sub-CU already processed in the considered parent CU.

#### Block partition on picture border

This section describes the block partitioning performed in the proposed solution, in CUs located near the bottom and right borders of the coded picture.

First, the HEVC-like process is illustrated on Figure 10. Red lines represent the quad-tree splitting and blue lines represent the binary tree splitting that is embedded in each quad-tree leaf.

In HEVC, if at current CU depth level, the CU size is such that a part of the CU is spatially outside the picture, then a quad-tree splitting of the considered CU is inferred.  Furthermore, in HEVC, if the minimum CU size is 8x8, and the size of the frame in one dimension is a multiple of 4 and not a multiple of 8, a CU may be split down to 4x4 to spatially match the border of the frame.

Figure 10. HEVC-like quad-tree splitting on the picture border.

The CU splitting rules on the picture bottom and right borders are modified here, according to the two following aspects:

- If a part of a given Coding Tree node (CU) is partially located outside the picture, then the binary symmetric splitting of the CU is always allowed, along the concerned border direction (horizontal split orientation along bottom border, vertical split orientation along right border). If the bottom-right corner of the current CU is outside the frame, then only the quad-tree splitting of the CU is allowed. In addition, if the current binary tree depth is greater than the maximum binary tree depth and current CU is on the frame border, then the binary split is enabled to ensure the frame border is reached.

- The asymmetric binary splitting is allowed, if the splitting line (boundary between the two sub-CU resulting from the split) exactly matches the border of the frame. With respect to the triple tree splitting process, the triple tree split is allowed in case the first or the second boundary between resulting sub-CU exactly lies on the border of the picture.

A typical picture block partitioning resulting from this new block partitioning policy on the picture borders is illustrated on Figure 11.

Figure 11. Block partitioning on the picture border, in the proposed codec.

### Entropy decoding

The same arithmetic decoding process as used in JVET-J0021 is applied.

### Conversion process from entropy decoded data to symbols

#### Intra prediction parameters

Figure 12 and Figure 13 provide an overview of the CU coding tree for intra syntax signaling, for luma and chroma respectively. In the current response, a new intra prediction mode, bi-directional intra prediction (BDIP) mode, is introduced. As shown in Figure 12, a corresponding CU-level flag is inserted in the coding tree, and one related context is added. The remaining intra prediction parameters are the same as in JVET-J0021.

Figure 12. Coding tree of intra modes (luma part). ”*” indicates the changes made in this proposal.

Figure 13. Coding tree of intra modes (chroma part).

#### Motion parameters

Figure 14 provides an overview of the CU coding tree for inter coding tools signaling and usage. The figure indicates when a tool requires signaling (blue ellipses) or when its activation/de-activation is inferred or inherited from a predictor (yellow/green ellipses).

ic

ic

mode

merge

dir

affine

Mv info

mvd

imv

1

0

obmc

ic

Mv info

mvd

obmc

ic

intra

x

1

0

0

0

0

1

0

0

AMVP

bio

bio

bio

bio

0

1

bio

bio

1

0

0

1

Figure 14. Coding tree of inter modes.

Compared to JVET-J0021, the proposed solution introduces a new template-based merge prediction mode called “template affine”, presented in 2.1.7. Thus, template-based merge inter prediction, also called template merge, includes two modes: template matching and template affine. Template matching was called “FRUC” or “Pattern matched motion vector derivation” in the JEM.

Two bins are context-based coded. They form the templateMergeMode syntax element. First bin indicates the use of template merge or classical merge. In case of template merge, the second bin indicates the use of template affine or template matching.

#### Quantization parameters

Quantization parameters usage is not modified compared to the JEM.

#### Transform coefficients

In this response, no additional context is introduced to the entropy decoding of transform coefficients, compared to contribution JVET-J0021. However, some contexts in the transform coefficients decoding depend on the size of the blocks.

To handle newly introduced block sizes 6, 12, 24 and 48 with the ABT tool, these contexts are shared between the new block sizes and block sizes equal to a power of 2.

Let us note c the block size equal to a multiple of three in width or height, i.e. of the form c=3×2n. The syntax elements that use block size dependent contexts are the following ones.

- The LastSignificantXY syntax element coding for block size c, employs the context that would normally be used for the smallest block size equal to a power of two, i.e. 43×c=2n+2. Apart from this context sharing between different block sizes, the coding of LastSignificantXY is identical to that of JVET-J0021.

- In the JEM and in JVET-J0021, the significant coefficient flag syntax element employs a set of contexts indexed according to the block size. This set is noted baseCtx. The context actually used for the decoding of one coefficient’s significant flag is baseCtxctxIndex. ctxIndex is computed according to the position of the coefficient in its block, and the significance of already processed surrounding coefficients. In JVET-J0021, the used set of contexts baseCtx is selected as a function of the block size represented by:

uiLog2BlockSize=log2(width)+log2(height)≫1

Therefore, for block sizes of the form 3×2n in width or height, the closest higher size equal to a power of 2 is considered when choosing the context to decode the significant coefficient flag.

#### In-loop filter parameters

The Sample Adaptive Offset (SAO) post-filtering introduces 3 additional syntax elements: two slice-level syntax elements (slice_sao_size_id, sao_palette_num_cand) and one CU-level element (sao_palette_idx). They are described in section 2.1.9.

### Decoder-side motion refinement

The current response modifies the template matching tool and introduces a new coding mode called template affine. As indicated in section 2.1.3.2, these tools are part of the template merge mode. Both tools perform decoder-side motion vector refinement, but, for clarity matters, their description is integrated to the motion compensation part 2.1.7.

### Inverse quantization

This section explains some adaptations of the inverse quantization needed for the new block sizes introduced in the ABT. It also explains how it is implemented in practice.

With the Asymmetric Binary Tree, blocks may have sizes equal to 3×2n in width and/or height. This leads to an overall block surface potentially equal to 3×2n×2m=3×2n+m, or 3×2n×3×2n=9×2n+m. To properly handle these cases, proper post-inverse-quantization scale factors are required. Their derivation is explained below.

Figure 15 depicts the inverse transform and inverse quantization process, for any rectangular block with generic size noted (s1,s2). The successively scaling operations applied during this overall process are the following ones.

- 26: scale factor applied during the inverse quantization stage

- SIQ: scale factor applied after the inverse quantization stage

- s1×26 : scale factor used during the first (horizontal) inverse transform stage

- SIT1: scale factor applied after the first inverse transform stage

- s2×26 : scale factor applied during the second (vertical) inverse transform stage

- SIT2: scale factor applied after the second inverse transform stage.

Figure 15. Inverse Transform and inverse quantization steps, with associated respective scaling factors used for the HEVC-like integer-based implementation, for a generic rectangular transform block with size (s1,s2).

Table 1 provides the values of each involved scale factor and the overall scale factor resulting from the product of all scale factors applied in the inverse 2D transform. It is equal to s1s2×2-(15-B).

Table 1. Value of each scaling factor in the HEVC-like inverse transform process, with generic rectangular blocks, where (s1,s2) is the size of the rectangular transform block and B is the signal bit-depth.

|  | Scale factor |
| --- | --- |
| First inverse transform stage (Sit) | s1.26 |
| After the first inverse transform stage (SIT1) | 2-7 |
| Second inverse transform stage (S’it) | s2.26 |
| After the second inverse transform stage (SIT2) | 2-(20-B) |
| Total scaling for the 2D inverse transform | s1s2.2-(15-B) |

To ensure the preservation of the residual norm for a quantization step Qstep=1 i.e. QP=4, a post-inverse quantization scaling factor SIQ is needed such that SIQ×g4×s1s2×2-(15-B)=1, where g4=64 (see  REF _Ref509212173 \h  \* MERGEFORMAT [2]). This leads to:

SIQ=1s1s2 ×2-(B-9)

To handle the cases where s1 or s2 is equal to a multiple of 3, the chosen implementation of the overall process consists in integrating the factor 4/3 into the series of scaling factors of Table 1 as follows.

- In case s1 is not a power of 2, then:

  - the scaling factor Sit is computed as: Sit=43s1∙26

  - this results in an overall scaling by 43 along the concerned orientation

- In case s2 is not a power of 2, then:

  - the scaling factor S'it is computed as: S'it=43s2∙26

  - this results in an overall scaling by 43 along the concerned orientation

This way, the overall inverse transform and inverse quantization process for the newly introduced block sizes is performed with multiplication and bit shift operations.

### Inverse transforms

#### Reduced pool of primary transforms in EMT

As in JVET-J0021, this solution utilizes the same signaling for the EMT flag as well as EMT index. However, the set of transforms is limited to three additional transforms, namely {DCT-VIII, DST-VII,  DST-IV}, beside the core DCT-II transform (Table 2 for mathematical definition). These transforms have the following properties:

- DST-VII is the optimal transform for intra predicted residual along the direction of prediction, when considering an autoregressive model of the residual signal.

- DST-IV has similar characteristics to DST-VII, where its lowest frequency basis function is increasing. Its inclusion adds additional flexibility for compacting the residual signal.

- DCT-II is included in the EMT set, as it represents the optimal transform for highly correlated signal.

- DCT-VIII is also included, as its basis function is time-reverse of the DST-VII. This transform suits other types of residual not covered by the rest of EMT set.

Additionally, as it can be mathematically proven the DST-VII is obtained by reversing the DCT-VIII basis functions, and negating the odd indexed coefficients, DST-VII is computed from the existing architecture/algorithm of DCT-VIII. Mathematically, this can be expressed as:

TDST-VII=S×TDCT-VIII×R

where TDST-VII and  TDCT-VIII are the transform matrices of DST-VII and DCT-VIII (resp.), and S and R are sign inverse and input reversal matrices, defined for i,j∈(0,N-1), N being the transform size, as:

Si,j=-1i, i=j0, else

R(i,j)= 1, i=N-1-j0, else

The interest behind keeping DCT-VIII, rather than DST-VII, is that this transform is self-inverse. In other words, the same architecture/algorithm can be used for the forward and inverse transform.

Table 2. Set of primary core transform used in this response.

| Transform Type | Basis function Ti(j), i, j=0 to N−1 |
| --- | --- |
| DCT-II | Ti(j)=ω0∙2N∙cosπ∙i∙2j+12N where ω0=2Ni=01i≠0 |
| DCT-VIII | Ti(j)=42N+1∙cosπ∙(2i+1)∙(2j+1)4N+2 |
| DST-IV | Ti(j)=2N∙sinπ∙(2i+1)∙(2j+1)4N |
| DST-VII | Ti(j)=42N+1∙sinπ∙(2i+1)∙(j+1)2N+1 |

The selection of the transform pair performed at the encoder, where four possible combinations, indexed by syntax element TrIdx, are tested, works in the exact same way as in JVET-J0021.

For intra-frame coding, the correspondence between the TrIdx value and the pair of used horizontal and vertical transforms is implemented by the following pseudo-code:

[VerTr, HorTr] = GetTransformPair(TrWidth, TrHeight, IntraPredMod, TrIdx)

SizeIdx = min(3, min(⌈log2(TrWidth)⌉ - 2 , ⌈log2(TrHeight)⌉ - 2)

IntraPredMod = IntraPredMod > DiagMode ? (nModes - IntraPredMod) : IntraPredMod

PredModeIdx = MapArray[ SizeIdx ][ IntraPredMod  ]

return TrSet[ PredModeIdx ][ TrIdx ]

where DiagMod is equal to 66, MapArray is defined in Table 3 and TrSet is defined in Table 4.

Table 3. MapArray value depending on the parameters sizeIdx and IntraPredMode.

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sizeIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 1 | 1 | 2 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 2 | 1 | 2 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 5 | 5 | 5 | 5 | 5 |
| 3 | 1 | 2 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| IntraPredMode | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 |
| sizeIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 1 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 |
| 2 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| 3 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| IntraPredMode | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 |  |  |
| sizeIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |  |  |
| 1 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |
| 2 | 5 | 5 | 5 | 5 | 5 | 5 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |  |  |
| 3 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |  |  |

Finally, TrSet is defined as follows, which differs from JVET-J0021 due to the reduced set of transforms:

Table 4. Set of 2D transforms used by decoding process (TrSet) depending on the parameters predModeIdx and TrIdx.

| TrIdx predModIdx | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| 0 | DST4,DST4 | DST7,DST7 | DST4,DCT8 | DCT8,DST4 |
| 1 | DST7,DST7 | DST7,DCT2 | DCT2,DST7 | DCT2,DCT8 |
| 2 | DST7,DST7 | DST7,DCT8 | DCT8,DST7 | DCT2,DST7 |
| 3 | DST4,DST4 | DST4,DCT2 | DCT8,DST4 | DCT2,DST7 |
| 4 | DST4,DST7 | DST7,DCT2 | DCT8,DST7 | DCT2,DST7 |
| 5 | DST7,DST7 | DST7,DCT2 | DCT8,DST7 | DCT2,DST7 |
| 6 | DST7,DST7 | DST7,DCT2 | DCT2,DST7 | DCT2,DST7 |

On the other hand, the selection of transform pair in inter coding is the same as in JVET, where the same transforms in EMT are used (DST-VII and DCT-VIII).

#### Interaction between the Primary and Secondary Transform

This section presents the tool called “EMT-NSST interaction”. Its basic idea consists in limiting the mutual usage of the primary and secondary transforms, based on the two following observations.

- In the JEM, the transform skip flag is signaled whatever the primary transform mode. This appears to be useless, given that the CU-level flag that signals the use of EMT is known before parsing the Transform Skip flag.

- The EMT and NSST combined usage is statistically very low when dealing with EMT indices higher than 0 in the considered codec.

The EMT-NSST interaction tool brings the following modifications to the decoding process.

- The Transform Skip flag is not decoded and is inferred to 0 when the CU-level EMT flag is 1.

- Depending on a configuration index interactionEMT_NSST decoded from the SPS, NSST activation depends on EMT:

- If interactionEMT_NSST is equal to 3

  - If the EMT CU-level flag is equal to 1 and the EMT transform index is different from 0, then the NSST index is inferred to zero on the decoder side.

- Otherwise, if interactionEMT_NSST is equal to 1

  - If the EMT CU-level flag is equal to 1, then the NSST index is inferred to zero on the decoder side.

- In all other cases, NSST index is decoded as usual. The presence of the NSST index does not depend on the primary transform used for the considered CU.

#### Enriched set of transforms to support the Asymmetric Binary Tree

The set of 1D transforms specified in section 2.1.6.1 is enriched, to support additional block sizes 6, 12, 24 and 48. For these new block sizes, the same transforms as on Table 4 are used. Moreover, the same EMT and NSST processes as for other block sizes are used.

### Motion compensation

The motion compensation is described relatively to the JEM in this section.

#### Overview

This sub-section provides an overview of the motion compensation tools contained in this proposal.

Figure 16 depicts the spatial position of potential spatial/temporal motion information predictors, which are referred to in the following tables. Each table corresponds to one branch of the inter coding tree in Figure 14 (one single table is used for the two “template merge” branches).

Table 5 to Table 9 summarize the motion vector coding processes for the different considered inter modes. The item "Motion information predictor composition" indicates the type of motion information stored in MV prediction candidate lists. The composition of a candidate list is summarized in "Candidate list". “Signaling” indicates the actual motion-related coded data.

Figure 16. Positions of spatial candidates of motion information.

Table 5. AMVP mode characteristics.

|  | JEM | Proposed |
| --- | --- | --- |
| Motion information predictor composition | inherited MV | inherited MV |
| Candidate list | 2 candidates, among: - two spatial positions among: left {a0, a1} and top {b0, b1, b2} - a temporal position {c} - a template matching refined candidate - zero MVs | 2 candidates, among: - two spatial positions among: left {a0, a1} and top {b0, b1, b2} - a temporal position {c} - a template matching refined candidate - zero MVs |
| Signaling | For each reference picture: - reference picture idx - predictor idx - MVD IMV flag IC flag (OBMC flag) Residual is coded | For each reference picture: - reference picture idx - predictor idx - MVD IMV flag IC flag (OBMC flag) Residual is coded |

Table 6. Affine AMVP mode characteristics.

|  | JEM | Proposed |
| --- | --- | --- |
| Motion information predictor composition | inherited MV triplet | inherited MV triplet |
| Candidate list | 2 candidates, among: - constructed affine models from spatial neighbors - classical AMVP candidates | 2 candidates, among: - spatial Affine positions {b1, a1, a0, b0, b2} - a template affine refined candidate - improved constructed affine models from spatial neighbors - classical AMVP candidates - zero MVs |
| Signaling | For each reference picture: - reference picture idx - predictor idx - 2 MVDs Affine flag Residual is coded | For each reference picture: - reference picture idx - predictor idx - 2 MVDs Affine flag Residual is coded |

Table 7. Merge mode characteristics.

|  | JEM | Proposed |
| --- | --- | --- |
| Motion information predictor composition | - inherited reference picture index - inherited associated MV - inherited associated IC flag | - inherited reference picture index - inherited associated MV (triplet) - inherited associated IC flag - inferred affine flag |
| Candidate list | 7 candidates, among: - spatial positions in {a1, b1, b0, a0} - ATMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - STMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - spatial positions in {b2} - a temporal position {c} - bi-directional combinations - zero MV for each reference picture | 19 candidates with up to 8 Affine ones, among: - spatial positions in {a1, b1, b0, a0} - spatial affine positions in {a1, b1, b0, a0} - ATMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - STMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - constructed affine candidate based on spatial non-affine positions - spatial positions in {b2} - spatial affine positions in {b2, a2, b3} - a temporal position {c} - temporal affine positions in {curr, c} - non-adjacent spatial positions - non-adjacent spatial affine positions - bi-directional combinations - zero MV for each reference picture |
| Signaling | For each reference picture: - predictor idx Residual is coded if mode != skip | For each reference picture: - predictor idx Residual is coded if mode != skip |

Table 8. Affine Merge mode characteristics.

|  | JEM | Proposed |
| --- | --- | --- |
| Motion information predictor composition | - inherited reference picture index - inherited associated MV triplet | N/A (in Merge) |
| Candidate list | 1 candidate, among: - spatial affine positions in {a1, b1, b0, a0, b2} | N/A (in Merge) |
| Signaling | Affine flag Residual is coded if mode != skip | N/A (in Merge) |

Table 9. Template modes characteristics.

|  | JEM | Proposed |
| --- | --- | --- |
| Motion information predictor composition | - inherited reference picture index - inherited associated MV | - inherited reference picture index - inherited associated MV |
| Candidate list | Up to 13-15 candidates, among: - AMVP candidates if current CU is in AMVP mode - merge candidates - template matching unilateral candidates - spatial neighboring MVs in {b1, a1} For sub-blocks, up to 15 candidates, among: - the best candidate of the corresponding entire CU - spatial neighboring motion vectors in {b1, a1, b2, b0} - scaled versions of co-located motion vectors - up to 4 ATMVP candidates - up to 4 STMVP candidates | Template matching AMVP Up to 2 candidates, among: - AMVP candidates - merge candidates (non-affine) - spatial neighboring MVs in {b1, a1} |
|  |  | Template matching merge Up to 4 candidates, among: - merge candidates (non-affine) No more sub-blocks |
|  |  | Template Affine (AMVP and merge) Up to 2 candidates, among: - merge candidates (non-affine) - spatial neighboring MVs in {b1, a1} |
| Signaling | Template Merge mode (1 bin) Residual is coded if mode != skip | Template Merge mode (2 bin) Residual is coded if mode != skip |

#### Unicity in Motion information candidates lists

All the inter coding modes need to apply a process for generating the lists of candidate motion information. In the current response, these processes are homogenized and designed to ensure the full diversity and unicity of candidates.

The first generic aspect consists in unicity checking before adding any new candidate to the list. If the new candidate is already present in the list, then it is not added. This check is performed for all inter coding modes: AMVP, affine AMVP, merge/skip, template matching merge and template affine merge.

In some inter coding modes, other processes also improve the diversity of the lists, as explained below.

- In AMVP, the unicity check is also ported into the spatial candidate collection which constructs temporary candidate list. Furthermore, in AMVP and affine AMVP, when rounding and/or clipping operations are performed, these operations are achieved as soon as a new candidate is reached. The rounded and/or clipped candidate is checked before being added or not into the list.

- In Merge, whereas in the JEM only the spatial candidate redundancy was checked, the unicity check is applied to all candidate types except the ATMVP and STMVP ones.

- Candidates used to build the Template Matching and Template Affine lists are the same as the ones used to build the merge list. The main difference with the merge list derivation is that, during the unicity check, the IC flag of those candidates is not considered, because it is not part of the derived/predicted motion information in these modes.

#### Template Merge modes

This section presents the modifications brought to the template merge modes. The coding results for these modifications correspond to the “Template Merge” category of the tool-off/tool-on tests in section 11.2.10.

##### Extended Template Matching

In the proposed solution, the template matching mode is slightly different from the one described in JVET-J0021.

First, the candidates, called template matching unilateral motion vectors, are completely removed. This reduces complexity at the encoder and decoder sides.

Then, in AMVP, instead of limiting the studied candidates to the AMVP ones, other candidates as the merge ones and the neighboring spatial candidates are considered. The number of candidates in that case is restricted to two in order to limit the complexity impact.

##### Template Matching and Bilateral Matching refinement

In template matching, when the refinement is performed with a diamond pattern, all directions are checked at the first step, then only a subset is explored in the following steps. This subset is only composed of the search directions selected in the previous step. For example, if at the first step the pure horizontal right direction is selected, then at the next step only directions to the right are considered (all the left ones are not considered). In bilateral matching refinement, as the search pattern is a square, the same approach can be used instead of testing all the directions at each step.

#### Extended Affine Motion Compensation

This section presents the modifications brought to affine and template affine prediction modes. The coding results of these tools correspond to the “Affine MC” category of the tool-off/tool-on tests in section 11.2.10.

##### Affine Motion Compensation adaptations

The affine motion compensation (and estimation) process is the same as in JVET-J0021. The differences introduced in the proposed solution stand in the way the affine CUs are represented, and in the candidate collection.

An affine CU is defined by its associated affine model composed of three motion vectors, called the affine model seeds, as the motion vectors from the top-left, top-right and bottom-left corners of the CU (v0, v1 and v2 in Figure 17). This affine model allows calculating the affine motion vector field within the CU, performed on a 4x4 sub-block basis (black motion vectors on Figure 17). In the JEM, these seeds are stored in the top-left, top-right and bottom-left 4x4 sub-blocks in the considered CU. In the proposed solution, the affine model seeds are stored separately as a motion information associated to the whole CU (like the IC flag, for example). The motion model is thus decoupled from the motion vectors used for actual motion compensation at the 4x4 block level. This storage allows preserving the complete motion vector field at the 4x4 sub-block level. It also allows using affine motion compensation for block of size 4 in width or height.

Figure 17. 4x4 sub-block based affine motion vector field and the associated affine model seeds.

In the merge process, a candidate list is created as presented in Table 7. The list contains classical merge candidates as well as affine ones. In the proposed solution, the affine candidates are enriched with new ones. The spatial affine candidates set in JEM {a1, b1, a0, b0, b2} becomes {a1, b1, b0, a0, b2, a2, b3} in this contribution.

Furthermore, in the JEM, affine does not benefit from the temporal candidates introduced in the proposed solution. To that aim, every available reference picture of each list is explored at the current CU position {curr} and at the bottom-right position {c} to get affine model seeds. When a temporal candidate was coded with an affine mode, its associated affine model seeds are retrieved. If the corresponding temporal reference picture is also available for the current CU in same reference picture list, then the current CU inherits the reference picture index of the temporal candidate. Otherwise the first reference picture in the considered list is chosen for current CU. As for classical temporal candidates, the retrieved affine model seeds are scaled according to the temporal distances between pictures and their reference pictures. The same process as for spatial candidates is applied, i.e. the current affine model seeds are derived from the scaled temporal candidate seeds according to the following equations (also used in JEM).

  - v0=v3+v5-v3Ycurr-YneighbHneighb+v4-v3Xcurr-XneighbWneighb

  - v1=v0+(v4-v3)WcurrWneighb

  - v2=v0+(v5-v3)HcurrHneighb

Equation 1. Affine model seeds derivation process from CU to CU.

This affine model seeds derivation from CU to CU is illustrated on Figure 18.

Figure 18. Affine model derivation starting from a neighboring CU to a current CU.

##### Template Affine mode

Template affine is a new coding mode introduced in the proposed solution compared to JEM. The tool is similar to template matching but dedicated to affine. It allows automatically finding an affine model for the currently processed CU and refining it, even if surrounding CUs are not in affine mode.

Template affine mode can only be used by CUs whose left and top neighbors are both available. The process is as follows.

- First, by using the template matching process, for each reference picture list, a motion vector with its associated reference picture is derived and refined automatically.

- Then a process close to the merge candidate list collection, but dedicated to affine, allows deriving affine model seeds for the current CU using Equation 1. The first spatial affine neighbor found in {a1, b1, b0, a0, b2, a2, b3} is chosen.

- The last step consists in a template-based refinement process of the derived affine seeds. For each reference picture list, if an affine model was found in the second step, it is set as the initial affine model to be refined for the current CU. Otherwise the template matching motion vector found in the first step is used as the initial affine model. The template used in this mode is a strip of 4 pixels on the top and left of the current CU including the top-left corner as shown in Figure 19. Then the affine model of the current template is derived with Equation 1, based on the model of the current CU. The obtained affine model seeds allow calculating the motion vectors of each 4x4 sub-block of the template and so getting the corresponding template in the reference picture as represented on Figure 19. The template matching cost associated to this affine model is the SAD between the template in the reference picture and the template around current CU. The refinement process of the affine model seeds is performed with a gradient descent search. This search is similar to the one performed in the motion estimation of the affine AMVP mode, but is based on the template instead of the current CU. It calculates, at each step, deltas to be added to each seed of the affine model. The refinement is stopped after 5 iterations, and the affine model with the smallest template matching cost is selected as the best one.

Figure 19. Templates used in Template Affine mode.

- As in template matching, if both reference picture lists are used, then a joint refinement between lists is performed. The list whose affine model reaches the smallest template matching cost is selected as the best one. The affine model in the other list is the one to be jointly refined. To that aim, the current template is modified using the best template and becomes:

T'curr=2* Tcurr- Tbest

The affine model of the second list is then further refined with 5 iterations using this new template. If the template matching cost associated to this refined affine model is not sufficiently low, then the motion is set unavailable for this list.

Template affine is used in affine AMVP mode to derive a candidate while template matching is used in AMVP mode (see Table 5 and Table 6). A candidate can be derived only if there are less than 2 candidates already found in the spatial neighborhood. The derived candidate is inserted in the top of the affine AMVP candidate list.

It is also used in merge as a separate coding mode (see Table 7), with a similar process as for the template matching in the JEM. If the decoded template merge mode indicates the use of template affine mode, then a best refined affine model is derived per list and used to reconstruct the CU.

Spatial propagation of affine seeds is applied between CUs, regardless they are coded in affine merge mode or template affine mode.

#### Generalized OBMC

The OBMC process is carried out with 4x4 sub-blocks. In the proposed solution, it has been unified so that the decision to filter 2 or 4 lines/columns of pixels is no more taken a-priori depending on the CU coding mode, but on-the-fly depending on motion vectors located at opposite 4x4 sub-block borders.

Instead of performing OBMC successively on left, above, right then bottom sub-block borders, these borders are considered by pairs: horizontal orientation (left, right) and vertical orientation (top, bottom). For each 4x4 sub-block, motion vectors corresponding to the pair of borders in the considered orientation are retrieved. If both are available and different from the motion vector of the current 4x4 sub-block, then OBMC filtering is performed on 2 lines/columns of pixels from each orientation. If only one is available or if only one is different from the motion vector of the current sub-block, OBMC filtering is achieved on 4 lines/columns of pixels on the border where the motion vector is different. If none is available or both motion vectors are equal to the current sub-block motion vector, no filtering is applied.

This proposed unified OBMC allows filtering a different number of samples for horizontal and vertical directions and having a better adaptation to motion transitions within a CU, in particular with merge ATMVP CUs as illustrated in Figure 20.

Figure 20. a) prior art OBMC and b) proposed unified OBMC applied on a 32x16 inter CU in Merge ATMVP mode
(dashed lines denote the splits retrieved from ATMVP, i.e. the motion transitions).

Furthermore, the OBMC process in the JEM limits filtering to 2 lines/columns for small CUs whose surface is lower than 64 samples. In the proposed unified OBMC, since the filtering size is decoupled between orientations, this limitation has been adapted. If the width of the CU is less than 8, only 2 columns of pixels are filtered. If the height is less than 8, only 2 lines of pixels are filtered. This allows a better adaptation of the OBMC filtering for thin CUs as shown in Figure 21.

Figure 21. OBMC applied on a 32x4 inter CU a) as achieved in JEM and b) as in proposed unified OBMC.

OBMC design simplification

The proposed generalized OBMC design is further lightened by removing IC parameters computation for each OBMC 4xS band. Instead the IC parameters are inherited from the considered neighboring 4x4 sub-blocks (Figure 22). For this purpose, the IC parameters of the current picture are stored with the motion field information at a 4x4 sub-block resolution.

Figure 22. OBMC process can inherit the IC parameters from the neighboring sub-blocks.

#### Bi-directional Illumination Compensation tool

Illumination Compensation (IC) allows correcting block prediction samples (SMC) obtained via motion compensation by possibly considering the spatial or temporal local illumination variation.

SIC = ai . SMC + bi

In uni-directional prediction, when IC is enabled for the current CU, the IC parameters (ai,bi) are estimated by comparing the reconstructed neighboring samples (L-shape-cur in Figure 23) with the neighboring samples (L-shape-ref-i) of the reference-i block (i=0 or 1). The IC parameters minimize the difference between the samples in the L-shape-cur and the samples of the L-shape-ref-i corrected with IC parameters as depicted in Figure 23.

Figure 23. Uni-directional IC parameters are estimated on the L-shape in grey.

In case of bi-prediction, the illumination compensation process for deriving the IC parameters is modified so that the parameters are estimated using the samples of the two reference CUs samples as depicted in Figure 24. First, the IC parameters (a,b) are estimated between the two reference blocks, next the IC parameters (ai,bi)i=0,1 between reference and current blocks are derived as follows:

a0=a.α+1-αb0= α.ba1= α+(1- α)/ab1= -b(1- α)/a		where: α=poccur-poc0poc1-poc0

Figure 24. Bi-directional IC parameters are estimated on the full reference CUs in grey.

If the CU size is smaller or equal to 8 in width or height, the IC parameters are estimated using the uni-directional process. When CU size is greater than 8, the choice between using IC parameters derived from bi-directional or uni-directional process is made by selecting the IC parameters which minimize the difference of the average of the IC compensated reference blocks. The bi-directional optical flow (BIO) is enabled with bi-directional illumination compensation tool.

#### IC parameters derivation for ABT related block sizes (also used in LM intra mode)

This section describes how the IC and LM prediction parameters (a,b) mentioned in previous section are derived, in the case of a block size not equal to a power of 2. The JEM parameters derivation employs a lest mean square method to determine (a,b), which solves the following: 
ai,bi=argmin(a,b)r∈L-shape-cur,s∈L-shape-ref0(MV)rec_cur(r)-a.rec_ref(s)-b2

In JEM, the least mean square process proceeds on a sub-sampled neighborhood. The same subsampling step is used along horizontal and vertical directions. This step is set equal to the minimum power of two, such that a bounded number of samples are considered along the smallest block dimension. Then, it is adjusted for each dimension, such that the same number of samples are considered in width and height.

The modified process consists in the following. For each dimension, an initial sub-sampling step size is chosen as follows.

- If the size is a power of two, the step size is set to the minimum power of 2 such that an upper-bounded number of samples are considered along the concerned direction.

- If the size if not a power of two, the step size is set to the minimum value of the form 3×2n, such that an upper-bounded number of samples are considered along the concerned direction.

Then, the two steps are adjusted such that a same number of samples is considered in the two directions during the least mean square minimization.

### Intra prediction

#### General

Intra prediction in this response uses the same basic design as in JVET-J0021. 129 intra directional modes are used instead of 65 for the JEM, in addition to the DC and Planar modes (cf Figure 25).

Two additional tools, described in the following subsections, are used on top of JVET-J0021, namely Multi-Reference Intra Prediction (MRIP) and Bi-Directional Intra Prediction (BDIP).

Figure 25. Intra prediction modes numbering.

#### Multi-Reference Intra Prediction

This tool facilitates the use of multiple reference layers for intra prediction of a block. In the CfP response, 2 reference layers are used for intra prediction, where each reference layer consists of a left reference array and a top reference array (Figure 26). Each reference layer is constructed by the reference sample substitution and then pre-filtered, as done in JEM.

Using each reference layer, a prediction for the block is constructed as in JEM. The final prediction is formed as a weighted average of the predictions made from the two reference layers. The prediction from the closest reference layer is given higher weight than the prediction from the farthest layer. The used weights are 3 and 1. Multi-reference intra prediction is not activated when for luma blocks coded as Planar or DC, or for chroma blocks coded as LMChroma. In the other cases, it is activated.

Figure 26. Intra prediction with 2 reference layers. The layer #1 consists of the usual reference array, the layer #2 consists of the next reference array on top and on left of layer #1.

Boundary samples filtering

In the last part of the prediction process, multiple reference layers are used to smoothen the boundary samples for certain prediction modes. The process is mode dependent.

- For DC prediction mode, the boundary filtering remains the same as in JEM.

- For the four modes closest to the purely vertical/horizontal modes, the predicted pixel values on the first column/line of a block are filtered as:

Modes 96, 97, 99, 100 – for y = 0 to H – 1

P(0,y) = Clip(P(0,y) + (3*((P(-1,y)-P(-1,-1)) >> 2) + ((P(-2,y)-P(-2,-2)) >> 2))/(4))

Modes 32, 33, 35, 36 – for x = 0 to W – 1

P(x,0) = Clip(P(x,0) + (3*((P(x,-1)-P(-1,-1)) >> 2) + ((P(x,-2)-P(-2,-2)) >> 2))/(4))

This filtering is applied to luma blocks having their width and height smaller than 32.

- For vertical/horizontal modes 101 to 130, a gradient based filtering is applied to the predicted samples on the left/top boundary as follows:

Modes 101 to 130 – for x = 0 to W – 1, y = 0 to H – 1

P(x,y) = Clip(P(x,y) + (D(y)* L(x) + 32)/64)

where: D(y) = P(-1,y)-((3*P(s1,-1)+P(s2,-2)+2) >> 2)

and:    L(x) = (32 >> ((x << 1) >> 2))

Modes 2 to 31 – for x = 0 to W – 1, y = 0 to H – 1

P(x,y) = Clip(P(x,y) + (D(x)* L(y) + 32)/64)

where: D(x) = P(x,-1)-((3*P(-1,s1)+P(-2,s2)+2) >> 2)

and:     L(y) = (32 >> ((y << 1) >> 2))

where D(y)/D(x) correspond to the gradient at the first left/top reference array at distance y/x from the top of the block (Figure 27). s1 and s2 are the reference samples in the first and second reference layers, respectively, of the sample P(x,y), for the considered directional mode.

To avoid the cases of sharp changes in intensity at the block boundaries, the afore-mentioned filtering is conditionally applied if the so-called gradient condition applies. The gradient condition is true if the average gradient magnitude at the second reference layer is less than two times that at the first reference layer.

Figure 27. Gradient based boundary filtering for a vertical mode. The gradient is computed at the left reference array by taking the difference of the actual sample value and its prediction using the reference arrays on the top.

- For modes 3 to 18 and modes 114 to 129, if the gradient based filtering is not applied due to the failure of the gradient condition, then the diagonal filtering in JEM is applied to the first row (for modes 3-18) or the first column (for modes 114-129) of predicted samples in the block.

- For the diagonal modes 2 and 130, after the gradient based filtering is applied, the first 4 rows on top and the first 4 columns on left of the block, respectively, are filtered as follows (weights wj being defined in Table 10):

  - for mode 2, for i=0 to 3

P(x,i) = ( w1[i] * P(x,i) + w2[i] * P(x+1,-1) + w3[i] * P(x+2,-2) + 16 ) >> 5

  - for mode 130, for i=0 to 3

P(i,y) = ( w1[i] * P(i,y) + w2[i] * P(-1,y+1) + w3[i] * P(-2,y+2) + 16 ) >> 5

Table 10. Weights for boundary samples filtering in diagonal modes.

| i | w1[i] | w2[i] | w3[i] |
| --- | --- | --- | --- |
| 0 | 16 | 12 | 4 |
| 1 | 20 | 9 | 3 |
| 2 | 24 | 6 | 2 |
| 3 | 28 | 3 | 1 |

The MPM set construction and the decoding of the intra prediction mode remains the same as in JEM. With multi-reference intra prediction there is no added syntax parameter.

#### Bi-directional intra prediction

This tool aims to improve the prediction efficiency for certain positive angular prediction modes using a linear model of luminance change. A CU-level flag (BDIPflag) indicates the application of bi-directional intra prediction. The flag is signaled only for luma blocks with prediction modes 2-17 and 114-129. For the remaining modes, the traditional uni-directional prediction is performed.

In bi-directional prediction a target pixel value is predicted using two reference samples on both ends of a positive prediction direction, as shown in the Figure 28.

Figure 28. In bi-directional prediction, two reference samples are used for computing the prediction for a target pixel. The second reference sample is located at the intersection of the prediction direction with the second reference array.

For a positive vertical prediction direction, the first reference sample is at displacement ∆x from x-coordinate of the target pixel at (x,y). As specified in JEM,

∆x =1+y*A

where A denotes the angle associated with the prediction mode. The second reference sample is at displacement ∆y from the y-coordinate of the target pixel at (x,y).  ∆y can be computed as:

∆y= 1+x≪120A

The values of  ∆y are pre-computed for all positive A and x = 0, and stored in a table. For any value of x, the value of  ∆y is computed by incrementing this stored value by its magnitude x times. Denoting Ay≡ 1≪10A, the tabulated values for different positive A values are given in Table 11 in case of using 65 modes as in JEM. When the number of angular prediction modes is increased to 129, corresponding to an angle resolution of 1/64, the values of Ay  are given as Ay≡ 1≪12A.

Table 11. Parameter Ay for different positive values of A in JEM.

| A | 1 | 2 | 3 | 5 | 7 | 9 | 11 | 13 | 15 | 17 | 19 | 21 | 23 | 26 | 29 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ay | 1024 | 512 | 341 | 205 | 146 | 114 | 93 | 79 | 68 | 60 | 54 | 49 | 45 | 39 | 35 | 32 |

If the coordinate of the second reference sample lies beyond the length of the left reference array, the last reference sample on the left reference array is taken to be the second reference sample value.

Using the first and the second reference samples, an update term is computed and added to the initial prediction (which is the first reference sample value):

ΔP = P2-P1*∆x1+x≪5+ ∆x

P=P1+ ∆P

Here P1 and P2 are the first and the second reference sample values for a target pixel, ∆P is the update, and P is the final prediction value. For positive horizontal prediction directions, the computations are similar.

#### Semantics and interactions between BDIP and MRIP

As shown in Figure 12, a flag is coded to signal the use of BDIP when applicable, whereas MRIP is inferred depending on the intra mode.

In the case of a luma block,

- If BDIPflag is true, then BDIP is on, MRIP is off.

- Else, BDIP is off:

  - If intra mode is directional, MRIP is on,

  - Else MRIP is off.

In the case of a chroma block,

- If the mode is directional (hence not LMChroma), then MRIP is on

- Else, MRIP is off

### In-loop filtering

In HEVC, the SAO process classifies the reconstructed samples of one block into several classes and the samples belonging to some classes are corrected with offsets. The SAO parameters are encoded per block or can be inherited from left or above neighboring blocks only.

This process is replaced in this response by the SAO palette mode. SAO palette keeps the same SAO parameters per block, but the blocks may inherit these parameters from all the other blocks. This brings more flexibility to SAO by widening the range of possible SAO parameters for one block. The SAO palette is composed of a set of different SAO parameters. For each block, the syntax element sao_palette_idx indicates which SAO parameter to use.

The size of the SAO palette is decoded first (sao_palette_num_cand), next the SAO parameters of the palette are decoded in their order of use, so they can be parsed progressively. The syntax of the SAO parameters is the same as HEVC. For each block, the syntax element sao_palette_idx is parsed.

In the slice header, a flag slice_sao_size_id indicates whether the SAO block size is same as CTU or if it is smaller (reduced by two both horizontally and vertically).

At the post-filtering stage, the list of SAO parameters in the palette is reordered so that the SAO parameters which were already used by the closest neighboring blocks appear on top of the list.

Figure 29 illustrates the SAO palette concept. The set of SAO parameters is first encoded for the slice, before coding the blocks SAO data. The blocks can then refer to any element from this set.

Figure 29. SAO palette is composed of all the different SAO parameters.

## 360° Video

### Projection format (including padding)

The projection format used is Padded Equirectangular Projection (PERP), as defined for the anchors.

### Quantization handling

Spatially adaptive quantization has been implemented. As described in JVET-F0038 and JVET-F0049, the Quantization Parameter QP can vary CTU-wise, as illustrated in Figure 30.

However, contrary to the method described in JVET-F0038 and JVET-F0049, the potential QP difference between CTUs (delta-QP) is not coded in the bitstream but inferred. It is based on the CTU’s location only and is derived at the decoder side.

The QP value for each CTU is computed based on the weight used in computing the Weighted Spherical PSNR:

w(y)=cosπyhhwy

where y is the vertical coordinate of the sample and h is the vertical size of the equirectangular picture.

CTUs in the proposed response can have a size up to 256×256 samples. An average weight over all the sample locations of the CTU is computed.

wctu(y0)=y=y0y0+syw(y)

where y0 corresponds to the CTU’s vertical coordinate of the top samples and sy denotes the vertical size of the CTU.

QP is calculated for each line of CTUs as follows:

QP(y0)=min(QPbase-3*log2(wctu(y0)), QPmax)

with QPmax=51, and QPbase corresponds to the QP of the current slice.

Figure 30 shows an exemplary variation of QP on the sequence Harbor PERP (4112×2048) at QPbase=29. The sample-based weighting w(y) is plotted in blue on the right as a function of the vertical coordinate, aligned with the left picture. The corresponding CTU based delta-QP is depicted in orange.

Figure 30. Variation of QP on the sequence Harbor PERP (4112×2048) at QPbase=29. Delta-QP is plotted on the right as a function of the vertical coordinate, aligned with the left picture.

### 360° video specific decoding tools

There is no 360° specific decoding tool other than all the tools included in the SDR solution proposed in this contribution. However, note that the provided bitstreams require the decoder to be able to derive the QP based on CTU’s location, as described in section 2.2.1.

## HDR

### Post-processing

#### Overview

The post-processing applied for HDR is made of two tools:

- inverse Dynamic Range Adaptation (DRA)

- post-decoding refinement

#### Inverse dynamic range adaptation

The DRA process is a signal adaptation applied as a pre-processing at the encoder side, with the aim to improve the coding efficiency. At the decoder side, the inverse process is applied.

DRA applies in the Y’CbCr 4:2:0 sample domain. DRA modifies luma and chroma samples in the picture based on the value of the luma or chroma samples and on the value of other component samples located at the same or closest relative location. It basically consists in a mapping of the Y, Cb and Cr samples, using the following process:

  - For luma, DRA is applied intra-component :

    - Y’ = fDRA(Y)

  - For chroma, a cross-component DRA (CC-DRA) applies :

    - U’ = 2Bchr + (U – 2Bchr) * fscale(Ya))

    - V’ = 2Bchr + (V – 2Bchr ) * fscale(Ya))

fDRA( ) is a 1D function, and Ya is a co-located luma value associated with the Cb and Cr samples, and fscale( ) is a scaling function. Both functions are built from a single scaling table, as shown in Figure 31. A more detailed description of DRA is provided in JVET-J0021. Inverse DRA is of course the inverse of the DRA process, adapted for a division-free implementation.

In this contribution, the DRA and inverse DRA are based on static tables. For HLG content, no DRA is used. For PQ content, the scaling table is independent from the content and is based on the dQP table used for the HM and JEM HDR anchors. The depicted scaling factors correspond to K.2dQP/6, K being a normalization factor avoiding the scaled signal to be clipped.

Figure 31. CC-DRA scaling function for chroma components (red curve) derived from luma DRA scales (blue curve).

#### Post-decoding refinement process

The post-decoding refinement process can apply per slice. It aims at compensating the distortion resulting from the compression that may impact the inverse DRA process. The inverse mapped signal (YIM,UIM, VIM) is refined as follows:

- For luma component, intra-component scaling is applied, corresponding to the following:

  - Yrefine = Min( 2Blum – 1, LUTrefineY[ YIM  ] * YIM  )

  - Note: this can be achieved using a direct LUT

- For chroma components, luma-dependent scaling is applied, corresponding to the following:

  - Urefine = Max(0, Min( 2Bchr – 1, 2Bchr-1 + LUTrefineU[ YIMcoloc ] * ( UIM – 2Bchr – 1 ))

  - Vrefine = Max(0, Min( 2Bchr – 1, 2Bchr-1 + LUTrefineV[ YIMcoloc ] * ( VIM – 2Bchr – 1 ))

  - where YIMcoloc is the co-located inverse mapped luma sample

The look-up-tables LUTrefineY, LUTrefineU, LUTrefineV are built from piece-wise linear (PWL) tables, that can be signaled per slice. The pivot points of PWL tables are equi-distant, and the tables size is equal to 2S+1, S being a parameter signaled in the slice header. In practice, a size of 17 was used in this response. When a table is not signaled for a component, the table previously decoded is used. In practice, in this CfP response, the tables were sent only for slices of temporal level 0.

### Quantization handling

The inverse quantization process is identical to the JEM one.

### HDR specific decoding tools

No HDR specific tool is introduced in the core decoding loop.

# Encoder algorithm description

## General algorithm description

### Rate-distortion optimization

#### Distortion metrics

For SDR and HDR categories, distortion metrics used in the encoder are the same as in the HM or the JEM, that is, SAD or SSE.

For 360° video, the QP adaptation is performed at CTU level. Apart from considering the new assigned QP, the RDO within each CTU remains unchanged.

#### Rate-distortion dependency models

No specific change compared to the JVET-J0021 was applied.

### ABT and coding tree configuration for Medium Complexity (Random Access)

#### Block partitioning RD search with ABT

The configuration used in the medium complexity encoder is the following one.

- The (1/3,2/3) asymmetric partitioning ratio introduced in Figure 7 is deactivated.

- The block size 48 is only supported in luma component in Intra slices.

In the medium complexity encoder, the rate distortion search for the block partitioning uses the following fast methods.

- For a given CU, the first evaluated split mode is NO_SPLIT. Then symmetric BT splitting is evaluated, then ABT splitting is evaluated. The ABT splitting is not evaluated if the BT splitting already evaluated in the same orientation is not competitive enough compared to the NO_SPLIT split mode. This takes the following form.

| - If ( RDCost(HOR_SPLIT) > α×RDCost(NO_SPLIT) ) | then disable asymmetric horizontal |
| --- | --- |
| - If( RDCost(VER_SPLIT) > α×RDCost(NO_SPLIT) ) | then disable asymmetric vertical |
| - If( RDCost(HOR_SPLIT) > β×RDCost(VER_SPLIT) ) | then disable asymmetric horizontal |
| - If( RDCost(VER_SPLIT) > β×RDCost(HOR_SPLIT) ) | then disable asymmetric vertical |

In practice the values α and β are equal to 1.1.

- Some caching mechanisms re-use coding parameters already determined when visiting a block in the past, having the same shape and position as current block, as illustrated in Figure 32.

Figure 32. Re-using already optimized coding parameter for same block with same causal coding environment.

If the current block has already been processed before, with exactly the same causal environment in terms of surround reconstructed pixels and motion vectors, then the already found split mode is chosen without any further RD search.

- The caching mechanism is also used to limit the number of tested binary tree split modes, when a block with same position and size as current block has already been processed in the past. In that case, only the M best split modes found in the past are considered as candidate split modes for current block. Thus, the caching process stores the evaluated split modes for each block position and size, together with the RD cost associated to each evaluated split mode. This caching and re-using process in commonly used in JVET-J0021 and JVET-J0022.

- The CNN-based (Convolutional Neuron Network) split mode prediction process, described in section 3.1.13, is used in Intra slices, to reduce the amount of tested split modes for a given CU at a given quad-tree plus binary-tree depth level.  In practice, the CU currently being processed and associated QT+BT depth is given to the CNN, which returns an array of Boolean values noted split_choices[NO_SPLIT, QT_SPLIT, …, VER_RIGHT]. Then, each split mode S such that the split_choices[S] is false is discarded from the set of evaluated split modes.

#### Coding tree configuration

The used coding tree configuration varies according to the slice type and the temporal layer. The maximum QT and BT depths allowed for each picture of a random-access sequence is shown on Figure 33 and is detailed on Table 12.

Figure 33. Maximum BT depth level associated to each slice type/temporal layer in the random-access coding structure.

As can be seen on Table 12, the maximum allowed QT depth varies according to the slice type, temporal level and component. Moreover, the maximum allowed BT depth varies according to the slice type, slice temporal layer and QT depth level.

Table 12. Maximum QT depth and BT depth used in the medium complexity profile, for each slice type, temporal layer, component, for Random-Access coding structure.

#### Temporal layer based on the fly evolution of the coding tree

The QT and BT multiple depths configuration of Table 12 is used as the starting configuration when coding a Random-Access intra period. It is used as the starting coding tree configuration for each Intra period, since parallel coding of intra periods is employed as for the JEM.

In addition to that starting configuration, a dynamic adaptation process of the maximum QT and BT depth for subsequent pictures is employed, based on the average quad- and binary- coding trees used in already coded picture in the same temporal level, in the considered intra period. This coding tree adaptation process is the same as the one used in contribution JVET-J0021.

#### Other fast methods used in medium complexity profile

Apart from the block partitioning and coding tree configurations previously explained, the implementation of RDOQ process has been improved compared to the JEM. The number of loops over coefficients is reduced. The optimization of coefficients is performed in one single loop, leading to encoder runtime reduction of about 3%.

### Low-Delay B encoder configuration

The difference between the random-access encoder configuration and the low-delay B encoder configuration are the following ones.

- With respect to the GOP structure, the same GOP structure with size 8 is used as in JVET-J0021, with same QP offset and QP model offset used to compute the picture’s QP inside the GOP.

- The slice level usage of affine MC is decided on the encoder side in a similar way as in random access (see JVET-J0021). In random access, the decision is dependent on the frequency of affine MC usage in past coded slices in the same temporal layer. In low-delay, no temporal layer concept is used. However, to re-use a similar process for past-slice based affine usage decision, each slice in the low-delay GOP is given a virtual temporal layer, defined as:

layerIdx=slicePOC%GOPsize

if layerIdx≥4then layerIdx=2+(layerIdx%2)

Given this virtual slice temporal layer, the usage of affine MC is decided according to the average frequency of affine MC usage in past coded slices in same virtual temporal layer.

- Regarding the coding tree, different maximum BT depth are assigned to each QT depth level in B slices. The maximum BT depths used in LDB are shown in Table 13.

- Regarding the block partitioning with ABT, the speed-up that limits ABT RD search, based on the α and β speed-up factors, is deactivated.

- The EMT-NSST interaction tool of section 2.1.6.2 is deactivated.

- An additional speed-up in the motion search is used. It consists in testing if, for a given CU at a given coding tree depth level, the rate distortion search for parent CUs at lower depth levels have led to the choice of the merge mode, then the RD search for current CU is constrained to the merge flag equal to true only.

- Adaptive search range is deactivated in the motion estimation process.

- Two additional caching and re-using mechanisms are used in the INTER mode search.

  - Regarding the merge mode, if the current block (in terms of position, size and same set of MV predictor candidates) was already processed by the RDO, then only the merge candidate that were not checked before and the best already-checked merge candidate are considered as candidate for the current block.

  - Regarding the INTER mode, if the current block (in terms of position, size and same surrounding motion vectors) was already processed by the RDO, and if the best coding mode found so far for current block has a lower RD cost than the cached INTER RD cost, then the INTER mode is not evaluated for current CU.

### Alternative High Efficiency Encoder Configuration for Random-Access

This section describes an alternative encoder configuration, aiming at higher compression efficiency. The same set of coding tools as in the proposed medium complexity profile is used. Thus, the same decoder is used as in the medium complexity.

The same fast methods as in the medium complexity are used, except the following ones.

- The CNN-based split prediction process is deactivated.

- The same relationship between the Luma QP and the Chroma QP as in the high complexity profile of contribution JVET-J0021 is employed.

- The coding tree configuration when starting to code an Intra period is modified for B slices compared to section 3.1.2. The maximum BT depth is always equal to 3, except for the quad-tree depth level 0. This corresponds to Table 13.

Table 13. Maximum QT depth and BT depth used in low-delay B and in the higher efficiency RA profile, for each slice type, temporal layer, component, for Random-Access coding structure.

Table 14 shows the BD-rate gain, encoding time and decoding time obtained in the higher efficiency profile, compared to the JEM and HM anchors of the JVET Call for Proposal, in random-access. 15.6%/5.9%/4.9% BD-rate gain over JEM7 is obtained in Y/Cb/Cr components. The decoding time relative to the JEM is the same as that of the medium complexity profile. The encoding time ratio is 127% over JEM7.

Table 14. Higher efficiency trade-off with the QT+BT+ABT coding tree structure and the coding tools described in this document.

### Alternative Low Complexity Encoder Configuration for Random-Access

This section describes an alternative encoder configuration, aiming at lower complexity. The same fast methods as in the medium complexity are used, except the following ones.

- The CNN-based split prediction process is used with a higher speed-up factor.

- The ABT rate distortion search is further accelerated compared to section 3.1.2: the speed-up factors α and β are equal to 1.05.

- The ABT tool is deactivated in B slices with a temporal layer strictly higher than 0.

- The template affine motion compensation tool described in section 2.1.7 is deactivated.

Table 15 shows the BD-rate gain, encoding time and decoding time obtained in this lower complexity profile, compared to the JEM and HM anchors of the JVET Call for Proposal, in random-access. 11.8%/0.5%/2% BD rate gain is obtained in Y/Cb/Cr components over JEM7. The decoding time ratio over JEM7 is 82%. The encoding time ratio is 59% over JEM7.

Table 15. Lower-complexity trade-off with the QT+BT+ABT coding tree structure and the coding tools described in this document.

As a conclusion, the diagram of Figure 34 summarizes three trade-offs between encoder complexity and BD-rate gain that can be achieved with the QT+BT+ABT coding tree arrangement and the coding tools presented in this contribution.

Figure 34. Achievable BD rate / encoding time trade-offs that can be obtained with the tools described in this document, with the QT+BT+ABT coding tree structure.

### Intra prediction mode estimation and type selection

The intra prediction mode is estimated using a multi-pass algorithm as in JEM where a pre-determined number of candidates are first short-listed based on fast RD optimization (numModesForFullRD). Then, a more complete RD optimization over the short-listed candidate modes refines the prediction mode and transforms. The prediction method employed in intra prediction step is identical to the decoder one.

As described in section 2.1.8.4, the syntax element BDIPflag is added to indicate if intra bi-directional prediction (BDIP) is used or not for luma blocks. The second added tool, Multi-Reference intra prediction (MRIF), being inferred depending on prediction modes (as explained in section 2.1.3.2), this section focuses on the encoder process for selecting BDIP prediction.

In the first RDO phase, candidates are evaluated using a fast SATD cost, with both uni-directional prediction and bi-directional prediction if the mode is eligible for BDIP. The first intra pass checks DC, Planar and even prediction direction indexes, sorts (mode, BDIPflag) pairs according to their RD costs and keeps the numModesForFullRD best ones. Then, the second fast pass tests odd directions around the previously found best ones with uni-directional prediction and bi-directional prediction if the mode is eligible, sorts (mode, BDIPflag) pairs according to their RD costs together with previously found ones. At the end of the first phase, MPM modes not short listed are added to the short list with both BDIP indexes if available.

In the second phase, the short-listed pairs (mode, BDIPflag) are evaluated with a full RD optimization including residual transform and encoding. This is done in two passes. The first one evaluates candidates using only one primary transform (EMT=0), all the secondary transforms (NSSTIndex) available and fast quantization optimization (no RDOQ). numRDModesSelFrNonRDOQ best (mode, BDIPflag, NSSTIndex) triplets are selected for the last pass where (mode, BDIPflag, NSSTIndex) triplets are tested with transform skip and EMT transform set using full RDO to select the best intra coding parameters.

### Motion estimation and motion segment selection

#### General

The overall encoding processes, i.e. RD searches, of AMVP, merge/skip and template merge modes are not modified by the proposed solution compared to JVET-J0021. Differences take place at the tool level as described in previous part on motion compensation 2.1.7.

On top of that, several adaptations have been introduced:

- During the RD search in an inter CU, at a given QT+BT depth level, if the skip mode was found as the best mode for 2 successive depth levels until the current depth level, then only the skip mode is tested for current CU.

- The motion estimation process employs an adaption motion search range with a minimum search window equal to 96, a search range equal to 384, and a search range for bi-prediction refinement of 4, as in JEM7.

- The activation of IC in the picture (slice_ic_enable_flag) is function both of the SADs between the current original picture and the reference pictures and also of the relative surfaces SIC of blocks coded with IC in the reference pictures. If SAD < ThSAD or SIC < ThIC (ThSAD and ThIC are set to 0.06 and 0.015, respectively, in the CfP response), then IC is deactivated in the current picture.

#### Usage of decoder-side motion refinement

The template affine merge mode follows the same encoding process as the template matching of JEM. During inter RD search, the template affine merge mode is evaluated as an independent tool. Its RD cost is compared with the template matching merge mode RD cost. The best mode then competes with the other tested modes.

In template affine merge mode, a candidate motion vector and its associated reference picture are derived and refined for each reference picture list using the process described in 2.1.7. The current CU is then compensated using this(these) motion vector(s) and reference picture(s) and fully encoded (merge flag, template merge mode set to template affine and residuals) to get the associated RD cost.

### Mode decisions

The same mode decision processes are used as in JVET-J0021. No specific additional modification in the mode decisions was inserted in the current response.

### In-loop filtering type selection and parameter estimation

As in HM encoder software, the determination of the SAO parameters is done from the collected statistics of the reconstructed samples, then the picture is only run through once.

Next, the derivation of the SAO parameters which will compose the SAO palette is achieved by an iterative process, with computations only involving the collected statistics. A first set of SAO parameters is determined as done in JEM, by raster scanning the SAO blocks of the picture to derive SAO parameters, possibly inherited from left or above. Next, the set of SAO parameters are refined: two SAO parameters can be merged to form one single SAO parameter and the unused SAO parameters are removed from the SAO palette. This process is performed for each possible value of enabled SAO block size. Finally, the SAO palette with minimum RD cost is selected.

### Transforms and transform type selection

With regards to the primary block transform, the set of transforms that can be chosen by the encoder is depicted by Table 4. It is of the same size as the one used in JVET-J0021. Therefore, the combinatory of the encoder side transform selection process is the same as in JVET-J0021.

With regards to the secondary transform, no modification of the process of JVET-J0021 is brought here.

### Quantization and quantization type selection

No specific change in the quantization process is made compared to the JEM, except for new blocks sizes introduced by the ABT tool, as described in the following.

In the case of asymmetric Coding Units, blocks may have size which are equal to 3×2n in width and/or height. This leads to an overall block surface potentially equal to 3×2n×2m=3×2n+m or 3×2n×3×2n=9×2n+m.

Figure 35 depicts the forward transform and forward quantization processes, for any rectangular block with generic size noted (s1,s2). The successive scaling factors that apply are the following ones.

- s1×26 : scale factor used during the first (horizontal) forward transform stage

- ST1: scale factor applied after the first forward transform stage

- s2×26 : scale factor applied during the second (vertical) forward transform stage

- ST2: scale factor applied after the second forward transform stage

- 214: scale factor applied during the forward quantization stage

- SQ: scale factor applied after the forward quantization stage

Figure 35. Forward transform and forward quantization steps, with associated respective scaling factors used for the HEVC-like integer-based implementation, for a generic rectangular transform block with size (s1,s2).

Concerning the forward transform, Table 16 provides the values of each scale factor involved, together with the overall scale factor resulting from the product of all scale factors used in the forward 2D transform of the block. It is equal to 1s1s2×215-B, where B is the considered bit-depth.

Table 16. Value of each scaling factor in the HEVC-like 2D forward transform process, where (s1,s2) is the size of the generic rectangular transform block and B is the signal bit-depth.

|  | Scale factor |
| --- | --- |
| First forward transform stage (St) | s1.26 |
| After the first transform stage (ST1) | 2-(B-9).1s1 |
| Second forward transform stage (S’t) | s2.26 |
| After the second transform stage (ST2) | 2-6.1s2 |
| Total scaling for the 2D forward transform | 1s1s2.215-B |

To ensure the preservation of the residual norm for Qstep=1 i.e. QP=4, the post-quantization scaling factor SQ is derived such that:

SQ×f4×1s1s2×215-B=1

which leads to:

SQ=s1s2×2-(29-B)

To handle the cases where s1 or s2 is equal to a multiple of 3, the chosen implementation of the overall process consists in integrating the factor 3/4 into the series of scaling factors of Table 16 as follows.

- In case s1 is not a power of 2:

  - the scaling factor St is computed as: St=43s1∙26

  - the scaling factor ST1is computed as: ST1=2-B-9×143×s1 , which can be performed by right bit shift

  - this results in an overall scaling by 34 along the concerned orientation

- In case s2 is not a power of 2:

  - the scaling factor S't is computed as: S't=43s2∙26

  - the scaling factor ST2 is computed as: ST2=2-6×143×s2 , which can be performed by right bit shift

  - this results in an overall scaling by 34 along the concerned orientation

This way, the overall transform and quantization process for the newly introduced block sizes is performed with multiplication and bit shift operations.

### Entropy coding

No specific encoder change of the entropy coding is made compared to the JEM.

### Additional tool – CNN-based driving of partitioning decisions

For Intra slices, a Convolutional Neural Network (CNN) based analysis is proposed to speed-up the encoding process. The method is based on the texture analysis of 64×64 root blocks, luma or chroma, for predicting the most probable splits inside each potential sub-block. The method enables the encoder to leverage the gains of BT and ABT partitioning, while keeping the combinatory reasonable.

This method is described in detail in the contribution JVET-J0034.

In Intra slices, the encoder always applies a first quadtree split to the CTU as a first splitting depth, leading to four 128×128 blocks in luma and 64×64 blocks in chroma. In the luma case, the second splitting depth only allows quadtree split, evaluated through a classical RDO process, leading to four 64×64 blocks. Root blocks for CNN based analysis then correspond to the third level of depth in luma, respectively the second level in chroma. 64×64 blocks are considered in both cases.

65×65 patches, luma or chroma, are considered as input to the CNN stage. They consist in the original texture of root-blocks and their causal neighboring top and left boundaries, as they participate to split decisions in classical RDO.

Figure 36. Overview of the split prediction process.

Figure 36 depicts the overall process of the split prediction module. This module precedes the usual RDO process, and pre-selects the split configurations to be tested by the RDO. The process is composed of the following 3 steps:

- CNN-based analysis – In the first step, each input 65×65 patch is analyzed by a CNN-based texture analyzer. The output of this step consists in a vector of probabilities associated to each elementary boundary that separate elementary 4x4 sub-blocks. The elementary size of blocks being 4×4, the vector contains n=480 probability values.

- Probable split selection - The second step takes as input the probability of each elementary boundary and outputs a first set of splitting modes among all possible options, that are: no split, QT, BT (vertical, horizontal), ABT (top, bottom, left, right).

- Encoder constraints and speed-ups - The third step selects the final set of splitting modes to be checked by classical RDO, depending on the first set provided by step 2, the contextual split constraints described in section 2.1.1.3 and the encoder speed-ups described in section 3.1.2.1.

A control parameter called speed-control has been added to the encoder. This parameter is a floating value ranging from 0.65 to 3.4. It drives:

- the structure of the tree used for the luma and chroma: maximum and minimum QT level and BT/ABT levels,

- the thresholds on boundary probabilities, used to prune the tree before the RDO.

It is observed in Figure 37 that the proposed split prediction and dynamic tree structure allow the encoder to cover a wide range of trade-offs. For example:

- for the same BD-rate as JEM7, the encoder runtime is reported to be 23% of the JEM runtime,

- at the same speed as JEM7, the BD-rate gain is around 6%,

- for a speed of approximately 850% of JEM7, the BD-rate gain reaches 9.1%.

From a speed ratio around 50% of the JEM7, the benefit of having more topologies in the proposed codec can be observed, since, QTBT+ABT offers a better trade-off than the QTBT tree only.

Figure 37. BD-rate gains of the proposed encoder over JEM7, as a function of the complexity, the latter being controlled by a user encoding parameter “speed control”.

The results provided in section 4 were produced using the same CNN models and a speedup factor of 1.5 for all tests (SDR, HDR, 360° video).

### Additional encoder optimization

No additional encoder optimization is used in addition to the fast coding methods described in sections 3.1.1 to 3.1.13.

No non-automatic encoding method is employed.  No multi-pass encoding is performed. No content-adaptive GOP structure is used. No specific optimization technique for subjective quality is used. No spatial adaptation of QP is done, except for PERP coding in 360° video (see section 3.2).

## 360°

### Usage of projection format adaptation

The native content in PERP format was used.  No projection format adaptation was applied.

### 360° Video specific encoding tools

Spatially adaptive quantization as described in 2.2.1 is used.

### Additional encoder optimization

No additional 360° specific encoder optimization was used.

## HDR

### Pre-processing

#### Dynamic range adaptation

As explained in the decoder section 2.3.1, the input HDR signal is pre-processed by a Dynamic Range Adaptation, implemented using 1D LUTs. The process is explained in section 2.3.1.

The DRA LUTs are derived from a unique scaling table, built from a dQP table. For a given dQP value, the corresponding scaling factor is derived as 2dQP/6. The scaling factors are used both for luma and chroma range adaptation.

The scaling table is defined per interval [ Yi, Yi+1 – l ], where i denotes the index of the interval between 0 and N – 1, N being the total number of intervals. Let’s note Sci the scaling factor for the interval i.

The mapping function map( ) for the luma component is defined as follows, for a value Y in an interval i:

map( Y ) = Min(2Bluma, map( Yi ) + Sci * ( Y – Yi ) )

which can be implemented by a 1D LUT LUTmap:

Ymap = LUTmap [ Yorig ]

where Ymap is the remapped version of the original luma value Yorig.

For the chroma components, the table of scaling factors is considered as a PWL model from which a 1D scaling LUT, LUTscalchr, is derived. Each chroma component (C = U or V), co-located with the original luma sample Ycoloc, is rescaled as follows:

Cmap = 2Bchr + LUTscalchr[Ycoloc] * (Corig – 2Bchr )

More details on DRA process can be found in JVET-J0021.

In this CfP response, for PQ content, the same table is used for all content. It is built from the dQP table used for the CfP anchors. For HLG content, DRA does not apply.

#### Refinement tables derivation

In addition, a process to derive the refinement tables also applies. This process aims at minimizing the distortion of the reconstructed samples, after post-processing, compared to the encoder input samples, before pre-processing. As explained in the related decoding section, the refinement tables correspond to scaling functions modelled by PWL models. The estimation of these models consists in defining the pivot points such as the rate-distortion over the slice is minimized. This is achieved by scanning one-by-one the pivot points (xp,yp)  (p being the index of the point; in practice 17 points were used), evaluating the resulting RDO cost for a range of possible yp values (32 values tested), and selecting the yp value providing the lowest RDO cost. The process can be iteratively repeated (in practice 2 iterations were achieved).

Fast implementations, avoiding full picture scanning, can be used for these optimization processes, as explained below.

For the luma, the distortion over the picture array I is computed as:

SSE=p in I(Yin(p)-Yrefine(p))2

where Yrefine(p) can be derived using a mapping LUT: Yrefine(p) = LUTY[Ydec(p)].

For the set SY ={p in I such that Ydec (p)= Y}, the cumulated error is

SSEY=p in SY(Yin(p)-LUTY[Y])2=p in SYYin(p)2-2.LUTY[Y].p in SYYinp+NY.LUTYY2

where NY is the number of occurrences in the decoded picture of the luma sample value Y (i.e. the number of elements of SY). The total distortion SSE can be computed as:

SSE=Y=02Bluma-1SSEY

When changing a pivot point of the luma refinement table, it is only required to recompute the new mapping values LUTY[Y] for Y values impacted by the modification of the pivot point, and then to update the SSE for these new mapping values.

For the chroma U (the same applies for V), the distortion over the picture is computed as:

SSE=p in I(Uin(p)-Urefine(p))2

Urefine(p) can be derived as Urefine(p) = off + LUTU[ Ydec(p) ] * ( Udec(p) – off ) (by neglecting the clipping), where off = 2Bchr-1

SSE=p in I(Uin(p)-off)2+LUTUYdec(p)2.p in I(Udec(p)-off)2-2.LUTUYdec(p).p in IUinp-off.(Udec(p)-off)

For the set SY ={p in I such that Ydec(p)= Y}, the cumulated error is

SSEY=p in SY(Uin(p)-off)2+LUTUY2.p in SY(Udec(p)-off)2-2.LUTUY.p in SYUinp-off.(Udec(p)-off)

And the total distortion SSE can be computed as:

SSE=Y=02Bluma-1SSEY

When changing a pivot point of the U (or V) refinement table, it is only required to recompute the new mapping values LUTU[Y] for Y values impacted by the modification of the pivot point, and then to update the SSE for these new mapping values.

### HDR specific encoding tools

No HDR specific tool is introduced in the core encoding loop.

### Additional encoder optimization

No HDR specific encoder optimization was used. In particular, it is not necessary, contrarily to the HM and JEM anchors, to modify the distortion or the lambda values used in the RDO process, to specialize the codec for the wPSNR HDR metric.

# Compression performance

## SDR: Constraint set 1 configuration relative to HM anchor

### Class SDR-A (RD-curves)

RD-curves for FoddMarket sequence.

RD-curves for CatRobot1 sequence.

RD-curves for DaylightRoad2 sequence.

RD-curves for ParkRunning3 sequence.

RD-curves for CampFire sequence.

### Class SDR-B (RD-curves)

RD-curves for BQTerrace sequence.

RD-curves for RitualDance sequence.

RD-curves for MarketPlace sequence.

RD-curves for BasketballDrive sequence.

RD-curves for Cactus sequence.

### Overall (BD-rate summary tables)

## SDR: Constraint set 2 configuration relative to HM anchor

### Class SDR-B

RD-curves for BQTerrace sequence.

RD-curves for RitualDance sequence.

RD-curves for MarketPlace sequence.

RD-curves for BasketballDrive sequence.

RD-curves for Cactus sequence.

### Overall

## SDR: Constraint set 1 configuration relative to JEM anchor

### Class SDR-A

RD-curves for FoddMarket sequence.

RD-curves for CatRobot1 sequence.

RD-curves for DaylightRoad2 sequence.

RD-curves for ParkRunning3 sequence.

RD-curves for CampFire sequence.

### Class SDR-B

RD-curves for BQTerrace sequence.

RD-curves for RitualDance sequence.

RD-curves for MarketPlace sequence.

RD-curves for BasketballDrive sequence.

RD-curves for Cactus sequence.

### Overall

## SDR: Constraint set 2 configuration relative to JEM anchor

### Class SDR-B

RD-curves for BQTerrace sequence.

RD-curves for RitualDance sequence.

RD-curves for MarketPlace sequence.

RD-curves for BasketballDrive sequence.

RD-curves for Cactus sequence.

### Overall

## HDR: Constraint set 1 configuration relative to HM anchor

### Class HDR-A

RD-curves for DayStreet sequence.

RD-curves for PeopleInShoppingCenter sequence.

RD-curves for SunsetBeach sequence.

### Class HDR-B

RD-curves for Market sequence.

RD-curves for ShowGirl2 sequence.

RD-curves for Hurdles sequence.

RD-curves for Starting sequence.

RD-curves for Cosmos1 sequence.

### Overall

## HDR: Constraint set 1 configuration relative to JEM anchor

### Class HDR-A

RD-curves for DayStreet sequence.

RD-curves for PeopleInShoppingCenter sequence.

RD-curves for SunsetBeach sequence.

### Class HDR-B

RD-curves for Market sequence.

RD-curves for ShowGirl2 sequence.

RD-curves for Hurdles sequence.

RD-curves for Starting sequence.

RD-curves for Cosmos1 sequence.

### Overall

## 360° Video: Constraint set 1 configuration relative to HM anchor

### Class 360

RD-curves for Balboa sequence.

RD-curves for Chairlift sequence.

RD-curves for KiteFlite sequence.

RD-curves for Harbor sequence.

RD-curves for Trolley sequence.

### Overall

## 360° Video: Constraint set 1 configuration relative to JEM anchor

### Class 360

RD-curves for Balboa sequence.

RD-curves for Chairlift sequence.

RD-curves for KiteFlite sequence.

RD-curves for Harbor sequence.

RD-curves for Trolley sequence.

### Overall

# Decoder complexity analysis

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

Due to varying workload on the used cluster, the following protocol was applied to make the decoding time measurements as reliable as possible.

For each coding tool test driven in the per-tool evaluation of section 11.2, a reference decoder and a test decoder were considered.

The test procedure was performed in parallel mode on a cluster, with CPU characteristics listed in section 5.2.

For each rate index of each sequence of the CfP test-set, the corresponding anchor codec and the corresponding test codec bitstreams were successively decoded several times in an interleaved way between anchor and test cases, in a single job. The average time for each reference and test case was computed, and the decoding time ratio was obtained as the ratio between test and anchor average decoding times.

This procedure ensured that the reference decoder and test decoder were run in similar conditions.

## Description of computing platform used to determine decoding times reported in section 5.1

The following platforms were used to encode and decode the bitstreams:

- Intel(R) Xeon(R) CPU E5-2697A v4 @ 2.60GHz, lock to 3.1GHz, 16 cores, no hyperthread

- Cache: 32KB/256KB/40MB

- SIMD extension: up to AVX1 was used

- Memory available: ~16GB/core

## Memory usage of decoder

The memory consumption was measured using the UNIX time command, giving the memory peak consumption. The memory required for encoding 4 GOPs in RA configuration at QP=32 was measured. The test was performed on two sequences, one from each SDR class.

Figure 38. Memory consumption peak at the decoder side.

It is observed on Figure 38 that MTT requires around 50-55% of the memory required by JEM7.0 and 68-85% of HM16.

## Complexity characteristics of decoder entropy decoding operation

The complexity level of the entropy decoding is the same as in contribution JVET-J0021.

## Complexity characteristics of decoder inverse quantization

Inverse quantization complexity is not impacted by the tools described in this contribution, compared to contribution JVET-J0021.

## Complexity characteristics of decoder inverse transform operation

In comparison with the MTT codec of JVET-J0021, the transform design requires 66.6% less memory for storing the transform matrices, and/or 66.6% less hardware components. The current transform set supported by JVET-J0021 are indicated in Table 17. Out of them, only DCT-VIII, DST-VII and DST-IV are kept in this proposal. In addition, DST-VII is implemented via simple pre- and post-processing of the existing DCT-VIII transform. Thus, no specific implementation of DST-VII is needed. For the purpose of comparison, Table 18 provides the numbers of matrices or hardware architectures needed for the present solution and JVET-J0021. As can be seen, 4 less matrices (or hardware architecture) are used for each block size in this contribution.

In the case of matrix multiplication implementation of the transform coding, the number of required matrix elements are calculated in Table 19. When short integer representation is considered (2 Bytes), the 24904 elements for each transform correspond to 298848 Bytes (2*24904*6) for JVET-J0021, and 99616 Bytes (2*24904*2) for the current proposal. Thus, 199 Kbytes less memory is required.

Table 17. Adaptive multiple transforms in JVET-J0021. Transforms in bold are self-inverse.

| MTT transform sets | DCT-V, DCT-VIII, DST-I, DST-VII, DST-IV |
| --- | --- |

Table 18. A comparison between the number of additional required additional transform matrices/hardware architectures in the proposed and MTT approaches.

| Transform Type (proposed) | Number of matrices or hardware architectures | Transform Type (MTT) | Number of matrices or hardware architectures |
| --- | --- | --- | --- |
| DST-VII | 0 | DST-VII | 2 |
| DST-IV | 1 | DCT-VIII | 1 |
| DCT-VIII | 1 | DST-I | 1 |
|  |  | DCT-V | 1 |
|  |  | DST-IV | 1 |
| Sum | 2 | Sum | 6 |

Table 19. Number of transform matrices elements needed for different transform block sizes.

| Transform block size | Number of transform matrix elements |
| --- | --- |
| 4 | 16 |
| 8 | 64 |
| 16 | 256 |
| 32 | 1024 |
| 64 | 4096 |
| 128 | 16384 |
| 2 | 4 |
| 6 | 36 |
| 12 | 144 |
| 24 | 576 |
| 48 | 2304 |
| Sum | 24904 |

Regarding the cost of ABT in terms of memory needed for the storage of the transform matrices, the estimated cost is 2×36+144+576+2304×2=12240 bytes.

The amount of memory needed in the MTT codec without ABT and without the reduction of the set of transforms is 2×16+64+256+1024+4096+16384×6=262080 bytes. This is to be compared with the 99616 bytes needed in the proposed codec.

Note in practice EMT transforms are used only until size 64, in the chosen encoder configuration. Table 20 summarizes the number of bytes needed in practice, respectively for the QTBT plus classical EMT case, and the proposed codec (QBTBT + ABT + reduced EMT).

Table 20. Memory storage need for the EMT transforms in used encoder configurations.

| QTBT + classical EMT | 2×16+64+256+1024+4096×6=65472 bytes |
| --- | --- |
| QTBT + ABT + reduced EMT | 2×16+64+256+1024+4096+36+144+576+2304×2=34064 bytes |

## Complexity characteristics of decoder-side motion refinement

Complexity of extended template modes

In the template merge modes, the template matching unilateral candidates are removed, even if the AMVP are enriched up to 2 candidates. Furthermore, in bilateral matching, the refinement steps are restricted. This results in a decrease of the overall decoding complexity of these tools. It involves 7% decrease of the decoding time in “tool on” tests and 3% decrease in “tool off” tests without BD rate loss in both cases.

Complexity of new template affine mode

The new template affine mode is conceptually based on a similar decoding process as the JEM “FRUC” mode, but the number of refinement steps is limited to 5. More specifically, in this mode, a template matching candidate is first calculated, an affine model is derived, then 6 affine motion compensations and 6 template matching cost calculations are performed (initial cost and 5 refinement steps). This generates, coupled with the other extended affine motion compensation adaptations, around 6% increase of the decoding time in “tool off” tests along with 0.3% BD rate gain and 16% increase in “tool on” tests with 0.5% BD rate gain.

## Complexity characteristics of decoder motion compensation

Complexity of extended affine motion compensation

The decoding complexity in merge mode, including affine candidates, is not modified compared to JVET-J0021.

Complexity of generalized OBMC

The proposed generalized OBMC does not aim at modifying the performance of OBMC, but at simplifying its design and at taking automatic (not mode dependent) decisions. The decoding complexity as well as the associated BD rate gains are preserved.

The generalized OBMC process can be lightened by removing IC parameters computation for each OBMC 4xS band but by inheriting the IC parameters from the considered neighboring 4x4 sub-block.

Complexity of Bi-directional Illumination Compensation

When IC is enabled for the block and the bi-prediction process for deriving the IC parameters is selected, then two sets of IC parameters are computed. Uni-directional IC parameters use the reconstructed samples of the L-shape, possibly sub-sampled for blocks larger than 8 and so that the sum of the samples does not exceed 30 bits. Bi-prediction IC parameters use the reference block samples, possibly sub-sampled for blocks larger than 8 and so that the sum of the samples does not exceed 30 bits.

The bi-directional optical flow (BIO) is enabled with bi-directional illumination compensation tool.

## Complexity characteristics of decoder intra-frame prediction operation

The added complexity in intra prediction depends on the prediction mode, block size, luma/chroma component, etc. If the decoded prediction mode is Planar, or DC, the prediction process is the same as in JEM. For chroma blocks, if the decoded prediction mode is a LMChroma mode, the prediction process also remains unchanged.

If the prediction mode is one of angular prediction modes and also belongs to the set of positive directions eligible for bi-directional prediction, then the flag BDIPflag will be decoded with the prediction mode. If BDIPflag has value 1, bi-directional prediction is performed, else uni-directional prediction with multi-reference is performed. In the latter case, depending on the prediction mode, the prediction block may be post-processed with boundary filtering.

In bi-directional prediction, for every target pixel, a second reference sample is estimated in the same manner as the first reference sample, and an update term is calculated. The update term calculation involves one division operation for higher accuracy. For an approximate update, the division operation can be replaced by multiplication and bit shift.

In multi-reference intra prediction, the decoder needs to fetch additionally one row and one column of decoded samples from the decoded frame buffer for constructing the second reference layer. For every target pixel, a second reference sample is calculated in the same manner as the first reference sample, and their weighted average is computed, which involves one multiplication, one addition, and a bit shift.

In the post-processing step, the additional complexity stems from the computation of gradients with two reference arrays and the subsequent boundary filtering with the gradient values, and for diagonal modes (2 and 130, cf Figure 25), filtering 4 rows or columns with fixed low-pass filters. The gradient calculation involves taking the difference between the known reference sample values and their prediction values over a length of target block width or height; it involves computing the prediction at two additional rows on top, or columns on left, of the target block. The final prediction value is obtained by adding a scaled gradient value to the initial prediction value.

## Complexity characteristics of decoder in-loop filtering operation

The parsing of the SAO parameters of the SAO palette is not changed compared to HEVC, except the replacement of sao_merge_left_flag and sao_merge_up_flag syntax elements by sao_palette_idx. The decoding process of the SAO parameters associated with sao_palette_idx needs to access the SAO palette (list of SAO parameters which compose the palette). The SAO filtering stage which adds offsets to the reconstructed samples is the same as HEVC.

## Complexity characteristics of additional decoder tools

n.a.

## Complexity characteristics of 360° video specific decoding tools

The proposed adaptive QP requires the encoder and the decoder to calculate the proper QP per line of CTUs. Delta QPs, corresponding to the difference between the slice QP and the new QP for each line of CTUs, can be computed at once for the whole sequence and stored in a table, whose size equals the number of CTUs rows.

As quantization is spatially modified, runtimes differ, due to the modified QP distribution over the frames. This results in the selection of different coding modes and block sizes, depending on the chosen QP per CTU.

## Complexity characteristics of HDR specific decoding tools

The HDR post-processing tools used in this response apply independently per luma or chroma sample.

For Dynamic range adaptation, a maximum of 3 look-up tables are built per SPS (denoted LUTinvDraY, LUTinvDraU and LUTinvDraV). In addition, a maximum of 3 refinement look-up tables are built per slice (denoted LUTrefineY, LUTrefineU and LUTrefineU).

For the luma component, Dynamic range adaptation and Refinement processes can be combined into one single LUT, computed per slice. The post-process for each luma sample involves one access to the LUT.

The post-process for each chroma sample (U/V) involves the following operations: one access to the LUT LUTrefineU/V indexed by the co-located luma sample, 1 multiplication, 2 additions, 1 shift and one clipping between 0 and (2Bchr – 1).

## Degree of capability for decoder parallel processing

The coding tools described in this document do not break any capability for decoder parallel processing, compared to JVET-J0021.

Note that the intra mode parsing depends on the decoded intra mode, both in this contribution and in JVET-J0021. This may imply a slightly higher amount of processing in the parsing, when parallelizing the parsing and decoding processes.

Additionally, as for JEM, the in-loop filtering processes introduce some dependencies in-between CTU blocks since the filtering of one CTU may need access to the values of neighboring reconstructed samples (de-blocking filter) or access to the decoded filter parameters from neighboring CTU (SAO).

# Encoder complexity analysis

## Encoding time and measurement methodology

For low-delay coding structure, the encoding time is simply taken from the encoder log file. For random-access, Intra period have been encoded in parallel on the cluster. Thus, the overall encoding time for a given segment is taken by summing all the encoding time of each frame, and by discarding the first picture of each IRAP segment except the first one. In other words, the same process as in JVET Coding Test Conditions has been employed.

## Description of computing platform used to determine encoding times reported in section 6.1

The computing platform described in section 5.2 has been used.

## Memory usage of encoder

The memory consumption was measured using the UNIX time command, giving the memory peak consumption. The memory required for encoding 4 GOPs in RA configuration of a single sequence of each CfP constrained set at QP=32 has been measured.

Figure 39. Memory consumption peak at the encoder side.

It is observed on Figure 39 that MTT requires around 50% of the memory required by JEM7.0 and around 150-180% of HM16.

## Complexity characteristics of encoder intra-frame prediction type selection

The added complexity in the intra prediction mode estimation stems from testing some directional candidates with both bi-directional prediction and uni-directional prediction. However, this is done only in the fast two passes where full RD optimization is not performed. In the third pass, which involves full RDO, each short-listed candidate mode is tested with either bi-directional prediction or uni-directional prediction, except the first two MPM modes which, if not already short-listed in first two passes, are included in the list of candidate modes for the third pass, first with uni-directional and then with bi-directional prediction, if eligible for it.

As mentioned for complexity in the decoder intra prediction, bi-directional prediction involves estimating a second reference sample for each target pixel and then computing an update term. The calculation of the update term involves a division operation for higher accuracy. The added complexity in uni-directional prediction comes from the usage of the second reference array. Based on the second reference array, a second prediction value is computed for each target pixel, and the final prediction is calculated as a weighted average of the two prediction values Furthermore, the complexity also comes from the added gradient-based post-filtering with two reference arrays.

Since the encoder has to check uni-directional prediction with all modes, it always constructs two reference layers, each consisting of a top and a left reference array, by accessing the decoded frame buffer.

## Complexity characteristics of encoder motion estimation and motion segmentation selection

### General

Complexity of extended affine motion compensation

The encoding complexity in merge mode, including affine candidates, is not modified compared to JVET-J0021.

Complexity of OBMC

As for the decoder, the proposed generalized OBMC aims at simplifying the design and at taking automatic (not mode dependent) decisions. The encoding complexity as well as the associated BD rate gains are preserved.

The proposed generalized OBMC process can be lightened by removing IC parameters computation for each OBMC 4xS band but inheriting the IC parameters from the considered neighboring 4x4 sub-block.

Complexity of Bi-directional Illumination Compensation

Two sets of IC parameters are computed, and BIO is enabled with bi-directional illumination compensation tool.

The activation of IC in the picture is function of the SADs between the current original picture and the reference pictures and of the relative portion of blocks coded with IC in the reference pictures.

### Complexity characteristics of usage of decoder-side motion refinement

Complexity of extended template modes

In the template merge modes, since some considered candidates are removed and some others are added, the overall encoding complexity and BD rate gains are not affected.

Complexity of new template affine mode

The new template affine mode is conceptually based on a similar encoding process as the JEM “FRUC” mode, but it has a limitation to 5 refinement steps. More specifically, a template matching candidate is first calculated, an affine model is derived, then 6 affine motion compensations and 6 template matching cost calculations are performed (initial cost and 5 refinement steps). This generates, coupled with the other extended affine motion compensation adaptations, around 10% increase of the encoding time in “tool off” tests along with 0.3% BD rate gains and around 12% increase in “tool on” tests with 0.5% BD rate gains.

## Complexity characteristics of encoder transforms and transform type selection

The same amount of memory to store the transform matrices and in hardware components is needed on the encoder side compared to the decoder. It is described in section 5.6.

## Complexity characteristics of encoder quantization and quantization type selection

The complexity of the encoder quantization and encoder quantizer type selection is the same as in HEVC. Only one quantization type is used, so no quantization type selection takes place on the encoder side.

## Complexity characteristics of encoder in-loop filtering type selection and parameter estimation

As in HM encoder software, the determination of the SAO parameters is done from the collected statistics of the reconstructed samples, then the picture is only run through once. Next, the determination of the SAO palette with RDO is made iteratively using these statistics, as described in 3.1.8. The average number of iterations when SAO is enabled in the slice, is 9.8 for SAO of same block size as the CTU and 8.9 for SAO of block size equal to ¼ of the CTU size for HD pictures (measured values for one second of RA configuration).

## Complexity characteristics of encoder entropy coding type selection

The proposed codec uses only context-based binary arithmetic coding and therefore there is no selection of entropy coding type necessary. No change compared to JVET-J0021 is brought here.

## Complexity characteristics of additional encoder tools

A stand-alone C++ version of the CNN module has been developed. It can be used as an inference engine by the encoder, without any dependency to external library.

The code footprint of the framework consists in approximately 1600 lines, mostly within header files. It executes on CPU, using a single core. The code is straightforward and only one hand-crafted vectorization for the convolution was done.

The CNN inference module works with the network described above but can be extended to other layers or layout. As a comparison, for a single HD frame, the GPU version takes around 180ms/frame (on a GTX 1070), the optimized CPU version of Tensorflow on one core around 2s/frame, and our module around 3.4s/frame.

## Complexity characteristics of 360° video specific encoding tools

As in 5.12, a table of delta QP is built for the whole sequence. Its size corresponds to the number of lines of CTUs. The spatial adaption of quantization leads to a different RDO complexity per CTU.

## Complexity characteristics of HDR specific encoding tools

The HDR pre-processing tools used in this response apply independently per luma or chroma sample.

The Dynamic range adaptation is based on static mapping functions. A maximum of 3 look-up tables are built (denoted LUTDraY, LUTDraU and LUTDraV), directly derived from one single scaling table coded in the PPS. The mapping process then consists in performing a per-sample conversion, involving for each luma sample one access to the LUT LUTDraY indexed by the luma sample, and for each chroma sample (U/V), one access to the LUT LUTDraU/V indexed by the co-located luma sample, 1 multiplication, 2 additions, 1 shift and one clipping between 0 and (2Bchr – 1).

For deriving the refinement tables, one full scanning of the picture is initially required to computed parameters such as sample mean, variance, cross-correlation between the decoded and the input samples. The tables optimization processes then only use these parameters and has a very limited encoding time impact.

## Degree of capability for encoder parallel processing

The same capability for encoding processing as in JVET-J0021 is provided. This includes the following potential parallelization methods.

- Frame level parallel coding.

- Pipelining of encoding operations (e.g. residual computation, transform and quantization, binarization process) as could already be done with HEVC.

# Algorithmic characteristics

The same GOP structures as in JVET-J0021 are used in this contribution.

## Random access characteristics

Same as JEM7.

## Lower Delay characteristics

Same as JVET-J0021.

# Software implementation description

## Software structure

The software is implemented using modern C++. Old C deprecated code has been removed.

It uses modern memory management and more separation of the different tools than in JEM (see statistics on the number of files below).

It follows the software guidelines for license, syntax and formatting as recommended in JCTVC-H1001.

A macro for each tool has been set, as well as a configuration file option. All tools can be disabled in order to go back to a version similar to HM, with added topology capabilities.

A set of speed-ups can be activated/deactivated from the configuration file.

The software includes the lib360 add-ons and the VR codec can be built using a cmake option.

The CNN code is integrated in the codec and can be used as an inference engine by the encoder, without any dependency to external library.

## Software statistics and portability

The number of lines of code (as measured by cloc) is the following:

|  | HM16.16 | JEM7 | MTT (*) |
| --- | --- | --- | --- |
| Encode | 48468 lines, 115 files | 105432 lines, 121 files | 93004 lines, 174 files |
| Decode | 32669 lines, 100 files | 72980 lines, 102 files | 63186 lines, 127 files |
| All | 58126 lines, 147 files | 119368 lines, 151 files | 104402 lines, 202 files |

(*) excluding CNN model files.

Note that ~10% of the the source code is code for retro-compatibility when tools macro are off.

Cmake is used for the build system, allowing to target several platforms and configurations. The software has been tested with the following configurations:

- Linux : gcc 4.8, gcc 5.3, gcc6.4, gcc 7.2, clang 5.0

- Windows: Visual Studio 2015, Visual Studio 2017, Cygwin gcc 6.4, mingw gcc 7.1

The code has been inspected using static code analyzer (cppcheck, clang-tidy) as well as memory profiler (valgrind memcheck).

# Closing remarks

## General

The MTT codec and software platform is being proposed in contribution JVET-J0021 and in the current contribution JVET-J0022, in response to the CfP on video coding technology, for the SDR, HDR and VR360 categories.

## Proposed new tools or adaptations

The current contribution JVET-J0022 describes a medium complexity version of MTT, with the following main coding tools evolutions compared to the JEM and to the low-complexity MTT version presented in JVET-J0021:

- Introduction of a new partitioning mode, named Asymmetric Binary Tree (ABT) on top of the QT and BT already defined in the JEM, plus an adapted picture border splitting

- Introduction of two intra coding modes: multi-reference intra prediction, and bi-direction intra prediction

- Introduction of a new inter coding mode: template matching affine

- Generalization of motion vector pruning and OBMC

- Bi-directional illumination compensation

- Limitation of the number of new transforms

- Adaptation of SAO using a palette-based process

- Introduction of two HDR tools: dynamic range adaptation and post-decoding refinement

- For the VR360 category, introduction of a normative adaptive quantization for PERP format

## Control of the encoding combinatory

Fast encoding solutions, based on heuristics and caching mechanisms in the RD-cost decision and on a deep-learning approach for Intra slice partitions prediction, have been implemented to control the encoding combinatory. This allows reaching a wide range of trade-offs coding efficiency / encoding complexity. This is illustrated on 3 SDR random access configurations, with performance compared to the JEM ranging from 11.8% BDR-Y gain with 59% JEM encoding runtime, to 15.6% BDR-Y gain with 127% JEM encoding runtime.

## Objective performance in the CfP response

The MTT version used for generating the bitstreams delivered to the CfP provides objective quality gain in SDR RA configuration of 41.9% over the HM at 7.3 times the HM encoding runtime and 5.8 times the HM decoding runtime, and of 13.6% over the JEM at 0.9 times the JEM encoding runtime and 0.8 times the JEM decoding runtime.

For the Low delay configuration, the objective quality gain is of 33.8% over the HM at 10.5 times the HM encoding runtime and 5.2 times the HM decoding runtime, and of 12.7% over the JEM at 1.2 times the JEM encoding runtime and 0.9 times the JEM decoding runtime.

## Software availability and characteristics

A medium complexity version of the MTT software is attached to the contribution. It has the same basis as the MTT version of JVET-J0021, incremented by the additional tools summarized above. The implementation is based on modern C++ programming, and follows the guidelines from JCTVC-H1001. The software allows producing a similar behavior as the HM. The various additional tools and adaptations can be controlled using macros or configuration parameters. The software integrates CNN code that can be used as an inference engine by the encoder, without any dependency to external library. The software is also compatible with the lib360 add-ons.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex 1: Questionnaire

## Codec level questions

| Codec Level Q&A |  | HM | JEM | MTT mode ABT |
| --- | --- | --- | --- | --- |
| 1 | Maximum number of reference frames used by encoder | 4 | 4 | 4 |
| 2 | Maximum coding unit | luma 64×64 chroma 32×32 | luma 128×128 chroma 64×64 | luma 256x256 chroma 128x128 |
| 3 | Minimum and maximum transform block size | luma 4×4/32×32 chroma 4×4/16×16 | luma 4×4/128×128 chroma 2×2/64×64 | luma 4×4/256x256 chroma 2×2/128x128 |
| 4 | Minimum and maximum intra prediction block | luma 4×4 chroma 4×4 | luma 4×4 chroma 2×2 | luma 4×4 to 256x256 chroma 2×2 to 128x128 |
| 5 | Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length | luma 4×8/8×4 (uni-pred) and 8×8 (bi-pred), 8-tap Chroma 2×4/4×2 (uni-pred) and 4×4 bi-pred, 4-tap | luma 4×4 (uni/bi-pred), 8-tap Chroma 2×2 (uni/bi-pred), 4-tap | luma 4×4 (uni/bi-pred), 8-tap Chroma 2×2 (uni/bi-pred), 4-tap |
| 6 | List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category) | n/a | template matching, LIC | template matching, template affine, LIC |
| 7 | List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access | n/a | 64-bit arithmetic for LIC, BIO and LMC | 64-bit arithmetic for LIC, BIO and LMC |
| 8 | List all the tools that use non-rectangular (e.g. triangle) partitions | n/a | n/a | n/a |
| 9 | Indicate whether reference frame resampling is used | n/a | n/a | n/a |
| 10 | Indicate whether luma and chroma use separate partitioning structures | No | Yes, in I-slices | Yes, in I-slices |
| 11 | Indicate whether there are parsing dependencies | Entropy parsing depends on intra prediction mode reconstruction, which requires an intra MPM list derivation | Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation | Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation |

## Tool level questions

### Partitioning structure

| Category 1. Partitioning structure |  | HM | JEM | MTT mode ABT |
| --- | --- | --- | --- | --- |
| 1.a | Whether the prediction unit partitioning is different from transform unit partitioning | Yes, PU and TU partitioning for a CU can differ | No | No |
| 1.b | Maximum tree depth | CU maximum depth 4 TU maximum depth 3 PU maximum depth 1 | QT maximum depth 5 BT maximum depth 3 | QT maximum depth 5 BT maximum depth - intra: 4 to 6 depending on encoder configuration - inter: 0 to 4, depending on QT depth level, temporal layer and complexity level |

### Entropy coding

| Category 2. Entropy coding |  | HM | JEM | MTT mode ABT |
| --- | --- | --- | --- | --- |
| 2.a | Total number of context models | 186 | 329 | 368 |
| 2.b | Memory storage size for each context model | 8-bit | 30-bit | 30-bit |
| 2.c | Range table size if table look up is used for range update | LPSTable[64×4×8bits]; RenormTable[32× 3bits] | LPSTable[512×64×9bits]; RenormTable[128×4 x3bits] | LPSTable[512×64×9bits]; RenormTable[128×4x3bits]; |
| 2.d | Initialization table size | 3×186×8 bits | 3×329×8 + 329×15×5 bits | 3x368x8 bits |
| 2.e | Other memory associated with context model adaptation | LPS transition table: 64*6 bits MPS transition table: 64*6 bits | n/a | n/a |

### Control data derivation process

| Category 3. Control data derivation process |  | HM | JEM | MTT mode ABT |
| --- | --- | --- | --- | --- |
| 3.a | Temporal motion data storage granularity | 16×16 | 4×4 | 4×4 |
| 3.b | Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation | 1 | 4 | 4 |

### Decoder-side motion refinement

| Category 4. Decoder-side motion refinement |  | HM | JEM | MTT mode ABT |
| --- | --- | --- | --- | --- |
| 4.a | Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement. | n/a | - BIO 2×2D separable 6 taps filter (for gradients), mults per sample in refinement calculation 5+5*4 +1/8+2, minimum processing unit same as regular bi-pred, no SAD calculation - FRUC minimum processing unit 44, sequential PU followed by sub-PU level refinement, number of SAD calculations and additional MC interpolations per unit for MV refinement is not constrained (refinement is terminated when cost no longer decreases); bi-linear filter at MV refinement stage | - BIO: same as JEM - Template matching: no more sub-blocks and limited refinement to 375 steps (or terminated when cost no longer decreases) - Bilateral matching refinement: no more sub-blocks, bi-linear filter for refinement limited to 26 steps - Template affine: 5 steps of refinement |
| 4.b | Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size. | n/a | - DMVR needs to access (H+7+2)×(W+7+2) MC instead of (H+7)×(W+7) in HEVC MC - FRUC not constrained | - Template modes not constrained as in JEM - No more DMVR |

### Intra prediction

| Category 5. Intra prediction |  | HM | JEM | MTT mode ABT |
| --- | --- | --- | --- | --- |
| 5.a | Line buffer size in terms of luma and chroma sample lines | luma 1, chroma 1 | luma 4, chroma 2 | luma 4, chroma 2 |
| 5.b | List all the filters used for intra prediction | Reference sample smoothing: {1,2,1}, Strong intra smoothing filter: 2-tap bilinear filter (only for 32×32 luma TU) Interpolation: 2×32×5 bits (bi-linear 32 phases) | Reference sample smoothing: {1,2,1}, {3,7,14,16,14,7,3}, {4,9,12,14,12,9,4} Interpolation: 2×4×32×8 bits (4 taps, 2 sets, 32 phase) | same as J0021 |
| 5.c | Indicate whether there are cross-component dependencies in intra prediction reconstruction | n/a | LM-CHROMA (Cb depends on Y, and Cr depends on Cb&Y) | same as J0021 |

### Inter prediction

| Category 6. Inter prediction |  | HM | JEM | MTT mode ABT |
| --- | --- | --- | --- | --- |
| 6.a | Motion vector accuracy | luma ¼ pel chroma 1/8 pel | luma 1/16 pel chroma 1/32 pel | luma 1/16 pel chroma 1/32 pel |
| 6.b | Storage size for a motion vector component | 16-bit | 16-bit | 16-bit |
| 6.c | List all the interpolation filters used for luma/chroma motion compensation | luma 8-tap, 8×4×6 bits chroma 4-tap, 4×8×6 bits | luma 8-tap, 8×16×6 bits chroma 4-tap, 4×32×6 bits | luma 8-tap, 8×16×6 bits chroma 4-tap, 4×32×6 bits |

### Quantization

| Category 7. Quantization |  | HM | JEM | MTT mode ABT |
| --- | --- | --- | --- | --- |
| 7.a | Storage size associated with quantization | n/a | n/a | n/a |

### Transforms

| Category 8. Transforms |  | HM | JEM | MTT mode ABT |
| --- | --- | --- | --- | --- |
| 8.a | List all the combinations of transform block size and transform type for primary transforms | - 8-bit 4×4, 8×8, 16×16 and 32×32 DCT-II, - 8-bit 4×4 DST VII | HEVC transform mode (10-bit DCT-II), transform size combinations: - (4, 8, 16, 32, 64, 128) × (4, 8, 16, 32, 64, 128) Tus for luma CTUs - (2, 4, 8, 16, 32, 64) × (2, 4, 8, 16, 32, 64) Tus for inter-coded chroma CTUs - (2, 4, 8, 16, 32, 64,) × (2, 4, 8, 16, 32, 64) TUs for intra-coded chroma CTUs EMT transform mode (10-bit DST VII, DCT-V, DCT-VIII, DST-I), only applied to CTUs up to 64×64, transform size combinations: (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TUs for luma CTUs | HEVC transform mode (10-bit DCT-II), transform size combinations: - (4, 8, 16, 32, 64, 128,256,12,24,48) × (4, 8, 16, 32, 64, 128,256,12,24,48) TUs for luma CTUs - (2, 4, 8, 16, 32, 64,128,256,6,12) × (2, 4, 8, 16, 32, 64,128,256,6,12) TUs for inter-coded chroma CTUs - (2, 4, 8, 16, 32, 64,128,6,12,24) × (2, 4, 8, 16, 32, 64,128,6,12,24) TUs for intra-coded chroma CTUs EMT transform mode in Intra (10-bit DST VII, DST-IV, DCT-VIII), applied to CTUs up to 64×64, transform size combinations EMT transform modes in inter: same as JVET-J0021 |
| 8.b | List all the combinations of transform block size and transform type for secondary transforms | n/a | 35×3 4×4 and 8×8 non-separable Hypercube-Givens Transform (HyGT) | 35×3 4×4 and 8×8 non-separable Hypercube-Givens Transform (HyGT), as in JVET-J0021 |
| 8.c | List all the non-separable transforms used | n/a | 4×4/8×8 HyGT | 4×4/8×8 HyGT, as in JVET-J0021 |
| 8.d | Indicate whether transforms can be fully/partially implemented using direct matrix multiply | partial factorization equivalent to matrix multiplication | factorized form not equivalent to matrix multiplication | factorized form not equivalent to matrix multiplication |
| 8.e | List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output) | n/a | 4×4 HyGT 2-interations, 8×8 HyGT 4-interations | 4×4 HyGT 2-interations, 8×8 HyGT 4-interations |

### In-loop filtering

| Category 9. In-loop filtering |  | HM | JEM | MTT mode ABT |
| --- | --- | --- | --- | --- |
| 9.a | List line buffer size in terms of luma and chroma lines (if any) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) ALF (luma 8, chroma 4) | Deblocking: same as JVET-J0021 |
| 9.b | List all the filters used for in-loop filtering | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. ALF: 9×9/7×7/5×5 diamond filter support for luma; and 5×5 diamond filter support for chroma | Deblocking: same as JVET-J0021 ALF: same as JVET-J0021 |
| 9.c | Minimum processing unit size | Deblocking (luma: 8×8 grid with 4×4 filter decision, chroma 8×8 grid with 4×4 filter decision) | Deblocking (luma 4×4 grid with 4×4 filter decision, chroma 8×8 grid with 2×2 filter decision) ALF: for luma, filter decision is based on 2×2 block Bilateral filter: sample level filter switch (luma only) | Deblocking: same as JVET-J0021 ALF: same as JVET-J0021 Bilateral filter: same as JEM |

### “tool-off” and “tool-on” tests

The following table reports the per-tool evaluation, focused on the specific tools presented in the current contribution. It completes the report made in JVET-J0021. The per-tool evaluation was done on 1 intra period, in RA configuration, over the complete sequence test set. Different tools (ABT, picture border partitioning, multi-reference and bi-directional intra prediction, improvements in motion coding, SAO palette) aim at providing coding gain. A few others are more focused on complexity reduction (generalized OBMC, reduction of transforms, limitation on EMT-NSST interaction). They are indicated in italic font in the table.  Different tool combinations were also tested.

| Tool name | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| TOPOLOGY | -5.0% | -5.6% | -5.5% | 185% | 98% | -4.3% | -6.4% | -6.6% | 201% | 101% |
| ABT (+QTBT) | -3.8% | -4.1% | -4.3% | 186% | 99% | -3.2% | -5.0% | -5.2% | 192% | 101% |
| Pict. Border partition. | -1.4% | -2.0% | -1.9% | 102% | 99% | -0.9% | -1.1% | -1.2% | 103% | 100% |
| INTRA | -0.3% | -0.7% | -0.5% | 104% | 100% | -0.4% | -0.5% | -0.3% | 113% | 103% |
| Multi-ref intra pred | -0.2% | -0.3% | -0.3% | 102% | 101% | -0.2% | -0.4% | -0.2% | 109% | 102% |
| Bi-dir intra pred | -0.2% | -0.3% | 0.0% | 102% | 100% | -0.2% | 0.0% | -0.1% | 103% | 100% |
| MVCODING+MC | -0.6% | -0.5% | -0.4% | 115% | 107% | -0.7% | -0.7% | -0.7% | 123% | 110% |
| Extended affine MC | -0.3% | -0.2% | -0.1% | 110% | 106% | -0.5% | -0.6% | -0.7% | 111% | 115% |
| Bi-dir IC | -0.1% | -0.1% | 0.1% | 104% | 105% | -0.2% | -0.2% | -0.1% | 104% | 108% |
| Template Merge | -0.1% | -0.1% | 0.2% | 102% | 97% | 0.0% | 0.1% | -0.1% | 100% | 93% |
| Generalized OBMC | -0.1% | -0.1% | 0.1% | 102% | 99% | 0.0% | -0.2% | 0.0% | 99% | 99% |
| TRANSFORM | 0.1% | 0.0% | -0.1% | 99% | 100% | 0.1% | -0.3% | -0.2% | 96% | 100% |
| Transforms Reduction | 0.0% | 0.2% | 0.2% | 100% | 100% | 0.0% | -0.1% | 0.4% | 100% | 100% |
| Interaction EMT-NSST | 0.1% | -0.1% | 0.2% | 97% | 100% | 0.1% | -0.3% | -0.3% | 96% | 101% |
| LOOP FILTER | -0.2% | -0.5% | -0.5% | 100% | 101% | -0.3% | -0.6% | -0.7% | 100% | 108% |
| SAO palette | -0.2% | -0.5% | -0.5% | 100% | 101% | -0.3% | -0.6% | -0.7% | 100% | 108% |

For HDR category, the per-tool evaluation is achieved in the tool-off configuration. Results are reported in the following table, only for PQ content since for HLG content, DRA was de-activated. Encoding and decoding times are not impacted by the tools (minor variations around 100%).

| Tool name | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | Enc | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DRA+refinement | -44.3% | -9.8% | -3.8% | -49.2% | -83.8% | 97% | 100% |
| DRA | -42.6% | -9.9% | -3.6% | -45.7% | -81.1% | 95% | 101% |
| refinement | -3.6% | 0.0% | -0.1% | -8.2% | -13.3% | 101% | 98% |

For VR360 category, the per-tool evaluation is achieved in the tool-off configuration. Results are reported in the following table. It should be noted that the encoding time is reduced to 93% and decoding time to 94%.

| Tool name | BD-rate E2E WS-PSNR |  |  | BD-rate E2E SPSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Enc | Dec |
| Adaptive QP | -3.0% | -3.1% | -4.6% | -3.0% | -3.0% | -4.6% | 93% | 94% |

# Annex 2: References

[1] JVET-J0021 Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions

[2] “Core Transform Design in the High Efficiency Video Coding (HEVC) Standard”, Madhukar Budagavi, Arild Fuldseth, Gisle Bjøntegaard, Vivienne Sze and Mangesh Sadafale, IEEE JOURNAL OF SELECTED TOPICS IN SIGNAL PROCESSING, VOL. 7, NO. 6, DECEMBER 2013

[end of document]