| buJoint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0025 |
| --- | --- |

| Title: | Description of SDR, HDR and 360° video coding technology proposal by Huawei, GoPro, HiSilicon, and Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen, Jianle Chen, Roman Chernyak, Semih Esenlik, Alexey Filippov, Shan Gao, Sergey Ikonin, Alexander Karabutov, Anand Meher Kotra, Xiaomu Lu, Xiang Ma, Vasily Rufitskiy, Timofey Solovyev, Victor Stepin, Maxim Sychev, Tian Wang, Ye-kui Wang, Weiwei Xu, Haitao Yang, Vladyslav Zakharchenko, Hong Zhang, Yin Zhao, Zhijie Zhao, Jiantong Zhou, Cheung Auyeung, Han Gao, Ivan Krasnov, Ruslan Mullakhmetov, Biao Wang, Yiu Fai Wong, Georgy Zhulikov(Huawei) Adeel Abbas, David Newman (GoPro) Jicheng An, Xu Chen, Yongbing Lin, Quanhe Yu, Jianhua Zheng (HiSilicon) Alexander Alshin, Elena Alshina, Kiho Choi, Narae Choi, Woongil Choi, Seungsoo Jeong, Chanyul Kim, Junghye Min, JeongHoon Park, Minsoo Park, Min Woo Park, Yinji Piao, Anish Tamse (Samsung) | Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1-408-330-5334 jianle.chen@huawei.com aabbas@gopro.com +86-010-82829521 zhengjianhua@hisilicon.com +82-10-8855-2932 elena_a.alshina@samsung.com kiho14.choi@samsung.com |
| Source: | Huawei, GoPro, HiSilicon, and Samsung Electronics |  |  |

_____________________________

# Abstract

This proposal is a joint response to the Call for Proposals (CfP) on Video Compression with Capability beyond HEVC, jointly issued by ITU-T SG16 Q.6 (VCEG) and ISO/IEC JTC1/SC29/WG11 (MPEG). It has been produced in collaboration with Samsung, Huawei, GoPro and HiSilicon. The goal of this proposal is to provide a video compression technology which has significantly higher compression capability than the state-of-the-art HEVC standard for all the three categories. The key highlights of this proposal are the following two aspects: 1) considering requirements of manufacture company and 2) using the same codec engine for all three categories. To achieve this goal, a number of algorithmic tools are proposed on top of a basic structure covering several aspects of prior art video compression technology. These include a flexible structure for representation of video content, inter/intra prediction, in-loop filtering, and entropy coding. When all the proposed algorithmic tools are used, the proposed video codec achieves approximately 37.2% bit-saving for SDR, 42.2% bit-saving for HDR, and 33.1% bit-saving for 360 degree contents, respectively, on average compared to HEVC anchors.

# Contents

Abstract	i

Contents	ii

Glossary of terminology	1

1	Introduction	2

1.1	Background	2

1.2	Design philosophy	2

1.3	New tools	2

2	Decoder algorithm description	5

2.1	General algorithm description	5

2.1.1	Bitstream structure	5

2.1.2	Entropy decoding	19

2.1.3	Conversion process from entropy decoded data to symbols	20

2.1.4	Decoder-side motion refinement	27

2.1.5	Inverse quantization	29

2.1.6	Inverse transforms	29

2.1.7	Motion compensation	30

2.1.8	Intra prediction	32

2.1.9	In-loop filtering	33

2.1.10	Additional tools	35

2.1.11	Additional algorithmic description topic(s)	36

2.2	360° Video	36

2.2.1	Projection format (including padding)	36

2.2.2	Quantization handling	39

2.2.3	360° video specific decoding tools	39

2.3	HDR	40

2.3.1	Post-processing	40

2.3.2	Quantization handling	41

2.3.3	HDR specific decoding tools	41

3	Encoder algorithm description	41

3.1	General algorithm description	41

3.1.1	Rate-distortion optimization	41

3.1.2	Partitioning structure selection	42

3.1.3	Intra prediction mode estimation and type selection	46

3.1.4	Motion estimation and motion segment selection	54

3.1.5	Mode decisions	74

3.1.6	De-blocking filtering	74

3.1.7	Transforms and transform type selection	80

3.1.8	Quantization and quantization type selection	83

3.1.9	Entropy coding	83

3.1.10	Additional tools	87

3.1.11	Additional encoder optimization	88

3.1.12	Additional algorithmic description topic(s)	88

3.2	360°	88

3.2.1	Usage of projection format adaptation	88

3.2.2	360° Video specific encoding tools	94

3.2.3	Additional encoder optimization	94

3.3	HDR	95

3.3.1	Pre-processing	95

3.3.2	HDR specific encoding tools	96

3.3.3	Additional encoder optimization	97

4	Compression performance	97

4.1	SDR: Constraint set 1 configuration relative to HM anchor	97

4.1.1	Class SDR-A (RD-curves)	98

4.1.2	Class SDR-B (RD-curves)	99

4.1.3	Overall (BD-rate summary tables)	99

4.2	SDR: Constraint set 2 configuration relative to HM anchor	100

4.2.1	Class SDR-B	100

4.2.2	Overall	101

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	102

4.3.1	Class SDR-A	102

4.3.2	Class SDR-B	103

4.3.3	Overall	103

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	104

4.4.1	Class SDR-B	105

4.4.2	Overall	105

4.5	HDR: Constraint set 1 configuration relative to HM anchor	106

4.5.1	Class HDR-A	106

4.5.2	Class HDR-B	107

4.5.3	Overall	107

4.6	HDR: Constraint set 1 configuration relative to JEM anchor	108

4.6.1	Class HDR-A	108

4.6.2	Class HDR-B	109

4.6.3	Overall	109

4.7	360° Video: Constraint set 1 configuration relative to HM anchor	110

4.7.1	Class 360	110

4.7.2	Overall	111

4.8	360° Video: Constraint set 1 configuration relative to JEM anchor	111

4.8.1	Class 360	111

4.8.2	Overall	112

5	Decoder complexity analysis	113

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	113

5.2	Description of computing platform used to determine decoding times reported in section 5.1	113

5.3	Memory usage of decoder	113

5.4	Complexity characteristics of decoder entropy decoding operation	113

5.5	Complexity characteristics of decoder inverse quantization	113

5.6	Complexity characteristics of decoder inverse transform operation	113

5.7	Complexity characteristics of decoder-side motion refinement	113

5.8	Complexity characteristics of decoder motion compensation	113

5.9	Complexity characteristics of decoder intra-frame prediction operation	114

5.10	Complexity characteristics of decoder in-loop filtering operation	114

5.11	Complexity characteristics of additional decoder tools	114

5.12	Complexity characteristics of 360° video specific decoding tools	114

5.13	Complexity characteristics of HDR specific decoding tools	114

5.14	Degree of capability for decoder parallel processing	114

6	Encoder complexity analysis	114

6.1	Encoding time and measurement methodology	114

6.2	Description of computing platform used to determine encoding times reported in section 6.1	114

6.3	Memory usage of encoder	114

6.4	Complexity characteristics of encoder intra-frame prediction type selection	115

6.5	Complexity characteristics of encoder motion estimation and motion segmentation selection	115

6.5.1	General	115

6.5.2	Complexity characteristics of usage of decoder-side motion refinement	115

6.6	Complexity characteristics of encoder transforms and transform type selection	115

6.7	Complexity characteristics of encoder quantization and quantization type selection	115

6.8	Complexity characteristics of encoder in-loop filtering type selection and parameter estimation	116

6.9	Complexity characteristics of encoder entropy coding type selection	116

6.10	Complexity characteristics of additional encoder tools	116

6.11	Complexity characteristics of 360° video specific encoding tools	116

6.12	Complexity characteristics of HDR specific encoding tools	116

6.13	Degree of capability for encoder parallel processing	116

7	Algorithmic characteristics	116

7.1	Random access characteristics	116

7.2	Delay characteristics	116

7.3	Additional characteristics discussion topic(s)	116

8	Software implementation description	116

9	Highlighted aspects discussion	116

10	Closing remarks	117

11	Patent rights declaration(s)	117

Reference	117

12	Annex 1: Questionnaire	119

12.1	Codec level questions	119

12.2	Tool level questions	119

13	Annex 2: Example of filling questionnaire HM vs. JEM	121

13.1	Codec level Q&A	121

13.2	Tool level Q&A	122

13.2.1	Category 1. Partitioning structure	122

13.2.2	Category 2. Entropy coding	123

13.2.3	Category 3. Control data derivation process	123

13.2.4	Category 4. Decoder-side motion refinement	123

13.2.5	Category 5. Intra prediction	124

13.2.6	Category 6. Inter prediction	125

13.2.7	Category 7. Quantization	125

13.2.8	Category 8. Transforms	125

13.2.9	Category 9. In-loop filtering	127

13.2.10	Category 10. “tool-off” and “tool-on” tests	128

# Glossary of terminology

Terminology used in this proposal is described in this section.

- ALF: adaptive loop filtering

- AMVR: adaptive motion vector resolution

- BIO: bi-directional optical flow

- BT: binary tree

- BTT: binary and triple tree

- CCIP: Cross-component intra prediction

- CU: coding unit

- CTU: coding tree unit

- DMVR: decoder-side motion refinement

- DMVD: decoder-side motion derivation

- DWDIP: distance weighted directional intra prediction

- IPR: inter prediction refinement

- LCCM: luma chroma coupling mode

- MC: motion compensation

- MCABAC: multi-hypothesis probability update

- ME: motion estimation

- MIP: multi-combined intra prediction

- MPM: most probable mode

- MTR: multiple core transform

- MV: motion vector

- MVD: motion vector difference

- MVP: motion vector predictor

- MVR: motion vector resolution

- MVSD: movtion vector difference sign derivation

- MsSSIM: multiple scales structural similarity

- NSF: noise suppression filtering

- QT: quad tree

- RDO: rate distortion optimization

- RDOQ: rate distortion optimized quantization

- SAO: sample adaptive offset

- STR: secondary transform

- SU: split unit

- SUCO: split unit coding order

- SVT: spatial varying transform

- TT: triple tree

- TD-RSP: transform domain residual sign prediction

- UMVE: ultimate motion vector expression

# Introduction

## Background

The technical obstacles for moving from 2K Full High Definition (FHD) broadcasting service quality to 4K Ultra-High Definition (UHD) were solved by adopting HEVC as the main feature of Next Generation 4K Broadcasting standard such as ATST3.0, DVB-UHD. However, the emergence of 8K UHD commercial broadcasting services, High Dynamic Range (HDR) and Wide Color Gamut (WCG) and 360 degree virtual reality videos that can captures all-around scenes accelerate the user requirements for both high quality video and high demanding bandwidth of network. According to the survey of a network provider, video content has much higher bit rates than other types of contents and it will generate three-quarters of data traffic by 2020. As OTT companies take a position of multimedia service market rulers, they have required more efficient compression of video than what is possible with the existing HEVC. This requirement has led to the formation of Joint Video Expert Team (JVET), which has jointly issued a Call for Proposals (CfP) on video compression technology.

This proposal is four companies’ joint response to the CfP on Video Compression with Capability beyond HEVC. It aims to provide a video compression technology which has significantly higher compression capability than HEVC/H.265 standard in all three categories. To achieve this goal, a number of new algorithmic tools are proposed, covering many aspects of video compression technology. These include a general structure for the representation of video content, inter/intra prediction, in-loop filtering, and entropy coding.

## Design philosophy

The philosophy followed for the codec design of this contribution is to 1) consider requirements of manufacturing companies, especially the decoder aspect, 2) design a minimum set of tools, 3) take prior art technologies and construct minimum sets of new tools on top of the basic structure using them, and 4) use the same codec engine for all the three categories.

In order to achieve mentioned goals, this proposal tried an approach to build a video codec using only conventional coding technologies which consist of traditional methods from the 1980s up to the end of the 1990s. Each proposed coding algorithm has the prior arts with disclosed documents such as patents, papers or standardization proposals [1]-[11]. On top of the basic structure, new tools were added to improve coding performance.

For HDR and 360 degree contents, pre/post processing was conducted on top of the codec engine that was used for SDR as the basic codec engine.

## New tools

For efficient and flexible representation of video content with various resolutions, a new flexible partitioning method with coding order is introduced as follows:

- Bi-tree and tri-tree mixture scheme (BTT)

- Split unit coding order (SUCO)

To improve inter prediction performance, a number of novel algorithmic tools are proposed as follows:

- Adaptive motion vector resolution (AMVR)

- Ultimate motion vector expression (UMVE)

- Affine motion prediction

- Inter prediction refinement (IPR)

- Decoder-side motion vector refinement (DMVR)

- Bi-directional optical flow (BIO)

- Motion vector difference signs derivation (MVDS)

- Overlapped block motion compensation (OBMC)

- Decoder-side motion derivation (DMVD)

To improve intra prediction performance, a number of novel algorithmic tools are proposed as follows:

- Extended intra prediction with 52 modes

- Multi-combined intra prediction (MIP)

- Distance-weighted direction intra prediction (DWDIP)

- Cross-component intra prediction (CCIP)

- Reference sample sharpening filter

To improve transform coding and entropy coding, a number of novel algorithmic tools are proposed as follows:

- Multiple core transform (MTR)

- Secondary transform (STR)

- Spatial varing transform (SVT)

- Adaptive quantization step size scaling

- Transform domain residual sign prediction (TD-RSP)

- Multi-hypothesis probability update (MCABAC)

To improve inloop filtering, a number of novel algorithmic tools are proposed as follows

- Noise suppression filtering (NSF)

- Adaptive loop filtering (ALF)

- Bilateral filtering (BLF)

To improve inloop filtering, adaptive clipping is applied:

- Adaptive clipping

To improve HDR contents coding, the following methods are applied:

- Pre-processing with Anisotropic SSD

- Remapping

To improve 360 degree contents coding, the following methods are applied:

- Rotated Sphere Projection (RSP) format with padding

The proposed solution in this contribution are developed on top of the proposed solution in JVET-J0024. The additional tools are in bold as lised above.

Figure 1-1. Encoder structure

Figure 1-2. Decoder structure

# Decoder algorithm description

## General algorithm description

### Bitstream structure

#### Sequence header syntax

| sequence_header( ) { | Descriptor |
| --- | --- |
| pic_width_in_luma_sample | u(16) |
| pic_height_in_luma_sample | u(16) |
| closed_gop_flag | u(1) |
| log2_ctu_size_minus1 | u(3) |
| log2_max_depth_minus1 | u(3) |
| seq_chroma_qp_offset | se(v) |
| slice_chroma_qp_offset_present_flag | u(1) |
| cu_delta_qp_enabled_flag | u(1) |
| chroma_qp_mapping_table_idc | u(1) |
| seq_chroma_qp_split_flag | u(1) |
| sao_chroma_enabled_flag | u(1) |
| 360_tool_info_signal_flag | u(1) |
| reserved_bits | f(12) |
| if( 360_tool_info_signal_flag ) { |  |
| in_loop_filter_off_flag | u(1) |
| beta_flag | u(1) |
| pad_flag | u(1) |
| geo_type | u(4) |
| yaw_rot | ue(v) |
| pitch_rot | ue(v) |
| roll_rot | ue(v) |
| if( beta_flag ) { |  |
| beta_val | ue(v) |
| } |  |
| if( pad_flag ) { |  |
| omni_arc_gran | ue(v) |
| omni_pad_sz | ue(v) |
| omni_active_pad_sz | ue(v) |
| } |  |
| } |  |
| if( seq_chroma_qp_split_flag ) { |  |
| delta_cb_qp_offset | se(v) |
| delta_cr_qp_offset | se(v) |
| } |  |
| log2_cu_11_ratio_max_minus2 | ue(v) |
| log2_cu_11_ratio_min_minus2 | ue(v) |
| log2_cu_12_ratio_max_minus2 | ue(v) |
| log2_cu_12_ratio_min_minus2 | ue(v) |
| log2_cu_14_ratio_max_minus2 | ue(v) |
| log2_cu_14_ratio_min_minus2 | ue(v) |
| log2_tri_split_max_minus2 | ue(v) |
| log2_tri_split_min_minus2 | ue(v) |
| seq_suco_depth_max | ue(v) |
| seq_suco_depth_min | ue(v) |
| seq_mip_flag | u(1) |
| seq_ccip_flag | u(1) |
| seq_mtr_flag | u(1) |
| seq_str_flag | u(1) |
| seq_amvr_flag | u(1) |
| seq_umve_flag | u(1) |
| seq_bio_flag | u(1) |
| seq_ipr_flag | u(1) |
| seq_sao_flag | u(1) |
| seq_adapt_clip_flag | u(1) |
| seq_nsf_flag | u(1) |
| seq_alf_flag | u(1) |
| seq_affine_flag | u(1) |
| seq_svt_flag | u(1) |
| seq_dmvr_flag | u(1) |
| seq_rsp_flag | u(1) |
| seq_dwdip_flag | u(1) |
| byte_align( ) |  |
| } |  |

#### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| dtr | u(10) |
| slice_type | u(3) |
| key_picture_flag | u(1) |
| slice_deblocking_filtering_flag | u(1) |
| if( seq_sao_flag ) { |  |
| slice_sao_flag | u(1) |
| } |  |
| if( slice_type ! = I && seq_ipr_flag ) { |  |
| slice_ipr_flag | u(1) |
| } |  |
| update_exist | u(1) |
| slice_qp | u(6) |
| if( slice_type ! = I ) { |  |
| dptr | se(v) |
| } |  |
| temporal_layer_id | u(3) |
| mmco_flag | u(1) |
| if( mmco_flag ) { |  |
| for( cnt = 0; cnt < 16; cnt++ ) { |  |
| mmco_type | ue(v) |
| if( mmco_type = = 1) |  |
| break |  |
| mmco_data | ue(v) |
| } |  |
| } |  |
| rmpni_flag | u(1) |
| if( rmpni_flag ) { |  |
| for( cnt = 0; cnt < 16; cnt++ ) { |  |
| rmpni_type_list0 | ue(v) |
| if( rmpni_type_list0 = = 0 ) |  |
| break |  |
| rmpni_data_list0 | ue(v) |
| } |  |
| for( cnt = 0; cnt < 16; cnt++ ) { |  |
| rmpni_type_list1 | ue(v) |
| if( rmpni_type_list1 = = 0 ) |  |
| break |  |
| rmpni_data_list1 | ue(v) |
| } |  |
| } |  |
| if( seq_adapt_clip_flag ) { |  |
| adapt_clip_max_signal_flag | u(1) |
| if( adapt_clip_max_signal_flag ) { |  |
| adapt_clip_max_val | u(4) |
| } |  |
| adapt_clip_min_signal_flag | u(1) |
| if( adapt_clip_min_signal_flag ) { |  |
| adapt_clip_min_val | u(4) |
| } |  |
| } |  |
| byte_align( ) |  |
| } |  |

#### Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| do { |  |
| coding_tree_unit( ) |  |
| if( slice_type = = I ) { |  |
| coding_tree_unit( ) |  |
| } |  |
| } while( !slice_end_flag ) |  |
| byte_align( ) |  |
| } |  |

#### Coding tree unit syntax

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| split_unit( x, y ) |  |
| if( slice_sao_flag ) { |  |
| sample_adaptive_offset( x, y ) |  |
| } |  |
| } |  |

#### Sample adaptive offset syntax

| sample_adaptive_offset( x, y ) { | Descriptor |
| --- | --- |
| sao_mode | ae(v) |
| if( sao_mode = = SAO_MODE_ON ) { |  |
| for( comp = 0; comp < (sao_chroma_enabled_flag ? 3 : 1); comp++ ) { |  |
| sao_type | ae(v) |
| if( sao_type = = SAO_EDGE_OFFSET ) { |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ comp ] [ i ] | ae(v) |
| } |  |
| else if( sao_type = = SAO_BAND_OFFSET ) { |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ comp ] [ i ] | ae(v) |
| if ( sao_offset_abs[ comp ] [ i ] ! = 0 ) |  |
| sao_offset_sign[ comp ] [ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

#### Split unit syntax

| split_unit( ) { | Descriptor |
| --- | --- |
| split_mode | ae(v) |
| if( split_mode = = QUAD_SPLIT ¦ ¦ split_ mode = = BI_VER_SPLIT ¦ ¦ |  |
| split_ mode = = TRI_VER_SPLIT ) { |  |
| suco_info | ae(v) |
| } |  |
| if( split_info = = QUAD_SPLIT ) { |  |
| if( suco_info = = 0 ) { |  |
| split_unit( sub_pos_0 ) |  |
| split_unit( sub_pos_1 ) |  |
| split_unit( sub_pos_2 ) |  |
| split_unit( sub_pos_3 ) |  |
| } |  |
| else { |  |
| split_unit( sub_pos_1 ) |  |
| split_unit( sub_pos_0 ) |  |
| split_unit( sub_pos_3 ) |  |
| split_unit( sub_pos_2 ) |  |
| } |  |
| } |  |
| else if( split_ mode = = BI_VER_SPLIT ) { |  |
| if( suco_info = = 0 ) { |  |
| split_unit( sub_pos_0 ) |  |
| split_unit( sub_pos_1 ) |  |
| } |  |
| else { |  |
| split_unit( sub_pos_1 ) |  |
| split_unit( sub_pos_0 ) |  |
| } |  |
| } |  |
| else if( split_ mode = = BI_HOR_SPLIT ) { |  |
| split_unit( sub_pos_0 ) |  |
| split_unit( sub_pos_1 ) |  |
| } |  |
| else if( split_ mode = = TRI_VER_SPLIT ) { |  |
| if( suco_info = = 0 ) { |  |
| split_unit( sub_pos_0 ) |  |
| split_unit( sub_pos_1 ) |  |
| split_unit( sub_pos_2 ) |  |
| } |  |
| else { |  |
| split_unit( sub_pos_2 ) |  |
| split_unit( sub_pos_1 ) |  |
| split_unit( sub_pos_0 ) |  |
| } |  |
| } |  |
| else if( split_mode = = TRI_HOR_SPLIT ) { |  |
| split_unit( sub_pos_0 ) |  |
| split_unit( sub_pos_1 ) |  |
| split_unit( sub_pos_2 ) |  |
| } |  |
| else { // split_mode = = NO_SPLIT |  |
| coding_unit( x0, y0, log2width, log2height ) |  |
| } |  |
| } |  |

#### Coding unit syntax

| coding_unit( x0, y0, log2width, log2height ) { | Descriptor |
| --- | --- |
| if( slice_type != I && !( log2width <= 2 && log2height <= 2 ) { |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| dmvd_flag[ x0 ][ y0 ] | ae(v) |
| if(dmvd_flag[ x0 ][ y0 ] ) { |  |
| dmvd_mode[ x0 ][ y0 ] | ae(v) |
| if(dmvd_mode[ x0 ][ y0 ]==TM ) |  |
| tm_dir[ x0 ][ y0 ] | ae(v) |
| } |  |
| else { |  |
| cu_skip_umve[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_umve[ x0 ][ y0 ] ) { |  |
| cu_mode = SKIP_UMVE |  |
| mvp_idx_umve | ae(v) |
| } |  |
| else if( log2width >= 3 && log2height >= 3 ) { |  |
| cu_skip_affine[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_affine[ x0 ][ y0 ] ) { |  |
| cu_mode = SKIP_AFFINE |  |
| } |  |
| else { |  |
| mvp_idx[ x0 ][ y0 ] | ae(v) |
| cu_mode = SKIP |  |
| } |  |
| } |  |
| else { |  |
| mvp_idx[ x0 ][ y0 ] | ae(v) |
| cu_mode = SKIP |  |
| } |  |
| } |  |
| } |  |
| else { |  |
| pred_mode_flag[ x0 ][ y0 ] | ae(v) |
| cu_mode = ( pred_mode_flag[ x0 ][ y0 ] = = 0 ) ? MODE_INTRA : MODE_INTER |  |
| if( pred_mode_flag[x0][y0] != MODE_INTRA ) |  |
| pred_mvr_idx[ x0 ][ y0 ] | ae(v) |
| if ( pred_mvr_idx[x0][y0] == 0 ) { |  |
| dmvd_flag[ x0 ][ y0 ] | ae(v) |
| if (dmvd_flag[ x0 ][ y0 ] ) { |  |
| dmvd_mode[ x0 ][ y0 ] | ae(v) |
| if (dmvd_mode[ x0 ][ y0 ]==TM ) |  |
| tm_dir[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| else { |  |
| pred_mode_flag[ x0 ][ y0 ] = MODE_INTRA |  |
| cu_mode = MODE_INTRA |  |
| } |  |
| if( cu_mode = = MODE_INTER ) { |  |
| if(slice_type = = P) { |  |
| if( pred_mvr_idx[ x0 ][ y0 ] = = 0 ) { |  |
| cu_direct[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( cu_direct[ x0 ][ y0 ] ) { |  |
| affine_dir[ x0 ][ y0 ] = 0 |  |
| if( log2width >= 3 && log2height >= 3 ) { |  |
| affine_dir[ x0 ][ y0 ] | ae(v) |
| if( affine_dir[ x0 ][ y0 ] ) { |  |
| affine_flag[ x0 ][ y0 ] = 1 |  |
| cu_mode = DIR_AFFINE |  |
| } |  |
| } |  |
| if( !affine_dir[ x0 ][ y0 ] ) { |  |
| mvp_idx_umve | ae(v) |
| cu_mode = DIR_UMVE |  |
| } |  |
| } |  |
| } |  |
| else { // slice_type = = B |  |
| if( pred_mvr_idx[ x0 ][ y0 ] = = 0 ) |  |
| cu_direct[ x0 ][ y0 ] | ae(v) |
| if( cu_direct[ x0 ][ y0 ] ) { |  |
| cu_direct_umve[ x0 ][ y0 ] | ae(v) |
| if( cu_direct_umve[ x0 ][ y0 ] ) { |  |
| cu_mode = DIR_UMVE |  |
| get_skip_cand( ) |  |
| mvp_idx_umve | ae(v) |
| } |  |
| else{ |  |
| affine_dir[ x0 ][ y0 ] = 0 |  |
| if( log2width >= 3 && log2height >= 3 ) { |  |
| affine_dir[ x0 ][ y0 ] | ae(v) |
| if( affine_dir[ x0 ][ y0 ] ) { |  |
| cu_mode = DIR_AFFINE |  |
| affine_flag[ x0 ][ y0 ] = 1 |  |
| } |  |
| } |  |
| if( !affine_dir[ x0 ][ y0 ] ) { |  |
| cu_mode = DIR |  |
| get_skip_cand() |  |
| mvp_idx_dir[ x0 ][ y0 ][ 0 ] | ae(v) |
| mvp_idx_dir[ x0 ][ y0 ][ 1 ] = mvp_idx_dir[ x0 ][ y0 ][ 0 ] |  |
| } |  |
| } |  |
| } |  |
| else { |  |
| inter_dir[ x0 ][ y0 ] | ae(v) |
| if( inter_dir[ x0 ][ y0 ] ) |  |
| inter_pred_dir[ x0 ][ y0 ] = PRED_BI |  |
| else { |  |
| inter_list[ x0 ][ y0 ] | ae(v) |
| inter_pred_dir[ x0 ][ y0 ] = inter_list[ x0 ][ y0 ] == 1 ? PRED_L1 : PRED_L0 |  |
| } |  |
| affine_flag[ x0 ][ y0 ] = 0 |  |
| if( log2width >= 3 && log2height >= 3 && affine_applicable ) { |  |
| affine_flag[ x0 ][ y0 ] | ae(v) |
| if( affine_flag[ x0 ][ y0 ] ) { |  |
| cu_mode = AFFINE |  |
| affine_flag[ x0 ][ y0 ] = 1 |  |
| affine_mode[ x0 ][ y0 ] | ae(v) |
| num_vertex = affine_flag[ x0 ][ y0 ] + affine_mode[ x0 ][ y0 ] + 1 |  |
| } |  |
| } |  |
| if( !affine_flag[ x0 ][ y0 ] ) { |  |
| if( inter_pred_dir[x0][y0] = = PRED_BI ) |  |
| inter_bi_idx | ae(v) |
| if( log2width + low2height > 5 && ipr_applicable && (pred_mvr_idx[ x0 ][ y0 ]==0 ¦ ¦ pred_mvr_idx[ x0 ][ y0 ] = = 3) |  |
| ipr_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_dir[x0][y0] = = PRED_L0 ¦ ¦ inter_pred_dir[x0][y0] = = PRED_BI ){ |  |
| if( inter_bi_idx = = BI_FL0 ¦ ¦ inter_bi_idx = = BI_FL1 ) |  |
| refi[ x0 ][ y0 ][ 0 ] = get_first_refi( ) |  |
| else |  |
| refi[ x0 ][ y0 ][ 0 ] = ref_idx[ x0 ][ y0 ][ 0 ] | ae(v) |
| if( inter_bi_idx = = BI_FL0 ) |  |
| mvd = ( 0, 0 ) |  |
| else |  |
| mvd[ x0 ][ y0 ][ 0 ] | ae(v) |
| } |  |
| if( inter_pred_dir[ x0 ][ y0 ] = = PRED_L1 ¦ ¦ inter_pred_dir[ x0 ][ y0 ] = = PRED_BI) { |  |
| if( inter_bi_idx = = BI_FL0 ¦ ¦ inter_bi_idx = = BI_FL1 ) |  |
| refi[ x0 ][ y0 ][ 1 ] = get_first_refi( ) |  |
| else |  |
| refi[ x0 ][ y0 ][ 1 ] = ref_idx[ x0 ][ y0 ][ 1 ] | ae(v) |
| if( inter_bi_idx = = BI_FL1 ) |  |
| mvd = ( 0, 0 ) |  |
| else |  |
| mvd[ x0 ][ y0 ][ 1 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| else if ( cu_mode = = MODE_INTRA ) { |  |
| if( slice_type != I) |  |
| lccm_flag[ x0 ][ y0 ] | ae(v) |
| if( lccm_flag[ x0 ][ y0 ] = = 0) { |  |
| mpm_flag | ae(v) |
| if( mpm_flag ) { |  |
| mpm_idx | ae(v) |
| ipm[ x0 ][ y0 ][ 0 ] = mpm[ mpm_idx ] |  |
| } |  |
| else { |  |
| mpm_ext_flag | ae(v) |
| if( mpm_ext_flag ) { |  |
| mpm_ext _idx | ae(v) |
| ipm[ x0 ][ y0 ][ 0 ] = mpm_ext[ mpm_ext_idx] |  |
| } |  |
| else { |  |
| remain_ipm | ae(v) |
| ipm[ x0 ][ y0 ][ 0 ] = remain_set[ remain_ipm ] |  |
| } |  |
| } |  |
| if( log2width > 4 ¦ ¦ log2height > 4 ) |  |
| mip_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| mip_idx[ x0 ][ y0 ] = 0 |  |
| if ( tree_mode != TREE_L ) { |  |
| ccip_flag | ae(v) |
| if( ccip_flag ) { |  |
| mdlm_flag | ae(v) |
| if( mdlm_flag ) |  |
| mdlm_l_flag | ae(v) |
| } |  |
| else |  |
| ipm[ x0 ][ y0 ][ 1 ] | ae(v) |
| } |  |
| } |  |
| else { |  |
| ipm[ x0 ][ y0 ][ 0 ] = LCCM_LUMA |  |
| mip_idx[ x0 ][ y0 ] = 1 |  |
| ipm[ x0 ][ y0 ][ 1 ] = LCCM_CHROMA |  |
| ccip_flag[ x0 ][ y0 ] = 1 |  |
| } |  |
| } |  |
| if( cu_mode = = SKIP_AFFINE ¦ ¦ cu_mode = = DIR_AFFINE ) { |  |
| affine_merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| else if( cu_mode = = AFFINE ) |  |
| { |  |
| if( inter_pred_dir[ x0 ][ y0 ] = = PRED_L0 ¦ ¦ inter_pred_dir[ x0 ][ y0 ] = = PRED_BI) { |  |
| ref_idx[ x0 ][ y0 ][ 0 ] | ae(v) |
| mvp_idx[ x0 ][ y0 ][ 0 ] | ae(v) |
| affine_mvd_flag | ae(v) |
| for( i = 0; i < num_vertex; i++ ) |  |
| { |  |
| if( affine_mvd_flag ) |  |
| mvd_affine = ( 0, 0 ) |  |
| else |  |
| mvd_affine | ae(v) |
| affine_mv[ x0 ][ y0 ][ I ][ 0 ] = recon_affine_mv(ref_idx, mvp_idx, mvd_affine) |  |
| } |  |
| } |  |
| if( inter_pred_dir[ x0 ][ y0 ] = = PRED_L1 ¦ ¦ inter_pred_dir[ x0 ][ y0 ] = = PRED_BI ) { |  |
| ref_idx[ x0 ][ y0 ][ 1 ] | ae(v) |
| mvp_idx[ x0 ][ y0 ][ 1 ] | ae(v) |
| affine_mvd_flag | ae(v) |
| for( i = 0; i < num_vertex; i++ ) { |  |
| if( affine_mvd_flag ) |  |
| mvd_affine = ( 0, 0 ) |  |
| else |  |
| mvd_affine | ae(v) |
| affine_mv[ x0 ][ y0 ][ I ][ 1 ] = recon_affine_mv( ref_idx, mvp_idx, mvd_affine ) |  |
| } |  |
| } |  |
| } |  |
| if( !cu_skip_flag[ x0 ][ y0 ] ) |  |
| transform_unit( ) |  |
| if( cu_mode = = MODE_INTRA ) { |  |
| if( lccm_flag[ x0 ][y0] ) { |  |
| init_mpm_info( ) |  |
| dwdip_flag[ x0] [ y0 ] | ae(v) |
| } |  |
| else |  |
| dwdip_flag[ x0 ][ y0 ] = 0 |  |
| } |  |
| } |  |

#### Transform unit syntax

| transform_unit( ) { | Descriptor |
| --- | --- |
| if( cu_mode != MODE_INTRA ) { |  |
| if( cu_mode = = DIR ¦ ¦ cu_mode = = DIR_AFFINE ¦ ¦ cu_mode = = DIR_UMVE ) |  |
| root_cbf = 1 |  |
| else |  |
| root_cbf | ae(v) |
| if( root_cbf = = 0 ) |  |
| cbf_y = cbf_u = cbf_v = 0 |  |
| else { |  |
| cbf_u | ae(v) |
| cbf_v | ae(v) |
| if( cbf_u + cbf_v = = 0 ) |  |
| cbf_y = 1 |  |
| else |  |
| cbf_y | ae(v) |
| } |  |
| } |  |
| else { |  |
| cbf_y = cbf_u = cbf_v = 0 |  |
| if( tree_mode = = TREE_L_C ¦ ¦ tree_mode = = TREE_L ) |  |
| cbf_y | ae(v) |
| if( tree_mode = = TREE_L_C ¦ ¦ tree_mode = = TREE_C) { |  |
| cbf_u | ae(v) |
| cbf_v | ae(v) |
| } |  |
| } |  |
| if( cbf_y ) { |  |
| svt_idx | ae(v) |
| log2tuw = svt_idx = = 1 ? log2cuw – 1 : log2cuw |  |
| log2tuh = svt_idx = = 2 ? log2cuh – 1 : log2cuh |  |
| residual coding( ) |  |
| svt_pos | ae(v) |
| } |  |
| if( cbf_u ) { |  |
| residual coding( ) |  |
| } |  |
| if( cbf_v ) { |  |
| residual coding( ) |  |
| } |  |
| } |  |

#### Residual coding syntax

| residual_coding( ) { | Descriptor |
| --- | --- |
| last_sig_coeff_x | ae(v) |
| last_sig_coeff_y | ae(v) |
| lastScanPos = 16 |  |
| lastSubBlock = ( 1 << ( log2width − 2 ) ) * ( 1 << ( log2height − 2 ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = 16 |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = ScanOrder[ log2width − 2 ][log2height-2][ scanIdx ][ lastSubBlock ][ 0 ] |  |
| yS = ScanOrder[ log2width − 2 ][log2height-2][ scanIdx ][ lastSubBlock ][ 1 ] |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| xS = ScanOrder[ log2width − 2 ][log2height-2] [ scanIdx ][ i ][ 0 ] |  |
| yS = ScanOrder[ log2width − 2 ][log2height-2] [ scanIdx ][ i ][ 1 ] |  |
| escapeDataPresent = 0 |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( ( i < lastSubBlock ) && ( i > 0 ) ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| for( n = ( i = = lastSubBlock ) ? lastScanPos − 1 : 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| } |  |
| firstSigScanPos = 16 |  |
| lastSigScanPos = −1 |  |
| numGreater1Flag = 0 |  |
| lastGreater1ScanPos = −1 |  |
| cg_nz[i] = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| if( numGreater1Flag < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numGreater1Flag++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && lastGreater1ScanPos = = −1 ) |  |
| lastGreater1ScanPos = n |  |
| else if( coeff_abs_level_greater1_flag[ n ] ) |  |
| escapeDataPresent = 1 |  |
| } else |  |
| escapeDataPresent = 1 |  |
| if( lastSigScanPos = = −1 ) |  |
| lastSigScanPos = n |  |
| firstSigScanPos = n |  |
| } |  |
| } |  |
| signHidden = lastSigScanPos − firstSigScanPos > 3 |  |
| if( lastGreater1ScanPos != −1 ) { |  |
| coeff_abs_level_greater2_flag[ lastGreater1ScanPos ] | ae(v) |
| if( coeff_abs_level_greater2_flag[ lastGreater1ScanPos ] ) |  |
| escapeDataPresent = 1 |  |
| } |  |
| numSigCoeff = 0 |  |
| sumAbsLevel = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = lastGreater1ScanPos) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| } |  |
| } |  |
| } |  |
| if( svt_idx = = 0 && ( pred_mvr_idx < 1 ¦ ¦ is_intra ) && is_luma ) |  |
| mtr_idx | ae(v) |
| else |  |
| mtr_idx = 0 |  |
| if( cu_mode = = MODE_INTRA && is_luma ) { |  |
| if( mip_idx = = 1 ) |  |
| str_idx = 0 |  |
| else |  |
| str_idx | ae(v) |
| } |  |
| else |  |
| str_idx = 0 |  |
| if( apply_rsp ) |  |
| generate_rsp_pred_signs( ) |  |
| for( i =0; i <= lastSubBlock; i++ ) { |  |
| xS = ScanOrder[ log2width − 2 ][log2height-2] [ scanIdx ][ i ][ 0 ] |  |
| yS = ScanOrder[ log2width − 2 ][log2height-2] [ scanIdx ][ i ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] ) { |  |
| if( rsp_apply ) { |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( blkpos != rsp_pos ) { |  |
| if( blkpos = = hidden_pos ) |  |
| sign[ xC ][ yC ] = hidden_sign |  |
| else |  |
| sign[ xC ][ yC ] | ae(v) |
| } |  |
| else |  |
| sign_rsp[ xC ][ yC ] | ae(v) |
| } |  |
| } |  |
| else { |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] && (!sign_data_hiding_enabled_flag ¦ ¦ !signHidden[i] ¦ ¦ ( n != firstSigScanPos ) ) ) |  |
| sign[ xC ][ yC ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

### Entropy decoding

#### Binary arithmetic coding engine and multi-hypothesis probability update

In this proposal, except initialization and probability update all other part of entropy coding are the same that described in [11]. Initialization and probability update in encoder and decoder coincide. Full description can be found in section 3.1.9.

#### Context initialization

The basic process for context initialization is the same as that in JEM. This proposal uses the initial probability stataes of context models for inter-coded slices by copying states from previously coded slice.

### Conversion process from entropy decoded data to symbols

#### Block partitions

##### Derivation process for split configuration parameter

Split configuration parameters in the sequence header are defined as below.

- log2_cu_11_ratio_max_minus2: the maximum allowed size of 1:1 ratio CU.

- log2_cu_11_ratio_min_minus2: the minimum allowed size of 1:1 ratio CU.

- log2_cu_12_ratio_max_minus2: the maximum allowed size of 1:2 or 2:1 ratio CU (for longer side).

- log2_cu_12_ratio_min_minus2: the minimum allowed size of 1:2 or 2:1 ratio CU (for longer side).

- log2_cu_14_ratio_max_minus2: the maximum allowed size of 1:4 or 4:1 ratio CU (for longer side).

- log2_cu_14_ratio_min_minus2: the minimum allowed size of 1:4 or 4:1 ratio CU (for longer side).

- log2_tri_split_max_ minus2: the maximum allowed size of tri-split (for longer side).

- log2_tri_split_min_ minus2: the minimum allowed size of tri-split (for longer side).

Each parameter is derived as follows.

- Input: a syntax element of a split configuration parameter.

- Output: value of a split configuration parameter.

- Process

  - The value of a split configuration parameter is set to a value of syntax element plus two.

##### Derivation process for split mode

Split modes in this proposal are defined as below.

- NO_SPLIT: Use current CU

- QUAD_SPLIT: Split current CU into four sub-CU as both vertical and horizontal direction in 1:1 ratio

- BI_VER_SPLIT: Split current CU into two sub-CU as vertical direction in 1:1 ratio

- BI_HOR_SPLIT: Split current CU into two sub-CU as horizontal direction in 1:1 ratio

- TRI_VER_SPLIT: Split current CU into three sub-CU as vertical direction in 1:2:1 ratio

- TRI_HOR_SPLIT: Split current CU into three sub-CU as horizontal direction in 1:2:1 ratio

Split mode for the current CU is derived as follows:

- Input: split configuration, split information of neighboring CUs, width and height of CU

- Output: split mode

- Process

  - If both width and height of CU are equal to CTU size

    - split mode is set to QUAD_SPLIT

  - Otherwise, if both width and height of CU are equal to the minimum CU size

    - split mode is set to NO_SPLIT

  - Otherwise, split mode is derived as follows (depending on the number of allowed split mode):

    - Parse one bit if there is at least one allowed split mode; otherwise, the bit is set to be zero

    - If the bit is equal to zero, split mode is set to NO_SPLIT

    - Otherwise, check a number of allowed split modes

      - If the number of allowed split modes is equal to four (Figure 2-1 (a))

        - Parse two bits and select split mode

      - If the number of allowed split modes is equal to three (Figure 2-1 (b))

        - Parse one bit.

        - Check the bit and if parsing one more bit is needed

        - If one more bit is needed

          - Parse one bit

          - Select split mode

        - Otherwise

          - Select split mode

      - If the number of allowed split modes is equal to two (Figure 2-1 (c))

        - Parse one bit

        - Select split mode

      - If the number of allowed split modes is equal to one (Figure 2-1 (d))

        - Select split mode

Allowed split modes for a CU are defined by split configuration as described in section 3.1.2. In other words, bits assignment of each split mode is conducted based on split configuration. All the possible combinations of split modes in this proposal are illustrated in Figure 2-1.

Figure 2-1. Bits assignment for split mode based on split configuration (i.e. allowed split modes)

##### Derivation process for SUCO information

- Input: split mode, parent_suco, su_width, su_height

- Output: suco_info

- Process

  - if a SU satisfies following all conditions, parse suco_info

    - if inter slice or luma tree of intra slice

    - if suco_minsize <= min(su_width, su_height) && max(su_width, su_height)<=suco_maxsize

    - if split_mode is BI_VER_SPLIT or TRI_VER_SPLIT or QUAD_SPLIT

    - if su_width > su_height

  - else suco_info = parent_suco

  - Where parent_suco = 0 if su_width = ctu_width, su_height = ctu_height

##### Spatial varing transform

- Input: cbf_Y, cu_width, cu_height, cu_mode

- Output: SVT mode

- Process

  - if cu_mode is not MODE_INTRA or DIR_UMVE and cbf_Y is 1

    - check whether svt_H and svt_V are allowed as follows

      - if cu_width <= 64 && cu_width >= 8 && cu_height <= 64, svt_H is allowed;

      - if cu_height <= 64 && cu_height >= 8 && cu_width <= 64, svt_V is allowed;

    - if svt_H or svt_V is allowed, parse svt_idx;

    - otherwise, svt_idx = 0

    - if svt_idx > 0, parse svt_pos

  - otherwise, svt_idx = 0

  - set SVT mode based on svt_idx and svt_pos as below.

|  | NO_SVT | SVT_V_0 | SVT_V_1 | SVT_V_2 | SVT_H_0 | SVT_H_1 | SVT_H_2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| svt_idx | 0 | 1 | 1 | 1 | 2 | 2 | 2 |
| svt_pos |  | 0 | 1 | 2 | 0 | 1 | 2 |

SVT modes in this proposal are defined as below

- NO_SVT: tu_height = cu_height; tu_width = cu_width

- SVT_V_n: tu_height = cu_height; tu_width = 1/2 * cu_height; tu_x = cu_x + n/4 * cu_width; tu_y = cu_y; where (cu_x, cu_y) and (tu_x, tu_y) are the coordinate of the top-left corner of CU and the coordinate of the top-left corner of TU, respectively, n= 0, 1, 2.

- SVT_H_n: tu_height = cu_height; tu_width = 1/2 * cu_height; tu_y = cu_y + n/4 * cu_height; tu_x = cu_x

#### Intra prediction parameters

##### Derivation process for intra prediction mode for luma component

- Input: MPM list, MPM_EXT list, LCCM flag

- Output: Intra mode, MIP flag

- Process_ext

  - If LCCM is 1

    - Skip parsing intra mode and MIP flag

    - Return with following setting

      - Luma mode: DC mode

      - MIP flag on

  - If LCCM is 0

    - Parse MPM flag

      - When the MPM flag is on, intra mode is determined with MPM flag and index.

        - MPM list is composed with two intra modes which are exist in left, upper or right CUs.

        - If neighbor CU information is not available, DC or Bilinear mode is used for MPM mode.

    - When the MPM flag is off, parse extended MPM flag.

      - When the MPM_EXT_flag is on, intra mode is determined with extened MPM index.

        - Two MPM and neighbor CU information are considered to compose the extended MPM set.

    - When the MPM_EXT_flag is off, parse remaining intra mode with a truncated binary code.

      - The remaining intra modes apart from the intra modes in the MPM list and extended MPM list are parsed by reading a “truncated binary” code. The truncated binary code can be five bits or six bits long. The parsed truncated binary code can be treated as an index into the “remaining intra modes list”. First nine modes  in the “remaining intra modes list” are filled from a “default mode list” which is ordered based on the “intra mode probability” occurrence.

    - Check the CU size. If CU width or height is bigger than 16, parse MIP flag

    - If MIP flag is 0

      - If DWDIP is not enabled set DWDIP flag (dwdip_flag) to 0.

      - If DWDIP is used by default, set DWDIP flag (dwdip_flag) to 1

      - Otherwise parse DWDIP flag

##### Cross-component intra prediction mode

- Input: LCCM flag

- Output: Chroma Intra mode

- Process

  - If LCCM is 1

    - Chroma mode: CCIP mode

  - If LCCM is 0

    - Parse Chroma mode index

    - Parse ccip_flag

      - If ccip_flag is equal to 1

        - Parse mdlm_flag

          - If mdlm_flag is equal to 1

            - Parse mdlm_l_flag

              - If mdlm_1_flag is equal to 1

                - Chroma mode: CCIP_L mode

              - If mdlm_l_flag is equal to 0

                - Chroma mode:CCIP_A mode

          - If mdlm_flag is equal to 0

            - Chroma mode: CCIP mode

      - If ccip_flag is equal to 0

        - Parse Chroma mode index

#### Motion parameters

##### General process in slice level

ptr, which specifies the picture count of the current picture, is derived as follows:

- Input: dtr (which is the LSB 10 bits of ptr) and dptr (which is (ptr – dtr) >> 10) in slice header

- Output: ptr

- Process

  - ptr = (dptr << 10) + dtr

identical_list0_list1 flag, which indicates whether two reference lists, i.e. list0 and list 1, are identical or not, is derived as follows:

- Input: reference list 0 and list 1

- Output: identical_list0_list1 flag

- Process

  - If all the ptrs of all the reference indices between list 0 and list 1 are the same, identical_list0_list1 is set to 1

  - Otherwise, identical_list0_list1 is set to 0

##### Merge mode

- Input: slice_type, cuw, cuh

- Output: motion information

- Process:

  - If (slice_type = = inter && !(cuw <= MIN_CU_LOG2 && cuh <= MIN_CU_LOG2))

    - parse skip_flag

  - if (skip_flag)

    - create MVP list

    - parse MVP index

    - derive motion information using MVP index from MVP list

  - else

    - parse pred_mode

    - if (pred_mode = = MODE_INTER)

      - parse inter_dir

    - if (inter_dir = = PRED_DIR)

      - create MVP list

      - parse MVP index

      - derive motion information using MVP index from MVP list

##### Parameter derivation process for AMVR

- Input : prediction direction

- Output : motion information

- Process

  - Parse MVR index

  - round_offset = MVR index > 0 ? 1 << (MVR index – 1) : 0

  - if( prediction direction = = bi-prediction )

    - Parse bi_type_idx

    - if( bi_type_idx = = 0 )

      - For reference list 0

        - Parse MVD for list 0

        - Parse reference index for list 0

        - Derive MVP for list 0

        - MVx = (( (MVPx + round_offset) >> MVR index ) << MVR index) + MVDx << MVR index

        - MVy = (( (MVPy + round_offset) >> MVR index ) << MVR index) + MVDy << MVR index

      - For reference list 1

        - Parse MVD for list 1

        - Parse reference index for list 1

        - Derive MVP for list 1

        - MVx = (( (MVPx+ round_offset) >> MVR index ) << MVR index) + MVDx << MVR index

        - MVy = (( (MVPy+ round_offset) >> MVR index ) << MVR index) + MVDy << MVR index

    - else if( bi_type_idx = = 1 )

      - For reference list 0

        - Derive reference index for list 0

        - Derive MVP for list 0

        - MVx = (( MVPx << MVR index ) >> MVR index)

        - MVy = (( MVPy << MVR index ) >> MVR index)

      - For reference list 1

        - Parse MVD for list 1

        - Derive reference index for list 1

        - Derive MVP for list 1

        - MVx = (( MVPx << MVR index ) >> MVR index) + MVDx << MVR index

        - MVy = (( MVPy << MVR index ) >> MVR index) + MVDy << MVR index

    - else if( bi_type_idx = = 2 )

      - For reference list 0

        - Parse MVD for list 0

        - Derive reference index for list 0

        - Derive MVP for list 0

        - MVx = (( MVPx << MVR index ) >> MVR index) + MVDx << MVR index

        - MVy = (( MVPy << MVR index ) >> MVR index) + MVDy << MVR index

      - For reference list 1

        - Derive reference index for list 1

        - Derive MVP for list 1

        - MVx = (( MVPx << MVR index ) >> MVR index)

        - MVy = (( MVPy << MVR index ) >> MVR index)

  - else if( prediction direction = = uni-prediction with list0 )

    - For reference list 0

      - Parse MVD for list 0

      - Parse reference index for list 0

      - Derive MVP for list 0

      - MVx = (( (MVPx+ round_offset) >> MVR index ) << MVR index) + MVDx << MVR index

      - MVy = (( (MVPy+ round_offset) >> MVR index ) << MVR index) + MVDy << MVR index

  - else if( prediction direction = = uni-prediction with list1 )

    - For reference list 1

      - Parse MVD for list 1

      - Parse reference index for list 1

      - Derive MVP for list 1

      - MVx = (( (MVPx+ round_offset) >> MVR index ) << MVR index) + MVDx << MVR index

      - MVy = (( (MVPy+ round_offset) >> MVR index ) << MVR index) + MVDy << MVR index

##### Parameter derivation process for UMVE

- Input : skip flag or direct flag, and identical_list0_list1 flag

- Output : prediction direction, motion vector, and reference index

- Process

  - If( (skip flag = = 1) or (direct flag = = 1) ) Parse UMVE flag

  - If( identical_list0_list1 flag != 1 )

    - Parse Group IDX

  - Else

    - Group IDX = 0

  - Parse Base candidate IDX

  - Parse Distance IDX

  - Parse Direction IDX

  - Base candidate is selected by using Base candidate IDX.

  - Prediction direction is set by using Group IDX.

  - Motion vector is derived by using base candidate, prediction direction, Distance IDX and Direction IDX.

##### Parameter derivation process for AFFINE mode

INTER_AFFINE mode:

- Input : Neighbor’s affine flags, neighbor’s num_vertex, MVR index, width and height of current CU, prediction direction

- Output : affine_flag, num_vertex, affine motion information

- Process :

  - if a CU satisfies following all conditions, parse affine_flag

    - mvr_idx == 0

    - cu_width >=8 && cu_height >= 8

    - Neighbor CU mode (at least one affine coded neighbor)

  - if affine_flag equals to 1, parse affine_mode

  - Once decoded, store the flag to map

  - If affine_flag is larger than 0, for reference list X

    - num_vertex = affine_flag + affine_mode + 1

    - Parse reference index for list X

    - Pare mvp index for list X

    - Derive affine_mvp[X][i] for list X for i in num_vertex

    - Parse affine_mvd_zero_flag for list X

    - For i in num_vertex:

      - If affine_mvd_zero_flag equals to 0, parse affine_mvd[X][i], else affine_mvd[X][i]= (0,0)

      - affine_mv[X][i] = affine_mvd[X][i] + affine_mvp[X][i]

SKIP_AFFINE or DIR_AFFINE mode:

- Input : Neighbor’s affine flags, neighbor’s num_vertex, width and height of current CU

- Output : affine_flag, num_vertex, affine motion information

- Process :

  - if a CU satisfies following all conditions, parse affine_flag

    - (skip_flag == 1 && skip_umve_flag == 0) or (direct_flag == 1 && direct_umve_flag == 0)

    - cu_width >=8 && cu_height >= 8

  - if affine_flag equals to 1:

    - Parse affine_merge_index K

      - Construct affine merge candidate list, AffineMergeCandList, every candidate N includes:

        - num_vertex[N]

        - pred_dir[N]

    - ref_idx[N][0], affine_mv[N][0][i] for i in num_vertex[N]

        - ref_idx[N][1], affine_mv[N][1][i] for i in num_vertex[N]

    - Assigns affine motion information

  - Once decoded, store the flag to map

##### Parameter derivation process for IPR

- Input: Neighbor’s skip and IPR flags, MVR index, width and height of current CU

- Output: IPR flag, refined prediction pixels

- Process:

  - Inter Prediction Refinement flag is decoded for non-merge/skip inter coded CU under certain condition. The condition includes

    - Size restriction (width * height > 32)

    - Neighbor CU mode (at least one IPR coded neighbor or no skip coded CU)

    - MVR index (MVR index = = 0 || MVR index = = 2)

  - Once decoded, store the flag to map.

  - If IPR flag is 1, apply IPR to the pixels obtained after motion compensation.

#### Quantization parameters

- Input: slice QP

- Output: CU QP

- Process:

  - Assign CU QP as slice QP

#### Transform coefficients

- Input: bitstream

- Output: transform coefficient

- Process

  - Parse last position (lastX, lastY)

  - Determine scan type (diagonal/horizontal/vertical) according to intra mode

  - Scan each 4x4 CG from last CG in inverse scan order

    - Parse Sig_CG_flag

    - If Sig_CG_flag is equal to 1, decode following values:

      - sig_flag

        - ctx_idx: derive with number of neighboring sig_flags, posX+posY, color, TU size

      - gt1_flag

        - ctx_idx: derive with number of neighboring gt1_flags, posX+posY, color, TU size

      - gt2_flag

        - ctx_idx: derive with number of neighboring gt2_flags, posX+posY, color, TU size

      - remaining_abs_level(ricePara)

        - ricePara: derive with sum of neighboring abs_levels

  - If TD-RSP is not enabled (seq_rsp_flag = 0), apply_rsp flag is set to false for all the coefficients:

    - If str_idx and svt_idx are 0 and top-left or top-right is available (apply_rsp = 1):

      - Generate truncated list of positions of coefficients (sign_pred_pos_list) which signs should be predicted.

      - Parse signs:

        - If coefficient in sign_pred_pos_list:

          - Decode sign prediction error (sign_rsp) using context.

            - If coefficient magnitude is higher than threshold use ctx_rsp_high

            - Otherwise use ctx_rsp_low

        - Otherwise sequentially decode signs of all coefficients as EP

          - Sign Hide: same with HEVC

    - Otherwise sequentially decode signs of all coefficients as EP

      - Sign Hide: same with HEVC

#### In-loop filter parameters

##### Parameter derivation process for SAO

- Input: reconstructed sample

- Output: SAO mode, SAO type, offsets

- Process

  - Parse SAO slice on flag for Luma

  - Parse SAO slice on flag for Chroma

  - SAO slice on flag for Luma is on

    - Parse merge flag for Luma

    - Merge flag is on

      - Get used SAO flags for Luma from neighboring CTU

    - Merge flag is off

      - Parse SAO mode for Luma

      - Parse offset for Luma

  - SAO slice on flag for Chroma is on

    - Parse merge flag for Chroma

    - Merge flag for Chroma is on

      - Get used SAO flags for Chroma from neighboring CTU

    - Merge flag for Chroma is off

      - Parse SAO mode for Chroma

      - Parse offset for Chroma

##### Noise suppression filter

- Input: bitstream

- Output: is_adaptive flag (1 bit flag) , is_whole_frame_filt flag (1 bit flag), block_size_idx index (2bit index determined block size of filtering application map), appl_map[] array(array of bit flags determined where NS filter should be used)

- Process

  - Parse is_adaptive flag

  - If is_adaptive is equal to 0, parse is_whole_frame_filt flag

  - Otherwise (is_adaptive is equal to 1)

    - Parse block_size_idx

    - Parse app_map array

##### Adaptive loop filter

- Input: reconstructed sample

- Output: ALF mode, ALF coefficients, taps, on/off control

- Process

  - Parse ALF slice on flag for Luma

  - ALF slice on flag for Luma is on

    - Parse ALF coefficients for Luma

    - Parse ALF taps for Luma

    - Parse ALF on/off control flags for Luma

    - Parse ALF slice on flag for Chroma

    - ALF slice on flag for Chroma is on

      - Parse separate ALF on flag

      - Separate ALF on flag in on

        - Parse ALF coefficients for Chroma

        - Parse ALF on/off control flags for Chroma

##### Bilateral filtering

The basic process for BLF is the same as in JEM. There is no BLF related parameters for filtering.

### Decoder-side motion refinement

#### Decoder-side motion refinement

- Input: Reconstructed motion vectors

- Output: Refined motion vectors

- Process:

  - DMVR is applied if the following conditions hold true:

    - If inter_dir is Bi-prediction

    - If forward reference and backward reference locate in each side of current picture

  - (cu_skip_flag = = true OR cu_direct = = true) AND UMVE = = false AND AFFINE = = false AND IPR = = false AND DMVD = = falseThe maximum number of iterations to be applied by DMVR is derived according to CU size as follows:

    - Max_ite = 2 if (cu_width = = 4 OR cu_height = = 4 OR cu_width = = 8 AND cu_height = = 8)

    - Else if (cu_width = = 8 AND cu_height = = 16 | | cu_width = =16 AND cu_height = = 8), Max_ite = 4

    - Max_ite = 8 otherwise.

- DMVR process applied with maximum number of iterations set to Max_ite.

#### Bi-predictive optical flow

- Input: motion compensated predictor

- Output: BIO applied predictor

- Process

  - BIO is applied to luma predictor if following conditions are valid,

    - inter_dir is bi-prediction,

    - Forward reference and backward reference are located on two sides of the current picture

    - AMVR_idx = 0

  - Calculate gradient x, gradient y for forward reference

  - Calculate gradient x, gradient y for backward reference

    - Gradients are calculated with 8-taps filters

  - Apply BIO at cluster level, the claster size is determined as follows,

    - if (cu_weight <= 4 or cu_height <= 4) bio_cluster_size = 2

    - else if (cu_weight <= 16 or cu_height <= 16) bio_cluster_size = 4

    - else bio_cluster_size = 8

  - For each cluster

    - Calculate motion refinement vector

      - Update predictor for each pixel with

Pred = I 0+  I 12 - τ 2vx Ix 0-Ix 1 +  vyIy 0- Iy 12

#### Decoder-side motion vector derivation

- Input : bitstream, SKIP flag, PRED mode, slice type

- Output : DMVD mode

- Process

  - Parse skip flag, and if SKIP flag = 1,

    - parse DMVD flag, If DMVD flag = 1,

      - parse DMVD mode

      - If DMVD flag = 1, parse DMVD mode

      - If DMVD mode = 0, template matching (TM) DMVD method is used.

      - If DMVD mode = 1, bilateral matching (BM) DMVD method is used.

      - If DMVD mode = TM, and slice type = B, parse TMP_DIR.

        - If the flag TMP_DIR is parsed to be ‘1’, bidirectional MCP is performed using the derived MV0 and MV1.

        - If the flag TMP_DIR is parsed to be ‘0’, unidirectional MCP will be performed using the derived MVX (X=0 or 1). If cost0 is less than cost1, X is set to 0, otherwise X is set to 1.

  - Parse PRED mode, if PRED mode = INTER,

    - parse AMVR index, if AMVR index = 0,

      - parse DMVD flag,

      - If DMVD flag = 1, parse DMVD mode

      - If DMVD mode = 0, template matching (TM) DMVD method is used.

      - If DMVD mode = 1, bilateral matching (BM) DMVD method is used.

      - If DMVD mode = TM, and slice type = B, parse TMP_DIR.

        - If the flag TMP_DIR is parsed to be ‘1’, bidirectional MCP is performed using the derived MV0 and MV1.

        - If the flag TMP_DIR is parsed to be ‘0’, unidirectional MCP will be performed using the derived MVX (X=0 or 1). If cost0 is less than cost1, X is set to 0, otherwise X is set to 1.

  - Parse INTER dir,

    - if INTER dir != DIR mode, and INTER dir != DIR UMVE mode, and INTER dir != AFFINE DIR mode,

    - If INTER dir = L0 mode, or INTER dir = L1 mode, or INTER dir = BI mode,

    - Get MVP candidate list for L0 and L1 separately

      - No syntax is transmitted, template matching (TM) DMVD method is used.

#### Motion vector difference sign derivation

- Input: absolute values of MVD, MVSD index

- Output: MVD with signs

- Process:

  - Generate list of MVD candidates based on absolute values of MVD (up to 4 candidates)

  - Calculate cost for each MVD candidate using template matching

  - Perform ranking of candidates by sorting based on calculated cost

  - Taking MVD with an index in the sorted list according to the MVSD index

### Inverse quantization

The basic process for inverse quantization is the same as in HEVC. This proposal uses the same QP range, quantization step, and corresponding scaling process for transformed coefficients.

When Adaptive Quantization step size Scaling is enabled, the quantization step size of a CU is further scaled by a scaling factor which is decided by the contrast and average values of the reconstructed luma sample around the CU.

- Input: QP of the CU, two lines of reconstructed luma samples around the CU, a normalizer from slice header

- Output: quantization step size

- Process:

  - Decide a basic quantization step size based on the QP of the CU

  - Calculate average luminance and contrast of the reconstructed luma samples around the CU

  - Decide a luminance factor and a contrast factor based on the average luminance and contrast

  - Decide a scaling factor based on the product of the luminance factor, the contrast factor and the normalizer

Decide the quantization step size as the product of the basic quantization step size and the scaling factor

### Inverse transforms

#### Multi-core transform

- Input: transform coefficient

- Output: residual samples

- Process:

  - If both width and height of CU are smaller than or equal to 32

    - Parse MTR flag

    - MTR flag is on

      - Parse horizontal mode

      - Parse vertical mode

      - Decide horizontal kernel type from transform set using CU type

      - Decide vertical kernel type from transform set using CU type

    - Apply inverse transform using horizontal kernel type and vertical kernel type

#### Secondary transform

- Input: transform coefficient

- Output: transform coefficient

- Process:

  - Parse STR flag

  - STR flag is on

    - Parse STR mode

    - Get grouped ID from intra prediction mode with CU size

    - Decide angle set from STR mode and grouped ID

    - Apply inverse STR using selected angle set

#### Transform domain residual sign prediction

- Input

  - transform coefficients magnitudes and signs prediction errors

  - sign_pred_pos_list

- Output: transform coefficients

- Process:

  - Calculate reference line from neighbored blocks

    - Transform reference line to transform domain according to MTR type of transform for rows and columns

  - Calculate hypothesis for all possible combinations of signs.

  - Calculate costs for all hypothesis.

  - Sequential loop for all items in sign_pred_pos_list:

    - Predict sign of the current coefficient.

    - If sign prediction error is equal to “1” set sign of coefficient as it was predicted.

      - Otherwise set it as inverted to predicted value.

### Motion compensation

#### Interpolation processfor inter prediction

- Input: motion parameters, reference picture samples

- Output: prediction of a CU

- Process

  - DCT-IF based motion compensation (1/4 pel for luma and 1/8 for chroma) is invoked for the CU by using interpolation filters for luma and chroma components as shown in Table 3-2 and Table 3-3.

#### Affine motion compensation

- Input: affine motion parameters, reference picture samples

- Output: prediction block of a CU

- Process

  - Derive sub-block size of the affine block

  - If both width and height of the sub-block are larger than 4 luma samples,

    - For each sub-block

      - Derive the motion vector for the sub-block.

      - DCT-IF based motion compensation (1/16 pel for luma and 1/32 for chroma) is invoked for the sub-block

  - Otherwise, enhanced bi-linear interpolation filter based motion compensation is invoked for the whole affine block

#### Inter prediction refinement

- Input: IPR flag, prediction pixels

- Output: refined prediction pixels

- Process:

  - A frame level parameter map is stored which describes the linear model relation between the prediction pixels and the corresponding reconstructed pixels.

  - The complete set of IPR model parameters for a CU is given by

    - a[DIR][N_C][TOTAL_SIDES] – scaling parameters

    - b[DIR][N_C][TOTAL_SIDES] – offset parameters

      - Here, DIR is the number of prediction directions i.e. 2, N_C the number of color components i.e. 3 and TOTAL_SIDES is 3.

  - When the prediction pixels and the reconstructed pixels are available for a given CU, compute these model parameters using linear regression and store them to the map.

  - Skip the model parameter computation and store trivial model parameters which correspond to y=x for skip coded CUs and intra CUs.

Figure 2-2 Parameter computation process

  - If the decoded IPR flag for a given CU is 1, parameters are borrowed from the neighboring CU and the linear model is applied to the pixels obtained after motion compensation.

  - Scanning order of left, followed by top neighbor followed by right neighbor is followed for deriving the IPR model parameters from the neighbors. Furthermore, neighbor which uses same reference frame as the current CU is given priority in the derivation process.

#### Overlapped block motion compensation

- Input: motion compensated predictor of a CU

- Output: OBMC applied predictor of a CU

- Process

  - Split the predictor into 4x4 block basis if luma, otherwise to 2x2 block basis

  - For each 4x4 block

    - Compare current MV with above, left, bottom and right neighboring MVs.

    - If different, get MC predictor with neighboring MV, else copy current predictor to corresponding predictor

      - If mv_c != mv_x,  pred(mv_x)=MC(mv_x)

      - Else pred(mv_x) = pred(mv_c)

      - Where x is above, left, bottom or right.

    - Weighted sum of five predictors (current, above, left, bottom and right MVs corresponding predictors) replaces current 4x4 block

Pred' = (pred(mv_c) * wC + pred(mv_a) * wA + pred(mv_l) * wL + pred(mv_b) * wB+pred(mv_r) * wR) >> 4

#### Boundary pixel padding using motion compensation

- Input: motion information of the boundary pixel

- Output: pixels for padding

- Process

  - Check boundary pixel and position of padded pixel

  - If boundary pixel has motion information and position of padded pixel is not top-left, top-right, bottom-left and bottom-right

    - Check neighbor of referenced block.

    - If the neighbor is available, MC is conducted in the area for padding

    - Otherwise, pad a boundary pixel using last pixel in image area

  - Otherwise, pad a boundary pixel using last pixel in image area

Figure 2-3 Boundary pixel padding using motion compensation

### Intra prediction

#### General intra prediction

- Input : intra prediction mode for luma (IPML) and chroma (IPMC)

- Output : prediction sample

- Process

  - Check neighboring CU availability to get reference pixels

    - Check availability of bottom-left, left, top-left, top, top-right, right, bottom-right CUs

    - If a CU is not available, the corresponding reference pixels are padded from nearest adjacent pixel among available CUs

  - Perform intra prediction process for luma

    - If IPML is equal to 0, the CU is predicted by DC value of the reference pixels

    - If IPML is equal to 1, the predicted CU is derived by applying Plane prediction

    - If IPML is equal to 2, the predicted CU is derived by applying Bi-linear prediction

    - If IPML is equal to 15, the predicted CU is derived by applying vertical prediction

    - If IPML is equal to 39, the predicted CU is derived by applying horizontal prediction

    - Otherwise, the predicted CU is derived by applying the following Angular prediction

      - The reference pixels are selectively predicted between left or right CUs according to IPML

        - If IPML is less than 15 and is less than 38, reference pixels of left and bottom-left CU are used. If left CU is not available and right CU is available, then reference pixels of right CU is used

        - Otherwise, reference pixels of right, top right and bottom-right CUs are used. If right CU is not available and left CU is available, then reference pixels of left CU is used

      - Each prediction sample is derived by filtering four reference pixels and the filter coefficients are decided by the distance of between predicted sample position and reference pixel position

  - Perform intra prediction process for chroma

    - If IPMC is equal to 0, the CU is predicted by the same prediction process of luma (DM mode)

    - If IPMC is equal to 1, the predicted CU is derived by applying Bi-linear prediction

    - If IPMC is equal to 2, the CU is predicted by DC value of reference pixels

    - If IPMC is equal to 3, the predicted CU is derived by applying vertical prediction

    - If IPMC is equal to 4, the predicted CU is derived by applying horizontal prediction

#### Multi-combined intra prediction

- Input: Predicted intra CU

- Output: MIP applied prediction CU

- Process

  - Generate filtered reference for MIP

  - If left is available, left and above references are used to MIP process.

  - If right is available, right and above references are used.

  - Both direction are available, all references are used.

#### Distance weighted directional intra prediction

- Input : directional intra prediction mode for luma (IPML) and chroma (IPMC)

- Output : predicted intra block

- Process

  - Check neighboring block availability to get reference pixels

    - Check availability of bottom-left, left, top-left, top, top-right, right, bottom-right blocks

  - Generate secondary reference samples for the sides where neighboring blocks are unavailable

  - Perform intra prediction process for luma by applying the following Angular prediction

      - Primary and secondary prediction samples are derived at the corresponding primary and secondary positions.

      - Resulting prediction sample is obtained by weighting primary and secondary prediction samples with respect to the distance to the reference sample positions.

  - Perform intra prediction process for chroma

    - If IPMC is equal to 0, the block is predicted by the same prediction process of luma (DM mode)

    - If IPMC is equal to 3 or 4, the predicted block is derived by applying a process described above for luma intra prediction modes 15 and 39 respectively.

#### Reference sample sharpening filter

- Input: Sharpening flag, reference samples

- Output: Sharpened reference samples

- Process

  - Calculate edge pattern of reference samples

  - Perform warping of reference samples based on edge pattern.

#### Cross-component intra prediction mode

- Input: intra prediction mode for chroma

- Output: prediction samples

- Process

  - Check the chroma modes

    - If chroma mode is CCIP mode

      - Check availability of left, top, right blocks

    - If chroma mode is CCIP_A mode

      - Check availability of top, top_right blocks

    - If chroma mode is CCIP_L mode

      - Check availability of left, left_bottom blocks

        - If none left block is available

          - Check availability of right, right_bottom blocks

  - Derive the CCIP model coefficients by using the available neighboring chroma pixels and the corresponding downsampled luma pixels.

    - If none block is available, α=0, β=1≪BitDepth-1.

  - Derive the prediction samples of current chroma block using the derived model coefficients and luma sampls in corresponding downsampled luma block.

### In-loop filtering

#### De-blocking filtering

- Input: reconstructed sample

- Output: reconstructed sample

- Process

  - Filter boundarious

    - Process is similar to HEVC. All edges which overlap with an 8x8 grid are filtered

  - Check boundary strength (bs) based on coding mode, number of coefficients, motion vectors in adjacent blocks of block boundary

  - If bs is greater than 0, additional filtering condition is checked for block edges to determine whether filtering process will be performed

  - Determine between normal and strong filter

    - If the strong filter is decided, then the following additional condition is checked to determine filter type of strong filter; If the following condition is true, the longer-tap filter is applied

      - In case of horizontal filtering process:

      - If both block width of adjacent blocks are greater than 8

p7-p0+ q7-q0 < β8

      - In case of vertical filtering process:

      - If both block height of adjacent blocks are greater than 8

p3-p0+ q7-q0 < β8

#### Sample adaptive offset

- Input: reconstructed sample

- Output: reconstructed sample

- Process

  - Check starting position, then set block size for the SAO process

    - If current block is the starting block of the slice, block size is adjusted from (width, height) to (width – 4, height – 4)

    - Else if current block is not a starting block of the slice, starting pixel position of the block is adjusted from (x, y) to (x – 4, y – 4) with same block size.

    - Else if current block is not a start block of the slice and located in last horizontal block in a slice, ending pixel position of the block is adjusted from (x + width, y + height) to (x + width – 4, y + height – 4).

    - Else if current block is not a starting block of the slice and is located in last horizontal block in a slice, ending pixel position of the block is adjusted from (x + width, y + height) to (x + width, y + height- 4).

    - Else if current block is not a starting block of the slice and is located in the last vertical block in a slice, ending pixel position of the block is adjusted from (x + width, y + height) to (x + width – 4, y + height).

  - If SAO is used for current block,

    - Check SAO type between edge offset type and band offset type

    - If edge offset type is used,

      - If sao_type is equal to 0, sample_left is set to (x – 1, y) and sample_right is set to (x + 1, y).

      - Else if sao_type is equal to 1, sample_left is set to (x, y – 1) and sample_right is set to (x, y + 1).

      - Else if sao_type is equal to 2, sample_left is set to (x – 1, y – 1) and sample_right is set to (x + 1, y + 1).

      - Else if sao_type is equal to 3, sample_left is set to (x + 1, y – 1) and sample_right is set to (x – 1, y + 1).

      - Calculate class using sample_right and sample left

      - Add an offset to reconstructed sample by using the offset based on a decided class

    - If band offset type is used,

      - Calculate piexels in 4 bands to be applied

      - Add offsets to reconstructed samples by using offsets based on parsed 4 offsets

#### Noise suppression filtering

- Input: reconstructed frame

- Output: filtered reconstructed frame

- Process

Noise Suppression (NS) filtering is applied for CTU and comprises the following steps:

  - CTU partitioning into a set of square non-overlapped blocks 128x128 size, which are named Super Blocks.

  - Super Block partitioning into a set of square sub-blocks 8x8 size, which are named root sub-blocks

  - Marking root sub-blocks, which should be processing, according filtering application map decompressed from received bits stream

    - for each of the root sub-blocks, which are marked as processed:

      - estimation of luma variance inside sub-block and making decision about further sub-block processing (if luma variance is less threshold then this sub-block is not processed)

      - determining a cluster of pixel-domain patches that are similar to the root sub-block (up to 8 patches) in the cluster;

      - for each pixel from root sub-block

        - scanning collocated pixel from all patches inside the cluster into linear buffer (buffer size is less 8)

        - 1D Hadamard transform (transform size is 8 , 4 or 2)

        - transform domain filtering of Hadamard spectrums using the following formula

fi=fi3fi2+σ2,

wherein fi is i-th transform coefficient in Hadamard spectrum, and σ is filtering parameters which is derived based on QP. Filtering is performed based on LUT reading

        - 1D inverse Hadamard transform of filtered spectrum

        - placing the filtered pixels to corresponding position of filtered cluster of similar patches

      - pixel-domain averaging of pixels with same spatial coordinates in different patches from different clusters

Determining the cluster of pixel-domain patches that are similar to the root sub-block comprises the following steps:

  - For sub-blocks on main diagonal of Super Block search best 8 offsets (in respect to root sub-block) within the search region (limited by current CTU) with minimal distance metrics between root sub-block of the cluster and candidate block

  - Collection best offsets for all diagonals sub-blocks together in one list of best offsets

  - For rest blocks of Super Block search best 8 offsets (in respect to root sub-block) with minimal distance metric between root sub-block and candidate sub-blocks (limited by current CTU) , wherein tested offset candidates are being chosen from list of best offsets.

#### Adaptive loop filtering

- Input: reconstructed sample

- Output: reconstructed sample

- Process

  - If the ALF is used for current picture

    - Derive the luma ALF coefficients, taps in picture level

    - Derive the luma ALF on/off control in block level, the block size is controlled by a quad tree depth parameter in picture level

    - Derive the luma ALF classification in 2x2 luma block level

    - Check ALF mode between unified ALF or separate ALF

    - If the separate ALF is used

      - Derive the chroma ALF coefficients in picture level

      - Derive the chroma ALF on/off control in CTU level

    - Else

      - Convert the luma ALF coefficients to 5x5 taps for chroma use.

      - Set the chroma ALF on/off control and classification the same as the collocated luma block.

    - Invoke ALF filtering process

#### Bilateral filtering

- Input: reconstructed sample

- Output: reconstructed sample

- Process

  - Check length of block and select index of block length

  - Derive weights based on block index and block type

  - Derive values to be applied by calculating weighted average using derived weights

  - Update reconstructed samples based on derived values

### Additional tools

#### Adaptive clipping

Clipping range parameters (clip_min and clip_max) are derived as follows:

- Input: adapt_clip_max_signal_flag, adapt_clip_max_val, adapt_clip_min_signal_flag, adapt_clip_min_val, bit_depth (bit depth of luma component)

- Output: clipping range parameters (clip_min and clip_max)

- Process

  - If adapt_clip_max_signal_flag is equal to 1

    - clip_max = (adapt_clip_max_val << (bit_depth – 6)) + (1 << (bit_depth – 6))

  - Otherwise

    - clip_max = (1 << bit_depth) – 1

  - If adapt_clip_min_signal_flag is equal to 1

    - clip_min = (1 << bit_depth) – (adapt_clip_min_val << (bit_depth – 6)) – (1 << (bit_depth – 6))

  - Otherwise

    - clip_min = 0

For each reconstructed and predicted luma sample, the following clipping process is applied.

s’(i, j) = Clip3(clip_min, clip_max, s(i, j))

where s(i, j) denotes a luma sample with sample position (i, j) and s’(i, j) denotes a clipped luma sample.

### Additional algorithmic description topic(s)

n/a

## 360° Video

In this section, we describe algorithms that were developed while answering CfP for 360° video category. Goal was to keep design as simple as possible, with picture level preprocessing and without coding tools that are specific to 360° video. The response is based on a relatively new projection called Rotated Sphere Projection (RSP). RSP splits and surrounds the sphere using two faces that are cropped from Equirectangular Projection (ERP), in the same way as two flat pieces of rubber are stitched to form a tennis ball. This approach allows RSP to get close to the sphere, achieving more continuity while having a desirable 3:2 aspect ratio similar to Cube Map Projection.

### Projection format (including padding)

RSP was previously proposed at JVET and has been included in 360Lib software as one of the projection formats. It is composed of two faces that are arranged in two rows. The first face can be obtained by directly cropping middle 270°x90° from ERP, as shown by white dotted line in Figure 2-4.

Figure 2-4 Cropping top face of RSP in Balboa sequence

In order to extract the top, bottom and back parts of sphere, ERP is spherically rotated such that pixels at the poles are brought to the equator and pixels corresponding to back side are brought to the front, as shown in Figure 2-5 . The rotated ERP in Figure 2-5 is cut along dotted line to get second face, which represents 90°x270° from top, back and bottom sides. The two faces in Figure 2-4 and Figure 2-5 are combined to obtain final RSP frame, as shown in Figure 2-6.

Figure 2-5 Cropping bottom face of RSP in Balboa sequence

Figure 2-6 Example RSP frame

RSP in Figure 2-6 has almost 6% redundant pixels on corners of each face. When goal is to achieve highest compression efficiency, these pixels can be greyed out or set inactive as shown in Figure 2-7. Since these pixels overlap with pixels from the other face, we do not lose any information during this step.

Figure 2-7 Example RSP frame with inactive pixels

#### Padding

Padding for RSP can be explained with the help of a Chart, which is shown in Figure 2-8. Chart is a computer rendered image with longitudes and latitudes drawn as horizontal and vertical lines as shown. Chart image in RSP format with width W and height H is shown in

Figure 2-9, where the arc splitting active and inactive region is perfectly circular and has no padding, i.e. all pixels from sphere are represented only once in the frame.

Figure 2-8 Chart image in ERP format

W

H

Figure 2-9 Chart image in RSP format with no padding

Padded version of RSP is obtained as follows:

- Move the arc splitting active and inactive area further out by 8 pixels. Furthermore, rather than drawing it as circular, draw on a grid of 8x8 blocks. This allows arc to align with 8x8 PU/TU sizes.

- Increase width of the picture by 16 pixels, by adding 8-pixel padded region on each side. This helps with prediction of active pixels in extreme corners.

Above two operations lead to a Chart RSP image that is shown in Figure 2-10.

H

W+16

Figure 2-10 Chart image in proposed RSP format

### Quantization handling

Quantization parameter is adapted at frame level. CTU level QP adjustment is not done. QP is changed across frames to meet the required bitrate for CfP bitrates for all five bit-streams under consideration.

### 360° video specific decoding tools

There is no 360° specific encoding tools are used for this proposal. Codec used here is generic which supports SDR and 360° with the same set of the tools.

## HDR

### Post-processing

Figure 2-11 HDR Post processing workflow

Three metadata, average/max/min are 12bits/10bits/10bits respectively, therefore totally 32bits are coded in sequence header as HDR metadata used for post-processing.

In Figure 2-11, HDR post processing is to process the decoded signal from decoder and obtained reconstructed 4:2:0 YCbCr signal as the reconstructed HDR output signal. It includes two modules, inverse adaptive quantization and inverse remapping, in post processing. The post-processing is a further post-decoding procedure, since the output reconstructed HDR signal is bit-matched with the HDR signal for pre-processing.

If HDR signal is PQ signal, the luminance decoded signal DY’ from the decoder is forward to inverse adaptive quantization and obtained de-quantized signal Y’ and forward to inverse remapping module to get luminance Y as the luminance component of the output reconstructed 4:2:0 YCbCr HDR signal; If HDR signal is HLG signal, the luminance decoded signal DY’ from the decoder is forward directly to inverse remapping module to get luminance Y as the luminance component of the output reconstructed 4:2:0 YCbCr HDR signal. For both PQ and HLG, the chrominance component of the decoded signal from the decoder is not changed and output as the chrominance component of the reconstructed 4:2:0 YCbCr HDR signal.

The inversed adaptive quantization is as follows,

where T is a threshold and equals to 0.15, w and b are parameters for inverse quantization and w equals to -0.33 and b equals to 0.05 respectively, weight and offset equal to 1/876 and 64 for 10bit HDR signal.

For PQ and HLG signal, the inverse remapping is as derived as follows,

In which, the function of PQ/InversePQ are defined in ST.2084, and the function of HLG/InverseHLG are defined in ARIB STD-B67, and

where, a, b both equal to 1.12762,

m equals to 0.14 and p equals to 1.4 for PQ signal, and m equals to 0.3 and p equals to 2.3 for HLG signal.

### Quantization handling

While coding unit based QP adaptation is not used for the proposed codec, separate QP controls for Cb and Cr components are applied for HDR contents. For Cb and Cr components, QP offsets are set independently. QP offsets are added to luma QP and used for chroma components. For HDR sequences, QP offsets are selected depending on HDR content types which can be specified by representation of color space. For HLG contents, QP offsets are set to 0 for both Cb and Cr, For Market, Hurdles and Starting sequences, which are PQ & BT 709 types, –4 and –9 were set to QP offsets for Cb and Cr respectively. For ShowGirl2 sequences which is PQ and BT2020 type, –2 and –3 were used, –3 were set to QP offsets for both Cb and Cr for Cosmos1 (PQ, BT2100) sequence. Separate QP controls using QP offsets provides similar effects of chroma scaling factors from both HM and JEM.

### HDR specific decoding tools

Motivated by the fact that HDR compression performance is largely dependent on chroma compression performance, SAO is enabled for chroma while SAO is used only for luma component for SDR. SAO parameters are signaled separately for Y, Cb, and Cr, supporting optimized SAO performance for each color component.

# Encoder algorithm description

## General algorithm description

### Rate-distortion optimization

#### Distortion metrics

n/a

#### Perceptual distortion metric

The distortion measurement of the luma component is a perceptually weighted SSD metric, in which the weight is 8×8 block based and reflects the luminance and contrast masking of each 8×8 block in an image. For chroma components, the distortion metric in RD cost calculation is simply SSD.

DR,O=j=0Jwj×SSD(Rj,Oj)

where Rj and Oj denotes a 4×4 sub-block of the reconstructed luma block R and the original luma block O, and wj denotes the weight associated with the sub-block.

The perceptual weight wk for the k-th 8×8 block Bk in a picture is derived by a multi-scale scheme. A picture is partitioned into N scales. For each scale n (n=0, 1,.., N), a contrast masking factor is derived for each (8×2n)×(8×2n) block in the scale. For the largest scale N, a luminance masking factor is derived for each (8×2N)×(8×2N) block in the scale. The multi-scale contrast factors and the single scale luminance factor are combined to derive the weight for each 8×8 block in the picture,

#### Rate-distortion dependency models

n/a

### Partitioning structure selection

#### General partitioning structure

A coding tree unit (CTU) is the basic unit of the proposed coding scheme. A CTU is automatically partitioned by using quad tree structure, thus the maximum coding unit (CU) size is always less than the CTU size. After the quad tree split, the maximum CU can be further partitioned recursively by binary and triple tree (BTT) structure. For instance, if the CTU size is 256x256 then the CTU is split into four 128x128 CUs and each 128x128 CU becomes the maximum CU size and can be split further. By using proposed CU partitioning rule with BTT structure, both square and non-square shapes for the CU are possible, whereas HEVC only allows square CU with quad tree structure. By introducing triple tree partition, CU can be located on the edges of the quad tree partition, not aligning with the boundaries of quad tree whereas CU split from quad tree partitioning is always aligned with its boundaries. With multiple tree structures, an image can be represented in a more flexible way and can hence provide better coding efficiency.

In this proposal, CU shapes are described by the ratio between width and height of a block. For instance, if width and height of a block are the same it can be represented as 1:1 ratio CU or a square CU, and if width is equal to 64 and height is equal to 16 it can be represented as 1:4 ratio CU. CU partitioning is conducted based on the allowed CU shapes and their allowed maximum and minimum sizes.

BTT structure has two CU split modes i.e. the binary split and the triple split mode, and each split mode has two directions, vertical and horizontal. Thus, a CU can be partitioned by four different split modes as defined in previous section: BI_VER_SPLIT, BI_HOR_SPLIT, TRI_VER_SPLIT, and TRI_HOR_SPLIT. These split modes are shown in Figure 3-1.

Figure 3-1 BTT split modes

Depending on the shape of the current CU, allowed split modes can be varied by sequence level signaling. In this proposal, for each shape of the CU the minimum and the maximum sizes are defined in the sequence header as below. (All the sizes are in log2 scale and minus2 implies original size in log2 scale minus two)

- log2_cu_11_ratio_max_minus2: the maximum allowed size of 1:1 ratio CU

- log2_cu_11_ratio_min_minus2: the minimum allowed size of 1:1 ratio CU

- log2_cu_12_ratio_max_minus2: the maximum allowed size of 1:2 or 2:1 ratio CU (for longer side)

- log2_cu_12_ratio_min_minus2: the minimum allowed size of 1:2 or 2:1 ratio CU (for longer side)

- log2_cu_14_ratio_max_minus2: the maximum allowed size of 1:4 or 4:1 ratio CU (for longer side)

- log2_cu_14_ratio_min_minus2: the minimum allowed size of 1:4 or 4:1 ratio CU (for longer side)

In addition to this, triple split modes can be configured by signaling the minimum and the maximum allowed CU size as below.

- log2_tri_split_max_ minus2: the maximum allowed size of tri-split (for longer side)

- log2_tri_split_min_ minus2: the minimum allowed size of tri-split (for longer side)

For example, if CU size is 64x64 and log2_cu_12_ratio_max_minus2 is larger than 4 (real size is 64), bi-vertical and bi-horizontal split modes can be applied. And if log2_tri_split_max_minus2 is larger than 4 (real size is 64) but log2_cu_14_ratio_max_minus2 is smaller than 4 (real size is 64), tri-splits are not allowed. Because tri-sub CU of the CU has shape of 64x16 or 16x64 CU but these shape are not allowed by log2_cu_14_ratio_max_minus2. If CU size is 32x64 and configuration is the same as above, split modes of bi-horizontal and tri-horizontal are allowed. This is shown in Figure 3-2.

Figure 3-2 Example of allowed split modes

A CTU is recursively partitioned by quad tree structure, binary tree structure or triple tree structure. The units in the partitioning process, which are further split into two, three, or four units are named split units (SUs). Usually the coding order of a split unit is from left to right and from above to bottom because of z scanning order of quad tree structure and raster scan of CTUs in a picture. However, normal left to right coding order is more beneficial to left inclined features than right inclined features. This is the case not just in intra prediction. Even in inter prediction, blocks with right inclined features cannot find similar motion information from left and above neighborhood.

Figure 3-3 Allowed coding order in QT / TT / BT split

The SUCO enables more flexible coding order, such as left to right (L2R) and right to left (R2L), to allow intra prediction from right reference pixels and inter prediction with right motion vector predictors. If a SU is partitioned horizontally (horizontal splitting), a flag is signaled to indicate L2R or R2L coding order of partitioned units as shown in Figure 3-3. If a SU is partitioned by quad tree structure, a flag is shared for above two units and bottom two units as shown in Figure 3-3. If no flag is signaled for coding order of an SU, the coding order follows its parent’s SU coding order.

The following parameters are defined for SUCO scheme.

- MaxSUCOSize: the maximum allowed SU size for which SUCO is signaled, is defined as MaxSUCOSize = 1<< (Log2CTUsize – MaxSUCODepthFromTop)

- MinSUCOSize: the minimum allowed SU size for which SUCO is signaled, is defined as MinSUCOSize = 1<< (Log2CTUsize – MinSUCODepthFromTop)

The condition of SUCO signaling is as follows,

- Min(su_width, su_height) >= MinSUCOSize

- Max(su_width, su_height) <= MaxSUCOSize

- split_mode is horizontal split or quardtree split

- su_width > su_height

Figure 3-4. Example if SUCO of a CTU

Figure 3-4 shows example of SUCO in a CTU. Due to more flexible coding order in SU level, the neighboring availability of a leaf CU become more diverse than common left and above neighbors in HEVC. As shown in Figure 3-5, there are four different availability cases if only left and right neighboring blocks are considered, i.e., LR_10, LR_01, LR_11 and LR_00. Above block is always available unless the current CU lies on the top boundary of a slice. Availability of the above left or above right corner block depends on the corresponding left or right neighbor availability.

Figure 3-5. Neighboring availability of a CU

All intra prediction methods are designed to utilize three neighboring reference pixel sets (Figure 3-6 (a)) for its prediction according to their availability. Inter prediction methods which utilize neighboring MVs are also designed to consider three-side neighboring block information (Figure 3-6 (b)).

| (a) | (b) |
| --- | --- |

Figure 3-6 (a) intra reference pixels, (b) Inter MVP position

In this contribution, when the CTU/CU is located on the bottom boundary, horizontal forced BT partition is used without partition flag signaling recursively. The forced horizontal BT partition is terminated when the right bottom sample of the leaf nodes is located within the slice/picture bottom boundary. After forced horizontal BT partition, the forced partitioned CUs from the boundary CTU are possibly further split based on RD-cost at the encoder and the further split flag is signaled from the encoder to the decoder. Similarly, for the CTU/CU located on the right boundary, vertical forced BT partition is used until the leaf nodes are entirely located inside of the boundary and for the right bottom corner boundary located CTU/CU, the forced QT partition is used without signaling until the leaf nodes are entirely located inside of the boundary or the corner located CU becoming bottom/right boundary case.

#### Spatial varing transform

For an inter-predicted CU with cbf_Y equal to 1, a svt_flag is signaled to indicate whether the whole residual block or a subpart of the residual block is decoded. The latter case is called a spatial varying transform (SVT), in which a part of the residual block is coded and the rest of the residual block is forced to be zero.

When SVT is used for a CU, SVT type and SVT position information are further decoded from the bitstream. There are two SVT types and three SVT positions, as indicated in Figure 3-7. For SVT-V (or SVT-H), the TU width (or height) equals to half of the CU width (or height), and three TU positions are equally spaced as (i/4) of the CU width (or height) to the top-left corner of the CU, where i=0, 1 and 2. The three positions are hence denoted as position 0, 1 and 2.

Both TU and CU boundaries are filtered in de-blocking filtering. SVT-V (or SVTH) is allowed when CU width (or height) is in the range of [8, 64]. To ensure the gap between two filtered boundaries are not smaller than 4 pixels, Position 1 of SVT-V (or SVT-H) is not allowed when CU width (or height) equals to 8.

Position-dependent core transform is applied in SVT. The three positions are associated with different core transforms. More specifically, DCT-8 is used for position 0 while DST-7 is used for position 1 and 2.

To harmonize with SUCO, when the right neighbor is available but the left neighbor is not, the reconstructed residual of a SVT TU is horizontally flipped before being added with the prediction block, if the core transform is DST-7.

Figure 3-7 Two types of SVT TU

At encoder side, a non-SVT candidate (i.e., TU=CU) and at most six allowed SVT candidates are competed by RDO cost and the best candidate is selected.

There are three fast algorithms to skip the RDO of the SVT candidates.

- If the SSD of the luma prediction block matches with the SSD of an already tried luma prediction block at the same location in the CTU, which means the residual signal is very likely identical to the previous case, the historical best transform candidates (one or two at most) are tried only.

- If the residual energy is distributed evenly in the CU by comparing the SSD of three SVT positions of SVT-V (or SVT-H), SVT-V (or SVT-H) candidates are skipped.

If the transform coefficients of non-SVT candidate and those of the SVT candidate at position 0 and 2 are all quantized to zero, which means the residual of the CU is very small, all other SVT candidates are skipped.

### Intra prediction mode estimation and type selection

#### General intra prediction

To exploit spatial correlation efficiently based on flexible coding structure, a total of 52 intra prediction modes for luma component and 5 modes for chroma component are applied as specified in Table 3-1. DC, Bi-linear and DM modes are similar to HEVC and just straightforward extended for flexible block size while plane and angular prediction mode are different from HEVC. Compared to HEVC angular prediction, the prediction direction is extended from 33 to 52 for all block sizes. The prediction directions are defined by integer pair information (dx, dy) for angles multiple of 3.75 degree. The (dx, dy) pair represents the direction which each mode uses for context extrapolation as illustrated in Figure 3-8. If we denote the sub-pixel offset of context pixel from the lower bound integer pixel as α, the reference pixel is derived by 4-tap linear prediction of the nearest integer pixels as follows.

refp = 32-α x-1 + 64-α x0+ 32+ α x1+ αx2+ 64≫ 7

Table 3-1 Intra prediction mode for luma and chroma

| Mode index | Luma | Chroma |
| --- | --- | --- |
| 0 | DC | DM |
| 1 | Plane | Bi-linear |
| 2 | Bi-linear | DC |
| 3 | Angular prediction | Horizontal |
| 4 |  | Vertical |
| … |  | N/A |
| 51 |  | N/A |

Figure 3-8 Prediction angle defined by (dx, dy)

Four-tap linear prediction is composed with interpolation filter and smoothing filter. Smoothing filter makes a prediction block natural and transform friendly but removes detail information especially near the reference pixels. Therefore, filter type is alternatingly chosen with the block size but still same filter is used in the same block. Multiple filters which have different degrees of smoothness in the same block can improve both accuracy and naturalness of intra prediction block.

In this proposal, five types of four-tap interpolation and smoothing filters are used. Each filter has different degree of smoothness and the degree is chosen according to the distance from the reference pixels. Four-tap filter is applied as in the following equation and the figure briefly describes this algorithm where px,y represents the predicted pixel, x and y represent the pixel position, fk denotes filter type, and ai represents the reference pixel. When the distance-weighted directional mode is used, the weakest filter is only applied.

px, y  = fk, 0 * a-1 + fk, 1 * a0 +fk, 2 * a1 + fk, 3 * a2 , where fk, i: filter coefficient (0 ≤ k ≤ 5, 0 ≤ i ≤ 3).

Figure 3-9 Multiple four-tap filter description

In the small CU, one filter is adapted according to intra mode. Otherwise, filter is adapted according to absolute distance from the reference pixels. With SUCO, reference is chosen adaptively depend on intra mode and reference availability.

#### Intra mode coding

Intra direction mode is composed with 49 angular modes and 3 non-angular modes (DC, Plane, and Bilinear mode). Currently MPM (most probable mode) is increased with increasing intra mode however this simple extension of MPM dilutes the purpose of MPM. Therefore, this proposal uses two MPM and additional probable intra modes which is called extended MPM. Extended MPM is aimed at reducing mode bits by using additional intra mode set.

Two MPMs are determined with intra mode of left, upper and right blocks. If information doesn’t exist or is not enough, DC or Bilinear mode is used for MPM. If MPM_flag=0, MPM_EXT_flag is checked. If MPM_flag=1, one bit is used for signaling the index of MPM.

Extended MPM is a secondary intra mode set composed with eight intra modes. Extended MPM is selected after MPM is determined. Neighbor block modes and MPM are considered to determine extended MPM set. If candidates are not enough to compose the mode set, statistically selected default candidate list is used. Default candidate is adaptively composed with reference availability and MPM mode. MPM_EXT flag is check when the MPM flag is 0. If MPM_EXT_flag=0, five or six bits are used to code intra mode. If MPM_EXT_flag=1, three bits are used for signaling the index of extended MPM.

The “remaining intra modes” apart from the intra modes in the MPM list and extended MPM list are signaled using a “truncated binary” code. The truncated binary code can be five bits or six bits long. The signaled truncated binary code can be treated as an index into the “remaining intra modes list”. First nine modes in the “remaining intra modes list” are filled from a “default mode list” which is ordered based on the “intra mode probability” occurrence.

Codec is composed with many compression algorithms and each tool has their own flags or indices. The ratio of flag is increased when the performance of codec become efficient. The purpose of LCCM is to improve efficiency of signaling by combining frequently selected tools as a one-bit flag. LCCM combines flag of major tools at a one-bit flag. Combination of tool is chosen by considering correlation of tools and selection ratio. In this proposal, intra mode of luma and chroma component, and MIP are combined, each mode is selected to DC, LM-chroma and MIP on. If LCCM_flag=1, other flag or index of combined intra tool is skipped from signaling. If LCCM_flag=0, every tool is coded.

#### Multi-combined intra prediction

Directional intra prediction often results in non-smooth and unnatural patterns which are not transform-friendly. It is possible to reduce this problem by using a smoothing filter, but conventional smoothing filters tend to also remove meaningful details from the predicted results.

The purpose of MIP is to generate more natural patterns by combining filtered and original reference pixels. Filtered reference is calculated with weighted summation of original reference pixels. Filtering direction is adaptively chosen by intra direction mode. Figure 3-10 describes how to generate the filtered references where a represents original reference and a’ represents filtered reference.

Reference filtering = L→R, U→D: a'j=14*(aj-1+3aj) R→L, D→U: a'j=14*(3aj+aj+1)

Figure 3-10 Reference filtering algorithm

For each prediction unit, MIP processing is applied after the intra predictions. If MIP_index = 0, no MIP processing is applied. If MIP_index = 1, then a 3 point based filter is applied as in the following equation and the figure (Figure 3-11). Cai and Caj represent coefficient of left and upper reference pixel, respectively and Cpij represents coefficient of predicted pixel. In the table, Cai and Caj are counted from down to up row and from right to left column, respectively.

p'ij= CaiCpijCaj * a'ipija'j

Figure 3-11 MIP description and coefficient table

#### Distance-weighted direction intra prediction

Distance-weighted directional intra prediction (DWDIP) is a generalization of bidirectional intra-prediction that uses two opposing reference samples for any direction. Unavailable set of reference samples (secondary reference samples) is generated by taking the available ones (primary reference samples). If DWDIP is enabled, a bi-directional prediction is involved using either:

- two primary reference samples (when dx∙dy>0, and when SUCO enables prediction from both left and right sides)

- primary and secondary reference samples (otherwise)

Process of secondary reference samples generation is performed when dx∙dy≤0. Secondary reference samples are located on those sides of the block to be predicted that do not have available neighboring blocks. Depending on the coding order specified by SUCO, either left, right or both sides could be available for intra prediction. Correspondingly, generation of secondary reference samples is performed either for bottom and right sides, bottom and left sides or just a buttom side of the predicted block. The process of secondary reference samples generation for a square block is shown in Figure 3-12. Values A, B and C in Figure 3-12 are primary reference samples’ values at positions adjacent to top-left and bottom-right corners. These corners aredefined accordingly to the availalble sides and thus depend on the coding order.

Figure 3-12 Process of secondary reference samples generation for a square block

For a non-square block, the value 2N is substituted by W+H, where W is width and H is height of a block to be predicted.

Figure 3-13 illustrates the process of obtaining predicted sample value using distance-weighting.

Figure 3-13 Distance-weighting process

A predicted block generated using DWDIP is usually smoother in such a direction where there is no reliable information on reference samples. The predicted block is adaptable to the difference between the primary and secondary reference samples along a selected direction that is not provided by other directional intra-prediction modes.

An exemplary DWDIP-based predictor is given in comparison with conventional one in Figure 3-14.

Figure 3-14 Exemplary comparison of conventional directional intra prediction and DWDIP

DWDIP-enabled intra prediction mode is selected at the encoder-side in the similar way as conventional intra-prediction mode selection is performed. RD-cost of the selected conventional intra prediction is compared with RD-cost of DWDIP intra prediction. Result of this comparison is a decision whether DWDIP is applied or not.

If DWDIP is applied by default, the candidate list of intra prediction modes for rate-distortion optimization does not include corresponding conventional intra prediction modes.

#### Cross-component intra prediction

To reduce the cross-component redundancy, a cross-component intra prediction (CCIP) mode is used in the proposal, for which the chroma samples are predicted based on the reconstructed luma samples of the same CU by using a linear model as follows:

predCi, j=α·recL'i, j+ β					(1)

where predCi, j represents the predicted chroma samples in a CU and recLi, j represents the downsampled reconstructed luma samples of the same CU. Parameters α and β are derived by minimizing the regression error between the neighbouring reconstructed luma and chroma samples around the current block as follows:

α= N·∑Ln·Cn-∑Ln·∑CnN·∑Ln·Ln-∑Ln·∑Ln				(2)

β= ∑Cn-α·∑LnN						(3)

where Ln represents the down-sampled top and left neighbouring reconstructed luma samples, Cn represents the top and left neighbouring reconstructed chroma samples, and N is equal to the value nearest power of 2 from the sum of width and height of current chroma coding block. For a non-square coding block, another (N-W-H) samples will be sampled from (W+H) samples with equal steps to meet the condition of power of 2. Figure 3-15 shows the location of the left and above causal samples and the sample of the current block involved in the CCIP mode.

This regression error minimization computation is performed as part of the decoding process, not just as an encoder search operation, so no syntax is used to convey the α and β values.

Figure 3-15 Locations of the samples used for the derivation of α and β

There are three CCIP modes totally, besides the above template and left template can be used to calculate the linear model coefficients together, they also can be used alternatively in the other 2 CCIP modes, called CCIP_A, and CCIP_Lmodes.

In CCIP_Amode, only the above template are used to calculate the linear model coefficients. To get more samples, the above template are extended to (W+H).

Figure 3-16 CCIP_A mode

In CCIP_L mode, only left template are used to calculate the linear model coefficients. To get more samples, the left template are extended to (H+W).

Figure 3-17 CCIP_Lmode

Those 3 modes are added as 3 additional chroma intra prediction modes. At the encoder side, 3 more RD cost checks for the chroma components are added for selecting the chroma intra prediction mode.

#### Reference sample sharpening filter

Sharpening filter is selectively applied to reference samples of intra prediction based on flag from bitstream. Sharpening filter can be applied: prediction mode is angular, if block size is block width and heights are less or equal to 32 and when DWDIP is off. The filter has the following structure:

Figure 3-18 Sharpening filter structure

Sharpening of reference samples of intra prediction mode is selected at the encoder-side in the similar way as conventional intra-prediction mode selection is performed. RD-cost of the selected conventional intra prediction is compared with RD-cost of sharpened intra prediction for cases when sharpening is allowed: angular prediction mode, block width and height is less or equal to 32. Result of this comparison is a decision whether sharpening filter is applied or not.

### Motion estimation and motion segment selection

#### General

##### Motion estimation algorithm

The ME algorithm used in the proposal is implemented in a similar manner to HM software. Uni-directional inter prediction uses the MVP as the starting point for the ME search and bi-directional inter prediction uses the MV from the uni-directional prediction search result as the starting MV.

As the first step of the search process, a box search is performed around the starting point with a search range of 5 pixels in each of the four directions. The cost is computed using MVD and SAD for at each position and the best position is computed. This is followed by a diamond search with varying step size. First iteration of diamond search is performed with a step size of 4 and the size is doubled for every further iteration. Three iterations are performed i.e. till the step size is 16. The cost at each position of each iteration is compared to locate the best position. If the best position found happens to be in diamond search and for a step size greater than 5, it implies that the MVD is relatively large. In this case, a raster search is performed around this best position found so far. The raster scan starts with a box search with step size based on the current CU size and the box search is performed. Hereon the step size is progressively reduced until it reaches 1 and the box search is performed at each step and the best position is stored. The MV obtained so far is further refined with one box searches followed by a diamond search performed in the first step.

The MV obtained after the above process is the integer pixel MV and is used as the starting point for sub-pixel MV search. First, a diamond search is performed around this MV to obtain the best half-pixel MV. This half pixel MV is further refined to obtain the final quarter-pixel MV.

When applying AMVR, the search area and the step size of the searches performed is varied adaptively depending on the MVR index.

##### Motion segment selection

Since this proposal does not allow segmentation of CU, each CU always has one motion segment.

##### MC constraint for managing memory bandwidth requirement

To manage the memory bandwidth requirement for motion compensation, when the current CU size is equal to 4x4, or equal to 8x4 or 4x8 coded with bi-prediction mode bi-linear interpolation filter is applied. Otherwise, 8-tap interpolation is used.

##### MC interpolation filter

Motion compensation interpolation uses 2-D separable filter with 6 bits coefficient precision. Filter length is 8 taps for Luma and 4 taps for Chroma. The same Luma filter was adopted in AVS-2. Filters coefficient set was extended in order to support 1/16 MV precision for Luma (1/32 for Chroma). MV precision higher than 1/4 appeared only in Affine mode.

Motion compensation interpolation filters for luma and chroma components are summarized in Table 3-2 and Table 3-3, repectively.

Table 3-2 Luma moction compensation interpolation filters

| Phase | 6-bit filter coefficients | Applicatin |
| --- | --- | --- |
| 1/16 | { 0, 1, –3, 63, 4, –2, 1, 0 }, | Only Affine |
| 1/8 | { –1, 2, –5, 62, 8, –3, 1, 0 }, | Only Affine |
| 3/16 | { –1, 3, –8, 60, 13, –4, 1, 0 }, | Only Affine |
| 1/4 | { –1, 3, –9, 57, 19, –7, 3, –1 }, |  |
| 5/16 | { –1, 4, –11, 52, 26, –8, 3, –1 }, | Only Affine |
| 3/8 | { –1, 3, –9, 47, 31, –10, 4, –1 }, | Only Affine |
| 7/16 | { –1, 4, –11, 45, 34, –10, 4, –1 }, | Only Affine |
| 1/2 | { –1, 4, –10, 39, 39, –10, 4, –1 }, |  |

Table 3-3 Chroma moction compensation interpolation filters

| Phase | 6-bit filter coefficients | Phase | 6-bit filter coefficients |
| --- | --- | --- | --- |
| 1/32 | { –3, 68, –1, 0 }, | 9/32 | { –5, 52, 20, –3 }, |
| 1/16 | { –3, 66, 2, –1 }, | 5/16 | { –5, 50, 23, –4 }, |
| 3/32 | { –4, 64, 5, –1 }, | 11/32 | { –5, 48, 25, –4 }, |
| 1/8 | { –4, 62, 7, –1 }, | 3/8 | { –5, 46, 28, –5 }, |
| 5/32 | { –4, 60, 10, –2 }, | 13/32 | { –5, 44, 30, –5 }, |
| 3/16 | { –4, 58, 12, –2 }, | 7/16 | { –6, 42, 33, –5 }, |
| 7/32 | { –4, 56, 15, –3 }, | 15/32 | { –6, 40, 36, –6 }, |
| 1/4 | { –5, 54, 18, –3 }, | 1/2 | { –6, 38, 38, –6 }, |

In order to ensure memory bandwith the worst case does not exceed HEVC for small blocks bi-linear interpolation I used.

#### Merge mode

A merge mode is specified whereby the motion parameters for the current CU are obtained from neighbouring CUs, including spatial and temporal candidates. The alternative to merge mode is the explicit transmission of motion parameters, where motion vector, corresponding reference picture index for each reference picture list and reference picture list usage are signaled explicitly per each CU. The merge mode in this contribution is almost same to the merge mode in HEVC beside some minor changes.

1) Derivation of candidates for merge mode

When a CU is predicted using merge mode, an index is used to retrieve the motion information in the merge candidates list. The construction of this list can be summarized according to following sequence of steps:

- Step 1: Initial candidates derivation

  - Step 1.1: Spatial candidates derivation

  - Step 1.2: Redundancy check for spatial candidates

  - Step 1.3: Temporal candidates derivation

- Step 2: Additional candidates insertion

  - Step 2.1: Creation of bi-predictive candidates

  - Step 2.2: Creation of refined candidates

  - Step 2.3: Insertion of zero motion candidates

These steps are also schematically depicted in Figure 3-19. For spatial merge candidate derivation, a maximum of four are selected among candidates that are located in five different positions. For temporal merge candidate derivation, a maximum of two merge candidates are selected. Since constant number of candidates for each CU is assumed at decoder, additional candidates are generated when the number of candidates does not reach to maximum number of merge candidate (5 is set under macro xxx ). Since the number of candidates is constant, index of best merge candidate is encoded using truncated unary binarization (TUB).

In the following, the operations associated with the aforementioned steps are detailed.

Figure 3-19 Derivation process for merge candidates list construction

- Spatial candidates derivation

In the derivation of spatial merge candidates, a maximum of four merge candidates are selected among candidates located in the positions depicted in Figure 3-20. The merge candidates located positions are related to the neighbouring CU availability, and the order of derivation is listed for each condition.

  - Left and right neighbouring CUs are not available

B->E->A->D->C

  - Right neighbouring CUs are not available

F->B_>E(D/H)->A->I

If E is not available or intra coded, 1) and width of current CU is larger or equal to height, D is selected; 2_ and width of current CU is smaller than height, H is selected.

  - Left neighbouring CUs are not available

G->D-A(B/J)->E->K

If A is not available or intra coded, 1) and width of current CU is larger or equal to height, B is selected; 2_ and width of current CU is smaller than height, J is selected.

  - Others

F->G->B->E->A

Figure 3-20 Positions of spatial merge candidates

After the first candidate is added, the addition of the remaining candidates is subject to a redundancy check which ensures that candidates with same motion information are excluded from the list.

- Temporal candidates derivation

In this step, two candidates are added to the list if the total selected candidate number does not exceed the max merge candidate number. In the derivation of temporal merge candidates, scaled motion vector is derived based on co-located CU belong to the co-located picture which is the first reference picture in reference list 1. The scaled motion vector for temporal merge candidate is obtained as illustrated by the dotted line in Figure 3-21 , which is scaled from the motion vector of the co-located CU using the POC distances, tb and td, where tb is defined to be the POC difference between the reference picture of the current picture and the current picture, and tb is defined to be the POC difference between the reference picture of the co-located picture and the co-located picture. The reference picture index of temporal merge candidate is set equal to zero. For a B-slice, the mv of co-located CU reference list 0 is scaled for current CU reference list 0 and reference list 1.

Figure 3-21 Illustration of motion vetor scaling for temporal merge candidate

In the co-located CU (Y) belonging to the reference frame, the position for temporal candidate are C0 and C1, as depicted in Figure 3-22 , and the order of temporal candidate derivation is C1 and C0.

Figure 3-22 Candidate positions for temporal merge candidate, C0 and C1

- Additional candidates insertion

Besides spatio-temporal merge candidates, there are three additional types of merge candidates: combined bi-predictive merge candidate, refined merge candidate and zero merge candidate, as done in HEVC.

  - Combined bi-predictive merge candidate

Generated by utilizing spatio-temporal merge candidates. Combined bi-predictive merge candidate is used for B-Slice only. The combined bi-predictive candidates are generated by combining the first reference picture list motion parameters of an initial candidate with the second reference picture list motion parameters of another. If these two tuples provide different motion hypotheses, they will form a new bi-predictive candidate.

  - Refined merge candidate

Candidates in the list are adjusted according to the refine pattern in Figure 3-22to generate new candidates. For example, the mv in reference list 0 of first candidate in the list is (mvx0, mvy0), which can be refined to (mvx0+1, mvy0), similarly, mv in reference list 1 can be refined to the opposite direction, (mvx1-1, mvy1), the refined mv tuples are combined as a new candidate.

  - Zero merge candidate

Zero motion candidates are inserted to fill the remaining entries in the merge candidates list and therefore hit the MaxNumMergeCand capacity. These candidates have zero spatial displacement and a reference picture index which starts from zero. Finally, no redundancy check is performed on these candidates.

#### Adaptive motion vector resolution

In HEVC, a motion vector (MVx, MVy) is represented with a 1/4–pel precision. Instead of directly coding MV, the difference between MV and MV predictor, which is derived from neighboring MVs, is coded to save bits for MV representation. Since MV of the current block and neighboring MVs are all in 1/4–pel precision, the difference (MVDx, MVDy) is in 1/4–pel precision too. The MVDx and MVDy is then independently encoded.

This proposal however supports multiple motion vector resolutions. Supported motion vector resolutions in proposed method range from 1/4–pel to 4–pel (1/4–pel, 1/2–pel, 1–pel, 2–pel and 4–pel). Information about the motion vector resolution is signaled at the CU level when MVD information is signaled.

Depending on the resolution of CU, both motion vector (MV) and motion vector predictor (MVP) of the CU are adjusted. If applied motion vector resolution is denoted as R, MV (MVx, MVy) and MVP (MVPx, MVPy) are represented as follows:

(MVx, MVy) = (Round(MVx / (R * 4)) * (R * 4), Round(MVy / (R * 4)) * (R * 4))

(MVPx, MVPy) = (Round(MVPx / (R * 4)) * (R * 4), Round(MVPy / (R * 4)) * (R * 4))

Since the motion vector predictor and MV are both adjusted by an adaptive resolution, MVD (MVDx, MVDy) is also aligned to the resolution and, is signaled according to the resolution as follows:

(MVDx, MVDy) = ((MVx – MVPx) / (R * 4), (MVy – MVPy) / R * 4))

In HEVC, AMVP mode uses MVP index for selecting the motion vector predictor among those in the MVP candidate list. In this proposal, motion vector resolution index (MVR index) indicates MVP index as well as the motion vector resolution. As result, the proposed method has no MVP index signaling. Table 3-4 shows what each value of MVR index represents.

Table 3-4 MVR Index

| MVR Index | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| Resolution (R) in pel | 1/4 | 1/2 | 1 | 2 | 4 |
| Nth MVP | 1st MVP | 2nd MVP | 3rd MVP | 4th MVP | 5th MVP |

In case of bi-prediction, AMVR has 3 modes for every resolution. AMVR Bi-Index indicates whether MVDx, MVDy of each reference list (list 0 or list 1) are signaled or not. AMVR Bi-Index in this proposal is defined as Table 3-5.

Table 3-5 AMVR Bi-Index

| AMVR Bi-Index | (MVDx, MVDy) of list 0 | (MVDx, MVDy) of list 1 |
| --- | --- | --- |
| 0 | Signaled | Signaled |
| 1 | Not signaled | Signaled |
| 2 | Signaled | Not signaled |

Figure 3-23 AMVR inter prediction

#### Ultimate motion vector expression

In inter prediction, one of the neighbor motion information is directly used to represent MV of current CU in skip mode and direct mode, while additional motion information is explicitly signaled with residual information in other modes. This motion information includes prediction direction, reference index and corresponding MVs.

Ultimate motion vector expression (UMVE) mode can be either skip or direct modes, which use proposed motion vector expression method with neighboring motion information. As the skip and the merge modes in HEVC, UMVE also makes a candidate list from neighboring motion information. Among those candidates in the list, a MV candidate is selected, and is further expanded by new motion vector expression method.

UMVE provides a new motion vector expression with simplified signaling. The expression method includes starting point, motion magnitude, and motion direction.

Figure 3-24 UMVE Search Process

Figure 3-25 UMVE Search Point

Base candidate index defines the starting point. Base candidate index indicates the best candidate among candidates in the list as follows.

Table 3-6. Base candidate IDX

| Base candidate IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Nth MVP | 1st MVP | 2nd MVP | 3rd MVP | 4th MVP |

Distance index is motion magnitude information. Distance index indicates the pre-defined distance from the starting point information. Pre-defined distance is as follows:

Table 3-7 Distance IDX

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel |

Direction index represents the direction of the MVD relative to the starting point. The direction index can represent of the four directions as shown below.

Table 3-8 Direction IDX

| Direction IDX | 00 | 01 | 10 | 11 |
| --- | --- | --- | --- | --- |
| x-axis | + | – | N/A | N/A |
| y-axis | N/A | N/A | + | – |

#### Affine motion prediction

There are three affine motion modes: AF_4_INTER mode, AF_6_INTER and AF_MERGE mode.

When merge/skip flag is true, and both width and height for the CU are larger than or equal to 8, an affine flag in CU level is signalled in the bitstream to indicate whether affine merge mode is used. And when the CU is coded as AF_MERGE, the merge candidate index with maximum value 5 is signalled for specifying which motion information candidate in the affine merge candidate list is used for the CU.

The affine merge candidate list is constructed as following steps:

- Insert model based affine candidates

Figure 3-26: Candidates for AF_MERGE

Model based affine candidate means that the candidate is derived from the valid neighbor reconstructed block coded with affine mode. As shown in Figure 3-26, the scan order for the candidate block is from left, above, above right, left bottom to above left.

If the neighbour left bottom block A is coded in 6-paramter affine mode, the motion vectors v4, v5, and v6 of the top left corner, above right corner and left bottom corner of the CU which contains the block A are obtained. And the motion vectors v0, v1, and v2 of the top left corner on the current CU is calculated according to v4, v5, and v6 by 6-parameter affine model.

If the neighbour left bottom block A is coded in 4-paramter affine mode, the motion vectors v4 and v5 of the top left corner and above right corner of the CU which contains the block A are obtained. And the motion vectors v0 and v1 of the top left corner on the current CU is calculated according to v4 and v5 by 4-parameter affine model.

- Insert control point based affine candidates

Control points based candidate means the candidate is constructed by combining the neighbor motion information of each control point.

The motion information for the control points is derived firstly from the specified spatial neighbors and temporal neighbor shown in Figure 3-27. CPk (k=1, 2, 3, 4) represents the k-th control point. A, B, C, D, E, F and G are spatial positions for predicting CPk (k=1, 2, 3); H is temporal position for predicting CP4.

The coordinates of CP1, CP2, CP3 and CP4 is (0, 0), (W, 0), (H, 0) and (W, H), respectively, where W and H are the width and height of current block.

The motion information of each control point is obtained according to the following priority order:

- For CP1, the checking priority is ABC, A is used if it is available. Otherwise, if B is available, B is used. If both A and B are unavailable, C is used. If all the three candidates are unavailable, the motion information of CP1 cannot be obtained.

- For CP2, the checking priority is ED;

- For CP3, the checking priority is GF;

- For CP4, H is used.

Secondly, the combinations of controls points are used to construct the motion model.

Motion vectors of two control points are needed to compute the transform parameters in 4-parameter affine model. The two control points can be selected from one of the following six combinations ({CP1, CP4}, {CP2, CP3}, {CP1, CP2}, {CP2, CP4}, {CP1, CP3}, {CP3, CP4}). For example, use the CP1 and CP2 control points to construct 4-parameter affine motion model, denoted as Affine (CP1, CP2).

Motion vectors of three control points are needed to compute the transform parameters in 6-parameter affine model. The three control points can be selected from one of the following four combinations ({CP1, CP2, CP4}, {CP1, CP2, CP3}, {CP2, CP3, CP4}, {CP1, CP3, CP4}). For example, use CP1, CP2 and CP3 control points to construct 6-parameter affine motion model, denoted as Affine (CP1, CP2, CP3).

Figure 3-27 Candidate positions for control point based merge candidate

When merge/skip flag is false, and both width and height for the CU are larger than or equal to 8, an affine flag in CU level is signalled in the bitstream to indicate whether affine inter mode is used. And when the CU is coded as affine inter mode, a model flag is signalled for specifying whether 4-parameter or 6-parameter affine model is used for this CU. If the model flag is true, AF_6_INTER mode (6-parameter affine model) is applied and 3 MVDs will be parsed; otherwise, AF_6_INTER mode (4-parameter affine model) is applied and 2 MVDs will be parsed.

In AF_4_INTER mode, similar to affine merge mode, affine motion vector pairs extrapolated from neighbour blocks coded with affine mode are constructed and insert into candidate list firstly.

After that, if the size of the candidate list is smaller than 4, candidates with motion vector pair v0,v1|v0={vA,vB,vc},v1={vD, vE} is constructed using the neighbour blocks. As shown in Figure 3-28, v0 is selected from the motion vectors of the block A, B or C. The motion vector from the neighbour block is scaled according to the reference list and the relationship among the POC of the reference for the neighbour block, the POC of the reference for the current CU and the POC of the current CU. And the approach to select v1 from the neighbour block D and E is similar. When the candidate list is larger than 4, the candidates are firstly sorted according to the consistency of the neighbouring motion vectors (similarity of the two motion vectors in a pair candidate) and only the first four candidates are kept.

If the number of candidate list is smaller than 4, the list is padded by the motion vector pair composed by duplicating each of the AMVP candidates.

In AF_6_INTER mode, similar to affine merge mode, affine motion vector triples extrapolated from neighbour blocks coded with affine mode are constructed and insert into candidate list firstly.

After that, if the size of the candidate list is smaller than 4, candidates with motion vector triples v0,v1,v2  |v0={vA,vB,vc},v1={vD, vE},v2={vG, vH} is constructed using the neighbour blocks. As shown in Figure 3-28, v0 is selected from the motion vectors of the block A, B or C. The motion vector from the neighbour block is scaled according to the reference list and the relationship among the POC of the reference for the neighbour block, the POC of the reference for the current CU and the POC of the current CU. And the approach to select v1 from the neighbour block D and E, and select v2 from F and G is similar. When the candidate list is larger than 4, the candidates are firstly sorted according to the consistency of the neighbouring motion vectors (similarity of the two motion vectors in a triple candidate) and only the first four candidates are kept.

If the number of candidate list is smaller than 4, the list is padded by the motion vector triple composed by duplicating each of the AMVP candidates.

Figure 3-28: MVP for AF_4_INTER and AF_6_INTER

After the CPMV of the current CU are derived, according to the number of affine parameters, the MVF of the current CU is generated according to Equation 4 for 4-parameter affine model, and according to Equation 5 for 6-parameters affine model.

vx=v1x-v0xWx-v1y-v0yWy+v0xvy=v1y-v0yWx+v1x-v0xWy+v0y						(4)

vx=v1x-v0xWx+v2x-v0xHy+v0xvy=v1y-v0yWx+v2y-v0yHy+v0y						(5)

The sub-block size M×N is derived as in Equation 6, where MvPre is the motion vector fraction accuracy (1/16 in JEM).

M=clip3(4,w,w×MvPremaxabsv1x-v0x,absv1y-v0y)N=clip3(4,h,h×MvPremaxabsv2x-v0x,absv2y-v0y)	            (6)

After derived by Equation 6, M and N should be adjusted downward if necessary to make it a divisor of w and h, respectively. If M or N is smaller than 8, WIF is applied; otherwise, sub-block based affine motion compensation is applied.

Figure 3-29: Affine MVF per sub-block

To derive motion vector of each M×N sub-block, the motion vector of the center sample of each sub-block, as shown in Figure 3-29, is calculated according to Equation 4 or Equation 5, and rounded to 1/16 fraction accuracy. Then the SHVC upsampling interpolation filters are applied to generate the prediction of each sub-block with derived motion vector.

SHVC upsampling interpolation filters, which have same filter length and normalization factor as HEVC motion compensation interpolation filters, are used as motion compensation interpolation filters for the additional fractional pel positions. The chroma component motion vector accuracy is 1/32 sample in the JEM, the additional interpolation filters of 1/32 pel fractional positions are derived by using the average of the filters of the two neighbouring 1/16 pel fractional positions.

AF_MERGE mode is selected at the encoder-side in the similar way as conventional merge mode selection is performed. The candidate list is constructed firstly, and minimum RD-cost inside the candidates is selected to compare with RD-cost of other inter modes. Result of this comparison is a decision whether AF_MERGE is applied or not.

For AF_4_INTER mode, a RD cost check is used to determine which motion vector pair candidate is selected as the control point motion vector prediction (CPMVP) of the current CU. After the CPMVP of the current affine CU is determined, affine motion estimation is applied and the control point motion vector (CPMV) is found. Then the difference of the CPMV and the CPMVP is decided.

In encoder side, AF_6_INTER mode will only be verified when AF_MERGE or AF_4_INTER mode is selected as the best mode in the previous mode selected stage.

#### Enhanced bi-linear interpolation filter

Enhanced bi-linear Interpolation Filter (EIF) is used for non-translational (affine) motion compensation. In EIF bilinear interpolation coupled with 3-tap high-pass filter ({-1,10, -1} with normalization factor 1/8) is used for inter prediction of both Luma and Chroma component. The bilinear interpolation is applied to interpolate all fractional offsets on pixel-level. High-pass filter applied after bilinear interpolation. The coefficients of the filters are fixed, the same filter is used for all predicted samples involved in non-translational (affine) motion compensation model excluding cases when both width and height of the sub-block unit are larger than 4 luma samples.

A new four-tap inter interpolation filters are used for inter prediction filtering of Chroma component with translational motion. The filter coefficients are derived based on bilinear transform coupled with a high-pass filter.

#### Inter prediction refinement

The idea behind Inter Prediction Refinement (IPR) is to compensate for the illumination differences between the reference frame and the current frame. IPR is applied on the prediction pixels obtained by motion compensation. The tool can be described in two parts

- Computing and storing the parameters:

At the end of processing of each CU, linear models (Ax + B = y) are computed by comparing the prediction pixels (x) and the reconstructed pixels (y). The models are computed using linear regression. Three sets of models are computed, one each for the left edge of the CU, the bottom edge of the CU and the right edge of the CU. Furthermore, each of these sets contains models for the three color component and two directions (if available). These parameters (sets of A and B) are then stored in a map for later access.

Figure 3-30 Parameter computation process

- Tool application

In case of a merge/skip block, the IPR flag for the current CU is derived from the corresponding neighbor. In case of other inter coded CU’s certain conditions are defined under which IPR is applicable. These conditions depend on the size of the current CU and the coding mode used by neighbors. A flag is signaled to indicate whether IPR is to be applied to a given inter coded CU or not. If the tool is to be applied, the motion compensation pixels (x) are refined using the application of the linear model (Ax + B) to obtain the new set of prediction pixels. The model used is derived from the current CU’s neighbor. Since multiple neighbors may be available, pruning is performed in order to consider only inter neighbors and give higher priority to neighbor which use same reference frames for prediction as the current CU.

Figure 3-31 Model parameter derivation

Additionally, when performing RDO check for determining whether IPR will be used or not, mean-removed SAD (Sum of Absolute Difference) is used instead of the regular SAD for the ME (Motion Estimation) process.

#### Overlapped block motion compensation

Overlapped block motion compensation (OBMC) in H.263 is directly extended to this proposal on 4x4 block basis. OBMC is always applied to motion compensated predictors regardless of CU size. After motion compensation, each 4x4 block in a CU further achieves four corresponding predictors with four neighboring MVs (Figure 3-32), which are denoted as pred(mvA), pred(mvL), pred(mvB), pred(mvR) respectively. Including predictor of current 4x4 block pred(mvC), OBMC applied predictor is achieved by weighted sum of five predictors as given by the  following equation:

Where, wC, wA, wL, wB, wR are weighting values for five predictors as given in Table 3-9. OBMC is applied to the corresponding chroma predictor on 2x2 block basis. To avoid duplicate motion compensation, predictor of current sub-block is used instead if neighboring MV is same as the current MV.

Figure 3-32 Illustration of 4x4 block OBMC for a CU

Table 3-9 Weighting values for 4x4 block OBMC

#### Usage of decoder-side motion refinement

##### Decoder-side motion vector refinement

In bi-prediction operation, for the prediction of one block region, two prediction blocks, formed using a motion vector (MV) of list0 and a MV of list1, respectively, are combined to form a single prediction signal. In the decoder-side motion vector refinement (DMVR) method, the two motion vectors of the bi-prediction are further refined by a bilateral template matching process. The bilateral template matching applied in the decoder to perform a distortion-based search between a bilateral template and the reconstruction samples in the reference pictures in order to obtain a refined MV without transmission of additional motion information.

In DMVR, a bilateral template is generated as the weighted combination (i.e. average) of the two prediction blocks, from the initial MV0 of list0 and MV1 of list1, respectively, as shown in Figure 3-33. The template matching operation consists of calculating cost measures between the generated template and the sample region (around the initial prediction block) in the reference picture. For each of the two reference pictures, the MV that yields the minimum template cost is considered as the updated MV of that list to replace the original one.

The two new MVs, i.e., MV0′ and MV1′ as shown in Figure 3-33, are used for generating new bi-prediction results. A sum of absolute differences (SAD) is used as the cost measure. Optionally, multi-iteration for the new MVs derivation is available to get the final bi-prediction results.

Figure 3-33: DMVR based on bilateral template matching

The DMVR method follows the block merging operation and modifies input motion vectors in order to improve the accuracy of the initial motion compensated prediction. The process is invoked if the following conditions are true:

- Current block is bi-predicted,

- One reference picture proceeds and other reference picture precedes the current picture in display order,

- Prediction mode is skip mode or direct mode,

- UMVE, AFFINE, DMVD, INTER_PREDICTION_REFINEMENT are signaled to be off.

The DMVR is composed of 2 stages, integer pel and half pel motion vector refinement. The flowchart provides a high level overview of the operation:

Figure 3-34  Flowchart of DMVR process

Interpolation filtering:

During the MV search process the motion compensated predictions are obtained using the bilinear interpolation filtering. After the DMVR process finishes, the final predictions are obtained using regular interpolation filters.

Maximum number of iterations:

Integer pel refinement stage is repeated multiple times depending on the width and the height of the current CU. The maximum number of allowed iterations (Max_ite) is determined as follows:

- If width or height of the current CU is smaller than 8, Max_ite is set to 2.

- If width and height of the current CU are both equal to 8, Max_ite is set to 2.

- If width is equal to 8 and height is equal to 16, or vice versa, Max_ite is set to 4.

- If none of above is true, Max_ite is set to 8.

Please note that one integer pel iteration consists a search operation either in L0 or in L1.

Half pel refinement stage is composed of exactly 2 iterations, one MV refinement search on list 0 reference picture and one search on list 1 reference picture. The half pel iteration stage is applied only if maximum number of iterations is not reached.

Construction of motion vector refinement search space:

Integer search space is composed of 6 search points, 1 center point and 5 surrounding points, in total. In the first step 5 points (CENTER, UP, LEFT, DOWN and RIGHT) are searched. According to the matching cost obtained for the 4 surrounding points, one of the 4 corner points (TOP-LEFT, TOP-RIGHT, BOTTOM-LEFT and BOTTOM-RIGHT) is included in the search space. Gradient of the cost is used in the selection process.

Half pel search space is composed of 5 points, namely CENTER, UP, LEFT, DOWN and RIGHT.

The predictions corresponding to the candidate search points are generated using bilinear interpolation.

Construction of template and matching function:

Before beginning of each L0 search iteration, the refined motion vectors of the previous iteration are used to obtain the motion compensated predictions P1[CENTER] and P2[CENTER]. The template is constructed as T = P1[CENTER] + P2[CENTER]. Division, rounding or clipping are not applied, P1 and P2 are generated using bilinear interpolation.

The normalized SAD is used as the cost function, the matching cost is given by SAD(T – mean(T), 2 * P[x] – 2 * mean(P[x])).

Early iteration termination:

The integer pel search iterations are terminated before the maximum number of iterations are reached if any one of the motion vectors is not changed after one search operation. Half pel search stage does not apply early termination.

##### Bi-directional optical flow

This method was initially proposed in [12]. It can be classified as decoder motion derivation method and can be applied only for Bi prediction. In the construction of prediction block, instead of conventional half-sum more precise combination of two references is used. Note that overhead is not needed for this method as the encoder calculates motion vector for each pixel and this procedure is repeated on the decoder side. Model of optical is supposed to be valid along motion trajectory

∂I  ∂t+ vx∂I  ∂x + vy∂I  ∂y= 0

This is a very reasonable assumption when illumination does not change from frame to frame. In addition we shall notice, that BIO is very effective in case of camera motion and zoom.

Main formula

Figure 3-35 Optical flow process for BIO

For most often used case τ0 = τ1 = τ using of Hermite interpolation leads to

Pred = Pτ0 = I 0 +  I 12 +  τ 2 It 0- It 12

Combining two equations finally we obtain

Pred = I 0+  I 12 - τ 2vx Ix 0-Ix 1 +  vyIy 0- Iy 12

Full proof of this formula with all details can be found in [13]. Note that formula has fourth order of accuracy while half-sum only has second.

Motion refinement vector calculation

The BIO is applied as modification of the classical bi-prediction. The motion refinement vector vx, vy minimizes the difference between samples and derivatives values in window  surrounding points A and B (Figure 3-35):

vx τ = σ1+ r > m ? Clip3-l, l, -σ3σ1+ r : 0

vyτ = σ5 + r > m ? Clip3-l, l,- σ6-vxσ22σ5+ r : 0

where

σ 1 =i', j'∈ ΩIx 1+ Ix 02

σ3 =i', j'∈ ΩIx 1+ τ0 Ix 0 I 1- I 0

σ2 =i', j'∈ ΩIx 1+ Ix 0 Iy 1+ Iy 0

σ5=i', j'∈ ΩIy1+Iy02

σ6  =i',  j'∈ ΩIy 1+ Iy 0 I 1- I 0

In order to prevent division by 0 small regularization term r is introduced in the denominator. Note that optical flow model can be used only for small motion vectors which is why clipping is introduced.

Clustering

For this proposal additional complexity reduction for motion vector refinement is introduced. Motion vectors are determined not for one pixel but for cluster of pixel. The size of cluster depends on size of prediction block. Suppose prediction block has size WxH then.

- if (W <= 4 or H <= 4) 		bio_cluster_size = 2

- else if (W <= 16 or H <= 16) 	bio_cluster_size = 4

- else				bio_cluster_size = 8

Gradients calculation

Gradients are calculated as convolution with 8-taps filters depending on fractional position

Table 3-10 8-tap filer coefficients

| Fractional part block MV | coefficients fracFilterk, |
| --- | --- |
| 0 | –4, 11, –39, –1, 41, –14, 8, –2 |
| 1/16 | –4, 10, –34, –10, 46, –14, 8, –2 |
| 1/8 | –3, 9, –29, –18, 49, –14, 8, –2 |
| 3/16 | –3, 7, –24, –23, 51, –13, 7, –2 |
| 1/4 | –2, 6, –19, –31, 53, –12, 7, –2 |
| 5/16 | –2, 4, –14, –37, 54, –10, 6, –1 |
| 3/8 | –1, 2, –9, –41, 53, –7, 4, –1 |
| 7/16 | –1, 0, –4, –46, 52, –4, 3, 0 |
| 1/2 | 0, –1, 0, –50, 50, 0, 1, 0 |
| 9/16 | 0, –3, 4, –52, 46, 4, 0, 1 |
| 5/8 | 1, –4, 7, –53, 41, 9, –2, 1 |
| 11/16 | 1, –6, 10, –54, 37, 14, –4, 2 |
| 3/4 | 2, –7, 12, –53, 31, 19, –6, 2 |
| 13/16 | 2, –7, 13, –51, 23, 24, –7, 3 |
| 7/8 | 2, –8, 14, –49, 18, 29, –9, 3 |
| 15/16 | 2, –8, 14, –46, 10, 34, –10, 4 |

Note that all above formulas are friendly to SIMD implementation.

##### Motion vector difference signs derivation

Signs of motion vector difference are not directly signaled to bistream. To define signs Motion vector Difference Signs Derivation (MVSD) procedure is performed during reconstruction process which consists from the following steps:

- generate list of MVD candidates based on received absolute values of MVD wherein each candidate results a different sign combination of the absolute values;

- calculate cost for each MVD candidate using template matching;

- ranking candidates by sorting based on calculated cost;

- taking MVD with an index in the sorted list according to the received index (rank) from the bistream.

In general case 4 MVD candidates are evaluated (+/– mvd_abs.x, +/– mvd_abs.y). If one of MVD components (x or y) is zero then only MVD 2 combinations are possible. Case when all MVD component are zero is trivial and no sign derivation is needed.

MVSD is applied only for translational blocks.

MVD candidates

MVP

Current Frame

Reference Frame

A

B

C

D

Figure 3-36. Illustration of Motion Vector difference Signs Derivation procedure

##### Decoder-side motion derivation

With decoder side motion derivation (DMVD) mode, motion information of a block is not signalled but derived at decoder side. A DMVD flag is signalled for a CU when its SKIP flag is true (DMVD SKIP mode), or this CU is inter coded and its AMVR flag is 0 (DMVD DIRECT mode). When the DMVD flag is true, an additional DMVD mode flag is signalled to indicate which method (bilateral matching or template matching) is to be used to derive motion information for the block. In DMVD mode, MV is derived by using luma samples only.

As shown in the Figure 3-37, the bilateral matching is used to derive motion information of the current CU by finding the closest match between two blocks along the motion trajectory of the current CU in two different reference pictures. Under the assumption of continuous motion trajectory, the motion vectors MV0 and MV1 pointing to the two reference blocks shall be proportional to the temporal distances, i.e., TD0 and TD1, between the current picture and the two reference pictures. As a special case, when the current picture is temporally between the two reference pictures and the temporal distance from the current picture to the two reference pictures is the same, the bilateral matching becomes mirror based bi-directional MV.

Figure 3-37 Bilateral matching

As shown in Figure 3-38, template matching is used to derive motion information of the current CU by finding the closest match between a template (top and/or left neighbouring blocks of the current CU) in the current picture and a block (same size to the template) in a reference picture.

Figure 3-38  Template matching

For the motion derivation process in DMVD SKIP/ DIRECT mode, a CU-level motion search is performed, an initial motion vector is derived for the whole CU based on bilateral matching or template matching. First, a list of MV candidates is generated and the candidate which leads to the minimum matching cost is selected as the starting point for further CU level refinement. Then a local search based on bilateral matching or template matching around the starting point is performed, the MV precision of each search point is above or equal to the AMVR flag indicated MV precision, and the MV results in the minimum matching cost is taken as the MV for the whole CU.

Besides the aforementioned DMVD SKIP/ DIRECT mode, it is also applied to generate the MVP candidate list of INTER mode. When applied to INTER mode, only CU level template matching search is applied. With template matching method, a new candidate is derived. The newly derived candidate by template matching is utilized as the default AMVP candidate.

The MV derivation of DMVD is performed in the following two steps:

First, a list of MV candidates is generated and the candidate which leads to the minimum matching cost is selected as the starting point for further CU level refinement.

The MV candidate set at CU level consists of:

- spatial neighboring motion vectors, including HEVC merge pattern left, top right, bottom left, top, top left neighboring MV and AVC pattern left, top neighboring MV.

- temporal neighboring motion vectors, including HEVC merge pattern temporal MV and 1 SFVC pattern temporal MV

When using bilateral matching, each valid MV of a merge candidate is used as an input to generate a MV pair with the assumption of bilateral matching. For example, one valid MV of a merge candidate is (MVa, refa) at reference list A. Then the reference picture refb of its paired bilateral MV is found in the other reference list B so that refa and refb are temporally at different sides of the current picture. If such a refb is not available in reference list B, refb is determined as a reference which is different from refa and its temporal distance to the current picture is the minimal one in list B. After refb is determined, MVb is derived by scaling MVa based on the temporal distance between the current picture and refa, refb.

When selecting the candidate from the candidate set at the CU level, the matching cost is the absolute sum difference (SAD) of bilateral matching or template matching.

Then, a local search based on bilateral matching or template matching around the starting point is performed, the MV precision of each search point is above or equal to the AMVR flag indicated MV precision, and the MV results in the minimum matching cost is taken as the MV for the whole CU.

MV refinement is a pattern based MV search with the criterion of bilateral matching cost or template matching cost. Two search patterns are supported – an unrestricted center-biased diamond search (UCBDS) and an adaptive cross search for MV refinement at the CU level. The MV is directly searched at the precision of requested luma sample MV accuracy for both search patterns. The search range of MV refinement for the CU step is set equal to 8 luma samples.

When performing a local search at the CU level, the matching cost is the absolute sum difference (SAD) of bilateral matching or template matching. When a motion vector points to a fractional sample position, motion compensated interpolation is needed. To reduce complexity, bi-linear interpolation instead of regular 8-tap HEVC interpolation is used for both bilateral matching and template matching.

In the template matching merge mode, the encoder can choose among uni-prediction from list0, uni-prediction from list1 or bi-prediction for a CU. A CU level flag, i.e. TMP_DIR, is signaled to indicate whether the template matching prediction is bidirectional or unidirectional.

Figure 3-39 Flowchart of the proposed template matching prediction

Flowchart of the proposed template matching prediction (TMP) is shown in Figure 3-39. Template matching process is firstly utilized to obtain motion vector MV0/MV1 from list0/list1 and corresponding template matching cost, i.e., cost0/cost1.

If the flag TMP_DIR is parsed to be ‘1’, bidirectional MCP will be performed using the derived MV0 and MV1.

If the flag TMP_DIR is parsed to be ‘0’, unidirectional MCP will be performed using the derived MVX (X=0 or 1). If cost0 is less than cost1, X is set to 0, otherwise X is set to 1.

The proposed template matching prediction is only applied to DMVD template matching mode for B frame.

The derived motion is used for both luma and chroma for MC inter prediction or as one of the MVP candidates. After MV is decided, final MC is performed using 8-taps interpolation filter for luma and 4-taps interpolation filter for chroma, when the derived motion is utilized for MC inter prediction.

### Mode decisions

The basic process for rule of mode decision is the same as that in HEVC and JEM. This proposal uses a same Rate-distortion optimization (RDO) and Rate-distortion optimized quantization (RDOQ) for selecting modes in the encoder that is in HEVC and JEM.

### De-blocking filtering

De-blocking filtering process is identical to HEVC except for the additional strong filter. It is observed that the blocking artifacts on the boundaries of large blocks is not effectively removed by the conventional strong filter. This happens because the filtering area among block boundary is not enough to remove artifact, so that additional strong filter with longer filter tap is applied for this case. The following conditions are added to the conventional strong filter condition to decide the type of strong filter. If the following condition is true, then longer tap strong filter is applied. The corresponding pixels on block boundary for filter condition are illustrated in Figure 3-40.

Condition 1:

p7-p0+ q7-q0 < β8

Condition 2: block width of P and Q are greater than 8 in horizontal direction filtering

block height of P and Q are greater than 8 in vertical direction filtering

Figure 3-40 Pixel position to check strong filtering condition

The filter coefficients of new strong filter are shown in Table 3-9 and Table 3-10. The pixel position filled by orange color indicates the filtered output pixel position. A total of 16 pixels (p7 ~ q7) are filtered for horizontal direction filtering while a total of 12 pixels (p3 ~ q7) are filtered for vertical direction filtering due to memory bandwidth efficiency.

Table 3-11 New strong filter coefficients for horizontal direction

| Output pixel | Filter coefficients | Input pixels |
| --- | --- | --- |
| p6 | {3, 5, 2, 1, 1, 1, 1, 1, 1} | p7 ~ q0 |
| p5 | {1, 2, 5, 2, 1, 1, 1, 1, 1, 1} | p7 ~ q1 |
| p4 | {1, 3, 3, 3, 1, 1, 1, 1, 1, 1} | p6 ~ q2 |
| p3 | {1, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1} | p6 ~ q3 |
| p2 | {1, 2, 2, 2, 2, 2, 2, 1, 1, 1} | p5 ~ q3 |
| p1 | {1, 1, 2, 2, 2, 2, 2, 2, 1, 1} | p5 ~ q3 |
| P0 | {1, 2, 2, 2, 2, 2, 2, 2, 1} | p4 ~ q3 |
| q0 | {1, 2, 2, 2, 2, 2, 2, 2, 1} | p3 ~ q4 |
| q1 | {1, 1, 2, 2, 2, 2, 2, 2, 1, 1} | p3 ~ q5 |
| q2 | {1, 1, 1, 2, 2, 2, 2, 2, 2, 1} | p3 ~ q5 |
| q3 | {1, 1, 1, 1, 1, 2, 2, 2, 2, 2, 1} | p3 ~ q6 |
| q4 | {1, 1, 1, 1, 1, 1, 3, 3, 3, 1} | p2 ~ q6 |
| q5 | {1, 1, 1, 1, 1, 1, 2, 5, 2, 1} | p1 ~ q7 |
| q6 | {1, 1, 1, 1, 1, 1, 2, 5, 3} | p0 ~ q7 |

Table 3-12 New strong filter coefficients for vertical direction

| Output pixel | Filter coefficients | Input pixels |
| --- | --- | --- |
| p2 | {3, 5, 2, 2, 1, 1, 1, 1} | p3 ~ q3 |
| p1 | {2, 2, 4, 2, 2, 2, 1, 1} | p3 ~ q3 |
| p0 | {2, 2, 2, 2, 2, 2, 2, 2} | p3 ~ q3 |
| q0 | {1, 2, 2, 2, 2, 2, 2, 2, 1} | p3 ~ q4 |
| q1 | {1, 1, 2, 2, 2, 2, 2, 2, 1, 1} | p3 ~ q5 |
| q2 | {1, 1, 1, 2, 2, 2, 2, 2, 2, 1} | p3 ~ q5 |
| q3 | {1, 1, 1, 1, 1, 2, 2, 2, 2, 2, 1} | p3 ~ q6 |
| q4 | {1, 1, 1, 1, 1, 1, 3, 3, 3, 1} | p2 ~ q6 |
| q5 | {1, 1, 1, 1, 1, 1, 2, 5, 2, 1} | p1 ~ q7 |
| q6 | {1, 1, 1, 1, 1, 1, 2, 5, 3} | p0 ~ q7 |

At the encoder, for each reconstructed block, the top and left boundaries are further filtered by the deblocking filter during RDO process. That is the influence of deblocking filter to reconstructed pixels are counted in the RDO process.

#### Sample adaptive offset

The key idea of SAO is to reduce mean sample distortion of a region by first classifying region samples into multiple categories with a selected classifier, obtaining an offset for each category, and then adding the offset to each sample of category, where the classifier index and the offsets of the region are coded in the bitstream while the classification of each sample is performed at both the encoder and decoder with the selected classifier to save the amount of side information to be signaled.

To achieve low latency of only one Coding tree unit (CTU), a CTU-based syntax design is specified to adapt SAO parameters for each CTU. A CTU -based optimization algorithm can be used to derive SAO parameters of each CTU, and the SAO parameters for each CTU are interleaved into slice data. To further reduce the bits dedicated on SAO parameters, a merge flag is employed to indicate whether the SAO parameters of the current CTU is exact the same with its neighbors. When merge flag is enabled, all the following SAO parameters are not signaled but inferred from neighbors.

SAO uses different offset values for each sample in a CTU depending on the sample classification, and SAO parameters are adapted from CTU to CTU, that is, SAO parameters are derived and signaled for each CTU. In this contribution, two types of classifiers are proposed: edge offset mode (EO) and band offset mode (BO). In edge offset mode, each sample in a CTU is classified into one of five categories based on the current sample and its neighbors specified by one of four pre-defined patterns shown in Figure 3-41. The offset values to be added to the reconstructed samples are signaled for each category except the category 0 in which the edge offset is not used. In band offset mode, the whole sample value range is divided into 32 bands, and offset values are derived for each band and signaled.

Figure 3-41 Four 1-D directional EO patterns

In order to keep a reasonable balance between complexity and coding efficiency, the edge offset mode uses four 1-D directional patterns for sample classification: horizontal, vertical, 135° diagonal and 45°diagonal shown as Figure 3-41. In Figure 3-41, the label “c” represents the current sample and the labels “a” and “b” represent two neighboring samples. For a given EO patterns, samples of current CTU are classified into one of five categories. The classification rules to determine the category for each sample are summarized in Table 3-13. For each category, an offset is derived by an encoder and transmitted to a decoder. The mode information and the index of classification pattern are transmitted as side information as well. In category 1 and 4 associated with a local valley and a local peak along the selected 1-D pattern, the range of offset is set to [–1, 6] and [–6, 1], respectively. In category 2 and 3 associated with concave and convex corners along the selected 1-D pattern, the range of offset is set to [0, 1] and [–1, 0] respectively. If the current sample does not belong to EO categories 1-4, then it is classified as category 0, and EO is not applied.

Table 3-13 Classification rules for edge offset

| Category | Condition | Offset Range |
| --- | --- | --- |
| 1 | c < a && c < b | –1 <= offset <= 6 |
| 2 | ( c < a && c==b) ¦¦ (c == a && c < b) | 0 <= offset <= 1 |
| 3 | ( c > a && c==b) ¦¦ (c == a && c > b) | –1 <= offset <= 0 |
| 4 | c > a && c > b | –6 <= offset <= 1 |
| 0 | None of the above | None |

For BO, the whole sample value range is equally divided into 32 bands. For 10-bit samples ranging from 0 to 1023, the width of a band is 32, and sample values from 32k to 32k+31 belong to band k, where k ranges from 0 to 31. Offsets are derived by an encoder for bands to be modified, and transmitted to a decoder. The mode information is transmitted as side information as well. Thus, there are 4 offsets in the bitstream if band offset mode is selected. The range of band offset for all the bands is [–7, 7].

#### Noise suppression filtering

Noise Suppression (NS) filtering is applied for CTU and consists of the following steps:

- CTU partitioning into a set of reference square sub-blocks 8x8 size which is called root sub-block

  - for each of the root sub-blocks:

    - estimation of luma variance inside sub-block and making decision about further sub-bloc processing (if luma variance is less threshold then this sub-block is not processed)

    - determining a cluster of pixel-domain patches that are similar to the reference sub-block up to 8 patches in cluster;

    - for each pixel of reference sub-block 1D Hadamard transform (transform size is 8) of all collocated pixels from all patches inside the cluster

    - transform domain filtering of Hadamard spectrums using following formula

  - fi=fi3fi2+σ2, where fi is a certain frequency in Hadamar spectrum, and σ is filtering parameters which is derived based on QP.

    - 1D inverse Hadamard transform

    - pixel-domain averaging of pixels with same spatial coordinates in different patches from different clusters

Determining the cluster of pixel-domain patches that are similar to the reference sub-block comprises the following steps:

  - For sub-blocks on main diagonal of Super Block (128x128 non overlapped blocks) perform search within the search region (limited by current CTU) of the best 8 offsets (in respect to reference sub-block) with minimal distance metrics between root sub-block

  - Collect best offsets for all diagonals sub-blocks together in one list of best offsets

  - For rest blocks of Super Block search best 8 offsets (in respect to root sub-block) with minimal distance metric between root sub-block and candidate sub-blocks (limited by current CTU) , wherein tested offset candidates are chosen from list of best offsets.

NS filtering application map is determined on encoder side and signaled to decoder to define whether sub-blocks from certain region of picture are filtered.

NS key features:

- non-linear collaborative filtering in transform domain (1D Hadamard)

- filtering parameters are not transferred and derived on information already presented in bitstream

- filtering process is strictly limited by CTU

- parallel-friendly filtering (each sub-block is processed independently from others)

- multiplication free filtering (only adding, subtraction and LUT reading are used)

On the encoder side NS filtering is performed the same manner as on decoder side. Transform domain filtering parameter σ is pre-calculated before processing based on QP (matching to original image is not needed). After NS filtering the RDO process determines which part of reconstructed frame should be filtered. During RDO process filtered frame is split into a set of square regions. The 4 different sub-block size are estimated namely 256x256, 128x128, 64x64, 32x32. Also whole frame filtering on/off option is supported. For each region cost value is estimated for filtered and reconstructed blocks. If cost value of filtered block is less in respect to cost value of original reconstructed block then current block should be processed by NS filter.

#### Adaptive loop filtering

The adaptive loop filter (ALF) in this proposal is based on ALF in JEM7.0 but the temporal ALF prediction and filter coefficient prediction from predefined fixed filters are prohibited, and the luma and chroma unified ALF is added as follows.

In the proposed unified ALF mode, the chroma sample classification and on/off decision directly re-use the results of the co-located luma sample so that there is no complexity increase for the chroma classification. The chroma ALF coefficients also re-use those of luma but with a constant 5x5 diamond taps to keep the complexity of chroma filtering unchanged. The ALF coefficients conversion from luma 9x9 taps to chroma 5x5 taps is shown in Figure 3-42 and equations (7)-(13). For luma 7x7 and 5x5 taps, the outermost coefficients in Figure 3-42 (a) are set to 0 accordingly, and the equations (7)-(13) still work. As a special case, for luma 5x5 taps, the luma ALF coefficients are exactly the same as chroma.

- (b)

Figure 3-42 9x9 diamond taps luma filter (a) and 5x5 diamond taps chroma filter (b)

|  | C0=L0+L1+L2+L3+L6 | (7) |
| --- | --- | --- |
|  | C1=L4+L5+L10+L11 | (8) |
|  | C2=L12 | (9) |
|  | C3=L7+L8+L13+L14 | (10) |
|  | C4=L9+L15+L16+L17+L18 | (11) |
|  | C5=L19 | (12) |
|  | C6=L20 | (13) |

Figure 3-43: illustrates the idea behind the coefficients conversion from luma to chroma as shown in (7)-(13). In the Figure 3-43:(a) the luma coefficients are directly applied to chroma pixels resulting in an increased number of pixels (9x9 diamond) participating in the chroma ALF processing. In order to keep the chroma ALF still using only 5x5 diamond pixels, the pixels out of the 5x5 diamond border are replaced by the pixels lying in the 5x5 diamond border as shown in Figure 3-43:(b). Apparently, applying the 9x9 diamond coefficients to a border-extended 5x5 diamond pixels as shown in Figure 3-43:(b) is equivalent to applying a 5x5 diamond coefficients to the 5x5 diamond pixels as shown in Figure 3-43:(c). The 5x5 diamond coefficients in Figure 3-43:(c) are converted from the 9x9 diamond coefficients as shown in (7)-(13).

- Apply 9x9 luma coeff.(left) to chroma pixels (right) directly

- Apply 9x9 luma coeff.(left) to border-extended chroma pixels (right) for simplification

- Apply 5x5 coeff. (left) to 5x5 chroma pixels (right). Equivalent to (b)

Figure 3-43:The evolution process of the coefficients conversion from luma 9x9 diamond to chroma 5x5 diamond.

Besides the proposed unified ALF mode, the original separate ALF for luma and chroma as in JEM is also supported. For the separate ALF mode, a CTU level on/off control for chroma ALF is newly added in this proposed method. A flag is added at picture level to indicate whether the unified mode or the separate mode is used.

Replacing multiplication with bit-shifting:

In the proposed ALF, the filter coefficients used in filtering of the luma component is divided into 2 groups. The first group contains that coefficients {L0, L1, L2, L3, L4, L5, L7, L8, L9, L10. L14, L15, L16, L17}, and the second group contains the rest of the coefficients. The first group coefficients are normatively restricted to use only the following set of values: {–64, –32, –16, –8, –4, 0, 4, 8, 16, 32, 64}, hence multiplication with those coefficients can be implemented as a single bit shit operation. The allowed filter coefficient values are mapped to {–5, –4, –3, –2, –1, 0, 1, 2, 3, 4, 5} respectively before binarization in order to reduce the signaling overhead.

#### Bilateral filtering

The basic process for BLF is the same as in JEM. After a block is reconstructed, each luma sample in the reconstructed block is replaced by a weighted average of itself and its neighbours within the block. The weights are calculated based on the distance from the center sample as well as the difference in sample values.

### Transforms and transform type selection

#### Multiple core transform

DCT is widely used in video compression for residual coding since it provides good energy compaction in low-frequency coefficients. HEVC used two kernel designs for transform coding i.e. DCT-II and DST-VII. In this contribution, DST-VII and DCT-VIII have been exploited additionally on top of DCT-II which is normally used for core transform. The concept of multiple core transform was introduced in [14], and this contribution follows the similar concept of the study in [14]. Table 3-14 shows the basis functions of for kernel design of MTR.

Table 3-14 Transform basis functions of DCT- VIII and DST-VII for N-point input

| Transform Type | Basis function Ti(j), i, j = 0, 1,…, N−1 |
| --- | --- |
| DCT-VIII | Ti(j )= 42N+ 1∙cosπ∙(2i+ 1)∙(2j+ 1)4N+ 2 |
| DST-VII | Ti(j) = 42N+ 1∙sinπ∙(2i+ 1)∙(j+ 1)2N+ 1 |

MTR is applied to the block that is smaller than 32 block size with both width and height. If width or height is over 32 pixels in length, the block is not considered to be applied for MTR and not signaled to decoder.

A flag is used to signal to the decoder whether MTR applied or not. Basically, encoder decides the use of MTR based on RDO process in CU block. If encoder selects using MTR in a CU block for core transform, two flags are additionally signaled to decoder to indicate which type is used for the horizontal and vertical directions. Each flag indicates the kernel type used among two candidates at a direction. For candidates for inter and intra block, DCT and DST types are pre-defined as {0: DST-VII, 1: DCT-VIII} for intra block and 0: DCT-VIII, 1: DST-VII} for inter block as shown in Table 3-15.

Table 3-15 Candidates for MTR

| Type | Candidates |
| --- | --- |
| Intra block | 0: DST-VII, 1: DCT-VIII |
| Inter block | 0: DCT-VIII, 1: DST-VII |

#### Secondary transform

The idea of secondary transform is to slightly redistribute energy after core transform and hence reduce the quantization error. Introducing full-sized additional transforms leads to significant increment of implementation cost. Also, in lossy compression, high-frequency transform coefficients are often quantized to zero. With this motivation, the proposed scheme applies secondary transforms only to the low-frequency components after core transform. This allows keeping the core transform kernels in a codec un-changed, but efficiently adapting transform by adding a small sized additional transform kernel.

The STR process is based on exchanging two values in a given angle. Such a process is known as lossless change while keeping energy. As shown in the Figure 3-44, the (x, y) can be changed to (x’, y’) or revert from (x’, y’) to (x, y) in a given angle (e.g. ). This is a basic process for STR to enhance every compaction in a low-frequency area.

Figure 3-44 Value exchange between x and y in a given angle

Applied block size for STR depends on the size of CU. If CU size is over 8x8, STR applied with 8x8 based value exchanging process; otherwise, 4x4 based value exchanging process is conducted. Those 4x4 and 8x8 based value exchange process can be implemented with butterfly structure. Figure 3-45 shows example of 4x4 based value exchange process in given angles depending on value position. In the Figure 3-45, the first and second stage is the process for exchanging values in the direction of x-direction in 2-dimension; the third and four stage is the process for exchanging values in the direction of y-direction. In order to make output values 32 angles are needed in a round.

Figure 3-45 4x4 based value exchange process

The process for 8x8 based value exchange is similar to the process for 4x4 based value exchange. Input and output are 64 values, and 16 stages are conducted in total with 192 angles.

Similar to MTR syntax structure, a flag is used to signal to decoder whether STR applied or not. The use of STR is decided based on RDO process in a CU block. If encoder selects using STR in a CU block for secondary transform, two flags are additionally signaled to decoder to indicate which mode is used for secondary transform. Total available modes in STR are four, the four modes are predefined with angle table based on a used intra mode for a block. Table 3-16 shows grouping of intra prediction for candidates of STR mode. STR uses one of group id for a predefined table containing four angles sets. Based on RDO process, one of modes is selected.

Table 3-16 Grouped id depending on index of intra prediction

| Grouped id | Intra prediction index |
| --- | --- |
| 0 | 1, 2 |
| 1 | 0 |
| 2 | 3, 4, 51 |
| 3 | 5, 6, 48, 49, 50 |
| 4 | 7, 8, 46, 47 |
| 5 | 9, 9, 11, 43, 44, 45 |
| 6 | 12, 13, 41, 42 |
| 7 | 14, 15, 39, 40 |
| 8 | 16, 38 |
| 9 | 17, 18, 19, 35, 36, 37 |
| 10 | 20, 21, 32, 33, 34 |
| 11 | 22, 23, 24, 30, 31 |
| 12 | 25, 26, 28, 29 |
| 13 | 27 |

In order to enhance energy compaction, additional round of STR and 90 angle based value exchange are conducted. Second rounds with a 90 angle based value exchange are used for 4x4 based value exchange process; fourth rounds with a 90 angle based value exchange are used for 8x8 based value exchange process.

### Quantization and quantization type selection

The basic process for quantization is the same as in HEVC. This proposal uses the same QP range, quantization step, and corresponding scaling process for transformed coefficients.

#### Adaptive quantization step size scaling

Visual distortion of a block is influenced by the background of the block. Luminance masking and contrast masking effect of the background are considered in the quantizer design, in order to make a more perceptually uniform quantizer. The supporting background are two lines around the current CU.

A luminance factor fl(Ba) is derived based on the average luminance of the background Ba (normalized to 8-bit),

flBa=1+60-Ba150,       Ba∈0,60   1,                      Ba∈60,170       1+Ba-170425,    Ba∈170,255.

A contrast masking factor fcBc is derived based on the Mean Absolute Difference (MAD) between the background luminance and the average luminance (i.e., a way to measure the contrast Bc).

fcBc=1,            Bc≤5Bc+510,     Bc>5.

The scaler is based on the product of the two factors and is constrained in a range of [2/2,2]. The scaler is then applied to scale the quantization step size. For the flexibility of the quantizer, a slice-header parameter s is signaled to further adjust the scaler. At encoder, the parameter s is derived from the geometric mean of masking factors calculated from original luma samples of 8x8 blocks inside the slice, and is written into the slice header.

qs'= qsQP×scaler=qsQP×clipflBa×fcBc×s2, 0.5, 2.0

To avoid floating-point calculation, the factors and scaler are all integerized into a 9-bit integers, and the squared root operation is realized by a look-up table with 33 entries.

The quantization step size scaler is derived based on the reconstructed luma samples around the current CU, which is the same as that in the decoder side. For a same position CU reached from different split routes, the surrounding reconstructed pixels may be different and hence the scaler may be different. The scaler is then used in the quantization process to adjust the quantization step size.

### Entropy coding

#### Arithmetic coding engine and multi-hypothesis probability update

In this proposal, except initialization and probability update, all other parts of entropy coding are same as that described in Annex D [11].

The binary arithmetic coder encodes a series of zeros and ones. The basis for the binary encoding is a well–known procedure of recursive interval subdivision. Suppose R is current interval and P is probability of Least probable symbol then from theoretical standpoint the optimal subdivision can be PR and (1–P)R. It should mentioned the more precise interval subdivision will be calculated the closer average bit saving will be to Shannon limit.

-p log2 p - (1-p) log2 (1-p)

It means that for LPS we spend approximately -log2 p bits for MPS -log2 (1-p) bits. This value can be pre-calculated and used for bits estimation in encoder side for wide diapason of probabilities.

Table 3-17 shows main difference between CABAC [16], Annex D and mCABAC. Note that the proposed range belongs to [215, 216]. It allows realization of optimal subdivision with high accuracy. However, for initialization process, interval subdivision and probability estimation is significantly different from Annex D.

Table 3-17 Comparison between proposed mCABAC with other known methods

| Theory | CABAC | Annex D | mCABAC |
| --- | --- | --- | --- |
| r = r * p | r = LUT[r, p][64][4] LPS < 0.5 range true | r = LUT[p][113] LPS < 0.5 range false | r = r * p LPS < 0.5 range false |
| p = update(p, b) (e.g. exponential smoothing) | p = b ? LUT0[p] : LUT1[p] exponential smoothing = 1 / 19.6 | p = b ? LUT0[p] : LUT1[p] Different from exponential smoothing | p1 = b + (p1 >> W1) p0 = (p0 >> W0) p = (p0 + p1) >> 1 Mixture of exponential models = 1 / 16, 1 / 128 |
| MPS / LPS | Yes | Yes | Yes |
| Range | [256, 512] | [215, 216] | [215, 216] |
| Carry | FF(11111111) | FF | FF |
| Termination bit | LPS = 2 | LPS = 1 | LPS = 1 |
| Initialization | (qp, slice) | 0.5 | (qp, slice) + index + switch to one model |
| Restriction for LPS | Needed >= 6 | >= 431 | >= 431 |

So-called “exponential smoothing” is most commonly used for prediction of time series. The exponential smoothing was first proposed by Charles C. Holt in 1957 [15]. In case of probability estimation this formula is:

Pi = αk∞bi - k 1-αk = α bi+ 1- α Pi - 1                                                            (14)

Where bi is equal “1” if bin is LPS and “0” if MPS. The parameter  controls the sensitivity of the model: smaller  corresponds slow decrease of weights in (14) and more members of time series are taken into account during probability estimation. It is also possible to say that value N=1/ determines average memory of system. In other words it describes how many of previously encoded bins have significant influence to the current probability estimation (for CABAC N 19.69). Sensitive system quickly reacts to real changing (Figure 3-46) but less sensitive model does not react to noise and random errors. Both properties are useful but contradictory. Our numerous experiments have proved that mixture of several estimation with different  can significantly improve final probability evaluation.

Implementation in integer arithmetic

Suppose that probability p in (2) is represented by integer number P from 0 to 2k on uniform mesh. If  is power of two (  = 1/2M) formula (2) can be written in multiplication free form:

Pnew = (B >> M) + Pold – (Pold >> M)                          (15)

Two-probabilities update model

As it was shown in [1] if signal has small positive autocorrelation one of the parameter α should be chosen as close as possible to zero. The best results were obtained with linear combination for probability estimation that consists of only two summands corresponding N0=16 (short window) and N1=128 (long window):

P0 = (Y >> 4) + P0 – (P0 >> 4)                              (16)

P1 = (Y >> 7) + P1 – (P0 >> 7)                             (17)

P = (P0 + P1 + 1) >> 1                                  (18)

Possible problem with wrong initialization

To start probability estimation it is necessary to initialize probability however there is no guarantee that it will be close to optimal for current context model. In Figure 3-46(left) optimal value 0.1 initial value 0.5. Black is Short window probability and red is long window size. For red line convergence to optimal value takes several hundred updates. That is why we introduce counter of updates. Before counter reaches 31 only short window size model is updated when number of updates exceeds 31 we start updating long window model with initial value borrowed from short window model (Figure 3-46 (right)). This approach significantly reduces negative effect from wrong initialization and also reduces complexity due to update of long window size model is not needed for first 31 updates.

Figure 3-46 Example of probability using different initialized value

Probability estimation process.

To determine context model state 32bits (4bytes) are needed

Table 3-18 Bit configuration

| Counter | Prob1 | Prob0 | Mps |
| --- | --- | --- | --- |
| 5 bits | 13 bits | 13 bits | 1 bit |

- bFastProbUpdate = counter < 31

- counter++

- MPS pass:

  - P0mps = P0 – (P0 >> 4)

  - P1mps = bFastProbUpdate ? P0mps: P1 – (P1 >> 7)

- LPS pass

  - P0lps = P0 + ((2^13 – P0) >> 4)

  - P1lps = bFastProbUpdate ? P0lps : P1 + ((2^13 – P1) >> 7)

  - If(P0 + P1 > 2^13)

    - mps = 1 – mps

    - P1lps = 2^13 – P1lps

    - P0lps = 2^13 – P0lps

- P= (P0 + P1) >> 1

#### Context initialization

The basic process for context initialization is the same as that of JEM. This proposal uses the initial probability states of context models for inter-coded slices by copying states from previously coded pictures.

#### Residual coding

Coefficient coding in HEVC is processed in coefficient group (CG) basis. A TU is partitioned into CGs with size of 4x4. Coefficients in each CG are coded from the last non-zero coefficient of last CG in one of three scan orders, which are horizontal scan, vertical scan, and diagonal scan. Last position (lastX, lastY) is coded first to determine the starting position of inverse scan. Then significant group flag is signaled for each CG to indicate whether there are non-zero coefficients in that CG. If there is at least one non-zero coefficient, all coefficients in that CG are inverse scanned to signal non-zero coefficients’ positions with significant flag. For those non-zero coefficients, greater than 1 flags (referred as GT1flag), and greater than 2 flags (GT2flag) are further signaled to indicate whether absolute levels are greater than 1, or greater than 2. If absolute levels are greater than 1 or greater than 2, the remaining levels (AbsMinusX) are coded by Truncated Golomb-Rice and Exp-Gobomb codes. Context modeling of significant flag depends on the position in a CG and corresponding right and bottom significant group flags. Context modeling of GT1flag depends previously coded GT1 flag in corresponding CG and on GT1 flag in previous CG, similar to GT2flag. Rice parameter for remaining level coding is updated based on previously coded level. Sign of each non-zero coefficient is bypass-coded except the hidden sign of last non-zero coefficient in each CG which can be implicitly derived by the parity of sum of previous decoded levels in that CG if the distance of first and last non-zero coefficients satisfy a given threshold.

Coefficient coding in JEM, follows basic coefficient coding scenario of HEVC except for the context modeling of significant flag, GT1flag, GT2 flag and rice parameter derivation. Context index of a significant flag is derived by the number of significant flag of five previously coded spatial neighboring coefficients. Context indices of GT1flag and GT2flag are derived by the number of corresponding flags in neighborhood similar to significant flag. Depending on the located coefficient position in a TU, color component, and transform size, different set of context models are used for significant flag, GT1 flag and GT2 flag coding. Rice parameter is determined by the sum of previously coded spatially neighboring coefficient as significant flag.

#### Transform domain residual sign prediction

Transform domain residual sign prediction (TD-RSP) is performed anywhere before inverse transform process. It contains the following steps:

- Preparation prediction of boundary residuals

- Calculation spectrum of boundary line

- Calculation cost function for hypothesis

- Sign values decoding

As the result of this process, decoded sign values for coefficients are obtained and stored in sign_pred_pos_list.

Preparation prediction of boundary residuals

Figure 3-47 Pixels used to derive cost value

The first step to get prediction of boundary residuals is to predict boundary pixels of current block. To realize it, a linear extrapolation of lines (Xi,j)/columns (Zi,j) is used:

PXi=2Xi,-1-Xi,-2PZi=2Z-1,i-Z-2,i

where PXi and PZi are linear extrapolations.

Pixels of a current block could be presented as a sum of residuals and an inter/intra-predictor:

Yi,j=Ri,j+Pi,j

where Ri,j and Pi,j denote residuals and a predictor, respectively.

The second step is to remove an inter/intra predictor and to perform forward transform:

xn=TPXi-Ri,0zm=TPZi-R0,i

Calculation spectrum of boundary line

Spectrum of neighboring boundary could be calculated as follows:

lyn=m=0M-1ωmRn,mcosπm2M,

tym=n=0N-1ωnRn,mcosπn2N,

where ωi=1M, i=02M, i>0, M is block height.

Calculation cost function for hypothesis

Cost function values should be calculated for every combination of signs to be predicted. So, the number of hypotheses is 2max_num_pred_signs . SSE is used as cost function:

costk=n=0N-1xn-lyn2+m=0M-1zm-tym2

Sign values decoding

Sign positions are consequently fetched from sign_pred_pos_list starting from the 1st one. At every step, hypothesis having minimal cost function value is chosen and coefficient signs are assigned in accordance with corresponding sign prediction errors. If a sign prediction error value is 1, a coefficient sign is assigned as specified by the hypothesis. Otherwise, the sign is inverted, i.e. “+” is changed to “–” and vice versa. Only those hypotheses are further checked that specify the same signs as previously restored using sign prediction error values as described above. This process is repeated for each element of sign_pred_pos_list.

### Additional tools

#### Adaptive Clipping

Similar to the adaptive clipping process in JEM, this proposal also applies the content adaptive clipping process to all reconstruction and prediction data. Unlike JEM, it is applied only to the luma component, and there is no smoothing filtering for residual. The clipping range, i.e. lower bound and upper bound, is signaled in the slice header, and it is quantized depending on bit depth of luma component.

### Additional encoder optimization

#### Multi-pass encoding

n/a

### Additional algorithmic description topic(s)

## 360°

There are many additional pre/post processing steps introduced over conventional 360°compression to improve the objective and subjective quality of compressed 360° video. These methods/algorithms are described in the following sections.

### Usage of projection format adaptation

#### Rotation

It has been observed that rotation helps in improving the coding gain for 360° video. Rotation of the content before projection can help in reducing the discontinuities in the complex region and align the edges in the content more suitable for the conventional coding scheme as well.

(b)

(a)

(d)

(c)

Figure 3-48 (a) Frame from Balboa sequence in RSP, (b) Optimally rotated frame, (c) Edge output of image in (a), (d) Edge output of image in (b)

It can be seen that figure Figure 3-48(c) has more curved edges while the optimally rotated Figure 3-48(d) has more horizontal and vertical edges. Optimally rotated image provides the highest BD bitrate among all the angles.

Figure 3-49: Block diagram of optimal rotation parameter estimation algorithm

To arrive at the optimal rotation angle without encoding the video we devised the following algorithm.

- Downsize the image to 2K size and generate 36 RSP images at angles multiple of 10 degrees starting from 10 to 360 degrees. (only for X axis rotation)

- Compute gradient magnitudes and angles for each of these 36 images

- Find the image with maximum number of vertical and horizontal gradients from these images. Here we consider gradients between -5 to 5 degrees as horizontal and -85 to 95 degrees as vertical.

- After arriving at the best angle at the granularity of 10 degree these step can be further repeated to reach to 1 degree resolution.

- This algorithm can also be applied at RAP level.

As of now this algorithm is only applied at the first frame of the sequence. Currently this algorithm computes only the optimal rotation in x axis but this can be extended to other two axes as well. Figure 3-50 shows the BD bitrate for luma component for 36 rotations starting from 0 to 360 at 10 degrees interval. It shows that the highest BD bitrate is correlated with highest number of Horizontal and Vertical edge gradients in the image. This supports the method which we used to arrive at the optimal rotation angle (x axis) without encoding the input frames.

Figure 3-50 Correlation between BD-bitrate and Horizontal + Vertical gradients in Balboa and KiteFlite sequence

Table 3-19 Optimal rotation angle computed from brute-force method vs proposed algorithm

| Sequence | Brute-force angle | Computed angle |
| --- | --- | --- |
| Balboa | 90 | 90 |
| ChairliftRide | 0 | 0 |
| KiteFlite | 80 | 0 |
| Harbor | 310 | 310 |
| Trolley | 10 | 10 |

Table 3-20 BD rate comparison for no rotation vs rotation of algorithm output

| Sequence | E2E WS PSNR Anchor (360 lib 5.0) |  |  | E2E WS PSNR - Rotated |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Balboa | -4.93% | -0.89% | -2.49% | -9.27% | -3.98% | -5.77% |
| ChairliftRide | -24.16% | -20.21% | -21.27% | -24.16% | -20.21% | -21.27% |
| KiteFlite | -6.13% | -8.41% | -10.06% | -6.13% | -8.41% | -10.06% |
| Harbor | -4.35% | -6.64% | -5.64% | -7.31% | -7.71% | -8.46% |
| Trolley | -4.99% | -1.42% | -7.54% | -5.34% | -1.35% | -7.23% |
| Average | -8.91% | -7.51% | -9.51% | -10.44% | -8.33% | -10.56 % |

Table 3-21 shows the BD bitrate gain using rotation for CfP sequences for the optimal rotation vs no rotation encoding. We see around 1.5% compression gain for luma component on an average with maximum gain for balboa sequence around 4.3% for the luma component.

#### Adjustment

Generally the 360° images have more details/high texture content present near equator as compared to poles. It has been observed that if equator is stretched and poles are shrunk in the same proportion then the coding gain is improved. With this assumption we tried to adjust ERP image before converting them to RSP format with different level of adjustment using below formula,

len= x2+y2+z2

org_pitch=sin-1(ylen)

yadj=-0.5*sinβ*org_pitchsin0.5β*π +0.5

And to transform from RSP to ERP, we use

pitch=1β*sin-10.5-v*sinπ*β2

where β ϵ (0, 1]. If β is equal to 0, adjustment is not applied.

Figure 3-51: (a) adjusted ERP image (b) actual ERP image from Balboa sequence

Figure 3-52: Effect of the adjustment on RSP format

Figure 3-52 indicates that which part of RSP gets stretched while which parts are shrunk with the adjustment. We propose an algorithm to arrive at the optimal value of β without explicitly encoding the frames.

Figure 3-53 Block diagram of β estimation algorithm

Steps of the algorithm

- To compute the spread of the information first the gradient image is computed

- Accumulate square of gradient along each rows of the image which would produce a column matrix

- Locate the region where 95% of the image energy is concentrated

- Optimal adjustment parameter is derived from this width parameter using below equation

Optimal BETA= abs( -5*10-8*w2-w*10-4+0.5801 )

where w is the derived width from step 3.

Figure 3-54 Accumulating the energy of gradients

Figure 3-55 Energy distribution over the height of image

Figure 3-55 shows that in case of balboa sequence more texture is concentrated near the center while for Trolley sequence the information is more distributed. This indicates that adjustment parameter shall be higher for Balboa and less for Trolley. To arrive at the relation between the optimal beta and width we plot the optimal beta vs width parameter for all five CfP sequences and fit a quadratic curve which yielded the equation illustrated in Figure 3-56. We observe around 0.2% BD bitrate gain on an average for five CfP sequences and maximum gain of 0.4% in Balboa sequence.

Figure 3-56 Optimal β vs width

Table 3-22 Comparison of β value derived from algorithm as compared to brute-force

| Sequence | β Value (Brute force) | β value (algorithm) |
| --- | --- | --- |
| Balboa | 0.55 | 0.39 |
| ChairliftRide | 0.40 | 0.06 |
| KiteFlite | 0.05 | 0.03 |
| Harbor | 0.30 | 0.31 |
| Trolley | 0.25 | 0.002 |

#### In-active region filling

In RSP, we observe in-active regions near the ends of lobes that need to be encoded. We experimented and found that it would be better to fill the in-active regions with content similar to neighboring active region rather than just grey color. This can be also referred to as color filling.

In RSP, we observe in-active regions near the ends of lobes that need to be encoded. It was observed that it would be better to fill the in-active regions with content similar to neighboring active region rather than just grey color. This can be also referred to as color filling. As shown in Figure 3-57, we try to fill up the non-active regions 1, 2 by using the whole or part of neighboring highlighted content. We can compute either mean/median/mode using the neighboring active pixels to fill up the non-active region. Setting all neighboring inactive pixels with this value improves prediction of active pixels on the boundary. Figure 3-58 shows one frame of Balboa sequence with inactive region filled as explained above.

Color filling values are computed every intra frame and all inter frames encoded within a group of pictures (GOP) use the same colored filling value. From heuristics, we found out that for Luma channel, median value is preferable, while for Chroma channels, mode value is preferable. We observe 0.2% BD bitrate gain on an average for five CfP sequences.

Figure 3-57 (a) RSP Image without color filling (b) Top-left lobe end before color filling (c) Top-left lobe end after color filling

Figure 3-58 Example RSP frame with inactive pixels set using colored filling

Table 3-23 BD rate comparison for no color filling vs color filling output

| Sequence | E2E WS PSNR Anchor (360 lib 5.0) |  |  | E2E WS PSNR – Color Filling |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Balboa | -4.93% | -0.89% | -2.49% | -5.23% | -1.03% | -3.10% |
| ChairliftRide | -24.16% | -20.21% | -21.27% | -24.63% | -20.54% | -21.34% |
| KiteFlite | -6.13% | -8.41% | -10.06% | -6.22% | -8.25% | -10.76% |
| Harbor | -4.35% | -6.64% | -5.64% | -4.56% | -6.62% | -5.67% |
| Trolley | -4.99% | -1.42% | -7.54% | -5.03% | -1.35% | -7.68% |
| Average | -8.91% | -7.51% | -9.51% | -9.13% | -7.56% | -9.71% |

#### Blending on the seam boundaries

In blending, boundary content is updated with weighted average of itself and equivalent content inside padded region. In RSP, circular boundary of the lobes is filled by using spherically neighboring content and those pixels are used for blending. On top of spherical padding, we add some extra padding for aligning boundary/seams with CU. This is represented as step padding in Figure 3-59. The remaining in-active region is filled using color filling method as described in section 3.2.1.3.

Figure 3-59 Representative image of RSP lobe, showing different regions

Figure 3-60 Representative image of RSP, showing color mapping between RSP content and their counterpart in spherical padding which is used for blending

For each pixel inside the colored region of RSP content, we find its corresponding pixel values in spherical padding region using bilinear interpolation. Denote Pold as pixel intensity value inside RSP content before blending, Ppad as pixel intensity mapped inside spherical padding, Pnew as pixel intensity value inside RSP content after blending, we compute Pnew by the following

Pnew= Pold*d+ Ppad*(D-d) D

where d refers to distance of the pixel from boundary while D is number of pixels to be modified after blending within same column. We have observed significant improvement in subjective quality but loss in BD bitrate gain after introducing padding and blending for the discontinuous regions.

### 360° Video specific encoding tools

There is no 360° specific encoding tools are used for this proposal. Codec used here is generic which supports SDR and 360° with the same set of the tools.

### Additional encoder optimization

There is no 360° specific encoder optimization but deblocking filter is selectively turned off on the seam boundary during encoding. Figure 3-60 shows packed RSP format 360° image. The middle part where two non-spherical neighbors placed next to each other in 2D format creates discontinuities. We avoid performing deblock filtering process across this discontinuity. Discontinuity is indicated with yellow line in figure 3-60.

Figure 3-61 Packed RSP image with the seam location

## HDR

### Pre-processing

Figure 3-62  HDR Pre-processing workflow

Input HDR signal is processed by the pre-processing workflow in Figure 3-62 to obtain the pre-encoded HDR signal for the encoder. It includes two modules, adaptive quantization and remapping, in pre- processing. The pre-processing acts as a pre-encoding procedure before the HDR signal is input to the encoder, the output reconstructed HDR signal is bit-matched with the HDR signal before pre-processing. Three metadata, average/max/min are 12bits/10bits/10bits respectively, therefore totally 32bits are coded in sequence header as HDR metadata transmitted by the encoder and used for post-processing.

If input HDR signal is PQ signal, the luminance component of input HDR signal Y is forward to remapping module and is remapping to Y’ and then forward to adaptive quantization module to obtain quantized/scaling signal DY’ as the luminance component of pre-encoded signal as the input of the encoder; If input HDR signal is HLG signal, the remapping result Y’ is the luminance component of pre-encoded signal as the input of the encoder. For both PQ and HLG, the chrominance component of the input HDR signal is not changed and output as the chrominance component of pre-encoded signal as the input of the encoder

The adaptive quantization is as follows,

where T is a threshold and equals to 0.15, w and b are parameters for inverse quantization and w equals to -0.33 and b equals to 0.05 respectively, weight and offset equal to 1/876 and 64 for 10bit HDR signal.

For PQ and HLG signal, the inverse remapping is as derived as follows,

In which, the function of PQ/InversePQ are defined in ST.2084, and the function of HLG/InverseHLG are defined in ARIB STD-B67, and

where, a, b both equal to 1.12762,

m equals to 0.14 and p equals to 1.4 for PQ signal, and m equals to 0.3 and p equals to 2.3 for HLG signal.

### HDR specific encoding tools

Anisotropic SSD

Initial HDR content consists of RGB each belongs to [0, 10000]. In order to encode it by conventional codec it is necessary to convert to YUV (10bits). It can be done by following consequent steps

1. Convertion to float RGB

R’ = PQ – TF(max(0, min(R/10000, 1)))

G’ = PQ – TF(max(0, min(G/10000, 1)))

B’ = PQ – TF(max(0, min(B/10000, 1)))

Figure 3-63 PQ transfer function

2. Conversion to float YUV

| BT.2020 | BT.709 |
| --- | --- |
| Y' = 0.2627 * R' + 0.6780 * G' + 0.0593 * B' Cb = B '- Y'1.8814 Cr = R '- Y'1.4746 | Y' = 0.2126 * R' + 0.7152 * G' + 0.722 * B' Cb = B '- Y'1.8556 Cr = R '- Y'1.5748 |

3. Conversion to integer YUV

DY’ = Clip1Y ( Round (( 1<< (BitDepthY – 8) * (219 * Y’ + 16)))

DCb’ = Clip1C ( Round (( 1<< (BitDepthC – 8) * (224 * Y’ + 128)))

DCr’ = Clip1C ( Round (( 1<< (BitDepthC – 8) * (224 * Y’ + 128)))

In inverse transform any errors for bright pixels will be dramatically amplified. To reduce this negative effect we introduce anisotropic SSD.

| SSD | Anisotropic SSD |
| --- | --- |
| SSD(O ,R) = iNOi- Ri 2 | ASSD(O ,R)= iNw Oi+Ri64Oi- Ri2∕ 64 |

where w[i]=49+i

It allows introducing additional penalty for errors in bright diapason during encoder decision.

Note that these changes have effect only for HDR metrics.

### Additional encoder optimization

n/a

# Compression performance

## SDR: Constraint set 1 configuration relative to HM anchor

### Class SDR-A (RD-curves)

### Class SDR-B (RD-curves)

### Overall (BD-rate summary tables)

|  | BD-rate (MsSSIM) | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FoodMarket4 | -41.02% | -35.24% | -28.31% | -31.85% | 1.77 | 0.76 | 0.95 |
| CatRobot1 | -46.94% | -41.43% | -51.66% | -45.97% | 1.63 | 0.94 | 1.31 |
| DaylightRoad2 | -46.57% | -41.08% | -54.77% | -40.80% | 1.15 | 1.16 | 0.76 |
| ParkRunning3 | -53.35% | -43.25% | -13.94% | -13.39% | 1.53 | 0.22 | 0.17 |
| Campfire | -52.83% | -46.72% | -25.02% | -53.50% | 1.45 | 0.87 | 1.68 |
| BQTerrace | -40.04% | -31.11% | -44.03% | -50.58% | 0.94 | 0.80 | 0.92 |
| RitualDance | -39.14% | -30.47% | -32.10% | -36.59% | 1.65 | 0.88 | 1.19 |
| MarketPlace | -38.50% | -32.63% | -29.52% | -26.49% | 1.17 | 0.47 | 0.44 |
| BasketballDrive | -43.23% | -34.72% | -41.86% | -40.53% | 1.27 | 0.92 | 1.23 |
| Cactus | -40.13% | -35.34% | -38.99% | -35.84% | 1.45 | 0.61 | 0.90 |
| Average SDR-A | -48.14% | -41.54% | -34.74% | -37.10% | 1.51 | 0.79 | 0.97 |
| Average SDR-B | -40.21% | -32.85% | -37.30% | -38.01% | 1.30 | 0.73 | 0.94 |
| Average all | -44.17% | -37.20% | -36.02% | -37.55% | 1.40 | 0.76 | 0.95 |

Beside BD-rate PSNR gain, the BD-rate MsSSIM is also provided in the above table. It shows that the proposed solution achieves 44.17% BD-rate MsSSIM gain compared to HM16.16.

The proposed method with the adaptive quantization disabled has also been tested. The results are as follows:

|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| --- | --- | --- | --- | --- | --- | --- |
| FoodMarket4 | -37.13% | -34.45% | -38.07% | 1.89 | 0.96 | 1.17 |
| CatRobot1 | -43.84% | -53.51% | -47.81% | 1.75 | 0.98 | 1.38 |
| DaylightRoad2 | -43.57% | -58.97% | -46.86% | 1.23 | 1.29 | 0.91 |
| ParkRunning3 | -42.97% | -15.86% | -15.68% | 1.52 | 0.25 | 0.20 |
| Campfire | -46.87% | -25.37% | -51.71% | 1.46 | 0.88 | 1.61 |
| BQTerrace | -34.22% | -41.67% | -48.56% | 1.04 | 0.74 | 0.87 |
| RitualDance | -32.08% | -35.70% | -40.18% | 1.76 | 1.01 | 1.34 |
| MarketPlace | -33.49% | -36.70% | -33.87% | 1.21 | 0.62 | 0.61 |
| BasketballDrive | -35.87% | -42.31% | -40.69% | 1.32 | 0.93 | 1.24 |
| Cactus | -37.59% | -42.82% | -39.54% | 1.57 | 0.68 | 1.01 |
| Average SDR-A | -42.88% | -37.63% | -40.02% | 1.57 | 0.87 | 1.05 |
| Average SDR-B | -34.65% | -39.84% | -40.57% | 1.38 | 0.80 | 1.01 |
| Average all | -38.76% | -38.74% | -40.30% | 1.47 | 0.83 | 1.03 |

## SDR: Constraint set 2 configuration relative to HM anchor

### Class SDR-B

### Overall

|  | BD-rate (MsSSIM) | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BQTerrace | -33.24% | -22.62% | -39.27% | -41.26% | 0.66 | 0.43 | 0.42 |
| RitualDance | -32.20% | -23.77% | -27.91% | -30.72% | 1.23 | 0.63 | 0.82 |
| MarketPlace | -28.54% | -23.15% | -28.44% | -24.69% | 0.75 | 0.34 | 0.32 |
| BasketballDrive | -37.78% | -29.67% | -37.07% | -40.40% | 1.14 | 0.76 | 1.17 |
| Cactus | -31.52% | -24.58% | -33.85% | -33.88% | 0.96 | 0.46 | 0.73 |
| Average SDR-B | -32.66% | -24.76% | -33.31% | -34.19% | 0.95 | 0.52 | 0.69 |

Beside BD-rate PSNR gain, the BD-rate MsSSIM is also provided in the above table. It shows that the proposed solution achieves avg. 32.66% BD-rate MsSSIM gain compared to HM16.16.

The test results with the adaptive quantization disabled are as follows:

|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| --- | --- | --- | --- | --- | --- | --- |
| BQTerrace | -27.36% | -39.05% | -40.57% | 0.82 | 0.43 | 0.41 |
| RitualDance | -24.81% | -32.50% | -34.89% | 1.29 | 0.77 | 0.96 |
| MarketPlace | -23.12% | -35.62% | -31.58% | 0.75 | 0.45 | 0.42 |
| BasketballDrive | -29.83% | -37.88% | -40.87% | 1.14 | 0.78 | 1.19 |
| Cactus | -28.22% | -38.24% | -38.96% | 1.13 | 0.53 | 0.86 |
| Average SDR-B | -26.67% | -36.66% | -37.37% | 1.03 | 0.59 | 0.77 |

## SDR: Constraint set 1 configuration relative to JEM anchor

### Class SDR-A

### Class SDR-B

### Overall

|  | BD-rate (MsSSIM) | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FoodMarket4 | -11.16% | -2.99% | 20.73% | 19.91% | 0.11 | -0.50 | -0.52 |
| CatRobot1 | -14.03% | -3.70% | -0.08% | -1.32% | 0.10 | -0.01 | 0.02 |
| DaylightRoad2 | -11.38% | -1.38% | 3.44% | -1.30% | 0.03 | -0.07 | 0.00 |
| ParkRunning3 | -30.63% | -18.12% | 15.13% | 19.47% | 0.54 | -0.21 | -0.21 |
| Campfire | -29.59% | -20.23% | 14.32% | -7.97% | 0.45 | -0.42 | 0.16 |
| BQTerrace | -18.82% | -1.14% | 0.68% | 6.91% | 0.03 | -0.02 | -0.11 |
| RitualDance | -16.26% | -4.44% | 6.21% | 3.84% | 0.20 | -0.18 | -0.13 |
| MarketPlace | -13.70% | -5.21% | 28.15% | 25.97% | 0.15 | -0.36 | -0.37 |
| BasketballDrive | -19.49% | -5.36% | 4.60% | 3.16% | 0.15 | -0.09 | -0.08 |
| Cactus | -12.38% | -0.50% | 7.94% | 7.20% | 0.02 | -0.11 | -0.17 |
| Average SDR-A | -19.36% | -9.28% | 10.71% | 5.76% | 0.24 | -0.24 | -0.11 |
| Average SDR-B | -16.13% | -3.33% | 9.52% | 9.42% | 0.11 | -0.15 | -0.17 |
| Average all | -17.74% | -6.31% | 10.11% | 7.59% | 0.18 | -0.20 | -0.14 |

Beside BD-rate PSNR gain, the BD-rate MsSSIM is also provided in the above table. It shows that the proposed solution achieves avg. 17.74% BD-rate MsSSIM gain compared to JEM7.0.

The test results with the adaptive quantization disabled are as follows:

|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| --- | --- | --- | --- | --- | --- | --- |
| FoodMarket4 | -6.02% | 13.51% | 12.42% | 0.23 | -0.30 | -0.30 |
| CatRobot1 | -7.83% | -3.39% | -4.23% | 0.21 | 0.03 | 0.09 |
| DaylightRoad2 | -5.58% | -5.30% | -9.69% | 0.11 | 0.07 | 0.15 |
| ParkRunning3 | -17.75% | 12.50% | 16.22% | 0.52 | -0.17 | -0.18 |
| Campfire | -20.50% | 13.85% | -4.33% | 0.46 | -0.41 | 0.09 |
| BQTerrace | -5.63% | 5.08% | 11.63% | 0.13 | -0.08 | -0.16 |
| RitualDance | -6.76% | 1.53% | -0.81% | 0.31 | -0.05 | 0.02 |
| MarketPlace | -6.40% | 14.34% | 11.96% | 0.19 | -0.21 | -0.20 |
| BasketballDrive | -7.11% | 4.03% | 3.01% | 0.20 | -0.08 | -0.07 |
| Cactus | -4.24% | 2.73% | 2.12% | 0.13 | -0.04 | -0.05 |
| Average SDR-A | -11.53% | 6.23% | 2.08% | 0.31 | -0.16 | -0.03 |
| Average SDR-B | -6.03% | 5.54% | 5.58% | 0.19 | -0.09 | -0.09 |
| Average all | -8.78% | 5.89% | 3.83% | 0.25 | -0.12 | -0.06 |

## SDR: Constraint set 2 configuration relative to JEM anchor

### Class SDR-B

### Overall

|  | BD-rate (MsSSIM) | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BQTerrace | -15.72% | 5.22% | 14.18% | 30.81% | -0.12 | -0.13 | -0.19 |
| RitualDance | -13.82% | -2.65% | 11.12% | 7.81% | 0.12 | -0.20 | -0.16 |
| MarketPlace | -11.22% | -3.15% | 19.93% | 23.19% | 0.09 | -0.21 | -0.26 |
| BasketballDrive | -19.39% | -5.59% | 16.76% | 6.02% | 0.17 | -0.23 | -0.13 |
| Cactus | -7.11% | 5.40% | 13.72% | 8.79% | -0.17 | -0.14 | -0.15 |
| Average SDR-B | -13.45% | -0.15% | 15.14% | 15.32% | 0.02 | -0.18 | -0.18 |

Beside BD-rate PSNR gain, the BD-rate MsSSIM is also provided in the above table. It shows that the proposed solution achieves 13.45% BD-rate MsSSIM gain compared to JEM7.0.

The test results with the adaptive quantization disabled are as follows:

|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| --- | --- | --- | --- | --- | --- | --- |
| BQTerrace | -1.51% | 16.13% | 34.10% | 0.03 | -0.13 | -0.20 |
| RitualDance | -4.04% | 3.12% | 0.91% | 0.19 | -0.05 | -0.01 |
| MarketPlace | -3.10% | 10.21% | 14.01% | 0.09 | -0.11 | -0.16 |
| BasketballDrive | -5.82% | 14.96% | 5.18% | 0.18 | -0.20 | -0.11 |
| Cactus | 0.06% | 8.07% | 1.35% | 0.00 | -0.08 | -0.03 |
| Average SDR-B | -2.88% | 10.50% | 11.11% | 0.10 | -0.11 | -0.10 |

## HDR: Constraint set 1 configuration relative to HM anchor

### Class HDR-A

### Class HDR-B

### Overall

| BDBR | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DayStreet | -54.1% | -38.0% | -39.7% | -46.6% | -59.1% | -30.2% | -43.0% | -54.5% |
| PeopleInShoppingCenter | -44.7% | -30.3% | -32.2% | -52.9% | -44.0% | -32.5% | -48.5% | -42.8% |
| SunsetBeach | -59.3% | -31.9% | -28.9% | -74.0% | -73.8% | -41.8% | -69.8% | -70.3% |
| Market | -24.5% | -22.0% | -24.4% | -8.0% | -13.6% | -41.9% | -15.8% | -19.6% |
| ShowGirl2 | -21.3% | -25.7% | -25.9% | -42.5% | -10.1% | -46.7% | -50.1% | -20.5% |
| Hurdles | -32.5% | -36.4% | -36.7% | -23.5% | -31.9% | -41.5% | -23.3% | -31.2% |
| Starting | -27.6% | -26.1% | -27.9% | -20.3% | -24.8% | -41.7% | -25.2% | -28.0% |
| Cosmos1 | -30.8% | -33.1% | -27.9% | -56.8% | -42.7% | -61.4% | -57.2% | -46.6% |
| Average HDR-A | -52.7% | -33.4% | -33.6% | -57.9% | -59.0% | -34.8% | -53.8% | -55.9% |
| Average HDR-B | -27.3% | -28.6% | -28.6% | -30.2% | -24.6% | -46.7% | -34.3% | -29.2% |
| Average all | -36.8% | -30.4% | -30.4% | -40.6% | -37.5% | -42.2% | -41.6% | -39.2% |

| BDPSNR | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DayStreet | 0.71 | 0.88 | 1.76 | 0.88 | 1.24 | 1.34 | 0.86 | 1.26 |
| PeopleInShoppingCenter | 0.77 | 0.82 | 1.76 | 1.74 | 1.42 | 1.83 | 1.51 | 1.36 |
| SunsetBeach | 0.45 | 0.35 | 0.76 | 1.04 | 0.94 | 1.17 | 0.94 | 0.70 |
| Market | 0.29 | 0.44 | 1.11 | 0.08 | 0.48 | 2.15 | 0.26 | 0.64 |
| ShowGirl2 | 0.22 | 0.60 | 1.23 | 0.69 | 0.06 | 2.37 | 0.83 | 0.27 |
| Hurdles | 0.43 | 0.74 | 1.73 | 0.58 | 1.15 | 2.03 | 0.54 | 1.08 |
| Starting | 0.39 | 0.77 | 1.89 | 0.52 | 1.02 | 2.85 | 0.62 | 1.07 |
| Cosmos1 | 0.36 | 0.50 | 0.80 | 1.00 | 0.56 | 2.27 | 1.01 | 0.61 |
| Average HDR-A | 0.65 | 0.68 | 1.43 | 1.22 | 1.20 | 1.45 | 1.10 | 1.11 |
| Average HDR-B | 0.34 | 0.61 | 1.35 | 0.57 | 0.65 | 2.33 | 0.65 | 0.73 |
| Average all | 0.45 | 0.64 | 1.38 | 0.82 | 0.86 | 2.00 | 0.82 | 0.87 |

## HDR: Constraint set 1 configuration relative to JEM anchor

### Class HDR-A

### Class HDR-B

### Overall

| BDBR | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DayStreet | 6.8% | -2.7% | -7.5% | 11.9% | 17.0% | 6.4% | 14.7% | 20.8% |
| PeopleInShoppingCenter | 6.2% | -1.1% | -5.3% | 6.3% | 12.1% | -6.1% | 8.8% | 12.2% |
| SunsetBeach | -9.6% | -8.1% | -10.2% | 38.6% | 20.8% | -26.1% | 30.9% | 42.9% |
| Market | 18.7% | 3.5% | 0.8% | 35.5% | 31.0% | -24.6% | 17.3% | 10.6% |
| ShowGirl2 | 7.0% | 1.1% | 2.3% | 27.1% | 21.9% | -28.7% | 3.6% | 4.5% |
| Hurdles | 29.6% | -3.2% | -5.1% | 51.4% | 42.3% | -15.9% | 37.8% | 30.8% |
| Starting | 4.2% | 7.1% | 3.4% | 9.7% | -11.1% | -18.5% | 1.7% | -19.7% |
| Cosmos1 | -1.5% | -11.9% | -5.0% | 1.9% | 36.3% | -50.3% | 0.4% | 34.5% |
| Average HDR-A | 1.2% | -4.0% | -7.7% | 18.9% | 16.6% | -8.6% | 18.1% | 25.3% |
| Average HDR-B | 11.6% | -0.7% | -0.7% | 25.1% | 24.1% | -27.6% | 12.1% | 12.1% |
| Average all | 7.7% | -1.9% | -3.3% | 22.8% | 21.3% | -20.5% | 14.4% | 17.1% |

| BDPSNR | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DayStreet | -0.06 | 0.05 | 0.25 | -0.19 | -0.24 | -0.22 | -0.26 | -0.36 |
| PeopleInShoppingCenter | -0.08 | 0.03 | 0.25 | -0.15 | -0.35 | 0.29 | -0.22 | -0.37 |
| SunsetBeach | 0.05 | 0.08 | 0.24 | -0.25 | -0.13 | 0.65 | -0.22 | -0.21 |
| Market | -0.19 | -0.06 | -0.03 | -0.66 | -0.20 | 1.09 | -0.33 | 0.14 |
| ShowGirl2 | -0.06 | -0.02 | -0.09 | -0.29 | -0.24 | 1.23 | -0.01 | 0.01 |
| Hurdles | -0.25 | 0.05 | 0.18 | -0.95 | -0.60 | 0.62 | -0.73 | -0.39 |
| Starting | -0.04 | -0.17 | -0.19 | -0.28 | 0.42 | 1.00 | -0.06 | 0.69 |
| Cosmos1 | 0.02 | 0.16 | 0.12 | 0.02 | -0.35 | 1.67 | 0.05 | -0.32 |
| Average HDR-A | -0.03 | 0.05 | 0.24 | -0.20 | -0.24 | 0.24 | -0.23 | -0.31 |
| Average HDR-B | -0.10 | -0.01 | 0.00 | -0.43 | -0.20 | 1.12 | -0.21 | 0.03 |
| Average all | -0.08 | 0.01 | 0.09 | -0.34 | -0.21 | 0.79 | -0.22 | -0.10 |

## 360° Video: Constraint set 1 configuration relative to HM anchor

### Class 360

### Overall

| E2E WS-PSNR | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| --- | --- | --- | --- | --- | --- | --- |
| Balboa | -45.11% | -54.85% | -57.98% | 1.97 | 1.65 | 1.70 |
| Chairlift | -46.02% | -64.18% | -57.29% | 1.81 | 2.36 | 1.35 |
| KiteFlite | -23.59% | -50.38% | -55.09% | 1.05 | 1.75 | 1.73 |
| Harbor | -29.14% | -55.36% | -57.09% | 1.13 | 2.10 | 1.90 |
| Trolley | -21.47% | -41.39% | -45.95% | 1.03 | 1.65 | 1.68 |
| Average | -33.07% | -53.23% | -54.68% | 1.40 | 1.90 | 1.67 |

| E2E SPSNR-NN | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| --- | --- | --- | --- | --- | --- | --- |
| Balboa | -45.11% | -54.85% | -58.00% | 1.97 | 1.64 | 1.70 |
| Chairlift | -45.93% | -64.15% | -57.24% | 1.81 | 2.36 | 1.34 |
| KiteFlite | -23.60% | -50.36% | -55.09% | 1.05 | 1.76 | 1.74 |
| Harbor | -28.87% | -55.40% | -57.09% | 1.12 | 2.09 | 1.90 |
| Trolley | -21.49% | -41.38% | -45.80% | 1.03 | 1.65 | 1.68 |
| Average | -33.00% | -53.23% | -54.64% | 1.40 | 1.90 | 1.67 |

## 360° Video: Constraint set 1 configuration relative to JEM anchor

### Class 360

### Overall

| E2E WS-PSNR | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| --- | --- | --- | --- | --- | --- | --- |
| Balboa | -15.11% | -17.45% | -15.34% | 0.52 | 0.32 | 0.25 |
| Chairlift | -22.24% | -28.94% | -21.86% | 0.74 | 0.60 | 0.32 |
| KiteFlite | -7.36% | -2.55% | -3.29% | 0.30 | 0.04 | 0.06 |
| Harbor | -11.07% | -15.08% | -17.49% | 0.39 | 0.34 | 0.34 |
| Trolley | -6.56% | -5.67% | -9.31% | 0.30 | 0.13 | 0.21 |
| Average | -12.47% | -13.94% | -13.46% | 0.45 | 0.29 | 0.24 |

| E2E SPSNR-NN | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| --- | --- | --- | --- | --- | --- | --- |
| Balboa | -15.08% | -17.37% | -15.31% | 0.52 | 0.32 | 0.25 |
| Chairlift | -22.13% | -28.83% | -21.88% | 0.74 | 0.60 | 0.32 |
| KiteFlite | -7.33% | -2.45% | -3.24% | 0.30 | 0.04 | 0.06 |
| Harbor | -10.81% | -15.25% | -17.64% | 0.38 | 0.35 | 0.35 |
| Trolley | -6.53% | -5.56% | -9.13% | 0.30 | 0.13 | 0.21 |
| Average | -12.38% | -13.89% | -13.44% | 0.45 | 0.29 | 0.24 |

# Decoder complexity analysis

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

Run time measurement was performed with YUV output enabled and reference input disabled for HM 16.16, additional JEM anchors and proposed codec solution.

The decoding time comparison to HM anchors: 283% of the decoding time of HM16.16 for constraint set 1 and 244% of the decoding time of HM16.16 for constraint set 2. When compared to JEM anchors, the proposed solution requires 45% of the decoding time of JEM7.0 for constraint set 1 and 48% of the decoding time of JEM7.0 for constraint set 2.

## Description of computing platform used to determine decoding times reported in section 5.1

Run-time measurement was performed during decoding on homogeneous LINUX cluster, 64 bits, GCC-4.8.3. Only one core was utilized for measuring the decoding time.

## Memory usage of decoder

In our simulation environment memory usage of HM, current proposal and JEM is 0.5GB, 1 GB and 1.5 GB correspondently.

## Complexity characteristics of decoder entropy decoding operation

Entropy coder doesn’t use any look-up tables. Each state has 2 hypothesis represented by 13-bits unsigned integer, up-dated independently using 2 adds and 1 shift operation.

## Complexity characteristics of decoder inverse quantization

The basic process for quantization is the same as in HEVC. This proposal uses the same QP range, quantization step, and corresponding scaling process for transformed coefficients.

## Complexity characteristics of decoder inverse transform operation

Core transform coefficients precision is 8 bits.  Maximum transform size is 128. Only one core transform kernel (DCT-II) is used for 1258 and 64 sizes. Additionally, 2 more full-size core transforms are allowed for block sizes 4, 8, 16 and 32. Core transform is followed by secondary transform of 4x4 or 8x8 size. More details can be found in section 13.2.8.

## Complexity characteristics of decoder-side motion refinement

Signaled per coding unit MV information is refined with precision ±1/2 sample in DMVR mode by choosing minimal SAD of bi-lateral matching among several candidates. The number of candidates is limited according to the block size in order not to exceed the worst case memory bandwidth in HEVC. DMVR uses bi-linear interpolation.

## Complexity characteristics of decoder motion compensation

DMVR mode uses bi-linear interpolation. Affine mode uses combination of 5 taps (with 3 non-zero coefficient) and bi-linear filters for explicit calculation of affine transformation. BIO requires additional calculation for gradients, using 2D separable 8-taps filter, which can be performed in parallel with regular MC prediction generation utilizing the same architecture (just filter coefficient are different). Small blocks (such as 4×8) use bi-linear interpolation in order to ensure memory bandwidth doesn’t exceed HEVC the worst case. The rest of the modes similar to HEVC use 2D separable 8/4-taps (Luma / Chroma) interpolation with 6 bits filter coefficient. All interpolation filter coefficients are predetermined (not content dependent).

## Complexity characteristics of decoder intra-frame prediction operation

Non-angular Intra prediction computational complexity doesn’t exceed complexity of Planar mode in HEVC. Angular prediction is performed using 4-taps interpolation filter, but no Intra reference sample processing is used.

## Complexity characteristics of decoder in-loop filtering operation

There are 4 in-loop filters: de-blocking (uses longer filter than HEVC), SAO (sample processing same as HEVC), noise-suppressor filter (new technology) and ALF. More details can be found in section 13.2.9.

## Complexity characteristics of additional decoder tools

There are no additional decoder tools.

## Complexity characteristics of 360° video specific decoding tools

There are no low-level 360° video specific tools.

## Complexity characteristics of HDR specific decoding tools

There are no low-level HDR specific tools.

## Degree of capability for decoder parallel processing

Decoder parallel processing is same level with HM and JEM.

# Encoder complexity analysis

## Encoding time and measurement methodology

For encoding time measurement, a clustered system with multiple CPUs and RAMs was used. Each CPU has at most similar clock-speed with LINUX cluster, 64 bits, GCC-4.8.3. Testing was performed using parallel processing and calculated encoding time by using the follow methods: encoding log files should be parsed for each IRAP segment (or equivalent parallelism segment), and the total encoding time should be calculated by adding the encoding time of all parallelism segments and then subtracting the encoding times of the first pictures (or equivalent overlapping elements) for all segments except the first.

The encoding time comparison to HM anchors: 1043% of the encoding time of HM16.16 for constraint set 1 and 1027% of the encoding time of HM16.16 for constraint set 2. When compared to JEM anchors, the proposed solution requires 139% of the encoding time of JEM7.0 for constraint set 1 and 125% of the encoding time of JEM7.0 for constraint set 2.

## Description of computing platform used to determine encoding times reported in section 6.1

Run-time measurement was performed during encoding on homogeneous LINUX cluster, 64 bits, 4G Ram, GCC-4.8.3. Testing was performed using parallel processing and calculated encoding time as noticed in 6.1 section.

## Memory usage of encoder

In our simulation environment memory usage of HM, current proposal and JEM is 0.5GB, 1 GB and 1.5 GB correspondently.

## Complexity characteristics of encoder intra-frame prediction type selection

Non-angular Intra prediction computational complexity doesn’t exceed complexity of Planar mode in HEVC. Angular prediction is performed using 4-taps interpolation filter, but no Intra reference sample processing is used. Detail is described in Annex 2 Tool level Q&A.

## Complexity characteristics of encoder motion estimation and motion segmentation selection

The ME algorithm used in the proposal is implemented in a similar manner to HM software. Uni-directional inter prediction uses the MVP as the starting point for the ME search and bi-directional inter prediction uses the MV from the uni-directional prediction search result as the starting MV.

As the first step of the search process, a box search is performed around the starting point with a search range of 5 pixels in each of the four directions. The cost is computed using MVD and SAD for at each position and the best position is computed. This is followed by a diamond search with varying step size. First iteration of diamond search is performed with a step size of 4 and the size is doubled for every further iteration. Three iterations are performed i.e. till the step size is 16. The cost at each position of each iteration is compared to locate the best position. If the best position found happens to be in diamond search and for a step size greater than 5, it implies that the MVD is relatively large. In this case, a raster search is performed around this best position found so far. The raster scan starts with a box search with step size based on the current CU size and the box search is performed. Hereon the step size is progressively reduced until it reaches 1 and the box search is performed at each step and the best position is stored. The MV obtained so far is further refined with one box searches followed by a diamond search performed in the first step.

The MV obtained after the above process is the integer pixel MV and is used as the starting point for sub-pixel MV search. First, a diamond search is performed around this MV to obtain the best half-pixel MV. This half pixel MV is further refined to obtain the final quarter-pixel MV.

When applying AMVR, the search area and the step size of the searches performed is varied adaptively depending on the MVR index.

### General

### Complexity characteristics of usage of decoder-side motion refinement

Signaled per coding unit MV information is refined with precision ±1/2 sample in DMVR mode by choosing minimal SAD of bi-lateral matching among several candidates. The number of candidates is limited according to the block size in order not to exceed the worst case memory bandwidth in HEVC. DMVR uses bi-linear interpolation. . Detail is described in Annex 2 Tool level Q&A.

## Complexity characteristics of encoder transforms and transform type selection

Core transform coefficients precision is 8 bits.  Maximum transform size is 128. Only one core transform kernel (DCT-II) is used for 1258 and 64 sizes. Additionally, 2 more full-size core transforms are allowed for block sizes 4, 8, 16 and 32. Core transform is followed by secondary transform of 4x4 or 8x8 size. More details can be found in section 13.2.8.

## Complexity characteristics of encoder quantization and quantization type selection

The basic process for quantization is the same as in HEVC. This proposal uses the same QP range, quantization step, and corresponding scaling process for transformed coefficients.

## Complexity characteristics of encoder in-loop filtering type selection and parameter estimation

There are 4 in-loop filters: de-blocking (uses longer filter than HEVC), SAO (sample processing same as HEVC), noise-suppressor filter (new technology) and ALF. More details can be found in section 13.2.9.

## Complexity characteristics of encoder entropy coding type selection

Entropy coder doesn’t use any look-up tables. Each state has 2 hypothesis represented by 13-bits unsigned integer, up-dated independently using 2 adds and 1 shift operation.

## Complexity characteristics of additional encoder tools

There are no additional encoder tools.

## Complexity characteristics of 360° video specific encoding tools

There are no low-level 360° video specific tools.

## Complexity characteristics of HDR specific encoding tools

There are no low-level HDR specific tools.

## Degree of capability for encoder parallel processing

Encoder parallel processing is same level with HM and JEM.

# Algorithmic characteristics

## Random access characteristics

The constraint set 1 anchor GOP conditions condition were also satisfied by this proposal. Therefore, the number of frames required to decode any given frame is exactly same as HM and JEM anchors.

## Delay characteristics

The constraint set 2 anchor GOP conditions condition were also satisfied by this proposal. Therefore, the number of frames required to decode any given frame is exactly same as HM and JEM anchors.

## Additional characteristics discussion topic(s)

# Software implementation description

The proposed coding scheme is implemented by using standard C programming language.

# Highlighted aspects discussion

# Closing remarks

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Hisilicon Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

GoPro, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G.J. Sullivan and R.L. Baker Efficient quadtree coding of images and video, IEEE Trans-actions on Image Processing pp327-331, May 1994.

[2] M.C. Chen and A. Willson, A Spatial and Temporal Motion Vector Coding Algorithm for Low-Bit-Rate Video Coding, International Conference on Image Processing, Oct. 1997.

[3] S.D. Kim, and J.B. Ra, An efficient motion vector coding scheme based on minimum bitrate prediction, IEEE Transaction on Image Processing, vol.8, no.8, Aug. 1999.

[4] K.R. Rao and P. Yip, Discrete Cosine Transform Algorithms, Advantage, Applications, New York Academic, 1990.

[5] Video Coding for Low bit rate Communication, ITU-T Recommendation H.263, version 1, Nov. 1995; version 2, Jan. 1998; version 3, Nov. 2000.

[6] T. Wiegand , X. Zhang, and B. Girod Long-Term Memory Motion-Compensated Prediction, ITU-T SG16 VCEG, Q15-C11, Dec. 1997.

[7] G. Bjontegaard, Simulation results using 4x4 blocks for motion vectors and transform, ITU-T SG16 VCEG, Q15-D29, Apr. 1998.

[8] Coding of audio-visual objects Part 2: Visual, ISO/IEC 14496-2 MPEG-4 visual, Apr. 1999

[9] G. Bjontegaard, H.26L Test Model Long Term Number 1 (TML-1) draft 2, Aug. 1999

[10] Advanced Video Coding (AVC), ITU-T Rec. H.264 and ISO/IEC 14496-10, May 2003

[11] CCITT Recommendation | ISO/IEC International Standard Digital Compression and Coding of Continuous-tone Still images Annex D

[12] A. Alshin, E. Alshina, “Bi-directional optical flow,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C204, Guangzhou, China, 10-15 October, 2010.

[13] A.Alshin, E.Alshina Bi-directional optical flow for future video codec. DCC 2016

[14] Puri, Atul, and H. M. Hang. "Adaptive schemes for motion-compensated coding." Visual Communications and Image Processing'88: Third in a Series. Vol. 1001. International Society for Optics and Photonics, 1988.

[15] C.C. Holt “Forecasting trends and seasonal by exponentially weighted moving averages.” O.N.R. Memorandum, Carnegie Inst. Of Technology. 1957.

[16] D. Marpe, H. Schwarz, and T. Wiegand, “Context-based Adaptive Binary Arithmetic Coding in the H.264 /AVC Video Compression Standard,” IEEE Transactions on Circuits and Systems for Video Technology, Vol. 13, No. 7, pp. 620-636, July 2003.

# Annex 1: Questionnaire

Proponents of CfP responses are required to answer the codec level questions, and are also encouraged to fill out the optional tool level questions in addition to the complexity analysis of their proposals. If some question is not applicable to your proposal, you may express that as well.

## Codec level questions

- Maximum number of reference frames used by encoder

- Maximum coding unit (e.g. CTU in HEVC)

- Minimum and maximum transform block size

- Minimum and maximum intra prediction block

- Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length

- List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category)

- List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples)

- List all tools that use non-rectangular (e.g. triangle) partitions

- Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa)

- Indicate whether luma and chroma use separate partitioning structures

- Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes)

- If the coded picture resolution is different from the input picture resolution, list the coded picture resolution for those sequences (e.g. 360° video, input is 8K ERP, coded video is in 4K CMP converted from the 8K input)

## Tool level questions

- Partitioning structure

  - Whether the prediction unit partitioning is different from transform unit partitioning

  - Maximum tree depth

- Entropy coding

  - Total number of context models

  - Memory storage size for each context model

  - Range table size if table look up is used for range update

  - Initialization table size

  - Other memory associated with context model adaptation

- Control data derivation process

  - Temporal motion data storage granularity

  - Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-predictive reference picture are used, count this picture as 2)

- Decoder-side motion refinement

  - Line buffer size in terms of motion vector block lines (assuming horizontal motion vector block size is 4 luma samples)

  - Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement

  - Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size

- Intra prediction

  - Line buffer size in terms of luma and chroma sample lines

  - List all the filters used for intra prediction

  - Indicate whether there are cross-component dependencies in intra prediction reconstruction

- Inter prediction

  - Motion vector accuracy

  - Storage size for a motion vector component

  - List all the interpolation filters used for luma/chroma motion compensation

- Quantization

  - Storage size associated with quantization

- Transforms

- List all the combinations of transform block size and transform type for primary transforms

- List all the combinations of transform block size and transform type for secondary transforms

- List all the non-separable transforms used

- Indicate whether transforms can be fully/partially implemented using direct matrix multiply

- List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output)

- In-loop filtering

- List line buffer size in terms of luma and chroma lines (if any)

- List all the filters used for in-loop filtering

- Minimum processing unit size

- “tool-off” and “tool-on” tests

- List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools (see Table 1 below for test procedure)

Table 1. Test procedure of “tool-off” and “tool-on” tests for a CfP response

| step | “tool-off” test | “tool-on” test |
| --- | --- | --- |
| 1 | Identify what you consider major tools in a CfP response that contribute to compression efficiency gain, and create a test candidates list |  |
| 2 | Turn on all the tools in the test candidates list, run the CfP response software and compute PSNRs, bit rates, and run times | Turn off all the tools in the test candidates list, run the CfP response software, compute PSNRs, bit rates, and run times, and compute the BD-rate and run-time numbers relative to the HM anchor |
| 3 | Turn off a tool in the test candidates list, and keep all the other tools in the test candidates list on, run the CfP response software and compute the BD-rate and run-time numbers relative to the CfP response software with all tools on | Turn on a tool in the test candidates list, and keep all the other tools in the test candidates list off, run the CfP response software and compute the BD-rate and run-time numbers relative to the CfP response software with all tools of the test candidates list off (i.e. step 2) |
| 4 | Repeat step 3 until all the tools in the test candidates list are tested | Repeat step 3 until all the tools in the test candidates list are tested |
| 5 | Compose the test data |  |

# Annex 2: Example of filling questionnaire HM vs. JEM

This annex is meant as an example how the questions could be answered. If some of the aspects are not applicable to an algorithm, the corresponding lines may not need to be filled. You may also add lines at the end of the table to compare additional aspects of a proposal versus HM.

## Codec level Q&A

| Codec Level Q&A |  | HM | JVET-J0025 | JEM |
| --- | --- | --- | --- | --- |
| 1 | Maximum number of reference frames used by encoder | 4 | 4 | 4 |
| 2 | Maximum coding unit (e.g. CTU in HEVC) | luma 64×64 chroma 32×32 | luma 128×128 chroma 64×64 | luma 128×128 chroma 64×64 |
| 3 | Minimum and maximum transform block size | luma 4×4/32×32 chroma 4×4/16×16 | luma 4×4/128×128 chroma 2×2/64×64 | luma 4×4/128×128 chroma 2×2/64×64 |
| 4 | Minimum and maximum intra prediction block | luma 4×4/32×32 chroma 4×4/32×32 | luma 4×4/128×128 chroma 2×2/64×64 | luma 4×4/128×128 chroma 2×2/64×64 |
| 5 | Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length | luma 4×8/8×4 (uni-pred) and 8×8 (bi-pred), 8-tap Chroma 2×4/4×2 (uni-pred) and 4×4 bi-pred, 4-tap | luma 4×4 (uni/bi-pred), 8-tap Chroma 2×2 (uni/bi-pred), 4-tap | luma 4×4 (uni/bi-pred), 8-tap Chroma 2×2 (uni/bi-pred), 4-tap |
| 6 | List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category) | n/a | n/a | template matching in FRUC, LIC |
| 7 | List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples, position unforeseeable) | n/a | n/a | 64-bit arithmetic for LIC, BIO and LMC |
| 8 | List all the tools that use non-rectangular (e.g. triangle) partitions | n/a | n/a | n/a |
| 9 | Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa) | n/a | n/a | n/a |
| 10 | Indicate whether luma and chroma use separate partitioning structures | no | Yes, in I-slices | Yes, in I-slices |
| 11 | Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes) | Entropy parsing depends on intra prediction mode reconstruction, which requires an intra MPM list derivation | n/a | Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation |

## Tool level Q&A

### Category 1. Partitioning structure

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1. Partitioning structure |  | HM | JVET-J0025 | JEM |
| 1.a | Whether the prediction unit partitioning is different from transform unit partitioning | Yes, PU and TU partitioning for a CU can differ | PU < TU (affine) | PU < TU (affine, subCU, OBMC) |
| 1.b | Maximum tree depth | CU maximum depth 4 TU maximum depth 3 PU maximum depth 1 | QT 256128 Max/Min size signaled in SPS, default BT/TT 1284 | QT maximum depth 5 BT maximum depth 3 |

### Category 2. Entropy coding

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 2. Entropy coding |  | HM | JVET-J0025 | JEM |
| 2.a | Total number of context models | 186 | 322 | 329 |
| 2.b | Memory storage size for each context model | 8-bit | 32-bit | 30-bit |
| 2.c | Range table size if table look up is used for range update | LPSTable[64×4×8bits]; RenormTable[32× 3bits] | n/a | LPSTable[512×64×9bits]; RenormTable[128×3bits] |
| 2.d | Initialization table size | 3×186×8 bits | 2× 322 ×16 + 322×13 ×5 bits | 3×329×8 + 329×15×5 bits |
| 2.e | Other memory associated with context model adaptation | LPS transition table: 64*6 bits MPS transition table: 64*6 bits | n/a | n/a |

### Category 3. Control data derivation process

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 3. Control data derivation process |  | HM | JVET-J0025 | JEM |
| 3.a | Temporal motion data storage granularity | 16×16 | 4×4 | 4×4 |
| 3.b | Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-directional reference picture are used, count this picture as 2) | 1 | 1 | 4 |

### Category 4. Decoder-side motion refinement

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 4. Decoder-side motion refinement |  | HM | JVET-J0025 | JEM |
| 4.a | Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement. | n/a | - BIO 2×2D separable 8 taps filter (for gradients), mults per sample in refinement calculation 5+5*4 +1/8+2, minimum processing unit same as regular bi-pred, no SAD calculation | - BIO 2×2D separable 6 taps filter (for gradients), mults per sample in refinement calculation 5+5*4 +1/8+2, minimum processing unit same as regular bi-pred, no SAD calculation - FRUC minimum processing unit 44, sequential PU followed by sub-PU level refinement, number of SAD calculations and additional MC interpolations per unit for MV refinement is not constrained (refinement is terminated when cost no longer decreases); bi-linear filter at MV refinement stage |
| 4.b | Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size. | n/a | - DMVR needs to access (H+7+L)×(W+7+L) MC instead of (H+7)×(W+7) in HEVC MC; depending on block size L is constrained in order not to exceed HEVC worst case memory bandwidth | - DMVR needs to access (H+7+2)×(W+7+2) MC instead of (H+7)×(W+7) in HEVC MC - FRUC not constrained |

### Category 5. Intra prediction

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 5. Intra prediction |  | HM | JVET-J0025 | JEM |
| 5.a | Line buffer size in terms of luma and chroma sample lines | luma 1, chroma 1 | luma 2, chroma 1 | luma 4, chroma 2 |
| 5.b | List all the filters used for intra prediction | Reference sample smoothing: {1,2,1}, Strong intra smoothing filter: 2-tap bilinear filter (only for 32×32 luma TU) Interpolation: 2×32×5 bits (bi-linear 32 phases) | Interpolation: 2×4×32×8 bits (4 taps, 2 sets, 32 phase) | Reference sample smoothing: {1,2,1}, {3,7,14,16,14,7,3}, {4,9,12,14,12,9,4} Interpolation: 2×4×32×8 bits (4 taps, 2 sets, 32 phase) |
| 5.c | Indicate whether there are cross-component dependencies in intra prediction reconstruction | n/a | LM-CHROMA (Cb, CR depends on Y only) | LM-CHROMA (Cb depends on Y, and Cr depends on Cb&Y) |

### Category 6. Inter prediction

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 6. Inter prediction |  | HM | JVET-J0025 | JEM |
| 6.a | Motion vector accuracy | luma ¼ pel chroma 1/8 pel | luma 1/16 pel chroma 1/32 pel | luma 1/16 pel chroma 1/32 pel |
| 6.b | Storage size for a motion vector component | 16-bit | 16-bit | 16-bit |
| 6.c | List all the interpolation filters used for luma/chroma motion compensation | luma 8-tap, 8×4×6 bits chroma 4-tap, 4×8×6 bits | luma 8-tap, 8×4×6 bits chroma 4-tap, 4×8×6 bits bi-linear | luma 8-tap, 8×16×6 bits chroma 4-tap, 4×32×6 bits bi-linear |

### Category 7. Quantization

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 7. Quantization |  | HM | JVET-J0025 | JEM |
| 7.a | Storage size associated with quantization | n/a | n/a | n/a |

### Category 8. Transforms

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 8. Transforms |  | HM | JVET-J0025 | JEM |
| 8.a | List all the combinations of transform block size and transform type for primary transforms | - 8-bit 4×4, 8×8, 16×16 and 32×32 DCT-II, - 8-bit 4×4 DST VII | - 8-bit 4×4, 8×8, 16×16, 32×32, 64×64 and 128×128 DCT-II, Multiple transform mode (10-bit DST VII, DCT-V, DCT-VIII, DST-I), only applied to CTUs up to 64×64, transform size combinations: (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TUs for luma CTUs DST-VII, DCT-V, DCT-VIII, DST-I (10-bits) - Sizes (2,4,8,16,32,64) | HEVC transform mode (10-bit DCT-II), transform size combinations: - (4, 8, 16, 32, 64, 128) × (4, 8, 16, 32, 64, 128) TUs for luma CTUs - (2, 4, 8, 16, 32, 64) × (2, 4, 8, 16, 32, 64) TUs for inter-coded chroma CTUs - (2, 4, 8, 16, 32, 64,) × (2, 4, 8, 16, 32, 64) TUs for intra-coded chroma CTUs EMT transform mode (10-bit DST VII, DCT-V, DCT-VIII, DST-I), only applied to CTUs up to 64×64, transform size combinations: - (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TUs for luma CTUs |
| 8.b | List all the combinations of transform block size and transform type for secondary transforms | n/a | Transform sizes (4,8) ; Transform types; 14 x 4(=56) for 4x4 and 8x8 ; Hypercube-Givens; Transform (HyGT); Partial residual transform | 35×3(=105) 4×4 and 8×8 non-separable Hypercube-Givens Transform (HyGT) |
| 8.c | List all the non-separable transforms used | n/a | 4×4/8×8 HyGT | 4×4/8×8 HyGT |
| 8.d | Indicate whether transforms can be fully/partially implemented using direct matrix multiply | partial factorization equivalent to matrix multiplication | factorized form not equivalent to matrix multiplication | factorized form not equivalent to matrix multiplication |
| 8.e | List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output) | n/a | 4×4 HyGT 2-interations, 8×8 HyGT 4-interations | 4×4 HyGT 2-interations, 8×8 HyGT 4-interations |

### Category 9. In-loop filtering

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 9. In-loop filtering |  | HM | JVET-J0025 | JEM |
| 9.a | List line buffer size in terms of luma and chroma lines (if any) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) NSF non ALF (luma 8, chroma 4) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) ALF (luma 8, chroma 4) |
| 9.b | List all the filters used for in-loop filtering | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. | Deblocking Luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, (2 3 1 1 1)/8 and {3, 5, 2, 1, 1, 1, 1, 1, 1}/16 {1, 2, 5, 2, 1, 1, 1, 1, 1, 1}/16 {1, 3, 3, 3, 1, 1, 1, 1, 1, 1}/16 {1, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1}/16 {1, 2, 2, 2, 2, 2, 2, 1, 1, 1}/16 {1, 1, 2, 2, 2, 2, 2, 2, 1, 1}/16 {1, 2, 2, 2, 2, 2, 2, 2, 1}/16 {3, 5, 2, 2, 1, 1, 1, 1}/16 {2, 2, 4, 2, 2, 2, 1, 1}/16 {2, 2, 2, 2, 2, 2, 2, 2}/16; For chroma: (1 4 4 –1)/8; ALF 9x9/7x7/5x5 diamond filter support for luma; and 5x5 diamond filter support for chroma; NSF (calculated on decoder side) | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. ALF: 9×9/7×7/5×5 diamond filter support for luma; and 5×5 diamond filter support for chroma |
| 9.c | Minimum processing unit size | Deblocking (luma: 8×8 grid with 4×4 filter decision, chroma 8×8 grid with 4×4 filter decision) | Deblocking (luma 8×8 grid with 4×4 filter decision, chroma 8×8 grid with 2×2 filter decision) ALF: for luma, filter decision is based on 2×2 block Bilateral filter: sample level filter switch (luma only) Noise suppressor: 8×8 grid | Deblocking (luma 4×4 grid with 4×4 filter decision, chroma 8×8 grid with 2×2 filter decision) ALF: for luma, filter decision is based on 2×2 block Bilateral filter: sample level filter switch (luma only) |

### Category 10. “tool-off” and “tool-on” tests

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 10. “tool-off” and “tool-on” tests |  | HM | JVET-J0025 | JEM |
| 10.a | List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools | None | TBD | QTBT, ALF, FRUC, BIO, DMVR, EMT, … See Table 2 below for an example of “tool-off” and “tool-on” tests |

Table 2. Summary of “tool-off” and “tool-on” tests

TBA

[end of document]