| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0027 |
| --- | --- |

| Title: | Description of SDR and HDR video coding technology proposal by NHK and SHARP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Kazuhisa Iguchi Atsuro Ichigaya (NHK) Kiran Misra Jie Zhao Andrew Segall Weijia Zhu Byeongdoo Choi Frank Bossen Michael Horowitz Phil Cowan Yukinobu Yasugi Tomonori Hashimoto Tianyang Zhou Tomohiro Ikai Takeshi Chujoh Tomoko Aono (Sharp) | Tel: Email: | +81-3-5494-3353 iwamura.s-gc@nhk.or.jp +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | NHK and SHARP |  |  |

_____________________________

- Abstract

This document describes the details of the response from NHK and SHARP to the Call for Proposals on Video Compression with Capability beyond HEVC issued jointly by ITU-T SG16 Q6 and ISO/IEC JTC 1/SC 29/WG 11.

There is another submission of JVET-J0026 which includes Sharp’s technologies. To help readability and avoid duplications, the common parts of JVET-J0026 and JVET-J0027 are described in JVET-J0026. Thus, please refer JVET-J0026 for those parts.

The response focuses on improved coding efficiency for the SDR and HDR categories with relatively low complexity. The tools include a large subset of the algorithms available in the JEM software, with additional contributions in the areas of intra prediction, inter prediction, in-loop filter, entropy coding. The combination of these tools achieves a measurable performance relative to the HM anchor, with gains of 37.5 % and 33.0 % for 4K-SDR and HD-SDR sequences, respectively, using the random access configuration; gains of 26.6 % for HD-SDR sequences using a low delay configuration, and gains of 30.1 % and 31.4 % for HLG-HDR and PQ-HDR sequences, respectively, using a random access configuration. It is asserted that the proposed approach combines a strong coding performance with a flexible software design, and it is proposed to use the response as a starting point for the next generation video coding standard.

- Contents

Abstract	i

Contents	i

1	Introduction	1

2	Decoder algorithm description	1

2.1	General algorithm description	1

2.1.1	Bitstream structure	1

2.1.2	Entropy decoding	1

2.1.3	Conversion process from entropy decoded data to symbols	1

2.1.4	Decoder-side motion refinement	4

2.1.5	Inverse quantization	4

2.1.6	Inverse transforms	4

2.1.7	Motion compensation	4

2.1.8	Intra prediction	6

2.1.9	In-loop filtering	6

2.1.10	Additional tools	9

2.1.11	Additional algorithmic description topic(s)	9

2.2	360° Video	9

2.2.1	Projection format (including padding)	9

2.2.2	Quantization handling	9

2.2.3	360° video specific decoding tools	9

2.3	HDR	9

2.3.1	Post-processing	9

2.3.2	Quantization handling	9

2.3.3	HDR specific decoding tools	10

3	Encoder algorithm description	10

3.1	General algorithm description	10

3.1.1	Rate-distortion optimization	10

3.1.2	Partitioning structure selection	10

3.1.3	Intra prediction mode estimation and type selection	10

3.1.4	Motion estimation and motion segment selection	10

3.1.5	Mode decisions	10

3.1.6	In-loop filtering type selection and parameter estimation	10

3.1.7	Transforms and transform type selection	10

3.1.8	Quantization and quantization type selection	10

3.1.9	Entropy coding	10

3.1.10	Additional tools	10

3.1.11	Additional encoder optimization	11

3.1.12	Additional algorithmic description topic(s)	11

3.2	360°	11

3.2.1	Usage of projection format adaptation	11

3.2.2	360° Video specific encoding tools	11

3.2.3	Additional encoder optimization	11

3.3	HDR	11

3.3.1	Pre-processing	11

3.3.2	HDR specific encoding tools	11

3.3.3	Additional encoder optimization	11

4	Compression performance	11

4.1	SDR: Constraint set 1 configuration relative to HM anchor	11

4.1.1	Class SDR-A (RD-curves)	11

4.1.2	Class SDR-B (RD-curves)	11

4.1.3	Overall (BD-rate summary tables)	12

4.2	SDR: Constraint set 2 configuration relative to HM anchor	12

4.2.1	Class SDR-B	12

4.2.2	Overall	12

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	13

4.3.1	Class SDR-A	13

4.3.2	Class SDR-B	13

4.3.3	Overall	13

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	13

4.4.1	Class SDR-B	13

4.4.2	Overall	13

4.5	HDR: Constraint set 1 configuration relative to HM anchor	14

4.5.1	Class HDR-A	14

4.5.2	Class HDR-B	14

4.5.3	Overall	14

4.6	HDR: Constraint set 1 configuration relative to JEM anchor	15

4.6.1	Class HDR-A	15

4.6.2	Class HDR-B	15

4.6.3	Overall	15

4.7	360° Video: Constraint set 1 configuration relative to HM anchor	18

4.7.1	Class 360	18

4.7.2	Overall	18

4.8	360° Video: Constraint set 1 configuration relative to JEM anchor	18

4.8.1	Class 360	18

4.8.2	Overall	18

5	Decoder complexity analysis	18

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	18

5.2	Description of computing platform used to determine decoding times reported in section 5.1	18

5.3	Memory usage of decoder	18

5.4	Complexity characteristics of decoder entropy decoding operation	18

5.5	Complexity characteristics of decoder inverse quantization	18

5.6	Complexity characteristics of decoder inverse transform operation	19

5.7	Complexity characteristics of decoder-side motion refinement	19

5.8	Complexity characteristics of decoder motion compensation	19

5.9	Complexity characteristics of decoder intra-frame prediction operation	19

5.10	Complexity characteristics of decoder in-loop filtering operation	19

5.11	Complexity characteristics of additional decoder tools	19

5.12	Complexity characteristics of 360° video specific decoding tools	19

5.13	Complexity characteristics of HDR specific decoding tools	19

5.14	Degree of capability for decoder parallel processing	19

6	Encoder complexity analysis	19

6.1	Encoding time and measurement methodology	19

6.2	Description of computing platform used to determine encoding times reported in section 6.1	19

6.3	Memory usage of encoder	20

6.4	Complexity characteristics of encoder intra-frame prediction type selection	20

6.5	Complexity characteristics of encoder motion estimation and motion segmentation selection	20

6.5.1	General	20

6.5.2	Complexity characteristics of usage of decoder-side motion refinement	20

6.6	Complexity characteristics of encoder transforms and transform type selection	20

6.7	Complexity characteristics of encoder quantization and quantization type selection	20

6.8	Complexity characteristics of encoder in-loop filtering type selection and parameter estimation	20

6.9	Complexity characteristics of encoder entropy coding type selection	20

6.10	Complexity characteristics of additional encoder tools	20

6.11	Complexity characteristics of 360° video specific encoding tools	20

6.12	Complexity characteristics of HDR specific encoding tools	20

6.13	Degree of capability for encoder parallel processing	21

7	Algorithmic characteristics	21

7.1	Random access characteristics	21

7.2	Delay characteristics	21

7.3	Additional characteristics discussion topic(s)	21

8	Software implementation description	21

9	Highlighted aspects discussion	21

10	Closing remarks	21

11	Patent rights declaration(s)	21

12	Annex 1: Questionnaire	22

12.1	Codec level questions	22

12.2	Tool level questions	23

13	Annex 2: Example of filling questionnaire HM vs. JEM	24

13.1	Codec level Q&A	24

13.2	Tool level Q&A	25

13.2.1	Category 1. Partitioning structure	25

13.2.2	Category 2. Entropy coding	26

13.2.3	Category 3. Control data derivation process	26

13.2.4	Category 4. Decoder-side motion refinement	26

13.2.5	Category 5. Intra prediction	27

13.2.6	Category 6. Inter prediction	27

13.2.7	Category 7. Quantization	28

13.2.8	Category 8. Transforms	28

13.2.9	Category 9. In-loop filtering	29

13.2.10	Category 10. “tool-off” and “tool-on” tests	29

Introduction

This response is an extension of JVET-J0026 in the areas of intra prediction, inter prediction, in-loop filter, and entropy coding as listed below:

| Tool names | Short description |
| --- | --- |
| chroma DM binarization bug fix | The bug-fix reported in JVET-H0077 on binarization for chroma intra prediction modes. |
| Bi-pred optimized TS | Transform-skipped coefficients are reordered using the estimated prediction accuracy calculated by the difference between L0 and L1 reference blocks when bi-prediction is applied. |
| MVPlanar | Sub-block motion vector derivation by interpolating the neighboring MV predictors with explicit signaling of inter prediction indices and reference frame indices. |
| PDIntrafilter | Two types of intra interpolation filters are alternatively applied depending on the position of the prediction samples. |
| Luma-adaptive deblocking filter | Deblocking filter strength increment according to the luma level |

Decoder algorithm description

General algorithm description

Bitstream structure

Refer to JVET-J0026.

Entropy decoding

Refer to JVET-J0026.

Conversion process from entropy decoded data to symbols

Intra prediction parameters

Chroma DM binarization bug fix

The intra prediction mode coding is almost the same as JEM other than a small bug fix reported in JVET-H0077.

In current JEM, a total of 11 chroma intra mode are allowed, 5 of which are traditional intra prediction modes and the others of which are cross-component prediction modes. In the pruning process of intra prediction modes of chroma blocks, the first flag to be signaled indicates whether traditional intra prediction is applied or not. If the traditional intra prediction is applied, a total of 5 intra prediction modes are derived from the co-located luma blocks and spatial neighbouring chroma blocks (which are calls as DM0, DM1, DM2, DM3 and DM4). Then a flag which intra mode is applied to the current chroma block among those 5 intra prediction modes is signaled by truncated rice coding with a rice parameter of 0.

However, one redundant CABAC-coded bit is signaled in the current JEM implementation when the last candidate of the DM list was selected. Figure 2.1.3.1-1 illustrates the pruning process of chroma intra mode. As shown in Figure 1, when the flag which indicates whether DM3 is applied or not is 0, always DM4 is selected. It means that a redundant CABAC-coded bit (always appears as 1) is signaled.

Figure 2.1.3.1.1-1. Pruning process of JEM

The proposed syntax change is to remove the last CABAC-coded bit of DM index signaling as shown in Figure 2.1.3.1.1-2.

Figure 2.1.3.1.1-2. Proposed pruning process

Motion parameters

Same as JEM.

Quantization parameters

Same as JEM.

Transform coefficients

Transform-skipped coefficient reordering (Bi-pred optimized TS)

In JEM, the entropy coding of both transform coefficients and transform-skipped coefficients is performed in predefined scan order from top left to bottom right. In the proposed scheme, the transform-skipped coefficients for inter-coded blocks with the bi-prediction mode is implicitly reordered by estimating prediction accuracy at the decoder side.

A prediction block of bi-prediction is generated by the average of two reference blocks in JEM. In general, regions with simple motion can be precisely predicted by motion compensation. When the bi-prediction is applied to those regions, the two reference blocks are highly correlated. On the contrary, when the current block contains complex motion or occlusion, the correlation in those parts of the block becomes lower. Since prediction samples are generated the average of the two reference blocks when bi-prediction is applied, the residual of those parts tends to have higher amplitude. Under this assumption, the prediction accuracy within the block can be estimated by taking a correlation between the reference blocks when bi-prediction is applied. The proposed scheme utilizes the estimated prediction accuracy for reordering transform-skipped coefficients to improve coding efficiency relating to significant coefficients signaling.

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma block relative to the top-left luma sample of the current picture,

- two variables nTbW and nTbH specifying the width and the height of the luma block,

- a (nTbW)x(nTbH) array d of luma transform coefficients with elements d[ x ][ y ],

- two (nTbW)x(nTbH) arrays predSamplesL0 and predSamplesL1 of list0 and list1 luma prediction samples derived by the inter prediction process of JEM.

Output of this process is:

- The (nTbW)x(nTbH) array r of reordered luma transform coefficients with elements r[ x ][ y ].

The variables inter_pred_idc and transform_skip_flag are derived by the decoding processes the same as JEM. Note that inter_pred_idc specifies whether list0, list1, or bi-prediction is used for the current block, and transform_skip_flag specifies whether a transform is applied to the current block or not, respectively.

The (nTbW)x(nTbH) array r of reordered luma transform coefficients is derived as follows:

- If inter_pred_idc[ xTbY ][ xTbY ] is equal to PRED_BI and transform_skip_flag[ xTbY ][ yTbY ] is equal to 1, the following ordered steps apply:

- Let g, nRbW, and nRbH be a predefined constant value of the granularity of the transform coefficients reordering, the number of reordering units in the current block width and height, respectively. The (nRbW)x(nRbH) array m of the estimated prediction accuracy map with elements m[ xR ][ yR ] is derived as follows:

m xR  yR =( i,j1≪g( predSampleL0 xR≪g+i  yR≪g+j 
- predSampleL1 xR≪g+i  yR≪g+j  ) )≫( g≪1 ).

Note that the constant value g was set equal to 2 when the submitted bitstreams were generated.

- The (nRbW)x(nRbH) array p of reordering scan order with elements p[ x ][ y ] is derived as follows:

count = 0
while ( count < nRbW * nRbH )
{
  for ( i = 0; i < nRbW * nRbH; i++ )
      if ( m[ i % nRbW ][ i / nRbW ] == max( m ) )
      {
        p[ i % nRbW ][ i / nRbW ] = count++
        m[ i / nRbW ][ i / nRbW ] = 0
        break
      }
}

- Let the (nRbW)x(nRbH) array diagScan of up-right diagonal scan order specified in HEVC specification. The (nRbW)x(nRbH) array r of reordered transform coefficients is derived as follows:

for ( i = 0; i < nRbW * nRbH; i++ )

for ( s = 0; s < ( 1 << g ); s++ )

for ( t = 0; t < (1 << g ); t++ )
      r[ diagScan[ i ][ 0 ] + s ][ diagScan[ i ][ 1 ] + t ]

= d[ diagScan[ p[ i ][ j ] ][ 0 ] + s ][ diagScan[ p[ i ][ j ] ][ 1 ] + t ]

- Otherwise, the (nTbW)x(nTbH) array r is set equal to the (nTbW)x(nTbH) array of transform coefficients d[ x ][ y ].

In-loop filter parameters

Same as JEM.

Decoder-side motion refinement

Refer to JVET-J0026.

Inverse quantization

Same as JEM.

Inverse transforms

Same as JEM.

Motion compensation

Interpolated motion compensation prediction (MVPlanar)

MVPlanar introduces an additional motion vector derivation mode. The proposed scheme generates sub-block motion vector field by interpolating motion vectors by weighted average of motion vectors of the above, above-right, left, and bottom-left neighboring blocks. In JEM, sub-block motion vector is also introduced by Spatial-temporal motion vector prediction (STMVP). The motion vectors are generated recursively by using the temporally and spatially neighboring MV predictors. The major difference between the proposed scheme and STMVP is the derivation of the inter prediction index and reference frame index specified as inter_pred_idc and ref_idx in HEVC specifications, which specifies whether list0, list1, or bi-prediction is used for the current block and specifies an index into a reference picture list, respectively. In STMVP, inter_pred_idc and ref_idx are derived by taking median of those of the neighboring blocks, whereas inter_pred_idc and ref_idx are signaled at coding block level in the proposed scheme. The proposed scheme is composed of the reference motion vector substitution process and the motion vector interpolation process as described below.

Inputs to the reference motion vector substitution process are:

- reference motion vectors refMV[ x ][ y ] with x = -1, y = -1..nCbH/nSb and x = 0..nCbW/nSb, y = -1 for motion vector interpolation, where nSb is the constant value of the sub-block size,

- two variables nCbW and nCbH specifying the width and the height of the luma coding block,

Outputs to this process are:

- The values of the modified reference motion vectors refMV[ x ][ y ] with x = -1, y = -1..nCbH/nSb and x = 0..nCbW/nSb, y = -1 for motion vector interpolation.

Note that the value nSb is set equal to 4 when the submitted bitstreams were generated.

The values of the reference motion vectors refMV[ x ][ y ], with x = -1, y = -1..nCbH/nSb and x = 0..nCbW/nSb, y = -1 aremodified as follows:

- If all the neighboring blocks are intra-coded blocks or marked as “not available”, zero motion vector is substituted for the values of all reference motion vectors refMV[ x ][ y ].

- Otherwise (at least one but not all neighboring blocks are intra-coded or marked as “not available”), the following ordered steps apply:

- When refMV[ -1 ][ nCbH/nSb ] is marked as “not available”, search sequentially starting from x = -1, y = nCbH/nSb to x = -1, y = -1, then from x = 0, y = -1 to x = nCbW/nSb, y = -1. Once a reference motion vector refMV[ x ][ y ] marked as “available” is found, the search is terminated and the value of refMV[ x ][ y ] is assigned to refMV[ -1 ][ nCbH/nSb ].

- Search sequentially starting from x = -1, y = nCbH/nSb to x = -1, y = -1, when refMV[ x ][ y ] is marked as “not available”, the value of refMV[ x ][ y + 1 ] is substituted for the value of refMV[ x ][ y ].

- For x = 0..nCbW/nSb, y = -1, when refMV[ x ][ y ] is marked as “not available”, the value of refMV[ x – 1 ][ y ] is substituted for the value of refMV[ x ][ y ].

All reference motion vectors refMV[ x ][ y ] with x = -1, y = -1..nCbH/nSb and x = 0..nCbW/nSb, y = -1 are marked as “available”.

Inputs to the motion vector interpolation process are:

- the reference motion vectors refMV[ x ][ y ], with x = -1, y = -1..nCbH/nSb and x = 0..nCbW/nSb, y = -1, where nSb is the constant value of the sub-block size,

- two variables nCbW and nCbH specifying the width and the height of the luma coding block,

Outputs to this process are:

- The values of the interpolated motion vectors interpMV[ x ][ y ], with x = 0..nCbW/nSb and y = 0..nCbH/nSb, where nSb is the constant value of sub-block size.

The value of the interpolated motion vectors interpMV[ x ][ y ] are derived as follows:

interpMV[ x ][ y ] = ( ( ( nCbW/nSb -1 – x ) * refMV[ -1 ][ y ]

+ ( x + 1 ) * refMV[ nCbW/nSb ][ -1 ] ) << Log( nCbW/nSb ) )

+ ( ( nCbH/nSb – 1 – y ) * refMV[ x ][ -1 ]

+ ( y + 1 ) * refMV[ -1 ][ nCbH/nSb ] ) << Log( nCbH/nSb ) ) )

>> ( Log( nCbW/nSb ) + Log( nCbH/nSb ) + nCbW * nCbH )

The individual performance of MVPlanar is shown in 12.10.2.1

Intra prediction

Position dependent intra interpolation filter (PDIntraFilter)

In JEM, two types of four-tap intra interpolation filters are used to generate intra prediction samples. Cubic interpolation filter is applied if the block size is smaller than or equal to 64 samples, otherwise, Gaussian interpolation filter is applied. In this proposal, these two intra interpolation filters are alternatively applied depending on the position of the prediction samples. When an intra prediction mode is ranging from 34 to 66, cubic interpolation filter is used if the prediction sample position is in the upper part of the block. Otherwise, Gaussian interpolation filter is used. In the same fashion, when intra prediction mode is ranging from 2 to 33, cubic interpolation filter is used for the left part of the block, and Gaussian interpolation filter is applied for the others.

Figure 2.1.8.1-1 Example of the proposed PDIntraFilter

Figure 2.1.8.1-1 shows an example of the proposed PDintraFilter, where N in the figure is a constant threshold value. Note that the constant value N was set equal to 16 when the submitted bitstreams were generated.

Inputs to this process are:

- the intra prediction mode predModeIntra,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbX + nTbY − 1 and x = 0..nTbX + nTbY − 1, y = −1,

- a variable nTbX and nTbY specifying the width and height of a block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbX − 1, y = 0..nTbY − 1.

The derivation processes of reference sample array ref[ x ], the angle parameter intraPredAngle, cubic filter coefficients c[ x ], and gaussian filter coefficients g[ x ] are same as JEM. Let N be a predefined constant threshold value of the line which belongs of the prediction sample. The values of the prediction samples predSamples are derived follows:

- If predModeIntra is greater than or equal to 34, the following ordered steps apply:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 ) * intraPredAngle ) >> 5

iFact = ( ( y + 1 ) * intraPredAngle ) & 31

- Depending on the value of iFact, the following applies:

- If iFact is not equal to 0 and y is less than N, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] =

( c0 * ref[ x + iIdx ] + c1 * ref[ x + iIdx + 1 ]

+ c2 * ref[ x + iIdx + 2 ] + c3 * ref[ x + iIdx + 3 ] + 128 ) >> 8

- Otherwise, if iFact is not equal to 0 and y is greater than or equal to N, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] =

( g0 * ref[ x + iIdx ] + g1 * ref[ x + iIdx + 1 ]

+ g2 * ref[ x + iIdx + 2 ] + g3 * ref[ x + iIdx + 3 ] + 128 ) >> 8

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]

- When predModeIntra is equal to 50 (vertical), the value of the prediction samples predSamples[ x ][ y ] is derived the same as JEM.

- Otherwise (predModeIntra is less than 34), the following ordered steps apply:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 ) * intraPredAngle ) >> 5

iFact = ( ( x + 1 ) * intraPredAngle ) & 31

- Depending on the value of iFact, the following applies:

- If iFact is not equal to 0 and x is less than N, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] =

( c0 * ref[ y + iIdx ] + c1 * ref[ y + iIdx + 1 ]

+ c2 * ref[ y + iIdx + 2 ] + c3 * ref[ y + iIdx + 3 ] + 128 ) >> 8

- Otherwise, if iFact is not equal to 0 and x is greater than or equal to N, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] =

( g0 * ref[ y + iIdx ] + g1 * ref[ y + iIdx + 1 ]

+ g2 * ref[ y + iIdx + 2 ] + g3 * ref[ y + iIdx + 3 ] + 128 ) >> 8

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]

- When predModeIntra is equal to 18 (horizontal), the value of the prediction samples predSamples[ x ][ y ] is derived the same as JEM.

Other intra prediction tools

Refer to JVET-J0026.

In-loop filtering

Luma-adaptive deblocking filter

This proposal modifies the deblocking filter process of HEVC. The proposed deblocking filter applies variable filter strength depending on the level of reconstructed samples before in-loop filters. The distortion of decoded pictures is caused by a quantization process in the video signal domain. The distortion is displayed through the nonlinear process called Electro-Optical Transfer Function (EOTF) in the linear light domain. Additionally, the scene referred video system such as conventional SDR format and one of the HDR formats, Hybrid Log-Gamma (HLG), requires an additional nonlinear process called Opto-Optical Transfer Function (OOTF) also known as a system gamma. The displayed subjective distortion is enhanced by the nonlinear transfer function depending on video signal level. Such mechanism makes block distortion visible at bright areas, even if the quantization noise is uniformly added at all video level. Especially, for HDR formats, this enhanced distortion becomes salient due to its strong nonlinearity.

The proposed deblocking filter manages the strength of the deblocking filter according to the luma level of the reconstructed samples before in-loop filters. In the derivation process of the parameters of deblocking filter, the variable qPL of averaged quantization parameters of the current block and neighboring block is increased according to the averaged luma level. Since the EOTF and OOTF vary among each video format, the conditions of the increment of qPL is different according to the transfer_characteristics which is specified as a VUI syntax in HEVC specification.

Inputs to this process are:

– a luma picture sample array recPictureL,

– a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

– a luma location ( xBl, yBl ) specifying the top-left sample of the current luma block relative to the top-left sample of the current luma coding block,

– a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered.

Outputs of this process are:

– the variables β and tC.

The variable LL of the reconstructed luma level is derived as follow:

LL= ( ( p0,0 + p0,3 + q0,0 + q0,3 ) >> 2 ) / ( 1 << iBitDepth ),

where, the sample values pi,k and qi,k with i = 0..3 and k = 0 and 3 are derived the same as HEVC as shown in Figure 2.1.9.1-1.

Figure 2.1.9.1-1 Sample position of pi,k and qi,k

The variables QpQ and QpP are set equal to the QpY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

The variable qPL is derived as follows:

qPL = ( ( QPQ + QPP +1 ) >> 1 ) + qpOffset,

where qpOffset is specified in Table 2.1.9.1-1.

The subsequent derivation processes of the variables β and tC are same as HEVC.

Table 2.1.9.1-1 Specification of qpOffset as a function of LL

| transfer_characteristics == 16 (HLG format) |  | transfer_characteristics == 18 (PQ format) |  | Otherwise (conventional SDR format) |  |
| --- | --- | --- | --- | --- | --- |
| Conditions | qpOffset | Conditions | qpOffset | Conditions | qpOffset |
| LL <= 0.84 | 0 | LL <= 0.8 | 0 | LL <= 0.875 | 0 |
| 0.84 < LL <= 0.91 | 1 | 0.8 < LL <= 0.9 | 1 | 0.875 < LL | 1 |
| 0.91 < LL <= 0.99 | 2 | 0.9 < LL | 2 |  |  |
| 0.99 < LL | 3 |  |  |  |  |

Additional tools

Refer to JVET-J0026.

Additional algorithmic description topic(s)

Refer to JVET-J0026.

360° Video

No additional tools for 360° coding are proposed.

HDR

Post-processing

None.

Quantization handling

Refer to JVET-J0026.

HDR specific decoding tools

Refer to JVET-J0026.

Encoder algorithm description

General algorithm description

Rate-distortion optimization

Distortion metrics

Refer to JVET-J0026.

Luma chroma quality balance adjustment

In JEM, chroma qp offset for Cb/Cr being equal to 1 is utilized in random access configuration to alleviate chroma gain balance issue. In addition to this offset, the following setting noted ChromaWeight is adjusted to get better luma chroma gain balance. Without this adjustment, chroma improvement is too much compared to luma improvement causing serious imbalance.

λchroma=λmode2QP-QPchromawchroma

The default chroma weight wchroma of HM and JEM is equal to 3.0. If the chroma weight is greater than 3.0, the luma gain is emphasized, if the chroma weight is smaller than 3.0, the chroma gain is emphasized.

|  | ChromaWeight | note |
| --- | --- | --- |
| JEM | 3.0 |  |
| SDR constraint set 1 (RA) | 3.2 | chroma bits are shifted to luma |
| SDR constraint set 2 (LB) | 3.1 | chroma bits are shifted to luma |
| HDR-A HLG | 3.0 | same as JEM |
| HDR-B HLG | 3.1 | chroma bits are shifted to luma |

Note: JVET-J0026 uses a different luma chroma quality balance adjustment method.

Rate-distortion dependency models

Same as JEM.

Partitioning structure selection

Refer to JVET-J0026.

Intra prediction mode estimation and type selection

Refer to JVET-J0026.

Motion estimation and motion segment selection

General

Same as JEM.

Usage of decoder-side motion refinement

Same as JEM.

Mode decisions

An additional RD cost calculation for MVPlanar mode is introduced.

In-loop filtering type selection and parameter estimation

Refer to JVET-J0026.

Transforms and transform type selection

The proposed bi-pred optimized TS is always applied if the prediction mode of the current block is bi-prediction and if transform skip mode is applied to the current block. The RD cost of the transform skip mode for bi-prediction CU is calculated accordingly.

Quantization and quantization type selection

Same as JEM.

Entropy coding

Same as JEM.

HDR

Pre-processing

None

HDR specific encoding tools

Refer to JVET-J0026.

Additional encoder optimization

Refer to JVET-J0026.

Compression performance

Compression performance of SDR and HDR categories are addressed in this section. Because the software generating the submitted bitstreams, which were delivered to the test chair by 15 February, has a minor bug which affects the compression performance for some sequences significantly, the performance of the software with bug fix is also included.

SDR: Constraint set 1 configuration relative to HM anchor

Class SDR-A (Submitted software)

Class SDR-B (Submitted software)

Overall (Submitted software)

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -34.6% | -46.0% | -49.9% | 1.73 | 1.35 | 1.61 |
| CatRobot1 | -39.1% | -53.8% | -46.8% | 1.53 | 0.99 | 1.33 |
| DaylightRoad2 | -41.1% | -60.3% | -44.6% | 1.15 | 1.33 | 0.84 |
| ParkRunning3 | -32.8% | -25.0% | -27.8% | 1.06 | 0.42 | 0.39 |
| Campfire | -36.4% | -34.6% | -50.3% | 1.08 | 1.29 | 1.55 |
| BQTerrace | -33.9% | -50.9% | -59.0% | 1.03 | 0.97 | 1.15 |
| RitualDance | -29.3% | -40.7% | -46.2% | 1.58 | 1.19 | 1.57 |
| MarketPlace | -30.4% | -51.3% | -49.1% | 1.07 | 0.93 | 0.93 |
| BasketballDrive | -33.3% | -49.6% | -46.7% | 1.21 | 1.12 | 1.44 |
| Cactus | -34.6% | -47.4% | -42.8% | 1.42 | 0.77 | 1.12 |
| Average SDR-A | -36.8% | -43.9% | -43.9% | 1.31 | 1.08 | 1.14 |
| Average SDR-B | -32.3% | -48.0% | -48.7% | 1.26 | 1.00 | 1.24 |
| Average all | -34.6% | -46.0% | -46.3% | 1.29 | 1.04 | 1.19 |
| Enc Time[%] | 1890% |  |  |  |  |  |
| Dec Time[%] | 1630% |  |  |  |  |  |
| Max Delta Rate[%] | 0.00% |  |  |  |  |  |
| Min Delta Rate[%] | -0.81% |  |  |  |  |  |
| Max Delta PSNR (YUV) | 2.07 |  | 1.55 |  | 1.82 |  |
| Min Delta PNSR (YUV) | 0.72 |  | 0.41 |  | 0.38 |  |

Overall (Bug-fixed software)

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -35.0% | -46.2% | -50.2% | 1.76 | 1.36 | 1.62 |
| CatRobot1 | -40.9% | -55.5% | -48.6% | 1.62 | 1.03 | 1.40 |
| DaylightRoad2 | -42.1% | -60.8% | -45.3% | 1.19 | 1.35 | 0.86 |
| ParkRunning3 | -33.1% | -25.2% | -28.0% | 1.07 | 0.43 | 0.40 |
| Campfire | -36.5% | -34.7% | -50.4% | 1.08 | 1.29 | 1.55 |
| BQTerrace | -33.9% | -50.8% | -59.0% | 1.03 | 0.97 | 1.15 |
| RitualDance | -29.5% | -40.8% | -46.3% | 1.59 | 1.19 | 1.57 |
| MarketPlace | -31.1% | -51.6% | -49.7% | 1.10 | 0.94 | 0.94 |
| BasketballDrive | -33.5% | -49.6% | -46.9% | 1.21 | 1.12 | 1.45 |
| Cactus | -37.2% | -49.0% | -44.2% | 1.55 | 0.81 | 1.17 |
| Average SDR-A | -37.5% | -44.5% | -44.5% | 1.34 | 1.09 | 1.17 |
| Average SDR-B | -33.0% | -48.4% | -49.2% | 1.30 | 1.01 | 1.26 |
| Average all | -35.3% | -46.4% | -46.9% | 1.32 | 1.05 | 1.21 |
| Enc Time[%] | 2175% |  |  |  |  |  |
| Dec Time[%] | 1955% |  |  |  |  |  |
| Max Delta Rate[%] | -0.04% |  |  |  |  |  |
| Min Delta Rate[%] | -3.63% |  |  |  |  |  |
| Max Delta PSNR (YUV) | 2.08 |  | 1.56 |  | 1.82 |  |
| Min Delta PNSR (YUV) | 0.72 |  | 0.41 |  | 0.38 |  |

SDR: Constraint set 2 configuration relative to HM anchor

Class SDR-B (Submitted software)

Overall (Submitted software)

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -28.7% | -47.9% | -54.2% | 0.86 | 0.57 | 0.65 |
| RitualDance | -23.4% | -36.6% | -38.8% | 1.21 | 0.89 | 1.09 |
| MarketPlace | -22.7% | -43.6% | -43.0% | 0.73 | 0.57 | 0.62 |
| BasketballDrive | -27.1% | -46.9% | -45.0% | 1.02 | 1.01 | 1.34 |
| Cactus | -28.8% | -43.3% | -40.4% | 1.16 | 0.62 | 0.91 |
| Average SDR-B | -26.2% | -43.7% | -44.3% | 1.00 | 0.73 | 0.92 |
| Enc Time[%] | 1293% |  |  |  |  |  |
| Dec Time[%] | 667% |  |  |  |  |  |
| Max Delta Rate | 0.00% |  |  |  |  |  |
| Min Delta Rate | -2.04% |  |  |  |  |  |
| Max Delta PSNR (YUV) | 1.30 |  | 1.11 |  | 1.46 |  |
| Min Delta PNSR (YUV) | 0.58 |  | 0.49 |  | 0.53 |  |

Overall (Bug-fixed software)

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -28.8% | -47.2% | -53.6% | 0.86 | 0.55 | 0.64 |
| RitualDance | -23.6% | -36.7% | -39.0% | 1.22 | 0.90 | 1.10 |
| MarketPlace | -23.1% | -43.4% | -42.5% | 0.75 | 0.57 | 0.61 |
| BasketballDrive | -27.4% | -47.2% | -45.4% | 1.04 | 1.02 | 1.35 |
| Cactus | -30.2% | -43.8% | -40.8% | 1.22 | 0.63 | 0.92 |
| Average SDR-B | -26.6% | -43.7% | -44.2% | 1.02 | 0.73 | 0.92 |
| Enc Time[%] | 1602% |  |  |  |  |  |
| Dec Time[%] | 942% |  |  |  |  |  |
| Max Delta Rate | -0.28% |  |  |  |  |  |
| Min Delta Rate | -2.31% |  |  |  |  |  |
| Max Delta PSNR (YUV) | 1.33 |  | 1.12 |  | 1.47 |  |
| Min Delta PNSR (YUV) | 0.58 |  | 0.48 |  | 0.51 |  |

SDR: Constraint set 1 configuration relative to JEM anchor

Class SDR-A (Submitted software)

Class SDR-A (Bug-fixed software)

Class SDR-B (Submitted software)

Class SDR-B (Bug-fixed software)

Overall (Submitted software)

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -2.1% | -4.3% | -6.0% | 0.08 | 0.09 | 0.14 |
| CatRobot1 | 0.3% | -3.8% | -2.4% | -0.01 | 0.04 | 0.04 |
| DaylightRoad2 | -1.3% | -8.8% | -6.3% | 0.02 | 0.11 | 0.08 |
| ParkRunning3 | -2.5% | 0.2% | -0.6% | 0.07 | 0.00 | 0.01 |
| Campfire | -3.3% | -0.1% | -1.6% | 0.07 | 0.00 | 0.03 |
| BQTerrace | -5.2% | -12.0% | -11.8% | 0.12 | 0.15 | 0.13 |
| RitualDance | -2.8% | -6.6% | -9.9% | 0.13 | 0.14 | 0.25 |
| MarketPlace | -1.9% | -8.1% | -8.6% | 0.05 | 0.10 | 0.12 |
| BasketballDrive | -3.2% | -7.9% | -5.9% | 0.09 | 0.11 | 0.13 |
| Cactus | 0.6% | -4.2% | -3.0% | -0.02 | 0.04 | 0.06 |
| Average SDR-A | -1.8% | -3.3% | -3.4% | 0.05 | 0.05 | 0.06 |
| Average SDR-B | -2.5% | -7.8% | -7.8% | 0.07 | 0.11 | 0.14 |
| Average all | -2.1% | -5.5% | -5.6% | 0.06 | 0.08 | 0.10 |
| Enc Time[%] | 237% |  |  |  |  |  |
| Dec Time[%] | 214% |  |  |  |  |  |
| Max Delta Rate[%] |  |  |  |  |  |  |
| Min Delta Rate[%] |  |  |  |  |  |  |
| Max Delta PSNR (YUV) | 0.22 |  | 0.16 |  | 0.26 |  |
| Min Delta PNSR (YUV) | -0.06 |  | -0.02 |  | -0.01 |  |

Overall (Bug-fixed software)

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -2.6% | -4.5% | -6.4% | 0.10 | 0.09 | 0.15 |
| CatRobot1 | -2.8% | -7.4% | -5.7% | 0.07 | 0.08 | 0.11 |
| DaylightRoad2 | -3.1% | -10.0% | -7.6% | 0.06 | 0.12 | 0.10 |
| ParkRunning3 | -2.9% | -0.1% | -0.8% | 0.08 | 0.00 | 0.01 |
| Campfire | -3.4% | -0.1% | -1.8% | 0.07 | 0.00 | 0.04 |
| BQTerrace | -5.2% | -11.9% | -11.8% | 0.12 | 0.15 | 0.13 |
| RitualDance | -3.0% | -6.6% | -10.1% | 0.14 | 0.14 | 0.25 |
| MarketPlace | -2.8% | -8.8% | -9.6% | 0.08 | 0.11 | 0.13 |
| BasketballDrive | -3.4% | -8.0% | -6.3% | 0.10 | 0.12 | 0.14 |
| Cactus | -3.4% | -7.2% | -5.5% | 0.10 | 0.08 | 0.10 |
| Average SDR-A | -3.0% | -4.4% | -4.5% | 0.08 | 0.06 | 0.08 |
| Average SDR-B | -3.6% | -8.5% | -8.7% | 0.11 | 0.12 | 0.15 |
| Average all | -3.3% | -6.5% | -6.6% | 0.09 | 0.09 | 0.12 |
| Enc Time[%] | 273% |  |  |  |  |  |
| Dec Time[%] | 257% |  |  |  |  |  |
| Max Delta Rate[%] |  |  |  |  |  |  |
| Min Delta Rate[%] |  |  |  |  |  |  |
| Max Delta PSNR (YUV) | 0.22 |  | 0.16 |  | 0.26 |  |
| Min Delta PNSR (YUV) | -0.04 |  | -0.02 |  | -0.01 |  |

SDR: Constraint set 2 configuration relative to JEM anchor

Class SDR-B (Submitted software)

Overall (Submitted software)

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -3.5% | -1.3% | -6.8% | 0.08 | 0.01 | 0.04 |
| RitualDance | -2.2% | -3.6% | -5.2% | 0.10 | 0.07 | 0.12 |
| MarketPlace | -2.6% | -1.3% | -3.5% | 0.08 | 0.01 | 0.04 |
| BasketballDrive | -2.0% | -2.2% | -2.0% | 0.06 | 0.03 | 0.04 |
| Cactus | -0.8% | -1.6% | -1.6% | 0.03 | 0.01 | 0.03 |
| Average SDR-B | -2.2% | -2.0% | -3.8% | 0.07 | 0.03 | 0.05 |
| Enc Time[%] | 161% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |
| Max Delta Rate |  |  |  |  |  |  |
| Min Delta Rate |  |  |  |  |  |  |
| Max Delta PSNR (YUV) | 0.14 |  | 0.06 |  | 0.10 |  |
| Min Delta PNSR (YUV) | -0.06 |  | -0.02 |  | -0.01 |  |

Overall (Bug-fixed software)

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -3.5% | 0.8% | -4.3% | 0.08 | -0.01 | 0.02 |
| RitualDance | -2.4% | -3.7% | -5.5% | 0.11 | 0.07 | 0.12 |
| MarketPlace | -3.1% | -1.1% | -2.7% | 0.09 | 0.01 | 0.03 |
| BasketballDrive | -2.4% | -2.7% | -2.7% | 0.07 | 0.04 | 0.06 |
| Cactus | -2.8% | -2.4% | -1.9% | 0.09 | 0.02 | 0.03 |
| Average SDR-B | -2.8% | -1.8% | -3.4% | 0.09 | 0.03 | 0.05 |
| Enc Time[%] | 199% |  |  |  |  |  |
| Dec Time[%] | 141% |  |  |  |  |  |
| Max Delta Rate |  |  |  |  |  |  |
| Min Delta Rate |  |  |  |  |  |  |
| Max Delta PSNR (YUV) | 0.14 |  | 0.06 |  | 0.11 |  |
| Min Delta PNSR (YUV) | -0.05 |  | -0.02 |  | -0.03 |  |

HDR: Constraint set 1 configuration relative to HM anchor

Class HDR-A (Submitted software)

Class HDR-B (Submitted software)

Overall (Submitted software)

|  | Constraint Set 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |  |  |
|  | BD-rate |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| DayStreet | -60.8% | -37.9% | -36.4% | -58.2% | -70.7% | -35.4% | -56.8% | -69.2% |
| PeopleInShoppingCenter | -53.5% | -32.0% | -30.7% | -62.8% | -58.5% | -30.5% | -60.4% | -58.4% |
| SunsetBeach | -60.8% | -27.2% | -22.1% | -83.7% | -83.5% | -22.9% | -78.9% | -84.9% |
| Market | -45.3% | -27.9% | -28.0% | -54.4% | -50.7% | -25.1% | -45.9% | -40.1% |
| ShowGirl2 | -43.8% | -30.2% | -30.6% | -74.8% | -61.8% | -28.1% | -74.6% | -61.6% |
| Hurdles | -55.4% | -38.2% | -37.0% | -65.3% | -58.3% | -33.6% | -59.0% | -53.2% |
| Starting | -41.0% | -34.6% | -32.8% | -41.2% | -47.0% | -30.4% | -38.5% | -32.5% |
| Cosmos1 | -38.8% | -28.3% | -27.3% | -69.3% | -84.1% | -25.3% | -68.9% | -83.6% |
| Average HDR-A | -58.4% | -32.3% | -29.7% | -68.2% | -70.9% | -29.6% | -65.3% | -70.8% |
| Average HDR-B | -44.9% | -31.9% | -31.1% | -61.0% | -60.4% | -28.5% | -57.4% | -54.2% |
| Average all | -49.9% | -32.0% | -30.6% | -63.7% | -64.3% | -28.9% | -60.4% | -60.4% |
| Enc Time[%] | 1786% |  |  |  |  |  |  |  |
| Dec Time[%] | 1726% |  |  |  |  |  |  |  |
| Max Delta Rate[%] | 0.0% |  |  |  |  |  |  |  |
| Min Delta Rate[%] | -1.1% |  |  |  |  |  |  |  |
| Max Delta PSNR | 1.08 | 1.15 | 2.43 | 2.19 | 2.99 | 2.13 | 2.03 | 2.50 |
| Min Delta PNSR | 0.32 | 0.27 | 0.44 | 0.80 | 0.51 | 0.44 | 0.61 | 0.25 |

|  | Constraint Set 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |  |  |
|  | BD-PSNR |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| DayStreet | 0.83 | 0.87 | 1.58 | 1.15 | 1.59 | 1.63 | 1.21 | 1.76 |
| PeopleInShoppingCenter | 0.95 | 0.88 | 1.66 | 2.13 | 2.01 | 1.69 | 1.96 | 1.98 |
| SunsetBeach | 0.48 | 0.29 | 0.55 | 1.42 | 1.28 | 0.55 | 1.28 | 1.13 |
| Market | 0.57 | 0.58 | 1.31 | 0.92 | 0.84 | 1.14 | 0.73 | 0.62 |
| ShowGirl2 | 0.47 | 0.73 | 1.49 | 1.38 | 0.72 | 1.25 | 1.26 | 0.70 |
| Hurdles | 0.81 | 0.78 | 1.75 | 1.75 | 1.92 | 1.57 | 1.41 | 1.59 |
| Starting | 0.53 | 1.07 | 2.27 | 1.01 | 0.77 | 1.98 | 0.85 | 0.49 |
| Cosmos1 | 0.45 | 0.42 | 0.78 | 1.05 | 1.14 | 0.68 | 1.05 | 1.12 |
| Average HDR-A | 0.75 | 0.68 | 1.27 | 1.57 | 1.63 | 1.29 | 1.48 | 1.62 |
| Average HDR-B | 0.57 | 0.72 | 1.52 | 1.22 | 1.08 | 1.33 | 1.06 | 0.90 |
| Average all | 0.64 | 0.70 | 1.42 | 1.35 | 1.28 | 1.31 | 1.22 | 1.17 |

Overall (Bug-fixed software)

|  | Constraint Set 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |  |  |
|  | BD-rate |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| DayStreet | -61.4% | -38.7% | -37.2% | -58.8% | -71.2% | -36.2% | -57.4% | -69.8% |
| PeopleInShoppingCenter | -53.6% | -32.1% | -30.9% | -62.8% | -58.7% | -30.6% | -60.5% | -58.5% |
| SunsetBeach | -60.9% | -27.3% | -22.2% | -83.8% | -83.6% | -22.9% | -79.0% | -85.1% |
| Market | -45.3% | -27.9% | -28.1% | -54.4% | -50.7% | -25.2% | -45.8% | -40.3% |
| ShowGirl2 | -43.8% | -30.3% | -30.7% | -75.1% | -61.6% | -28.1% | -74.9% | -61.2% |
| Hurdles | -55.5% | -38.4% | -37.2% | -65.4% | -58.1% | -33.9% | -59.0% | -53.2% |
| Starting | -41.4% | -35.1% | -33.4% | -41.4% | -47.4% | -31.0% | -38.6% | -33.0% |
| Cosmos1 | -39.0% | -28.5% | -27.4% | -69.5% | -84.2% | -25.5% | -69.0% | -83.6% |
| Average HDR-A | -58.6% | -32.7% | -30.1% | -68.5% | -71.2% | -29.9% | -65.6% | -71.1% |
| Average HDR-B | -45.0% | -32.1% | -31.4% | -61.2% | -60.4% | -28.7% | -57.5% | -54.3% |
| Average all | -50.1% | -32.3% | -30.9% | -63.9% | -64.4% | -29.2% | -60.5% | -60.6% |
| Enc Time[%] | 2073% |  |  |  |  |  |  |  |
| Dec Time[%] | 1958% |  |  |  |  |  |  |  |
| Max Delta Rate[%] | 0.0% |  |  |  |  |  |  |  |
| Min Delta Rate[%] | -1.5% |  |  |  |  |  |  |  |
| Max Delta PSNR | 1.08 | 1.16 | 2.46 | 2.20 | 2.98 | 2.15 | 2.03 | 2.50 |
| Min Delta PNSR | 0.33 | 0.28 | 0.44 | 0.79 | 0.52 | 0.45 | 0.61 | 0.26 |

|  | Constraint Set 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |  |  |
|  | BD-PSNR |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| DayStreet | 0.85 | 0.90 | 1.63 | 1.17 | 1.61 | 1.68 | 1.23 | 1.78 |
| PeopleInShoppingCenter | 0.95 | 0.88 | 1.67 | 2.14 | 2.01 | 1.70 | 1.97 | 1.98 |
| SunsetBeach | 0.48 | 0.29 | 0.55 | 1.42 | 1.29 | 0.55 | 1.28 | 1.13 |
| Market | 0.57 | 0.58 | 1.31 | 0.91 | 0.84 | 1.14 | 0.72 | 0.61 |
| ShowGirl2 | 0.47 | 0.73 | 1.49 | 1.39 | 0.72 | 1.26 | 1.27 | 0.70 |
| Hurdles | 0.81 | 0.79 | 1.76 | 1.75 | 1.92 | 1.58 | 1.41 | 1.59 |
| Starting | 0.53 | 1.09 | 2.32 | 1.01 | 0.78 | 2.02 | 0.86 | 0.50 |
| Cosmos1 | 0.45 | 0.42 | 0.78 | 1.05 | 1.15 | 0.69 | 1.05 | 1.13 |
| Average HDR-A | 0.76 | 0.69 | 1.28 | 1.58 | 1.64 | 1.31 | 1.49 | 1.63 |
| Average HDR-B | 0.57 | 0.72 | 1.53 | 1.22 | 1.08 | 1.34 | 1.06 | 0.91 |
| Average all | 0.64 | 0.71 | 1.44 | 1.36 | 1.29 | 1.33 | 1.22 | 1.18 |

HDR: Constraint set 1 configuration relative to JEM anchor

lass HDR-A (Submitted software)

Class HDR-B (Submitted software)

Overall (Submitted software)

|  | Constraint Set 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |  |  |
|  | BD-rate |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| DayStreet | -7.1% | -2.5% | -2.1% | -7.0% | -9.3% | -2.0% | -7.0% | -9.6% |
| PeopleInShoppingCenter | -7.5% | -3.5% | -3.2% | -12.4% | -10.5% | -3.2% | -11.2% | -11.2% |
| SunsetBeach | -13.8% | -1.4% | -1.3% | -15.7% | -25.7% | -1.1% | -12.9% | -29.9% |
| Market | -9.4% | -4.7% | -4.1% | -13.9% | -15.9% | -2.3% | -10.8% | -11.4% |
| ShowGirl2 | -23.9% | -5.3% | -4.3% | -41.8% | -51.7% | -3.3% | -45.8% | -55.5% |
| Hurdles | -13.1% | -5.9% | -5.5% | -15.2% | -8.7% | -4.2% | -10.3% | -6.3% |
| Starting | -10.9% | -5.6% | -3.7% | -11.0% | -15.3% | -2.5% | -9.9% | -12.2% |
| Cosmos1 | -12.1% | -5.6% | -4.2% | -8.5% | -32.2% | -3.4% | -8.4% | -26.7% |
| Average HDR-A | -9.5% | -2.5% | -2.2% | -11.7% | -15.2% | -2.1% | -10.4% | -16.9% |
| Average HDR-B | -13.9% | -5.4% | -4.4% | -18.1% | -24.8% | -3.1% | -17.0% | -22.4% |
| Average all | -12.2% | -4.3% | -3.6% | -15.7% | -21.2% | -2.7% | -14.5% | -20.3% |
| Enc Time[%] | 224% |  |  |  |  |  |  |  |
| Dec Time[%] | 227% |  |  |  |  |  |  |  |
| Max Delta Rate[%] | 0.0% |  |  |  |  |  |  |  |
| Min Delta Rate[%] | -1.1% |  |  |  |  |  |  |  |
| Max Delta PSNR | 0.20 | 0.15 | 0.26 | 0.49 | 0.48 | 0.21 | 0.51 | 0.50 |
| Min Delta PNSR | 0.04 | 0.01 | 0.01 | 0.06 | 0.08 | 0.01 | 0.06 | 0.05 |

|  | Constraint Set 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |  |  |
|  | BD-PSNR |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| DayStreet | 0.06 | 0.04 | 0.07 | 0.09 | 0.11 | 0.07 | 0.10 | 0.14 |
| PeopleInShoppingCenter | 0.10 | 0.08 | 0.15 | 0.24 | 0.24 | 0.15 | 0.23 | 0.26 |
| SunsetBeach | 0.07 | 0.01 | 0.03 | 0.13 | 0.21 | 0.02 | 0.12 | 0.21 |
| Market | 0.09 | 0.08 | 0.16 | 0.18 | 0.16 | 0.09 | 0.14 | 0.11 |
| ShowGirl2 | 0.19 | 0.10 | 0.17 | 0.40 | 0.42 | 0.12 | 0.43 | 0.44 |
| Hurdles | 0.12 | 0.09 | 0.20 | 0.22 | 0.17 | 0.16 | 0.14 | 0.13 |
| Starting | 0.10 | 0.14 | 0.20 | 0.20 | 0.16 | 0.13 | 0.17 | 0.12 |
| Cosmos1 | 0.12 | 0.07 | 0.10 | 0.08 | 0.23 | 0.08 | 0.08 | 0.19 |
| Average HDR-A | 0.08 | 0.05 | 0.08 | 0.15 | 0.18 | 0.08 | 0.15 | 0.20 |
| Average HDR-B | 0.12 | 0.10 | 0.17 | 0.22 | 0.23 | 0.12 | 0.19 | 0.20 |
| Average all | 0.11 | 0.08 | 0.14 | 0.19 | 0.21 | 0.10 | 0.18 | 0.20 |

Overall (Bug-fixed software)

|  | Constraint Set 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |  |  |
|  | BD-rate |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| DayStreet | -8.4% | -3.8% | -3.4% | -8.3% | -10.6% | -3.3% | -8.2% | -11.0% |
| PeopleInShoppingCenter | -7.7% | -3.7% | -3.4% | -12.5% | -10.7% | -3.4% | -11.4% | -11.4% |
| SunsetBeach | -14.0% | -1.5% | -1.4% | -15.9% | -26.3% | -1.1% | -13.1% | -30.4% |
| Market | -9.4% | -4.7% | -4.2% | -13.7% | -15.9% | -2.3% | -10.5% | -11.4% |
| ShowGirl2 | -24.0% | -5.4% | -4.4% | -42.4% | -51.8% | -3.3% | -46.3% | -55.3% |
| Hurdles | -13.2% | -6.2% | -5.8% | -15.3% | -8.3% | -4.6% | -10.4% | -6.3% |
| Starting | -11.5% | -6.3% | -4.5% | -11.3% | -15.9% | -3.3% | -10.2% | -12.8% |
| Cosmos1 | -12.3% | -5.8% | -4.4% | -8.6% | -33.3% | -3.7% | -8.5% | -27.8% |
| Average HDR-A | -10.1% | -3.0% | -2.8% | -12.2% | -15.9% | -2.6% | -10.9% | -17.6% |
| Average HDR-B | -14.1% | -5.7% | -4.7% | -18.3% | -25.1% | -3.4% | -17.2% | -22.7% |
| Average all | -12.6% | -4.7% | -3.9% | -16.0% | -21.6% | -3.1% | -14.8% | -20.8% |
| Enc Time[%] | 260% |  |  |  |  |  |  |  |
| Dec Time[%] | 257% |  |  |  |  |  |  |  |
| Max Delta Rate[%] | 0.0% |  |  |  |  |  |  |  |
| Min Delta Rate[%] | -1.5% |  |  |  |  |  |  |  |
| Max Delta PSNR | 0.20 | 0.16 | 0.28 | 0.47 | 0.48 | 0.21 | 0.50 | 0.51 |
| Min Delta PNSR | 0.04 | 0.01 | 0.01 | 0.07 | 0.09 | 0.01 | 0.07 | 0.06 |

|  | Constraint Set 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |  |  |
|  | BD-PSNR |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| DayStreet | 0.07 | 0.07 | 0.11 | 0.10 | 0.12 | 0.12 | 0.12 | 0.16 |
| PeopleInShoppingCenter | 0.10 | 0.09 | 0.16 | 0.25 | 0.24 | 0.16 | 0.23 | 0.26 |
| SunsetBeach | 0.07 | 0.01 | 0.03 | 0.13 | 0.21 | 0.02 | 0.12 | 0.22 |
| Market | 0.09 | 0.08 | 0.17 | 0.17 | 0.16 | 0.09 | 0.13 | 0.11 |
| ShowGirl2 | 0.19 | 0.11 | 0.17 | 0.41 | 0.42 | 0.12 | 0.43 | 0.44 |
| Hurdles | 0.12 | 0.09 | 0.21 | 0.23 | 0.17 | 0.17 | 0.15 | 0.13 |
| Starting | 0.10 | 0.15 | 0.25 | 0.21 | 0.17 | 0.17 | 0.18 | 0.12 |
| Cosmos1 | 0.12 | 0.07 | 0.11 | 0.08 | 0.24 | 0.09 | 0.08 | 0.19 |
| Average HDR-A | 0.08 | 0.06 | 0.10 | 0.16 | 0.19 | 0.10 | 0.16 | 0.21 |
| Average HDR-B | 0.12 | 0.10 | 0.18 | 0.22 | 0.23 | 0.13 | 0.19 | 0.20 |
| Average all | 0.11 | 0.08 | 0.15 | 0.20 | 0.22 | 0.12 | 0.18 | 0.20 |

Decoder complexity analysis

Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

Decoding time is measured using decoder log as the same as HM anchor. JEM anchor decoding time is calculated using the HM anchor decoding time where the relative decoding time ratio is provided in JVET-I1003.

Description of computing platform used to determine decoding times reported in section 5.1

The proposed encoder and decoder software were compiled by gcc-7.2.0 and ran on the 64bit Linux computers.

Memory usage of decoder

It is confirmed that 1.9 G bytes (unit is 1024) are used to decode 4K sequences.

Complexity characteristics of decoder entropy decoding operation

The proposed modification on chroma intra prediction mode coding removes one redundant CABAC-coded bit if the forth candidate of intra prediction mode (DM4) in the derived mode list is applied to the current block.

The proposed bi-pred optimized TS requires additional derivation processes of estimated prediction accuracy map and reordering scan order. This derivation processes are conducted if and only if both bi-prediction and transform skip mode are applied to the current block.

Complexity characteristics of decoder inverse quantization

Same as JEM.

Complexity characteristics of decoder inverse transform operation

Same as JEM.

Complexity characteristics of decoder-side motion refinement

Same as JEM.

Complexity characteristics of decoder motion compensation

Same as JEM.

Complexity characteristics of decoder intra-frame prediction operation

The complexity of the proposed PDIntrafilter is almost the same as JEM other than the decision whether cubic or Gaussian intra interpolation filter is applied, which is negligible.

Complexity characteristics of decoder in-loop filtering operation

The complexity of the proposed luma-adaptive deblocking filter is almost same as HEVC and JEM other than the derivation process of luma level LL and qpOffset, which is negligible.

Complexity characteristics of additional decoder tools

Refer to JVET-J0026.

Complexity characteristics of HDR specific decoding tools

Refer to JVET-J0026.

Degree of capability for decoder parallel processing

Refer to JVET-J0026.

Encoder complexity analysis

Encoding time and measurement methodology

Decoding time is measured using encoder log as the same as HM anchor. JEM anchor encoding time is calculated using the HM anchor encoding time where the relative encoding time ratio is provided in JVET-I1003.

Description of computing platform used to determine encoding times reported in section 6.1

Memory usage of encoder

t.b.d. The description will be supplemented.

Complexity characteristics of encoder intra-frame prediction type selection

Same as JEM.

Complexity characteristics of encoder motion estimation and motion segmentation selection

General

Refer to JVET-J0026.

Complexity characteristics of usage of decoder-side motion refinement

Refer to JVET-J0026.

Complexity characteristics of encoder transforms and transform type selection

Same as JEM.

Complexity characteristics of encoder quantization and quantization type selection

Same as JEM.

Complexity characteristics of encoder in-loop filtering type selection and parameter estimation

Same as JEM.

Complexity characteristics of encoder entropy coding type selection

Same as JEM.

Complexity characteristics of additional encoder tools

Refer to JVET-J0026.

Complexity characteristics of 360° video specific encoding tools

No additional tools for 360° coding are proposed.

Complexity characteristics of HDR specific encoding tools

Refer to JVET-J0026.

Degree of capability for encoder parallel processing

Refer to JVET-J0026.

Algorithmic characteristics

Random access characteristics

Same as JEM.

Delay characteristics

Same as JEM.

Additional characteristics discussion topic(s)

None

Software implementation description

Based on JEM software (C++ implementation with SIMD code).

Highlighted aspects discussion

Closing remarks

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Annex 1: Answer for the questionnaire JEM vs. JVET-J0027

This annex is meant as an example how the questions could be answered. If some of the aspects are not applicable to an algorithm, the corresponding lines may not need to be filled. You may also add lines at the end of the table to compare additional aspects of a proposal versus HM.

Codec level Q&A

| Codec Level Q&A |  | JEM | JVET-J0027 |
| --- | --- | --- | --- |
| 1 | Maximum number of reference frames used by encoder | 4 | Same as JEM |
| 2 | Maximum coding unit (e.g. CTU in HEVC) | luma 128×128 chroma 64×64 | Same as JEM |
| 3 | Minimum and maximum transform block size | luma 4×4/128×128 chroma 2×2/64×64 | Same as JEM |
| 4 | Minimum and maximum intra prediction block | luma 4×4 chroma 2×2 | Same as JEM |
| 5 | Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length | luma 4×4 (uni/bi-pred), 8-tap Chroma 2×2 (uni/bi-pred), 4-tap | Same as JEM |
| 6 | List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category) | template matching in FRUC, LIC | Same as JEM |
| 7 | List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples, position unforeseeable) | 64-bit arithmetic for LIC, BIO and LMC | Same as JEM |
| 8 | List all the tools that use non-rectangular (e.g. triangle) partitions | n/a | Same as JEM |
| 9 | Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa) | n/a | Same as JEM |
| 10 | Indicate whether luma and chroma use separate partitioning structures | Yes, in I-slices | Yes |
| 11 | Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes) | Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation | Same as JEM |

Tool level Q&A

Category 1. Partitioning structure

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 1. Partitioning structure |  | JEM | JVET-J0027 |
| 1.a | Whether the prediction unit partitioning is different from transform unit partitioning | No | Same as JEM. |
| 1.b | Maximum tree depth | QT maximum depth 5 BT maximum depth 3 | Same as JEM. |

Category 2. Entropy coding

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 2. Entropy coding |  | JEM | JVET-J0027 |
| 2.a | Total number of context models | 329 | N = 329 + M (Refer to JVET-J0026) + 1 (MVPlanar) |
| 2.b | Memory storage size for each context model | 30-bit | Same as JEM. |
| 2.c | Range table size if table look up is used for range update | LPSTable[512×64×9bits]; RenormTable[128×3bits] | LPSTable[64×4×8bits]; RenormTable[32× 3bits] |
| 2.d | Initialization table size | 3×329×8 + 329×15×5 bits | 3×N×24 + N×15×5 bits |
| 2.e | Other memory associated with context model adaptation | n/a | Same as JEM. |

Category 3. Control data derivation process

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 3. Control data derivation process |  | JEM | JVET-J0027 |
| 3.a | Temporal motion data storage granularity | 4×4 | Same as JEM. |
| 3.b | Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-directional reference picture are used, count this picture as 2) | 4 | Same as JEM. |

Category 4. Decoder-side motion refinement

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 4. Decoder-side motion refinement |  | JEM | JVET-J0027 |
| 4.a | Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement. | - BIO 2×2D separable 6 taps filter (for gradients), mults per sample in refinement calculation 5+5*4 +1/8+2, minimum processing unit same as regular bi-pred, no SAD calculation - FRUC minimum processing unit 44, sequential PU followed by sub-PU level refinement, number of SAD calculations and additional MC interpolations per unit for MV refinement is not constrained (refinement is terminated when cost no longer decreases); bi-linear filter at MV refinement stage | Same as JEM. |
| 4.b | Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size. | - DMVR needs to access (H+7+2)×(W+7+2) MC instead of (H+7)×(W+7) in HEVC MC - FRUC not constrained | Same as JEM. |

Category 5. Intra prediction

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 5. Intra prediction |  | JEM | JVET-J0027 |
| 5.a | Line buffer size in terms of luma and chroma sample lines | luma 4, chroma 2 | Same as JEM. |
| 5.b | List all the filters used for intra prediction | Reference sample smoothing: {1,2,1}, {3,7,14,16,14,7,3}, {4,9,12,14,12,9,4} Interpolation: 2×4×32×8 bits (4 taps, 2 sets, 32 phase) | Same as JEM. |
| 5.c | Indicate whether there are cross-component dependencies in intra prediction reconstruction | LM-CHROMA (Cb depends on Y, and Cr depends on Cb&Y) | Same as JEM. |

Category 6. Inter prediction

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 6. Inter prediction |  | JEM | JVET-J0027 |
| 6.a | Motion vector accuracy | luma 1/16 pel chroma 1/32 pel | Same as JEM. |
| 6.b | Storage size for a motion vector component | 16-bit | Same as JEM. |
| 6.c | List all the interpolation filters used for luma/chroma motion compensation | luma 8-tap, 8×16×6 bits chroma 4-tap, 4×32×6 bits | Same as JEM. |

Category 7. Quantization

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 7. Quantization |  | JEM | JVET-J0027 |
| 7.a | Storage size associated with quantization | n/a | n/a |

Category 8. Transforms

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 8. Transforms |  | JEM | JVET-J0027 |
| 8.a | List all the combinations of transform block size and transform type for primary transforms | HEVC transform mode (10-bit DCT-II), transform size combinations: - (4, 8, 16, 32, 64, 128) × (4, 8, 16, 32, 64, 128) TUs for luma CTUs - (2, 4, 8, 16, 32, 64) × (2, 4, 8, 16, 32, 64) TUs for inter-coded chroma CTUs - (2, 4, 8, 16, 32, 64,) × (2, 4, 8, 16, 32, 64) TUs for intra-coded chroma CTUs EMT transform mode (10-bit DST VII, DCT-V, DCT-VIII, DST-I), only applied to CTUs up to 64×64, transform size combinations: - (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TUs for luma CTUs | Same as JEM. |
| 8.b | List all the combinations of transform block size and transform type for secondary transforms | 35×3 4×4 and 8×8 non-separable Hypercube-Givens Transform (HyGT) | Same as JEM. |
| 8.c | List all the non-separable transforms used | 4×4/8×8 HyGT | Same as JEM. |
| 8.d | Indicate whether transforms can be fully/partially implemented using direct matrix multiply | factorized form not equivalent to matrix multiplication | Same as JEM. |
| 8.e | List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output) | 4×4 HyGT 2-interations, 8×8 HyGT 4-interations | Same as JEM. |

Category 9. In-loop filtering

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 9. In-loop filtering |  | JEM | JVET-J0027 |
| 9.a | List line buffer size in terms of luma and chroma lines (if any) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) ALF (luma 8, chroma 4) | Same as JEM. |
| 9.b | List all the filters used for in-loop filtering | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. ALF: 9×9/7×7/5×5 diamond filter support for luma; and 5×5 diamond filter support for chroma | Same as JEM. |
| 9.c | Minimum processing unit size | Deblocking (luma 4×4 grid with 4×4 filter decision, chroma 8×8 grid with 2×2 filter decision) ALF: for luma, filter decision is based on 2×2 block Bilateral filter: sample level filter switch (luma only) | Same as JEM. |

Category 10. “tool-off” and “tool-on” tests

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 10. “tool-off” and “tool-on” tests |  | JEM | JVET-J0027 |
| 10.a | List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools | QTBT, ALF, FRUC, BIO, DMVR, EMT, … See Table 2 below for an example of “tool-off” and “tool-on” tests | MVPlanar Refer to JVET-J0026 for other tools. |

“Tool-off” and “tool-on” test for MVPlanar

- (a) “Tool-off” test

The performance comparison between the bug-fixed software and the software without MVPlanar is shown in Table 12.2.10.1-1.

Table 12.2.10.1-1. Summary of “tool-off” tests for MVPlanar under CfP constraint sets

|  |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| CS1 | SDR-A | 0.4% | 0.3% | 0.4% | 84% | 106% |
|  | SDR-B | 0.2% | 0.2% | 0.2% | 119% | 97% |
| CS2 | SDR-B | 0.2% | 0.4% | 0.5% | 97% | 96% |
| CS1 | HDR-A | 0.2% | 0.1% | 0.1% | 81% | 108% |
|  | HDR-B | 0.3% | 0.5% | 0.7% | 114% | 101% |

- (b) “Tool-on” test

To show the individual improvement of MVPlanar, the performance comparison between the customized JEM software and the software with MVPlanar is described in Table 12.2.10.1-2. The customized JEM disabled the following inter prediction tools:

- Alternative temporal motion vector prediction

- Higher motion vector storage accuracy

- Local illumination compensation

- Affine motion compensation prediction

- Pattern matched motion vector derivation

- Bi-directional optical flow

- Decoder-side motion vector refinement

Table 12.2.10.1-2. Summary of “tool-on” tests for MVPlanar on JEM6.0 w/o inter prediction tools under the common test condition.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | JEM6 w/o DMVR,ATMVP,FRUC,MVPrecision,IC,BIO,AFFINE |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.3% | -0.3% | -0.2% | 96% | 99% |
| Class A2 | -0.6% | -0.3% | -0.4% | 94% | 99% |
| Class B | -0.4% | -0.3% | -0.2% | 102% | 108% |
| Class C | -0.2% | -0.3% | -0.2% | 97% | 108% |
| Class D | -0.2% | -0.1% | 0.0% | 87% | 99% |
| Class E | -0.4% | -0.2% | -0.2% | 103% | 102% |
| Overall (Ref) | -0.4% | -0.3% | -0.2% | 96% | 103% |
| Class F(Optional) | -0.1% | -0.2% | -0.2% | 99% | 101% |

[end of document]