| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0028 |
| --- | --- |

| Title: | Description of SDR and HDR video coding technology proposal by Sony |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Call for Proposals |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki Masaru Ikeda Karl Sharman Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2782 teruhiko.s@sony.com masaru.ikeda@sony.com karl.sharman@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution presents a description of SDR and HDR video coding technology proposal by Sony which is in response to the Joint CfP on Video Compression with Capability beyond HEVC. It reports a coding performance and both encoder and decoder complexity. The proposed techniques were developed on top of JEM and the codec design is common between SDR and HDR. The proposed techniques address six categories of video coding tool which are entropy coding, transform/quantization, motion compensation, intra prediction, in-loop filtering and partitioning structure. There is no use of pre-processing outside the codec and no specific optimizing encoding parameters using non-automatic means (e.g. on per sequence) in both SDR and HDR. Quantization settings are kept static except for a one-time change of the settings to meet the target bit rate. The contribution reports　a coding gain for Y, U and V, on average, of 2.41%, 4.85% and 5.1%, and 2.25%, 6.74% and 7.34% over JEM at SDR constraint set 1 and 2, respectively. For HDR, it reports a coding gain for Y, U and V, on average, of 2.35%, 5.14% and 7.73%, and 1.78%, 6.69% and 8.89% over JEM at HDR-A and HDR-B constraint set 1, respectively.

- Contents

Abstract	i

Contents	ii

1	Introduction	1

2	Decoder algorithm description	1

2.1	General algorithm description	1

2.1.1	Bitstream structure	1

2.1.2	Entropy decoding	1

2.1.3	Conversion process from entropy decoded data to symbols	1

2.1.4	Decoder-side motion refinement	1

2.1.5	Inverse quantization	1

2.1.6	Inverse transforms	2

2.1.7	Motion compensation	2

2.1.8	Intra prediction	2

2.1.9	In-loop filtering	2

2.1.10	Additional tools	2

2.1.11	Additional algorithmic description topic(s)	2

2.2	360° Video	2

2.2.1	Projection format (including padding)	2

2.2.2	Quantization handling	2

2.2.3	360° video specific decoding tools	2

2.3	HDR	2

2.3.1	Post-processing	2

2.3.2	Quantization handling	2

2.3.3	HDR specific decoding tools	2

3	Encoder algorithm description	3

3.1	General algorithm description	3

3.1.1	Rate-distortion optimization	3

3.1.2	Partitioning structure selection	3

3.1.3	Intra prediction mode estimation and type selection	3

3.1.4	Motion estimation and motion segment selection	3

3.1.5	Mode decisions	3

3.1.6	In-loop filtering type selection and parameter estimation	3

3.1.7	Transforms and transform type selection	3

3.1.8	Quantization and quantization type selection	3

3.1.9	Entropy coding	3

3.1.10	Additional tools	3

3.1.11	Additional encoder optimization	4

3.1.12	Additional algorithmic description topic(s)	4

3.2	360°	4

3.2.1	Usage of projection format adaptation	4

3.2.2	360° Video specific encoding tools	4

3.2.3	Additional encoder optimization	4

3.3	HDR	4

3.3.1	Pre-processing	4

3.3.2	HDR specific encoding tools	4

3.3.3	Additional encoder optimization	4

4	Compression performance	4

4.1	SDR: Constraint set 1 configuration relative to HM anchor	4

4.1.1	Class SDR-A (RD-curves)	4

4.1.2	Class SDR-B (RD-curves)	4

4.1.3	Overall (BD-rate summary tables)	5

4.2	SDR: Constraint set 2 configuration relative to HM anchor	5

4.2.1	Class SDR-B	5

4.2.2	Overall	5

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	5

4.3.1	Class SDR-A	5

4.3.2	Class SDR-B	5

4.3.3	Overall	5

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	5

4.4.1	Class SDR-B	5

4.4.2	Overall	5

4.5	HDR: Constraint set 1 configuration relative to HM anchor	5

4.5.1	Class HDR-A	5

4.5.2	Class HDR-B	5

4.5.3	Overall	5

4.6	HDR: Constraint set 1 configuration relative to JEM anchor	6

4.6.1	Class HDR-A	6

4.6.2	Class HDR-B	6

4.6.3	Overall	6

4.7	360° Video: Constraint set 1 configuration relative to HM anchor	6

4.7.1	Class 360	6

4.7.2	Overall	6

4.8	360° Video: Constraint set 1 configuration relative to JEM anchor	6

4.8.1	Class 360	6

4.8.2	Overall	6

5	Decoder complexity analysis	6

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	6

5.2	Description of computing platform used to determine decoding times reported in section 5.1	6

5.3	Memory usage of decoder	7

5.4	Complexity characteristics of decoder entropy decoding operation	7

5.5	Complexity characteristics of decoder inverse quantization	7

5.6	Complexity characteristics of decoder inverse transform operation	7

5.7	Complexity characteristics of decoder-side motion refinement	7

5.8	Complexity characteristics of decoder motion compensation	7

5.9	Complexity characteristics of decoder intra-frame prediction operation	7

5.10	Complexity characteristics of decoder in-loop filtering operation	7

5.11	Complexity characteristics of additional decoder tools	7

5.12	Complexity characteristics of 360° video specific decoding tools	7

5.13	Complexity characteristics of HDR specific decoding tools	7

5.14	Degree of capability for decoder parallel processing	7

6	Encoder complexity analysis	8

6.1	Encoding time and measurement methodology	8

6.2	Description of computing platform used to determine encoding times reported in section 6.1	8

6.3	Memory usage of encoder	8

6.4	Complexity characteristics of encoder intra-frame prediction type selection	8

6.5	Complexity characteristics of encoder motion estimation and motion segmentation selection	8

6.5.1	General	8

6.5.2	Complexity characteristics of usage of decoder-side motion refinement	8

6.6	Complexity characteristics of encoder transforms and transform type selection	8

6.7	Complexity characteristics of encoder quantization and quantization type selection	8

6.8	Complexity characteristics of encoder in-loop filtering type selection and parameter estimation	9

6.9	Complexity characteristics of encoder entropy coding type selection	9

6.10	Complexity characteristics of additional encoder tools	9

6.11	Complexity characteristics of 360° video specific encoding tools	9

6.12	Complexity characteristics of HDR specific encoding tools	9

6.13	Degree of capability for encoder parallel processing	9

7	Algorithmic characteristics	9

7.1	Random access characteristics	9

7.2	Delay characteristics	9

7.3	Additional characteristics discussion topic(s)	9

8	Software implementation description	9

9	Highlighted aspects discussion	9

10	Closing remarks	9

11	Patent rights declaration(s)	9

12	Annex 1: Questionnaire	10

12.1	Codec level questions	10

12.2	Tool level questions	11

13	Annex 2: Example of filling questionnaire HM vs. JEM	14

13.1	Codec level Q&A	14

13.2	Tool level Q&A	15

13.2.1	Category 1. Partitioning structure	15

13.2.2	Category 2. Entropy coding	15

13.2.3	Category 3. Control data derivation process	15

13.2.4	Category 4. Decoder-side motion refinement	16

13.2.5	Category 5. Intra prediction	16

13.2.6	Category 6. Inter prediction	17

13.2.7	Category 7. Quantization	17

13.2.8	Category 8. Transforms	17

13.2.9	Category 9. In-loop filtering	18

13.2.10	Category 10. “tool-off” and “tool-on” tests	19

Introduction

This contribution provides a description of SDR and HDR video coding technology proposal by Sony and reports a coding performance and both encoder and decoder complexity of proposed techniques. The codec design is common between SDR and HDR and the detailed algorithm for decoder and encoder is described in sections 2 and 3, respectively. The compression performance is reported in section 4. The encoder and decoder complexity is described in sections 5, 6 and annex.

Decoder algorithm description

General algorithm description

Bitstream structure

Same as for JEM

Entropy decoding

Entropy coding is as per JEM7.1, baring the syntax changes required by the respective tools described in this document. There is also one entropy coding tool that has been added: sign bit prediction.

In the spatial domain, the values in the residual of inter blocks is, in general, distributed evenly in blocks. Conversely, for intra blocks, the energy is typically higher in residual samples further from the reference samples, (i.e. towards the bottom right of a block) than in those nearer the reference samples (i.e. towards the top left of a block). This has implications in the transform/frequency domain, with some correlation between signs of neighbouring coefficients.

Sign prediction uses the neighbourhood of previously coded coefficients around the coefficient being coded, in a similar method to that used for calculating contexts for the significance map. In some circumstances, the sign bit of X is also dependent on its position (x,y) with transformed block. The neighbourhood is defined in Figure 1 below, around the current coefficient X:

Figure 1 – Neighbourhood of previously coded coefficients around X (the current coefficient).

Rules have been developed for transform skip intra/inter blocks and for DCT coded inter blocks. Rules for DCT coded intra coded blocks have not yet been formalised, as the emphasis has to this point been on inter coding. In general the tool operates better when NSST is disabled.

The rules for transform skip (intra or inter) blocks are as follows:

Let Right = previously decoded coefficient A (if it is not outside the current block);
		0 otherwise.

Let Below = previously decoded coefficient C (if it is not outside the current block); 
		0 otherwise.

Let BelowRight = previously decoded coefficient D (if it is not outside the current block);
		0 otherwise

Then locate the value of predSign and subContext from Table 1, where sgn(x) returns the sign of x, that being -1 for negative values of x, 0 for x=0 and 1 for positive values of x.

Table 1 – Derivation of predicted sign and subContext for transform skipped blocks.

| Neighbourhood condition |  |  | Predicted sign, predSign | subContext |
| --- | --- | --- | --- | --- |
| sgn(Right) | sgn(Below) | Additional condition |  |  |
| ±1 | Sgn(Right) | - | sgn(Right) | 0 |
| ±1 | 0 | - | sgn(Right) | 1 |
| 0 | ±1 | - | sgn(Below) | 1 |
| ±1 | -Sgn(Right) | BelowRight!=0 | -sgn(BelowRight) | 2 |
|  |  | Otherwise | Do not predict. | Use EP bin |
| 0 | 0 | - | Do not predict. | Use EP bin |

For DCT coded inter blocks, the rules are as follows:

Let Right = value of previously decoded coefficient B (if it is not outside the current block)
 				for DCThorxDCTver or DCThorxDSTver blocks;
		0 otherwise.

Let Below = value of previously decoded coefficient E (if it is not outside the current block)
 				for DCThorxDCTver or DSThorxDCTver blocks
		0 otherwise.

Then locate the value of predSign and subContext from Table 2.

Table 2 – Derivation of predicted sign and subContext for transformed Inter-predicted blocks.

| Neighbourhood condition |  |  | Predicted sign, predSign | subContext |
| --- | --- | --- | --- | --- |
| sgn(Right) | sgn(Below) | Additional condition |  |  |
| ±1 | Sgn(Right) | - | -sgn(Right) | 6 |
| ±1 | 0 | - | -sgn(Right) | 7 |
| 0 | ±1 | - | -sgn(Below) | 7 |
| ±1 | -Sgn(Right) | abs(Right)>abs(Below) | -sgn(Right) | 8 |
|  |  | Otherwise | -sgn(Below) | 8 |
| 0 | 0 | y==0 && (x&1)!=0 && (2x≥width && DCThor | -1 | 9 |
|  |  | x==0 && (y&1)!=0 && (2y≥heigth && DCTver | -1 | 10 |
|  |  | Otherwise | Do not predict. | Use EP bin |

In JEM7, sign bits within each coefficient group are coded together, after their full magnitudes are coded. With the sign prediction tool, this process is the same, except that instead of only using EP bins, if a sign can be predicted, a CABAC context-coded bin would be used. If that CABAC context-coded bin is a 1 then the sign bit matches the prediction, predSign; if the CABAC context-coded bin is a 0, then the sign bit is the opposite of the prediction predSign.

The CABAC context to use for the coding is derived from the subContext value indicated in the above tables:

contextIndex = subContext + (x!=0||y!=0 ? 3:0)

There are 14 contexts for luma and 14 contexts for chroma in the design. These are initialised with the value of 0x9b (which equates to Probability(1)=67%), with the exception of the transform skip contexts, which have yet to be optimised and some have been initialised to 0x9a (which equates to Probability(1)=50%).

Finally, in order to realise the sign bit prediction tool, some syntax changes are required:

- Sign bit data hiding in JEM7.1 hides the lowest frequency coefficient’s sign. However, this is the most predictable sign. Hence, this has been modified to hide the highest frequency coefficient’s sign.

- Adaptive/Explicit Multi Transform (AMT/EMT) signalling in JEM7.1 is after a block has been coded, as it is dependent on the number of coefficients present. In this design, it has been modified to be dependent on the lastXY (or more specifically the scanPosLast being greater than 2) position instead, and it is coded prior to any coefficients.

- Although NSST is currently not used in the above description, the coding has been modified for NSST, so that in the future intra can also be coded using sign prediction. As with AMT/EMT, NSST signalling has been modified so that it is dependent on scanPosLast being greater than 2, rather than the number of coefficients present in the block.

Conversion process from entropy decoded data to symbols

Intra prediction parameters

Not in particular

Motion parameters

Not in particular

Quantization parameters

Not in particular

Transform coefficients

Not in particular

In-loop filter parameters

Not in particular

Decoder-side motion refinement

Same as for JEM

Inverse quantization

Please refer to 2.1.6.3 Adaptive Scaling for Transform and Quantization.

Inverse transforms

Adaptive Multiple core Transforms for Luma and Chroma

In JEM, Adaptive Multiple core Transforms, called AMT, is used as first transform for luma while DCT2 is always used for chroma. AMT can adapt the different residual statistics using multiple transforms so that applying it for chroma is also expected to improve chroma coding efficiency. Thus, our proposal is to apply AMT for both luma and chroma as shown in Figure 2-2.

Figure 2-2 AMT extension for chroma

[Transform type selection]

Transform type for horizontal and vertical for luma is determined by eq. (A1) based on a transform set given by Table 2-3 and transform candidate per transform set given by Table 2-4, and additionally signalled index per components. To improve luma coding efficiency, # of transform candidate is set to 9.

trTypeH = TransformCandidate [ mode ] [ index ] [ H ]

trTypeV = TransformCandidate [ mode ] [ index ] [ V ]                      - (A1)

For chroma/intra case, transform type is also determined by eq. (A1), where # of transform candidate is set to 3 in order to reduce signaling cost.

For chroma/inter case, transform type is set to DCT2 (index==0) or inferred from luma’s transform type of the collocated block (index==1), which results in # of max transform candidate is limited to 2.
For luma/intra&inter and chroma/intra case, # of transform candidate is optionally controlled in a certain parameter set. In addition, on/off flag of chroma AMT is optionally controlled in a certain parameter set.

Table 2-3 Transform set given by prediction mode

Table 2-4 Transform Candidate per Transform Set

Transform basis replacement

In this proposal, in order to reduce the number of unique transforms, the following transforms used in JEM are replaced with alternate transforms as specified in Table 2-5:

Table 2-5 Transform basis replacement

| Replaced Transform | Alternate transform |
| --- | --- |
| DCT5 | DCT2 |
| DCT8 | Flipped DST7 (called FlipDST7) |
| DST1 | Transposed DST7 (called TrDST7, identical to DST6) |

Consequently, there are only two unique transforms: DCT2 and DST7. Alternate transforms FlipDST7 and TrDST7 are easily derived from DST7 as below.

FlipDST7:  FlipSN7[l,k]=SN7[l, N-k-1]   - (A2)

TrDST7:  TrSN7[l,k]=SN7[k,l]    - (A3)

, where the l-th transform coefficients of k-th basis vector of DST7 and DCT2 are defined as below:

DCT2: CN2k,l=round2m22Nδkcosl+12kπN ,     (l,k=0,1,…,N-1)  - (A4)

DST7:  SN7k,l=round2m22N+1sin2l+1k+12π2N+1,    (l,k=0,1,…,N-1)  - (A5)

δk=12     for k=01       othwerwise      , m: same as for JEM

Figure 2-3 shows the basis vectors of DCT2/DST7/FlipDST7/TrDST7 with N=8, respectively.

Figure 2-3 Basis vectors of DCT2, DST7, FlipDST7, and TrDST7

Adaptive Scaling for Transform and Quantization

In JEM/HEVC, the calculation precision of both inverse Quantization and inverse transform depends on Input sequence’s bit depth called channelBitDepth.

For 10 bit sequence, its residuals are inverse quantized and inverse transformed assuming that the dynamic range of abs(its residuals) is 10 bit (=channelBitDepth). However, there are sometimes signals with narrower ranges than the assumed input sequence’s bit depth: Some HLG and PQ sequences have narrower range chroma signals than 10 bit. In such a case, there is room to reduce rounding error by extending calculation precision with an extra scaling factor deltaBD at low QPs.

The concept of Inverse quantization and inverse transform with extra scaling factor deltaBD is summarized as below:

- A) Inverse Quantization of quantized coefficient X1 and clipping

  - X2 = clip3(minCoef, maxCoef, invQScale*X1 << (invQShift – deltaBD) )

- B) Inverse vertical transform of the input inversed quantized coefficients X2

  - X3 = Tv . X2

- C) Intermediate scaling and clipping

  - X4 = clip3( minCoef, maxCoef, (X3>>invShift1))

- D) Inverse Horizontal transform of the matrix X4

  - X5 = X4 . ThT

- E) Final Scaling

  - Y = X5 >> (invShift2 + deltaBD)

, where variables are defined as below:

  - invQScale: a scaling parameter at inverse Quantization

  - invQShift: a shift parameter at inverse Quantization

  - invShift1: a shift parameter at inverse vertical transform

  - invShift2: a shift parameter at inverse horizontal transform

  - Tv : a certain vertical transform matrix

  - Th : a certain horizontal transform matrix

  - minCoef: min value of transform coefficient

  - maxCoef: max value of transform coefficient

As can be seen, coefficient precision is virtually extended by deltaBD from A) through D). Finally, extended coefficient precision is cancelled out at E). Note that rounding offset is omitted here to simplify the explanation.

An extra scaling factor deltaBD is derived using slice-level min/max pixel value signalled in Adaptive Clipping as follow:

- Max residual signal maxResi = maxPixelSignalled - minPixelSignalled

- Min residual signal minResi = minPixelSignalled - maxPixelSignalled

- Dynamic range of residual signals log2ResiRange = ceil( log2(max( maxResi, abs(minResi)))

= ceil( log2( maxPixelSignalled – minPixelSignalled) )

- An extra scaling factor deltaBD = channelBitDepth – log2ResiRange

An on/off flag for adaptive scaling is signalled in SPS.

Motion compensation

Motion model extension

In JEM software, the simplified affine transform is used in motion compensation. It can improve the quality of inter prediction but it increases the overhead as a motion information. To reduce the overhead and find better trade-off, this technique adds two motion modes which are a rotation and a scaling.

An encoder can choose four modes which are a conventional translate, scaling, rotation and Affine. A number of parameters are 2 (one motion vector), 3, 3 and 4 (two MVs) for translate mode, scaling mode, rotation mode and Affine mode.

As shown in the below figure, the motion model index is defined and it is sent to the decoder. The number in parenthesises shows the number of parameters of differential motion vector that are sent to a decoder.

Optionally change sub-block size

In these scaling, rotation and Affine mode, the inter prediction process is divided to sub-blocks. If smaller sub-blocks are used, it can improve the accuracy but the complexity will be larger. To have flexibility of the trade-off, this technique allows the sub-block size to change among 2x2, 4x4 or 8x8. The control index is sent in sequence parameter set.

Scaling mode

As mentioned above, a scaling mode is added in this technique. If the test sequence has zoom in or zoom out, the inter prediction needs a scaling translate. This scaling mode can predict the zoom in/out and it can reduce the number of parameters of Affine mode.

When the scaling mode is chosen, three parameters are used to predict as shown in the below figure. It is one motion vector (v0x, v0y) and one element (v1x). For the prediction process, the frame work is used the same as for Affine transform in JEM software. The parameter (v1y) is implicitly assumed as the same as (v0y).

Rotation mode

As mentioned above, a rotation mode is added in this technique. If the motion in a test sequence has rotation, the inter prediction can use a rotational translate. This rotation mode can predict the rotation and it can reduce the number of parameters than Affine mode.

When the rotation mode is chosen, three parameters are used to predict as shown in the below figure. It is one motion vector (v0x, v0y) and one element (v1y). For the prediction process, the frame work is used the same as for Affine transform in JEM software. The parameter (v1x) is implicitly calculated by below equation. Here, W is width in CU size.
v1x=v0x-v1y-v0y22W

Intra prediction

Use of multiple reference samples

In JEM, a single row or column of reference samples is projected at a range of angles (65) to generate a range of angular predictions.  The use of two or more rows (or columns) of samples can provide a better angular prediction in most cases.  In the diagram below the blue arrow shows how a reference sample is projected to generate a predicted sample:

Where the light blue squares are reference samples and the white squares are samples to be predicted.

In addition, a second row of reference samples is used to make an alternate angular prediction value, shown below by the green arrow:

These two prediction values are then combined by simple averaging to generate a prediction value that is less affected by noise in the reference samples.

The second prediction value (green arrow) may be less accurate then the first (blue arrow) as it is derived from samples spatially further away.  This is most pronounced in small blocks so use of the second prediction values is avoided for small blocks.

The second prediction value is also unused for blocks at the top of the CTU as the second row of reference samples would not be available in a hardware implementation without requiring significant extra storage (line stores etc.).

PDPC-Planar modification

In JEM, the PDPC planar tool applies a blend of the unfiltered reference samples with the sample planar-predicted from the filtered reference samples, with the amount of blend determined by the distance from the reference sample. For large blocks (those with a dimension greater than or equal to 64), a banding was noticed in the prediction. This was found to be due to a logical shift of greater than 32 being applied to a 32-bit integer in the software. To resolve this issue the derivations of Coeff_Top and Coeff_Left have been modified so that a 0 weighting is used when shiftRow or shiftCol respectively exceed 31.

In-loop filtering

Deblocking Filter extension

In JEM, both luma and chroma design of deblocking filter is the same as for HEVC. The extended filter for both luma and chroma can provide a better performance which is not only subjectively but also objectively improved. Proposed wide strong filter for luma component is applied to seven samples in each side at the block boundary. The filter equation for one side is as shown below.

p6′= (7*p7+2*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (6*p7+p6+2*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (5*p7+p6+p5+2*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (4*p7+p6+p5+p4+2*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (3*p7+p6+p5+p4+p3+2*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (2*p7+p6+p5+p4+p3+p2+2*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

The decision which has two additional conditions is also modified, accordingly. The first one is for large block decision since the proposed filter can be applied when the block width or height which orthogonally crosses the block edge is equal to or larger than 32. The second one is for wide strong filter decision. It is extended based on strong filter decision.

Condition 1:

If( EDGE_VER && block_width>=32) ¦¦ (EDGE_HOR && block_height>=32 )

Conditon 2:

xUseStrongFiltering4LargeBlock( Pel *R, beta, tc ) {

dp0 = xCalcDP( R[0] ); dp1 = xCalcDP( R[-2] ); dp2 = xCalcDP( R[-4] );

dq0 = xCalcDQ( R[0] ); dq1 = xCalcDQ( R[2] ); dq2 = xCalcDQ( R[4] );

dd0 = 2 * (dp0+dq0); dd2 = 2 * (dp1+dq1); dd4 = 2 * (dp2+dq2);

bdd = ( dd0 < (beta>>2) && dd2 < (beta>>2) && dd4 < (beta>>2) );

dd_strong0 = abs(R[-8] – R[-1]) + abs(R[0] – R[7]);

dd_strong1 = abs(R[-8] – R[-3]) + abs(R[2] – R[7]);

dd_strong2 = abs(R[-8] – R[-5]) + abs(R[4] – R[7]);

dd_strong_flag = ( (dd_strong0<(beta>>3)) && (dd_strong0<(beta>>3)) && (dd_strong0<(beta>>3)) );

return ( dd_strong_flag && bdd && (abs(R[-1]-R[0]) < ((tc*5+1)>>1)) );

}

For chroma component, proposed wide filter is applied to three samples in each side at the block boundary. The filter equation for one side is as follows.

p2′= (3*p3+2*p2+p1+p0+q0+4) >> 3

p1′= (2*p3+p2+2*p1+p0+q0+q1+4) >> 3

p0′= (p3+p2+p1+2*p0+q0+q1+q2+4) >> 3

The decision which has three additional conditions is also modified, accordingly. The first one is for decision of boundary strength as well as large block. The proposed filter can be applied when the block width or height which orthogonally crosses the block edge is larger than 16. The second and third one are basically similar to luma’s decision, which are on/off decision and strong filter decision, respectively.

Condition 1:

If(bS==2 ¦¦ (bS==1 && Large block decision#1))

#1: (EDGE_VER && block_width>TH2) ¦¦ (EDGE_HOR && block_height>TH2)

Condition 2:

d < beta

d = Abs(p2,0−2*p1,0+ p0,0) + Abs( q2,0−2*q1,0+q0,0) + Abs(p2,1−2*p1,1+p0,1) + Abs(q2,1−2*q1,1+q0,1);

Condition 3:

xUseStrongFiltering( ); // same as luma’s decision

Adaptive Loop Filter modification

In JEM, each 2x2 block for the luma component is classified based on its directionality and 2D Laplacian activity. Proposed gradient feature can provide a better classification for the ALF. Values of horizontal and vertical gradients are calculated using Sobel filter:

∂∂x=-101-202-101    ∂∂y=-1-2-1000121

The representative vector is calculated so that a given condition below is satisfied for gradient vectors within a certain window size (e.g. 6x6). According to θ, the direction and transformation are identified.

argmaxθJθ=argmaxθieθ, ti=argmaxθiti2cosθ-∅i2

Then the similarity between the representative vector and each gradient vector within the window are calculated using the inner product.

S=ieθ, ti

If S >th1 , D is set to 0.

If θ ∈D0 or D1 and S <th2 , D is set to 2, otherwise D is set to 1.

If θ ∈V or H and S <th2 , D is set to 4, otherwise D is set to 3.

The total number of class which is the same as for JEM is twenty-five.

Additional tools

Nothing in particular

Additional algorithmic description topic(s)

Nothing in particular

360° Video

Projection format (including padding)

N/A

Quantization handling

N/A

360° video specific decoding tools

N/A

HDR

Post-processing

No use of post-processing.

Quantization handling

No change of quantization handling (same as for JEM).

HDR specific decoding tools

No use of HDR specific decoding tools.

Encoder algorithm description

General algorithm description

Rate-distortion optimization

Distortion metrics

Same as for JEM

Rate-distortion dependency models

Same as for JEM

Partitioning structure selection

For inter slice, CTU size changes to 256x256. And max BT depth changes to 5. But the maximum of transform size keeps with 128 the same as JEM. If the CU size is larger than 128, the residual coding is implicitly disabled to avoid usage of large transforms beyond 128.

Intra prediction mode estimation and type selection

This is the same as JEM but with the use of two rows/columns of reference samples as described in 2.1.8.1

Motion estimation and motion segment selection

General

Same as for JEM

Usage of decoder-side motion refinement

Same as for JEM

Mode decisions

Same as for JEM

In-loop filtering type selection and parameter estimation

The parameter estimation is the same as for JEM. For filtering type selection, refer to 2.1.9.

Transforms and transform type selection

The transform encoding is basically the same as JEM, except primary transform type selection. As described in 2.1.6.1, # of transform candidates is changed as below:

| Component | PredMode | # of transform candidates |
| --- | --- | --- |
| Luma | Intra | set to 9 |
| Luma | Inter | set to 9 |
| Chroma | Intra | set to 3 |
| Chroma | Inter | 2 |

The best transform type is selected from the transform candidates based on Rate-Distortion Optimization.

Quantization and quantization type selection

Same as for JEM

Entropy coding

The entropy encoding is the same as JEM, except for the coding of additional control symbols, and also the use of context-coded variables to code sign bits instead of using equi-probable bins.

Additional tools

Nothing in particular

Additional encoder optimization

There is no non-automatic means of encoding parameter optimization, multi-pass encoding, content-adaptive GOP and special subjective optimization.

Additional algorithmic description topic(s)

Nothing in particular

360°

Usage of projection format adaptation

N/A

360° Video specific encoding tools

N/A

Additional encoder optimization

N/A

HDR

Pre-processing

No use of pre-processing.

HDR specific encoding tools

No use of HDR specific encoding tools.

Additional encoder optimization

No use of additional encoder optimization.

Compression performance

SDR: Constraint set 1 configuration relative to HM anchor

Class SDR-A (RD-curves)

The RD-curves of Y, U and V for SDR-A sequences at CS1 are shown below, respectively.

FoodMarket4

CatRobot1

DaylightRoad2

ParkRunnning3

CampfireParty2

Class SDR-B (RD-curves)

The RD-curves of Y, U and V for SDR-B sequences at CS1 are shown below, respectively.

BQTerrace

RitualDance

MarketPlace

BasketballDrive

Cactus

Overall (BD-rate summary tables)

The BD-rate for SDR-A and SDR-B sequences at CS1 are summarized below.

|  | Constraint Set 1 |  |  |
| --- | --- | --- | --- |
|  | Over HM |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| FoodMarket4 | -34.68% | -45.90% | -49.51% |
| CatRobot1 | -41.04% | -55.19% | -48.40% |
| DaylightRoad2 | -42.29% | -58.79% | -44.14% |
| ParkRunning3 | -32.08% | -27.25% | -30.22% |
| Campfire | -35.92% | -36.87% | -52.35% |
| BQTerrace | -32.69% | -47.71% | -56.38% |
| RitualDance | -28.65% | -39.56% | -42.90% |
| MarketPlace | -30.75% | -49.97% | -47.38% |
| BasketballDrive | -32.57% | -48.48% | -46.09% |
| Cactus | -36.34% | -48.34% | -44.57% |
| Average SDR-A | -37.20% | -44.80% | -44.93% |
| Average SDR-B | -32.20% | -46.81% | -47.46% |
| Average all | -34.70% | -45.81% | -46.19% |

SDR: Constraint set 2 configuration relative to HM anchor

The RD-curves of Y, U and V for SDR-B sequences at CS2 are shown below, respectively.

Class SDR-B

BQTerrace

RitualDance

MarketPlace

BasketballDrive

Cactus

Overall

The BD-rate for SDR-B sequences at CS2 is summarized below.

|  | Constraint Set 2 |  |  |
| --- | --- | --- | --- |
|  | Over HM |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| BQTerrace | -29.17% | -49.07% | -52.91% |
| RitualDance | -22.94% | -38.29% | -39.74% |
| MarketPlace | -21.78% | -49.29% | -47.78% |
| BasketballDrive | -26.89% | -48.56% | -46.60% |
| Cactus | -29.92% | -45.91% | -44.92% |
| Average SDR-B | -26.14% | -46.22% | -46.39% |

SDR: Constraint set 1 configuration relative to JEM anchor

Class SDR-A

See the RD-curves in 4.1.1.

Class SDR-B

See the RD-curves in 4.1.2.

Overall

The BD-rate for SDR-A and SDR-B sequences at CS1 are summarized below.

|  | Constraint Set 1 |  |  |
| --- | --- | --- | --- |
|  | Over JEM |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| FoodMarket4 | -2.13% | -4.09% | -5.16% |
| CatRobot1 | -2.98% | -7.08% | -5.53% |
| DaylightRoad2 | -3.37% | -4.60% | -5.51% |
| ParkRunning3 | -1.43% | -2.89% | -4.00% |
| Campfire | -2.41% | -3.35% | -5.66% |
| BQTerrace | -3.38% | -4.89% | -4.68% |
| RitualDance | -1.86% | -4.26% | -4.09% |
| MarketPlace | -2.26% | -5.50% | -5.35% |
| BasketballDrive | -2.09% | -5.82% | -4.85% |
| Cactus | -2.17% | -5.99% | -6.57% |
| Average SDR-A | -2.46% | -4.40% | -5.17% |
| Average SDR-B | -2.35% | -5.29% | -5.11% |
| Average all | -2.41% | -4.85% | -5.14% |

SDR: Constraint set 2 configuration relative to JEM anchor

Class SDR-B

See the RD-curves in 4.2.1.

Overall

The BD-rate for SDR-B sequences at CS2 is summarized below.

|  | Constraint Set 2 |  |  |
| --- | --- | --- | --- |
|  | Over JEM |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| BQTerrace | -4.14% | -4.36% | -4.06% |
| RitualDance | -1.60% | -6.28% | -6.70% |
| MarketPlace | -1.40% | -11.40% | -11.33% |
| BasketballDrive | -1.74% | -5.36% | -4.96% |
| Cactus | -2.37% | -6.30% | -9.67% |
| Average SDR-B | -2.25% | -6.74% | -7.34% |

HDR: Constraint set 1 configuration relative to HM anchor

The RD-curves of Y, U and V for HDR-A sequences at CS1 are shown below, respectively.

Class HDR-A

DayStreet

PeopleInShoppingCenter

SunsetBeach

Class HDR-B

The RD-curves of Y, U and V for HDR-B sequences at CS1 are shown below, respectively.

Market

ShowGirl2

Hurdles

Starting

Cosmos1

Overall

|  | Over HM |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| DayStreet | -60.79% | -38.17% | -36.80% | -57.45% | -70.32% | -35.74% | -55.97% | -68.32% |
| PeopleInShoppingCenter | -51.30% | -30.79% | -29.75% | -60.05% | -55.89% | -29.50% | -57.69% | -55.65% |
| SunsetBeach | -59.51% | -28.85% | -23.18% | -82.65% | -81.70% | -24.04% | -77.84% | -82.33% |
| Market | -39.23% | -23.06% | -24.54% | -46.54% | -45.88% | -24.75% | -40.94% | -38.73% |
| ShowGirl2 | -32.65% | -27.32% | -29.06% | -67.16% | -40.21% | -28.00% | -66.68% | -40.59% |
| Hurdles | -48.64% | -34.01% | -33.65% | -58.06% | -55.06% | -31.36% | -53.69% | -50.94% |
| Starting | -26.54% | -29.89% | -30.73% | -33.64% | -36.83% | -29.73% | -32.90% | -26.72% |
| Cosmos1 | -33.84% | -25.45% | -25.27% | -68.64% | -80.97% | -24.11% | -68.19% | -80.92% |
| Average HDR-A | -57.20% | -32.60% | -29.91% | -66.72% | -69.31% | -29.76% | -63.83% | -68.77% |
| Average HDR-B | -36.18% | -27.94% | -28.65% | -54.81% | -51.79% | -27.59% | -52.48% | -47.58% |
| Average all | -44.06% | -29.69% | -29.12% | -59.27% | -58.36% | -28.40% | -56.74% | -55.53% |

HDR: Constraint set 1 configuration relative to JEM anchor

Class HDR-A

See the RD-curves in 4.5.1.

Class HDR-B

See the RD-curves in 4.5.2.

Overall

|  | Over JEM |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| DayStreet | -6.89% | -3.03% | -2.77% | -5.29% | -7.89% | -2.60% | -5.03% | -6.83% |
| PeopleInShoppingCenter | -2.99% | -1.85% | -1.88% | -4.70% | -3.92% | -1.84% | -4.53% | -4.51% |
| SunsetBeach | -9.64% | -3.77% | -2.68% | -7.60% | -11.73% | -2.61% | -5.88% | -11.87% |
| Market | 1.06% | 2.06% | 0.73% | -0.43% | -3.31% | -1.72% | -3.05% | -6.36% |
| ShowGirl2 | -7.71% | -1.23% | -2.14% | -17.47% | -16.89% | -3.14% | -23.37% | -24.36% |
| Hurdles | 1.73% | 1.15% | -0.10% | 3.09% | 1.80% | -0.73% | 1.46% | 0.86% |
| Starting | 12.32% | 1.42% | -0.66% | 1.04% | -0.48% | -1.49% | -1.47% | -5.17% |
| Cosmos1 | -4.82% | -1.70% | -1.49% | -7.15% | -11.22% | -1.82% | -7.00% | -9.44% |
| Average HDR-A | -6.51% | -2.88% | -2.44% | -5.86% | -7.84% | -2.35% | -5.14% | -7.73% |
| Average HDR-B | 0.52% | 0.34% | -0.73% | -4.18% | -6.02% | -1.78% | -6.69% | -8.89% |
| Average all | -2.12% | -0.87% | -1.37% | -4.81% | -6.70% | -1.99% | -6.11% | -8.46% |

360° Video: Constraint set 1 configuration relative to HM anchor

Class 360

N/A

Overall

N/A

360° Video: Constraint set 1 configuration relative to JEM anchor

Class 360

N/A

Overall

N/A

Decoder complexity analysis

Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

- YUV file output enable

- It was measured by using the same method as HM16.16 and JEM software.

  - It was checked in the output log.

Decoding time for SDR Cs.1

|  |  |  | HM16.16 | Proposal | Ratio |
| --- | --- | --- | --- | --- | --- |
|  |  | Target rates (kbps) | Dec T [s] | Dec T [s] | Dec |
| SDR-A | FoodMarket4 | QP30, QPIncFrame=128 |  |  |  |
|  |  | 3800 | 104.86 | 1021.66 | 9.74 |
|  |  | 2200 | 97.91 | 971.62 | 9.92 |
|  |  | 1400 | 92.93 | 913.13 | 9.83 |
|  |  | 950 | 90.65 | 867.80 | 9.57 |
|  | CatRobot1 | QP26, QPIncFrame=288 |  |  |  |
|  |  | 6000 | 85.16 | 968.66 | 11.37 |
|  |  | 3800 | 87.12 | 1023.73 | 11.75 |
|  |  | 2400 | 76.51 | 1040.94 | 13.61 |
|  |  | 1500 | 73.72 | 1054.11 | 14.30 |
|  | DaylightRoad2 | QP26, QPIncFrame=128 |  |  |  |
|  |  | 5500 | 84.48 | 927.16 | 10.97 |
|  |  | 3500 | 79.74 | 930.28 | 11.67 |
|  |  | 2200 | 76.23 | 895.13 | 11.74 |
|  |  | 1400 | 75.72 | 831.43 | 10.98 |
|  | ParkRunning3 | QP34, QPIncFrame=168 |  |  |  |
|  |  | 10000 | 77.90 | 875.02 | 11.23 |
|  |  | 6000 | 71.68 | 893.71 | 12.47 |
|  |  | 3300 | 68.39 | 892.19 | 13.05 |
|  |  | 2000 | 66.35 | 892.22 | 13.45 |
|  | Campfire | QP28, QPIncFrame=272 |  |  |  |
|  |  | 10000 | 50.46 | 402.16 | 7.97 |
|  |  | 6000 | 45.68 | 383.28 | 8.39 |
|  |  | 3300 | 41.62 | 392.03 | 9.42 |
|  |  | 2000 | 40.39 | 411.41 | 10.19 |
| SDR-B | BQTerrace | QP30, QPIncFrame=288 |  |  |  |
|  |  | 1500 | 23.09 | 209.96 | 9.09 |
|  |  | 1000 | 22.89 | 203.58 | 8.89 |
|  |  | 600 | 22.25 | 182.84 | 8.22 |
|  |  | 400 | 21.24 | 168.86 | 7.95 |
|  | RitualDance | QP27 |  |  |  |
|  |  | 3800 | 28.96 | 231.99 | 8.01 |
|  |  | 2300 | 27.12 | 226.03 | 8.33 |
|  |  | 1500 | 24.58 | 223.42 | 9.09 |
|  |  | 900 | 23.15 | 224.88 | 9.71 |
|  | MarketPlace | QP31, QPIncFrame=288 |  |  |  |
|  |  | 2000 | 24.85 | 235.76 | 9.49 |
|  |  | 1200 | 23.68 | 224.16 | 9.47 |
|  |  | 800 | 21.62 | 209.32 | 9.68 |
|  |  | 500 | 21.54 | 192.62 | 8.94 |
|  | BasketballDrive | QP26, QPIncFrame=144 |  |  |  |
|  |  | 3500 | 24.13 | 208.59 | 8.64 |
|  |  | 2000 | 22.41 | 205.01 | 9.15 |
|  |  | 1200 | 20.68 | 203.92 | 9.86 |
|  |  | 800 | 20.33 | 203.39 | 10.00 |
|  | Cactus | QP30, QPIncFrame=456 |  |  |  |
|  |  | 2000 | 20.34 | 185.86 | 9.14 |
|  |  | 1200 | 18.72 | 179.12 | 9.57 |
|  |  | 800 | 18.64 | 165.71 | 8.89 |
|  |  | 500 | 18.11 | 161.83 | 8.94 |
|  | Average SDR-A |  | 71.95 | 787.91 | 10.95 |
|  | Average SDR-B |  | 22.26 | 201.12 | 9.03 |
|  | Average all |  | 40.02 | 398.08 | 9.95 |

Decoding time for SDR Cs.2

|  |  |  | HM16.16 | Proposal | Ratio |
| --- | --- | --- | --- | --- | --- |
|  |  | Target rates (kbps) | Dec T [s] | Dec T [s] | Dec |
| SDR-B | BQTerrace | QP31, QPIncFrame=416 |  |  |  |
|  |  | 1500 | 24.03 | 185.26 | 7.71 |
|  |  | 1000 | 23.11 | 173.91 | 7.53 |
|  |  | 600 | 21.53 | 159.01 | 7.39 |
|  |  | 400 | 20.37 | 137.85 | 6.77 |
|  | RitualDance | QP28, QPIncFrame=160 |  |  |  |
|  |  | 3800 | 29.47 | 232.92 | 7.90 |
|  |  | 2300 | 27.11 | 219.78 | 8.11 |
|  |  | 1500 | 24.82 | 212.12 | 8.55 |
|  |  | 900 | 23.49 | 205.69 | 8.76 |
|  | MarketPlace | QP32, QPIncFrame=32 |  |  |  |
|  |  | 2000 | 24.91 | 233.08 | 9.36 |
|  |  | 1200 | 23.12 | 215.73 | 9.33 |
|  |  | 800 | 22.60 | 201.50 | 8.92 |
|  |  | 500 | 21.17 | 188.57 | 8.91 |
|  | BasketballDrive | QP28, QPIncFrame=312 |  |  |  |
|  |  | 3500 | 25.34 | 204.72 | 8.08 |
|  |  | 2000 | 22.95 | 196.91 | 8.58 |
|  |  | 1200 | 20.99 | 191.35 | 9.12 |
|  |  | 800 | 19.35 | 187.28 | 9.68 |
|  | Cactus | QP30, QPIncFrame=72 |  |  |  |
|  |  | 2000 | 20.07 | 166.53 | 8.30 |
|  |  | 1200 | 18.26 | 161.93 | 8.87 |
|  |  | 800 | 17.56 | 160.47 | 9.14 |
|  |  | 500 | 16.18 | 161.05 | 9.95 |
|  | SDR-B |  | 22.09 | 187.98 | 8.51 |
|  | All |  | 22.09 | 187.98 | 8.51 |

Decoding time for HDR Cs.1

|  |  |  | HM16.16 | Proposal | Ratio |
| --- | --- | --- | --- | --- | --- |
|  |  | Target rates (kbps) | Dec T [s] | Dec T [s] | Dec |
| HDR-A | DayStreet | QP32 |  |  |  |
|  |  | 2800 | 83.67 | 886.29 | 10.59 |
|  |  | 1750 | 82.69 | 821.98 | 9.94 |
|  |  | 1100 | 81.90 | 833.31 | 10.17 |
|  |  | 680 | 85.26 | 775.90 | 9.10 |
|  | PeopleInShoppingCenter | QP33 |  |  |  |
|  |  | 2150 | 81.63 | 558.74 | 6.84 |
|  |  | 1450 | 76.23 | 539.95 | 7.08 |
|  |  | 960 | 80.18 | 563.73 | 7.03 |
|  |  | 640 | 80.68 | 529.94 | 6.57 |
|  | SunsetBeach | QP33 |  |  |  |
|  |  | 10000 | 90.93 | 991.12 | 10.90 |
|  |  | 6000 | 92.30 | 1019.84 | 11.05 |
|  |  | 3000 | 88.55 | 1067.25 | 12.05 |
|  |  | 1500 | 74.83 | 1140.43 | 15.24 |
| HDR-B | Market | QP28_switch380 |  |  |  |
|  |  | 2400 | 15.94 | 140.40 | 8.81 |
|  |  | 1500 | 15.32 | 135.72 | 8.86 |
|  |  | 900 | 14.32 | 120.93 | 8.44 |
|  |  | 600 | 13.73 | 111.65 | 8.13 |
|  | ShowGirl2 | QP23_switch120 |  |  |  |
|  |  | 1200 | 14.59 | 124.76 | 8.55 |
|  |  | 750 | 13.76 | 117.23 | 8.52 |
|  |  | 500 | 13.13 | 115.53 | 8.80 |
|  |  | 350 | 13.08 | 111.97 | 8.56 |
|  | Hurdles | QP28_switch140 |  |  |  |
|  |  | 2000 | 21.44 | 194.53 | 9.07 |
|  |  | 1500 | 21.13 | 193.98 | 9.18 |
|  |  | 800 | 19.75 | 193.97 | 9.82 |
|  |  | 450 | 18.87 | 184.35 | 9.77 |
|  | Starting | QP24_switch320 |  |  |  |
|  |  | 1400 | 19.18 | 175.50 | 9.15 |
|  |  | 850 | 18.96 | 162.15 | 8.55 |
|  |  | 600 | 18.27 | 146.81 | 8.04 |
|  |  | 450 | 18.29 | 144.39 | 7.89 |
|  | Cosmos1 | QP25_switch30 |  |  |  |
|  |  | 3000 | 10.24 | 95.84 | 9.36 |
|  |  | 1500 | 8.98 | 87.81 | 9.78 |
|  |  | 900 | 8.20 | 80.53 | 9.82 |
|  |  | 500 | 7.28 | 74.10 | 10.18 |
|  | Average HDR-A |  | 83.08 | 781.91 | 9.41 |
|  | Average HDR-B |  | 14.57 | 130.30 | 8.94 |
|  | Average all |  | 27.99 | 255.14 | 9.11 |

Description of computing platform used to determine decoding times reported in section 5.1

- OS: Linux CentOS 6.9

- Compiler: GCC (g++) 4.4.7

  - 64 bit executable

  - No use multi-thread

- Storage: Network File System

- CPU: Intel Core i7-7700K 4.2GHz

Memory usage of decoder

The number of reference frame of inter-frame prediction is the same as common test conditions in JVET. Other memory to store parameters is described in the following sub-clause if any.

Complexity characteristics of decoder entropy decoding operation

The entropy decoding process is the same as for JEM, except for the addition of the context variables required for the aforementioned tools and the decoding of coefficient sign bits using CABAC context-coded variables instead of equi-probable bins.Complexity characteristics of decoder inverse quantization

Complexity characteristics of decoder inverse quantization

The inverse quantization process is the same as JEM, except Adaptive Scaling (see 2.1.6.3), which requires an additional derivation of an extra scaling factor deltaBD using slice-level min/max of sample value at slice header.

Complexity characteristics of decoder inverse transform operation

The inverse transform operation is the same as JEM, except AMT extension (2.1.6.1 and 2.1.6.2) and Adaptive Scaling (2.1.6.3). For Adaptive Scaling, please refer to section 5.5. Chroma AMT which requires additional transform selections is introduced for both intra and inter coding.

Complexity characteristics of decoder-side motion refinement

Same as for JEM

Complexity characteristics of decoder motion compensation

The motion compensation is the same as for JEM except for affine (2.1.7) which requires additional computation for two modes introduced and small granularity, 2x2, which can be selected by additional flag in slice header

Complexity characteristics of decoder intra-frame prediction operation

This is largely the same as JEM but the use of an additional row/column of reference samples requires two predictions to be performed (except for small blocks).

Complexity characteristics of decoder in-loop filtering operation

The deblocking filter process is basically the same as for JEM, except for the addition of the wide filter for large block boundaries for both luma and chroma. The decision for the wide filter is also added, accordingly. Those filter and decision are described in detailed in 2.1.9.1.

The ALF process is the same as for JEM except for the modification of the classification for luma. The features used for the classification are slightly different, but the number of class and the filter condition which is tap size and shape is the same as for JEM.

Complexity characteristics of additional decoder tools

No use of additional decoder tools.

Complexity characteristics of 360° video specific decoding tools

N/A

Complexity characteristics of HDR specific decoding tools

No use of HDR specific decoding tools.

Degree of capability for decoder parallel processing

Degree of capability is the same as JEM.

Encoder complexity analysis

Encoding time and measurement methodology

- For Cs.1, parallel encoding was used to generate the bitstreams.

  - The total encoding time was calculated by adding the encoding time of all parall segments and then subtracting the encoding times of the first pictures

- It was measured by using the same method as HM16.16 and JEM software.

  - It was checked in the output log.

Encoding time for SDR Cs.1

|  |  |  | HM16.16 | Proposal | Ratio |
| --- | --- | --- | --- | --- | --- |
|  |  | Target rates (kbps) | Enc T [h] | Enc T [h] | Enc |
| SDR-A | FoodMarket4 | QP30, QPIncFrame=128 |  |  |  |
|  |  | 3800 | 14.40 | 439.18 | 30.49 |
|  |  | 2200 | 13.50 | 333.41 | 24.70 |
|  |  | 1400 | 12.74 | 277.40 | 21.77 |
|  |  | 950 | 12.40 | 241.08 | 19.44 |
|  | CatRobot1 | QP26, QPIncFrame=288 |  |  |  |
|  |  | 6000 | 11.55 | 364.45 | 31.56 |
|  |  | 3800 | 10.82 | 305.98 | 28.28 |
|  |  | 2400 | 10.19 | 254.71 | 24.99 |
|  |  | 1500 | 9.70 | 221.74 | 22.85 |
|  | DaylightRoad2 | QP26, QPIncFrame=128 |  |  |  |
|  |  | 5500 | 11.99 | 401.68 | 33.50 |
|  |  | 3500 | 11.20 | 327.74 | 29.25 |
|  |  | 2200 | 10.59 | 276.34 | 26.09 |
|  |  | 1400 | 10.12 | 234.95 | 23.22 |
|  | ParkRunning3 | QP34, QPIncFrame=168 |  |  |  |
|  |  | 10000 | 11.43 | 363.37 | 31.80 |
|  |  | 6000 | 10.38 | 316.93 | 30.52 |
|  |  | 3300 | 9.65 | 259.13 | 26.86 |
|  |  | 2000 | 9.38 | 248.96 | 26.54 |
|  | Campfire | QP28, QPIncFrame=272 |  |  |  |
|  |  | 10000 | 6.99 | 431.38 | 61.68 |
|  |  | 6000 | 6.29 | 362.94 | 57.70 |
|  |  | 3300 | 5.69 | 288.38 | 50.68 |
|  |  | 2000 | 5.34 | 229.42 | 42.96 |
| SDR-B | BQTerrace | QP30, QPIncFrame=288 |  |  |  |
|  |  | 1500 | 2.32 | 50.35 | 21.67 |
|  |  | 1000 | 2.27 | 43.26 | 19.08 |
|  |  | 600 | 2.20 | 36.22 | 16.44 |
|  |  | 400 | 2.18 | 32.89 | 15.11 |
|  | RitualDance | QP27 |  |  |  |
|  |  | 3800 | 3.98 | 179.41 | 45.07 |
|  |  | 2300 | 3.63 | 164.42 | 45.35 |
|  |  | 1500 | 3.36 | 145.19 | 43.26 |
|  |  | 900 | 3.09 | 118.83 | 38.48 |
|  | MarketPlace | QP31, QPIncFrame=288 |  |  |  |
|  |  | 2000 | 3.10 | 96.77 | 31.21 |
|  |  | 1200 | 2.91 | 81.16 | 27.92 |
|  |  | 800 | 2.74 | 70.74 | 25.80 |
|  |  | 500 | 2.61 | 61.25 | 23.50 |
|  | BasketballDrive | QP26, QPIncFrame=144 |  |  |  |
|  |  | 3500 | 3.10 | 121.77 | 39.34 |
|  |  | 2000 | 2.80 | 103.96 | 37.10 |
|  |  | 1200 | 2.57 | 90.51 | 35.28 |
|  |  | 800 | 2.41 | 78.14 | 32.43 |
|  | Cactus | QP30, QPIncFrame=456 |  |  |  |
|  |  | 2000 | 2.33 | 81.64 | 35.10 |
|  |  | 1200 | 2.17 | 70.08 | 32.36 |
|  |  | 800 | 2.05 | 61.34 | 29.87 |
|  |  | 500 | 1.96 | 51.35 | 26.20 |
|  | Average SDR-A |  | 9.88 | 302.28 | 30.60 |
|  | Average SDR-B |  | 2.64 | 78.17 | 29.64 |
|  | Average all |  | 5.10 | 153.71 | 30.12 |

Encoding time for SDR Cs.2

|  |  |  | HM16.16 | Proposal | Ratio |
| --- | --- | --- | --- | --- | --- |
|  |  | Target rates (kbps) | Enc T [h] | Enc T [h] | Enc |
| SDR-B | BQTerrace | QP31, QPIncFrame=416 |  |  |  |
|  |  | 1500 | 2.95 | 43.71 | 14.84 |
|  |  | 1000 | 2.87 | 37.39 | 13.04 |
|  |  | 600 | 2.76 | 33.30 | 12.08 |
|  |  | 400 | 2.66 | 30.88 | 11.62 |
|  | RitualDance | QP28, QPIncFrame=160 |  |  |  |
|  |  | 3800 | 3.98 | 133.08 | 33.46 |
|  |  | 2300 | 3.60 | 126.09 | 35.05 |
|  |  | 1500 | 3.38 | 107.95 | 31.95 |
|  |  | 900 | 3.13 | 96.04 | 30.67 |
|  | MarketPlace | QP32, QPIncFrame=32 |  |  |  |
|  |  | 2000 | 3.34 | 79.60 | 23.83 |
|  |  | 1200 | 3.11 | 71.40 | 22.97 |
|  |  | 800 | 3.00 | 61.86 | 20.63 |
|  |  | 500 | 2.86 | 52.35 | 18.29 |
|  | BasketballDrive | QP28, QPIncFrame=312 |  |  |  |
|  |  | 3500 | 3.22 | 98.40 | 30.53 |
|  |  | 2000 | 2.96 | 88.41 | 29.91 |
|  |  | 1200 | 2.68 | 73.64 | 27.50 |
|  |  | 800 | 2.58 | 65.44 | 25.40 |
|  | Cactus | QP30, QPIncFrame=72 |  |  |  |
|  |  | 2000 | 2.70 | 76.24 | 28.24 |
|  |  | 1200 | 2.49 | 61.21 | 24.62 |
|  |  | 800 | 2.37 | 57.52 | 24.27 |
|  |  | 500 | 2.26 | 49.61 | 21.92 |
|  | SDR-B |  | 2.92 | 66.66 | 22.86 |
|  | All |  | 2.92 | 66.66 | 22.86 |

Encoding time for HDR Cs.1

|  |  |  | HM16.16 | Proposal | Ratio |
| --- | --- | --- | --- | --- | --- |
|  |  | Target rates (kbps) | Enc T [h] | Enc T [h] | Enc |
| HDR-A | DayStreet | QP32 |  |  |  |
|  |  | 2800 | 9.64 | 215.24 | 22.32 |
|  |  | 1750 | 9.27 | 187.79 | 20.26 |
|  |  | 1100 | 9.01 | 160.69 | 17.84 |
|  |  | 680 | 8.85 | 142.54 | 16.11 |
|  | PeopleInShoppingCenter | QP33 |  |  |  |
|  |  | 2150 | 8.72 | 144.00 | 16.52 |
|  |  | 1450 | 8.47 | 127.78 | 15.08 |
|  |  | 960 | 8.30 | 115.40 | 13.90 |
|  |  | 640 | 8.18 | 108.63 | 13.28 |
|  | SunsetBeach | QP33 |  |  |  |
|  |  | 10000 | 10.87 | 352.14 | 32.41 |
|  |  | 6000 | 10.15 | 293.84 | 28.95 |
|  |  | 3000 | 9.33 | 222.32 | 23.84 |
|  |  | 1500 | 8.82 | 175.99 | 19.96 |
| HDR-B | Market | QP28_switch380 |  |  |  |
|  |  | 2400 | 1.51 | 30.67 | 20.28 |
|  |  | 1500 | 1.46 | 26.00 | 17.87 |
|  |  | 900 | 1.41 | 22.47 | 15.92 |
|  |  | 600 | 1.39 | 20.64 | 14.82 |
|  | ShowGirl2 | QP23_switch120 |  |  |  |
|  |  | 1200 | 1.69 | 64.41 | 38.12 |
|  |  | 750 | 1.58 | 55.81 | 35.39 |
|  |  | 500 | 1.50 | 47.47 | 31.74 |
|  |  | 350 | 1.44 | 41.68 | 28.85 |
|  | Hurdles | QP28_switch140 |  |  |  |
|  |  | 2000 | 2.31 | 63.18 | 27.33 |
|  |  | 1500 | 2.23 | 60.57 | 27.17 |
|  |  | 800 | 2.09 | 50.63 | 24.24 |
|  |  | 450 | 1.99 | 41.83 | 20.98 |
|  | Starting | QP24_switch320 |  |  |  |
|  |  | 1400 | 1.86 | 34.26 | 18.45 |
|  |  | 850 | 1.80 | 29.26 | 16.21 |
|  |  | 600 | 1.78 | 26.98 | 15.15 |
|  |  | 450 | 1.76 | 25.76 | 14.60 |
|  | Cosmos1 | QP25_switch30 |  |  |  |
|  |  | 3000 | 1.21 | 54.06 | 44.74 |
|  |  | 1500 | 1.05 | 44.38 | 42.19 |
|  |  | 900 | 0.96 | 36.51 | 38.23 |
|  |  | 500 | 0.87 | 29.00 | 33.35 |
|  | Average HDR-A |  | 9.10 | 175.68 | 19.30 |
|  | Average HDR-B |  | 1.54 | 37.92 | 24.56 |
|  | Average all |  | 3.00 | 67.38 | 22.43 |

Description of computing platform used to determine encoding times reported in section 6.1

- OS: Linux CentOS 6.9

- Compiler: GCC (g++) 4.4.7

  - 64 bit executable

  - No use multi-thread

- Storage: Network File System

- CPU: Intel Core i7-7700K 4.2GHz

Memory usage of encoder

The number of reference frames of inter-frame prediction is the same as for JEM. Other memory to store parameters is described in the following sub-clause.

Complexity characteristics of encoder intra-frame prediction type selection

A small amount of additional storage is required for the extra row/column, but restricting the use of the additional samples at the top of the CTU limits the amount of additional storage required.

Complexity characteristics of encoder motion estimation and motion segmentation selection

General

Same as for JEM

Complexity characteristics of usage of decoder-side motion refinement

Same as for JEM

Complexity characteristics of encoder transforms and transform type selection

The transform operation is the same as JEM, except AMT extension (2.1.6.1, and 2.1.6.2) and Adaptive Scaling (2.1.6.3). As described in 2.1.6.1 (3.1.7), # of transform candidates is varied depending on component and prediction mode. As a result extra RDO for transform type selection introduced is required for each component and prediction mode, respectively.

Complexity characteristics of encoder quantization and quantization type selection

Same as for JEM

Complexity characteristics of encoder in-loop filtering type selection and parameter estimation

The deblocking filter process is basically the same as for JEM, except for the addition of the wide filter for large block boundaries for both luma and chroma. The decision for the wide filter is also added, accordingly. Those filter and decision are described in detailed in 2.1.9.1.

The ALF process is the same as JEM except for the modification of the classification for luma. The features used for the classification are slightly different, but the number of class and the filter condition which is tap size and shape is the same as JEM.

Complexity characteristics of encoder entropy coding type selection

Entropy encoding is the same as for JEM, except for the addition of the coding of the context-coded variables to code the aforementioned tools, including the coding of coefficient sign bits using CABAC contexts, instead of using only equi-probable bins.

Complexity characteristics of additional encoder tools

No use of additional encoder tools.

Complexity characteristics of 360° video specific encoding tools

N/A

Complexity characteristics of HDR specific encoding tools

No use of HDR specific encoding tools.

Degree of capability for encoder parallel processing

Degree of capability is the same as JEM.

Algorithmic characteristics

Random access characteristics

Random access characteristics are the same as for JEM.

Delay characteristics

Delay characteristics are the same as common test condition in JVET. No change of GOP structure at constraint set 1 and no picture reordering between decoder processing and output at constraint set 2, and no frame-level multi-pass encoding.

Additional characteristics discussion topic(s)

Nothing in particular

Software implementation description

The decoder software submitted is implemented on top of JEM.

Highlighted aspects discussion

Nothing in particular

Closing remarks

This contribution presents a description of SDR and HDR video coding technology proposal by Sony as a candidate of starting point of next generation coding standardization. It reports a compression performance and both encoder and decoder complexity of proposed techniques which addresses six categories of coding tools on top of JEM. The codec design is common between SDR and HDR.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Annex 1: Questionnaire

Refer to Annex 2.

Codec level questions

- Maximum number of reference frames used by encoder

- Maximum coding unit (e.g. CTU in HEVC)

- Minimum and maximum transform block size

- Minimum and maximum intra prediction block

- Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length

- List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category)

- List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples)

- List all tools that use non-rectangular (e.g. triangle) partitions

- Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa)

- Indicate whether luma and chroma use separate partitioning structures

- Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes)

- If the coded picture resolution is different from the input picture resolution, list the coded picture resolution for those sequences (e.g. 360° video, input is 8K ERP, coded video is in 4K CMP converted from the 8K input)

Tool level questions

- Partitioning structure

  - Whether the prediction unit partitioning is different from transform unit partitioning

  - Maximum tree depth

- Entropy coding

  - Total number of context models

  - Memory storage size for each context model

  - Range table size if table look up is used for range update

  - Initialization table size

  - Other memory associated with context model adaptation

- Control data derivation process

  - Temporal motion data storage granularity

  - Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-predictive reference picture are used, count this picture as 2)

- Decoder-side motion refinement

  - Line buffer size in terms of motion vector block lines (assuming horizontal motion vector block size is 4 luma samples)

  - Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement

  - Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size

- Intra prediction

  - Line buffer size in terms of luma and chroma sample lines

  - List all the filters used for intra prediction

  - Indicate whether there are cross-component dependencies in intra prediction reconstruction

- Inter prediction

  - Motion vector accuracy

  - Storage size for a motion vector component

  - List all the interpolation filters used for luma/chroma motion compensation

- Quantization

  - Storage size associated with quantization

- Transforms

- List all the combinations of transform block size and transform type for primary transforms

- List all the combinations of transform block size and transform type for secondary transforms

- List all the non-separable transforms used

- Indicate whether transforms can be fully/partially implemented using direct matrix multiply

- List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output)

- In-loop filtering

- List line buffer size in terms of luma and chroma lines (if any)

- List all the filters used for in-loop filtering

- Minimum processing unit size

- “tool-off” and “tool-on” tests

- List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools (see Table 1 below for test procedure)

Table 1. Test procedure of “tool-off” and “tool-on” tests for a CfP response

| step | “tool-off” test | “tool-on” test |
| --- | --- | --- |
| 1 | Identify what you consider major tools in a CfP response that contribute to compression efficiency gain, and create a test candidates list |  |
| 2 | Turn on all the tools in the test candidates list, run the CfP response software and compute PSNRs, bit rates, and run times | Turn off all the tools in the test candidates list, run the CfP response software, compute PSNRs, bit rates, and run times, and compute the BD-rate and run-time numbers relative to the HM anchor |
| 3 | Turn off a tool in the test candidates list, and keep all the other tools in the test candidates list on, run the CfP response software and compute the BD-rate and run-time numbers relative to the CfP response software with all tools on | Turn on a tool in the test candidates list, and keep all the other tools in the test candidates list off, run the CfP response software and compute the BD-rate and run-time numbers relative to the CfP response software with all tools of the test candidates list off (i.e. step 2) |
| 4 | Repeat step 3 until all the tools in the test candidates list are tested | Repeat step 3 until all the tools in the test candidates list are tested |
| 5 | Compose the test data |  |

Annex 2: Example of filling questionnaire HM vs. JEM

Codec level Q&A

| Codec Level Q&A |  | HM | JEM | Sony |
| --- | --- | --- | --- | --- |
| 1 | Maximum number of reference frames used by encoder | 4 | 4 | Same as for JEM |
| 2 | Maximum coding unit (e.g. CTU in HEVC) | luma 64×64 chroma 32×32 | luma 128×128 chroma 64×64 | Same as for JEM except for inter case which is as follows luma 256×256 chroma 128×128 |
| 3 | Minimum and maximum transform block size | luma 4×4/32×32 chroma 4×4/16×16 | luma 4×4/128×128 chroma 2×2/64×64 | Same as for JEM |
| 4 | Minimum and maximum intra prediction block | luma 4×4 chroma 4×4 | luma 4×4 chroma 2×2 | Same as for JEM |
| 5 | Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length | luma 4×8/8×4 (uni-pred) and 8×8 (bi-pred), 8-tap Chroma 2×4/4×2 (uni-pred) and 4×4 bi-pred, 4-tap | luma 4×4 (uni/bi-pred), 8-tap Chroma 2×2 (uni/bi-pred), 4-tap | Same as for JEM except for affine sub-block size in Cs.1: luma sub-block 2×2 (uni/bi-pred), 8-tap Chroma sub-block 1×1 (uni/bi-pred), 4-tap |
| 6 | List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category) | n/a | template matching in FRUC, LIC | Same as for JEM |
| 7 | List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples, position unforeseeable) | n/a | 64-bit arithmetic for LIC, BIO and LMC | Same as for JEM |
| 8 | List all the tools that use non-rectangular (e.g. triangle) partitions | n/a | n/a | Same as for JEM |
| 9 | Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa) | n/a | n/a | Same as for JEM |
| 10 | Indicate whether luma and chroma use separate partitioning structures | no | Yes, in I-slices | Same as for JEM |
| 11 | Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes) | Entropy parsing depends on intra prediction mode reconstruction, which requires an intra MPM list derivation | Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation | Same as for JEM |

Tool level Q&A

Category 1. Partitioning structure

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1. Partitioning structure |  | HM | JEM | Sony |
| 1.a | Whether the prediction unit partitioning is different from transform unit partitioning | Yes, PU and TU partitioning for a CU can differ | No | Same as for JEM |
| 1.b | Maximum tree depth | CU maximum depth 4 TU maximum depth 3 PU maximum depth 1 | QT maximum depth 5 BT maximum depth 3 | Same as for JEM except for inter case which is as follows: QT maximum depth 6 BT maximum depth 5 |

Category 2. Entropy coding

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 2. Entropy coding |  | HM | JEM | Sony |
| 2.a | Total number of context models | 186 | 329 | 389 |
| 2.b | Memory storage size for each context model | 8-bit | 30-bit | Same as for JEM |
| 2.c | Range table size if table look up is used for range update | LPSTable[64×4×8bits]; RenormTable[32× 3bits] | LPSTable[512×64×9bits]; RenormTable[128×3bits] | Same as for JEM |
| 2.d | Initialization table size | 3×186×8 bits | 3×329×8 + 329×15×5 bits |  |
| 2.e | Other memory associated with context model adaptation | LPS transition table: 64*6 bits MPS transition table: 64*6 bits | n/a | Same as for JEM |

Category 3. Control data derivation process

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 3. Control data derivation process |  | HM | JEM | Sony |
| 3.a | Temporal motion data storage granularity | 16×16 | 4×4 | Same as for JEM |
| 3.b | Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-directional reference picture are used, count this picture as 2) | 1 | 4 | Same as for JEM |

Category 4. Decoder-side motion refinement

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 4. Decoder-side motion refinement |  | HM | JEM | Sony |
| 4.a | Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement. | n/a | - BIO 2×2D separable 6 taps filter (for gradients), mults per sample in refinement calculation 5+5*4 +1/8+2, minimum processing unit same as regular bi-pred, no SAD calculation - FRUC minimum processing unit 44, sequential PU followed by sub-PU level refinement, number of SAD calculations and additional MC interpolations per unit for MV refinement is not constrained (refinement is terminated when cost no longer decreases); bi-linear filter at MV refinement stage | Same as for JEM |
| 4.b | Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size. | n/a | - DMVR needs to access (H+7+2)×(W+7+2) MC instead of (H+7)×(W+7) in HEVC MC - FRUC not constrained | Same as for JEM |

Category 5. Intra prediction

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 5. Intra prediction |  | HM | JEM | Sony |
| 5.a | Line buffer size in terms of luma and chroma sample lines | luma 1, chroma 1 | luma 4, chroma 2 | Same as for JEM |
| 5.b | List all the filters used for intra prediction | Reference sample smoothing: {1,2,1}, Strong intra smoothing filter: 2-tap bilinear filter (only for 32×32 luma TU) Interpolation: 2×32×5 bits (bi-linear 32 phases) | Reference sample smoothing: {1,2,1}, {3,7,14,16,14,7,3}, {4,9,12,14,12,9,4} Interpolation: 2×4×32×8 bits (4 taps, 2 sets, 32 phase) | Same as for JEM |
| 5.c | Indicate whether there are cross-component dependencies in intra prediction reconstruction | n/a | LM-CHROMA (Cb depends on Y, and Cr depends on Cb&Y) | Same as for JEM |

Category 6. Inter prediction

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 6. Inter prediction |  | HM | JEM | Sony |
| 6.a | Motion vector accuracy | luma ¼ pel chroma 1/8 pel | luma 1/16 pel chroma 1/32 pel | Same as for JEM |
| 6.b | Storage size for a motion vector component | 16-bit | 16-bit | Same as for JEM |
| 6.c | List all the interpolation filters used for luma/chroma motion compensation | luma 8-tap, 8×4×6 bits chroma 4-tap, 4×8×6 bits | luma 8-tap, 8×16×6 bits chroma 4-tap, 4×32×6 bits | Same as for JEM |

Category 7. Quantization

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 7. Quantization |  | HM | JEM | Sony |
| 7.a | Storage size associated with quantization | n/a | n/a | Same as for JEM |

Category 8. Transforms

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 8. Transforms |  | HM | JEM | Sony |
| 8.a | List all the combinations of transform block size and transform type for primary transforms | - 8-bit 4×4, 8×8, 16×16 and 32×32 DCT-II, - 8-bit 4×4 DST VII | HEVC transform mode (10-bit DCT-II), transform size combinations: - (4, 8, 16, 32, 64, 128) × (4, 8, 16, 32, 64, 128) TUs for luma CTUs - (2, 4, 8, 16, 32, 64) × (2, 4, 8, 16, 32, 64) TUs for inter-coded chroma CTUs - (2, 4, 8, 16, 32, 64,) × (2, 4, 8, 16, 32, 64) TUs for intra-coded chroma CTUs EMT transform mode (10-bit DST VII, DCT-V, DCT-VIII, DST-I), only applied to CTUs up to 64×64, transform size combinations: - (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TUs for luma CTUs | (a) HEVC transform mode(10-bit DCT-II): same as JEM’s (b) AMT transform mode (10-bit DCT-II, DST-VII, + two alternate transforms: FlipDST7, TrDST7), is applied to Transform block (TB)s up to 64x64, transform size combinations for both luma and chroma: - (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TBs for luma - (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TBs for chroma |
| 8.b | List all the combinations of transform block size and transform type for secondary transforms | n/a | 35×3 4×4 and 8×8 non-separable Hypercube-Givens Transform (HyGT) | Same as JEM’s |
| 8.c | List all the non-separable transforms used | n/a | 4×4/8×8 HyGT | Same as JEM’s |
| 8.d | Indicate whether transforms can be fully/partially implemented using direct matrix multiply | partial factorization equivalent to matrix multiplication | factorized form not equivalent to matrix multiplication | Same as JEM’s |
| 8.e | List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output) | n/a | 4×4 HyGT 2-interations, 8×8 HyGT 4-interations | Same as JEM’s |

Category 9. In-loop filtering

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 9. In-loop filtering |  | HM | JEM | Sony |
| 9.a | List line buffer size in terms of luma and chroma lines (if any) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) ALF (luma 8, chroma 4) | Same as for JEM except for deblocking (luma 8 & chroma 4) |
| 9.b | List all the filters used for in-loop filtering | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. ALF: 9×9/7×7/5×5 diamond filter support for luma; and 5×5 diamond filter support for chroma | Deblocking: For luma: (1 1 1 1 1 1 1 2 1 1 1 1 1 1 1)/16; (2 1 1 1 1 1 2 1 1 1 1 1 1 1)/16; (3 1 1 1 1 2 1 1 1 1 1 1 1)/16, (4 1 1 1 2 1 1 1 1 1 1 1)/16, (5 1 1 2 1 1 1 1 1 1 1)/16 (6 1 2 1 1 1 1 1 1 1)/16 and (7 2 1 1 1 1 1 1 1)/16 in addition to for JEM. For chroma: (1 1 1 2 1 1 1)/8 (2 1 2 1 1 1)/8 and (3 2 1 1 1)/8 in addition to for JEM. ALF: same as for JEM |
| 9.c | Minimum processing unit size | Deblocking (luma: 8×8 grid with 4×4 filter decision, chroma 8×8 grid with 4×4 filter decision) | Deblocking (luma 4×4 grid with 4×4 filter decision, chroma 8×8 grid with 2×2 filter decision) ALF: for luma, filter decision is based on 2×2 block Bilateral filter: sample level filter switch (luma only) | Same as for JEM |

Category 10. “tool-off” and “tool-on” tests

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 10. “tool-off” and “tool-on” tests |  | HM | JEM | Sony |
| 10.a | List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools | None | QTBT, ALF, FRUC, BIO, DMVR, EMT, … See Table 2 below for an example of “tool-off” and “tool-on” tests |  |

Table 2. Summary of “tool-off” and “tool-on” tests for JEM6.0 vs. HM

| Tested tool name | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| AFFINE | −1.2% | −1.0% | −1.1% | 105% | 101% | −2.7% | −2.2% | −2.2% | 135% | 107% |
| ALF | −5.0% | −2.8% | −2.0% | 101% | 115% | −5.8% | −2.3% | −1.7% | 111% | 175% |
| BIO | −1.1% | −0.2% | −0.1% | 115% | 147% | −2.3% | −0.8% | −0.5% | 113% | 187% |
| DMVR | −0.3% | −0.4% | −0.4% | 101% | 100% | −2.6% | −2.5% | −2.5% | 105% | 128% |
| EMT | −2.6% | −0.6% | −0.9% | 130% | 99% | −3.0% | −0.7% | −0.5% | 135% | 99% |
| FRUC | −3.8% | −3.8% | −4.1% | 125% | 130% | −6.1% | −5.8% | −5.9% | 119% | 180% |
| IMV | −1.0% | −1.4% | −1.5% | 111% | 97% | −2.2% | −2.7% | −2.9% | 117% | 95% |
| LM−CHROMA | −1.7% | −24.1% | −29.2% | 100% | 98% | −2.8% | −5.8% | −6.9% | 101% | 98% |
| NSST | −1.7% | −3.6% | −4.0% | 119% | 99% | −2.5% | −5.1% | −5.4% | 111% | 100% |

[end of document]