| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0029-v1 |
| --- | --- |

| Title: | Description of SDR video coding technology proposal by Tencent |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Xiaozhong Xu, Xin Zhao, Jing Ye, Liang Zhao, Shan Liu | Tel: Email: | {xlxiangli, xiaozhongxu, xinzzhao, jingye, leolzhao, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This proposal reports Tencent’s response to the Call for Proposals (CfP) on video compression with capability beyond HEVC, jointly issued by ITU-T SG16 Q.6 (VCEG) and ISO/IEC JTC1/SC29/WG11 (MPEG). This response is on top of Next software which is an alternative implementation of JEM. The additional or modified coding tools in this proposal include: block coding structure, intra block copy, intra mode coding, intra prediction with arbitrary reference tier, transform, and merge candidate list construction. It is reported that 36.17% (36.66% by new bdrateExtend fuction) and 27.78%, (28.21% by new bdrateExtend function) luma BD rate reduction over HM anchor for SDR constraint set 1 and 2 was achieved, respectively. When compared to JEM anchor, 4.70% and 4.47% (same results by the two bdrate functions) luma BD rate reduction was observed.

# Contents

Abstract	i

Contents	i

1	Introduction	3

2	Decoder algorithm description	3

2.1	General algorithm description	3

2.1.1	Bitstream structure	3

2.1.2	Entropy decoding	3

2.1.3	Conversion process from entropy decoded data to symbols	3

2.1.4	Decoder-side motion refinement	3

2.1.5	Inverse quantization	4

2.1.6	Inverse transforms	4

2.1.7	Motion compensation	5

2.1.8	Intra prediction	5

2.1.9	In-loop filtering	6

2.1.10	Additional tools	6

2.1.11	Additional algorithmic description topic(s)	8

3	Encoder algorithm description	9

3.1	General algorithm description	9

3.1.1	Rate-distortion optimization	9

3.1.2	Partitioning structure selection	9

3.1.3	Intra prediction mode estimation and type selection	9

3.1.4	Motion estimation and motion segment selection	9

3.1.5	Mode decisions	9

3.1.6	In-loop filtering type selection and parameter estimation	9

3.1.7	Transforms and transform type selection	9

3.1.8	Quantization and quantization type selection	11

3.1.9	Entropy coding	11

3.1.10	Additional tools	11

3.1.11	Additional encoder optimization	11

3.1.12	Additional algorithmic description topic(s)	11

4	Compression performance	11

4.1	SDR: Constraint set 1 configuration relative to HM anchor	11

4.1.1	Class SDR-A (RD-curves)	11

4.1.2	Class SDR-B (RD-curves)	14

4.1.3	Overall (BD-rate summary tables)	16

4.2	SDR: Constraint set 2 configuration relative to HM anchor	16

4.2.1	Class SDR-B	16

4.2.2	Overall	18

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	18

4.3.1	Class SDR-A	18

4.3.2	Class SDR-B	21

4.3.3	Overall	23

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	24

4.4.1	Class SDR-B	24

4.4.2	Overall	26

5	Decoder complexity analysis	26

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	26

5.2	Description of computing platform used to determine decoding times reported in section 5.1	26

5.3	Memory usage of decoder	27

5.4	Complexity characteristics of decoder entropy decoding operation	27

5.5	Complexity characteristics of decoder inverse quantization	27

5.6	Complexity characteristics of decoder inverse transform operation	27

5.6.1	Inverse primary transform	27

5.6.2	Inverse secondary transform	27

5.7	Complexity characteristics of decoder-side motion refinement	27

5.8	Complexity characteristics of decoder motion compensation	28

5.9	Complexity characteristics of decoder intra-frame prediction operation	28

5.10	Complexity characteristics of decoder in-loop filtering operation	28

5.11	Complexity characteristics of additional decoder tools	28

5.12	Degree of capability for decoder parallel processing	28

6	Encoder complexity analysis	28

6.1	Encoding time and measurement methodology	28

6.2	Description of computing platform used to determine encoding times reported in section 6.1	28

6.3	Memory usage of encoder	29

6.4	Complexity characteristics of encoder intra-frame prediction type selection	29

6.5	Complexity characteristics of encoder motion estimation and motion segmentation selection	29

6.5.1	General	29

6.5.2	Complexity characteristics of usage of decoder-side motion refinement	29

6.6	Complexity characteristics of encoder transforms and transform type selection	29

6.6.1	Forward primary transform	29

6.6.2	Forward secondary transform	30

6.7	Complexity characteristics of encoder quantization and quantization type selection	30

6.8	Complexity characteristics of encoder in-loop filtering type selection and parameter estimation	30

6.9	Complexity characteristics of encoder entropy coding type selection	30

6.10	Complexity characteristics of additional encoder tools	30

6.11	Degree of capability for encoder parallel processing	30

7	Algorithmic characteristics	30

7.1	Random access characteristics	30

7.2	Delay characteristics	30

7.3	Additional characteristics discussion topic(s)	30

8	Software implementation description	30

9	Highlighted aspects discussion	31

10	Closing remarks	31

11	Refences	31

12	Patent rights declaration(s)	32

13	Annex 1: Questionnaire	32

13.1	Codec level questions	32

# Introduction

This response is on top of Next software 1.8.5 [1] which is an alternative implementation of JEM 7.0 software [2]. The additional or modified coding tools are block coding structure, intra block copy, intra mode coding, intra prediction with arbitrary reference tier, transform and merge candidate list construction.

# Decoder algorithm description

## General algorithm description

### Bitstream structure

The same as JEM/Next.

### Entropy decoding

Instead of the default CABAC engine in Next software, the multi-parameter CABAC engine (CABACEngine=1) is used in this response.

### Conversion process from entropy decoded data to symbols

#### Intra prediction parameters

The intra mode coding process is modified according to the method proposed in [9]. More specifically, for Most Probable Mode (MPM) list construction, the checking order of DC mode is moved after checking the above left block. In addition, for non-MPM coding, the remaining 61 non-MPMs are separated to 2 sets, one has 16 non-MPMs, namely selected modes, the other covers the remaining 45 non-MPMs, namely non-selected modes. The 16 selected modes are derived by looping over all the directional modes in MPM list, and adding the adjacent directional mode with an offset 2 and -2. The magnitude of the offset is increased by 2 each time, i.e., +4, -4, +6, -6…, until the number of selected modes reaches 16.

#### Motion parameters

The same as that in Next software.

#### Quantization parameters

The same as that in Next software.

#### Transform coefficients

The same as that in Next software.

#### In-loop filter parameters

The same as that in Next software.

### Decoder-side motion refinement

The same as that in Next software.

### Inverse quantization

The same as that in Next software.

### Inverse transforms

#### Inverse primary transform

Similar to the primary transform design in Next/JEM software, the DCT-II and Adaptive Multiple Transform (AMT) are jointly used. To support 256×256 Coding Tree Unit (CTU) while keeping relatively low complexity, 256-point DCT-II is approximated by Hadamard transform.

For large inverse primary transform size, i.e., 64-point, 128-point and 256-point, a block-size dependent zero-out scheme is applied. More specifically, when 64-point, 128-point and 256-point inverse primary horizontal transform is applied, only coefficients of the first 32, 16 and 4 columns are used and the remainings are not used and considered as 0. When 64-point, 128-point and 256-point inverse primary vertical transform is applied, only coefficients of the first 32, 16 and 4 rows are used and the remaining are not used and considered as 0. For example, consider a 16×128 transform coefficient block, the inverse primary transform is only applied on the top-left 16×16 transform coefficients, and the remaining coefficients are considered as 0. For another example, consider a 64×256 transform coefficient block, the inverse primary transform is only applied on the top-left 32×4 transform coefficients, and the remaining coefficients are considered as 0.

The AMT in Next software is further modified by the methods proposed in [7] and [8]. For inverse primary transform applied on intra transform coefficients, in addition to DCT-II, DCT-V, DCT-VIII, DST-I and DST-VII, which are already applied in Next software, DST-4 and Identity (ID) transform are utilized as well.

Seven transform sets are pre-defined, and each transform set includes 4 pairs of horizontal and vertical transform, as depicted in below table.

|  | Transform Pair 0 |  | Transform Pair 1 |  | Transform Pair 2 |  | Transform Pair 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Vertical | Horizontal | Vertical | Horizontal | Vertical | Horizontal | Vertical | Horizontal |
| Transform Set 0 | DST4 | DST4 | DST7 | DST7 | DST4 | DCT8 | DCT8 | DST4 |
| Transform Set 1 | DST7 | DST7 | DST7 | DCT5 | DCT5 | DST7 | DST1 | DCT5 |
| Transform Set 2 | DST7 | DST7 | DST7 | DCT8 | DCT8 | DST7 | DCT5 | DCT5 |
| Transform Set 3 | DST4 | DST4 | DST4 | DCT5 | DCT8 | DST4 | DST1 | DST7 |
| Transform Set 4 | DST4 | DST7 | DST7 | DCT5 | DCT8 | DST7 | DST1 | DST7 |
| Transform Set 5 | DST7 | DST7 | DST7 | DCT5 | DCT8 | DST7 | DST1 | DST7 |
| Transform Set 6 | DST7 | DST7 | DST7 | DCT5 | DCT5 | DST7 | DST1 | DST7 |

Given the intra prediction mode and block size, one of the 7 transform sets are selected according to the below table, where bs is calculated as min (4, log2(width)-2, log2(height)-2).

| Intra Prediction Mode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 ~ 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bs = 0 | 0 | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| bs = 1 | 1 | 2 | 5 | 5 | 5 | 5 | 5 | 5 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| bs = 2 | 1 | 2 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| bs = 3 | 1 | 2 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| bs = 4 | 1 | 2 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |

Given the selected transform set, one transform pair of the transform set is further identified by the transform index (0~3) which is signaled in the bitstream. After that, for blocks smaller than or equal to 16×16, the absolute difference between the intra prediction mode index and horizontal (or vertical) intra prediction mode index is calculated and compare to the threshold given in the following table.

|  |  | Log2(width)-2 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 |
| Log2(height)-2 | 0 | 8 | 6 | 4 |
|  | 1 | 8 | 8 | 6 |
|  | 2 | 4 | 2 | -1 |

If the absolute difference is smaller than the given threshold, the vertical (or horizontal) transform of the last (3rd) transform pair is changed to be ID transform. Finally, the identified horizontal and vertical transform are applied on the transform coefficients as the inverse primary transform.

For primary transform applied on inter transform coefficients, the methods proposed in [8] are applied on top of Next software. More specifically, for 4-point, 8-point and 16-point primary transforms, the transform cores applied in Advanced Multiple Transform (AMT) are replaced by KLT.

#### Inverse secondary transform

In this proposal, instead of Hyper-Givens transform [9] as used in Next/JEM software, the secondary transform is implemented based on matrix multiplication. There are 35x3 pre-defined 16×16 and 64×64 matrices which are applied as 4×4 and 8×8 non-separable secondary transform (NSST) cores, respectively. The elements in the proposed secondary transform are represented by 8-bit integers.

The signaling and conditions of applying secondary transform are unchanged and kept as same as those in Next software.

### Motion compensation

The same as that in Next software.

### Intra prediction

For Planar, DC, horizontal and vertical intra prediction modes, the prediction process has been replaced by the methods proposed in [11].

For Planar and DC intra prediction modes, the prediction value pred[x][y] for a sample located at (x, y) is derived as follows,

pred[x][y] = wL * R-1,y + wT * Rx,-1 + wTL * R-1,-1 + (64 – wL – wT – wTL) * pred[x][y] + 32 )  >>  6,

where pred[x][y] is derived using the HEVC intra prediction function of the corresponding mode, Rx,-1, R-1,y and R-1,-1 denote the unfiltered reference samples at top, left and top-left position of current sample (x, y), respectively, and wL, wT and wTL represent the weightings calculated as below,

wT = 32 >> ( ( y<<1 ) >> shift ),  wL = 32 >> ( ( x<<1 ) >> shift),  wTL = – ( wL>>4 ) – ( wT>>4 ),

with shift = ( log2( width ) + log2( height ) + 2 ) >> 2;

For horizontal and vertical intra prediction modes, the prediction value is derived using the same above procedure, but the top-left weighting is modified as wTL = – wT and wTL = – wL, respectively.

### In-loop filtering

The same as that in Next software.

### Additional tools

#### Larger CTU

The CTU, which represents the largest coding unit, is increased from 128×128 to 256×256. To support 256×256 CTU size, the largest primary transform size is also increased from 128-point to 256-point accordingly.

#### Improvement to block structure – Triple tree

Block coding structure is improved by further introducing triple tree proposed in [3]. As shown in Figure 1, two new partitioning types, i.e., vertical and horizontal triple trees are added in addition to quad-tree and binary trees. With the triple trees, a block is split into three sub-blocks vertically or horizontally such that the size of two side sub-blocks are the same and the sum of the two side sub-block sizes is the same as the center sub-block.

Figure 1 (a) quad-tree partitioning (b) vertical binary-tree partitioning (c) horizontal binary-tree partitioning (d) vertical triple-tree partitioning (e) horizontal triple-tree partitioning

To support the new tree types, an additional bin is added to signal whether a splitting type is binary tree or triple tree.

#### Intra Block Copy

Intra Block Copy (IBC) [5] is implemented as a block level coding mode. A block vector in integer resolution is used to indicate the displacement from the current block to a reference block inside the current picture.

When signaling the prediction mode of a Coding Unit (CU), the IBC mode is signaled as a third mode, which is different from intra or inter mode. At CU level, a 1-bin flag is signaled to indicate if the current block is coded in IBC mode or not. There are two different coding methods for the IBC mode in this response.

- If a block is coded in merge mode, a merge candidate index is used to indicate which of the block vectors in the list from neighboring candidate IBC coded blocks will be used to predict the current block. The merge candidate list consists of five spatial neighboring blocks, in the similar way as in HEVC.

- If a block is coded as a non-merge mode, block vector difference is coded in the same way as a motion vector difference. The block vector prediction method uses two candidates as predictors, one from left neighbor and one from above neighbor (if IBC coded). When either neighbor is not available, a default block vector will be used as a predictor. A 1-bin flag is signaled to indicate the block vector predictor index.

When separate partition trees for luma and chroma are used, such as intra slices, a chroma CU may be coded in IBC mode, with the corresponding chroma block vector derived from the corresponding luma samples. More specifically, a chroma CU’s corresponding luma samples are checked in terms of their samples coverage by IBC mode. The following methods are proposed to handle different cases when chroma separate coding tree is enabled:

- Full coverage: all the corresponding luma sub-blocks are coded in IBC mode. In this case, the current chroma CU can choose to be coded in IBC mode. For each chroma sub-block, its block vector is derived from its corresponding luma block’s block vector, or it can choose from other available modes (intra, PCM, etc.) or split further.

- No coverage: none of the corresponding luma sub-blocks is coded in IBC mode. In this case, the current chroma CU will be coded in one of the available modes (intra mode, PCM, etc.) or split further.

- Partial coverage: at least one, but not all, of the corresponding luma sub-blocks are coded in IBC mode. In this case, the current chroma CU will signal a flag to indicate if a further split is necessary. If yes, it will be split into smaller CUs and the decision will be made by each split CU, using the same criteria here; if no, the current chroma CU will be coded in one of the available modes but not IBC.

In the current implementation, the IBC mode is enabled for block length (luma width or height) of 16 and under.

#### Intra prediction with arbitrary reference tier

The arbitrary reference tier (ART) [12] was proposed to increase the number of reference tiers, and an intra directional mode can arbitrarily decide which reference tier is selected to generate intra predictors. Suppose there are N reference tiers allowed for intra predictions. Figure 2 depicts an example of ART for N = 4. Each of the farther reference tiers is composed of the above-left pixel and six segments from Segment A to Segment F. To reduce the complexity and possible dependencies, Segment A and Segment F are padded with the closest samples from Segment B and Segment E, respectively. An index is signaled to indicate which reference tier is chosen for an intra directional mode. If N is set as 1, the intra directional prediction method is the same as the method in Next/JEM software. To reduce the coding complexity, the NSST index coding is skipped when the third or fourth reference tier is chosen for an intra directional mode.

Figure 2 Four reference tiers adjacent to the coding block unit [12].

#### Additional merge candidates

Additional merge candidates are proposed to the merge candidates lists. The maximum number of merge candidates is increased to 23. The proposed method gets merge candidates from neighbors that are not direct spatial neighbors to the current block. Figure 3 shows the neighboring block that is checked when constructing the merge candidates list.

Figure 3 Additional merge candidates

In the above figure, the offset of the reference region to the current is (-96, -96). The searching grid is 16×16. For each 16×16 grid, the top left 4×4 unit is checked. If it is inter coded and the motion information is different from the existing merge candidates, the motion information of this block is inserted into the merge candidate list. The searching order is from the reference blocks that are near to the current block to the reference blocks that are far from the current block. These newly added merge candidates are inserted after TMVP and before artificial candidates.

### Additional algorithmic description topic(s)

None.

# Encoder algorithm description

## General algorithm description

### Rate-distortion optimization

#### Distortion metrics

The same as that in Next software.

#### Rate-distortion dependency models

The same as that in Next software.

### Partitioning structure selection

The same as that in Next software.

### Intra prediction mode estimation and type selection

In Next software, there are 3~6 intra prediction modes (IPMs) selected to perform RDO checks for intra mode decision, where 3 IPMs are selected from 67 IPMs based on SATD costs, and 3 IPMs at most are selected out of most probable modes (MPMs). To implement ART at the encoder, there are 3N to 3N+3 IPMs selected to perform RDO checks for intra mode decision, where 3N IPMs are selected from the IPMs in different reference tiers based on SATD costs, and 3 IPMs at most are selected out of most probable modes (MPMs). Therefore, there are 12~15 IPMs to be further checked using RDO based intra mode decision.

### Motion estimation and motion segment selection

#### General

The same as that in Next software.

#### Usage of decoder-side motion refinement

The same as that in Next software.

### Mode decisions

The same as that in Next software.

### In-loop filtering type selection and parameter estimation

The same as that in Next software.

### Transforms and transform type selection

To match with the proposed changes on inverse transform process, at the encoder, the transform type selection in Next software is also modified accordingly by the methods proposed in [7] and [8].

For forward primary transform applied on intra prediction residuals, in addition to DCT-II, DCT-V, DCT-VIII, DST-I and DST-VII, which are already applied in Next software, DST-4 and Identity (ID) transform are utilized as well.

Seven transform sets are pre-defined, and each transform set includes 4 pairs of horizontal and vertical transform, as depicted in below table.

|  | Transform Pair 0 |  | Transform Pair 1 |  | Transform Pair 2 |  | Transform Pair 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Vertical | Horizontal | Vertical | Horizontal | Vertical | Horizontal | Vertical | Horizontal |
| Transform Set 0 | DST4 | DST4 | DST7 | DST7 | DST4 | DCT8 | DCT8 | DST4 |
| Transform Set 1 | DST7 | DST7 | DST7 | DCT5 | DCT5 | DST7 | DST1 | DCT5 |
| Transform Set 2 | DST7 | DST7 | DST7 | DCT8 | DCT8 | DST7 | DCT5 | DCT5 |
| Transform Set 3 | DST4 | DST4 | DST4 | DCT5 | DCT8 | DST4 | DST1 | DST7 |
| Transform Set 4 | DST4 | DST7 | DST7 | DCT5 | DCT8 | DST7 | DST1 | DST7 |
| Transform Set 5 | DST7 | DST7 | DST7 | DCT5 | DCT8 | DST7 | DST1 | DST7 |
| Transform Set 6 | DST7 | DST7 | DST7 | DCT5 | DCT5 | DST7 | DST1 | DST7 |

Given the intra prediction mode and block size, one of the 7 transform sets are selected according to the below table, where bs is calculated as min (4, log2(width)-2, log2(height)-2).

| Intra Prediction Mode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 ~ 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bs = 0 | 0 | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| bs = 1 | 1 | 2 | 5 | 5 | 5 | 5 | 5 | 5 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| bs = 2 | 1 | 2 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| bs = 3 | 1 | 2 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| bs = 4 | 1 | 2 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |

Given the selected transform set, one transform pair of the transform set is further identified by the transform index (0~3) which is signaled in the bitstream. After that, for blocks smaller than or equal to 16×16, the absolute difference between the intra prediction mode index and horizontal (or vertical) intra prediction mode index is calculated and compare to the threshold given in the following table.

|  |  | Log2(width)-2 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 |
| Log2(height)-2 | 0 | 8 | 6 | 4 |
|  | 1 | 8 | 8 | 6 |
|  | 2 | 4 | 2 | -1 |

If the absolute difference is smaller than the given threshold, the vertical (or horizontal) transform of the last (3rd) transform pair is changed to be ID transform. Finally, the identified horizontal and vertical transform are applied on the residual block as the forward primary transform.

For primary transform applied on inter prediction residuals, the methods proposed in [8] are applied on top of Next software. More specifically, for 4-point, 8-point and 16-point primary transforms, the transform cores applied in Advanced Multiple Transform (AMT) are replaced by KLT.

The encoder mode decision process on the selecting the AMT flag and index values is unchanged and kept as same as that in Next software.

### Quantization and quantization type selection

The same as that in Next software.

### Entropy coding

The same as that in Next software.

### Additional tools

None.

### Additional encoder optimization

To improve encoding speed, a fast method called adaptive force skip is proposed based on [4]. Before coding a block, a forceSkip flag is first derived based on whether the best modes of its parent and its grandparents are both skip. If true, non-skip coding modes are not checked for this block except further splitting.

### Additional algorithmic description topic(s)

None.

# Compression performance

## SDR: Constraint set 1 configuration relative to HM anchor

### Class SDR-A (RD-curves)

#### FoodMarket4

#### CatRobot1

#### DaylightRoad2

#### ParkRunning3

#### CampfireParty2

### Class SDR-B (RD-curves)

#### BQTerrace

#### RitualDance

#### MarketPlace

#### BasketballDrive

#### Cactus

### Overall (BD-rate summary tables)

Please note that the BD-rate is calculated by the new function bdrateExtend from JVET-H0030 which was adopted in Macau meeting.

## SDR: Constraint set 2 configuration relative to HM anchor

### Class SDR-B

#### BQTerrace

#### RitualDance

#### MarketPlace

#### BasketballDrive

#### Cactus

### Overall

Please note that the BD-rate is calculated by the new function bdrateExtend from JVET-H0030 which was adopted in Macau meeting.

## SDR: Constraint set 1 configuration relative to JEM anchor

### Class SDR-A

#### FoodMarket4

#### CatRobot1

#### DaylightRoad2

#### ParkRunning3

#### CampfireParty2

### Class SDR-B

#### BQTerrace

#### RitualDance

#### MarketPlace

#### BasketballDrive

#### Cactus

### Overall

Please note that the BD-rate is calculated by the new function bdrateExtend from JVET-H0030 which was adopted in Macau meeting.

## SDR: Constraint set 2 configuration relative to JEM anchor

### Class SDR-B

#### BQTerrace

#### RitualDance

#### MarketPlace

#### BasketballDrive

#### Cactus

### Overall

Please note that the BD-rate is calculated by the new function bdrateExtend from JVET-H0030 which was adopted in Macau meeting.

# Decoder complexity analysis

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

The run times of the proposed decoder software are shown in Table below.

|  | Constraint Set 1 configuration | Constraint Set 2 configuration |
| --- | --- | --- |
| Decoding Time | 954% | 754% |

## Description of computing platform used to determine decoding times reported in section 5.1

The computing platforms are summarized as below.

|  | CPU | Multi-core | Memory | OS | Storage |
| --- | --- | --- | --- | --- | --- |
| Platform | Intel(R) Xeon(R) CPU E5-2680 v4(14 cores * 2 , 2.4 GHz) | Yes | 500GB | CentOS Linux release 7.2.1511 64-bits | Local HDD |

## Memory usage of decoder

For RA UHD1 highest rate, maximally 1.2 GB memory is used by the proposed decoder.

For LDB HD1 highest rate, maximally 0.4 GB memory is used by the proposed decoder.

## Complexity characteristics of decoder entropy decoding operation

The same as that in Next software.

## Complexity characteristics of decoder inverse quantization

The same as that in Next software.

## Complexity characteristics of decoder inverse transform operation

### Inverse primary transform

In this proposal, the additional inverse primary transforms are 4-point and 8-point DST4, 4-point, 8-point and 16-point ID transform, and 256-point Hadamard transform with zero-out.

For 1-D inverse 4-point DST4, 16 multiplications and 12 adds are used. For 1-D inverse 8-point DST4, 64 multiplications and 56 adds are used. For 1-D inverse 4-point, 8-point and 16-point ID transform, 4, 8 and 16 shifts are used. For 1-D inverse 256-point Hadamard transform with zero-out, 4 shifts and 12 adds are needed.

Moreover, the inverse primary 128-point transform only keeps the first 16 coefficients and the remaining coefficients are considered to be 0. As a result, for 1-D inverse 128-point transform, 2048 multiplications and 1920 adds are needed.

In Next software, the total memory for storing the primary transform cores is 26.525 Kbytes. While in this response, the total memory for storing the primary transform cores is 23.965 Kbytes due to different zero-out schemes for large transforms as compared to Next software.

### Inverse secondary transform

In this proposal, the inverse secondary transform is implemented as matrix multiplication. For one 4x4 inverse secondary transform, 256 multiplications and 240 adds are needed. For one 8x8 inverse secondary transform, 4096 multiplications and 4032 adds are needed.

In Next software, the total memory for storing the secondary transform cores is 94.004 KBytes, while in our proposal, the total memory for storing the secondary transform cores is 456.96 Kbytes as HyGT is replaced by matrix multiplication.

## Complexity characteristics of decoder-side motion refinement

The decoder-side motion refinement modules are unchanged from Next software. However, a larger merge list is proposed so that the seed number of FRUC mode is increased.

## Complexity characteristics of decoder motion compensation

The same as that in Next software.

## Complexity characteristics of decoder intra-frame prediction operation

The decoding process requirement for the IBC mode is very similar to inter motion compensation with uni-prediction and integer vector resolution. There is not additional computational cost of NSST or LIC when a block is coded in IBC mode as they are normatively disabled. EMT is also signaled as off by the encoder.

## Complexity characteristics of decoder in-loop filtering operation

The same as that in Next software.

## Complexity characteristics of additional decoder tools

The reference sample for IBC mode is the reconstructed samples prior to all loop-filtering operations. This means that potentially, such unfiltered samples need to be written off-chip to memory. The calculated memory bandwidth cost is 1 access/pixel.

## Degree of capability for decoder parallel processing

The same as that in Next software.

# Encoder complexity analysis

## Encoding time and measurement methodology

The run times of the proposed encoder software compared to HM anchor are shown below.

|  | Constraint Set 1 configuration | Constraint Set 2 configuration |
| --- | --- | --- |
| Encoding Time | 1928% | 1951% |

## Description of computing platform used to determine encoding times reported in section 6.1

The computing platforms are summarized as below.

|  | CPU | Multi-core | Memory | OS | Storage |
| --- | --- | --- | --- | --- | --- |
| Platform | Intel(R) Xeon(R) CPU E5-2680 v4(14 cores * 2 , 2.4 GHz) | Yes | 500GB | CentOS Linux release 7.2.1511 64-bits | Local HDD |

## Memory usage of encoder

For RA UHD1 highest rate, maximally 4.9 GB memory is used by the proposed encoder.

For LDB HD1 highest rate, maximally 1.2 GB memory is used by the proposed encoder.

## Complexity characteristics of encoder intra-frame prediction type selection

The major complexity in encoder intra-frame prediction type selection is from IBC.

The encoder search of the IBC mode has two parts. A hash-based global search is performed first, if no valid block vector is found, block matching based local search will be performed.

In the hash-based search, hash key matching between the current block and a reference block is extended to all allowed block sizes. The hash key calculation for every position in current picture is based on 4x4 blocks. For the current block of a larger size, a hash key matching to a reference block happens when all its 4×4 blocks match the hash keys in the corresponding reference locations. If multiple reference blocks are found to match the current block with the same hash key, the block vector costs of each candidates are calculated and the one with minimum cost is selected.

In block matching search, the search range is set to be 128 pixels to the left and on top of current block.

## Complexity characteristics of encoder motion estimation and motion segmentation selection

### General

The same as that in Next software.

### Complexity characteristics of usage of decoder-side motion refinement

The decoder-side motion refinement modules are unchanged from Next software. However, a larger merge list is proposed so that the seed number of FRUC mode is increased.

## Complexity characteristics of encoder transforms and transform type selection

### Forward primary transform

In this proposal, the additional forward primary transforms are 4-point and 8-point DST4, 4-point, 8-point and 16-point ID transform, and 256-point Hadamard transform with zero-out.

For 1-D forward 4-point DST4, 16 multiplications and 12 adds are used. For 1-D inverse 8-point DST4, 64 multiplications and 56 adds are used. For 1-D forward 4-point, 8-point and 16-point ID transform, 4, 8 and 16 shifts are used. For 1-D forward 256-point Hadamard transform with zero-out, 256 shifts and 268 adds are needed.

In addition, the inverse 128-point transform only keeps the first 16 coefficients and the remaining coefficients are considered to be 0. As a result, for 1-D forward 128-point transform, 2048 multiplications and 1920 adds are needed.

In Next software, the total memory for storing the primary transform cores is 26.525 KBytes, while in this proposal, the total memory for storing the primary transform cores is 23.965 KBytes due to different zero-out schemes for large transforms as compared to Next software.

### Forward secondary transform

In this proposal, the forward secondary transform is implemented as matrix multiplication. For one 4x4 forward secondary transform, 256 multiplications and 240 adds are needed. For one 8x8 forward secondary transform, 4096 multiplications and 4032 adds are needed.

In Next software, the total memory for storing the secondary transform cores is 94.004 KBytes, while in our proposal, the total memory for storing the secondary transform cores is 456.96 KBytes.

## Complexity characteristics of encoder quantization and quantization type selection

The same as that in Next software.

## Complexity characteristics of encoder in-loop filtering type selection and parameter estimation

The same as that in Next software.

## Complexity characteristics of encoder entropy coding type selection

The same as that in Next software.

## Complexity characteristics of additional encoder tools

In block coding structure, triple tree is introduced so that additional block R-D checking is needed during encoding.

## Degree of capability for encoder parallel processing

Due to newly introduced triple tree, a bit higher capability for encoder parallel processing than Next software may be achieved by multi-threading at CTU level. Basically, at CTU level (CU depth 0), each tree type may be R-D evaluated by a single thread. In other words, R-D checking on tree types may be in parallel.

# Algorithmic characteristics

## Random access characteristics

The same as that in Next software.

## Delay characteristics

The same as that in Next software.

## Additional characteristics discussion topic(s)

None.

# Software implementation description

The proposed software is based on Next software 1.8.5 which is implemented with C++. All newly introduced coding tools are implemented with the same coding style as that of Next software.

# Highlighted aspects discussion

IBC mode is tested for coding of screen content materials, in addition to CfP sequences. In Table 1 below, the BD rate changes are shown for CfP sequences. In Table 2 below, the BD rate changes for a set of HD (1080p) sequences in Text and Graphics with Motion category are shown. The sequences and testing QPs are selected from HEVC SCC CTC [6]. Other test conditions follow the simulation conditions in this document. In these two tests, the anchor is the CfP response software. The tested one is the software without IBC mode.

- Simulation Results for CfP Sequences

| Sequences | RA | Enc Time | Dec Time |
| --- | --- | --- | --- |
| FoodMarket4 | -0.01% | 101% | 100% |
| CatRobot1 | 0.37% | 102% | 100% |
| DaylightRoad2 | 0.54% | 97% | 96% |
| ParkRunning3 | -0.06% | 100% | 93% |
| CampfireParty2 | 0.06% | 98% | 96% |
| BQTerrace | 1.42% | 98% | 100% |
| RitualDance | 0.00% | 106% | 99% |
| MarketPlace | 0.02% | 100% | 98% |
| BasketballDrive | 0.15% | 99% | 93% |
| Cactus | 0.33% | 102% | 100% |
| Average | 0.28% | 100% | 98% |

- Simulation Results for SCC Sequences

| Sequences | RA | Enc Time | Dec Time |
| --- | --- | --- | --- |
| Console | 54.67% | 91% | 127% |
| Desktop | 100.79% | 93% | 115% |
| FlyingGraphics | 18.42% | 94% | 106% |
| ChinaEditing | 35.05% | 96% | 105% |
| HD Average | 52.23% | 94% | 113% |

# Closing remarks

This proposal describes Tencent’s response to the joint CfP on video compression with capability beyond HEVC by ITU-T SG16 Q.6 (VCEG) and ISO/IEC JTC1/SC29/WG11 (MPEG). Several coding tools including block coding structure, intra block copy, intra mode coding, intra prediction with arbitrary reference tier, transform, and merge candidate list construction, are proposed. Compared to HM anchor, 36.66%, 28.21% luma BD rate reduction for SDR constraint set 1 and 2 was achieved, respectively. Compared to JEM anchor, 4.70% and 4.47% luma BD rate reduction was observed accordingly.

# Refences

- Wieckowski, T. Hinz, V. George, J. Brandenburg, J. Ma, S. De-Luxán-Hernández, B. Bross, H. Schwarz, D. Marpe, T. Wiegand, “AHG10: Updated NextSoftware as an alternative implementation of the Joint Exploration Model (JEM)”, JVET-I0034, Gwangju, KR, 20–26 Jan. 2018.

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of Joint Exploration Test Model 7 (JEM7)”, JVET-G1001, Torino, IT, 13–21 July 2017.

- X. Li, H.-C. Chuang, J. Chen, M. Karczewicz, L. Zhang, X. Zhao, A. Said, “Multi-Type-Tree”, JVET-D0117, Chengdu, CN, 15–21 October 2016.

- L. Shen, Z. Zhang and Z. Liu, "Adaptive Inter-Mode Decision for HEVC Jointly Utilizing Inter-Level and Spatiotemporal Correlations," in IEEE Transactions on Circuits and Systems for Video Technology, vol. 24, no. 10, pp. 1709-1722, Oct. 2014.

- X. Xu, et al, “Intra Block Copy in HEVC Screen Content Coding Extensions,” IEEE J. Emerg. Sel. Topics Circuits Syst., vol. 6, no. 4, pp. 409-419, 2016.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, Common Test Conditions for Screen Content Coding, JCTVC-U1015, Proceeding of 19th JCT-VC meeting, Jun. 2015.

- V. Lorcy, P. Philippe, T. Biatek, X. Zhao, V. Seregin, M. Karczewicz, “EE2: Adaptive Primary Transform improvement”, JVET-D0065, Chengdu, CN, 15–21 Oct. 2016.

- X. Zhao, V. Seregin, M. Karczewicz, J. Chen, “Improvements on AMT for inter prediction residuals”, JVET-D0126, Chengdu, CN, 15–21 Oct. 2016.

- X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, “EE1: Improvements on non-separable secondary transform”, JVET-D0120, Chengdu, CN, 15–21 Oct. 2016.

- Y. Han, J. An, J. Zheng, “EE1: Improvements for Intra Prediction Mode Coding”, JVET-H0024, Macao, CN, 18–25 Oct. 2017.

- X. Zhao, V. Seregin, A. Said, M. Karczewicz, “EE1 related: Simplification and extension of PDPC”, JVET-H0057, Macao, CN, 18–25 Oct. 2017.

- Y.-J. Chang, P.-H. Lin, C.-L. Lin, J.-S. Tu, and C.-C. Lin, “Arbitrary reference tier for intra directional modes,” JVET-C0043, Geneva, CH, May 2016.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex 1: Questionnaire

Proponents of CfP responses are required to answer the codec level questions, and are also encouraged to fill out the optional tool level questions in addition to the complexity analysis of their proposals. If some question is not applicable to your proposal, you may express that as well.

## Codec level questions

- Maximum number of reference frames used by encoder

4 frames.

- Maximum coding unit (e.g. CTU in HEVC)

Luma 256×256, chroma 128×128.

- Minimum and maximum transform block size

Luma 4×4 and 256×256. Chroma 2×2 and 128×128.

- Minimum and maximum intra prediction block

Luma 4×4 and 256×256. Chroma 2×2 and 128×128.

- Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length

Luma 4×4 (uni/bi-pred), 8-tap

Chroma 2×2 (uni/bi-pred), 4-tap

- List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category)

Template matching in FRUC, LIC

- List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples)

64-bit arithmetic for LIC, BIO and LMC.

- List all tools that use non-rectangular (e.g. triangle) partitions

N/A.

- Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa)

N/A.

- Indicate whether luma and chroma use separate partitioning structures

Yes, in I-slices.

- Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes)

Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation.

- If the coded picture resolution is different from the input picture resolution, list the coded picture resolution for those sequences (e.g. 360° video, input is 8K ERP, coded video is in 4K CMP converted from the 8K input)

No.