| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, USA, 10–20 Apr. 2018 | Document: JVET-J0030 |
| --- | --- |

| Title: | Description of 360° video coding technology proposal by TNO. |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Call for Proposals |  |  |
| Author(s) or Contact(s): | Alexandre Gabriel Emmanuel Thomas | Tel: Email: | +31611843111 / +31652779148 alexandre.dasilvapratasgabriel@tno.nl emmanuel.thomas@tno.nl |
| Source: | TNO |  |  |

_____________________________

# Abstract

This proposal is for the Call for Proposals for the 360 video category. In the proposal a method of encoding by subsampling is proposed whereby each frame is divided into 4 different frames which are subsequently ordered and encoded as normal. The proposal has 360 videos as a target because the resolution is typically much higher meaning that aliasing is less likely to occur. It is also the intention that with the distortions present in an ERP that the subsampling process will not affect the subjective quality. The GOP structure parameters intend to also allow for scalability with frames with a higher Temporal Id corresponding to one 4K stream and the remaining frames allowing for the reconstruction of the full 8K allowing for scalability.

# Contents

Abstract	i

Contents	i

0	Definition and abbreviations	1

1	Introduction	1

2	Decoder algorithm description	1

2.1	General algorithm description	1

2.1.1	Additional tools	1

2.1.2	Additional algorithmic description topic(s)	2

2.2	360° Video	3

2.2.1	Projection format (including padding)	3

2.2.2	Quantization handling	3

2.2.3	360° video specific decoding tools	3

3	Encoder algorithm description	3

3.1	General algorithm description	3

3.1.1	Additional tools	4

3.1.2	Additional algorithmic description topic(s)	4

3.2	360°	4

3.2.1	Usage of projection format adaptation	4

3.2.2	360° Video specific encoding tools	4

3.2.3	Additional encoder optimization	5

4	Compression performance	5

4.1	360° Video: Constraint set 1 configuration relative to HM anchor	5

4.1.1	Class 360	5

4.1.2	Overall	6

4.2	360° Video: Constraint set 1 configuration relative to JEM anchor	6

5	Decoder complexity analysis	7

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	7

5.2	Description of computing platform used to determine decoding times reported in section 5.1	7

5.3	Memory usage of decoder	8

5.4	Complexity characteristics	9

5.5	Complexity characteristics of additional decoder tools	10

5.5.1	Polyphase reconstruction	10

5.6	Degree of capability for decoder parallel processing	10

6	Encoder complexity analysis	11

6.1	Encoding time and measurement methodology	11

6.2	Description of computing platform used to determine encoding times reported in section 6.1	11

6.3	Memory usage of encoder	12

6.4	Complexity characteristics of encoder intra-frame prediction type selection	12

6.5	Complexity characteristics of encoder motion estimation and motion segmentation selection	12

7	Algorithmic characteristics	13

7.1	Random access characteristics	13

7.2	Delay characteristics	14

8	Software implementation description	14

9	Highlighted aspects discussion	14

10	Closing remarks	17

11	Patent rights declaration(s)	17

12	Annex 1: Questionnaire	17

12.1	Codec level questions	17

12.2	Tool level questions	18

13	Reference	20

# Definition and abbreviations

low resolution frame

A low resolution frame is a quarter of a frame that results of one of the phases once a frame has been polyphase subsampled

super frame

A super frame is a frame generated based on low resolution frames coherent in time whose resolution is higher than the one of the low resolution frames

# Introduction

The technique described in this document enables spatial scalability of a video bitstream by reusing existing coding tools of non-scalable video coding system. This technique requires at least an ultra-high resolution (i.e. 4K) source input and performs better with even higher resolutions so that no noticeable artifacts such as aliasing are produced. Therefore, the 360-degree video category with 8K test sequences is a relevant category for this technique. In particular, the technique decomposes the input video signal (luminance and chrominance components) into lower resolution descriptions. In this case, the proposed method uses a polyphase filter to subsample the input video bitstream but other filters could also be applied. The multiple lower resolution descriptions of the signal, called resolution components in this contribution, are encoded and transported in the same video bitstream. On the decoder side, the various resolution components are decoded and recombined according to the down-subsampling performed before the encoding (here polyphase subsampling) which provides the high resolution output of the decoded bitstream. If only one resolution component is decoded, the output picture corresponds to a downscaled version of the original sequence, here by a factor of 4 in terms of number pixels due to the property of the polyphase filtering.

# Decoder algorithm description

## General algorithm description

All the following parameters are the same as the HEVC Test Model (HM) :

- Bitstream structure

- Entropy decoding

- Conversion process from entropy decoded data to symbols

- Decoder-side motion refinement

- Inverse quantization

- Inverse transforms

- Motion compensation

- Intra prediction

- In-loop filtering

### Additional tools

#### Polyphase decomposition and composition

Polyphase subsampling is a known filtering technique in the field the signal processing that decomposes a signal into lower frequency output signals. The process is reversible and does not incur loss of data. In other words, the union of the output signals is equal to the input signal. Figure 1 shows a polyphase decomposition example considering a 4x4 2D signal. The four output signals constitute four subsets of samples of the input signal.

Figure 1 - Polyphase decomposition

The reverse operation takes as input the four low resolution signals and combine them adequately to recompose the original signal.

#### High resolution frame reconstruction

In addition to performing the usual decoding steps of HM, the decoder maintains a second output video buffer whose pixel resolution is 2 times higher than the pixel resolution of the coded bitstream. Every time a frame is decoded, the YUV buffer of this frame is also stored in the high resolution frame output buffer. The way the low resolution frames are stored in the high resolution frame YUV buffer is done according to the index of each low resolution frame in the current Group of Picture which determines the resolution component they represent as pictured in Figure 1.

As a result, the decoder can output two decoded video sequences. The first contains all the low resolution frames temporally multiplexed. The second is the high resolution video sequences reconstructed by combining the low resolution frames.

### Additional algorithmic description topic(s)

When decoding the coded sequence, the decoder also comprises the reconstruction step of the high resolution sequence by recombining the decoded low resolution frames. Figure 2 shows the logic implemented in the decoder.

Figure 2 - High-level decoding loop block diagram

## 360° Video

### Projection format (including padding)

The source videos are not preprocessed and are encoded in their original projection format, equirectangular projection (ERP) – without any padding.

### Quantization handling

No specific quantization is applied.

### 360° video specific decoding tools

The key advantage of the 360-degree category is the pixel resolution which is higher than 4K compared to other categories such as the SDR category. In addition, 360-degree videos have the property of warping straight lines from the real world in the projected frame especially when using the equirectangular projection (ERP). This may tend to reduce the likelihood of aliasing effect since high frequency regular patterns in the scene may be distorted in the projected frame.

The decoder does not implement 360° specific tools.

# Encoder algorithm description

## General algorithm description

All the following parameters are the same as HEVC Test Model (HM):

- Rate-distortion optimization

- Partitioning structure selection

- Intra prediction mode estimation and type selection

- Motion estimation and motion segment selection

- Usage of decoder-side motion refinement

- Mode decisions

- In-loop filtering type selection and parameter estimation

- Transforms and transform type selection

- Quantization and quantization type selection

- Entropy coding

### Additional tools

#### Polyphase decomposition

The polyphase decomposition generates four low resolution signals from a single input signal. The polyphase filtering is explained in 2.1.1.1.

#### Low resolution frame generation

Before performing the encoding steps of HM, the encoder down-samples the original frame and produces four smaller frames which constitutes the internal bitstream format in terms of pixel resolution. In addition, the internal framerate is set to the quadruple of the input framerate so that it corresponds to the rate at which the bitstream should be decoded to provide the high resolution decoded sequence. The encoder also maintains both original frames and subsampled frames so that metrics can be computed for both type of frames. Note though that metrics such as PSNR of a super frame can be calculated based on the PSNR of the lower resolution frames constituting this super frame, see 3.2.3.

### Additional algorithmic description topic(s)

When encoding the sequence, the encoder executes the polyphase decomposition step on the input resolution sequence to produce the low resolution frames. Figure 3 shows the logic implemented in the encoder.

Figure 3 - High-level encoding loop block diagram

The lr frame i (see figure above) is the ith frame of the coded sequence. Subsequently, this frame is added to the GoP. When sufficient pictures have been received, the GoP is compress otherwise the block "Compress GoP" returns without actually compressing the GoP. This behaviour is the one of the function compressing GoPs in HM software to the best of our knowledge.

## 360°

### Usage of projection format adaptation

The source videos are not preprocessed and are encoded in their original projection format, equirectangular projection (ERP) – without any padding.

### 360° Video specific encoding tools

The key advantage of the 360° category is the pixel resolution higher than 4K. The encoder does not implement 360° specific tools but merely leverages the extremely high resolution of the test sequences.

### Additional encoder optimization

When calculating metrics such as PSNR, a possible optimization to reduce memory consumption is to derive these metrics for the super frame based on the one computed for the low resolution frames. It can be indeed demonstrated that:

PNSRsuper=10log4-10log⁡i=0310-PNSRi/10
∀i∈0,3, i being the resolution component index

# Compression performance

## 360° Video: Constraint set 1 configuration relative to HM anchor

### Class 360

Table 1 - Constraint Set 1 over HM16.16

### Overall

Overall the at the same bitrates there is roughly a 0,52 dB loss on average (both WSPSNR and SPSNR) although it is important to note that the number of samples being encoded is quadrupled.

## 360° Video: Constraint set 1 configuration relative to JEM anchor

These results are not shown. Since the technique has been implemented in the HM software, it would not be relevant to compare it again JEM. Moreover, the technique is agnostic to the underlying coding architecture and could similarly be implemented in JEM or another codec.

# Decoder complexity analysis

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

Figure 4 shows the measured encoding times and Table 2 the raw data.

Figure 4 - Decoding times comparison graph in seconds

Table 2- Decoding times

|  | Anchor | P20 | Ratio |
| --- | --- | --- | --- |
| Balboa | 154,428 | 386,658 | 2,50 |
| Chairlift | 86,905 | 365,739 | 4,21 |
| Kiteflite | 74,436 | 402,109 | 5,40 |
| Harbor | 67,263 | 394,388 | 5,86 |
| Trolley | 70,621 | 396,733 | 5,62 |
| AVERAGE | 90,731 | 389,125 | 4,72 |

To generate the decoding times a batch file was prepared with all the decoding to run in parallel. The server then allocated each of the decoding processes to a physical core for the entire duration of the decoding. The decoder binary was always located on the SSD storage and writing occurred always to the RAID 5 disk of 6.6 TB. Writing of the YUV occurred in both cases both for the anchor decoding times and the P20 decoding times.

The decoding times were extracted from the HM log files.

Decoding times are naturally longer as there are four times more frames to decode and there is also the additional polyphase reconstruction step compared to the anchor.

## Description of computing platform used to determine decoding times reported in section 5.1

The server used to generate all results has the following properties (the results are the output of standard commands to view server information):

| Parameter | Sub parameter | Details |
| --- | --- | --- |
| Server model | Manufacturer: Dell Inc. Product Name | Dell Inc. PowerEdge R730 |
| CPU | Architecture CPU op-mode(s) Byte Order CPU(s) On-line CPU(s) list Thread(s) per core Core(s) per socket Socket(s) NUMA node(s) Vendor ID CPU family Model Model name Stepping CPU max MHz CPU min MHz BogoMIPS Virtualization L1d cache L1i cache L2 cache L3 cache | x86_64 32-bit, 64-bit Little Endian 64 0-63 2 16 2 2 GenuineIntel 6 63 Intel(R) Xeon(R) CPU E5-2698 v3 @ 2.30GHz 2 3600.000 1200.000 4600.63 VT-x 32K 32K 256K 40960K |
| OS details | OS Distribution Description | Ubuntu 16.04.4 LTS |
| Hard drive data | Model Number (OS) Model Number (storage) | Samsung SSD 850 EVO 120GB [RAID 5 6.6TB] Dell 1.2TB 10K SAS HDD, 12 Gbps, 1.2 TB, 10,000 rpm |
| Memory | Speed Size Memory Swap Total | 2133 MHz 10 x 32 GB 314 GB 9.8 GB 324 GB |

## Memory usage of decoder

Figure 5 shows the measured decoder memory and Table 3 the raw data in a table.

Figure 5 - Mean decoding memory usage comparison in GB

Table 3 - Mean decoding memory usage in GB

|  | Anchor | P20 | Ratio |
| --- | --- | --- | --- |
| Balboa | 1,022 | 0,628 | 0,61 |
| Chairlift | 1,012 | 1,094 | 1,08 |
| Kiteflite | 1,008 | 1,087 | 1,08 |
| Harbor | 1,009 | 1,090 | 1,08 |
| Trolley | 1,014 | 1,087 | 1,07 |
| AVERAGE | 1,013 | 0,997 | 0,99 |

The memory usage and the time calculation were measured in the same run. While the decoding processes ran, the program pidstat [1] periodically (every second) checked current memory consumption of each of these processes with the average of each of these measurements being made at the end.

The memory consumption is very similar for the four 8K sequences but is slightly larger since the GoPs are longer. For Balboa, the memory usage is significantly lower because the frames being encoded are 3072x1536 pixels which is significantly smaller than the 4096x2048 pixels in the anchor. This is then compensated by the fact that the decoding process takes much longer.

## Complexity characteristics

All the following characteristics are the same as the HEVC Test Model (HM):

- Complexity characteristics of decoder entropy decoding operation

- Complexity characteristics of decoder inverse quantization

- Complexity characteristics of decoder inverse transform operation

- Complexity characteristics of decoder-side motion refinement

- Complexity characteristics of decoder motion compensation

- Complexity characteristics of decoder intra-frame prediction operation

- Complexity characteristics of decoder in-loop filtering operation

The following characteristics are not applicable for this technique:

- Complexity characteristics of 360° video specific decoding tools

- Complexity characteristics of HDR specific decoding tools

## Complexity characteristics of additional decoder tools

### Polyphase reconstruction

The following is a C++ example function which performs the polyphase reconstruction in HM. The function accepts the low resolution frame, the index of this resolution component and the high resolution frame as input parameter. The first two parameters are constant since the goal of the function to copy the samples from the low resolution frame at the right location in the high resolution frame.

As can be seen, the polyphase reconstruction comes down to a sample copy performed by the instruction recomposed_picture[2 * j] = subsampled_picture[j]. Therefore, the cost in term of complexity is minimal for the decoder.

void add_polyphase_component(TComPicYuv const& subsampled_yuv, int const index, TComPicYuv& recomposed_yuv) { for (Int component = 0; component < recomposed_yuv.getNumberValidComponents(); ++component) { ComponentID const component_id = ComponentID(component); Pel const* subsampled_picture = subsampled_yuv.getAddr(component_id); Pel* recomposed_picture = recomposed_yuv.getAddr(component_id); Int const start_offset = (index % 2) + (index >> 1) * recomposed_yuv.getStride(component_id); recomposed_picture += start_offset; for (Int i = 0; i < subsampled_yuv.getHeight(component_id); ++i, recomposed_picture += 2 * recomposed_yuv.getStride(component_id), subsampled_picture += subsampled_yuv.getStride(component_id)) { for (Int j = 0; j < subsampled_yuv.getWidth(component_id); ++j) { recomposed_picture[2 * j] = subsampled_picture[j]; } } } }

## Degree of capability for decoder parallel processing

The presented technique allows two levels of parallelisation:

- Frame-level (depends on the codec design)

- Super frame-level (property of the technique)

The frame-level parallelisation is determined by the ability of the decoder to decode a single frame in parallel. Since this technique was implemented in HM, the well-know parallelisation tools can be used in conjunction of this technique.

The super frame-level of parallelisation is the ability of the decoder to process the low resolution frames and the super frames separately. Since the super frame reconstruction is an out of loop process, this process can run in parallel to the decoding process of the low resolution frames.

# Encoder complexity analysis

## Encoding time and measurement methodology

Figure 6 shows the measured encoding times and Table 4 the raw data.

Figure 6 - Encoding times comparison in seconds

Table 4 - Mean encoding memory usage in GB

|  | Anchor | P20 | Ratio |
| --- | --- | --- | --- |
| Balboa | 78956,472 | 117212,529 | 1,48 |
| Chairlift | 43270,588 | 105513,973 | 2,44 |
| Kiteflite | 32664,775 | 94956,288 | 2,91 |
| Harbor | 34288,003 | 97328,685 | 2,84 |
| Trolley | 34045,044 | 96882,18 | 2,85 |
| AVERAGE | 44644,976 | 102378,731 | 2,50 |

As before all the encodings were launched in parallel by a single bash script. A background script analysed every minute if the processes running were sharing a physical CPU core. If yes, these processes were moved to a separate core. This way, the variations in execution time due to the hyperthreading (two threads per core) were minimized to a negligible impact. It was indeed observed that when one CPU core (with two threads and hyperthreading enabled) had two threads with processes running on it, the execution time of these two encoding was slower than an encoding process running alone on a core. It is suspected that this is due to the limitation of hyperthreading for long running process and memory throughput in the core. The script also made sure to move the processes from one core to another core only if it was needed so that time due to the copying of the thread in the CPU cache was minimized and can be considered null over the duration of an encoding.

The encoding times were extracted from the HM log files.

Encoding times are longer because there are four times more frames to encode. Balboa has less of a noticeable difference and this is most likely because of the lower pixel resolution of the sequence.

## Description of computing platform used to determine encoding times reported in section 6.1

See section 5.2.

## Memory usage of encoder

The measured decoder memory usage is below in the graph and the raw data is in the table.

Figure 7 - Memory usage of the encoder in GB

Table 5 - Encoding times in seconds

|  | Anchor | P20 | Ratio |
| --- | --- | --- | --- |
| Balboa | 3,349 | 1,809 | 0,54 |
| Chairlift | 3,792 | 3,050 | 0,80 |
| Kiteflite | 3,794 | 3,042 | 0,80 |
| Harbor | 3,769 | 3,052 | 0,81 |
| Trolley | 3,797 | 2,944 | 0,78 |
| AVERAGE | 3,700 | 2,780 | 0,75 |

The memory usage was calculated simultaneously with the time calculation. While the encoding processes ran, in the background a program periodically (once a second) checked current memory consumption of each of these processes. Then the average of each of these measurements was made.

## Complexity characteristics of encoder intra-frame prediction type selection

Intra-frame prediction remains the same as HM. The only difference that occurs is when an Intra frame is decoded, every 128 low resolution frames, it must wait for the next 3 frames (presentation order) to be decoded to be display the corresponding super frame. This is further explained in section 7.1.

## Complexity characteristics of encoder motion estimation and motion segmentation selection

Motion estimation occurs as normal although it was noted that typically low resolution frames that reference other low resolution frames of the same super frame will typically have a uniform motion vector field. This in turn benefits motion vector encoding in these frames.

# Algorithmic characteristics

## Random access characteristics

The GOP structure for low resolution pictures can be seen below.

Figure 8 – GOP structure of encodings

The GoP structure much resembles that of the random_access_main10.cfg, a configuration file available from the HM software repository with some alterations. Changes have been made for 3 frames regarding their type – they have been changed from B types to P types – they are frames 1, 2 and 3. This was done because when an I frame is decoded originally it would be necessary to decode more frames (namely 4, 8 and 16) before it would be possible to decode 1, 2 and 3 and show the super frame. In the current configuration it is only necessary to decode frame 16 before 1, 2 and 3.

The I frame rate has also been changed. I frame frequency has been decreased to ¼, i.e. to every 128 frames instead of 32. Since the frame rate was quadrupled, this keeps the refresh period the same.

QP offset values have also been changed for some frames in light of experimental results. In particular, I frames have higher quality offset than normal (-8 vs -3) – the idea being that an I frame only comes every 128 frames in this configuration meaning it is even more important than normal. Also, some frames – 7, 11 and 15 have worse quality as their motion estimation is not as efficient as others, so a conscious choice was made to not spend more bits. This happens because the frames that they reference with the highest quality are diagonal to them, meaning that they are spatially furthest away. The figure below shows the phases that exist: the blue phase is normally the highest quality; the orange and purple phase can perform better motion estimation while the green phase is both a pixel lower and to the right meaning that motion estimation is not as efficient.

Figure 9 – Relation between the 4 phases – between green-blue and between purple-orange there is not as high a correlation as orange-blue, blue-purple, purple-green and green-orange

Important to highlight is the scalability feature previously mentioned and how this works with this GoP structure. Every fourth low resolution frame belongs to the 4K resolution (see blue phase in Figure 9) while the following 3 low resolution frames (orange, purple and green phase in Figure 9) act as enhancement layers. This means that by decoding frames with TId ≥ 0 then the user would get an 8K sequence while as TId ≤ 2 will get a 4K stream, the fact that the other frames are not decoded does not influence these frames. Figure 8 shows this by colouring in the base layer (BL) and not filling in the enhancement layer (EL). Also important to note is that other strategies for the QP offset can be chosen, for instance to balance the ratio BL bitrate vs. EL bitrate.

## Delay characteristics

Figure 10 – GOP structure with encode/decode order

The encoding order is illustrated in Figure 10. The random access time is defined as 128 frames, as this is the difference in time between the first and the next full I frame shown. This corresponds to 32 super frames (equal to 128 low resolution frames divided by 4) which is in line with the CfP requirements.

# Software implementation description

The polyphase decomposition and reconstruction constitutes a new library called super resolution in the HM software. All implantation was done in C++ and ran on Ubuntu 16.04 LTS.

# Highlighted aspects discussion

The technique presented here presents the following features:

- 6K/8K number of samples encoded in this proposal as opposed to the 4K samples in the anchors

- Maximum block size and input sequence resolution decoupling

- No apparent aliasing in low resolution frames

- Low complexity spatial scalability at the cost of relatively low compression efficiency loss

Figure 11 – Anchor vs P20 high-level comparison

Figure 11 shows the several operations performed by the technique and the anchor. The first observation is that the technique actually encodes an 8K sequence in terms of number of coded samples as opposed to 4K for the anchor. Even with four times more coded samples, the technique remains in a range of 11.4% to 26% for an average of 16.6% Y-BD-rate loss. A more relevant comparison for the technique might be against the HM16.16 with 8K ERP encoded sequences, although this is not provided in this contribution.

In a coding system like HM, the block size has a maximum value. This value is chosen in such a way that the encoding of the highest possible resolution video can be efficiently performed with this block size. That is, larger block sizes would not yield further compression efficiency due to the lack of correlation between pixels for intra coded blocks for instance. Consequently, this would lead the encoder to choose block sizes below this maximum size. Therefore, it is common to see the maximum block size of video coding system being increased as the highest possible video resolution supported increases as well. In this technique, the encoded frames are essentially downscaled versions by a factor 2 of the input video in both horizontal and vertical directions. This means that an existing coding system could efficiently exploit spatial redundancy using the current maximum block sizes.

This technique is applied to 360-degree video although not exclusive to this type of videos. Indeed, the technique is theoretically applicable to any video. However, the polyphase down-sampling used can generate aliasing in the low resolution frames when the input video contains high frequencies that are higher than twice of the subsampling rate of the polyphase filter. Therefore, the higher the input video resolution, the less aliasing is generated by the polyphase subsampling. In this 360-degree sequence, the encoding results do not show abnormal bitrate increase which are typical of the presence of aliasing the picture. As a result, the assumption can be made that the polyphase down-sampling as presented here does not cause aliasing when applied to the 8K ERP test sequences.

As listed in "Requirements for a Future Video Coding Standard v5", N17074, the requirements comprise spatial scalability. The desired goal of spatial scalability is to offer the possibility to decode a given bitstream in different resolutions. With the presented technique, the decoder can provide two output resolutions when decoding the bitstream where the low resolution is half of the high resolution. Although current scalable video coding system offer flexible resolutions, the practice shows that the industry relies on a list of few standardised resolutions. From the resolutions, many are defined by a factor 2 compared to the previous, e.g. 1920x1080 (Full HD) and 3840x2160 (4K). This make the presented technique suitable for a wide range of resolutions used in the industry. The figure below shows the percentage of the bitstreams – a 4K bitstream as the base layer and the additional bits for 8K as the enhancement layer in each of the highest quality encodings. Although true that the difference between 4K and 8K is very small (average of 4,6%) this is because encoding compression was the main goal in this submission. It is expected that if optimizations were made to more evenly distribute information throughout the 5 temporal layers there could be a more real life scalable solution.

Figure 12 – Bitrates of the 4K high quality and the remaining bits constituting the remaining 3 phases

As a side note there are many cases for spatial scalability such as;

Figure 13 - Zero-footprint cloud transcoding

Figure 14 - Single broadcast channel

Figure 15 - Adaptive streaming client

# Closing remarks

In the context of 8K video, it seems natural that all pixels should be encoded, and this submission aims to not only do that but also provide functionality beyond video compression. The results here shown exemplify a technique that allows for spatial scalability in a way that is easy to implement and at low cost of compression efficiency end encoder complexity (based on the HEVC standard) when compared to a solution that encodes one quarter of the pixels. There are undoubtedly optimizations to be made given the specific nature of the frame being encoded, i.e. coherent in time but slightly shifted in space, but as a proof of concept it is shown that this technique has the potential to treat 8K encodings efficiently and do not harm the signal with aliasing effect.

# Patent rights declaration(s)

TNO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex 1: Questionnaire

Proponents of CfP responses are required to answer the codec level questions, and are also encouraged to fill out the optional tool level questions in addition to the complexity analysis of their proposals. If some question is not applicable to your proposal, you may express that as well.

## Codec level questions

- Maximum number of reference frames used by encoder

Same as HM

- Maximum coding unit (e.g. CTU in HEVC)

Same as HM

- Minimum and maximum transform block size

Same as HM

- Minimum and maximum intra prediction block

Same as HM

- Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length

Same as HM

- List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category)

Same as HM

- List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples)

None specific to this technique. If exists, same as HM.

- List all tools that use non-rectangular (e.g. triangle) partitions

Same as HM, none.

- Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa)

All the input frames are resamples to a quarter of the number of samples. However, the current picture and their reference frames are always of the same pixel resolution.

- Indicate whether luma and chroma use separate partitioning structures

Same as HM.

- Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes)

Same as HM.

- If the coded picture resolution is different from the input picture resolution, list the coded picture resolution for those sequences (e.g. 360° video, input is 8K ERP, coded video is in 4K CMP converted from the 8K input)

When the input is WxH ERP, the coded video is W/2xH/2 ERP polyphase down-sampled at 4x the original framerate with 4 times more frames than the original sequence.

## Tool level questions

The technique has been implemented in HM and no aspect asked below of HM was modified.

- Partitioning structure

  - Whether the prediction unit partitioning is different from transform unit partitioning

  - Maximum tree depth

- Entropy coding

  - Total number of context models

  - Memory storage size for each context model

  - Range table size if table look up is used for range update

  - Initialization table size

  - Other memory associated with context model adaptation

- Control data derivation process

  - Temporal motion data storage granularity

  - Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-predictive reference picture are used, count this picture as 2)

- Decoder-side motion refinement

  - Line buffer size in terms of motion vector block lines (assuming horizontal motion vector block size is 4 luma samples)

  - Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement

  - Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size

- Intra prediction

  - Line buffer size in terms of luma and chroma sample lines

  - List all the filters used for intra prediction

  - Indicate whether there are cross-component dependencies in intra prediction reconstruction

- Inter prediction

  - Motion vector accuracy

  - Storage size for a motion vector component

  - List all the interpolation filters used for luma/chroma motion compensation

- Quantization

  - Storage size associated with quantization

- Transforms

- List all the combinations of transform block size and transform type for primary transforms

- List all the combinations of transform block size and transform type for secondary transforms

- List all the non-separable transforms used

- Indicate whether transforms can be fully/partially implemented using direct matrix multiply

- List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output)

- In-loop filtering

- List line buffer size in terms of luma and chroma lines (if any)

- List all the filters used for in-loop filtering

- Minimum processing unit size

- “tool-off” and “tool-on” tests

- List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools (see Table 1 below for test procedure)

Table 1. Test procedure of “tool-off” and “tool-on” tests for a CfP response

| step | “tool-off” test | “tool-on” test |
| --- | --- | --- |
| 1 | Identify what you consider major tools in a CfP response that contribute to compression efficiency gain, and create a test candidates list |  |
| 2 | Turn on all the tools in the test candidates list, run the CfP response software and compute PSNRs, bit rates, and run times | Turn off all the tools in the test candidates list, run the CfP response software, compute PSNRs, bit rates, and run times, and compute the BD-rate and run-time numbers relative to the HM anchor |
| 3 | Turn off a tool in the test candidates list, and keep all the other tools in the test candidates list on, run the CfP response software and compute the BD-rate and run-time numbers relative to the CfP response software with all tools on | Turn on a tool in the test candidates list, and keep all the other tools in the test candidates list off, run the CfP response software and compute the BD-rate and run-time numbers relative to the CfP response software with all tools of the test candidates list off (i.e. step 2) |
| 4 | Repeat step 3 until all the tools in the test candidates list are tested | Repeat step 3 until all the tools in the test candidates list are tested |
| 5 | Compose the test data |  |

# Reference

- pidstat - Report statistics for Linux tasks, source: http://sebastien.godard.pagesperso-orange.fr/man_pidstat.html

[end of document]