| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0032-v1 |
| --- | --- |

| Title: | Description of SDR video coding technology proposal by IEEE 1857.10 Study Group |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Wu, Dong Liu, Jizheng Xu, Bin Li, Houqiang Li, Zhibo Chen, Li, Fangdong Chen, Yuanying Dai, Lei Guo, Ye Li, Yue Li, Jianping Lin, Changyue Ma, Ning Yan University of Science and Technology of China, Hefei, China Wen Gao, Siwei Ma, Ruiqin Xiong, Yiqun Xu, Jiahao Li Peking University, Beijing, China Xiaopeng Fan, Na Zhang, Yang Wang, Tao Zhang, Min Gao Harbin Institute of Technology, Harbin, China Zhenzhong Chen, Yan Zhou, Xiang Pan, Yiming Li, Feiyang Liu, Yingbin Wang Wuhan University, Wuhan, China | Tel: Email: | +86-15755129766 +86-18156517881 fengwu@ustc.edu.cn dongeliu@ustc.edu.cn |
| Source: | University of Science and Technology of China (USTC) |  |  |

_____________________________

- Abstract

This document describes the proposed SDR video coding technology as the response to the JVET-H1002 Joint call-for-proposal by IEEE 1857.10 Study Group. The proposal is referred to as Deep Learning-Based Video Coding (DLVC), because it contains two coding tools, convolutional neural network-based loop filter (CNNLF) and convolutional neural network-based block adaptive resolution coding (CNN-BARC), which are based on deep convolutional neural network (CNN). In addition to the two CNN-based coding tools, a set of regular coding tools are proposed, focusing on block partition, inter coding, loop filtering and background modeling. It is reported that the proposal achieves the BD-rate reduction of 11.0%, 9.3%, 11.8%, for SDR-A CS1, SDR-B CS1 and SDR-B CS2, respectively, compared with JEM anchor. And it achieves the BD-rate reduction of 42.5%, 36.8% and 33.0%, respectively, compared with HM anchor. The compression performance of each individual technique including the two CNN-based tools is also reported in detail in this document.

- Contents

Abstract	i

Contents	iii

1	Introduction	1

2	Decoder algorithm description	1

2.1	General algorithm description	1

2.1.1	Bitstream structure	1

2.1.2	Entropy decoding	5

2.1.3	Conversion process from entropy decoded data to symbols	6

2.1.4	Partition structure	8

2.1.5	Decoder-side motion refinement	9

2.1.6	In-loop filtering	12

2.1.7	Additional tools	13

2.1.8	Additional algorithmic description topics	13

3	Encoder algorithm description	16

3.1	General algorithm description	16

3.1.1	Partitioning structure selection	16

3.1.2	Motion estimation and motion segment selection	17

3.1.3	In-loop filtering type selection and parameter estimation	18

3.1.4	Additional encoder optimization	19

3.1.5	Additional algorithmic description topic(s)	20

4	Compression performance	21

4.1	SDR: Constraint set 1 configuration relative to HM anchor	21

4.1.1	Class SDR-A (RD-curves)	21

4.1.2	Class SDR-B (RD-curves)	21

4.1.3	Overall (BD-rate summary tables)	22

4.2	SDR: Constraint set 2 configuration relative to HM anchor	23

4.2.1	Class SDR-B	23

4.2.2	Overall	23

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	24

4.3.1	Class SDR-A	24

4.3.2	Class SDR-B	25

4.3.3	Overall	25

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	26

4.4.1	Class SDR-B	26

4.4.2	Overall	26

5	Decoder complexity analysis	27

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	27

5.2	Description of computing platform used to determine decoding times reported in section 5.1	29

5.3	Memory usage of decoder	29

5.4	Complexity characteristics of decoder-side motion refinement	29

5.4.1	DMVRI	29

5.5	Complexity characteristics of decoder motion compensation	29

5.5.1	FRUCI	29

5.5.2	MERGEI	29

5.5.3	AFFINEI	29

5.6	Complexity characteristics of decoder in-loop filtering operation	29

5.6.1	CNNLF	29

5.6.2	NF	30

5.7	Complexity characteristics of additional decoder tools	30

5.7.1	BCBR	30

5.7.2	BARC	30

5.7.3	TT	30

5.7.4	FBT	30

6	Encoder complexity analysis	30

6.1	Encoding time and measurement methodology	30

6.2	Description of computing platform used to determine encoding times reported in section 6.1	32

6.3	Memory usage of encoder	32

6.4	Complexity characteristics of encoder motion estimation and motion segmentation selection	32

6.4.1	General	32

6.4.2	Complexity characteristics of usage of decoder-side motion refinement	33

6.5	Complexity characteristics of encoder in-loop filtering type selection and parameter estimation	33

6.5.1	CNNLF	33

6.5.2	NF Filter	33

6.6	Complexity characteristics of additional encoder tools	33

6.6.1	BCBR	33

6.6.2	BARC	33

6.6.3	TT	34

6.6.4	FBT	34

7	Software implementation description	34

8	Highlighted aspects discussion	34

8.1	CNN-BARC	34

8.2	CNNLF	34

9	Closing remarks	34

10	Acknowledgement	35

11	Patent rights declaration(s)	35

12	Annex: Questionnaire	36

12.1	Codec level Q&A	36

12.2	Tool level Q&A	37

12.2.1	Category 1. Partitioning structure	37

12.2.2	Category 2. Entropy coding	37

12.2.3	Category 3. Control data derivation process	37

12.2.4	Category 4. Decoder-side motion refinement	38

12.2.5	Category 5. Intra prediction	39

12.2.6	Category 6. Inter prediction	39

12.2.7	Category 7. Quantization	39

12.2.8	Category 8. Transforms	39

12.2.9	Category 9. In-loop filtering	41

12.2.10	Category 10. “tool-off” and “tool-on” tests	42

Introduction

This contributed document describes the proposal for SDR video coding technology by the IEEE 1857.10 Study Group.

The proposal is built upon the reference model—JEM version 6.0 with no change on the existing techniques in JEM 6.0, but with added techniques including

- convolutional neural network-based loop filter (CNNLF)

- convolutional neural network-based block adaptive resolution coding (CNN-BARC)

- triple-tree partition (TT)

- forced boundary partition (FBT)

- non local filter (NF)

- frame-rate up-conversion improvement (FRUCI)

- decoder side motion vector refinement improvement (DMVRI)

- merge improvement (MERGEI)

- affine improvement (AFFINEI)

- block-composed background reference (BCBR)

The proposal is referred to as Deep Learning-Based Video Coding (DLVC), because the added CNNLF and CNN-BARC techniques are based on deep learning, or more specifically, based on deep convolutional neural network (CNN) models that are built by learning technology.

This proposal reports that, compared with the specific JEM anchor, the BD-rate reduction is 10.95%, 9.27%, 11.83%, for SDR-A Constraint Set 1, SDR-B Constraint Set 1, SDR-B Constraint Set 2, respectively; compared with the specific HM anchor, the BD-rate reduction is 42.45%, 36.81%, and 33.00%, respectively. This proposal also reports the BD-rate reduction of each individual technique, especially the CNN-based techniques.

Decoder algorithm description

General algorithm description

Bitstream structure

Several SPS-level, picture-level, and CTU-level flags are added into the bitstream for the added techniques. Most of them are simple binary flags.

SPS level flags

The following table lists the added SPS level flags (in red).

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| BCBR_enable_flag | u(1) |
| CNNLF_enable_flag | u(1) |
| BARC_intra_luma_enable_flag | u(1) |
| NF_enable_flag | u(1) |
| … |  |
| } |  |

Picture level flags

The following table lists the added picture level flags (in red).

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if (NF_enable_flag) { |  |
| adaptive_NF_flag | u(1) |
| if (adaptive_NF_flag == FALSE) { |  |
| NF_frame_flag | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

CTU level flags

The following table lists the added CTU level flags (in red) and their locations.

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| … |  |
| if (BCBR_enable_flag && slice_type != I) |  |
| background_flag_ctu | ae(v) |
| if (BARC_intra_luma_enable_flag && slice_type == I) { |  |
| barc_flag | ae(v) |
| if (barc_flag) |  |
| cnn_up_flag | ae(v) |
| } |  |
| if (CNNLF_enable_flag) { |  |
| cnnlf_luma_flag | ae(v) |
| cnnlf_chroma_flag | ae(v) |
| } |  |
| if (adaptive_NF_flag) { |  |
| NF_block_size_indicator | u(2) |
| for (idx=0; idx<block_count; idx++) |  |
| NF_block_flag | ae(v) |
| } |  |
| coding_tree(xCtb, yCtb, CtbLog2SizeY, CtbLog2SizeY, 0) |  |
| } |  |

| coding_tree( x0, y0, log2CbSizeX, log2CbSizeY, depth ) { |  |
| --- | --- |
| split _flag | ae(v) |
| If (split _flag == NO_SPLIT) |  |
| coding_uint(x0, y0, log2CbSizeX, log2CbSizeY) |  |
| Else { |  |
| split_mode_flag | ae(v) |
| If (split_mode_flag == QT_SPLIT){ |  |
| coding_tree(x0, y0, log2CbSizeX0, log2CbSizeY0, depth+1) |  |
| coding_tree(x1, y1, log2CbSizeX1, log2CbSizeY1, depth+1) |  |
| coding_tree(x2, y2, log2CbSizeX2, log2CbSizeY2, depth+1) |  |
| coding_tree(x3, y3, log2CbSizeX3, log2CbSizeY3, depth+1) |  |
| } |  |
| Else { |  |
| BT_TT_split_dir_flag | ae(v) |
| If (BT_TT_split_dir_flag == HOR){ |  |
| BT_TT_flag | ae(v) |
| if (BT_TT_flag == BT){ |  |
| coding_tree(x0, y0, log2CbSizeX0, log2CbSizeY0, depth+1) |  |
| coding_tree(x1, y1, log2CbSizeX1, log2CbSizeY1, depth+1) |  |
| } Else{ |  |
| coding_tree(x0, y0, log2CbSizeX0, log2CbSizeY0, depth+1) |  |
| coding_tree(x1, y1, log2CbSizeX1, log2CbSizeY1, depth+1) |  |
| coding_tree(x2, y2, log2CbSizeX2, log2CbSizeY2, depth+1) |  |
| } |  |
| } |  |
| Else { |  |
| BT_TT_flag | ae(v) |
| if (BT_TT_flag == BT){ |  |
| coding_tree(x0, y0, log2CbSizeX0, log2CbSizeY0, depth+1) |  |
| coding_tree(x1, y1, log2CbSizeX1, log2CbSizeY1, depth+1) |  |
| } |  |
| Else{ |  |
| coding_tree(x0, y0, log2CbSizeX0, log2CbSizeY0, depth+1) |  |
| coding_tree(x1, y1, log2CbSizeX1, log2CbSizeY1, depth+1) |  |
| coding_tree(x2, y2, log2CbSizeX2, log2CbSizeY2, depth+1) |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

| coding_unit(x, y, CUWidth, CUHeight) { |  |
| --- | --- |
| … |  |
| FRUCI() |  |
| MERGEI() |  |
| ... |  |
| Affine_flag |  |
| if (Affine_flag){ |  |
| Affine_para_flag | ae(v) |
| } |  |
| } |  |

| FRUCI() { |  |
| --- | --- |
| if (num_ref_idx_List0>1 && num_ref_idx_List1>1) { |  |
| ... |  |
| if (FRUCMode == FRUC_ template) |  |
| FRUC_template_uni_flag | ae(v) |
| else { |  |
| FRUC_merge_flag | ae(v) |
| if (FRUC_merge_flag) |  |
| FRUC_template_uni_flag | ae(v) |
| } |  |
| } |  |
| } |  |

| MERGEI() { |  |
| --- | --- |
| if (CUWidth>= 16 ¦¦ CUHeight>= 16) { |  |
| merge_complex_flag | ae(v) |
| if (merge_complex_flag) |  |
| merge_complex_index | ae(v) |
| } |  |
| } |  |

Entropy decoding

All the SPS level flags are written by plain bins without CABAC. At CTU level, for some of the added bins, there are added designed context models, while the other bins are written in bypass mode (i.e. equal probability).

TT

The split_mode_flag is decoded using 3 context models: the left, above or none, and the BT_TT_split_dir_flag is decoded using 5 context models for different split directions: quad, horizontal BT or TT, vertical BT or TT, and the BT_TT_flag is decoded using 3 context models for BT/TT depth 0, 1 and above 2.

BCBR

At CTU level, one new flag, background_flag_ctu, is decoded through CABAC. There is 1 context model for this flag.

CNNLF

Both cnnlf_luma_flag and cnnlf_chroma_flag are decoded through CABAC. There are 3 context models for each, similar to the skip flag.

NF

The NF_block_flag is decoded through CABAC with 1 context model. Other bins are decoded with equal probability.

MERGEI

If the width or the height of the current CU is larger than or equal to 16, a flag is decoded whether the current CU uses complex merge mode. If the current CU uses complex merge mode, the complex merge index is further decoded to indicate which candidate is used.

merge_complex_flag uses 3 context models, similar to the skip flag. For the complex merge index, 8 candidates are used, which are equally binarized into 3 bins. The first bin is decoded using 1 context model. The following two bins are bypass decoded.

AFFINEI

In the AMVP mode, if the affine flag of the current coding unit is true, an AFFINE_PARA_FLAG is further parsed to indicate whether the current coding unit will use the four-parameter affine mode or the six-parameter affine mode. AFFINE_PARA_FLAG uses 3 context models, similar to the skip flag.

Conversion process from entropy decoded data to symbols

Motion parameters

MERGEI

The complex merge mode combines several blocks with similar motions into a single region. As a result, all blocks within this region share the same motion information, and thus the motion parameters need to be transmitted only once. This method considers bilinear interpolation model, six-parameter affine model and four-parameter affine model simultaneously.

The control points are needed to determine the complex motion models. Therefore, the first step of the complex merge mode is to determine the candidates of the control points. The candidates of the control points are shown in Fig. 1. CPk represents the kth control point. The spatial candidates for predicting the motion information of CP1, CP2, and CP3 are shown in Fig. 1(a). The temporal candidate position for predicting the motion information of CP4 is shown in Fig. 1(b).

Fig. 1. Candidate positions for the complex merge mode (a) spatial (b) temporal

The motion information of each control point is determined in the following priority order.

- For CP1, the checking priority is B2, A2, and B3.

- For CP2, the checking priority is B0 and B1.

- For CP3, the checking priority is A0 and A1.

- For CP4, TRB is used.

The control points are then used to construct the candidate list. The candidate model list is constructed in the following order. Only when the motion information of all the selected control points for the candidate model can be derived and they are not identical in at least one reference list, the model can be put into the candidate model list. The decoded complex merge index can be derived from the 3 bins using equal length binarization. When the candidate index is equal to the decoded complex merge index, the process stops.

- Affine (CP2, CP3)

- Affine (CP1, CP3)

- Affine (CP1, CP2, CP3)

- Affine (CP1, CP2)

- Affine (CP2, CP4);

- Affine (CP3, CP4);

- Affine (CP1, CP4);

- Bilinear

- Affine (CP1, CP2, CP4);

- Affine (CP2, CP3, CP4);

- Affine (CP1, CP3, CP4);

The motion information of the control points and the high-order motion model are then used to interpolate the MV of each sub-block in the current coding unit. As mentioned above, the interpolation function can be bilinear, six-parameter affine, and four-parameter affine. The reference index with the highest utilization rate among all the reference indices is selected as the final reference index. The control points with different reference indices are scaled to the final reference index.

AFFINEI

- AFFINE_MERGE_IMPROVE: instead of finding the first neighboring block with affine mode, the improvement tries to find the neighboring block with the largest coding unit size as the affine merge candidate.

- AFFINE_AMVP_IMPROVE: add the neighboring blocks with affine mode to the affine AMVP candidate list similar to the traditional AMVP process.

The detailed affine AMVP candidate list construction process is as follows.

First, the below left neighboring block is checked whether it is using the affine motion model and has the reference index with the current reference index. If it does not exist, the left neighboring block is then checked in the same way. If it does not exist, the below left neighboring block is checked whether it is using the affine motion model and with the different reference index. If it exists, the scaled affine motion vector is added to the reference picture list. If it does not exist, the left neighboring block with be checked in the same way.

Second, the above right neighboring block, the above neighboring block, and above left neighboring block will then be checked in the same way.

If we have found two candidates after the above processes, we will have finished constructing the affine AMVP candidate lists. If we have not found two candidates, the original process in the JEM software will be performed to construct the affine AMVP candidate lists.

- AFFINE_SIX_PARAM: besides the four-parameter affine motion model, six-parameter affine motion model is also added as an additional model.

The six parameter affine motion model is as follows.

(1)

Since there are six-parameters in the above motion model, three motion vectors in the above left position MV0, the above right position MV1, and the bottom left position MV2 are needed to determine the model. The three motion vectors are determined in a similar way as the two motion vectors in the four-parameter affine motion model. Note that the affine model merge is always set as the six-parameter affine motion model.

- AFFINE_CLIP_REMOVE: delete the motion vector constraints for all the affine motion vectors. Let the motion compensation process handle the motion vector constraints themselves.

In-loop filter parameters

NF_block_size_indicator uses two bins. After decoding the two bins, the NF block size is determined by:

| NF_block_size_indicator | 00 | 01 | 10 | 11 |
| --- | --- | --- | --- | --- |
| NF block size | 128 | 64 | 32 | 16 |

Partition structure

QT-BT/TT tree

In this contribution, a QT plus BT/TT tree (QT-BT/TT) structure is proposed. In addition to quad-tree and binary-tree, triple-tree partition is employed to capture objects in the center of blocks while the quad-tree and binary-tree are splitting along block center. There are horizontal and vertical triple-tree splits, as reported in Fig. 2 (d) and (e).

(a)            (b) 	        (c) 	          (d) 	     (e)

Fig. 2. (a) quad-tree split; (b) horizontal binary-tree split; (c) vertical binary-tree split;

(d) horizontal triple-tree split; (e) vertical triple-tree split.

In the proposed coding tree structure, a CTU is first portioned using a Quad-tree, and any of the Quad-tree leaf nodes is further partitioned by a BT/TT tree with maximum BT/TT depth of 3. For boundary CTU, the maximum BT/TT depth is extended (c.f. 2.1.4.2). In the BT/TT tree, any node may be split by one of the four BT/TT split modes. The allowed BT/TT split modes of a BT/TT tree node is decided by the width and height of the node, the aspect ratio of the node, and split mode of the parent node and that of a previous node.

A BT/TT leaf node is a coding unit (CU). Prediction and transform are both applied on a CU in the same way as JEM. The width and height of all CUs are power of 2.

FBT

This proposal presents that for non-boundary CTU, the maximum BT/TT depth is set as uiMaxBTD = 3 as JVET CTC. For CUs which are located in boundary CTU, uiMaxBTD is set according to the following equation:

uiMaxBTD = max(3, ForceSplitLevel – ForceQTLevel + Extension)

Where the ForceSplitLevel is needed split level to ensure the boundary CU is fully located within the frame boundary, the ForceQTLevel is the needed QT split level before forced BT/TT, the Extension controls BT/TT partition freedom.

For instance, in HD sequences, 4 level (56 = 32+16+8, 8=128>>4 and the forced level is 4) forced splitting (QT/BT) are needed until the boundary CU is fully located within the frame boundary. When the ForceQTLevel and Extension are 0 and 2, the uiMaxBTD is 6 (4 – 0 + 2), as shown in Fig. 3, the yellow CU has used 2 levels BT split, the maximal number of possible BT/TT levels is 4; the green CU has used 3 levels BT//TT split, 3 levels BT/TT partition is possible; the blue CU has used 4 levels BT/TT split, 2 levels BT/TT is possible.

Fig. 3. Example of ForceQTLevel is 0 and Extension is 2

Decoder-side motion refinement

DMVRI

In the decoder-side motion vector refinement (DMVR) technique in JEM, the two motion vectors are further refined by a bilateral template matching process. This contribution proposes two improvements to the current DMVR method, including an iterative motion vector search method and a DMVR_BM method.

Iterative motion vector search method

Step1:

Obtain the forward prediction and the backward prediction according to the motion information of the current block.

Step2:

Derive the predicted block of the current block using the forward and backward prediction by the average weighting method. The predicted block is used as the template for further motion search. Let predSamplesL0 and predSamplesL1 be the forward and backward prediction, for the pixel at position (x,y), the average weighting operation is conducted as follow:

predSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1,
								( predSamplesL0[ x ][ y ] + predSamplesL1[ x ][ y ] + offset2 )  >>  shift2 )

where

Clip3( x, y, z ) =

Shift2 is the parameter for bit depth shifting and offset2 equals 1<<(shift2-1).

Step3:

Iteratively perform integer-pixel and half-pixel refinement. The total iteration number is set to be N. When the iteration time is odd, conduct motion search for list0, else for list1.

(1) integer-pixel motion search from several candidates

Fig. 4. The illustration of the points to be searched

As shown in Fig. 4, (0,0) is the start point to be searched, in addition the four points around the start points are also used for search. According to the matching cost obtained for the 4 surrounding points, one of the 4 corner points (-1, -1), (-1, 1), (1, -1), (1, 1) is included as the sixth point in the search space.

predSamplesListX’[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1,
								( predSamplesListX[ x ][ y ]  + offset2 )  >>  shift2 )

(2) compute the SAD of the predicted blocks in (1) and the template generated in Step2, and the candidate which leads to minimum SAD is selected as the best candidate and the second best candidates.

(3) derive the motion vector in the other direction

MV(1-X)'=MV(1-X)-(MVX' – MVX)

where X is 0 or 1, MVX and MVX' represent the initial and best motion vector in the direction X, and MV(1-X) is the initial motion vector in the direction (1-X).

(4) update the predicted blocks in both directions with the motion vector derived in (2) and (3)

(5) generate the predicted block of the current block by averaging the two prediction generated in (4)

(6) determine the half-pixel search points according to the best candidate (P(Best) in Fig. 5) and the second best candidate (P(Second best) in Fig. 5) in (2).

Fig. 5. The illustration of half-pixel motion search

(7) motion search using the half-pixel search points and compute the SAD of the candidate predicted blocks and the template generated in (5), and the candidate which leads to minimum SAD is selected as the best candidate.

(8) update the motion vector of the other direction as (3)

(9) update the predicted blocks in both directions with the motion vector derived in (8)

(10) generate the predicted block of the current block by averaging the two prediction generated in (9)

DMVR_BM method

The steps of DMVR_BM method are as follow. It should be noted that the modification of the DMVR_BM method is in the step4 below.

step1:

Obtain the initial motion information of the current block, including the reference index and the motion vector. Determine the location of the reference block in both directions.

step2:

Obtain the start points for motion search of the forward reference picture. Note the forward motion vector be (MV0x，MV0y) and the location of the current block is (B0x, B0y), then the forward search start point is (MV0x+B0x, MV0y+B0y).

step3:

Obtain the start points for motion search of the backward reference picture. Note the backward motion vector be (MV1x，MV1y) and the location of the current block is (B1x, B1y), then the backward search start point is (MV1x+B1x, MV1y+B1y).

step4:

Determine the most matching blocks in both directions under the MVD mirror constraint. Let the position offset to the start point in the forward reference be MVD0 (delta0x, delta0y) and the position offset to the start point in the backward reference be MVD1 (delta1x, delta1y), then:

delta0x = - delta1x；

delta0y = - delta1y；

Calculate the SAD between the candidates in the forward reference and the backward reference, the candidates which lead to the minimum SAD will be selected the best candidates. And the best forward motion vector and backward motion vector can be obtained accordingly.

FRUCI

This technique deals with two aspects of the current FRUC template matching mode: (1) bi-directional template matching which jointly exploits list0 and list1 reference pictures; (2) selection between the bi-directional template matching and the uni-directional matching based on SAD.

In the original implementation of the template matching mode, the matched template is searched from list0 and list1, respectively. This is a kind of uni-directional template matching since only one list of reference pictures is utilized during each template matching. Such a uni-directional template matching is suitable to derive motion information for uni-prediction instead of bi-prediction. However, motion compensation prediction for the existing FRUC template matching mode always uses bi-prediction to generate predictor for current block. In this contribution, this issue for FRUC template matching mode is solved by introducing bi-directional template matching and adaptive selection between uni-prediction and bi-prediction.

The modification of the template matching FRUC mode is only applied to the PU-level motion refinement. Sub-PU level motion refinement keeps unchanged. The process of deriving the motion vector of the current PU is as follow:

(i) uni-directional template matching

• Use the template Tc to find the matched template T0 and corresponding motion vector MV0 from   reference picture in list0 and calculate the cost0.

• Use the template Tc to find the matched template T1 and corresponding motion vector MV1 from   reference picture in list1 and calculate the cost1.

(ii) bi-directional template matching

• Update the template for list1:

T’C = 2*TC – T0

• Use the template T’C to find the matched template T1 and corresponding motion vector MV1 from   reference picture in list1 and calculate the costBi:

(iii) FRUC template mode selection:

If costBi<= factor×min(cost0, cost1),

bi-prediction with MV0 and MV1 is used;

Else If cost0<=cost1,

uni-prediction with MV0 in list0 is used;

Else,

uni-prediction with MV1 in list1 is used;

In-loop filtering

NF

NF filter is used as in-loop filter after SAO and before ALF. The NF filtering algorithm is performed for the whole frame. If adaptive_NF_flag is false and NF_frame_flag is true, the whole reconstructed frame is replaced by the filtered reconstructed frame. If adaptive_NF_flag is true, for each NF block, if the NF_block_flag is true, the reconstructed block is replaced by the filtered reconstructed block.

- First, a reconstructed video frame from a previous stage is divided into overlapped S image patches. Each patch is reorganized into a vector. Then for each patch, K nearest neighbors are found according to the Euclidean distance, thus generating patches groups. Here, contains all the patches which called a group.

- After that, patches in each group can be represented sparsely. Singular value decomposition(SVD) is applied to get sparse representation.

Whereis a diagonal matrix.

- Last, thresholding the singular values of image patches in each group is used. In this proposal, there are hard and soft thresholding methods: and , and the τ is thresholding-level parameter.

For hard thresholding,

For soft thresholding,

Where theand is standard deviation of noise and original patch, and Bs is patch size.

- After thresholding singular values, the restored group of image patches is given by:

Where theis thresholded .Then the final filtered image is the average of overlapped patches.

Additional tools

BCBR

If BCBR_enable_flag is true, then the number of reference pictures increases by 1. A long-term reference is added into the reference list.

A long-term reference picture, also called background reference picture, is added as a long-term reference picture.

The background reference picture is put at the last of the reference list, thus, its reference index is the number of reference pictures minus 1.

All the motion compensation processes are not changed, only referring to the background reference picture, if needed.

Additional algorithmic description topics

BCBR

BCBR is currently used for Constraint Set 2 (low-delay coding) only. After finishing the first (I) frame reconstruction, the I frame is copied to the background reference picture. Next, after finishing one frame reconstruction, the flag background_flag_ctu is checked for each CTU. If this flag is true, the CTU is copied to replace the corresponding location in the background reference picture.

CNN-BARC

The basic idea of CNN-BARC is that, one CTU can be either normally compressed, or down-sampled, compressed, and then up-sampled. This is called block adaptive resolution coding (BARC).

Currently, CNN-BARC is adopted in I frame only. It is intentionally turned off in P and B frames.

Currently, CNN-BARC is applied for luma only, but not for chroma.

All the down-sampling and up-sampling are at the scaling factor of 2×2, i.e. down- or up-sampling by a factor of 2 in both horizontal and vertical directions.

The basic usage of CNN-BARC is as follows:

If(barc_flag){

xDecodeCU(depth=1,width=half_ctu_width, height=half_ctu_width); // this is down-sampled coded

}else{

xDecodeCU(depth=0,width=ctu_width, height=ctu_width); // this is normally coded

}

If(barc_flag){

xDecompressCU(depth=1, width=half_ctu_width, height=half_ctu_width); // this is down-sample coded

If(cnn_up_flag){ // need to do up-sampling

cnn_up_sample_rec();

} else{

filter_up_sample_rec();

}

}

else {

xDecompressCU(depth=0, width=ctu_width, height=ctu_width); // this is normally coded

down_sample_rec();

}

During implementation, we have a new buffer called pic_yuv_down_scale_ref for every I frame. This frame stores the down-sampled reconstructed picture. Its memory is then 1/4 of the memory of normal picture.

The reference sample derivation for each CU and the filling of pic_yuv_down_scale_ref buffer is specified as follows:

If barc_flag is true – derive reference samples from pic_yuv_down_scale_ref buffer and store the reconstruction value in the buffer.

If barc_flag is false – derive reference samples from pic_yuv_rec (normal reconstructed picture), fill the pic_yuv_down_scale_ref by using down_sample_rec(), since the following CTUs may be down-sample coded and need the down-sampled reconstruction.

down_sample_rec(): if the CTU is normally compressed, then the reconstruction resolution is the same as original resolution. The down-sampling is implemented by another CNN, called CRCNN, which is depicted in Fig. 6. Please note that Conv1 and Conv11 adopt convolutions with a stride value of 2, which effectively reduces the size by a factor of 2. Similar to the previously mentioned CNNLF, the operations are done in floating-point numbers. Only one network model is used here. Before input, the samples are divided by (1<<bitDepth-1) for normalization. And after output, the samples are multiplied by (1<<bitDepth-1) and rounded to integers.

Fig. 6. CRCNN used for down-sampling

filter_up_sample_rec(): if the CTU is down-sample coded, the reconstruction needs to be up-sampled to its original resolution. There are two up-sampling methods for choice. If cnn_up_flag is false, use a fixed FIR interpolation filter for up-sampling. The FIR filter coefficients are:

-1,4,-11,40,40,-11,4,-1

cnn_up_sample_rec(): if the CTU is down-sample coded and cnn_up_flag is true, use a CNN model to up-sample the CTU. There are 4 network models corresponding to QPs {27,32,37,42}, a model with the QP value closest to the QP of the current I frame is chosen for the up-sampling. Again, operations are in floating-point numbers, division by (1<<bitDepth-1) before input and multiplication by (1<<bitDepth-1) after output are used, too. The network structure is shown in Fig. 7. There are 16 residual blocks, each of which has the same structure as shown in Fig. 7 (b). The network configuration is as follows: convolutional kernel size is 3×3, deconvolutional kernel size is 6×6, number of feature map is 64 (but 1 for the last Conv).

(a) The overall EDSR structure           (b) The residual block structure

Fig. 7. CNN for up-sampling

CNNLF

In the contribution JVET-I0022 (Convolutional Neural Network Filter (CNNF) for intra frame), a CNN-based filter is proposed for intra frame. In this contribution, we apply CNN-based filter for not only intra frame but also inter frame.

CNNLF is an in-loop filter, it is applied after DF, before SAO.

CNNLF operates in floating-point numbers, indicating potential dependence on computational environment to ensure reproducible results.

For different QPs, different CNN models are prepared. Currently there are four models with QP equal to 28, 34, 40, 46. One model is selected according to the QP of the current frame, it selects the model that has most similar QP.

After deblocking, the luma samples of current picture is re-scaled by dividing by (1<<bitDepth-1) so that the sample values are in the range of [0, 1]. Then, the entire luma picture is sent to the following CNN:

Fig. 8. CNNLF

As shown in Fig. 8, the adopted CNN has 34 layers. The output of the last layer is added back to the input. The configuration of the network is shown as follows.

| Residual block number | 16 |
| --- | --- |
| Filter size in each convolutional layer | 3*3 |
| Filter number in each convolutional layer | 64 (1 in the last layer) |

The CNN output is re-scaled by multiplying by (1<<bitDepth-1) and then rounding, and stored in a buffer.

Next, for each CTU, if cnnlf_luma_flag is true, the luma samples in this CTU are replaced by the corresponding samples in the buffer; if cnnlf_luma_flag is false, the samples are not changed.

The chroma (Cb and Cr) samples are processed similarly. Cb is processed by the network, then Cr. For each CTU, if cnnlf_chroma_flag is true, both Cb and Cr samples are replaced; otherwise, both Cb and Cr samples are not changed.

Encoder algorithm description

General algorithm description

Partitioning structure selection

TT

In encoder, the RDO is used to determine the best partition for an input block. First, the block is encoded without any partition in intra or inter mode. And save the best mode. When the block is less than maximum allowed binary tree size, the block tries the horizontal and vertical binary split. Note that the binary split is recursively performed until reaching the minimum allowed size. Similarly, when the block is less than maximum allowed triple tree size, the block tries triple split recursively. The best partition structure is decided according to the RD-costs of each partition structure.

FBP

This proposal presents that for non-boundary CTU, the maximum BT/TT depth is set as uiMaxD = 3 as JVET CTC. For CUs which are located in boundary CTU, uiMaxD is set according to the following equation:

uiMaxD = max(3, ForceSplitLevel – ForceQTLevel + Extension)

Where the ForceSplitLevel is needed split level to ensure the boundary CU is fully located within the frame boundary, the ForceQTLevel is the needed QT split level before forced BT/TT, the Extension controls BT/TT partition freedom.

The process of FBP can be illustrated as below: if the block is not at bottom, right and corner boundary, normal partition is performed. And the FBP is carried out recursively until reaching the stop condition. If the block is at bottom, right or corner, then judge the boundary location of the block. If the block is at corner, then forced quad-tree partition is performed. And FBP is again carried out recursively. If the block is at bottom or right, then judge the condition of uiDepth< =FQT_LV -1(FQT_LV indicates the forced QT level used before forced BT/TT). If the condition is true, then forced quad-tree partition is performed, and then FBP is again carried out recursively. If false, then forced binary or triple tree partition is performed, and then FBP is again carried out recursively.

Motion estimation and motion segment selection

General

FRUCI

In the encoder side, the three FRUC mode including FRUC_MERGE_BILATERALMV, FRUC_MERGE_TEMPLATE and FRUC_MERGE_TEMPLATE_UNI are checked, and the FRUC mode which leads to the minimum RD cost is selected as the final FRUC mode.

MERGEI

In the encoder side, we first construct the complex merge candidate lists. Then for all the candidates in the candidate list, they will be measured using SATD to derive the candidate with the minimum cost. Then the full RDO will be used to calculate the cost of the best candidate and compare with the other modes. Both the modes with and without residue are measured to achieve the best performance.

AFFINEI

- AFFINE_SIX_PARA: In the affine merge mode, if the affine merge candidate exists, it is always considered as six-parameter affine mode.

In the affine AMVP mode, we develop the gradient-based fast affine motion estimation algorithm to update the three motion vectors. The essence of the gradient based fast affine motion estimation algorithm is to adjust the three motion vectors MV0, MV1, and MV2 according to the MSE between the current block and the prediction block. Under the gradient based motion estimation algorithm, only a system of linear equations is needed to be solved to derive the updated motion vectors. Since the proposed algorithm will update the three motion vectors simultaneously, the proposed algorithm can reduce the encoder complexity significantly. And according to our experiments, the proposed algorithm will converge very quickly. Only 3 and 5 iterations are performed for uni-directional motion estimation and bi-directional motion estimation, respectively. Under each iteration, the sum of the absolute difference of the residue plus the square root of lambda multiplied by the bits of the motion vectors will be calculated to obtain the best affine motion vectors.

The sum of the absolute difference plus the square root of lambda multiplied by the bits of the motion vectors will also be used to compare between the six-parameter affine motion model and four-parameter affine motion model. The one with smaller cost will be chosen as the final affine motion model to compare with the normal inter mode.

- AFFINE_CLIP_REMOVE:  Delete the affine motion vector constraints for all the affine motion vectors in the encoder motion estimation process.

In-loop filtering type selection and parameter estimation

CNNLF

Distortion of the CTU with the proposed CNN filter and distortion without CNN processing are compared. If the image quality is improved after being processed by the network, CNNLF is enabled for the CTU. Note that the distortion of luma and chroma are calculated separately.

NF

Fig. 9. The Flowchart of Non Local Filter

The flowchart of NF filter is shown in Fig. 9. The RDO is used to determine whether to use NF filter.

First the filtering algorithm is performed for the whole reconstructed frame after deserving the filter parameters from quantization parameter and reconstructed picture.

Then calculate the RD-costs of reconstructed whole frame, filtered reconstructed whole frame, and adaptive NF filtering for different block size: CTU size, CTU size>>1, CTU size>>2 and CTU size>>3 (Depth0, Depth1, Depth2 and Depth3).

If the minimum of them is reconstructed frame, the whole frame NF flag is false. If the minimum of them is filtered reconstructed frame, the whole frame flag is true. If neither of them, the adaptive NF flag is true. And according to the minimum of the cost of different size filtering, the application map, i.e. the NF flag map for different size, is gotten, and each block is replaced by the corresponding filtered block according to the NF flag.

Additional encoder optimization

(Proponents are requested to report any additional encoder optimization methods used in their proposal that are not used by the anchors, for example, non-automatic means of encoding parameter optimization, multi-pass encoding, content-adaptive GOP, special subjective optimization techniques, etc.)

BCBR

Fig. 10. Framework of the proposed background CTU detection method

The framework of the proposed background CTU detection method is depicted as Fig. 10. Initially, the background reference picture is the reconstructed IDR picture. When encoding a new picture, this contribution proposes a background CTU detection method to select the background CTUs. After encoding current picture, the background reference picture in the location of the CTU which is detected as background CTU is updated by the reconstructed ones. And the new background picture will be indicted as the long term reference and utilized for encoding of the next picture.

The framework of the proposed background CTU detection method is summarized as follows.

1)Virtual Background Anchor Generation

For every newly arrived pixel intensity pt, it is matched against themean of the existing model to verify whether the intensity isbelong to the category. When the difference is within 1.5 times of standard deviation σt of the Gaussian model, or a relatively small value, the new observation is accepted in the model. In this case, the mean μt, variance σt2 and the correspondingweight wtof that model should be updated. In other cases, namely, the difference is not small enough, the processing of the pixel intensity will be based on the weight of that model. When the model is deployed with a high weight, i.e., not less than th2, it is considered as a background model, and the pixel of current picture will be rejected; otherwise, the model parameters will be initialized again using the pixel intensity.

2)Obtaining BCTU Candidates with Spatial-temporal Constraints

When the collocated CTU in the VBA has enough high weight pixels, i.e., more than 80%, the CTU of VBA is employed as an anchor to test current CTU. And if mean square error (MSE) between the current CTU and collocated CTU in the VBA does not less than or equal to th1, it should be removed from the BCTU candidate set.When the collocatedCTU in the VBA has no enough high-weight pixels, the mean square error between the current CTU and collocated CTU in the previous coded picture is considered instead. If the luminance MSE is less than or equal to th1 and chrominance MSE is less than or equal to th1/4, it should be kept in the BCTU candidate set. Otherwise, it is removed, and there is no need to check the spatial condition followed.

Considering that some parts of moving foreground may keep still in a short period, the gradient smoothness needs to be tested in the CTU boundaries even if the current CTU satisfies the temporal conditions above. Specifically, the current BCTU will be put into the background reference, and an edge detector (Sobel’s operator) is applied at the boundaries of the current BCTU to identify whether or not there are abnormal edges. The abnormal edge pixel refers to the pixel at the boundaries of the current BCTU, which has a high gradient, but its neighboring pixels are not edge pixels. If such abnormal edge pixel exists, the spatial condition is not satisfied and the current BCTU is no longer considered.

Finally, when the collocated CTU in the BCBR has been updated, the luminance MSE between the detected CTU and the collocated CTU in the BCBR has to be calculated. When it exceeds th1, the detected CTU is a new background region, and it should be kept to conduct the following process. Otherwise, the similar part has been maintained in the BCBR, and there is no need to update the collocated CTU in the BCBR for this case. The thresholdsth1 and th2are setto 6 and 10,respectively.

3)Selecting the Optimal BCTUs with Ranking Process

If the number of BCTU candidates that satisfy the spatial-temporal conditions is not more than the specified number of BCTU for one picture, i.e., Mbctu, which is one twelfth of theCTU number in a frame, all the candidates will be selected as the optimal BCTUs. Otherwise, a ranking process has to be conducted on the candidates, and the Mbctu CTUs with the highest scores will be the final choices. Finally, the Mbctu candidates that have higherscores will be selected to compose the background reference in the scheme.

Additional algorithmic description topic(s)

CNN-BARC

The proposed intra frame coding method is depicted in Fig. 11, each CTU can be coded at its original full-resolution version or the low-resolution version. For the low-resolution coding scheme, the CTU should be down-sampled by half before coding and up-sampled to its original resolution after reconstruction. Here we propose two down/up-sampling methods, i.e. the FIR based and CNN based. For FIR based method, the down/up-sampling is implemented by FIR interpolation filters, while for CNN based method, we use convolutional neural network to first down-sample the original pixel and then up-sample the output pixel. In Fig. 11 both the “Low-resolution coding” block and “Full-resolution coding” block mean the standard codec of JEM7.0. In our scheme, two Rate-Distortion Optimization (RDO) processes are used to select the best coding mode. The first RDO is used to determine the CNN-based down/up-sampling method or the FIR-based. The second RDO determines whether the CTU is coded at low-resolution or full-resolution. The second stage up-sampling in Fig. 11 is used to refine the regions of each up-sampled CTU around its boundaries.

Fig. 11. The framework of proposed intra frame coding method

- FIR based method

The down-sampling filter coefficients are defined as follows:

4,0,-12,0,40,64,40, 0, -12,0,4,

The up-sampling filter coefficients are defined the same as in 2.1.11.

- CNN based method

Two convolutional neural networks are trained for the CNN-based down/up sampling method, as is the same as decoder side as in 2.1.11.

Compression performance

SDR: Constraint set 1 configuration relative to HM anchor

Note that the red line is our proposed DLVC, and the black line is HM anchor. The horizontal axis is bit rate, and the vertical axis is PSNR. There are four rates in our tests corresponding to four blocks.

Class SDR-A (RD-curves)

Class SDR-B (RD-curves)

Overall (BD-rate summary tables)

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -38.42% | -45.57% | -50.10% | 1.97 | 1.33 | 1.61 |
| CatRobot1 | -46.87% | -59.26% | -53.80% | 1.89 | 1.12 | 1.61 |
| DaylightRoad2 | -47.34% | -60.65% | -46.19% | 1.35 | 1.35 | 0.89 |
| ParkRunning3 | -38.43% | -29.65% | -31.58% | 1.30 | 0.52 | 0.46 |
| Campfire | -41.20% | -38.07% | -54.53% | 1.25 | 1.46 | 1.73 |
| BQTerrace | -37.53% | -50.06% | -56.97% | 1.16 | 0.95 | 1.10 |
| RitualDance | -33.30% | -42.15% | -45.71% | 1.85 | 1.25 | 1.55 |
| MarketPlace | -35.34% | -51.78% | -49.99% | 1.29 | 0.94 | 0.94 |
| BasketballDrive | -37.09% | -52.86% | -51.20% | 1.37 | 1.21 | 1.62 |
| Cactus | -40.80% | -50.92% | -46.82% | 1.74 | 0.85 | 1.27 |
| Average SDR-A | -42.45% | -46.64% | -47.24% | 1.55 | 1.16 | 1.26 |
| Average SDR-B | -36.81% | -49.55% | -50.14% | 1.48 | 1.04 | 1.30 |
| Average all | -39.63% | -48.10% | -48.69% | 1.52 | 1.10 | 1.28 |

SDR: Constraint set 2 configuration relative to HM anchor

Note that the red line is our proposed DLVC, and the black line is HM anchor. The horizontal axis is bit rate, and the vertical axis is PSNR. There are four rates in our tests corresponding to four blocks.

Class SDR-B

Overall

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -35.56% | -52.79% | -59.11% | 1.10 | 0.66 | 0.73 |
| RitualDance | -28.23% | -41.19% | -43.19% | 1.51 | 1.04 | 1.26 |
| MarketPlace | -26.85% | -46.65% | -47.83% | 0.89 | 0.62 | 0.70 |
| BasketballDrive | -32.98% | -50.91% | -52.69% | 1.29 | 1.14 | 1.66 |
| Cactus | -41.37% | -56.66% | -54.68% | 1.79 | 0.90 | 1.37 |
| Average SDR-B | -33.00% | -49.64% | -51.50% | 1.32 | 0.87 | 1.14 |

SDR: Constraint set 1 configuration relative to JEM anchor

Note that the red line is our proposed DLVC, and the black line is JEM anchor. The horizontal axis is bit rate, and the vertical axis is PSNR. There are four rates in our tests corresponding to four blocks.

Class SDR-A

Class SDR-B

Overall

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -7.97% | -3.45% | -6.23% | 0.31 | 0.07 | 0.14 |
| CatRobot1 | -12.90% | -16.46% | -16.52% | 0.35 | 0.17 | 0.33 |
| DaylightRoad2 | -11.82% | -10.12% | -9.58% | 0.23 | 0.12 | 0.13 |
| ParkRunning3 | -10.93% | -6.19% | -5.95% | 0.31 | 0.09 | 0.07 |
| Campfire | -11.13% | -5.12% | -10.30% | 0.24 | 0.17 | 0.21 |
| BQTerrace | -10.34% | -10.82% | -7.22% | 0.25 | 0.13 | 0.07 |
| RitualDance | -8.41% | -9.17% | -9.08% | 0.40 | 0.19 | 0.22 |
| MarketPlace | -9.20% | -8.87% | -9.69% | 0.27 | 0.11 | 0.13 |
| BasketballDrive | -8.93% | -14.18% | -13.86% | 0.25 | 0.21 | 0.31 |
| Cactus | -9.47% | -11.50% | -11.29% | 0.30 | 0.12 | 0.21 |
| Average SDR-A | -10.95% | -8.27% | -9.72% | 0.29 | 0.13 | 0.18 |
| Average SDR-B | -9.27% | -10.91% | -10.23% | 0.29 | 0.15 | 0.19 |
| Average all | -10.11% | -9.59% | -9.97% | 0.29 | 0.14 | 0.18 |

SDR: Constraint set 2 configuration relative to JEM anchor

Note that the red line is our proposed DLVC, and the black line is JEM anchor. The horizontal axis is bit rate, and the vertical axis is PSNR. There are four rates in our tests corresponding to four blocks.

Class SDR-B

Overall

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -13.36% | -12.00% | -17.21% | 0.32 | 0.10 | 0.12 |
| RitualDance | -8.50% | -11.10% | -12.03% | 0.40 | 0.22 | 0.28 |
| MarketPlace | -7.90% | -6.53% | -10.72% | 0.24 | 0.07 | 0.12 |
| BasketballDrive | -10.30% | -10.23% | -15.97% | 0.32 | 0.16 | 0.37 |
| Cactus | -19.09% | -26.21% | -26.51% | 0.66 | 0.29 | 0.48 |
| Average SDR-B | -11.83% | -13.22% | -16.49% | 0.39 | 0.17 | 0.27 |

Decoder complexity analysis

Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

| Constraint Set 1 |  |  | HM16.16 | DLVC |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Target rates (kbps) | Dec T [s] | Dec T [s] | Dec |
| SDR-A | FoodMarket4 | 3800 | 505.78 | 744284.20 | 1471.56 |
|  |  | 2200 | 229.15 | 789508.99 | 3445.40 |
|  |  | 1400 | 497.17 | 816099.32 | 1641.48 |
|  |  | 950 | 227.19 | 916093.64 | 4032.21 |
|  | CatRobot1 | 6000 | 300.27 | 633825.81 | 2110.87 |
|  |  | 3800 | 346.55 | 607796.44 | 1753.85 |
|  |  | 2400 | 293.45 | 574920.92 | 1959.19 |
|  |  | 1500 | 251.03 | 657239.14 | 2618.17 |
|  | DaylightRoad2 | 5500 | 227.47 | 606436.60 | 2666.04 |
|  |  | 3500 | 226.75 | 644938.87 | 2844.31 |
|  |  | 2200 | 225.75 | 655520.43 | 2903.78 |
|  |  | 1400 | 223.83 | 637957.66 | 2850.14 |
|  | ParkRunning3 | 10000 | 260.11 | 494134.22 | 1899.68 |
|  |  | 6000 | 175.94 | 528009.07 | 3001.14 |
|  |  | 3300 | 203.71 | 522123.30 | 2563.13 |
|  |  | 2000 | 193.33 | 555358.22 | 2872.56 |
|  | Campfire | 10000 | 274.70 | 324874.36 | 1182.65 |
|  |  | 6000 | 112.44 | 352176.86 | 3132.19 |
|  |  | 3300 | 91.85 | 337455.29 | 3674.02 |
|  |  | 2000 | 93.38 | 319298.96 | 3419.31 |
| SDR-B | BQTerrace | 1500 | 80.20 | 201365.40 | 2510.92 |
|  |  | 1000 | 44.55 | 220294.42 | 4945.44 |
|  |  | 600 | 41.91 | 185447.00 | 4424.78 |
|  |  | 400 | 46.16 | 194496.97 | 4213.27 |
|  | RitualDance | 3800 | 96.97 | 195209.73 | 2013.01 |
|  |  | 2300 | 78.73 | 172840.55 | 2195.39 |
|  |  | 1500 | 51.64 | 183017.71 | 3543.90 |
|  |  | 900 | 50.02 | 181509.97 | 3628.60 |
|  | MarketPlace | 2000 | 53.27 | 182631.03 | 3428.47 |
|  |  | 1200 | 98.03 | 180224.85 | 1838.52 |
|  |  | 800 | 50.30 | 196848.88 | 3913.89 |
|  |  | 500 | 42.88 | 205703.15 | 4797.18 |
|  | BasketballDrive | 3500 | 67.21 | 158698.84 | 2361.31 |
|  |  | 2000 | 44.43 | 150224.10 | 3381.14 |
|  |  | 1200 | 53.24 | 161398.93 | 3031.76 |
|  |  | 800 | 43.25 | 174118.37 | 4025.67 |
|  | Cactus | 2000 | 76.69 | 158986.72 | 2073.05 |
|  |  | 1200 | 44.70 | 189511.47 | 4239.35 |
|  |  | 800 | 68.19 | 163485.41 | 2397.67 |
|  |  | 500 | 43.13 | 172326.48 | 3995.88 |
|  | Average SDR-A |  | 226.37 | 561645.33 | 2481.07 |
|  | Average SDR-B |  | 56.41 | 176025.00 | 3201.23 |
|  | Average all |  | 113.00 | 318466.57 | 2818.24 |

| Constraint Set 2 |  |  | HM16.16 | DLVC |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Target rates (kbps) | Dec T [s] | Dec T [s] | Dec |
| SDR-B | BQTerrace | 1500 | 52.71 | 409128.54 | 7761.73 |
|  |  | 1000 | 54.02 | 228030.63 | 4221.38 |
|  |  | 600 | 46.83 | 196297.33 | 4191.97 |
|  |  | 400 | 86.89 | 225707.04 | 2597.77 |
|  | RitualDance | 3800 | 74.29 | 260958.22 | 3512.55 |
|  |  | 2300 | 100.87 | 206133.06 | 2043.61 |
|  |  | 1500 | 47.29 | 181064.68 | 3828.49 |
|  |  | 900 | 46.88 | 257869.96 | 5500.99 |
|  | MarketPlace | 2000 | 64.84 | 227611.83 | 3510.36 |
|  |  | 1200 | 62.09 | 188455.36 | 3035.39 |
|  |  | 800 | 54.70 | 228486.57 | 4177.47 |
|  |  | 500 | 43.43 | 290071.03 | 6679.66 |
|  | BasketballDrive | 3500 | 53.92 | 166131.26 | 3081.13 |
|  |  | 2000 | 49.10 | 304277.13 | 6196.84 |
|  |  | 1200 | 44.50 | 178875.75 | 4019.68 |
|  |  | 800 | 48.50 | 178788.85 | 3686.29 |
|  | Cactus | 2000 | 64.75 | 176840.26 | 2731.33 |
|  |  | 1200 | 40.22 | 179963.06 | 4474.47 |
|  |  | 800 | 34.62 | 173897.20 | 5023.03 |
|  |  | 500 | 31.96 | 153636.04 | 4806.98 |
|  | Average all |  | 52.98 | 213998.06 | 4039.60 |

Description of computing platform used to determine decoding times reported in section 5.1

System: Microsoft Server 2012 64 bits

CPU: Inter Xeon CPU E5-2690 v2, 16 cores

Memory: 65528 MB

Hard drive: 1 TB

Compiling: Visual studio 2015 (x64)

Memory usage of decoder

As measured, the highest memory usage of decoder is about 14.6 GB.

Complexity characteristics of decoder-side motion refinement

DMVRI

Let the decoding time of the original DMVR technique be O(T). Since an iterative search method is adopted in the proposed improvement, therefore the decoding time increase is proportional to the total iterative number: O(N*T), where N is the total iterative number.

Complexity characteristics of decoder motion compensation

FRUCI

There is negligible complexity change in the proposed FRUC mode improvement.

The main complexity of FRUC comes from the motion search of template matching, while both the uni-directional template matching and bi-directional template matching need to find two template in the two directions, therefore the complexity of uni-directional and bi-directional template matching is similar.

MERGEI

With the complex merge mode, the percentage of the codingunitsusinghigh-ordermotionmodelssuchasfour-parameteraffine, six-parameter affine, and bilinear motion model will increase by a little. Therefore, this mode will lead to a little decoding time increase. Besides, there are also some complexity increase caused by the candidate construction.

AFFINEI

There is slight complexity increase for the decoder under the affine improvement. Due to enrollment of the six-parameter affine motion model, there will be more blocks choosing affine motion model. Since the motion compensation of the affine motion model is slightly more complex compared with the translational motion model, the decoder complexity will increase slightly.

Complexity characteristics of decoder in-loop filtering operation

CNNLF

The proposed CNNLF is not optimized for computational speed, and thus the decoding time of the proposed method is much longer than the highly optimized codec.

The complexity of CNNLF decoding mainly comes from the calculation of the network. The image width I_w, image height I_h, the input filters F_in and output filters F_out per layer are related to complexity. The complexity of the CNN is

O(I_w*I_h*F_in*F_out*CNN_layers).

NF

In NF filter algorithm, the first parameter group such as local variance value from reconstructed frame. The second group parameters are derived from codec QP. The above complexity is negligible. Then running the NF filter for the whole frame: the clustering patches are performed 2D Harr transforming and collaborative Wiener filtering in frequency domain, and then 2D Harr inverse transforming into pixel domain. There are also many copying operations. The complexity is reportedly increased largely compared with anchor which has small complexity.

Complexity characteristics of additional decoder tools

BCBR

The additional complexity of decoder comes from the following aspects:

Updating the background reference picture with CTU block according to the decoded background flag;

BARC

- Complexity characteristics for down/up-sampling process with FIR filter

The interpolation with FIR filter is implemented by weighted summation, and complexity is

O(H∙W∙T)

Where H, W, T means the CTU height, width and number of filter taps.

- Complexity characteristics for down/up-sampling process with CNN

Convolution complexity is measured by FLOPs(floating-point operations),for a single convolutional layer,

O(M2∙K2∙Cin∙Cout)

Where M is the size of output feature maps, K is the kernel size, Cin and Cout represent the input channel number and output channel number.

For the EDSR used, M=half_ctu_size(or ctu_size after deconv layer), K=3, Cin=Cout=64 except for the first layer and the last layer (for the first layer Cin=1 and for the last layer Cout=1)

For the CRCNN used, M=half_ctu_size, K and Cin, Cout are specialized for each layer as in Fig.2

TT

The decoder chooses the partition structure from binary tree or triple tree according to the symbol when splitting within the prediction-tree. After the partition structure is set as one of binary or triple tree, the below process is similar to the JEM. The complexity of this technique in the decoder increases reportedly slightly.

FBT

The process of boundary partition structure is set according the decoded split symbol. It is the same as JEM and its complexity is negligible.

Encoder complexity analysis

Encoding time and measurement methodology

| Constraint Set 1 |  |  | HM16.16 | DLVC |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Target rates (kbps) | Enc T [s] | Enc T [s] | Enc |
| SDR-A | FoodMarket4 | 3800 | 149547.57 | 2726511.00 | 18.23 |
|  |  | 2200 | 119759.45 | 2327131.00 | 19.43 |
|  |  | 1400 | 133420.39 | 2070806.00 | 15.52 |
|  |  | 950 | 111645.59 | 1733010.00 | 15.52 |
|  | CatRobot1 | 6000 | 119613.45 | 2988278.00 | 24.98 |
|  |  | 3800 | 137784.54 | 2392171.00 | 17.36 |
|  |  | 2400 | 130587.95 | 1974545.00 | 15.12 |
|  |  | 1500 | 125572.12 | 1776469.00 | 14.15 |
|  | DaylightRoad2 | 5500 | 126857.19 | 3133227.00 | 24.70 |
|  |  | 3500 | 119799.57 | 2587927.00 | 21.60 |
|  |  | 2200 | 110627.35 | 2126203.00 | 19.22 |
|  |  | 1400 | 106391.44 | 1748435.00 | 16.43 |
|  | ParkRunning3 | 10000 | 122848.56 | 2946759.00 | 23.99 |
|  |  | 6000 | 95763.68 | 2552288.00 | 26.65 |
|  |  | 3300 | 98468.00 | 2072114.00 | 21.04 |
|  |  | 2000 | 100356.10 | 2030704.00 | 20.23 |
|  | Campfire | 10000 | 89012.63 | 2148008.00 | 24.13 |
|  |  | 6000 | 58086.72 | 1903489.00 | 32.77 |
|  |  | 3300 | 45642.27 | 1600282.00 | 35.06 |
|  |  | 2000 | 50464.95 | 1306567.00 | 25.89 |
| SDR-B | BQTerrace | 1500 | 26568.52 | 513955.00 | 19.34 |
|  |  | 1000 | 19423.11 | 455369.00 | 23.44 |
|  |  | 600 | 19087.25 | 367548.00 | 19.26 |
|  |  | 400 | 19951.53 | 348935.00 | 17.49 |
|  | RitualDance | 3800 | 41063.91 | 1140688.00 | 27.78 |
|  |  | 2300 | 42095.13 | 947426.00 | 22.51 |
|  |  | 1500 | 28686.30 | 860840.00 | 30.01 |
|  |  | 900 | 36768.04 | 679683.00 | 18.49 |
|  | MarketPlace | 2000 | 31855.66 | 783175.00 | 24.59 |
|  |  | 1200 | 36211.05 | 630075.00 | 17.40 |
|  |  | 800 | 29777.66 | 535040.00 | 17.97 |
|  |  | 500 | 22078.55 | 460460.00 | 20.86 |
|  | BasketballDrive | 3500 | 33035.75 | 932921.00 | 28.24 |
|  |  | 2000 | 23325.10 | 726502.00 | 31.15 |
|  |  | 1200 | 26656.05 | 690850.00 | 25.92 |
|  |  | 800 | 22897.43 | 593868.00 | 25.94 |
|  | Cactus | 2000 | 27012.60 | 643506.00 | 23.82 |
|  |  | 1200 | 21374.77 | 578060.00 | 27.04 |
|  |  | 800 | 25732.09 | 493808.00 | 19.19 |
|  |  | 500 | 19572.87 | 447810.00 | 22.88 |
|  | Average SDR-A |  | 102940.96 | 2154778.73 | 20.93 |
|  | Average SDR-B |  | 26822.91 | 611114.39 | 22.78 |
|  | Average all |  | 52546.90 | 1147526.16 | 21.84 |

| Constraint Set 2 |  |  | HM16.16 | DLVC |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Target rates (kbps) | Enc T [s] | Enc T [s] | Enc |
| SDR-B | BQTerrace | 1500 | 29079.08 | 777872.00 | 26.75 |
|  |  | 1000 | 30650.68 | 508467.00 | 16.59 |
|  |  | 600 | 26737.17 | 512627.00 | 19.17 |
|  |  | 400 | 37177.77 | 418843.00 | 11.27 |
|  | RitualDance | 3800 | 47525.08 | 1175031.00 | 24.72 |
|  |  | 2300 | 45009.29 | 1014627.00 | 22.54 |
|  |  | 1500 | 27845.01 | 852922.00 | 30.63 |
|  |  | 900 | 29399.39 | 968755.00 | 32.95 |
|  | MarketPlace | 2000 | 35792.55 | 953848.00 | 26.65 |
|  |  | 1200 | 33834.41 | 685582.00 | 20.26 |
|  |  | 800 | 32673.46 | 720796.00 | 22.06 |
|  |  | 500 | 27135.60 | 650442.00 | 23.97 |
|  | BasketballDrive | 3500 | 27640.34 | 970059.00 | 35.10 |
|  |  | 2000 | 29974.86 | 878366.00 | 29.30 |
|  |  | 1200 | 26368.70 | 676215.00 | 25.64 |
|  |  | 800 | 25506.58 | 587820.00 | 23.05 |
|  | Cactus | 2000 | 45427.82 | 758825.00 | 16.70 |
|  |  | 1200 | 27058.45 | 700666.00 | 25.89 |
|  |  | 800 | 19576.22 | 543425.00 | 27.76 |
|  |  | 500 | 20004.25 | 467139.00 | 23.35 |
|  | Average all |  | 30374.33 | 713600.54 | 23.49 |

Description of computing platform used to determine encoding times reported in section 6.1

System: Microsoft Server 2012 64 bits

CPU : Inter Xeon CPU E5-2690 v2, 16 cores

Memory: 65528 MB

Hard drive: 1 T

Compiling: Visual studio 2015 (x64)

Memory usage of encoder

The memory usage is not measured during encoding.

Complexity characteristics of encoder motion estimation and motion segmentation selection

General

FRUCI

For each CU, one more FRUC mode will be checked, therefore the encoding time of FRUC mode will increase by half of the time of the original FRUC merge mode.

Complex merge

For each CU, this method needs to check all the complex merge candidates in the candidate list. The main complexity comes from the motion compensation of the high-order motion models. Since no motion estimation process is involved in the mode decision process, the complexity will only increase a little.

Affine improvement

Due to the enrollment of the six-parameter affine motion model in the AMVP process, more motion compensations and RDO operations will be performed in the encoding process. Therefore, the encoding complexity will increase a little. All the other affine improvements will not lead to obvious encoder complexity increase.

Complexity characteristics of usage of decoder-side motion refinement

DMVR

Let the decoding time of the original DMVR technique be O(T). Since an iterative search method is adopted in the proposed improvement, therefore the decoding time increase is proportional to the total iterative number: O(N*T), where N is the total iterative number.

Complexity characteristics of encoder in-loop filtering type selection and parameter estimation

CNNLF

The complexity of CNNLF encoding is mainly comes from the computation of the CTU distortion. It is

O(Pic_width*Pic_height).

NF Filter

In NF filter algorithm, there only once NF filtering for the whole frame. Then calculate six costs, that is, original, filtered for whole frame, blocks at depth 0, 1, 2 and 3. And then he best filter results are chosen. Although there will be many MSE calculating, compare and copy operations, the complexity is still negligible compared with anchor which has large complexity.

Complexity characteristics of additional encoder tools

BCBR

The additional complexity of encoder comes from the following aspects:

- Detecting the background CTU with the proposed method;

- Conducting rate-distortion optimization on the background reference picture.

BARC

For the encoding process, each CTU needs to be down/up sampled twice with FIR filter and CNN respectively. The down-up sampling process complexity is characterized in 5.11. Additional complexity comes from multi-compression of the CTU.

- Normal compress with original resolution

- Compress with FIR down-sampled low-resolution

- Compress with CNN down-sampled low-resolution

TT

In the proposed partition structure, a quad-tree leaf can split into binary tree or triple tree. And the RDO is used to select the best partition structure from binary or triple tree. Since the partition is recursively performed, the complexity is increased greatly.

FBT

This technique is proposed for the boundary partition. The BT/TT partition structure can be used for the bottom, right and corner boundary block. Because the number of performed blocks are small, and the partition are limited, the complexity is negligible.

Software implementation description

DLVC was developed on top of the JEM reference software. In addition, DLVC uses a well-known deep learning framework—Caffe to fulfill the CNN-BARC and CNNLF tools. Currently, DLVC was developed under Windows OS with Visual Studio 2015 and x64 configuration. The Caffe for Windows (https://github.com/BVLC/caffe/tree/windows) is adopted for DLVC. More specifically, the Caffe for Windows is compiled and built as a DLL, this DLL as well as Caffe’s dependencies DLL’s are necessary when running DLVC executables. In addition, the related header files and library files are included when compiling and building DLVC executables.

It is worth noting that Caffe for Windows provides the flexibility to use CPU or GPU. The current DLVC is however stick to CPU only.

Highlighted aspects discussion

CNN-BARC

1000 images from DIV2K dataset are collected for training the two CNNs in BARC. The dataset consists of 800 training images, 100 validation images, and 100 test images. Two loss functions, i.e. reconstruction loss Jrec and regularization loss Jreg are defined for training of the two neural networks

Jrec=gfX-X22

Jreg=fX-F(X)22

Where X is the original patch, and F(X) means the output of bicubic down-sampling patch. f∙ and g∙ represent the down-sampling and up-sampling by CNN, respectively. By iteratively training the two networks, finally one CRCNN model and four EDSR models (corresponding to four QPs) are obtained.

CNNLF

DIV2K dataset is used for intra picture training. For intra picture training, each original image is compressed by intra coding at four different quantization parameters (QPs): 28, 34, 40, and 46. For each QP, a separate network is trained out. Only the luminance channel (i.e. Y out of YUV) is considered for training. The original image and compressed image are both divided into 70*70 sub-images without overlap. Zero padding is used before each convolution so that the output is of the same size as the input, and therefore the loss is computed over the entire sub-image. Adam optimizer is used to train the model by setting learning rate to 0.0001.

Closing remarks

This contributed document summarizes the most important characteristics of DLVC, a proponent for compression technology beyond HEVC. Most noticeably, DLVC has two CNN-based tools namely CNN-BARC and CNNLF.

Acknowledgement

The IEEE 1857.10 Study Group is a sub-unit of the IEEE 1857 Working Group (http://www.ieee1857.org/). The Study Group had been working for over three years to research and develop video coding technology beyond HEVC. The authors of this proposal are very grateful to the efforts of all the members of the Study Group. Especially, the authors of this proposal would like to thank the following persons/institutions:

- Haitao Yang, from Huawei Technologies, Shenzhen, China, for co-chairing Study Group meetings and for coordinating the software development of this proposal.

- Li Song, from Shanghai Jiaotong University, Shanghai, China, for active contribution in several techniques including intra prediction tool and fractional interpolation tool.

- Yongfei Zhang, from Beihang University, Beijing, China, for active contribution in background modeling tool.

Patent rights declaration(s)

University of Science and Technology of China, Peking University, Harbin Institute of Technology, and Wuhan University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Annex: Questionnaire

Codec level Q&A

| Codec Level Q&A |  | HM | JEM | DLVC |
| --- | --- | --- | --- | --- |
| 1 | Maximum number of reference frames used by encoder | 4 | 4 | 5 |
| 2 | Maximum coding unit (e.g. CTU in HEVC) | luma 64×64 chroma 32×32 | luma 128×128 chroma 64×64 | luma 128×128 chroma 64×64 |
| 3 | Minimum and maximum transform block size | luma 4×4/32×32 chroma 4×4/16×16 | luma4×4/128×128 chroma 2×2/64×64 | luma4×4/128×128 chroma 2×2/64×64 |
| 4 | Minimum and maximum intra prediction block | luma 4×4 chroma 4×4 | luma 4×4 chroma 2×2 | luma 4×4 chroma 2×2 |
| 5 | Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length | luma 4×8/8×4 (uni-pred) and 8×8 (bi-pred), 8-tap Chroma 2×4/4×2 (uni-pred) and 4×4 bi-pred, 4-tap | luma 4×4 (uni/bi-pred), 8-tap Chroma 2×2 (uni/bi-pred), 4-tap | luma 4×4 (uni/bi-pred), 8-tap Chroma 2×2 (uni/bi-pred), 4-tap |
| 6 | List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category) | n/a | template matching in FRUC, LIC | template matching in FRUC, LIC |
| 7 | List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples, position unforeseeable) | n/a | 64-bit arithmetic for LIC, BIO and LMC | 64-bit arithmetic for LIC, BIO and LMC Floating-point operations for CNNLF, CNN-BARC |
| 8 | List all the tools that use non-rectangular (e.g. triangle) partitions | n/a | n/a | n/a |
| 9 | Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa) | n/a | n/a | n/a |
| 10 | Indicate whether luma and chroma use separate partitioning structures | no | Yes, in I-slices | Yes, in I-slices |
| 11 | Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes) | Entropy parsing depends on intra prediction mode reconstruction, which requires an intra MPM list derivation | Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation | Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation |

Tool level Q&A

Category 1. Partitioning structure

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 1. Partitioning structure |  | HM | JEM | DLVC |
| 1.a | Whether the prediction unit partitioning is different from transform unit partitioning | Yes, PU and TU partitioning for a CU can differ | No | No |
| 1.b | Maximum tree depth | CU maximum depth 4 TU maximum depth 3 PU maximum depth 1 | QT maximum depth 5 BT maximum depth 3 | QT maximum depth 5 BT/TT maximum depth 3 |

Category 2. Entropy coding

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 2. Entropy coding |  | HM | JEM | DLVC |
| 2.a | Total number of context models | 186 | 329 | 357 |
| 2.b | Memory storage size for each context model | 8-bit | 30-bit | 30-bit |
| 2.c | Range table size if table look up is used for range update | LPSTable[64×4×8bits]; RenormTable[32× 3bits] | LPSTable[512×64×9bits]; RenormTable[128×3bits] | LPSTable[512×64×9bits]; RenormTable[128×3bits] |
| 2.d | Initialization table size | 3×186×8 bits | 3×329×8 + 329×15×5 bits | 3×357×8+357×15×5 bits |
| 2.e | Other memory associated with context model adaptation | LPS transition table: 64*6 bits MPS transition table: 64*6 bits | n/a | n/a |

Category 3. Control data derivation process

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 3. Control data derivation process |  | HM | JEM | DLVC |
| 3.a | Temporal motion data storage granularity | 16×16 | 4×4 | 4×4 |
| 3.b | Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-directional reference picture are used, count this picture as 2) | 1 | 4 | 4 |

Category 4. Decoder-side motion refinement

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 4. Decoder-side motion refinement |  | HM | JEM | DLVC |
| 4.a | Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement. | n/a | - BIO 2×2D separable 6 taps filter (for gradients), mults per sample in refinement calculation 5+5*4 +1/8+2, minimum processing unit same as regular bi-pred, no SAD calculation - FRUC minimum processing unit 44, sequential PU followed by sub-PU level refinement, number of SAD calculations and additional MC interpolations per unit for MV refinement is not constrained (refinement is terminated when cost no longer decreases); bi-linear filter at MV refinement stage | 1. BIO 2×2D separable 6 taps filter (for gradients), mults per sample in refinement calculation 5+5*4 +1/8+2, minimum processing unit same as regular bi-pred, no SAD calculation 2. FRUC minimum processing unit 44, sequential PU followed by sub-PU level refinement, number of SAD calculations and additional MC interpolations per unit for MV refinement is not constrained (refinement is terminated when cost no longer decreases); bi-linear filter at MV refinement stage |
| 4.b | Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size. | n/a | - DMVR needs to access (H+7+2)×(W+7+2) MC instead of (H+7)×(W+7) in HEVC MC - FRUC not constrained | 1. DMVR needs to access (H+7+2)×(W+7+2) MC instead of (H+7)×(W+7) in HEVC MC 2. FRUC not constrained |

Category 5. Intra prediction

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 5. Intra prediction |  | HM | JEM | DLVC |
| 5.a | Line buffer size in terms of luma and chroma sample lines | luma 1, chroma 1 | luma 4, chroma 2 | luma 4, chroma 2 |
| 5.b | List all the filters used for intra prediction | Reference sample smoothing: {1,2,1}, Strong intra smoothing filter: 2-tap bilinear filter (only for 32×32 luma TU) Interpolation: 2×32×5 bits (bi-linear 32 phases) | Reference sample smoothing: {1,2,1}, {3,7,14,16,14,7,3}, {4,9,12,14,12,9,4} Interpolation: 2×4×32×8 bits (4 taps, 2 sets, 32 phase) | Reference sample smoothing: {1,2,1}, {3,7,14,16,14,7,3}, {4,9,12,14,12,9,4} Interpolation: 2×4×32×8 bits (4 taps, 2 sets, 32 phase) |
| 5.c | Indicate whether there are cross-component dependencies in intra prediction reconstruction | n/a | LM-CHROMA (Cb depends on Y, and Cr depends on Cb&Y) | LM-CHROMA (Cb depends on Y, and Cr depends on Cb&Y) |

Category 6. Inter prediction

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 6. Inter prediction |  | HM | JEM | DLVC |
| 6.a | Motion vector accuracy | luma¼pel chroma 1/8 pel | luma1/16 pel chroma 1/32 pel | luma1/16 pel chroma 1/32 pel |
| 6.b | Storage size for a motion vector component | 16-bit | 16-bit | 16-bit |
| 6.c | List all the interpolation filters used for luma/chroma motion compensation | luma 8-tap, 8×4×6 bits chroma 4-tap, 4×8×6 bits | luma 8-tap, 8×16×6 bits chroma 4-tap, 4×32×6 bits | luma 8-tap, 8×16×6 bits chroma 4-tap, 4×32×6 bits |

Category 7. Quantization

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 7. Quantization |  | HM | JEM | DLVC |
| 7.a | Storage size associated with quantization | n/a | n/a | n/a |

Category 8. Transforms

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 8. Transforms |  | HM | JEM | DLVC |
| 8.a | List all the combinations of transform block size and transform type for primary transforms | - 8-bit 4×4, 8×8, 16×16 and 32×32 DCT-II, - 8-bit 4×4 DST VII | HEVC transform mode (10-bit DCT-II), transform size combinations: - (4, 8, 16, 32, 64, 128) × (4, 8, 16, 32, 64, 128) TUs for luma CTUs - (2, 4, 8, 16, 32, 64) × (2, 4, 8, 16, 32, 64) TUs for inter-coded chroma CTUs - (2, 4, 8, 16, 32, 64,) × (2, 4, 8, 16, 32, 64) TUs for intra-coded chroma CTUs EMT transform mode (10-bit DST VII, DCT-V, DCT-VIII, DST-I), only applied to CTUs up to 64×64, transform size combinations: - (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TUs for luma CTUs | HEVC transform mode (10-bit DCT-II), transform size combinations: - (4, 8, 16, 32, 64, 128) × (4, 8, 16, 32, 64, 128) TUs for luma CTUs - (2, 4, 8, 16, 32, 64) × (2, 4, 8, 16, 32, 64) TUs for inter-coded chroma CTUs - (2, 4, 8, 16, 32, 64,) × (2, 4, 8, 16, 32, 64) TUs for intra-coded chroma CTUs EMT transform mode (10-bit DST VII, DCT-V, DCT-VIII, DST-I), only applied to CTUs up to 64×64, transform size combinations: - (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TUs for luma CTUs |
| 8.b | List all the combinations of transform block size and transform type for secondary transforms | n/a | 35×3 4×4 and 8×8 non-separable Hypercube-Givens Transform (HyGT) | 35×3 4×4 and 8×8 non-separable Hypercube-Givens Transform (HyGT) |
| 8.c | List all the non-separable transforms used | n/a | 4×4/8×8 HyGT | 4×4/8×8 HyGT |
| 8.d | Indicate whether transforms can be fully/partially implemented using direct matrix multiply | partial factorization equivalent to matrix multiplication | factorized form not equivalent to matrix multiplication | factorized form not equivalent to matrix multiplication |
| 8.e | List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output) | n/a | 4×4 HyGT2-interations, 8×8 HyGT4-interations | 4×4 HyGT2-interations, 8×8 HyGT4-interations |

Category 9. In-loop filtering

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 9. In-loop filtering |  | HM | JEM | DLVC |
| 9.a | List line buffer size in terms of luma and chroma lines (if any) | Deblocking (luma 4 &chroma 2) SAO (luma 1 &chroma 1) | Deblocking (luma 4 &chroma 2) SAO (luma 1 &chroma 1) ALF (luma 8, chroma 4) | Deblocking (luma 4 &chroma 2) CNNLF (entire image) SAO (luma 1 &chroma 1) ALF (luma 8, chroma 4) NF (entire image) |
| 9.b | List all the filters used for in-loop filtering | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and(2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. ALF: 9×9/7×7/5×5 diamond filter support for luma; and 5×5 diamond filter support for chroma | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and(2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. CNNLF: Trained CNN filters ALF: 9×9/7×7/5×5 diamond filter support for luma; and 5×5 diamond filter support for chroma |
| 9.c | Minimum processing unit size | Deblocking (luma: 8×8 grid with 4×4 filter decision, chroma 8×8 grid with 4×4 filter decision) | Deblocking (luma 4×4 grid with 4×4 filter decision, chroma 8×8 grid with 2×2 filter decision) ALF: for luma, filter decision is based on 2×2 block Bilateral filter: sample level filter switch (luma only) | Deblocking (luma 4×4 grid with 4×4 filter decision, chroma 8×8 grid with 2×2 filter decision) CNNLF: sample level ALF: for luma, filter decision is based on 2×2 block Bilateral filter: sample level filter switch (luma only) NF: sample level |

Category 10. “tool-off” and “tool-on” tests

| Tool Level Q&A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category 10. “tool-off” and “tool-on” tests |  | HM | JEM | DLVC |
| 10.a | List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools | None | QTBT, ALF, FRUC, BIO, DMVR, EMT, … | CNNLF, CNN-BARC, TT, FBT, NF, FRUCI, DMVRI, MERGEI, AFFINEI, BCBR See the following table for the results of “tool-off” and “tool-on” tests |

Summary of “tool-off” and “tool-on” tests for DLVC

Test conditions:

For “tool-off” tests, anchor is DLVC with one tool turned off, and tested is DLVC.

For “tool-on” tests, anchor is HM version 16.16 plus QTBT, and tested is anchor plus one tool.

In addition to the specified Constrained Set 1 and Constrained Set 2, All-Intra configuration is also tested for selected tools.

Due to time limit, some “tool-off” test results are not ready yet. They will be included in the future version of this document.

| SDR-A Constraint Set 1 | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tested tool | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CNNLF | −2.0% | −2.1% | −2.0% | 152% | 12863% | −5.1% | −3.8% | −4.5% | 677% | 143169% |
| TT |  |  |  |  |  | −3.2% | −5.0% | −5.5% | 198% | 78% |
| CNN-BARC+TT |  |  |  |  |  | −5.0% | −4.4% | −5.1% | 118% | 903% |
| FBT |  |  |  |  |  | −0.3% | −0.3% | −0.3% | 97% | 90% |
| NF |  |  |  |  |  | −1.2% | −2.9% | −3.2% | 102% | 244% |
| DMVR |  |  |  |  |  | −4.6% | −4.0% | −4.2% | 100% | 88% |
| DMVRI |  |  |  |  |  | −7.5% | −6.8% | −7.0% | 95% | 121% |
| FRUC |  |  |  |  |  | −8.6% | −8.4% | −8.9% | 91% | 141% |
| FRUCI |  |  |  |  |  | −9.0% | −9.0% | −9.6% | 147% | 236% |
| AFFINE |  |  |  |  |  | −4.9% | −3.8% | −3.9% | 98% | 68% |
| AFFINEI |  |  |  |  |  | −7.8% | −6.3% | −6.5% | 167% | 116% |

| SDR-B Constraint Set 1 | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tested tool | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CNNLF | −2.4% | −3.7% | −3.4% | 163% | 15241% | −5.9% | −5.8% | −5.8% | 776% | 235587% |
| TT |  |  |  |  |  | −3.2% | −5.9% | −5.8% | 214% | 83% |
| CNN-BARC+TT | −3.0% | -7.0% | -7.1% | 130% | 72% | −4.2% | −5.8% | −5.9% | 111% | 447% |
| FBT |  |  |  |  |  | −1.2% | −1.7% | −1.7% | 99% | 72% |
| NF |  |  |  |  |  | −1.4% | −4.5% | −4.8% | 102% | 244% |
| DMVR |  |  |  |  |  | −2.8% | −2.3% | −2.6% | 106% | 80% |
| DMVRI |  |  |  |  |  | −4.3% | −3.9% | −4.1% | 103% | 91% |
| FRUC |  |  |  |  |  | −7.4% | −7.1% | −7.0% | 123% | 123% |
| FRUCI |  |  |  |  |  | −7.7% | −7.5% | −7.5% | 158% | 185% |
| AFFINE |  |  |  |  |  | −4.5% | −3.0% | −3.3% | 118% | 64% |
| AFFINEI |  |  |  |  |  | −6.2% | −4.2% | −4.7% | 178% | 100% |

| SDR-B Constraint Set 2 | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tested tool | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CNNLF | −2.0% | −4.2% | −5.4% | 138% | 37071% | −5.2% | −7.2% | −8.4% | 1187% | 254605% |
| TT |  |  |  |  |  | −3.0% | −5.1% | −5.3% | 231% | 88% |
| CNN-BARC+TT |  |  |  |  |  | −3.3% | −5.4% | −5.3% | 152% | 154% |
| FBT |  |  |  |  |  | −1.5% | −2.4% | −2.3% | 119% | 96% |
| NF |  |  |  |  |  | −1.7% | −7.2% | −8.7% | 113% | 328% |
| BCBR |  |  |  |  |  | −0.8% | −1.1% | −0.8% | 119% | 103% |
| FRUC |  |  |  |  |  | −3.7% | −4.4% | −4.8% | 144% | 149% |
| FRUCI |  |  |  |  |  | −7.7% | −7.5% | −7.5% | 158% | 185% |
| AFFINE |  |  |  |  |  | −4.5% | −3.0% | −3.3% | 118% | 64% |
| AFFINEI |  |  |  |  |  | −6.2% | −4.2% | −4.7% | 178% | 100% |

| SDR-A All Intra | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tested tool | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec |
| CNNLF | −5.7% | −5.1% | −5.9% | 508% | 143169% |
| TT | −1.7% | −3.7% | −4.0% | 377% | 77% |
| CNN-BARC+TT | −8.8% | −1.6% | -1.9% | 583% | 31187% |
| FBT | −0.1% | −0.1% | −0.1% | 118% | 96% |
| NF | −1.2% | −2.9% | −3.5% | 131% | 392% |

| SDR-B All Intra | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tested tool | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec |
| CNNLF | −7.1% | −6.5% | −7.8% | 693% | 235587% |
| TT | −2.0% | −5.6% | −5.7% | 529% | 122% |
| CNN-BARC+TT | −5.6% | −5.1% | -5.3% | 639% | 33872% |
| FBT | −0.2% | −1.1% | −1.0% | 131% | 145% |
| NF | −1.3% | −4.7% | −5.1% | 135% | 453% |

[end of document]