| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0034 |
| --- | --- |

| Title: | AHG9: CNN-based driving of block partitioning for intra slices encoding |  |  |
| --- | --- | --- | --- |
| Status: | Input document approved by JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Galpin, F. Racapé, P. Bordes, F. Le Léannec, E. François (Technicolor) | Tel: Email: | franck.galpin@technicolor.com fabien.racape@technicolor.com philippe.bordes@technicolor.com fabrice.leleannec@technicolor.com edouard.francois@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution describes in more details the Convolutional Neural Network (CNN) based algorithm, used in the MTT codec presented in the CfP response JVET-J0022, for driving the block partitioning in intra slices encoding.

A CNN-based encoding approach is explored to partly substitute heuristics-based encoder speed-ups by a systematic and automatic process. The solution allows controlling the trade-off between complexity and coding gains, in intra slices, with one single parameter. This contribution reports, in AI configuration, a BD-rate gain of 6% for the method MTT codec presented in JVET-J0022, compared to JEM7 at the same encoding runtime, whereas for the same BD-rate performance as JEM7, the average encoding runtime is reportedly reduced by a factor of 4.3.

# Introduction

This document describes a deep-learning-based approach used in the CfP response JVET-J0022, for driving the encoder by estimating probabilities of blocks or Coding Units (CU) splitting in intra slices. The approach is primarily based on a texture analysis of the original blocks, and partly replaces the costly Rate Distortion Optimization (RDO) potentially involved for testing all potential partitioning configurations. All experiments described in this document are using the codec described and used for the CfP response JVET-J0022.

Considering the evolution of the last video compression standards, one can observe that an increased coding efficiency has been obtained by extending the picture partitioning options. In HEVC, the basic partitioning of a CTU is based on a recursive quad-tree (QT) decomposition into square CUs (with in addition the possibility to use square or rectangular Prediction Units (PUs) inside CUs). In JEM, the concept was extended by the introduction of Binary Tree splitting (BT) that allows splitting a CU into two symmetric rectangular sub-CUs. In several CfP responses, as in JVET-J0021 or JVET-J0022, even more flexible partitioning modes are enabled. This generalization of partitioning modes potentially increases the RDO combinatory.

This is illustrated in Table 1 that provides an estimate of the combinatory variations when coding a single block of size 32×32, using the software implementation provided in JVET-J0022, in which most of the encoder speed-ups were disabled. The table shows estimated average numbers of explored luma sub-blocks for a frame of size 416×240, encoded at QP=32, measured as the number of calls of the checkIntraRDCost function. The estimation is performed for the three types of partitioning supported in JVET-J0022, i.e. QT, BT and Asymmetric Binary Tree (ABT).  Note that some encoder speed-up heuristics were still enabled. Table 1 indicates an increase of the checks by a factor of 22 when QT is replaced by BT for all depths, and by an additional factor of 3 when ABT is added on top of BT (66 times compared to QT).

Encoder speed-ups are usually based on heuristics typically built from the analysis of the behavior of a full RDO. Deep-learning offers a complementary way to address the combinatory issue, with the possibility to develop automatically trained engines. This is what is proposed in JVET-J0022 and further explained in the current contribution, with a focus on intra slices.

Table 1. Number of luma sub-blocks checked at the encoder side.

| Tree structure | (almost) no heuristics |
| --- | --- |
| QT | 8340 |
| BT | 186932 (×22/QT) |
| BT+ABT | 577689 (×3/QTBT) |

# Algorithm overview

A Convolutional Neural Network (CNN) based analysis is proposed to speed-up the intra slices encoding process. The method is based on texture analysis of original 64×64 root blocks, luma or chroma, for predicting the most probable splits inside each potential sub-block. The method aims at enabling the encoder to leverage the gains of BT and ABT partitioning, while reducing the combinatory for the RDO checks.

The solution described in JVET-J0022 considers Coding Tree Units (CTUs) of size 256×256. In intra slices, the encoder always applies a quadtree split to the CTU as a first splitting depth, leading to four 128×128 blocks in luma and 64×64 blocks in chroma. In the luma case, the second splitting depth only allows quadtree split, evaluated through a classical RDO process, leading to one 128×128 or four 64×64 blocks. Root blocks for CNN based analysis then correspond to the third level of depth in luma, respectively the second level in chroma. 64×64 blocks are considered in both cases.

In fact, 65×65 samples patches, luma or chroma, are considered as input to the CNN stage. They consist in the original texture of root blocks plus their causal neighboring top and left boundary samples, as they participate to split decisions in classical RDO.

Figure 1. Overview of the split prediction process.

Figure 1 depicts the overall process of the split prediction module. This module precedes the usual RDO process, and pre-selects the split configurations to be tested by the RDO. It is composed of the following 3 steps:

- CNN-based analysis – In the first step, each input 65×65 patch is analyzed by a CNN-based texture analyzer. The output of this step consists in a vector of probabilities associated to each one of the elementary boundaries that separate elementary sub-blocks. Figure 2 illustrates the mapping between elementary boundary locations and the vector of probabilities. The size of elementary blocks being 4×4, the vector contains n=480 probability values. The CNN is described in section 3.

- Probable split selection – The second step takes as input the probability of each elementary boundary and outputs a first set of splitting modes among all possible options, which are: no split, QT, BT (vertical, horizontal), ABT (top, bottom, left, right). This step is further detailed in section 4.1.

- Encoder constraints and speed-ups –  The third step selects the final set of splitting modes to be checked by classical RDO, depending on the first set provided by step 2, the contextual split constraints described in JVET-J0022 section 3.1.1.3 and the encoder speed-ups described in JVET-J0022, section 3.1.2.1. This step is further detailed in section 4.2.

Figure 2. Mapping of the boundary locations onto a vector n×1.

# CNN-based analysis

## CNN architecture

Two CNNs are used, one dedicated to the luma component and one for the chroma components. They are trained and inferred independently, since luma and chroma blocks may have separate partitions in intra slices.

Figure 3 depicts the layout of the overall network used in the boundary prediction process.

Figure 3. Overview of the CNN layout for boundary prediction.

The CNN is analogous to a ResNet as described in [1], composed of a set of convolutional layers with several skip connections. The main differences with a ResNet (i.e. 18-layer version with 2 building blocks) are:

- the adaptation of the number of filters and layers,

- the absence of batch norm, average pooling and double fully connected layers,

- the pooling at the end of each scaling block,

- and no stride during convolution.

It takes as inputs:

- a block of luma samples of size 65×65, as well as a normalized luma Quantization step (Qstep) at the very end during the fully connected layer, for the luma CNN.

- a chroma block 65×65×2 and a normalized chroma Qstep at the very end during the fully connected layer, for the chroma CNN.

It outputs a vector describing the probability of the 480 elementary boundary locations in the input root block as described in Figure 2. Figure 4 provides another illustration, with the probabilities of boundaries of an exemplary root block represented by gray levels (from white corresponding to most probable, to dark gray corresponding to least probable).

Figure 4. Boundary probabilities represented in gray levels.

The overall CNN model contains around 225k parameters.

## CNN training

The data set was created by encoding a set of images in intra mode with the JVET-J0022 codec, using a classical RDO in ABT mode, for several QPs. The ground truth corresponds to the boundary choices made by the RDO. During this encoding, several speed-ups, based on heuristics, were disabled, leading to a higher number of potential partitions compared to the actual codec version used in JVET-J0022. For each block 64×64, a patch of size 65×65 containing the 64×64 block + 1 pixel-wide causal border was extracted, as depicted in Figure 5.

The quantization step (Qstep) used for encoding was also extracted for each block and normalized to consider a value ranging between 0 and 1 for the whole considered Qstep range.

Finally, the ground truth of each elementary boundary was extracted from the decoded bitstream and mapped onto a vector of size 480 using the inverse process as shown in Figure 5.

Figure 5. Dataset creation for CNN training.

The loss function corresponds to a L2 norm between the ground truth vector V, which contains values 1 or 0 depending on whether a boundary exists or not, and the output of the network V, plus a regularization term corresponding to the L2 norm of the weights of the convolution ck:

L=iVi-Vi+λkck

A dataset containing a large number of source images, excluding the JVET test sequences, was created. It contains more than 10M patches, encoded with QPs ranging from 19 to 41.

The CNN is classically trained using the deep learning framework Tensorflow [2]. Specifically, the Adam minimizer was used with a learning rate of 10-3, a regularization weight of 10-5 and a batch size of 256. The model was trained on 4 epochs in a few hours on a single recent GPU.

# Probable split selection

## Probable split selection

Figure 6. From boundaries to split selection.

From the output vector of the CNN, an image of boundaries is constructed, as shown in Figure 6 (a). For each sub-block potentially explored by the RDO, the relevant boundaries are extracted, represented as segments in Figure 6 (b, c).

At each location of possible splits, the average value of elementary boundaries corresponding to each half-split boundary is computed, as depicted in yellow in Figure 6 (c). These values are then compared to pre-determined thresholds for each split choice. A decision to explore or not each choice is finally made. Figure 6 (d) provides an example where, at this step, i.e. before performing the third step, QT, BT horizontal, and ABT bottom will be considered by the classical RDO.

The thresholds depend on the depth and a speed-control parameter described in section 5.2.

Figure 7 shows an exemplary case focusing on the first 3 depths of split only. The considered data consist in all the blocks in the training set for which a boundary exists (i.e. excluding blocks which are not split anymore). The x-axis corresponds to the threshold to decide whether a split exists or not. The y-axis corresponds to the ratio of blocks having a boundary probability below this threshold but that were split by classical RDO. For example, if the split threshold is set to 0.8, the ratio is around 0.1, which means that approximately 10% of the blocks were split, although the calculated boundary probability was lower than the threshold. In other words, the CNN analysis would make the wrong decision on only 10% of the split blocks.

Figure 7. Exemplary relationship between threshold and incorrectly classified blocks, i.e. that are split by classical RDO, although the corresponding boundary probability is lower than the threshold in abscissa.

The decision logic on each split depends on the type of partitioning choice. For example, when only QT is considered:

- No split is explored if all corresponding boundary probabilities are lower than a pre-determined threshold.

- Quadtree split is explored if at least one vertical and one horizontal half boundary probabilities are higher than a pre-determined threshold.

The remaining combinatory, for RDO exploration, directly depends on the above thresholds. Note that the thresholds can be different for the luma tree and the chroma tree, enabling the encoder to make different trade-offs for each tree.

## Encoder constraints and speed-ups

This last step selects the final set of splitting modes to be checked by classical RDO.

Some splitting modes may be removed to be consistent with encoder heuristics, for example,

- when they are not compatible with the current tree structure, e.g. quadtree emulation with an horizontal and a vertical split,

- when a splitting mode has already been explored in a previous configuration.

Finally, when no remaining splitting mode is available, the RDO falls back to the no-split mode exploration.

# Speed-control parameter and results

This section details the combinatory reduction of the RDO and describes how it is driven by a single parameter. It also provides results in terms of BD-rate gains versus encoder complexity.

## Estimated number of RDO checks per 32×32 block

Table 2 depicts the estimate of the combinatory for coding a single luma block of size 32×32, using the full combinatory for each tree structure, as done for Table 1, using three software versions of the JVET-J0022 codec. The first version corresponds to the one used for Table 1, almost without heuristics. The second version activates encoding heuristics. The third version, in addition, implements the CNN-based algorithm. The benefit in terms of reduction of number of RD checks, resulting from the CNN approach, is observed in Table 2, e.g. -68% instead of -28% with heuristics for BT+ABT.

Table 2. Number of explored sub-CUs explored at the encoder side.

| Tree structure | (almost) no heuristics | With heuristics | With CNN |
| --- | --- | --- | --- |
| QT | 8340 | 8181 (-2%) | Not tested |
| BT | 186932 (×22/QT) | 58388 (-69%) | 49769 (-73%) |
| BT+ABT | 577689 (×3/QTBT) | 416281 (-28%) | 182156 (-68%) |

## Tree exploration trade-offs

A single control parameter called speed-control has been added to the encoder. This parameter is a floating value. It drives:

- the structure of the tree used for the luma and chroma: maximum and minimum QT level and BT /ABT levels (c.f. Table 3),

- the thresholds used to prune the tree before the RDO.

Table 3 reports the link between the speed-control parameter and the maximum QT depth and the maximum number of BT/ABT splits.

Table 3. Tree structure explored depending on the speed-control parameter.

| speed-control parameter | Max QT depth | ABT/BT depth[2] (64×64) | ABT/BT BT depth[3] (32×32) | ABT/BT BT depth[4] (16×16) |
| --- | --- | --- | --- | --- |
| [0.65,1.04[ | 2 (64×64) | 8 | 0 | 0 |
| [1.04,1.7[ | 3(32×32) | 2 | 8 | 0 |
| [1.7,3.2[ | 4(16×16) | 0 | 4 | 8 |
| [3.2,-[ | 3(32×32) | 0 | 8 | 0 |

In the following, some results are reported using the CfP test set, in AI mode, using the codec of JVET-J0022 including the ABT mode. Encoding and performance measurements were made on the first frame of each sequence.

Figure 8 shows the trade-off between speed ratio (in %) and BD-rate gain, over JEM7, with the speed-control parameter varying from 0.65 to 3.4. The tree structure is adapted per range of speed-control values, as can be seen in the varying colors of the plotted line, each color corresponding to a specific tree structure.

For example, for the legend tree 00028XX (in green), corresponding to a speed-up range between 25% and 60%:

- levels 0,1 and 2 (QT size 256 to 64) do not allows BT splits (max level 0); this is indicated by “000”.

- level 3 (QT size 32) allows a BT tree with a depth of 2; this is indicated by “2”.

- level 4 (QT size 16) allows a BT tree with a depth 8 (maximum depth) ; this is indicated by “8”.

- level 5 and 6 (QT size 8 and 4) are not explored by QT; this is indicated by “XX”.

The other tree types consist of other configurations that can be derived using this nomenclature and the above example.

Figure 8. BD-rate gains as a function of the relative encoding time (in % of JEM7) with dynamic tree structure.

Figure 9 shows the behavior of the encoder when the tree structure is kept and does not depend on the speed-control factor. For low complexity tuning, the QTBT mode is slightly faster at similar BD-rate performance. At similar compression efficiency, the encoding runtime of the codec of JVET-J0022, configured with QTBT only, is approximately 21% of the JEM7 runtime, while the QTBT+ABT is around 23%.

On the other hand, QTBT+ABT offers a better trade-off than the QTBT when the speed ratio is above 50% of the JEM7 speed. It is also observed in Figure 9 that the maximum gain over JEM is around 4.8% with QTBT only configuration, while it can increase to a maximum gain of ~7.3% when extending the partitioning to the QTBT+ABT configuration.

Figure 9. BD-rate gains in AI configuration, as a function of the relative encoding time (in % of HM) for QTBT and QTBT+ABT.

## Actual results

The results provided in the document JVET-J0022 were produced using the same CNN models and a speed-control factor of 1.5 for all tests. The relation between the speed-up and the BD rate gains of the MTT codec described in JVET-J0022 compared to JEM7 is provided in Figure 10.

It is observed in Figure 10 that the proposed split prediction and dynamic tree structure allows the encoder to cover a wide range of trade-offs. For example:

- for the same BD-rate as JEM7, the encoder runtime is reported to be 23% of the JEM runtime, which corresponds to a speed-up of factor 4.3,

- at the same speed as JEM7, the BD-rate gain is around 6%,

- for a speed of approximately 850% of JEM7, the BD-rate gain reaches 9.1%.

Figure 10. Result curve showing the BD-rate gains of the proposed codec in JVET-J0022 over JEM7, as a function of the complexity, the latter being targeted using a single “speed-control” encoding parameter.

# CNN implementation

A stand-alone C++ version of the above CNN software has been developed. It can be used as an inference engine by the encoder, without any dependency to external library.

## Model conversion

Figure 11 describes the CNN model conversion process to a format compliant with the codec software. The model of the CNN is first converted from the Tensorflow original format to a basic binary format. The binary format is then directly integrated into the encoder using two files (model_data.h and model_data_chroma.h). Both, training and conversion, are performed once and the CNN-based texture analysis is robust to tools change inside the codec.

Figure 11. Interfacing the CNN with the encoder.

## CNN inference in the codec

The CNN inference module implements the minimum number of layers, without any dependency. This implementation includes the required layer types: reshape, fully connected, convolution 2D, add, bias, max pool and concatenation.

The code footprint of the framework consists in approximately 1600 lines, mostly within header files. It executes on CPU, using a single core. The code is straightforward and only one hand-crafted vectorization for the convolution was done.

The CNN inference module works with the network described above but can be extended to other layers or layout. As a comparison, for a single HD frame, the implemented GPU version requires around 180ms/frame (on a GTX 1070), the optimized CPU version of Tensorflow on one core around 2s/frame, and our module around 3.4s/frame.

In the encoder, for each new block 64×64 of an intra slice, the model is inferred, taking as inputs: the block 64×64, its causal border, and the normalized QP. The resulting boundary probability vector is saved for processing the sub-blocks inside the root-block.

# Conclusion

A deep-learning-based method for driving the partitioning of intra slices has been described. Operating independently on luma and chroma 64×64 root blocks, it determines a subset of most probable split decisions to be explored by the classical RDO, speeding-up the encoder of the CfP response JVET-J0022. A single parameter drives the proper thresholds and tree structure to find the targeted trade-off between encoder complexity (speed-up) and efficiency (BD-rate gain). In AI configuration, the codec, presented in the response JVET-J0022, is found to achieve a BD-rate gain of 6% compared to JEM7 at the same encoding runtime, whereas for the same BD-rate performance as JEM7, the average encoding runtime is reportedly reduced by a factor of 4.3. The method was implemented in C++ and integrated in the software provided with the response JVET-J0022.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Cited Documents

[1]	K. He, X. Zhang, S. Ren, et J. Sun, « Deep Residual Learning for Image Recognition », Computer Vision and Pattern Recognition (cs.CV), 2015.

[2]	M. Abadi et al., « TensorFlow, Large-Scale Machine Learning on Heterogeneous Systems ». Software available from tensorflow.org, 2015.