| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0035 |
| --- | --- |

| Title: | Quadtree plus binary tree with shifting |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Jackie Ma Adam Wieckowski Valerie George Tobias Hinz Jens Brandenburg Santiago De-Luxán-Hernández Heiner Kichhoffer Robert Skupin Heiko Schwarz Detlev Marpe Thomas Schierl Thomas Wiegand | Email: | jackie.ma@hhi.fraunhofer.de adam.wieckowski@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de detlev.marpe@hhi.fraunhofer.de thomas.schierl@hhi.fraunhofer.de thomas.wiegand@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |  |

_____________________________

# Abstract

This contribution reports a quadtree plus binary tree with shifting (QT+BTS). The reported technology extends the QTBT split modes by asymmetric modes with split ratio 1:2, 1:3, 1:4, 2:3, 3:5. It is reported that for the random access configuration 9.33% gain in luma is achieved for 109% encoding time compared to HM 16.14. In another configuration of the proposed method 10.37% gain in luma are reported for 183% encoding time compared to HM 16.14. For low-delay B 12.12% gain in luma are reported for 111% encoding time over HM 16.14 and in another configuration 12.90% luma gain for 193% encoding time over HM 16.14 is reported.

# Introduction

Extensions of QTBT [1] have been reported by Qualcomm Inc. in JVET-D0117 [2] and by Technicolor in JVET-D0064 [3]. Both extensions added additional split modes to the ones from QTBT. In JVET-D0117 [2] triple tree splits are proposed which are splits of one block into three subblocks with the center block being twice as large as the equal sized boundary blocks, cf. Figure 1. In JVET-D0064 [3] asymmetric binary splits for power of 2 sizes are presented. The asymmetric splits produce two sub blocks with a ratio of 1:3 and can occur vertically as well as horizontally, cf. Figure 2.

Figure 1: triple tree partitioning of a block as reported in JVET-D0117 [2]

Figure 2: Asymmetric partitioning of a block as reported in JVET-D0064 [3]

In this contribution shifted binary splits are presented that produce a superset of the above mentioned partitioning methods.

# Proposed method

The proposed quad tree plus binary tree with shifting (QT+BTS) the QTBT split modes by:

- 1/4 and 3/4: if the split side is 2n, n∈N and at least 16 in luma pixels,

- 3/8 and 5/8: if the split side is 2n, n∈N and at least  32 in luma pixels.

Figure 3: Possible splits for power of 2 sizes. First row: QTBT splits. Second row: 1:3 splits with different orientations. Third row: 3:5 splits with different orientations.

The quad split is not needed for the presented method to work and the shifted binary splits can be used without it as the quad split can be replaced by a sequence of orthogonal 1/2 splits. However, the inclusion of a quad split proves to be superior to the underlying method without the quad split.

If the size of the side that is to be split is not 2n, n∈N, then it is either 3·2n, n∈N or 5·2n, n∈N. It is proposed to split these non-power of 2 sizes by either

- 1/2: if the side to be split is at least 8 in luma pixels,

- 1/3 and 2/3: if the split side is 3·2n,

- 1/5, 2/5, 3/5, 4/5: if the split side is 5·2n.

Figure 4: Possible shifted binary splits for non-power of 2 sizes. First row: 1:2 splits with different orientations. Second and third row: 1:4 and 2:3 splits with different orientations.

## Syntax signaling

A shifted binary split is uniquely determined by its split direction and its split ratio. The direction of the binary splits are coded depending on the previous split, i.e. instead of being signaled as horizontal or vertical split, they are signaled as perpendicular split or parallel split which, however, can always be translated to a horizontal or vertical split. The first split has to be set always by convention. There are two binary flags perpend_split_flag and parallel_split_flag for the two possible directions (perpendicular and parallel). The split ratio then describes the location where to split binarily, see Figure 1 and Figure 2. It is coded using a binary decision tree. The full signaling tree for the quadtree plus binary splits with shifting (QT+BTS) without any restrictions is shown in Figure 5.

Figure 5: Full signaling tree, the perpendicular part is not displayed as it has the same sub tree as the parallel case. Branches or leafs of tree can be omitted, if the options are not available in a specific context. If a node only contains one child, the decision is not signaled.

Figure 6: Signaling tree, with different available options highlighted. The signaling a) would be used for splitting of a quadratic partition within a QT (possibly finalizing it and opening a binary tree with shifting (BTS). Signaling b) would be used for a partition within a BTS for which the side perpendicular to the last split is of size 3·2n, n∈N, thus allowing only x/3 and ½ splits parallel to the last splits.

Table 1: Recursive coding tree syntax in tabular form

| coding_tree( x0, y0, cbSizeX, cbSizeY, treeType, qtDepth, btDepth, numBtImpl, lastSplitDir ) { | Descriptor |
| --- | --- |
| if( !QtbtsNonLog2CUs && ( ( 1 << log2( cbSizeX ) ) != cbSizeX ¦¦ ( 1 << log2( cbSizeY ) ) != cbSizeY ) ) { |  |
| btDepth = btDepth - 1; |  |
| } |  |
| if( canQtSplit ) |  |
| split_cu_flag[x0][y0][cbSizeX][cbSizeY] | ae(v) |
| if( split_cu_flag[x0][y0][cbSizeX][cbSizeY] == 0 ) |  |
| split_mode( x0, y0, cbSizeX, cbSizeY, treeType, btDepth, numBtImple, lastSplitDir ) |  |
| } |  |
| if( cu_qp_delta_enabled_flag && floor( log2[cbSizeX] ) >= Log2MinCuQpDeltaSize && floor( log2[cbSizeY] ) >= Log2MinCuQpDeltaSize ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| } |  |
| if( split_cu_flag[x0][y0][cbSizeX][cbSizeY] ) { |  |
| x1 = x0 + cbSizeX/2 |  |
| y1 = y0 + cbSizeY/2 |  |
| coding_tree( x0, y0, cbSizeX/2, cbSizeY/2, treeType, qtDepth + 1, 0, 0, lastSplitDir ) |  |
| coding_tree( x1, y0, cbSizeX/2, cbSizeY/2, treeType, qtDepth + 1, 0, 0, lastSplitDir ) |  |
| coding_tree( x0, y1, cbSizeX/2, cbSizeY/2, treeType, qtDepth + 1, 0, 0, lastSplitDir ) |  |
| coding_tree( x1, y1, cbSizeX/2, cbSizeY/2, treeType, qtDepth + 1, 0, 0, lastSplitDir ) |  |
| } else if( split_perpendicular_flag[x0][y0][cbSizeX][cbSizeY] ¦¦ split_parallel_flag[x0][y0][cbSizeX][cbSizeY] ) { |  |
| x1 = x0 + ( splitDir[x0][y0][cbSizeX][cbSizeY] == 0 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : 0 ) |  |
| y1 = y0 + ( splitDir[x0][y0][cbSizeX][cbSizeY] == 1 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : 0 ) |  |
| cbSizeX0 = cbSizeX - ( splitDir[x0][y0][cbSizeX][cbSizeY] == 0 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : 0 ) |  |
| cbSizeX1 = ( splitDir[x0][y0][cbSizeX][cbSizeY] == 0 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : cbSizeX ) |  |
| cbSizeY0 = cbSizeY - ( splitDir[x0][y0][cbSizeX][cbSizeY] == 1 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : 0 ) |  |
| cbSizeY1 = ( splitDir[x0][y0][cbSizeX][cbSizeY] == 1 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : cbSizeY ) |  |
| if( x0 + cbSizeX > pic_width[treeType] ¦¦ y0 + cbSizeY > pic_height[treeType] ) |  |
| { |  |
| numBtImpl = numBtImpl + 1 |  |
| } |  |
| coding_tree( x0, y0, cbSizeX0, cbSizeY0, treeType, qtDepth, btDepth + 1, numBtImpl, splitDir[x0][y0][cbSizeX][cbSizeY] ) |  |
| coding_tree( x1, y1, cbSizeX1, cbSizeY1, treeType, qtDepth, btDepth + 1, numBtImpl, splitDir[x0][y0][cbSizeX][cbSizeY] ) |  |
| } else { |  |
| coding_unit( x0, y0, cbSizeX, cbSizeY, treeType ) |  |
| } |  |
| } |  |

Table 2: split mode syntax in tabular form

| split_mode( x0, y0, cbSizeX, cbSizeY, treeType, btDepth, numBtImple, lastSplitDir ) { | Descriptor |
| --- | --- |
| if( canNotSplit && ( canParlSplit ¦¦ canPerpSplit ) ) { |  |
| bt_split_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| if( bt_split_flag[x0][y0] ) { |  |
| if( canParlSplit && canPerpSplit ) { |  |
| split_perpendicular_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| split_parallel_flag_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] = 1 - split_perpendicular_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] |  |
| if( split_perpendicular_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ¦¦ split_parallel_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ) { |  |
| splitDir[x0][y0][cbSizeX][cbSizeY] = split_perpendicular_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ? 1 - lastSplitDir : lastSplitDir |  |
| splitSize = splitDir[x0][y0][cbSizeX][cbSizeY] == 0 ? cbSizeX : cbSizeY |  |
| if( canHalfSplit && ( canFourthsSplit ¦¦ canEightsSplit ¦¦ canThirdsSplit ¦¦ canFifthSplit ) ) { |  |
| split_symmetrical_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| if( !split_symmetrical_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ) { |  |
| if( splitSize == ( 1 << log2( splitSize ) ) ) { |  |
| if( canFourthsSplit && canEightsSplit ) { |  |
| split_eights_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| } |  |
| if( canFourthsSplit ¦¦ canEightsSplit ¦¦ canThirdsSplit ) { |  |
| if( ( canThirdsSplit && is13Avail && is23Avail ) ¦¦ ( split_eights_flag[x0][y0][cbSizeX][cbSizeY] && is38Avail && is58Avail ) ¦¦ ( !split_eights_flag[x0][y0][cbSizeX][cbSizeY] && is14Avail && is34Avail ) ) { |  |
| is_split_offset_large[x0][y0][cbSizeX][cbSizeY] | ae(v) |
| } |  |
| splitBase = splitSize / ( canThirdsSplit ? 3 : ( split_eights_flag[x0][y0][cbSizeX][cbSizeY] ? 8 : 4 ) ) |  |
| splitOffset[x0][y0][cbSizeX][cbSizeY] = ( ( canThirdsSplit ¦¦ !split_eights_flag[x0][y0][cbSizeX][cbSizeY] ) ? splitBase : 3 * splitBase ) + is_split_offset_large[x0][y0][cbSizeX][cbSizeY] ? ( canThirdsSplit ? 1 : 2 ) * splitBase : 0 |  |
| } else { |  |
| if( ( is25Avail ¦¦ is35Avail ) && ( is15Avail ¦¦ is45Avail ) ) { |  |
| split_fifths_is235_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| fifths_split_id = 1 |  |
| if( split_fifths_is235_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ) { |  |
| if( is25Avail && is35Avail ) { |  |
| split_fifths_is35_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| fifths_split_id = split_fifths_is35_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ? 3 : 2 |  |
| } else { |  |
| if( is15Avail && is45Avail ) { |  |
| split_fifths_is45_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| if( split_fifths_is45_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ) { |  |
| fifths_split_id = 4 |  |
| } |  |
| } |  |
| splitOffset[x0][y0][cbSizeX][cbSizeY] = fifths_split_id * splitSize / 5 |  |
| } |  |
| } else { |  |
| splitOffset[x0][y0][cbSizeX][cbSizeY] = splitSize / 2 |  |
| } |  |
| } |  |
| } else { |  |
| split_perpendicular_flag[x0][y0][cbSizeX][cbSizeY] = 0 |  |
| split_parallel_flag[x0][y0][cbSizeX][cbSizeY] = 0 |  |
| } |  |
| } |  |

The values of the split decision variable canParlSplit, canPerpSplit, canFourthsSplit, canEightsSplit, canThirdsSplit and canFifthsSplit depend on availability constraints such as minimal block sizes, maximal tree depths and the prohibitions of redundant splits given in Table 3 of this proposal.

Figure 7: Notion of different types of layouts presented in Table 3.  All layouts represent the splitting pattern after a binary split is used.
a): First block is to be split and second block not. b): First block is not split and second part is split. c): First and second blocks are split. d) Split first block and then the second subblock of the resulting block. e) Split second block and then the first subblock of the resulting block. f) Split second block and then the second subblock of the resulting block.

Table 3: prohibited redundancies

| Layout | A-Dir | A-Ratio | B-Dir | B-Ratio | C-Dir | C-Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| a) | * | 2/3 | parallel | 1/2 | - | - |
| a) | * | 3/4 | parallel | 1/3 | - | - |
| a) | * | 3/4 | parallel | 2/3 | - | - |
| a) | * | 3/5 | parallel | 1/3 | - | - |
| a) | * | 3/5 | parallel | 2/3 | - | - |
| a) | * | 2/5 | parallel | 1/2 | - | - |
| a) | * | 4/5 | parallel | 1/2 | - | - |
| a) | * | 5/8 | parallel | 4/5 | - | - |
| a) | * | 5/8 | parallel | 3/5 | - | - |
| a) | * | 4/5 | parallel | 1/4 | - | - |
| a) | * | 4/5 | parallel | 3/4 | - | - |
| b) | * | 1/4 | parallel | 1/3 | - | - |
| b) | * | 3/8 | parallel | 1/5 | - | - |
| c) | * | 1/4 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 3/4 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 1/3 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 2/3 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 1/5 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 2/5 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 3/5 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 4/5 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | ¾ | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 1/3 | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 2/3 | perpendicular | 1/4 | Perpendicular | 1/4 |
| c) | * | 1/5 | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 2/5 | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 3/5 | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 4/5 | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 1/3 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 2/3 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 1/5 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 2/5 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 3/5 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 4/5 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 2/3 | perpendicular | 1/3 | perpendicular | 1/3 |
| c) | * | 1/5 | perpendicular | 1/3 | perpendicular | 1/3 |
| c) | * | 2/5 | perpendicular | 1/3 | perpendicular | 1/3 |
| c) | * | 3/5 | perpendicular | 1/3 | perpendicular | 1/3 |
| c) | * | 4/5 | perpendicular | 1/3 | perpendicular | 1/3 |
| c) | * | 1/5 | perpendicular | 2/3 | perpendicular | 2/3 |
| c) | * | 2/5 | perpendicular | 2/3 | perpendicular | 2/3 |
| c) | * | 3/5 | perpendicular | 2/3 | perpendicular | 2/3 |
| c) | * | 4/5 | perpendicular | 2/3 | perpendicular | 2/3 |
| c) | * | 2/5 | perpendicular | 1/5 | perpendicular | 1/5 |
| c) | * | 3/5 | perpendicular | 1/5 | perpendicular | 1/5 |
| c) | * | 4/5 | perpendicular | 1/5 | perpendicular | 1/5 |
| c) | * | 3/5 | perpendicular | 2/5 | perpendicular | 2/5 |
| c) | * | 4/5 | perpendicular | 2/5 | perpendicular | 2/5 |
| c) | * | 4/5 | perpendicular | 3/5 | perpendicular | 3/5 |
| d) | * | 3/4 | parallel | 1/3 | parallel | 1/3 |
| e) | * | 1/4 | parallel | 2/3 | parallel | 3/4 |
| f) | * | 1/4 | parallel | 1/2 | parallel | 1/3 |

## High-level syntax elements

There are several high-level parameters that can be used to configure the splitting tree. The proposed QT+BTS can be configured by

- CTUSize: size of largest CTU (default set to 128)

- MinQTLumaISlice: no quad splits for sizes smaller than this in luma for I slices (default set to 8)

- MinQTChromaISlice: no quad splits for sizes smaller than this in chroma for I slices (default set to 4)

- MinQTNonISlice: no quad splits for sizes smaller than this for non I slices (default set to 8)

- MaxBTDepth: maximal depth for non-quad splits for intra (default set to 3)

- MaxBTDepthI: maximal depth for non-quad splits for inter (default set to 3)

- MaxBTDepthIChroma: maximal depth for non-quad splits for chroma (default set to 3)

- MaxBTSize: maximal size for non- quad splits for intra (default set to 32)

- MaxBTSizeI: maximal size for non- quad splits for inter (default set to 128)

- MaxBTSizeChroma: maximal size for non- quad splits for chroma (default set to 64)

- QtbtsAllowFourths: use 1/4 and 3/4 splits (default set to 1)

- QtbtsAllowEights: use 3/8 and 5/8 splits (default set to 0)

- QtbtsNonLog2Halving: allow non power of 2 sized blocks to be split by 1/2  (default set to 1)

- QtbtsNonLog2CUs: allow non power of 2 sized blocks (default set to 0)

- QtbtsForceSplitToLog2: if split, the partitioning is forced to split into power of 2 sizes (default set to 1)

- MaxAsymTSize: maximal size for non symmetric splits for intra (default set to 32)

- MaxAsymTISize: maximal size for non symmetric splits for inter (default set to 64)

- MaxAsymTSizeIChroma: maximal size for non symmetric splits for chroma (default set to 32)

These high-level parameters may be transmitted in a sequence parameter set (SPS), picture parameter set (PPS), or slice header. Moreover, they can be used to restrict the QT+BTS and thereby reduce the availability in the full tree only to a subtree. For instance, if QtbtsNonLog2CUs is set to 0 and QtbtsForceSplitToLog2 is set to 1, then a 3/4 split is never available. In order to prevent transmitting unnecessary decisions the binary decision tree is pruned according to the availability of the nodes.

## Non-power of 2 adaptations

The tools in the following list needed modifications:

- Enhanced multiple transform (EMT): adapted to non-power of two block shapes.

- Frame-rate up conversion (FRUC [7]): adapted to non-power of two block shapes.

Additionally, the entropy coding of transform coefficients is adapted:

- Transform coefficient level scan-patterns adapted to non-power of two block shapes.

- Indication of last non-zero coefficient is using the HEVC defined binarization scheme including tables referred by block size index rounded up to next larger power of two value.

## Quadtree plus binary tree with shifting on edge

Figure 8: Implicit splitting of a block located on a coded picture boundary. a) Using QT splits, as in H.265/HEVC and [1]. b) using QT and binary splits.

When a block within a partitioning tree extents beyond the coded picture boundaries, it has to be split into sub-blocks, so that the sub-blocks fully contained in the coded picture can be encoded. The block which lie outside of the coded picture boundaries are ignored. In H.265/HEVC and [1], to accomplish such splitting, a QT split is implicitly inferred and no further signaling is required (see Figure).

In QT+BTS, if only two corners of a current block extent beyond coded picture boundaries, the block can be additionally split using a binary split with ratio ½ and split direction parallel to the picture boundary the block extents over. The encoder can choose, based on RD-cost, if the QT or implicit BTS split is preferred. The decision has to be signaled using a single bin (see Figure 9). If the current block is a part of a QT+BTS rather than the QT tree, the QT will not be available and the split will be implicitly inferred as the BTS split with the ratio 1/2 parallel to the boundary edge the block extents over. If the current block extents over a coded picture boundary, QT+BTS-tree restriction like maximal tree depth are ignored.

Figure 9: Reduced split signaling for block within a QT tree, for which two of its corners lie outside of the coded picture boundaries.

If a different number of corners of a current block extent over the coded picture boundaries, a QT split is implicitly inferred and no further signaling is required.

## Encoder speed ups

Due to the greater amount of split modes in this presented method several speed ups are used to reduce the RD search complexity. In addition to some of the speed ups used for QTBT the presented method has the following early determination rules.

- If at least n levels have been tested and skip was the best mode for these past n levels, then no split is performed at the current level. The number n is exemplarily set as 3.

- If a parallel and a perpendicular 1/2 split has been tested, only splits in the direction which improved the RD cost will be tested next.

- If the parallel 1/2 split and the perpendicular 1/2 split did not improve the result and skip is best, then no other split is tested.

Furthermore, there are additional restrictions for a subset of splits depending on the targeted encoder time.

- Horizontal and vertical gradients are computed and splits are performed in accordance to these values – e.g. if one gradient is much higher than the other gradient, then only splits perpendicular to the higher gradient direction are tested.

- Additional maximal depths and cu sizes are assigned to a subset of splits in order to limit the RD search.

- Partitions can be forced to end up in sizes that are powers of 2 which shrinks the range of possible splits. To balance this option, only levels than can contain a CU (e.g. with power of 2 width and height), are counted for enforcement of the depth restrictions.

Prohibitions of splits that reduce redundant representations are included in the presented method.

# Experimental results

The proposed method is tested below in different configurations:

|  | QT+BTS | QT+BTS-2-2 | QT+BTS-2-3 | QT+BTS-3-3 | QT+BTS-5-3 |
| --- | --- | --- | --- | --- | --- |
| MaxBTDepth | 3 | 2 | 3 | 3 | 3 |
| QtbtsAllowFourths | 0 | 1 | 1 | 1 | 1 |
| QtbtsAllowEights | 0 | 0 | 0 | 0 | 1 |
| QtbtsNonLog2CUs | 0 | 0 | 0 | 1 | 1 |
| QtbtsForceSplitToLog2 | 0 | 1 | 1 | 0 | 0 |

For the comparison HM 16.14 is used as a reference (100% encoder run time, 0% gain). The test set consists of the sequences from the Common Test Conditions [9] and the Call for Proposal [10]. In Figure 9 the BD [10] rate gain in luma is depicted versus the encoder runtime for QPs 27, 32, 37, and 42. For the detailed results (such as results for all intra, per class, and QP 22) see the attached tables. The number of frames is set to 49 in the presented results.

Figure 7: BD-rate gain in luma versus encoder times. First: Random access on full set. Second: Random access with CfP sequences only. Third: Low-delay B results on CfP test set.

# Conclusion

This proposal reports a quadtree plus binary tree with shifting. The method obtains more than 10% BD-rate gain in luma for less than 200% encoder runtime compared to HM 16.14 in the random access configuration. For low-delay B the proposed achieves almost 13% BD-rate gain in luma for roughly 200% encoder run time. For both cases the decoding time is overall equal to the decoding time of HM 16.14.

# References

- J. Chen, E. Alshina, G.J. Sullivan, J.-R. Ohm, J. Boyce, Algorithm Description of Joint Exploration Test Model 7 (JEM 7), JVET, doc. JVET-G1001, July 2017.

- X. Li, H.-C. Chuang, J. Chen, M. Karczewicz, L. Zhang, X. Zhao, A. Said, Multi-Type-Tree, JVET, doc. JVET-D0117, Oct. 2016.

- F. Le Léannec, T. Poirier, F. Urban, Asymmetric Coding Units in QTBT, JVET, doc. JVET-D0064, Oct. 2016.

- J. Chen, E. Alshina, G. Sullivan, J.-R. Ohm, J. Boye, Algorithm Description of Joint Exploration Test Model 3, JVET, doc. JVET-C1001, May 2016.

- J. Chen, Y. Chen, M. Karczewicz, X. Li, H. Liu, L. Zhang, X. Zhao, “Coding tools investigation for next generation video coding”, ITU-T/STUDY GROUP 16 Doc. COM 16 – C806 – E, Jan. 2015

- T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm and G. J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E603, 5th Meeting: Geneva, CH, 16-23 March, 2011.

- J. Chen, W.-J. Chien, M. Karczewicz, X. Li, H. Liu, A. Said, L. Zhang, X. Zhao, “Further improvements to HMKTA-1.0”, ITU-T SG16/Q6 Doc. VCEG-AZ07, Jun. 2015.

- M. Budagavi, A. Fuldseth, G. Bjøntegaard, V. Sze, M. Sadafale, “Core Transform Design in the High Efficiency Video Coding (HEVC) Standard,” IEEE Journal of Selected Topics in Signal Processing, December 2013.

- K. Sühring and X. Li, “JVET common test conditions and software configurations”, JVET, doc JVET-H1010, October 2017.

- A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC”, JVET, doc. JVET-H1002, October 2017.

- G. Bjøntegaard, “Calculation of average PSNR differences between RD-curves,” VCEG-M33, Austin, Mar. 2001.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).