| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0035-v3 |
| --- | --- |

| Title: | Quadtree plus binary tree with shifting (including software) |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Jackie Ma Adam Wieckowski Valerie George Tobias Hinz Jens Brandenburg Santiago De-Luxán-Hernández Heiner Kichhoffer Robert Skupin Heiko Schwarz Detlev Marpe Thomas Schierl Thomas Wiegand | Email: | jackie.ma@hhi.fraunhofer.de adam.wieckowski@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de detlev.marpe@hhi.fraunhofer.de thomas.schierl@hhi.fraunhofer.de thomas.wiegand@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |  |

_____________________________

version 2:

- additional results (in particular for demonstrating the effect of modifying the luma-chroma-QP relationship)

- link to software

- revised abstract

version 3:

- link to another version of software (partitioning + JEM tools)

# Abstract

This contribution reports a quadtree plus binary tree with shifting (QT+BTS). The proposed technology extends the QTBT split modes by asymmetric modes with split ratio of 1:2, 1:3, 1:4, 2:3, and 3:5.

The document contains a link to a reportedly clean software, which does not include any new coding tools (compared to HEVC), but provides three block partitioning options: QTBT, QTBT plus triple splits (MTT), and QT+BTS as proposed in this contribution. For combining the block partitioning with an adaptation of the chroma QP, the following BD-rate savings are reported relative to HEVC (HM) for simulations with 49 frames:

Random access SDR-A:

- QTBT:          16.85%,   5.83%,   6.55% (Y,U,V) at   87% encoder and 84% decoder run time;

- MTT:            19.23%,   9.32%, 10.55% (Y,U,V) at 196% encoder and 83% decoder run time;

- QT+BTS-A: 17.80%,   7.08%,   8.15% (Y,U,V) at   87% encoder and 86% decoder run time;

- QT+BTS-B: 18.90%,   8.75%,   9.94% (Y,U,V) at 133% encoder and 87% decoder run time;

- QT+BTS-C: 19.62%, 10.05%, 11.29% (Y,U,V) at 222% encoder and 87% decoder run time.

Random access SDR-B:

- QTBT:          10.12%,   5.55%,   5.28% (Y,U,V) at   78% encoder and 88% decoder run time;

- MTT:            12.72%,   9.52%,   9.60% (Y,U,V) at 170% encoder and 89% decoder run time;

- QT+BTS-A: 11.86%,   7.91%,   7.61% (Y,U,V) at   81% encoder and 91% decoder run time;

- QT+BTS-B: 13.02%,   9.35%,   9.32% (Y,U,V) at 120% encoder and 90% decoder run time;

- QT+BTS-C: 14.04%, 11.13%, 11.30% (Y,U,V) at 204% encoder and 93% decoder run time.

Random access HDR:

- QTBT:            7.89%,   6.84%, 15.53% (Y,U,V) at   59% encoder and 85% decoder run time;

- MTT:            10.23%, 11.93%, 20.43% (Y,U,V) at 111% encoder and 85% decoder run time;

- QT+BTS-A:   9.53%, 11.17%, 19.62% (Y,U,V) at   63% encoder and 86% decoder run time;

- QT+BTS-B: 10.30%, 12.49%, 20.98% (Y,U,V) at   88% encoder and 86% decoder run time;

- QT+BTS-C: 11.16%, 14.68%, 23.48% (Y,U,V) at 147% encoder and 89% decoder run time.

Low delay SDR-B:

- QTBT:          10.28%, 14.72%, 15.31% (Y,U,V) at   61% encoder and 93% decoder run time;

- MTT:            12.79%, 18.53%, 19.21% (Y,U,V) at 147% encoder and 94% decoder run time;

- QT+BTS-A: 12.33%, 17.29%, 18.25% (Y,U,V) at   70% encoder and 98% decoder run time;

- QT+BTS-B: 13.39%, 18.81%, 19.60% (Y,U,V) at 110% encoder and 97% decoder run time;

- QT+BTS-C: 14.22%, 19.82%, 20.94% (Y,U,V) at 196% encoder and 98% decoder run time.

It is further reported that an adaptation of the luma-chroma-QP relationship can have a significant impact on the obtained luma BD rate savings. For the QT+BTS-B configuration, a modification of the chroma QP setting reportedly increases the average luma BD rate savings by 5.8% for RA SDR-A and 2.0% for RA SDR-B.

# Introduction

Extensions of QTBT [1] have been reported by Qualcomm Inc. in JVET-D0117 [2] and by Technicolor in JVET-D0064 [3]. Both extensions added additional split modes to the ones from QTBT. In JVET-D0117 [2] triple tree splits are proposed which are splits of one block into three subblocks with the center block being twice as large as the equal sized boundary blocks, cf. Figure 1. In JVET-D0064 [3] asymmetric binary splits for power of 2 sizes are presented. The asymmetric splits produce two sub blocks with a ratio of 1:3 and can occur vertically as well as horizontally, cf. Figure 2.

Figure 1: triple tree partitioning of a block as reported in JVET-D0117 [2]

Figure 2: Asymmetric partitioning of a block as reported in JVET-D0064 [3]

In this contribution shifted binary splits are presented that produce a superset of the above mentioned partitioning methods.

# Proposed method

The proposed quad tree plus binary tree with shifting (QT+BTS) the QTBT split modes by:

- 1/4 and 3/4: if the split side is 2n, n∈N and at least 16 in luma pixels,

- 3/8 and 5/8: if the split side is 2n, n∈N and at least  32 in luma pixels.

Figure 3: Possible splits for power of 2 sizes. First row: QTBT splits. Second row: 1:3 splits with different orientations. Third row: 3:5 splits with different orientations.

The quad split is not needed for the presented method to work and the shifted binary splits can be used without it as the quad split can be replaced by a sequence of orthogonal 1/2 splits. However, the inclusion of a quad split proves to be superior to the underlying method without the quad split.

If the size of the side that is to be split is not 2n, n∈N, then it is either 3·2n, n∈N or 5·2n, n∈N. It is proposed to split these non-power of 2 sizes by either

- 1/2: if the side to be split is at least 8 in luma pixels,

- 1/3 and 2/3: if the split side is 3·2n,

- 1/5, 2/5, 3/5, 4/5: if the split side is 5·2n.

Figure 4: Possible shifted binary splits for non-power of 2 sizes. First row: 1:2 splits with different orientations. Second and third row: 1:4 and 2:3 splits with different orientations.

## Syntax signaling

A shifted binary split is uniquely determined by its split direction and its split ratio. The direction of the binary splits are coded depending on the previous split, i.e. instead of being signaled as horizontal or vertical split, they are signaled as perpendicular split or parallel split which, however, can always be translated to a horizontal or vertical split. The first split has to be set always by convention. There are two binary flags perpend_split_flag and parallel_split_flag for the two possible directions (perpendicular and parallel). The split ratio then describes the location where to split binarily, see Figure 1 and Figure 2. It is coded using a binary decision tree. The full signaling tree for the quadtree plus binary splits with shifting (QT+BTS) without any restrictions is shown in Figure 5.

Figure 5: Full signaling tree, the perpendicular part is not displayed as it has the same sub tree as the parallel case. Branches or leafs of tree can be omitted, if the options are not available in a specific context. If a node only contains one child, the decision is not signaled.

Figure 6: Signaling tree, with different available options highlighted. The signaling a) would be used for splitting of a quadratic partition within a QT (possibly finalizing it and opening a binary tree with shifting (BTS). Signaling b) would be used for a partition within a BTS for which the side perpendicular to the last split is of size 3·2n, n∈N, thus allowing only x/3 and ½ splits parallel to the last splits.

Table 1: Recursive coding tree syntax in tabular form

| coding_tree( x0, y0, cbSizeX, cbSizeY, treeType, qtDepth, btDepth, numBtImpl, lastSplitDir ) { | Descriptor |
| --- | --- |
| if( !QtbtsNonLog2CUs && ( ( 1 << log2( cbSizeX ) ) != cbSizeX ¦¦ ( 1 << log2( cbSizeY ) ) != cbSizeY ) ) { |  |
| btDepth = btDepth - 1; |  |
| } |  |
| if( canQtSplit ) |  |
| split_cu_flag[x0][y0][cbSizeX][cbSizeY] | ae(v) |
| if( split_cu_flag[x0][y0][cbSizeX][cbSizeY] == 0 ) |  |
| split_mode( x0, y0, cbSizeX, cbSizeY, treeType, btDepth, numBtImple, lastSplitDir ) |  |
| } |  |
| if( cu_qp_delta_enabled_flag && floor( log2[cbSizeX] ) >= Log2MinCuQpDeltaSize && floor( log2[cbSizeY] ) >= Log2MinCuQpDeltaSize ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| } |  |
| if( split_cu_flag[x0][y0][cbSizeX][cbSizeY] ) { |  |
| x1 = x0 + cbSizeX/2 |  |
| y1 = y0 + cbSizeY/2 |  |
| coding_tree( x0, y0, cbSizeX/2, cbSizeY/2, treeType, qtDepth + 1, 0, 0, lastSplitDir ) |  |
| coding_tree( x1, y0, cbSizeX/2, cbSizeY/2, treeType, qtDepth + 1, 0, 0, lastSplitDir ) |  |
| coding_tree( x0, y1, cbSizeX/2, cbSizeY/2, treeType, qtDepth + 1, 0, 0, lastSplitDir ) |  |
| coding_tree( x1, y1, cbSizeX/2, cbSizeY/2, treeType, qtDepth + 1, 0, 0, lastSplitDir ) |  |
| } else if( split_perpendicular_flag[x0][y0][cbSizeX][cbSizeY] ¦¦ split_parallel_flag[x0][y0][cbSizeX][cbSizeY] ) { |  |
| x1 = x0 + ( splitDir[x0][y0][cbSizeX][cbSizeY] == 0 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : 0 ) |  |
| y1 = y0 + ( splitDir[x0][y0][cbSizeX][cbSizeY] == 1 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : 0 ) |  |
| cbSizeX0 = cbSizeX - ( splitDir[x0][y0][cbSizeX][cbSizeY] == 0 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : 0 ) |  |
| cbSizeX1 = ( splitDir[x0][y0][cbSizeX][cbSizeY] == 0 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : cbSizeX ) |  |
| cbSizeY0 = cbSizeY - ( splitDir[x0][y0][cbSizeX][cbSizeY] == 1 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : 0 ) |  |
| cbSizeY1 = ( splitDir[x0][y0][cbSizeX][cbSizeY] == 1 ? splitOffset[x0][y0][cbSizeX][cbSizeY] : cbSizeY ) |  |
| if( x0 + cbSizeX > pic_width[treeType] ¦¦ y0 + cbSizeY > pic_height[treeType] ) |  |
| { |  |
| numBtImpl = numBtImpl + 1 |  |
| } |  |
| coding_tree( x0, y0, cbSizeX0, cbSizeY0, treeType, qtDepth, btDepth + 1, numBtImpl, splitDir[x0][y0][cbSizeX][cbSizeY] ) |  |
| coding_tree( x1, y1, cbSizeX1, cbSizeY1, treeType, qtDepth, btDepth + 1, numBtImpl, splitDir[x0][y0][cbSizeX][cbSizeY] ) |  |
| } else { |  |
| coding_unit( x0, y0, cbSizeX, cbSizeY, treeType ) |  |
| } |  |
| } |  |

Table 2: split mode syntax in tabular form

| split_mode( x0, y0, cbSizeX, cbSizeY, treeType, btDepth, numBtImple, lastSplitDir ) { | Descriptor |
| --- | --- |
| if( canNotSplit && ( canParlSplit ¦¦ canPerpSplit ) ) { |  |
| bt_split_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| if( bt_split_flag[x0][y0] ) { |  |
| if( canParlSplit && canPerpSplit ) { |  |
| split_perpendicular_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| split_parallel_flag_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] = 1 - split_perpendicular_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] |  |
| if( split_perpendicular_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ¦¦ split_parallel_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ) { |  |
| splitDir[x0][y0][cbSizeX][cbSizeY] = split_perpendicular_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ? 1 - lastSplitDir : lastSplitDir |  |
| splitSize = splitDir[x0][y0][cbSizeX][cbSizeY] == 0 ? cbSizeX : cbSizeY |  |
| if( canHalfSplit && ( canFourthsSplit ¦¦ canEightsSplit ¦¦ canThirdsSplit ¦¦ canFifthSplit ) ) { |  |
| split_symmetrical_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| if( !split_symmetrical_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ) { |  |
| if( splitSize == ( 1 << log2( splitSize ) ) ) { |  |
| if( canFourthsSplit && canEightsSplit ) { |  |
| split_eights_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| } |  |
| if( canFourthsSplit ¦¦ canEightsSplit ¦¦ canThirdsSplit ) { |  |
| if( ( canThirdsSplit && is13Avail && is23Avail ) ¦¦ ( split_eights_flag[x0][y0][cbSizeX][cbSizeY] && is38Avail && is58Avail ) ¦¦ ( !split_eights_flag[x0][y0][cbSizeX][cbSizeY] && is14Avail && is34Avail ) ) { |  |
| is_split_offset_large[x0][y0][cbSizeX][cbSizeY] | ae(v) |
| } |  |
| splitBase = splitSize / ( canThirdsSplit ? 3 : ( split_eights_flag[x0][y0][cbSizeX][cbSizeY] ? 8 : 4 ) ) |  |
| splitOffset[x0][y0][cbSizeX][cbSizeY] = ( ( canThirdsSplit ¦¦ !split_eights_flag[x0][y0][cbSizeX][cbSizeY] ) ? splitBase : 3 * splitBase ) + is_split_offset_large[x0][y0][cbSizeX][cbSizeY] ? ( canThirdsSplit ? 1 : 2 ) * splitBase : 0 |  |
| } else { |  |
| if( ( is25Avail ¦¦ is35Avail ) && ( is15Avail ¦¦ is45Avail ) ) { |  |
| split_fifths_is235_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| fifths_split_id = 1 |  |
| if( split_fifths_is235_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ) { |  |
| if( is25Avail && is35Avail ) { |  |
| split_fifths_is35_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| fifths_split_id = split_fifths_is35_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ? 3 : 2 |  |
| } else { |  |
| if( is15Avail && is45Avail ) { |  |
| split_fifths_is45_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] | ae(v) |
| } |  |
| if( split_fifths_is45_flag[ x0 ][ y0 ][ cbSizeX ][ cbSizeY ] ) { |  |
| fifths_split_id = 4 |  |
| } |  |
| } |  |
| splitOffset[x0][y0][cbSizeX][cbSizeY] = fifths_split_id * splitSize / 5 |  |
| } |  |
| } else { |  |
| splitOffset[x0][y0][cbSizeX][cbSizeY] = splitSize / 2 |  |
| } |  |
| } |  |
| } else { |  |
| split_perpendicular_flag[x0][y0][cbSizeX][cbSizeY] = 0 |  |
| split_parallel_flag[x0][y0][cbSizeX][cbSizeY] = 0 |  |
| } |  |
| } |  |

The values of the split decision variable canParlSplit, canPerpSplit, canFourthsSplit, canEightsSplit, canThirdsSplit and canFifthsSplit depend on availability constraints such as minimal block sizes, maximal tree depths and the prohibitions of redundant splits given in Table 3 of this proposal.

Figure 7: Notion of different types of layouts presented in Table 3.  All layouts represent the splitting pattern after a binary split is used.
a): First block is to be split and second block not. b): First block is not split and second part is split. c): First and second blocks are split. d) Split first block and then the second subblock of the resulting block. e) Split second block and then the first subblock of the resulting block. f) Split second block and then the second subblock of the resulting block.

Table 3: prohibited redundancies

| Layout | A-Dir | A-Ratio | B-Dir | B-Ratio | C-Dir | C-Ratio |
| --- | --- | --- | --- | --- | --- | --- |
| a) | * | 2/3 | parallel | 1/2 | - | - |
| a) | * | 3/4 | parallel | 1/3 | - | - |
| a) | * | 3/4 | parallel | 2/3 | - | - |
| a) | * | 3/5 | parallel | 1/3 | - | - |
| a) | * | 3/5 | parallel | 2/3 | - | - |
| a) | * | 2/5 | parallel | 1/2 | - | - |
| a) | * | 4/5 | parallel | 1/2 | - | - |
| a) | * | 5/8 | parallel | 4/5 | - | - |
| a) | * | 5/8 | parallel | 3/5 | - | - |
| a) | * | 4/5 | parallel | 1/4 | - | - |
| a) | * | 4/5 | parallel | 3/4 | - | - |
| b) | * | 1/4 | parallel | 1/3 | - | - |
| b) | * | 3/8 | parallel | 1/5 | - | - |
| c) | * | 1/4 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 3/4 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 1/3 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 2/3 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 1/5 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 2/5 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 3/5 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | 4/5 | perpendicular | 1/2 | perpendicular | 1/2 |
| c) | * | ¾ | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 1/3 | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 2/3 | perpendicular | 1/4 | Perpendicular | 1/4 |
| c) | * | 1/5 | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 2/5 | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 3/5 | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 4/5 | perpendicular | 1/4 | perpendicular | 1/4 |
| c) | * | 1/3 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 2/3 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 1/5 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 2/5 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 3/5 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 4/5 | perpendicular | 3/4 | perpendicular | 3/4 |
| c) | * | 2/3 | perpendicular | 1/3 | perpendicular | 1/3 |
| c) | * | 1/5 | perpendicular | 1/3 | perpendicular | 1/3 |
| c) | * | 2/5 | perpendicular | 1/3 | perpendicular | 1/3 |
| c) | * | 3/5 | perpendicular | 1/3 | perpendicular | 1/3 |
| c) | * | 4/5 | perpendicular | 1/3 | perpendicular | 1/3 |
| c) | * | 1/5 | perpendicular | 2/3 | perpendicular | 2/3 |
| c) | * | 2/5 | perpendicular | 2/3 | perpendicular | 2/3 |
| c) | * | 3/5 | perpendicular | 2/3 | perpendicular | 2/3 |
| c) | * | 4/5 | perpendicular | 2/3 | perpendicular | 2/3 |
| c) | * | 2/5 | perpendicular | 1/5 | perpendicular | 1/5 |
| c) | * | 3/5 | perpendicular | 1/5 | perpendicular | 1/5 |
| c) | * | 4/5 | perpendicular | 1/5 | perpendicular | 1/5 |
| c) | * | 3/5 | perpendicular | 2/5 | perpendicular | 2/5 |
| c) | * | 4/5 | perpendicular | 2/5 | perpendicular | 2/5 |
| c) | * | 4/5 | perpendicular | 3/5 | perpendicular | 3/5 |
| d) | * | 3/4 | parallel | 1/3 | parallel | 1/3 |
| e) | * | 1/4 | parallel | 2/3 | parallel | 3/4 |
| f) | * | 1/4 | parallel | 1/2 | parallel | 1/3 |

## High-level syntax elements

There are several high-level parameters that can be used to configure the splitting tree. The proposed QT+BTS can be configured by

- CTUSize: size of largest CTU (default set to 128)

- MinQTLumaISlice: no quad splits for sizes smaller than this in luma for I slices (default set to 8)

- MinQTChromaISlice: no quad splits for sizes smaller than this in chroma for I slices (default set to 4)

- MinQTNonISlice: no quad splits for sizes smaller than this for non I slices (default set to 8)

- MaxBTDepth: maximal depth for non-quad splits for intra (default set to 3)

- MaxBTDepthI: maximal depth for non-quad splits for inter (default set to 3)

- MaxBTDepthIChroma: maximal depth for non-quad splits for chroma (default set to 3)

- MaxBTSize: maximal size for non- quad splits for intra (default set to 32)

- MaxBTSizeI: maximal size for non- quad splits for inter (default set to 128)

- MaxBTSizeChroma: maximal size for non- quad splits for chroma (default set to 64)

- QtbtsAllowFourths: use 1/4 and 3/4 splits (default set to 1)

- QtbtsAllowEights: use 3/8 and 5/8 splits (default set to 0)

- QtbtsNonLog2Halving: allow non power of 2 sized blocks to be split by 1/2  (default set to 1)

- QtbtsNonLog2CUs: allow non power of 2 sized blocks (default set to 0)

- QtbtsForceSplitToLog2: if split, the partitioning is forced to split into power of 2 sizes (default set to 1)

- MaxAsymTSize: maximal size for non symmetric splits for intra (default set to 32)

- MaxAsymTISize: maximal size for non symmetric splits for inter (default set to 64)

- MaxAsymTSizeIChroma: maximal size for non symmetric splits for chroma (default set to 32)

These high-level parameters may be transmitted in a sequence parameter set (SPS), picture parameter set (PPS), or slice header. Moreover, they can be used to restrict the QT+BTS and thereby reduce the availability in the full tree only to a subtree. For instance, if QtbtsNonLog2CUs is set to 0 and QtbtsForceSplitToLog2 is set to 1, then a 3/4 split is never available. In order to prevent transmitting unnecessary decisions the binary decision tree is pruned according to the availability of the nodes.

## Non-power of 2 adaptations

The tools in the following list needed modifications:

- Enhanced multiple transform (EMT): adapted to non-power of two block shapes.

- Frame-rate up conversion (FRUC [7]): adapted to non-power of two block shapes.

Additionally, the entropy coding of transform coefficients is adapted:

- Transform coefficient level scan-patterns adapted to non-power of two block shapes.

- Indication of last non-zero coefficient is using the HEVC defined binarization scheme including tables referred by block size index rounded up to next larger power of two value.

## Quadtree plus binary tree with shifting on edge

Figure 8: Implicit splitting of a block located on a coded picture boundary. a) Using QT splits, as in H.265/HEVC and [1]. b) using QT and binary splits.

When a block within a partitioning tree extents beyond the coded picture boundaries, it has to be split into sub-blocks, so that the sub-blocks fully contained in the coded picture can be encoded. The block which lie outside of the coded picture boundaries are ignored. In H.265/HEVC and [1], to accomplish such splitting, a QT split is implicitly inferred and no further signaling is required (see Figure).

In QT+BTS, if only two corners of a current block extent beyond coded picture boundaries, the block can be additionally split using a binary split with ratio ½ and split direction parallel to the picture boundary the block extents over. The encoder can choose, based on RD-cost, if the QT or implicit BTS split is preferred. The decision has to be signaled using a single bin (see Figure 9). If the current block is a part of a QT+BTS rather than the QT tree, the QT will not be available and the split will be implicitly inferred as the BTS split with the ratio 1/2 parallel to the boundary edge the block extents over. If the current block extents over a coded picture boundary, QT+BTS-tree restriction like maximal tree depth are ignored.

Figure 9: Reduced split signaling for block within a QT tree, for which two of its corners lie outside of the coded picture boundaries.

If a different number of corners of a current block extent over the coded picture boundaries, a QT split is implicitly inferred and no further signaling is required.

## Encoder speed ups

Due to the greater amount of split modes in this presented method several speed ups are used to reduce the RD search complexity. In addition to some of the speed ups used for QTBT the presented method has the following early determination rules.

- If at least n levels have been tested and skip was the best mode for these past n levels, then no split is performed at the current level. The number n is exemplarily set as 3.

- If a parallel and a perpendicular 1/2 split has been tested, only splits in the direction which improved the RD cost will be tested next.

- If the parallel 1/2 split and the perpendicular 1/2 split did not improve the result and skip is best, then no other split is tested.

Furthermore, there are additional restrictions for a subset of splits depending on the targeted encoder time.

- Horizontal and vertical gradients are computed and splits are performed in accordance to these values – e.g. if one gradient is much higher than the other gradient, then only splits perpendicular to the higher gradient direction are tested.

- Additional maximal depths and cu sizes are assigned to a subset of splits in order to limit the RD search.

- Partitions can be forced to end up in sizes that are powers of 2 which shrinks the range of possible splits. To balance this option, only levels than can contain a CU (e.g. with power of 2 width and height), are counted for enforcement of the depth restrictions.

Prohibitions of splits that reduce redundant representations are included in the presented method.

# Experimental results

## Configurations

The proposed method was tested in different configurations:

|  | QT+BTS | QT+BTS-2-2 | QT+BTS-2-3 | QT+BTS-2-4 | QT+BTS-3-4 | QT+BTS-5-4 |
| --- | --- | --- | --- | --- | --- | --- |
| MaxBTDepth | 3 | 2 | 3 | 4 | 4 | 4 |
| QtbtsAllowFourths | 0 | 1 | 1 | 1 | 1 | 1 |
| QtbtsAllowEights | 0 | 0 | 0 | 0 | 0 | 1 |
| QtbtsNonLog2CUs | 0 | 0 | 0 | 0 | 1 | 1 |
| QtbtsForceSplitToLog2 | 0 | 1 | 1 | 1 | 0 | 0 |

For the comparison HM 16.14 is used as a reference (100% encoder run time, 0% gain). The test set consists of the sequences from the Common Test Conditions [9] and the Call for Proposal [10]. In Figure 9 the BD [10] rate gain in luma is plotted over the encoder runtime for QPs 27, 32, 37, and 42. For detailed results (such as results for all intra, per class, and QPs 22, 27, 32, 37), the reader is referred to the accompanying Excel documentsttached tables. The number of frames is set to 49 in the presented results.

Figure 7: BD-rate gain in luma versus encoder times. First: Random access on full set. Second: Random access with CfP sequences only. Third: Low-delay B results on CfP test set.

When using the configuration QT+BTS-2-3, the proposed method has about the same average encoding time as HM 16.14 in the random access scenario for QP values 27, 32, 37, 42, but achieves roughly 10% luma bit rate savings (for the CfP sequences).  For the low delay configuration, luma bit rate savings of about 12% are achieved at 111% encoding time (CfP sequences at QP values 27, 32, 37, 42).

Comparison to block partitioning used in CfP submission

In Fraunhofer HHI’s response [12] to the Call for proposals, a binary block partitioning without quadtree splits was used. In the diagrams of Figure 7, this configuration is marked using the label “CfP”.  For the random access scenario (CfP sequences, QP values 27, 32, 37, 42), the configuration QT+BTS-2-3 increases the luma bit rate saving from 9.7% to 10.0% in comparison to the CfP setting and at the same time it reduces the encoder run time from 166% to 100%. For the low-delay scenario, the bit rate savings is increased from 11.5% to 12.1% and the encoder time is reduced from 190% to 111%.

When comparing the configuration QT+BTS-2-4 against the CfP setting (for CfP sequences and QP values 27, 32, 37, 42), the following is observed. For the random access scenario, the bit rate saving is increased from 9.7% to 10.9% while the encoding time is approximately the same (167% vs 166%). For the low delay configuration, the bit rate saving is increased from 11.5% to 12.9% at roughly the same encoding time (193% vs 190%).

Details for the CfP configuration and the settings QT+BTS-2-3 and QT+BTS-2-4 can be found in the following tables.

Table 1: Results for configuration QT+BTS-2-3 (low complexity).

Table 2: Results for configuration QT+BTS-2-4 (medium complexity).

Table 3: Results for CfP configuration (used in Fraunhofer HHI’s response to the CfP).

## Chroma QP Adaptation

The modified block partitioning generally has the effect that the resulting BD rate gains in chroma are much higher than the BD rate gains in luma (see results in sec. 3.1).  By increasing the chroma QP relative to the luma QP it is possible to obtain a setting in which a larger part of the total bit rate is used for luma.  Such a modification yields results with larger luma BD rate gains and smaller chroma BD rate gains.  In order to demonstrate this effect, we used the luma-to-chroma-QP mapping table suggested in the MediaTek proposal JVET-J0018 [13].  This table specified the following mapping:

| qPi | < 30 | 30 | 31 | 32 | 33 | 34 | > 34 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 29 | 30 | 31 | 32 | 33 | = qPi − 2 |

In addition, a chroma QP offset of 1 was used in the random access configuration for HDR and SDR-B sequences.  Simulations with and without the chroma QP adaptation were conducted for the SDR and HDR sequences specified in the CfP. As for the results in the previous section, we coded 49 frames of each test sequences.  BD-rate savings were calculated for the QP values 27, 32, 37, 42, which represent a reasonable approximation for the bit rate points specified in the CfP.

We tested three configurations of the proposed partitioning. In addition, a QTBT and a QTBT plus triple split (MTT) configuration were tested.  In Figure 8, Figure 9, Figure 10, the average luma BD rates are plotted over the encoding time for the random access SDR, low delay SDR, and random access HDR configurations, respectively. All figures contain two diagrams, one without chroma QP adaptation (left) and another with the mentioned chroma QP adaptation (right).

Detailed results with luma and chroma BDR savings are given in Table 4 and Table 5.  The largest differences between the results with and without chroma QP adaptation are observed for the random access SDR-A scenario. Here, the luma BD rate saving is increased by up to 6% by applying the chroma QP adaptation.

Figure 8: Average luma BD-rates versus encoder run times for the random access configuration and the SDR sequences of the CfP: (left) without chroma QP adaptation; (right) with chroma QP adaptation.

Figure 9: Average luma BD-rates versus encoder run times for the low delay configuration and the SDR-B sequences of the CfP: (left) without chroma QP adaptation; (right) with chroma QP adaptation.

Figure 10: Average luma BD-rates versus encoder run times for the random access configuration and the HDR sequences of the CfP: (left) without chroma QP adaptation; (right) with chroma QP adaptation.

Table 4: Average results with and without chroma QP adaptation for the CfP sequences (SDR and HDR) and QP values of 27, 32, 37, 42.

Table 5: Average results with and without chroma QP adaptation for the random access configuration and the SDR sequences of the CfP (QP values of 27, 32, 37, 42).

## Results for entire sequences (SDR)

In the following, we present additional experimental results for the complete SDR sequences.  In addition to QTBT, we tested three configurations of the proposed partitioning.  The results are obtained without any changes to the luma-chroma-QP mapping, i.e., the HEVC mapping is used without chroma QP offsets.  For the configuration “QT+BTS-2-3”, additional results with the chroma QP adaptation described in sec. 3.2 are presented. As in our other simulations, the QP values of 27, 32, 37, 42 were used, since they reasonably approximate the bit rates specified in the CfP.

Figure 11, Figure 12, and Figure 13 show diagrams, in which the luma BD rate is plotted over the encoding time.  All figures show two diagrams, one for encodings of the entire sequences and another for 49 frames. Detailed results are summarized in Table 6.

It should be noted that the differences between the tested configurations are very similar, regardless of whether 49 frames or the entire sequences are encoded.  In addition, the results demonstrated again that the luma BD rate savings can be significantly increased if the relationship between luma and chroma QP is modified (in particular for SDR-A).

Figure 11: Average luma BD-rates versus encoder run times for the random access configuration and the SDR-A sequences of the CfP: (left) complete sequences coded; (right) 49 frames coded.

Figure 12: Average luma BD-rates versus encoder run times for the random access configuration and the SDR-B sequences of the CfP: (left) complete sequences coded; (right) 49 frames coded.

Figure 13: Average luma BD-rates versus encoder run times for the low delay configuration and the SDR-B sequences of the CfP: (left) complete sequences coded; (right) 49 frames coded.

Table 6: Average results for the SDR CfP sequences and QP values of 27, 32, 37, 42.  The tables compare the results for all frames and 49 frames.  For the configuration QT+BTS-2-3, results with chroma QP adaptation (marked as “cQP”) are additionally included.

# Software

Software for testing the presented partitioning options is available in the following repository:

https://ipbt.hhi.fraunhofer.de/svn/svn_hhi-next-software/tags/NextSoftware-JVET-J0035

Except for the block partitioning, the software does not include any JEM coding tools or other coding tools beyond HEVC. The software supports the following block partitioning options:

- QTBT

- QTBT + triple tree split (MTT)

- QT+BTS (as proposed in this document)

The software package includes sample configuration files for each of these partitioning options.

Another version of the software, which additionally includes JEM coding tools, is available in the following repository:

https://ipbt.hhi.fraunhofer.de/svn/svn_hhi-next-software/tags/NextSoftware-JVET-J0035_plus_JEMTools

The JEM tools are marked with the compiler flag “JEM_TOOLS”.

# Conclusion

This proposal reports a quadtree plus binary tree with shifting. The method obtains more than 10% BD-rate gain in luma for less than 200% encoder runtime compared to HM 16.14 in the random access configuration. For low-delay B the proposed achieves almost 13% BD-rate gain in luma for roughly 200% encoder run time. For both cases the decoding time is overall equal to the decoding time of HM 16.14.

# References

- J. Chen, E. Alshina, G.J. Sullivan, J.-R. Ohm, J. Boyce, Algorithm Description of Joint Exploration Test Model 7 (JEM 7), JVET, doc. JVET-G1001, July 2017.

- X. Li, H.-C. Chuang, J. Chen, M. Karczewicz, L. Zhang, X. Zhao, A. Said, Multi-Type-Tree, JVET, doc. JVET-D0117, Oct. 2016.

- F. Le Léannec, T. Poirier, F. Urban, Asymmetric Coding Units in QTBT, JVET, doc. JVET-D0064, Oct. 2016.

- J. Chen, E. Alshina, G. Sullivan, J.-R. Ohm, J. Boye, Algorithm Description of Joint Exploration Test Model 3, JVET, doc. JVET-C1001, May 2016.

- J. Chen, Y. Chen, M. Karczewicz, X. Li, H. Liu, L. Zhang, X. Zhao, “Coding tools investigation for next generation video coding”, ITU-T/STUDY GROUP 16 Doc. COM 16 – C806 – E, Jan. 2015

- T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm and G. J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E603, 5th Meeting: Geneva, CH, 16-23 March, 2011.

- J. Chen, W.-J. Chien, M. Karczewicz, X. Li, H. Liu, A. Said, L. Zhang, X. Zhao, “Further improvements to HMKTA-1.0”, ITU-T SG16/Q6 Doc. VCEG-AZ07, Jun. 2015.

- M. Budagavi, A. Fuldseth, G. Bjøntegaard, V. Sze, M. Sadafale, “Core Transform Design in the High Efficiency Video Coding (HEVC) Standard,” IEEE Journal of Selected Topics in Signal Processing, December 2013.

- K. Sühring and X. Li, “JVET common test conditions and software configurations”, JVET, doc JVET-H1010, October 2017.

- A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC”, JVET, doc. JVET-H1002, October 2017.

- G. Bjøntegaard, “Calculation of average PSNR differences between RD-curves,” VCEG-M33, Austin, Mar. 2001.

- Albrecht, et. al, “Description of SDR, HDR, and 360° video coding technology proposal by Fraunhofer HHI”, JVET-J0014, Apr. 2018.

- Hsu, et. al., “Description of SDR video coding technology proposal by MediaTek,” JVET-J0018, Apr. 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).