| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0040-v1 |
| --- | --- |

| Title: | Set of Transforms (SoT) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mischa Siekmann, Björn Stallenberger, Christian Bartnik, Jonathan Pfaff, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI, Einsteinufer 37, 10587 Berlin, Germany | Tel: Email: | +49 30 31002-440 Mischa.Siekmann@hhi.fraunhofer.de |
| Source: | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute. |  |  |

_____________________________

# Abstract

In this document, an adaptive selection of transforms for the residual coding is proposed. For each residual block a set of 5 transform candidates is chosen from the variety of EMT, NSST and additionally trained transforms. It is reported that relative to testing the product space of EMT and NSST transforms, as implemented in JEM, the coding efficiency is increased while at the same time the encoder run time is reduced.

# Technical description

For each residual block a list of transform candidates is constructed and if required an index is signaled which identifies the selected transform among the candidates. Which transforms are considered as candidates depends on the prediction mode, the channel type and the size of the residual block. While all transforms defined by EMT [2] and NSST [2] can serve as candidates, also new transforms are defined, which have been especially trained for the newly introduced prediction modes and for dedicated block sizes. The set of transforms comprises primary and secondary transforms. Secondary transforms are used for transform blocks which width (W) or height (H) exceed 8 and are applied to the low-frequency part of the primary transform coefficients. The size of the secondary transform is given by min(8,W) x min(8,H).

For coding units with inter predicted mode, the candidate list comprises the DCT-II transform plus the 4 transforms defined by EMT for the Inter case.

In the case of line-based intra prediction [6] the transform is set to be DST7 while in the case of Intra Region-based Template Matching [7] the used transform is fixed to DCT-II.

For DC, Planar and Angular modes:

Additional to the transforms defined by EMT and NSST, a dedicated set of secondary transforms with sizes in {4x8,8x8,8x4} are provided for non-quadratic TUs.

Which of the transforms serve as candidates for a residual block depends on the prediction mode and the actual TB size. The mapping from block size and intra mode to the candidate list has been found experimentally and is given by a look-up table.

For chroma TBs the DCT-II is taken as primary transform and can be combined with the secondary transforms defined by NSST. If luma and chroma share the same partitioning, each candidate in the luma list also provides an associated nsst-index for the chroma components, which is also given by the look-up table. In the dual-tree case, chroma TUs are handled the same way as in JEM.

In the proposed configurations of SoT, each candidate list is composed of 5 transforms. For quadratic transform blocks and transform blocks which sizes do not exceed 8, either in vertical nor horizontal direction, these 5 candidates comprise a selection of EMT transforms as primary transforms possibly combined with secondary transforms defined by NSST. Which exact combinations are candidates for which intra direction and block size is given by a look-up table. For non-quadratic transform blocks of the remaining sizes, the candidates comprise a selection of EMT transforms as primary only transforms and the DCT-II as primary transform combined with trained secondary non-separable transforms implemented as full matrix multiplication.

For intra-NN prediction modes [5]:

A dedicated intraNN transform set has been trained for intra-NN predicted residuals. It comprises primary transforms of sizes in {4x4,4x8,8x8,8x4} and secondary transforms of sizes in {4x8,8x8,8x4} combined with the DCT-II as their primary transform. The candidate list for a residual block is formed by the DCT-II transform plus 4 transforms taken from the intraNN set. A direct mapping from residual block size and intra-NN prediction mode to the 4 candidates is given for sizes in S = {4, 8, 16, 32} x {4, 8, 16, 32} by a look-up table. TBs having a size not in S, use the candidates given by the next larger block size s’, which is in S. Precisely, the size s’ is obtained by taking the size in S which is in each, the vertical and horizontal dimension, the smallest in S, but has at least the size of the block. Note, these candidates are given as secondary transforms applied to the low-frequency coefficients of the DCT-II primary transform, which is defined for all occurring sizes.

The actual identification of the selected transforms among the candidates is done after the cu residual data has been parsed. First, a flag is parsed which indicates if the first candidate in list is chosen. If this is not the case, for intra coded CUs it is checked if the number of non-zero and non-transform-skip luma coefficients is at least 3. In this case, and also for inter coded CUs, a transform index is decoded which identifies the selected transform among the remaining 4 candidates. Otherwise, if the required condition on the number of significant coefficient does not hold, the first of the remaining 4 candidates is selected.

For coding the flag a dedicated context model is used for different block sizes. The transform index for the remaining 4 candidates is coded by a fixed length code with two context modeled bins, where a dedicated set of context models is used for intra and inter coded blocks.

## Transform selection

The transform type decision for an inter-coded CU is done like in JEM.

For an intra-coded CU a pre-selection of available modes is performed to reduce the number of RD checks. This pre-selection or rough mode decision is based on SATD cost. Each remaining intra mode candidate is tested together with each of the transform candidates and compared by the resulting RD cost.

# Experimental Results

The following results have been obtained with an enhanced version of the proposed Next-software [3,6]. As reference a HEVC configuration extended to the “Quadtree plus binary tree with shifting” (QT-BTS) [4] partitioning structure is used. The tools are tested on top of that configuration. The test conditions are based on those given by [1], whereas the number of frames has been reduced to 49. Test results for an extended set of test-sequences are also provided.

## Configurations:

Two different configurations of SoT are presented and compared to two different combinations of EMT and NSST. While in JEM, each transform defined by EMT can be combined with each secondary transform of NSST, an alternative implementation of the Joint Exploration Model (NextSoftware) [3] restricts NSST transforms to be applied together with DCT-II as their primary transform. Hence, EMT and NSST are used exclusive in this case. These configurations are referred to as EMT+NSST (JEM) and EMT+NSST (NEXT), respectively. The test results have been generated with EMT-Fast-Mode [2] being enabled in both cases.

The first configuration of SoT (SOT CfP) is the configuration used in [6]. Here, the encoder is configured such, that with SoT the number of obtained intra mode candidates that are tested with full RD-cost are possibly (depending on the actual most probable mode list) increased by 2. In doing so, it is taken advantage from the reduction of transform checks per mode due to SoT, by testing more candidates.

The second provided configuration (SOT v.2) does not make use of extra candidate checks. Hence, the encoder runtime is decreased by SoT, compared to both versions of EMT+NSST. Compared to the first configuration, also the additionally trained transforms have been further optimized.

## Results:

Table 1 shows the direct comparison of the results of the different configurations tested on the CfP sequences [1]. It can be seen that a set of 5 transform candidates per transform block is sufficient to maintain the coding efficiency in terms of BD-rate savings of the full product space of EMT and NSST transforms, while the encoder run-time can remarkably be decreased. Furthermore, the BD-rate saving can be even increased by further specializing the transform candidates to their prediction modes and block shapes. Of course, this specialization leads to an increased variety of transforms that has to be supplied and hence, comes with an increased demand on read-only-memory. Further investigations are needed to find an appropriate trade-off.

More detailed results can be found in the tables 2-5.

Table 1. BD-rates and run-time comparison on the bases of CfP test conditions restricted to 49 frames.

Table 2. EMT+NSST (JEM)

Table 3. EMT+NSST (NEXT)

Table 4. SoT (CfP)

Table 5. SOT (v.2)

# References

- JVET, “Joint Call for Proposals on Video Compression with Capabilities beyond HEVC,” JVET-H1002, Oct. 2017

- JVET, “Algorithmic Description of Joint Exploration Test Model 7 (JEM 7),” JEVT-G1001, July 2017.

- Wieckowski, et al., “AHG10: Updated NextSoftware as an alternative implementation of the Joint Exploration Model (JEM),” JVET-I0034, Jan. 2018.

- Ma, et al.  ”Quadtree plus binary tree with shifting”, JVET-J0035, April 2018.

- Pfaff, et al. , ”Intra Prediction Modes based on Neural Networks”, JVET-J0037, April 2018.

- Schwarz, et al. “Description of SDR, HDR and 360° video coding technology proposal by Fraunhofer HHI”, JVET-J0014, April 2018.

- Venugopal, et al. „Intra Region-based Template Matching“, JVET-J0039, April 2018.

# Patent rights declaration

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).