| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0042-v3 |
| --- | --- |

| Title: | Intra block copy for intra-frame coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuguang Zuo Li Wang Fangdong Chen Xiaodan Song | Email: | zuoxuguang@hikvision.com wangli7@hikvision.com chenfangdong@hikvision.com songxiaodan@hikvision.com |
| Source: | Hangzhou Hikvision Digital Technology Co. Ltd., Hangzhou, China |  |  |

_____________________________

- Abstract

This contribution proposes an intra block copy (IBC) method for the intra-frame coding of both natural video and screen content video. Different from the IBC technique in HEVC screen content coding (SCC) extension, the proposed IBC is implemented separately for luma and chroma in accordance with JEM. For luma frame, IBC mode is regarded as an inter mode and the frame is coded similarly to an inter frame. While for chroma frame, all blocks are treated as intra block and IBC mode is a new mode added to the candidate list.

Introduction

IBC is a block matching technique wherein the current block is predicted by a reference block located in the already reconstructed region in the same frame. Figure 1 gives a brief illustration of IBC technique. IBC can exploit the non-local similarity in the frame and is especially effective for the coding of screen content video. That is why IBC method has been adopted in the HEVC SCC extension standard.

Fig.1 Illustration of IBC

Joint Exploration Model (JEM) is the platform used to implement and evaluate new coding tools for the next generation of video coding standard currently. To improve the accuracy of intra prediction, JEM increases the number of intra modes from 35 in HEVC to 67. Also it uses an improved planar mode P-PDPC to better handle the prediction of blocks with non-directional textures. Thus the efficiency of intra frame coding is improved. However, for blocks with complex non-directional textures, the modes in JEM may still not provide a good prediction. If there are blocks in the reconstructed region sharing similar pattern with the coding blocks, IBC technique can help generate more accurate prediction. So in this contribution the IBC technique is proposed on JEM and evaluated with both natural and screen content videos. The details of the technique are descried in Section 2.

Discription of proposed IBC

In HEVC SCC, IBC mode is regarded as inter mode, and the original intra frame is changed to P frame. In JEM, the coding of luma and chroma in intra frame is separate. So the partition of luma and chroma might be different. The final coding efficiency benefits from this new coding structure. To maintain the coding structure, the implementation of IBC for luma and chroma is also different in the proposed method.

IBC for luma frame

First, for luma, the IBC mode is also regarded as inter mode. So in luma frame there are intra blocks and inter blocks. Then the luma frame can be regarded as inter frame and the coding method is basically inherit from the inter frame. Each block should be signaled to indicate its prediction type. Also for blocks predicted by IBC, the residual is transformed using only one transform set, which consists of DST-VII and DCT-VIII. This is just the same way that the inter prediction residual is transformed in JEM.

(a)                                       (b)

Fig.2 (a)The syntax of inter block of inter frame. (b)the syntax of inter block in proposed IBC method

However, considering the proportions of inter blocks and intra blocks, the syntax is changed. In the coding of inter picture, a skip_flag syntax is first coded for each block. If the value of the syntax is false, then the pred_mode_flag syntax is coded to indicate the prediction type of the block, as shown in Fig.1 (a). However, for the luma frame in intra frame, it is no longer efficient to code the skip_flag syntax for each block because the proportion of blocks choosing skip mode is rather low. Therefore, in the proposed method the skip_flag syntax is omitted when encoding luma frame. Then for each luma block, the pred_mode_flag syntax is first encoded. Also, in inter frame, it does not need to encode the cbf flag of block with merge mode because the flag value is inferred to be true, as shown in Fig. 2(a). (otherwise the block is with skip mode). Now in the proposed method, there is no skip mode block. For block with merge mode, a cbf flag is needed to be encoded to indicate the residual is zero or not, as shown in Fig. 2(b).

IBC for chroma frame

For intra-frame coding in JEM, all chroma blocks are intra coded. A candidate list of intra prediction mode is built for each block. The intra mode candidate is composed of 6 cross-component linear model (CCLM) modes and 5 traditional intra modes. The CCLM modes are in front of the traditional intra prediction modes. In the proposed method, sometimes the IBC mode is put into the candidate list of chroma block and serves as a new intra prediction mode. And the MV of the IBC mode is derived from the collocated luma blocks.

Given a chroma block, a check process is carried out to decide if the IBC mode should be put into the candidate list. The check process includes:

--Check the prediction type of the luma blocks covering the co-located five positions of the current chroma block. The five positions are checked in the order of center (CR), top-left (TL), top-right (TR), bottom-left (BL) and bottom-right (BR).

--Scale the motion vectors (MVs) of inter luma blocks according to the resolution of luma and chroma. Then check if the scaled MVs is valid for current chroma block. The scale and check operations are also in the order of CR, TL, TR, BL and BR.

If an inter luma block is found and its MV is valid for current chroma block after scaling, then the IBC mode is put into the candidate list just behind the CCLM modes. Otherwise, no IBC mode is added to the list. It does not change the length of the list when putting IBC mode into the list, which means a traditional intra prediction mode will be removed. Also the MV associated with the IBC mode is just the first scaled valid MV from the inter luma blocks.

Fig. 3 An illustration of chroma block and its collocated luma blocks

Motion estimation for IBC

The complexity of IBC mainly comes from the motion estimation (ME) process deriving the best MV for the block. To control the complexity of IBC, only blocks smaller than or equal to 32x32 employ the ME process to find the optimal MV. Also the search range is set as a rectangular area surrounding current block. The coordinates of the TL and BR points of the search area are (-SHor, -SVer) and (SHor, SVer) with the origin being the TL point of current block. The values of Shor and SVer are related to the width and height of current block, which can be expressed as

SHor=CWidth≪2,   SVer=CHeight≪2

where CWidth and CHeight are the width and height of current block, respectively.

Fig. 4 The search range of current blovk

After the search range defined, a horizontal search, vertical direction search and interlaced search is firstly executed to find 8 MVs with mimimum cost. As the interlaced search method shown in Fig. 5, only the positions in blue color are tested. Then the neighbor positions of the 8 MVs, which are in red color in Fig. 5 are further tested. Finally the position with the minimum cost is chosen as the final MV.

Fig. 5 Interlaces search and the further refinement

Experimental Results

The IBC technique is implemented on JEM 7.1 and tested under all intra configuration. The results are shown in Table 1. It can be seen that the IBC technique can achieve -0.75%, -1.03% and -1.04% gain on Y, U, V component for natural videos. While for screen content videos, the coding gain is -19.92%, -20.22 and 20.41%. Besides, the complexity increase of encoding for natural video and screen content video is 51% and 30%. The complexity of decoding remains almost the same.

Table 1 Comparison of IBC with JEM 7.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.05% | -0.07% | 175% | 101% |
| Class A2 | -1.42% | -1.75% | -1.89% | 146% | 101% |
| Class B | -1.04% | -1.60% | -1.58% | 181% | 100% |
| Class C | -0.30% | -0.42% | -0.38% | 147% | 101% |
| Class D | -0.29% | -0.55% | -0.47% | 121% | 101% |
| Class E | -1.48% | -1.88% | -1.91% | 133% | 100% |
| Overall | -0.75% | -1.03% | -1.04% | 151% | 101% |
| Class F | -19.92% | -20.22% | -20.41% | 130% | 97% |

Conclusion

This contribution proposes an IBC method on JEM. It is verified that the proposed method can improve the coding efficiency of not only screen content videos but also natural videos. Also the complexity is not too much and can be further reduced by elaborately designed ME method. It is suggested to further study the technique in CE activity..

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).