| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0043-v2 |
| --- | --- |

| Title: | AHG9:Convolutional Neural Network Filter for inter frame |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jiabao Yao Xiaodan Song Shuqing Fang Li Wang | Tel: Email:yaojiabao@hikvison.com songxiaodan@hikvision.com fangshuqing@hikvision.com wangli7@hikvision.com |  |
| Source: | Hangzhou Hikvision Digital Technology Co. Ltd. |  |  |

_____________________________

- Abstract

This contribution provides a convolutional neural network filter (CNNF) for inter frames. For intra frames, it keeps the same as JVET-I0022. For inter frames, a flag is coded to indicate whether to use CNNF or traditional filters on JEM 7.1, i.e., bilateral filter (BF), deblocking filter (DF), sample adaptive offset (SAO) and adaptive loop filters (ALF). Simulation results report -2.71%, -10.66% and -11.52% BD-rate savings for luma, and both chroma components compared with JEM 7.1 with RA configuration, and -2.82%, -9.41%, -9.56% for LDP configuration, and -2.43%, -8.01%, -8.75%for LDB configuration.

Introduction

In JVET-I0022, it is proposed to use CNNF for intra frames to replace BF, DF and SAO. 3.57%, 6.17% and 7.06% BD-rate savings are reported for luma, and both chroma components for JEM-7.0 with AI configuration. In this proposal, the possibility of CNNF applying in inter frames is investigated. In this scheme, CNNF competes with traditional filters in block level. A flag is coded to indicate which is used. Moreover, a different set of parameters is trained for inter frames within CNNF.

Technical description

The proposed scheme for inter frames

CNNF is introduced to compete with the traditional filters, i.e. DF, SAO and ALF for inter frames. Fig.1 shows the proposed encoding scheme. Reconstructions are filtered by traditional filters and CNNF, respectively. After that, the filtered reconstruction are divided into blocks with 64Ⅹ64. For each block, rate distortion optimization (RDO) is established to decide to use CNNF or traditional filtered. Note that RDO is established for luma, Cb and Cr, respectively.

Fig.1 The proposed filtering process for inter frames

A flag is encoded by CABAC to indicate the above decision, in which one represents for using CNNF and zero for traditional filters. The flags of the left and above blocks are used to generate contexts. Thus, three contexts are used to help encode the flag of the current block. For unavailable blocks, the flag is set to 0.

Network

The adopted network structure is presented in Fig.2. It is similar to that in JVET-I0022 except that network compression and fixed point operations are not established due to limited time. For inter  frames, a different model is trained, which means two sets of parameters are used, one for intra frames and one for inter frames.

Fig.2 Network structure

Experiments Results

In evaluation, JEM 7.1 serves as the reference software. The test environment is listed in Table 2.

Table 1 Evaluation environment

|  | CPU |
| --- | --- |
| CPU | Intel(R) Xeon(R) CPU E5-2680 v4 @ 2.40GHz |
| GPU | —— |
| Library | OpenBLAS 0.2.18 |

|  | Random Access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | CPU |  |  |  |
|  |  |  |  | EncT | DecT |  |  |  |  |  |
| Class A1 | -1.58% | -11.41% | -10.60% | 113% | 12426% |  |  |  |  |  |
| Class A2 | -3.39% | -10.36% | -10.84% | 124% | 13840% |  |  |  |  |  |
| Class B | -2.50% | -10.67% | -10.73% | 124% | 13361% |  |  |  |  |  |
| Class C | -3.01% | -12.16% | -14.20% | 115% | 10794% |  |  |  |  |  |
| Class D | -3.10% | -8.68% | -11.41% | 121% | 8040% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Overall (Ref) | -2.71% | -10.66% | -11.52% | 119% | 11564% |  |  |  |  |  |
|  | Low Delay B Main10 |  |  |  |  |  |  |
|  | Y | U | V | CPU |  |  |  |
|  |  |  |  | EncT | DecT |  |  |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.79% | -8.01% | -9.84% | 117% |  | 14916% |  |  |  |  |
| Class C | -2.39% | -7.44% | -8.97% | 111% |  | 10828% |  |  |  |  |
| Class D | -2.58% | -6.01% | -7.88% | 114% |  | 7511% |  |  |  |  |
| Class E | -3.37% | -11.42% | -7.82% | 149% |  | 22444% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | -2.43% | -8.01% | -8.75% | 120% |  | 12522% |  |  |  |  |
|  | Low Delay P Main10 |  |  |  |  |  |  |
|  | Y | U | V | CPU |  |  |  |
|  |  |  |  | EncT | DecT |  |  |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.10% | -9.66% | -10.30% | 132% |  | 21864% |  |  |  |  |
| Class C | -2.60% | -8.64% | -10.19% | 123% |  | 16977% |  |  |  |  |
| Class D | -2.98% | -6.57% | -8.48% | 129% |  | 11405% |  |  |  |  |
| Class E | -4.12% | -13.81% | -8.90% | 183% |  | 33750% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | -2.82% | -9.41% | -9.56% | 137% |  | 18922% |  |  |  |  |

Conclusions

In this proposal, a CNN-based in-loop filter for inter frame is investigated. Simulation results report -2.71%, -10.66% and -11.52% BD-rate savings for luma, and both chroma components compared with JEM 7.1 with RA configuration, and  -2.82%, -9.41%, -9.56% for LDP configuration, and -2.43%, -8.01%, -8.75% for LDB configuration.

Patent rights declaration(s)

Hikvision may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

[1] HM-16.6-JEM-7.1,https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-7.1/.