| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0044 |
| --- | --- |

| Title: | AHG8: Geometry padding for PERP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

The padded ERP (PERP) format was shown to effectively reduce the seam artifacts in reconstructed viewports that encompass the left and right boundaries of the ERP picture (JVET- G0098). Therefore, the PERP format was selected as the format used for the HM and JEM anchors for the 360° category in the joint CfP (JVET-H1002). However, padding and blending may not be sufficient to completely resolve the seam issue, as a seam artifact is still visible in the HM and JEM anchors for the Chairlift sequence. The seam artifact in Chairlift is particularly visible during video playback, as the video appears to flicker where face seams occur.

Geometry padding of the reference pictures was previously proposed for motion compensated prediction (JVET-D0075). Compared to the repetitive padding method used in HEVC, geometry padding can provide meaningful samples and improve continuity of neighboring samples for areas outside of the reference picture boundaries. For HM-16.12 and 8K ERP sequences, average BD-rate reduction of 0.3% and 0.8% were previously reported for ERP and cubemap formats, respectively.

This proposal investigates the performance of geometry padding for motion compensated prediction when encoding in the PERP format. The simulation results using the CfP settings show that geometry padding achieves average BD-rate reduction of 0.23%, with up to 0.98% savings for Chairlift, at a similar complexity as JEM. More importantly, it is reported that geometry padding combined with the PERP format completely solves the seam artifact problem. No seam artifact was visible in our expert viewing for the reconstructed viewports located near the PERP boundaries.

# Background

## PERP

In the PERP format [1][4], padding is applied on each of the left and the right sides of the current ERP picture. After decoding, the reconstructed ERP with padding is converted back to ERP by blending the duplicated samples (using a distance-based weighted averaging) or cropping the padded areas, as illustrated in Figure 1. In the JVET CTC for 360° video [5] and CfP anchors [2], padding of 8 luma samples is applied on each side of the ERP picture. Therefore, PERP slightly increases the number of coded samples in each picture.

| (a) | (b) |
| --- | --- |

Figure 1. Post-processing of ERP with padding: convert padded ERP (recPERP) to unpadded ERP (recERP) by (a) blending or (b) cropping.

## Geometry padding of reference pictures for motion compensated prediction in ERP

Geometry padding of reference picture was previously proposed in [3]. For the ERP format, geometry padding is straightforward. Given a point (u, v) to be padded (outside of the ERP picture), the point (u’, v’) used to derive the padding sample can be calculated as follows

|  | (1) |
| --- | --- |
|  | (2) |
|  | (3) |

where W and H are the width and height of the ERP picture. Compared to the repetitive padding of the reference pictures used in HEVC, little additional complexity is introduced by geometry padding for the ERP format.

Figure 2 shows the geometry padding process for ERP. In the left and right boundary, those samples at A, B, C, D, E and F are padded with the sample at A’, B’, C’, D’, E’ and F’. In the top boundary, those samples at G, H, I and J are padded with the sample at G’, H’, I’ and J’. In the bottom boundary, those samples at K, L, M and N are padded with the sample at K’, L’, M’ and N’. Figure 3 shows an example of the extended ERP picture using geometry padding. As shown, meaningful spherical samples are used to pad outside of the reference picture boundaries.

Figure 2. Geometry padding for ERP: the padding to be filled in at arrow (A) would be taken along arrow (A’), and so forth. The alphabetic labels show the correspondence.

Figure 3. Extended ERP picture with geometry padding: padded boundary is outside of the red box

# Geometry padding for motion compensated prediction in PERP

To extend the reference PERP picture for motion compensated prediction, the reconstructed duplicated samples are first blended using the same blending method as for the post-processing of PERP, i.e., by applying a distance-based weighted averaging. Then, for motion compensated prediction, the geometry padding method for ERP is applied using those blended samples at boundaries to extend the reference picture. Same as geometry padding for the ERP format, the geometry padding for PERP only involves a direct copy of samples already available in the blended ERP picture and can thus be performed with no additional complexity.

# Simulation results

We implemented geometry padding based on the JEM-7.0 [6] and 360Lib-5.0 [7]. The encoding was done in the PERP format using the Random Access configuration. The same QP settings as for the JEM CfP anchor [8] were used. Table 1 reports the BD-rate for PERP with geometry padding compared to the JEM anchor with repetitive padding. Geometry padding can improve the coding performance of the JEM by providing an average BD-rate savings of 0.23%, 0.31%, and 0.30% for the Y, U, and V components, respectively, based on the E2E WS-PSNR metric. The maximum BD rate savings of geometry padding can reach 0.98% for sequence Chairlift. The complexity of geometry padding is similar to that of repetitive padding.

Besides providing about 1% BD rate saving for Chairlift, the most important highlight of geometry padding is that it significantly improves the subjective quality of the viewport video. In the JEM CfP anchor, a seam artifact is clearly visible when the viewport encompasses the left and right boundaries of the 360 video. Though less visible on still images, the seam artifact is particularly objectionable during video playback, as the video appears to flicker where face seams occur. The proposed geometry padding method resolves the seam artifact issue for the Chairlift sequence, as no seam artifact was observed in our expert viewing.

Table 1. Comparison of geometry padding and repetitive padding for PERP.

|  | E2E WS-PSNR |  |  |
| --- | --- | --- | --- |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Balboa | -0.01% | -0.06% | 0.16% |
| Chairlift | -0.98% | -1.44% | -1.33% |
| KiteFlite | -0.01% | -0.01% | 0.13% |
| Harbor | 0.01% | 0.09% | -0.06% |
| Trolley | -0.14% | -0.14% | -0.41% |
| Average | -0.23% | -0.31% | -0.30% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 106% |  |  |

# Conclusion

The PERP format was proposed to resolve the seam artifact observed in the ERP format. Whereas PERP reduces the seam artifact in some sequences, it is not sufficient to resolve the issue for all sequences, as a seam artifact can still be observed in some sequences, e.g., Chairlift. Based on our findings, geometry padding can efficiently resolve the seam artifact for cases where PERP alone is not sufficient. Therefore, it is suggested that JVET establish focused group (such as CE and/or AHG) to explore the benefit of geometry padding for resolving the seam artifact issue. To demonstrate the benefits of geometry padding in resolving the seam artifact issue, we would like to request for a viewing session.

# References

- J. Boyce, Z. Deng, “EE4: Padded ERP (PERP) projection format”, JVET-G0098, July 2017, Torino, IT.

- A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC”, JVET-H1002, Oct. 2017, Macao, CN.

- Y. He, Y. Ye, P. Hanhart, X. Xiu, “AHG8: Geometry padding for 360 video coding”, JVET-D0075, Oct. 2016, Chengdu, CN.

- Y. Ye, E. Alshina, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 5”, JVET-H1004, Oct. 2017, Macao, CN.

- J. Boyce, E. Alshina, A. Abbas, Y. Ye, “JVET common test conditions and evaluation procedures for 360° video”, JVET-H1030, Oct. 2017, Macao, CN.

- JEM-7.0, https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-7.0

- 360Lib-5.0, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-5.0

- M. Zhou, J. Chen, E. Francois, P. Hanhart, “Draft template of proposal description document”, JVET-H1003, Oct. 2017, Macao, CN.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).