| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0045 |
| --- | --- |

| Title: | On low-latency reduction for template-based inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

In the JEM-7.0, two template-based inter prediction modes, namely the template-matching based frame-rate up conversion mode (FRUC) and the local illumination compensation (LIC) mode, are included.  These template-based inter prediction modes need to refer to the neighboring reconstructed samples of the current block when deriving parameters such as motion vectors or weight and offset needed to obtain the prediction signal. These template-based inter prediction techniques could complicate hardware implementation because they introduce interdependency among the decoding of spatial neighboring blocks and therefore increase decoding latency. This contribution proposes to reduce the latency of the existing template-based inter prediction techniques. Instead of using the reconstructed neighboring samples as the template sample, the proposed method uses the prediction samples of the spatial neighbors as the template samples. This way, the decoding of the current block can be invoked as soon as its neighboring prediction samples become available without waiting for the neighboring samples to be fully reconstructed (that is, residual reconstruction is bypassed). Additionally, for a better coding performance, it is proposed to add the reconstructed DC value onto the prediction samples to form the template samples. Simulation results show that the proposed method can maintain the majority of the coding gain achieved by the template-based inter prediction modes, while offering the benefit of reduced encoding/decoding latency.

# Problem statement

Like its predecessors, HEVC employs motion compensated prediction (MCP) to reduce the temporal redundancy between neighboring pictures. Because the MCP only uses the samples from already decoded pictures to predict the samples in the current picture, there is no dependency between the MCPs of spatial neighboring blocks. This means that the decoding processes of multiple inter coding units (CUs) in one picture can performed in parallel.

In the JEM, there are two template-based inter prediction methods, i.e., the template-matching based frame-rate up conversion (FRUC) and local illumination compensation (LIC). To reduce the signaling overhead, some coding parameters used by the template-based inter prediction (e.g., the MVs for the FRUC, and the scaling factor and the offset for the LIC) are derived at both encoder and decoder using the neighboring reconstructed samples of one CU. Although it can provide good coding gains, the current design of the template-based inter prediction is reported to have serious latency issue, because the decoding of one CU cannot start until all its neighboring samples are fully reconstructed.

# Proposed solution

Given the problem mentioned above, in this contribution, it is proposed to reduce the latency of the existing template-based inter prediction modes. Specifically, the main modules of the template-based inter prediction methods, e.g., the template-matching based FRUC motion search and the LIC-based sample adjustment, remain the same as the existing design. However, the derivation of the template samples is modified in the proposed method: instead of using reconstructed samples, the proposed method uses the prediction samples as the template samples for the template-based inter prediction. In this way, the decoding of one template-based CU can be immediately started as soon as the prediction signals of its spatial neighboring blocks are generated. When OBMC is enabled for the current block, to maximize the latency reduction, the prediction samples that are generated before the OBMC are used as the template samples in the proposed method.

Additionally, when the prediction samples are used as template samples, the following changes are also applied in this contribution:

- Because reconstructed residuals are not applied, prediction signals are less accurate than the fully reconstructed signals as template samples, therefore resulting in some coding performance degradation. To achieve a better performance, it is proposed to add the reconstructed DC component of one block onto its prediction samples to form the template samples. Given that the reconstruction of the DC can be easily done by de-quantizing the transform coefficient at the lowest frequency, available after parsing, without applying the full inverse quantization and inverse transform process, this method does not increase decoding latency.

- In the JEM, the FRUC template-matching mode uses a fixed template size, i.e., 4 rows/columns of neighboring samples as the top/left template samples for the current block. In this contribution, it is proposed to use an adaptive template size. Assume the size of the current block is WxH. For the top template samples, if H <= 8, then the top template size is Wx2; otherwise, the top template size is Wx4. Similarly, for the template samples to the left of the block, if W <= 8, then the left template size is 2xH; otherwise, the left template size is 4xH.  Thus, the proposed template size is smaller than or equal to the existing template size in the JEM, and therefore reduces computational complexity.

# Simulation results

The proposed method has been implemented on top of the CfP response software from InterDigital Communications and Dolby Laboratories [1]. All the simulations are conducted based on the random-access configuration and by following the test conditions as specified in the CfP document [2]. The results of [1] are used as anchor. To verify the performance of the proposed method, two tests are conducted:

- The template-based inter prediction method is disabled (i.e., either FRUC template-matching or LIC or both are disabled)

- The proposed method of forming template samples using prediction samples and adding reconstructed DC component is used for that template-based inter prediction method

Additionally, to understand the respective impact of the proposed methods on the FRUC template-matching and the LIC, these tests are performed for the FRUC template-matching only, the LIC only, and both the FRUC template-matching and the LIC. Table I summarizes the average BD-rate performance of different test cases compared to the anchor, which uses the existing template-matching based modes using the reconstructed samples as template samples. Readers are referred to the spreadsheet attached with the contribution for detailed results.

Table I. The average BD-rate performance of different tests compared to the anchor

| Test cases |  | Average |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| FRUC template-matching only | Disable | 2.55% | 2.17% | 2.36% |
|  | Proposed | 0.30% | 0.20% | 0.19% |
| LIC only | Disable | 0.91% | 0.39% | 0.49% |
|  | Proposed | 0.17% | -0.06% | -0.15% |
| FRUC template-matching and LIC | Disable | 3.46% | 2.56% | 2.85% |
|  | Proposed | 0.46% | 0.23% | 0.16% |

As shown in Table I, disabling the template-based inter prediction could incur an average luma BD-rate loss by 2.55% and 0.91% for the FRUC template-matching and the LIC, respectively. These are essentially the “tool off” test results for these two template-matching based modes.  Such coding loss can be significantly reduced by enabling the respective template-matching based mode, but using the proposed method to form the template samples based on prediction samples plus the reconstructed DC. Compared to disabling these tools, the luma BD-rate loss is reduced to 0.3% and 0.17% for the FRUC template-matching and the LIC, respectively. Finally, when applying the proposed method to both the FRUC template-matching and the LIC, the proposed method can maintain most of the coding gains from these two modes: compared to luma BD-rate loss of 3.46% when both tools are disabled, the proposed method incurs only an average luma BD-rate loss of 0.46%.

Table II. The average BD-rate performance of the proposed method vs. disabling FRUC template-matching and/or LIC

| Test cases | Average |  |  |
| --- | --- | --- | --- |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| FRUC template only | -2.19% | -1.92% | -2.11% |
| LIC only | -0.73% | -0.45% | -0.64% |
| FRUC template + LIC | -2.96% | -2.37% | -2.75% |

# References

- X. Xiu, P. Hanhart, R .Vanam, Y. He, Y. Ye, T. Lu, F. Pu, P. Yin, W. Husak, T. Chen, “Description of SDR, HDR, and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories,” JVET-J0015, San Diego, USA, Apr. 2018.

- A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC,” JVET-H1002, Oct. 2017.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).