| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0049 |
| --- | --- |

| Title: | Coding performance of Tencent’s structure only scheme |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Xin Zhao, Shan Liu | Tel: Email: | {xlxiangli, xinzzhao, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the coding performance of Tencent’s structure only scheme. The scheme is on top of Next Software with newly introduced structure modifications while disabling Next tools by cfg options. Compared to HM-16.6 anchor, over 12% luma BD rate reduction for constraint set 1 and 2 are reportedly achieved, respectively.

# Introduction

The improvement of block structure contributes a big portion of coding gain over HM and JEM anchors in Tencent’s CfP response [1] and later improvement [2] which are on top of Next software [4]. In this proposal, the coding performance of structure only scheme of [2] is studied by disabling Next tools with cfg options while keeping structure related tools on.

# Proposed block structure

In JVET-J0029 [1], block structure is based on quad-tree (QT) + binary-tree (BT) + triple tree (TT). In [2], a new tree type SplitToSquare is further proposed. With this tree type, a block is split into largest same-size square sub-blocks. In the current implementation, Split2Square can only be used when the width to height ratio of an input block is 1:1, 1:4, or 4:1.

# Simulation results

The proposed scheme is on top of [2]. All Next tools except structure related changes and multi-parameter CABAC are disabled by cfg options. Please note that HEVC CABAC engine cannot be directly used since it does not support small (like 2×2) coefficient group (CG) which is resulted from non-quad-tree modes. It is reported that the coding performance improvement by multi-parameter CABAC engine was around 1.2% for random access configuration [3].

Simulations are performed under the same QP settings used in Constraint Set 1 and 2 of JVET-J0029. The results are summarized in Table 1 and Table 2. It can be observed that 14.76% and 13.25% luma BD-rate reduction is obtained with moderate coding time increase for constraint set 1 and 2, respectively. Considering the impact by multi-parameter CABAC engine is around 1.2%, the gain by structure only is quite significant.

Table 1 Coding performance for constraint set 1 (random access)

|  | Over HM-16.6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class UHD | -16.87% | -17.95% | -19.57% | 320% | 123% |
| Class HD | -12.66% | -22.41% | -25.48% | 266% | 114% |
| Overall (Ref) | -14.76% | -20.18% | -22.52% | 292% | 118% |

Table 2 Coding performance for constraint set 2 (low delay)

|  | Over HM-16.6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| HD | -13.25% | -35.18% | -37.43% | 209% | 105% |
| Overall (Ref) | -13.25% | -35.18% | -37.43% | 213% | 106% |

# Conclusions

In this contribution, the coding performance of Tencent’s structure only scheme is reported. Compared to HM-16.6 anchor, over 14.76% and 13.25% luma BD rate reduction for constraint set 1 and 2 are achieved with multi-parameter CABAC on. It is proposed to include structure coding tools into test model.

# References

- X. Li, X. Xu, X. Zhao, J. Ye, L. Zhao, S. Liu, “Description of SDR video coding technology proposal by Tencent”, doc. JVET-J0029, April 2018.

- X. Li, X. Xu, X. Zhao, J. Ye, L. Zhao, S. Liu, M. Xu, G. Li, “Improvement on top of Tencent’s CfP response”, doc. JVET-J0047, April 2018.

- E. Alshina, A. Alshin, K. Choi, M. Park, "Performance of JEM 1 tools analysis", doc. JVET-B0022, February 2016.

- Wieckowski, T. Hinz, V. George, J. Brandenburg, J. Ma, S. De-Luxán-Hernández, B. Bross, H. Schwarz, D. Marpe, T. Wiegand, “AHG10: Updated NextSoftware as an alternative implementation of the Joint Exploration Model (JEM)”, JVET-I0034, Gwangju, KR, 20–26 Jan. 2018.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).