| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0050-v2 |
| --- | --- |

| Title: | Intra block copy improvement on top of Tencent’s CfP response |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu, Xiang Li, Guichun Li, and Shan Liu Tencent 661 Bryant St, Palo Alto, CA 94301 | Tel: Email: | +1 650-798-3300 xiaozhongxu@tencent.com xlxiangli@tencent.com guichunli@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution describes the technical aspects of intra block copy (IBC) implementation improvements on top of Tencent’s CfP response. It is reported that 0.31% luma BD rate change over Tencent’s software anchor for SDR constraint set 1 was observed when IBC mode is turned off. It is also reported that 58.49% RA luma BD rate change was observed over Tencent’s software anchor for screen content test sequences (TGM 1080p category).

# Introduction

Intra block copy (IBC) [2] is a tool adopted in HEVC extensions on SCC. It is well known that it significantly improves the coding efficiency of screen content materials. As screen contents become more and more popular, it is desirable that the next generation video coding standard includes screen content as one of its target applications. Therefore, IBC is implemented as part of Tencent’s CfP response [1]. The software platform is based on the NEXT software [3] which is an alternative implementation of JEM software [4].

# Algorithm description

The implementation of IBC in this contribution is based on the NEXT software, as part of Tencent’s corresponding CfP response [1]. After the submission of CfP, a few improvements have been achieved. The following are a list of technical aspects for IBC improvements in this document. Other aspects of the IBC implementation can be referred from Tencent’s CfP response [1].

- Unification of IBC merge candidate list and inter merge candidate list into one list

  - The current block is coded in IBC mode if the merge index points to a merge candidate that is coded in IBC mode; if it points to an inter coded candidate, the current block is coded in inter mode

- Move IBC flag signaling from before merge flag to after merge flag, and explicitly signal this flag when merge flag is false (non-merge case).

- If a block is coded as a non-merge IBC mode, the BVD resolution is switchable between 1-pel integer and 4-pel integer at block level

The flowchart of IBC syntax signaling is summarized in Figure 1.

Figure 1 Flowchart of IBC syntax signaling

# Simulations

The proposed intra block copy improvements are based on Tencent’s CfP response software, where the IBC mode has already been implemented.

IBC mode is tested for coding of screen content materials, in addition to CfP sequences. In Table 1 below, the BD rate changes are shown for CfP sequences. The testing QPs are chosen for the sequences to work approximately on the targeted bitrates, as indicated in joint CfP document [5]. In Table 2 below, the BD rate changes for a set of HD (1080p) sequences in Text and Graphics with Motion (TGM) category are shown. The sequences and testing QPs are selected from HEVC SCC CTC [6]. Other test conditions follow the simulation conditions in Tencent’s corresponding CfP response document. In these two tests, the anchor is the CfP response software with IBC mode. The tested one is the software without IBC mode.

- Simulation Results for CfP Sequences

| Sequences | RA | Enc Time | Dec Time |
| --- | --- | --- | --- |
| FoodMarket4 | -0.04% | 98% | 97% |
| CatRobot1 | 0.34% | 97% | 98% |
| DaylightRoad2 | 0.61% | 97% | 98% |
| ParkRunning3 | -0.03% | 96% | 98% |
| CampfireParty2 | 0.01% | 96% | 99% |
| BQTerrace | 1.94% | 95% | 93% |
| RitualDance | -0.05% | 96% | 98% |
| MarketPlace | -0.03% | 95% | 98% |
| BasketballDrive | 0.06% | 96% | 97% |
| Cactus | 0.28% | 96% | 98% |
| Average | 0.31% | 96% | 97% |

- Simulation Results for SCC Sequences

| Sequences | RA | Enc Time | Dec Time |
| --- | --- | --- | --- |
| Console | 65.56% | 87% | 115% |
| Desktop | 110.43% | 89% | 100% |
| FlyingGraphics | 20.63% | 93% | 111% |
| ChinaEditing | 37.34% | 93% | 92% |
| Average | 58.49% | 91% | 104% |

# Closing remarks

This proposal describes the intra block copy improvements on top of Tencent’s response to the joint CfP on video compression with capability beyond HEVC by ITU-T SG16 Q.6 (VCEG) and ISO/IEC JTC1/SC29/WG11 (MPEG) [1]. Compared to the anchor with this tool, when IBC mode is turned off, 0.31% luma BD rate change for SDR constraint set 1 was observed. In addition, 58.49% RA luma BD rate change for SCC HD sequences was observed accordingly.

# Refences

- Xiang Li, et al, “Description of video coding technology proposal by Tencent”, JVET-J0029, San Diego, US, April 2018

- X. Xu, et al, “Intra Block Copy in HEVC Screen Content Coding Extensions,” IEEE J. Emerg. Sel. Topics Circuits Syst., vol. 6, no. 4, pp. 409-419, 2016.

- Wieckowski, T. Hinz, V. George, J. Brandenburg, J. Ma, S. De-Luxán-Hernández, B. Bross, H. Schwarz, D. Marpe, T. Wiegand, “AHG10: Updated NextSoftware as an alternative implementation of the Joint Exploration Model (JEM)”, JVET-I0034, Gwangju, KR, 20–26 Jan. 2018.

- JVET Reference software Ver. 7.0 (JEM-7.0):  https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-7.0

- A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC”, JVET-H1002, Macau, CN, Oct. 2017

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, Common Test Conditions for Screen Content Coding, JCTVC-U1015, Proceeding of 19th JCT-VC meeting, Jun. 2015.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).