| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0052-v1 |
| --- | --- |

| Title: | Tencent test sequences for video coding development |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Ye, Xiang Li, Shan Liu, Lori Wu, Chemin Xie, Keane Liu, Berton Wang, Ping Liu, Kane Dong, Ying Kuang, Weizhong Feng Tencent America LLC 661 Bryant St, Palo Alto, CA 94301 | Tel: Email: | +1 650-798-3300 jingye@tencent.com xlxiangli@tencent.com shanl@tencent.com loriwu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution describes 10 test sequences for the development of next generation video coding standard. Among them, there are 6 concert related sequence (1920x1080, 25 fps, 4:2:0), 2 eSports sequences (1920x1080, 30 fps, 4: 2: 0), and other 2 eSports sequences (2176x1080, 30 fps, 4: 2: 0).

# Introduction

As the online streaming service is more and more popular. More and more contents can be streamed on the internet and consume a lot of bandwidth. Millions of viewers can watch live concert, or live eSports on the internet. Those contents are quite different from the traditional nature video, and have their unique characteristic, such as frequently light changing, larger scenes with zoom-in, zoom-out, panning, rotation, and a lot computer generated texture details.

In 121 MPEG meeting, Gwangju, Korean, January 2018, some experts expressed the interests on concert sequences and eSports sequences. We believe these contents should be included in the test set for developing next generation video coding standards. Therefore, we are pleased to contribute such sequences to JVET.

# Sequence description

The test set is composed of 8 HD (1920x1080) sequences in YUV 4:2:0 8 bits, and 2 resolutions of 2176x080 sequences in YUV 4:2:0 8 bits.

| Sequence | Capture way | Frame rate | Resolution | Color primaries | Frame number |
| --- | --- | --- | --- | --- | --- |
| Concert1_man_n_robot | Atomos Shogun Studio | 25 fps | 1920x1080 | 709 | 250 |
| Concert2_group_dance1 | Atomos Shogun Studio | 25 fps | 1920x1080 | 709 | 250 |
| Concert3_band_rising | Atomos Shogun Studio | 25 fps | 1920x1080 | 709 | 390 |
| Concert4_robot_band | Atomos Shogun Studio | 25 fps | 1920x1080 | 709 | 250 |
| Concert5_drum | Atomos Shogun Studio | 25 fps | 1920x1080 | 709 | 250 |
| Concert6_group_dance2 | Atomos Shogun Studio | 25 fps | 1920x1080 | 709 | 250 |
| AOV_middle_group_fight | Capture screen | 30 fps | 2176x1080 | 709 | 300 |
| AOV_small_group_fight | Capture screen | 30 fps | 2176x1080 | 709 | 300 |
| PUBG_MOBILE_car | Capture screen | 30 fps | 1920x1080 | 709 | 300 |
| PUBG_MOBILE_parachute | Capture screen | 30 fps | 1920x1080 | 709 | 300 |

Table 1 – Test set

They have been choosen for the purpose of compression research. They cover a large variety of contents with studio or external shooting and contain challenging video characteristics for codecs, like:

- camera panning,

- Frequently light changing,

- challenging texture structures,

- saturated colour,

- large contrast/dynamic range.

| Concert1_man_n_robot |  |
| --- | --- |
| Concert2_group_dance1 |  |
| Concert3_band_rising |  |
| Concert4_robot_band |  |
| Concert5_drum |  |
| Concert6_group_dance2 |  |
| AOV_middle_group_fight |  |
| AOV_small_group_fight |  |
| PUBG_MOBILE_car |  |
| PUBG_MOBILE_parachute |  |

# Conversion process

The six concert sequences are captured by Atomos Shogun Studio and created bitrate of 115Mb/s MOV files. FFMPEG [1] is used to generate the YUV from the MOV files. The command line is as below:

ffmpeg -i input.mov -c:v rawvideo -pix_fmt yuv422p -deinterlace output.yuv

Then in YUV 4:2:2 is converted into YUV 4:2:0 by FFMPEG, and the command line is as below:

ffmpeg.exe -y -pix_fmt yuv422p -s 1920x1080 -r 25 -i 1520014897673_ yuv422.yuv -f rawvideo -pix_fmt yuv420p -s 1920x1080 -r 25 output_420.yuv

The two 1920x1080 eSports sequences are captured by monitor/phone/tablet screen with a modified eGame [2] platform that can capture the YUV information via GPU and store the YUV in the local HDD. The captured YUV is 4:4:4 format, and it is converted into 4:2:0 format by the HDRtools [3].

The rest two 2176x1080 eSports sequences are captured by modify the android code and generate an APK file to capture the RGB information of the screen. Then the RGB is converted into YUV 4:4:4. The YUV 4:4:4 is converted into YUV 4:2:0 by the HDRtools [3].

# Test sequence evaluation

RD performance of those test sequences are evaluated by HM16.16 [4] and JEM7.0[5] platform. They have been encoded using RA and LDB configurations. For each sequence and configuration, four QP values 22-27-32-37 have been tested. We have used the configuration files provided on Aachen server. The test results are in Table 1.

The intra period in RA configuration has been set to:

- 32 for 25 fps content,

- 32 for 30 fps content

Table 1. JEM and HM BD rate of the proposed test sequences

The RD curves of the proposed method is in Fig. 1 to Fig 4.

Fig. 1 Rate-Distortion curves of proposed sequences (RA JEM 7.0)

Fig. 1 Rate-Distortion curves of proposed sequences (RA HM 7.0)

Fig. 1 Rate-Distortion curves of proposed sequences (LDB JEM 7.0)

Fig. 1 Rate-Distortion curves of proposed sequences (LDB HM 7.0)

- Reference

- https://www.ffmpeg.org/

- https://egame.qq.com/

- HDRTools0.16, https://gitlab.com/standards/HDRTools/tags/v0.16

- https://vceg.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6

- https://vceg.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags//HM-16.6-JEM-7.0

- Copyright information

This sequence and all intellectual property rights therein remain the property of Shenzhen city Tencent computer system Co. Ltd. and Tencent America LLC. This sequence may only be used for the purpose of developing, testing and promulgating technology standards (the Purpose). Shenzhen city Tencent computer system Co. Ltd. and Tencent America LLC. make no warranties with respect to this sequence and expressly disclaim any warranties regarding its fitness for any purpose. This disclaimer shall remain attached to this sequence except when this sequence is used for the Purpose.