| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0053 |
| --- | --- |

| Title: | Intra-prediction Mode Propagation for Inter-pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1-858-845-0281 zhangkai@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents an Intra-Prediction Mode (IPM) propagation approach for inter-pictures. Each 4×4 sub-block in an inter-coded Coding Unit (CU) is assigned with a propagated IPM, which is fetched from a reference block in a reference picture, located by motion vectors of the 4×4 sub-block. The propagated IPM can be used in two aspects. First, an inter-coded CU with the merge mode can be predicted by a weighted sum of inter-prediction and intra-prediction with the propagated IPM. Second, the propagated IPM can be used as a predictor for intra mode coding. Simulation results reportedly show 0.5% and 0.3% BD rate savings on average for Random Access (RA) and Low Delayed B (LDB) configurations, respectively, compared to JEM-7.1.

# Introduction

In JEM, a Coding Unit (CU) can be either inter-coded or intra-coded in an inter-picture. When a CU is inter-coded, it is predicted with inter-prediction known as motion compensation (MC), controlled by motion information including Motion Vectors (MV) and reference picture indices. When a CU is intra-coded, it is predicted by intra-prediction, controlled by an Intra-Prediction Mode (IPM). As a result, an inter-coded CU is assigned with the motion information only, leaving its IPM invalid; while an intra-coded CU is assigned with the IPM only, leaving its motion information invalid. This uni-mode design has two problems. First, only one of inter-prediction and intra-prediction modes can be applied in a block, meaning that the temporal correlation and spatial correlation cannot be utilized at the same time. Second, inter-coded neighbouring blocks cannot be used to predict the IPM of an intra-coded CU in an inter-picture, meaning that the coding of IPM in an inter-picture is usually less efficient than in an intra-coded picture.

# Proposed Methods

This contribution presents an IPM propagation approach for inter-pictures. Each inter-coded block is additionally assigned with a propagated IPM. The propagated IPM can be used to conduct inter-intra prediction and predict IPMs of following intra-coded CUs.

## Intra Prediction Mode Propagation

IPM propagation is performed for each 4×4 sub-block in an inter-coded CU. The propagated IPM of a sub-block is fetched from a reference block in a reference picture. The reference block is located with the MV used in MC. The reference block may be intra-coded or inter-coded. In the latter case, the propagated IPM of the reference block is used. Fig. 1 shows an example how IPM X is propagated from an intra-coded block A, through an inter-coded block B, to another inter-coded block C.

Fig.1 An example of IPM propagation.

If a sub-block is bi-predictive coded, a priority-based method is designed to select one of two IPMs. That is, the IPM of a reference block which is intra-coded, or has an IPM not equal to DC/Planar mode, or located in a reference picture closer to the current picture, or has a lower QP, is selected.

## Inter-intra Prediction

With inter-intra prediction on the luma component, a prediction block P1 is generated using intra-prediction with the propagated IPM of the top-left 4×4 sub-block in the current CU, and another prediction block P2 is generated using inter-prediction with motion information of the current CU. The final prediction block P is generated as the weighted sum of P1 and P2 as the formula shown below:

Px,y=w1x,y×P1x,y+w2x,y×P2x,y≫s

where (x, y) is the coordinate of a sample in the current block. w1 is the weighting value for P1, w2 is the weighting value for P2, and w1+ w2=2s.

For luma coding, the weighting values are position-dependent, which can be calculated by a quadratic function as

w1x,y=a×x2+b×y2+c×x×y+d×x+e×y+f≫sPx,y=w1x,y×P1x,y+w2(x,y)×P2(x,y)

where the six parameters a, b, c, d, e, and fPx,y=w1x,y×P1x,y+w2(x,y)×P2(x,y) are off-line trained.

For chroma coding, P1 is generated using intra-prediction with the MMLM mode employed in JEM [1]. Moreover, a linear function instead of a quadratic function is used to derive w1, i.e., a=b=c=0.

The proposed inter-intra prediction can be applied to inter-coded blocks with merge-mode or FRUC-mode. For a CU with merge-mode, up to four more inter-intra merge candidates are inserted into the candidate list. An inter-intra merge candidate copies the motion information of a normal merge candidate previously inserted. Accordingly, the size of merge candidate list is increased by four. At encoder, only four more SATD checks are required while no additional RDO checks are needed. For a CU with FRUC-mode, an additional flag is signaled when inter-intra prediction is applicable. Inter-intra prediction is used if the flag is equal to 1. At encoder, one more RDO test with inter-intra prediction is added for each FRUC mode.

Inter-intra prediction is only applicable when the both the width and height of the CU is no larger than 64.

## Intra prediction mode coding in inter-pictures

Propagated IPMs of neighboring blocks are used to construct the Most Probable Mode (MPM) list. Fig. 2 shows the five neighbouring blocks used to construct the MPM list in JEM [1]. In this contribution, the checking order of five spatial neighbors (L, A, BL, AR, AL) is modified as follows: IPMs of L and A if they are intra coded, propagated IPMs of L and A if they are available, IPM of BL, AR and AL if they are intra-coded, and propagated IPMs of BL, AR and AL if they are available.

Fig. 2 Neighbouring blocks for MPM derivation.

# Experimental Results

The proposed IPM propagation method has been implemented on JEM-7.1 [2], and results of Random Access (RA) and Low-Delayed B (LDB) configurations are summarized in the Table 1 and Table 2 respectively. The simulations were performed following the JVET CTC [3].

To investigate the inter-action of the proposed method and existing tools in JEM, the proposed IPM propagation method has also been tested on a simplified JEM-7.1 with all JEM tools off except the QT/BT structure, which is equal to HM+QT/BT structure. Since there is no MMLM mode in simplified JEM-7.1, P1 is generated using intra-prediction with the Planar mode for chroma coding. Results of Random Access (RA) are summarized in the Table 3.

More detail results can be found in the accompanying Excel tables.

Table 1 Propagated IPM Results with all JEM tools on (RA)

|  | RA Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.0% | -1.7% | -2.3% | 111% | 111% |
| Class A2 | -0.5% | -1.6% | -1.2% | 108% | 105% |
| Class B | -0.4% | -0.8% | -0.2% | 107% | 104% |
| Class C | -0.3% | -0.8% | -0.8% | 110% | 105% |
| Class D | -0.1% | -0.4% | -0.7% | 111% | 103% |
| Class E |  |  |  |  |  |
| Overall (Ref) | -0.5% | -1.0% | -1.0% | 109% | 106% |

Table 2 Propagated IPM Results with all JEM tools on (LDB)

|  | LDB Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.4% | -3.0% | -2.3% | 112% | 112% |
| Class C | -0.4% | -2.5% | -2.5% | 109% | 108% |
| Class D | -0.1% | -1.9% | -2.4% | 111% | 106% |
| Class E | -0.2% | -1.4% | -1.6% | 108% | 111% |
| Overall (Ref) | -0.3% | -2.3% | -2.3% | 110% | 109% |

Table 3 Propagated IPM Results with only QT/BT on (RA)

|  | RA Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.1% | -0.3% | -0.3% | 109% | 121% |
| Class A2 | -0.6% | 0.4% | 0.5% | 108% | 119% |
| Class B | -0.6% | 0.6% | 0.8% | 110% | 116% |
| Class C | -0.5% | -0.1% | 0.1% | 110% | 109% |
| Class D | -0.1% | 0.4% | 0.2% | 112% | 104% |
| Class E |  |  |  |  |  |
| Overall (Ref) | -0.6% | 0.2% | 0.3% | 110% | 114% |

# References

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of Joint Exploration Test Model 7 (JEM7)”, JVET-G1001, Aug. 2017.

- JEM7.1: https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-7.1/

- K. Suehring, X. Li, JVET-H1010, “JVET common test conditions and software reference configurations”, Nov. 2017.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).