| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0056-v1 |
| --- | --- |

| Title: | Multi-Dimensional Filter Selection for Deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong, Yung-Hsuan Chao, Wei-Jung Chien, Li Zhang, Marta Karczewicz | Tel: Email: | +1-408-533-9515 jiedong@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution presents a multi-dimensional filter selection scheme for deblocking. The filter selection for a sample is four-dimensional, i.e., determined by four facts: the average local activities of Blocks P and Q, the difference of local activities of Blocks P and Q, the type of the belonged block (Type 0 or Type 1), and the distance from the segment. Given a combination of the four facts, the filter index is not fixed, but adaptively determined by the encoder and signaled in the bitstream. This contribution leverages a bit more computation resource for significant coding efficiency improvement, while still being easy for parallel processing.

# Introduction

Blocking artifacts, the horizontal and vertical discontinuities that do not exist in the original picture, are caused by moderate to high compression. Deblocking modifies the values of the samples located near block boundaries, by filtering, clipping, or any other means, such that the discontinuity is smoothed and not so visible. The deblocking in HEVC is applied to the boundaries of coding units (CU), prediction units (PU), or transform units (TU). A boundary to be processed is divided into multiple non-overlapped 4-sample segments, and a segment is the unit that the deblocking method applies on.

The HEVC deblocking on a segment includes filtering the nearby samples and clipping the change of sample magnitudes. The filtering part offers three levels of filter strengths: strong, normal, or zero (i.e., no filtering), corresponding to increasing levels of local activities. For the strong filtering mode, 3 samples on each side of a segment are filtered; for the normal filtering mode, up to 2 samples on each side are filtered.  There are totally 5 pre-defined 4- or 5-tap filters, pre-assigned to the 3 samples in strong filtering mode and the 2 samples in normal filtering mode, respectively. As can be seen, the filter selection for a certain sample depends on two facts: the level of local activity and its distance to the segment. Given a possible combination of these two facts, the filter is pre-assigned.

The clipping part is controlled by the parameter tC. In strong filtering, the change of sample magnitude is no greater than 2tC; in normal filtering, the magnitude changes of the 1st and 2nd samples are no greater than tC and tC/2, respectively. BS, determined based on coding conditions for each segment, has an impact on tC selection, i.e., the segment with BS equal to 2 has greater tC than the segment with BS equal to 1.

The HEVC deblocking has two consequential stages: (1) filter all the vertical block boundaries and (2) filter all the horizontal block boundaries. In Stage (2), the samples used for mode decision and filtering are the output of Stage (1). In each stage, where boundaries being deblocked are all parallel and at least 8 samples apart, the samples involved in deblocking one boundary do not overlap with the samples involved in deblocking any other boundaries and therefore one boundary can be deblocked in parallel to any other boundaries.

The HEVC deblocking essentially follows the framework of H.264/AVC deblocking, but enables easier parallel processing and a better fit into HEVC’s larger and more flexible block-size coding structure. However, HEVC deblocking, which was considered a good trade-off between computational complexity and coding efficiency at the finalization of HEVC in 2013, may be oversimplified, considering today’s highly developed computation capability.

# Proposed Deblocking Scheme

This contribution tries to leverage a bit more computation resource for significant coding efficiency improvement, while still being easy for parallel processing. It features the following design elements.

1. The filter selection for a sample is four-dimensional, i.e., determined by four facts: the average local activities of Blocks P and Q, the difference of local activities of Blocks P and Q, the type of the belonged block (Type 0 or Type 1), and the distance from the segment.

2. Given a combination of the four facts, the filter index is not fixed, but adaptively determined by the encoder and signaled in the bitstream.

3. More samples on either side of a segment are used in local activities calculation. More specifically, all the four lines of a segment are involved, and in each line, up to 8 samples on either side of the boundary are used.

4. While deblocking one line of a segment, up to 4 samples on either side are filtered.

5. The number of pre-defined filters increases to 15, with the filter length increased to 11-tap.

6. The derivation of BS values is simplified for inter slice. In an inter slice, the boundary of an intra block, which has BS equal to 2 in HEVC deblocking, has BS equal to 1 in this contribution, meaning in an inter slice, BS is always 1, if not 0. Thus, BS equal to 2 only happens in an intra slice.

7. BS values have an direct impact on filter selection. Four filters, which are a subset of the 15 pre-defined filters, can be used for deblocking BS2 segments. For BS1 segments, all the 15 filters can be used.

More details of this contribution are introduced in the following sub-sections.

## Segment Classification Based on Local Activity

Local activity means how smooth/complex the area near a block boundary is, and this contribution uses the 2nd derivative to measure it. As shown in Fig. 1, the absolute values of the 2nd derivative of all the samples in the dashed boxes are calculated and averaged. The results are denoted as PA and QA for Block P and Block Q, respectively.

Fig. 1 Samples involved in calculating the 2nd derivative

The samples closest to the block boundary (p0,j and q0,j) are not included in calculating PA or QA, because the 2nd derivatives of these samples are supported by samples across the boundary and may not precisely reflect the inner activity of either side.

Note the case that Block P or Block Q is not wide enough. For example, when the width of P or Q, denoted as WP and WQ, is 4 or 8, the remote sides of the dash boxes in Fig. 1 should not exceed WP/2 and WQ/2 for Block P and Block Q, respectively.

Two indices, levelave and leveldiff, representing the average and the difference of the local activities in the two adjacent blocks, respectively, are calculated using (1) and (2).

levelave = Clip3(0, 6, floor ( log2( (PA + QA) / 2 + 2) ) – 1)                                                 (1)

leveldiff = Clip3(0, 6, floor ( log2( |PA - QA| / 2 + 2) ) – 1)                                                  (2)

Since PA and QA are both non-negative, leveldiff cannot be greater than levelave. Therefore, there are totally 28 different combinations of (levelave, leveldiff), and each combination is a category. A segment is classified into one of the 28 categories, according to its (levelave, leveldiff).

## Four-Dimensional Filter Selection

Like HEVC deblocking, a segment is deblocked by filtering line by line. Here, we only describe how one line is filtered. Given a line to be filtered, min(4, WP/2) samples on the Block P side and min(4, WQ/2) samples on the Block Q side can be modified by filtering.

Unlike HEVC deblocking, which assumes the local activities of Block P and Block Q are similar and uses the same filter for pi and qi, this contribution considers the different local activities in the two adjacent blocks and applies filtering differently. The local activities PA and QA are first compared. If PA is smaller than QA, meaning Block P is smoother, the samples in Block P will have Type 0 filtering and the samples in Block Q will have Type 1 filtering. Vise versa. For each filtering type, the 4 samples (or 2 samples if the block width is 4) have their own filters, and therefore up to 8 different filters will be used to deblock one line.

To summarize, four arguments are used to select the right filter for a sample: (1) the category of the segment, which is two dimensional with levelave and leveldiff, (2) the filtering type, i.e., Type 0 or Type 1, and (3) the distance from the segment (i.e., the closest, the 2nd closest, etc.). The total number of all the possible combinations of the four arguments is 224 (28×2×4). A 224-entry 4-D LUT, determined, coded, and transmitted from the encoder, stores the filter indices for all the possible cases, and takes the four arguments as the indices for searching.

## Pre-Defined Filters and Filtering Process

The integer representation of the 15 pre-defined filters is shown in Table 1. For each filter, the highlighted tap is the filter center, which should be multiplied to the sample to be filtered. As can be seen, these pre-defined filters are designed to have more supporting samples close to and even across the segment than the supporting samples far away from the segment. These filters can be directly used for filtering the samples in Block P, but need to be flipped left right before used for filtering the samples in Block Q.

The identity filter is included with index 0, because no filtering is also one option for a certain sample. The segment with BS equal to 2 can only use the four filters with indices from 1 to 4 and the identity filter.

Since the filter gain is 512, the filtering results are right shifted 9 bits for normalization.

Note that the supporting samples located outside P and Q should not be used. When the filter taps reach beyond the remote boundary of P or Q, padded samples will be used to substitute the outside ones.

Table 1 Integer representation of the 15 pre-defined filters

| Identity filter (0) | 0 | 0 | 0 | 0 | 512 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | -4 | 13 | -26 | 31 | 481 | 40 | -33 | 18 | -10 | 2 | 0 |
| 2 | -21 | -14 | 6 | 264 | 130 | 56 | 138 | -22 | -18 | 7 | -14 |
| 3 | -11 | 24 | -185 | 456 | 95 | 106 | 63 | -37 | 1 | 3 | -3 |
| 4 | -2 | 12 | -94 | 208 | 293 | 119 | -9 | -17 | 1 | 6 | -5 |
| 5 | 9 | -46 | -6 | 235 | 108 | 314 | -154 | 105 | -87 | 55 | -21 |
| 6 | -12 | 14 | -3 | -17 | 547 | -29 | 18 | -5 | -2 | 4 | -3 |
| 7 | -13 | 31 | -69 | 132 | 305 | 225 | -164 | 120 | -80 | 35 | -10 |
| 8 | -18 | 46 | -142 | 318 | 149 | 222 | -67 | 8 | -4 | 7 | -7 |
| 9 | -64 | 96 | -55 | 188 | 327 | -119 | 106 | 67 | 20 | -50 | -4 |
| 10 | -10 | -41 | -16 | 348 | 34 | 157 | 102 | -29 | -52 | 45 | -26 |
| 11 | -46 | -40 | 128 | 315 | -66 | 48 | 207 | 49 | -115 | 87 | -55 |
| 12 | -32 | 18 | 63 | -39 | 365 | 255 | -208 | 130 | -24 | -28 | 12 |
| 13 | -3 | 4 | -7 | 3 | 492 | 58 | -71 | 72 | -54 | 27 | -9 |
| 14 | 17 | -57 | 90 | -105 | 631 | -110 | 73 | -29 | -1 | 14 | -11 |
| 15 | -40 | 124 | -99 | -25 | 685 | -268 | 185 | -57 | 13 | -33 | 27 |

## Clipping

In this contribution, two clipping values, tC,VER and tC,HOR, are signaled at the picture level, for vertical filtering and horizontal stages, respectively. In the first stage, when all the vertical boundaries are being filtered, tC,VER is used for clipping, as shown in (3),

p' = p + Clip3 (-tC,VER, tC,VER, Δp)                                                             (3)

where Δp is the change of the sample magnitude and p is the sample value before filtering.

In the second stage, when all the horizontal boundaries are being filtered, tC,HOR is used for clipping, as shown in (4).

p' = p + Clip3 (-tC,HOR, tC,HOR, Δp)                                                             (4)

## Parallelization

The proposed deblocking is applied to the boundaries of CUs, which may not be at least 8-sample apart. However, allowing the neighbouring boundaries to be closer does not lose the advantage of parallelization. In each stage (e.g., horizontal or vertical), the samples involved in deblocking one boundary do not overlap with the samples involved in deblocking any other boundaries.

First, the to-be-filtered samples of one segment do not overlap with the to-be-filtered samples of any other segments, because the number of samples to be filtered is min(4, WP/2) for Block P and min(4, WQ/2) for Block Q.

Second, the supporting samples for filtering are all obtained from the output of the previous stage, i.e., in Stage 1, they are from the reconstructed samples, and in Stage 2, they are form the output samples of Stage 1. The supporting samples are never from the filtered samples in the current stage.

Third, the samples used for making deblock-related decisions, such as local activity calculation and classification, are all from the output of the previous stage.

# Experimental Results

The proposed deblocking scheme is integrated into JEM7.0 and tested under the common test condition, except that all the advanced coding tools but QTBT are turned off. Therefore, the performance of the anchor is close to HM+QTBT.

# Conclusion

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).