| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0059-v1 |
| --- | --- |

| Title: | Enhanced Merge Mode based on JEM7.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An Na Zhang Xu Chen Jianhua Zheng | Tel: Email: | +86 13439745728 anjicheng@hisilicon.com zhengjianhua@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution presents an enhanced merge mode based on JEM7.0. Firstly, the extended spatial merge candidates are added into the merge candidates list, followed by more MV offsets added to the first merge candidate, then the combined average merge candidates are generated to replace the original combined bi-predictive candidates, followed by a template matching based adaptive reorder method to finalize the merge candidates list. Finally, a dual merge mode is proposed to allow each reference list of one merge candidate to use two sets of motion information. The proposed technologies can provide 1.7%, 1.0%, 0.77% gain for RA, LB, LP respectively compared to JEM7.0 anchor with around 13% encoding time increase.

# Introduction

In current JEM7.0, the set of possible candidates in the merge mode consists of spatial neighbour candidates, ATMVP candidates, a temporal candidate, and generated candidates. Figure 1 shows the positions of five spatial candidates. For each candidate position, the availability is checked according to the numerical order {1, 2, 3, 4, 5}.

Figure 1. Positions of spatial candidates in HEVC/JEM.

# Proposed improvements

## Extended Spatial Merge Candidates

In this proposed method, more spatial positions are checked as shown in Figure 2. The extended spatial positions from 6 to 27 are checked according to their numerical order after the temporal candidate. In order to save the MV line buffer, all the spatial candidates are restricted within two CTU lines. That is, the spatial candidates beyond the CTU line above the current CTU line are excluded.

Figure 2. Positions of extended spatial candidates in the proposed method.

The number of candidates within the merge candidate list is controlled by NumMrgCands. In HEVC, NumMrgCands=5. In JEM, the NumMrgCands is added by 2 when ATMVP is turned on. In this proposed method, the NumMrgCands is added by 6, so the NumMrgCands equals to 11 when ATMVP is off and 13 when ATMVP is on.

The whole process of adding candidates will stop as soon as the number of candidates reaches NumMrgCands during the merge candidate list construction process. The redundancy check is done for all the merge candidates except the generated ones. That is, only unique motion candidate from spatial positions, ATMVP, and temporal candidate, can be included the merge candidate list. The merge candidate list is constructed as follows:

- Add the spatial positions from position 1 to 5, ATMVP candidates, and temporal candidate, which is the same as in current JEM; (include redundancy check)

- Add the extended spatial positions from position 6 to 27; (include redundancy check)

## Merge offset extension

In current JEM, the set of possible candidates in the merge mode consists of spatial neighbour candidates, ATMVP candidates, a temporal candidate, and generated candidates.

In Merge offset extension, more candidates are checked based on existed candidate. As Figure 3 shows, new candidates has mv offset to the mv of existed candidate (except ATMVP candidate).

Figure 3.  New MV candidate

The number of candidates within the merge candidate list is controlled by NumMrgCands. In HEVC, NumMrgCands=5. In JEM, the NumMrgCands is added by 2 when ATMVP is turned on. For extended spatial merge candidates, the NumMrgCands is added by 6, so the NumMrgCands equals to 11 when ATMVP is off and 13 when ATMVP is on. In this proposed method, four new MV candidates with one luma sample offset, so the NumMrgCands is added by 4. The NumMrgCands equals to 15 when ATMVP is off and 17 when ATMVP is on (set Extended spatial merge candidates on). The NumMrgCands equals to 9 when ATMVP is off and 11 when ATMVP is on (set Extended spatial merge candidates off).

If one of new MV candidates (one luma sample offset) is selected, another eight new MV candidates with half luma sample offset be candidates in the list and one more index need to be transferred for higher precision.

Figure 4 New merge candidate list construction

As Figure 4 shows, new extended mv offset candidates are constructed only based on the first candidate of merge candidate list. New extended mv offset candidates only has mv offset to the current mv of first candidate and other prediction information is same to first candidate.

In the proposed method, twelve new MV candidates include four MV' surrounding candidates with one luma sample offset to the first candidate MV in either the horizontal or vertical direction, eight MV' surrounding candidates with half luma sample offset to the first candidate MV in either the horizontal or vertical direction, or both. Finally, the best refined MV' are used for generating the final prediction results. For bi-prediction with one MV from a reference picture in the past and another from a reference picture in the future case, add the mv offset to first candidate MV0, and add the reverse mv offset to first candidate MV1. Other bi-prediction cases add same mv offset to first candidate MV0 and MV1 respectively.

## Combined Average Merge candidates

Combined bi-predictive merge candidates in current JEM are created by combining original merge candidates. They are added following original merge candidates to decrease coding loss by using fixed list size. However, it is invalid for the P slice. Combined bi-predictive merge candidates are replaced by the proposed combined average merge candidates in random access configuration and low delay P configuration for the higher coding efficiency.

Combined average merge candidates are generated as follows. The motion information of the original merge candidates for each list is scaled to the derived reference picture for a given list. The scaled motion information is stored in a new array. The derived reference picture for a given list is the reference picture with the highest utilization rate among the reference pictures of the original merge candidates in that list. After scaling the motion information, all available motion vectors (up to 2) of the combined merge candidates are averaged separately for each reference list. The averaged motion vector is assigned as the motion vector of the combined average merge candidate. The derived reference picture for a given list is the reference picture of all the combined average merge candidates in that list.

In Figure 5, three combined average merge candidates are added into the merge candidate list.

| Merge_idx | L0 | L1 |
| --- | --- | --- |
| 0 | mvL0_A, ref0 | mvL1_A, ref1 |
| 1 | mvL0_B, ref0 | mvL1_B, ref1 |
| 2 | - | mvL1_C, ref1 |

(a)Original merge candidate list

| Merge_idx | L0 | L1 |
| --- | --- | --- |
| 0 | mvL0_A, ref0 | mvL1_A, ref1 |
| 1 | mvL0_B, ref0 | mvL1_B, ref1 |
| 2 | - | mvL1_C, ref1 |
| 3 | ( mvL0_A + mvL0_B)/2, ref0 | ( mvL1_A + mvL1_B)/2, ref1 |
| 4 | mvL0_A, ref0 | ( mvL1_A + mvL1_C)/2, ref1 |
| 5 | mvL0_B, ref0 | ( mvL1_B + mvL1_C)/2, ref1 |

(b)Merge Candidate List after adding combined average candidates

Figure 5．An example for combined average merge candidates

The combined average merge candidates have six combinations at most if the number of available original merge candidates is 4. The six combinations are described as follows:

uiPriority0[6] = {0 , 0,  1,  0,  1,  2};

uiPriority1[6] = {1 , 2,  2,  3,  3,  3};

## Template matching based adaptive merge candidates list

To improve the coding efficiency, after the merge candidate list is constructed, the order of each merge candidate is adjusted according to the template matching cost. The merge candidates are arranged in the list in accordance with the template matching cost of ascending order. It is operated in the form of sub-group.

The template matching cost is measured by the SAD (Sum of absolute differences) between the neighbouring samples of the current CU and their corresponding reference samples.  If a merge candidate includes bi-predictive motion information, the corresponding reference samples are the average of the corresponding reference samples in reference list0 and the corresponding reference samples in reference list1, as illustrated in Figure 6.  If a merge candidate includes sub-CU level motion information, the corresponding reference samples consist of the neighbouring samples of the corresponding reference sub-blocks, as illustrated in Figure 7.

The sorting process is operated in the form of sub-group, as illustrated in Figure 8. The first three merge candidates are sorted together. The following three merge candidates are sorted together.

The template size (width of the left template or height of the above template) is 1. The sub-group size is 3.

Figure 6. Neighbouring samples used for calculating SAD

Figure 7. Neighbouring samples used for calculating SAD for sub-CU level motion information

Figure 8. The sorting process

## Dual merge mode

It may not be effective that each reference list has only one set of motion information. The dual merge mode allows each reference list to use two sets of motion information. It is determined by the SATD criterion whether each reference list uses two sets of motion information or a set of motion information and indicated with a flag. The new set motion information of each reference list is obtained as follows:

RefX’= (RefX == 0) ? 1 : 0

MvX’ = dX’ * MVX /dX

As shown in Figure 9, the X represents the reference list 0 or the reference list 1. RefX represents the reference index of the original motion information of the reference list X. RefX' represents the reference index of the new set of motion information in the reference list X. Similarly, MVX represents the motion vector of the original motion information of the reference list X. MVX' represents the motion vector of the new set of motion information in the reference list X. dX represents the temporal distance between the current picture and the reference picture specified by the RefX. dX’ represents the temporal distance between the current picture and the reference picture specified by the RefX’.

Figure 9. The sketch map of dual merge mode

Original prediction value is

PredUni = PredX

PredBi=(Pred0+Pred1) >>1

Modified prediction value is

PredUni = (PredX+ PredX’) >>1

PredBi=( (Pred0+Pred1) >>1+ (Pred0’+Pred1’) >>1 ) >>1

where PredUni represents the prediction value of uni-prediction, PredBi represents the prediction value of bi-prediction, PredX represents the prediction value compensated by the original motion information of the reference list X, and PredX’ represents the prediction value compensated by the new set of motion information in the reference list X.

For the merge mode, the original motion information can be the motion information of each candidate.

A flag under merge mode is signalled to indicate whether this dual merge mode is used.

This scheme is used only in low delay P configuration.

# Experimental Results

All of the above improvements have been implemented based on JEM7.0. In the experiments, anchor is JEM7.0, and the test is JEM7.0+all of the above proposed technologies. Two kinds of tests have been done: tools off test and tools on test. For the tools off test, both anchor and test use the following command line parameters to disable the JEM tools:

--EMT=0 --NSST=0 --PDPC=0 --FRUC=0 --BIO=0 --OBMC=0 --IlluCompEnable=0 --AFFINE=0 --ATMVP=0 --IMV=0 --DMVR=0 --BilateralFilter=0 --ALF=0

For tools on test, both anchor and test use the default setting, i.e., no additional command line parameters.

The test sequences are those used in call for proposals [1]. The QP values used in the RA and LD test cases are the base QP used in CFP JEM anchor.

Table 1. Results of tools on test

Table 2. Results of tools off test

# Reference

[1] A. Segal, V. Baroncini, J. Boyce, J. Chen, and T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC”, Macao, CN, Oct., 2017.

# Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).