| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0061-v2 |
| --- | --- |

| Title: | Planar Motion Vector Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang Jicheng An Jianhua Zheng | Tel: Email: | +86 18601967458 zhangna25@huawei.com zhengjianhua@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd |  |  |

_____________________________

# Abstract

To generate a smooth fine granularity motion field, this contribution presents a planar motion vector prediction method based on JEM7.0. Planar motion vector prediction is achieved by averaging a horizontal and vertical linear interpolation on 4x4 block basis. The proposed technology can provide 0.16%, 0.33%, 0.34% BD bitrate savings for RA, LB, LP, respectively compared to JEM7.0 anchor with around 7% encoding time increase.

# Introduction

Merge mode derives the motion information from spatially or temporally neighboring blocks. It forms a merged region sharing all motion information.

It may not be effective that each block uses one set of motion information.

# Proposed Method

To generate a smooth fine granularity motion field, Figure 1 gives a brief description of the planar motion vector prediction process.

Figure 1. Planar motion vector prediction process

Planar motion vector prediction is achieved by averaging a horizontal and vertical linear interpolation on 4x4 block basis as follows.

W and H denote the width and the height of the block. (x,y) is the coordinates of current sub-block relative to the above left corner sub-block. All the distances are denoted by the pixel distances divided by 4.is the motion vector of current sub-block.

The horizontal prediction  and the vertical prediction  for location (x,y) are calculated as follows:

where  and  are the motion vectors of the 4x4 blocks to the left and right of the current block.  and  are the motion vectors of the 4x4 blocks to the above and bottom of the current block.

The reference motion information of the left column and above row neighbour blocks are derived from the spatial neighbour blocks of current block.

The reference motion information of the right column and bottom row neighbour blocks are derived as follows.

- Derive the motion information of the bottom right temporal neighbour 4x4 block

- Compute the motion vectors of the right column neighbour 4x4 blocks, using the derived motion information of the bottom right neighbour 4x4 block along with the motion information of the above right neighbour 4x4 block, as described in Equation 1.

- Compute the motion vectors of the bottom row neighbour 4x4 blocks, using the derived motion information of the bottom right neighbour 4x4 block along with the motion information of the bottom left neighbour 4x4 block, as described in Equation 2.

|  | R(W,y) = ((H-y-1)AR + (y+1)BR)/H | Equation 1 |
| --- | --- | --- |
|  | B(x,H) = ((W-x-1)BL+ (x+1)BR)/W | Equation 2 |

where AR is the motion vector of the above right spatial neighbour 4x4 block, BR is the motion vector of the bottom right temporal neighbour 4x4 block, and BL is the motion vector of the bottom left spatial neighbour 4x4 block.

The motion information obtained from the neighbouring blocks for each list is scaled to the first reference picture for a given list.

# Experimental Results

The proposed method has been implemented based on JEM7.0. In the experiments, anchor is JEM7.0, and the test is JEM7.0+ the proposed method. Two kinds of tests have been done: tools off test and tools on test. For the tools off test, both anchor and test use the following command line parameters to disable the JEM tools:

--EMT=0 --NSST=0 --PDPC=0 --FRUC=0 --BIO=0 --OBMC=0 --IlluCompEnable=0 --AFFINE=0 --ATMVP=0 --IMV=0 --DMVR=0 --BilateralFilter=0 --ALF=0

For the tools on test, both anchor and test use the default setting, i.e., no additional command line parameters.

The test sequences are those used in call for proposals [1]. The QP values used in the RA and LD test cases are the base QP used in CFP JEM anchor.

Table 1. Results of tools on test

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 AllToolsOn |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class UHD1 | -0.19% | -0.17% | -0.27% | 107% | 105% |
| Class HD1 | -0.14% | -0.24% | -0.15% | 107% | 105% |
| Overall (Ref) | -0.16% | -0.21% | -0.21% | 107% | 105% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over JEM7.0 AllToolsOn |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class UHD1 |  |  |  |  |  |
| Class HD1 | -0.33% | 0.37% | -0.85% | 106% | 110% |
| Overall (Ref) | -0.33% | 0.37% | -0.85% | 106% | 110% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over JEM7.0 AllToolsOn |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class UHD1 |  |  |  |  |  |
| Class HD1 | -0.34% | -0.38% | -0.47% | 107% | 110% |
| Overall (Ref) | -0.34% | -0.38% | -0.47% | 107% | 110% |

Table 2. Results of tools off test

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 ToolsOff |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class UHD1 | -2.22% | -2.22% | -2.24% | 105% | 150% |
| Class HD1 | -1.68% | -1.65% | -1.91% | 106% | 134% |
| Overall (Ref) | -1.95% | -1.93% | -2.07% | 105% | 142% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over JEM7.0 ToolsOff |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class UHD1 |  |  |  |  |  |
| Class HD1 | -1.67% | -1.19% | -1.34% | 109% | 122% |
| Overall (Ref) | -1.67% | -1.19% | -1.34% | 109% | 122% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over JEM7.0 ToolsOff |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class UHD1 |  |  |  |  |  |
| Class HD1 | -2.27% | -1.73% | -2.26% | 109% | 114% |
| Overall (Ref) | -2.27% | -1.73% | -2.26% | 109% | 114% |

# Reference

[1] A. Segal, V. Baroncini, J. Boyce, J. Chen, and T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC”, Macao, CN, Oct., 2017.

# Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).