| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, USA, 10–21 Apr. 2018 | Document: JVET-J0063 |
| --- | --- |

| Title: | Symmetrical mode for bi-prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution provides a symmetrical mode for motion information coding in bi-prediction. In this mode, only motion information for list 0, and mvp index for list 1 are explicitly signaled, and the reference index and MVD for the list 1 is derived based on the assumption of linear motion. Simulation results state that 0.93% BD-rate saving can be achieved for RA configuration with 9% encoding time increase.

Introduction

In previous standards, when bi-prediction is applied, 2 sets of motion information should be signaled in the bit-streams. It is not an efficient way for motion information coding in bi-prediction. In this contribution, a symmetrical mode is proposed to reduce the bit-rate of motion information coding.

Proposal method

In this proposal, a symmetrical mode flag indicating whether symmetrical mode is used or not is explicitly signaled if the prediction direction is bi-prediction. When the flag is true, only mvp index for list 1 is signaled, and the reference index and MVD for list 1 are derived from motion information of list 0.

As shown in Fig.1, firstly, the ref_idx_l1 is decided by searching a reference picture in list 1, which is mirrored with the reference picture for list 0. If not exist such picture, the nearest reference picture to current picture is used. After the reference picture in list 1 is decided, MVD1 is derived by scaling MVD0. The scaling factor is calculated by following formula:

MVD1=POCCUR-POClist1POCCUR-POClist0×MVD0                         (1)

Fig. 1 Illustration for symmetrical MVD

Experimental results

The proposed method is implemented on top of the CfP response software by Huawei and Samsung. The QP settings for HM CfP anchor are used for both anchor and test. Results for tool-on test are shown in Table 1, which show that 0.93% BD-rate saving can be achieved for tool-on RA configuration.

Table 1: Tool on test results in RA configuration

| Tool On | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| FoodMarket4 | -1.07% | -0.80% | -0.87% | 109% | 99% |
| CatRobot1 | -1.09% | -1.17% | -1.26% | 108% | 99% |
| DaylightRoad2 | -2.05% | -1.72% | -1.69% | 108% | 100% |
| ParkRunning3 | -1.19% | -1.12% | -0.97% | 109% | 99% |
| Campfire | -0.14% | -0.07% | -0.17% | 109% | 99% |
| BQTerrace | -0.36% | -0.32% | -0.37% | 110% | 100% |
| RitualDance | -0.51% | -0.43% | -0.43% | 109% | 100% |
| MarketPlace | -1.02% | -0.76% | -0.75% | 109% | 100% |
| BasketballDrive | -0.81% | -0.50% | -0.70% | 108% | 100% |
| Cactus | -1.10% | -1.03% | -0.90% | 109% | 100% |
| Class A1 | -1.11% | -0.97% | -0.99% | 109% | 99% |
| Class A2 | -0.76% | -0.61% | -0.63% | 109% | 100% |
| Overall(A1+A2) | -0.93% | -0.79% | -0.81% | 109% | 100% |

Conclusions

A symmetrical mode for bi-prediction is proposed to reduce the bit-rate of motion information for bi-prediction. Simulation results report 0.93% BD-rate saving in luma component for RA configuration.

Patent rights declaration(s)

Huawei Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).