| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0067r1 |
| --- | --- |

| Title: | Additional information on HDR video coding technology proposal by Qualcomm and Technicolor |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | D. Rusanovskyy, A.K. Ramasubramonian M.Karczewicz (Qualcomm) E. François (Technicolor) | Tel: Email: | dmytror@qti.qualcomm.com Edouard.Francois@technicolor.com |
| Source: | Qualcomm, Technicolor |  |  |

_____________________________

# Abstract

This contribution provides additional information on the HDR video coding technology proposals by Qualcomm and Technicolor presented in JVET-J0021 and JVET-J0022. The proposed HDR technology is Color Volume Transform (CVT) which is applied in the Y’CbCr 4:2:0 sample domain. The CVT is implemented through Dynamic Range Adjustment (DRA) process which is applied as pre-processing at the encoder side, with the aim of improving the coding efficiency. At the decoder side, the inverse DRA process is applied.

Simulation results reported in this document show that the proposed CVT implemented on top of JEM7.0 software and tested on Class HDR-B test sequences provides around 34.0% and 8.3% of bitrate reduction (for PSNR-L100 metrics) against HM and JEM HDR anchors of the CfP, respectively.

As it is shown in J0021, proposed CVT being integrated in the core technology of J0021 (High Complexity mode), provides for class HDR-B on average 41.3% and 18.8% BD-rate gain (PSNR-L100) against HM and JEM HDR anchors, respectively. In Low Complexity Mode, proposed CVT provides for class HDR-B on average 38.8% and 15.2% BD-rate gain (PSNR-L100).

Additionally, this document reports, that for for HDR-B class sequences proposed CVT utilized in the JVET-J0021 core design provides 14% of bitrate reduction (for PSNR-L100 metric) over the default (SDR) coding configuration in the High Complexity mode and 13.6% of bitrate reduction over the SDR configuration in the Low Complexity mode.

# Introduction

To allow evaluation of HDR tools without interfering with the generic video coding tools, proposed by Qualcomm and Technicolor technology in the HDR category was implemented as an add-on to the core video coding system. The technology was implemented on top of High, Medium and Low complexity submissions from Qualcomm and Technicolor which are described in JVET documents J0021 and J0022. This document reports implementation of the proposed HDR technology on the top JEM7.0 software. Also additional results are reported for proposed HDR technolog implemented in the JVET-J0021 core design.

The proposed HDR technology is Color Volume Transform (CVT) which is applied in the Y’CbCr 4:2:0 sample domain. The CVT is performed on the input video samples prior to coding and the inverse process is performed on the samples after decoding. The CVT modifies luma and chroma samples in the picture based on the value of the luma or chroma samples and on the value of other component samples located at the same or closest relative location. A single CVT was used for coding of all video sequences of the CfP; parameters of the CVT in the submitted response were configured to be static for a sequence across of all rate points. The response was configured to utilize no multi-pass processing and no dynamic change in parameters of CVT within a rate-point.

The proposed CVT consists of the following major components:

- Dynamic Range Adjustment (DRA)

- Cross-Component DRA (CC-DRA)

- DRA and scalar quantizer harmonization

The response, like HDR anchors of the CfP, also employs different encoder-side configurations to address the utilized HDR formats (HLG/PQ) and the color gamut of the sequences.

# Technology Description

## HDR Video Codec design

Block diagram of the response in the HDR category is shown in Figure 1. At the encoder, the CVT is applied to the input Y’CbCr 4:2:0 picture (10 bits per sample) which results in an intermediate picture (stored at 4:2:0 10 bits per sample); this intermediate picture is encoded. Parameters of the inverse CVT are encapsulated in the coded bitstream and used at the decoder side to configure the inverse CVT which is applied to the decoded picture (stored in 4:2:0 chroma format, at 10 bits per sample).

Figure 1: Block diagram of the response in HDR category.

## Color Volume Transform

Dynamic Range Adjustment (DRA) is a main component of the proposed CVT and it was originally described in VCEG document T13-SG16-C-1027 and in the MPEG document m37064. The main principle of the DRA is range-adaptive scaling of luma and chroma samples; scaling which is applied at the pre-processing stage and its inverse is applied at the decoder side as post-processing. Inverse DRA operations at post-processing are described as:

XY=xY*SYi+OYi				( 1 )

XCb=xCb*SCbj+OCbj				( 2 )

XCr=xCr*SCrk+OCrk				( 3 )

where the terms S and O define the piece-wise linear function of the DRA for a particular dynamic range partition of color component, x is a decoded sample (with xY, xCb and xCr being the Y, Cb and Cr sample), X is the post-processed sample (with XY, XCb and XCr being the Y, Cb and Cr components of the post-processed sample) and i, j and k are indices of the  dynamic range partition to which the Y, Cb and Cr components, respectively, of sample x belong.

Parameters of the DRA are derived from a piece-wise linear function defined through dynamical range partitions (called ranges) and scale values which are applied to all samples belonging to a particular range partition. Independent DRA curve is defined for each color component.

Cross-Component DRA (CC-DRA) is carried to preserve luma/chroma bitrate allocation for different brightness ranges following the application of the DRA on input chroma samples. At the decoder side, inverse CC-DRA precedes the inverse DRA process. In this approach, for each decoded chroma sample xCx, the chroma scaling component S2cxY is derived from luma inverse DRA scales SYi of the associated luma sample:

S2cxY=funSYxY					( 4 )

The CC-DRA scales S2cxY is obtained by linear interpolation of luma DRA scales of adjacent intervals, e.g. SYi and SYi+1. Figure 2 shows an example of such interpolation.

The derived CC-DRA scales are applied to zero-mean chroma samples xCb and xCr based on the luma values xY associated with these chroma samples.

X2Cx=xCx-2bitDepth-1*S2cxY*S3xQP+2bitDepth-1		( 5 )

Figure 2: CC-DRA scaling function for chroma components (red curve) derived from luma DRA scales (blue curve)

DRA scales and scalar quantizer harmonization is conducted to preserve luma/chroma bit allocation assumed by the core coding design for a given QP setting. Example of such bit allocation mechanism is a chroma QP, Qpcx, dependency on luma QP, QpY, through Table 8-10 of HEVC, effect of which on chroma QP is illustrated in Figure 3:

qPi=function(QpY, chromaQpOffsetBitstream)  	                      ( 6 )

Qpcx=qPi+chromaQpTable

is qPi is the  QP index derived from luma QP value QpY, chromaQpOffsetBitstream is the cumulative chroma QP offset signalled in the bitstream and chromaQpTable is an offset introduced by Table 8-10.

As shown in Figure 3, QpY value (QpY = 36) that is applied to quantize/dequantize luma component would produce a chroma QP offset chromaQpTable= − 2. DRA scale SYi which is applied to luma component at the pixel domain will result in deviation from QpY which is not carried through QP signaling, thus would not be reflected in the value of chromaQPTable.

Low Luma Value (Dark)

High Luma Value (Bright)

QpY

Figure 3: Internal chroma QP offset introduced by Table 8-10 of HEVC

To correct this luma/chroma bit re-allocation, CC-DRA scale S2cxY is adjusted by a multiplication factor S3cxQP:.

X2Cx=XCx-2bitDepth-1*S2cxY*S3xQP+2bitDepth-1			( 7 )

Let QPY be a QP with which used to code luma component sample; the chroma DRA associated with this luma sample correction term S3xQP is derived as follows:

QP1=QPy + cQPOffset1

QP2=QPy+ cQPOffset1 + cQPOffset2Lin QPy+ scale2QP SYxY

qpShift1=QP1-TableQPLinQP1

qpShift2=QP2-TableQPLinQP2 )

S3xQP=qp2ScaleqpShift2-qpShift1			( 8 )

where cQPOffset1 is the fixed chroma QP offset which is signalled in the bitstream, cQPOffset2( ) is the chroma QP offset derived as color gamut of HDR content, and TableQPLin() accounts for chroma QP offset in Table 8-10  and chQPOffset2Lin () describes the chroma QP offset term based on the gamut of the video, and functions qp2Scale( x ) and scale2QP( x ) describe function to convert the QP parameter to the scale parameter  and vice versa.

# CVT Configurations:

Identical CVT was utilized for coding of all video sequences and rate points of the CfP response. However, parameters of the CVT have been adjusted to addressed properties of HDR video content represented with HLG and PQ transfer functions and diversity of properties of the HDR content in PQ representation.

Similar to HDR anchors encoder configurations, three different CVT configurations have been used in submitted response for HLG, PQ/P3 and PQ/BT709 and video sequences.

Table 1 describes DRA scales which were utilized in the CfP response submissions described in JVET document J0021, Y value row specifies the lower boundary of dynamic range partitions, and scales raw specify scale value applicable for each dynamic range partition. Shown scale values have been renormalized to a single set of dynamic ranges. Luma DRA function for PQ/BT.709 test sequences have been derived from the dQP table used for the HM and JEM HDR anchors. Luma DRA functions for HLG and PQ/P3 sequences have been derived to maximized objective gain in HDR metrics (L100 and deltaE). These derivations were also based on some assumption on HDR grading process involved during production of the HDR content.

Parameters of DRA function for chroma components have been derived from chroma QP offset configurations utilized by the HDR CfP anchors. Parameters of these configurations for tested sequences are shown in Table 2.

Table 1: DRA functions for Luma component (defined as scale per range):

| Y value | 0 | 301 | 367 | 434 | 501 | 567 | 634 | 701 | 767 | 834 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HLG scales | 0.8738 | 0.8634 | 0.8636 | 0.8636 | 0.8634 | 0.8636 | 1.0415 | 1.1091 | 1.1986 | 1.3143 |
| PQ/709 scales | 0.5835 | 0.6475 | 0.7266 | 0.8156 | 0.9156 | 1.0273 | 1.1535 | 1.2940 | 1.4531 | 1.6471 |
| PQ/P3 scales | 0.5051 | 0.8527 | 0.8705 | 0.9726 | 1.0985 | 1.1366 | 1.2324 | 1.2688 | 1.3713 | 1.5036 |

Table 2: DRA function parameters for Chroma component:

|  | WCGPPSCbQpScale | WCGPPSCrQpScale | WCGPPSChromaQpScale | WCGPPSChromaQpOffset |
| --- | --- | --- | --- | --- |
| HLG | 1.0 | 1.0 | 1.0 | 0.0 |
| PQ/709 | 1.14 | 1.79 | -0.46 | 9.26 |
| PQ/P3 | 1.04 | 1.39 | -0.46 | 9.26 |

Parameters of the CVT in all submitted responses described in JVET documents J0021 and J0022 were configured to be static for a sequence across of all rate points. No multi-pass processing and no dynamic change in parameters within a sequence of CVT were utilized.

# Simulation results: JEM7.0 code base

Complete simulation results produced under the CfP conditions and for all quality metrics are provided in attached Excel document. Note, that reported results in this contribution are not rate matched to CfP target bitrates, since provided for information purpose only.

However, parameters of the CVT for this contribution were configured to maximize video coding performance in HDR metrics (PSNR L100 and DeltaE) as they were found well-correlated with visual quality during our experiments. Moreover, special effort was put to assess “aggregate gain”, thus redistributing observed chroma gain to the luma component. Therefore, this and other sections of this document summarizes results by quoting gain for PSNR-L100 HDR metric.

Comparative performance of the proposed technology against CfP Anchors measured for class HDR-B under the CfP test conditions are reported in Table 3, Table 4, Table 5 and Table 6. Being integrated in the JEM7.0, proposed technology provides 34% of dBR reduction or 0.8 dB of BD-PSNR gain in PSNR-L100 metric on average comparing to HM HDR Anchor.

In Table 5 and Table 6 comparative performance against the JEM HDR Anchor is reported. Proposed technology provides in 8.3% of dBR reduction or 0.15 dB of BD-PSNR gain in PSNR L100 metric on average comparing to JEM HDR Anchor.

Table 3: Comparative performance in terms of BD-Rate [%] against HM HDR Anchor:

|  | Constraint Set 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM HDR Anchor |  |  |  |  |  |  |  |
|  | BD-rate, % |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | -38.7% | -29.5% | -28.3% | -31.9% | -38.2% | -22.0% | -11.8% | -2.5% |
| ShowGirl2 | -31.8% | -29.6% | -29.2% | -47.0% | 15.2% | -24.7% | -71.1% | 9.4% |
| Hurdles | -51.0% | -39.0% | -36.8% | -66.3% | -61.7% | -32.6% | -57.4% | -49.9% |
| Starting | -34.8% | -36.3% | -33.6% | -20.5% | -34.5% | -29.8% | -9.3% | -0.3% |
| Cosmos1 | -30.7% | -35.5% | -18.1% | 46.9% | -19.2% | -10.6% | 53.9% | 26.3% |
| Average HDR-B | -37.4% | -34.0% | -29.2% | -23.8% | -27.7% | -23.9% | -19.1% | -3.4% |

Table 4: Comparative performance in terms of BD-PSNR [dB] against HM HDR Anchor:

|  | Constraint Set 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM HDR Anchor |  |  |  |  |  |  |  |
|  | BD-PSNR, dB |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | 0.46 | 0.63 | 1.33 | 0.46 | 0.48 | 0.98 | 0.15 | 0.16 |
| ShowGirl2 | 0.29 | 0.71 | 1.42 | 0.75 | 0.19 | 1.09 | 0.59 | 0.07 |
| Hurdles | 0.69 | 0.80 | 1.73 | 1.63 | 1.63 | 1.51 | 1.22 | 1.18 |
| Starting | 0.42 | 1.13 | 2.35 | 0.39 | 0.50 | 1.94 | 0.13 | 0.06 |
| Cosmos1 | 0.34 | 0.56 | 0.49 | -0.35 | -0.15 | 0.26 | -0.47 | -0.28 |
| Average HDR-B | 0.44 | 0.77 | 1.46 | 0.58 | 0.53 | 1.16 | 0.32 | 0.24 |

Table 5: Comparative performance in terms of BD-Rate [%] against JEM HDR Anchor:

|  | Constraint Set 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM HDR Anchor |  |  |  |  |  |  |  |
|  | BD-rate, % |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | 1.9% | -6.9% | -4.6% | 26.9% | 29.7% | 1.9% | 48.0% | 45.4% |
| ShowGirl2 | -3.0% | -4.1% | -2.1% | 59.2% | 86.1% | 1.7% | -38.2% | 72.0% |
| Hurdles | -0.8% | -7.1% | -5.1% | -11.4% | 9.2% | -2.7% | -2.2% | 19.4% |
| Starting | -1.8% | -8.2% | -5.1% | 25.9% | 2.8% | -1.8% | 40.8% | 34.8% |
| Cosmos1 | -0.2% | -15.2% | 8.1% | 100.0% | 100.0% | 16.0% | 100.0% | 100.0% |
| Average HDR-B | -0.8% | -8.3% | -1.7% | 40.1% | 45.5% | 3.0% | 29.7% | 54.3% |

Table 6: Comparative performance in terms of BD-PSNR [dB] against JEM HDR Anchor:

|  | Constraint Set 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM HDR Anchor |  |  |  |  |  |  |  |
|  | BD-PSNR, dB |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | -0.02 | 0.13 | 0.19 | -0.28 | -0.19 | -0.07 | -0.44 | -0.34 |
| ShowGirl2 | 0.01 | 0.08 | 0.08 | -0.26 | -0.13 | -0.06 | -0.27 | -0.21 |
| Hurdles | 0.01 | 0.11 | 0.19 | 0.12 | -0.10 | 0.10 | -0.04 | -0.27 |
| Starting | 0.00 | 0.20 | 0.28 | -0.42 | -0.06 | 0.09 | -0.55 | -0.27 |
| Cosmos1 | 0.00 | 0.21 | -0.19 | -1.32 | -1.07 | -0.34 | -1.44 | -1.22 |
| Average HDR-B | 0.00 | 0.15 | 0.11 | -0.43 | -0.31 | -0.06 | -0.55 | -0.46 |

Figures 4-6 show rate-distortion curves (a. – PSNR L100 and b. – DeltaE) for the proposed HDR scheme and two CfP anchors.

a.								b.

Figure 4: Rate-distortion curves for proposed HDR scheme and 2 CfP anchor, Hurdles test sequence

a.								b.

Figure 5: Rate-distortion curves for proposed HDR scheme and 2 CfP anchor, Cosmos test sequence

a.								b.

Figure 6: Rate-distortion curves for proposed HDR scheme and 2 CfP anchor, Market test sequence

# Simulation results: JVET-J0021 code base

Additionally, proposed HDR technology integrated in the JVET-J0021 coding core was tested against the default coding configuration (SDR) of this core technology.

As it is shown in J0021, proposed CVT being integrated in the core technology of J0021 (High Complexity mode), provides for class HDR-B on average 41.3% and 18.8% BD-rate gain (PSNR-L100) against HM and JEM HDR anchors, respectively. In Low Complexity Mode, proposed CVT provides for class HDR-B on average 38.8% and 15.2% BD-rate gain (PSNR-L100)

Table 7 and 8 show comparative results (BD-rate, % and BD-PSNR, dB respectively) produced with High Complexity mode for Class HDR-B test sequences. It is reported that proposed HDR technology provides in 14% of dBR reduction or 0.24 dB of BD-PSNR gain in PSNR L100 metric on average comparing to SDR configuration.

Table 9 and 10 show comparative results (BD-rate, % and BD-PSNR, dB respectively) produced with Low Complexity mode for Class HDR-B test sequences. It is reported that proposed HDR technology provides in 13.6% of dBR reduction or 0.22 dB of BD-PSNR gain in PSNR L100 metric on average comparing to SDR configuration.

Table 7: Comparative performance in terms of BD-Rate [%] against SDR configuration of JVET-0021 (High Complexity mode):

|  | Constraint Set 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over SDR configuration of JVET0021 |  |  |  |  |  |  |  |
|  | BD-rate, % |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | -46.6% | -18.9% | -13.3% | -71.1% | -100.0% | 10.3% | -54.4% | -70.6% |
| ShowGirl2 | -15.8% | -6.5% | 0.8% | -6.1% | -15.5% | 19.6% | 30.4% | 195.8% |
| Hurdles | -63.6% | -9.0% | -3.6% | -66.6% | -100.0% | 6.1% | -61.7% | -100.0% |
| Starting | -40.2% | -12.3% | -2.5% | -33.9% | -43.5% | 9.6% | -23.4% | -17.6% |
| Cosmos1 | -19.0% | -23.5% | 14.3% | 100.0% | -82.2% | 32.0% | 100.0% | 86.7% |
| Average HDR-B | -37.0% | -14.0% | -0.9% | -15.5% | -68.2% | 15.5% | -1.8% | 18.9% |

Table 8: Comparative performance in terms of BD-PSNR [dB] against SDR configuration of JVET-0021 (High Complexity mode):

|  | Constraint Set 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over SDR configuration of JVET0021 |  |  |  |  |  |  |  |
|  | BD-PSNR, dB |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | 0.59 | 0.35 | 0.54 | 1.61 | 1.46 | -0.38 | 1.09 | 1.05 |
| ShowGirl2 | 0.15 | 0.12 | -0.03 | 0.09 | 0.23 | -0.59 | -0.40 | -0.16 |
| Hurdles | 0.93 | 0.13 | 0.12 | 2.17 | 2.93 | -0.20 | 1.73 | 2.51 |
| Starting | 0.54 | 0.28 | 0.13 | 0.91 | 1.58 | -0.41 | 0.53 | 1.10 |
| Cosmos1 | 0.19 | 0.33 | -0.32 | -0.94 | -0.57 | -0.64 | -1.17 | -0.89 |
| Average HDR-B | 0.48 | 0.24 | 0.09 | 0.77 | 1.13 | -0.44 | 0.36 | 0.72 |

Table 9: Comparative performance in terms of BD-Rate [%] against SDR configuration of JVET-0021 (Low Complexity mode):

|  | Constraint Set 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over SDR configuration of JVET0021 |  |  |  |  |  |  |  |
|  | BD-rate, % |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | -44.8% | -18.8% | -13.0% | -71.3% | -100.0% | 10.6% | -55.7% | -60.0% |
| ShowGirl2 | -12.8% | -6.3% | 0.9% | -1.9% | 12.2% | 19.6% | 41.1% | 23.5% |
| Hurdles | -64.8% | -8.1% | -2.5% | -67.9% | -100.0% | 7.2% | -63.1% | -100.0% |
| Starting | -39.7% | -11.8% | -2.3% | -34.3% | -36.9% | 9.7% | -23.6% | -11.8% |
| Cosmos1 | -18.8% | -23.0% | 14.5% | 100.0% | 435.4% | 32.2% | 100.0% | 100.0% |
| Average HDR-B | -36.2% | -13.6% | -0.5% | -15.1% | 42.1% | 15.9% | -0.3% | -9.7% |

For additional information, Figures 7-8 show rate-distortion curves (a. – PSNR L100 and b. – DeltaE) for the proposed HDR scheme and two CfP anchors.

a.								b.

Figure 7: Rate-distortion curves for proposed HDR scheme and 2 CfP anchor, Market test sequence

a.								b.

Figure 8: Rate-distortion curves for proposed HDR scheme and 2 CfP anchor, Hurdles test sequence

a.								b.

Figure 9: Rate-distortion curves for proposed HDR scheme and 2 CfP anchor, Cosmos test sequence

# Conclusions:

This contribution provides additional information on HDR video coding technology proposal by Qualcomm and Technicolor presented in JVET documents J0021 and J0022. Simulation results reported in this document show that proposed CVT implemented on top of JEM7.0 software and tested on class HDR-B test set provides around 34.0% and 8.3% of bitrate reduction (for PSNR L100 HDR metrics) against HM and JEM and CfP HDR anchors respectively.

Additionally, this document reports, that for for HDR-B class sequences proposed CVT utilized in the JVET-J0021 core design provides 14% of bitrate reduction (for PSNR-L100 metric) over the default (SDR) coding configuration in the High Complexity mode and 13.6% of bitrate reduction over the SDR configuration in the Low Complexity mode.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)