| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0071-v1 |
| --- | --- |

| Title: | Non-local Structure-based Filter with integer operation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Xuewei Meng, Chuanmin Jia, Zhao Wang, Shanshe Wang, Siwei Ma Peking University Xiaozhen Zheng DJI | Tel: Email: Tel: Email: | swma@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., Peking University |  |  |

_____________________________

- Abstract

This contribution is a continuation of NLSF (Non-local Structure-based Filter) technique that was proposed in JVET-J0011. It proposes a solution for integer NLSF algorithm. The NLSF design in J0011 contains two modules: group construction by block matching and SVD-based filtering. The collaborative filtering is achieved by iterative singular value decomposition (SVD) that calculates the singular values with their singular vectors by iterative power method whose internal data type utilizes double precision float-point representation. To adapt the video coding standard as well as being hardware friendly, this proposal addresses this issue by eliminating the double precision values via the decimal digits clipping after shifting the intermediate results to large numbers during iterations. The simulation results show that the proposed fix-point algorithm for SVD module could achieve comparable performance with original NLSF algorithm.

Introduction

In current Non-local Structure-based Filter (NLSF) design, there are two procedures, group construction by block matching and SVD-based filtering. As illustrated in proposal JVET-J0011 [2], NLSF can improve the coding efficiency significantly. However, there are some complexity issues related to NLSF design in J0011 that are addressed below:

- The decimal operations during SVD and threshold estimation are not good for hardware realization;

- Hardware implementation of decimal operations has high complexity;

- Different processors might deal with decimals in different ways.

In order to solve these problems, an integer NLSF solution is proposed in this document. The rest of the document is organized as follows. Section two briefly reviews NLSF algorithm in proposal J0011. In section three, the proposed integer NLSF and its implementation details are elaborated. Section four shows experimental results of the proposed method. Finally, conclusions are drawn in section five.

Review of NLSF

NLSF locates between Deblocking Filter (DF) and Sample Adaptive Offset (SAO) in JEM, which includes group construction by block matching and SVD-based filtering.

Group construction by block matching

As illustrated in Fig 1, the input reconstructed frame is firstly divided into K overlapped blocks, and each block serves as the basic loop unit. Let Bs × Bs denote the size of loop unit, which is set to 6 by default. For each loop unit, it is stretched as a vector xk∈RBs × Bs, k=1,2,…,K. Then, for each block xk, no more than c (defaulted to 20) similar blocks are collected within the corresponding search window to construct the block group. As the example of Fig 1, the red square represents the current loop unit, meanwhile the blue dotted square denotes the search window and the golden dotted square denotes the CTU current block located in. We use m to represent the number of similar blocks selected in block matching process. After block matching process, all the similar blocks selected are stacked into a matrix of size Bs2×m, denoted by XGk, where each column of the matrix corresponds to one block, i.e. XGk=[xGk⊗1, xGk⊗2, …, xGk⊗m].

Fig 1. Illustrations for Non-Local Structure-based Filter

SVD-based Filtering

For each group XGk=xGk⊗1, xGk⊗2, …, xGk⊗m, k=1,2,…,K, it is decomposed by SVD as follows,

XGk= UGkGkVGkT= i=1mγxGk⊗i(uGk⊗ivGk⊗iT)              (1)

where γxGk=[γxGk⊗1;γxGk⊗2;…;γxGk⊗m] is a column vector, Gk=diag(γxGk) is a diagonal matrix with the elements of γxGk on its main diagonal, and uGk⊗i, vGk⊗i are the columns of UGk and VGk, separately. To suppress the compression noise, the hard thresholding operation is applied to the singular values, γxGk

αGk=hard(γxGk,τ)                               (2)

where hardx,a=x⊙1 (absx-a) denotes the operator of hard thresholding and ⊙ stands for the element-wise product of two vectors. τ denotes the threshold.

As the threshold τ used in hard thresholding process is related to Qstep. The relationship between QP and Qstep for an orthonormal transform is formulated as follow,

Qstep= 2QP-46                                   (3)

The relationship between τ and Qstep is linear as follow,

τ=a×Qstep+b×Bs2+ c                        (4)

where a and b represent the parameter trained offline. a and b are double type variables.

The filtered image can be reconstructed from the shrunk singular values αGk, and the reconstruction for group XGk can be formulated as,

XGk= i=1mαGk⊗i(uGk⊗ivGk⊗iT)                        (5)

This process is applied for all K groups to achieve XGk, k=1,2,…,K. At last, all XGk are averaged to get the frame x.

Proposed method

The fix-point algorithm for integer NLSF concentrates on two sub-modules, power method based iterative SVD and threshold estimation during each iteration. For simplification discussion, the size of similar block group matrix XGk is set to be p×q.

Overall Description

The original implementation for SVD is realized based on iterative power method, each iteration of which calculates the largest singular value with its singular vectors. The obtained largest singular value with its left/right singular vectors is used to get the approximation (XGk) of the original block group matrix (XGk). Then the difference between XGk and XGk is calculated. Subsequently, the difference is feed into the singular value calculation process to get the largest singular value and its singular vectors. Until the Singular value meets the filtering threshold or meets the rank of matrix XGk, the iteration is terminated. The algorithm description is depicted below.

Fig 2. The algorithm description of SVD

SVD

This section discusses the singular_value_decom process in Fig. 2. To obtain such singular value and vectors, the numerical iteration is done by first column-wise normalizing the similar block group XGk as the initialization of its right singular vector VGkT whose dimension is 1×q. Second, row-wise normalize the result after dot multiplication between the XGk and VGkT to generate the left singular vector UGk with size 1×p. Third, update the right singular value by column-wise normalizing the result after dot multiplication between the UGk and XGk. Note that the UGk in this step is calculated in step two. As such, one iteration is done and the left singular vector UGk and right singular vector VGkT can also be obtained. The largest singular value is the L2 norm of the right singular vector VGkT. The iteration for the largest singular value calculation is convergence until the singular value meets the precision requirement.

The objective of singular_value_decom function is to calculate the largest singular value γxGk with its left-singular vector UGk and right-singular vector VGkT. Most variables used during original SVD are double precision. In the integer SVD, we use 64-bit integers to store γxGk, UGk, VGkT, and normalization factor during SVD process. Other variables are stored by 32-bit integers. The details are given below,

As shown in Fig 3, the matrix X denotes the group XGi in Fig.2.. X is a double-precision variable. Hence, the first step is to left shifting the X by shift1 bits to transform X to a integer-type variable. Then sum the matrix X by rows and get a one-dimension matrix V1, V1=[v11, v12,…,v1q]. Then calculate the normalization factor norm of matrix X.

norm= j=1qv1j2                                   (6)

Integer square root is used to calculate norm. And after obtain norm, right shift it by shift2 bits. We use V1', V1'=[v1'1,v1'2,…,v1'q] to represent the normalized matrix of V1, where V1 is the initialization of right singular vector VGkT as illustrated before. The normalization operation is as follows,

v1'i= v1i(norm≪shift2)                                   (7)

After the normalization of matrix V1, row-wise normalize the result after dot multiplication between XGk and V1'to generate the left singular vector UGk with size 1×p. The integer square root process and the shrink of normalization factor during the normalization process of left singular vector UGk are the same as before. Like the calculation of left-singular vector, the calculate of right-singular vector uses similar manners.

Fig 3. Flowchart steps of singular_value_decom

Threshold estimation

The filtering threshold is determined by QP values which are integers between 1 and 51 and there is a one-to-one relationship between QP and Q_step, we use a look-up table named Qstep_T with the length of 52 to store Qstep. And the values in Qstep_T are all calculated in advance. The calculation of Qstep_T is as follows,

Qstep_Ti= 2i-46×(1≪12)                           (8)

where i (0≤i≤51) represents the ith element of Qstep_T.

The parameter a and b trained offline are both enlarged, wherein, a is enlarged by 212 times and b is enlarged by 224 times.

Bitstream structure

Compare to NLSF in proposal J0011, there are three frame level on/off controls of the proposed integer NLSF, slice_nlsf_luma_flag, slice_nlsf_cb_flag and slice_nlsf_cr_flag, which are signalled in the slice header to indicate whether NLSF are enabled for luma, cb and cr components, respectively.

| slice_segment_header( ){ | Descriptor |
| --- | --- |
| …… |  |
| if (nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP){ |  |
| …… |  |
| if(sps_temporal_mvp_enable_flag) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| } |  |
| if(nonlocal_structure_filter_enabled_flag){ |  |
| slice_nlsf_luma_flag | u(1) |
| if(slice_nlsf_luma_flag){ |  |
| slice_nlsf_cb_flag | u(1) |
| slice_nlsf_cr_flag | u(1) |
| } |  |
| } |  |
| if(sample_adaptive_offset_enabled_flag){ |  |
| slice_sao_luma_flag | u(1) |
| if(slice_sao_luma_flag) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| …… |  |
| } |  |

Experimental results

The proposed integer NLSF is implemented on JEM7.0. To evaluate the coding performance efficiently, a QP set that approaches the bitrates described in CfP document [4] is used at anchor data and the test data generation. It should be noted that QP setting is varied by sequences because the bitrates are determined by sequences in JVET-H1002 document.

Compared to JEM7.0 data, the performance of integer NLSF is show below:

Table 1. Performance of the proposed integer NLSF

The performance of J0011’s NLSF design is also provided below. The data comes from “tools-off” test described in J0011 whose anchor data is almost same as JEM7.0 anchor.

Table 2. Performance of J0011’s NLSF (“tools-off” test)

Conclusion

This contribution proposes an integer NLSF solution, which can achieve hardware-friendly design. Compare to J0011’s NLSF design that use float point operation, it has minor coding lost but the coding performance improvement is still substantial.

References

[1] J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, JVET-F1001, “Algorithm description of Joint Exploration Test Model 7 (JEM7),” 2017.

[2] Z. Wang, X. Meng, C. Jia, et al. “Description of SDR video coding technology proposal by DJI and Peking University,” JVET-J0011, April., 2018.

[3] K. Suehring, X. Li, “JVET common test conditions and software reference configurations,” JVET-C1010, May, 2016.

[4] A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC”, JVET-H1002, Macao, CN, 18–24 Oct.2017.

Patent rights declaration(s)

SZ DJI Technology Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).