| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0072_v2 |
| --- | --- |

| Title: | SW for technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario (JVET-J0024) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexander Alshin, Elena Alshina, Kiho Choi, Narae Choi, Woongil Choi, Seungsoo Jeong, Bora Jin, Chanyul Kim, Junghye Min, JeongHoon Park, Minsoo Park, Min Woo Park, Yinji Piao, Anish Tamse, Heechul Yang (Samsung) Huanbang Chen, Jianle Chen, Roman Chernyak, Semih Esenlik, Alexey Filippov, Shan Gao, Sergey Ikonin, Alexander Karabutov, Anand Meher Kotra, Xiaomu Lu, Xiang Ma, Vasily Rufitskiy, Timofey Solovyev, Victor Stepin, Maxim Sychev, Tian Wang, Ye-kui Wang, Weiwei Xu, Haitao Yang, Vladyslav Zakharchenko, Hong Zhang, Yin Zhao, Zhijie Zhao, Jiantong Zhou, Cheung Auyeung, Han Gao, Ivan Krasnov, Ruslan Mullakhmetov, Biao Wang, Yiu Fai Wong, Georgy Zhulikov(Huawei) Adeel Abbas, David Newman (GoPro) Jicheng An, Xu Chen, Yongbing Lin, Quanhe Yu, Jianhua Zheng (HiSilicon) | Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +82-10-8855-2932 elena_a.alshina@samsung.com kiho14.choi@samsung.com +1-408-330-5334 jianle.chen@huawei.com +14437398055 aabbas@gopro.com +86-010-82829521 zhengjianhua@hisilicon.com |
| Source: | Samsung Electronics, Huawei, GoPro, and HiSilicon |  |  |

_____________________________

# Abstract

This contribution provides IFVC SW package. This is implementation for all tools described in technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario. After CfP bit-stream submission SW was optimized, cleaned and also several minor bugs have been fixed. Under CS1 test conditions IFVC provides 36%, 36% 37% BD-rate gain over CfP anchor for Y, Cb, Cr components correspondently while encoding and decoding run time is ×5 and ×3 compare to CfP anchor. For example, SW can be configured to show 13% gain over CfP anchor with 15% faster encoder or to provide 19% while encoder speed is the same as HM. Decoder time is significantly lower (almost half) than HM anchor in these configurations.

# Introduction

Software used for CfP response results generation by Samsung, Huawei, GoPro, and HiSilicon internally called Industry Future Video Codec (IFVC). Tools set for CfP response was agreed between four companies and designed under consideration of mobile application scenario. This tools set was used for CfP bit-streams generation [1]. Technology proposal description document [1] contains detailed description for all tools including syntax table.

After CfP bit-streams submission joint team of four companies have been working on SW optimization and clean-up. Also several minor bugs have been fixed.

As a result of this work SW speed has been improved twice compare to original CfP submission with around 1% performance difference. Under CS1 test conditions IFVC provides 36%, 36% 37% BD-rate gain over CfP anchor for Y, Cb, Cr components correspondently while encoding and decoding run time is ×5 and ×3 compare to CfP anchor. IFVC SW is twice faster than JEM7.1.

# Performance summary

For IFVC SW testing in CS1 scenario we used the same base QP per sequence as described in [1] submission but w/o QP increment (for simplicity). Relatively to CfP anchor performance and run time for response [1] and IFVC SW is shown in Table. 1.

Table 1. Performance in CS1 for IFVC SW compared to JVET-J0024 CfP response results.

| Over HM CfP anchor | JVET-J0024 CfP response |  |  | IFVC software |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Average SDR-A | -41.2% | -35.2% | -37.1% | -40.2% | -35.0% | -37.0% |
| Average SDR-B | -32.8% | -36.7% | -37.7% | -31.3% | -37.3% | -37.9% |
| Average all | -37.0% | -36.0% | -37.4% | -35.8% | -36.2% | -37.4% |
| Enc Time[%] | 902% |  |  | 494% |  |  |
| Dec Time[%] | 274% |  |  | 292% |  |  |

# Software

The source code is attached to this contribution. All tools mentioned in JVET-J0024 have macro which can be enabled and disabled. SW allows testing multiple combination of JVET-J0024 tools. Parallel and sequential encoding provides identical results.

SW usage is described in IFVC_readMe.txt file.

# Additional operating points

Donated SW is highly configurable.

With minimal set of tools 13% gain over CfP anchor can be achieved while encoder is 15% faster.

Operating with HM encoder speed IFVC shows 19% gain over CfP anchor while decoder is only 60% of HM. Noticeably faster decoder indicates this tools combination is significantly simple than HEVC.

If twice slower encoder speed is acceptable then IFVC shows 29% gain over CfP anchor. For this combination of IFVC tools decoder speed is the same as HM (complexity of tools is roughly the same).

More operation points of IFVC are demonstrated in excel attached to this contribution. IFVC macro settings for any of those operation points provided upon request.

Table 2. IFVC operation points.

|  | BD-rate Y | BD-rate Cb | BD-rate Cr | EncTime | DecTime |
| --- | --- | --- | --- | --- | --- |
| “minimal set of tools” | -12.5% | -13.5% | -13.7% | 84% | 57% |
| “×1 HM encoder speed” | -18.6% | -18.6% | -18.8% | 101% | 60% |
| “×2 HM encoder speed” | -29.2% | -30.1% | -30.5% | 197% | 100% |
| “J0024 set of tools” | -35.8% | -36.2% | -37.4% | 494% | 292% |

# Conclusion

The proponents suggest considering the provided SW as the basis for tools testing during Test Model selection.

# References

[1] Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario, JVET-J0024.