| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0080-v1 |
| --- | --- |

| Title: | Results of Subjective Testing of Responses to the Joint CfP on Video Compression Technology with Capability beyond HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input to JVET |  |  |
| Purpose: | Test report |  |  |
| Authors | Vittorio Baroncini Director EVATech Via Pisanelli, 4 00196 – Rome, Italy | Tel: Email: | +39 3335474643 baroncini@gmx.com |
| Source: | Test Group |  |  |

_____________________________

# Abstract

This document reports the work to design, organise and conduct the Formal Subjective Test for assessing the quality of Submissions to the Joint Call for Proposals on Video Compression Technology with Capability beyond HEVC, issued by MPEG and VCEG [1].

# Summary of Activities

After receiving expressions of interest, the Test Coordinator prepared a plan to accommodate the large number of submissions expected; more in detail, JVET received a total of 46 submissions in the following category groups:

  - 22 SDR submissions with 36 test cases each,

  - 12 HDR submissions with 32 test cases each,

  - 12 360° submissions with 20 test cases.

In addition, HM and JEM anchors had to be tested in all three categories, which made a total of 52 category-specific tests to be performed.

Additional partners were added to some proposals later, bringing the total number of institutions to 32. All proponent groups properly completed the submission process.

# Logistics and conduction of subjective tests

A total of 7 test labs, which are experienced in their respective fields, had agreed to contribute to the test effort. Tests were partially overlapping (conducted in at least two labs) such that verification about the validity and comparability of the tests would be possible. In particular, the test labs were as follows:

  - For SDR @ HD resolution: BBC (England), Queen Mary University of London (QMUL - England), University of West Scotland (UWS - Scotland);

  - For SDR @UHD resolution: Centrum Wiskunde & Informatica (CWI - Netherlands)

  - For 360°: University of Padova, GBTech (Italy);

  - For HDR - PQ: RAI, Sisvel (Italy);

  - For HDR – HLG: DBW and EVATech (Italy).

GBTech, EVATech and CWI collaborated in general coordination and taking care of contacts and administrative matters with the proponents; designing the test sessions and creating the scripts, as well as visually checking all files on the disks received. Furthermore, they created the dynamic viewports for the 360° video test cases (not known to proponents beforehand), which had been selected with help from Jill Boyce (Intel) as independent expert.

A wide number of human viewing subjects were hired and participated to the test in the above test sites; the number of subjects was very high (around twice the theoretical number) to allow a low level of fatigue and maintain high the response and efficiency of the participants.

Following this approach, more than 2000 people were involved as test participant to perform a total of 58 test sessions, for a total testing time exceeding 500 hours of work including test execution and laboratory set-up time.

# Design of the tests

The design of the 58 test sessions was entirely done by the test coordinator, with the support of CWI expert.

SDR testing effort required 36 test sessions, HDR tests were done designing 14 test sessions, while 360 required 8 test sessions. The medium duration of a test session never exceeded 20 minutes; this solution allowed a low fatigue level and high efficiency of the viewers.

All scores were collected by paper sheets; a novel data acquisition technique was experienced (already successful tested in other JVET testing experiments) assigning the data entry task to the viewers; this was very safe, efficient (two viewers dialled in the data; other two checked the inputs) and allowed to lower the level of stress of the participants changing the cognitive load of the viewing activities.

The exact test procedure is described in [1]

# Test results and graphs

This paragraph provides the graphs that depict the performances of the submissions to the Call. Each graph represents the results of all submissions in the respective category, as well as the performances of the two anchor encoders, i.e. HM (HEVC) and JEM [2].

In the tables below the Anchors are labeled with red circles (HM) and with blue circles (JEM). Other proposals are not identified.

Note: Pending confirmation that the error bars indicating 95% confidence intervals have to be doubled in 4.1-4.5.

## Results for the 360 category

## Results for the SDR HD constrain 1 category

## Results for the SDR HD constrain 2 category

## Results for the SDR UHD constrain 1 category

## Results for the HDR HLG category

## Results for the HDR PQ category

# Conclusions

The results of the subjective tests clearly confirm the existence of new video compression technology which is superior in terms of subjective quality in most test cases, when compared against HEVC, and also superior in a relevant number of test cases, when compared against JEM.

# References

[1] Joint Call for Proposals on Video Compression with Capability beyond HEVC, JVET-H1002, Macao, Oct. 2017

[2] Algorithm description of Joint Exploration Test Model 7 (JEM7), JVET-G1001, Torino, Jul. 2017