| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0081_r1 |
| --- | --- |

| Title: | Comments on Test Model Development |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Wade Wan Tim Hellman Peisong Chen Ola Hugosson Dominic Symes Alberto Duenas Eric Chai | Email: | minhua.zhou@broadcom.com wade.wan@broadcom.com tim.hellman@broadcom.com peisong.chen@broadcom.com Ola.Hugosson@arm.com Dominic.Symes@arm.com Alberto.Duenas@arm.com eric.chai@ublnx.com |
| Source: | Broadcom, ARM, RealTek |  |  |

_____________________________

# Abstract

The response to the Joint CfP on Video Compression with Capability beyond HEVC provides many interesting coding technologies with different tradeoffs between coding performance and codec complexity.  This document is a joint contribution from several companies focused on the feasibility of implementation that did not submit responses to the CfP and intended to provide some recommendations for tool selection for an initial test model as well as other comments for the long term standardization development.

# Recommendations for tool selection for an initial test model

The following recommendations are suggested to aid the discussion of tool selection for an initial test model:

- Quadtree plus binary tree and triple tree (QTBTT) appears to be a good starting point for block partitioning

- Suggest setting the maximum TU size to 64x64 (luma)/32x32 (chroma) as the initial starting point with further study on the impact of larger TU sizes of 128x128 (luma)/64x64 (chroma) in CEs

- Inter coding tools (e.g. template matching in FRUC, uni-directional LIC, diffusion filter for inter prediction) which require access to the reconstructed samples from neighboring blocks have a substantial implementation impact so it is recommended not to include them in the test model

- Neural Network (e.g. CNN) based tools have a substantial implementation impact for decoder so it is recommended to further study the tradeoff between coding efficiency and complexity before deciding whether to include them in the test model

Additional detail and reasoning for the recommendations above is provided in the attached slides.

# Comments on the long term standardization development

The following comments are feedback the authors have on some of the other tools that do not necessarily have to be addressed now but believe it is useful for the group to consider for the long term standardization development

- Small transform and intra prediction block sizes less than 4x4 should be avoided for both luma and chroma

- The MPM list derivation may need to be decoupled from CABAC parsing to avoid parsing dependency issues limiting throughput.

- CABAC context initialization from previous frames should be carefully studied as it makes it harder to parallelize across multiple cores.

- New coding tools inside the intra prediction critical loop, such as bilateral filter, multiple line intra prediction, PDPC + planar, LMC, boundary filters, smoothing/sharpening filters and etc., should be closely studied as there is concern over throughput and/or implementation cost issues with these tools.

- Decoder side motion refinement and inter prediction tools, such as bilateral matching in FRUC, DMVR, BIO and bi-prediction LIC, should be closely studied as there is concern over memory bandwidth consumption, throughput and/or implementation cost with these tools. For example,

- Block to block serial dependency and memory latency issues for bilateral FRUC needs to be seriously investigated

- Memory latency caused by the feedback of refined motion vectors to the MV reconstruction process also needs investigation

- 4x4 based motion compensation such as currently being done in JEM7.0 is a memory bandwidth concern.

- Non-separable 8x8 secondary transforms is another significant cost issue that should be addressed.

# Patent rights declaration(s)

Broadcom Incorporation does not have patent rights relating to the technology described in this contribution.

ARM does not have patent rights relating to the technology described in this contribution.

RealTek does not have patent rights relating to the technology described in this contribution.