| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0083_r1 |
| --- | --- |

| Title: | Memory usage analysis in available software packages of the responses |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 80 5066 9075 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution presents a memory usage analysis in available software packages during the first week of the meeting. Some parameters like QP and resolution are varying to confirm the dependency to them. The memory usage of both encoder and decoder is one of the point of complexity analysis. In general case, the memory usage of encoder is larger than that of decoder. When an encoder memory usage is small, such the encoder can be tested simultaneously with multiple conditions under the given amount of memory.

Introduction

In this meeting, three new encoding software package is provided in SVN or in contribution. An anchor is the Joint Exploration Test Model (JEM) software package [1]. The first new software package is announced in the reflector on 11th April corresponding to the JVET-J0014 [2]. The second new software package is JVET-J0075 corresponding to the JVET-J0021 [3]. The third new software package is JVET-J0072 corresponding to the JVET-J0024 [2].

Memory usage comparison

Measurement methodology and conditions (revision 1)

All software is compiled by GCC 4.8.5 on the linux environment (CentOS 7.2 64bit). Running environment is Xeon E5-2640 v4 with 128GB memory or Xeon E5-2697 v3 with 128GB. Memory usage of each software is collected by “ps” command at every 1 second. The maximum memory usage timing is extracted as an memory usage value. The reported value in this contribution select “RES” column. Coding structure is Constraint 1.

2K sequences are BasketballDrive, BQTerrace, and Cactus with QP 22 and 37. 4K sequences are Trafficflow with QP 22 and 37, CampfireParty2 with QP 37, CatRobot with QP37, and DaylightRoad2 with QP37. Some 4K sequences with QP22 are skipped due to the limited time. Encoded duration is one intra period, e.g. 32 frames for 30fps sequence.

Command examples are as follows.

[JEM7.0]$ ../bin/TAppEncoderStatic -c ../cfg/encoder_randomaccess_jvet10.cfg -c ../cfg/per-seq/TrafficFlow.cfg --ReconFile= -q 37 --IntraPeriod=32 --FramesToBeEncoded=32 -b ../str/str_RA10_TrafficFlow_qp37.bin

[J0014]$ ../bin/EncoderApp -c ../cfg/encoder_randomaccess_jem7.cfg -c ../cfg/per-seq/TrafficFlow.cfg --ReconFile= -q 37 --IntraPeriod=32 --FramesToBeEncoded=32 -b ../str/str_RA10_TrafficFlow_qp37.bin

[J0021]$ ../bin/encoder -c ../cfg/encoder_randomaccess_main10_QTBT.cfg -c ../cfg/per-seq/TrafficFlow.cfg --ReconFile= -q 37 --IntraPeriod=32 --FramesToBeEncoded=32 -b ../str/str_RA10_TrafficFlow_qp37.bin

[J0024]$ ../bin/ifvca_encoder -i TrafficFlow_3840x2160_30fps_10bit_420_jvet.yuv -o ../str/str_RA10_TrafficFlow_3840x2160_30fps_10bit_420_jvet_qp37.bin -w 3840 -h 2160 -q 37 -g 15 -z 30 -p 32 -v 1 -f 32 -d 10 --output_bit_depth 10 -s

Additional conditions (revision 2)

For further consideration, another configuration provided in the software packages are also tested.

Regarding JVET-J0014 response, SVN repository provides full tool set while the JVET-J0035 provides partition only configuration. Therefore, four configurations, e.g. jem7, mtt10, qtbt10, and qtbts10, are tested for JVET-J0014. Three configurations, e.g. mtt10, qtbt10, and qtbts10, are also tested for JVET-J0035.

Regarding JVET-J0021 response, JVET-J0075 provides partition only configuration. Five configurations, e.g. QTBT+ABT, QTBT+TT,QTBT+ABT+TT, QTBT+TThc, and QTBT, are tested.

Regarding JVET-J0024 response, JVET-J0072 provides full tool set and minimum tool set. Two configurations are tested.

Comparison results (revision 1)

Memory usages of 2K and 4K for each software package are following.

Table 1 Memory usage comparison of encoders [GiB]

|  |  | 2K | 4K |
| --- | --- | --- | --- |
| G1001 | JEM7.0 | 1.8 | 6.0 |
| SVN | J0014 | 0.8 | 3.6 |
| J0075 | J0021 | 0.8 | 2.3 |
| J0072 | J0024 | 1.5 | 3.2 |

More detailed memory usage is found in the attached Excel file.

From the detailed results, following items are confirmed. The amount of memory usage of encoder is larger than that of decoder for all software package. The memory usage is

- independent from QP values and content,

- dependent on the frame rate, i.e. intra period, for J0014 and J0021,

- dependent on the image resolution.

Comparison results (revision 2)

From the above discussion, we shrink the test condition into QP=37 only. Another conditions like sequence variation and the number of encoded frame are the same as previous test. Detailed results are attached as the Excel file. For the 4K cases, measurement results are shown in Figure 1.

Figure 1 Encoder peak memory usage for TrafficFlow with QP37. Unit is [GiB]

When we consider the partition only configuration, all software realizes the memory usage reduction significantly. It is noted that this contribution does not investigate the definition of “partition only” or “minimum tool set.”

Conclusion

We compared the encoder memory usage of four software. For further development of video coding technology, less memory usage is encouraged. Consequently, the software package in J0075/J0021 is much among them.

References

- “Algorithm Description of Joint Exploration Test Model 7 (JEM7)”, Joint Video Exploration Team (JVET) of ITU-T VCEG (Q6/16) and ISO/IEC MPEG (JTC 1/SC 29/WG 11), 7th Meeting, Torino, Italy, July 2017, Doc. JVET-G1001.

- https://ipbt.hhi.fraunhofer.de/svn/svn_hhi-next-software/tags/NextSoftware-JVET-J0014

- “Partition only software of the video coding technology proposal by Qualcomm and Technicolor”, Joint Video Exploration Team (JVET) of ITU-T VCEG (Q6/16) and ISO/IEC MPEG (JTC 1/SC 29/WG 11), 10th Meeting, San Diego, US, 2018, Apr. JVET-J0075.

- “SW for technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario (JVET-J0024)”, Joint Video Exploration Team (JVET) of ITU-T VCEG (Q6/16) and ISO/IEC MPEG (JTC 1/SC 29/WG 11), 10th Meeting, San Diego, US, 2018, Apr. JVET-J0072.

Patent rights declaration(s)

KDDI Corporation does not have any current or pending patent rights relating to the technology described in this contribution.