| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0085-v3 |
| --- | --- |

| Title: | BoG report on 360 video |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill.boyce@intel.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met on Apr 14 with the primary goal of preparing a survey of the proposed technologies includes in responses to the Call for Proposals in the 360° video category. A spreadsheet containing the prepared summary is attached. A revised version is attached in the v2 version of this document, to provide improved clarity on whether the responses proposed changes within the coding loop or not.

Some questions were raised during the BoG meeting regarding the plans for the 360Lib software. Currently, the 360Lib software is integrated with the HM and JEM. When a Test Model is defined, should the 360Lib software or a variant of it be integrated with it as well? For experiments involving 360°-specific coding tools such as those included in some CfP responses, some sort of integration of projection mapping and the codec is desirable.

Several new projection formats were proposed by proponents for inclusion in 360Lib, but BoG did not discuss yet, since it depends on the general plans for 360Lib. It was suggested to define a CE on projection formats.

The BoG met again on Apr 19 from 6:30 pm – 8:00 pm. A number of recommendations were made to the Common Test Conditions for 360 Video, as detailed in section 2.1. One item was marked for revisit. A draft of the CE on projection formats was discussed, with several changes decided, as detailed in section 2.2.

# Discussion

In addition to the summary include in the spreadsheet table, some additional points were captured when discussing some of the contributions, which are noted below.

## JVET-J0019

Proposes MCP projection format plus coding tools.

MCP Format:

- For top and bottom faces use EAC

- For other four faces, use EAC in horizontal dimension, and other projection in vertical dimension

- Proposing MCP be added to 360Lib

Coding tools:

- For inter, derive projected MV from spherical neighbor block

- For other cases, consider neighbors unavailable if across dis. boundary

Coding gains (not included in CfP response individually):

- 0.35% gain for MCP vs. EAC with HM

- 1.08% for inter

- 0.45% for intra

- 0.2% for loop filter (asserts bigger impact on subjective quality)

## JVET-J0021

Coding tools average 2.2% gain for Low Complexity

## JVET-J0024

RSP with content-adaptive spatial adjustment, signaled per sequence. Should really be signaled per RAP.

0.2% gain for RSP change.

## JVET-J0022

CU-adaptive QP based on spatial position.

## JVET-J0033

Proposes PAU projection format. Similar to J0019 in that one dimension’s projection function is changed, but new projection function differs.

No coding tool changes.

0.2% coding gain for PAU vs EAC.

Proposing PAU be added to 360Lib.

## JVET-J0015

Proposes HAC adaptive projection mapping function. Is a generalization of EAC.

Signal 2 parameters per face, 12 total. Can change for each RAP.

Conversion of reference frames between projection mapping formats is used (for open GOP.)

Proposing HAC be added to 360Lib.

From tool-on test, coding gains:

- 0.32% for HAC over EAC

- 0.54% for adaptive frame packing

## 0.33% for face discontinuity (but more subjective impact)

- 1.62% for geometry padding

## Other discussion

Question: Should there be some type of a “Test Model” for 360Lib type functionality? Should some 360Lib-like software have some new status?

Suggest to have a CE on projection formats, to study proposed new formats and existing formats.

The CE would bring experimental results using the CTC. Will need to define CTC for projection formats and for 360 coding tools using the new test Model.

Which codec to use for projection formats? New Test Model would require integration work

Consider removing unused formats from 360Lib.

Raise question about 360Lib status in track, and hold additional BoG session afterwards.

# Discussion at Apr 19 meeting

# Topics for discussion:

# Updates to CTC

# Cosmetic changes to align to new SDR CTC

# Test sequences?

- CE for projections

  - Which projection formats to include?

# Limit to those in CfP responses or already in 360Lib

# 8 listed in the draft CE description

# Objective metrics

# Plan for expert viewing at next meeting

# CfP dynamic viewport paths?

# Static viewports?

# Evil viewports?

# Any scoring, or just informal viewing

# What viewing equipment requirements?

## Updates to CTC

BoG recommends the following:

Eliminate the requirement to have coding tools required to test with ERP.

For coding tool proposals, provide gains relative to the same projection format without the coding tool.

If the coding tool is tested using a variation of cube map such as EAC, it is encouraged to also provide data for the basic CMP format.

Remove the 4K source sequences.

Don’t add the moving camera versions of KiteFliteWalking and HarborBiking this meeting, but sequences are available on ftp site, and may be used to provide additional information. Consider at next meeting if should add to the CTC.

Suggest to have a face size that is a multiple of the CTU size. Check to see if that would exceed the level limit for HEVC.  Revisit.

Restrict coded pixel sizes to be +/- 3% of ERP size.

Switch to the viewport sizes from the CfP.

Metrics: only required codec based metrics for coding tool based proposals and be optional for projection format based proposals.

Provide anchors for PERP, CMP.

## CE for projections

Tests to be performed in the CE:

For RSP:

- Basic format that was present in 360Lib before CfP

- Rotation

- Rotation + Inactive area filling + blending

For MCP

- No padding

- With padding

Add the EAC w/ padding, even though proponent not asking for inclusion in the CE,  and try to get someone to perform the test.

For PAU

- No padding

- With padding

For CMP

- No padding

- With padding

Proponents can select the amount of padding and type of blending for their format, but need to specify by the CE document deadline.

Objective metrics – follow CTC.

Use evil viewports for informal subjective viewing at next meting, especially for padding vs. no padding.