| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J10XX |
| --- | --- |

| Title: | JVET common test conditions and evaluation procedures for HDR/WCG video |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Segall E. François D. Rusanovskyy | Email: | asegall@sharplabs.com Edouard.Francois@technicolor.com dmytror@qti.qualcomm.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document defines common test conditions (CTC), conversion practices and software reference configurations to be used in the context of HDR/WCG video coding experiments after the 10th JVET meeting. These common test conditions are recommended for use in technical contributions to the 11th and following JVET meetings, as applicable.

# Introduction

Common test conditions (CTC) are desirable to conduct experiments in a well-defined environment and to ease the comparison of the outcome of experiments.

This document defines four test conditions for HDR/WCG content, reflecting intra-only, random-access, and low-delay settings:

- Intra, 10 bit

- Random access, 10 bit

- Low delay, 10 bit

- Low delay, P slices only, 10 bit

A subset of these test conditions might be used for a particular experiment. For example, when testing an intra coding tool, only intra configurations might be used.

Version 1.0 or newer of the XXX software is expected to be used for most experiments. More recent versions are encouraged where applicable.

The following sections define test sequences, quantization parameter values, evaluation processes, encoder configuration files, and compile-time options to be used.

People bringing input contributions to JVET meetings should provide a set of results that is as complete as possible and uses the common test conditions that apply to the proposal.

# Test sequences

Table 1 and Table 2 define the sets of test sequences to be used for intra, random-access, and low-delay experiment conditions. All frames (as defined by frame count in the table) shall be encoded for all sequences in the random-access and low-delay test cases described below. For the intra configuration, the first of every eight frames shall be encoded starting with the first frame in the sequence for all sequences, and bit rates shall be calculated using the sequence frame rate (as defined by frame rate in the table) divided by eight. Please note that temporal sub-sampling may be enabled in the XXX software using the parameter TemporalSubsampleRatio.

The column corresponding to each configuration is interpreted as follows:

- “M” indicates that the test sequence is mandatory in the JVET CTC for the given configuration

- “O” indicates that the test sequence is optional (but encouraged) in the JVET CTC for the given configuration

- “-” indicates that the test sequence is not requested in the JVET CTC or the given configuration

Original versions of the test sequences in Table 1 and Table 2 are available on ftp://jvet@ftp.ient.rwth-aachen.de/testsequences/testset_hdr/  (properly qualified participants of JVET may please contact the JVET chairs for login information).

Table 1 Test sequences

| Class | Sequence name | Frame count | Frame rate | Bit depth | Transfer Function | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H1 | Market | 400 | 50 fps | 10 | PQ | M | M | - |
| H1 | SunRise | 200 | 25 fps | 10 | PQ | M | M | - |
| H1 | ShowGirl | 339 | 25 fps | 10 | PQ | M | M | - |
| H1 | BalloonFestival | 240 | 24 fps | 10 | PQ | M | M | - |
| H1 | EBU_Hurdles | 500 | 50 fps | 10 | PQ | M | M | - |
| H1 | EBU_Starting | 500 | 50 fps | 10 | PQ | M | M | - |
| H1 | Cosmos1_TreeTrunk | 240 | 24 fps | 10 | PQ | M | M | - |

Note: The capture frame rate of the EBU_Hurdles and EBU_Starting sequences was 100fps. However, these sequences are treated as 50fps sequences for the evaluation processes defined in this document.

Table 2 Test sequences

| Class | Sequence name | Frame count | Frame rate | Bit depth | Transfer Function | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H2 | DayStreet | 600 | 60 fps | 10 | HLG | M | M | - |
| H2 | FlyingBirds | 600 | 60 fps | 10 | HLG | M | M | - |
| H2 | PeopleInShoppingCenter | 600 | 60 fps | 10 | HLG | M | M | - |
| H2 | SunsetBeach | 600 | 60 fps | 10 | HLG | M | M | - |

# Quantization parameter values

# This document defines two configurations for the quantization parameter values.  The first condition uses a fixed QP setting and is intended to be used primarily for objective evaluation.  The second configuration requires the QP to be selected to achieve bit-rate targets, and it is intended to be used for both subjecdtive and objective evaluation.  Unless otherwise stated, it is required to provide results for both Configuration 1 and Configuration 2.

# Configuration 1 (Fixed QP)

For the first configuration, each test sequence shall use the four quantization parameter values 22, 27, 32 and 37. These values define the initial QP values that are specified for the I-frames in a sequence. Note that the configuration files may enable the adaptation of the used QP value on a per-frame and CU basis.

# Configuration 2 (Fixed Bit-rate)

For the second configuration, results shall be provided using the target bit rate listed in Table 3.  The results shall not exceed the target bit rate points and shall only use the random access configuration. Simulations shall use the provided configuration files, but the quantizatition configuration shall be chosen to achieve the target bit rate points. Quantization settings should be kept static except that a one-time change of the quantization settings to meet the target bit rate is allowed. Only sequences listed in Table 3 need to be provided in this configuration.

Table 3 – Target bit rate points not to be exceeded for Configuration 2

|  | Target bit rates [kbit/s] |  |  |  |
| --- | --- | --- | --- | --- |
| Sequences | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| DayStreet | 680 | 1100 | 1750 | 2800 |
| PeopleInShoppingCenter | 640 | 960 | 1450 | 2150 |
| SunsetBeach | 1500 | 3000 | 6000 | 10000 |
| Market3 | 600 | 900 | 1500 | 2400 |
| ShowGirl2 | 350 | 500 | 750 | 1200 |
| Hurdles | 450 | 800 | 1500 | 2000 |
| Starting | 450 | 600 | 850 | 1400 |
| Cosmos1 | 500 | 900 | 1500 | 3000 |

# Configuration files

The following sections define encoder configuration files to be used for each test case. Parameters to be changed for each test point are:

- InputFile to reflect the location of the source video sequence on the test system

- FrameRate to reflect the frame rate of a given sequence as per the tables above

- SourceWidth to reflect the width of the source video sequence

- SourceHeight to reflect the height of the source video sequence

- FramesToBeEncoded to reflect the frame count of a given sequence as per Tables above

- IntraPeriod to reflect the intra refresh period in the random access test cases. The intra refresh period is dependent on the frame rate of the source and the GOP size in use: a value 16 shall be used for sequences with a frame rate equal to 20fps, 32 for 24fps, 32 for 30fps, 48 for 50fps, 64 for 60fps, and 96 for 100fps.

- QP to reflect the quantization parameter values defined in section 3.

Configuration files are provided in https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-7.0rc1/cfg/. When available, newer versions of the configuration files, attached to the JEM software, should be used. There are 4 configurations provided as follows:

- “All Intra” (AI): encoder_intra_jvet10.cfg

- “Random access” (RA): encoder_randomaccess_ jvet10.cfg

- “Low-delay B” (LB): encoder_lowdelay_ jvet10.cfg

- “Low-delay P” (LP, optional): encoder_lowdelay_P_ jvet10.cfg

Sequence-specific parameters are to be found in the https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-7.0rc1/cfg/per-sequence-HDR/ folder. When available, newer versions of the configuration files, attached to the XXX software, should be used. Note that the adjustment of the QP value may differ for different test classes and may employ methods similar to those defined in JCTVC-Y1017, “Conversion and Coding Practices for HDR/WCG Y′CbCr 4:2:0 Video with PQ Transfer Characteristics”.

# Evaluation

Results reported shall include the following metrics:

- PSNR for Y′, Cb and Cr

- wPSNR for Y’, Cb and Cr

- deltaE100

- PSNR-L100

The metrics deltaE100 and PSNR-L100 shall be calculated using the HDRMetrics v0.17 or newer utility available at https://gitlab.com/standards/HDRTools.

All metrics shall use the sequence data provided as input to the encoder as the original sequence. For example, when the sequence data provided to the encoder is in the Y′CbCr 4:2:0 10 bits PQ format, metrics requiring an EXR version of the original sequence shall obtain the EXR version from this Y′CbCr 4:2:0 10 bits PQ original sequence.  Similarly, when the sequence data provided to the encoder is in the Y′CbCr 4:2:0 10 bits HLG format, metrics requiring an EXR version of the original sequence shall obtain the EXR version from this Y′CbCr 4:2:0 10 bits HLG original sequence.

Additional information for calculating wPSNR, deltaE100 and PSNR-L100 metrics is provided in Annex A.

# Parallel encoding/decoding

Parallel encoding/decoding as described in JVET-B0036 may be applied for RA configurations. If parallel encoding and decoding is used, this method shall be applied for both versions, the reference and the version under test. When reporting results, it shall also be reported whether parallel encoding/decoding was used or not.

Example scripts are provided with the XXX software.

# Results Template

A template excel files for reporting test results for Configuration 1 and Configuration 2 are attached to this document.

# Compile-time settings

Compile-time settings are defined mostly in the TypeDef.h file located in the source/Lib/TLibCommon folder of the common software. The default settings provided in the source code should be used.

# Annex A: Objective distortion metrics for HDR quality evaluations

The common test conditions described in this document requires a number of metrics to be reported for evaluation. In this Annex, further information about the wPSNR, deltaE100, and PSNR-L100 metrics are provided.

wPSNR

The wPSNR, or weighted PSNR, metric is calculated from the weighted mean squared error of the pixel values. An implementation of the metric is provided in the JEM software, and that implementation shall be used for calculation of the metric using the weighting functions provided below.

As an informative description of the metric follows:

The wPSNR metric is calculated as:

wPSNR=10*logX2wMSE,

where X is the maximum pixel value for the specific bit depth and wMSE is given as

wMSE= all pixels i in blockwilumaxorig,i*(xorig, i-xdec,i)2,

where wilumaxorig,iis a weight that is a function of the luma value corresponding to the pixel xorig,i, xorig,i is the original value at location i, and xdec,i is the reconstructed value at location i.

The calculation of the weight is computed as:

yi = 0.015*luma(xorig,i) − 1.5 − 6;

yi = yi < −3 ? −3 : (yi>6 ? 6 : yi);

wi(luma(xorig,i)) = pow(2.0, yi÷3.0);

However, for the testing of sequences in Table 1, an alternative calculation of the weight shall be used and computed as:

yi = 0.03*luma(xorig,i) − 3.0;

yi = yi<0 ? 0 : (yi>12 ? 12 : yi);

wi(luma(xorig,i)) = pow(2.0, yi÷3.0);

In all cases, the metric is calculated individually for a single luma or chroma channel and then used to compute a Bjøntegaard Delta-Rate and Delta-PSNR rate.

deltaE100-based metric

An implementation of the deltaE100 distortion metric is provided in the HDRTools software, and that implementation shall be used for calcuatlion of the metric using the HDRMetric utility and configuration provided with the HDRTools software.

An informative description of the metric follows.

The orignal and test material must first be coverted to linear-light 4:4:4 RGB EXR. For example, if the material is in the YCbCr BT.2100 4:2:0 PQ 10 bit format, it must be converted to a 4:4:4 RGB BT.2100 OpenEXR file using configuration provided with the HDRTools software.

Subsequently, the following steps should be applied for each (R,G,B) sample,within the content to be compared, i.e. original source (content 1) and test material (content 2) :

- Convert the content to the XYZ colour space

- Convert the content from the XYZ to Lab space using the following equations. Computations are performed using double floating point precision.

  - invYn = 1.0 ÷ Yn, with Yn = 100

  - invXn = invYn ÷ 0.95047

  - invZn = invYn ÷ 1.08883

  - yLab = convToLab( y * invYn )

  - L = 116.0 * yLab − 16.0

  - a = 500.0 * (convToLab(x * invXn) − yLab )

  - b = 200.0 * ( yLab − convToLab (z * invZn) )

with convToLab(x) is defined as

if x >= 0.008856

convToLab(x) = x(1÷3)

else

convToLab(x) = 7.78704 * x + 0.137931

- The deltaE100 distance DE between two samples (L1,a1,b1) and (L2,a2,b2) is then computed as follows:

cRef = sqrt( a1 * a1 + b1 * b1 )

cIn  = sqrt( a2 * a2 + b2 * b2 )

cm = (cRef + cIn) ÷ 2.0

g = 0.5 * ( 1.0 − sqrt( cm7.0 ÷ ( cm7.0 + 257.0 ) ) )

aPRef = ( 1.0 + g ) * a1

aPIn  = ( 1.0 + g ) * a2

cPRef = sqrt( aPRef * aPRef + b1 * b1 )

cPIn  = sqrt( aPIn * aPIn + b2 * b2 )

hPRef = arctan( b1, aPRef )

hPIn  = arctan( b2, aPIn )

deltaLp = L1 − L2

deltaCp = cPRef − cPIn

deltaHp = 2.0 * sqrt( cPRef * cPIn ) * sin( (hPRef − hPIn) ÷ 2.0 )

lpm = ( L1 + L2 ) ÷ 2.0

cpm = ( cPRef + cPIn ) ÷ 2.0

hpm = ( hPRef + hPIn ) ÷ 2.0

rC = 2.0 * sqrt( cpm7.0 ÷ ( cpm7.0 + 257.0 ) )

dTheta = DEG30 * exp(−((hpm − DEG275) ÷ DEG25) * ((hpm − DEG275) ÷ DEG25))

rT = −sin( 2.0 * dTheta ) * rC

t = 1.0 − 0.17 * cos(hpm − DEG30) + 0.24 * cos(2.0 * hpm) +

0.32 * cos(3.0 * hpm + DEG6) − 0.20 * cos(4.0 * hpm − DEG63)

sH = 1.0 + ( 0.015 * cpm * t )

sC = 1.0 + ( 0.045 * cpm )

sL = 1.0 + ( 0.015 * (lpm − 50) * (lpm-50) ÷ sqrt(20 + (lpm − 50) * (lpm − 50)) )

deltaLpSL = deltaLp ÷ sL

deltaCpSC = deltaCp ÷ sC

deltaHpSH = deltaHp ÷ sH

deltaE100 = sqrt( deltaLpSL * deltaLpSL + deltaCpSC * deltaCpSC +

deltaHpSH * deltaHpSH + rT * deltaCpSC * deltaHpSH )

with

DEG275 = 4.7996554429844

DEG30 = 0.523598775598299

DEG6 = 0.1047197551196598

DEG63 = 1.099557428756428

DEG25 = 0.436332

After this process the deltaE100 values for each frame are averaged within the “Distortion” specified window. Finally, a PSNR based value is derived as:

PSNR_DE = 10 * log10( PeakValue ÷ deltaE100 )

where PeakValue is set to 10,000.

PSNR-L100

An implementation of the PSNR-L100 distortion metric is provided in the HDRTools software, and that implementation shall be used for calculation of the metric using the HDRMetric utility and configuration provided with the HDRTools software.

An informative description of the metric follows:

PSNR-L100 represents the distortion in the lightness domain of the CIELab colour space. The derivation of Lab values from the linear representation of the signal is similar as given in the description of deltaE100. The mean absolute error (MAE) in the L domain is used to compute the PSNR-L100 as follows:

PSNR-L100 = 10 * log10( PeakValue ÷ MAE )

where PeakValue is set to 10,000.