| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J1024r2 |
| --- | --- |

| Title: | Description of CE4: Inter prediction and motion vector coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Haitao Yang Shan Liu Kai Zhang | Email: | haitao.yang@huawei.com shanl@tencent.com zhangkai.video@bytedance.com |
| Source: | CE coordinators |  |  |

_____________________________

Introduction

The goal of this Core Experiment (CE) is to investigate various inter prediction and motion vector coding techniques in the Test Model and the Benchmark Set (BMS). It comprises 7 categories, 1) affine motion compensation, 2) merge mode enhancement, 3) motion vector coding, 4) generalized bi-prediction, 5) reference picture boundary padding, 6) local illumination compensation, and 7) interpolation filter improvement.

Coding tools studied different categories in this CE covers various topics and may have interaction with each other. In addition, interaction with coding tools in CE9 on decoder side MV derivation, CE10 on combined and multi-hypothesis prediction, and CE11 on combined and multi-hypothesis prediction. The interaction of related coding tools in terms of compression performance and the joint design will also be studied.

CE4.1: Affine motion compensation

Summary

| Proposal | Affine model | Affine MC | Affine MVP |
| --- | --- | --- | --- |
| JEM | 4-param model | 1/16 8-tap DCT-IF for luma; 1/32 4-tap DCT-IF for chroma Adaptive sub-block size (min 4x4) Refine sub-block boundary by OBMC | Candidate derivation MV duplet from spatial neighboring MVs Candidate ordering Consistency checking of each candidate to an affine model Number of candidates : 2 |
| J0012 (Ericsson and Nokia) | Same as JEM | Flexing Affine (replace OBMC to refine sub-block boundary) | Same as JEM |
| J0017 (LGE) | 1) 4-para/6-para model switching at CU level 2) Conditional affine | Same as JEM | Candidate derivation MV duplet/triplet from spatial neighboring affine model |
| J0018 (MediaTek) | Same as JEM | Same as JEM | Candidate derivation (1) MV duplet from spatial neighboring affine models (2) MV duplet from spatial neighboring MVs |
| J0021 (Qualcomm & Technicolor) | 4-para/6-para model switching at CU level and slice level | Same as JEM | Candidate derivation (1) MV duplet/triplet from spatial neighboring affine models (2) MV duplet/triplet from spatial neighboring MVs Others MVD of one CP is used to refine MVP of another CP |
| J0022 (Technicolor & Qualcomm) | 4-para/6-para model switching at CU level and slice level | Same as JEM | Same as J0021 Candidate derivation (1) Added spatial affine neighbors and temporal affine candidates in Merge (2) Template Affine MVP (Merge and AMVP) |
| J0024 (Samsung, Huawei, Hisilicon, GoPro) | 4-para/6-para model switching at CU level | bi-linear filter / DCT-IF switching at CU level | Candidate derivation (1) MV duplet/triplet from spatial neighboring affine models (2) MV duplet/triplet from spatial neighboring MVs Candidate ordering Consistency checking of each candidate to an affine model, MV duplet/triplet from spatial neighboring affine models is put in the front of the list. Number of candidates : 4 Others Optional MVD coding in each direction. |
| J0028 (Sony) | 3-para/4-para model switching at CU level | Sub-block size could be chosen such as 4x4 or 8x8 to limit complexity | Basically, same as JEM but the MV predictor predicts 3 parameters |

Technical description

Affine motion compensation in JEM

Please refer to JVET-G1001 for details.

Affine flexing motion compensation in J0012 (Nokia)

This test is set up to verify performance of the affine motion compensation approach proposed in JVET-J0012. The approach performs block-based motion compensation followed by a resampling or "flexing" filtering of the prediction block. The original proposal is based on 4x4 luma block motion compensation as in JEM 7.0. In addition, a version with 8x8 luma block compensation is tested. The test will be using the 16-position JEM motion compensation filter for the flexing operation, but may also provide further results using the 64-position KTA interpolation filter.

The affine flexing tool calculates the horizontal and vertical motion vector differences on the affine sub-PU boundaries. Based on whether the nature of motion is more rotational or zooming, the method decides to apply either lateral compensation (moving the sub-PU boundary samples laterally with respect to the neighboring sub-PU samples) or perpendicular compensation (moving the sub-PU boundary samples away or closer to the samples of the neighboring sub-PU), respectively. The remapping offsets applied to different lines are calculated based on motion vector difference and distance from the sub-PU border. A look-up table implementation is provided in the CE software. An example of lateral processing is illustrated in the figure below.

Figure: Example of horizontal remapping of samples in a prediction unit in the case of 4x4 sub-PUs.

Further details:

- Flexing is applied to both horizontally and vertically, and the result of the flexing in first dimension is used as an input to the second dimension

- Flexing is applied only for the luma channel in the case of 4:2:0 video

- Line ends are padded with the last available samples on the line

- OBMC, if available, is switched off for affine blocks

Expected results with respect to the test model: over 2 % gain on CfP sequences, significantly higher on sequences with affine like motion characteristics, results of 8x8 approach close to that of 4x4 based, run-time similar to JEM affine implementation.

Affine Motion Vector Prediction in J0017 (LGE)

In JVET-J0017, affine inter mode constructs up to two candidates from the affine motion models of neighboring affine coded blocks (inherited affine candidate). If the number of candidates is less than two, JEM affine process is performed until the number of candidate is two.

Adaptive four and six parameter motion model in J0017 (LGE)

In JVET-J0017, adaptive four and six parameter motion model is used when there are the neighboring affine coded blocks. Otherwise, only four parameter model is used to perform the affine prediction.

Affine inter mode in J0018 (MediaTek)

In J0018, a four-parameter (two-control-point) affine inter mode is used to predict the content with the motion model of zoom-in/out and rotation. If the affine prediction is applied, the syntax of inter_dir, ref_idx, mvp_index, and two MVDs (mvd_x and mvd_y) are signaled. An affine MVP pair candidate list containing two affine MVP pairs is generated. The signaled mvp_index is used to select one of them. The affine MVP pair is generated by two kinds of affine MVP candidates. One is the spatial inherited affine candidate, and the other is the corner derived affine candidate. If the neighboring CUs are coded with affine mode, the spatial inherited affine candidates can be generated. The affine motion model of the neighboring affine coded block is used to generate the motion vectors of the two-control-point MVP pair. The MVs of the two-control-point MVP pair of the spatial inherited affine candidate are derived by using the following equations.

V0x = VB0x + (VB2_x - VB0x ) * ( posCurCU_Y - posRefCU_Y ) / RefCU_height
+ (VB1x - VB0x ) * (posCurCU_X - posRefCU_X) / RefCU_width

(3)

V0y = VB0y + (VB2_y - VB0y ) * (posCurCU_Y - posRefCU_Y) / RefCU_height
+ (VB1y - VB0y ) * (posCurCU_X - posRefCU_X) / RefCU_width    ,

(4)

where VB0, VB1, and VB2 can be replaced by the top-left MV, top-right MV, and bottom-left MV of any reference/neighboring CU, (posCurCU_X, posCurCU_Y) is the position of the top-left sample of the current CU relative to the top-left sample of the frame, (posRefCU_X, posRefCU_Y) is the position of the top-left sample of the reference/neighboring CU relative to the top-left sample of the frame.

V1x = VB0x + (VB1x - VB0x) * CU_width / RefCU_width

(5)

V1y = VB0y + (VB1y - VB0y) * CU_width / RefCU_width

(6)

If the number of the MVP pairs is smaller than 2, the corner derived affine candidate is used. The neighboring motion vectors, as shown in Figure below, are used to derive the affine MVP pair. For the first corner derived affine candidate, the first available MV in set A (A0, A1, and A2) and first available MV in set B (B0 and B1) are used to construct the first MVP pair. For the second corner derived affine candidate, the first available MV in set A and first available MV in set C (C0 and C1) are used to calculate the MV of top-right control point. The first available MV in set A and the calculated top-right control point MV are the second MVP pair.

Adaptive affine model selection in J0021 (Qualcomm, Technicolor)

4-parameter affine model with two control points, and an additional 6-parameter affine model with three control point MVs, are used. The affine models are adaptively selected at both slice level and block level. Slice header flags affine4_flag and affine6_flag indicate whether the 4-parameter affine and 6-parameter affine model are applied. If one of them is equal to 1, affine_flag signaled at block level indicates whether the allowed affine model is used for this block. If both are equal to 1, there is an affine_type flag signaled at block level. If affine_type is equal to 1, the 6-parameter affine model is used, otherwise the 4-parameter affine model is applied.

Affine motion vector prediction in J0021 (Qualcomm, Technicolor)

- Simplified Affine MVP

  - Extrapolated affine MVP

Neighboring blocks A0, A1, B0, B1, and B2 are checked. If the neighboring block is coded using affine motion model and its reference frame is same as the reference frame of the current block, MVs at two (for 4-parameter affine model) or three (for 6-parameter affine model) control points of the current block are derived from the affine model of this neighbor.

The neighboring MVs are divided into three groups: S0=mvA,mvB,mvC , S1=mvD,mvE and S2=mvF,mvG. mv0  is the first MV in S0 that refers to the same reference picture as the current block; mv1 is the first MV in S1 that refers to the same reference picture of the current block; and mv2 is the first in S2 that refers to the same reference picture of the current block.

- If only mv0 and mv1 can be found, mv2 is derived as:

mv2x=mv0x-hmv1y-mv0yw, mv2y=mv0y+hmv1x-mv0xw,

where the current block size is W×H.

- If only mv0 and mv2 can be found, mv1 is derived as:

mv1x=mv0x+hmv2y-mv0yw, mv1y=mv0y-hmv2x-mv0xw.

{mv0, mv1} serves as a MVP candidate for the 4-parameter affine model and {mv0, mv1, mv2} serves as a MVP candidate for the 6-parameter affine model

- Mvd prediction

Given motion vector difference vectors, mvd0, mvd1, mvd2, the control points are calculated:

mv0=mv0+mvd0

mv1=mv1+mvd1+mvd0

mv2=mv2+mvd2+mvd0  (for the 6-parameter model only)

Affine motion vector prediction adaptations in J0022 (Technicolor, Qualcomm)

Table 1. Merge mode characteristics.

|  | JEM | Proposed |
| --- | --- | --- |
| Motion information predictor composition | - inherited reference picture index - inherited associated MV - inherited associated IC flag | - inherited reference picture index - inherited associated MV (triplet) - inherited associated IC flag - inferred affine flag |
| Candidate list | 7 candidates, among: - spatial positions in {a1, b1, b0, a0} - ATMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - STMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - spatial positions in {b2} - a temporal position {c} - bi-directional combinations - zero MV for each reference picture | 19 candidates with up to 8 Affine ones, among: - spatial positions in {a1, b1, b0, a0} - spatial affine positions in {a1, b1, b0, a0} - ATMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - STMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - constructed affine candidate based on spatial non-affine positions - spatial positions in {b2} - spatial affine positions in {b2, a2, b3} - a temporal position {c} - temporal affine positions in {curr, c} - non-adjacent spatial positions - non-adjacent spatial affine positions - bi-directional combinations - zero MV for each reference picture |
| Signaling | For each reference picture: - predictor idx Residual is coded if mode != skip | For each reference picture: - predictor idx Residual is coded if mode != skip |

Affine model storage

In the JEM, the affine model seeds are stored in the top-left, top-right and bottom-left 4x4 sub-blocks in the considered CU. In the proposed solution, the affine model seeds are stored separately as a motion information associated to the whole CU. The motion model is thus decoupled from the motion vectors used for actual motion compensation at the 4x4 block level. This storage allows preserving the complete motion vector field at the 4x4 sub-block level. It also allows using affine motion compensation for block of size 4 in width or height.

Affine candidate derivation

Figure 1. Positions of spatial candidates of motion information.

In the merge process described in J0021, a candidate list is created as presented in Table 1. The list contains classical merge candidates as well as affine ones. In the proposed solution, the affine candidates are enriched with new ones. The spatial affine candidates set in JEM {a1, b1, a0, b0, b2} becomes {a1, b1, b0, a0, b2, a2, b3} in this contribution.

Furthermore, affine does not benefit from the temporal candidates introduced in the proposed solution. To that aim, every available reference picture of each list is explored at the current CU position {curr} and at the bottom-right position {c} to get affine model seeds. When a temporal candidate was coded with an affine mode, its associated affine model seeds are retrieved. If the corresponding temporal reference picture is also available for the current CU in same reference picture list, then the current CU inherits the reference picture of the temporal candidate. Otherwise the first reference picture in the considered list is chosen for current CU. As for classical temporal candidates, the retrieved affine model seeds are scaled according to the temporal distances between pictures and their reference pictures. The same process as for spatial candidates is applied, i.e. the current affine model seeds are derived from the scaled temporal candidate seeds according to the following equations (also used in JEM).

  - v0=v3+v5-v3Ycurr-YneighbHneighb+v4-v3Xcurr-XneighbWneighb

  - v1=v0+(v4-v3)WcurrWneighb

  - v2=v0+(v5-v3)HcurrHneighb

Equation 1. Affine model seeds derivation process from CU to CU.

Template affine derivation

Template affine is a new coding mode introduced in the proposed solution compared to JEM and J0021. The tool is close to template matching but dedicated to affine. It allows automatically finding an affine model for the currently processed CU and refining it, even if surrounding CUs are not in affine mode.

Template affine mode can only be used by CUs whose left and top neighbors are both available. The process is as follows.

- First, by using the template matching process, for each reference picture list, a motion vector with its associated reference picture is derived and refined automatically.

- Then a process close to the merge candidate list collection, but dedicated to affine, allows deriving affine model seeds for the current CU using Equation 1. The first spatial affine neighbor found in {a1, b1, b0, a0, b2, a2, b3} is chosen.

- The last step consists in a template-based refinement process of the derived affine seeds. For each reference picture list, if an affine model was found in the second step, it is set as the initial affine model to be refined for the current CU. Otherwise the template matching motion vector found in the first step is used as the initial affine model. The template used in this mode is a strip of 4 pixels on the top and left of the current CU including the top-left corner as shown in Figure 19. Then the affine model of the current template is derived with Equation 1, based on the model of the current CU. The obtained affine model seeds allow calculating the motion vectors of each 4x4 sub-block of the template and so getting the corresponding template in the reference picture as represented on Figure 2. The template matching cost associated to this affine model is the SAD between the template in the reference picture and the template around current CU. The refinement process of the affine model seeds is performed with a gradient descent search. This search is similar to the one performed in the motion estimation of the affine AMVP mode, but is based on the template instead of the current CU. It calculates, at each step, deltas to be added to each seed of the affine model. The refinement is stopped after 5 iterations, and the affine model with the smallest template matching cost is selected as the best one.

Figure 2. Templates used in Template Affine mode.

- As in template matching, if both reference picture lists are used, then a joint refinement between lists is performed. The list whose affine model reaches the smallest template matching cost is selected as the best one. The affine model in the other list is the one to be jointly refined. To that aim, the current template is modified using the best template and becomes:

T'curr=2* Tcurr- Tbest

The affine model of the second list is then further refined with 5 iterations using this new template. If the template matching cost associated to this refined affine model is not sufficiently low, then the motion is set unavailable for this list.

Template affine is used in affine AMVP mode to derive a candidate while template matching is used in AMVP mode. A candidate can be derived only if there are less than 2 candidates already found in the spatial neighborhood. The derived candidate is inserted in the top of the affine AMVP candidate list.

It is also used in merge as a separate coding mode, with a similar process as for the template matching in the JEM. If the decoded template merge mode indicates the use of template affine mode, then a best refined affine model is derived per list and used to reconstruct the CU.

Spatial propagation of affine seeds is applied between CUs, regardless they are coded in affine merge mode or template affine mode.

Affine inter prediction and Affine MVD coding in J0024 (Samsung, Huawei, Hisilicon, GoPro)

When merge/skip flag is false, and both width and height for the CU are larger than or equal to 8, an affine flag in CU level is signalled in the bitstream to indicate whether affine inter mode is used. And when the CU is coded as affine inter mode, a model flag is signalled for specifying whether 4-parameter or 6-parameter affine model is used for this CU. If the model flag is true, AF_6_INTER mode (6-parameter affine model) is applied and 3 MVDs will be parsed; otherwise, AF_6_INTER mode (4-parameter affine model) is applied and 2 MVDs will be parsed.

In AF_4_INTER mode, similar to affine merge mode, affine motion vector pairs extrapolated from neighbour blocks coded with affine mode are constructed and insert into candidate list firstly.

After that, if the size of the candidate list is smaller than 4, candidates with motion vector pair v0,v1|v0={vA,vB,vc},v1={vD, vE} is constructed using the neighbour blocks. As shown in Figure 3-27, v0 is selected from the motion vectors of the block A, B or C. The motion vector from the neighbour block is scaled according to the reference list and the relationship among the POC of the reference for the neighbour block, the POC of the reference for the current CU and the POC of the current CU. And the approach to select v1 from the neighbour block D and E is similar. When the candidate list is larger than 4, the candidates are firstly sorted according to the consistency of the neighbouring motion vectors (similarity of the two motion vectors in a pair candidate) and only the first four candidates are kept.

If the number of candidate list is smaller than 4, the list is padded by the motion vector pair composed by duplicating each of the AMVP candidates.

In AF_6_INTER mode, similar to affine merge mode, affine motion vector triples extrapolated from neighbour blocks coded with affine mode are constructed and insert into candidate list firstly.

After that, if the size of the candidate list is smaller than 4, candidates with motion vector triples v0,v1,v2  |v0={vA,vB,vc},v1={vD, vE},v2={vG, vH} is constructed using the neighbour blocks. As shown in Figure 3-27, v0 is selected from the motion vectors of the block A, B or C. The motion vector from the neighbour block is scaled according to the reference list and the relationship among the POC of the reference for the neighbour block, the POC of the reference for the current CU and the POC of the current CU. And the approach to select v1 from the neighbour block D and E, and select v2 from F and G is similar. When the candidate list is larger than 4, the candidates are firstly sorted according to the consistency of the neighbouring motion vectors (similarity of the two motion vectors in a triple candidate) and only the first four candidates are kept.

If the number of candidate list is smaller than 4, the list is padded by the motion vector triple composed by duplicating each of the AMVP candidates.

Affine motion compensation in J0024 (Huawei, Samsung, Hisilicon, GoPro)

According to the number of affine parameters, the MVF of the affine CU is generated according to Equation 4 for 4-parameter affine model, and according to Equation 5 for 6-parameters affine model.

vx=v1x-v0xWx-v1y-v0yWy+v0xvy=v1y-v0yWx+v1x-v0xWy+v0y						(4)

vx=v1x-v0xWx+v2x-v0xHy+v0xvy=v1y-v0yWx+v2y-v0yHy+v0y						(5)

The sub-block size M×N is derived as in Equation 6, where MvPre is the motion vector fraction accuracy (1/16 in JEM).

M=clip3(4,w,w×MvPremaxabsv1x-v0x,absv1y-v0y)N=clip3(4,h,h×MvPremaxabsv2x-v0x,absv2y-v0y)	            (6)

After derived by Equation 6, M and N should be adjusted downward if necessary to make it a divisor of w and h, respectively.

If M or N is smaller than 8, Enhanced interpolation filter (EIF) is applied; otherwise, sub-block based affine motion compensation is applied.

To derive motion vector of each M×N sub-block, the motion vector of the center sample of each sub-block, as shown in Figure above, is calculated according to Equation 4 or Equation 5, and rounded to 1/16 fraction accuracy. Then the SHVC upsampling interpolation filters are applied to generate the prediction of each sub-block with derived motion vector.

SHVC upsampling interpolation filters, which have same filter length and normalization factor as HEVC motion compensation interpolation filters, are used as motion compensation interpolation filters for the additional fractional pel positions. The chroma component motion vector accuracy is 1/32 sample in the JEM, the additional interpolation filters of 1/32 pel fractional positions are derived by using the average of the filters of the two neighboring 1/16 pel fractional positions.

Enhanced bi-linear Interpolation Filter (EIF) is used for non-translational (affine) motion compensation. In EIF bilinear interpolation coupled with 3-tap high-pass filter ({-1,10, -1} with normalization factor 1/8) is used for inter prediction of both Luma and Chroma component. The bilinear interpolation is applied to interpolate all fractional offsets on pixel-level. High-pass filter applied after bilinear interpolation. The coefficients of the filters are fixed, the same filter is used for all predicted samples involved in non-translational (affine) motion compensation model excluding cases when both width and height of the sub-block unit are larger than 4 luma samples.

Sony’s proposal in CfP (JVET-J0028)

Motion model extension

In JEM software, the simplified affine transform is used in motion compensation. It can improve the quality of inter prediction but it increases the overhead as a motion information. To reduce the overhead and find better trade-off, this technique adds two motion modes which are a rotation and a scaling.

An encoder can choose four modes which are a conventional translate, scaling, rotation and Affine. A number of parameters are 2 (one motion vector), 3, 3 and 4 (two MVs) for translate mode, scaling mode, rotation mode and Affine mode.

As shown in the below figure, the motion model index is defined and it is sent to the decoder. The number in parenthesises shows the number of parameters of differential motion vector that are sent to a decoder.

Optionally change sub-block size

In these scaling, rotation and Affine mode, the inter prediction process is divided to sub-blocks. If smaller sub-blocks are used, it can improve the accuracy but the complexity will be larger. To have flexibility of the trade-off, this technique allows the sub-block size to change among 2x2, 4x4 or 8x8. The control index is sent in sequence parameter set.

Scaling mode

As mentioned above, a scaling mode is added in this technique. If the test sequence has zoom in or zoom out, the inter prediction needs a scaling translate. This scaling mode can predict the zoom in/out and it can reduce the number of parameters of Affine mode.

When the scaling mode is chosen, three parameters are used to predict as shown in the below figure. It is one motion vector (v0x, v0y) and one element (v1x). For the prediction process, the frame work is used the same as for Affine transform in JEM software. The parameter (v1y) is implicitly assumed as the same as (v0y).

Rotation mode

As mentioned above, a rotation mode is added in this technique. If the motion in a test sequence has rotation, the inter prediction can use a rotational translate. This rotation mode can predict the rotation and it can reduce the number of parameters than affine mode.

When the rotation mode is chosen, three parameters are used to predict as shown in the below figure. It is one motion vector (v0x, v0y) and one element (v1y). For the prediction process, the frame work is used the same as for Affine transform in JEM software. The parameter (v1x) is implicitly calculated by below equation. Here, W is width in CU size.
v1x=v0x-v1y-v0y22W

Test specification

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.1.1 | BMS a) BMS Affine with 4x4 sub-block only b) BMS Affine with 8x8 sub-block only | Huanbang Chen (Huawei) | Tzu-Der Chuang (MediaTek) |
| 4.1.2 | J0012 a) Affine flexing using sub-block size of 4x4 b) Affine flexing using sub-block size of 8x8 | Jani Lainema (Nokia) | Solovyev Timofey (Huawei) |
| 4.1.3 | J0021 a) Affine MVP construction b) a + MVD prediction c) b + CU level 4-para/6-para switching d) c + slice level 4-para/6-para switching | Yu Han (Qualcomm) | Huanbang Chen (Huawei) |
| 4.1.4 | J0022 a) Affine model seed storage b) Affine MVP construction with added spatial and temporal c) Template affine derivation for MVP construction d) a + b + c | A. Robert (Technicolor) | Huanbang Chen (Huawei) |
| 4.1.5 | J0024 a) Affine MVP construction b) a) + MVD coding c) b) + 4 para/6-para switching d) EIF on top of BMS Affine e) EIF on top c) | Huanbang Chen (Huawei) | Fangdong Chen (Hikvision) |
| 4.1.6 | J0018 Affine MVP construction | Tzu-Der Chuang (MediaTek) | Guichun Li (Tencent) |
| 4.1.7 | J0017 a) Up to two affine candidate for affine inter b) Four and six parameter model c) Four and six parameter model with the conditional affine parameter signaling | Jaeho Lee (LGE) | K. Kondo (Sony) |
| 4.1.8 | J0028 Affine model extension (additional 2 3-param models) a) 3-para (scaling) / 4-para switching b) 3-para (rotation) / 4-para switching c) a + b | K. Kondo (Sony) | P. Bordes (Technicolor) Anish Tamse (Samsung) |

Description of encoder operations is required if a dedicated algorithm is used in the proposed tool for, e.g., motion estimation, mode decision, etc.

Test condition and evaluation criteria

| Test# | Tool on case 1 (VTM as reference) | Tool off case 1 (BMS as reference) | Tool on case 2 (VTM+ToolX as reference) | Tool off case 2 (BMS-ToolX as reference) |
| --- | --- | --- | --- | --- |
|  | BD-Rate/enc./dec. | BD-Rate/enc./dec. | BD-Rate/enc./dec. | BD-Rate/enc./dec. |

For all tests in this category, “Tool on case 1” and “Tool off case 2” are mandatory, where ToolX is affine motion in BMS.

For complexity evaluation, in addition to encoding and decoding time, additional computational complexity brought by a proposed technique should be analyzed.

The suggested granularity for temporal motion vector storage is 16x16 in tool on test, and 4x4 for tool off test. It should be explicitly described if a different granularity is applied.

Subjective quality evaluation is encouraged if a technique is claimed to be able to improve it.

CE4.2: Merge mode enhancement

Summary

| Proposal | Candidate derivation | Candidate list construction |
| --- | --- | --- |
| JEM | Translational motion candidate Spatial Candidates, ATMVP, STMVP, B2, TMVP, combined bi, zero, Affine motion candidate From neighboring affine blocks | Translational motion candidate list Max candidate number is 7 Affine motion candidate list Separate list with a single candidate |
| J0012 (Ericsson and Nokia) | All JEM candidates Non-adjacent spatial candidates before TMVP candidate | Single merge list, max candidate number 7 |
| J0014 (HHI) | All JEM candidates A set of uni-directional candidate | JEM list Alternative uni-directional candidate list |
| J0015 (InterDigital, Dolby) | 4x4/8x8 sub-block size switching at slice level for the two sub-block candidates Fixed collocated picture for ATMVP derivation | Same as JEM |
| J0017 (LGE) | All JEM candidates Affine candidates derived from affine model of neighboring blocks Fixed collocated picture for ATMVP | TM based candidate reordering for JEM candidate list Alternative affine candidate list with two candidates |
| J0018 (MediaTek) | All JEM candidates Affine candidates derived from affine model of neighboring blocks Affine candidates derived from motion vectors of neighboring blocks | A unified merge list with all kinds of candidates, max candidate number 10 |
| J0021 (Qualcomm & Technicolor) | All JEM candidates Non-adjacent spatial / affine candidates Affine candidates derived from affine model of neighboring blocks Affine candidates derived from motion vectors of neighboring blocks | A unified merge list with all kinds of candidates, max candidate number 19 |
| J0022 (Technicolor & Qualcomm) | All J0021 candidates Add spatial and temporal affine candidates | A unified merge list with all kinds of candidates, max candidate number 19 Template affine MVP |
| J0024 (Samsung&Huawei, etc.) | All JEM candidates without sub-block ones Progressive motion candidates Affine candidates derived from affine model of neighboring blocks Affine candidates derived from motion vectors of neighboring blocks | JEM list is extended to support a set of (max 96) progressive motion candidates A separate affine candidate list with 5 candidates |
| J0032 (USTC, etc.) | All JEM candidates Affine candidates derived from affine model of neighboring blocks Affine candidates derived from motion vectors of neighboring blocks | JEM list Alternative affine list with a single affine candidate from the affine model of the largest neighboring block Alternative affine list with the rest of the all affine candidate |
| J0058 (Tencent) | Additional merge candidates | Additional Merge candidates, the number of maximum number of merge candidates can be further reduced. |
| J0059 (HiSilicon) | All JEM candidates Extended spatial merge candidates Merge candidate offsets Combined average merge candidates TM based candidates reorder | Increase the number of merge list to 13. TM based reorder |
| J0061 (HiSilicon) | Additional planar MV prediction |  |

Technical description

Subblock merge candidate in BMS

Please refer to ATMVP merge candidate in JVET-G1001 for details.

Motion vector look back in J0012 (Ericsson, Nokia)

The contribution proposes to derive additional spatial candidates from two non-adjacent neighboring positions. One from the closest non-neighboring block in straight horizontal spatial distance and the other one from the closest non-neighboring block in straight vertical spatial distance. The blocks are limited within a maximum distance of 1 CTU to the current block. The fetching process of non-adjacent neighboring candidates starts with tracing the previous decoded blocks in the vertical direction. The vertical inverse tracing stops when an inter block is encountered or the traced back distance reaches 1 CTU size. The fetching process then traces the previous decoded blocks in the horizontal direction. The criterion for stopping the horizontal fetching process depends on whether there is a vertical non-adjacent candidate successfully being fetched or not. If no vertical non-adjacent candidate is fetched, the horizontal fetching process stops when an inter is encountered or the traced back distance exceed one CTU size threshold. If there is a vertical non-adjacent candidate fetched, then the horizontal fetching process stops when an inter block which contains different MV from the vertical non-adjacent candidate is encountered or the traced back distance exceed on CTU size threshold.

The fetched non-adjacent neighboring candidates are added before the TMVP candidate in the merge candidate list when the number of already added spatial candidates is less than a threshold.

Restricted merge in J0014 (HHI)

In this test, a restricted merge mode is to be studied, which operates as follows: In B slices, the merging candidate list can be modified such that it only contains merging candidates using uni prediction.  The application of this “restricted merge mode” as well as the designated prediction list are indicated via a syntax element merge_ref_pic_list_idc, which can take the values LIST_0, LIST_1, and LIST_NONE.  When merge_ref_pic_list_idc != LIST_NONE, the merging candidate list modification process proceeds as specified by the following ordered steps:

- All merging candidates using uni prediction are removed from the merging candidate list.

- For the remaining bi-prediction merging candidates, the prediction list utilization flags are modified, such that the designated prediction list is used (e.g., for merge_ref_pic_list_idc == LIST_0, predFlagL1N is set to 0, with N taking all bi prediction merging candidates). Furthermore, all potentially available additional multi hypothesis prediction parameters are removed from these remaining merging candidates.

- Possibly empty entries at the end of the merging candidate list (due to the removal in step 1) are filled with zero motion vector entries having the appropriate prediction list utilization flag and iterating over all reference frames of this prediction list.

Sub-block merge mode simplification in J0015 (InterDigital, Dolby)

In this test, simplification methods are to be studied for reducing both worst-case and average complexity of the advanced temporal motion vector prediction (ATMVP) in the BMS.

- Test 1: Simplified ATMVP with one fixed collocated picture

In the current ATMVP design of the BMS, all the reference pictures in L0 and L1 can be used as the collocated picture to derive the motion vectors of the CUs that are coded by the ATMVP mode. In this test, a simplified design is proposed to use the same collocated picture as in HEVC (signaled at the slice header) as the collocated picture for ATMVP. At the block level, if the reference picture of a neighboring block A is different from the collocated picture, the MV of block A is scaled using the HEVC temporal MV scaling method, and the scaled MV of block A is used to locate the block in the collocated picture for ATMVP-based motion derivation.  Denote the motion vector used to fetch the motion field in the collocated picture Rcol as MVcol. To minimize the impact due to MV scaling, the MV in the spatial candidate list used to derive MVcol is selected in the following way. If the reference picture of a candidate MV is the collocated picture, this MV is selected and used as MVcol without any scaling. Otherwise, the MV having a reference picture closest to the collocated picture is selected to derive MVcol with scaling.

- Test 2: picture/slice-level adaptation of ATMVP sub-block-size

The picture/slice-level adaptation of the granularity of the ATMVP-based motion derivation is studied in this test. Specifically, one default sub-block size that is used to derive the ATMVP motion vectors is signaled at sequence level. Additionally, one flag is signaled in picture/slice level to indicate if the default sub-block size is used for the current slice. If the flag is false, the specific ATMVP sub-block size is further signaled in bit-stream for the picture/slice. Additionally, a simple encoder method is used to decide the ATMVP sub-block size for the current picture/slice based on the average size of the ATMVP CUs from the last coded picture in the same temporal layer.

Test 3: Test 1 + Test 2

Adaptive affine candidate selection for Affine merge in J0017 (LGE)

In JVET-J0017, affine merge mode uses up to two candidates to generate affine merge candidates. Affine merge mode is skipped when there is no neighboring affine coded block. Affine merge index is signaled only when the available neighboring affine coded block is more than one.

Modified ATMVP in J0017 (LGE)

Related test in subclause3.2.4.

- Test 1: Simplified ATMVP with one fixed collocated picture

The MV in spatial candidate list used to derive MVcol is selected in the following way: If the reference picture of a candidate MV is the collocated picture, this MV is used as MVcol. Otherwise, zero MV corresponding with collocated picture is used as MVcol.

- Test 2: Test 1+ InterDigital’s Test 2 (picture/slice-level adaptation of ATMVP sub-block-size in J0015)

Unified merge modes in J0018 (MediaTek)

Several merge candidate types are proposed, including affine merge candidate, middle spatial candidates, average candidates, and double ATMVP candidates with pruning.

The affine merge mode is integrated within the normal merge mode, and the affine merge candidates are added into the merge candidate list. If the affine merge candidate is selected, the affine prediction is applied to current CU. Two kinds of affine merge candidates are added, which include spatial inherited affine candidates and corner derived affine candidates. Spatial inherited affine candidates insert spatial candidates which derived from the neighboring affine coded blocks, for example the block A1, B1, B0, A0, B2, T, and L. Corner derived affine candidates simply set three spatial candidates as control points, for example, the MV sets of block {(B2, A1, B1), (B2, A0, B1), (B2, A1, B0), (B2, A0, B0)}. Available motion triplets of spatial neighbors with the same reference frame are inserted.

Middle spatial candidates fetch motion information from center-left (block L) and center-top (block T) blocks of the current CU. Temporal candidates fetch motion information from temporal collocated blocks of block RB, CT, RT, and LB.

Pairwise-average candidates average two different candidates in the merge candidate list and the averaged motion vectors is calculated separately for each reference picture lists. Pairwise-average candidates could apply motion vector scaling before averaging if reference pictures of two motion vectors are different.

Double ATMVP candidates derive two ATMVP candidates by two different temporal vectors. For ATMVP decoder-side speedup, ATMVP only derives all sub-blocks motion when current merge candidate is ATMVP. For ATMVP uniformity check, ATMVP compares motions at partial sub-blocks, e.g, top-left, top-right, bottom-left, and bottom-right blocks in current CU, are all the same or not at merge candidate list construction. If these sub-blocks are all the same, treat this ATMVP candidate like a normal merge candidate.

The maximum merge candidate number is 10.

Merge/Skip modes in J0021 (Qualcomm, Technicolor)

- Merge candidate list reordering

The merge candidate list is reordering. Certain kinds of candidates such as bi-predicted candidates are put forward.

- Non-adjacent neighbouring merge candidate

The contribution proposes to derive additional spatial merge candidates from non-adjacent neighboring positions which are marked as 6 to 49 in Figure 4.

The derived candidates are added after TMVP candidates in the merge candidate list.

- Unified affine/normal merge candidate list (Qualcomm, Technicolor)

Unlike in JEM there is no separate merge mode for affine. A merge candidate in the merge candidate list can be an affine merge candidate or a normal merge candidate. For each adjacent/non-adjacent spatial neighboring block, if the it is coded using affine motion model, MVs at two or control points of the current block are derived from the affine model of this neighbor.

In addition, a virtual affine merge candidate constructed as combination of motion vectors from spatial neighboring blocks is inserted into the list. This candidate is constructed in the same way as the virtual affine MVP for the affine motion vector prediction described in the section “Affine motion vector prediction in J0021 (Qualcomm, Technicolor)”, except that the target reference picture is not known beforehand

Added affine candidates in unified affine merge mode in J0022 (Technicolor, Qualcomm)

J0022 proposes a modified version of the Merge/Skip mode proposed in J0021 (3.2.8).

The affine model seeds are stored at the CU level as presented in 2.2.8.1 allowing merge mode to be applied on CU of size 4 in width or height.

Two further spatial affine positions are explored to retrieve affine model for the current CU. These neighbors are {a2, b3} as presented on Figure 1 and associated derived affine model, if exist, are inserted after the {b2} affine candidate.

Furthermore, several temporal affine candidates are also added to the merge candidate list. The derivation of such temporal affine models that are get from {curr, c} positions is described in 2.2.8.2. These temporal affine candidates are inserted after the classical temporal candidate.

The complete list of merge candidates (normal/affine) is presented in Table 1.

Ultimate motion vector expression in J0024 (Samsung, Huawei, Hisilicon, GoPro)

Ultimate motion vector expression (UMVE) in J0024 can be either skip or direct(or merge) modes, which use proposed motion vector expression method with neighboring motion information. As the skip and the merge modes in HEVC, UMVE also makes a candidate list from neighboring motion information. Among those candidates in the list, a MV candidate is selected, and is further expanded by new motion vector expression method.

UMVE provides a new motion vector expression with simplified signaling. The expression method includes starting point, motion magnitude, and motion direction.

UMVE search process

UMVE search point

Base candidate index defines the starting point. Base candidate index indicates the best candidate among candidates in the list as follows.

| Base candidate IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Nth MVP | 1st MVP | 2nd MVP | 3rd MVP | 4th MVP |

Distance index is motion magnitude information. Distance index indicates the pre-defined distance from the starting point information. Pre-defined distance is as follows:

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel |

Direction index represents the direction of the MVD relative to the starting point. The direction index can represent of the four directions as shown below.

| Direction IDX | 00 | 01 | 10 | 11 |
| --- | --- | --- | --- | --- |
| x-axis | + | – | N/A | N/A |
| y-axis | N/A | N/A | + | – |

The UMVE method can be implemented on the test model or the Benchmark Set (BMS) in two variances.

1)  as additional merge candidates with additional information signaled.

2) as independent UMVE candidates with additional information signaled.

Affine merge in J0024 (Samsung, Huawei, Hisilicon, GoPro)

When merge/skip flag is true, and both width and height for the CU are larger than or equal to 8, an affine flag in CU level is signalled in the bitstream to indicate whether affine merge mode is used. And when the CU is coded as AF_MERGE, the merge candidate index with maximum value 5 is signalled for specifying which motion information candidate in the affine merge candidate list is used for the CU.

The affine merge candidate list is constructed as following steps:

- Insert model based affine candidates

Figure 3-25: Candidates for AF_MERGE

Model based affine candidate means that the candidate is derived from the valid neighbor reconstructed block coded with affine mode. As shown in Figure 3-25, the scan order for the candidate block is from left, above, above right, left bottom to above left.

If the neighbour left bottom block A is coded in 6-paramter affine mode, the motion vectors v4, v5, and v6 of the top left corner, above right corner and left bottom corner of the CU which contains the block A are obtained. And the motion vectors v0, v1, and v2 of the top left corner on the current CU is calculated according to v4, v5, and v6 by 6-parameter affine model.

If the neighbour left bottom block A is coded in 4-paramter affine mode, the motion vectors v4 and v5 of the top left corner and above right corner of the CU which contains the block A are obtained. And the motion vectors v0 and v1 of the top left corner on the current CU is calculated according to v4 and v5 by 4-parameter affine model.

- Insert control point based affine candidates

Control points based candidate means the candidate is constructed by combining the neighbor motion information of each control point.

The motion information for the control points is derived firstly from the specified spatial neighbors and temporal neighbor shown in Figure 3-26. CPk (k=1, 2, 3, 4) represents the k-th control point. A, B, C, D, E, F and G are spatial positions for predicting CPk (k=1, 2, 3); H is temporal position for predicting CP4.

The coordinates of CP1, CP2, CP3 and CP4 is (0, 0), (W, 0), (H, 0) and (W, H), respectively, where W and H are the width and height of current block.

The motion information of each control point is obtained according to the following priority order:

- For CP1, the checking priority is ABC, A is used if it is available. Otherwise, if B is available, B is used. If both A and B are unavailable, C is used. If all the three candidates are unavailable, the motion information of CP1 cannot be obtained.

- For CP2, the checking priority is ED;

- For CP3, the checking priority is GF;

- For CP4, H is used.

Secondly, the combinations of controls points are used to construct the motion model.

Motion vectors of two control points are needed to compute the transform parameters in 4-parameter affine model. The two control points can be selected from one of the following six combinations ({CP1, CP4}, {CP2, CP3}, {CP1, CP2}, {CP2, CP4}, {CP1, CP3}, {CP3, CP4}). For example, use the CP1 and CP2 control points to construct 4-parameter affine motion model, denoted as Affine (CP1, CP2).

Motion vectors of three control points are needed to compute the transform parameters in 6-parameter affine model. The three control points can be selected from one of the following four combinations ({CP1, CP2, CP4}, {CP1, CP2, CP3}, {CP2, CP3, CP4}, {CP1, CP3, CP4}). For example, use CP1, CP2 and CP3 control points to construct 6-parameter affine motion model, denoted as Affine (CP1, CP2, CP3).

Figure 3-26 Candidate positions for control point based merge candidate

Interpolated motion compensation prediction (MVPlanar) in J0027 (NHK, Sharp)

MVPlanar proposed in JVET-J0027 introduces an additional motion vector derivation mode. The proposed scheme generates sub-block motion vector field by interpolating motion vectors by weighted average of motion vectors of the above, above-right, left, and bottom-left neighboring blocks. In JEM, sub-block motion vector is also introduced by Spatial-temporal motion vector prediction (STMVP). The motion vectors are generated recursively by using the temporally and spatially neighboring MV predictors. The major difference between the proposed scheme and STMVP is the derivation of the inter prediction index and reference frame index specified as inter_pred_idc and ref_idx in HEVC specifications, which specifies whether list0, list1, or bi-prediction is used for the current block and specifies an index into a reference picture list, respectively. In STMVP, inter_pred_idc and ref_idx are derived by taking median of those of the neighboring blocks, whereas inter_pred_idc and ref_idx are signaled at coding block level in the proposed scheme. If some of the surrounding MVs are not available, the nearest available MV are copied in same way as intra reference sample substitution.

Figure 3-X MVPlanar

Merge Improvement in J0032 (USTC, etc.)

The merge improvement in J0032 will be called as complex merge mode in the following descriptions. If the width or the height of the current CU is larger than or equal to 16, a flag is decoded whether the current CU uses complex merge mode. If the current CU uses complex merge mode, the complex merge index is further decoded to indicate which candidate is used. The complex merge flag uses 3 context models, similar to the skip flag. For the complex merge index, 8 candidates are used, which are equally binarized into 3 bins. The first bin is decoded using 1 context model. The following two bins are bypass decoded.

The complex merge mode combines several blocks with similar motions into a single region with high-order motion models. As a result, all blocks within this region share the same motion information, and thus the motion parameters need to be transmitted only once. This method considers the high-order motion models including bilinear interpolation model, six-parameter affine model, and four-parameter affine model simultaneously.

The control points are needed to determine the complex motion models. Therefore, the first step of the complex merge mode is to determine the candidates of the control points. The candidates of the control points are shown in Fig. 1. CPk represents the kth control point. The spatial candidates for predicting the motion information of CP1, CP2, and CP3 are shown in Fig. 1(a). The temporal candidate position for predicting the motion information of CP4 is shown in Fig. 1(b).

Fig. 1. Candidate positions for the complex merge mode (a) spatial (b) temporal

The motion information of each control point is determined in the following priority order.

- For CP1, the checking priority is B2, A2, and B3.

- For CP2, the checking priority is B0 and B1.

- For CP3, the checking priority is A0 and A1.

- For CP4, TRB is used.

The control points are then used to construct the candidate list. The candidate model list is constructed in the following order. Only when the motion information of all the selected control points for the candidate model can be derived and they are not identical in at least one reference list, the model can be put into the candidate model list. The decoded complex merge index can be derived from the 3 bins using equal length binarization. When the candidate index is equal to the decoded complex merge index, the process stops.

- Affine (CP2, CP3)

- Affine (CP1, CP3)

- Affine (CP1, CP2, CP3)

- Affine (CP1, CP2)

- Affine (CP2, CP4);

- Affine (CP3, CP4);

- Affine (CP1, CP4);

- Bilinear

- Affine (CP1, CP2, CP4);

- Affine (CP2, CP3, CP4);

- Affine (CP1, CP3, CP4);

The motion information of the control points and the high-order motion model are then used to interpolate the MV of each sub-block in the current coding unit. As mentioned above, the interpolation function can be bilinear, six-parameter affine, and four-parameter affine. The reference index with the highest utilization rate among all the reference indices is selected as the final reference index. The control points with different reference indices are scaled to the final reference index.

Affine Merge Improvement in J0032 (USTC, etc.)

Instead of finding the first neighboring block with affine mode, the improvement tries to find the neighboring block with the largest coding unit size as the affine merge candidate.

Additional merge candidates in J0058 (Tencent)

Figure 5 additional merge candidates

The contribution proposes to derive additional spatial merge candidates from positions in an outer reference area which has an offset of (-96, -96) to the current block.

As shown in Figure 5, the positions are marked as A(i,j), B(i,j), C(i,j), D(i,j) and E(i,j). Each candidate B (i, j) or C (i, j) has an offset of 16 in the vertical direction compared to its previous B or C candidates. Each candidate A (i, j) or D (i, j) has an offset of 16 in the horizontal direction compared to its previous A or D candidates. Each E (i, j) has an offset of 16 in both horizontal direction and vertical direction compared to its previous E candidates. The candidates are checked from inside to the outside. And the order of the candidates is A (i, j), B (i, j), C (i, j), D (i, j), and E (i, j). To further study whether the number of merge candidates can be further reduced.

The candidates are added after TMVP candidates in the merge candidate list.

Enhanced merge mode in J0059 (HiSilicon)

Merge candidates list is constructed as follows with increased number of candidates 13

- Spatial neighbor 1~5

- ATMVP candidates

- A temporal candidate

- Extended spatial candidates 6~27

- Merge index 0 with MV offsets

- Combined Average Merge Candidates

A template matching based method is used to reorder the candidates.

The extended spatial positions from 6 to 27 are checked according to their numerical order after the temporal candidate. In order to save the MV line buffer, all the spatial candidates are restricted within two CTU lines.

In Merge offset extension, more candidates are checked based on existed candidate. New candidates has mv offset to the mv of existed candidate (except ATMVP candidate).

If one of new MV candidates (one luma sample offset) is selected, another eight new MV candidates with half luma sample offset be candidates in the list and one more index need to be transferred for higher precision

Combined average merge candidates are generated as follows. The motion information of the original merge candidates for each list is scaled to the derived reference picture for a given list. The scaled motion information is stored in a new array. The derived reference picture for a given list is the reference picture with the highest utilization rate among the reference pictures of the original merge candidates in that list. After scaling the motion information, all available motion vectors (up to 2) of the combined merge candidates are averaged separately for each reference list. The averaged motion vector is assigned as the motion vector of the combined average merge candidate. The derived reference picture for a given list is the reference picture of all the combined average merge candidates in that list.

Planar MV prediction in J0061 (HiSilicon)

Planar motion vector prediction is achieved by averaging a horizontal and vertical linear interpolation on 4x4 block basis as follows.

W and H denote the width and the height of the block. (x,y) is the coordinates of current sub-block relative to the above left corner sub-block. All the distances are denoted by the pixel distances divided by 4.is the motion vector of current sub-block.

The horizontal prediction  and the vertical prediction  for location (x,y) are calculated as follows:

where  and  are the motion vectors of the 4x4 blocks to the left and right of the current block.  and  are the motion vectors of the 4x4 blocks to the above and bottom of the current block.

The reference motion information of the left column and above row neighbour blocks are derived from the spatial neighbour blocks of current block.

The reference motion information of the right column and bottom row neighbour blocks are derived as follows.

- Derive the motion information of the bottom right temporal neighbour 4x4 block

- Compute the motion vectors of the right column neighbour 4x4 blocks, using the derived motion information of the bottom right neighbour 4x4 block along with the motion information of the above right neighbour 4x4 block, as described in Equation 1.

- Compute the motion vectors of the bottom row neighbour 4x4 blocks, using the derived motion information of the bottom right neighbour 4x4 block along with the motion information of the bottom left neighbour 4x4 block, as described in Equation 2.

|  | R(W,y) = ((H-y-1)AR + (y+1)BR)/H | Equation 1 |
| --- | --- | --- |
|  | B(x,H) = ((W-x-1)BL+ (x+1)BR)/W | Equation 2 |

where AR is the motion vector of the above right spatial neighbour 4x4 block, BR is the motion vector of the bottom right temporal neighbour 4x4 block, and BL is the motion vector of the bottom left spatial neighbour 4x4 block.

The motion information obtained from the neighbouring blocks for each list is scaled to the first reference picture for a given list.

Test specification

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.1 | J0012 Adding non-adjacent spatial merge candidates, on/off test | Ruoyang Yu (Ericsson) | Saverio Blasi (BBC) |
| 4.2.2 | J0014 Restricted merge | Martin Winken (HHI) | Yuling Hsiao (MediaTek) |
| 4.2.3 | J0021 a) Non-adjacent spatial merge candidate candidates b) a + Enlarge merge candidate list size to 10 c) a + Enlarge merge candidate list size to 15 d) c + Merge candidate list reordering e) d + Unified merge candidate list | Yi-Wen Chen (Qualcomm) | Jing Ye (Tencent) d),e): Byeongdoo Choi (Sharp) |
| 4.2.4 | J0022 a) Combined test 4.2.3 with model storage at CU level b) Combined test 4.2.3 with added affine candidates c) a+b | A. Robert (Technicolor) |  |
| 4.2.5 | J0015 a) ATMVP with one fixed collocated picture b) Picture/slice-level adaptation of ATMVP sub-block-size c) a+b | Xiaoyu Xiu (InterDigital) | a) Yue Yu (Arris) b) Hyeongmun Jang (LGE) c) Hyeongmun Jang (LGE) |
| 4.2.6 | J0017 ATMVP modification a) Simplified ATMVP with one fixed collocated picture b) a+5.b | Hyeongmun Jang (LGE) | Xiaoyu Xiu (InterDigital) |
| 4.2.7 | J0017 affine merge mode Affine merge with up to two affine merge candidate. | Jaeho LEE (LGE) | Xiaoyu Xiu (InterDigital) |
| 4.2.8 | J0018 a) Adding affine merge candidates b) Adding middle spatial and multiple temporal candidates c) Adding pairwise-average candidates and removing HEVC combined candidates d) Double ATMVP candidates with uniformity check and decoder-side speedup | Tzu-Der Chuang (MediaTek) | Huanbang Chen (Huawei) |
| 4.2.9 | J0024 Ultimate motion vector expression a) UMVE candidates as additional merge candidates with additional information signaled b) UMVE candidates as independent merge candidates (using new merge candidate list) with additional information signaled (including conventional merge candidates, “best“ set of candidate from this subCE, etc.) | Seungsoo Jeong (Samsung) | a) Ruoyang Yu (Ericsson) b)Byeongdoo Choi (Sharp) |
| 4.2.10 | J0024 Affine merge mode in a separate list a) Model based affine candidates (separate test for different number) b) Additional control point based affine candidates (separate test for different number) | Huanbang Chen (Huawei) | Fangdong Chen (Hikvision) |
| 4.2.11 | J0027 MV Planar | Shunsuke Iwamura (NHK) | Xiaozhong Xu (Tencent) |
| 4.2.12 | J0032 affine merge mode a) Modify the simple affine merge mode in BMS (use the largest coding unit size in affine mode as the affine merge candidate) b) Add a complex merge mode with 8 candidates, using different models. Separate test of different kinds of candidates will be provided. | Yang Wang (HIT) | Jaeho LEE (LGE) |
| 4.2.13 | J0058 Additional merge candidates a) Additional merge candidates b) Further reduce the additional merge candidate number | Jing Ye (Tencent) | Jungwon Kang (ETRI) Solovyev Timofey (Huawei) b) Byeongdoo Choi (Sharp) |
| 4.2.14 | J0061 MV Planar | Na Zhang (HiSilicon) | Xiaozhong Xu (Tencent) |
| 4.2.15 | J0059 enhanced merge mode a) Extended spatial positions from 6 to 27 b) Merge index 0 with MV offsets c) Combined average merge candidates | Jicheng An Xu Chen Na Zhang (Hisilicon) | Tzu-Der Chuang (MediaTek) Jing Ye (Tencent) |
| 4.2.16 | Sub-block merge candidates in BMS and JEM a) ATMVP b) STMVP c) Combination of a) and b) | Yi-Wen Chen (Qualcomm) |  |

Description of encoder operations is required if a dedicated algorithm is used in the proposed tool for, e.g., motion estimation, mode decision, etc.

Additional cost on line buffer due to new merge candidates should be described.

Number of additional merge candidates is required to be reported for each test.

The description of the RDO process for merge candidate selection is required if it is different from the default behavior in BMS/VTM. E.g., change of NUM_MRG_SATD_CAND is required to be reported.

Test condition and evaluation criteria

| Test# | Tool on case 1 (VTM as reference) | Tool off case 1 (BMS as reference) | Tool on case 2 (VTM+ToolX as reference) | Tool off case 2 (BMS-ToolX as reference) |
| --- | --- | --- | --- | --- |
|  | BD-Rate/enc./dec. | BD-Rate/enc./dec. | BD-Rate/enc./dec. | BD-Rate/enc./dec. |

For the tests on affine model based merge candidates, “Tool on case 1” and “Tool off case 2” are mandatory, where ToolX is affine motion in BMS.

For the tests on sub-block motion candidate related techniques, including various ATMVP modifications and MV planar techniques, “Tool on case 1” and “Tool off case 2” are mandatory, where ToolX is ATMVP in BMS.

For the tests on additional regular merge candidates (each candidate contains a motion vector for the whole CU), “Tool on case 1” and “Tool off case 1” are mandatory.

The suggested granularity for temporal motion vector storage is 16x16 in tool on test, and 4x4 for tool off test. It should be explicitly described if a different granularity is applied.

CE4.3: Motion vector coding

Summary

| Proposal | MV resolution | MVP | Inter Dir/Ref Idx | MVD coding |
| --- | --- | --- | --- | --- |
| J0012 (Ericsson etc.) |  | MVP between two directions |  |  |
| J0018 (MediaTek) | Adaptive motion vector difference resolutions (1/4, 1, 4), signal the index always |  |  |  |
| J0021 (Qualcomm) | Adaptive motion vector difference resolutions (1/4, 1, 4) |  |  | Modified MVD binarization and context modeling |
| J0022 (Technicolor etc.) |  | Unicity Check for AMVP candidate list. |  |  |
| J0024 (Samsung etc.) | adaptive motion vector difference resolutions (1/4,1/2, 1, ½, 4) | A combined signaling for MVP index and MVR index | Signal AMVR Bi-Index | Signal AMVR Bi-Index |
| J0063 (Huawei) |  |  | Symmetric derived ref_idx_l1 from ref_idx_l0 | Symmetric derived MVD1 from MVD0 |

Technical description

Motion vector prediction between two directions in J0012 (Ericsson, Nokia)

The MV prediction between two directions tool in J0012 exploits correlation between the two motion vectors for bi-prediction when the current block is coded using the AMVP mode. The tool allows a motion vector in a bi-prediction pair to be scaled and used as a predictor for the second vector. Furthermore, the tool makes the reconstruction order of the bi-prediction pair adaptively depending on how promising the L1 motion vector prediction candidate list is. A promising candidate list here means a list containing a non-scaled prediction candidate.

No change is made to the parsing of the motion vector differences. The reconstruction process of the bi-prediction pair starts with deriving the two candidate lists for L0 and L1 as normal. If the L1 candidate list does not contain a scaled candidate, the L1 vector is determined to be promising and therefore get reconstructed first. After that, if the L0 prediction list contains a scaled candidate, the first scaled candidate in the L0 candidate list is replaced by a scaled version of the L1 vector. Otherwise, if the L1 candidate list does contain a scaled candidate, the L0 vector is reconstructed first. After that, the first scaled candidate in the L1 candidate list is replaced with a scaled version of the L0 vector.

The tool is not enabled when the two reference picture lists contains the same reference pictures with mvd_l1_zero_flag being enabled.

Adaptive motion vector resolution in J0018 (MediaTek)

In J0018, an adaptive motion vector difference resolutions (AMVR) is proposed. Three kinds of motion vector resolutions are supported, including quarter-luma samples, integer-luma samples, and four-luma samples. When AMVP mode is used, an AMVR flag is always signaled to indicate whether a coarse-grand MVD resolution is applied. If yes, a flag is further signaled to indicate whether to use the integer-luma sample resolution or and four-luma sample resolution. The MVP is also rounded into the corresponding resolution.

Adaptive motion vector difference resolution in J0021 (Qualcomm, Technicolor)

A locally adaptive motion vector resolution (LAMVR) is used. As in JEM, MVD can be coded in units of quarter luma samples, integer luma samples or four luma samples. The MVD resolution is controlled at the block level, and MVD resolution flags are conditionally signaled for each block that has at least one non-zero MVD component.

When signaling MVD, the binarization and context modeling are dependent on the MVD precision and the POC distance between the current frame and the reference frame.

Adaptive motion vector resolution in J0024 (Samsung, Huawei, Hisilicon, GoPro)

AMVR in J0024 supports multiple motion vector resolutions. Supported motion vector resolutions in proposed method range from 1/4–pel to 4–pel (1/4–pel, 1/2–pel, 1–pel, 2–pel and 4–pel). Information about the motion vector resolution is signaled at the CU level when MVD information is signaled.

Depending on the resolution of CU, both motion vector (MV) and motion vector predictor (MVP) of the CU are adjusted. If applied motion vector resolution is denoted as R, MV (MVx, MVy) and MVP (MVPx, MVPy) are represented as follows:

(MVx, MVy) = (Round(MVx / (R * 4)) * (R * 4), Round(MVy / (R * 4)) * (R * 4))

(MVPx, MVPy) = (Round(MVPx / (R * 4)) * (R * 4), Round(MVPy / (R * 4)) * (R * 4))

Since the motion vector predictor and MV are both adjusted by an adaptive resolution, MVD (MVDx, MVDy) is also aligned to the resolution and, is signaled according to the resolution as follows:

(MVDx, MVDy) = ((MVx – MVPx) / (R * 4), (MVy – MVPy) / R * 4))

In this proposal, motion vector resolution index (MVR index) indicates MVP index as well as the motion vector resolution. As result, the proposed method has no MVP index signaling. Below table shows what each value of MVR index represents.

| MVR Index | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| Resolution (R) in pel | 1/4 | 1/2 | 1 | 2 | 4 |
| Nth MVP | 1st MVP | 2nd MVP | 3rd MVP | 4th MVP | 5th MVP |

In case of bi-prediction, AMVR has 3 modes for every resolution. AMVR Bi-Index indicates whether MVDx, MVDy of each reference list (list 0 or list 1) are signaled or not. AMVR Bi-Index in this proposal is defined as below table. AMVR Bi-Index is only signaled for bi-prediction. If motion information of List X is not signaled indicated by AMVR Bi-Index, reference index and MVD of List X are not signaled. Reference index of list X is set to be 0. MVD of List X is also set to be Zero. MVP of List X is derived from the combined signaling of MVR index and MVP index.

| AMVR Bi-Index | (MVDx, MVDy) of list 0 | (MVDx, MVDy) of list 1 |
| --- | --- | --- |
| 0 | Signaled | Signaled |
| 1 | Not signaled | Signaled |
| 2 | Signaled | Not signaled |

Unicity in motion information candidate list in J0022 (Technicolor, Qualcomm)

All the inter coding modes need to apply a process for generating the lists of candidate motion information. In the current response, these processes are homogenized and designed to ensure the full diversity and unicity of candidates.

The first generic aspect consists in unicity checking before adding any new candidate to the list. If the new candidate is already present in the list, then it is not added. This check is performed for all inter coding modes: AMVP, affine AMVP, merge/skip, template matching merge and template affine merge.

In some inter coding modes, other processes also improve the diversity of the lists, as explained below.

- In AMVP, the unicity check is also ported into the spatial candidate collection which constructs temporary candidate list. Furthermore, in AMVP and affine AMVP, when rounding and/or clipping operations are performed, these operations are achieved as soon as a new candidate is reached. The rounded and/or clipped candidate is checked before being added or not into the list.

- In Merge, whereas in the JEM only the spatial candidate redundancy was checked, the unicity check is applied to all candidate types except the ATMVP and STMVP ones.

- Candidates used to build the Template Matching and Template Affine lists are the same as the ones used to build the merge list. The main difference with the merge list derivation is that, during the unicity check, the IC flag of those candidates is not considered, because it is not part of the derived/predicted motion information in these modes.

Symmetrical mode for bi-prediction in J0063 (Huawei)

In this proposal, a symmetrical mode flag indicating whether symmetrical mode is used or not is explicitly signaled if the prediction direction is bi-prediction. When the flag is true, only mvp index for list 1 is signaled, and the reference index and MVD for list 1 are derived from motion information of list 0.

As shown in Fig.1, firstly, the ref_idx_l1 is decided by searching a reference picture in list 1, which is mirrored with the reference picture for list 0. If not exist such picture, the nearest reference picture to current picture is used. After the reference picture in list 1 is decided, MVD1 is derived by scaling MVD0. The scaling factor is calculated by following formula:

MVD1=POCCUR-POClist1POCCUR-POClist0×MVD0                         (1)

Test specification

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.3.1 | J0012 MV prediction between two directions for AMVP mode, on/off test. | Ruoyang Yu (Ericsson) | Huanbang Chen (Huawei) |
| 4.3.2 | J0063 Symmetrical mode for bi-prediction in J0063. | Huanbang Chen (Huawei) | Krit Panusopone (Arris) |
| 4.3.3 | J0021 a) Adaptive MV resolution (1/4 or 1 or 4). b) a) + Efficient MVD coding. | Yan Zhang (Qualcomm) | Xiaoyu Xiu (InterDigital) |
| 4.3.4 | Adaptive motion vector resolution in J0018 (1/4 or 1 or 4). | Tzu-Der Chuang (MediaTek) | Seungsoo Jeong (Samsung) |
| 4.3.5 | J0024 a) AMVR in J0024 with three MV resolutions ( ¼ or 1 or 4) b) AMVR in J0024 with five MV resolutions (1/4 or ½ or 1 or 2 or 4) c) Combined signaling of MVR index and MVP index with three MV resolutions ( ¼ or 1 or 4) in J0024 d) Combined signaling of MVR index and MVP index with five MV resolutions (1/4 or ½ or 1 or 2 or 4) in J0024 | Anish Tamse (Samsung) | Guichun Li (Tencent) |
| 4.3.6 | J0022 Unicity Check for AMVP candidate list | A. Robert (Technicolor) |  |

Description of encoder operations is required if a dedicated algorithm is used in the proposed tool for, e.g., motion estimation, mode decision, etc.

Test condition and evaluation criteria

| Test# | Tool on case 1 (VTM as reference) | Tool off case 1 (BMS as reference) | Tool on case 2 (VTM+ToolX as reference) | Tool off case 2 (BMS-ToolX as reference) |
| --- | --- | --- | --- | --- |
|  | BD-Rate/enc./dec. | BD-Rate/enc./dec. | BD-Rate/enc./dec. | BD-Rate/enc./dec. |

For tests on AMVR techniques (test 3-5) in this category, “Tool on case 1” and “Tool off case 2” are mandatory, where ToolX is AMVR in BMS.

For the other tests in this category, “Tool on case 1” and “Tool off case 1” are mandatory.

CE4.4: Generalized bi-direction

Technical description

Generalized bi-prediction in J0018 (MediaTek)

Generalized bi-prediction (GBi) is presented in JVET-J0018. The concept of GBi is to allow using different weights for predictors from L0 and L1, respectively, instead of using equal weights in traditional bi-prediction. The predictor generation can be summarized in the following equations.

PTradBiPred = (PL0 + PL1 + RoundingOffset) >> shiftNum

PGBi = (w0 * PL0 + w1 * PL1 + RoundingOffsetGBi) >> shiftNumGBi    ,

where PTradBiPred and PGBi are final predictors for traditional bi-prediction and GBi respectively, PL0 and PL1 are predictors from L0 and L1, w0 and w1 are selected GBi weights for L0 and L1, and RoundingOffset, shiftNum, RoundingOffset1, and shiftNum1 are used to normalize the final predictors in traditional bi-prediction and GBi respectively. In AMVP mode, the selection of weights is signaled, and in Merge mode, the weights are inherited from merge candidate. In this CE test, different sets of supported weights for {w0 , w1} are evaluated. The encoding algorithm is similar to that in JVET-D0102. For each supported weight, one high-complexity mode decision is used to evaluate the coding performance. However, in order to achieve a better tradeoff point between coding gain and encoding time increase, some early terminations and reusing MVs among different supported weights are also utilized.

Test specification

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.4.1 | Generalized bi-prediction in J0018 | Ching-Yeh Chen (MediaTek) | Saverio Blasi (BBC) |

Description of encoder operations is required if a dedicated algorithm is used in the proposed tool for, e.g., motion estimation, mode decision, etc.

Test condition and evaluation criteria

| Test# | Tool on case 1 (VTM as reference) | Tool off case 1 (BMS as reference) | Tool on case 2 (VTM+ToolX as reference) | Tool off case 2 (BMS-ToolX as reference) |
| --- | --- | --- | --- | --- |
|  | BD-Rate/enc./dec. | BD-Rate/enc./dec. | BD-Rate/enc./dec. | BD-Rate/enc./dec. |

For the test in this category, “Tool on case 1” and “Tool off case 1” are mandatory.

CE4.5: Reference picture boundary padding

Technical description

Multi-directional boundary padding (MDBP) in J0014 (HHI)

Multi-directional boundary padding (MDBP) uses angular intra prediction to extend the reference frame border, whenever the referenced pixel area is partially or entirely outside the area of the reconstructed reference frame.

The best intra mode is estimated in both encoder and decoder by using a template area that is located inside the reconstructed reference frame as shown below. The template area is filled by using all available intra modes revered and deriving sample values of the template area by intra prediction from the outer-most pixel line. The best intra mode lead to minimal SAD with respect to the original reconstructed sample values.

Figure 8: Template area used for estimating the MDBP angular mode

Motion Compensated Boundary Pixel Padding (MC Padding) in J0021 (Qualcomm, Technicolor)

When the decoder performs motion compensation, if the motion vector points to a block outside the frame boundary, a part of the reference block is not available. For each region with size of 4xM or Mx4 along the boundary of the reference picture to be padded, M being the desired frame boundary extension, a motion vector is derived from the nearest 4x4 block inside the frame. If the nearest 4x4 block is intra coded, a zero motion vector is used.  If the nearest 4x4 block is coded with bi-directional inter prediction, only the motion vector, which points to the pixel farther away from the frame boundary, is used in motion compensation for padding. After the motion vector derivation, motion compensation is then performed to obtain the pixels in the padding region with the consideration of average pixel value offset between the nearest 4x4 block and its corresponding block in its reference picture.

Boundary pixel padding using motion compensation in J0025 (Huawei, Samsung, Hisilicon, GoPro)

In J0025, if a boundary pixel of picture is derived by motion compensation, padded pixel is also derived by using the motion compensation.

Figure 9: Boundary pixel padding using motion compensation

For using neighbor pixel of reference block, motion compensation of the area which is referenced by padded pixel is conducted. If the area is smaller than padded area or boundary block is not derived by MC, left padded pixel is derived using last pixels. And top-left, top-right, bottom-left, bottom-right of padding area are derived by using last pixel of each position. This process is conducted after in-loop filtering.

Test specification

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.5.1 | Multi-directional boundary padding (MDBP) in J0014 | Jens Brandenburg (HHI) | Yuling Hsiao (MediaTek) |
| 4.5.2 | MC Padding in J0021 | Yan Zhang (Qualcomm) | Minsoo Park (Samsung) |
| 4.5.3 | Boundary pixel padding using motion compensation in J0025 | Minsoo Park (Samsung) | Robert Skupin (HHI) |

Test condition and evaluation criteria

| Test# | Tool on case 1 (VTM as reference) | Tool off case 1 (BMS as reference) | Tool on case 2 (VTM+ToolX as reference) | Tool off case 2 (BMS-ToolX as reference) |
| --- | --- | --- | --- | --- |
|  | BD-Rate/enc./dec. | BD-Rate/enc./dec. | BD-Rate/enc./dec. | BD-Rate/enc./dec. |

For the test in this category, “Tool on case 1” and “Tool off case 1” are mandatory.

CE4.6: Local illumination compensation

Technical description

Local illumination compensation in JEM

Please refer to JVET-G1001 for details.

Constraint on usage of LIC in J0020 (Panasonic)

LIC is disabled for normal inter prediction and lic_flag does not appear in the mode, i.e. LIC can only be used together with skip/merge modes.

Bi-directional illumination compensation (BIC) in J0022 (Technicolor)

For the sake of memory bandwidth reduction, the traditional Local Illumination Compensation (LIC) has been removed both from the test model and the benchmark model set. Indeed, traditional uni-directional LIC requires access to the reconstructed neighboring samples to estimate the illumination compensation parameters (ICP) (Figure 7).

It is proposed to modify the estimation of the IC parameters so that the reconstructed neighboring samples are no more used but the reference samples (ref-0 and ref-1), for bi-prediction only.

Traditional LIC

LIC allows correcting block prediction samples (SMC) obtained via motion compensation by possibly considering the spatial or temporal local illumination variation.

SIC = ai . SMC + bi

In traditional uni-directional prediction, the IC parameters (ai,bi) are estimated by comparing the reconstructed neighboring samples (L-shape-cur in Figure 10) with the neighboring samples (L-shape-ref-i) of the reference-i block (i=0 or 1). The IC parameters minimize the difference between the samples in the L-shape-cur and the samples of the L-shape-ref-i corrected with IC parameters as depicted in Figure 10.

Figure 10: traditional uni-directional LIC uses reconstructed neighboring samples (L-shape in grey) to estimate the IC parameters.

Proposed bi-directional IC (BIC)

The illumination compensation process for deriving the IC parameters is modified so that the parameters are estimated using the samples of the two reference CUs samples as depicted in Figure 11. First, the IC parameters (a,b) are estimated between the two reference blocks, next the IC parameters (ai,bi)i=0,1 between the reference blocks and the current block are derived as follows:

a0=a.α+1-αb0= α.ba1= α+(1- α)/ab1= -b(1- α)/a		where: α=poccur-poc0poc1-poc0

Figure 11. Bi-directional IC parameters are estimated on the full reference CUs in grey.

Simplified BIC parameters derivation for generalized OBMC in J0022 (Technicolor)

The OBMC process is carried out with 4x4 sub-blocks. In the proposed solution, it has been unified so that the decision to filter 2 or 4 lines/columns of pixels is no more taken a-priori depending on the CU coding mode, but on-the-fly depending on motion vectors located at opposite 4x4 sub-block borders.

Instead of performing OBMC successively on left, above, right then bottom sub-block borders, these borders are considered by pairs: horizontal orientation (left, right) and vertical orientation (top, bottom). For each 4x4 sub-block, motion vectors corresponding to the pair of borders in the considered orientation are retrieved. If both are available and different from the motion vector of the current 4x4 sub-block, then OBMC filtering is performed on 2 lines/columns of pixels from each orientation. If only one is available or if only one is different from the motion vector of the current sub-block, OBMC filtering is achieved on 4 lines/columns of pixels on the border where the motion vector is different. If none is available or both motion vectors are equal to the current sub-block motion vector, no filtering is applied.

This proposed unified OBMC allows filtering a different number of samples for horizontal and vertical directions and having a better adaptation to motion transitions within a CU, in particular with merge ATMVP CUs as illustrated in Figure 12.

Figure 12. a) prior art OBMC and b) proposed unified OBMC applied on a 32x16 inter CU in Merge ATMVP mode
(dashed lines denote the splits retrieved from ATMVP, i.e. the motion transitions).

Furthermore, the OBMC process in the JEM limits filtering to 2 lines/columns for small CUs whose surface is lower than 64 samples. In the proposed unified OBMC, since the filtering size is decoupled between orientations, this limitation has been adapted. If the width of the CU is less than 8, only 2 columns of pixels are filtered. If the height is less than 8, only 2 lines of pixels are filtered. This allows a better adaptation of the OBMC filtering for thin CUs as shown in Figure 13.

Figure 13. OBMC applied on a 32x4 inter CU a) as achieved in JEM and b) as in proposed unified OBMC.

OBMC design simplification

The proposed generalized OBMC is further enriched to account for the bi-directional IC parameters (section 5.1.2) associated to the neighboring CU of current CU.

Indeed, in the JEM, some LIC parameters were computed for each OBMC 4xS band. Instead of that, here the bi-directional IC parameters are inherited from the considered neighboring 4x4 sub-blocks (Figure 14). For this purpose, the BIC parameters of the current picture are stored with the motion field information at a 4x4 sub-block resolution.

Figure 14: OBMC process inherits the BIC parameters from the neighboring sub-blocks

Inter prediction refinement in J0024 (Samsung, Huawei, Hisilicon, GoPro)

The idea behind Inter Prediction Refinement (IPR) in J0024 is to compensate for the illumination differences between the reference frame and the current frame. IPR is applied on the prediction pixels obtained by motion compensation. The tool can be described in two parts.

- Computing and storing the parameters

At the end of processing of each CU, linear models (Ax + B = y) are computed by comparing the prediction pixels (x) and the reconstructed pixels (y). The models are computed using linear regression. Three sets of models are computed, one each for the left edge of the CU, the bottom edge of the CU and the right edge of the CU. Furthermore, each of these sets contains models for the three color component and two directions (if available). These parameters (sets of A and B) are then stored in a map for later access.

Parameter computation process

- Tool application

In case of a merge/skip block, the IPR flag for the current CU is derived from the corresponding neighbor. In case of other inter coded CU’s certain conditions are defined under which IPR is applicable. These conditions depend on the size of the current CU and the coding mode used by neighbors. A flag is signaled to indicate whether IPR is to be applied to a given inter coded CU or not. If the tool is to be applied, the motion compensation pixels (x) are refined using the application of the linear model (Ax + B) to obtain the new set of prediction pixels. The model used is derived from the current CU’s neighbor. Since multiple neighbors may be available, pruning is performed in order to consider only inter neighbors and give higher priority to neighbor which use same reference frames for prediction as the current CU.

Model parameter derivation

Additionally, when performing RDO check for determining whether IPR will be used or not, mean-removed SAD (Sum of Absolute Difference) is used instead of the regular SAD for the ME (Motion Estimation) process.

Test specification

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.6.1 | Bi-directional illumination compensation (BIC) in J0022 | A. Robert (Technicolor) | Kiyofumi Abe (Panasonic) |
| 4.6.2 | J0022 Combination of BIC, generalized OBMC and Simplified BIC parameters derivation | A. Robert (Technicolor) | V. Lorcy (Bcom) |
| 4.6.3 | J0024 Inter prediction refinement | Anish Tamse (Samsung) | P. Bordes (Technicolor) |

Description of encoder operations is required if a dedicated algorithm is used in the proposed tool for, e.g., motion estimation, mode decision, etc.

Test condition and evaluation criteria

| Test# | Tool on case 1 (VTM as reference) | Tool off case 1 (BMS as reference) | Tool on case 2 (VTM+ToolX as reference) | Tool off case 2 (BMS-ToolX as reference) |
| --- | --- | --- | --- | --- |
|  | BD-Rate/enc./dec. | BD-Rate/enc./dec. | BD-Rate/enc./dec. | BD-Rate/enc./dec. |

For the test in this category, “Tool on case 1” and “Tool off case 1” are mandatory.

CE4.7: Chroma Interpolation filters

Technical description

Chroma interpolation filters in JEM

Please refer to JVET-G1001 for details.

Chroma interpolation filters in J0024 (Samsung, Huawei, Hisilicon, GoPro)

Four-tap interpolation filters are used for inter prediction filtering of Chroma component with translational motion. The filter coefficients are derived based on bilinear transform coupled with a high-pass filter. Motion compensation interpolation filters for luma and chroma components are summarized in Table 8-3.

Table 8-3. Chroma motion compensation interpolation filters

| Phase | 6-bit filter coefficients | Phase | 6-bit filter coefficients |
| --- | --- | --- | --- |
| 1/32 | { -3, 68, -1, 0 } | 9/32 | { -5, 52, 20, -3 } |
| 1/16 | { -3, 66, 2, -1 } | 5/16 | { -5, 50, 23, -4 } |
| 3/32 | { -4, 64, 5, -1 } | 11/32 | { -5, 48, 25, -4 } |
| 1/8 | { -4, 62, 7, -1 } | 3/8 | { -5, 46, 28, -5 } |
| 5/32 | { -4, 60, 10, -2 } | 13/32 | { -5, 44, 30, -5 } |
| 3/16 | { -4, 58, 12, -2 } | 7/16 | { -6, 42, 33, -5 } |
| 7/32 | { -4, 56, 15, -3 } | 15/32 | { -6, 40, 36, -6 } |
| 1/4 | { -5, 54, 18, -3 } | 1/2 | { -6, 38, 38, -6 } |

Test specification

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.7.1 | J0024 Chroma interpolation filter | Sychev Maxim (Huawei) | Anastasia Henkel (HHI) |

Test condition and evaluation criteria

| Test# | Tool on case 1 (VTM as reference) | Tool off case 1 (BMS as reference) | Tool on case 2 (VTM+ToolX as reference) | Tool off case 2 (BMS-ToolX as reference) |
| --- | --- | --- | --- | --- |
|  | BD-Rate/enc./dec. | BD-Rate/enc./dec. | BD-Rate/enc./dec. | BD-Rate/enc./dec. |

For the test in this category, “Tool on case 1” and “Tool off case 1” are mandatory.

Expected interdependency with other CE

This CE is related to CE9 on Decoder side MV derivation, CE10 on Combined and multi-hypothesis prediction, and CE11 on Combined and multi-hypothesis prediction. It is recommended to do combined tests with selected outcomes of these CEs to provide information for the group.

Test condition and evaluation criteria

In general, each testing item is required to be compared against 1) VTM and 2) BMS, which is “Tool on case 1” and “Tool off case 1” respectively in the table below.

“Tool on case 2” and “Tool off case 2” are also specified in the table in order to provide additional information especially for encoding and decoding time comparison with ToolX. ToolX is the BMS tool that is similar to the techniques being tested, which is specified in each category of the CE.

| Test# | Tool on case 1 (VTM as reference) | Tool off case 1 (BMS as reference) | Tool on case 2 (VTM+ToolX as reference) | Tool off case 2 (BMS-ToolX as reference) |
| --- | --- | --- | --- | --- |
|  | BD-Rate/enc./dec. | BD-Rate/enc./dec. | BD-Rate/enc./dec. | BD-Rate/enc./dec. |

In case ToolX is identified for a testing item, “Tool on case 1” and “Tool off case 2” are mandatory. Otherwise, “Tool on case 1” and “Tool off case 1” are mandatory.

For the other two cases that are optional, it is encouraged to provide results as well. Considering the simulation burden, the simulation data provided by AHG13 could be used for obtaining BD-Rate when the simulation by a tester/crosschecker is not finished.

Each testing item will be evaluated using BD rates, encoding time, decoding time according to Common Test Conditions (CTC) in JVET-J1010. Simulation with RA and LDB configurations are mandatory.

Sub-CE specific test condition and evaluation criteria may be defined for a sub-CE if considered informational for tool evaluation. Please refer to the section of test condition and evaluation criteria in each sub-CE for information.

For getting encoding and decoding time ratio of each test, the same SIMD setting, either SSE or AVX2, must be used for generating both anchor and a test. For easy comparison of encoding and decoding time of different techniques, it is suggested to use SSE as the default SIMD setting by encoder configuration, and a different setting is requested to be reported. It is requested to report if the SW implementation involves any kind of SIMD speed-up.

SW integration guideline

According to the information from SW coordinator,

- BMS with JEM_TOOLS macro disabled should produce the same bitstream as VTM.

- Bitstreams of BMS with VTM config file and BMS with JEM_TOOLS macro disabled should only differ by the tool indication flags in the SPS, which should have a very minor impact on bit rate.

Implementing a proposed technique on BMS SW only could serve the purpose of both tool on tests and tool off tests. Therefore, following guideline for SW integration is suggested,

- Proponent implement the technique in BMS SW, for both tool on tests and tool off tests

- For the two type of tool on tests, the results of VTM or VTM+ToolX are obtained by disabling BMS tools (except ToolX) via configuring encoder parameters of BMS SW (no change of macro setting)

Timeline

| T1 | May 11, 2018 | Finalize CE description |
| --- | --- | --- |
| T2 | June 1, 2018 | Deliver software packages and begin cross-checking (proponents can continue to modify the software until T3) |
| T3 | June 19, 2018 | Finish cross-checking and report results to CE group |

Participant list

| Company/ Organization | Participant Name | Email Address |
| --- | --- | --- |
| Technicolor | Antoine Robert Philippe Bordes Tangi Poirier Fabrice Le Léannec Franck Galpin Ya Chen | antoine.robert@technicolor.com philippe.bordes@technicolor.com tangi.poirier@technicolor.com fabrice.leleannec@technicolor.com franck.galpin@technicolor.com Ya.CHEN@technicolor.com |
| HHI | Martin Winken Robert Skupin Jens Brandenburg Anastasia Henkel | martin.winken@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de jens.brandenburg@hhi.fraunhofer.de anastasia.henkel@hhi-extern.fraunhofer.de |
| Huawei | Huanbang Chen Haitao Yang Jianle Chen Sychev Maxim Solovyev Timofey | chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com Sychev.Maxim@huawei.com solovyev.timofey@huawei.com |
| LGE | Seung Hwan Kim Jaehyun Lim Jaeho Lee Hyeongmun Jang | seunghwan3.kim@lge.com Jaehyun.lim@lge.com jhlee.lee@lge.com hm.jang@lge.com |
| Tencent | Xiaozhong Xu Jing Ye Guichun Li Shan Liu Meng Xu Xiang Li | xiaozhongxu@tencent.com jingye@tencent.com guichunli@tencent.com shanl@tencent.com mengxxu@tencent.com xlxiangli@tencent.com |
| USTC | Yue Li Li Li | lytt@mail.ustc.edu.cn lili90th@gmail.com |
| HiSilicon | Jicheng An Na Zhang Xu Chen Jianhua Zhang | anjicheng@hisilicon.com zhangna25@huawei.com anderson.chen@hisilicon.com zhengjianhua@hisilicon.com |
| BBC | Saverio Blasi | saverio.blasi@bbc.co.uk |
| ETRI | Jungwon Kang Sung-Chang Lim Hahyun Lee | jungwon@etri.re.kr sclim@etri.re.kr hanilee@etri.re.kr |
| Panasonic | Kiyofumi Abe Jingya Li | abe.kiyo@jp.panasonic.com jingya.li@sg.panasonic.com |
| Fujitsu | Jianqing Zhu Kimihiko Kazui | zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Sony | Kenji Kondo | kenji.kondo@sony.com |
| Qualcomm | Yi-Wen Chen Chun-Chi Chen Yan Zhang Yu Han Wei-Jung Chen | yiwench@qti.qualcomm.com chunchic@qti.qualcomm.com yzh@qti.qualcomm.com yuhan@qti.qualcomm.com wchien@qti.qualcomm.com |
| Alibaba | Yu-Chen Sun | yuchen.s@alibaba-inc.com |
| Hikvision | Fangdong Chen | chenfangdong@hikvision.com |
| Sharp | Weijia Zhu Byeongdoo Choi Tomonori Hashimoto Takeshi Chujoh Tomohiro Ikai | zhuw@sharplabs.com choib@sharplabs.com tomonori.hashimoto@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Ericsson | Ruoyang Yu | ruoyang.yu@ericsson.com |
| Samsung Electronics | Min Woo Park Seungsoo Jeong Anish Tamse Kiho Choi Jie Chen Minsoo Park | m.w.park@samsung.com ss00.jeong@samsung.com tamse.anish@samsung.com kiho14.choi@samsung.com jie1222.chen@samsung.com mss.park@samsung.com |
| Nokia | Jani Lainema | jani.lainema@nokia.com |
| InterDigital | Yuwen He Xiaoyu Xiu Yan Ye | Yuwen.He@interdigital.com Xiaoyu.Xiu@interdigital.com Yan.Ye@interdigital.com |
| Arris | Krit Panusopone Yue Yu David Baylon Seungwook Hong Limin Wang | Krit.panusopone@arris.com Yue.yu@arris.com David.baylone@arris.com Seungwook.hong@arris.com Limin.wang@arris.com |
| MediaTek | Tzu-Der Chuang Ching-Yeh Chen Chih-Wei Hsu Yu-Wen Huang Shih-Ta Hsiang | Peter.Chuang@mediatek.com chingyeh.chen@mediatek.com cw.hsu@mediatek.com yuwen.huang@mediatek.com shih-ta.hsiang@mediatek.com |
| NHK | Shunsuke Iwamura | iwamura.s-gc@nhk.or.jp |
| Netflix | Andrey Norkin | anorkin@netflix.com |
| Orange | Gordon Clare | gordon.clare@orange.com |
| Broadcom | Minhua Zhou Pensong Chen Wade Wan | minhua.zhou@broadcom.com peisong.chen@broadcom.com wade.wan@broadcom.com |
| Canon | Jonathan Taquet | Jonathan.TAQUET@crf.canon.fr |
| PKU | Zhao Wang | zhaowang@pku.edu.cn |
| Chips&Media | Wook Je Jeong | nick.jeong@chipsnmedia.com |
| Aachen University | Johannes Sauer | sauer@ient.rwth-aachen.de |
| HIT | Yang Wang Xiaopeng Fan | wangyang.cs@hit.edu.cn fxp@hit.edu.cn |
| WILUS | Greg Ko | greg.ko@wilusgroup.com |
| ITRI | Ching-Chieh Lin | JackLin@itri.com |
| Byte Dance | Kai Zhang | zhangkai.video@bytedance.com |