| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J1030 |
| --- | --- |

| Title: | Description of Core Experiment 10: Combined and multi-hypothesis prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu Martin Winken Xiaoyu Xiu | Email: | cw.hsu@mediatek.com martin.winken@hhi.fraunhofer.de Xiaoyu.Xiu@InterDigital.com |
| Source: | CE coordinators |  |  |

_____________________________

- Abstract

The main goal of this Core Experiment (CE) is to improve the inter prediction of the Test Model by the use of combined predictions. CE10.1 is to test prediction to be combined coming from multiple hypothesis, where one hypothesis refers to prediction from inter mode or from intra mode. CE10.2 is to test prediction to be combined from using motions of neighboring coding units (CUs). CE10.3 is to test prediction to be combined from non-rectangular prediction partitions within one CU. CE10.4 is to test prediction to be combined using filtering.

Participants

| Nr. | Name | Company | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
| 1 | Chih-Wei Hsu | MediaTek | cw.hsu@mediatek.com | X | X |
| 2 | Martin Winken Jonathan Pfaff | Fraunhofer HHI | martin.winken@hhi.fraunhofer.de | X | X |
| 3 | Kai Zhang | Qualcomm | zhangkai@qti.qualcomm.com | X |  |
| 4 | Jicheng An | HiSilicon | anjicheng@hisilicon.com | X |  |
| 5 | Yinji Piao | Samsung | yj1003.piao@samsung.com | X |  |
| 6 | Xiu, Xiaoyu | InterDigital | Xiaoyu.Xiu@InterDigital.com | X |  |
| 7 | Antoine Robert | Technicolor | antoine.robert@technicolor.com | X |  |
| 8 | Ru-Ling Liao | Panasonic | ruling.liao@sg.panasonic.com | X |  |
| 9 | Max Blaeser | RWTH Aachen University | blaeser@ient.rwth-aachen.de | X |  |

CE10.1: Multi-hypothesis prediction

JVET-J0018 MediaTek

Tree inter combined prediction modes are tested in this experiment to combine merge indexed prediction with AMVP/uni-prediction mode, skip/merge mode, and intra mode, respectively. When combined with AMVP/uni-prediction mode, one motion is acquired using the original syntax elements in AMVP mode while the other is acquired using the merge index, where total two hypothesis are used. When combined with skip or merge mode, implicitly selection of succeeding merge candidate without signaling any additional merge index is used to acquire the motions, where at most two more hypothesis are used. When combined with intra mode, explicit signaling of intra mode for merge mode in inter CU and explicit signaling of merge index for intra mode in intra CU are applied to generate the combined predictions, where one more hypothesis from intra mode or from merge index is used. Different weights to generate the final prediction depending on the combinations are also tested in this experiments.

JVET-J0014 HHI

The aim of this test is to study inter prediction with more than two constituent motion-compensated prediction signals. The signaling of the uni/bi-prediction signal itself is unchanged, but it is possible to specify one or more additional inter prediction signal(s).  The syntax elements which are transmitted for each of these additional inter prediction signals are very similar to the ones used for uni/bi-prediction (i.e., reference index, motion vector prediction index, and motion vector differences). For the additional prediction signals, the concept of prediction list0/list1 is not applied, and instead one combined list is used.

The resulting overall prediction signal is accumulated iteratively with each additional prediction signal.  A weighting factor specifies how the so far accumulated prediction signal and the current additional prediction signal are to be combined.  With the uni/bi prediction signal puni/bi and the first additional inter prediction signal h3, the resulting prediction signal p3 is obtained as follows:

p3=(1-α)puni/bi+αh3

The weighting factor α is specified by the syntax element add_hyp_weight_idx, according to the following mapping:

| add_hyp_weight_idx | α |
| --- | --- |
| 0 | 1/4 |
| 1 | -1/8 |

Analogously to above, further additional prediction signals can be used:

pn+1=1-αn+1pn+αn+1hn+1

The resulting overall prediction signal is obtained as the last pn (i.e., the pn having the largest index n).

Note that also for inter prediction blocks using MERGE mode (but not SKIP mode), additional inter prediction signals can be specified.  Further note, that in case of MERGE, not only the uni/bi prediction parameters, but also the additional prediction parameters of the selected merging candidate can be used for the current block. The impact of different configuration settings (i.e., max. number of additional hypotheses, number of reference pictures used for the additional hypotheses, etc.) will be studied.

JVET-J0053 Qualcomm

Two tests are based on Intra-Prediction Mode (IPM) propagation approach for inter-pictures. Each 4×4 sub-block in an inter-coded Coding Unit (CU) is assigned with a propagated IPM, which is fetched from a reference block in a reference picture, located by motion vectors of the 4×4 sub-block. In test 1, the propagated IPM can be used as a predictor for intra mode coding. Based on test 1, an inter-coded CU with the merge mode can be predicted by a weighted sum of inter-prediction and intra-prediction with the propagated IPM in test 2.

JVET-J0059 HiSilicon

The dual merge mode allows each reference list to use two sets of motion information. It is determined by the SATD criterion whether each reference list uses two sets of motion information or a set of motion information and indicated with a flag. The new set motion information of each reference list is obtained as follows:

RefX’= (RefX == 0) ? 1 : 0

MvX’ = dX’ * MVX /dX

As shown in the figure below, the X represents the reference list 0 or the reference list 1. RefX represents the reference index of the original motion information of the reference list X. RefX' represents the reference index of the new set of motion information in the reference list X. Similarly, MVX represents the motion vector of the original motion information of the reference list X. MVX' represents the motion vector of the new set of motion information in the reference list X. dX represents the temporal distance between the current picture and the reference picture specified by the RefX. dX’ represents the temporal distance between the current picture and the reference picture specified by the RefX’.

Original prediction value is

PredUni = PredX

PredBi=(Pred0+Pred1) >>1

Modified prediction value is

PredUni = (PredX+ PredX’) >>1

PredBi=( (Pred0+Pred1) >>1+ (Pred0’+Pred1’) >>1 ) >>1

where PredUni represents the prediction value of uni-prediction, PredBi represents the prediction value of bi-prediction, PredX represents the prediction value compensated by the original motion information of the reference list X, and PredX’ represents the prediction value compensated by the new set of motion information in the reference list X.

For the merge mode, the original motion information can be the motion information of each candidate.

A flag under merge mode is signalled to indicate whether this dual merge mode is used.

Test scenario

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 | Multi-hypothesis prediction for uni-prediction as described in 2.1 - Explicit signaling with merge index - Additional 1 hypothesis for uni-prediction - Total 2 hypothesis | Chih-Wei Hsu (MediaTek) | Martin Winken (HHI) |
| 2 | Multi-hypothesis prediction for skip/merge mode as described in 2.1 - Implicit derivation of hypothesis - Additional 1 or 2 hypothesis - Total at most 4 hypothesis | Chih-Wei Hsu (MediaTek) | Martin Winken (HHI) |
| 3 | Multi-hypothesis prediction for intra and inter mode as described in 2.1 - Explicit signaling - Additional 1 hypothesis for merge mode - Additional 1 hypothesis for intra mode - Total 2 hypothesis | Chih-Wei Hsu (MediaTek) | Martin Winken (HHI) |
| 4 | Multi-hypothesis prediction as described in 2.2 - Additional 1 hypothesis - Total at most 3 hypothesis | Martin Winken (HHI) | ETRI |
| 5 | Multi-hypothesis prediction as described in 2.2 - Additional 2 hypothesis - Total at most 4 hypothesis | Martin Winken (HHI) | ETRI |
| 6 | Multi-hypothesis prediction as described in 2.3 | Kai Zhang (Qualcomm) | BBC |
| 7 | Multi-hypothesis prediction as described in 2.4 | Jicheng An (HiSilicon) | SHARP |

CE10.2: Overlapped block motion compensation

JVET-J0024 Samsung

OBMC in H.263 is directly extended to this proposal on 4x4 block basis. OBMC is always applied to motion compensated predictors regardless of CU size. After motion compensation, each 4x4 block in a CU further achieves four corresponding predictors with four neighboring MVs, respectively. Including predictor of current 4x4 block, OBMC applied predictor is achieved by weighted sum of five predictors.

JVET-J0015 InterDigital

The purpose of this test is to reduce the complexity of the OBMC in JEM. Specifically, in the proposed OBMC design, the whole OBMC process is performed in two steps. In the first step (referred to as “boundary OBMC”) the samples on the top/left boundaries of the current CU are adjusted using the MVs from its spatial neighboring CUs. If the CU is not coded in sub-block mode, the OBMC process ends after the first step. Otherwise, the second step (referred as “internal OBMC”) is performed. In the second step, the samples on the top, left, bottom and right boundaries of each sub-block, except the samples on the CU boundaries that have already been adjusted in the first step, are adjusted using the MVs of its neighboring sub-blocks in the same CU. During both the boundary and internal OBMC processes, the motion compensations (MCs) of the sub-blocks with identical motion are combined to one single MC with a larger block size for reducing the complexity.

Additionally, in the tested method, some OBMC processes can be skipped when the following conditions are met: 1) both of the neighboring motion and the current motion are either uni-predicted ro bi-predicted; 2) the same reference picture(s) are used by the neighboring motion and the current motion; 3) the MV difference between the neighboring motion and the current motion is no larger than 1/2-pel.

JVET-J0022 Technicolor - Generalized OBMC

The OBMC process is carried out with 4x4 sub-blocks. In the proposed solution, it has been unified so that the decision to filter 2 or 4 lines/columns of pixels is no more taken a-priori depending on the CU coding mode, but on-the-fly depending on motion vectors located at opposite 4x4 sub-block borders.

Instead of performing OBMC successively on left, above, right then bottom sub-block borders, these borders are considered by pairs: horizontal orientation (left, right) and vertical orientation (top, bottom). For each 4x4 sub-block, motion vectors corresponding to the pair of borders in the considered orientation are retrieved. If both are available and different from the motion vector of the current 4x4 sub-block, then OBMC filtering is performed on 2 lines/columns of pixels from each orientation. If only one is available or if only one is different from the motion vector of the current sub-block, OBMC filtering is achieved on 4 lines/columns of pixels on the border where the motion vector is different. If none is available or both motion vectors are equal to the current sub-block motion vector, no filtering is applied.

This proposed unified OBMC allows filtering a different number of samples for horizontal and vertical directions and having a better adaptation to motion transitions within a CU, in particular with merge ATMVP CUs as illustrated in Figure 1.

Figure 1. a) prior art OBMC and b) proposed unified OBMC applied on a 32x16 inter CU in Merge ATMVP mode
(dashed lines denote the splits retrieved from ATMVP, i.e. the motion transitions).

Furthermore, the OBMC process in the JEM limits filtering to 2 lines/columns for small CUs whose surface is lower than 64 samples. In the proposed unified OBMC, since the filtering size is decoupled between orientations, this limitation has been adapted. If the width of the CU is less than 8, only 2 columns of pixels are filtered. If the height is less than 8, only 2 lines of pixels are filtered. This allows a better adaptation of the OBMC filtering for thin CUs as shown in Figure 2.

Figure 2. OBMC applied on a 32x4 inter CU a) as achieved in JEM and b) as in proposed unified OBMC.

Test scenario

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 | OBMC as described in 3.1 | Yinji Piao (Samsung) | Panasonic |
| 2 | OBMC as described in 3.2 | Xiu, Xiaoyu (InterDigital) |  |
| 3 | OBMC as described in 3.2 | Antoine Robert (Technicolor) |  |

CE10.3: Non-rectangular partitions

JVET-J0020 Panasonic – Triangular Prediction unit mode

A new triangular partition is introduced for motion compensated prediction. A CU can be split using two triangular prediction units, either in diagonal or inverse diagonal direction. Each triangular prediction unit in the CU is inter-predicted using its own motion vectors and reference frame index which are derived from a merge candidate list. A weighting process is applied to the diagonal or inverse diagonal edge between the two triangular prediction units to derive the final prediction for the whole CU using the prediction pixels of the two triangular prediction units. Triangular partitioning is only applied to motion compensated prediction, which means that the transform and quantization process is applied to the whole CU formed by combining together the two triangles.

The triangular prediction unit mode is only applied to CUs in skip or merge mode. For a CU coded in a skip or a merge mode, a CU level flag is signalled to indicate whether the triangular prediction unit mode is applied or not for the current CU. When the triangular prediction unit mode is applied to the CU, an index indicating the direction of splitting the CU into two triangles and the motion vectors of the two triangles is signalled.

JVET-J0023 RWTH Aachen University – Geometric partitioning (GEO)

Geometric partitioning allows the splitting of a block into two wedge shaped prediction units / segments. Each segment may be inter- or intra predicted and the final result is composed using an overlapped wedge / feathering filter. The geometric partitioning is signalled by a flag on the CU level, which is followed by additional syntax elements required to specify the partitioning line.

JVET-J0023 includes an additional specialized transform (Shape-Adaptive DCT) which is not considered in the context of this this CE.

Test scenario

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 | Triangular Prediction unit mode as described in 4.1 - Explicit signal - Two triangular PUs - Weighting prediction for triangle boundary | Ru-Ling Liao (Panasonic) | Samsung |
| 2 | Geometric partitioning as described in 4.2 | Max Blaeser (RWTH Aachen University) | Tencent |

CE10.4: Diffusion filtering of inter- and intra-prediction signals

JVET-J0038 HHI

The aim of this experiment is to study the filtering of the inter- or intra-prediction signal by a signal adaptive diffusion filter as proposed by HHI in JVET-J0038. Two types of diffusion filters are proposed. Either a linear diffusion filter, which filters the prediction signal uniformly in all directions, or a non-linear diffusion filter, whose goal is to perform a smoothing while preserving significant edges. The diffusion filtering is implemented by applying n correlations of the prediction signal with a mask that depends on the prediction signal in the case of nonlinear diffusion.

A flag is signaled in the bitstream that indicates whether diffusion is to be applied. If diffusion is to be applied, additional diffusion parameters are either signaled in the bitstream or merged in the case of inter prediction in merge mode.

Test scenario

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 | Diffusion filtering as described in 5.1 Fast Encoder Decisions | Jonathan Pfaff (HHI) | Ericsson |
| 2 | Diffusion filtering as described in 5.1 - More Extensive Search | Jonathan Pfaff (HHI) | Ericsson |
| 3 | Diffusion filtering as described in 5.1 - Fast Encoder Decisions - No neighboring block samples used | Jonathan Pfaff (HHI) | Ericsson |
| 4 | Diffusion filtering as described in 5.1 - More Extensive Search - No neighboring block samples used | Jonathan Pfaff (HHI) | Ericsson |

Test conditions and evaluation criteria

The Test Model [1] will be used as one anchor. The Benchmark Set (BMS) will be used as another anchor, where all the tools in the BMS will be enabled. Proposals will be evaluated using BD rates, encoder run time, decoder time with Common Test Conditions (CTC) [2] as agreed at the San Diego meeting. Each proponent is suggested to provide additional information related to memory bandwidth.

Expected interdependency with other CE

This CE is related to CE3: Intra prediction and mode coding, and CE4: Inter prediction and MV coding. It is recommended to do combined tests with selected outcomes of CE3 and CE4 to provide information for the group.

Time-line and Responsibilities

2018-05-11:	Finalize CE description

2018-05-18:	Deliver software packages to CE group and begin cross-checking

2018-06-15:	Report results to CE group and begin summarizing CE report

2018-xx-xx:	Upload CE report

Reference

[1] Description of Test Model

[2] Common test conditions and software reference configurations

Appedix: Full participant list

| Company/ Organization | Participant Name | Email Address |
| --- | --- | --- |
| MediaTek | Chih-Wei Hsu Yu-Wen Huang ChingYeh Chen Peter Chuang | cw.hsu@mediatek.com yuwen.huang@mediatek.com chingyeh.chen@mediatek.com peter.chuang@mediatek.com |
| HHI | Martin Winken Jonathan Pfaff | martin.winken@hhi.fraunhofer.de jonathan.pfaff@hhi.fraunhofer.de |
| Qualcomm | Kai Zhang Yi-Wen Chen Chun-Chi Chen | zhangkai@qti.qualcomm.com yiwench@qti.qualcomm.com chunchic@qti.qualcomm.com |
| HiSilicon | Jicheng An | anjicheng@hisilicon.com |
| Samsumg | Yinji Piao Kiho Choi Jie Chen | yj1003.piao@samsung.com kiho14.choi@samsung.com jie1222.chen@samsung.com |
| InterDigital | Xiu, Xiaoyu Yuwen He | Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com |
| Panasonic | Ru-Ling Liao Chong Soon Lim | ruling.liao@sg.panasonic.com chongsoon.lim@sg.panasonic.com |
| RWTH Aachen University | Max Blaeser Mathias Wien | blaeser@ient.rwth-aachen.de wien@ient.rwth-aachen.de |
| LGE | Seung Hwan Kim Ling Li Sunmi Yoo | seunghwan3.kim@lge.com l.li@lge.com sunmi.yoo@lge.com |
| Huawei | Xu Wei Wei Hai Tao Yang Jianle Chen An Ji Cheng Zhang Na Zheng Jian Hua | xuweiwei3@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com anjicheng@hisilicon.com zhangna25@huawei.com zhengjianhua@hisilicon.com |
| Tencent | Xiaozhong Xu Guichun Li JingYe | xiaozhongxu@tencent.com guichunli@tencent.com jingye@tencent.com |
| ETRI | Jung Won Kang Sung-Chang Lim | jungwon@etri.re.kr sclim@etri.re.kr |
| Alibaba | YuChen Sun | yuchen.s@alibaba-inc.com |
| Apple | Alexis Tourapis | atourapis@apple.com |
| Technicolor | Leleannec Fabrice | Fabrice.Leleannec@technicolor.com antoine.robert@technicolor.com tangi.poirier@technicolor.com Edouard.francois@technicolor.com fabrice.leleannec@technicolor.com |
| PIXTREE | Jaeseob Shin | jsshin@pixtree.com |
| Netflix | Andrey Norkin | anorkin@netflix.com |
| Sony | Masaru Ikeda | Masaru.Ikeda@sony.com |
| BBC | Saverio Blasi | Saverio.Blasi@bbc.co.uk |
| Canon | Chris Rosewarne | Chris.Rosewarne@cisra.canon.com.au |
| Peking University | Zhao Wang, | zhaowang@pku.edu.cn |
| Nvidia | Ankur | ankur81saxena@gmail.com |
| Yonsei University | Sang-hyo Park | spark@barunict.kr |
| Arris | Panusopone, Krit Yu, Yue Hong, Seungwook Wang, Limin | Krit.Panusopone@arris.com Yue.Yu@arris.com Seungwook.Hong@arris.com Limin.Wang@arris.com |
| ITRI | Po-Han Lin Ching-Chieh Lin | pohan@itri.com JackLin@itri.com; |