| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, USA, 10–20 Apr. 2018 | Document: JVET-J1032r1 |
| --- | --- |

| Title: | Description of Core Experiment 12 (CE12): mapping for HDR content |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: |  |  |  |
| Author(s) or Contact(s): | E. Francois, D. Rusanovskyy, P. Yin | Email: | edouard.francois@technicolor.com dmytror@qti.qualcomm.com pyin@dolby.com |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

The purpose of this CE is to investigate the performance of HDR-related coding tools submitted to the 10th JVET meeting. The considered technologies are adaptive quantization, out-of-loop reshaping and in-loop reshaping. The considered technologies will be compared using objective metrics and using subjective viewing at the next meeting.

# Summary

The Test Model ™ with enabled HDR encoder optimization will be used as an anchor. Tested tools shall be integrated to the TM and configured according to the specified (aligned among proposals) distortion-normalization (bitrate allocation) model.

The following are considered as main goals of this CE:

- Assess performance of the pixel-level mapping (a.k.a. DRA, reshaping, CVT) compared to the QP-based anchor approach (using deltaQP and weighted distortion).

- Assess performance of the out-of-loop and in-loop mapping approaches.

- Assess performance of harmonization of DRA scale and QP for pixel-level mapping.

- Assess performance of the region-based mapping.

- Access performance of loop filter applied in original or in mapped domain of coded video data.

# Participants

| Nr. | Name | Company | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
| 1 | Taoran Lu, Fangjun Pu, Peng Yin | Dolby | tlu@dolby.com pyin@dolby.com | X | X |
| 2 | D.Rusanovskyy, A.K.Ramasubramonian | Qualcomm | Dmytror@qti.qualcomm.com | X | X |
| 3 | Edouard Francois | Technicolor | Edouard.francois@technicolor.com | X | X |
| 4 | Jianhua Zheng Quanhe Yu | HiSilicon | zhengjianhua@hisilicon.com |  | X |
| 5 | Kiran Misra, | Sharp | <misrak@sharplabs.com> |  | X |
| 6 | David Baylon Ajay Luthra | Arris | David.Baylon@arris.com Ajay.Luthra@arris.com |  | X |
| 7 | Minhua Zhou Wade Wan Peisong Chen | Broadcom | minhua.zhou@broadcom.com wade.wan@broadcom.com peisong.chen@broadcom.com |  |  |

| CE12 test nb | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE12-1 | Qualcomm/Technicolor (J0021/J0022) | Dolby |
| CE12-2 | Qualcomm/Technicolor (J0021/J0022) | Dolby |
| CE12-3 | Qualcomm/Technicolor (J0021/J0022) | Arris (tentatively) |
| CE12-4 | Qualcomm/Technicolor (J0021/J0022) | HiSilicon |
| CE12-5 | Dolby (J0015) | Qualcomm |
| CE12-6 | Dolby (J0015) | Sharp |
| CE12-7 | Dolby (J0015) | Technicolor |
| CE12-8 | Dolby (J0015) | Technicolor/Qualcomm (tentatively) |

# Tools description

This CE will study the following tools:

- [JVET-J0021, JVET-J0022]: out-of-loop dynamic range adaptation (DRA) and cross-component DRA

- [JVET-J0021]: same as CEx-1, plus DRA scale and QP harmonization

- [JVET-J0015]: in-loop reshaping

## Anchor: average luma-based adaptive QP adaptation

The CE anchors are based on TM, with luma-based QP adaptation. The approach consists in deriving a local dQP value per CTU, based on the average of luma sample values. The solution is non-normative (the dQP values are explicitly coded in the bitstream).

In addition, chroma QP adaptation described in [1] is used. Chroma QP offsets for Cb and Cr are based on the luma QP and on a factor depending on the capture and representation colour primaries. The model is expressed as:

QPoffsetCb = Clip3-12, 0,Roundccb * k * QP+l

QPoffsetCr = Clip3-12, 0,Roundccr * k * QP+l

where parameters ccb, ccr, k and l depend on the capture and representation colour primaries, as described in annex.

Distortion in the rate-distortion optimization is modified, by using a weighted distortion. The weights are derived from the dQP adaptation values.

## [JVET-J0021, JVET-J0022, Qualcomm, Technicolor]: out-of-loop dynamic range adaptation (DRA) and cross component DRA

Tested scheme implements Color Volume Transform (CVT) as proposed in JVET-J0021/J0022. CVT is applied on input video data, Y’CbCr 4:2:0 picture (10 bits per sample) prior to coding (pre-processing). Inverse CVT is implemented after decoding loop as a post- processing (output process). Parameters of the inverse CVT are encapsulated in PPS of the coded bitstream and signaled every IRAP.

Figure 1. Block diagram of the response in HDR category.

Proposed CVT consist of the following components.

Dynamic Range Adjustment (DRA) is range-adaptive scaling of luma and chroma samples; scaling is applied at the pre-processing stage and its inverse is applied at the decoder side as post-processing. Inverse DRA operations at post-processing are described as:

XY=xY*SYi+OYi

XCb=xCb*SCbj+OCbj

XCr=xCr*SCrk+OCrk

where the terms S and O define the piece-wise linear function of the DRA for a particular dynamic range partition of color component, x is an input sample (with xY, xCb and xCr being the Y, Cb and Cr sample), X is the post-processed sample (with XY, XCb and XCr being the Y, Cb and Cr components of the post-processed sample) and i, j and k are indices of the dynamic range partition to which the Y, Cb and Cr components, respectively, of sample x belong.

Cross-Component DRA (CC-DRA) is carried to preserve luma/chroma bitrate allocation for different brightness ranges following the application of the DRA on input chroma samples. At the decoder side, inverse CC-DRA precedes the inverse DRA process. In this approach, for each decoded chroma sample xCx, a chroma scaling component S2cxY is derived from luma inverse DRA scales SYi of the associated luma sample:

S2cxY=funSYxY

The CC-DRA scale S2cxY is obtained by linear interpolation of luma DRA scales of adjacent intervals, e.g. SYi and SYi+1.

The derived CC-DRA scales are applied to a zero-mean chroma samples xCb and xCr based on the luma values xY associated with these chroma samples.

X2Cx=xCx-2bitDepth-1*S2xY+2bitDepth-1

DRA refinement process aims at compensating the distortion resulting from the compression that may impact the inverse DRA process. The process applies per slice. The inverse mapped signal (YIM,UIM,VIM) is refined as follows:

- For luma component, intra-component scaling is applied, corresponding to the following (achievable using a direct LUT):

Yrefine = Min( 2Blum – 1, LUTrefineY[ YIM  ] * YIM  )

- For chroma components, luma-dependent scaling is applied, corresponding to the following:

Urefine = Max( 0, Min( 2Bchr – 1, 2Bchr-1 + LUTrefineU[ YIMcoloc ] * ( UIM – 2Bchr – 1 ) )

Vrefine = Max( 0, Min( 2Bchr – 1, 2Bchr-1 + LUTrefineV[ YIMcoloc ] * ( VIM – 2Bchr – 1 ) )

where YIMcoloc is the co-located inverse mapped luma sample

The look-up-tables LUTrefineY, LUTrefineU, LUTrefineV are built from piece-wise linear (PWL) tables, that can be signaled per slice. The pivot points of PWL tables are equi-distant, and the tables size is equal to 2S+1, S being a parameter signaled in the slice header. When a table is not signaled for a component, the table previously decoded is used.

## [JVET-J0021, Qualcomm, Technicolor]: out-of-loop dynamic range adaptation with harmonization of DRA scale and QP

In this experiment DRA scales and scalar quantizer harmonization proposed in JVET-J0021/22 is studied. The CC-DRA scale S2cxY is adjusted by a multiplication factor S3cxQP to preserve a luma/chroma bit allocation assumed by the core coding design for a given QP settings introduced by chroma QP offset table.

The CC-DRA applies to chroma samples xCb and xCr as follows:

X2Cx=xCx-2bitDepth-1*S2xY*S3xQP+2bitDepth-1

The term S3cxQP aims at compensating the impact of the luma-based QP derivation, which eventually impacts the chroma QP, noted Qpcx. Qpcx is dependent on luma QP, QpY, similar to Table 8-10 of HEVC specification:

qPi = function( QpY, chromaQpOffsetBitstream )

Qpcx = qPi + chromaQpTable

where qPi is the QP index derived from luma QP value QpY, chromaQpOffsetBitstream is the cumulative chroma QP offset signalled in the bitstream and chromaQpTable is an offset introduced by Table 8-10.

## [JVET-J0015, Dolby]: in-loop reshaping

The in-loop reshaper proposed in JVET-J0015 maps the luma code values based on the slice type received by the decoder. The luma reshaping process is defined as following.

For an intra slice, coding process is performed in the reshaped domain, followed by inverse reshaping before storing in DPB. The simplified HDR intra slice decoding workflow is shown in Figure 2.

Figure 2. HDR Intra slice reconstruction with in-loop Luma Reshaper.

For an inter slice, reshaping is performed within the prediction loop. At encoder, the residue is generated by Yres=FwdLUT[Yi]-FwdLUT[Ypred] instead of the traditional method where the residue is the direct difference between the input and prediction (e.g., Yres=Yi-Ypred). Correspondingly, the decoding reconstruction process (e.g., Yr= Yres+Ypred) is modified by Yr=InvLUT[Yres+FwdLUT[Ypred]]. All the other inter decoding tools are used in the same way as the SDR decoding process, such as entropy decoding, motion compensation, reference picture handling, etc. The simplified HDR inter slice decoding workflow is shown in Figure 3.

Figure 3. HDR Inter slice reconstruction with in-loop Luma Reshaper

Reshaping parameters are transmitted once per IRAP picture.

The in-loop Luma reshaper with the following coding features are also going to be tested:

- Luma-based Chroma QP Offset: to compensate for the Luma signal interaction with the Chroma signal, an additional Chroma QP Offset is used to adjust Chroma quantization parameters for Chroma component decoding. The Chroma QP Offset is derived as follows:

cQPO=-6*log2(FwdLUT'YCU)

where FwdLUT' is the slope (first order derivative) of the FwdLUT and YCU is the average Luma value of the CU.

- Loop Filter application: if the control flag for loop filter optimization is enabled in the SPS, then the reconstructed luma component Yr is first forward reshaped to apply Loop Filtering, then inverse reshaped to store Yf in the DPB.

- Region-of-Interest reshaping: a rectangular area in multiples of CTU size that can be specified as a region of interest (ROI), where reshaping is enabled for the coding units inside the ROI. The ROI location is signaled in the SPS and can be modified at the intra slice level if needed. To avoid boundary artifacts in the intra slice, the inverse reshaping is applied for the ROI before the loop filter.

# Tests to be performed

The following tests aim at evaluating the pixel-level mapping, and at comparing the out-of-loop and in-loop implementations performance.

| CE12-1 | Assess performance of the out-of-loop pixel-level mapping compared to the QP-based anchor approach. J0021/J0022 vs. Anchor |
| --- | --- |
| CE12-2 | Assess performance of the in-loop vs out-of-loop pixel-level mapping. J0021/J0022 vs. J0015, using same DRA/reshaping functions as CE12-1 |
| CE12-3 | Same as CE12-1, with DRA scale and QP harmonization J0021/J0022 with DRA scale and QP harmonization vs. Anchor |
| CE12-4 | Same as CE12-2, with DRA scale and QP harmonization J0021/J0022 vs. J0015, using same DRA/reshaping functions as CE12-1 |

The following tests aim at further evaluating the variants of the in-loop approach.

| CE12-5 | Assess performance of the in-loop luma pixel-level mapping compared to the QP-based anchor approach from J0015, without usage of the Luma-based Chroma QP Offset. J0015 vs. Anchor |
| --- | --- |
| CE12-6 | Assess performance of Luma-based Chroma QP Offset on top of the in-loop luma based reshaping from J0015. J0015 with Luma-based Chroma QP Offset vs. Anchor |
| CE12-7 | Assess performance of Loop Filter application on top of the in-loop reshaping from J0015. Compare application of LF always in original domain, always in mapped domain, or mix of both (as done in J0015). J0015 with Loop Filter application vs. Anchor |
| CE12-8 | Assess performance of Region-of-Interest feature on top of the in-loop reshaping from J0015. J0015 with Region-of-Interest reshaping vs. Anchor with Region-of-Interest QP adaptation |

For all tests, parameters of the mapping/reshaping are derived to be aligned to bitrate redistribution function (deltaQP settings) employed by the HDR anchor.

# Experimental Test Conditions

## Software

TM software shall be used for these experiments.

## Test Sequences, Bit Rates, Coding Conditions

Tests sequences, bit rates, and coding conditions defined in the common tests conditions described in document JVET-J1011 will be used.

The TM with the tools described in section 4.1 will be used as anchor.

Proposed tools will be implemented in the TM and performance will be compared to the TM anchor.

The anchors encoder settings for the luma-based QP adaptation and for the chroma QP adaptation are provided in an informative annex.

## Evaluation of Results

The performance will be evaluated using the common tests conditions described in document JVET- J1011.

In addition, expert viewing assessments will be made during the 11th JVET meeting.

# Time-line

| T1 | 2018-May-04 | Final CE description |
| --- | --- | --- |
| T2 | 4 weeks after TM software availability | Completion of tool integration, test configuration announcement, made software available |
| T3 | 3 weeks before next meeting | Proponents deliver the results and start cross-check |
| T4 | 2 weeks before next meeting | Completion of cross-check Reporting of all results to CE coordinators |
| T5 | 2018-July-02 | Input documents uploaded |
| T6 | 2018-July-06 | CE summary report uploaded |

# Annex – anchor encoder settings for luma-based QP adaptation and for the chroma QP adaptation

## Settings for BT.2100 PQ content

Luma-based QP adaptation will use the table depicted below.

| Luma range (avgY) | dQP |
| --- | --- |
| 0 – 300 | 3 |
| 301 – 366 | 2 |
| 367 – 433 | 1 |
| 434 – 500 | 0 |
| 501 – 566 | -1 |
| 567 – 633 | -2 |
| 634 – 700 | -3 |
| 701 – 766 | -4 |
| 767 – 833 | -5 |
| 834 - 1023 | -6 |

Chroma QP adaptation will be activated with the following settings:

- for content with BT.709 primaries, ccb=1.14 and ccr=1.79

- for content with P3D65 primaries, ccb=1.04 and ccr=1.39

- k = – 0.46 and l = 9.26

## Settings for BT.2100 HLG content

The luma-based QP adaptation will be de-activated for HLG content.

The Chroma QP adaptation will be de-activated for HLG content.