| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0366-v2 |
| --- | --- |

| Title: | CE1 related: Partial CU for picture boundary handling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu Xiang Li Shan Liu | Tel: Email: | {mengxxu, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, partial CU is proposed for picture boundary handling. It is reported that 0.33% and 0.56% luma BD-rate reduction is achieved on average over VTM-1.0, for RA and LDB configurations, respectively.

# Introduction

It is quite common that the picture width or height is not power of 2. Thus at the picture boundary, a CU may not fit the picture boundary and is forced to split into smaller coding blocks, so that each CU will have its area entirely inside the picture boundary.

# Proposed method

In this contribution, partial CU is proposed. A partial CU is defined as a CU which contains both areas inside and outside a picture. For partial CU, only the area inside the picture boundary is considered valid and coded, as shown in Figure 1.

Figure 1 a partial CU with both valid and invalid regions

Therefore, with partial CU, it is not necessary to force split until all smaller CUs fit the picture boundary. A bin is signaled in the boundary CUs to indicate whether quad tree split is used. If not used, another bin is signaled to indicate whether it is binary split or non-split. When it is non-split, it is inferred as partial CU. It is further restricted that the valid CU area has either width or height equal to non-power-of-2. For partial CU, skip mode or CBF zero (no coefficient) is inferred for the valid CU area.

# Simulation results

The proposed method is implemented on BMS-1.0. The VTM RA and LDB configurations are used for the simulation. In current implementation, partial CU is disabled for intra slices.

Reference: BMS-1.0 + VTM RA configuration

Test: BMS-1.0 + VTM RA configuration + proposed method

Reference: BMS-1.0 + VTM LDB configuration

Test: BMS-1.0 + VTM LDB configuration + proposed method

# Conclusions

In this contribution, partial CU is proposed for picture boundary handling. It is reported that 0.33% coding gain for RA and 0.56% coding gain for LDB on average is achieved over VTM-1.0. It is proposed to further study this method in CE.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.1/