| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0031-v3 |
| --- | --- |

| Title: | CE11: Summary Report on Deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin Anand Meher Kotra | Tel: Email: | anorkin@netflix.com anand.meher.kotra@huawei.com |
| Source: | CE11 coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report of Core Experiment 11 on deblocking filtering. Three categories of proposals are covered by this CE, split into three sub-tests. These sub-tests are 1) long-tap deblocking filters, 2) general aspects of deblocking filters, and 3) deblocking at 4x4 block boundaries.

The corresponding compression performance of each coding tool evaluated in CE11 is summarized in this contribution. In addition, answers to questions mentioned in [3], such as complexity of the proposals and cross-checking results are also provided.

The software basis for this CE was VTM-2.0.1. Configurations and test conditions in JVET-K1010 [1] for SDR sequences are used. Results for additional configuration with ALF turned off are also reported. For the subjective viewing, additional encodes with different QPs have been used.

# Test conditions and evaluation criteria

The proposals have been encoded according to two test conditions, which correspond to two anchors.

Anchor-1 is VTM-2.0.1 according to the CTC [1].

Anchor-2 is VTM-2.0.1 with ALF switched off (other conditions are the same as in Anchor 2).

# Tests specification

| Test# | Description | Document# |
| --- | --- | --- |
| 11.1.1 | CE11: Long deblocking filters for luma (CE11.1.1) | JVET-L0072 |
| 11.1.2 | Extended Deblocking Filter (only luma) | JVET-L0380 |
| 11.1.3 | Long deblocking filters (only luma) | JVET-L0102 |
| 11.1.4 | Test on long deblocking filtering from JVET-J0021/K0334 | JVET-L0403 |
| 11.1.5 | CE11: Test results of CE11.1.5 long-tap deblocking filter | JVET-L0062 |
| 11.1.6 | Long-tap deblocking filter for luma and chroma | JVET-L0327 |
| 11.1.7 | Deblocking modifications for Large CUs both luma and chroma | JVET-L0405 |
| 11.1.8 | Longer tap Luma deblocking filter | JVET-L0224 |
| 11.1.9 | CE11: Long deblocking filters for both luma and chroma (CE11.1.9) | JVET-L0072 |
| 11.1.10 | Combination of CE11.1.6 and CE11.1.7 | JVET-L0140 |
| 11.1.11 | Combination of CE11.1.1 and CE11.1.7 | JVET-L0337 |
| 11.2.1 | Higher precision modification for VVC deblocking filter | JVET-L0192 |
| 11.2.2 | Luma-adaptive deblocking filter (QP offset change based on luma level) | JVET-L0414 |
| 11.3.1 | Non-recursive deblocking of luma on 4x4 grid (CE11.3.1) | JVET-L0073 |
| 11.3.2 | Deblocking of sub-block boundaries for luma (CE11.3.2) | JVET-L0074 |
| 11.3.3 | CE2-related: Deblocking for 4 x N and N x 4 block boundaries | JVET-L0225 |
| 11.3.4 | Parallel deblocking for 4 x N and N x 4 block boundaries | JVET-L0103 |
| 11.3.5 | Parallel deblocking filter | JVET-L0170 |

# CE 11.1: long-tap deblocking filter

Proposals in this sub-CE investigate longer deblocking filters. Some proposals in this sub-CE only experiment with luma deblocking, while several proposals test changes in both luma and chroma deblocking.

Common aspect of most proposals is applying longer deblocking filters for blocks of larger sizes. By larger block sizes >=32 samples in the direction orthogonal to the block boundary is usually understood. Some proposals differentiate between blocks of size 16 and >=32 with longer filters applied to larger blocks. Another aspect different among the proposals is applying the same or different length of deblocking filters to the sides of the block boundary. The decisions for choosing strong deblocking filtering (in addition to large block sizes) are studied.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE11.1.1 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-L0072 | Masaru Ikeda Masaru.Ikeda@sony.com; D.Rusanovskyy dmytror@qti.qualcomm.com |
| CE11.1.2 | Kei Kawamura ki-kawamura@kddi.com JVET-L0380 | Woong Il Choi woongil.choi@samsung.com |
| CE11.1.3 | Chia-Ming Tsai chia-ming.tsai@mediatek.com JVET-L0102 | Hyeongmun Jang hm.jang@lge.com |
| CE11.1.4 | Dmytro Rusanovskyy dmytror@qti.qualcomm.com JVET-L0403 | Kenneth Andersson kenneth.r.andersson@ericsson.com |
| CE11.1.5 | Woong IL Choi woongil.choi@samsung.com JVET-L0062 | Kei Kawamura ki-kawamura@kddi.com |
| CE11.1.6 | Masaru Ikeda masaru.ikeda@sony.com JVET-L0327 | Kenneth Andersson kenneth.r.andersson@ericsson.com |
| CE11.1.7 | Kiran Misra misrak@sharplabs.com JVET-L0405 | Masaru Ikeda Masaru.Ikeda@sony.com Adam Wieckowski adam.wieckowski@hhi.fraunhofer.de |
| CE11.1.8 | Anand Meher Kotra Anand.meher.kotra@huawei.com JVET-L0224 | Patrice Onno patrice.onno@crf.canon.fr Woong Il Choi woongil.choi@samsung.com |
| CE11.1.9 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-L0072 | Christian Helmrich christian.helmrich@hhi.fraunhofer.de |
| CE11.1.10 | Masaru Ikeda masaru.ikeda@sony.com Kiran Misra misrak@sharplabs.com JVET-L0140 | Anand Meher Kotra anand.meher.kotra@huawei.com |
| CE11.1.11 | Kenneth Andersson kenneth.r.andersson@ericsson.com misrak@sharplabs.com JVET-L0337 | Jie Zhao jie.zhao@lge.com |

## Complexity of technologies in CE11.1

### Scope of proposals in CE11.1

| Tests | Luma modified (Y/N) | Chroma modified (Y/N) |
| --- | --- | --- |
| CE11.1.1 | Y | N |
| CE11.1.2 | Y | N |
| CE11.1.3 | Y | N |
| CE11.1.4 S1 | Y | N |
| CE11.1.4 S2 | Y | N |
| CE11.1.5 | Y | N |
| CE11.1.6 | Y | Y |
| CE11.1.7 S1 | Y | Y |
| CE11.1.7 S2 | Y | Y |
| CE11.1.8 | Y | N |
| CE11.1.9 | Y | Y |
| CE11.1.10 | Y | Y |
| CE11.1.11 | Y | Y |

### Luma deblocking complexity (CE11.1)

| Tests | Samples from block bound. modified | Samples from block bound. deblocking decision | Max num. oper for filtering per line (add/mult/compar/shift) | Max number of oper. for decision for 8-sample boundary (add/mult/compar/shift) | Num. line buffers |
| --- | --- | --- | --- | --- | --- |
| VTM2.0.1 | 3+3 | 4+4 | 56 (28/2/12/14) | 94 per 8 line segment | 4 |
| CE11.1.1 | 7+7, 5+5 | 16 | 120 (46,24,28,22) | 30 (15,0,13,2) per line for 32x32 blocks * 8 = + 240 op | 8 |
| CE11.1.2 | 7+7 | 7+7 | 246 (138, 12, 28, 68) | 20(10,0,6,4) per 4 line segment* 2 segments = + 20 op | 8 |
| CE11.1.3 | 7+7, 7+4, 4+7, 4+4 | 16 | 266 (168, 34, 28, 36) | 25 (12,0,11,2) per line | 8 |
| CE11.1.4 S1 | 4+4/4+3 | 6+6/6+4 | 123 (56/39/0/28) | 32(20/0/2/10) per line*4 = + 120op | 6 |
| CE11.1.4 S2 | 4+4/4+3 | 6+6/6+4 | 123 (56/39/0/28) | 32(20/0/2/10) per line* 4 = + 128 op | 6 |
| CE11.1.5 | 7+7, 3+7 | 16 | 190(124/12/0/54) | 5(3/0/1/1) per line* 4 = + 20 op | VTM |
| CE11.1.6 | 7+7 | 16 | 146 (104,0,28,14) | 48(25,0,14,9) per line* 4 =+ 192 op | 8 |
| CE11.1.7 S1 | 7+7, 7+3, 3+7 | 16, 12 | 97* (55, 0, 28, 14) | Worst case is same as HEVC. 20 (11, 0, 5, 4) * 4 = + 80 op | 8 |
| CE11.1.7 S2 | 7+7, 7+3, 3+7 | 16, 12 | 97* (55, 0, 28 14) | Worst case is same as HEVC. 20 (11, 0, 5, 4) per line*4 = + 80 op | 8 |
| CE11.1.8 | 7+7, 3 + 7 for hor. edges between CTUs | 16 | 142 (96/4/0/42) | 20(14, 2,2,2) per line * 4 lines = + 80 op | VTM |
| CE11.1.9 | 7+7, 5+5 | 16 | 120 (46,24,28,22) | 30 (15,0,13,2) per line for 32x32 blocks* 8 = + 240 op | 8 |
| CE11.1.10 | 7+7, 7+3, 3+7 | 16, 12 | 97* (55, 0, 28, 14) | Worst case is same as HEVC. 20 (11, 0, 5, 4) per line*4 = + 80 op | 8 |
| CE11.1.11 | 7+7, 7+3, 3+7 | 16, 12 | 120 (46,24,28,22) | Worst case is same as HEVC. 20 (11, 0, 5, 4) per line*4 = + 80 op | 8 |

* Numbers obtained with taking into account certain possible optimizations

### Chroma deblocking complexity (CE11.1)

| Tests | Samples from block bound. modified | Samples from block bound. for deblocking decision | Max num. oper for filtering per line (add/mult/compar/shift) | Max number of oper. for decision for 8-sample boundary (add/mult/compar/shift) | Num. line buffers |
| --- | --- | --- | --- | --- | --- |
| VTM2.0.1 | 1+1 | None | 10 (6/0/2/2) | None | 2 |
| CE11.1.1 | VTM | VTM | VTM | VTM | VTM |
| CE11.1.2 | VTM | VTM | VTM | VTM | VTM |
| CE11.1.3 | VTM | VTM | VTM | VTM | VTM |
| CE11.1.4 S1 | VTM | VTM | VTM | VTM | VTM |
| CE11.1.4 S2 | VTM | VTM | VTM | VTM | VTM |
| CE11.1.5 | VTM | VTM | VTM | VTM | VTM |
| CE11.1.6 | 3+3 | 8 | 58(32,0,12,14) | 17 (17, 0, 12, 5)/2 per line | 4 |
| CE11.1.7 S1 | 3+3 | 8 | 41 (21, 0, 12, 8) | 17 (17, 0, 12, 5)/2 per line | 4 |
| CE11.1.7 S2 | 3+3 | 8 | 41 (21, 0, 12, 8) | 17 (17, 0, 12, 5)/2 per line | 4 |
| CE11.1.8 | VTM | VTM | VTM | VTM | VTM |
| CE11.1.9 | 3+3, 2+2, 2+1,1+2, 1+1 | 8 | 52 | 24 (9,0,13,2) per line | 4 |
| CE11.1.10 | 3+3 | 8 | 41 (21, 0, 12, 8) | 17 (17, 0, 12, 5)/2 per line | 4 |
| CE11.1.11 | 3+3 | 8 | 41 (21, 0, 12, 8) | 17 (17, 0, 12, 5)/2 per line | 4 |

### Parallel processing capabilities (CE11.1)

| Tests | Min unit size in luma samples that can be processed separately |
| --- | --- |
| CE11.1.1 | 8x8 grid (boundary segment size 16 samples, min block sizes P16x16/Q16x16 for 5+5, P32x32/Q32x32 for 7+7) |
| CE11.1.2 | 8x8 |
| CE11.1.3 | 8x8 grid |
| CE11.1.4 S1 | 4x4 grid, 8x8 block asymmetric filtering: filter support 4 for P8/Q8, filter support 6 for longer P/Q |
| CE11.1.4 S2 | 4x4 grid, 8x8 block asymmetric filtering: filter support 4 for P8/Q8, filter support 6 for longer P/Q |
| CE11.1.5 | 8x8 grid |
| CE11.1.6 | 8x8 grid |
| CE11.1.7 S1 | 8x8 grid |
| CE11.1.7 S2 | 8x8 grid |
| CE11.1.8 | 8 x 8 grid (longer tap filter only applied when both blocks sharing the edge have a size >=16 samples) |
| CE11.1.9 | 8x8 grid (Luma: boundary segment size 16 samples, min block sizes P16x16/Q16x16 for 5+5, P32x32/Q32x32 for 7+7; Chroma: boundary segment size 1 sample, min block sizes P2x4/Q2x4 for 1+1, P2x8/Q2x8 for 2+2, P2x16/Q2x16 for 3+3) |
| CE11.1.10 | 8x8 grid |
| CE11.1.11 | 8x8 grid |

## Objective results of technologies in CE 11.1

### VTM 2.0.1 CTC (ALF on)

| Test | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.1.1 | 0.0% | 0.0% | 0.0% | 106%* | 99% | 0.0% | 0.0% | -0.1% | 99% | 100% |
| CE11.1.2 | 0.2% | 0.0% | 0.0% | 100% | 101% | 0.1% | 0.0% | 0.0% | 100% | 101% |
| CE11.1.3 | 0.0% | 0.0% | 0.0% | 100% | 101% | 0.0% | 0.0% | 0.0% | 100% | 103% |
| CE11.1.4 S1 | 0.2% | 0.0% | 0.0% | 100% | 100% | -0.2% | -0.1% | -0.3% | 99% | 100% |
| CE11.1.4 S2 | 0.0% | 0.0% | 0.0% | 99% | 100% | -0.1% | -0.1% | -0.2% | 99% | 100% |
| CE11.1.5 | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | 0.0% | -0.1% | 100% | 100% |
| CE11.1.6 | 0.0% | -0.7% | -0.6% | 100% | 102% | -0.1% | -1.2% | -1.1% | 100% | 103% |
| CE11.1.7 S1 | 0.0% | 1.0% | 1.3% | 102% | 102% | 0.2% | 1.0% | 1.1% | 100% | 102% |
| CE11.1.7 S2 | 0.0% | 0.4% | 0.4% | 102% | 102% | 0.2% | 0.3% | 0.3% | 100% | 102% |
| CE11.1.8 | 0.0% | 0.0% | 0.0% | 100% | 102% | 0.0% | 0.0% | 0.0% | 99% | 101% |
| CE11.1.9 | 0.0% | -0.6% | -0.6% | 103%* | 103% | 0.0% | -1.8% | -2.1% | 98%* | 104% |
| CE11.1.10 | 0.0% | -0.7% | -0.6% | 100% | 100% | 0.1% | -1.1% | -1.2% | 99% | 103% |
| CE11.1.11 | 0.0% | -0.4% | -0.4% | 103% | 102% | 0.3% | -0.4% | -0.4% | 96% | 104% |

| Test | LD-B |  |  |  |  | LD-P |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.1.1 | 0.0% | 0.0% | -0.1% | 101% | 100% | 0.0% | 0.1% | 0.2% | 102%* | 101% |
| CE11.1.2 | 0.1% | -0.1% | -0.1% | 100% | 101% | 0.1% | 0.0% | 0.2% | 100% | 101% |
| CE11.1.3 | 0.0% | -0.1% | -0.1% | 100% | 102% | 0.0% | 0.1% | 0.1% | 100% | 102% |
| CE11.1.4 S1 | -0.2% | -0.3% | -0.4% | 99% | 101% | -0.2% | -0.4% | -0.1% | 100% | 101% |
| CE11.1.4 S2 | -0.1% | -0.2% | -0.4% | 99% | 101% | -0.1% | -0.3% | -0.1% | 100% | 102% |
| CE11.1.5 | 0.1% | 0.0% | -0.2% | 100% | 100% | 0.1% | 0.0% | 0.2% | 100% | 101% |
| CE11.1.6 | -0.1% | -1.5% | -1.7% | 96% | 101% | -0.1% | -1.7% | -1.7% | 100% | 104% |
| CE11.1.7 S1 | 0.5% | 0.5% | 0.4% | 100% | 102% | 0.3% | 0.8% | 0.8% | 100% | 102% |
| CE11.1.7 S2 | 0.4% | 0.3% | 0.0% | 100% | 102% | 0.3% | 0.3% | 0.4% | 102% | 103% |
| CE11.1.8 | 0.1% | -0.1% | -0.3% | 100% | 100% | 0.0% | 0.1% | 0.1% | 100% | 100% |
| CE11.1.9 | -0.1% | -2.0% | -2.1% | 101% | 105% | -0.1% | -2.2% | -2.0% | 100% | 104% |
| CE11.1.10 | 0.3% | -1.5% | -1.4% | 96% | 103% | 0.2% | -1.5% | -1.5% | 100% | 101% |
| CE11.1.11 | 0.4% | -0.2% | -0.1% | 99% | 101% | 0.3% | 0.1% | 0.0% | 100% | 98% |

### VTM 2.0.1 ALF off

| Test | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.1.1 | 0.0% | 0.0% | 0.0% | 99% | 99% | -0.1% | 0.0% | 0.0% | 100% | 101% |
| CE11.1.2 | 0.2% | 0.0% | 0.0% | 99% | 100% | 0.1% | -0.1% | -0.1% | 100% | 102% |
| CE11.1.3 | 0.0% | 0.0% | 0.0% | 100% | 102% | 0.0% | 0.0% | 0.0% | 100% | 103% |
| CE11.1.4 S1 | 0.1% | 0.0% | 0.0% | 100% | 101% | -0.3% | -0.2% | -0.3% | 100% | 101% |
| CE11.1.4 S2 | 0.0% | 0.0% | 0.0% | 100% | 99% | -0.1% | -0.1% | -0.2% | 100% | 101% |
| CE11.1.5 | 0.0% | 0.0% | 0.0% | 100% | 101% | -0.1% | 0.0% | 0.0% | 101% | 100% |
| CE11.1.6 | 0.0% | -0.8% | -0.7% | 100% | 103% | -0.1% | -1.4% | -1.2% | 90% | 103% |
| CE11.1.7 S1 | 0.0% | 2.2% | 2.7% | 100% | 106% | 0.1% | 1.4% | 1.5% | 101% | 103% |
| CE11.1.7 S2 | 0.0% | 0.7% | 0.8% | 100% | 106% | 0.1% | 0.5% | 0.5% | 102% | 103% |
| CE11.1.8 | 0.0% | 0.0% | 0.0% | 99% | 103% | -0.1% | 0.0% | 0.0% | 99% | 101% |
| CE11.1.9 | 0.0% | -0.6% | -0.6% | 99% | 103% | -0.1% | -2.2% | -2.4% | 99% | 106% |
| CE11.1.10 | 0.0% | -0.8% | -0.7% | 114% | 103% | 0.0% | -1.4% | -1.3% | 104% | 102% |
| CE11.1.11 | 0.0% | -0.6% | -0.5% | 100% | 102% | 0.2% | -0.5% | -0.4% | 102% | 103% |

| Test | LD-B |  |  |  |  | LD-P |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.1.1 | 0.1% | 0.0% | 0.0% | 100% | 101% | -0.1% | 0.1% | -0.1% | 101% | 101% |
| CE11.1.2 | 0.1% | -0.1% | -0.1% | 100% | 102% | 0.0% | 0.0% | -0.1% | 100% | 101% |
| CE11.1.3 | 0.0% | 0.0% | 0.0% | 100% | 102% | 0.0% | 0.0% | -0.1% | 100% | 102% |
| CE11.1.4 S1 | -0.2% | -0.2% | -0.1% | 100% | 102% | -0.4% | -0.4% | -0.4% | 100% | 101% |
| CE11.1.4 S2 | 0.0% | -0.1% | -0.2% | 100% | 101% | -0.1% | -0.1% | -0.1% | 100% | 101% |
| CE11.1.5 | 0.1% | 0.0% | 0.0% | 100% | 99% | 0.0% | 0.1% | -0.1% | 100% | 103% |
| CE11.1.6 | 0.0% | -1.6% | -2.0% | 99% | 101% | -0.1% | -1.9% | -2.1% | 98% | 103% |
| CE11.1.7 S1 | 0.4% | 0.8% | 1.0% | 102% | 105% | 0.2% | 1.1% | 1.2% | 100% | 103% |
| CE11.1.7 S2 | 0.4% | 0.3% | 0.2% | 102% | 104% | 0.1% | 0.4% | 0.2% | 101% | 103% |
| CE11.1.8 | 0.1% | 0.0% | 0.0% | 100% | 101% | -0.1% | 0.0% | 0.1% | 100% | 101% |
| CE11.1.9 | 0.0% | -1.8% | -2.0% | 100% | 107% | -0.2% | -2.3% | -2.6% | 100% | 105% |
| CE11.1.10 | 0.3% | -1.6% | -1.6% | 94% | 101% | 0.0% | -1.8% | -1.8% | 99% | 101% |
| CE11.1.11 | 0.5% | -0.1% | -0.1% | 100% | 102% | 0.2% | 0.0% | -0.1% | 99% | 102% |

## Cross-checks for CE11.1

| Test | Cross-checker | PSNR& BR Match (Y/N) | RunT matched? (Y/N) | SW studied? (Y/N) | Significant inconsistencies between description and SW? (Y/N) | Studied visual quality? (Y/N) | Other comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CE11.1.1 | M. Ikeda | Y | Y | Y | N | N |  |
|  | D.Rusanovskyy | Y | Y | Y | N | N |  |
| CE11.1.2 | W. I. Choi | Y | Y | Y | N | N |  |
| CE11.1.3 | H. Jang | Y | Y | Y | N | N |  |
| CE11.1.4 S1 | K. Andersson | Y except 1 point with ALF=0 | Y | Y | N | N | Not parallel friendly due to 4x4 grid. Can be solved by asymmetric filtering as in CE11.3.1. |
| CE11.1.4 S2 | K. Andersson | Y for completedso far.. In progress |  | Y | N | N |  |
| CE11.1.5 | K. Kawamura | Y | Y | Y | N | N |  |
| CE11.1.6 | K.Andersson | Y | Y | Y | N | N |  |
| CE11.1.7 S1 | M. Ikeda | Y | Y | Y | N | N |  |
| CE11.1.7 S2 | A. Wieckowski | Y | Y | Y | N | N |  |
| CE11.1.8 | P. Onno | Y | Y | Y | N | N |  |
|  | W. I. Choi | Y | Y | Y | N | N |  |
| CE11.1.9 | C. Helmrich | Y | Y | Y | N | Y | See a comment below |
| CE11.1.10 | A. M. Kotra | Y | Y | Y | N | N |  |
| CE11.1.11 | J. Zhao | Y | Y | Y | N | N |  |

Other comments from cross-checkers

CE11.1.9 (C. Helmrich): BD-WPSNR gains are a bit higher than the corresponding PSNR gains, indicating the visual relevance of CE 11.1.9. Subjectively, cross-checker looked at the RA sequences with the highest BD-(W)PSNR gains, i.e., BasketballDrive, Tango2, and FoodMarket4. The cross-checker sees only very slight visual improvements on the latter two (e. g., the ruby/burgundy colored dress of the Tango2 lady on the far right). 
On BasketballDrive (and BQTerrace, which the cross-checker looked at briefly) the cross-checker sees no improvement at all. Regarding Campfire, the cross-checker confirms that the improvement over CE11.1.1 is quite obvious. The cross-checker reports that with CE11.1.1 and 11.1.9, VTM 2.x now looks much better on Campfire than VTM 1.x.

## Description of technologies in CE11.1

#### CE11.1.1 JVET-L0072

HEVC deblocking filters are extended with longer deblocking filters for luma. One of 7 and 5 sample filtering on respective side of the block boundary can be applied for a 16-sample segment of the block boundary if current and neighboring block size >= 32 and >=16 respectively and corresponding conditions for 8 lines of the boundary are fulfilled as shown below.

| Size | Conditions for each of the 8 lines of a 16 sample segment (line 0, 3, 4, 7, 8, 11, 12, 15) |
| --- | --- |
| 7 | HEVCstrong_weak&& p0-2*p2+p4+q0-2*q2+q4<3*β32 && p0-p2-p3+p5+q0-q2-q3+q5<3*β32&& p0-2*p3+p6+q0-2*q3+q6<3*β32 && p0-p3-p4+p7+q0-q3-q4+q7<3*β32 |
| 5 | HEVCstrong_weak&& p0-p4+q0-q4<β8 && p0-p5+q0-q5<β8 |
| 0-3 | HEVC filter and decisions |

Block boundary samples pi and qi for i=0 to S-1 are then replaced by linear interpolation as follows:

- pi´=(fi*Middles+64-fi*Ps)+32)≫6, clipped to pi±tc

- qi´=(fi*Middles+64-fi*Qs+32)≫6), clipped to qi±tc

where f, Middle, P and Q are given below:

| S | Filter kernels |
| --- | --- |
| 7 | fi=59-i*9, can also be described as f=59,50,41,32,23,14,5 Middle7=(2*po+qo+p1+q1+p2+q2+p3+q3+p4+q4+p5+q5+p6+q6+8)≫4 P7=(p6+p7+1)≫1, Q7=(q6+q7+1)≫1 |
| 5 | fi=58-i*13, can also be described as f=58,45,32,19,6 Middle5=(2*po+qo+p1+q1+p2+q2+p3+q3+p4+q4+8)≫4 P5=(p4+p5+1)≫1, Q5=(q4+q5+1)≫1 |

#### CE11.1.2 JVET-L0380

HEVC deblocking filters are extended with longer deblocking filters for luma. New blocks are shown with red dashed line.

The proposed method consisted of two parts; edge boundary detection of large intra block and extension of the filtered samples on edge filtering process.

When the following conditions are true, the proposed decision process is continued:

- Block boundary is between two blocks of size of >=16 in the direction orthogonal to the boundary.

- The filter type is strong.

- At least one of the blocks forming the boundary is intra coded.

It is noted that a block size is replaced by an edge size. Direction of edge size depends on that of edge boundary like EDGE_VER or EDGE_HOR.

When the above criteria is not true, the default short deblocking filter may be applied.

If the above criteria is true, at block P and block Q, following conditions are checked respectively. If (11.1.2-1) is true, extended filter is applied to block P. In the same manner, if (11.1.2-2) is true, extended filter is applied to block Q.

p60-2*p50+p40+p63-2*p53+p43<β2# SEQ Equation \* ARABIC 11.1.2-1

q60-2*q40+q40+q63-2*q53+q43<β2#11.1.2-2

Following filter kernels are used in the extended filtering process.

p6 = 3,5,2,1,1,1,1,1,1,0,0,0,0,0,0,0
p5 = 1,2,5,2,1,1,1,1,1,1,0,0,0,0,0,0
p4 = 0,1,3,3,3,1,1,1,1,1,1,0,0,0,0,0
p3 = 0,1,2,2,2,2,2,1,1,1,1,1,0,0,0,0
p2 = 0,0,1,2,2,2,2,2,2,1,1,1,0,0,0,0
p1 = 0,0,1,1,2,2,2,2,2,2,1,1,0,0,0,0
p0 = 0,0,0,1,2,2,2,2,2,2,2,1,0,0,0,0

There are no changes for chroma deblocking filter process.

#### CE11.1.3 JVET-L0102

HEVC deblocking filters are extended with longer strong filters for luma. The flowchart of the proposed filtering decisions if shown below.

When all the following conditions are true, the proposed long strong filtering is applied. Otherwise, the HEVC strong filtering is applied for p-side and q-side

- The slice type is inter.

- The CB length at p-side and the CB length at q-side are either larger than or equal to 16.

- Both of the two inter-side difference values representing difference between p-side and q-side are larger than a threshold, inter_side_diff_th. The two inter-side difference values are defined as

- InterSideDiffi =, where i = 0 or 3.

- Both of the two intra-side difference values representing variation within p-side plus variation within q-side are smaller than a threshold, intra_side_diff_th. The two intra-side difference values are defined as

IntraSideDiffi =, where i = 0 or 3.

- Any one of adjacent block edges also satisfies all the above four conditions.

When the long strong filtering decision output is yes, the long strong filter set selection is activated to select one of the three long strong filter sets for p-side and one of the three long strong filter sets for q-side. The selection based on the p-side and q-side length of the current block edge is described as follows, where lenp and lengq denote the CB length at p-side and the CB length at q-side, respectively.

- If “lenp ≥ 32 and lenq ≥ 32”, R8W7-ultra or R8W7-strong filtering is selected as follows:

  - If any one of adjacent block edges of the current block edge is also satisfied “lenp ≥ 32 and lenq ≥ 32”, R8W7-ultra filtering is applied to both p-side and q-side of the current block edge.

  - Otherwise (i.e., “lenp ≥ 32 and lenq ≥ 32” is only true for the current block edge), R8W7-strong filtering is applied to both p-side and q-side of the current block edge.

- If “lenp > 16 and lenq = 16”, R8W7-strong filtering is applied to p-side of the current block edge, and R4W4-strong filtering is applied to q-side of the current block edge.

- If “lenp = 16 and lenq > 16”, R4W4-strong filtering is applied to p-side of the current block edge, and R8W7-strong filtering is applied to q-side of the current block edge.

- If “lenp = 16 and lenq = 16”, R4W4-strong filtering is applied to both p-side and q-side of the current block edge.

The filtered values are clipped to stay in the bounds of -2tc to 2tc of the original values, as in HEVC strong filtering.

If R8W7-ultra filtering is applied to p-side, filtered sample values from  to  are as follows:

If R8W7-ultra filtering is applied to q-side, filtered sample values from  to  are as follows:

If R8W7-strong filtering is applied to p-side, filtered sample values from  to  are as follows:

If R8W7-strong filtering is applied to q-side, filtered sample values from  to  are as follows:

If R4W4-strong filtering is applied to p-side, filtered sample values from  to  are as follows:

If R4W4-strong filtering is selected at q-side, filtered sample values from  to  are as follows:

Within this CE, the performance of the deblocking method described in JVET-K0232, the filter coefficients, and the samples used in long strong filtering are to be studied.

#### CE11.1.4 JVET-L0403

HEVC deblocking filters are extended with longer deblocking filters for luma. The deblocking process tested in CE11.1.4 is similar to that in JEM with a long-tap filter (instead of strong HEVC filter) being conditionally applied to the boundaries of the blocks with size (orthogonal to the filtered boundary) being larger or equal to 16. A long deblocking filter is applied to large blocks boundary samples if smoothness criteria are met on both sides of the boundary:

- HEVC strong filter is selected

- Boundary separates blocks of size larger than 8, with either side being large than 16

- Long filter smoothness criteria return TRUE

The following pseudo code provides an example of implementation:

minBLkSize = min(lengthP, lengthQ);

maxBLkSize = max(lengthP, lengthQ);

if ((maxBLkSize > LARGE_BLOCK_SIZE_DEBLOCK ) && (minBLkSize >= 8))

if ( isSmoothAsymArea() )

{

if (minBLkSize >= LARGE_BLOCK_SIZE_DEBLOCK)

{

ApplyLongSymmetricFiltering()

}

else if ((MODE_INTRA != cuP.predMode) && (MODE_INTRA != cuQ.predMode))

{

ApplyAsymmetricFiltering();

}

}

else

{

ApplyHEVCFiltering();

}

Long filter smoothness criterion ( isSmoothAsymArea() ) is computed as an absolute value of second derivatives computed for P and Q blocks independently and this estimate is compared against the thresholds thresholdP and thresholdQ. Decision making is conducted for group of 4 lines, with only 2 out of these 4 lines are analyzed.

The thresholds thresholdP and thresholdQ are derived from the block size parameters, coded QP value and the type of the slice. Derivation of these threshold values can be tabulated.

The following pseudo code provides an example of implementation of the function isSmoothAsymArea():

function isSmoothAsymArea()
{

countQ = min(LARGE_BLOCK_SIZE_CLASSIFIER, (lengthQ >> 1)) - 2);

countP = min(LARGE_BLOCK_SIZE_CLASSIFIER, (lengthP >> 1)) - 2);

thresholdP = fun (countP, perCountTh, m_deblockSmoothThQP[iQP]);

thresholdQ = fun (countP, perCountTh, m_deblockSmoothThQP[iQP]);

varP = sum(abs(LaplacianOverP))

varQ = sum(abs(LaplacianOverQ))

if (varP < thresholdP && varQ < thresholdQ)

{

return true;

}

else

return false;

}

When long filter is selected, the following long filters are used to modify pixels q0,q1, q2,q3 and  p0,p1, p2,p3, with filter hi is centered at pixel pi, where i=0,1,2,3:

h0=0, 2, 9, 12, 1, 3, 13, 16, 5, 3, 0,

h1=-1, 6, 7, 8, -2, 13, 15, 10, 8, 0, 0,

h2=-1, 4, 12, -3, 6, 14, 14, 16, 2, 0, 0,

h3=-3, 16, -3, 1, 5, 16, 18, 14, 0, 0, 0

It should be noted, that filters hi defined above are shown in 11-tap representation for simplicity of writing, effective filter support area is spanning to 6 boundary samples.

A side of the boundary for which long filter is not applicable either due to length of the filter or not meeting smoothness criteria, is being filtered with HEVC filter.

Clipping is used to limit the amount of filtering. A boundary strength of 3 is used to determine tc for both luma and chroma. Increase tc offset if QP > 42 and decrease if QP<= 35, used for both luma and chroma. Tc is then scaled by 3 for first two samples near the boundary and scaled by 2 for the other samples.

Long filter can be turned on for respective 4 sample segment. Deblocking applied for 4x4 grid for luma.

#### CE11.1.5 JVET-L0062

HEVC deblocking filters are extended with longer deblocking filters for luma. 7 samples on a side of the boundary is filtered when length orthogonal to block boundary is >= 16, strong/weak filter determines use strong filter, sum of absolute pixel difference for pixel p8 and p0 and q8 and q0 compared to a threshold. Only 3 samples are filtered for the upper block when filtering horizontal boundaries. Decision on 4 sample segments of an 8x8 grid.

Following filters are used for left block in vertical filtering:

p6	{3, 5, 2, 1, 1, 1, 1, 1, 1}	      p7 ~ q0

p5	{1, 2, 5, 2, 1, 1, 1, 1, 1, 1}	   p7 ~ q1

p4	{1, 3, 3, 3, 1, 1, 1, 1, 1, 1}	   p6 ~ q2

p3	{1, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1}	p6 ~ q3

p2	{1, 2, 2, 2, 2, 2, 2, 1, 1, 1}	   p5 ~ q3

p1	{1, 1, 2, 2, 2, 2, 2, 2, 1, 1}	   p5 ~ q3

p0	{1, 2, 2, 2, 2, 2, 2, 2, 1}	      p4 ~ q3

For the upper block in horizontal filtering only p0 to p2.

p2    {3, 5, 2, 2, 1, 1, 1, 1}  p3 ~ q3

p1    {2, 2, 4, 2, 2, 2, 1, 1}  p3 ~ q3

p0    {2, 2, 2, 2, 2, 2, 2, 2}  p3 ~ q3

#### CE11.1.6 JVET-L0327

HEVC deblocking filters are extended with longer deblocking filters for luma and chroma. 7 samples on each side of the boundary is filtered when the minimal length orthogonal to block boundary is >= 32, sum of absolute pixel difference and also Laplacians, and comparison with absolute value of pixel difference across boundary in comparison with tc. Decision is taken on two lines of 4 sample segments on a 8x8 grid.

The decision which has two additional conditions is also modified, accordingly. The first one is for large block decision since the proposed filter can be applied when the block width or height which orthogonally crosses the block edge is equal to or larger than 32. The second one is for proposed wide strong filter. The decision is extended based on HEVC strong filter decision (xUseStrongFiltering) as follows.

dp0 = xCalcDP( R[0] ); dp1 = xCalcDP( R[-2] ); dp2 = xCalcDP( R[-4] );

dq0 = xCalcDQ( R[0] ); dq1 = xCalcDQ( R[2] ); dq2 = xCalcDQ( R[4] );

dd0 = 2 * (dp0+dq0); dd2 = 2 * (dp1+dq1); dd4 = 2 * (dp2+dq2);

bdd = ( dd0 < (beta>>2) && dd2 < (beta>>2) && dd4 < (beta>>2) );

dd_strong0 = abs(R[-8] – R[-1]) + abs(R[0] – R[7]);

dd_strong1 = abs(R[-8] – R[-3]) + abs(R[2] – R[7]);

dd_strong2 = abs(R[-8] – R[-5]) + abs(R[4] – R[7]);

dd_strong_flag =

( (dd_strong0<(beta>>3)) && (dd_strong0<(beta>>3)) && (dd_strong0<(beta>>3)) );

If ( dd_strong_flag && bdd && (abs(R[-1]-R[0]) < ((tc*5+1)>>1)) ) is TRUE, and then the following filter is applied to P side in the same with Q side.

p6′= (7*p7+2*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (6*p7+p6+2*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (5*p7+p6+p5+2*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (4*p7+p6+p5+p4+2*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (3*p7+p6+p5+p4+p3+2*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (2*p7+p6+p5+p4+p3+p2+2*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

In this conribution, long-tap filter is also proposed for chroma. The decision which has three additional conditions is also modified, accordingly. The first one is for decision of boundary strength as well as large block. The proposed filter can be applied when the block width or height which orthogonally crosses the block edge is equal to or larger than 8 in chroma sample domain. The second and third one are basically the same as for HEVC luma decison, which are on/off decision and strong filter decision, respectively.

In the first decision, boundary strength (bS) is modified for chroma filtering as follows.

| Conditions | Y | U | V |
| --- | --- | --- | --- |
| At least one of the adjacent blocks is intra | 2 | 2 | 2 |
| At least one of the adjacent blocks has non-zero transform coefficients | 1 | 1 | 1 |
| Absolute difference between the motion vectors that belong to the adjacent blocks is greater than or equal to one integer luma sample | 1 | N/A | N/A |
| Motion prediction in the adjacent blocks refers to vectors is different | 1 | N/A | N/A |
| Otherwise | 0 | 0 | 0 |

The following two decisions are the same as HEVC luma strong filter decision:

The second decision:

d < beta

d = Abs(p2,0−2*p1,0+ p0,0) + Abs( q2,0−2*q1,0+q0,0) + Abs(p2,1−2*p1,1+p0,1) + Abs(q2,1−2*q1,1+q0,1);

The third decision:

xUseStrongFiltering( );    // same as luma’s decision

If those criteria are satisfied, and then the following filter is applied to P side in the same with Q side.

p2′= (3*p3+2*p2+p1+p0+q0+4) >> 3

p1′= (2*p3+p2+2*p1+p0+q0+q1+4) >> 3

p0′= (p3+p2+p1+2*p0+q0+q1+q2+4) >> 3

#### CE11.1.7 JVET-L0405

The proposal incorporates stronger filters for both luma and chroma, compared to JEM deblocking. Additionally, the proposed modification includes a control process that considers the block sizes on both sides of the boundary being deblocked. The stronger filters are used for luma samples that correspond to larger block sizes, while the JEM deblocking filters are still used for luma samples corresponding to the smaller block sizes.  For chroma, the filter selection uses a different approach, and a stronger filter is applied when chroma samples on either sides of the deblocking boundary belong to a large block.

Luma filtering.

The flowchart for a stronger filter decision for luma is provided in the picture below:

Luma is deblocked on 8x8 luma sample grid.

Luma stronger filters (same as JVET-J0038):

p6′= (7*p7+2*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (6*p7+p6+2*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (5*p7+p6+p5+2*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (4*p7+p6+p5+p4+2*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (3*p7+p6+p5+p4+p3+2*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (2*p7+p6+p5+p4+p3+p2+2*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

q6′= (7*q7+2*q6+q5+q4+q3+q2+ q1+q0+p0+ 8) >> 4

q5′= (6*q7+q6+2*q5+q4+q3+q2+q1+q0+p0+p1+ 8) >> 4

q4′= (5*q7+q6+q5+2*q4+q3+q2+q1+q0+p0+p1+p2+ 8) >> 4

q3′= (4*q7+q6+q5+q4+2*q3+q2+q1+q0+p0+p1+p2+p3+ 8) >> 4

q2′= (3*q7+q6+q5+q4+q3+2*q2+q1+q0+p0+p1+p2+p3+p4+8) >> 4

q1′= (2*q7+q6+q5+q4+q3+q2+2*q1+q0+p0+p1+p2+p3+p4+p5+8) >> 4

q0′= (q7+q6+q5+q4+q3+q2+q1+2*q0+p0+p1+p2+p3+p4+p5+p6+8) >> 4

Luma stronger filters are used for large blocks where large block corresponds to width >= 32 for a vertical edge, and height>=32 for a horizontal edge. The control process is further based on gradients computed for two lines of 4 sample segments; comparison of absolute pixel value difference with tc; and comparison of other absolute pixel value differences with β. More gradients are computed for large block side.

- The variables dpq0, dpq3, dp, dq, and d are derived as follows:

dp0, dp3, dq0, dq3 are first derived as in HEVC

if (p side is greater than or equal to 32)

dp0 = ( dp0 + Abs( p5,0 − 2 * p4,0 + p3,0 ) + 1 ) >> 1

dp3 = ( dp3 + Abs( p5,3 − 2 * p4,3 + p3,3 ) + 1 ) >> 1

if (q side is greater than or equal to 32)

dq0 = ( dq0 + Abs( q5,0 − 2 * q4,0 + q3,0 ) + 1 ) >> 1

dq3 = ( dq3 + Abs( q5,3 − 2 * q4,3 + q3,3 ) + 1 ) >> 1

dpq0, dpq3, dp, dq, d are then derived as in HEVC

- As in HEVC, When d is less than β, the following ordered steps apply:

- dpq is derived as in HEVC.

- sp3 = Abs( p3 − p0 ), derived as in HEVC

if (p side is greater than or equal to 32)

sp3 = ( sp3 + Abs( p7 − p3 ) + 1) >> 1

- sq3 = Abs( q0 − q3 ), derived as in HEVC

if (q side is greater than or equal to 32)

sq3 = ( sq3 + Abs( q7 − q3 ) + 1) >> 1

- As in HEVC derive, StrongFilterCondition = (dpq is less than ( β  >>  2 ), sp3 + sq3 is less than ( β  >>  3 ), and Abs( p0 − q0 ) is less than ( 5 * tC + 1 )  >>  1) ? TRUE : FALSE

- When StrongFilterCondition is TRUE, use luma stronger filter on the side with length perpendicular to boundary edge greater than or equal to 32.

Chroma filtering.

Chroma deblocks on a 4x4 chroma sample grid.

Chroma stronger filters (i.e. HEVC luma strong filter):

p0′ = ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 )  >>  3

p1′ = ( p2 + p1 + p0 + q0 + 2 )  >>  2

p2′ = ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 )  >>  3

q0′ = ( p1 + 2 * p0 + 2 * q0 + 2 * q1 + q2 + 4 )  >>  3

q1′ = ( p0 + q0 + q1 + q2 + 2 )  >>  2

q2′= ( p0 + q0 + q1 + 3 * q2 + 2 * q3 + 4 )  >>  3

Two sub-tests for chroma are evaluated.

Sub-test 1. Chroma stronger filters are used on both sides of the block boundary.  Specifically, the strong filter is used when any of the following conditions is true:

- The edge type is vertical an p0 belongs to CU with width >= 16 (chroma samples)

- The edge type is horizontal and p0 belongs to CU with height >= 16 (chroma samples)

- The edge type is vertical and q0 belongs to CU with width >= 16 (chroma samples)

- The edge type is horizontal and q0 belongs to CU with height >= 16 (chroma samples)

Sub-test 2. In another test (b), is same as above, except chroma stronger filters are used on both sides of the block boundary. Specifically, the strong filter is used when HEVC luma strong filter condition when computed for chroma is TRUE and any of the following conditions is true:

- The edge type is vertical an p0 belongs to CU with width >= 16 (chroma samples) and q0 belongs to CU with width >= 16 (chroma samples)

- The edge type is horizontal and p0 belongs to CU with height >= 16 (chroma samples) and q0 belongs to CU with height >= 16 (chroma samples)

| Sub-test 1. | Sub-test 2. |
| --- | --- |

#### CE11.1.8 JVET-L0224

New luma longer tap filters for large blocks are proposed.

The flowchart for the filtering decisions is as follows:

New luma longer tap filters:

Filter condition:                       HEVCstrongweak

&&

p0-p3- p4+p7+q0-q3- q4+q7<3*β32 &&

p0-p2-p3+p5+q0-q2-q3+q5<3*β32

Proposed filter coefficients for longer tap filter:

p6′= (6*p7+3*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (5*p7+p6+3*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (4*p7+p6+p5+3*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (3*p7+p6+p5+p4+3*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (2*p7+p6+p5+p4+p3+3*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (p7+p6+p5+p4+p3+p2+3*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

q6′= (6*q7+3*q6+q5+q4+q3+q2+ q1+q0+p0+ 8) >> 4

q5′= (5*q7+q6+3*q5+q4+q3+q2+q1+q0+p0+p1+ 8) >> 4

q4′= (4*q7+q6+q5+3*q4+q3+q2+q1+q0+p0+p1+p2+ 8) >> 4

q3′= (3*q7+q6+q5+q4+3*q3+q2+q1+q0+p0+p1+p2+p3+ 8) >> 4

q2′= (2*q7+q6+q5+q4+q3+3*q2+q1+q0+p0+p1+p2+p3+p4+8) >> 4

q1′= (q7+q6+q5+q4+q3+q2+3*q1+q0+p0+p1+p2+p3+p4+p5+8) >> 4

q0′= (q7+q6+q5+q4+q3+q2+q1+2*q0+p0+p1+p2+p3+p4+p5+p6+8) >> 4

Proposed longer tap filter condition for horizontal boundaries overlapping with CTU boundaries:

HEVCstrongweak &&

p3-p2-p1+p0+q0-q3- q4+q7<3*β32 &&

p3-p2-p1+p0+q0-q2-q3+q5<3*β32

Proposed longer tap filter coefficients for horizontal boundaries overlapping with CTU boundaries:

p2′= (6*p3+3*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (5*p3+p2+3*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (5*p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

q6′= (6*q7+3*q6+q5+q4+q3+q2+ q1+q0+p0+ 8) >> 4

q5′= (5*q7+q6+3*q5+q4+q3+q2+q1+q0+p0+p1+ 8) >> 4

q4′= (4*q7+q6+q5+3*q4+q3+q2+q1+q0+p0+p1+p2+ 8) >> 4

q3′= (3*q7+q6+q5+q4+3*q3+q2+q1+q0+p0+p1+p2+p3+ 8) >> 4

q2′= (2*q7+q6+q5+q4+q3+3*q2+q1+q0+p0+p1+p2+2*p3+8) >> 4

q1′= (q7+q6+q5+q4+q3+q2+3*q1+q0+p0+p1+p2+3*p3+8) >> 4

q0′= (q7+q6+q5+q4+q3+q2+q1+2*q0+p0+p1+p2+4*p3+8) >> 4

#### CE11.1.9 JVET-L0072

This test is done on top of long-tap luma filter in test CE11.1.1:

Use HEVC luma filter and decisions with following modifications for chroma deblocking filtering.

Determine the number of samples to modify in block P and Q as follows:

lengthP = edge==EDGE_HOR ? (CUP_height_chroma >> 2) : (CUP_width_chroma >>2)

lengthQ = edge==EDGE_HOR ? (CUQ_height_chroma >> 2) : (CUQ_width_chroma >>2)

Only filter boundary if both lengthQ and lengthP is larger than 0.

The HEVC strong filter (reading 4 samples and modifying 3 samples on respective side) is only applied if both sides have a maximum deblocking filter length larger than 2.

Strong/weak filter and number of samples to filter is determined for each line. Inter blocks are only filtered if Laplacian gradients near the boundary on respective side is below the structure threshold “beta” but for intra blocks that check is by passed.

“beta” parameter is divided by 8 compared to what is used for luma.

#### CE11.1.10 Combination of CE11.1.6 and CE11.1.7 JVET-L0140

In this test, the luma part comes from CE11.1.7 and the chroma part from CE11.1.6. The stronger filters are used for luma samples that correspond to larger block sizes, while the JEM deblocking filters are still used for luma samples corresponding to the smaller block sizes. Luma deblocks on 8x8 luma sample grid.

Luma Stronger Filter :

p6′= (7*p7+2*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (6*p7+p6+2*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (5*p7+p6+p5+2*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (4*p7+p6+p5+p4+2*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (3*p7+p6+p5+p4+p3+2*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (2*p7+p6+p5+p4+p3+p2+2*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

Luma stronger filters are used for large blocks where large block corresponds to width >= 32 for a vertical edge, and height>=32 for a horizontal edge. The adjacent block must be greater than or equal to 16. The control process is further based on gradients computed for two lines of 4 sample segments; comparison of absolute pixel value difference with tc; and comparison of other absolute pixel value differences with β. More gradients are computed for large block side.

- The variables dpq0, dpq3, dp, dq, and d are derived as follows:

dp0, dp3, dq0, dq3 are first derived as in HEVC

if (p side is greater than or equal to 32)

dp0 = ( dp0 + Abs( p5,0 − 2 * p4,0 + p3,0 ) + 1 ) >> 1

dp3 = ( dp3 + Abs( p5,3 − 2 * p4,3 + p3,3 ) + 1 ) >> 1

if (q side is greater than or equal to 32)

dq0 = ( dq0 + Abs( q5,0 − 2 * q4,0 + q3,0 ) + 1 ) >> 1

dq3 = ( dq3 + Abs( q5,3 − 2 * q4,3 + q3,3 ) + 1 ) >> 1

dpq0, dpq3, dp, dq, d are then derived as in HEVC

- As in HEVC, When d is less than β, the following ordered steps apply:

- dpq is derived as in HEVC.

- sp3 = Abs( p3 − p0 ), derived as in HEVC

if (p side is greater than or equal to 32)

sp3 = ( sp3 + Abs( p7 − p3 ) + 1) >> 1

- sq3 = Abs( q0 − q3 ), derived as in HEVC

if (q side is greater than or equal to 32)

sq3 = ( sq3 + Abs( q7 − q3 ) + 1) >> 1

- As in HEVC derive, StrongFilterCondition = (dpq is less than ( β  >>  2 ), sp3 + sq3 is less than ( β  >>  3 ), and Abs( p0 − q0 ) is less than ( 5 * tC + 1 )  >>  1) ? TRUE : FALSE

- When StrongFilterCondition is TRUE, use luma stronger filter on the side with length perpendicular to boundary edge greater than or equal to 32.

Chroma deblocks on a 4x4 chroma sample grid

Use of HEVC luma strong filter is also proposed for chroma when both sides of the chroma edge are greater than or equal to 8 and the following decision with three conditions are satisfied. The first one is for decision of boundary strength as well as large block. The proposed filter can be applied when the block width or height which orthogonally crosses the block edge is equal to or larger than 8 in chroma sample domain. The second and third one are basically the same as for HEVC luma decison, which are on/off decision and strong filter decision, respectively.

In the first decision, boundary strength (bS) is modified for chroma filtering as follows.

| Conditions | Y | U | V |
| --- | --- | --- | --- |
| At least one of the adjacent blocks is intra | 2 | 2 | 2 |
| At least one of the adjacent blocks has non-zero transform coefficients | 1 | 1 | 1 |
| Absolute difference between the motion vectors that belong to the adjacent blocks is greater than or equal to one integer luma sample | 1 | N/A | N/A |
| Motion prediction in the adjacent blocks refers to vectors is different | 1 | N/A | N/A |
| Otherwise | 0 | 0 | 0 |

The second and third decision is basically the same as HEVC luma strong filter decision as follows.

The second decision:

d < beta

d = Abs(p2,0−2*p1,0+ p0,0) + Abs( q2,0−2*q1,0+q0,0) + Abs(p2,1−2*p1,1+p0,1) + Abs(q2,1−2*q1,1+q0,1);

The third decision:

xUseStrongFiltering( );    // same as luma’s decision

If those decisions are satisfied, and then the following filter is applied to P side in the same with Q side.

p0′ = ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 )  >>  3

p1′ = ( p2 + p1 + p0 + q0 + 2 )  >>  2

p2′ = ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 )  >>  3

#### CE11.1.11 Combination of CE11.1.1 and CE11.1.7 JVET-L0337

The filtering method in CE11.1.1 (JVET-K0307) is modified to enable use of long asymmetric filters. For long asymmetric filters the number of samples deblocked on the larger block side is greater than that on the smaller block side. The deblocking decision processes described in CE11.1.1 (JVET-K0307) and CE11.1.7 (JVET-K0315) are combined to select from this extended filter set. The extended filter set may be used for strong deblocking. The luma filtering process is mostly adopted from CE11.1.1 whereas the filtering decisions from CE11.1.7 are used.

Luma stronger filters are used when either side has a large block and modified strong filter condition is met. Luma large block corresponds to width >= 32 for a vertical edge, and height>=32 for a horizontal edge.

Luma Stronger Filter :

Block boundary samples pi and qi for i=0 to S-1 are then replaced by linear interpolation as follows:

- pi´=(fi*Middles,t+64-fi*Ps+32)≫6), clipped to pi±tc

- qi´=(gi*Middles,t+64-gi*Qs+32)≫6), clipped to qi±tc

where f_i, Middle_s,t, P_s and Q_s are given below:

| 7, 7 (p side: 7, q side: 7) | fi=59-i*9, can also be described as f=59,50,41,32,23,14,5 gi=59-i*9, can also be described as g=59,50,41,32,23,14,5 Middle7,7=(2*po+qo+p1+q1+p2+q2+p3+q3+p4+q4+p5+q5+p6+q6+8)≫4 P7=(p6+p7+1)≫1, Q7=(q6+q7+1)≫1 |
| --- | --- |
| 7, 3 (p side: 7 q side: 3) | fi=59-i*9, can also be described as f=59,50,41,32,23,14,5 gi=53-i*21, can also be described as g=53,32,11 Middle7,3=(2*po+qo+q0+2*(q1+q2)+p1+q1+p2+p3+p4+p5+p6+8)≫4 P7=(p6+p7+1)≫1, Q3=(q2+q3+1)≫1 |
| 3, 7 (p side: 3 q side: 7) | gi=59-i*9, can also be described as g=59,50,41,32,23,14,5 fi=53-i*21, can also be described as f=53,32,11 Middle3.7=(2*qo+po+p0+2*(p1+p2)+q1+p1+q2+q3+q4+q5+q6+8)≫4 Q7=(q6+q7+1)≫1, P3=(p2+p3+1)≫1 |

The control process is further based on gradients computed for two lines of 4 sample segments; comparison of absolute pixel value difference with tc; and comparison of other absolute pixel value differences with β. More gradients are computed for large block side.

- The variables dpq0, dpq3, dp, dq, and d are derived as follows:

dp0, dp3, dq0, dq3 are first derived as in HEVC

if (p side is greater than or equal to 32)

dp0 = ( dp0 + Abs( p5,0 − 2 * p4,0 + p3,0 ) + 1 ) >> 1

dp3 = ( dp3 + Abs( p5,3 − 2 * p4,3 + p3,3 ) + 1 ) >> 1

if (q side is greater than or equal to 32)

dq0 = ( dq0 + Abs( q5,0 − 2 * q4,0 + q3,0 ) + 1 ) >> 1

dq3 = ( dq3 + Abs( q5,3 − 2 * q4,3 + q3,3 ) + 1 ) >> 1

dpq0, dpq3, dp, dq, d are then derived as in HEVC

- As in HEVC, When d is less than β, the following ordered steps apply:

- dpq is derived as in HEVC.

- sp3 = Abs( p3 − p0 ), derived as in HEVC

if (p side is greater than or equal to 32)

sp3 = ( sp3 + Abs( p7 − p3 ) + 1) >> 1

- sq3 = Abs( q0 − q3 ), derived as in HEVC

if (q side is greater than or equal to 32)

sq3 = ( sq3 + Abs( q7 − q3 ) + 1) >> 1

- As in HEVC derive, StrongFilterCondition = (dpq is less than ( β  >>  2 ), sp3 + sq3 is less than ( β  >>  3 ), and Abs( p0 − q0 ) is less than ( 5 * tC + 1 )  >>  1) ? TRUE : FALSE

- When StrongFilterCondition is TRUE, use luma stronger filter.

Use of HEVC luma strong filter is also proposed for chroma (as chroma stronger filter) when both sides of the chroma edge are greater than or equal to 16 (other threshold, such as 8 may be tested) and HEVC luma strong filter condition when computed for chroma is TRUE.

# CE11.2: General aspects of deblocking filter

The proposals in this sub-CE have not been included into either sub-CE 1 or sub-CE3. In particular proposal CE11.2.1, experiments with derivation of tC table values for different bit depths as well as chroma deblocking. Proposal CE11.2.2 investigates adaptively choosing QP offset for deblocking depending on the average luma values adjacent to the edge.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE11.2.1 | Christophe Gisquet christophe.gisquet@crf.canon.fr JVET-L0192 | Anand Meher Kotra anand.meher.kotra@huawei.com Biao Wang: biao.wang@huawei.com |
| CE11.2.2 | Atsuro Ichigaya ichigaya.a-go@nhk.or.jp JVET-L0414 | Anand Meher Kotra anand.meher.kotra@huawei.com Biao Wang: biao.wang@huawei.com |

## Complexity of technologies in CE11.2

### Luma deblocking complexity (CE11.2)

| Tests | Samples from block bound. modified | Samples from block bound. fdeblocking decision | Max num. oper for filtering per line (add/mult/compar/shift) | Max number of oper. for decision for 8-sample boundary (add/mult/compar/shift) | Num. line buffers |
| --- | --- | --- | --- | --- | --- |
| CE11.2.1 S1 | VTM | VTM | VTM | VTM | VTM |
| CE11.2.1 S2 | VTM | VTM | VTM | VTM | VTM |
| CE11.2.1 S3 | VTM | VTM | VTM | VTM | VTM |
| CE11.2.1 S4 | VTM | VTM | VTM | VTM | VTM |
| CE11.2.1 S5 | VTM | VTM | VTM | VTM | VTM |
| CE11.2.1 S6 | VTM | VTM | VTM | VTM | VTM |
| CE11.2.2 S1 | VTM | VTM | VTM | VTM | VTM |
| CE11.2.2 S2 | VTM | VTM | VTM | VTM | VTM |

### Chroma deblocking complexity ( CE11.2 )

| Tests | Samples from block bound. modified | Samples from block bound. fdeblocking decision | Max num. oper for filtering per line (add/mult/compar/shift) | Max number of oper. for decision for 8-sample boundary (add/mult/compar/shift) | Num. line buffers |
| --- | --- | --- | --- | --- | --- |
| CE11.2.1 S1 | VTM | VTM | VTM | VTM | VTM |
| CE11.2.1 S2 | VTM | VTM | VTM | VTM | VTM |
| CE11.2.1 S3 | VTM | VTM | VTM | VTM | VTM |
| CE11.2.1 S4 | VTM | VTM | VTM | VTM | VTM |
| CE11.2.1 S5 | VTM | VTM | VTM | VTM | VTM |
| CE11.2.1 S6 | 2+2 | VTM | VTM+1 shift+2 add | VTM | VTM |
| CE11.2.2 S1 | VTM | VTM | VTM | VTM | VTM |
| CE11.2.2 S2 | VTM | VTM | VTM | VTM | VTM |

### Parallel processing (CE11.2)

| Tests | Min unit size in luma samples that can be processed separately |
| --- | --- |
| CE11.2.1 S1 | VTM |
| CE11.2.1 S2 | VTM |
| CE11.2.1 S3 | VTM |
| CE11.2.1 S4 | VTM |
| CE11.2.1 S5 | VTM |
| CE11.2.1 S6 | VTM |
| CE11.2.2 S1 | VTM |
| CE11.2.2 S2 | VTM |

## Objective results of technologies in CE11.2

### VTM 2.0.1 CTC (ALF on)

| Test | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.2.1 S1 | 0.0% | -0.7% | -0.5% |  |  | 0.0% | -1.7% | -1.8% |  |  |
| CE11.2.1 S2 | -0.4% | -0.6% | -0.4% |  |  | -0.1% | -0.6% | -0.6% |  |  |
| CE11.2.1 S3 | -0.3% | -0.6% | -0.4% |  |  | -0.1% | -0.4% | -0.3% |  |  |
| CE11.2.1 S4 | -0.3% | -0.6% | -0.4% |  |  | 0.1% | -0.7% | -0.5% |  |  |
| CE11.2.1 S5 | -0.4% | -0.8% | -0.5% |  |  | -0.2% | -1.8% | -1.9% |  |  |
| CE11.2.1 S6 | -0.4% | -1.0% | -0.8% |  |  | -0.2% | -2.3% | -2.4% |  |  |
| CE11.2.2 S1 | 0.0% | 0.0% | 0.0% |  | 98% | 0.0% | 0.1% | -0.1% |  | 99% |
| CE11.2.2 S2 | 0.1% | 0.0% | 0.0% |  | 98% | 0.1% | 0.0% | -0.1% |  | 98% |

| Test | LD-B |  |  |  |  | LD-P |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.2.1 S1 | -0.1% | -1.9% | -2.1% |  |  | -0.1% | -1.9% | -1.8% |  |  |
| CE11.2.1 S2 | -0.2% | -0.3% | -0.3% |  |  | -0.2% | -0.2% | -0.2% |  |  |
| CE11.2.1 S3 | -0.2% | -0.4% | -0.2% |  |  | -0.2% | -0.3% | -0.3% |  |  |
| CE11.2.1 S4 | -0.2% | -0.4% | -0.2% |  |  | 0.0% | -0.2% | -0.1% |  |  |
| CE11.2.1 S5 | -0.3% | -2.0% | -1.9% |  |  | -0.3% | -2.0% | -1.9% |  |  |
| CE11.2.1 S6 | -0.3% | -1.8% | -1.9% |  |  | -0.3% | -1.9% | -1.9% |  |  |
| CE11.2.2 S1 | 0.0% | -0.1% | -0.1% |  | 98% | 0.0% | 0.0% | 0.3% |  | 98% |
| CE11.2.2 S2 | 0.1% | 0.2% | -0.1% |  | 97% | 0.1% | 0.0% | 0.2% |  | 99% |

### VTM 2.0.1 (ALF off)

| Test | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.2.1 S1 | 0.0% | -0.1% | 0.2% |  |  | 0.0% | -2.1% | -2.2% |  |  |
| CE11.2.1 S2 | -0.3% | -0.5% | -0.3% |  |  | 0.0% | -0.8% | -0.6% |  |  |
| CE11.2.1 S3 | -0.2% | -0.5% | -0.3% |  |  | 0.1% | -0.4% | -0.4% |  |  |
| CE11.2.1 S4 | -0.3% | -0.3% | 0.0% |  |  | 0.3% | -0.7% | -0.6% |  |  |
| CE11.2.1 S5 | -0.3% | -0.1% | 0.3% |  |  | 0.0% | -2.3% | -2.2% |  |  |
| CE11.2.1 S6 | -0.3% | -0.5% | -0.1% |  |  | 0.0% | -2.6% | -2.5% |  |  |
| CE11.2.2 S1 | 0.0% | 0.0% | 0.0% | 100% | 99% | 0.0% | -0.1% | 0.0% | 100% | 99% |
| CE11.2.2 S2 | 0.1% | 0.0% | 0.0% | 100% | 99% | 0.0% | 0.0% | 0.0% | 100% | 100% |

| Test | LD-B |  |  |  |  | LD-P |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.2.1 S1 | 0.0% | -1.6% | -1.6% |  |  | -0.1% | -2.0% | -2.2% |  |  |
| CE11.2.1 S2 | -0.1% | -0.3% | -0.3% |  |  | 0.1% | 0.1% | -0.1% |  |  |
| CE11.2.1 S3 | -0.1% | 0.0% | 0.0% |  |  | 0.2% | -0.2% | 0.2% |  |  |
| CE11.2.1 S4 | 0.0% | -0.2% | 0.0% |  |  | 0.6% | -0.1% | 0.3% |  |  |
| CE11.2.1 S5 | -0.2% | -1.6% | -1.6% |  |  | 0.1% | -2.0% | -1.8% |  |  |
| CE11.2.1 S6 | -0.2% | -1.3% | -1.5% |  |  | 0.0% | -2.2% | -2.1% |  |  |
| CE11.2.2 S1 | 0.0% | -0.1% | -0.1% | 99% | 101% | 0.0% | 0.0% | 0.0% | 99% | 101% |
| CE11.2.2 S2 | 0.1% | 0.0% | 0.0% | 100% | 102% | 0.0% | 0.2% | 0.1% | 100% | 101% |

## Cross-checks of tests in CE11.2

| Test | Cross-checker | PSNR&BR Match (Y/N) | RunT matched? (Y/N) | SW studied? (Y/N) | Significant inconsistencies between description and SW? (Y/N) | Studied visual quality? (Y/N) | Other comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CE11.2.1 S1 |  | Y | N | Y | N | N |  |
| CE11.2.1 S2 |  | Y | N | Y | N | N |  |
| CE11.2.1 S3 | A.M. Kotra | N | N | Y | N | N | Due to a VTM encoder bug (see issue #93 on VVC bug tracker) |
| CE11.2.1 S4 | B.Wang | N | N | Y | N | N | Due to a VTM encoder bug (see issue #93 on VVC bug tracker) |
| CE11.2.1 S5 |  | Y | N | Y | N | N |  |
| CE11.2.1 S6 |  | Y | N | Y | N | N |  |
| CE11.2.2 S1 |  | Y | Y | Y | N | N |  |
| CE11.2.2 S2 |  | Y | Y | Y | N | N |  |

Other comments from cross-checkers

## Description of technologies in CE11.2

#### CE11.2.1 Higher-precision modifications to VVC deblocking filters JVET-L0192

S1 (formerly TEST 1): Chroma deblocking decision from luma. Chroma filtering ON/OFF decision switches on the same pixel granularity as luma one does, following CTCs (10 bits). This is related to section 1.2.3.1 of contribution JVET-K0203.

TEST 2: tc parameter link to bit depth

This corresponds to section 1.1 of contribution JVET-K0203. This test requires 3 types of encoding. The goal is to show that the rule from HEVC equation 8-353 is sub-optimal. As a consequence, encoding at 2 bit depths is required, and showcasing “traditional (as in HEVC)” vs “scaling” is required. The following encodings are thus required:

- S2 (10 bits): Per 10 bits CTC, using an “optimal” scaling from a high-precision tc curve

- Encoding with an internal bit depth of 8 bits:

  - S4 (8 bits): Using the 10-bits scaling, and dividing it by 4 to mimic HEVC rule in equation  8-353

  - S3 (8 bits): Proposing an “optimal” 8-bit scaling instead

S5 (formerly TEST 3): combination of TEST1 and TEST2, following CTCs (10 bits)

S6 (formerly TEST 4): TEST3 with longer chroma filter, following CTCs (10 bits)

The addition  in S6 (compared to S5) corresponds to what is described in section 1.3.2.2 of contribution JVET-K0203. Current filtering process for chroma samples (section 8.7.2.5.8 of H.265 v5) is as follows:

Δ = Clip3( −tC, tC, ( ( ( ( q0 − p0 )  <<  2 ) + p1 − q1 + 4 )  >>  3 ) )				(8-405)

p0′ = Clip1C( p0 + Δ )													(8-406)

q0′ = Clip1C( q0 − Δ ) 													(8-407)

The added filtering is:

Δ1 = Δ >> 1

p1′ = Clip1C( p1 + Δ1 )

q1′ = Clip1C( q1 – Δ1 )

#### CE11.2.2 Luma-adaptive deblocking filter JVET-L0414

The variable LL of the reconstructed luma level is derived as follow:

LL= ( ( p0,0 + p0,3 + q0,0 + q0,3 ) >> 2 ) / ( 1 << iBitDepth ),               (1)

where, the sample values pi,k and qi,k with i = 0..3 and k = 0 and 3 are derived the same as HEVC as shown in Figure 1.

Figure 1: Sample position of pi,k and qi,k

The variables QpQ and QpP are set equal to the QpY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

The variable qPL is derived as follows:

qPL = ( ( QPQ + QPP +1 ) >> 1 ) + qpOffset,                    (2)

where qpOffset is specified in Table 1.

The subsequent derivation processes of the variables β and tC are same as HEVC.

Table 1. Specification of qpOffset as a function of LL (Test 1)

| transfer_characteristics == 16 (PQ format) |  | transfer_characteristics == 18 (HLG format) |  | Otherwise (conventional SDR format) |  |
| --- | --- | --- | --- | --- | --- |
| Conditions | qpOffset | Conditions | qpOffset | Conditions | qpOffset |
| LL <= 0.84 | 0 | LL <= 0.8 | 0 | LL <= 0.875 | 0 |
| 0.84 < LL <= 0.91 | 1 | 0.8 < LL <= 0.9 | 1 | 0.875 < LL | 1 |
| 0.91 < LL <= 0.99 | 2 | 0.9 < LL | 2 |  |  |
| 0.99 < LL | 3 |  |  |  |  |

Table 2. Specification of qpOffset as a function of LL (Test2)

| transfer_characteristics == 1 (conventional SDR format) |  | transfer_characteristics == 16 (PQ format) |  | transfer_characteristics == 18 (HLG format) |  |
| --- | --- | --- | --- | --- | --- |
| Conditions | qpOffset | Conditions | qpOffset | Conditions | qpOffset |
| LL≤0.326 | 1 | LL≤0.608 | 2 | LL≤0.504 | 2 |
| 0.326<LL≤0.878 | 0 | 0.608<LL≤0.685 | 1 | 0.504<LL≤0.612 | 1 |
| 0.878 < LL | 1 | 0.685<LL≤0.837 | 0 | 0.612<LL≤0.822 | 0 |
|  |  | 0.837<LL≤0.910 | 1 | 0.822<LL≤0.926 | 1 |
|  |  | 0.910< LL | 2 | 0.926 < LL | 2 |

# CE 11.3: Deblocking at 4x4 block boundaries

The proposals in this CE study application of deblocking filtering to the 4x4 boundaries. Common feature of proposals studied in this CE is limiting number of modified samples and samples accessed for filtering decisions when the block size is 4 samples in the dimension orthogonal to the block boundary. This is done to avoid introducing a dependency of deblocking for an edge on a parallel edge preceding in a filtering order. Test CE11.3.5 investigates whether filtering any edges adjacent to the blocks of size 4 can be avoided.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE11.3.1 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-L0073 | Dmytro Rusanovskyy dmytror@qti.qualcomm.com |
| CE11.3.2 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-L0074 | Chia-Ming Tsai chia-ming.tsai@mediatek.com |
| CE11.3.3 | Anand Meher Kotra Anand.meher.kotra@huawei.com JVET-L0225 | Kiran Misra misrak@sharplabs.com |
| CE11.3.4 | Chia-Ming Tsai chia-ming.tsai@mediatek.com JVET-L0103 | Kenneth Andersson kenneth.r.andersson@ericsson.com |
| CE11.3.5 | Hyeongmun Jang hm.jang@lge.com JVET-L0170 | Chia-Ming Tsai chia-ming.tsai@mediatek.com |

## Complexity of technologies in CE11.3

### Luma deblocking complexity (CE11.3)

| Tests | Samples from block bound. modified | Samples from block bound. for deblocking decision | Max num. oper for filtering per line (add/mult/compar/shift) | Max number of oper. for decision for 8-sample boundary (add/mult/compar/shift) | Num. line buffers |
| --- | --- | --- | --- | --- | --- |
| CE11.3.1 | VTM | VTM | VTM | VTM | VTM |
| CE11.3.2 | VTM | VTM | VTM | VTM | VTM |
| CE11.3.3 S1 | VTM | VTM | VTM | VTM | VTM |
| CE11.3.3 S2 | VTM | VTM | VTM | VTM | VTM |
| CE11.3.4 | VTM | VTM | VTM | VTM | VTM |
| CE11.3.5 | VTM | VTM | VTM | VTM | VTM |

### Chroma deblocking complexity (CE11.3)

| Tests | Samples from block bound. modified | Samples from block bound. for deblocking decision | Max num. oper for filtering per line (add/mult/compar/shift) | Max number of oper. for decision for 8-sample boundary (add/mult/compar/shift) | Num. line buffers |
| --- | --- | --- | --- | --- | --- |
| CE11.3.1 | VTM | VTM | VTM | VTM | VTM |
| CE11.3.2 | VTM | VTM | VTM | VTM | VTM |
| CE11.3.3 S1 | VTM | VTM | VTM | VTM | VTM |
| CE11.3.3 S2 | VTM | VTM | VTM | VTM | VTM |
| CE11.3.4 | VTM | VTM | VTM | VTM | VTM |
| CE11.3.5 | VTM | VTM | VTM | VTM | VTM |

### Parallel processing (CE11.3)

| Tests | Min unit size in luma samples that can be processed separately |
| --- | --- |
| CE11.3.1 | 4x4 grid |
| CE11.3.2 | VTM |
| CE11.3.3 S1 | 4x4 grid |
| CE11.3.3 S2 | 4x4 grid |
| CE11.3.4 | 4x4 grid |
| CE11.3.5 | 4x4 grid |

## Objective results of technologies in CE11.3

### VTM2.0.1 CTC (ALF on)

| Test | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.3.1 | 0.1% | 0.0% | 0.0% | 103%* | 97% | -0.1% | 0.0% | -0.1% | 98%* | 100% |
| CE11.3.2 | 0.0% | 0.0% | 0.0% | 101% | 100% | 0.0% | 0.0% | 0.0% | 98%* | 100% |
| CE11.3.3 S1 | 0.1% | 0.0% | 0.0% | 100% | 103% | -0.1% | 0.0% | -0.1% | 100% | 100% |
| CE11.3.3 S2 | 0.0% | 0.0% | 0.0% | 100% | 103% | -0.2% | 0.0% | -0.1% | 99% | 100% |
| CE11.3.4 | 0.0% | 0.0% | 0.0% | 100% | 100% | -0.2% | 0.0% | 0.0% | 100% | 100% |
| CE11.3.5 | -0.1% | 0.3% | 0.2% | 99% | 98% | 0.0% | 0.1% | 0.0% | 100% | 97% |

| Test | LD-B |  |  |  |  | LD-P |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.3.1 | -0.2% | -0.2% | -0.2% | 99% | 99% | -0.2% | 0.0% | 0.1% | 100% | 100% |
| CE11.3.2 | -0.1% | -0.1% | 0.1% | 98%* | 100% | 0.0% | 0.0% | 0.1% | 98%* | 100% |
| CE11.3.3 S1 | -0.2% | -0.1% | -0.2% | 100% | 100% | -0.2% | 0.1% | 0.2% | 100% | 101% |
| CE11.3.3 S2 | -0.2% | -0.1% | -0.2% | 100% | 100% | -0.2% | 0.0% | 0.1% | 100% | 101% |
| CE11.3.4 | -0.3% | 0.0% | -0.2% | 101% | 100% | -0.3% | 0.0% | 0.0% | 100% | 101% |
| CE11.3.5 | 0.0% | -0.2% | -0.3% | 99% | 95% | 0.0% | 0.0% | 0.1% | 100% | 98% |

### VTM 2.0.1 CTC (ALF off)

| Test | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.3.1 | 0.0% | 0.0% | 0.0% | 101% | 98% | -0.3% | -0.1% | -0.1% | 99% | 99% |
| CE11.3.2 | 0.0% | 0.0% | 0.0% | 99% | 100% | -0.1% | 0.0% | 0.0% | 99% | 99% |
| CE11.3.3 S1 | 0.0% | 0.0% | 0.0% | 100% | 103% | -0.3% | -0.1% | -0.1% | 99% | 101% |
| CE11.3.3 S2 | 0.0% | 0.0% | 0.0% | 99% | 103% | -0.3% | 0.0% | -0.1% | 99% | 100% |
| CE11.3.4 | 0.0% | 0.0% | 0.0% | 100% | 101% | -0.3% | -0.1% | 0.0% | 100% | 101% |
| CE11.3.5 | 0.0% | -0.2% | -0.4% | 100% | 99% | 0.0% | 0.2% | 0.1% | 101% | 100% |

| Test | LD-B |  |  |  |  | LD-P |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.3.1 | -0.2% | -0.1% | -0.1% | 100% | 100% | -0.4% | -0.1% | -0.2% | 101% | 100% |
| CE11.3.2 | -0.1% | 0.0% | -0.1% | 100% | 100% | -0.1% | 0.0% | 0.0% | 99% | 100% |
| CE11.3.3 S1 | -0.2% | -0.1% | -0.1% | 100% | 101% | -0.4% | -0.1% | -0.1% | 100% | 101% |
| CE11.3.3 S2 | -0.3% | -0.1% | -0.1% | 100% | 101% | -0.4% | 0.1% | -0.2% | 99% | 101% |
| CE11.3.4 | -0.2% | 0.0% | -0.1% | 100% | 100% | -0.4% | 0.0% | 0.1% | 100% | 100% |
| CE11.3.5 | 0.0% | -0.2% | -0.3% | 101% | 101% | 0.0% | -0.2% | -0.4% | 100% | 99% |

## Cross-checks of tests in CE11.3

| Test | Cross-checker | PSNR&BR Match (Y/N) | RunT matched? (Y/N) | SW studied? (Y/N) | Significant inconsistencies between description and SW? (Y/N) | Studied visual quality? (Y/N) | Other comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CE11.3.1 | D. Rusanovskyy | Y | Y | Y | N | N |  |
| CE11.3.2 | C.-M. Tsai | Y | Y | Y | N | N |  |
| CE11.3.3 S1 | K. Misra | Y | Y | Y | N | N |  |
| CE11.3.3 S2 | K. Misra | Y | Y | Y | N | N |  |
| CE11.3.4 | K. Andersson | Y | Y | Y | N | N |  |
| CE11.3.5 | C.-M. Tsai | Y | Y | Y | N | N |  |

Other comments from cross-checkers

## Description of technologies in CE11.3

#### CE11.3.1 CU deblocking on 4x4 grid for luma JVET-L0073

Determine the number of samples to modify in block P and Q as follows:

lengthP = edge==EDGE_HOR ? (CUP_height >> 1) – 1 : (CUP_width>>1) - 1

lengthQ = edge==EDGE_HOR ? (CUQ_height >> 1) – 1 : (CUQ_width>>1) - 1

This means that only one sample will be filtered with the weak VVC filter on side P when the distance between boundaries on P side is 4 and only one sample will be filtered with the weak VVC filter on side Q when the distance between the boundaries on Q side is 4.

The weak VVC filter can be used with two sample filtering on at least one side if lengthP or lengthQ is larger than 1. The strong VVC filter can be used if both lengthP and lengthQ are larger than 2.

#### CE11.3.2 Filtering of sub-block boundaries for luma JVET-L0074

Deblocking is done on sub-block boundaries on 8x8 grid for CUs that use ATMVP.

#### CE11.3.3 Deblocking for 4 x N and N x 4 CUs JVET-L0225

Sub-tests: 1 and 2

Two solutions are proposed to be studied here:

Sub-test 1:

When block width/height is equal to 4 for vertical edge/horizontal edge respectively, then a maximum of three samples are used in filter decision and only one sample is modified by the filter. i.e. p2i is used to replace p3i in strong/weak filter condition checks and both strong and weak filter are only allowed to modify p0 and q0

Sub-test 2:

This subtests uses filtering on 4x4 grid with only one sample from the block boundary modified if at least one of the adjacent blocks has the corresponding size of 4.

The steps performed for a vertical edge are illustrated below:

- Check if filter on/off condition is evaluated to true i.e. d < Beta, where d = d0 + d3 and d0 = dp0 + dq0 and d3= dp3 + dq3) is evaluated to true for the respective edge. If true then

- Check if the one of the blocks has a block width of 4 samples, if true, then

- Enforce weak filter with maximum one sample modification. Therefore the following condition is checked δ<10tC where δ=(9*q0-p0- (3*q1-p1+8≫4, and if the condition is evaluated to true, then samples p0 and q0 are modified, otherwise no filtering is applied.

#### CE11.3.4 Parallel deblocking for 4 x N and N x 4 block boundaries JVET-L0103

To support deblocking all 4xN and Nx4 block boundaries and achieve parallel deblocking for all parallel block boundaries, the following changes are made to HEVC deblocking filters (DF):

- When HEVC filtering decision and filtering operations are performed, if the CB length at p-side is equal to four, then p1i is used to replace p2i and p3i.

- When HEVC filtering decision and filtering operations are performed, if the CB length at q-side is equal to four, then q1i is used to replace q2i and q3i.

- When HEVC normal/weak filtering is used and CB length at p-side or q-side is equal to four, then only samples p0i and q0i are modified.

When HEVC strong filtering is used and CB length at p-side or q-side is equal to four, then only samples p1i, p0i, q0i, and q1i are modified.

#### CE11.3.5 Parallel deblocking filter JVET-L0170

Deblocking filter can be processed in parallel because it is not applied to 4x4 TU and PU boundaries. Within this CE, the performance of the parallel deblocking method described in JVET-D0044 is to be studied in terms of subjective quality and computational complexity.

For the luma component, deblocking filtering process is not applied on block boundary of 4xN block for vertical edge and Nx4 block for horizontal edge

For the chroma component, deblocking filtering process is not applied on block boundary of 2xN block for vertical edge and Nx2 block for horizontal edge.

# Annex A. Subjective test.

A subjective test has been performed before the Macao meeting.

## Test material and coding conditions of the subjective test

A formal subjective test was conducted by the JVET test chair Vittorio Baroncini before the Macao meeting.

The following test sequences and conditions have been used in the subjective test. The sequence length is 10 seconds. The following sequences and QP have been used.

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | FoodMarket | 3840x2160p | 60fps | 600 | RA | 39 |
| 02 | Park Running | 3840x2160p | 50fps | 500 | RA | 39 |
| 03 | Campfire | 3840x2160p | 30fps | 300 | RA | 39 |
| 04 | Kimono | 1920x1080p | 24fps | 240 | LDB | 34 |
| 05 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 34 |

The anchors for the subjective viewing were provided by Huawei (Anand). The test conditions for the subjective test were CTC (VTM-2.0.1) except ALF switched off and the base QP used as in the table above.

Two anchors have been used Anchor 1 with ALF switched on and Anchor-2 with ALF switched off.

## Subjective test description

The results of the formal subjective test of the proposals in this CE are reported in JVET-L0611 [4].

# Acknowledgements

The CE would like to thank JVET test chair Vittorio Baroncini and EVATech Labs for performing a formal subjective test for evaluating performance of the proposals in this CE before the Macao meeting.

# References

- “JVET common test conditions and software reference configurations”, JVET-K1010.

- “Algorithm description for Versatile Video Coding and Test Model 2 (VTM2)”, JVET-K1002.

- A. Norkin and A. M. Kotra, "Description of Core Experiment 11 (CE11): Deblocking", JVET-K1031.

- V. Baroncini, A. Norkin, and A.M. Kotra, “Subjective assessment of CE11 proposals”, JVET-L0611.